Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document

LLD: Notification Of SATK File Change

(Edit via File/Properties/Summary!)

| Document Number: | 20_04_04_02483 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.2(Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2005-Jun-1 |
| Last changed: | 2005-Jul-25 by Andrew Spruce |
| File Name: | input.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2005-Jun-1 | Andrew Spruce |  | 0.1 | Draft | 1 |
| 2005-Jul-25 | Andrew Spruce |  | 0.2 | Draft | 2 |

Notes:

- Initial version.

- File renamed.

Table of Contents

LLD: Notification Of SATK File Change	1

1	Introduction	5

1.1	File Change Notification	5

1.2	Existing MFW Implementation	5

1.3	Proposed ACI/ATI Implementation	6

1.4	Proposed EXP/BAT Implementation	6

2	ACI	8

3	ATI	11

4	BAT	13

5	Testing	15

6	Documentation	16

6.1	AT Command Interface Description (8415.052)	16

6.1.1	%SIMEF: SIM EF Update Indication	16

6.1.2	%EFRSLT: SIM EF Update Indication Response	16

6.2	ACI Functional Interface Description (8411.802)	17

6.2.1	T_ACI_EFRSLT_RES	17

6.2.2	T_ACI_SIMEF_MODE	17

6.2.3	sAT_PercentEFRSLT( ) - SIM EF Update Indication Response	17

6.2.4	sAT_PercentSIMEF( ) - SIM EF Update Indication	18

6.2.5	qAT_PercentSIMEF( ) - SIM EF Update Indication	19

6.2.6	rAT_PercentSIMEF( ) - SIM EF Update Indication	19

Appendices	20

A.	Abbreviations	20

# Introduction

During the design phase of the Atlas (XT6) project, it has been determined that enhancements to the ACI, ATI and BAT are required in order to provide notification of SIM elementary file updates.

This document details the work required to implement this feature, which is based on the proposals described in the documents "SIM_FILE_UPDATE_IND handling" by Liyi Yu and "SATK refresh for T&A" by Marcus Oakland. These are summarised below.

## File Change Notification

The Protocol Stack SIM handler sends a SIM_FILE_UPDATE_IND primitive to the ACI with a T_SIM_FILE_UPDATE_IND structure containing the list of the files to be updated.

typedef struct

{

U16 val_nr;               /*<  0:  2> valid entries of a file ID list */

U16 file_id[MAX_FILE_ID]; /*<  2:128> Identifier of an EF             */

U8 _align0;               /*<130:  1> alignment                       */

U8 _align1;               /*<131:  1> alignment                       */

} T_SIM_FILE_UPDATE_IND;

The "file_id" array contains the file IDs for the SIM files that need to be re-read, the "val_nr" indicates the number of valid elements in this array.

The ACI’s psa_sim_file_update_ind( ) function handles the received SIM_FILE_UPDATE_IND primitive, and ultimately causes the response (SIM_FILE_UPDATE_RES) to be sent to the stack.

## Existing MFW Implementation

MFW

ACI

SIM

Registration of callback functions satRefreshNotification( ) and satChangeNotify( ).

SIM_FILE_UPDATE_IND

satChangeNotify( )

satRefreshNotification( )

SIM_FILE_UPDATE_RES

MFW

ACI

SIM

The T_SIM_FILE_UPDATE_IND structure is passed to a callback function, satRefreshNotification( ) that has previously been registered using psaSAT_FUNotifyRegister( ). When the MFW has finished updating the indicated files, or has failed to do so, it calls the ACI function psaSAT_FUConfirm( ). This causes the SIM_FILE_UPDATE_RES primitive to be sent.

## Proposed ACI/ATI Implementation

The ACI and ATI will be enhanced to provide :

A new unsolicited result code (%SIMEF:) that reports the information received in the T_SIM_FILE_UPDATE_IND structure.

A new command (%EFRSLT) that is used to indicate success or failure to update the indicated SIM files.

A new command (%SIMEF) that is used to turn this feature on or off.

APP

ACI/ATI

SIM

AT%SIMEF=1

OK

SIM_FILE_UPDATE_IND

%SIMEF: <ef>,<ef>...

AT%EFRSLT=<res>

SIM_FILE_UPDATE_RES

APP

ACI/ATI

SIM

## Proposed EXP/BAT Implementation

BAT will be enhanced to provide :

A new unsolicited result code (BAT_RES_UNS_PERCENT_SIMEF) that passes the information received in the T_SIM_FILE_UPDATE_IND structure to the EXP.

A new BAT command (BAT_CMD_SET_PERCENT_EFRSLT) that is used by the EXP to indicate success or failure to update the indicated SIM files.

BAT will respond to the new BAT command with BAT_RES_AT_OK if there is a file change notification in progress, or will respond with BAT_RES_PLUS_CME_ERROR if there is not.

EXP

BAT

SIM

SIM_FILE_UPDATE_IND

BAT_RES_UNS_PERCENT_SIMEF

BAT_CMD_SET_PERCENT_EFRSLTT

SIM_FILE_UPDATE_RES

EXP

BAT

SIM

# ACI

Definitions for the command parameters will be added to aci_cmh.h :

typedef enum

{

EFRSLT_RES_FAIL,

EFRSLT_RES_OK

}

T_ACI_EFRSLT_RES;

typedef enum

{

SIMEF_MODE_OFF,

SIMEF_MODE_ON

}

T_ACI_SIMEF_MODE;

RAT_P_SIMEF will be added to RAT_ID in cmh.h, and the corresponding modification to RATJmpTbl in cmh_f.c will be made.

GLOBAL const T_VOID_FUNC RATJmpTbl[RAT_MAX][CMD_MODE_MAX] =

{

.

.

.

CB_VC( PercentCPRI ),   /*RAT_P_CPRI*/

CB_VC( PercentSIMEF ),  /*RAT_P_SIMEF*/

#ifdef _SIMULATION_

,

CB_VC( PercentCNIV )    /*RAT_P_CNIV*/

#endif

};

The SIMEF mode will be added to the T_MM_CMD_PRM structure in cmh.h.

typedef struct mmCmdPrm          /* command parameters related to MM */

{

.

.

.

T_ACI_SIMEF_MODE SIMEFMode;

} T_MM_CMD_PRM;

This new parameter will be initialised with the rest of the T_MM_CMD_PRM values in cmh_Reset( ) in cmh_f.c, to SIMEF_MODE_OFF.

The function psa_sim_file_update_ind( ) in psa_satp.c, which is called when the primitive SIM_FILE_UPDATE_IND is received, will be modified in order to generate the %SIMEF unsolicited event. The function psaSAT_FUConfirm( ) will then only be called if MFW is present - it is expected that any other application will use %EFRSLT.

GLOBAL const void psa_sim_file_update_ind

( T_SIM_FILE_UPDATE_IND *sim_file_update_ind )

{

SHORT i;

BOOL send_confirm_now=TRUE;

TRACE_FUNCTION ("psa_sim_file_update_ind()");

/*

*-------------------------------------------------------------------

* store the primitive and start processing

*-------------------------------------------------------------------

*/

satShrdPrm.fu_ind = sim_file_update_ind;

/*

*   Broadcast %SIMEF unsolicited event to all interested parties.

*/

for(i=CMD_SRC_LCL;i<CMD_SRC_MAX;i++)

{

if ((cmhPrm[i].mmCmdPrm.SIMEFMode==SIMEF_MODE_ON)

|| (aci_cmd_src_mode_get(i)==CMD_MODE_BAT))

{

/*

*   As we have reported the EF update, it is not now the

*   responsibility of this function to send the response

*   immediately.

*/

send_confirm_now=FALSE;

R_AT(RAT_P_SIMEF,i)(

(SHORT *)sim_file_update_ind->file_id,

(UBYTE)sim_file_update_ind->val_nr);

}

}

/*

*   If we didn't report the EF update to anyone we must send

*   the response now.

*/

if (send_confirm_now==TRUE)

psaSAT_FUConfirm (-1, NOT_PRESENT_16BIT);

}

In cmh_sats.c (corresponding prototypes in aci_cmh.h), the functions sAT_PercentEFRSLT( ) and sAT_PercentSIMEF( ) will be implemented.

GLOBAL T_ACI_RETURN sAT_PercentEFRSLT (T_ACI_CMD_SRC srcId,

T_ACI_EFRSLT_RES result)

{

if(!cmh_IsVldCmdSrc(srcId))

{

ACI_ERR_DESC(ACI_ERR_CLASS_Ext,EXT_ERR_Parameter);

return(AT_FAIL);

}

switch(result)

{

case EFRSLT_RES_FAIL:

psaSAT_FUConfirm(simShrdPrm.fuRef,SIM_FU_ERROR);

break;

case EFRSLT_RES_OK:

psaSAT_FUConfirm(simShrdPrm.fuRef,SIM_FU_SUCCESS);

break;

default:

ACI_ERR_DESC(ACI_ERR_CLASS_Ext,EXT_ERR_Parameter);

return(AT_FAIL);

}

return(AT_CMPL);

}

GLOBAL T_ACI_RETURN sAT_PercentSIMEF (T_ACI_CMD_SRC srcId,

T_ACI_SIMEF_MODE mode)

{

if(!cmh_IsVldCmdSrc(srcId))

{

ACI_ERR_DESC(ACI_ERR_CLASS_Ext,EXT_ERR_Parameter);

return(AT_FAIL);

}

switch(mode)

{

case SIMEF_MODE_ON:

case SIMEF_MODE_OFF:

cmhPrm[srcId].mmCmdPrm.SIMEFMode=mode;

break;

default:

ACI_ERR_DESC(ACI_ERR_CLASS_Ext,EXT_ERR_Parameter);

return(AT_FAIL);

}

return(AT_CMPL);

}

In cmh_satq.c (corresponding prototype in aci_cmh.h), qAT_PercentSIMEF( ) will be implemented.

GLOBAL T_ACI_RETURN qAT_PercentSIMEF (T_ACI_CMD_SRC srcId,

T_ACI_SIMEF_MODE *mode)

{

TRACE_FUNCTION ("qAT_PercentSIMEF()");

/*

*   Check command source

*/

if(!cmh_IsVldCmdSrc (srcId))

return(AT_FAIL);

*mode=cmhPrm[srcId].mmCmdPrm.SIMEFMode;

return (AT_CMPL);

}

# ATI

AT_CMD_P_SIMEF and AT_CMD_P_EFRSLT will be added to T_ACI_AT_CMD in aci_cmh.h. The new commands will be added to the 'cmds' table in ati_cmd.c :

LOCAL const ATCommand cmds [] =

{

.

.

.

SAT_CMD("%SIMEF",AT_CMD_P_SIMEF,setatPercentSIMEF,test_gen,queatPercentSIMEF,0)

SAT_CMD("%EFRSLT",AT_CMD_P_EFRSLT,setatPercentEFRSLT,test_gen,0,0)

{NULL,0,0,0,0,0}

};

In ati_sat.c , setatPercentEFRSLT( ), setatPercentSIMEF( ) and queatPercentSIMEF( ) will be added.

GLOBAL T_ATI_RSLT setatPercentEFRSLT(char *cl,UBYTE srcId)

{

T_ACI_EFRSLT_RES res;

T_ACI_RETURN ret;

TRACE_FUNCTION("setatPercentEFRSLT()");

switch(*cl)

{

case '0':

res=EFRSLT_RES_FAIL;

break;

case '1':

res=EFRSLT_RES_OK;

break;

default:

cmdCmeError(CME_ERR_Unknown);

return(ATI_FAIL);

}

ret=sAT_PercentEFRSLT(srcId,res);

return(map_aci_2_ati_rslt(ret));

}

GLOBAL T_ATI_RSLT setatPercentSIMEF(char *cl,UBYTE srcId)

{

T_ACI_SIMEF_MODE mode;

T_ACI_RETURN ret;

TRACE_FUNCTION("setatPercentSIMEF()");

switch(*cl)

{

case '0':

mode=SIMEF_MODE_OFF;

break;

case '1':

mode=SIMEF_MODE_ON;

break;

default:

cmdCmeError(CME_ERR_Unknown);

return(ATI_FAIL);

}

ret=sAT_PercentSIMEF(srcId,mode);

return(map_aci_2_ati_rslt(ret));

}

GLOBAL T_ATI_RSLT queatPercentSIMEF(char *cl, UBYTE srcId)

{

T_ACI_RETURN ret;

T_ACI_SIMEF_MODE mode;

TRACE_FUNCTION("queatPercentSIMEF");

ret=qAT_PercentSIMEF(srcId,&mode);

if (ret==AT_CMPL)

{

sprintf(g_sa,"%s: %d","%SIMEF",mode);

io_sendMessage(srcId,g_sa,ATI_NORMAL_OUTPUT);

}

return(map_aci_2_ati_rslt(ret));

}

In ati_ret.c , rCI_PercentSIMEF( ) will be added.

GLOBAL void rCI_PercentSIMEF(

SHORT *ef,

UBYTE count)

{

UBYTE srcId=srcId_cb;

SHORT pos;

UBYTE n;

TRACE_FUNCTION("rCI_PercentSIMEF()");

pos=sprintf(g_sa,"%s: ","%SIMEF");

for (n=0;n<count;n++)

{

pos+=sprintf(g_sa+pos,"%04X",ef[n]);

if (n!=count-1)

*(g_sa+pos++)=',';

}

io_sendMessage(srcId,g_sa,ATI_NORMAL_OUTPUT);

}

# BAT

The script file cmd_list.log will be modified to include the new messages :

{

%SIMEF : SIM EF Update Indication

[uns rsp

-ef : EF : U16[0..MAX_SIMEF_EF_LEN=64] : man : List of files changed

]

}

{

%EFRSLT : SIM EF Update Indication Response

[cmd

-result : RES : U8 : man : Result of update

0 : FAIL : Update failed

1 : OK   : Update was successful

]

}

This will result in the following structures being added to p_bat.h,

typedef struct

{

U8            _align0;                  /*<  0:  1> alignment */

U8            c_ef;                     /*<  1:  1> counter */

U16           ef[BAT_MAX_SIMEF_EF_LEN]; /*<  2: 64> List of files changed */

U8            _align1;                  /*<130:  1> alignment */

U8            _align2;                  /*<131:  1> alignment */

} T_BAT_res_uns_percent_simef;

typedef struct

{

T_BAT_percent_efrslt_result result;     /*<  0:  4> Result of update */

} T_BAT_cmd_set_percent_efrslt;

and in p_bat_val.h, definitions for BAT_CMD_SET_PERCENT_EFRSLT and BAT_RES_UNS_PERCENT_SIMEF will be created.

In aci_bat.c, the new command and response will be inserted into the appropriate tables :

LOCAL const BATCommand bat_cmds [] =

{

.

.

.

{ /* BAT_CMD_SET_PERCENT_EFRSLT     */ sBAT_PercentEFRSLT  },

static T_map_response_2_size unsolicited_2_size [] =

{

.

.

.

{/* BAT_RES_UNS_PERCENT_SIMEF      */ sizeof(T_BAT_res_uns_percent_simef)},

In aci_bat_cb.c, rBAT_PercentSIMEF( ) will be implemented (corresponding prototype in aci_bat_cb.h and aci_cmh.h) :

GLOBAL void rBAT_PercentSIMEF(

SHORT *ef,

UBYTE count)

{

T_ACI_CMD_SRC src_id;

T_ACI_DTI_PRC_PSI *src_infos;

T_BAT_cmd_response resp;

T_BAT_res_uns_percent_simef simef_data;

UBYTE n;

TRACE_FUNCTION ("rBAT_PercentSIMEF()");

/*

*   Get the source ID and a pointer to the PSI source information.

*/

aci_bat_src_info(&src_id,&src_infos);

resp.ctrl_response=BAT_RES_UNS_PERCENT_SIMEF;

resp.response.ptr_res_percent_simef=&simef_data;

/*

*   Copy the data in. Note that any excess data will be lost, however

*   this should not happen as the BAT message has been designed to

*   carry the entire contents of the T_SIM_FILE_UPDATE_IND primitive.

*/

for (n=0;((n<count) && (n<BAT_MAX_SIMEF_EF_LEN));n++)

{

simef_data.ef[n]=(U16)ef[n];

}

simef_data.c_ef=(U8)n;

aci_bat_send(src_infos,&resp);

}

In aci_bat_sim.c, sBAT_PercentEFRSLT( ) will be implemented (the corresponding prototype will be in aci_bat_cmh.h).

GLOBAL T_ACI_BAT_RSLT sBAT_PercentEFRSLT   (T_ACI_DTI_PRC_PSI *src_infos_psi,

T_BAT_cmd_send    *cmd)

{

T_ACI_EFRSLT_RES result;

T_ACI_BAT_RSLT ret;

TRACE_FUNCTION ("sBAT_PercentEFRSLT()");

/*

*   This relies on T_BAT_percent_efrslt_result being identical to

*   T_ACI_EFRSLT_RES.

*/

result=(T_ACI_EFRSLT_RES)cmd->params.ptr_set_percent_efrslt->result;

/*

*   Call the corresponding sAT function. T_ACI_BAT_RSLT is

*   assumed to be equivalent to T_ACI_RESULT.

*/

ret=(T_ACI_BAT_RSLT)sAT_PercentEFRSLT(src_infos_psi->srcId,result);

return(ret);

}

# Testing

Tapcaller test cases will be written, for example :

T_CASE ACISAT621()

{

BEGIN_CASE ("simef_efrslt_acisat621")

{

power_on_with_various_terminal_profiles_acisat012('F');

SEND (aci_cmd_req("AT%SIMEF=1"));

AWAIT (aci_cmd_ind ("OK"));

SEND (sim_file_update_ind_1('A'));

AWAIT (aci_cmd_ind ("%SIMEF: 6F20"));

SEND (aci_cmd_req("AT%EFRSLT=1"));

AWAIT (sim_file_update_res_1());

AWAIT (aci_cmd_ind ("OK"));

}

}

It is anticipated that the BAT messages will be tested the same way. In order to achieve this it will be necessary to modify the corresponding ATI routines to generate and process BAT messages. As this area is currently under development, and the mechanism may change significantly before the work described in this document is implemented, this will not be detailed at present.

# Documentation

This section describes changes that will be made to existing documents.

## AT Command Interface Description (8415.052)

### %SIMEF: SIM EF Update Indication

| Command | Possible Response(s) |
| --- | --- |
| %SIMEF=<mode> | OK +CME ERROR: <err> |
| %SIMEF? | %SIMEF: <mode> |

Description

This command is used to select or deselect the reporting of EF updates.

When <mode>=1, and the ACI receives the primitive SIM_FILE_UPDATE_IND, unsolicited result code %SIMEF: <ef>,<ef>,<ef>... (where each <ef> is a 4-digit hexadecimal number representing an elementary file ID) is sent to the TE.

Defined Values

<mode> : EF update indication mode

0	Off - EF updates will not be reported.

1	On - EF updates will be reported (and will have to be acknowledged using %EFRSLT).

### %EFRSLT: SIM EF Update Indication Response

| Command | Possible Response(s) |
| --- | --- |
| %EFRSLT=<res> | OK +CME ERROR: <err> |

Description

This command is sent in response to the %SIMEF unsolicited event in order to report the success or otherwise of the update. It causes the ACI to send the primitive SIM_FILE_UPDATE_RES.

Defined Values

<res> : Indicates whether the update was successfully handled or not.

0	Not successfully handled

1	Successfully handled

## ACI Functional Interface Description (8411.802)

The following will be added :

### T_ACI_EFRSLT_RES

| enum type | value | symbolic constant | comment |
| --- | --- | --- | --- |
| T_ACI_EFRSLT_RES | 0 | EFRSLT_RES_FAIL | Refresh failed |
|  | 1 | EFRSLT_RES_OK | Refresh was successful |

### T_ACI_SIMEF_MODE

| enum type | value | symbolic constant | comment |
| --- | --- | --- | --- |
| T_ACI_SIMEF_MODE | 0 | SIMEF_MODE_OFF | No reporting of EF update. |
|  | 1 | SIMEF_MODE_ON | EF update reported, application must respond with %EFRSLT. |

### sAT_PercentEFRSLT( ) - SIM EF Update Indication Response

Command Reference :

8415.052

Function Definition :

T_ACI_RETURN sAT_PercentEFRSLT(

T_ACI_CMD_SRC srcId,

T_ACI_EFRSLT_RES result);

Parameters:

| name | buffer size | comment |  |
| --- | --- | --- | --- |
| srcId | --- | command source identifier | IN |
| result | --- | success or failure of update | IN |

Return:

| symbolic constant | comment |
| --- | --- |
| AT_CMPL | successfully completed |
| AT_FAIL | failed |

### sAT_PercentSIMEF( ) - SIM EF Update Indication

Command Reference :

8415.052

Function Definition :

T_ACI_RETURN sAT_PercentSIMEF(

T_ACI_CMD_SRC srcId,

T_ACI_SIMEF_MODE mode);

Parameters:

| name | buffer size | comment |  |
| --- | --- | --- | --- |
| srcId | --- | command source identifier | IN |
| mode | --- | enable/disable SIM EF update indication | IN |

Return:

| symbolic constant | comment |
| --- | --- |
| AT_CMPL | successfully completed |
| AT_FAIL | failed |

### qAT_PercentSIMEF( ) - SIM EF Update Indication

Command Reference :

8415.052

Function Definition :

T_ACI_RETURN qAT_PercentSIMEF(

T_ACI_CMD_SRC srcId,

T_ACI_SIMEF_MODE *mode);

Parameters:

| name | buffer size | comment |  |
| --- | --- | --- | --- |
| srcId | --- | command source identifier | IN |
| mode | --- | enable/disable SIM EF update indication | OUT |

Return:

| symbolic constant | comment |
| --- | --- |
| AT_CMPL | successfully completed |
| AT_FAIL | failed |

### rAT_PercentSIMEF( ) - SIM EF Update Indication

Command Reference:

8415.052

Function Definition:

void rAT_PercentSIMEF(

SHORT *ef,

UBYTE count);

Parameters:

| name | buffer size | comment |  |
| --- | --- | --- | --- |
| ef | MAX_FILE_ID | pointer to array of elementary file IDs | IN |
| count | --- | number of files changed | IN |

Appendices

  - Abbreviations

This is just an example!:

| ACI | Application Control Interface |
| --- | --- |
| EF | Elementary File |
| EXP | Alcatel Exploitation Layer |
| SAP | Service Access Point |
| SIM | Subscriber Identity Module |