| Date: | 20 May, 2003 |
| --- | --- |
| ID: | 6147.401.97.010 |
| Status: | Being Processed |

| Author: | Condat AG Alt Moabit 90a 10559 Berlin Germany |
| --- | --- |

| Condat Proprietary Information NDA - Confidential Do Not Copy |  |
| --- | --- |

G23-GSM Protocol Stack

SS

Entity Test Specification

GSM

Table of Contents

0	Document Control	3

0.1	Document History	3

0.2	References	3

0.3	Abbreviations	6

0.4	Terms	8

1	Overview	9

2	Parameters	12

3	TEST CASES	17

3.1	Routing (internal)	17

3.1.1	SS000: Setup the routing and PCO view for the SS test	17

3.2	SS establishment	18

3.2.1	SS001: IDLE_1- MO establishment with a facility, requesting MM connection	18

3.2.2	SS002: PENDING_1 - MM connection establishment failure	19

3.2.3	SS004: IDLE_3 - MT establishment, without facility	20

3.2.4	SS005: IDLE_4 - MT establishment, with facility	21

3.2.5	SS011: PENDING_2 - Successful MO establishment, completion	22

3.2.6	SS050: SS002 with Engineering Mode interaction	23

3.3	Error handling	24

3.3.1	SS003: IDLE_2 - Primitives to ignore in idle state	24

3.3.2	SS006: IDLE_5 – MT establishment attempted by false message, FACILITY	25

3.3.3	SS030: IDLE_10 – MT message, RELEASE COMPLETE on unused transaction	26

3.3.4	SS007: IDLE_6 - Establishment with a reserved TI value	27

3.3.5	SS008: IDLE_7 - Establishment with set TI flag	28

3.3.6	SS009: IDLE_8 - Network establishment with a reserved TI	29

3.3.7	SS010: IDLE_9 - Network establishment with set TI flag	30

3.3.8	SS040: IDLE_3 - MT establishment, with erroneous message type	31

3.3.9	SS041: IDLE_3 - MT establishment, with missing mandatory IEI	32

3.3.10	SS042: Error Handling - MT establishment, with FACILITY message containing a incorrect TI value	33

3.3.11	SS043: Error Handling - MT establishment, with message containing a incorrect TI value	34

3.3.12	SS012: PENDING_3 - Error reports during establishment	35

3.3.13	SS025: CONNECTED_13 – Call re-establishment during the transaction	36

3.3.14	SS026: CONNECTED_14 - Network sends a false message	37

3.3.15	SS031: CONNECTED_18 – MT message, RELEASE COMPLETE on unused transaction	38

3.4	Data transfer	39

3.4.1	SS013: CONNECTED_1 – MO transaction, higher layer sends a facility IE	39

3.4.2	SS014: CONNECTED_2 – MT transaction, higher layer sends a facility IE	40

3.4.3	SS019: CONNECTED_7 – MO transaction, network sends a FACILITY message	41

3.4.4	SS020: CONNECTED_8 – MT transaction, network sends a FACILITY message	42

3.5	Connection release	43

3.5.1	SS015: CONNECTED_3 – MO transaction, higher layer releases without facility IE	43

3.5.2	SS016: CONNECTED_4 – MO transaction, higher layer releases with facility IE	44

3.5.3	SS021: CONNECTED_9 - MO transaction, network releases without facility IE	45

3.5.4	SS022: CONNECTED_10 – MT transaction, network releases without facility IE	46

3.5.5	SS023: CONNECTED_11 – MO transaction, network releases with facility IE	47

3.5.6	SS029: CONNECTED_17 – MO transaction, net releases with facility IE, no cause	48

3.5.7	SS024: CONNECTED_12 – MT transaction, network releases with facility IE	49

3.5.8	SS027: CONNECTED_15 – MO transaction, MM releases the connection	50

3.5.9	SS028: CONNECTED_16 – MT transaction, MM releases the connection	51

3.5.10	SS017: CONNECTED_5 – MT transaction, higher layer releases without facility	53

3.5.11	SS018: CONNECTED_6 - MT transaction, higher layer releases with facility	54

# Document Control

ã Copyright Condat AG, 2002-2003

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Condat AG reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Condat AG. Condat AG accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a license agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Condat AG.

Condat AG

Alt Moabit 90a

10559 Berlin

Germany

Telephone:	+49.30.39 49 0

Fax:		+49.30.39 49 1300

Internet:	www.condat.de

## Document History

ID	Author	Date	Status
6147.401.97.001	LE et al.	20 August, 1997	Being Processed

Initial version.

6147.401.97.002	SBK	5 Apil, 2001	Being Processed

Changed to TAP2, corrected TIMEOUTs to MUTEs, improved formal aspects like "Condat AG"

6147.401.97.003	SBK	27 July, 2001	Being Processed

Added TC for Release Complete without cause

6147.401.97.004	HM	04 January, 2002	Being Processed

Made SS050 pass with TAP_PARK=NO

6147.401.97.005	SBK	31 January, 2002	Being Processed

Revert unwanted changes preventing compilation

6147.401.97.006	SBK	31 January, 2002	Being Processed

Switched to new template; add new test cases for improved coverage, i.e. areas not tested yet via PC tests: SS030 and SS031.

6147.401.97.007	SBK	6 February, 2002	Being Processed

Realized Cause Concept. Corrected title of TC SS001.

6147.401.97.008	HM	25 September, 2002	Being Processed

Adaption after SS SAP change (cause concept related).

6147.401.97.009	ZM	27 Mar, 2003	Being Processed

Error handling refined

6147.401.97.010	HM	20 May, 2003	Being Processed

Made some failed testcases pass, specification 4.10 clause 3.7.3 handling in code changed

## References

[1]	GSM 2.81, Line Identification Supplementary Services - Stage 1
	ETS 300 514, ETSI, September 1994

[2]	GSM 2.82, Call Forwarding Supplementary Services - Stage 1
	ETS 300 515, ETSI, September 1994

[3]	GSM 2.83, Call Waiting and Call Hold Supplementary Services - Stage 1
	ETS 300 516, ETSI, September 1994

[4]	GSM 2.84, Multi Party Supplementary Services - Stage 1
	ETS 300 517, ETSI, September 1994

[5]	GSM 2.85, Closed User Group Supplementary Services - Stage 1
	ETS 300 518, ETSI, September 1994

[6]	GSM 2.86, Advice of Charge Supplementary Services - Stage 1
	ETS 300 519, ETSI, September 1994

[7]	GSM 2.88, Call Barring Supplementary Services - Stage 1
	ETS 300 520, ETSI, September 1994

[8]	GSM 3.14, Support of Dual Tone Multi Frequency Signalling via the GSM System
	ETS 300 532, ETSI, April 1994

[9]	GSM 3.40, Technical Realization of the Short Message Service Point-to-Point
	ETS 300 536, ETSI, January 1996

[10]	GSM 3.41, Technical Realization of Short Message Service Cell Broadcast
	ETS 300 537, ETSI, June 1995

[11]	GSM 3.81, Line Identification Supplementary Services - Stage 2
	ETS 300 542, ETSI, February 1995

[12]	GSM 3.82, Call Forwarding Supplementary Services - Stage 2
	ETS 300 543, ETSI, February 1995

[13]	GSM 3.83, Call Waiting and Call Hold Supplementary Services - Stage 2
	ETS 300 544, ETSI, November 1994

[14]	GSM 3.84, Multi Party Supplementary Services - Stage 2
	ETS 300 545, ETSI, November 1994

[15]	GSM 3.85, Closed User Group Supplementary Services - Stage 2
	ETS 300 546, ETSI, January 1996

[16]	GSM 3.86, Advice of Charge Supplementary Services - Stage 2
	ETS 300 547, ETSI, March 1995

[17]	GSM 3.88, Call Barring Supplementary Services - Stage 2
	ETS 300 548, ETSI, November 1994

[18]	GSM 4.01, MS-BSS Interface General Aspects and Principles
	ETS 300 550, ETSI, September 1994

[18a]	GSM 4.03, MS-BSS Interface Channel Structures and Access Capabilities
	ETS 300 552, ETSI, September 1994

[19]	GSM 4.05, Data Link Layer General Aspects
	ETS 300 554, ETSI, September 1994

[20]	GSM 4.06, MS-BSS Interface Data Link Layer Specification
	ETS 300 555, ETSI, September 1994

[21]	GSM 4.07, Mobile Radio Interface Signalling Layer 3 General Aspects
	ETS 300 556, ETSI, February 1995

[22]	GSM 4.08, Mobile Radio Interface Layer 3 Specification
	ETS 300 557, ETSI, January 1996

[23]	GSM 4.10, Mobile Radio Interface Layer 3 Supplementary Services Specification 
	General Aspects
	ETS 300 558, ETSI, February 1995

[24]	GSM 4.11, Point-to-Point Short Message Service Support on Mobile Radio Interface
	ETS 300 559, ETSI, October 1995

[25]	GSM 4.12, Short Message Service Cell Broadcast Support on Mobile Radio Interface
	ETS 300 560, ETSI, January 1996

[26]	GSM 4.80, Mobile Radio Interface Supplementary Services Specification Formats and Coding
	ETS 300 564, ETSI, February 1995

[27]	GSM 4.81, Line Identification Supplementary Services - Stage 3
	ETS 300 565, ETSI, February 1995

[28]	GSM 4.82, Call Forwarding Supplementary Services - Stage 3
	ETS 300 566, ETSI, February 1995

[29]	GSM 4.83, Call Waiting and Call Hold Supplementary Services - Stage 3
	ETS 300 567, ETSI, February 1995

[30]	GSM 4.84, Multi Party Supplementary Services - Stage 3
	ETS 300 568, ETSI, February 1995

[31]	GSM 4.85, Closed User Group Supplementary Services - Stage 3
	ETS 300 569, ETSI, February 1995

[32]	GSM 4.86, Advice of Charge Supplementary Services - Stage 3
	ETS 300 570, ETSI, February 1995

[33]	GSM 4.88, Call Barring Supplementary Services - Stage 3
	ETS 300 571, ETSI, February 1995

[34]	GSM 5.01, Physical Layer on the Radio Path General Description
	ETS 300 573, ETSI, October 1995

[35]	GSM 5.02, Multiplexing and Multiple Access on the Radio Path
	ETS 300 574, ETSI, January 1996

[36]	GSM 5.08, Radio Sub-system Link Control
	ETS 300 578, ETSI, January 1996

[37]	GSM 5.10, Radio Sub-system Synchronisation
	ETS 300 579, ETSI, October 1995

[38]	Service Access Point MMREG
	6147.100.96.100; Condat GmbH

[39]	Service Access Point MNCC
	6147.101.96.100; Condat GmbH

[40]	Service Access Point MNSS
	6147.102.96.100; Condat GmbH

[41]	Service Access Point MNSMS
	6147.103.96.100; Condat GmbH

[42]	Service Access Point MMCC
	6147.104.97.100; Condat GmbH

[43]	Service Access Point MMSS
	6147.105.97.100; Condat GmbH

[44]	Service Access Point MMSMS
	6147.106.97.100; Condat GmbH

[45]	Service Access Point RR
	6147.107.97.100; Condat GmbH

[46]	Service Access Point SIM
	6147.108.97.100; Condat GmbH

[47]	Service Access Point MPH
	6147.109.96.100; Condat GmbH

[48]	Service Access Point DL
	6147.110.96.100; Condat GmbH

[49]	Service Access Point MDL
	6147.111.96.100; Condat GmbH

[50]	Service Access Point PH
	6147.112.97.100; Condat GmbH

[51]	Service Access Point MMI
	6147.113.96.100; Condat GmbH

[52]	Message Sequence Charts CC
	6147.200.97.100; Condat GmbH

[53]	Message Sequence Charts SS
	6147.201.97.100; Condat GmbH

[54]	Message Sequence Charts SMS
	6147.202.97.100; Condat GmbH

[55]	Message Sequence Charts MM
	6147.203.97.100; Condat GmbH

[56]	Message Sequence Charts RR
	6147.204.96.100; Condat GmbH

[57]	Message Sequence Charts DL
	6147.205.96.100; Condat GmbH

[58]	Users Guide
	6147.300.96.100; Condat GmbH

[59]	Test Specification CC
	6147.400.97.100; Condat GmbH

[60]	Test Specification SS
	6147.401.97.100; Condat GmbH

[61]	Test Specification SMS
	6147.402.97.100; Condat GmbH

[62]	Test Specification MM
	6147.403.97.100; Condat GmbH

[63]	Test Specification RR
	6147.404.97.100; Condat GmbH

[64]	Test Specification DL
	6147.405.97.100; Condat GmbH

[65]	Test Specification CCD
	6147.406.97.100; Condat GmbH

[66]	SDL Specification CC
	6147.500.97.100; Condat GmbH

[67]	SDL Specification SS
	6147.501.97.100; Condat GmbH

[68]	SDL Specification SMS
	6147.502.97.100; Condat GmbH

[69]	SDL Specification MM
	6147.503.97.100; Condat GmbH

[70]	SDL Specification RR
	6147.504.97.100; Condat GmbH

[71]	SDL Specification DL
	6147.505.97.100; Condat GmbH

[72]	Message Specification CC
	6147.600.97.100; Condat GmbH

[73]	Message Specification SS
	6147.601.97.100; Condat GmbH

[74]	Message Specification SMS
	6147.602.97.100; Condat GmbH

[75]	Message Specification MM
	6147.603.97.100; Condat GmbH

[76]	Message Specification RR
	6147.604.97.100; Condat GmbH

[77]	Message Specification DL
	6147.605.97.100; Condat GmbH

[78]	Technical Documentation CC
	6147.700.97.100; Condat GmbH

[79]	Technical Documentation SS
	6147.701.97.100; Condat GmbH

[80]	Technical Documentation SMS
	6147.702.97.100; Condat GmbH

[81]	Technical Documentation MM
	6147.703.97.100; Condat GmbH

[82]	Technical Documentation RR
	6147.704.97.100; Condat GmbH

[83]	Technical Documentation DL
	6147.705.97.100; Condat GmbH

[84]	Technical Documentation CCD
	6147.706.97.100; Condat GmbH

## Abbreviations

AGCH	Access Grant Channel

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
C/R	Command / Response
C1	Path Loss Criterion
C2	Reselection Criterion

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station

NCC	National Colour Code
NECI	New Establishment Causes included
N(R)	Receive Number
N(S)	Send Number

OTD	Observed Time Difference

P	Poll Bit
PCH	Paging Channel
PDU	Protocol Description Unit
P/F	Poll / Final Bit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

VPLMN	Visiting Public Land Mobile Network
V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

The GSM specification do not set out any implementation of the Protocol Stack. The following diagrams show the implementation described in all these documents for the mobile station. All entities except the Man Machine Interface and Physical Layer are implemented as part of the Protocol Stack.

- Figure 1: Mobile-station protocol architecture

This document describes the tests for Supplementary Services.

# Parameters

/* DECLARATIONS:  */
DECLARATION (SS_FACILITY_A)
DECLARATION (SS_FACILITY_A_CONTENT)
DECLARATION (SS_FACILITY_A_INF)

DECLARATION (SS_FACILITY_B)
DECLARATION (SS_FACILITY_B_CONTENT)
DECLARATION (SS_FACILITY_B_INF)

DECLARATION (EMPTY_FACILITY)

DECLARATION (SS_VERSION)
DECLARATION (SS_CAUSE_50)
DECLARATION (SS_CAUSE_51)
DECLARATION (SS_CAUSE_16)
DECLARATION (SS_CAUSE_61)
DECLARATION (SS_CAUSE_60)

DECLARATION (SS_VERSION_INDICATOR)
DECLARATION (SS_VERSION_PRIM)
DECLARATION (SS_VERSION_MSG)
DECLARATION (REGISTER_CMD_MID_ERROR)
DECLARATION (REGISTER_CMD_NO_FACILITY)
DECLARATION (FACILITY_CMD_MAND_ERROR)
DECLARATION (FACILITY_CMD_OK_BAD_TI) 
DECLARATION (RELEASE_COMP_CMD_OK_BAD_TI) 
DECLARATION (REGISTER_CMD_OK_BAD_TI)

/* Constants */
SHORT	L_FAC_0	0
SHORT	L_FAC_32	32
SHORT	O_FAC_0	0

/* ti */
BYTE	TI_3	3
BYTE	TI_7	7
BYTE	TI_8	8
BYTE	TI_10	10
BYTE	TI_11	11
BYTE	TI_15	15

/* ss cause */
/* 0x60– Missing Mandatory IEI */
SHORT	CAUSE_60	0x60
/* 0x61 – Message Type Not Implemented */
SHORT	CAUSE_61	0x61

/* 0xC301 corresponds to normal abort, see RR SAP */
SHORT	CAUSE_C301	0xC301

/* used for ss_cause */
/* CAUSE_50 is according to 04.80 / 04.08 the cause value 50 "Requested facility not subscribed" */
BYTE	CAUSE_50	50

/* CAUSE_51 is according to 04.80 / 04.08 the cause value 81 - Invalid TI */
BYTE	CAUSE_51	0x51

/* CAUSE_16, i.e. cause value 16 according to 04.80 / 04.08, "Normal call clearing", is not very meaningful as it
    is used for CC usually but the purpose here is simply to test if causes are passed transparently to higher layers */
BYTE	CAUSE_16	16

/* CAUSE_0632 is used for MNSS_END_IND, case when cause value decimal 50 was received over the air */
SHORT	CAUSE_0632	0x0632

/* CAUSE_0610 is used for MNSS_END_IND, sample for the case when cause dec. 16 was received over the air */
SHORT	CAUSE_0610	0x0610

/* According to cause concept 0x86FF is used for MNSS_END_IND, case when no cause was received over the air */
SHORT	CAUSE_NOT_PRESENT	0x86FF

/* chosen arbitrary value, for the scenario when lower layers indicate an error (MMSS_ERROR_IND) */
/* SS shall transparently pass the cause up to higher layers */
/* note that this value would not occur at the interface in a real target case */
SHORT	SS_ERROR_CAUSE_C397	0xC397

/*
 * Definitions for EM
 */
SHORT	Bitm		0x04

BYTE	EM_ENTITY	0x06

/* facility */
BEGINARRAY_PART (SS_FACILITY_A_CONTENT, 4)
	0xA0, /* These are not valid bytes */
	0x03, 
	0x82, 
	0x56
ENDARRAY
BEGIN_PSTRUCT ("fac_inf", SS_FACILITY_A_INF)
	SET_COMP ("l_fac",	L_FAC_32)
	SET_COMP ("o_fac",	O_FAC_0)
	SET_COMP ("fac",	SS_FACILITY_A_CONTENT)
ENDSTRUCT
BEGIN_MSTRUCT ("ss_facility", SS_FACILITY_A)
	SET_COMP ("fac_info",	SS_FACILITY_A_CONTENT)
ENDSTRUCT

BEGINARRAY_PART (SS_FACILITY_B_CONTENT, 4)
	0xA0, /* These are not valid bytes */
	0x03, 
	0x43, 
	0x78
ENDARRAY
BEGIN_PSTRUCT ("fac_inf", SS_FACILITY_B_INF)
	SET_COMP ("l_fac",	L_FAC_32)
	SET_COMP ("o_fac",	O_FAC_0)
	SET_COMP ("fac",	SS_FACILITY_B_CONTENT)
ENDSTRUCT
BEGIN_MSTRUCT ("ss_facility", SS_FACILITY_B)
	SET_COMP ("fac_info",	SS_FACILITY_B_CONTENT)
ENDSTRUCT

BEGIN_PSTRUCT ("fac_inf", EMPTY_FACILITY)
	SET_COMP ("l_fac",	L_FAC_0)
	SKIP_COMP ("o_fac")
	SKIP_COMP ("fac")
ENDSTRUCT

/* version */
/* version for primitives (MNSS SAP) */
BEGINARRAY (SS_VERSION_INDICATOR, 1)
	0xAA
ENDARRAY
BEGIN_PSTRUCT ("ss_ver", SS_VERSION_PRIM)
	SET_COMP ("len",	1)
	SET_COMP ("ver",	SS_VERSION_INDICATOR)
ENDSTRUCT
BEGIN_MSTRUCT ("ss_version", SS_VERSION_MSG)
	SET_COMP ("ver",	SS_VERSION_INDICATOR)
ENDSTRUCT

/* mstructs */
BEGIN_MSTRUCT ("ss_cause", SS_CAUSE_50)
	SET_COMP ("cs2",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER) 
	SKIP_COMP ("rec")
	SET_COMP ("cs",	CAUSE_50)
	SKIP_COMP ("diag")
ENDSTRUCT
BEGIN_MSTRUCT ("ss_cause", SS_CAUSE_16)
	SET_COMP ("cs2",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER) 
	SKIP_COMP ("rec")
	SET_COMP ("cs",	CAUSE_16)
	SKIP_COMP ("diag")
ENDSTRUCT

BEGIN_MSTRUCT ("ss_cause", SS_CAUSE_51)
	SET_COMP ("cs2",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_PUB_NET_REMOTE_USER) 
	SKIP_COMP ("rec")
	SET_COMP ("cs",	CAUSE_51)
	SKIP_COMP ("diag")
ENDSTRUCT

BEGIN_MSTRUCT ("ss_cause", SS_CAUSE_60)
	SET_COMP ("cs2",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_PUB_NET_REMOTE_USER) 
	SKIP_COMP ("rec")
	SET_COMP ("cs",	CAUSE_60)
	SKIP_COMP ("diag")
ENDSTRUCT

BEGIN_MSTRUCT ("ss_cause", SS_CAUSE_61)
	SET_COMP ("cs2",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_PUB_NET_REMOTE_USER) 
	SKIP_COMP ("rec")
	SET_COMP ("cs",	CAUSE_61)
	SKIP_COMP ("diag")
ENDSTRUCT

#if 0
SET_SDU(REGISTER_CMD_NO_FACILITY, 16, 0)

0x3B,  /* ti+pd */
	0x3B  /* Message Type */

ENDSDU

#endif

BEGINARRAY (REGISTER_CMD_NO_FACILITY, 6)
	0x10, 0x00,	/* length in bits */
	0x00, 0x00,	/* offset in bits */
	0x3B,	/* ti=3, pd=0xb (SS) */
	0x3B	/* Message Type=0x3b (REGISTER) */

ENDARRAY

SET_SDU(REGISTER_CMD_MID_ERROR, 16, 0)

0x0B,  /* ti+pd */
	0x30  /* Message Type */

ENDSDU

SET_SDU(FACILITY_CMD_MAND_ERROR, 16, 0)

0x0B,  /* ti+pd */
	0x3A  /* Message Type */

ENDSDU

SET_SDU(FACILITY_CMD_OK_BAD_TI, 80, 0)

0x2B,  /* ti=2,pd=B */
	0x3A,  /* Message Type */
	0xA2, /* Invoke */
	0x06, /* Length */
	0x02, /* Invoke ID Tag */
	0x01, /* Invoke ID Tag - Length */
	0x02, /* Invoke ID */
	0x02, /* Operation Code Tag */
	0x01, /* Operation Code - Length */
	0x02 /* Operation Code */
ENDSDU

SET_SDU(REGISTER_CMD_OK_BAD_TI, 88, 0)

0xBB,  /* ti=0,pd=B */
	0x3B,  /* Message Type */
	0x1C,  /* FACILITY */
	0xA2, /* Invoke */
	0x06, /* Length */
	0x02, /* Invoke ID Tag */
	0x01, /* Invoke ID Tag - Length */
	0x02, /* Invoke ID */
	0x02, /* Operation Code Tag */
	0x01, /* Operation Code - Length */
	0x02 /* Operation Code */
ENDSDU

SET_SDU(RELEASE_COMP_CMD_OK_BAD_TI, 16, 0)

0x2B,  /* ti=2,pd=B */
	0x2A  /* Message Type */
ENDSDU

# TEST CASES

## Routing (internal)

### SS000: Setup the routing and PCO view for the SS test

Description:	Routings for the SS tests are set. Only those routings are set which are really required.

Preamble:	None

MMI		 SS		MM
	 |		  |		  |
COMMAND (TAP RESET)
COMMAND (MMI RESET)
COMMAND (SS RESET)
COMMAND (MM RESET)
	 |		  |		  |
COMMAND (TAP REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
	 |		  |		  |
COMMAND (SS REDIRECT MMI TAP)
COMMAND (SS REDIRECT MM TAP)
	 |		  |		  |
COMMAND (TAP REDIRECT TAP SS)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

History:	04-Jul-97	PZ	Initial
17-Mar-01	SBK	Removed unnecessary COMMANDs to speed up

## SS establishment

### SS001: IDLE_1- MO establishment with a facility, requesting MM connection

Description:	Higher layers request a SS transaction establishment. The request includes facility data. SS requests the establishment of an MM connection for the SS transaction.

Preamble:	SS000

MMI		 SS		  MM
	 |		  |		  |
(1)	 |	MNSS_BEGIN_REQ	  | 		  |
	 *===================================>*		  |
(2)	 |		  |	MMSS_ESTABLISH_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNSS_BEGIN_REQ	
		ti	TI_3
		fac_inf	SS_FACILITY_A_INF
		ss_ver	SS_VERSION_PRIM

- MMSS_ESTABLISH_REQ
		ti	TI_3

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	18-Oct-00	HM	Revised

### SS002: PENDING_1 - MM connection establishment failure

Description: 	Mobile originated SS transaction establishment is pending. SS receives the indication that MM connection establishment is not possible, e.g. because due to an RR connection establishment failure.

Preamble:	SS001

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  | 	    MMSS_RELEASE_IND	  |
	 |		  *<====================================*
(2)	 |	MNSS_END_IND	  |		  |
	 *<===================================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_RELEASE_IND
		ti	TI_3
		cause	CAUSE_C301

- MNSS_END_IND
		ti	TI_3
		cause	CAUSE_C301
		fac_inf	EMPTY_FACILITY

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS004: IDLE_3 - MT establishment, without facility

Description:	Mobile terminated SS transaction establishment is triggered by the network sending a REGISTER message containing no Facility IE to the MS. The scenario is for the successful case.

Preamble:	SS000

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	   MMSS_ESTABLISH_IND	  |
	 |		  *<====================================*
(2)	 |	MNSS_BEGIN_IND	  |		  |
	 *<===================================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	REGISTER_CMD_NO_FACILITY

- MNSS_BEGIN_IND
		ti	TI_11
		fac_inf	EMPTY_FACILITY

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	13-Mar-03	ZM	tidy up

### SS005: IDLE_4 - MT establishment, with facility

Description:	Mobile terminated SS transaction establishment is triggered by the network sending a REGISTER message containing a Facility IE to the MS. The scenario is for the successful case.

Preamble:	SS000

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	   MMSS_ESTABLISH_IND	  |
	 |		  *<====================================*
(2)	 |	MNSS_BEGIN_IND	  |		  |
	 *<===================================*	 	  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	D_SS_REGISTER
		ti	TI_3
		ss_facility	SS_FACILITY_A
		}

- MNSS_BEGIN_IND
		ti	TI_11
		fac_inf	SS_FACILITY_A_INF

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	18-Oct-00	HM	Revised

### SS011: PENDING_2 - Successful MO establishment, completion

Description:	Normal, successful mobile originated SS transaction establishment is completed: MM confirms the establishment of the MM connection and SS sends a REGISTER message to the network containing the facility data previously received from the higher layer.

Preamble:	SS001

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	 MMSS_ESTABLISH_CNF	  |
	 |		  *<====================================*
(2)	 |		  |	   MMSS_DATA_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_ESTABLISH_CNF
		ti	TI_3

- MMSS_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	UPLINK
		pd	U_SS_REGISTER
		ti	TI_3
		ss_facility	SS_FACILITY_A
		ss_version	SS_VERSION_MSG
		}

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS050: SS002 with Engineering Mode interaction

Description: 	Mobile originated SS transaction establishment is pending. SS receives the indication that MM connection establishment is not possible, e.g. because due to an RR connection establishment failure.

Preamble:	SS001

MMI		 SS		  MM
	 |		  |		  |
(1)	 |	EM_SS_EVENT_REQ	  | 		  |
	 *===================================>*		  |
(2)	 |		  | 	    MMSS_RELEASE_IND	  |
	 |		  *<====================================*
(3)	 |	EM_DATA_IND	  |		  |
	 *<===================================*		  |
(4)	 |	MNSS_END_IND	  |		  |
	 *<===================================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- EM_SS_EVENT_REQ
		bitmask_ss	Bitm

- MMSS_RELEASE_IND
		ti	TI_3
		cause	CAUSE_C301

- EM_DATA_IND
		entity	EM_ENTITY

- MNSS_END_IND
		ti	TI_3
		cause	CAUSE_C301
		fac_inf	EMPTY_FACILITY

History:	23-Oct-01	OT	Initial
	04-Jan-02	HM	Changed order of primitives

## Error handling

### SS003: IDLE_2 - Primitives to ignore in idle state

Description:	SS receives primitives to be ignored while in idle state.

Preamble: 	SS000

MMI		 SS		  MM
	 |		  |		  |
(1)	 |	MNSS_END_REQ	  | 		  |
	 *===================================>*		  |
MUTE (10000)		  |		  |
(2)	 |		  |	   MMSS_ERROR_IND	  |
	 |		  *<====================================*
MUTE (10000)		  |		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNSS_END_REQ
		ti	TI_3
		fac_inf	EMPTY_FACILITY

- MMSS_ERROR_IND
		ti	TI_3
		cause	SS_ERROR_CAUSE_C397

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS006: IDLE_5 – MT establishment attempted by false message, FACILITY

Description:	The network attempts establishment of an SS transaction by sending a message inconsistent with the protocol state, i.e. no REGISTER message. Here the example of the FACILITY message is taken.
Unrecognised TI : "If a FACILITY message is received specifying a transaction identifier that is not recognised as relating to a call independent SS transaction that is in progress then a RELEASE COMPLETE message shall be sent with cause value #81 "invalid call reference value".
[TS 24.010 Section 3.7.3 b)]

Preamble:	SS000

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_ESTABLISH_IND	  |
	 |		  *<====================================*
(2)	 |		  |	MMSS_DATA_REQ	  |
	 |		  *====================================>*
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	D_SS_FACILITY
		ti	TI_3
		ss_facility	SS_FACILITY_A
		}

- MMSS_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	UPLINK
		pd	B_SS_REL_COMP
		ti	TI_11
		ss_cause	SS_CAUSE_51
		}

- MMSS_RELEASE_REQ
		ti	TI_11

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	19-May-03	HM	corrected test case expectations

### SS030: IDLE_10 – MT message, RELEASE COMPLETE on unused transaction

Description:	The network sends a RELEASE COMPLETE on an transition (identifier) not used. SS shall ignore the message. The corresponding MM connection is released.
GSM 04.10 clause 3.7.3:
”If a RELEASE COMPLETE message is received specifying a transaction identifier that is not recognized as relating to a call independent SS transaction that is in progress then the message shall be ignored.”

Preamble:	SS000

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_ESTABLISH_IND	  |
	 |		  *<====================================*
(2)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	B_SS_REL_COMP
		ti	TI_3
		ss_cause	NOT_USED
		ss_facility	SS_FACILITY_B
		}

- MMSS_RELEASE_REQ
		ti	TI_11

History:	31-Jan-02	SBK	Initial

### SS007: IDLE_6 - Establishment with a reserved TI value

Description:	A transaction with a reserved TI value is requested by the higher layer (TI =7). This is assessed as wrong higher layer behaviour.

Preamble:	SS000

MMI		 SS		  MM
	 |		  |		  |
(1)	 |	MNSS_BEGIN_REQ	  | 		  |
	 *===================================>*		  |
MUTE (10000)		  |		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNSS_BEGIN_REQ
		ti	TI_7
		fac_inf	SS_FACILITY_A_INF
		ss_ver	SS_VERSION_PRIM

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	18-Oct-00	HM	Revised

### SS008: IDLE_7 - Establishment with set TI flag

Description:	A transaction with a TI flag which is set is requested by the higher layer (TI > 7). This is assessed as wrong higher layer behaviour.

Preamble:	SS000

MMI		 SS		  MM
	 |		  |		  |
(1)	 |	MNSS_BEGIN_REQ	  | 		  |
	 *===================================>*		  |
MUTE (10000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNSS_BEGIN_REQ
		ti	TI_11
		fac_inf	EMPTY_FACILITY
		ss_ver	SS_VERSION_PRIM

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS009: IDLE_8 - Network establishment with a reserved TI

Description:	A transaction is attempted to be established by the network with a REGISTER message with TI == 7.This is interpreted as wrong network behaviour and the message is ignored. The corresponding MM connection is released.
GSM 04.10 clause 3.7.3:
”The MS shall ignore messages with the transaction identifier value set to "111".

Preamble:	SS000

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_ESTABLISH_IND	  |
	 |		  *<====================================*
(2)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	D_SS_REGISTER
		ti	TI_7
		ss_facility	NOT_USED
		}

- MMSS_RELEASE_REQ
		ti	TI_15

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS010: IDLE_9 - Network establishment with set TI flag

Description:	A transaction is attempted to be established by the network with a REGISTER message with set TI Flag (T>7).This is interpreted as wrong network behaviour. . The corresponding MM connection is released.
GSM 04.10 clause 3.7.3:
”If a REGISTER message is received specifying a transaction identifier that is not recognized as relating to a call independent SS transaction that is in progress and with a transaction identifier flag incorrectly set to "1", this message shall be ignored.”

Preamble:	SS000

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_ESTABLISH_IND	  |
	 |		  *<====================================*
(2)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	D_SS_REGISTER
		ti	TI_11
		ss_facility	NOT_USED
		}

- MMSS_RELEASE_REQ
		ti	TI_3

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS040: IDLE_3 - MT establishment, with erroneous message type

Description:	Mobile terminated SS transaction establishment is triggered by the network sending a REGISTER message containing an incorrect message ID. (24.010, section 3.7.4)

Preamble:	SS000

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	   MMSS_ESTABLISH_IND	  |
	 |		  *<====================================*
(2)	 |		  |	MMSS_DATA_REQ	  |
	 |		  *====================================>*
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	REGISTER_CMD_MID_ERROR

- MMSS_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	UPLINK
		pd	B_SS_REL_COMP
		ti	TI_8
		ss_cause	SS_CAUSE_61
		}

- MMSS_RELEASE_REQ
		ti	TI_8

History:	13-Mar-03	ZM	Initial

### SS041: IDLE_3 - MT establishment, with missing mandatory IEI

Description:	Mobile terminated SS transaction establishment is triggered by the network sending a FACILITY message containing a missing mandatory IEI. (24.010, section 3.7.5)

Preamble:	SS011

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_DATA_IND	  |
	 |		  *<====================================*
(2)	 |		  |	MMSS_DATA_REQ	  |
	 |		  *====================================>*
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	FACILITY_CMD_MAND_ERROR

- MMSS_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	UPLINK
		pd	B_SS_REL_COMP
		ti	TI_8
		ss_cause	SS_CAUSE_60
		}

- MMSS_RELEASE_REQ
		ti	TI_8

History:	13-Mar-03	ZM	Initial

### SS042: Error Handling - MT establishment, with FACILITY message containing a incorrect TI value

Description:	A FACILITY message is sent with an invalid TI (24.010, section 3.7.3). A REL COMP is returned

Preamble:	SS011

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_DATA_IND	  |
	 |		  *<====================================*
(2)	 |		  |	MMSS_DATA_REQ	  |
	 |		  *====================================>*
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	FACILITY_CMD_OK_BAD_TI

- MMSS_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	UPLINK
		pd	B_SS_REL_COMP
		ti	TI_10
		ss_cause	SS_CAUSE_51
		}

- MMSS_RELEASE_REQ
		ti	TI_10

History:	13-Mar-03	ZM	Initial

### SS043: Error Handling - MT establishment, with message containing a incorrect TI value

Description:	A REGISTER/REL COMP message is sent with an invalid TI (24.010, section 3.7.3). There is no response.

Preamble:	SS011

Variants:	<A> .. <B>

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_DATA_IND	  |
	 |		  *<====================================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
<A>	sdu	RELEASE_COMP_CMD_OK_BAD_TI
<B>	sdu	REGISTER_CMD_OK_BAD_TI

History:	13-Mar-03	ZM	Initial

### SS012: PENDING_3 - Error reports during establishment

Description:	While waiting for an MM connection confirmation, an error is reported. SS releases the transaction, indicates to higher layers that the transaction is cleared and releases the MM connection.

Preamble:	SS001

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	  MMSS_ERROR_IND	  |
	 |		  *<====================================*
(2)	 |	MNSS_END_IND	  | 		  |
	 *<===================================*		  |
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_ERROR_IND
		ti	TI_3
		cause	SS_ERROR_CAUSE_C397

- MNSS_END_IND
		ti	TI_3
		cause	SS_ERROR_CAUSE_C397
		fac_inf	EMPTY_FACILITY

- MMSS_RELEASE_REQ
		ti	TI_3

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	18-Oct-00	HM	Under revision
	15-Feb-01	LE	Test case modified

### SS025: CONNECTED_13 – Call re-establishment during the transaction

Description:	The SS connection is in CONNECTED state after the mobile originated transaction establishment. MM reports an error in the RR connection with the primitive MMSS_ERR_IND. Call Reestablishment is not applicable to SS transactions. SS informs MM about the end of connection with an MNSS_END_IND and requests release of the MM connection with an MMSS_REL_REQ.

Preamble:	SS011

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_ERROR_IND	  |
	 |		  *<====================================*
(2)	 |	MNSS_END_IND	  | 		  |
	 *<===================================*		  |
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_ERROR_IND
		ti	TI_3
		cause	SS_ERROR_CAUSE_C397

- MNSS_END_IND
		ti	TI_3
		cause	SS_ERROR_CAUSE_C397
		fac_inf	EMPTY_FACILITY

- MMSS_RELEASE_REQ
		ti	TI_3

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS026: CONNECTED_14 - Network sends a false message

Description:	The SS connection is in CONNECTED state after the mobile originated transaction establishment. MM passes a message on to SS that is neither a FACILITY nor a RELEASE COMPLETE message. SS ignores this message.

Preamble:	SS011

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_DATA_IND	  |
	 |		  *<====================================*
MUTE (5000)	 	  |		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	D_SS_REGISTER
		ti	TI_11
		ss_facility	SS_FACILITY_A
		}

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS031: CONNECTED_18 – MT message, RELEASE COMPLETE on unused transaction

Description:	The network sends a RELEASE COMPLETE on an transition (identifier) not used while SS is in the connected state (on another transaction). SS shall ignore the transaction. The corresponding MM connection is released.
GSM 04.10 clause 3.7.3:
”If a RELEASE COMPLETE message is received specifying a transaction identifier that is not recognized as relating to a call independent SS transaction that is in progress then the message shall be ignored.”

Preamble:	SS011

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_ESTABLISH_IND	  |
	 |		  *<====================================*
(2)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	B_SS_REL_COMP
		ti	TI_3
		ss_cause	NOT_USED
		ss_facility	SS_FACILITY_B
		}

- MMSS_RELEASE_REQ
		ti	TI_11

History:	31-Jan-02	SBK	Initial

## Data transfer

### SS013: CONNECTED_1 – MO transaction, higher layer sends a facility IE

Description:	After the mobile originated transaction establishment, a facility information element is passed from the higher layer on to SS. SS has to convert this into a FACILITY message.

Preamble:	SS011

MMI		 SS		  MM
	 |		  |		  |
(1)	 |	MNSS_FACILITY_REQ	  | 		  |
	 *====================================>*		  |
(2)	 |		  |	MMSS_DATA_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNSS_FACILITY_REQ
		ti	TI_3
		fac_inf	SS_FACILITY_B_INF
		ss_ver	NOT_USED

- MMSS_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	UPLINK
		pd	U_SS_FACILITY
		ti	TI_3
		ss_facility	SS_FACILITY_B
		}

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	18-Oct-00	HM	Revised

### SS014: CONNECTED_2 – MT transaction, higher layer sends a facility IE

Description:	After the network initiated transaction establishment, a facility information element is passed from the higher layer on to SS. SS has to convert this into a FACILITY message.

Preamble:	SS005

MMI		 SS		  MM
	 |		  |		  |
(1)	 |	MNSS_FACILITY_REQ	  | 		  |
	 *====================================>*		  |
(2)	 |		  |	MMSS_DATA_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNSS_FACILITY_REQ
		ti	TI_11
		fac_inf	SS_FACILITY_B_INF
		ss_ver	NOT_USED

- MMSS_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	UPLINK
		pd	U_SS_FACILITY
		ti	TI_11
		ss_facility	SS_FACILITY_B
		}

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	18-Oct-00	HM	Revised

### SS019: CONNECTED_7 – MO transaction, network sends a FACILITY message

Description:	After the mobile originated transaction establishment, a FACILITY is passed from MM on to SS. SS has to convert this into facility data of the MNSS_FACILITY_IND.

Preamble:	SS011

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_DATA_IND	  |
	 |		  *<====================================*
(2)	 |	MNSS_FACILITY_IND	  | 		  |
	 *<====================================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	D_SS_FACILITY
		ti	TI_11
		ss_facility	SS_FACILITY_A
		}

- MNSS_FACILITY_IND
		ti	TI_3
		fac_inf	SS_FACILITY_A_INF

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	18-Oct-00	HM	Revised

### SS020: CONNECTED_8 – MT transaction, network sends a FACILITY message

Description:	After the network initiated transaction establishment, a FACILITY message is passed from MM on to SS. SS has to convert this into facility data of the MNSS_FACILITY_IND.

Preamble:	SS005

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_DATA_IND	  |
	 |		  *<====================================*
(2)	 |	MNSS_FACILITY_IND	  | 		  |
	 *<====================================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	D_SS_FACILITY
		ti	TI_3
		ss_facility	SS_FACILITY_A
		}

- MNSS_FACILITY_IND
		ti	TI_11
		fac_inf	SS_FACILITY_A_INF

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	18-Oct-00	HM	Revised

## Connection release

### SS015: CONNECTED_3 – MO transaction, higher layer releases without facility IE

Description:	After the mobile originated transaction establishment, the connection release is initiated by higher layer with the primitive MNSS_END_REQ. The primitive contains no facility information element. SS must inform the network with the RELEASE COMPLETE message and terminate the MM connection locally with MMSS_REL_REQ.

Preamble:	SS011

MMI		 SS		  MM
	 |		  |		  |
(1)	 |	MNSS_END_REQ	  | 		  |
	 *====================================>*		  |
(2)	 |		  |	MMSS_DATA_REQ	  |
	 |		  *====================================>*
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNSS_END_REQ
		ti	TI_3
		fac_inf	EMPTY_FACILITY

- MMSS_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	UPLINK
		pd	B_SS_REL_COMP
		ti	TI_3
		ss_cause	NOT_USED
		ss_facility	NOT_USED
		}

- MMSS_RELEASE_REQ
		ti	TI_3

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS016: CONNECTED_4 – MO transaction, higher layer releases with facility IE

Description:	After the mobile originated connection establishment, the connection release is initiated through the higher layer with the primitive MNSS_END_REQ. The primitive contains a facility information element. SS must inform the network with RELEASE COMPLETE and terminate the connection locally with MMSS_REL_REQ.

Preamble:	SS011

MMI		 SS		  MM
	 |		  |		  |
(1)	 |	MNSS_END_REQ	  | 		  |
	 *====================================>*		  |
(2)	 |		  |	MMSS_DATA_REQ	  |
	 |		  *====================================>*
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNSS_END_REQ
		ti	TI_3
		fac_inf	SS_FACILITY_A_INF

- MMSS_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	UPLINK
		pd	B_SS_REL_COMP
		ti	TI_3
		ss_cause	NOT_USED
		ss_facility	SS_FACILITY_A
		}

- MMSS_RELEASE_REQ
		ti	TI_3

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	18-Oct-00	HM	Revised

### SS021: CONNECTED_9 - MO transaction, network releases without facility IE

Description:	After the mobile originated transaction establishment, the SS transaction release is initiated with the RELEASE COMPLETE message by the network. The RELEASE COMPLETE message contains no facility information element. SS informs the higher layer about the release with MNSS_END_IND. The primitive contains no facility information element. SS requests MM connection release with MMSS_REL_REQ.

Preamble:	SS011

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_DATA_IND	  |
	 |		  *<====================================*
(2)	 |	MNSS_END_IND	  | 		  |
	 *<====================================*		  |
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	B_SS_REL_COMP
		ti	TI_11
		ss_cause	SS_CAUSE_50
		}

- MNSS_END_IND
		ti	TI_3
		cause	CAUSE_0632
		fac_inf	EMPTY_FACILITY

- MMSS_RELEASE_REQ
		ti	TI_3

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS022: CONNECTED_10 – MT transaction, network releases without facility IE

Description:	After the network initiated transaction establishment, the SS connection release is initiated with the RELEASE COMPLETE message. The RELEASE COMPLETE message contains no facility information element. SS informs the higher layer about the release with MNSS_END_IND. The primitive contains no facility information element. SS requests MM connection release with MMSS_REL_REQ.

Preamble:	SS005

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_DATA_IND	  |
	 |		  *<====================================*
(2)	 |	MNSS_END_IND	  | 		  |
	 *<====================================*		  |
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	B_SS_REL_COMP
		ti	TI_3
		ss_cause	SS_CAUSE_50
		}

- MNSS_END_IND
		ti	TI_11
		cause	CAUSE_0632
		fac_inf	EMPTY_FACILITY

- MMSS_RELEASE_REQ
		ti	TI_11

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS023: CONNECTED_11 – MO transaction, network releases with facility IE

Description:	After the mobile originated transaction establishment, the SS transaction release is initiated with the RELEASE COMPLETE message by the network. The RELEASE COMPLETE message contains a facility information element. SS informs the higher layer about the release with MNSS_END_IND. The primitive contains no facility information element. SS requests MM connection release with MMSS_REL_REQ.

Preamble:	SS011

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_DATA_IND	  |
	 |		  *<====================================*
(2)	 |	MNSS_END_IND	  | 		  |
	 *<====================================*		  |
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	B_SS_REL_COMP
		ti	TI_11
		ss_cause	SS_CAUSE_50
		ss_facility	SS_FACILITY_B
		}

- MNSS_END_IND
		ti	TI_3
		cause	CAUSE_0632
		fac_inf	SS_FACILITY_B_INF

- MMSS_RELEASE_REQ
		ti	TI_3

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	18-Oct-00	HM	Revised

### SS029: CONNECTED_17 – MO transaction, net releases with facility IE, no cause

Description:	After the mobile originated transaction establishment, the SS transaction release is initiated with the RELEASE COMPLETE message by the network, carrying no cause IE. The RELEASE COMPLETE message contains a facility information element. SS informs the higher layer about the release with MNSS_END_IND. The primitive contains no facility information element. SS requests MM connection release with MMSS_REL_REQ.

Preamble:	SS011

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_DATA_IND	  |
	 |		  *<====================================*
(2)	 |	MNSS_END_IND	  | 		  |
	 *<====================================*		  |
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	B_SS_REL_COMP
		ti	TI_11
		ss_cause	NOT_USED
		ss_facility	SS_FACILITY_B
		}

- MNSS_END_IND
		ti	TI_3
		cause	CAUSE_NOT_PRESENT
		fac_inf	SS_FACILITY_B_INF

- MMSS_RELEASE_REQ
		ti	TI_3

History:	27-Jul-01	SBK	Initial

### SS024: CONNECTED_12 – MT transaction, network releases with facility IE

Description:	After the network initiated transaction establishment, the SS connection release is initiated with the RELEASE COMPLETE message. The RELEASE COMPLETE message contains a facility information element. SS informs the higher layer about the release with MNSS_END_IND. The primitive contains no facility information element. SS requests MM connection release with MMSS_REL_REQ.

Preamble:	SS005

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_DATA_IND	  |
	 |		  *<===================================*
(2)	 |	MNSS_END_IND	  | 		  |
	 *<====================================*		  |
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	DOWNLINK
		pd	B_SS_REL_COMP
		ti	TI_3
		ss_cause	SS_CAUSE_16
		ss_facility	SS_FACILITY_A
		}

- MNSS_END_IND
		ti	TI_11
		cause	CAUSE_0610
		fac_inf	SS_FACILITY_a_INF

- MMSS_RELEASE_REQ
		ti	TI_11

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	18-Oct-00	HM	Revised

### SS027: CONNECTED_15 – MO transaction, MM releases the connection

Description:	After mobile originated transaction establishment, SS is informed about the MM connection release by MM. SS has to indicate that therefore the SS transaction is terminated to the higher layer.

Preamble:	SS011

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_RELEASE_IND	  |
	 |		  *<====================================*
(2)	 |	MNSS_END_IND	  | 		  |
	 *<====================================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_RELEASE_IND
		ti	TI_3
		cause	CAUSE_C301

- MNSS_END_IND
		ti	TI_3
		cause	CAUSE_C301
		fac_inf	EMPTY_FACILITY

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS028: CONNECTED_16 – MT transaction, MM releases the connection

Description:	After mobile terminated transaction establishment, SS is informed about the MM connection release by MM. SS has to indicate that therefore the SS transaction is terminated to the higher layer.

Preamble:	SS005

MMI		 SS		  MM
	 |		  |		  |
(1)	 |		  |	MMSS_RELEASE_IND	  |
	 |		  *<====================================*
(2)	 |	MNSS_END_IND	  | 		  |
	 *<====================================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMSS_RELEASE_IND
		ti	TI_11
		cause	CAUSE_C301

- MNSS_END_IND
		ti	TI_11
		cause	CAUSE_C301
		fac_inf	EMPTY_FACILITY

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS017: CONNECTED_5 – MT transaction, higher layer releases without facility

Description:	After the network initiated transaction establishment, the transaction release is initiated by the higher layer with the primitive MNSS_END_REQ. The primitive contains no facility information element. SS must inform the network with RELEASE COMPLETE and terminate the MM connection locally with MMSS_REL_REQ.

Preamble:	SS005

MMI		 SS		  MM
	 |		  |		  |
(1)	 |	MNSS_END_REQ	  | 		  |
	 *====================================>*		  |
(2)	 |		  |	MMSS_DATA_REQ	  |
	 |		  *====================================>*
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNSS_END_REQ
		ti	TI_11
		fac_inf	EMPTY_FACILITY

- MMSS_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	UPLINK
		pd	B_SS_REL_COMP
		ti	TI_11
		ss_cause	NOT_USED
		ss_facility	NOT_USED
		}

- MMSS_RELEASE_REQ
		ti	TI_11

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction

### SS018: CONNECTED_6 - MT transaction, higher layer releases with facility

Description:	After the network initiated transaction establishment, the transaction release is initiated by the higher layer with the primitive MNSS_END_REQ. The primitive contains a facility information element. SS must inform the network with RELEASE COMPLETE and terminate the MM connection locally with MMSS_REL_REQ.

Preamble:	SS005

MMI		 SS		  MM
	 |		  |		  |
(1)	 |	MNSS_END_REQ	  | 		  |
	 *====================================>*		  |
(2)	 |		  |	MMSS_DATA_REQ	  |
	 |		  *====================================>*
(3)	 |		  |	MMSS_RELEASE_REQ	  |
	 |		  *====================================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNSS_END_REQ
		ti	TI_11
		fac_inf	SS_FACILITY_B_INF

- MMSS_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	SS
		direction	UPLINK
		pd	B_SS_REL_COMP
		ti	TI_11
		ss_cause	NOT_USED
		ss_facility	SS_FACILITY_B
		}

- MMSS_RELEASE_REQ
		ti	TI_11

History:	21-July-97	MS	Initial
	20-Aug-97	SZ	error correction
	18-Oct-00	HM	Revised