Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document

LLD Specification. Notification of CCM for Individual Calls %CCCM.(Edit via File/Properties/Summary!)

| Document Number: | 20_04_04_02520 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.1 (Edit via File/Properties/Summary!) |
| Status: | Approved (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 06 June 2005 |
| Last changed: | 2005-Sep-13 by x0001198 |
| File Name: | input.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2005-06-06 | x0001198 |  | 0.1 | Approved | 1 |

Notes:

- Initial version

Table of Contents

1	Introduction	4

2	Interface changes	5

3	Proposed Low Level Design.	5

3.1	Interface Changes	6

3.2	ATI Modifications.	6

3.2.1	New Functions:	6

3.2.2	Affected Global variables:	6

3.2.3	Description of the changes:	7

3.3	CMH Modications	8

3.3.1	New Callback:	8

3.4	BAT	10

3.4.1	BAT Testing	10

4	Simulation Tests	12

5	List of References	15

# Introduction

The New proprietary AT%CCCM command and unsolicited result code relate to Advice of Charge supplementary service that enables subscriber to get information about the cost of individual calls.

# Interface changes

%CCCM parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| %CCCM=[<n>] | OK |
| %CCCM? | %CCCM: <n> OK |
| %CCCM=? | %CCCM: (list of supported <n>s) OK |

Description

This refers to Advice of Charge supplementary service (3GPP TS 22.024 [26] and 3GPP TS 22.086 [27]) that enables subscriber to get information about the cost of individual calls. With <n>=1, the execute command enables an unsolicited event reporting the CCM information. The unsolicited result code %CCCM: < LineID>,<ccm> is sent when the CCM value for the call identified by <LineID> changes. Deactivation of the unsolicited event reporting is made with the same command with <n>=0.

Defined values

<n> (parameter sets/shows the result code presentation status in the MT/TA):

0   disable

1   enable

<LineID> identifies ALS Line ID

1               Speech Mode
2               Auxiliary Speech Mode

<ccm> string type; three bytes of the current call meter value in hexadecimal format (e.g. "00001E" indicates decimal value 30); value is in home units and bytes are similarly coded as ACMmax value in the SIM card or in the active application in the UICC (GSM or USIM)

# Proposed Low Level Design.

## Interface Changes

The %CCCM command will be defined as described above.

## ATI Modifications.

### New Functions:

-setatPercentCCCM() will handle the %CCCM set command.

-queatPlusCCCM () will handle the %CCCM query command

### Affected Global variables:

ati_cmd.c:

The ATI array "cmds" will be updated with the new functions as follow:

setatPercentCCCM()

queatPlusCCCM ()

For the test command echo the following string will be added to the ATI array “cmds” to be handled via test_gen.

"%s: (0, 1)"

aci_cmh.h:

Addition of the ID AT_CMD_P_CCCM will be necessary to type T_ACI_AT_CMD for call to the functions above.

<n> Sets/shows the result code presentation status in the MT/TA.

Percent_CCCM_stat will be added to type T_ATI_USER_OUTPUT_CFG for the global variable ati_user_output_cfg[CMD_SRC_MAX] for set/query functionality of the %CCCM command.

### Description of the changes:

Ati_cc.c:

%CCCM Set and Query functions will do the following:

setatPercentCCCM() will call GenAtCR_C( cl, srcId, AT_CMD_P_CCCM ).

queatPercentCCCM() will display the current settings for the parameter <n>.

GenAtCR_C () will be modified to include case for AT_CMD_P_CCCM where ati_user_output_cfg[] will be updated as shown below for the AT command parameter <n>.

case( AT_CMD_P_CCCM ):

ati_user_output_cfg[srcId].Percent_CCCM_stat = x;

## CMH Modications

### New Callback:

A Callback is required for when the presentation of the Advice of Charge supplementary service at the TE is enabled, result code will be as below:

%CCCM: <cid>,<pcccm>.

Here Advice of charge information is given for a particular call identified by <cid>. The results code is sent when a change is identified in the Advice of charge for a call.

As with all other unsolicited messaged we make use of the RAT_ macro to display the result code.

aci_cmh.h

RAT_P_CCCM will be added to type RAT_ID.

Additionally the prototype will be added as below:

EXTERN void rAT_PercentCCCM ( SHORT cid,

ULONG *pcccm );

The rAT_PercentCCCM function definition will be implemented either in mfw_cm.c (like rAT_PlusCCCM) or in a dummy functions module. All it needs to do is return.

Ati_ret.c

Here a new ATI function rCI_PercentCCCM as below will be implemented which corresponds to the rAT_PercentCCCM function. The setting of ati_user_output_cfg[srcId].CCCM_stat will be used to generate the +CDIP: result code (comparable to rCI_PlusCLIP).

GLOBAL void rAT_PercentCCCM ( SHORT cid,

ULONG *pcccm );

Cmh_f.c:

New element for %CCCM will be added to the array RATJmpTbl

CB_VC( PercentCCCM ),      /* RAT_P_CCCM */

Aoc.c:

The existing function aoc_send_ccm() already searches for connected calls via function qAT_CallActive.

The function qAT_CallActive( &cId ) function will be modified to return the call ID <cId> of the active call. The <cId> is necessary to find the srcId for the originating caller.

The %ALS setting is then queried with the following function:

cmhCC_get_active_als_mode( srcId, &mode );

Returned value <mode> has the following structure and will be used for <LineID> when sending the unsolicited results code:

typedef enum               /* %ALS parameter <mod> bit_field*/

{

ALS_MOD_NOTPRESENT = 0,

ALS_MOD_SPEECH     = 1,

ALS_MOD_AUX_SPEECH = 2

} T_ACI_ALS_MOD;

Sending Unsolicited Results Code:

R_AT( RAT_P_CCCM, idx )(< LineID>,<ccm>);

The function aoc_send_ccm() is already being called from various locations to yield initial AOC info and AOC incremental updates for connected call(s).

## BAT

For BAT the equivalent is:

BAT_RES_UNS_PERCENT_CCCM T_BAT_res_uns_percent_cccm;

typedef struct
{
  U16                       cid;                      /* Call identification number     */
  U32                       pcccm;                  /* Current Call Meter for call with <cid> */
} T_BAT_res_uns_percent_cccm;

There will be no application control over the generation of this unsolicited result code (i.e. no need to enable and disable with a BAT_CMD_SET_PERCENT_CCCM equivalent to AT%CCCM=[<n>] as it is always enabled with BAT).

If there is more than one connected call then more BAT results code instances T_BAT_res_uns_percent_cccm will be sent to match the number of connected calls.

### BAT Testing

The BAT_RES_UNS_PERCENT_CCCM results code will occur whenever there is a change in the Advice of Charge for a call.

## Individual Call Timer Notification - %CCCM

USER

Power on SIM

CC

AOC

aoc_info (0, AOC_START_AOC)

Initialise SIM for AOC Charge

Make New Call or Accept Call (cId)

psa_mncc_setup_ind

aoc_info (cId, AOC_START_TIME)

%CCCM Uns. Results Code for connected calls

psa_mncc_setup_cnf

aoc_info (cId, AOC_CALL_CONNECTED)

End  Call (cId)

Initialise pcccm[cId] call timer

aoc_info (cId, AOC_STOP_TIME)

psa_mncc_sync_ind

aoc_info (cId, AOC_SUSPEND_AOC)

%CCCM Uns. Results Code for call (Final value)

AOC Timeout Occurs

%CCCM Uns. Results Code for the call (In not already sent)

USER

CC

AOC

Abbreviations:

AOC       Advance of Charge

CC          Call Control

cId          Call Id

# Simulation Tests

The following simulation test cases will be added:

-Testcase 1 - %CCCM Test Command

%CCCM=?

%CCCM: (0, 1)

-Testcase 2 - %CCCM Disable, Enable and Query Command

%CCCM=0                                 /* This will disable AOC unsolicited results code for this command */

OK,+CME ERROR

%CCCM?                                  /* Query when Disabled */

%CCCM: 0

%CCCM=1                                 /* This will Enable AOC unsolicited results code for this command */

OK,+CME ERROR

%CCCM?                                  /* Query when Enabled */

%CCCM: 1

Call Session …

No %CCCM Unsolicited results code is received

-Testcase 3 - Disable %CCCM, +CAOC Enabled (1 Connected Call)

%CCCM=0                            /* This will Disable %CCCM unsolicited results code */

OK,+CME ERROR

Call Session … giving rise to change in Advice of Charge (1 Connected Call).

Time Delay in call …20 seconds

No %CCCM Unsolicited results code is received

AT+CAOC=0                       /* Query CCM value */

+CAOC: 000014                   /* CCM value for all calls */

-Testcase 4 - Enable %CCCM,  Query CCM value (1 Connected Call)

%CCCM=1                            /* This will Enable %CCCM unsolicited results code */

OK,+CME ERROR

Setup Call Session … giving rise to change in Advice of Charge (1 Connected Call).

Time Delay in call …25s

%CCCM: 0, 00000A                        /* Format is %CCCM: <Call Id1>, <PCCCM>   for the Call */

5s Delay….

%CCCM: 0, 00000C

5s Delay….

%CCCM: 0, 00000E

More delays…

%CCCM: 0, 000014

Terminate Call...

AT+CAOC=0                       /* Query CCM value */

+CAOC: 000014                   /* CCM value for  call */

-Testcase 5 - Enable %CCCM,  Query CCM value (2 Connected Calls)

%CCCM=1                            /* This will enable %CCCM unsolicited results code */

OK,+CME ERROR

Set up the 1st Call Session … giving rise to change in Advice of Charge.

Time Delay in call …25 Seconds

%CCCM: 0, 00000A                        /* Format is %CCCM: <Call Id1>, <PCCCM>   for the Call */

5s Delay….

%CCCM: 0, 00000C

5s Delay….

%CCCM: 0, 00000E

Set up the 2nd Call Session … giving rise to change in Advice of Charge. (2 Connected Calls)

More delays…

%CCCM: 0, 000012                                   /* For the 1st Call */

%CCCM: 1, 000002                                   /* For the 2nd Call */

10s Delay

%CCCM: 0, 000016

%CCCM: 1, 000006

Terminate All Calls...

AT+CAOC=0                       /* Query CCM value */

+CAOC: 00001E                /* CCM value for  call, which is the total of individual call timers above */

/* N.B. This total CCM value 0x1E above is 2 units more than the total of the %CCCM value  for the 2 calls. This discrepancy is due to delays caused when setting up calls during which the %CCCM timers are not incremented. */

Similar functionality will be tested in the target.

# List of References

This is just an example!:

| [1] | 3GPP TS 27.007 v3.13.0 |
| --- | --- |
| [2] | 3GPP TS 22.024 |
| [3] | 3GPP TS 22.086 |