/*---"ERROR"— */

GSM Fax & Data Services

Test Specification

ACIPHB

Author:	Condat DV-Beratung
	Organisation Software AG
	Alt-Moabit 91d
	D-10559 Berlin
	Germany

Date:	
Document No.:	
File:	input.doc

# Table of Contents

0	Table of Contents	2

1	Document Control	2

1.1	References	3

1.2	Abbreviations	3

1.3	Terms	4

2	Overview	4

2.1	RA - Rate Adaptation	4

2.2	RLP - Radio Link Protocol	4

2.3	L2R - Layer 2 Relay Functionality	4

2.4	FAD 03.45 - Fax Adaptation Protocol	4

2.5	T.30 - Fax Protocol Entity	4

2.6	ACI - AT Command Interpreter	4

2.7	USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	4

3	Parameters	5

4	TEST CASES	7

4.1	Initialisation (ACIPHB001 – ACIPHB10)	7

4.1.1	ACIPHB001: Initialisierung	7

4.1.2	ACIPHB002: initialize phonebook	7

4.1.3	ACIPHB003: use verbose <err> values	7

4.2	Select phonebook memory storrage "+CPBS" (ACIPHB011 – ACIPHB20)	7

4.2.1	ACIPHB011: getting list of supported memory storages	7

4.2.2	ACIPHB012: setting PHB memory storage	7

4.2.3	ACIPHB013: reading memory storage info	7

4.2.4	ACIPHB014: performe some illegal commands	7

4.3	read phonebook entries  "+CPBR" (ACIPHB021 – ACIPHB30)	7

4.3.1	ACIPHB021: getting supported location ranges of phb storage (ADN_Alpha_length=20)	7

4.3.2	ACIPHB022: getting supported location ranges of phb storage (ADN_Data_length=24)	9

4.3.3	ACIPHB023: getting supported location ranges of phb storage (ADN_Alpha_length=40)	10

4.3.4	ACIPHB024: getting supported location ranges of phb storage (ADN_Alpha_length=80)	12

4.3.5	ACIPHB025: reading ad entries (ADN_Alpha_length = 20)	12

4.3.6	ACIPHB026: reading ad entries (ADN_Alpha_length > 20)	14

4.3.7	ACIPHB027: performe some illegal commands	14

4.4	find phonebook entries  "+CPBF" (ACIPHB031 – ACIPHB40)	16

4.4.1	ACIPHB031: getting list of supported modes	16

4.4.2	ACIPHB032: searching for ADN entities starting with "H"	16

4.4.3	ACIPHB033: searching for several ADN entities	18

4.4.4	ACIPHB034: performe some "illegal" commands (query-command, searchings without hints)	18

4.5	write phonebook entries  "+CPBW" (ACIPHB041 – ACIPHB50)	18

4.5.1	ACIPHB041: getting supported location ranges of phb storage	18

4.5.2	ACIPHB042: write and overwrite of entries	18

4.6	test ADN  (ACIPHB045 – ACIPHB47)	18

4.6.1	ACIPHB045: power on, on SIM card could be max phonebook entries 255	18

4.6.2	ACIPHB046: ADN  phonebook entry call via atD	18

4.7	test BDN  "+" (ACIPHB048 – ACIPHB50)	18

4.7.1	ACIPHB048:	18

4.8	test FDN	18

4.8.1	ACIPHB051: initialize phonebook	18

4.8.2	ACIPHB052: use verbose <err> values	18

4.8.3	ACIPHB053: setting PHB memory storage	18

4.8.4	ACIPHB054: reading memory storage info	18

4.8.5	ACIPHB055: getting supported location ranges of phb storage	18

4.8.6	ACIPHB056: select and reading an entries (FDN_Alpha_length = 20)	18

4.8.7	ACIPHB057: getting supported location ranges of phb storage	18

4.9	test LDN-entries on SIM (ACIPHB061 – ACIPHB070)	20

4.9.1	ACIPHB061: initialize phonebook	20

4.9.2	ACIPHB062: setting PHB memory storage	21

4.9.3	ACIPHB063: reading memory storage info	21

4.9.4	ACIPHB064: reading existing LD entries (LDN_Alpha_length = 20)	21

4.9.5	ACIPHB065: Dial some number (0174 4444444) ??????	21

4.9.6	ACIPHB066: reading memory storage info after ter the call	21

4.9.7	ACIPHB067: reading existing+new LD entries (LDN_Alpha_length = 20)	21

# Document Control

•	Copyright Condat DV-Beratung Organisation und Software GmbH, 1998.

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Condat GmbH reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Condat GmbH. Condat GmbH accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a licence agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Condat GmbH.

Condat DV-Beratung

Organisation und Software GmbH

Alt Moabit 91d

10559 Berlin

Germany

Telephone:	+49.30.39094-0

Fax:	+49.30.39094-300

Internet:	http://www.condat.de

E-mail:	gsm@condat.deDocument History

Document Id.	Date	Author	Remarks

8411.417.99.???	5 Oktober 1999	DAK	Initial

## References

[1]	Rec. T.4 Standardisation of group 3 facsimile apparatus for document transmission;
(CCITT-T.4, 1984)

[2]	ITU-T Recommendation T.30; Series T: Terminal equipments and protocols for telematic services;
Procedures for document facsimile transmission in the general switched
telephone network;
(ITU-T.30, 1996)

[3]	ITU-T Recommendation T.31; Terminals for telematic services;
Asynchronous facsimile DCE control - service class 1
(ITU-T.31, 1995)

[4]	ITU-T Recommendation T.32; Terminals for telematic services;
Asynchronous facsimile DCE control - service class 2
(ITU-T.32, 1995)

[5]	Rec. T.35; Terminal equipment and protocols for telematic services;
Procedures for the allocation of CCITT definde codes for non-standard facilities;
(CCITT-T.35, 1991)

[6]	ITU-T Recommendation V.25 ter; Series V: data communication over the telephone network;
Interfaces and voiceband modems; Serial asynchronous automatic dialling and control
(ITU-T V.25 ter, 1997)

[7]	Rec. V.42 bis Data compression procedures for data circuit terminating equipment (DCE) using error correction procedures;
(CCITT-V.42 bis, 1990)

[8]	Rec. V.110 (Blue book, Vol. VIII, Fascicle VIII.1) Support of data terminal equipments (DTEs) with V-series type interfaces by an integrated services digital network (ISDN);
(CCITT-V.110, 1988)

[9]	European digital cellular telecommunications system (Phase 2);
GSM Public Land Mobile Network (PLMN) connection types;
(GSM 3.10, September 1994, version 4.3.1)

[10]	European digital cellular telecommunications system (Phase 2);
Technical realisation of facsimile group 3 transparent;
(GSM 3.45, September 1995, version 4.5.0)

[11]	Digital cellular telecommunications system (Phase 2);
Mobile radio interface layer 3 specification;
(GSM 4.08, November 1996, version 4.17.0)

[12]	European digital cellular telecommunications system (Phase 2);
Rate adaptation on the Mobile Station - Base Station System (MS - BSS) Interface;
(GSM 4.21, May 1995, version 4.6.0)

[13]	European digital cellular telecommunications system (Phase 2);
Radio Link Protocol (RLP) for data and telematic services on the Mobile Station - Base Station System (MS - BSS) interface and the Base Station System - Mobile-service Switching Centre (BSS - MSC) interface
(GSM 4.22, September 1994, version 4.3.0)

[14]	European digital cellular telecommunications system (Phase 2);
Radio Link Protocol (RLP) for data and telematic services on the Mobile Station - Base Station System (MS - BSS) interface and the Base Station System - Mobile-service Switching Centre (BSS - MSC) interface
(Amendment prA1 for GSM 4.22, version 4.3.0)
(GSM 4.22, March 1995, version 4.4.0)

[15]	European digital cellular telecommunications system (Phase 2);
General on Terminal Adaptation Functions (TAF) for Mobile Stations (MS);
(GSM 7.01, December 1995, version 4.10.0)

[16]	European digital cellular telecommunications system (Phase 2);
Terminal Adaptation Functions (TAF) for services using asynchronous bearer capabilities;
(GSM 7.02, September 1994, version 4.5.1)

[17]	European digital cellular telecommunications system (Phase 2);
Terminal Adaptation Functions (TAF) for services using synchronous bearer capabilities;
(GSM 7.03, September 1994, version 4.5.1)

[18]	Digital cellular telecommunications system (Phase 2);
Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Services (CBS);
(GSM 7.05, November 1996, version 4.8.0)

[19]	Digital cellular telecommunications system (Phase 2);
AT command set for GSM Mobile Equipment (ME)
(GSM 7.07, May 1996, version 4.1.0)

[20]	Digital cellular telecommunication system (Phase 2);
Mobile Station (MS) conformance specification;
Part 1: Conformance specification
(GSM 11.10-1, November 1996, version 4.17.0)

[21]	Digital cellular telecommunications system (Phase 2);
Mobile Station (MS) conformance specification;
Part 2: Protocol Implementation Conformance Statement (PICS)
proforma specification
(GSM 11.10-2, May 1996, version 4.15.0)

[22]	Digital cellular telecommunications system (Phase 2);
Mobile Station (MS) conformance specification;
Part 3: Layer 3 (L3) Abstract Test Suite (ATS)
(GSM 11.10-3, November 1996, version 4.17.0)

[23]	Proposal for Rate Adaptation implemented on a DSP;
(C. Bianconi, Texas Instruments, January 1998, version 1.0)

[24]	MCU-DSP Interfaces for Data Applications;
Specification S844
(C. Bianconi, Texas Instruments, March 1998, version 0.1)

[25]	Users Guide
6147.300.96.100; Condat GmbH

[26]	Service Access Point RA
8411.100.98.100; Condat GmbH

[27]	Service Access Point RLP
8411.101.98.100; Condat GmbH

[28]	Service Access Point L2R
8411.102.98.100; Condat GmbH

[29]	Service Access Point FAD
8411.103.98.100; Condat GmbH

[30]	Service Access Point T30
8411.104.98.100; Condat GmbH

[31]	Service Access Point ACI
8411.105.98.100; Condat GmbH

[32]	Message Sequence Charts RLP
8411.201.98.100; Condat GmbH

[33]	Message Sequence Charts L2R
8411.202.98.100; Condat GmbH

[34]	Message Sequence Charts FAD
8411.203.98.100; Condat GmbH

[35]	Message Sequence Charts T30
8411.204.98.100; Condat GmbH

[36]	Message Sequence Charts ACI
8411.205.98.100; Condat GmbH

[37]	Proposal for Fax & Data Integration; March 1998
8411.300.98.100; Condat GmbH

[38]	Test Specification RLP
8411.401.98.100; Condat GmbH

[39]	Test Specification L2R
8411.402.98.100; Condat GmbH

[40]	Test Specification FAD
8411.403.98.100; Condat GmbH

[41]	Test Specification T30
8411.404.98.100; Condat GmbH

[42]	Test Specification ACI
8411.405.98.100; Condat GmbH

[43]	SDL Specification RLP
8411.501.98.100; Condat GmbH

[44]	SDL Specification L2R
8411.502.98.100; Condat GmbH

[45]	SDL Specification FAD
8411.503.98.100; Condat GmbH

[46]	SDL Specification T30
8411.504.98.100; Condat GmbH

[47]	SDL Specification ACI
8411.505.98.100; Condat GmbH

[48]	Technical Documentation RLP
8411.701.98.100; Condat GmbH

[49]	Technical Documentation L2R
8411.702.98.100; Condat GmbH

[50]	Technical Documentation FAD
8411.703.98.100; Condat GmbH

[51]	Technical Documentation T30
8411.704.98.100; Condat GmbH

[52]	Technical Documentation ACI
8411.705.98.100; Condat GmbH

## Abbreviations

ACI	AT Command Interpreter
AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BCS	Binary Coded Signals
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface
EOL	End Of Line

F	Final Bit
F&D	Fax and Data Protocol Stack
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HDLC	High level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LISR	Low level Interrupt Service Routine
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MSG	Message phase in the GSM 3.45 protocol

N(R)	Receive Number
N(S)	Send Number
NCC	National Colour Code
NECI	New Establishment Causes included

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Description Unit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

T.4	CCITT Standardisation for Document coding of Group 3 Facsimile Apparatus
TAP	Test Application Program
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable
VPLMN	Visiting Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for fax and data transmission consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity. The entity, which is described in this document, is coloured grey in the following figure :

Figure 2-1: Architecture of the fax and data protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the fax and data protocol stack are:

## RA - Rate Adaptation

This entity performs an adaptation between an asynchronous or synchronous data stream with several bit rates on to the fixed bit rate used at the TCH. This is performed by the rate adaptation functions RA1’ and RA0 described in GSM 04.21.

## RLP - Radio Link Protocol

This entity provides a Layer 2 protocol for asynchronous reliable data transfer as specified in GSM 04.22. It includes error correction, sequence numbers and a mechanism for repeating corrupted and lost messages.

## L2R - Layer 2 Relay Functionality

The L2R provides relay functions in order to adapt the character-oriented data received from the TE via USART to the bit-oriented RLP protocol.

## FAD 03.45 - Fax Adaptation Protocol

The fax adaptation protocol, as specified in GSM 03.45, provides synchronisation with the BCS and MSG modems of the peer entity. It uses byte repetition in conjunction with a voting algorithm to handle corruption on the TCH data stream. The non-transparent fax protocol in accordance with GSM 03.46 is not part of this implementation.

The fax adapter enables T.30 to send BCS at 300 BPS and T.4 MSG in 2400, 4800, 7200 and 9600 BPS.

## T.30 - Fax Protocol Entity

The protocol uses binary coded signals packed in HDLC frames to set up and release a connection in the message phase of the FAX transmission. This entity is specified in the ITU-T.30. The main tasks of this unit are:

- Building the HDLC frames with CRC.

- Performing bit stuffing/de-stuffing.

- Executing a sequence of 5 phases: 1.) set up, 2.) pre-message procedures, 3.) transmission/reception, 4.) post message procedures, 5.) waiting for call release.

## ACI - AT Command Interpreter

The ACI is specified in GSM 07.07. It is responsible for call establishment via the GSM voice protocol stack and terminal adaptation for asynchronous transparent character-oriented data transmission. The ACI is able to receive AT commands and send the replies over the USART driver to a remote PC. This makes it possible to control the voice and data protocol stack from a remote application running on a PC. The ACI also provides a unique interface for an internal MMI in the MS.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the F&D. The driver has to be able to perform flow control.

# Parameters

/* --- array declarations --------------------------------------------------------------------------------------------- */

- DECLARATION (A_PIN_1234 )

- DECLARATION (EC_CODES)
DECLARATION (NO_PREF_LANG)
DECLARATION (IMSI_FIELD_DATA)
DECLARATION (SIM_SERV_ADN_BDN)
DECLARATION (PREF_PLMN_DATA)
DECLARATION (DATA_BDN)
DECLARATION (DATA_ADN34_1)
DECLARATION (DATA_ADN34_2)
DECLARATION (DATA_EMPTY_ADN34)
DECLARATION (DATA_ADN38_1)
DECLARATION (DATA_ADN38_2)
DECLARATION (DATA_EMPTY_ADN38)
DECLARATION (DATA_ADN54_1)
DECLARATION (DATA_ADN54_2)
DECLARATION (DATA_EMPTY_ADN54)
DECLARATION (DATA_ADN94_1)
DECLARATION (DATA_ADN94_2)
DECLARATION (DATA_EMPTY_ADN94)
DECLARATION (EMPTY_FDN)

DECLARATION (DATA_FDN_1)

DECLARATION (DATA_FDN_2)

DECLARATION (DATA_EMPTY_FDN)

DECLARATION (DATA_SDN)
DECLARATION (STK_PRO_FILE)
DECLARATION (SIM_SERV_FDN)

DECLARATION (DATA_ADNP1)
DECLARATION (DATA_ADNP)
DECLARATION (DATA_ADNP3)
DECLARATION(PHN_NUM0)

DECLARATION (SIM_SERV_LND)

DECLARATION (EMPTY_LND)

DECLARATION (DATA_LND_1)

DECLARATION (DATA_LND_2)

DECLARATION (DATA_LND_3)

DECLARATION (DATA_LND_4)

DECLARATION (DATA_EMPTY_LND)

DECLARATION (S_BS_VOICE)

DECLARATION (S_BS_NOT_PRESENT)
DECLARATION (S_CLD_PARTY)
DECLARATION (S_CLD_PARTY_SUB)

- DECLARATION ( A_CLD_NUM )

DECLARATION (S_CHN_SPEECH)

DECLARATION (S_CLG_PARTY)

DECLARATION (S_CLG_PARTY_SUB)

DECLARATION (A_CLG_NUM)

/* --- structure declarations  --------------------------------------------------------------------------------------- */

DECLARATION (IMSI_FIELD)
DECLARATION(PREF_PLMN)
DECLARATION (A_ECC_FIELD )

DECLARATION (BC_PARA_SPEECH)
DECLARATION (BC_PARA_NO_SERVICE)
DECLARATION(CLED_PARTY0) 
DECLARATION(CLED_PARTY_SUB_NONE) 
DECLARATION(CONNECTED_NUMBER0)
DECLARATION(EMPTY_PHN_NUM)

/* Number definitions */
BYTE NOT_SPEC 255
BYTE MAX_DATAS 0xFF

BYTE NUM_0 0

BYTE NUM_1 1

BYTE NUM_2 2

BYTE NUM_3 3

BYTE NUM_4 4

BYTE NUM_5 5

BYTE NUM_6 6

BYTE NUM_7 7

BYTE NUM_8 8

BYTE NUM_9 9

BYTE NUM_10 10

BYTE NUM_11 11

BYTE NUM_12 12

BYTE NUM_20 20

BYTE NUM_22 22

BYTE NUM_33 33

BYTE NUM_50 50

BYTE NUM_82 82

BYTE NUM_FF 255

SHORT NUM_9600 9600

SHORT NUM_4800 4800

BYTE LDATE_EMPTY 255

BYTE LDATA_BDN 34

BYTE LDATA_ADN34 34

BYTE LDATA_ADN38 38

BYTE LDATA_ADN54 54

BYTE LDATA_ADN94 94

BYTE LDATA_FDN 34

BYTE LDATA_SDN 34

BYTE LDATA_ADNP1 26

BYTE LDATA_ADNP 26

BYTE LDATA_LND 34

/* MMI profile parameter, ADN, FDN, BDN supported */

BYTE MMI_PRO_FILE 0xE0

/*---"OK"—(successful operation) */

//	Message:

STRING(M_OK, "OK" )

BYTE LM_OK 2

/*---"NO CARRIER"---*/

//	Message:

STRING(M_NO_CARRIER, "NO CARRIER" )

BYTE LM_NO_CARRIER 10

/*---"NO ANSWER"---*/

//	Message:

STRING(M_NO_ANSWER, "NO ANSWER" )

BYTE LM_NO_ANSWER 9

/*---"BUSY"---*/

//	Message:

STRING(M_BUSY, "BUSY" )

BYTE LM_BUSY 4

/*---"BUSY"---*/

//	Message:

STRING(M_RING, "RING" )

BYTE LM_RING 4

/*---" CONNECT"---*/

//	Message:

STRING(M_CONNECT, "CONNECT" )

BYTE LM_CONNECT 7

/*---"NO DIALTONE"---*/

//	Message:

STRING(M_NO_DIALTONE, "NO DIALTONE" )

BYTE LM_NO_DIALTONE 11

/* PIN 1234 array */

//	Message:

STRING(M_ERROR, "ERROR" )

BYTE LM_ERROR 5

ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
<A>	cmd_seq	C_CPBS_EN
<B>	cmd_seq	C_CPBS_LD
<C>	cmd_seq	C_CPBS_LR
<D>	cmd_seq	C_CPBS_AD
<E>	cmd_seq	C_CPBS_LM
<F>	cmd_seq	C_CPBS_AF

- STRING(C_PLUS_CPIN_1234, "AT+CPIN=\"1234\" " )

- BYTE LC_PLUS_CPIN_1234 14

/*---"CLIR_COLP"—(CLIR_COLP_S) */

//	Command:

- STRING(C_CLIR_COLP_S, "AT+CLIR=2;+COLP=1" )

- BYTE LC_CLIR_COLP_S 18

/*---"CME ERROR"—(verbose error result code) */

//	Message:

STRING(M_CME_ERR_INV_OPP, "+CME ERROR: operation not allowed" )

BYTE LM_CME_ERR_INV_OPP 33

STRING(M_CME_ERR_INV_IDX, "+CME ERROR: invalid index" )

BYTE LM_CME_ERR_INV_IDX 25

STRING(M_CME_ERR_UNKN, "+CME ERROR: unknown" )

BYTE LM_CME_ERR_UNKN 19

STRING(M_CME_ERR_OTHER, "+CME ERROR: other error" )

BYTE LM_CME_ERR_OTHER 23

STRING(M_CME_ERR_PIN1_REQ, "+CME ERROR: SIM PIN required" )

BYTE LM_CME_ERR_PIN1_REQ 28

STRING(M_CME_ERR_PIN2_REQ, "+CME ERROR: SIM PIN2 required" )

BYTE LM_CME_ERR_PIN2_REQ 29

STRING(M_CME_ERR_PUK1_REQ, "+CME ERROR: SIM PUK required" )

BYTE LM_CME_ERR_PUK1_REQ 28

STRING(M_CME_ERR_PUK2_REQ, "+CME ERROR: SIM PUK2 required" )

BYTE LM_CME_ERR_PUK2_REQ 29

STRING(M_CME_ERR_NO_SIM, "+CME ERROR: SIM not inserted" )

BYTE LM_CME_ERR_NO_SIM 28

STRING(M_CME_ERR_SIM_FAIL, "+CME ERROR: SIM failure" )

BYTE LM_CME_ERR_SIM_FAIL 23

STRING(M_CME_ERR_SIM_WRONG, "+CME ERROR: SIM wrong" )

BYTE LM_CME_ERR_SIM_WRONG 21

STRING(M_CME_ERR_INV_PWD, "+CME ERROR: incorrect password" )

BYTE LM_CME_ERR_INV_PWD 30

STRING(M_CME_ERR_NOT_FOUND, "+CME ERROR: not found" )

BYTE LM_CME_ERR_NOT_FOUND 21

/*---"CFUN=1"—(CFUN_S) */

//	Command:

STRING(C_CFUN_S, "AT+CFUN=1" )

BYTE LC_CFUN_S 10

/*---"CMEE"—(CMEE_S) */

//	Command:

STRING(C_CMEE_S2, "AT+CMEE=2" )

BYTE LC_CMEE_S 10

/*---"CPBS=?"—(CPBS_T) */

//	Command:

STRING(C_CPBS_T, "AT+CPBS=?" )

BYTE LC_CPBS_T 10

//	Message:

STRING(M_CPBS_T, "+CPBS: (\"EN\",\"BD\",\"FD\",\"LD\",\"LR\",\"AD\",\"SD\",\"LM\",\"AF\")" )

BYTE LM_CPBS_T 53

/*---"CPBS?"—(CPBS_Q) */

//	Command:

STRING(C_CPBS_Q, "AT+CPBS?" )

BYTE LC_CPBS_Q 9

//	Message:

STRING(M_CPBS_EN, "+CPBS: \"EN\",3,4" )

STRING(M_CPBS_BD, "+CPBS: \"BD\",1,1" )

STRING(M_CPBS_FD, "+CPBS: \"FD\",2,4" )

STRING(M_CPBS_LD, "+CPBS: \"LD\",0,10" )

STRING(M_CPBS_LD0, "+CPBS: \"LD\",0,10" )

STRING(M_CPBS_LD1, "+CPBS: \"LD\",1,10" )

STRING(M_CPBS_LD3, "+CPBS: \"LD\",3,10" )

STRING(M_CPBS_LD4, "+CPBS: \"LD\",4,10" )

STRING(M_CPBS_LR, "+CPBS: \"LR\",0,10" )

STRING(M_CPBS_AD, "+CPBS: \"AD\",2,4" )

STRING(M_CPBS_SD, "+CPBS: \"SD\",1,1" )

STRING(M_CPBS_LM, "+CPBS: \"LM\",0,10" )

STRING(M_CPBS_AF, "+CPBS: \"AF\",2,4" )

BYTE LM_CPBS_EN 15

BYTE LM_CPBS_BD 15

BYTE LM_CPBS_FD 15

BYTE LM_CPBS_LD 16

BYTE LM_CPBS_LD0 16

BYTE LM_CPBS_LD1 16

BYTE LM_CPBS_LD3 16

BYTE LM_CPBS_LD4 16

BYTE LM_CPBS_LR 16

BYTE LM_CPBS_AD 15

BYTE LM_CPBS_SD 15

BYTE LM_CPBS_LM 16

BYTE LM_CPBS_AF 15

/*---"CPBS="—(CPBS_S) */

//	Command:

STRING(C_CPBS_EN, "AT+CPBS=\"EN\"" )

STRING(C_CPBS_BD, "AT+CPBS=\"BD\"" )

STRING(C_CPBS_FD, "AT+CPBS=\"FD\"" )

STRING(C_CPBS_LD, "AT+CPBS=\"LD\"" )

STRING(C_CPBS_LR, "AT+CPBS=\"LR\"" )

STRING(C_CPBS_AD, "AT+CPBS=\"AD\"" )

STRING(C_CPBS_SD, "AT+CPBS=\"SD\"" )

STRING(C_CPBS_LM, "AT+CPBS=\"LM\"" )

STRING(C_CPBS_AF, "AT+CPBS=\"AF\"" )

STRING(C_CPBS_SM, "AT+CPBS=\"SM\"" )

STRING(C_CPBS_AA, "AT+CPBS=\"AA\"" )

STRING(C_CPBS_XX, "AT+CPBS=\"XX\"" )

BYTE LC_CPBS_S 13

/*---"CPBR=?"—(CPBR_T) */

//	Command:

STRING(C_CPBR_T, "AT+CPBR=?" )

BYTE LC_CPBR_T 10

//	Message:  ADN_Length = 34

STRING(M_CPBR_T_EN1, "+CPBR: (1-4),20,0" )

STRING(M_CPBR_T_LD1, "+CPBR: (1-10),20,0" )

STRING(M_CPBR_T_LR1, "+CPBR: (1-10),20,0" )

STRING(M_CPBR_T_AD1, "+CPBR: (1-4),20,20" )

STRING(M_CPBR_T_LM1, "+CPBR: (1-10),20,0" )

STRING(M_CPBR_T_AF1, "+CPBR: (1-4),20,20" )

STRING(M_CPBR_T_FD1, "+CPBR: (1-4),20,20" )

BYTE LM_CPBR_T_EN1 17

BYTE LM_CPBR_T_LD1 18

BYTE LM_CPBR_T_LR1 18

BYTE LM_CPBR_T_AD1 18

BYTE LM_CPBR_T_LM1 18

BYTE LM_CPBR_T_AF1 18

BYTE LM_CPBR_T_FD1 18

//	Message:  ADN_Length = 38

STRING(M_CPBR_T_AD2, "+CPBR: (1-4),20,20" )

STRING(M_CPBR_T_AF2, "+CPBR: (1-4),20,20" )

BYTE LM_CPBR_T_AD2 18

BYTE LM_CPBR_T_AF2 18

//	Message:  ADN_Length = 54

// PHB_MAX_TAG_LEN is limited to 20… STRING(M_CPBR_T_AD3, "+CPBR: (1-4),20,40" )

STRING(M_CPBR_T_AD3, "+CPBR: (1-4),20,20" )

// PHB_MAX_TAG_LEN is limited to 20… STRING(M_CPBR_T_AF3, "+CPBR: (1-4),20,40" )

STRING(M_CPBR_T_AF3, "+CPBR: (1-4),20,20" )

BYTE LM_CPBR_T_AD3 18

BYTE LM_CPBR_T_AF3 18

//	Message:  ADN_Length = 94

// PHB_MAX_TAG_LEN is limited to 20… STRING(M_CPBR_T_AD4, "+CPBR: (1-4),20,80" )

STRING(M_CPBR_T_AD4, "+CPBR: (1-4),20,20" )

// PHB_MAX_TAG_LEN is limited to 20… STRING(M_CPBR_T_AF4, "+CPBR: (1-4),20,80" )

STRING(M_CPBR_T_AF4, "+CPBR: (1-4),20,20" )

BYTE LM_CPBR_T_AD4 18

BYTE LM_CPBR_T_AF4 18

/*---"CPBR?"—(CPBR_Q) */

//	Command:

STRING(C_CPBR_Q, "AT+CPBR?" )

BYTE LC_CPBR_Q 9

/*---"CPBR=<n>"—(CPBR_S) */

//	Command:

STRING(C_CPBR_ALL, "AT+CPBR=1,2")

BYTE LC_CPBR_ALL 11

STRING(C_CPBR_S0, "AT+CPBR=1" )

STRING(C_CPBR_S1, "AT+CPBR=2" )

STRING(C_CPBR_S2, "AT+CPBR=3" )

STRING(C_CPBR_S7, "AT+CPBR=7" )

BYTE LC_CPBR_S 10

STRING(C_CPBR_LDS0, "AT+CPBR=1" )

BYTE LC_CPBR_LDS0 10

STRING(M_CPBR_LD, "+CPBR: 1,\"03076903367\",129,\"toto\"" )

BYTE LM_CPBR_LD 42

STRING(M_CPBR_LD1, "+CPBR: 1,\"03039094336\",129,\"\"" )

BYTE LM_CPBR_LD1 29

STRING(M_CPBR_LD2, "+CPBR: 1,\" 03076903367\",129,\"\"" )

BYTE LM_CPBR_LD2 32

//	Message: 	(ADN_Alpha_length = 20)

STRING(M_CPBR_AD1, "+CPBR: 1,\"00491239876543\",129,\"Heier, Max\"" )

STRING(M_CPBR_AD2, "+CPBR: 2,\"9123456789\",161,\"Hans Egon\"" )

BYTE LM_CPBR_AD1 42

BYTE LM_CPBR_AD2 37

//	Message: 	(ADN_Alpha_length > 20)

STRING(M_CPBR_AD10, "+CPBR: 1,\"9123456789\",161,\"ABCDEFGHIJABCDEFGHIJ\"" )

STRING(M_CPBR_AD11, "+CPBR: 2,\"9123456789\",161,\"abcdefghijabcdefghij\"" )

BYTE LM_CPBR_AD10 48

//	Message: 	(FDN_Alpha_length = 20)

STRING(M_CPBR_FD1, "+CPBR: 1,\"00491239876543\",129,\"Heier, Max\"" )

STRING(M_CPBR_FD2, "+CPBR: 2,\"9123456789\",161,\"Hans Egon\"" )

STRING(M_CPBR_FD3, "+CPBR: 3,\"12345678901234567890\",129,\"toto\"" )

BYTE LM_CPBR_FD1 42

BYTE LM_CPBR_FD2 37

//	Message: 	(SDN_Alpha_length = 20)

STRING(M_CPBR_SD, "+CPBR: 1,\"003076903367\",129,\"Heier, Max\"" )

BYTE LM_CPBR_SD 40

STRING(M_CPBR_BD, "+CPBR: 1,\"003076903367\",129,\"Heier, Max\"" )

BYTE LM_CPBR_BD 40

STRING(C_CPBR_LD1TO3,”AT+CPBR=1,3”)

BYTE LC_CPBR_LD1TO3 11

//	Message: 	(LDN_Alpha_length = 20)

STRING(M_CPBR_LD1OF3, "+CPBR: 1,\"01733333333\",129,\"Dial 3\"" )

STRING(M_CPBR_LD2OF3, "+CPBR: 2,\"01722222222\",129,\"Dial 2\"" )

STRING(M_CPBR_LD3OF3, "+CPBR: 3,\"01711111111\",129,\"Dial 1\"" )

BYTE LM_CPBR_LD1OF3 35

BYTE LM_CPBR_LD2OF3 35

BYTE LM_CPBR_LD3OF3 35

//	The last dialed entry will be the first record. The older ACI does not store the alpha-tag!

// STRING(M_CPBR_LD1OF3, "+CPBR: 1,\"01733333333\",129,\"\"" )

// STRING(M_CPBR_LD2OF3, "+CPBR: 2,\"01722222222\",129,\"\"" )

// STRING(M_CPBR_LD3OF3, "+CPBR: 3,\"01711111111\",129,\"\"" )

// BYTE LM_CPBR_LD1OF3 29

// BYTE LM_CPBR_LD2OF3 29

// BYTE LM_CPBR_LD3OF3 29

// and the same entries after one call shifted one position down:

STRING(C_D_VOICE,”ATD01744444444;”)

STRING(C_CPBR_LD1TO4,”AT+CPBR=1,4”)

BYTE LC_CPBR_LD1TO4 12

STRING(M_CPBR_LD1OF4, "+CPBR: 1,\"01744444444\",129,\"\"" )

STRING(M_CPBR_LD2OF4, "+CPBR: 2,\"01733333333\",129,\"Dial 3\"" )

STRING(M_CPBR_LD3OF4, "+CPBR: 3,\"01722222222\",129,\"Dial 2\"" )

STRING(M_CPBR_LD4OF4, "+CPBR: 4,\"01711111111\",129,\"Dial 1\"" )

BYTE LM_CPBR_LD1OF4 29

BYTE LM_CPBR_LD2OF4 35

BYTE LM_CPBR_LD3OF4 35

BYTE LM_CPBR_LD4OF4 35

/*---"CPBF=?"—(CPBF_T) */

//	Command:

STRING(C_CPBF_T, "AT+CPBF=?" )

BYTE LC_CPBF_T 10

//	Message:

STRING(M_CPBF_T, "+CPBF: 20,20" )

BYTE LM_CPBF_T 12

/*---"CPBF?"—(CPBF_Q) */

//	Command:

STRING(C_CPBF_Q, "AT+CPBF?" )

BYTE LC_CPBF_Q 9

/*---"CPBF="...""—(CPBF_S) */

//	Command:

STRING(C_CPBF_S0, "AT+CPBF=\"H\"" )

STRING(C_CPBF_S1, "AT+CPBF=\"Hans\"" )

STRING(C_CPBF_S2, "AT+CPBF=\"Heier\"" )

STRING(C_CPBF_S3, "AT+CPBF=\"Max\"" )

STRING(C_CPBF_S4, "AT+CPBF=\"XXX\"" )

STRING(C_CPBF_S5, "AT+CPBF=\"0123\"" )

STRING(C_CPBF_S6, "AT+CPBF=2" )

BYTE LC_CPBF_S0 12

BYTE LC_CPBF_S1 15

BYTE LC_CPBF_S2 16

BYTE LC_CPBF_S3 14

BYTE LC_CPBF_S4 14

BYTE LC_CPBF_S5 15

BYTE LC_CPBF_S6 10

//	Message:

STRING(M_CPBF_S0, "+CPBF: 1,\"00491239876543\",129,\"Heier, Max\"" )

STRING(M_CPBF_S1, "+CPBF: 2,\"9123456789\",161,\"Hans Egon\"" )

BYTE LM_CPBF_S0 42

BYTE LM_CPBF_S1 37

/*---"CPBW=?"—(CPBW_T) */

//	Command:

STRING(C_CPBW_T, "AT+CPBW=,\"03076903367\",129,\"toto\"" )

BYTE LC_CPBW_T 33

STRING(C_CPBW_T1, "AT+CPBW=,\"12345678901234567890\",129,\"toto\"" )

STRING(C_CPBW_S2, "AT+CPBW=,\"03012345678\",129" )

BYTE LC_CPBW_S2 26

STRING(C_CPBW_P, "AT+CPBW=2,\"03076903367\",129,\"P\"" )

BYTE LC_CPBW_P 31

STRING(C_CPBW_AD1, "AT+CPBW= 1,\"03012345678\",129,\"\"" )

BYTE LC_CPBW_AD1 31

STRING(C_CPBW_AD10, "AT+CPBW= 10,\"03012345678\",129,\"\"" )

BYTE LC_CPBW_AD10 32

STRING(C_CPBW_AD255, "AT+CPBW= 255,\"03012345678\",129,\"\"" )

BYTE LC_CPBW_AD255 33

STRING(C_CPBR_A2, "+CPBR: 1,\"03012345678\",129,\"\"" )

BYTE LC_CPBR_A2 29

STRING(atD_forb, "ATD003076903367;" )

BYTE LatD_forb 16

STRING(atD_ATD_AD, "ATD>AD2;" )

BYTE LatD_ATD_AD 8

STRING(atD_ATD_AD1, "ATD>AD1;" )

BYTE LatD_ATD_AD1 8

STRING(atD_ATD_AD10, "ATD>AD10;" )

BYTE LatD_ATD_AD10 9

STRING(atD_ATD_AD255, "ATD>AD255;" )

BYTE LatD_ATD_AD255 10

//	Message:

STRING(M_CPBW_T_EN, "+CPBW: (1-5),20,(128-201),0" )

STRING(M_CPBW_T_AD, "+CPBW: (1-4),20,(128-201),20")

STRING(M_CPBW_T_LD, "+CPBW: (1-10),20,(128-201),0" )

STRING(M_CPBW_T_LR, "+CPBW: (1-10),20,(128-201),0" )

STRING(M_CPBW_T_LM, "+CPBW: (1-10),20,(128-201),0" )

BYTE LM_CPBW_T_EN 27

BYTE LM_CPBW_T_AD 28

BYTE LM_CPBW_T_LD 28

BYTE LM_CPBW_T_LR 28

BYTE LM_CPBW_T_LM 28

STRING(M_D0, "+COLP: \"03012345678\",129,,128" )

BYTE LM_D0 29

/*----------------------------------------  fields ------------------------------------------------------ */

/* --- phone number  "03012345678"---*/

BEGINARRAY_PART (PHN_NUM0, 11)

0x00, 0x03, 0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08

ENDARRAY

BEGINARRAY (STK_PRO_FILE, 12)

0x0C,

0x00,

0x00,

0x00,

0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00

ENDARRAY

BEGINARRAY (EC_CODES, 15)

0x11, 0xF2, 0xFF,
	0x99, 0xF9, 0xFF,
	0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF

ENDARRAY

BEGINARRAY (NO_PREF_LANG, 5)

0xFF,0xFF,0xFF,0xFF,0xFF

ENDARRAY

BEGINARRAY (IMSI_FIELD_DATA, 9)

0x02,0x21,0x30,0xFF,0xFF,0xFF,0xFF,0xFF,0xFF

ENDARRAY

BEGINARRAY (SIM_SERV_ADN_BDN, 16)
	0xCF, 	//	1100 1111
	0x3C,	//	0011 1100
	0x3C,	//	0011 1100
//	0x03,	//	0000 0011	Disable SIM-LND
	0x00,	//	0000 0000
	0xFF,	//	1111 1111
	0x00,	//	0000 0000
	0x00,	//	0000 0000
	0x30,	//	0011 0000
	0x00,	//	0000 0000
	0x00,	//	0000 0000
	0x00,	//	0000 0000
	0x00,	//	0000 0000
	0x00,	//	0000 0000
	0x00,	//	0000 0000
	0x00,	//	0000 0000
	0x00	//	0000 0000
ENDARRAY

BEGINARRAY (PREF_PLMN_DATA, 96)

0x21, 0xF3, 0x89,	0x89, 0xF7, 0x21 ,	0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF, 	0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF, 	0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF, 	0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF, 	0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF, 	0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF, 	0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF, 	0xFF, 0xFF, 0xFF ,	 0xFF, 0xFF, 0xFF

ENDARRAY

// BDN entries , length of alpha idenitfier = 20

//	"30 76 90 33 67",129,"Heier, Max"

BEGINARRAY(DATA_BDN, 34)

0x48, 0x65, 0x69, 0x65, 0x72, 0x2C, 0x20, 0x4D, 0x61, 0x78,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0x08,

0x81,

0x00, 0x03, 0x67, 0x09, 0x33, 0x76, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

// ADN entries , length of alpha idenitfier = 20

//	"00 49 123 9876543",129,"Heier, Max"

BEGINARRAY (DATA_ADN34_1, 34)

0x48, 0x65, 0x69, 0x65, 0x72, 0x2C, 0x20, 0x4D, 0x61, 0x78,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0x08,

0x81,

0x00, 0x94, 0x21, 0x93, 0x78, 0x56, 0x34, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

//	"9123 456789",161,"Hans Egon"

BEGINARRAY (DATA_ADN34_2, 34)

0x48, 0x61, 0x6E, 0x73, 0x20, 0x45, 0x67, 0x6F, 0x6E,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0x06,

0xA1,

0x19, 0x32, 0x54, 0x76, 0x98, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

//	empty entry

BEGINARRAY (DATA_EMPTY_ADN34, 33)

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0x00,

0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

//	"00 49 123 9876543",129,"P1"

BEGINARRAY (DATA_ADNP1, 26)

0x50, 0x31,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0x08,

0x81,

0x00, 0x94, 0x21, 0x93, 0x78, 0x56, 0x34, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

//	"00 49 123 9876543",129,"P"

BEGINARRAY (DATA_ADNP, 25)

0x50,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0x08,

0x81,

0x00, 0x94, 0x21, 0x93, 0x78, 0x56, 0x34, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

//	"00 49 123 9876543",129,"P3"

BEGINARRAY (DATA_ADNP3, 26)

0x50, 0x33,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0x08,

0x81,

0x00, 0x94, 0x21, 0x93, 0x78, 0x56, 0x34, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

// ADN entryies , length of alpha idenitfier = 24

BEGINARRAY (DATA_ADN38_1, 38)

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x41, 0x42, 0x43,0x44,
	0x06,

0xA1,

0x19, 0x32, 0x54, 0x76, 0x98, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

BEGINARRAY (DATA_ADN38_2, 38)

0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x61, 0x62, 0x63,0x64,

0x06,

0xA1,

0x19, 0x32, 0x54, 0x76, 0x98, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

BEGINARRAY (DATA_EMPTY_ADN38, 38)

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF,0xFF,
	0x00,

0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

// ADN entries , length of alpha idenitfier = 40

BEGINARRAY (DATA_ADN54_1, 54)

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x06,

0xA1,

0x19, 0x32, 0x54, 0x76, 0x98, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

BEGINARRAY (DATA_ADN54_2, 54)

0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x06,

0xA1,

0x19, 0x32, 0x54, 0x76, 0x98, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

BEGINARRAY (DATA_EMPTY_ADN54, 54)

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0x00,

0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

// ADN entries , length of alpha idenitfier = 80

BEGINARRAY (DATA_ADN94_1, 94)

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x49, 0x4A,
	0x06,

0xA1,

0x19, 0x32, 0x54, 0x76, 0x98, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

BEGINARRAY (DATA_ADN94_2, 94)

0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x61, 0x62, 0x63, 0x64, 0x65, 0x66, 0x67, 0x68, 0x69, 0x6A,
	0x06,

0xA1,

0x19, 0x32, 0x54, 0x76, 0x98, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

BEGINARRAY (DATA_EMPTY_ADN94, 94)

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0x00,

0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

BEGINARRAY (SIM_SERV_FDN, 10)
//	0xEF,	//	1110 1111   should be 0xFF to allocate+activate FDN! MSC
	0xFF,	//	1111 1111
	0x3C,	//	0011 1100
	0x3C,	//	0011 1100
//	0x03,	//	0000 0011	Disable SIM-LND
	0x00,	//	0000 0000
	0xF3,	//	1111 0011
	0x00,	//	0000 0000
	0x30,	//	1100 0000
	0x00,	//	0000 0000
	0x00,	//	0000 0000
	0x00	//	0000 0000
ENDARRAY

BEGINARRAY(EMPTY_FDN, 1)

0xFF
ENDARRAY

// FDN entries , length of alpha idenitfier = 20

//	"00 49 123 9876543",129,"Heier, Max"

BEGINARRAY(DATA_FDN_1, 34)

0x48, 0x65, 0x69, 0x65, 0x72, 0x2C, 0x20, 0x4D, 0x61, 0x78,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0x08,

0x81,

0x00, 0x94, 0x21, 0x93, 0x78, 0x56, 0x34, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

//	"9123 456789",161,"Hans Egon"

BEGINARRAY(DATA_FDN_2, 34)

0x48, 0x61, 0x6E, 0x73, 0x20, 0x45, 0x67, 0x6F, 0x6E,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0x06,

0xA1,

0x19, 0x32, 0x54, 0x76, 0x98, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

//	empty entry

BEGINARRAY(DATA_EMPTY_FDN, 34)

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0x00,

0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

BEGINARRAY (SIM_SERV_LND, 16) 
	0xCF, 	//	1100 1111

0x3C,	//	0011 1100

0x3C,	//	0011 1100

0x03,	//	0000 0011
	0x00,	//	0000 0000

0x00,	//	0000 0000

0x00,	//	0000 0000

0x00,	//	0000 0000
	0x00,	//	0000 0000

0x00,	//	0000 0000

0x00,	//	0000 0000
	0x00,	//	0000 0000
	0x00,	//	0000 0000
	0x00,	//	0000 0000
	0x00,	//	0000 0000
	0x00	//	0000 0000
ENDARRAY

// LDN entries , length of alpha idenitfier = 20

//	"0173 3333333",129,"Dial 3"  (newest entry)

BEGINARRAY(DATA_LND_1, 34)

0x44, 0x69, 0x61, 0x6C, 0x20, 0x33, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0x07,

0x81,

0x10, 0x37, 0x33, 0x33, 0x33, 0xF3, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

//	"0172 222222",129,"Dial 2"

BEGINARRAY(DATA_LND_2, 34)

0x44, 0x69, 0x61, 0x6C, 0x20, 0x32, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0x07,

0x81,

0x10, 0x27, 0x22, 0x22, 0x22, 0xF2, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

//	"0171 1111111",129,"Dial 1"  (oldest entry)

BEGINARRAY(DATA_LND_3, 34)

0x44, 0x69, 0x61, 0x6C, 0x20, 0x31, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0x07,

0x81,

0x10, 0x17, 0x11, 0x11, 0x11, 0xF1, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

//	empty entry

BEGINARRAY(DATA_EMPTY_LND, 34)

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0x00,

0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

//	"0174 4444444",129,""  (manual dialed)

BEGINARRAY_PART(DATA_LND_4, 34)

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0x07,

0x81,

0x10, 0x47, 0x44, 0x44, 0x44, 0xF4, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

/* called number array */

BEGINARRAY_PART (A_CLD_NUM,11) 0x0, 0x1, 0x7, 0x4, 0x4, 0x4, 0x4, 0x4, 0x4, 0x4, 0x4 ENDARRAY

BYTE LA_CLD_NUM 11

/* calling number array */

BEGINARRAY_PART (A_CLG_NUM,11) 0x0, 0x1, 0x7, 0x4, 0x4, 0x4, 0x4, 0x4, 0x4, 0x4, 0x4 ENDARRAY

BYTE LA_CLG_NUM 11

/*----------------------------------------  arrays ------------------------------------------------------ */

// SDN entries , length of alpha idenitfier = 20

//	"30 76 90 33 67",129,"Heier, Max"

BEGINARRAY(DATA_SDN, 34)

0x48, 0x65, 0x69, 0x65, 0x72, 0x2C, 0x20, 0x4D, 0x61, 0x78,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,0xFF,

0x08,

0x81,

0x00, 0x03, 0x67, 0x09, 0x33, 0x76, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF,

0xFF

ENDARRAY

/*----------------------------------------  structures ------------------------------------------------------ */

/* */
BEGIN_PSTRUCT ("imsi_field", IMSI_FIELD)
	SET_COMP ("c_field",	0x09)
	SET_COMP ("field",	IMSI_FIELD_DATA)

ENDSTRUCT

BEGIN_PSTRUCT ("pref_plmn", PREF_PLMN)
	SET_COMP ("c_pref",	0x60)
	SET_COMP ("pref",	PREF_PLMN_DATA)

ENDSTRUCT

/* bearer service not present */
 BEGIN_PSTRUCT ("bcpara", S_BS_NOT_PRESENT)
	SET_COMP ("rate",	UR_NOT_PRES)
	SET_COMP ("bearer_serv",	BEARER_SERV_NOT_PRES)
	SET_COMP ("conn_elem",	CONN_ELEM_NOT_PRES)
	SET_COMP ("stop_bits",	STOP_1_BIT)
	SET_COMP ("data_bits",	DATA_8_BIT)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("flow_control",	NO_FLOW_CONTROL)
	SET_COMP ("modem_type",	MT_NONE)
ENDSTRUCT

/* bearer service voice */
 BEGIN_PSTRUCT ("bcpara", S_BS_VOICE)
	SET_COMP ("rate",	UR_NOT_PRES)
	SET_COMP ("bearer_serv",	BEARER_SERV_SPEECH)
	SET_COMP ("conn_elem",	CONN_ELEM_NOT_PRES)
	SET_COMP ("stop_bits",	STOP_1_BIT)
	SET_COMP ("data_bits",	DATA_8_BIT)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("flow_control",	NO_FLOW_CONTROL)

SET_COMP ("modem_type",	MT_NONE)
ENDSTRUCT

/* called party address national*/
 BEGIN_PSTRUCT ("called_party", S_CLD_PARTY)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num",	LA_CLD_NUM)
	SET_COMP ("called_num",	A_CLD_NUM)
ENDSTRUCT

/* called party sub address */
 BEGIN_PSTRUCT ("called_party_sub", S_CLD_PARTY_SUB)
	SET_COMP ("tos",	TOS_NOT_PRES)
	SET_COMP ("odd_even",	OE_EVEN)
	SET_COMP ("c_subaddr",	NUM_0)
	SKIP_COMP ("subaddr")
ENDSTRUCT

BEGIN_PSTRUCT ("chm", S_CHN_SPEECH)
	SET_COMP ("ch_type",	CH_TCH_F)
	SET_COMP ("ch_mode",	CHM_SPEECH)
ENDSTRUCT

BEGIN_PSTRUCT ("calling_party", S_CLG_PARTY)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("present",	PRES_PRES_ALLOW)
	SET_COMP ("screen",	SCREEN_IND_NOT_PRES)
	SET_COMP ("c_num",	LA_CLG_NUM)
	SET_COMP ("num",	A_CLG_NUM)
ENDSTRUCT

/* calling party sub address */
 BEGIN_PSTRUCT ("calling_party_sub", S_CLG_PARTY_SUB)
	SET_COMP ("tos",	TOS_NOT_PRES)
	SET_COMP ("odd_even",	OE_EVEN)
	SET_COMP ("c_subaddr",	NUM_0)
	SKIP_COMP ("subaddr")
ENDSTRUCT

BEGINARRAY (A_ECC_FIELD,12) 0x11, 0xF2, 0xFF, 0x99, 0xF9, 0xFF, 0x21, 0x43, 0x65, 0xFF, 0xFF, 0xFF ENDARRAY

/* PIN 1234 array */

BEGINARRAY (A_PIN_1234,8) 0x31, 0x32, 0x33, 0x34, 0xFF, 0xFF, 0xFF, 0xFF ENDARRAY

/* --- barear capability info, speech only ----*/

BEGIN_PSTRUCT ("bcpara", BC_PARA_SPEECH)
	SET_COMP ("rate",	UR_NOT_PRES)
	SET_COMP ("bearer_serv",	BEARER_SERV_SPEECH)
	SET_COMP ("conn_elem",	CONN_ELEM_NOT_PRES)
	SET_COMP ("stop_bits",	STOP_1_BIT)
	SET_COMP ("data_bits",	DATA_8_BIT)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("flow_control",	NO_FLOW_CONTROL)
	SET_COMP ("modem_type", 	MT_NONE)
ENDSTRUCT

/* --- barear capability info, no service ----*/

BEGIN_PSTRUCT ("bcpara2", BC_PARA_NO_SERVICE)
	SET_COMP ("rate",	UR_NOT_PRES)
	SET_COMP ("bearer_serv",	BEARER_SERV_NOT_PRES)
	SET_COMP ("conn_elem",	CONN_ELEM_NOT_PRES)
	SET_COMP ("stop_bits",	STOP_1_BIT)
	SET_COMP ("data_bits",	DATA_8_BIT)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("flow_control",	NO_FLOW_CONTROL)
	SET_COMP ("modem_type", 	MT_NONE)
ENDSTRUCT

/* called_party (Called party BCD number) */
BEGIN_PSTRUCT ("called_party", CLED_PARTY0)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num",	NUM_11)
	SET_COMP ("called_num",	PHN_NUM0)
ENDSTRUCT

/* called_party_sub (Called party subaddress) */
 BEGIN_PSTRUCT ("called_party_sub", CLED_PARTY_SUB_NONE)
	SET_COMP ("tos",	TOS_NOT_PRES)
	SET_COMP ("odd_even",	OE_EVEN)
	SET_COMP ("c_subaddr",	NUM_0)
	SET_COMP ("subaddr",	EMPTY_PHN_NUM)
ENDSTRUCT

BEGIN_PSTRUCT ("connected_number", CONNECTED_NUMBER0)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("present",	TOS_NOT_PRES)
	SET_COMP ("screen",	SCREEN_USER_PROV_NOT_SCREEN)
	SET_COMP ("c_num",	NUM_11)
	SET_COMP ("num",	PHN_NUM0)
ENDSTRUCT

/* --- empty phone number ---*/

BEGINARRAY_PART (EMPTY_PHN_NUM, 1)

0x00

- ENDARRAY

# TEST CASES

## Initialisation (ACIPHB001 – ACIPHB10)

### ACIPHB001: Initialisierung

Description:

Preamble:

None

APL		ACI		PS

|		|		|

COMMAND (TAP RESET)
COMMAND (CC RESET)
COMMAND (MM RESET)
COMMAND (SIM RESET)
COMMAND (SS RESET)
COMMAND (MMI RESET)
COMMAND (SMS RESET)
COMMAND (RR RESET)
COMMAND (hCommGRR RESET)
COMMAND (hCommGMM RESET)

COMMAND (PL RESET)
	 |		|		|
COMMAND (TAP REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
COMMAND (MM REDIRECT CLEAR)
COMMAND (SIM REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
COMMAND (MMI REDIRECT CLEAR)
COMMAND (SMS REDIRECT CLEAR)

COMMAND (RR REDIRECT CLEAR)
COMMAND (hCommGRR REDIRECT CLEAR)
COMMAND (hCommGMM REDIRECT CLEAR)
COMMAND (PL REDIRECT CLEAR)

|		|		|
COMMAND (MMI REDIRECT CC TAP)
COMMAND (MMI REDIRECT MM TAP)
COMMAND (MMI REDIRECT SIM TAP)
COMMAND (MMI REDIRECT SS TAP)
COMMAND (MMI REDIRECT MMI TAP)
COMMAND (MMI REDIRECT SMS TAP)
COMMAND (MMI REDIRECT T30 TAP)
COMMAND (MMI REDIRECT L2R TAP)
COMMAND (MMI REDIRECT RA TAP)

COMMAND (MMI REDIRECT RR TAP)
COMMAND (MMI REDIRECT hCommGRR TAP)
COMMAND (MMI REDIRECT hCommGMM TAP)
COMMAND (PL REDIRECT MMI NULL)
	|		|		|
COMMAND (TAP REDIRECT TAP MMI)
COMMAND (MMI REDIRECT MMI TAP)
COMMAND (PL CONFIG STD=3)
	|		|		|

|		|		|

Parametrization:

Primitive	Parameter	Value

History:	14.12.98	AK	Initial

### ACIPHB002: initialize phonebook

Description:

initialize phonebook

Preamble:

ACIPHB001

Variants:

<A>...<D>

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CFUN=1)	|		|
	*	==========================>	*		|

- |		|	SIM_ACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	SIM_ACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*

- |		|	SIM_READ_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_CNF	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

- |		|	MNSMS_REPORT_IND	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CFUN_S
	cmd_seq	C_CFUN_S

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	NOT_USED
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_3
	pin2_cnt	NUM_3
	puk2_cnt	NUM_3
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	SIM_SERV_ADN_BDN
	imsi_field	NOT_USED
	pref_plmn	NOT_USED
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNSMS_REPORT_IND
	state	SMS_STATE_READY

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	NUM_1
	length	MAX_DATAS

- SIM_READ_RECORD_CNF
	datafield	SIM_ADN
	cause	SIM_NO_ERROR
	record	NUM_1
	max_record	NUM_4
<A>	length	LDATA_ADN34
<A>	linear_data	DATA_ADN34_1
<B>	length	LDATA_ADN38
<B>	linear_data	DATA_ADN38_1
<C>	length	LDATA_ADN54
<C>	linear_data	DATA_ADN54_1
<D>	length	LDATA_ADN94
<D>	linear_data	DATA_ADN94_1

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	NUM_2
<A>	length	LDATA_ADN34
<B>	length	LDATA_ADN38
<C>	length	LDATA_ADN54
<D>	length	LDATA_ADN94

- SIM_READ_RECORD_CNF
	datafield	SIM_ADN
	cause	SIM_NO_ERROR
	record	NUM_2
	max_record	NUM_4
<A>	length	LDATA_ADN34
<A>	linear_data	DATA_ADN34_2
<B>	length	LDATA_ADN38
<B>	linear_data	DATA_ADN38_2
<C>	length	LDATA_ADN54
<C>	linear_data	DATA_ADN54_2
<D>	length	LDATA_ADN94
<D>	linear_data	DATA_ADN94_2

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	NUM_3
<A>	length	LDATA_ADN34
<B>	length	LDATA_ADN38
<C>	length	LDATA_ADN54
<D>	length	LDATA_ADN94

- SIM_READ_RECORD_CNF
	datafield	SIM_ADN
	cause	SIM_NO_ERROR
	record	NUM_3
	max_record	NUM_4
<A>	length	LDATA_ADN34
<A>	linear_data	DATA_EMPTY_ADN34
<B>	length	LDATA_ADN38
<B>	linear_data	DATA_EMPTY_ADN38
<C>	length	LDATA_ADN54
<C>	linear_data	DATA_EMPTY_ADN54
<D>	length	LDATA_ADN94
<D>	linear_data	DATA_EMPTY_ADN94

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	NUM_4
<A>	length	LDATA_ADN34
<B>	length	LDATA_ADN38
<C>	length	LDATA_ADN54
<D>	length	LDATA_ADN94

- SIM_READ_RECORD_CNF
	datafield	SIM_ADN
	cause	SIM_NO_ERROR
	record	NUM_4
	max_record	NUM_4
<A>	length	LDATA_ADN34
<A>	linear_data	DATA_EMPTY_ADN34
<B>	length	LDATA_ADN38
<B>	linear_data	DATA_EMPTY_ADN38
<C>	length	LDATA_ADN54
<C>	linear_data	DATA_EMPTY_ADN54
<D>	length	LDATA_ADN94
<D>	linear_data	DATA_EMPTY_ADN94

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_BDN
	record	NUM_1
	length	LDATE_EMPTY

- SIM_READ_RECORD_CNF
	datafield	SIM_BDN
	cause	SIM_NO_ERROR
	record	NUM_1
	max_record	NUM_1
	length	LDATA_BDN
	linear_data	DATA_BDN

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_SDN
	record	NUM_1
	length	LDATE_EMPTY

- SIM_READ_RECORD_CNF
	datafield	SIM_SDN
	cause	SIM_NO_ERROR
	record	NUM_1
	max_record	NUM_1
	length	LDATA_SDN
	linear_data	DATA_SDN

History:	11.11.99	DAK	Initial

### ACIPHB003: use verbose <err> values

Description:

Variants:

<A>...<D>

Preamble:

<A> ACIPHB002A

<B> ACIPHB002B

<C> ACIPHB002C

<D> ACIPHB002D

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CMEE=2)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CMEE_S
	cmd_seq	C_CMEE_S2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	11.11.99	DAK	Initial

## Select phonebook memory storrage "+CPBS" (ACIPHB011 – ACIPHB20)

### ACIPHB011: getting list of supported memory storages

Description:

Select phonebook memory storrage, getting list of supported memory storages

Preamble:

ACIPHB003A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS=?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBS: (...))	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_T
	cmd_seq	C_CPBS_T

- ACI_CMD_IND
	cmd_len	LM_CPBS_T
	cmd_seq	M_CPBS_T

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	27.01.2000	DAK	Initial

### ACIPHB012: setting PHB memory storage

Description:

Select phonebook memory storrage, setting storrage

Preamble:

ACIPHB003A

Variants:

<A>...<I>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS=...)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
<A>	cmd_seq	C_CPBS_EN
<B>	cmd_seq	C_CPBS_BD
<C>	cmd_seq	C_CPBS_FD
<D>	cmd_seq	C_CPBS_LD
<E>	cmd_seq	C_CPBS_LR
<F>	cmd_seq	C_CPBS_AD
<G>	cmd_seq	C_CPBS_SD
<H>	cmd_seq	C_CPBS_LM
<I>	cmd_seq	C_CPBS_AF

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	27.01.2000	DAK	Initial

### ACIPHB013: reading memory storage info

Description:

Select phonebook memory storrage, readin memory storrage info

Variants:

<A>...<F>

Preamble:

<A>ACIPHB012A
<B>ACIPHB012D

<C>ACIPHB012E

<D>ACIPHB012F

<E>ACIPHB012H

<F>ACIPHB012I

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBS: ...)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_Q
	cmd_seq	C_CPBS_Q

- ACI_CMD_IND
<A>	cmd_len	LM_CPBS_EN
<A>	cmd_seq	M_CPBS_EN
<B>	cmd_len	LM_CPBS_LD
<B>	cmd_seq	M_CPBS_LD
<C>	cmd_len	LM_CPBS_LR
<C>	cmd_seq	M_CPBS_LR
<D>	cmd_len	LM_CPBS_AD
<D>	cmd_seq	M_CPBS_AD
<E>	cmd_len	LM_CPBS_LM
<E>	cmd_seq	M_CPBS_LM
<F>	cmd_len	LM_CPBS_AF
<F>	cmd_seq	M_CPBS_AF

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	27.01.2000	DAK	Initial

### ACIPHB014: performe some illegal commands

Description:

Read phonebook entries, performe some illegal commands

Preamble:

ACIPHB003A

Variants:

<A>...<C>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS*)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: CME ERROR)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
<A>	cmd_seq	C_CPBS_SM
<B>	cmd_seq	C_CPBS_AA
<C>	cmd_seq	C_CPBS_XX

- ACI_CMD_IND
	cmd_len	LM_CME_ERR_INV_OPP
	cmd_seq	M_CME_ERR_INV_OPP

History:	09.02.2000	DAK	Initial

## read phonebook entries  "+CPBR" (ACIPHB021 – ACIPHB30)

### ACIPHB021: getting supported location ranges of phb storage (ADN_Alpha_length=20)

Description:

Select phonebook memory storrage, getting supported location ranges of phb storage

AND entries length 34

Variants:

<A>...<F>

Preamble:

ACIPHB003A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS=..)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBR=? (...))	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBR: (...))	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBR_T
	cmd_seq	C_CPBR_T

- ACI_CMD_IND
<A>	cmd_len	LM_CPBR_T_EN1
<A>	cmd_seq	M_CPBR_T_EN1
<B>	cmd_len	LM_CPBR_T_LD1
<B>	cmd_seq	M_CPBR_T_LD1
<C>	cmd_len	LM_CPBR_T_LR1
<C>	cmd_seq	M_CPBR_T_LR1
<D>	cmd_len	LM_CPBR_T_AD1
<D>	cmd_seq	M_CPBR_T_AD1
<E>	cmd_len	LM_CPBR_T_LM1
<E>	cmd_seq	M_CPBR_T_LM1
<F>	cmd_len	LM_CPBR_T_AF1
<F>	cmd_seq	M_CPBR_T_AF1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
<A>	cmd_seq	C_CPBS_EN
<B>	cmd_seq	C_CPBS_LD
<C>	cmd_seq	C_CPBS_LR
<D>	cmd_seq	C_CPBS_AD
<E>	cmd_seq	C_CPBS_LM
<F>	cmd_seq	C_CPBS_AF

History:	27.01.2000	DAK	Initial
	18.02.2000	DAK	changes for bigger ADN_length

### ACIPHB022: getting supported location ranges of phb storage (ADN_Data_length=24)

Description:

Select phonebook memory storrage, getting supported location ranges of phb storage

AND entries length 38

Variants:

<A>...<F>

Preamble:

ACIPHB003B

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS=..)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBR=? (...))	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBR: (...))	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBR_T
	cmd_seq	C_CPBR_T

- ACI_CMD_IND
<A>	cmd_len	LM_CPBR_T_EN1
<A>	cmd_seq	M_CPBR_T_EN1
<B>	cmd_len	LM_CPBR_T_LD1
<B>	cmd_seq	M_CPBR_T_LD1
<C>	cmd_len	LM_CPBR_T_LR1
<C>	cmd_seq	M_CPBR_T_LR1
<D>	cmd_len	LM_CPBR_T_AD2
<D>	cmd_seq	M_CPBR_T_AD2
<E>	cmd_len	LM_CPBR_T_LM1
<E>	cmd_seq	M_CPBR_T_LM1
<F>	cmd_len	LM_CPBR_T_AF2
<F>	cmd_seq	M_CPBR_T_AF2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
<A>	cmd_seq	C_CPBS_EN
<B>	cmd_seq	C_CPBS_LD
<C>	cmd_seq	C_CPBS_LR
<D>	cmd_seq	C_CPBS_AD
<E>	cmd_seq	C_CPBS_LM
<F>	cmd_seq	C_CPBS_AF

History:	18.02.2000	DAK	Initial

### ACIPHB023: getting supported location ranges of phb storage (ADN_Alpha_length=40)

Description:

Select phonebook memory storrage, getting supported location ranges of phb storage

AND entries length 54

Variants:

<A>...<F>

Preamble:

ACIPHB003C

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS=..)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBR=? (...))	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBR: (...))	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBR_T
	cmd_seq	C_CPBR_T

- ACI_CMD_IND
<A>	cmd_len	LM_CPBR_T_EN1
<A>	cmd_seq	M_CPBR_T_EN1
<B>	cmd_len	LM_CPBR_T_LD1
<B>	cmd_seq	M_CPBR_T_LD1
<C>	cmd_len	LM_CPBR_T_LR1
<C>	cmd_seq	M_CPBR_T_LR1
<D>	cmd_len	LM_CPBR_T_AD3
<D>	cmd_seq	M_CPBR_T_AD3
<E>	cmd_len	LM_CPBR_T_LM1
<E>	cmd_seq	M_CPBR_T_LM1
<F>	cmd_len	LM_CPBR_T_AF3
<F>	cmd_seq	M_CPBR_T_AF3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
	cmd_seq	C_CPBS_AD

History:	18.02.2000	DAK	Initial

### ACIPHB024: getting supported location ranges of phb storage (ADN_Alpha_length=80)

Description:

Select phonebook memory storrage, getting supported location ranges of phb storage

AND entries length 94

Variants:

<A>...<F>

Preamble:

ACIPHB003D

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS=..)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBR=? (...))	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBR: (...))	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
<A>	cmd_seq	C_CPBS_EN
<B>	cmd_seq	C_CPBS_LD
<C>	cmd_seq	C_CPBS_LR
<D>	cmd_seq	C_CPBS_AD
<E>	cmd_seq	C_CPBS_LM
<F>	cmd_seq	C_CPBS_AF

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBR_T
	cmd_seq	C_CPBR_T

- ACI_CMD_IND
<A>	cmd_len	LM_CPBR_T_EN1
<A>	cmd_seq	M_CPBR_T_EN1
<B>	cmd_len	LM_CPBR_T_LD1
<B>	cmd_seq	M_CPBR_T_LD1
<C>	cmd_len	LM_CPBR_T_LR1
<C>	cmd_seq	M_CPBR_T_LR1
<D>	cmd_len	LM_CPBR_T_AD4
<D>	cmd_seq	M_CPBR_T_AD4
<E>	cmd_len	LM_CPBR_T_LM1
<E>	cmd_seq	M_CPBR_T_LM1
<F>	cmd_len	LM_CPBR_T_AF4
<F>	cmd_seq	M_CPBR_T_AF4

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	18.02.2000	DAK	Initial

### ACIPHB025: reading ad entries (ADN_Alpha_length = 20)

Description:

Select phonebook memory storrage, getting supported location ranges of phb storage

Variants:

<A>...<B>

Preamble:

ACIPHB003A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS=..)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBR=<n>)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBR: (...))	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBR_S
<A>	cmd_seq	C_CPBR_S0
<B>	cmd_seq	C_CPBR_S1

- ACI_CMD_IND
<A>	cmd_len	LM_CPBR_AD1
<A>	cmd_seq	M_CPBR_AD1
<B>	cmd_len	LM_CPBR_AD2
<B>	cmd_seq	M_CPBR_AD2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
	cmd_seq	C_CPBS_AD

History:	08.02.2000	DAK	Initial

### ACIPHB026: reading ad entries (ADN_Alpha_length > 20)

Description:

Select phonebook memory storrage, getting supported location ranges of phb storage

Variants:

<A>...<F>

Preamble:

<A> ACIPHB003B
	<B> ACIPHB003B

<C> ACIPHB003C
	<D> ACIPHB003C
	<E> ACIPHB003D

<F> ACIPHB003D

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS=..)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBR=<n>)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBR: (...))	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
	cmd_seq	C_CPBS_AD

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBR_S
<A>	cmd_seq	C_CPBR_S0
<B>	cmd_seq	C_CPBR_S1
<C>	cmd_seq	C_CPBR_S0
<D>	cmd_seq	C_CPBR_S1
<E>	cmd_seq	C_CPBR_S0
<F>	cmd_seq	C_CPBR_S1

- ACI_CMD_IND
	cmd_len	LM_CPBR_AD10
<A>	cmd_seq	M_CPBR_AD10
<B>	cmd_seq	M_CPBR_AD11
<C>	cmd_seq	M_CPBR_AD10
<D>	cmd_seq	M_CPBR_AD11
<E>	cmd_seq	M_CPBR_AD10
<F>	cmd_seq	M_CPBR_AD11

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	18.02.2000	DAK	Initial

### ACIPHB027: performe some illegal commands

Description:

Select phonebook memory storrage, performe some illegal commands

Preamble:

ACIPHB003A

Variants:

<A>...<C>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS="AD")	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBR=?)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBR: ...)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(6)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBR*)	|		|

*	==========================>	*		|

(7)	|	ACI_CMD_IND	|		|

|	(cmd: CME ERROR)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBR_T
	cmd_seq	C_CPBR_T

- ACI_CMD_IND
	cmd_len	NOT_USED
	cmd_seq	NOT_USED

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CPBR_S
<A>	cmd_seq	C_CPBR_S2
<B>	cmd_len	LC_CPBR_S
<B>	cmd_seq	C_CPBR_S7
<C>	cmd_len	LC_CPBR_Q
<C>	cmd_seq	C_CPBR_Q

- ACI_CMD_IND
<A>	cmd_len	LM_OK
<A>	cmd_seq	M_OK
<B>	cmd_len	LM_CME_ERR_INV_IDX
<B>	cmd_seq	M_CME_ERR_INV_IDX
<C>	cmd_len	LM_CME_ERR_INV_OPP
<C>	cmd_seq	M_CME_ERR_INV_OPP

ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
	cmd_seq	C_CPBS_AD

History:	09.02.2000	DAK	Initial

## find phonebook entries  "+CPBF" (ACIPHB031 – ACIPHB40)

### ACIPHB031: getting list of supported modes

Description:

find phb entries, getting list of supported modes

Preamble:

ACIPHB003A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBF=?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBF: ...)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBF_T
	cmd_seq	C_CPBF_T

- ACI_CMD_IND
	cmd_len	LM_CPBF_T
	cmd_seq	M_CPBF_T

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	09.02.2000	DAK	Initial

### ACIPHB032: searching for ADN entities starting with "H"

Description:

find phb entries, searching for ADN entities starting with "H", 2 hints expexted

Preamble:

ACIPHB003A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS="AD")	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBR=1,4)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBR: 1,...)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBF: 2,...)	|		|

*	<==========================	*		|

(6)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBF="H")	|		|

*	==========================>	*		|

(8)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBF: ...)	|		|

*	<==========================	*		|

(9)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBF: ...)	|		|

*	<==========================	*		|

(10)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBR_ALL
	cmd_seq	C_CPBR_ALL

- ACI_CMD_IND
	cmd_len	NOT_USED
	cmd_seq	NOT_USED

- ACI_CMD_IND
	cmd_len	NOT_USED
	cmd_seq	NOT_USED

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBF_S0
	cmd_seq	C_CPBF_S0

- ACI_CMD_IND
	cmd_len	LM_CPBF_S1
	cmd_seq	M_CPBF_S1

- ACI_CMD_IND
	cmd_len	LM_CPBF_S0
	cmd_seq	M_CPBF_S0

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History: 24.03.2003	RM	Initial

History:	22.02.2000	DAK	Initial

### ACIPHB033: searching for several ADN entities

Description:

find phb entries, searching for several ADN entities "H", 1 hint expexted

Variants:
	<A>...<B>

Preamble:

ACIPHB003A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS="AD")	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBF="..")	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBF: ...)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
	cmd_seq	C_CPBS_AD

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CPBF_S1
<A>	cmd_seq	C_CPBF_S1
<B>	cmd_len	LC_CPBF_S2
<B>	cmd_seq	C_CPBF_S2

- ACI_CMD_IND
<A>	cmd_len	LM_CPBF_S1
<A>	cmd_seq	M_CPBF_S1
<B>	cmd_len	LM_CPBF_S0
<B>	cmd_seq	M_CPBF_S0

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	22.02.2000	DAK	Initial

### ACIPHB034: performe some "illegal" commands (query-command, searchings without hints)

Description:

find phb entries, performe some "illegal" commands (query-command, searchings without hints)

Variants:
	<A>...<E>

Preamble:

ACIPHB003A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS="AD")	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBF..)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
	cmd_seq	C_CPBS_AD

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CPBF_Q
<A>	cmd_seq	C_CPBF_Q
<B>	cmd_len	LC_CPBF_S3
<B>	cmd_seq	C_CPBF_S3
<C>	cmd_len	LC_CPBF_S4
<C>	cmd_seq	C_CPBF_S4
<D>	cmd_len	LC_CPBF_S5
<D>	cmd_seq	C_CPBF_S5
<E>	cmd_len	LC_CPBF_S6
<E>	cmd_seq	C_CPBF_S6

- ACI_CMD_IND
<A>	cmd_len	LM_CME_ERR_INV_OPP
<A>	cmd_seq	M_CME_ERR_INV_OPP
<B>	cmd_len	LM_CME_ERR_NOT_FOUND
<B>	cmd_seq	M_CME_ERR_NOT_FOUND
<C>	cmd_len	LM_CME_ERR_NOT_FOUND
<C>	cmd_seq	M_CME_ERR_NOT_FOUND
<D>	cmd_len	LM_CME_ERR_NOT_FOUND
<D>	cmd_seq	M_CME_ERR_NOT_FOUND
<E>	cmd_len	LM_CME_ERR_INV_OPP
<E>	cmd_seq	M_CME_ERR_INV_OPP

History:	22.02.2000	DAK	Initial

## write phonebook entries  "+CPBW" (ACIPHB041 – ACIPHB50)

### ACIPHB041: getting supported location ranges of phb storage

Description:

Select phonebook memory storrage, getting supported location ranges of phb storage

Preamble:

ACIPHB003A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CPBS="..")	|		|
	*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CPBW="")	|		|
	*	==========================>	*		|

(4)	|		|	SIM_UPDATE_RECORD_REQ	|
	|		*	==========================>	*

(5)	|		|	SIM_UPDATE_RECORD_CNF	|
	|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
	cmd_seq	C_CPBS_AD

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBW_T
	cmd_seq	C_CPBW_T

- SIM_UPDATE_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	NUM_3
	length	NOT_USED
	linear_data	NOT_USED

- SIM_UPDATE_RECORD_CNF
	datafield	SIM_ADN
	record	NUM_3
	cause	SIM_NO_ERROR

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	27.01.2000	DAK	Initial

### ACIPHB042: write and overwrite of entries

Description:

Select phonebook memory storrage, write three entries, overwrite the second entry

Preamble:

ACIPHB001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CFUN=1)	|		|
	*	==========================>	*		|

(2)	|		|	SIM_ACTIVATE_REQ	|
	|		*	==========================>	*

(3)	|		|	SIM_ACTIVATE_CNF	|
	|		*	<==========================	*

(4)	|		|	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*

(5)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(6)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(7)	|	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

(8)	|		|	MNSMS_REPORT_IND	|
	|		*	<==========================	*

(9)	|		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

(10)	|		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

(11)	|		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

(12)	|		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

(13)	|		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

(14)	|		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

(15)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CPBS="..")	|		|
	*	==========================>	*		|

(16)	|	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

(17)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CBPW="P")	|		|
	*	==========================>	*		|

(18)	|		|	SIM_UPDATE_RECORD_REQ	|
	|		*	==========================>	*

(19)	|		|	SIM_UPDATE_RECORD_CNF	|
	|		*	<==========================	*

(20)	|	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
			cmd_src	CMD_SRC_EXT
			cmd_len	LC_CFUN_S
			cmd_seq	C_CFUN_S

- SIM_ACTIVATE_REQ
			proc	SIM_INITIALISATION
			mmi_pro_file	NOT_USED
			stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
			cause	SIM_NO_ERROR
			pin_cnt	NUM_3
			puk_cnt	NUM_3
			pin2_cnt	NUM_3
			puk2_cnt	NUM_3
			ec_code	EC_CODES
			pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
			func	SIM_ADN_BDN_ENABLED
			sim_serv	SIM_SERV_ADN_BDN
			imsi_field	NOT_USED
			pref_plmn	NOT_USED
			phase	PHASE_2_SIM
			access_acm	ACCESS_ALWAYS
			access_acmmax	ACCESS_ALWAYS
			access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
			source	SRC_MMI
			offset	NUM_0
			datafield	SIM_ECC
			length	NOT_PRESENT_8BIT
			max_length	NUM_0

- SIM_READ_CNF
			datafield	SIM_ECC
			cause	SIM_NO_ERROR
			length	NUM_12
			trans_data	A_ECC_FIELD

- ACI_CMD_IND
			cmd_len	LM_OK
			cmd_seq	M_OK

- MNSMS_REPORT_IND
			state	SMS_STATE_READY

- SIM_READ_RECORD_REQ
			source	SRC_MMI
			datafield	SIM_ADN
			record	NUM_1
			length	MAX_DATAS

- SIM_READ_RECORD_CNF
			datafield	SIM_ADN
			cause	SIM_NO_ERROR
			record	NUM_1
			max_record	NUM_3
			length	LDATA_ADNP1
			linear_data	DATA_ADNP1

- SIM_READ_RECORD_REQ
			source	SRC_MMI
			datafield	SIM_ADN
			record	NUM_2
			length	LDATA_ADNP

- SIM_READ_RECORD_CNF
			datafield	SIM_ADN
			cause	SIM_NO_ERROR
			record	NUM_2
			max_record	NUM_3
			length	LDATA_ADNP
			linear_data	DATA_ADNP

- SIM_READ_RECORD_REQ
			source	SRC_MMI
			datafield	SIM_ADN
			record	NUM_3
			length	LDATA_ADNP1

- SIM_READ_RECORD_CNF
			datafield	SIM_ADN
			cause	SIM_NO_ERROR
			record	NUM_3
			max_record	NUM_3
			length	LDATA_ADNP1
			linear_data	DATA_ADNP3

- ACI_CMD_REQ
			cmd_src	CMD_SRC_EXT
			cmd_len	LC_CPBS_S
			cmd_seq	C_CPBS_AD

- ACI_CMD_IND
			cmd_len	LM_OK
			cmd_seq	M_OK

- ACI_CMD_REQ
			cmd_src	CMD_SRC_EXT
			cmd_len	LC_CPBW_P
			cmd_seq	C_CPBW_P

- SIM_UPDATE_RECORD_REQ
			source	SRC_MMI
			datafield	SIM_ADN
			record	NUM_2
			length	NOT_USED
			linear_data	NOT_USED

- SIM_UPDATE_RECORD_CNF
			datafield	SIM_ADN
			record	NUM_2
			cause	SIM_NO_ERROR

- ACI_CMD_IND
			cmd_len	LM_OK
			cmd_seq	M_OK

History:	17.03.2003	RM	Initial

## test ADN  (ACIPHB045 – ACIPHB47)

### ACIPHB045: power on, on SIM card could be max phonebook entries 255

Description:

Preamble:

ACIPHB001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CFUN=1)	|		|
	*	==========================>	*		|

(2)	|		|	SIM_ACTIVATE_REQ	|
	|		*	==========================>	*

(3)	|		|	SIM_ACTIVATE_CNF	|
	|		*	<==========================	*

(4)	|		|	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*

(5)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(6)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(7)	|	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

(8)	|		|	MNSMS_REPORT_IND	|
	|		*	<==========================	*

(9)	|		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

(10)	|		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

TIMEOUT(10000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
			cmd_src	CMD_SRC_EXT
			cmd_len	LC_CFUN_S
			cmd_seq	C_CFUN_S

- SIM_ACTIVATE_REQ
			proc	SIM_INITIALISATION
			mmi_pro_file	NOT_USED
			stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
			cause	SIM_NO_ERROR
			pin_cnt	NUM_3
			puk_cnt	NUM_3
			pin2_cnt	NUM_3
			puk2_cnt	NUM_3
			ec_code	EC_CODES
			pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
			func	SIM_ADN_BDN_ENABLED
			sim_serv	SIM_SERV_ADN_BDN
			imsi_field	NOT_USED
			pref_plmn	NOT_USED
			phase	PHASE_2_SIM
			access_acm	ACCESS_ALWAYS
			access_acmmax	ACCESS_ALWAYS
			access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
			source	SRC_MMI
			offset	NUM_0
			datafield	SIM_ECC
			length	NOT_PRESENT_8BIT
			max_length	NUM_0

- SIM_READ_CNF
			datafield	SIM_ECC
			cause	SIM_NO_ERROR
			length	NUM_12
			trans_data	A_ECC_FIELD

- ACI_CMD_IND
			cmd_len	LM_OK
			cmd_seq	M_OK

- MNSMS_REPORT_IND
			state	SMS_STATE_READY

- SIM_READ_RECORD_REQ
			source	SRC_MMI
			datafield	SIM_ADN
			record	NUM_1
			length	MAX_DATAS

- SIM_READ_RECORD_CNF
			datafield	SIM_ADN
			cause	SIM_NO_ERROR
			record	NUM_1
			max_record	NUM_FF
			length	LDATA_ADNP1
			linear_data	DATA_ADNP1

History:	21.03.2003	RM	Initial

### ACIPHB046: ADN  phonebook entry call via atD

Description:

Variants:

<A>...<B>

Preamble:

ACIPHB045

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS="..")	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CBPW="")	|		|
	*	==========================>	*		|

(4)	|		|	SIM_UPDATE_RECORD_REQ	|
	|		*	==========================>	*

(5)	|		|	SIM_UPDATE_RECORD_CNF	|
	|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

(7)	|	ACI_CMD_REQ	|		|

|	(cmd: +CLIR=,+COLP=)	|		|

*	==========================>	*		|

(8)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(9)	|	ACI_CMD_REQ	|		|

|	(cmd: atD)	|		|

*	==========================>	*		|

(10)	|		|	MNCC_SETUP_REQ	|
	|		*	==========================>	*

(11)	|		|	MNCC_CALL_PROCEED_IND	|
	|		*	<==========================	*

(12)	|		|	MNCC_ALERT_IND	|
	|		*	<==========================	*

(13)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

(14)	|		|	SIM_SYNC_REQ	|
	|		*	==========================>	*

(15)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

(16)	|	ACI_CMD_IND	|		|
	|	(cmd: +COPL: ...)	|		|
	*	<==========================	*		|

(17)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
		cmd_src	CMD_SRC_EXT
		cmd_len	LC_CPBS_S
		cmd_seq	C_CPBS_AD

- ACI_CMD_IND
		cmd_len	LM_OK
		cmd_seq	M_OK

- ACI_CMD_REQ
		cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CPBW_AD1
<B>	cmd_len	LC_CPBW_AD10
<A>	cmd_seq	C_CPBW_AD1
<B>	cmd_seq	C_CPBW_AD10

- SIM_UPDATE_RECORD_REQ
		source	SRC_MMI
		datafield	SIM_ADN
<A>	record	NUM_1
<B>	record	NUM_10
		length	NOT_USED
		linear_data	NOT_USED

- SIM_UPDATE_RECORD_CNF
		datafield	SIM_ADN
<A>	record	NUM_1
<B>	record	NUM_10
		cause	SIM_NO_ERROR

- ACI_CMD_IND
		cmd_len	LM_OK
		cmd_seq	M_OK

- ACI_CMD_REQ
		cmd_src	CMD_SRC_EXT
		cmd_len	LC_CLIR_COLP_S
		cmd_seq	C_CLIR_COLP_S

- ACI_CMD_IND
		cmd_len	LM_OK
		cmd_seq	M_OK

- ACI_CMD_REQ
		cmd_src	CMD_SRC_EXT
<A>	cmd_len	LatD_ATD_AD1
<B>	cmd_len	LatD_ATD_AD10
<A>	cmd_seq	atD_ATD_AD1
<B>	cmd_seq	atD_ATD_AD10

- MNCC_SETUP_REQ
		ti	NUM_0
		prio	PRIO_NORM_CALL
		ri	RI_NOT_PRES
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY0
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	CLR_SUP
		fac_inf	NOT_USED

- MNCC_CALL_PROCEED_IND
		ti	NUM_0
		progress_desc	NOT_SPEC
		ri	RI_NOT_PRES
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE

- MNCC_ALERT_IND
		ti	NUM_0
		progress_desc	NOT_SPEC

- MNCC_SETUP_CNF
		ti	NUM_0
		cause	MNCC_CAUSE_SUCCESS
		progress_desc	NOT_SPEC
		connected_number	CONNECTED_NUMBER0
		connected_number_sub	NOT_USED

- SIM_SYNC_REQ
		synccs	0x01

- SIM_SYNC_CNF
		cause	NOT_USED

- ACI_CMD_IND
		cmd_len	LM_D0
		cmd_seq	M_D0

- ACI_CMD_IND
		cmd_len	LM_OK
		cmd_seq	M_OK

ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
	cmd_seq	C_CPBS_LD

## test BDN  "+" (ACIPHB048 – ACIPHB50)

### ACIPHB048:

Description:

Preamble:

ACIPHB003A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS="..")	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: atD)	|		|

*	==========================>	*		|

(5)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
	cmd_seq	C_CPBS_BD

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LatD_forb
	cmd_seq	atD_forb

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	29.05.2000	CLB	Initial

## test FDN

### ACIPHB051: initialize phonebook

Description:

initialize phonebook

Preamble:

ACIPHB001

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CFUN=1)	|		|
	*	==========================>	*		|

- |		|	SIM_ACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	SIM_ACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*

- |		|	SIM_READ_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_CNF	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

- |		|	MNSMS_REPORT_IND	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CFUN_S
	cmd_seq	C_CFUN_S

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	NOT_USED
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_3
	pin2_cnt	NUM_3
	puk2_cnt	NUM_3
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_FDN_ENABLED
	sim_serv	SIM_SERV_FDN
	imsi_field	NOT_USED
	pref_plmn	NOT_USED
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNSMS_REPORT_IND
	state	SMS_STATE_READY

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_FDN
	record	NUM_1
	length	MAX_DATAS

- SIM_READ_RECORD_CNF
	datafield	SIM_FDN
	cause	SIM_NO_ERROR
	record	NUM_1
	max_record	NUM_4
	length	LDATA_FDN
	linear_data	DATA_FDN_1

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_FDN
	record	NUM_2
	length	LDATA_FDN

- SIM_READ_RECORD_CNF
	datafield	SIM_FDN
	cause	SIM_NO_ERROR
	record	NUM_2
	max_record	NUM_4
	length	LDATA_FDN
	linear_data	DATA_FDN_2

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_FDN
	record	NUM_3
	length	LDATA_FDN

- SIM_READ_RECORD_CNF
	datafield	SIM_FDN
	cause	SIM_NO_ERROR
	record	NUM_3
	max_record	NUM_4
	length	LDATA_FDN
	linear_data	DATA_EMPTY_FDN

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_FDN
	record	NUM_4
	length	LDATA_FDN

- SIM_READ_RECORD_CNF
	datafield	SIM_FDN
	cause	SIM_NO_ERROR
	record	NUM_4
	max_record	NUM_4
	length	LDATA_FDN
	linear_data	DATA_EMPTY_FDN

- History:	11.11.99	DAK	Initial

### ACIPHB052: use verbose <err> values

Description:

Preamble:

ACIPHB051

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CMEE=2)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CMEE_S
	cmd_seq	C_CMEE_S2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	11.11.99	DAK	Initial

### ACIPHB053: setting PHB memory storage

Description:

Select phonebook memory storrage, setting storrage

Preamble:

ACIPHB052

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS=...)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
	cmd_seq	C_CPBS_FD

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	27.01.2000	DAK	Initial

### ACIPHB054: reading memory storage info

Description:

Select phonebook memory storrage, readin memory storrage info

Preamble:

ACIPHB053

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBS: ...)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_Q
	cmd_seq	C_CPBS_Q

- ACI_CMD_IND
	cmd_len	LM_CPBS_FD
	cmd_seq	M_CPBS_FD

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	27.01.2000	DAK	Initial

### ACIPHB055: getting supported location ranges of phb storage

Description:

Select a FDN-Phonebook memory storrage and write in it

Preamble:

ACIPHB054

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CPBS="..")	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: OK )	|		|
	*	<==========================	*		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CPBS?)	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: "+CPBS: "FD",...")	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: OK )	|		|
	*	<==========================	*		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CPBW="")	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: PIN2 required)	|		|
	*	<==========================	*		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CPIN=1234)	|		|
	*	==========================>	*		|

- |		|	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*

- |		|	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CPBW="")	|		|
	*	==========================>	*		|

- |		|	SIM_UPDATE_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_UPDATE_RECORD_CNF	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
			cmd_src	CMD_SRC_EXT
			cmd_len	LC_CPBS_S
			cmd_seq	C_CPBS_FD

- ACI_CMD_IND
			cmd_len	LM_OK
			cmd_seq	M_OK

- ACI_CMD_REQ
			cmd_src	CMD_SRC_EXT
				cmd_len	LC_CPBS_Q
			cmd_seq	C_CPBS_Q

- ACI_CMD_IND
			cmd_len	LM_CPBS_FD
			cmd_seq	M_CPBS_FD

- ACI_CMD_IND
			cmd_len	LM_OK
			cmd_seq	M_OK

- ACI_CMD_REQ
			cmd_src	CMD_SRC_EXT
			cmd_len	LC_CPBW_T
			cmd_seq	C_CPBW_T

- ACI_CMD_IND
			cmd_len	LM_CME_ERR_PIN2_REQ
			cmd_seq	M_CME_ERR_PIN2_REQ

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CPIN_1234
	cmd_seq	C_PLUS_CPIN_1234

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin		A_PIN_1234
	pin_id		PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
			cause	SIM_NO_ERROR
			pin_id	PHASE_2_PIN_2
			pin_cnt	NUM_1
			puk_cnt	NUM_10
			pin2_cnt	NUM_3
			puk2_cnt	NUM_10

- ACI_CMD_IND
			cmd_len	LM_OK
			cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	NUM_ELEMENTS(C_CPBW_T1)
	cmd_seq	C_CPBW_T1

- SIM_UPDATE_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_FDN
	record	NUM_3
	length	LDATA_FDN
	linear_data	NOT_USED

- SIM_UPDATE_RECORD_CNF
			datafield	SIM_FDN
			record	NUM_3
			cause	SIM_NO_ERROR

- ACI_CMD_IND
			cmd_len	LM_OK
			cmd_seq	M_OK

History:	07.11.2000	ENZ	Initial

### ACIPHB056: select and reading an entries (FDN_Alpha_length = 20)

Description:

Select phonebook memory storrage, getting supported location ranges of phb storage

Variants:
	<A>...<C>

Preamble:

ACIPHB055

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS=..)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBR=<n>)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(cmd: +CPBR: (...))	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(cmd: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
	cmd_seq	C_CPBS_FD

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBR_S
<A>	cmd_seq	C_CPBR_S0
<B>	cmd_seq	C_CPBR_S1
<C>	cmd_seq	C_CPBR_S2

- ACI_CMD_IND
<A>	cmd_len	LM_CPBR_FD1
<A>	cmd_seq	M_CPBR_FD1
<B>	cmd_len	LM_CPBR_FD2
<B>	cmd_seq	M_CPBR_FD2
<C>	cmd_len	NUM_ELEMENTS(M_CPBR_FD3)
<C>	cmd_seq	M_CPBR_FD3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	07.11.2000	ENZ	Initial

### ACIPHB057: getting supported location ranges of phb storage

Description:

Select a LDN-Phonebook memory storrage and write in it

Preamble:

ACIPHB041

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPBS="LD")	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|
	|	(cmd: OK )	|		|
	*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CPBS?)	|		|
	*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|
	|	(cmd: ACI_CMD_IND )	|		|
	*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|
	|	(cmd: OK )	|		|
	*	<==========================	*		|

(6)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CPBW="")	|		|
	*	==========================>	*		|

(7)	|	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

(8)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CPBS?)	|		|
	*	==========================>	*		|

(9)	|	ACI_CMD_IND	|		|
	|	(cmd: ACI_CMD_IND )	|		|
	*	<==========================	*		|

(10)	|	ACI_CMD_IND	|		|
	|	(cmd: OK )	|		|
	*	<==========================	*		|

(11)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CPBR=<n>)	|		|
	*	==========================>	*		|

(12)	|	ACI_CMD_IND	|		|
	|	(cmd: +CPBR: (...))	|		|
	*	<==========================	*		|

(13)	|	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_Q
	cmd_seq	C_CPBS_Q

- ACI_CMD_IND
	cmd_len	LM_CPBS_LD0
	cmd_seq	M_CPBS_LD0

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBW_S2
	cmd_seq	C_CPBW_S2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_Q
	cmd_seq	C_CPBS_Q

- ACI_CMD_IND
	cmd_len	LM_CPBS_LD1
	cmd_seq	M_CPBS_LD1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBR_S
	cmd_seq	C_CPBR_S0

- ACI_CMD_IND
	cmd_len	LC_CPBR_A2
	cmd_seq	C_CPBR_A2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

initialize phonebook

History:	07.11.2000	ENZ	Initial

## test LDN-entries on SIM (ACIPHB061 – ACIPHB070)

### ACIPHB061: initialize phonebook

Description:

Preamble:

ACIPHB001

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CFUN=1)	|		|
	*	==========================>	*		|

- |		|	SIM_ACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	SIM_ACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*

- |		|	SIM_READ_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_CNF	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

- |		|	MNSMS_REPORT_IND	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

- |		|	SIM_READ_RECORD_REQ	|
	|		*	==========================>	*

- |		|	SIM_READ_RECORD_CNF	|
	|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	NUM_ELEMENTS(C_CFUN_S)
	cmd_seq	C_CFUN_S

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	NOT_USED
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_3
	pin2_cnt	NUM_3
	puk2_cnt	NUM_3
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	SIM_SERV_LND
	imsi_field	NOT_USED
	pref_plmn	NOT_USED
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNSMS_REPORT_IND
	state	SMS_STATE_READY

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	NUM_1
	length	MAX_DATAS

- SIM_READ_RECORD_CNF
	datafield	SIM_ADN
	cause	SIM_NO_ERROR
	record	NUM_1
	max_record	NUM_1
	length	NUM_ELEMENTS(DATA_ADN34_1)
	linear_data	DATA_ADN34_1

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_LND
	record	NUM_1
	length	MAX_DATAS

- SIM_READ_RECORD_CNF
	datafield	SIM_LND
	cause	SIM_NO_ERROR
	record	NUM_1
	max_record	NUM_10
	length	NUM_ELEMENTS(DATA_LND_1)
	linear_data	DATA_LND_1

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_LND
	record	NUM_2
	length	LDATA_LND

- SIM_READ_RECORD_CNF
	datafield	SIM_LND
	cause	SIM_NO_ERROR
	record	NUM_2
	max_record	NUM_10
	length	NUM_ELEMENTS(DATA_LND_2)
	linear_data	DATA_LND_2

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_LND
	record	NUM_3
	length	LDATA_LND

- SIM_READ_RECORD_CNF
	datafield	SIM_LND
	cause	SIM_NO_ERROR
	record	NUM_3
	max_record	NUM_10
	length	NUM_ELEMENTS(DATA_LND_3)
	linear_data	DATA_LND_3

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_LND
	record	NUM_4
	length	LDATA_LND

- SIM_READ_RECORD_CNF
	datafield	SIM_LND
	cause	SIM_NO_ERROR
	record	NUM_4
	max_record	NUM_10
	length	LDATA_LND
	linear_data	DATA_EMPTY_LND

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_LND
	record	NUM_5
	length	LDATA_LND

- SIM_READ_RECORD_CNF
	datafield	SIM_LND
	cause	SIM_NO_ERROR
	record	NUM_5
	max_record	NUM_10
	length	LDATA_LND
	linear_data	DATA_EMPTY_LND

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_LND
	record	NUM_6
	length	LDATA_LND

- SIM_READ_RECORD_CNF
	datafield	SIM_LND
	cause	SIM_NO_ERROR
	record	NUM_6
	max_record	NUM_10
	length	LDATA_LND
	linear_data	DATA_EMPTY_LND

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_LND
	record	NUM_7
	length	LDATA_LND

- SIM_READ_RECORD_CNF
	datafield	SIM_LND
	cause	SIM_NO_ERROR
	record	NUM_7
	max_record	NUM_10
	length	LDATA_LND
	linear_data	DATA_EMPTY_LND

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_LND
	record	NUM_8
	length	LDATA_LND

- SIM_READ_RECORD_CNF
	datafield	SIM_LND
	cause	SIM_NO_ERROR
	record	NUM_8
	max_record	NUM_10
	length	LDATA_LND
	linear_data	DATA_EMPTY_LND

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_LND
	record	NUM_9
	length	LDATA_LND

- SIM_READ_RECORD_CNF
	datafield	SIM_LND
	cause	SIM_NO_ERROR
	record	NUM_9
	max_record	NUM_10
	length	LDATA_LND
	linear_data	DATA_EMPTY_LND

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_LND
	record	NUM_10
	length	LDATA_LND

- SIM_READ_RECORD_CNF
	datafield	SIM_LND
	cause	SIM_NO_ERROR
	record	NUM_10
	max_record	NUM_10
	length	LDATA_LND
	linear_data	DATA_EMPTY_LND

History:	16.12.02	MSC	Initial

### ACIPHB062: setting PHB memory storage

Description:

Select phonebook memory storrage, setting storrage

Preamble:

ACIPHB061

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CPBS=...)	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_S
	cmd_seq	C_CPBS_LD

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	16.12.2002	MSC	Initial

### ACIPHB063: reading memory storage info

Description:

Select phonebook memory storrage, readin memory storrage info

Preamble:

ACIPHB062

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CPBS?)	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: +CPBS: ...)	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_Q
	cmd_seq	C_CPBS_Q

- ACI_CMD_IND
	cmd_len	LM_CPBS_LD3
	cmd_seq	M_CPBS_LD3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	16.12.2002	MSC	Initial

### ACIPHB064: reading existing LD entries (LDN_Alpha_length = 20)

Description:

Read the entries 1 to 3 of the LND Phonebook

Preamble:

ACIPHB062

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CPBR=<n>)	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: +CPBR: (...))	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: +CPBR: (...))	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: +CPBR: (...))	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBR_LD1TO3
	cmd_seq	C_CPBR_LD1TO3

- ACI_CMD_IND
	cmd_len	LM_CPBR_LD1OF3
	cmd_seq	M_CPBR_LD1OF3

- ACI_CMD_IND
	cmd_len	LM_CPBR_LD2OF3
	cmd_seq	M_CPBR_LD2OF3

- ACI_CMD_IND
	cmd_len	LM_CPBR_LD3OF3
	cmd_seq	M_CPBR_LD3OF3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	17.12.2000	MSC	Initial

### ACIPHB065: Dial some number (0174 4444444) ??????

Description:

Force a new entry in the LDN list (=setup a call)

Preamble:

ACIPHB062

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: ATD...;)	|		|
	*	==========================>	*		|

- |		| 	MNCC_SETUP_REQ	|
	|		*	==========================>	*

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|

- |		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	NUM_ELEMENTS(C_D_VOICE)
	cmd_seq	C_D_VOICE

- MNCC_SETUP_REQ
	ti	NUM_0
	prio	PRIO_NORM_CALL
	ri	RI_NOT_PRES
	bcpara	S_BS_VOICE
	bcpara2	S_BS_NOT_PRESENT
	called_party	S_CLD_PARTY
	called_party_sub	S_CLD_PARTY_SUB
	clir_sup	CLR_NOT_PRES
	fac_inf	NOT_USED

- ACI_CMD_IND
	cmd_len	NUM_ELEMENTS(M_OK)
	cmd_seq	M_OK

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

History:	18.12.2002	MSC	Initial

### ACIPHB066: reading memory storage info after ter the call

Description:

Select phonebook memory storrage, readin memory storrage info

Preamble:

ACIPHB065

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CPBS?)	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: +CPBS: ...)	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBS_Q
	cmd_seq	C_CPBS_Q

- ACI_CMD_IND
	cmd_len	LM_CPBS_LD4
	cmd_seq	M_CPBS_LD4

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	17.12.2002	MSC	Initial

### ACIPHB067: reading existing+new LD entries (LDN_Alpha_length = 20)

Description:

Read the entries 1 to 4 of the LND Phonebook

Preamble:

ACIPHB065

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CPBR=<n>)	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: +CPBR: (...))	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: +CPBR: (...))	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: +CPBR: (...))	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: +CPBR: (...))	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(cmd: OK)	|		|
	*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPBR_LD1TO4
	cmd_seq	C_CPBR_LD1TO4

- ACI_CMD_IND
	cmd_len	LM_CPBR_LD1OF4
	cmd_seq	M_CPBR_LD1OF4

- ACI_CMD_IND
	cmd_len	LM_CPBR_LD2OF4
	cmd_seq	M_CPBR_LD2OF4

- ACI_CMD_IND
	cmd_len	LM_CPBR_LD3OF4
	cmd_seq	M_CPBR_LD3OF4

- ACI_CMD_IND
	cmd_len	LM_CPBR_LD4OF4
	cmd_seq	M_CPBR_LD4OF4

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	17.12.2000	MSC	Initial