Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document - Confidential

GSM Protocol Stack

G23

EMI-Electrical Man-Machine Interface

Driver Interface

(Edit via File/Properties/Summary!)

| Document Number: | 8415.003.99.008 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.9(Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1998-Sep-2 |
| Last changed: | 2003-May-14 by XGUTTEFE |
| File Name: | 8415_003.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1998-Sep-2 | LM et al. |  | 0.1 |  | 1 |
| 1998-Sep-9 | LM et al. |  | 0.2 |  | 2 |
| 1998-Nov-3 | LM et al. |  | 0.3 |  | 3 |
| 1999-Mar-9 | LM et al. |  | 0.4 |  | 4 |
| 1999-Mar-18 | MS et al. |  | 0.5 |  | 5 |
| 1999-Sep-6 | LM et al. |  | 0.6 |  | 6 |
| 1999-Nov-16 | FR et al |  | 0.7 |  | 7 |
| 2000-Jan-18 | MP et al. |  | 0.8 |  | 8 |
| 2003-May-13 | XINTEGRA |  | 0.9 |  |  |

Notes:

- Initial version

- Now complies with [C_8415.0026]

- API changed

- Now complies with new 8415.026.99.012 version

- Format/English check

- API changed. Now complies with socket API

- API changed again to fit in test interface layer struct configuration via string; some extra explanations

- Adaption to new GDI

Table of Contents

1.1	References	5

2	Introduction	6

2.1	G23 Test Interface Layer 2	6

2.2	G23 Test Interface Layer 1	7

3	Interface description of the EMI driver	7

3.1	Data Types	7

3.1.1	T_DRV_FUNC – Diver Functions	7

3.1.2	T_DRV_EXPORT – Driver Properties	7

3.1.3	T_DRV_SIGNAL – Driver Signal	8

3.1.4	T_DRV_CB_FUNC – Driver Callback Function	9

3.2	Constants	10

3.2.1	Return Codes	10

3.2.2	Signal Codes	10

3.3	Signals	11

3.4	Functions	12

3.4.1	emi_Init – Driver Initialization	13

3.4.2	emi_Exit – Driver Finalization	14

3.4.5	emi_Read - Read Data from the Driver	17

3.4.6	emi_Write – Write Data to the Driver	18

3.4.7	emi_SetSignal – Setup a Signal	19

3.4.8	emi_ResetSignal – Remove a Signal	20

3.4.9	emi_SetConfig – Set the Driver Configuration	21

3.4.10	emi_GetConfig – Retrieve the Driver Configuration	22

3.4.11	emi_Callback – Callback Function	23

Appendices	24

A.	Acronyms	24

B.	Glossary	24

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 4.14, Individual equipment type requirements and interworking;                                            Special conformance testing functions	TS 101 293 V6.0.0, ETSI, April 1999

[2]	Generic Driver Interface – Functional Specification; Condat 
8415.026.99.012; March 19, 1999

[3]	VSI/PEI – Frame/Body Interfaces - Next Generation; Condat
8415.033.99.100

[4]	Concept for Integration of Drivers into the Test Interface; Condat internal, Nov 99

# Introduction

- G23 is a software package implementing Layers 2 and 3 of the ETSI-defined GSM air interface signaling protocol, and as such represents the part of a GSM mobile station’s protocol software which is both, platform and manufacturer independent. Therefore, G23 can be viewed as a building block providing standardized functionality through generic interfaces for easy integration.

The G23 suite of products consists of the following items:

- Layers 2 and 3 for speech & short message services,

- Layers 2 and 3 for fax & data services,

- Application Control Interface,

- Slim MMI [02.30] and

- Test and integration support tools.

This document describes the functional interface of the G23 Layer 2 test interface driver, EMI. The EMMI functionality was originally defined by GSM in the recommendation 11.10, later moved do 4.14 [1]. This functionality has been extended to fulfill the needs of the G23 Protocol Stack, including higher data rates and larger data frames than the original EMMI.

This driver allows secure data transmission and reception. Layer 1 may be implemented by any driver that fits the test interface driver structure, e.g. USART or SOCKET.

In addition, this implementation of the EMI Layer 2 protocol includes a watchdog functionality. This means that the driver is able to send heartbeats (XONs) to the remote end to indicate the active and ready to receive state. If the remote end does not send any data or heartbeats, the driver indicates the status DRV_SIGTYPE_DISCONNECT.

## G23 Test Interface Layer 2

In Layer 2, frames are used to carry data from higher layers via Layer 1. Special Layer 2 frames (control frames) are used to control the flow and allow a secure transmission of the frames.

The flow control used by the G23 Test Interface Layer 2 is in line with the flow control used by EMMI. A second kind of I frame has been introduced that can carry more data in a single frame.

The following tables show the structure of both I frames.

| Field name | # of octets | Value | Start at octet # |
| --- | --- | --- | --- |
| Start | 1 | Character STX | 1 |
| Length | 1 | Length of data | 2 |
| Data | 0-255 | Content of data | 3 |
| Check | 1 | Error detection | Length + 3 |
| End | 1 | Character ETX | Length + 4 |

Table 1: Original I frame

| Field name | # of octets | Value | Start at octet # |
| --- | --- | --- | --- |
| Start | 1 | Character SI | 1 |
| Length | 2 | Length of data | 2 |
| Data | 0-65535 | Content of data | 4 |
| Check | 1 | Error detection | Length + 4 |
| End | 1 | Character ETX | Length + 5 |

Table 1: Extended I Frame

The EMI Driver accepts both frames in receiving. It sends the original frame, if the data length is lower than 256, else the extended frame.

The retransmission of I frames by the G23 Test Interface Layer 2 is limited to a maximum of 3 retransmissions. If the I frame could not be transmitted it will be discarded and the higher layer is notified about the failed retransmission.

The EMI driver described in this document implements this Layer 2.

## G23 Test Interface Layer 1

Layer 1 is performed by another driver, which is called by EMI. The functional interface of this driver has to be the same in structure as it is for EMI. The design of the test interface driver structure is explained in more detail in [4].

# Interface description of the EMI driver

## Data Types

For types not defined here see [2]

### T_DRV_FUNC – Diver Functions

Definition:	typedef struct

{

void (*drv_Exit)();

USHORT (*drv_Read)();

USHORT (*drv_Write)();

USHORT (*drv_Look)();

USHORT (*drv_Clear)();

USHORT (*drv_Flush)();

USHORT (*drv_SetSignal)();

USHORT (*drv_ResetSignal)();

USHORT (*drv_SetConfig)();

USHORT (*drv_GetConfig)();

void (*drv_Callback)();

} T_DRV_FUNC

Description: The structure of the type T_DRV_FUNC contains the addresses of the driver entry functions.

### T_DRV_EXPORT – Driver Properties

Definition:

typedef struct
{
	char *	Name
	USHORT	Flags
	T_DRV_FUNC	DrvFunc

} T_DRV_EXPORT

Description:

This data type defines the properties exported by the driver.

Name		Name of the driver

Flags		Bit (0): Callback function is called during ISR(1)/not called during ISR(0)

DrvFunc		functions to access the driver

### T_DRV_SIGNAL – Driver Signal

Definition:

typedef struct
{
	USHORT	SignalType
	USHORT	DataLength
	void *	UserData
	USHORT	DrvHandle
} T_DRV_SIGNAL

Description:

This type defines the signal information data used to identify a signal. This data type is used to define and to report a signal. A signal is defined by a process by calling the driver function drv_SetSignal(). An event is signaled by a driver by calling the signal call-back function (see 3.1.4, 3.3, 3.4.1).

### T_DRV_CB_FUNC – Driver Callback Function

Definition:

typedef void (*T_DRV_CB_FUNC) (T_DRV_SIGNAL * Signal) ;

Description:

This type defines a call-back function used to signal driver events, e.g. driver is ready to accept data. The driver calls the signal call-back function when a specific event occurs and the driver has been instructed to signal the event to a specific process.

A process can set or reset event signaling by calling one of the driver functions emi_SetSignal(), emi_ResetSignal(). Event signaling can only be performed when a signal call-back function has been installed at driver initialization.

For more information about the T_DRV_SIGNAL data type, refer to 3.1.3.

## Constants

### Return Codes

Name	Description

DRV_OK	Return value indicating the function completed successfully

DRV_INPROCESS	The requested function is currently being executed

DRV_NOTCONFIGURED	Driver is not configured

DRV_INITFAILURE	Driver initialization failed

DRV_BUFFER_FULL	The internal buffer is exhausted

DRV_INVALID_PARAMS	One or more parameters are out of range or invalid

DRV_NOCONNECT	No connection exists to a peer entity

DRV_SIGFCT_NOTAVAILABLE	Event signaling functionality is not available

DRV_ERROR	Other error

### Signal Codes

Name	Description

DRV_SIGTYPE_READ	Used to specify read event signaling

DRV_SIGTYPE_WRITE	Used to specify write event signaling

DRV_SIGTYPE_CONNECT	Used to specify connection established event signaling

DRV_SIGTYPE_DISCONNECT	Used to specify connection release event signaling

## Signals

Signals are used to asynchronously inform the process using services about selected events. A signal call-back function is passed to the driver at the time of initialization (see “3.4.1 emi_Init – Driver Initialization”). When no call-back is defined, event signaling cannot be performed. A signal can be set using the function emi_SetSignal(), which can be found in Chapter 3.4.7. Event signaling can be disabled by calling the function emi_ResetSignal().

The signalling is actually performed by calling the callback function, which has the following proto type:

void callback( T_DRV_SIGNAL *Signal );

The EMI driver uses only the DrvHandle and SignalType of the Signal, UserData and DataLength are nor used. The following table shows the values for SignalType as used by EMI:

| SignalType | meaning |
| --- | --- |
| DRV_SIGTYPE_READ | The driver has received data which can be read by the emi_Read() function. |
| DRV_SIGTYPE_WRITE | The driver has written data so that the write buffer is ready to take new data (using emi_Write()). |
| DRV_SIGTYPE_CONNECT | EMI detected that the peer entity is working, i.e. there is a connection. Data may be sent. |
| DRV_SIGTYPE_DISCONNECT | EMI detected that the connection to the peer entity is lost. |

## Functions

Name	Description

emi_Init	Initialization of EMI
emi_Exit	Termination of EMI
emi_Open	Open a connection to a peer entity
emi_Close	Close a connection
emi_Write	Send data to the remote end
emi_Read	Read data from the driver
emi_Clear	Clear internal buffers
emi_SetSignal	Define a signal which the driver uses to indicate an event
emi_ResetSignal	Un-define a signal which the driver uses to indicate an event
emi_SetConfig	Set driver configuration 
emi_GetConfig	Get driver configuration
emi_Callback	Callback entry for layer 1 driver

The GDI standard functions emi_Look() and emi_Flush() are not implemented.

### emi_Init – Driver Initialization

Definition:

USHORT emi_Init
(
	USHORT 	DrvHandle, 
	T_DRV_CB_FUNC 	CallbackFunc,
	T_DRV_EXPORT ** 	DrvInfo
) ;

Parameters:

Name	Description

DrvHandle	This is the number, which identifies the driver in the system. The driver has to store it. It is needed, when the callback function is called.

CallbackFunc	The address of the callback function. The events at which to call back are configured using emi_SetSignal(). If this address is equal to NULL the driver will not try to call back.

DrvInfo 	Via this parameter the driver returns the adresses of it's functions. Some function addresses may be set to NULL, which means that these functions are not implemented and cannot be called.

Return values:

Name	Description

DRV_OK	Initialization successful
DRV_INITFAILURE	Initialization failed

Description

This function initializes the driver’s internal data and returns the addresses of it's entry functions. emi_Init() returns DRV_OK in the case of a successful completion.

In case of an initialization failure, which means that the driver cannot be used, the function returns DRV_INITFAILURE.

All signals have to be disabled during initialization and may be enabled later.

The next call to EMI after emi_Init() shall be emi_SetConfig() !

### emi_Exit – Driver Finalization

Definition:

void emi_Exit
(
	void
) ;

Parameters:

Name	Description

-

Return values:

Name	Description

-	-

Description

The function is called when the driver functionality is no longer required. The data exchange terminates immediately regardless of any outstanding data to be sent.

### emi_Open - Open a Connection to the Remote Side

Definition:

USHORT emi_Open
(
	T_HANDLE 	caller
) ;

Parameters:

Name	Description

caller	task handle of the caller

Return values:

Name	Description

DRV_OK	Function successful, connected
DRV_NOTCONFIGURED	The driver is not yet configured
DRV_ERROR	An unspecified error occurred
DRV_NOCONNECT	Not connected (yet)

Description

This function is used to establish a connection to the peer entity. As soon as a peer is connected the CONNECT signal is generated. However, the driver does not wait for the connection.

It returns DRV_OK if it is already connected. If there is no error and the connection is assumed to be established in a while then the driver returns DRV_NOCONNECT.

To get notified about establishing the connection or about the reception of data the appropriate signal has to be set (emi_SetSignal()).

Before calling emi_Open() EMI has to be configured by emi_SetConfig().

### emi_Close – Close Connection

Definition:

void emi_Close
(
	T_HANDLE 	caller
) ;

Parameters:

Name	Description

caller	task handle of the caller

Return values:

Name	Description

-	-

Description

This function is used to close the connection to the peer entity.

In case of a successful completion the connection is shut down and neither emi_Read() nor emi_Write() will be successful. To get notified about the termination of a connection the appropriate signal has to be set (emi_SetSignal()).

### emi_Read - Read Data from the Driver

Definition:

USHORT emi_Read
(
	void *	Buffer,
	USHORT *	Length
) ;

Parameters:

Name	Description

Buffer	This parameter points to the buffer wherein the data is to be copied

Length	On call: number of characters to read. If the function returns DRV_OK, it contains the number of characters read. If the function returns DRV_INPROCESS, it contains 0.

Return values:

Name	Description

DRV_OK	Function successful
DRV_INPROCESS	The driver is currently reading data. The data is incomplete.
DRV_NOTCONFIGURED	The driver is not yet configured
DRV_NOCONNECT	Connection not available

Description

This function is used to read one data frame from the driver. The data is copied into the buffer to where Buffer points. The parameter *Length contains the size of the buffer in characters.

If the driver is not configured, the function returns DRV_ NOTCONFIGURED.

If no connection exists the driver returns DRV_NOCONNECT.

NOTE: When calling the function with a buffer size of 0, the function will return DRV_OK. The size of the buffer needed to store the available data is stored in the parameter *Length. In this case,  Buffer can be set to NULL.

### emi_Write – Write Data to the Driver

Definition:

USHORT emi_Write
(
	void *	Buffer,
	USHORT *	Length
) ;

Parameters:

Name	Description

Buffer	This parameter points to the buffer that is passed to the driver for further processing

Length	On call: number of characters to write. 
If the function returns DRV_OK, it contains the number of characters written. This number is always equal to the requested number.
In all other cases it contains 0.

Return values:

Name	Description

DRV_OK	Function successful, data frame written
DRV_BUFFER_FULL	Not enough space
DRV_NOTCONFIGURED	The driver is not yet configured
DRV_NOCONNECT	Connection not available

Description

This function is used to write one data frame to the driver. The parameter *Length contains the number of characters to write.

In the case of a successful completion, the function returns DRV_OK.

In all other cases the data was not written and the call to emi_Write() should be repeated later.

The emi_Write() function has to call vsi_d_write() – see prototype below - to call the layer 1 drv_Write() function.

USHORT vsi_d_write ( USHORT Caller, USHORT DrvHandle, void *Buffer, USHORT *Length )

The parameter Caller is the handle of the EMI driver passed to emi_Init(). The parameter DrvHandle has to be set to 0 to indicate to the frame that it must call the corresponding lower layer drv_Write() function. Buffer and Length are the parameters passed to emi_Write(). For detailed information about vsi_d_write() refer to [3].

NOTE:	When calling the function with a buffer size of 0, the function will return the number of characters that can be written in the parameter *Length. In this case,  Buffer can be set to NULL, no data will be written.

### emi_SetSignal – Setup a Signal

Definition:

USHORT emi_SetSignal
(
	USHORT	SignalType
) ;

Parameters:

Name	Description

SignalType	Signal type to be set

Return values:

Name	Description

DRV_OK	Function completed successfully
DRV_INVALID_PARAMS	Signal type not supported by EMI
DRV_SIGFCT_NOTAVAILABLE	Event signaling functionality is not available

Description

This function is used to define a single signal that the driver should indicated via the callback function.

To remove a signal, call the function emi_ResetSignal().

If no signal call-back function has been defined at the time of initialization, the driver returns DRV_SIGFCT_NOTAVAILABLE.

### emi_ResetSignal – Remove a Signal

Definition:

USHORT emi_ResetSignal
(
	USHORT	SignalType
) ;

Parameters:

Name	Description

SignalType	Signal type to be reset

Return values:

Name	Description

DRV_OK	Function completed successfully
DRV_INVALID_PARAMS	Signal type not supported by EMI
DRV_SIGFCT_NOTAVAILABLE	Event signaling functionality is not available

Description

This function is used to remove a previously set signal.

If no signal call-back function has been defined at the time of initialization, the driver returns DRV_SIGFCT_NOTAVAILABLE.

### emi_SetConfig – Set the Driver Configuration

Definition:

USHORT emi_SetConfig
(
	char *	Config
) ;

Parameters:

Name	Description

Config	Pointer to driver configuration string

Return values:

Name	Description

DRV_OK	Function successfully completed
DRV_INVALID_PARAMS	Error in configuration parameters

Description

This function is used to configure the driver and must be called after emi_Init() but before any other function.

### emi_GetConfig – Retrieve the Driver Configuration

Definition:

USHORT emi_GetConfig
(
	char *	Buffer
) ;

Parameters:

Name	Description

Buffer	Address to copy the configuration data string to.

Return values:

Name	Description

DRV_OK	Function successfully completed
DRV_INVALID_PARAMS	Driver not configured

Description

This function is used retrieve the configuration of the driver. The configuration data is copied to the Buffer provided by the caller.

If the driver has not been configured DRV_NOTCONFIGURED is returned.

### emi_Callback – Callback Function

Definition:

void emi_Callback
(
	T_DRV_SIGNAL *	Signal
) ;

Parameters:

Name	Description

Signal	Pointer to Signal sent by layer 1 driver

Return values:

Name	Description

-

Description

This function is called by the frame if the lower layer driver calls the vsi_d_callback() function.

If the signal type in the passed signal is DRV_SIGTYPE_READ the emi_Callback() function has to call vsi_d_read() – see prototype below - to read the layer 1 data via its drv_Read() function.

USHORT vsi_d_read ( USHORT Caller, USHORT DrvHandle, void *Buffer, USHORT *Length )

The parameter Caller is the handle of the EMI driver passed to emi_Init(). The parameter DrvHandle has to be set to 0 to indicate to the frame that it must call the corresponding lower layer drv_Read() function. For detailed information about vsi_d_read() refer to [3].

The data read from the lower layer driver has to be analyzed and when a complete message frame has been received and the DRV_SIGTYPE_READ is set then the callback function, which address  was passed to emi_Init(), has to be called with a pointer to a signal of the type T_DRV_SIGNAL (refer to 3.1.3). The signal type has to be set to DRV_SIGTYPE_READ and the DrvHandle is the one passed to emi_Init().

Signal types different from DRV_SIGTYPE_READ like DRV_SIGTYPE_CONNECT and DRV_SIGTYPE_DISCONNECT also have to be forwarded to the upper layer driver - if the corresponding signal type is enabled - by calling the callback function, which address  was passed to emi_Init().

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |