Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document

GSM Protocol Stack

GPF

OSX – Customer Frame Interface

Functional Interface Description

(Edit via File/Properties/Summary!)

| Document Number: | 06-03-10-ISP-0004 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.5 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2000-Oct-25 |
| Last changed: | 2003-Sep-08 by MP |
| File Name: | osx_api.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2000-Oct-25 | MP et al. |  | 0.1 | Being Processed | 1 |
| 2001-May-04 | MP et al. |  | 0.2 | Being Processed | 2 |
| 2001-Sep-07 | MP et al. |  | 0.3 | Submitted | 3 |
| 2003-May-20 | XINTEGRA |  | 0.4 | Draft |  |
| 2003-Sep-08 | MP |  | 0.5 | Submitted | 4 |

Notes:

- Initial version

- Function names changed

- Document number changed

- new API function osx_config()

xSignalHeaderRec modified

Table of Contents

Functional Interface Description	1

1.1	Abbreviations	5

2	Frame/Body Concept	6

3	Customer Frame Interface	7

3.1	Data Types	7

3.1.1	T_VOID_STRUCT	7

3.1.2	T_ENUM_OS_QUEUE	7

3.1.3	xSignalHeaderRec	8

3.2	Constants	8

3.2.1	Return Codes	8

3.3	int_osx Functions	9

3.3.1	int_osx_alloc_prim()	9

3.3.2	int_osx_alloc_mem()	10

3.3.3	int_osx_free_prim()	11

3.3.4	int_osx_free_mem()	12

3.3.5	int_osx_send_prim()	13

3.3.6	int_osx_receive_prim()	14

3.3.7	int_osx_send_sig()	15

3.4	osx Functions	16

3.4.1	_osx_init	16

3.4.2	_osx_config()	17

3.4.3	_osx_open()	18

3.4.4	osx_alloc_prim()	19

3.4.5	osx_alloc_mem()	20

3.4.6	osx_free_prim()	21

3.4.7	osx_free_mem()	22

3.4.8	osx_send_prim()	23

3.4.9	osx_receive_prim()	24

3.4.10	osx_send_sig()	25

Appendices	26

A.	Acronyms	26

B.	Glossary	26

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## Abbreviations

RTOS	Real-time Operating System

VSI	Virtual System Interface

PEI	Protocol Stack Entity Interface

# Frame/Body Concept

The frame body concept has been designed in the context of the G23 Protocol Stack. In the case of the G23 Protocol Stack, a process represents the protocol logic of a protocol stack entity. This architecture separates the process functionality into two logical modules, the process frame and the process body. Common process functionality is located in the process frame. The main process functionality is located in the process body.

### L1 Entity

PS Functionality

pei_

### L23 Entity

PS Functionality

pei_

OSX

Content of this Document

### Frame      vsi_

Virtual OS

Operating System Extensions:

Passive Model

Routing

Trace

Memory Supervision

Timer Configuration

Start up

### Driver
Hardware Abstraction

_init

call back

call back

### OS-Layer			os_

Adaption to RTOS

call back

### RTOS

Nucleus, Win32, pSOS, ...

- Figure 1: Structure of GSM Protocol Stack

The process frame has two interfaces. The Virtual System Interface (VSI) is the frames functional interface to be accessed by the bodies. The Operating System Interface (OS) is also a functional interface and provides the interface to the Real-time Operating System (RTOS). This interface is encapsulated in the "OS Layer" in order to keep the frame itself independent from the underlying RTOS.

In the OS Layer, the request of system resources by the protocol stack entities via the VSI is adapted to the implemented RTOS.

The intention of this interface is to provide a set of function calls that is independent of the underlying RTOS. If an RTOS does not supply all the features described in the following, e.g. the possibility of periodic timers, this must be adapted within the OS Interface.

Some functionality of the interface described in the following may not be necessary for all RTOSs and therefore some functions do not have to be filled with code. The releasing of queue handles if they are not longer used might not always be necessary when communication is closed.

The OSX interface has been designed for customers who do not like to access the Condat frame directly in order to keep there software independent of the Condat VSI.

# Customer Frame Interface

The customer frame interface is split up into two parts. There are functions beginning with osx and functions beginning with int_osx. The difference between the two variants is that the osx functions do not have a parameter caller and the access to queues is done via identifiers that are determined by the customer. The int_osx_ functions do also have a parameter caller that is needed for the memory supervision feature and they access the queues via a frame queue handles.

For performance reasons it is recommended to use the int_osx interface.

The int_osx_ functions will be used by customers who use the queue handles that have been retrieved in the pei_init() function of the specific task to access the  interface, refer to 3.3.

Also there are customers who do not really use Condat’s PEI interface because they run their task in active body variant. These customers also have a PEI interface but it is provided by Condat and consists only of a pei_create(), pei_init() and pei_run() function. In this case Condat need to know which communication channels are required for this task and which identifiers the customer will use to access them. For this reason the communication channels for the osx have to be opened by calling an osx_open() function for every channel to be used later to assign a queue handle to every identifier. Also this task has to be registered as the osx caller by a call of osx_open() with specific parameters, refer to 3.4.3. Before osx_open() is called the first time the function osx_init() has to be called. Due to this registration and opening of communication channels osx functions internally know their caller and are able to convert the passed queue identifiers into queue handles. For osx functions refer to 3.4.

## Data Types

### T_VOID_STRUCT

Definition:	typedef unsigned long T_VOID_STRUCT

Description:  Pointers of type T_VOID_STRUCT are passed to functions in order to avoid warnings when using void pointers with a subsequent cast operation within the called function.

### T_ENUM_OS_QUEUE

Definition:

typedef enum

{

L1_QUEUE,        	/* internal L1 communication          	*/

DL_QUEUE,        	/* L1->DL                             		*/

RR_QUEUE,       	/* L1->RR                            		*/

GRR_QUEUE,   		/* L1->GRR                            		*/

LLC_QUEUE,   		/* L1->LLC e.g. ciphering via CCI     	*/

SNDCP_QUEUE, 		/* L1->SNDCP e.g. compression via CCI 	*/

MAX_OSX_QUEUE

} T_ENUM_OS_QUEUE;

Description:	The values of this enum are used are used as queue_types for the osx functions.

### xSignalHeaderRec

Definition:

typedef struct xSignalHeaderStruct

{

int           SignalCode;

int           _dummy1;

int           _dummy2;

int           _dummy3;

void          *SigP;

int           _dummy4;

} xSignalHeaderRec;

Description:	Pointers of this type are used as parameter for the (int_)osx_send_prim and (int_)osx_receive_prim functions.

## Constants

### Return Codes

OSX_OK		 0	successful execution

OS_ERROR	-1	error

## int_osx Functions

### int_osx_alloc_prim()

- Function definition:

xSignalHeaderRec* int_osx_alloc_prim ( short caller, unsigned long len )

- Parameters:

Type	Name	Meaning

short	caller	handle of calling task 	IN

unsigned long	len	size of partition to be allocated 	IN

- Return:

Type	Meaning

xSignalHeaderRec*	pointer to allocated partition

- Description:

The function int_osx_alloc_prim() allocates a partition of a size that is sufficient to hold len bytes.

If no free partition is available at calling time the calling task is suspended. If the request has been satisfied but the underlying OS-layer function had to wait for a free partition a corresponding message

“SYSTEM WARNING: Waited for partition, task task, size size, opc opc" is traced.

If the caller is a non-task thread, the function returns immediately regardless whether or not the request can be satisfied. In this case the calling task is suspended forever and an error message

“SYSTEM ERROR: No Partition available, task task size size, opc opc ” is traced and the system is stopped.

In both cases the complete partition pool of the requested size is dumped via the test interface.

### int_osx_alloc_mem()

- Function definition:

void * int_osx_alloc_mem ( short caller, unsigned long len )

- Parameters:

Type	Name	Meaning

short	caller	handle of calling task 	IN

unsigned long	len	size of memory buffer to be allocated 	IN

- Return:

Type	Meaning

void *	pointer to allocated memory buffer

- Description:

The function int_osx_alloc_mem() allocates a memory buffer of a size that is sufficient to hold len bytes.

If no free partition is available at calling time the calling task is suspended. If the request has been satisfied but the underlying OS-layer function had to wait for a free partition a corresponding message

“SYSTEM WARNING: Waited for memory, task task, size size" is traced.

If the caller is a non-task thread, the function returns immediately regardless whether or not the request can be satisfied. If no partition is available the message

“SYSTEM ERROR: No memory available, task task size size ” is traced and the system is stopped.

In both cases the complete partition pool of the requested size is dumped via the test interface.

### int_osx_free_prim()

- Function definition:

void osx_free_prim ( short caller, xSignalHeaderRec *prim_ptr )

- Parameters:

Type	Name	Meaning

short	caller	handle of calling task 	IN

xSignalHeaderRec *	prim_ptr	pointer to partition to be freed 	IN

- Return:	      ---

- Description:

The function int_osx_free_prim() deallocates the memory partition to which prim_ptr points.

### int_osx_free_mem()

- Function definition:

void int_osx_free_mem ( void * mem_ptr )

- Parameters:

Type	Name	Meaning

short	caller	handle of calling task 	IN

void *	mem_ptr	pointer to memory to be freed 	IN

- Return:	      ---

- Description:

The function int_osx_free_prim() deallocates the memory partition to which prim_ptr points.

### int_osx_send_prim()

- Function definition:

void int_osx_send_prim ( short caller, xSignalHeaderRec *prim_ptr, short queue_handle )

- Parameters:

Type	Name	Meaning

short	caller	handle of calling task 	IN

xSignalHeaderRec *	prim_ptr	pointer to message to be sent 	IN

short	queue_handle	destination queue handle  	IN

- Return:      ---

- Description:

The function int_osx_send_prim() sends the message to which prim_ptr points to the message queue identified by queue_handle.

If there is no space available in the destination queue at calling time the calling task is suspended. If the request has been satisfied but the underlying OS-layer function had to wait for a space in the destination queue a corresponding message

"SYSTEM WARNING: task waited for space in task queue" is traced.

If the caller is a non-task thread, the function returns immediately regardless whether or not the request can be satisfied. If no space in the destination queue is available the message

"SYSTEM ERROR: task write attempt to task queue failed" is traced.

### int_osx_receive_prim()

- Function definition:

xSignalHeaderRec * int_osx_receive_prim ( short caller, short queue_handle )

- Parameters:

Type	Name	Meaning

short	caller	handle of calling task 	IN

short	queue_handle	queue handle	IN

- Return:

Type	Meaning

xSignalHeaderRec	pointer to received message

- Description:

The function int_osx_receive_prim() supervises the message queue specified by the parameter queue_handle.

The calling task is suspended until a message is available in the queue.

### int_osx_send_sig()

- Function definition:

void int_osx_send_sig ( short caller, unsigned long opc, void *signal_ptr, short queue_handle )

- Parameters:

Type	Name	Meaning

short	caller	handle of calling task 	IN

unsigned long	opc	operation code of signal	IN

void *	signal_ptr	pointer to signal to be sent 	IN

short	queue_handle	destination queue handle  	IN

- Return:      ---

- Description:

The function int_osx_send_sig() sends the message to which signal_ptr points to the message queue identified by queue_handle.  Messages sent as a signal have a higher priority than primitives when the destination queue is read out.

If there is no space available in the destination queue at calling time the calling task is suspended. If the request has been satisfied but the underlying OS-layer function had to wait for a space in the destination queue a corresponding message

"SYSTEM WARNING: task waited for space in task queue" is traced.

If the caller is a non-task thread, the function returns immediately regardless whether or not the request can be satisfied. If no space in the destination queue is available the message

"SYSTEM ERROR: task write attempt to task queue failed" is traced.

## osx Functions

### _osx_init

- Function definition:

void _osx_init ( void )

- Parameters:  ---

- Return:	        ---

- Description:

The function _osx_init() initializes the table that is used for the assignment of queue_types to queue_handles that is done in osx_open(), refer to 3.4.3.

This function must only be called by processes that intend to access the queues via queue_types instead of the handles retrieved in the pei_init() function.

### _osx_config()

- Function definition:

short _osx_config ( const char * config )

- Parameters:

Type	Name	Meaning

char *	config	configuration string 	IN

- Return:

Type	Meaning

short	OSX_OK	success

OSX_ERROR	error

- Description:

The function _osx_config() allows dynamic configuration of the OSX layer. Currently only the disabling of L1S traces is implemented.

L1S traces are sent from L1S to L1A via primitives using the primitive id TRACE_INFO (0x7d). When L1S traces are dynamically disabled these primitives are discarded during a osx_send_prim() command and the memory used for the primitive is freed.

To disabled L1S traces send a configuration primitive “CONFIG L1S_TRACE_DISABLE” to the L1 entity. To enable the traces send primitive “CONFIG L1S_TRACE_ENABLE” to L1.

### _osx_open()

- Function definition:

short _osx_open ( short caller, unsigned short queue_type, short queue_handle )

- Parameters:

Type	Name	Meaning

short	caller	task handle of the caller 	IN

unsigned short	queue_type	index of a queue	IN

short	queue_handle	handle of a queue 	IN

- Return:

Type	Meaning

int	OSX_OK	success

OSX_ERROR	error

- Description:

The function _osx_open() assigns a queue_type to a queue_handle. As a consequence of this assignment the queue_type is converted to a queue_handle in the following queue accesses via osx_send_prim() and osx_receive_prim().

The queue_handles are the handles returned by vsi_c_open() calls in the pei_init() function of the xxx_pei.c module.

This function must only be called by processes that intend to access the queues via queue_types instead of the handles retrieved in the pei_init() function.

### osx_alloc_prim()

- Function definition:

xSignalHeaderRec* osx_alloc_prim ( unsigned long len)

- Parameters:

Type	Name	Meaning

unsigned long	len	size of partition to be allocated 	IN

- Return:

Type	Meaning

xSignalHeaderRec*	pointer to allocated partition

- Description:

The function osx_alloc_prim() retrieves the handle of the caller and calls the function int_osx_alloc_prim() to allocate a partition of a size that is sufficient to hold len bytes.

If no free partition is available at calling time the calling task is suspended. If the request has been satisfied but the underlying OS-layer function had to wait for a free partition a corresponding message

“SYSTEM WARNING: Waited for memory, task task, size size" is traced.

If the caller is a non-task thread, the function returns immediately regardless whether or not the request can be satisfied. If no partition is available the message

“SYSTEM ERROR: No memory available, task task size size, opc opc ” is traced and the system is stopped.

In both cases the complete partition pool of the requested size is dumped via the test interface.

This function must only be called by processes that have registered themselves to access the osx in the pei_init() function. The caller that is needed internally for memory supervision functionality is set to the caller of osx_open() when called with queue_type and queue_handle set to zero.

### osx_alloc_mem()

- Function definition:

void* osx_alloc_mem ( unsigned long len)

- Parameters:

Type	Name	Meaning

unsigned long	len	size of memory buffer to be allocated 	IN

- Return:

Type	Meaning

void *	pointer to allocated memory

- Description:

The function osx_alloc_mem() retrieves the handle of the caller and calls the function int_osx_alloc_mem() to allocate a memory buffer of a size that is sufficient to hold len bytes.

If no free partition is available at calling time the calling task is suspended. If the request has been satisfied but the underlying OS-layer function had to wait for a free partition a corresponding message

“SYSTEM WARNING: Waited for memory, task task, size size" is traced.

If the caller is a non-task thread, the function returns immediately regardless whether or not the request can be satisfied. If no partition is available the message

“SYSTEM ERROR: No memory available, task task size size ” is traced and the system is stopped.

In both cases the complete partition pool of the requested size is dumped via the test interface.

This function must only be called by processes that have registered themselves to access the osx in the pei_init() function. The caller that is needed internally for memory supervision functionality is set to the caller of osx_open() when called with queue_type and queue_handle set to zero.

### osx_free_prim()

- Function definition:

void osx_free_prim ( xSignalHeaderRec *prim_ptr )

- Parameters:

Type	Name	Meaning

xSignalHeaderRec *	prim_ptr	pointer to partition to be freed 	IN

- Return:	      ---

- Description:

The function osx_free_prim()retrieves the handle of the caller and calls the function int_osx_free_mem() to  deallocate the memory to which prim_ptr points.

This function must only be called by processes that have registered themselves to access the osx in the pei_init() function. The caller that is needed internally for memory supervision functionality is set to the caller of osx_open() when called with queue_type and queue_handle set to zero.

### osx_free_mem()

- Function definition:

void osx_free_mem ( void * mem_ptr )

- Parameters:

Type	Name	Meaning

void *	mem_ptr	pointer to memory to be freed 	IN

- Return:	      ---

- Description:

The function _osx_free_mem() retrieves the handle of the caller and calls the function int_osx_free() to deallocates the memory to which mem_ptr points.

This function must only be called by processes that have registered themselves to access the osx in the pei_init() function. The caller that is needed internally for memory supervision functionality is set to the caller of osx_open() when called with queue_type and queue_handle set to zero.

### osx_send_prim()

- Function definition:

void osx_send_prim ( xSignalHeaderRec *prim_ptr, T_ENUM_OS_QUEUE queue_type )

- Parameters:

Type	Name	Meaning

xSignalHeaderRec *	prim_ptr	pointer to message to be sent 	IN

T_ENUM_OS_QUEUE	queue_type	destination queue identifier  	IN

- Return:      ---

- Description:

The function osx_send_prim() retrieves the handle of the caller and calls the function int_osx_send_prim() to send the message to which prim_ptr points to the message queue identified by queue_type.

The parameter queue_type is converted to a queue handle by evaluation of a registry were the assignment of queue identifiers (queue_type) to queue handles is stored during previous osx_open() calls.

If there is no space available in the destination queue at calling time the calling task is suspended. If the request has been satisfied but the underlying OS-layer function had to wait for a space in the destination queue a corresponding message

"SYSTEM WARNING: task waited for space in task queue" is traced.

If the caller is a non-task thread, the function returns immediately regardless whether or not the request can be satisfied. If no space in the destination queue is available the message

"SYSTEM ERROR: task write attempt to task queue failed" is traced.

is traced.

This function must only be called by processes that have registered themselves to access the osx in the pei_init() function. The caller that is needed internally for memory supervision functionality is set to the caller of osx_open() when the queue_handle is assigned to the queue_type.

### osx_receive_prim()

- Function definition:

xSignalHeaderRec *osx_receive_prim ( T_ENUM_OS_QUEUE queue_type )

- Parameters:

Type	Name	Meaning

T_ENUM_OS_QUEUE	queue_type	queue identifier 	IN

- Return:

Type	Meaning

xSignalHeaderRec	pointer to received message

- Description:

The function osx_receive_prim() retrieves the handle of the caller and calls the function int_osx_receive_prim() to supervise the message queue specified by the parameter queue_type.

The parameter queue_type is converted to a queue handle by evaluation of a registry were the assignment of queue identifiers (queue_type) to queue handles is stored during previous osx_open() calls.

The calling task is suspended until a message is available in the queue.

This function must only be called by processes that have registered themselves to access the osx in the pei_init() function. The caller that is needed internally for memory supervision functionality is set to the caller of osx_open() when the queue_handle is assigned to the queue_type.

### osx_send_sig()

- Function definition:

void osx_send_sig ( unsigned long opc, void *signal_ptr, T_ENUM_OS_QUEUE queue_type)

- Parameters:

Type	Name	Meaning

unsigned long	opc	operation code of signal	IN

void *	signal_ptr	pointer to signal to be sent 	IN

T_ENUM_OS_QUEUE	queue_type	destination queue identifier  	IN

- Return:      ---

- Description:

The function osx_send_sig() retrieves the handle of the caller and calls the function int_osx_send_sig() to send the message to which signal_ptr points as signal to the message queue identified by queue_type. Messages sent as a signal have a higher priority than primitives when the destination queue is read out.

The parameter queue_type is converted to a queue handle by evaluation of a registry were the assignment of queue identifiers (queue_type) to queue handles is stored during previous osx_open() calls.

If there is no space available in the destination queue at calling time the calling task is suspended. If the request has been satisfied but the underlying OS-layer function had to wait for a space in the destination queue a corresponding message

"SYSTEM WARNING: task waited for space in task queue" is traced.

If the caller is a non-task thread, the function returns immediately regardless whether or not the request can be satisfied. If no space in the destination queue is available the message

"SYSTEM ERROR: task write attempt to task queue failed" is traced.

is traced.

This function must only be called by processes that have registered themselves to access the osx in the pei_init() function. The caller that is needed internally for memory supervision functionality is set to the caller of osx_open() when the queue_handle is assigned to the queue_type.

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |