Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document

G23-UMTS Protocol Stack

Syntax description for air interface message documents

User Guide

(Edit via File/Properties/Summary!)

| Document Number: | 06-03-22-UDO-0001 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.4 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2001-Jul-20 |
| Last changed: | 2003-Sep-16 by XGUTTEFE |
| File Name: | 8350_300_MSG_Syntax.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2001-Jul-20 | MVJ |  | 0.1 | Being Processed | 1 |
| 2001-Aug-24 | MVJ |  | 0.2 | Being Processed |  |
| 2001-Sep-06 | MVJ |  | 0.3 | Being Processed |  |
| 2003-May-07 | XGUTTEFE |  | 0.4 | Draft |  |

Notes:

- Initial version

Table of Contents

1	Introduction	6

2	Message Document Structure	6

3	Constants	7

3.1	Description of Subsections	7

3.1.1	Description	7

3.1.2	Pragma	7

3.1.3	Definition	8

3.1.4	History	8

3.2	Example	8

4	Types	8

4.1	Description of Subsections	8

4.1.1	Description	8

4.1.2	Definition	8

4.1.3	History	8

4.2	Example	9

5	Messages	9

5.1	Description of Subsections	9

5.1.1	Definition	9

5.1.2	Elements	10

5.1.3	History	10

5.2	Example	10

6	Structured Elements	11

6.1	Description of Subsections	11

6.1.1	Definition	11

6.1.2	Elements	12

6.1.3	History	12

6.2	Example	12

7	Basic Elements	13

7.1	Description of Subsections	13

7.1.1	Definition	14

7.1.2	Values	14

7.1.3	History	14

7.2	Example	14

8	Common Sections/Table Columns	15

8.1	Subsections Described	15

8.1.1	Description	15

8.1.2	Pragma	15

8.1.3	Definition	15

8.1.4	Elements	15

8.1.5	Values	15

8.1.6	History	15

8.1.7	Combination matrix	15

8.2	Table Columns	16

8.2.1	Short name	16

8.2.2	Long name	16

8.2.3	Comment	16

8.2.4	Version	16

8.2.5	Reference	17

8.2.6	Spec ref	17

8.2.7	Link	17

8.2.8	Ctrl	17

8.2.9	ID	17

8.2.10	Direction	18

8.2.11	Pres	18

8.2.12	Type	18

8.2.13	Length	18

8.2.14	Bit len	18

8.2.15	Value	18

8.2.16	C-Macro	18

8.3	Table Column/Section Combination Matrix	19

9	Formal Grammar	19

9.1	Ctrl column	20

10	Example	21

Appendices	22

A.	Acronyms	22

B.	Glossary	22

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

# Introduction

This document describes how to write Condat Air Interface Message documents.

Air interface messages are peer-to-peer messages between a MS and its network peer. These messages are standardized by ETSI/3GPP, and thus have well defined formats.

The Condat Air Interface Message documents describe how data in air interface messages are organized, and how they can be used in entities. They operate at bit-level as opposed to Service Access Points (SAPs) which normally operate at byte level.

Air Interface Messages are documents written in Word as part of the high level design phase. When the air interface messages they describe are needed in actual code, the documents are run through the Condat tool chain (FIXME: reference) which produce header files and other data needed in program code.

This document is divided into sections, which describe how the Air Interface Message documents are organized, and what possibilities are available in each section.

# Message Document Structure

The Air Interface Message document is created from a Condat Word template (FIXME: reference) by choosing “air interface message” document type when invoking the template.

This will create the title page, initialise the document history, and an introductory headline.

The rest of the document sections must be structured as in the list below.  The order must be preserved, but some sections may be left out. These sections are marked [optional].

| 2 | Constants [optional] | Contains global constants |
| --- | --- | --- |
| 3 | Types | Lists coding types used (e.g. type 4 TLV) |
| 4 | Messages | The actual air interface message descriptions (e.g. ATTACH REQUEST) |
| 5 | Structured Elements [optional] | Elements in air interface messages (e.g. MS class mark) |
| 6 | Basic Elements | Basic types/values (e.g. MS type 2) |

The section numbering shown in the list above is the one traditionally use in Condat Air Interface Message documents, so it is recommended to stick to it.

The sections above contain a number of subsections which act as keywords, separating different types of information in each subsection, see Figure 1.

Figure 1 – Use of subsections as keywords in an AIM document

All subsections are listed in section 8.1.

Most sections have an associated Word style in Air Interface Message documents, i.e. a table of elements has Word style “ElementsTabelle”.  In each section where such a style is applicable, it will be mentioned. While using these styles is not mandatory, you are strongly urged to do so – it will make Condat’s planned transition from Word documents into XML much easier.

The air interface message document sections listed above are described in detail in the following chapters.

# Constants

This section contains information about global constants used in the document. The section must contain a table of constants, and a “History” list to track changes.

## Description of Subsections

In the Constants section, the following subsections are possible:

- Description

- Pragma

- Definition

- History

The use of each subsection is described below.

### Description

The “description” subsection is optional (but strongly recommended) – It contains a textual description of the information in the contants table below.

### Pragma

This section is optional. It is used for modifying the behaviour of the Condat tool chain, e.g. for defining a prefix for identifiers (constants, values, etc).

For more information, please refer to section 8.1.2.

### Definition

The Definition table in the constant section is similar to Basic Elements (section 7) definitions. It contains a name column, a value column, and optionally a comment column.

The value from the value column is then bound to the identifier named by entry in the name column.

### History

The “History” subsection contains a brief changelog for the table. For more information, see the “History” section.

## Example

The example below shows a section defining the constant “L3MAX” to the value “251”. The value field may specify the constant value in a number of formats, e.g. decimal (including negative values), hexadecimal, binary etc.  Please see 8.2.15 for a complete list.

Definition:

| name | value | comment |
| --- | --- | --- |
| L3MAX | 251 | maximum size of a L3 buffer |

History:

01-Jan-2001	DEV	Initial

# Types

This section contains information about the air interface message coding types used in the document. The section must contain a table of coding types, and a “History” list to track changes.

## Description of Subsections

In the Types section, the following subsections are possible:

- Description

- Definition

- History

The use of each subsection is described below.

### Description

The “description” subsection is optional (but strongly recommended) – It contains a textual description of the information in the contants table below.

### Definition

The Definition table in the types section contains a name column, an add bit column, and optionally ctrl and comment columns.

For each type mentioned in the name column, the add bit column tells how many bits are allocated to type and length fields, and the ctrl field may set a type to be optional by default.

For example, a GSM1_TV type is a type 1 IE with 4 extra bits of type information. Therefore, its add bits column should be set to 4. The GSM1_TV type is also optional by default, so its ctrl column could be set to “optional”.

### History

The “History” subsection contains a brief changelog for the table. For more information, see the “History” section.

## Example

Definition:

| name | add bit | ctrl | comment |
| --- | --- | --- | --- |
| GSM1_V | 0 |  | mandatory IE V-component in one nibble |
| GSM1_TV | 4 | optional | optional with V-Component |
| GSM2_T | 8 | optional | optional IE, contains IEI only (no V-component) |
| GSM3_V | 0 |  | mandatory / conditional, no IEI, V-component only |
| GSM3_TV | 8 | optional | optional IE with V-component |
| GSM4_LV | 8 |  | mandatory / conditional, length and V-component |
| GSM4_TLV | 16 | optional | optional IE with length indicator and V-component |
| GSM5_V | 0 | optional | optional IE (bitstream to end of message) |
| BCD | 4 |  | binary coded decimal number |
| BCDODD | 4 |  | binary coded decimal number starting with digit1 |
| BCDEVEN | 4 |  | binary coded decimal number starting with digit2 |

History:

01-Jan-2001	DEV	Initial

# Messages

This section defines the air interface message descriptions. Typically, the air interface message definitions contain a structure of a number of complex and simple elements.

Each air interface interface definition subsection must contain a ”Definition“ table, and a “History” list to track changes. Having a “Description” subsection is also strongly recommended.

## Description of Subsections

In the Messages section, the following subsections are possible:

- Description

- Definition

- Elements

- History

The use of each subsection is described below.

### Definition

The “definition” subsection defines the name of an air interface message, its type, and which direction(s) (uplink/downlink) it applies. Optionally, a field may define when to include the message in the output (using Version) or, if the element is defined in another document, a Link column detailing where to find it. In case the message is included using the link facility, no elements are allowed.

The “definition” subsection consists of a subsection caption and a definition table which contains the information mentioned above. The subsection caption must be “Definition:” and have Word type “Definition”.

The ”Definition” table in the messages section contains a Long name column, a Short name column, an ID column, and a Direction column. Optionally, Version and Comment columns may be present.

The Long name column is informational only. The Short name column names the C identifier, with which this message is associated (C struct). This struct should be used in program code to read from/fill in the message structure.

The ID column contains the air interface message msgid (message id) as defined in ETSI/3GPP specs. Used to differentiate messages, and code/decode messages appropriately.

Finally, the Direction column specifies whether a message can travel “uplink”, “downlink”, or “both”.

### Elements

The “elements” subsection defines which structured and/or basic elements form the air interface message, as well as their coding type, optionality, and size. These elements correspond to IEs (information elements) in 3GPP specs. Optionally, a field may define when to include the message in the output (using Version).

The “elements” subsection consists of a subsection caption and a elements table which contains the information mentioned above. The subsection caption must be “Elements:” and have Word type “Elements”.

A minimal ”Elements” table (which must be of Word type “ElementsTabelle”) in the messages section contains a Long name column, a Short name column, a Reference column (typically abbreviated to “ref”), a Type column, and a Length column (typically abbreviated to “len”). Optionally, an ID defines IE identifiers for optional TLV elements. Elements may be declared optional using a Pres column. Optional inclusion of an element can be realized using a Version column, and finally a Comment column can give further information about the elements. For easy back-tracking, a Spec ref column is recommended. It should point to the 3GPP specification that defines the element in question.

The Long name column is informational only. The Short name column names the C identifier, with which this element is associated (in C). This struct should be used in program code to read from/fill in the message structure.

The Reference column tells where to find the element definition, in case it is not a spare. Spares have no reference (See Short name for more information on spares). The reference column may be substituted with a Link column, if the element is defined outside the current document.

Type is the coding type to use for the element. It may be any of the type suffixes defined in the Types section, i.e. the types without “GSM” and type number (BCD, 1, 2, …). This usually boils down to a “[T][L]V” suffix. For example, an optional type 4 TLV element would have a Type of “TLV”.

Either Bit len or Length must be present and define the bit or byte length of the element, respectively. The length may be variable. For example, a type 4 TLV element which can be from 4 to 16 bytes would have a Length field of “4-16”.

The ID column contains the information element identifier (IEI) as defined in ETSI/3GPP specs. An element with a value in the ID column is automatically marked as optional, as its presence depends whether the IEI is included in a message.

### History

The “History” subsection contains a brief changelog for the table. For more information, see the “History” section.

## Example

The table below shows an example of an air interface message section (Session Management ACTIVATE PDP CONTEXT ACCEPT message).

Note: The ref column is for example only – it refers to sections in the originating document, and its content is invalid here.

Description:

This message is sent by the network to the MS to acknowledge activation of a PDP context.

If the MS did not request a static address in the corresponding ACTIVATE PDP CONTEXT REQUEST message, the network shall include the PDP address IE in this message, otherwise not. Protocol configuration options are included in the message when the network wishes to transmit protocol configuration options for the external PDN. Likewise for the GSM only packet flow identifier (PFI).

Reference : [3G 24.008, 9.5.2]

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| activate PDP context accept | activate_pdp_acc | 66 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len | version |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | message type | msg_type | 6.4 | [3G 24.008, 10.4] | M | V3 | 1 |  |
|  | negotiated LLC SAPI | llc_sapi | 5.2 | [3G 24.008, 10.5.6.9] | M | V | 1 |  |
|  | negotiated QoS | qos | 5.3 | [3G 24.008, 10.5.6.5] | M | LV | 12 |  |
|  | radio priority | radio_prio | 5.7 | [3G 24.008, 10.5.7.2] | M | V | ½ |  |
|  | spare half octet | .0000 |  | [3G 24.008, 10.5.1.8] | M | V | ½ |  |
| 0x2B | packet data protocol address | address | 5.4 | [3G 24.008, 10.5.6.4] | O | TLV | 4-20 |  |
| 0x27 | protocol configuration options | pco | 5.6 | [3G 24.008, 10.5.6.3] | O | TLV | 3-253 |  |
| 0x34 | packet flow identifier | pfi | 5.9 | [3G 24.008, 10.5.6.11] | O | TLV | 3 | R99 ¦ UMTS |

History:

01-Jan-2001	DEV	Initial

# Structured Elements

This section contains information about the complex elements used in air interface message descriptions. Typically, the complex element definitions contain a structure or array of (possibly optional) bit-coded elements. This section is optional – that is if there are no complex parameters in any of the air interface messages, this sections should be skipped.

While identical types may be declared in the same section, all different element definitions must be declared in a separate subsection (except for definitions imported from other documents; see 8.2.7). Each subsection must contain a “Definition” table, and a “History” list to track changes. Having a “Description” subsection is also strongly recommended.

## Description of Subsections

In the Structured Elements section, the following subsections are possible:

- Description

- Definition

- Elements

- History

The use of each subsection is described below.

### Definition

The “definition” subsection defines the name of a structured element, and its type and size. Optionally, a field may define when to include the message in the output (using Version) or, if the element is defined in another document, a Link field detailing where to find it. In case the element is included using the link facility, no other element content is allowed (i.e. elements or values).

The “definition” subsection consists of a subsection caption and a definition table which contains the information mentioned above. The subsection caption must be “Definition:” and have Word type “Definition”.

The definition table (which must have type “DefinitionTabelle”) must have the following columns: Short name, Type, and Bit len or Length (the latter typically abbreviated to “len”).

The Short name column names the identifier with which this basic element is associated. This will end up being the name of the C struct containing the elements defined in the table.

Type is the coding type to use for the element. It may be any of the type numbers defined in the “Types” section, i.e. without “GSM” and “[T][L]V”. The “[T][L]V” suffix is determined where the structured element is used (i.e. in the air interface message). For example, an optional type 4 TLV element would have a Type of “4”.

Either Bit len or Length must be present and define the bit or byte length of the structured element, respectively. The length may be variable. For example, a type 4 TLV element which can be from 4 to 16 bytes would have a Length field of “4-16”.

Optionally, a Version column may dictate when to include the basic element.

### Elements

The “elements” subsection defines the basic elements that form the structured element. For each element, their names and sizes are defined, and there must be a reference where to find them. Optionally, a field may declare elements optional, while a Version column may define when to include the message in the output (using) or, if the element is defined in another document, a Link field detailing where to find it.

The “elements” subsection consists of a subsection caption and a elements table which contains the information mentioned above. The subsection caption must be “Elements:” and have Word type “Elements”.

A minimal ”Elements” table (which must be of Word type “ElementsTabelle”) in the messages section contains a Long name column, a Short name column, a Reference column (typically abbreviated to “ref”), and a Bit len column. Elements may be declared optional using a Pres column. Optional inclusion of an element can be realized using a Version column, and finally a Comment column can give further information about the elements.

The Long name column is informational only. The Short name column names the C identifier, with which this element is associated (in C). This struct should be used in program code to read from/fill in the message structure.

The Reference column tells where to find the basic element definition, in case it is not a spare. Spares have no reference (See Short name for more information on spares). The reference column may be substituted with a Link column, if the element is defined outside the current document.

Type is the coding type to use for the element. It may be any of the type suffixes defined in the “Types” section, i.e. the types without “GSM” and type number (BCD, 1, 2, …). This usually boils down to a “[T][L]V” suffix. For example, an optional type 4 TLV element would have a Type of “TLV”.

Either Bit len or Length must be present and define the bit or byte length of the basic element, respectively. The length may be variable. For example, a type 4 TLV element which can be from 4 to 16 bytes would have a Length field of “4-16”.

### History

The “history” subsection contains a brief changelog for the table. For more information, see the “History” section.

## Example

The example below shows the structured element definitions for the Session Management “Quality of Service” parameter. Note: The “ref” column is for example only – it refers to sections in the originating document, and its content is invalid here.

Description:

The purpose of the quality of service information element is to specify the QoS parameters for a PDP context.

Reference : [3G 24.008, 10.5.6.5]

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| quality of service | qos | 4 | 12 |

Elements:

| long name | short name | ref | bit len | version |
| --- | --- | --- | --- | --- |
| spare | .00 |  | 2 |  |
| delay class | delay | 6.2 | 3 |  |
| reliability class | reliability | 6.3 | 3 |  |
| peak throughput | peak | 6.6 | 4 |  |
| spare | .0 |  | 1 |  |
| precedence class | precedence | 6.7 | 3 |  |
| spare | .000 |  | 3 |  |
| mean throughput | mean | 6.8 | 5 |  |
| traffic class | tc | 6.9 | 3 | R99 ¦ UMTS |
| delivery order | order | 6.10 | 2 | R99 ¦ UMTS |
| delivery of erroneous SDU | del_err_sdu | 6.11 | 3 | R99 ¦ UMTS |
| maximum SDU size | max_sdu | 6.12 | 8 | R99 ¦ UMTS |
| maximum uplink bit-rate | max_br_ul | 6.13 | 8 | R99 ¦ UMTS |
| maximum downlink bit-rate | max_br_dl | 6.14 | 8 | R99 ¦ UMTS |
| residual BER | ber | 6.15 | 4 | R99 ¦ UMTS |
| SDU error ratio | sdu_err_ratio | 6.16 | 4 | R99 ¦ UMTS |
| transfer delay | xfer_delay | 6.17 | 6 | R99 ¦ UMTS |
| traffic handling priority | handling_pri | 6.18 | 2 | R99 ¦ UMTS |
| guaranteed upink bit-rate | guar_br_ul | 6.19 | 8 | R99 ¦ UMTS |
| guaranteed downlink bit-rate | guar_br_dl | 6.20 | 8 | R99 ¦ UMTS |

History:

01-Jan-2001	DEV	Initial

# Basic Elements

This section contains information about the basic elements used in air interface message descriptions. Typically, the basic element definitions contain a single bit-coded element with a number of possible values.

While identical types may be declared in the same section, all different element definitions must be declared in a separate subsection (except for definitions imported from other documents; see 8.2.7). Each subsection must contain a ”Definition“ table, and a “History” list to track changes. Having a “Description” subsection is also strongly recommended.

## Description of Subsections

- Description

- Definition

- Values

- History

The Values subsection is mandatory only when actually defining values.

The use of each subsection is described below.

### Definition

The “definition” subsection, as the name says, defines the basic element. In fact, it defines the basic element’s name and (bit) size, and optionally when to include it (using Version) or, if the element is defined in another document, a Link field detailing where to find it. In case the element is included using the link facility, no other element content is allowed (i.e. elements or values).

The “definition” subsection consists of a subsection caption and a definition table which contains the information mentioned above. The subsection caption must be “Definition:” and have Word type “Definition”.

The definition table (which must have type “DefinitionTabelle”) must have the following columns: Short name and Bit len or Length. The Short name column names the identifier with which this basic element is associated. Typically, this will end up being a byte/short/int in a structure which uses this basic element. Either Bit len or Length must be present and define the bit or byte length of the basic element, respectively.

Optionally, a Version column may dictate when to include the basic element.

### Values

If the basic element has values, they must be declared in a “values” subsection. The “values” subsection consists of a values subsection caption and a value table.

The table contains value definitions, and how these values are bound to C identifiers (#defines). The values are defined in a Value column, while the C identifier, to which the value is bound, is named in a C-Macro column. Normally, a Comment column is appended in order to explain the individual values. See section 7.2 below for an example.

### History

The “history” subsection contains a brief changelog for the table. For more information, see the “History” section.

## Example

The example below shows the basic element definitions for the Session Management “SDU Error Ratio” parameter.

Description:

Reference: [3G 24.008, 10.5.6.5] (Quality of service, R99)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| SDU error ratio | sdu_err_ratio | 4 |

Values:

| value | c-macro | Comment |
| --- | --- | --- |
| 0 | QOS_SDU_ERR_SUB | Subscribed SDU error ratio |
| 0 | QOS_SDU_ERR_RES_DL | Reserved (downlink only) |
| 1 | QOS_SDU_ERR_1E_2 | 1*10-2 |
| 2 | QOS_SDU_ERR_7E_3 | 7*10-3 |
| 3 | QOS_SDU_ERR_1E_3 | 1*10-3 |
| 4 | QOS_SDU_ERR_1E_4 | 1*10-4 |
| 5 | QOS_SDU_ERR_1E_5 | 1*10-5 |
| 6 | QOS_SDU_ERR_1E_6 | 1*10-6 |
| 7 | QOS_SDU_ERR_1E_1 | 1*10-1 |
| 15 | QOS_SDU_ERR_RES | Reserved |

History:

01-Jan-2001	DEV	Initial

# Common Sections/Table Columns

This section describes the subsections and table columns that are used in the Air Interface Message documents.

## Subsections Described

### Description

The “description” subsection is for describing the object in the section. It is entirely informational, i.e. it is not used by the tool chain. Therefore, it is not mandatory in any section, but it is strongly recommended that a description is included in any section.

The “description” subsection caption must be of Word type “Description” and all descriptive text must be of Word type “DescriptionText”.

### Pragma

The “pragma” subsection is used for modifying the behaviour of the Condat tool chain. It must contain a table with the same format as a constant definition table (section 3.1.2).

Currently only two pragmas are supported. The pragma PREFIX allows all constants, elements and types generated from the SAP document to be automatically prefixed with a letter combination. The pragma COMPATIBILITY_DEFINES makes the tool chain generate C pre-processor directives, redefining legacy style declarations to the current standard.

### Definition

The “definition” subsection names an object, and defines a number of high level attributes for the object. The attributes defined depend on the object type. The “definition” may be used in the “Constants”, “Types”, “Messages”, “Structured Elements”, and “Basic Elements” sections.

The definition subsection caption must be of Word type “Definition” and all history entries must be of Word type “DefinitionTabelle”.

### Elements

The “elements” subsection consists of a subsection caption and a table. The table contains element definitions, and how these values are bound to C identifiers (C struct or type definitions).

The elements subsection caption must be of Word type “Elements” and all history entries must be of Word type “ElementsTabelle”.

### Values

The “values” subsection consists of a subsection caption and a table. The table contains value definitions, and how these values are bound to C identifiers (#defines).

The values subsection caption must be of Word type “Values” and all history entries must be of Word type “ValuesTabelle”.

### History

The “history” subsection contains a manually updated list of changes. It is mandatory in the “Constants” and “Types” sections as well as in all subsections of the remaining sections (i.e. subsections of “Messages”, “Structured Elements”, and “Basic Elements”). The developer working on the document is responsible for updating the history list.

The history subsection caption must be of Word type “History” and all history entries must be of Word type “HistoryText”.

### Combination matrix

Table 1 shows which subsections may be present in which sections. Note that the description and pragma sections are not mandatory; all other sections are mandatory.

|  | Description | Pragma | Definition | Elements | Values | History |
| --- | --- | --- | --- | --- | --- | --- |
| Constants |  |  |  |  |  |  |
| Types |  |  |  |  |  |  |
| Messages |  |  |  |  |  |  |
| Structured Elements |  |  |  |  |  |  |
| Basic Elements |  |  |  |  |  |  |

Table 1 – Subsections (keywords) allowed in Air Interface Message sections

Legend:
		Allowed
		Not allowed

## Table Columns

The subsections below lists all table columns allowed in Air Interface Documents. Some columns are allowed in all tables, while others are restricted to one or more specific tables. Such restrictions are mentioned in the respective subsections below.

Worth noting is that table column names are not case sensitive, so “ShoRt NaME” will work the same as “short name”.  Also, the order of columns is insignificant, so for example a “comment” column may be inserted at any position.

### Short name

The short name column defines the C identifier with which this object is associated. In other words, the name in the short name column will be the name of the object in the C header file. There can be no more than one short name column.

Spare elements may be declared using the special “.<binary>” notation. For example, a 4-bit zero-filled spare element would be declared by  “.0000”. A 7-bit spare element filled with 0x2b would be declared as “.0101011”. Spare elements are automatically inserted/removed by the Condat tools, and therefore are not included in C structs.

### Long name

The long name column is for human readability only. It is not visible in the C code/header files. It is included in the test application for debugging purposes, though. There can be no more than one long name column.

### Comment

Is a comment, as the name says. It is not included anywhere but the Air Interface Message document. Use it for descriptive purposes, or to hide columns with other content (the tools will skip the column when it is a comment). There may be any number of comment columns, and it may be in any table.

### Version

The version column provides a way to conditionally include definitions, sections, elements etc. The version column contains a Boolean expression, which must evaluate to true in order for the element to be included. If the expression is false, the object is not included in the output C code, so all identifiers will remain undefined.

In the Boolean expression, an identifier that has a value (i.e. has a value other than the empty string) evaluates to true, while an undefined identifier evaluates to false. Between identifiers, Boolean operators may be used to refine the expression. See below for a list of operators allowed.

If, for example, a version column for an air interface message (in section 4 by convention) says “R99”, the message will be included if and only if the identifier R99 is defined. A version column which says “R99 && ! UMTS” will be included if and only if the identifier R99 is defined, and the identifier UMTS is not defined.

The following Boolean expressions are allowed in the version column:

| ! | Logical not. True if the identifier is not defined |
| --- | --- |
| ¦ | Logical or. True if either the expression on the left or right is true. |
| & | Logical and. True only if both expressions on the left and right are true. |
| ( ) | Grouping operators. Changes operator precedence. |

Note: The version column syntax is not yet finalized, so the table above should serve as information on what is possible. If the syntax differs when finalized, a later version of this document should reflect it.

### Reference

The reference column (typically shortened to “ref”) is used for linking from a table to an element. The ref column may appear once in message element tables and structured elements element tables. In place of a ref column, one may use a “Link” column if the element is defined in an external document.

### Spec ref

The spec ref column is informational only. It allows the developer to tell where the object in question was defined in external specifications (such as ETSI/3GPP). It is allowed to appear once in message, element, and value tables.

### Link

The Link column allows inclusion of elements defined in other documents. It contains a hyperlink to a section in another air interface/SAP document, from which the Condat tool chain fetches the definitions.

A Link columns is mutually exclusive with a Reference column. They serve the same purpose, only Link columns refer to information outside the current document.

Condat convensions say that all linked elements in a section should go in the same table. That is, the Link column should be placed in one message definition, structured elements definition, and basic elements definition tables, and may contain a number of elements.

### Ctrl

The ctrl column is the most complex of all columns. It contains instruction for the Condat tool chain on how to define or interpret the object in question. Section 9.1 contains the formal definition of the possible contents in the Ctrl column.

Arrays are defined through the “[<from> .. <to>]” or “[<number>]” notation. The latter defines a constant length array with <number> elements, while the former defines a variable length array with anything from <from> to <to> elements, inclusive.

Note: Arrays of unions are not supported. Unions must be encapsulated in a structure in order to create an array. The structure requirement is due to the extra union controller (“ctrl_”) element inserted by the Condat tool chain; This element is outside the union, and thus needs a structure to contain it.

Conditional inclusion of elements is possible by using the “{<variable> <operator> <number/constant>}” syntax. The condition is checked at runtime, so for example “{ TI = 7 }” in an element’s ctrl column would include the element only if the TI (transaction identifier) has the value 7. <variable> may be either a defined constant or an element defined earlier in the message/structured element. Operator may be “=”, “#”, “<”, “<=”, “>”, and “>=”.

Calculations are possible by using the calculator built into Condat’s runtime tools. The calculator uses reverse polish notation, i.e. it is a stack machine (not unlike the venerable HP48). With it, one can write quite complex expressions, and move the bit-pointer back and forth in the message. Please refer to the ctrl column syntax description in section 9.1 for a list of possible operations.

Dynamic arrays (or pointer types) may be declared using the keywords “ptr” or “dyn”.

### ID

The ID column is allowed in air interface message definition and element tables only. In message definition tables, ID defines the air interface message ID; In message element tables, ID defines the IEI value for the (optional) element is included in the air interface message.

Message definition example: An air interface message has an ID of 66 (session management ACTIVATE PDP CONTEXT ACCEPT). This tells the Condat tool chain that the elements in the table below are associated with the ID 66. For an incoming message (downlink) with ID 66, the Condat tools will unpack data into the fields/variables defined in the elements below. For outgoing messages (uplink) with ID 66, a message will be formed using ID and the value from the elements below.

Message element example: A type 4 TLV element may have an IEI of 0x2b. This tells the Condat tools chain that this element is included in its parent message only if the IEI 0x2b appears after the mandatory elements. For an incoming message, this means that a valid flag is set to true for this element by the Condat tools, indicating to your code, that the element is present. For an outgoing message, your code need only set the valid flag to true for the element, and the Condat tools will include the IEI and the element in the air interface message.

### Direction

The direction column is allowed in message definition tables only. Its value may be uplink, download, and both. Apart from its informational value, the tells the Condat tools whether to create insert the message in the list of allowed uplink and/or downlink messages.

If a message is defined as uplink, the tools will allow sending the message, but report an error if received. Note that the tools will not recognize the message, if it has the wrong direction.

### Pres

The pres column (short for “presence”) indicates whether elements are mandatory or optional. It is purely informational, i.e. it is not used by the Condat tool chain (it checks the type in stead). The pres column may contain the values optional or mandatory (or any shorthand hereof, including just “O” and “M”). It may appear in message element and structured element tables only, and only once.

### Type

The type column defines the coding type for the element in question. The type column is allowed in air interface message element tables and structured element tables only.

### Length

The length column (which may be shortened to len) defines the length in bytes for the object in question. It is mandatory in basic element definition rows, whereas it is optional (and informational, i.e. not used) in message element table rows, structured element rows, structured element definition rows. In most places, the len column may be replaced by a Bit len column.

For variable length IEs, the length may be expressed as a range, i.e. <min> - <max>, where <min> and <max> may be either a number or a constant.

### Bit len

The bit len column defines the length in bits for the object in question. It may be used in message element table rows, structured element rows, structured element definition rows, and basic element definition rows, where it may replace the Len column.

### Value

The value column binds individual values to C identifiers when paired with a C-Macro column. It is allowed in value tables only.

The value parameter defines the value bound to the C identifier named in the C-Macro column. The value parameter may be entered in decimal, hexadecimal (e.g. 0x0a), or binary (e.g. 0b00001010) notation.

### C-Macro

The C-Macro column binds individual values to C identifiers when paired with a “Value” column. It is allowed in value tables only.

The C-Macro parameter is a C identifier as it may be used in program code later. Be careful concerning name clashes!

## Table Column/Section Combination Matrix

Table 2 below lists the Air Interface Message document sections, and which columns are allowed in the respective sections table.

|  | Active Section / Table |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Constants | Types | Messages |  | Structured elements |  | Basic elements |  |
|  | Definition | Definition | Definition | Element | Definition | Element | Definition | Value |
| name |  |  |  |  |  |  |  |  |
| long name |  |  |  |  |  |  |  |  |
| short name |  |  |  |  |  |  |  |  |
| comment |  |  |  |  |  |  |  |  |
| version |  |  |  |  |  |  |  |  |
| reference / ref |  |  |  |  |  |  |  |  |
| spec ref |  |  |  |  |  |  |  |  |
| link |  |  |  |  |  |  |  |  |
| ctrl |  |  |  |  |  |  |  |  |
| ID |  |  |  |  |  |  |  |  |
| direction |  |  |  |  |  |  |  |  |
| pres / presence |  |  |  |  |  |  |  |  |
| type |  |  |  |  |  |  |  |  |
| length / len |  |  |  |  |  |  |  |  |
| bit len |  |  |  |  |  |  |  |  |
| add bit |  |  |  |  |  |  |  |  |
| value |  |  |  |  |  |  |  |  |
| c-macro |  |  |  |  |  |  |  |  |

Table 2 - List of Columns Allowed in Different AIM Tables

Legend:
		Allowed
		Allowed (with restrictions)
		Not allowed

# Formal Grammar

At some point in time, this section may end up containing a formal grammar of what may be contained in an Air Interface Message document.

So far, only the Ctrl column is specified formally.

## Ctrl column

CtrlColumnContents:
	CtrlInfos

CtrlInfos:
	CtrlInfo
	CtrlInfos  CtrlInfo

CtrlInfo:
	CtrlGroup
	CtrlCondition
	CtrlArray
	CtrlPreamble
	CtrlLength
	CtrlType

CtrlGroup:
	“+”			/* Signals start of multi-octet GSM extended group:  Bit 7 set = present */
	“-“			/* Signals end of multi-octet GSM extended group */
	“*”			/* Signals single octet GSM extended group:  Bit 7 set = present */

CtrlCondition:
	“{“  ConditionalExpression  “}”

CtrlArray:
	DynamicOpt?  “[“  ArrayInfo  “]”

CtrlPreamble:
	“(“  PreambleInfo  “)”

CtrlLength:
	Digits

CtrlType:
	<any word>

ConditionalExpression:
	OperatorExpression
	OperatorExpression  BindingOperator ConditionalExpression

OperatorExpression:
	Identifier  ComparisonExpression  Digits
	Identifier  ComparisonExpression  Constant
	Identifier  ComparisonExpression  Identifier

ComparisonOperator: 
	“=“			/* Comparison for equality */
	“#“			/* Comparison for inequality */
	“<“			/* Less than comparison */
	“>“			/* Greater than comparison */

BindingOperator:
	“A“			/* Logical AND */
	“O“			/* Logical OR */
	“X“			/* Logical XOR (exclusive or) */

PreambleInfo:
	RPExpression
	RPExpression “,” PreambleInfo

RPExpression:			/* Expressions in reverse polish notation */
	Digits			/* Constant to push on stack */
	Identifier		/* Push value of identifier on to stack */
	StackOperator
	NumericOperator

StackOperator:
	“SetPos”			/* Push bit-pointer (in message) on stack */
	“GetPos”		/* Pop bit-pointer (in message) from stack */
	“:”			/* Duplicate top element in stack */
	“^”			/* Swap upper two elements in stack */

NumericOperator:
	“+”			/* Adds upper two elements on stack and leaves result */
	“-”			/* Subtracts upper two elements on stack and leaves result */
	“*”			/* Multiplies upper two elements on stack and leaves result */
	“/”			/* Divides upper two elements on stack and leaves result */
	“&”			/* Performs logical “and” between upper two elements and leaves result */
	“|”			/* Performs logical “or” on upper two elements and leaves result */

DynamicOpt:
	“DYN”			/* Dynamic array specifier – code transparent */
	“PTR”			/* Dynamic array specifier – non-code transparent */

ArrayInfo:
	“.”Min “..” Max		/* Variable size – length specified in bits */
	Min “..” Max		/* Variable size – length specified in bytes */
	“.”Size			/* Constant size – length specified in bits */
	Size			/* Constant size – length specified in bytes */

Min, Max, Size:
	Identifier
	Digits

Digits: 
	Digit
	Digits Digit

Digit:
	“0” “1” “2” “3” “4” “5” “6” “7” “8” “9”

# Example

[FIXME! Refer to some ClearCase based AIM document.]

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |