- GSM Protocol Stack

- Air-Interface Messages

- RR

Author:	Condat DV-Beratung
	Organisation Software GmbH
	Alt-Moabit 91d
	D-10559 Berlin
	Germany

Date:	14-Februar-2001 
Document No.:	6147.604.97.103 
File:	RR.DOC

- Table of Contents

- 0	Document Control	10

0.1	Document History	10

0.2	References	10

0.3	Abbreviations	13

0.4	Terms	14

1	Overview	16

2	Constants	18

3	Types	19

4	Messages	20

4.1	Additional assignment	20

4.2	Assignment command	21

4.3	Assignment complete	22

4.4	Assignment failure	22

4.5	Channel mode modify	23

4.6	Channel mode modify acknowledge	23

4.7	Channel release	24

4.8	Ciphering mode command	24

4.9	Ciphering mode complete	25

4.10	Classmark change	25

4.11	Classmark enquiry	26

4.12	Configuration Change Command	26

4.13	Configuration Change Acknowledge	27

4.14	Configuration Change Reject	27

4.15	Extended Measurement Order	28

4.16	Extended Measurement Report	28

4.17	Frequency redefinition	29

4.18	GPRS suspension request	29

4.19	Handover command	30

4.20	Handover complete	31

4.21	Handover failure	31

4.22	Immediate assignment	32

4.23	Immediate assignment extended	33

4.24	Immediate assignment reject	34

4.25	Measurement report	34

4.26	Notification/NCH	35

4.27	Paging request type 1	35

4.28	Paging request type 2	36

4.29	Paging request type 3	36

4.30	Paging response	37

4.31	Partial release	37

4.32	Partial release complete	38

4.33	PDCH Assignment Command	38

4.34	Physical information	39

4.35	RR-Cell Change Order	39

4.36	RR Initialisation Request	40

4.37	RR Status	40

4.38	System information Type 1	41

4.39	System information Type 2	41

4.40	System information Type 2bis	42

4.41	System information Type 2ter	42

4.42	System information Type 3	43

4.43	System information Type 4	43

4.44	System information Type 5	44

4.45	System information Type 5bis	44

4.46	System information Type 5ter	44

4.47	System information Type 6	45

4.48	System information Type 7	45

4.49	System information Type 8	46

4.50	System information Type 9	46

4.51	System information Type 13	47

4.52	System information Type 16	47

4.53	System information Type 17	48

4.54	Talker Indication	48

4.55	Uplink busy	49

4.56	Uplink release	49

4.57	VGCS uplink grant	50

5	Structured Elements	51

5.1	Alpha Gamma	51

5.2	BA Range	51

5.3	BA List Pref	52

5.4	Cell Description	52

5.5	Cell Options (BCCH)	53

5.6	Cell Options (SACCH)	53

5.7	Cell Selection Parameters	54

5.8	Channel Description	54

5.9	Channel Needed	55

5.10	Channel Needed for Mobile ID 3 and 4	55

5.11	Channel Request Description	56

5.12	Ciphering Key Sequence Number	56

5.13	Cipher Mode Setting	57

5.14	Cipher Response	57

5.15	Codec Properties	57

5.16	Control Channel Description	58

5.17	Descriptive Group Call Reference	58

5.18	Dynamic Allocation	59

5.19	Early Classmark sending Tag	59

5.20	Extended Measurement Results	60

5.21	Extensions Bits IE	60

5.22	Fixed Allocation	61

5.23	Frequency Channel Sequence	61

5.24	Frequency List	62

5.25	Frequency Range	62

5.26	GPRS Cell Options	63

5.27	GPRS Indicator	63

5.28	GPRS Measurement Result	63

5.29	GPRS Mobile Allocation	64

5.30	GPRS Resumption	64

5.31	Group Call Information	65

5.32	Group Channel Description	65

5.33	Group Channel Description 2	66

5.34	Hopping Frequencies	66

5.35	IA Assignment Parameters	67

5.36	IA Frequency Parameters	67

5.37	IA Rest Octets	68

5.38	IA Rest Octet Parameters	68

5.39	IA Second Part Packet Assignment	69

5.40	Info Type and Positions	69

5.41	Location Area Identification	70

5.42	LSA identity	70

5.43	LSA ID Information	71

5.44	LSA Parameters	71

5.45	LSA Parameters and LSA ID	72

5.46	MAC Mode And Channel Coding	72

5.47	Measurement Results	73

5.48	Mobile Allocation	73

5.49	Mobile Identity	74

5.50	Mobile Station Classmark 2	75

5.51	Mobile Station Classmark 3	76

5.52	Mobile Time Difference	76

5.53	MS Measurement Capability	77

5.54	Multi Rate Configuration	77

5.55	Multislot Allocation	78

5.56	NC Mode For Target Cell	78

5.57	Ncell	78

5.58	NLN(PCH) and Status	79

5.59	NT/N Rest Octets	79

5.60	Optional Mobile Country Code	79

5.61	Optional Selection Parameters	80

5.62	P0 and BTS Power Control Mode	80

5.63	P0 and PR Mode	80

5.64	P0 BTS PR Mode	81

5.65	P1 Rest Octets	81

5.66	P2 Rest Octets	82

5.67	P3 Rest Octets	82

5.68	Packet Channel Description	83

5.69	Packet Downlink Assignment (IA)	83

5.70	Packet Downlink Assignment (RR)	84

5.71	Packet Measurement Parameters	84

5.72	Packet Page Indication	85

5.73	Packet Timing Advance	86

5.74	Packet Uplink Assignment (IA)	86

5.75	Packet Uplink Assignment (RR)	87

5.76	Page Mode	87

5.77	PAN Structure	87

5.78	PBCCH Description	88

5.79	PCH and NCH Info	89

5.80	Position	89

5.81	Positions	89

5.82	Power Command	90

5.83	Power Command & Access Type	90

5.84	Power Control Parameters	91

5.85	RACH Control Parameters	91

5.86	Request Reference	92

5.87	Routing Area Identification	92

5.88	Scheduling Info	93

5.89	SI 1 Rest Octets	93

5.90	SI 3 Rest Octets	94

5.91	SI 4 Rest Octets	95

5.92	SI 6 Rest Octets	96

5.93	SI 9 Rest Octets	97

5.94	SI 13 Info	98

5.95	SI 13 Rest Octets	98

5.96	SI 16 Rest Octets	99

5.97	Single Block Allocation	99

5.98	Starting Time	100

5.99	System Information 2ter Indicator	100

5.100	Synchronization Indication	101

5.101	Tagged Gamma	101

5.102	Tagged Priority	101

5.103	Tagged USF For Timeslot	102

5.104	Timing Advance	102

5.105	Timing Advance Index and Number	103

5.106	TFI Assignment and Allocation	103

5.107	TFI Assignment and RLC Mode	104

5.108	USF For Timeslot And Gamma (CSN1)	104

5.109	USF For Timeslot And Gamma	105

5.110	VBS/VGCS Options	105

5.111	VGCS Target Mode Indication	106

5.112	Fixed Allocation Block Structure	106

6	Basic Elements	107

6.1	A5/1	107

6.2	A5/2	107

6.3	A5/3	107

6.4	A5/4	108

6.5	A5/5	108

6.6	A5/6	108

6.7	A5/7	109

6.8	Absolute RF Channel Number	109

6.9	Access Burst Type	109

6.10	Access Control Class	110

6.11	Access Type Control	110

6.12	Acknowledge Flag	110

6.13	ACS	111

6.14	Algorithm Identifier	111

6.15	Alpha	112

6.16	Allocation Bitmap	112

6.17	Allocation Bitmap Length	112

6.18	Allocation Bitmap Var	113

6.19	ARFCN Index	113

6.20	Associated Radio Capability 1	113

6.21	Associated Radio Capability 2	113

6.22	Attach / detach allowed	114

6.23	BA used	114

6.24	Multiband Support	114

6.25	BCCH ARFCN (hi part)	114

6.26	BCCH ARFCN (lo part)	115

6.27	BCCH Change Mark	115

6.28	BCCH Frequency Ncell	115

6.29	BCCH Type	115

6.30	Blocks Or Block Periods	116

6.31	BS Access Grant Blocks reserved	116

6.32	BS Colour Code	116

6.33	BS_CV_MAX	117

6.34	BSIC	117

6.35	BS Paging Multiframes	117

6.36	BTS Power Control Mode	118

6.37	C Value	118

6.38	Call Priority	118

6.39	Call Reestablishment allowed	119

6.40	Cell Bar Qualify	119

6.41	Cell Barred for Access	119

6.42	Cell Channel Description	120

6.43	Cell Identity	120

6.44	Cell Reselect Hysteresis	120

6.45	Cell Reselect Offset	121

6.46	Change Mark 1	122

6.47	Channel Type and TDMA Offset	123

6.48	Channel Needed Info	123

6.49	Channel Mode	124

6.50	Channel Mode 2	125

6.51	Channel Request Description Priority	125

6.52	Channel Set Bitmap	126

6.53	Cipher Response Info	126

6.54	Classmark 3	126

6.55	CMSP Supported	127

6.56	Codec Hysteresis	127

6.57	Codec Threshold	127

6.58	Coding Scheme	128

6.59	Common Control Channel Configuration	128

6.60	CONTROL_ACK_TYPE	129

6.61	Count Of Codec Properties	129

6.62	Dedicated Mode or TBF	129

6.63	Discontinuous Transmission (BCCH)	130

6.64	Discontinuous Transmission (SACCH) (part I)	130

6.65	Discontinuous Transmission (SACCH) (part II)	130

6.66	Discontinuous Transmission used	131

6.67	Downlink Assignment	131

6.68	Downlink Assignment Bitmap	131

6.69	DRX_TIMER_MAX	132

6.70	Early Classmark Sending	132

6.71	Extended Dynamic Allocation	132

6.72	Extended Measurement Capability	133

6.73	Extended Measurement Frequency List	133

6.74	Extension Length	133

6.75	Extension Spare Bits	134

6.76	Final Allocation	134

6.77	Flag	134

6.78	Frequency Capability	135

6.79	Frequency List Contents	135

6.80	Frequency Range Lower	135

6.81	Frequency Range Higher	135

6.82	Frequency Short List	136

6.83	Frequency Short List 2	136

6.84	Gamma	136

6.85	Group Call Reference	137

6.86	Group Cipher Key Number	137

6.87	Half Duplex Mode	137

6.88	Handover Reference	138

6.89	Hopping	138

6.90	Hopping Sequence Number	138

6.91	Identity digit	139

6.92	If And Where	139

6.93	Inband Notifications	139

6.94	Inband Pagings	140

6.95	Increment Skip of ARFCNn	140

6.96	Indirect	140

6.97	Info Type 4	141

6.98	Info Type 5	141

6.99	Initial Codec Mode Indicator	141

6.100	Key Sequence	142

6.101	Length Of Frequency Parameters	142

6.102	LLC Frame Type	142

6.103	Location Area Code	143

6.104	Lowest ARFCN	143

6.105	LSA ID	143

6.106	LSA Offset	144

6.107	MAC Mode	144

6.108	MA Number Indication	144

6.109	Maximum Number of Retransmissions	145

6.110	Measurement Bitmap	145

6.111	Measurement Interval	145

6.112	Measurement valid	146

6.113	Message Type	146

6.114	Mobile Allocation Contents	146

6.115	Mobile Allocation Index Offset	146

6.116	Mobile Allocation Length	147

6.117	Mobile Country Code	147

6.118	Mobile Network Code	147

6.119	Modulus	147

6.120	MS TXPWR MAX CCH	148

6.121	MT/MO Origination type	148

6.122	Multiband Reporting	148

6.123	Multi Rate Speech Version	149

6.124	Multi Slot Class	149

6.125	N_AVG_I Value	149

6.126	NC Mode	150

6.127	NCC Permitted	150

6.128	NCH Position	151

6.129	Neighbour Cells Description	151

6.130	NECI	152

6.131	Network Control Order	152

6.132	Network Mode of Operation	153

6.133	Notification List Number	153

6.134	Notification List Number Status	153

6.135	Normal cell indication	154

6.136	Number of Ranges	154

6.137	Number of Cells	154

6.138	Odd/ Even indication	155

6.139	P0	155

6.140	Packet Channel Type	155

6.141	Paging Channel Restructuring	156

6.142	Page Mode	156

6.143	PAN_DEC	156

6.144	PAN_INC	157

6.145	PAN_MAX	157

6.146	Pb	157

6.147	PC_MEAS_CHAN	158

6.148	Penalty Time	158

6.149	PLMN Colour Code	158

6.150	Polling	159

6.151	Power Control	159

6.152	Power Level	159

6.153	Power Offset	160

6.154	PR Mode	161

6.155	Priority Access Threshold	161

6.156	Priority Threshold	161

6.157	Pseudo Synchronisation Capability	162

6.158	PSI1 Repeat Period	162

6.159	RA Color	162

6.160	Radio Link Timeout	163

6.161	Random Access Information	163

6.162	Relative Position	163

6.163	Report Observed Time Difference	164

6.164	Requested Bandwidth	164

6.165	Resumption ACK	164

6.166	Revision Level	165

6.167	RFL Number	165

6.168	RF power capability	165

6.169	RLC Data Blocks Granted	166

6.170	RLC Mode	166

6.171	RLC Octet Count	166

6.172	Routing Area Code	167

6.173	RR Cause	167

6.174	R Support	168

6.175	RX Level Access Minimum	168

6.176	RX Level Full Serving Cell	168

6.177	RX Level Sub Serving Cell	168

6.178	RX Level Ncell	169

6.179	RX Quality Full Serving Cell	169

6.180	RX Quality Sub Serving Cell	169

6.181	Sequence Code Used	169

6.182	Service Flag	170

6.183	Set of AMR Codec Modes	170

6.184	ShortLSA ID	170

6.185	Short Message Capability	171

6.186	SI 13 Position	171

6.187	SI13 Change Mark	171

6.188	SI Change Field	172

6.189	Signal Variance	172

6.190	SoLSA Exclusive Access	172

6.191	SoLSA Supported	173

6.192	Split PG Cycle Support	173

6.193	SS Screening Indicator	173

6.194	Start Ciphering	174

6.195	Start Mode	174

6.196	Suspension Cause	175

6.197	Switch Measurement Time	175

6.198	Synchronisation indication	176

6.199	T1'	176

6.200	T2	176

6.201	T3	176

6.202	T3122 time-out value	177

6.203	T3168 timeout value	177

6.204	T3192 timeout value	177

6.205	T3212 time-out value	178

6.206	T_AVG_T Value	178

6.207	T_AVG_W Value	178

6.208	Target Mode	178

6.209	Temporary Logical Link Identifier	179

6.210	Temporary Offset	179

6.211	TFI Assignment	179

6.212	Time Difference	180

6.213	Time Difference	180

6.214	Time Slot	180

6.215	Timeslot Allocation	181

6.216	Timing Advance Index	181

6.217	Timing Advance Timeslot Number	181

6.218	Timing Advance Valid	182

6.219	Timing Advance value	182

6.220	TLLI Block Channel Coding	182

6.221	TMSI	183

6.222	Training Sequence Code	183

6.223	Two-message Assignment	183

6.224	Tx-integer	184

6.225	Type of identity	184

6.226	UCS2 Treatment	185

6.227	Uplink State Flag	186

6.228	USF Granularity	187

6.229	VBS Notification Reception	188

6.230	VGCS Notification Reception	189

# Document Control

ã Copyright Condat DV-Beratung Organisation und Software GmbH, 1997.

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Condat GmbH reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Condat GmbH. Condat GmbH accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a licence agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Condat GmbH.

- Condat DV-Beratung

Organisation und Software GmbH

Alt Moabit 91d

10559 Berlin

Germany

- Telephone:	+49.30.39094-0

Fax:	+49.30.39094-300

Internet:	http://www.condat.de

E-mail:	gsm@condat.de

## Document History

Document Id.	Date	Author	Remarks

6147.604.97.100	07-May-97	VK	Initial
6147.604.97.101	29-Oct-97	LE	rest octets
6147.604.97.102	03-Feb-98	VK	ARFCN in cell_desc
6147.604.97.103	11-Aug-98	VK	Values for Screening Indicator added

## References

[1]	Digital celluar telecommunications system (Phase 2+);
	Mobile radio interface layer 3 specification
	(GSM 4.08, December 1995, version 5.0.0)

[1a]	GSM 2.81, Line Identification Supplementary Services - Stage 1
	ETS 300 514, ETSI, September 1994

[2]	GSM 2.82, Call Forwarding Supplementary Services - Stage 1
	ETS 300 515, ETSI, September 1994

[3]	GSM 2.83, Call Waiting and Call Hold Supplementary Services - Stage 1
	ETS 300 516, ETSI, September 1994

[4]	GSM 2.84, Multi Party Supplementary Services - Stage 1
	ETS 300 517, ETSI, September 1994

[5]	GSM 2.85, Closed User Group Supplementary Services - Stage 1
	ETS 300 518, ETSI, September 1994

[6]	GSM 2.86, Advice of Charge Supplementary Services - Stage 1
	ETS 300 519, ETSI, September 1994

[7]	GSM 2.88, Call Barring Supplementary Services - Stage 1
	ETS 300 520, ETSI, September 1994

[8]	GSM 3.14, Support of Dual Tone Multi Frequency Signalling via the GSM System
	ETS 300 532, ETSI, April 1994

[9]	GSM 3.40, Technical Realization of the Short Message Service Point-to-Point
	ETS 300 536, ETSI, January 1996

[10]	GSM 3.41, Technical Realization of Short Message Service Cell Broadcast
	ETS 300 537, ETSI, June 1995

[11]	GSM 3.81, Line Identification Supplementary Services - Stage 2
	ETS 300 542, ETSI, February 1995

[12]	GSM 3.82, Call Forwarding Supplementary Services - Stage 2
	ETS 300 543, ETSI, February 1995

[13]	GSM 3.83, Call Waiting and Call Hold Supplementary Services - Stage 2
	ETS 300 544, ETSI, November 1994

[14]	GSM 3.84, Multi Party Supplementary Services - Stage 2
	ETS 300 545, ETSI, November 1994

[15]	GSM 3.85, Closed User Group Supplementary Services - Stage 2
	ETS 300 546, ETSI, January 1996

[16]	GSM 3.86, Advice of Charge Supplementary Services - Stage 2
	ETS 300 547, ETSI, March 1995

[17]	GSM 3.88, Call Barring Supplementary Services - Stage 2
	ETS 300 548, ETSI, November 1994

[18]	GSM 4.01, MS-BSS Interface General Aspects and Principles
	ETS 300 550, ETSI, September 1994

[18a]	GSM 4.03, MS-BSS Interface Channel Structures and Access Capabilities
	ETS 300 552, ETSI, September 1994

[19]	GSM 4.05, Data Link Layer General Aspects
	ETS 300 554, ETSI, September 1994

[20]	GSM 4.06, MS-BSS Interface Data Link Layer Specification
	ETS 300 555, ETSI, September 1994

[21]	GSM 4.07, Mobile Radio Interface Signalling Layer 3 General Aspects
	ETS 300 556, ETSI, February 1995

[22]	GSM 4.08, Mobile Radio Interface Layer 3 Specification
	ETS 300 557, ETSI, January 1996

[23]	GSM 4.10, Mobile Radio Interface Layer 3 Supplementary Services Specification 
	General Aspects
	ETS 300 558, ETSI, February 1995

[24]	GSM 4.11, Point-to-Point Short Message Service Support on Mobile Radio Interface
	ETS 300 559, ETSI, October 1995

[25]	GSM 4.12, Short Message Service Cell Broadcast Support on Mobile Radio Interface
	ETS 300 560, ETSI, January 1996

[26]	GSM 4.80, Mobile Radio Interface Supplementary Services Specification Formats and Coding
	ETS 300 564, ETSI, February 1995

[27]	GSM 4.81, Line Identification Supplementary Services - Stage 3
	ETS 300 565, ETSI, February 1995

[28]	GSM 4.82, Call Forwarding Supplementary Services - Stage 3
	ETS 300 566, ETSI, February 1995

[29]	GSM 4.83, Call Waiting and Call Hold Supplementary Services - Stage 3
	ETS 300 567, ETSI, February 1995

[30]	GSM 4.84, Multi Party Supplementary Services - Stage 3
	ETS 300 568, ETSI, February 1995

[31]	GSM 4.85, Closed User Group Supplementary Services - Stage 3
	ETS 300 569, ETSI, February 1995

[32]	GSM 4.86, Advice of Charge Supplementary Services - Stage 3
	ETS 300 570, ETSI, February 1995

[33]	GSM 4.88, Call Barring Supplementary Services - Stage 3
	ETS 300 571, ETSI, February 1995

[34]	GSM 5.01, Physical Layer on the Radio Path General Description
	ETS 300 573, ETSI, October 1995

[35]	GSM 5.02, Multiplexing and Multiple Access on the Radio Path
	ETS 300 574, ETSI, January 1996

[36]	GSM 5.08, Radio Sub-system Link Control
	ETS 300 578, ETSI, January 1996

[37]	GSM 5.10, Radio Sub-system Synchronisation
	ETS 300 579, ETSI, October 1995

[38]	Service Access Point MMREG
	6147.100.96.100; Condat GmbH

[39]	Service Access Point MNCC
	6147.101.96.100; Condat GmbH

[40]	Service Access Point MNSS
	6147.102.96.100; Condat GmbH

[41]	Service Access Point MNSMS
	6147.103.96.100; Condat GmbH

[42]	Service Access Point MMCC
	6147.104.97.100; Condat GmbH

[43]	Service Access Point MMSS
	6147.105.97.100; Condat GmbH

[44]	Service Access Point MMSMS
	6147.106.97.100; Condat GmbH

[45]	Service Access Point RR
	6147.107.97.100; Condat GmbH

[46]	Service Access Point SIM
	6147.108.97.100; Condat GmbH

[47]	Service Access Point MPH
	6147.109.96.100; Condat GmbH

[48]	Service Access Point DL
	6147.110.96.100; Condat GmbH

[49]	Service Access Point MDL
	6147.111.96.100; Condat GmbH

[50]	Service Access Point PH
	6147.112.97.100; Condat GmbH

[51]	Service Access Point MMI
	6147.113.96.100; Condat GmbH

[52]	Message Sequence Charts CC
	6147.200.97.100; Condat GmbH

[53]	Message Sequence Charts SS
	6147.201.97.100; Condat GmbH

[54]	Message Sequence Charts SMS
	6147.202.97.100; Condat GmbH

[55]	Message Sequence Charts MM
	6147.203.97.100; Condat GmbH

[56]	Message Sequence Charts RR
	6147.204.96.100; Condat GmbH

[57]	Message Sequence Charts DL
	6147.205.96.100; Condat GmbH

[58]	Users Guide
	6147.300.96.100; Condat GmbH

[59]	Test Specification CC
	6147.400.97.100; Condat GmbH

[60]	Test Specification SS
	6147.401.97.100; Condat GmbH

[61]	Test Specification SMS
	6147.402.97.100; Condat GmbH

[62]	Test Specification MM
	6147.403.97.100; Condat GmbH

[63]	Test Specification RR
	6147.404.97.100; Condat GmbH

[64]	Test Specification DL
	6147.405.97.100; Condat GmbH

[65]	Test Specification CCD
	6147.406.97.100; Condat GmbH

[66]	SDL Specification CC
	6147.500.97.100; Condat GmbH

[67]	SDL Specification SS
	6147.501.97.100; Condat GmbH

[68]	SDL Specification SMS
	6147.502.97.100; Condat GmbH

[69]	SDL Specification MM
	6147.503.97.100; Condat GmbH

[70]	SDL Specification RR
	6147.504.97.100; Condat GmbH

[71]	SDL Specification DL
	6147.505.97.100; Condat GmbH

[72]	Message Specification CC
	6147.600.97.100; Condat GmbH

[73]	Message Specification SS
	6147.601.97.100; Condat GmbH

[74]	Message Specification SMS
	6147.602.97.100; Condat GmbH

[75]	Message Specification MM
	6147.603.97.100; Condat GmbH

[76]	Message Specification RR
	6147.604.97.100; Condat GmbH

[77]	Message Specification DL
	6147.605.97.100; Condat GmbH

[78]	Technical Documentation CC
	6147.700.97.100; Condat GmbH

[79]	Technical Documentation SS
	6147.701.97.100; Condat GmbH

[80]	Technical Documentation SMS
	6147.702.97.100; Condat GmbH

[81]	Technical Documentation MM
	6147.703.97.100; Condat GmbH

[82]	Technical Documentation RR
	6147.704.97.100; Condat GmbH

[83]	Technical Documentation DL
	6147.705.97.100; Condat GmbH

[84]	Technical Documentation CCD
	6147.706.97.100; Condat GmbH

## Abbreviations

AGCH	Access Grant Channel

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
C/R	Command / Response
C1	Path Loss Criterion
C2	Reselection Criterion

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station

NCC	National Colour Code
NECI	New Establishment Causes included
N(R)	Receive Number
N(S)	Send Number

OTD	Observed Time Difference

P	Poll Bit
PCH	Paging Channel
PDU	Protocol Description Unit
P/F	Poll / Final Bit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

VPLMN	Visiting Public Land Mobile Network
V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages. Additionally the reception of cell broadcast short messages is included.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

The GSM specification do not set out any implementation of the Protocol Stack. The following diagrams show the implementation described in all these documents for the mobile station. All entities except the Man Machine Interface and Physical Layer are implemented as part of the Protocol Stack.

- Figure 1: Mobile-station protocol architecture

- This document describes the messages processed by the Radio Resource entity.

# Constants

Definition:

| name | value | comment |
| --- | --- | --- |
| L3MAX_ACK | 251 | GSM 4.06, section 5.8.5 |
| L3MAX | 251 |  |
| MAX_ARFCN | 20 | (L3MAX_ACK-3-3) * 8 / (10 + 1) =178 |
| MAX_ARFCNI | 5 |  |
| MAX_GR_C_INFO | 5 |  |
| MAX_ITP | 5 |  |
| MAX_LSA_ID | 2 |  |
| MAX_POS | 5 |  |
| MAX_RANGE | 10 | (L3MAX_ACK-3-3) * 8 / (2 * 10 + 1)=93 |
| MAX_RFL | 5 |  |
| N_MAX_RANGE | 30 | (L3MAX_ACK-3-3) * 8 / (2 * 10) = 98 |
| N_CELL_MRES | 6 |  |
| N_FREQ_SHORT_LIST | 9 | sizeof(IE) - sizeof(IEI) |
| N_FREQ_SKIP | 16 |  |
| N_MAX_FREQ_LIST | 250 | L3MAX - sizeof(IEI) |
| N_MOB_ALLOC | 8 |  |

History:

07-May-97	VK	Initial

05-Apr-00	LG	added MAX_ARFCN, MAX_ARFCNI, MAX_CR_C_INFO, MAXITP,       		MAX_LSA_ID, MAX_POS, MAX_RANGE, MAX_RFL, changed value for 		N_MAX_RANGE

# Types

Definition:

| name | add bit | ctrl | comment |
| --- | --- | --- | --- |
| GSM1_V | 0 |  | mandatory IE V-component in one nibble |
| GSM1_TV | 4 | optional | optional with V-Component |
| GSM2_T | 8 | optional | optional IE, contains IEI only (no V-component) |
| GSM3_V | 0 |  | mandatory / conditional, no IEI, V-component only |
| GSM3_TV | 8 | optional | optional IE with V-component |
| GSM4_LV | 8 |  | mandatory / conditional, length and V-component |
| GSM4_TLV | 16 | optional | optional IE with length indicator and V-component |
| GSM5_V | 0 | optional | optional IE (bitstream to end of message) |
| BCDODD | 4 |  | binary coded decimal number starting with digit1 |
| BCDEVEN | 4 |  | binary coded decimal number starting with digit2 |
| CSN1_S1 | 1 | optional | CSN1 coded value with 0/1 tag |
| CSN1_SHL | 1 | optional | CSN1 coded value with L/H tag |
| S_PADDING | 0 | optional | spare padding |

History:

07-May-97	VK	Initial

16-Oct-00	ID	S_PADDING optional

# Messages

## Additional assignment

Description:

This message is sent on the main DCCH by the network to the mobile station to allocate an additional dedicated channel while keeping the previously allocated channels.
Reference : [GSM_04.08] section 9.1.1

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Additional assignment | d_add_assign | 0b00111011 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Channel Description | chan_desc | 5.8 | 10.5.2.5 | M | V | 3 |
| 0x72 | Mobile Allocation | mob_alloc | 5.48 | 10.5.2.21 | C | TLV | 3-10 |
| 0x7C | Starting Time | start_time | 5.98 | 10.5.2.38 | O | TV | 3 |

History:

07-May-97	VK	Initial

## Assignment command

Description:

This message is sent on the main DCCH by the network to the mobile station to change the channel configuration to another independent dedicated channel configuration, when no timing adjustment is needed.
Reference : [GSM_04.08] section 9.1.2

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Assignment command | d_assign_cmd | 0b00101110 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Description of the First Channel, after time | chan_desc | 5.8 | 10.5.2.5 | M | V | 3 |
|  | Power Command | pow_cmd | 5.82 | 10.5.2.28 | M | V | 1 |
| 0x05 | Frequency List, after time | freq_list_after | 5.24 | 10.5.2.13 | C | TLV | 4-132 |
| 0x62 | Cell Channel Description | cell_chan_desc | 6.42 | 10.5.2.1b | O | TV3 | 17 |
| 0x10 | Multislot Configuration | multislot_alloc | 5.55 | 10.5.2.21b | C | TLV | 3-12 |
| 0x63 | Mode of the 1st Channel | chan_mode | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x11 | Mode of the 2nd Channel Set | chan_mode2 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x13 | Mode of the 3rd Channel Set | chan_mode3 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x14 | Mode of the 4th Channel Set | chan_mode4 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x15 | Mode of the 5th Channel Set | chan_mode5 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x16 | Mode of the 6th Channel Set | chan_mode6 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x17 | Mode of the 7th Channel Set | chan_mode7 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x18 | Mode of the 8th Channel Set | chan_mode8 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x64 | Description of the 2nd Channel, after time | chan_desc_after_2 | 5.8 | 10.5.2.5 | O | TV | 4 |
| 0x66 | Mode of the 2nd Channel | chan_mode_2 | 6.50 | 10.5.2.7 | O | TV3 | 2 |
| 0x72 | Mobile Allocation, after time | mob_alloc_after | 5.48 | 10.5.2.21 | C | TLV | 3-10 |
| 0x7C | Starting Time | start_time | 5.98 | 10.5.2.38 | O | TV | 3 |
| 0x19 | Frequency List, before time | freq_list_before | 5.24 | 10.5.2.13 | C | TLV | 4-132 |
| 0x1C | Description of the 1st Channel, before time | chan_desc_before | 5.8 | 10.5.2.5 | O | TV | 4 |
| 0x1D | Description of the 2nd Channel, before time | chan_desc_before_2 | 5.8 | 10.5.2.5 | O | TV | 4 |
| 0x1E | Frequency channel sequence before time | freq_chan_seq | 5.23 | 10.5.2.12 | C | TV | 10 |
| 0x21 | Mobile Allocation, before time | mob_alloc_before | 5.48 | 10.5.2.21 | C | TLV | 3-10 |
| 0x09 | Cipher Mode Setting | ciph_mode_set | 5.13 | 10.5.2.9 | O | TV | 1 |
| 0x01 | VGCS Target Mode Indication | vgcs_tmi | 5.111 | 10.5.2.42a | O | TLV | 3 |
| 0x03 | Multi Rate Configuration | multirate_conf | 5.54 | 10.5.2.21aa | O | TLV | 4-8 |

History:

07-May-97	VK	Initial
14-Apr-00	LG	conform to V7.1.2

## Assignment complete

Description:

This message is sent on the main DCCH from the mobile station to the network to indicate that the mobile station has established the main signaling link successfully.
Reference : [GSM_04.08] section 9.1.3

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Assignment complete | u_assign_comp | 0b00101001 | uplink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | RR Cause | rr_cause | 6.173 | 10.5.2.31 | M | V3 | 1 |

History:

07-May-97	VK	Initial
28-Jul-97	LE	short name is u_assign_comp instead of d_assign_comp

## Assignment failure

Description:

This message is sent on the main DCCH on the old channel from the mobile station to the network to indicate that the mobile station has failed to seize the new channel.
Reference : [GSM_04.08] section 9.1.4

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Assignment failure | u_assign_fail | 0b00101111 | uplink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | RR cause | rr_cause | 6.173 | 10.5.2.31 | M | V3 | 1 |

History:

07-May-97	VK	Initial
28-Jul-97	LE	short name is u_assign_fail instead of d_assign_fail

## Channel mode modify

Description:

This message is sent on the main DCCH by the network to the mobile station to request the setting of the mode for the indicated channel.
Reference : [GSM_04.08] section 9.1.5

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Channel mode modify | d_chan_mod | 0b00010000 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Channel Description | chan_desc | 5.8 | 10.5.2.5 | M | V | 3 |
|  | Channel Mode | chan_mode | 6.49 | 10.5.2.6 | M | V3 | 1 |
| 0x01 | VGCS Target Mode Indication | vgcs_tmi | 5.111 | 10.5.2.42a | O | TLV | 3 |
| 0x03 | Multi-Rate Configuration | multirate_conf | 5.54 | 10.5.2.21aa | O | TLV | 4-8 |

History:

07-May-97	VK	Initial

17-Apr-00	LG	conform to V7.1.2

## Channel mode modify acknowledge

Description:

This message is sent on the main DCCH by the mobile station to the network to indicate the successful or unsuccessful execution of a channel mode modify request.
Reference : [GSM_04.08] section 9.1.6

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Channel mode modify acknowledge | u_chan_mod_ack | 0b00010111 | uplink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Channel Description | chan_desc | 5.8 | 10.5.2.5 | M | V | 3 |
|  | Channel Mode | chan_mode | 6.49 | 10.5.2.6 | M | V3 | 1 |

History:

07-May-97	VK	Initial

## Channel release

Description:

This message is sent on the main DCCH from the network to the mobile station to initiate deactivation of the dedicated channel used.
Reference : [GSM_04.08] section 9.1.7

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Channel release | d_chan_rel | 0b00001101 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | RR Cause | rr_cause | 6.173 | 10.5.2.31 | M | V3 | 1 |
| 0x73 | BA Range | ba_range | 5.2 | 10.5.2.1 | O | TLV | 6-* |
| 0x74 | Group Channel Description | group_chan_desc | 5.32 | 10.5.2.14b | O | TLV4 | 4-13 |
| 0x08 | Group Cipher Key Number | group_ckn | 6.86 | 10.5.1.10 | C | TV3 | 1 |
| 0x0C | GPRS Resumption | gprs_resum | 5.30 | 10.5.2.14c | O | TV | 1 |
| 0x75 | BA List Pref | ba_list_pref | 5.3 | 10.5.2.1c | O | TLV | 6-* |

History:

07-May-97	VK	Initial

17-Apr-00	LG	conform to V7.1.2

## Ciphering mode command

Description:

This message is sent on the main DCCH from the network to the mobile station to indicate that the network has started deciphering and that enciphering and deciphering shall be started in the mobile station, or to indicate that ciphering will not be performed.
Reference : [GSM_04.08] section 9.1.9

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Ciphering mode command | d_ciph_cmd | 0b00110101 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Ciphering Mode Setting | ciph_mode_set | 5.13 | 10.5.2.9 | M | V | 1/2 |
|  | Cipher Response | ciph_res | 5.14 | 10.5.2.10 | M | V | 1/2 |

History:

07-May-97	VK	Initial

## Ciphering mode complete

Description:

This message is sent on the main DCCH from the mobile station to the network to indicate that enciphering and deciphering has been started in the mobile station.
Reference : [GSM_04.08] section 9.1.10

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Ciphering mode complete | u_ciph_comp | 0b00110010 | uplink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
| 0x17 | Mobile Equipment Identity | mob_ident | 5.49 | 10.5.1.4 | O | TLV | 3-11 |

History:

07-May-97	VK	Initial

## Classmark change

Description:

This message is sent on the main DCCH by the mobile station to the network to indicate a classmark change or as a response to a classmark enquiry.
Reference : [GSM_04.08] section 9.1.11

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Classmark change | u_class_chng | 0b00010110 | uplink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Mobile Station Classmark | mob_class_2 | 5.50 | 10.5.1.6 | M | LV | 4 |
| 0x20 | Additional Mobile Station Classmark Information | mob_class_3 | 5.51 | 10.5.1.7 | C | TLV | 3-14 |

History:

07-May-97	VK	Initial

17-Apr-00	LG	conform to V7.1.2

## Classmark enquiry

Description:

This message is sent on the main DCCH by the network to the mobile station to request classmark information.
Reference : [GSM_04.08] section 9.1.12

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Classmark enquiry | d_class_enq | 0b00010011 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |

History:

07-May-97	VK	Initial

## Configuration Change Command

Description:

This message is sent on the main DCCH by the network to the mobile station to change the channel configuration of a multi slot configuration.

Reference : [GSM_04.08] section 9.1.12b

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Configuration Change Command | d_conf_change_cmd | 0b00110000 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Multislot Configuration | multislot_alloc | 5.55 | 10.5.2.21b | M | TLV | 2-11 |
| 0x63 | Mode of the 1st Channel Set | chan_mode | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x11 | Mode of the 2nd Channel Set | chan_mode2 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x13 | Mode of the 3rd Channel Set | chan_mode3 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x14 | Mode of the 4th Channel Set | chan_mode4 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x15 | Mode of the 5th Channel Set | chan_mode5 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x16 | Mode of the 6th Channel Set | chan_mode6 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x17 | Mode of the 7th Channel Set | chan_mode7 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x18 | Mode of the 8th Channel Set | chan_mode8 | 6.49 | 10.5.2.6 | O | TV3 | 2 |

History:

17-Apr-00	LG	Initial

## Configuration Change Acknowledge

Description:

This message is sent on the main DCCH from the mobile station to the network to indicate that the mobile station has changed to the ordered channel configuration successfully.

Reference : [GSM_04.08] section 9.1.12c

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Configuration Change Acknowledge | d_conf_change_ack | 0b00110001 | uplink |

Elements:

| ID | long name | short name | Ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |

History:

17-Apr-00	LG	Initial

## Configuration Change Reject

Description:

This message is sent on the main DCCH from the mobile station to the network to indicate that the mobile station has not managed to switch to the channel configuration ordered by the configuration change command and is still using the previous configuration.

Reference : [GSM_04.08] section 9.1.12d

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Configuration Change Reject | d_config_change_rej | 0b00110011 | uplink |

Elements:

| ID | long name | short name | Ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | RR Cause | rr_cause | 6.173 | 10.5.2.31 | M | V3 | 1 |

History:

17-Apr-00	LG	Initial

## Extended Measurement Order

Description:

This message is sent on the SACCH by the network to order the mobile station to send one extended measurement report.
Reference : [GSM_04.08] section 9.1.51

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Extended Measurement Order | d_ext_meas_order | 0b00110111 | downlink |

Elements:

| ID | long name | Short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Extended Measurement Frequency List | ext_meas_freq | 6.73 | 10.5.2.46 | M | V3 | 16 |

History:

19-Apr-00	LG	Initial

## Extended Measurement Report

Description:

This message is sent on the SACCH by the mobile station to the network to report extended measurement results.
Reference : [GSM_04.08] section 9.1.5

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Extended Measurement Report | u_ext_meas_report | 0b00110110 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Extended Measurement Results | ext_meas_res | 5.20 | 10.5.2.45 | M | V | 16 |

History:

19-Apr-00	LG	Initial

## Frequency redefinition

Description:

This message is sent on the main DCCH from the network to the mobile station to indicate that the frequencies and the hopping sequence of the allocated channels shall be changed.
Reference : [GSM_04.08] section 9.1.13

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Frequency redefinition | d_freq_redef | 0b00010100 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Channel Description | chan_desc | 5.8 | 10.5.2.5 | M | V | 3 |
|  | Mobile Allocation | mob_alloc | 5.48 | 10.5.2.21 | M | LV | 1-9 |
|  | Starting Time | start_time | 5.98 | 10.5.2.38 | M | V | 2 |
| 0x62 | Cell Channel Description | cell_chan_desc | 6.42 | 10.5.2.1b | O | TV3 | 17 |

History:

07-May-97	VK	Initial

## GPRS suspension request

Description:

This message is sent on the main DCCH by the mobile station to request a suspension of the GPRS services.
Reference : [GSM_04.08] section 9.1.13b

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| GPRS suspension request | u_gprs_susp_req | 0b00110100 | uplink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Temporary Logical Link Identifier | ded_tlli | 6.209 | 10.5.2.41a | M | V3 | 4 |
|  | Routing Area Identification | rout_area_id | 5.87 | 10.5.5.15 | M | V | 6 |
|  | Suspension Cause | susp_cause | 6.196 | 10.5.2.47 | M | V3 | 1 |

History:

19-Apr-00	LG	Initial

## Handover command

Description:

This message is sent on the main DCCH by the network to the mobile station to change the dedicated channel configuration, timing adjustment needed.
Reference : [GSM_04.08] section 9.1.15

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Handover command | d_handov_cmd | 0b00101011 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Cell Description | cell_desc | 5.4 | 10.5.2.2 | M | V | 2 |
|  | Description of 1st channel, after time | chan_desc_after | 5.8 | 10.5.2.5 | M | V | 3 |
|  | Handover Reference | handov_ref | 6.88 | 10.5.2.15 | M | V3 | 1 |
|  | Power Command and Access type | pow_cmd_access | 5.83 | 10.5.2.28a | M | V | 1 |
| 0x0D | Synchronization Indication | synch_ind | 5.100 | 10.5.2.39 | O | TV | 1 |
| 0x02 | Frequency Short List, after time | freq_short_list_after | 6.82 | 10.5.2.14 | C | TV3 | 10 |
| 0x05 | Frequency List, after time | freq_list_after | 5.24 | 10.5.2.13 | C | TLV | 4-131 |
| 0x62 | Cell Channel Description | cell_chan_desc | 6.42 | 10.5.2.1b | C | TV3 | 17 |
| 0x63 | Mode of the First Channel | chan_mode | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x11 | Mode of the 2nd Channel Set | chan_mode2 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x13 | Mode of the 3rd Channel Set | chan_mode3 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x14 | Mode of the 4th Channel Set | chan_mode4 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x15 | Mode of the 5th Channel Set | chan_mode5 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x16 | Mode of the 6th Channel Set | chan_mode6 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x17 | Mode of the 7th Channel Set | chan_mode7 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x18 | Mode of the 8th Channel Set | chan_mode8 | 6.49 | 10.5.2.6 | O | TV3 | 2 |
| 0x64 | Description of 2nd Channel, after time | chan_desc_after_2 | 5.8 | 10.5.2.5 | O | TV | 4 |
| 0x66 | Mode of the 2nd Channel | chan_mode_2 | 6.50 | 10.5.2.7 | O | TV3 | 2 |
| 0x69 | Freq. Channel Sequence, after time | freq_chan_seq_after | 5.23 | 10.5.2.12 | C | TV | 10 |
| 0x72 | Mobile Allocation, after time | mob_alloc_after | 5.48 | 10.5.2.21 | C | TLV | 3-10 |
| 0x7C | Starting Time | start_time | 5.98 | 10.5.2.38 | O | TV | 3 |
| 0x7B | Real Time Difference | time_diff | 6.213 | 10.5.2.41 | C | TLV4 | 3 |
| 0x7D | Timing Advance | time_advance | 5.104 | 10.5.2.40 | C | TV | 2 |
| 0x12 | Frequency Short List, before time | freq_short_list_before | 6.82 | 10.5.2.14 | C | TV3 | 10 |
| 0x19 | Frequency List, before time | freq_list_before | 5.24 | 10.5.2.13 | C | TLV | 4-131 |
| 0x1C | Description of 1st Channel, before time | chan_desc_before | 5.8 | 10.5.2.5 | O | TV | 4 |
| 0x1D | Description of 2nd Channel, before time | chan_desc_before_2 | 5.8 | 10.5.2.5 | O | TV | 4 |
| 0x1E | Freq. channel sequence before time | freq_chan_seq_before | 5.23 | 10.5.2.12 | C | TV | 10 |
| 0x21 | Mobile Allocation, before time | mob_alloc_before | 5.48 | 10.5.2.21 | C | TLV | 3-10 |
| 0x09 | Cipher Mode Setting | ciph_mode_set | 5.13 | 10.5.2.9 | O | TV | 1 |
| 0x01 | VGCS Target Mode Indication | vgcs_tmi | 5.111 | 10.5.2.42a | O | TLV | 3 |
| 0x03 | Multi Rate Configuration | multirate_conf | 5.54 | 10.5.2.21aa | O | TLV | 4-8 |

History:

07-May-97	VK	Initial

19-Apr-00	LG	conform to V7.1.2

## Handover complete

Description:

This message is sent on the main DCCH from the mobile station to the network to indicate that the mobile station has established the main signalling link successfully.
Reference : [GSM_04.08] section 9.1.16

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Handover complete | u_handov_comp | 0b00101100 | uplink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | RR Cause | rr_cause | 6.173 | 10.5.2.31 | M | V3 | 1 |
| 0x77 | Mobile Observed Time Difference | mob_time_diff | 5.52 | 10.5.2.21a | O | TLV4 | 5 |

History:

07-May-97	VK	Initial

## Handover failure

Description:

This message is sent on the main DCCH on the old channel from the mobile station to the network to indicate that the mobile station has failed to seize the new channel.
Reference : [GSM_04.08] section 9.1.17

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Handover failure | u_handov_fail | 0b00101000 | uplink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | RR Cause | rr_cause | 6.173 | 10.5.2.31 | M | V3 | 1 |

History:

07-May-97	VK	Initial

## Immediate assignment

Description:

This message is sent on the CCCH by the network to the mobile station in idle mode to change the channel configuration to a dedicated configuration while staying in the same cell.
Reference : [GSM_04.08] section 9.1.18

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Immediate assignment | d_imm_assign | 0b00111111 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len | ctrl |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |  |
|  | Dedicated Mode Or TBF Spare | .0 |  | 10.5.2.25b | M | V | 1/8 |  |
|  | Two-message Assignment | tma | 6.223 | 10.5.2.25b | M |  | 1/8 |  |
|  | Downlink Assignment | dl | 6.67 | 10.5.2.25b | M |  | 1/8 |  |
|  | Dedicated Mode or TBF | d_t | 6.62 | 10.5.2.25b | M |  | 1/8 |  |
|  | Page Mode | page_mode | 5.76 | 10.5.2.26 | M | V | 1/2 |  |
|  | Channel Description | chan_desc | 5.8 | 10.5.2.5 | C | V | 3 | {d_t=0} |
|  | Packet Channel Description | pck_chan_desc | 5.68 | 10.5.2.25a | C | V | 3 | {d_t=1} |
|  | Request Reference | req_ref | 5.86 | 10.5.2.30 | M | V | 3 |  |
|  | Timing Advance | time_advance | 5.104 | 10.5.2.40 | M | V | 1 |  |
|  | Mobile Allocation | mob_alloc | 5.48 | 10.5.2.21 | M | LV | 1-9 |  |
| 0x7C | Starting Time | start_time | 5.98 | 10.5.2.38 | O | TV | 3 |  |
|  | IA Rest Octets (freq. parameters, before time) | ia_rest_oct | 5.37 | 10.5.2.16 | M |  | 0-11 |  |

History:

07-May-97	VK	Initial

19-Apr-00	LG	conform to V7.1.2

## Immediate assignment extended

Description:

This message is sent on the CCCH by the network to two mobile stations in idle mode to change their channel configurations to different dedicated configurations while they stay in the same cell.
Reference : [GSM_04.08] section 9.1.19

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Immediate assignment extended | d_imm_assign_ext | 0b00111001 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len | ctrl |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |  |
|  | Page Mode | page_mode | 5.76 | 10.5.2.26 | M | V | ½ |  |
|  | Spare Half Octet | .0000 |  | 10.5.1.8 | M | V | ½ |  |
|  | Channel Description 1 | chan_desc | 5.8 | 10.5.2.5 | M | V | 3 |  |
|  | Request Reference 1 | req_ref | 5.86 | 10.5.2.30 | M | V | 3 |  |
|  | Timing Advance 1 | time_advance | 5.104 | 10.5.2.40 | M | V | 1 |  |
|  | Channel Description 2 | chan_desc_2 | 5.8 | 10.5.2.5 | M | V | 3 |  |
|  | Request Reference 2 | req_ref_2 | 5.86 | 10.5.2.30 | M | V | 3 |  |
|  | Timing Advance 2 | time_advance_2 | 5.104 | 10.5.2.40 | M | V | 1 |  |
|  | Mobile Allocation | mob_alloc | 5.48 | 10.5.2.21 | M | LV | 1-5 |  |
| 0x7C | Starting Time | start_time | 5.98 | 10.5.2.38 | O | TV | 3 |  |
|  | IAX Rest Octets | .00101011 |  | 10.5.2.18 | O | S_PADDING | 1 | (22) |

History:

07-May-97	VK	Initial

25-Jul-97	LE	Rest Octet removed
26-Oct-99	VK	Introduce rest octets
25-Apr-00	LG	IAX Rest Octets as spare padding

## Immediate assignment reject

Description:

This message is sent on the CCCH by the network to up to four mobile stations to indicate that no channel is available for assignment.
Reference : [GSM_04.08] section 9.1.20

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Immediate assignment reject | d_imm_assign_rej | 0b00111010 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len | ctrl |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |  |
|  | Page Mode | page_mode | 5.76 | 10.5.2.26 | M | V | ½ |  |
|  | Spare Half Octet | .0000 |  | 10.5.1.8 | M | V | ½ |  |
|  | Request Reference 1 | req_ref | 5.86 | 10.5.2.30 | M | V | 3 |  |
|  | T3122 Wait Indication 1 | t3122 | 6.202 | 10.5.2.43 | M | V3 | 1 |  |
|  | Request Reference 2 | req_ref_2 | 5.86 | 10.5.2.30 | M | V | 3 |  |
|  | T3122 Wait Indication 2 | t3122_2 | 6.202 | 10.5.2.43 | M | V3 | 1 |  |
|  | Request Reference 3 | req_ref_3 | 5.86 | 10.5.2.30 | M | V | 3 |  |
|  | T3122 Wait Indication 3 | t3122_3 | 6.202 | 10.5.2.43 | M | V3 | 1 |  |
|  | Request Reference 4 | req_ref_4 | 5.86 | 10.5.2.30 | M | V | 3 |  |
|  | T3122 Wait Indication 4 | t3122_4 | 6.202 | 10.5.2.43 | M | V3 | 1 |  |
|  | IAR Rest Octets | .00101011 |  | 10.5.2.17 | O | S_PADDING | 1 | (22) |

History:

07-May-97	VK	Initial
25-Jul-97	LE	Rest octet removed
26-Oct-99	VK	Introduce rest octets

25-Apr-00	LG	rest octets as spare padding

## Measurement report

Description:

This message is sent on the SACCH by the mobile station to the network to report measurement results about the dedicated channel and about neighbor cells.
Reference : [GSM_04.08] section 9.1.21

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Measurement report | u_meas_rep | 0b00010101 | uplink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Measurement Results | meas_result | 5.47 | 10.5.2.20 | M | V | 16 |

History:

07-May-97	VK	Initial

## Notification/NCH

Description:

This message is sent on the NCH by the network to notify the mobile stations of VBS or VGCS calls in the current cell.
Reference : [GSM_04.08] section 9.1.21b

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Notification/NCH | d_notify_nch | 0b00100000 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | NT/N Rest Octets | nt_rest_oct | 5.59 | 10.5.2.22c | M | V3 | 1-20 |

History:

19-Apr-00	LG	Initial

## Paging request type 1

Description:

This message is sent on the CCCH by the network to up to two mobile stations to trigger channel access by these. The mobile stations are identified by their TMSI or IMSI.
Reference : [GSM_04.08] section 9.1.22

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Paging request type 1 | d_pag_req_1 | 0b00100001 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Page Mode | page_mode | 5.76 | 10.5.2.26 | M | V | 1/2 |
|  | Channels Needed for Mobiles 1 and 2 | chan_needed | 5.9 | 10.5.2.8 | M | V | 1/2 |
|  | Mobile Identity 1 | mob_ident | 5.49 | 10.5.1.4 | M | LV | 2-9 |
| 0x17 | Mobile Identity 2 | mob_ident_2 | 5.49 | 10.5.1.4 | O | TLV | 3-10 |
|  | P1 Rest Octets | p1_rest_oct | 5.65 | 10.5.2.23 | M |  | 0-17 |

History:

07-May-97	VK	Initial

25-Jul-97	LE	Rest Octet removed
19-Apr-00	LG	Rest Octet inserted

## Paging request type 2

Description:

This message is sent on the CCCH by the network to two or three mobile stations to trigger channel access by these. Two of the mobile stations are identified by their TMSI while the third is identified by its TMSI or IMSI.
Reference : [GSM_04.08] section 9.1.23

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Paging request type 2 | d_pag_req_2 | 0b00100010 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Page Mode | page_mode | 5.76 | 10.5.2.26 | M | V | 1/2 |
|  | Channels Needed for Mobiles 1 and 2 | chan_needed | 5.9 | 10.5.2.8 | M | V | 1/2 |
|  | Mobile Identity 1 | tmsi | 6.221 | 10.5.2.42 | M | V3 | 4 |
|  | Mobile Identity 2 | tmsi_2 | 6.221 | 10.5.2.42 | M | V3 | 4 |
| 0x17 | Mobile Identity 3 | mob_ident | 5.49 | 10.5.1.4 | O | TLV | 3-10 |
|  | P2 Rest Octets | p2_rest_oct | 5.66 | 10.5.2.24 | M |  |  |

History:

07-May-97	VK	Initial
26-Apr-00	LG	P2 Rest Octets as a structured IE

## Paging request type 3

Description:

This message is sent on the CCCH by the network to four mobile stations to trigger channel access by these. The mobile stations are identified by their TMSIs.
Reference : [GSM_04.08] section 9.1.24

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Paging request type 3 | d_pag_req_3 | 0b00100100 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Page Mode | page_mode | 5.76 | 10.5.2.26 | M | V | 1/2 |
|  | Channels Needed for Mobiles 1 and 2 | chan_needed | 5.9 | 10.5.2.8 | M | V | 1/2 |
|  | Mobile Identity 1 | tmsi | 6.221 | 10.5.2.42 | M | V3 | 4 |
|  | Mobile Identity 2 | tmsi_2 | 6.221 | 10.5.2.42 | M | V3 | 4 |
|  | Mobile Identity 3 | tmsi_3 | 6.221 | 10.5.2.42 | M | V3 | 4 |
|  | Mobile Identity 4 | tmsi_4 | 6.221 | 10.5.2.42 | M | V3 | 4 |
|  | P3 Rest Octets | p3_rest_oct | 5.67 | 10.5.2.25 | M |  |  |

History:

07-May-97	VK	Initial
26-Apr-00	LG	P3 Rest Octets as a structured IE

## Paging response

Description:

This message is sent on the main DCCH by the mobile station to the network in connection with establishment of the main signaling link as a response to the paging request message.
Reference : [GSM_04.08] section 9.1.25

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Paging response | u_pag_res | 0b00100111 | uplink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Ciphering Key Sequence Number | ciph_key_num | 5.12 | 10.5.1.2 | M | V | 1/2 |
|  | Spare Half Octet | .0000 |  | 10.5.1.8 | M | V | 1/2 |
|  | Mobile Station Classmark | mob_class_2 | 5.50 | 10.5.1.6 | M | LV | 4 |
|  | Mobile Identity | mob_ident | 5.49 | 10.5.1.4 | M | LV | 2-9 |

History:

07-May-97	VK	Initial

## Partial release

Description:

This message is sent on the main DCCH by the network to the mobile station to deactivate part of the dedicated channels in use.
Reference : [GSM_04.08] section 9.1.26

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Partial release | d_part_rel | 0b00001010 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Channel Description | chan_desc | 5.8 | 10.5.2.5 | M | V | 3 |

History:

07-May-97	VK	Initial

## Partial release complete

Description:

This message is sent on the main DCCH by the mobile station to the network to indicate that a part of the dedicated channels has been deactivated.
Reference : [GSM_04.08] section 9.1.27

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Partial release complete | u_part_rel_comp | 0b00001111 | uplink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |

History:

07-May-97	VK	Initial

## PDCH Assignment Command

Description:

This message is sent on the main DCCH by the network to the mobile station to change to a PDCH, when no timing adjustment is needed.
Reference : [GSM_04.08] section 9.1.13a

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| PDCH Assignment Command | d_pdch_ass_cmd | 0b00100011 | downlink |

Elements:

| ID | long name | short name | ref |  | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 |  | 10.4. | M | V3 | 1 |
|  | Description of the Channel, after time | chan_desc | 5.8 |  | 10.5.2.5 | M | V | 3 |
| 0x62 | Cell Channel Description | cell_chan_desc | 6.42 |  | 10.5.2.1b | O | TV3 | 17 |
| 0x05 | Frequency List, after time | freq_list_after | 5.24 |  | 10.5.2.13 | C | TLV | 4-132 |
| 0x72 | Mobile Allocation, after time | mob_alloc_after | 5.48 |  | 10.5.2.21 | C | TLV | 3-10 |
| 0x7C | Starting Time | start_time | 5.98 |  | 10.5.2.38 | O | TV | 3 |
| 0x1C | Description of the Channel, before time | chan_desc_before | 5.8 |  | 10.5.2.5 | O | TV | 4 |
| 0x1E | Frequency channel sequence before time | freq_chan_seq | 5.23 | 10.5.2.12 |  | C | TV | 10 |
| 0x21 | Mobile Allocation, before time | mob_alloc_before | 5.48 | 10.5.2.21 |  | C | TLV | 3-10 |
| 0x22 | RR Packet Uplink Assignment | pck_ul_ass | 5.75 |  | 10.5.2.25c | C | TLV | 3-* |
| 0x23 | RR Packet Downlink Assignment | pck_dl_ass | 5.70 |  | 10.5.2.25d | C | TLV | 3-* |

History:

19-Apr-00	LG	Initial

## Physical information

Description:

This message is sent on the main DCCH by the network to the mobile station to stop the sending of access bursts from the mobile station.
Reference : [GSM_04.08] section 9.1.28

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Physical information | d_phys_info | 0b00101101 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Timing Advance | time_advance | 5.104 | 10.5.2.40 | M | V | 1 |

History:

07-May-97	VK	Initial

## RR-Cell Change Order

Description:

This message is sent on the main DCCH by the network to the mobile station to order it to reselect a cell.
Reference : [GSM_04.08] section 9.1.21e

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| RR-Cell Change Order | d_change_order | 0b00001000 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Cell Description | cell_desc | 5.4 | 10.5.2.2 | M | V | 2 |
|  | NC Mode For Target Cell | nc_mode | 5.56 | 10.5.2.21c | M | V | 1/2 |

History:

19-Apr-00	LG	Initial

## RR Initialisation Request

Description:

This message is sent on the main DCCH by the mobile station to request the establishment of dedicated mode.
Reference : [GSM_04.08] section 9.1.28a

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| RR Initialisation Request | d_rr_init_req | 0b00111100 | uplink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Ciphering Key Sequence Number | ciph_key_num | 5.12 | 10.5.1.2 | M | V | 1/2 |
|  | MAC Mode And Channel Coding | chan_coding | 5.46 | 10.5.2.4a | M | V | 1/2 |
|  | Mobile Station Classmark | mob_class_2 | 5.50 | 10.5.1.6 | M | LV | 4 |
|  | Temporary Logical Link Identifier | ded_tlli | 6.209 | 10.5.2.41a | M | V3 | 4 |
|  | Channel Request Description | chan_req_desc | 5.11 | 10.5.2.8a | M | V | 5 |
|  | GPRS Measurement Results | gprs_meas_res | 5.28 | 10.5.2.20a | M | V | 2 |

History:

19-Apr-00	LG	Initial

## RR Status

Description:

This message is sent by the mobile station or the network at any time to report certain error conditions as described in section 8.
Reference : [GSM_04.08] section 9.1.29

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| RR Status | b_rr_status | 0b00010010 | both |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | RR Cause | rr_cause | 6.173 | 10.5.2.31 | M | V3 | 1 |

History:

07-May-97	VK	Initial

## System information Type 1

Description:

This message is sent on the BCCH by the network to all mobile stations within the cell giving information of control of the RACH and of the cell allocation.
Reference : [GSM_04.08] section 9.1.31

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 1 | d_sys_info_1 | 0b00011001 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Cell Channel Description | cell_chan_desc | 6.42 | 10.5.2.1b | M | V3 | 16 |
|  | RACH Control Parameter | rach_ctrl | 5.85 | 10.5.2.29 | M | V | 3 |
|  | SI 1 Rest Octets | si1_rest_oct | 5.89 | 10.5.2.32 | M |  | 1 |

History:

07-May-97	VK	Initial

23-Jul-97	LE	rest octet removed, no content
26-Oct-99	VK	Introduce rest octets
13-Apr-00	LG	si1_rest_octet now a structured IE

## System information Type 2

Description:

This message is sent on the BCCH by the network to all mobile stations within the cell giving information of control of the RACH and of the BCCH allocation in the neighbor cells.
Reference : [GSM_04.08] section 9.1.32

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 2 | d_sys_info_2 | 0b00011010 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | BCCH Frequency List | neigh_cell_desc | 6.129 | 10.5.2.22 | M | V3 | 16 |
|  | NCC Permitted | ncc_permit | 6.127 | 10.5.2.27 | M | V3 | 1 |
|  | RACH Control Parameter | rach_ctrl | 5.85 | 10.5.2.29 | M | V | 3 |

History:

07-May-97	VK	Initial

## System information Type 2bis

Description:

This message is sent optionally on the BCCH by the network to all mobile stations within the cell giving information on control of the RACH and of the extension of the BCCH allocation in the neighbour cells.
Reference : [GSM_04.08] section 9.1.33

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 2bis | d_sys_info_2bis | 0b00000010 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len | ctrl |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |  |
|  | Extended BCCH Frequency List | neigh_cell_desc | 6.129 | 10.5.2.22 | M | V3 | 16 |  |
|  | RACH Control Parameters | rach_ctrl | 5.85 | 10.5.2.29 | M | V | 3 |  |
|  | SI 2bis Rest Octets | .00101011 |  | 10.5.2.33 | O | S_PADDING | 1 | (22) |

History:

07-May-97	VK	Initial

23-Jul-97	LE	rest octet removed, no content
26-Oct-99	VK	Introduce rest octets
13-Apr-00	LG	si2bis_rest_oct as spare padding

## System information Type 2ter

Description:

This message is sent optionally on the BCCH by the network to all mobile stations within the cell giving information on the extension of the BCCH allocation in the neighbour cells.
Reference : [GSM_04.08] section 9.1.34

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 2ter | d_sys_info_2ter | 0b00000011 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len | ctrl |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |  |
|  | Extended BCCH Frequency List | neigh_cell_desc | 6.129 | 10.5.2.22 | M | V3 | 16 |  |
|  | SI 2ter Rest Octets | .00101011 |  | 10.5.2.33a | O | S_PADDING | 4 | (22) |

History:

07-May-97	VK	Initial

23-Jul-97	LE	rest octet removed, no content
26-Oct-99	VK	Introduce rest octets

13-Apr-00	LG	si2bis_rest_oct as spare padding

## System information Type 3

Description:

This message is sent on the BCCH by the network giving information of control on the RACH, the location area identification, the cell identity and various other information about the cell.
Reference : [GSM_04.08] section 9.1.35

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 3 | d_sys_info_3 | 0b00011011 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Cell Identity | cell_ident | 6.43 | 10.5.1.1 | M | V3 | 2 |
|  | Location Area Identification | loc_area_ident | 5.41 | 10.5.1.3 | M | V | 5 |
|  | Control Channel Description | ctrl_chan_desc | 5.16 | 10.5.2.11 | M | V | 3 |
|  | Cell Options | cell_opt_bcch | 5.5 | 10.5.2.3 | M | V | 1 |
|  | Cell Selection Parameters | cell_select | 5.7 | 10.5.2.4 | M | V | 2 |
|  | RACH Control Parameters | rach_ctrl | 5.85 | 10.5.2.29 | M | V | 3 |
|  | SI 3 Rest Octets | si3_rest_oct | 5.90 | 10.5.2.34 | M |  | 4 |

History:

07-May-97	VK	Initial
05-Apr-00	LG	si3_rest_octet now a structured IE

## System information Type 4

Description:

This message is sent on the BCCH by the network giving information on control of the RACH, the location area identification, the cell identity and various other information about the cell.
Reference : [GSM_04.08] section 9.1.36

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 4 | d_sys_info_4 | 0b00011100 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Location Area Identification | loc_area_ident | 5.41 | 10.5.1.3 | M | V | 5 |
|  | Cell Selection Parameters | cell_select | 5.7 | 10.5.2.4 | M | V | 2 |
|  | RACH Control Parameters | rach_ctrl | 5.85 | 10.5.2.29 | M | V | 3 |
| 0x64 | CBCH Channel Description | chan_desc | 5.8 | 10.5.2.5 | O | TV | 4 |
| 0x72 | CBCH Mobile Allocation | mob_alloc | 5.48 | 10.5.2.21 | C | TLV | 3-6 |
|  | SI 4 Rest Octets | si4_rest_oct | 5.91 | 10.5.2.35 | M | V5 | 0-10 |

History:

07-May-97	VK	Initial
05-Apr-00	LG	si4_rest_octet now a structured IE

20-Sep-00	PZ/MPA	added V5 type to si4_rest_oct

## System information Type 5

Description:

This message is sent on the SACCH by the network to mobile stations within the cell giving information on the BCCH allocation in the neighbour cells.
Reference : [GSM_04.08] section 9.1.37

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 5 | d_sys_info_5 | 0b00011101 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | BCCH Frequency List | neigh_cell_desc | 6.129 | 10.5.2.22 | M | V3 | 16 |

History:

07-May-97	VK	Initial

## System information Type 5bis

Description:

This message is sent optionally on the SACCH by the network to mobile stations within the cell giving information on the extension of the BCCH allocation in the neighbour cells.
Reference : [GSM_04.08] section 9.1.38

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 5bis | d_sys_info_5bis | 0b00000101 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Extension of the BCCH Frequency List Description | neigh_cell_desc | 6.129 | 10.5.2.22 | M | V3 | 16 |

History:

07-May-97	VK	Initial

## System information Type 5ter

Description:

This message is sent optionally on the SACCH by the network to mobile stations within the cell giving information on the extension of the BCCH allocation in the neighbour cells.
Reference : [GSM_04.08] section 9.1.39

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 5ter | d_sys_info_5ter | 0b00000110 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Extended BCCH Frequency List | neigh_cell_desc | 6.129 | 10.5.2.22 | M | V3 | 16 |

History:

07-May-97	VK	Initial

## System information Type 6

Description:

This message is sent on the SACCH by the network to mobile stations within the cell giving information of location area identification, of cell identity and various other information.
Reference : [GSM_04.08] section 9.1.40

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 6 | d_sys_info_6 | 0b00011110 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Cell Identity | cell_ident | 6.43 | 10.5.1.1 | M | V3 | 2 |
|  | Location Area Identification | loc_area_ident | 5.41 | 10.5.1.3 | M | V | 5 |
|  | Cell Options | cell_opt_sacch | 5.6 | 10.5.2.3 | M | V | 1 |
|  | NCC Permitted | ncc_permit | 6.127 | 10.5.2.27 | M | V3 | 1 |
|  | SI 6 Rest Octets | si6_rest_oct | 5.92 | 10.5.2.35a | M |  | 7 |

History:

07-May-97	VK	Initial

14-Apr-00	LG	added si6_rest_oct

## System information Type 7

Description:

This message is sent on the BCCH by the network giving information about cell reselection parameters to be used in that cell.
Reference : [GSM_04.08] section 9.1.41

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 7 | d_sys_info_7 | 0b00011111 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | SI 7 Rest Octets | si7_rest_oct | 5.91 | 10.5.2.36 | M | V5 | 20 |

History:

07-May-97	VK	Initial

05-Apr-00	LG	si7_rest_octet now a structured IE

## System information Type 8

Description:

This message is sent on the BCCH by the network giving information about cell reselection parameters to be used in that cell.
Reference : [GSM_04.08] section 9.1.42

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 8 | d_sys_info_8 | 0b00011000 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | SI 8 Rest Octets | si8_rest_oct | 5.91 | 10.5.2.37 | M | V5 | 20 |

History:

07-May-97	VK	Initial

05-Apr-00	LG	si8_rest_octet now a structured IE

## System information Type 9

Description:

This message is sent on the BCCH by the network giving information on the scheduling of information on the BBCH.
Reference : [GSM_04.08] section 9.1.43

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 9 | d_sys_info_9 | 0b00000100 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | RACH Control Parameter | rach_ctrl | 5.85 | 10.5.2.29 | M | V | 3 |
|  | SI 9 Rest Octets | si9_rest_oct | 5.93 | 10.5.2.37a | M |  | 17 |

History:

19-Apr-00	LG	Initial

## System information Type 13

Description:

This message is sent on the BCCH by the network giving information related to GPRS in the cell.
Reference : [GSM_04.08] section 9.1.43a

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 13 | d_sys_info_13 | 0b00000000 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | SI 13 Rest Octets | si13_rest_oct | 5.95 | 10.5.2.37b | M |  | 21 |

History:

19-Apr-00	LG	Initial

## System information Type 16

Description:

This message is sent on the BCCH by the network giving information about cell selection and reselection parameters to be used in that cell.
Reference : [GSM_04.08] section 9.1.43d

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 16 | d_sys_info_16 | 0b00111101 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | SI 16 Rest Octets | si16_rest_oct | 5.96 | 10.5.2.37e | M |  | 20 |

History:

19-Apr-00	LG	Initial

## System information Type 17

Description:

This message is sent on the BCCH by the network giving information about cell selection and reselection parameters to be used in that cell.
Reference : [GSM_04.08] section 9.1.42

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| System information Type 17 | d_sys_info_17 | 0b00111110 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | SI 17 Rest Octets | si17_rest_oct | 5.96 | 10.5.2.37f | M |  | 20 |

History:

19-Apr-00	LG	Initial

## Talker Indication

Description:

This message is sent on the main DDCH by the mobile station to the network to give the talker information when a new layer 2 connection is established on a VGCS channel after an uplink access.
Reference : [GSM_04.08] section 9.1.44

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Talker Indication | talker_ind | 0b00010001 | uplink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Mobile Station Classmark | mob_class_2 | 5.50 | 10.5.1.6 | M | LV | 5 |
|  | Mobile Identity | mob_ident | 5.49 | 10.5.1.4 | M | LV | 2-9 |

History:

19-Apr-00	LG	Initial

## Uplink busy

Description:

This message is broadcasted on the voice group call channel on the main DCCH, SAPI 0, by the network to inform of the uplink status of the voice group call channel.
Reference : [GSM_04.08] section 9.1.46

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Uplink Busy | d_uplink_busy | 0b00101010 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |

History:

19-Apr-00	LG	Initial

## Uplink release

Description:

This message is sent on the uplink of the voice group call channel to initiate a deactivation of the group transmit mode or on the downlink to reject an uplink access which was already granted by the network.
Reference : [GSM_04.08] section 9.1.48

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| Uplink release | b_uplink_rel | 0b00001110 | both |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | RR Cause | rr_cause | 6.173 | 10.5.2.31 | M | V3 | 1 |

History:

19-Apr-00	LG	Initial

## VGCS uplink grant

Description:

This message is sent on the main signaling channel by the network to stop the sending of access bursts from the mobile station and to change the channel configuration to a dedicated configuration.
Reference : [GSM_04.08] section 9.1.49

Definition:

| long name | short name | ID | direction |
| --- | --- | --- | --- |
| VGCS uplink grant | d_vgcs_uplink_grant | 0b00001001 | downlink |

Elements:

| ID | long name | short name | ref | spec ref | pres | type | len |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Message Type | msg_type | 6.113 | 10.4. | M | V3 | 1 |
|  | Request Reference | req_ref | 5.86 | 10.5.2.30 | M | V | 3 |
|  | Timing Advance | time_advance | 5.104 | 10.5.2.40 | M | V | 1 |

History:

19-Apr-00	LG	Initial

# Structured Elements

## Alpha Gamma

Description:

xxx

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Alpha Gamma | alpha_gamma | 9 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Alpha | alpha | 6.15 | 4 |
| Gamma | gamma | 6.84 | 5 |

History:

23-Oct-00	FRI	Initial

## BA Range

Description:

The purpose of the BA Range information element is to provide the mobile station with ARFCN range information which can be used in the cell selection procedure.
Reference : [GSM_04.08] section 10.5.2.1a

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| BA Range | ba_range | 4 | 6..L3MAX |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| Number of Ranges | num_range | 6.136 | 8 |  |
| Frequency Range | freq_range | 5.25 | 20 | [num_range..N_MAX_RANGE] |

History:

07-May-97	VK	Initial

## BA List Pref

Description:

The purpose of the BA List Pref information element is to provide the mobile station with ARFCN information which can be used in the cell selection/reselection procedure.
Reference : [GSM_04.08] section 10.5.2.1c

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| BA List Pref | ba_list_pref | 4 | 6..L3MAX |

Elements:

| long name | short name | ref | bit len | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| Range Limits | freq_range | 5.25 | 20 | CSN1_S1 | [0..MAX_RANGE] |
| BA Frequency | arfcn | 6.8 | 10 | CSN1_S1 | [0..MAX_ARFCN] |
| Spare Padding | .00101011 |  | 0-7 | S_PADDING | (0) |

History:

20-Apr-00	LG	Initial

## Cell Description

Description:

The purpose of the Cell Description information element is to provide a minimum description of a cell, e.g. to allow the mobile station to use its preknowledge about synchronization.
Reference : [GSM_04.08] section 10.5.2.2

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Cell Description | cell_desc | 3 | 3 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| BCCH ARFCN (hi part) | bcch_arfcn_hi | 6.25 | 2 |
| PLMN Colour Code | ncc | 6.149 | 3 |
| BS Colour Code | bcc | 6.32 | 3 |
| BCCH ARFCN (lo part) | bcch_arfcn_lo | 6.26 | 8 |

History:

07-May-97	VK	Initial

## Cell Options (BCCH)

Description:

The purpose of the Cell Options (BCCH) information element is to provide a variety of information about a cell.
Reference : [GSM_04.08] section 10.5.2.3

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Cell Options (BCCH) | cell_opt_bcch | 3 | 2 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| spare | .0 |  | 1 |
| Power Control | pow_ctrl | 6.151 | 1 |
| Discontinuous Transmission (BCCH) | dtx_b | 6.63 | 2 |
| Radio Link Timeout | rlt | 6.160 | 4 |

History:

07-May-97	VK	Initial

## Cell Options (SACCH)

Description:

The purpose of the Cell Options (SACCH) information element is to provide a variety of information about a cell.
Note that this IE has a split field : dtx. The values for ‘dtx’ span 'dtx_s' and 'dtx2_s'.
Reference : [GSM_04.08] section 10.5.2.3a

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Cell Options (SACCH) | cell_opt_sacch | 3 | 2 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Discontinuous Transmission (SACCH) high | dtx2_s | 6.64 | 1 |
| Power Control | pow_ctrl | 6.151 | 1 |
| Discontinuous Transmission (SACCH) low | dtx_s | 6.65 | 2 |
| Radio Link Timeout | rlt | 6.160 | 4 |

History:

07-May-97	VK	Initial

## Cell Selection Parameters

Description:

The purpose of the Cell Selection Parameters information element is to provide a variety of information about a cell.
Reference : [GSM_04.08] section 10.5.2.4

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Cell Selection Parameters | cell_select | 3 | 3 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Cell Reselect Hysteresis | cell_resel_hyst | 6.44 | 3 |
| MS-TXPWR-MAX-CCH | ms_txpwr_max_cch | 6.120 | 5 |
| ACS | acs | 6.13 | 1 |
| NECI | neci | 6.130 | 1 |
| RX Level Access Minimum | rxlev_access_min | 6.175 | 6 |

History:

07-May-97	VK	Initial

## Channel Description

Description:

The purpose of the Channel Description information element is to provide a description of an allocatable channel together with its SACCH.
Reference : [GSM_04.08] section 10.5.2.5

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Channel Description | chan_desc | 3 | 4 |
| Channel Description | chan_desc_2 | 3 | 4 |
| Channel Description | chan_desc_before | 3 | 4 |
| Channel Description | chan_desc_before_2 | 3 | 4 |
| Channel Description | chan_desc_after | 3 | 4 |
| Channel Description | chan_desc_after_2 | 3 | 4 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| Channel type and TDMA offset | chan_type | 6.47 | 5 |  |
| Time Slot | tn | 6.214 | 3 |  |
| Training Sequence Code | tsc | 6.222 | 3 |  |
| Hopping | hop | 6.89 | 1 |  |
| spare | .00 |  | 2 | {hop=0} |
| Absolute RF Channel Number | arfcn | 6.8 | 10 | {hop=0} |
| Mobile Allocation Index Offset | maio | 6.115 | 6 | {hop=1} |
| Hopping Sequence Number | hsn | 6.90 | 6 | {hop=1} |

History:

07-May-97	VK	Initial

## Channel Needed

Description:

The purpose of the Channel Needed information element is to indicate to up to two mobile stations which type of channel is needed (for each mobile station) for the transaction linked to the paging procedure.
Reference : [GSM_04.08] section 10.5.2.8

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Channel Needed | chan_needed | 1 | 1 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| CHANNEL | cn2 | 6.48 | 2 |
| CHANNEL | cn1 | 6.48 | 2 |

History:

07-May-97	VK	Initial

## Channel Needed for Mobile ID 3 and 4

Description:

The purpose of the Channel Needed for Mobile ID 3 and 4 field is to indicate to up to two mobile stations which type of channel is needed (for each mobile station) for the transaction linked to the paging procedure.
Reference : [GSM_04.08] section 10.5.2.25 (P3 Rest Octets)

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| Channel Needed for Mobile ID 3 and 4 | chan_needed3_4 |  | 4 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| CHANNEL | cn3 | 6.48 | 2 |
| CHANNEL | cn4 | 6.48 | 2 |

History:

26-Apr-00	LG	Initial

## Channel Request Description

Description:

The purpose of the Channel Request Description information element is to indicate to the network the type of requested uplink resources or to indicate the type of paging that is being responded to.
Reference : [GSM_04.08] section 10.5.2.8a

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Channel Request Description | chan_req_desc | 3 | 5 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| MT/MO Origination type | or_ty | 6.121 | 1 |  |
| Spare | .000 |  | 3 |  |
| Spare | .0000 |  | 4 | {or_ty=0} |
| Spare | .00000000 |  | 8 | {or_ty=0} [3] |
| Channel Request Description Priority | crd_prio | 6.51 | 2 | {or_ty=1} |
| RLC Mode | rlc_mode | 6.170 | 1 | {or_ty=1} |
| LLC Frame Type | llc_fr_type | 6.102 | 1 | {or_ty=1} |
| Requested Bandwidth | rbw | 6.164 | 16 | {or_ty=1} |
| RLC Octet Count | rlc_c_oct | 6.171 | 16 | {or_ty=1} |

History:

20-Apr-00	LG	Initial

## Ciphering Key Sequence Number

Description:

The purpose of the Ciphering Key Sequence Number information element is to make it possible for the network to identify the ciphering key Kc which is stored in the mobile station without invoking the authentication procedure. The ciphering key sequence number is allocated by the network and sent with the AUTHENTICATION REQUEST message to the mobile station where it is stored together with the calculated ciphering key Kc.
Reference : [GSM_04.08] section 10.5.1.2

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Ciphering Key Sequence Number | ciph_key_num | 1 | 1 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| spare | .0 |  | 1 |
| Key Sequence | key_seq | 6.100 | 3 |

History:

07-May-97	VK	Initial

## Cipher Mode Setting

Description:

The purpose of the Cipher Mode Setting information element is to indicate whether stream ciphering shall be started or not and if it is to be started, which algorithm to use.
Reference : [GSM_04.08] section 10.5.2.9

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Cipher Mode Setting | ciph_mode_set | 1 | 1 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Algorithm Identifier | algo_ident | 6.14 | 3 |
| Start Ciphering | sc | 6.194 | 1 |

History:

07-May-97	VK	Initial

## Cipher Response

Description:

The Cipher Response information element is used by the network to indicate to the mobile station which information the mobile station has to include in the CIPHERING MODE COMPLETE message.
Reference : [GSM_04.08] section 10.5.2.10

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Cipher Response | ciph_res | 1 | 1 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| spare | .000 |  | 3 |
| Cipher Response | cr | 6.53 | 1 |

History:

07-May-97	VK	Initial

## Codec Properties

Description:

The parameter is used to indicate the properties of a codec in a Multi-Rate Configuration.
Reference : [GSM_04.08] section 10.5.2.21aa

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Codec Properties | cod_prop | 10 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Codec Threshold | codec_thr | 6.57 | 6 |
| Codec Hysteresis | codec_hyst | 6.56 | 4 |

History:

25-Apr-00	LG	Initial

## Control Channel Description

Description:

The purpose of the Control Channel Description information element is to provide a variety of information about a cell.
Reference : [GSM_04.08] section 10.5.2.11

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Control Channel Description | ctrl_chan_desc | 3 | 4 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| spare | .0 |  | 1 |
| Attach / detach allowed | att | 6.22 | 1 |
| BS Access Grant Blocks reserved | bs_ag_blks_res | 6.31 | 3 |
| Common Control Channel Configuration | ccch_conf | 6.59 | 3 |
| spare | .00000 |  | 5 |
| BS Paging Multiframes | bs_pa_mfrms | 6.35 | 3 |
| T 3212 time-out value | t3212 | 6.205 | 8 |

History:

07-May-97	VK	Initial

## Descriptive Group Call Reference

Description:

The purpose of the Descriptive Group Or Broadcast Call Reference information element is to provide information describing a voice group or broadcast call. 
Reference : [GSM_04.08] section 10.5.1.9

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| Descriptive Group Call Reference | dgcr |  | 36 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Group Call Reference | gcr | 6.85 | 27 |
| Service Flag | sf | 6.182 | 1 |
| Acknowledge Flag | af | 6.12 | 1 |
| Call Priority | call_prio | 6.38 | 3 |
| Ciphering information | group_ckn | 6.86 | 4 |

History:

25-Apr-00	LG	Initial

## Dynamic Allocation

Description:

The Dynamic Allocation information element contains parameters necessary to define the radio resources of a dynamic allocation or an extended dynamic allocation. 
Reference : [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Dynamic Allocation | dyn_alloc | 69-93 |

Elements:

| long name | short name | ref | bit len | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| Extended Dynamic Allocation | ext_dyn_all | 6.71 | 1 |  |  |
| P0 | p0 | 6.139 | 4 | CSN1_S1 |  |
| USF Granularity | usf_gran | 6.228 | 1 |  |  |
| Uplink TFI Assignment | tfi | 6.211 | 5 | CSN1_S1 |  |
| RLC Data Blocks Granted | rlc_blks | 6.169 | 8 | CSN1_S1 |  |
| Flag Parameters Present | flag_pp | 6.77 | 1 |  |  |
| Tagged USF For Timeslot | tagged_usf_tn | 5.103 | 1-4 |  | [8] {flag_pp=0} |
| Alpha | alpha | 6.15 | 4 |  | {flag_pp=1} |
| USF For Timeslot And Gamma (CSN1) | usf_gamma_csn1 | 5.108 | 1-9 |  | [8] {flag_pp=1} |

History:

26-Apr-00	LG	Initial

23-Oct-00		FRI	added P0 & USF Granularity;
		changed usf_gamma to usf_gamma_csn1

## Early Classmark sending Tag

Description:

The parameter is used to indicate the support of early classmark sending inside the SI 3 rest octets.
Reference : [GSM_04.08] section 10.5.2.34

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Early Classmark Sending Tag | es_ind_tag | 0 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |

History:

05-Apr-00	LG	Initial

## Extended Measurement Results

Description:

The purpose of Extended Measurement Results information element is to provide the results of the measurement made by the mobile station on the carriers specified in the Extended Measurement Order.
Reference : [GSM_04.08] section 10.5.2.45

Definition:

| long name | short name | len | type |
| --- | --- | --- | --- |
| Extended Measurement Results | ext_meas_res | 17 | 3 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| Sequence Code Used | sc_used | 6.181 | 1 |  |
| DTX Used | dtx_used | 6.66 | 1 |  |
| RX Level Ncell | rx_lev_ncell | 6.178 | 6 | [21] |

History:

28-Apr-00	LG	Initial

## Extensions Bits IE

Description:

The Extension Bits information element is used to provide a generalised means for possible future extension within a message. This information element is variable length and contains the length indicator and spare bits.
Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24 (GPRS Cell Options)

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| Extensions Bits IE | exten |  | 6-70 |

Elements:

| long name | short name | ref | bit len | pres | ctrl |
| --- | --- | --- | --- | --- | --- |
| extension length | ext_len | 6.74 | 6 | M |  |
| spare bit extensions IE | sp_ext | 6.75 | 64 | M | [.ext_len..64] |

History:

31-May-99	SL	Initial

## Fixed Allocation

Description:

The Fixed Allocation information element contains parameters necessary to define the radio resources of a fixed allocation.
Reference : [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Fixed Allocation | fix_alloc | 27-200 |

Elements:

| long name | short name | ref | bit len | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| Uplink TFI Assignment | tfi | 6.211 | 5 | CSN1_S1 |  |
| Final Allocation | final | 6.76 | 1 |  |  |
| Downlink Control Timeslot | dl_ctrl_tn | 6.214 | 3 |  |  |
| P0 and BTS Power Control Mode | p0_pwr_ctrl_mode | 5.62 | 5 | CSN1_S1 |  |
| Flag | flag | 6.77 | 1 |  |  |
| Timeslot Allocation | ts_all | 6.215 | 8 |  | {flag=0} |
| Power Control Parameters | pwr_ctrl | 5.84 | 12-52 |  | {flag=1} |
| Half Duplex Mode | hdm | 6.87 | 1 |  |  |
| spare | .0 |  | 1 |  |  |
| Flag1 | flag1 | 6.77 | 1 |  |  |
| Fixed Allocation Block Structure | fa_blk_ia | 5.112 | 8 |  | {flag1=0} |
| Allocation Bitmap Var | allo_var | 6.18 | 1 |  | {flag1=1} [allo_len7..127] |

History:

26-Apr-00	LG	Initial

23-Oct-00	FRI	changed parameter sequence for Uplink TFI Assign,
		inserted P0 and BTS Power Control Mode,
		inserted spare and flag1, changed flaggingfor Blocks & Alloc

12-Feb-01	CAN	Fixed Allocation Block Structure changed

## Frequency Channel Sequence

Description:

The purpose of the Frequency Channel Sequence information element is to provide the absolute radio frequency channel numbers used in the mobile hopping sequence. This information element shall only be used for radio frequency channels in the primary GSM band (see TS GSM 05.05).
Reference : [GSM_04.08] section 10.5.2.12

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Frequency Channel Sequence | freq_chan_seq | 3 | 10 |
| Frequency Channel Sequence | freq_chan_seq_before | 3 | 10 |
| Frequency Channel Sequence | freq_chan_seq_after | 3 | 10 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| spare | .0 |  | 1 |  |
| Lowest ARFCN | low_arfcn | 6.104 | 7 |  |
| inc skip of ARFCNn | inc_skip | 6.95 | 4 | [N_FREQ_SKIP] |

History:

07-May-97	VK	Initial

## Frequency List

Description:

The purpose of the Frequency List information element is to provide the list of the absolute radio frequency channel numbers used in a frequency hopping sequence. There are several formats for the Frequency List information element, distinguished by the "format indicator" sub field. Some formats are frequency bit maps, the others use a special encoding scheme.
Reference : [GSM_04.08] section 10.5.2.13

The purpose of the Mobile Allocation information element is to provide that part of the RF channels belonging to the cell allocation (coded with a "1" in the cell channel description information element) which is used in the mobile hopping sequence.
Reference : [GSM_04.08] section 10.5.2.21

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Frequency List | freq_list | 4 | 3-32 |
| Frequency List | freq_list_after | 4 | 3-32 |
| Frequency List | freq_list_before | 4 | 3-32 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| Frequency List Contents | flist | 6.79 | 8 | [1..32] |

History:

08-Feb-99	LE	Initial

## Frequency Range

Description:

The Frequency Range is coded as the binary representation of the ARFCNs used as the limits of a range of frequencies to be used by the mobile station in cell selection (see TS GSM 05.08 and TS GSM 03.22)
Reference : [GSM_04.08] section 10.5.2.1a (BA Range)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Frequency Range | freq_range | 20 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Frequency RANGE LOWER | freq_lower | 6.80 | 10 |
| Frequency RANGE HIGHER | freq_higher | 6.81 | 10 |

History:

07-May-97	VK	Initial

## GPRS Cell Options

Description:

The GPRS Cell Options information element is used to control a set of cell options related to GPRS.
Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), GSM_04.60 section 12.24

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| GPRS Cell Options | gprs_opt |  | 34-98 |

Elements:

| long name | short name | ref | bit len | pres | type |
| --- | --- | --- | --- | --- | --- |
| Network Mode of Operation | nmo | 6.132 | 2 | M |  |
| T3168 | t3168 | 6.203 | 3 | M |  |
| T3192 | t3192 | 6.204 | 3 | M |  |
| DRX_TIMER_MAX | drx_t_max | 6.69 | 3 | M |  |
| ACCESS_BURST_TYPE | ab_type | 6.9 | 1 | M |  |
| CONTROL_ACK_TYPE | ctrl_ack_type | 6.60 | 1 | M |  |
| BS_CV_MAX | bs_cv_max | 6.33 | 4 | M |  |
| PAN Structure | pan | 5.77 | 9 | O | CSN1_S1 |
| Extension Structure | exten | 5.21 | 6-70 | O | CSN1_S1 |

History:

31-May-99	SL	Initial

20-Jul-99	SL	consider GSM 0460d640

## GPRS Indicator

Description:

The GPRS indicator is part of SI3 and SI4 rest octets.

Reference : [GSM_04.08] section 10.5.2.34, section 10.5.2.35

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| GPRS Indicator | gprs_indic | 4 |

Elements:

| long name | short name | ref | bit len | pres |
| --- | --- | --- | --- | --- |
| RA Color | ra_color | 6.159 | 3 | M |
| SI13 Position | si13_pos | 6.186 | 1 | M |

History:

05-Apr-00	LG	Initial

## GPRS Measurement Result

Description:

The purpose of the GPRS Measurement Result information element is to provide the results of GPRS measurements made by the GPRS mobile station an the serving cell. 
Reference : [GSM_04.08] section 10.5.2.20a

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| GPRS Measurement Results | gprs_meas_res | 3 | 3 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| C Value | c_val | 6.37 | 6 |
| RX Quality | rxqual | 6.179 | 3 |
| Spare | .0 |  | 1 |
| Signal Variance | sign_var | 6.189 | 6 |

History:

19-Apr-00	LG	Initial

## GPRS Mobile Allocation

Description:

The GPRS Mobile Allocation information element defines a set of radio frequency channels and a hopping sequence number, which may be allocated to a mobile station to define its channel configuration.

Reference : [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets) GSM_04.60 section 12.10a

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| GPRS Mobile Allocation | gprs_ma |  | 9-97 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| Hopping Sequence Number | hsn | 6.90 | 6 | M |  |  |
| RFL Number | rfln | 6.167 | 4 | O | CSN1_S1 | [0..MAX_RFL] |
| Flag | hop | 6.89 | 1 | M |  |  |
| Allocation Bitmap Length | allo_len6 | 6.17 | 6 | C |  | {hop=0} |
| Allocation Bitmap | allo_bmp6 | 6.16 | 1 | C |  | {hop=0} [allo_len6+1..64] |
| ARFCN Index | arfcn_idx | 6.19 | 6 | C | CSN1_S1 | {hop=1} [0..MAX_ARFCNI] |

History:

27-Apr-00	LG	Initial

9-Feb-01		PZ	       Allocation Bitmap fixed

12-Feb-01	CAN	Allocation Bitmap len set to 1

## GPRS Resumption

Description:

The purpose of the GPRS Resumption information element is to indicate whether the network has successfully resumed GPRS services or not.

Reference : [GSM_04.08] section 10.5.2.14c

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| GPRS Resumption | gprs_resum | 1 | 1 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Spare | .000 |  | 3 |
| Resumption ACK | res_ack | 6.165 | 1 |

History:

05-Apr-00	LG	Initial

## Group Call Information

Description:

Reference : [GSM_04.08] section 10.5.2.22c, 9.1.21a

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| Group Call Information | gr_call_info |  | 37-132 |

Elements:

| long name | short name | ref | bit len | type |
| --- | --- | --- | --- | --- |
| Descriptive Group Call Reference | dgcr | 5.17 | 36 |  |
| Group Channel Description 2 | gr_ch_desc | 5.33 | 24-95 | CSN1_S1 |

History:

25-Apr-00	LG	Initial

## Group Channel Description

Description:

The purpose of the Group Channel Description information element is to provide a description of an allocable voice group call or voice broadcast call channel together with its SACCH and part of the RF channels belonging to the cell allocation which is used in the mobile hopping sequence if applicable.

Reference : [GSM_04.08] section 10.5.2.14b

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Group Channel Description | group_chan_desc | 4 | 5-13 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| Channel type and TDMA offset | chan_type | 6.47 | 5 |  |
| Time Slot | tn | 6.214 | 3 |  |
| Training Sequence Code | tsc | 6.222 | 3 |  |
| Hopping | hop | 6.89 | 1 |  |
| spare | .00 |  | 2 | {hop=0} |
| Absolute RF Channel Number | arfcn | 6.8 | 10 | {hop=0} |
| Mobile Allocation Index Offset | maio | 6.115 | 6 | {hop=1} |
| Hopping Sequence Number | hsn | 6.90 | 6 | {hop=1} |
| Mobile Allocation RF Channels | mac | 6.114 | 8 | [1..N_MOB_ALLOC]{hop=1} |

History:

20-Apr-00	LG	Initial

## Group Channel Description 2

Description:

Reference : [GSM_04.08] section 9.1.21a (Notification/FACCH), 10.2.22c (NT/N Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Group Channel Description 2 | gr_ch_desc | 24-95 |

Elements:

| long name | short name | ref | bit len | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| Channel type and TDMA offset | chan_type | 6.47 | 5 |  |  |
| Time Slot | tn | 6.214 | 3 |  |  |
| Training Sequence Code | tsc | 6.222 | 3 |  |  |
| Hopping | hop | 6.89 | 1 |  |  |
| spare | .00 |  | 2 |  | {hop=0} |
| Absolute RF Channel Number | arfcn | 6.8 | 10 |  | {hop=0} |
| Mobile Allocation Index Offset | maio | 6.115 | 6 |  | {hop=1} |
| Hopping Sequence Number | hsn | 6.90 | 6 |  | {hop=1} |
| Hopping Frequencies | hop_freq | 5.34 | 17-71 | CSN1_S1 |  |

History:

26-Apr-00	LG	Initial

## Hopping Frequencies

Description:

Reference : [GSM_04.08] section 9.1.21a (Notification/FACCH), 10.2.22c (NT/N Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Hopping Frequencies | hop_freq | 17-71 |

Elements:

| long name | short name | ref | bit len | pres | ctrl |
| --- | --- | --- | --- | --- | --- |
| Flag | flag | 6.77 | 1 | M |  |
| Mobile Allocation Length | ma_len | 6.116 | 8 | C | {flag = 0} |
| Mobile Allocation Content | mac | 6.114 | 8 | C | {flag = 0}[ma_len..N_MOB_ALLOC] |
| Frequency Short List 2 | freq_short_list2 | 6.83 | 56 | C | {flag = 1} |

History:

26-Apr-00	LG	Initial

## IA Assignment Parameters

Description:

The IA Assignment parameters contains either a packet uplink assignment construction and a packet downlink assignment construction or a second part packet assignment construction.
Reference : [GSM_04.08] section 10.5.2.16 (IA Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| IA Assignment Parameters | ia_assign_par | 1-83 |

Elements:

| long name | short name | ref | bit len | pres | ctrl |
| --- | --- | --- | --- | --- | --- |
| Flag | flag | 6.77 | 1 | M |  |
| Flag | flag1 | 6.77 | 1 | C | {flag = 0} |
| Packet Uplink Assignment (IA) | pck_upl_ass_ia | 5.74 | 9-81 | C | {flag = 0 AND flag1 = 0} |
| Packet Downlink Assignment (IA) | pck_downl_ass_ia | 5.69 | 36-79 | C | {flag = 0 AND flag1 # 0} |
| IA Second Part Packet Assignment | ia_2nd_part | 5.39 | 0 | C | {flag = 1} |

History:

20-Apr-00	LG	Initial

## IA Frequency Parameters

Description:

The IA Frequency Parameters contains a frequency parameters, before time field.
Reference : [GSM_04.08] section 10.5.2.16 (IA Rest Octets)

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| IA Frequency Parameters | ia_freq_par |  | 102 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| Length Of Frequency Parameters | fp_len | 6.101 | 6 |  |
| Spare | .00 |  | 2 | {fp_len#0} |
| Mobile Allocation Index Offset | maio | 6.115 | 6 | {fp_len#0} |
| Mobile Allocation Contents | mac | 6.114 | 8 | [fp_len..11] {fp_len#0} |

History:

20-Apr-00	LG	Initial

## IA Rest Octets

Description:

The IA Rest Octets information element contains spare bits and possibly either a packet uplink assignment construction, a packet downlink assignment construction, a second part packet assignment construction or a frequency parameters, before time field, which combines a mobile allocation and a MAIO (see the channel description information element).
Reference : [GSM_04.08] section 10.5.2.16

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| IA Rest Octets | ia_rest_oct |  | 1-12 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| IA Rest Octets Parameters | ia_rest_oct_par | 5.38 | 2-103 | O | CSN1_SHL |  |
| Spare Padding | .00101011 |  | 0-7 | O | S_PADDING | (22) |

History:

07-May-97	VK	Initial
25-Oct-99	VK	change bit len to 88 (was 96)
20-Apr-00	LG	now a structured IE

## IA Rest Octet Parameters

Description:

The IA Rest Octets parameters contains either a packet uplink assignment construction, a packet downlink assignment construction, a second part packet assignment construction or a frequency parameters, before time field.
Reference : [GSM_04.08] section 10.5.2.16

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| IA Rest Octet Parameters | ia_rest_oct_par |  | 2-103 |

Elements:

| long name | short name | ref | bit len | pres | ctrl |
| --- | --- | --- | --- | --- | --- |
| Flag | flag | 6.77 | 1 | M |  |
| IA Frequency Parameters | ia_freq_par | 5.36 | 102 | C | {flag = 0} |
| IA Assignment Parameters | ia_assign_par | 5.35 | 1-83 | C | {flag = 1} |

History:

20-Apr-00	LG	Initial

## IA Second Part Packet Assignment

Description:

The IA Second Part Packet Assignment is empty in the current version. The presence of the Second Part Packet Assignment is the indication , that this message is the second message of two Immediate Assignment messages in an assignment of an uplink or downlink Temporary Block Flow (TBF). 
Reference : [GSM_04.08] section 10.5.2.16

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| IA Second Part Packet Assignment | ia_2nd_part |  | 0 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |

History:

20-Apr-00	LG	Initial

## Info Type and Positions

Description:

Reference : [1] section 10.5.2.37a (SI 9 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Info Type and Positions | itp | 11-133 |
| Info Type and Positions Optional | itpo | 11-133 |

Elements:

| long name | short name | ref | bit len | pres | ctrl |
| --- | --- | --- | --- | --- | --- |
| Flag | flag | 6.77 | 1 | M |  |
| Info Type 4 | it4 | 6.97 | 4 | O | {flag=0} |
| Flag1 | flag1 | 6.77 | 1 | O | {flag=1} |
| Info Type 5 | it5 | 6.98 | 5 | O | {flag=1 AND flag1=0} |
| Info Type 6 | it6 | 6.97 | 6 | O | {flag=1 AND flag1=6} |
| Positions | poss | 5.81 | 6-126 | M |  |

History:

27-Apr-00	LG	Initial

## Location Area Identification

Description:

The purpose of the Location Area Identification information element is to provide an unambiguous identification of location areas within the area covered by the GSM system.
Reference : [GSM_04.08] section 10.5.1.3

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Location Area Identification | loc_area_ident | 3 | 6 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| Mobile Country Code | mcc | 6.117 | 4 | BCDEVEN[3] |
| Mobile Network Code | mnc | 6.118 | 4 | BCDEVEN[2] |
| Location Area Code | lac | 6.103 | 16 |  |

History:

07-May-97	VK	Initial

## LSA identity

Description:

The LSA identity is part of the SI4 rest octets.

Reference : [GSM_04.08] section 10.5.2.35

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| LSA identity | lsa_ident | 11-25 |
| Additional LSA identity | lsa_ident_add | 11-25 |

Elements:

| long name | short name | ref | bit len | pres | ctrl |
| --- | --- | --- | --- | --- | --- |
| Flag | flag | 6.77 | 1 | M |  |
| LSA ID | lsa_id | 6.105 | 24 | O | {flag = 0} |
| ShortLSA ID | short_lsa_id | 6.184 | 10 | O | {flag = 1} |

History:

07-Apr-01	xba	Initial

## LSA ID Information

Description:

The LSA ID Information is part of SI4 rest octets.

Reference : [GSM_04.08] section 10.5.2.35

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| LSA ID Information | lsa_id_info | 25-76 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| LSA identity | lsa_ident | 5.42 | 11-25 | M |  |  |
| LSA ID | lsa_id | 6.105 | 24 | M |  |  |
| Additional LSA IDs | lsa_id_add | 6.105 | 24 | M | CSN1_S1 | [0..MAX_LSA_ID] |
| Additional LSA identity | lsa_ident_add | 5.42 | 11-25 | M | CSN1_S1 | [0..MAX_LSA_ID] |

History:

05-Apr-00	LG	Initial

07-Apr-01	xba	Change: GSM 04.08 ver. 7.1.2 ---> GSM 04.08 ver. 7.6.1

## LSA Parameters

Description:

The LSA parameters are part of SI4 Rest Octets and SI16 Rest Octets.

Reference : [GSM_04.08] section 10.5.2.35, section 10.5.2.37e

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| LSA parameters | lsa_param |  | 20-32 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| Priority Threshold | prio_thr | 6.156 | 3 | M |  |  |
| LSA Offset | lsa_offs | 6.106 | 3 | M |  |  |
| SoLSA Exclusive Access | solsa_exc_acc | 6.190 | 1 | M |  |  |
| Optional MCC | opt_mcc | 5.60 | 12 | O | CSN1_S1 |  |
| MNC | mnc | 6.118 | 4 | M |  | [3] |

History:

05-Apr-00	LG	Initial

## LSA Parameters and LSA ID

Description:

The LSA parameters and ID are part of SI16 Rest Octets.

Reference : [GSM_04.08] section 10.5.2.37e

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| LSA Parameters and LSA ID | lsa_par_id |  | 45-108 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| LSA parameters | lsa_param | 5.44 | 20-32 |
| LSA ID Information | lsa_id_info | 5.42 | 25-76 |

History:

28-Apr-00	LG	Initial

## MAC Mode And Channel Coding

Description:

The purpose of the MAC Mode and Channel Coding Requested information element is for the mobile station to indicate which channel coding rate is desired.
Reference : [GSM_04.08] section 10.5.2.4a

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| MAC Mode And Channel Coding | chan_coding | 1 | 1 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| MAC Mode | mac_mode | 6.107 | 2 |
| Coding Scheme | cod_scheme | 6.58 | 2 |

History:

20-Apr-00	LG	Initial

## Measurement Results

Description:

The purpose of the Measurement Results information element is to provide the results of the measurements made by the mobile station on the serving cell and the neighbour cells.
Reference : [GSM_04.08] section 10.5.2.20

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Measurement Results | meas_result | 3 | 17 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| BA used | ba_used | 6.23 | 1 |  |
| DTX used | dtx_used | 6.66 | 1 |  |
| RX Level Full Serving Cell | rxlev_full | 6.176 | 6 |  |
| spare | .0 |  | 1 |  |
| Measurement valid | meas_valid | 6.112 | 1 |  |
| RX Level Sub Serving Cell | rxlev_sub | 6.177 | 6 |  |
| spare | .0 |  | 1 |  |
| RX Quality Full Serving Cell | rxqual_full | 6.179 | 3 |  |
| RX Quality Sub Serving Cell | rxqual_sub | 6.180 | 3 |  |
| Number of Cells | num_ncell | 6.137 | 3 |  |
| Ncell | ncell | 5.57 | 17 | [N_CELL_MRES] |

History:

07-May-97	VK	Initial

## Mobile Allocation

Description:

The purpose of the Mobile Allocation information element is to provide that part of the RF channels belonging to the cell allocation (coded with a "1" in the cell channel description information element) which is used in the mobile hopping sequence.
Reference : [GSM_04.08] section 10.5.2.21

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Mobile Allocation | mob_alloc | 4 | 8 |
| Mobile Allocation | mob_alloc_before | 4 | 8 |
| Mobile Allocation | mob_alloc_after | 4 | 8 |

Elements:

| long name | short name | ref | bit len | Ctrl |
| --- | --- | --- | --- | --- |
| Mobile Allocation Contents | mac | 6.114 | 8 | [1..N_MOB_ALLOC] |

History:

07-May-97	VK	Initial

## Mobile Identity

Description:

The purpose of the Mobile Identity information element is to provide either the international mobile subscriber identity, IMSI, the temporary mobile subscriber identity, TMSI, the international mobile equipment identity, IMEI or the international mobile equipment identity together with the software version number, IMEISV.
Reference : [GSM_04.08] section 10.5.1.4

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Mobile Identity | mob_ident | 4 | 3-10 |
| Mobile Identity | mob_ident_2 | 4 | 3-10 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| Type of identity | ident_type | 6.225 | 3 | (GETPOS,:,4,+,:,1,+,SETPOS) |
| Odd/ Even indication | odd_even | 6.138 | 1 | (SETPOS) |
| Identity digit | ident_dig | 6.91 | 4 | (SETPOS) {ident_type # ID_TYPE_NO_IDENT AND ident_type # ID_TYPE_TMSI} BCDODD [0..16] |
| spare | .1111 |  | 4 | (:,SETPOS,8,+){ident_type = ID_TYPE_TMSI} |
| TMSI | tmsi | 6.221 | 32 | (SETPOS){ident_type = ID_TYPE_TMSI} [.32] |

History:

07-May-97	VK	Initial

17-Jul-97	LE	tmsi added

## Mobile Station Classmark 2

Description:

The purpose of the Mobile Station Classmark 2 information element is to provide the network with information concerning aspects of both high and low priority of the mobile station equipment. This affects the manner in which the network handles the operation of the mobile station.
Reference : [GSM_04.08] section 10.5.1.6

Definition:

| long name | Short name | type | len |
| --- | --- | --- | --- |
| Mobile Station Classmark 2 | mob_class_2 | 4 | 5 |

Elements:

| long name | Short name | ref | bit len |
| --- | --- | --- | --- |
| spare | .0 |  | 1 |
| Revision level | rev_lev | 6.166 | 2 |
| Early Classmark Sending | es_ind | 6.70 | 1 |
| A5/1 | a5_1 | 6.1 | 1 |
| RF power capability | rf_pow_cap | 6.168 | 3 |
| spare | .0 |  | 1 |
| pseudo-synchronisation cap. | ps_cap | 6.157 | 1 |
| SS Screen. Indicator | ss_screen | 6.193 | 2 |
| Short Message cap. | sm_cap | 6.185 | 1 |
| VBS Notification Reception | vbs_not_rec | 6.229 | 1 |
| VGCS Notification Reception | vgcs_not_rec | 6.230 | 1 |
| Frequency capability | freq_cap | 6.78 | 1 |
| Classmark 3 | class_3 | 6.54 | 1 |
| spare | .000 |  | 3 |
| SoLSA Supported | solsa_supp | 6.191 | 1 |
| CMSP Supported | cmsp_supp | 6.55 | 1 |
| A5/3 | a5_3 | 6.3 | 1 |
| A5/2 | a5_2 | 6.2 | 1 |

History:

07-May-97	VK	Initial
19-Apr-00	LG	conform to V7.1.2

## Mobile Station Classmark 3

Description:

The purpose of the Mobile Station Classmark 3 information element is to provide the network with information concerning aspects of the mobile station. The contents might affect the manner in which the network handles the operation of the mobile station.
Reference : [GSM_04.08] section 10.5.1.7

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Mobile Station Classmark 3 | mob_class_3 | 4 | 4-14 |

Elements:

| long name | short name | Ref | bit len | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| Spare | .0 |  | 1 |  |  |
| Multiband_support | mb_sup | 6.24 | 3 |  |  |
| A5/7 | a5_7 | 6.7 | 1 |  |  |
| A5/6 | a5_6 | 6.6 | 1 |  |  |
| A5/5 | a5_5 | 6.5 | 1 |  |  |
| A5/4 | a5_4 | 6.4 | 1 |  |  |
| Associated Radio Capability 2 | radio_cap_2 | 6.21 | 4 |  | { mb_sup # 0 } |
| Associated Radio Capability 1 | radio_cap_1 | 6.20 | 4 |  | { mb_sup # 0 } |
| R Support | r_support | 6.174 | 3 | CSN1_S1 |  |
| Multi Slot Class | m_s_class | 6.124 | 5 | CSN1_S1 |  |
| UCS2 Treatment | ucs2_treat | 6.226 | 1 | CSN1_S1 | { m_s_class = 1 } |
| Extended Measurement Capability | ext_meas_cap | 6.72 | 1 | CSN1_S1 | { m_s_class = 1 } |
| MS Measurement Capability | meas_cap | 5.53 | 8 | CSN1_S1 |  |
| Spare Padding | .00000 |  | 5 |  |  |

History:

07-May-97	VK	Initial

20-Jan-99	LE	structure changed
20-Apr-00	LG	R Support until Spare Padding and ctrl column added, E bit replaced 		by spare

## Mobile Time Difference

Description:

A Mobile Time Difference information element encodes a time related to the synchronization difference between the time bases of two base stations. This type of information is used in conjunction with the HANDOVER COMPLETE message.
Reference : [GSM_04.08] section 10.5.2.21a

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Mobile Time Difference | mob_time_diff | 4 | 3 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| time difference | diff | 6.212 | 21 |
| spare | .000 |  | 3 |

History:

07-May-97	VK	Initial

02-Jun-98	LE	Structure changed

## MS Measurement Capability

Description:

The measurement capability field provides times to perform an extended measurement.

Reference : [GSM_04.08] section 10.5.1.7

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| MS Measurement Capability | meas_cap |  | 8 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Switch Measurement Time | sw_time | 6.197 | 4 |
| Switch Measurement Switch Time | sws_time | 6.197 | 4 |

History:

20-Apr-00	LG	Initial

## Multi Rate Configuration

Description:

The Multi Rate Configuration information element gives all parameters related to a multi-rate speech codec. 
Reference : [GSM_04.08] section 10.5.2.21aa

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Multi Rate Configuration | multirate_conf | 4 | 4-8 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| Multi Rate Speech Version | mr_vers | 6.123 | 3 |  |
| Spare | .0 |  | 1 |  |
| Initial Codec Mode Indicator | icmi | 6.99 | 1 |  |
| Spare | .0 |  | 1 |  |
| Start Mode | st_mode | 6.195 | 2 |  |
| Set of AMR Codec Modes | set_amr | 6.183 | 8 |  |
| Count Of Codec Properties | cnt_cod_prop | 6.61 | 2 | {0} |
| Spare | .00 |  | 2 | {cnt_cod_prop#0} |
| Codec Properties | cod_prop | 5.15 | 10 | [cnt_cod_prop..3] |
| Spare | .00 |  | 2 | {cnt_cod_prop=1} |
| Spare | .00 |  | 2 | {cnt_cod_prop=1 OR cnt_cod_prop=2} |

History:

25-Apr-00	LG	Initial

## Multislot Allocation

Description:

The purpose of the Multislot Allocation information element is to provide a description of which channels are used in downlink and uplink respectively, in a multislot configuration. 
Reference : [GSM_04.08] section 10.5.2.21b

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Multislot Configuration | multislot_alloc | 4 | 3-12 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| Downlink Assignment Bitmap | dab | 6.68 | 7 | + |
| Uplink Assignment Bitmap | uab | 6.68 | 7 | * |
| Channel Set Bitmap | chan_set | 6.52 | 8 | [0..8] |

History:

25-Apr-00	LG	Initial

## NC Mode For Target Cell

Description:

The purpose of the NC mode information element is to inform the mobile station of the NC mode to be implemented on the target cell. 
Reference : [GSM_04.08] section 10.5.2.21c

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| NC Mode For Target Cell | nc_mode | 1 | 1/2 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Spare | .00 |  | 2 |
| NC Mode | ncm | 6.126 | 2 |

History:

25-Apr-00	LG	Initial

## Ncell

Description:

Reference : [GSM_04.08] section 10.5.2.20 (Measurement Results)

Definition:

| long name | short name |
| --- | --- |
| Ncell | ncell |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| RX Level Ncell | rx_lev_ncell | 6.178 | 6 |
| BCCH Frequency Ncell | bcch_ncell | 6.28 | 5 |
| BSIC Ncell | bsic | 6.34 | 6 |

History:

07-May-97	VK	Initial

## NLN(PCH) and Status

Description:

Reference : [GSM_04.08] section 10.5.2.23 (P1 Rest Octets), section 10.5.2.24 (P2 Rest Octets),section 10.5.2.25 (P3 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| NLN(PCH) and Status | nln_stat | 3 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Notification List Number PCH | nln_pch | 6.133 | 2 |
| Notification List Number Status | nln_status | 6.134 | 1 |

History:

26-Apr-00	LG	Initial

## NT/N Rest Octets

Description:

Reference : [GSM_04.08] section 10.5.2.22c

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| NT/N Rest Octets | nt_rest_oct |  | 1-20 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| NLN(PCH) | nln_pch | 6.133 | 2 | O | CSN1_S1 |  |
| Group Call Information | gr_call_info | 5.31 | 37-132 | O | CSN1_S1 | [0..MAX_GR_C_INFO] |
| Spare padding | .00101011 |  | 0-7 | O | S_PADDING | (0) |

History:

05-Apr-00	LG	Initial

## Optional Mobile Country Code

Description:

The optional mobile country code is part of LSA parameters in SI4 rest octets.

Reference : [GSM_04.08] section 10.5.2.35

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Optional MCC | opt_mcc | 12 |

Elements:

| long name | short name | ref | bit len | pres | ctrl |
| --- | --- | --- | --- | --- | --- |
| Mobile Country Code | mcc | 6.117 | 4 | M | [3] |

History:

05-Apr-00	LG	Initial

## Optional Selection Parameters

Description:

Optional selection parameters are part of SI3 and SI4 rest octets.  
Reference : [GSM_04.08] section 10.5.2.34, section 10.5.2.35

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| Optional Selection Parameters | opt_sel_par |  | 15 |

Elements:

| long name | short name | ref | bit len | pres |
| --- | --- | --- | --- | --- |
| Cell Bar Qualify | cell_bar_qual | 6.40 | 1 | M |
| Cell Reselect Offset | cell_resel_offs | 6.45 | 6 | M |
| Temporary Offset | temp_offs | 6.210 | 3 | M |
| Penalty Time | penalty_time | 6.148 | 5 | M |

History:

05-Apr-00	LG	Initial

## P0 and BTS Power Control Mode

Description:

The P0 and BTS Power Control Mode field is part of the packet downlink assignment of IA rest octets.

Reference : [GSM_04.08] section 10.5.2.16

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| P0 and BTS Power Control Mode | p0_pwr_ctrl_mode |  | 5 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| P0 | p0 | 6.139 | 4 |
| BTS Power Control Mode | pwr_ctrl_mode | 6.36 | 1 |

History:

25-Apr-00	LG	Initial

## P0 and PR Mode

Description:

Reference : [GSM_04.60] section 11.2.29 (Packet Uplink Assignment) 0460-670

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| P0 PR Mode | p0_prmode |  | 5 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| P0 | p0 | 6.139 | 4 |
| PR_Mode | pr_mode | 6.154 | 1 |

History:

22-Sep-00	FRI	Initial

## P0 BTS PR Mode

Description:

Reference : [GSM_04.60] section 11.2.29 (Packet Uplink Assignment) 0460-670

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| P0 BTS Power Control PR Mode | p0_bts_prmode |  | 6 |

Elements:

| Long name | short name | ref | bit len |
| --- | --- | --- | --- |
| P0 | p0 | 6.139 | 4 |
| BTS Power Control Mode | pwr_ctrl_mode | 6.36 | 1 |
| PR_Mode | pr_mode | 6.154 | 1 |

History:

22-Sep-00	FRI	Initial

## P1 Rest Octets

Description:

The P1 Rest Octets information element contains information about the status of an existing NCH, priority levels and packet page indications applied for mobile station identities.

Reference : [GSM_04.08] section 10.5.2.23

Definition:

| long name | short name | len |
| --- | --- | --- |
| P1 Rest Octets | p1_rest_oct | 1-18 |

Elements:

| long name | short name | ref | bit len | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| NLN(PCH) and Status | nln_stat | 5.58 | 3 | CSN1_SHL |  |
| Priority 1 and 2 | tagged_call_prio | 5.102 | 1-4 |  | [2] |
| Group Call Information | gr_call_info | 5.31 | 37-132 | CSN1_SHL |  |
| Packet Page Indication 1 | pck_page1 | 5.72 | 0 | CSN1_SHL |  |
| Packet Page Indication 2 | pck_page2 | 5.72 | 0 | CSN1_SHL |  |
| Spare Padding | .00101011 |  | 0-7 | S_PADDING | (23) |

History:

26-Apr-00	LG	Initial

## P2 Rest Octets

Description:

The P2 Rest Octets information element contains information on the channel needed by the network and information about the status of an existing NCH, priority levels and packet page indications applied for mobile station identities.

Reference : [GSM_04.08] section 10.5.2.24

Definition:

| long name | short name | len |
| --- | --- | --- |
| P2 Rest Octets | p2_rest_oct | 2-12 |

Elements:

| long name | short name | ref | bit len | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| Channel Needed for Mobile ID 3 | cn3 | 6.48 | 2 | CSN1_SHL |  |
| NLN(PCH) and Status | nln_stat | 5.58 | 3 | CSN1_SHL |  |
| Priority 1 until 3 | tagged_call_prio | 5.102 | 1-4 |  | [3] |
| Packet Page Indication 3 | pck_page3 | 5.72 | 0 | CSN1_SHL |  |
| Spare Padding | .00101011 |  | 0-7 | S_PADDING | (23) |

History:

26-Apr-00	LG	Initial

## P3 Rest Octets

Description:

The P3 Rest Octets information element contains information on the channel needed by the network and information about the status of an existing NCH, priority levels and packet page indications applied for mobile station identities.

Reference : [GSM_04.08] section 10.5.2.24

Definition:

| long name | short name | len |
| --- | --- | --- |
| P3 Rest Octets | p3_rest_oct | 4 |

Elements:

| long name | short name | ref | bit len | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| Channel Needed for Mobile ID 3 and 4 | chan_needed3_4 | 5.10 | 4 | CSN1_SHL |  |
| NLN(PCH) and Status | nln_stat | 5.58 | 3 | CSN1_SHL |  |
| Priority 1 until 4 | tagged_call_prio | 5.102 | 1-4 |  | [4] |
| Spare Padding | .00101011 |  | 0-7 | S_PADDING | (23) |

History:

26-Apr-00	LG	Initial

## Packet Channel Description

Description:

The purpose of the Packet Channel Description information element is to provide a description of a packet data physical channel (PDCH).

Reference : [GSM_04.08] section 10.5.2.25a

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Packet Channel Description | pck_chan_desc | 3 | 4 |

Elements:

| long name | short name | ref | bit len | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| Packet Channel type | pck_chan_type | 6.140 | 5 |  |  |
| Time Slot | tn | 6.214 | 3 |  |  |
| Training Sequence Code | tsc | 6.222 | 3 |  |  |
| Hopping | hop | 6.89 | 1 |  |  |
| Indirect | indir | 6.96 | 1 |  | {hop=0} |
| Spare | .0 |  | 1 |  | {hop=0} |
| Absolute RF Channel Number | arfcn | 6.8 | 10 |  | {hop=0 AND indir=0} |
| Mobile Allocation Index Offset | maio | 6.115 | 6 |  | {hop=1 OR hop=0 AND indir=1} |
| MA Number Indication | ma_num | 6.108 | 1 |  | {hop=0 AND indir=1} |
| Change Mark 1 | ch_mark1 | 6.46 | 2 | CSN1_S1 | {hop=0 AND indir=1} |
| Hopping Sequence Number | hsn | 6.90 | 6 |  | {hop=1} |

History:

26-Apr-00	LG	Initial

## Packet Downlink Assignment (IA)

Description:

The Packet Downlink Assignment IA is part of IA rest octets.

Reference : [GSM_04.08] section 10.5.2.16

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| Packet Downlink Assignment (IA) | pck_downl_ass_ia |  | 36-79 |

Elements:

| long name | short name | ref | bit len | pres | type |
| --- | --- | --- | --- | --- | --- |
| Temporary Logical Link Identifier | ded_tlli | 6.209 | 32 | M |  |
| TFI Assignment and RLC Mode | tfi_ass_rlc | 5.107 | 14-18 | O | CSN1_S1 |
| Timing Advance Index | ta_idx | 6.216 | 4 | O | CSN1_S1 |
| TBF Starting Time | tbf_start_time | 5.98 | 16 | O | CSN1_S1 |
| P0 BTS PR Mode | p0_bts_prmode | 5.64 | 6 | 0 | CSN1_S1 |

History:

25-Apr-00	LG	Initial

## Packet Downlink Assignment (RR)

Description:

The RR Packet Downlink Assignment is sent by the network to the mobile station to indicate the assigned  downlink resources.

Reference : [GSM_04.08] section 10.5.2.25d

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| RR Packet Downlink Assignment | pck_dl_ass | 4 | 16-120 |

Elements:

| long name | short name | ref | bit len | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| MAC Mode | mac_mode | 6.107 | 2 |  |  |
| RLC Mode | rlc_mode | 6.170 | 1 |  |  |
| Timeslot Allocation | ts_all | 6.215 | 8 |  |  |
| Packet Timing Advance | pck_ta | 5.73 | 2-15 |  |  |
| Power Control Parameters | pwr_ctrl | 5.84 | 12-52 | CSN1_S1 |  |
| Downlink TFI Assignment | tfi | 6.211 | 5 | CSN1_S1 |  |
| Packet Measurement Parameters | pck_meas_par | 5.71 | 29 | CSN1_S1 |  |
| N Spare Padding | .00000000 |  | 0-7 | S_PADDING | (0) |

History:

27-Apr-00	LG	Initial

## Packet Measurement Parameters

Description:

Reference : [GSM_04.08] section 10.5.2.25d (RR Packet Downlink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Packet Measurement Parameters | pck_meas_par | 29 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Measurement Starting Time | meas_start | 5.98 | 16 |
| Measurement Interval | meas_int | 6.111 | 5 |
| Measurement Bitmap | meas_bmp | 6.110 | 8 |

History:

27-Apr-00	LG	Initial

## Packet Page Indication

Description:

The Packet Page Indication field indicates the kind of paging procedure associated with the mobile station identity. The field is implemented as an optional empty composition. Absence of the composition means "paging procedure for RR connection establishment", presence means "packet paging procedure".  
Reference : [GSM_04.08] section 10.5.2.23 (P1 Rest Octets), section 10.5.2.24 (P2 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Packet Page Indication 1 | pck_page1 | 0 |
| Packet Page Indication 2 | pck_page2 | 0 |
| Packet Page Indication 3 | pck_page3 | 0 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |

History:

26-Apr-00	LG	Initial

## Packet Timing Advance

Description:

The Packet Timing Advance field describes the timing advance mode and the timing advance value assigned to the mobile station.
Reference : [GSM_04.08] section 10.5.2.26

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| Packet Timing Advance | pck_ta |  | 2-15 |

Elements:

| long name | short name | ref | bit len | type |
| --- | --- | --- | --- | --- |
| Timing Advance value | ta | 6.219 | 6 | CSN1_S1 |
| Timing Advance Index and Number | ta_idx_nm | 5.105 | 7 | CSN1_S1 |

History:

26-Apr-00	LG	Initial

## Packet Uplink Assignment (IA)

Description:

The Packet Uplink Assignment IA is part of IA rest octets.

Reference : [GSM_04.08] section 10.5.2.16

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| Packet Uplink Assignment (IA) | pck_upl_ass_ia |  | 9-81 |

Elements:

| long name | short name | ref | bit len | pres | type |
| --- | --- | --- | --- | --- | --- |
| TFI Assignment and Allocation | tfi_ass_alloc | 5.106 | 15-48 | O | CSN1_S1 |
| Alpha | alpha | 6.15 | 4 | O | CSN1_S1 |
| Gamma | gamma | 6.84 | 5 | M |  |
| Timing Advance Index | ta_idx | 6.216 | 4 | O | CSN1_S1 |
| TBF Starting Time | tbf_start_time | 5.98 | 16 | O | CSN1_S1 |

History:

25-Apr-00	LG	Initial

## Packet Uplink Assignment (RR)

Description:

The RR Packet Uplink Assignment is sent by the network to the mobile station to indicate the assigned  uplink resources. 
Note: The recommendation is ambiguous: In the definition of IE by CSN1 Timeslot Allocation is omitted, in the description of details it occurs.

Reference : [GSM_04.08] section 10.5.2.25c

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| RR Packet Uplink Assignment | pck_ul_ass | 4 |  |

Elements:

| long name | short name | ref | bit len | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| Channel Coding Command | ccm | 6.58 | 2 |  |  |
| TLLI Block Channel Coding | tlli_bcc | 6.220 | 1 |  |  |
| Packet Timing Advance | pck_ta | 5.73 | 2-15 |  |  |
| Flag | flag | 6.77 | 1 |  |  |
| Flag1 | flag1 | 6.77 | 1 |  |  |
| Dynamic Allocation | dyn_alloc | 5.18 | 69-93 |  | {flag=0 AND flag1=1} |
| Single Block Allocation | single_alloc | 5.97 | 9-13 |  | {flag=1 AND flag1=0} |
| Fixed Allocation | fix_alloc | 5.22 | 27-200 |  | {flag=1 AND flag1=1} |
| N Spare Padding | .00000000 |  | 0-7 | S_PADDING | (0) |

History:

26-Apr-00	LG	Initial

23-Oct-00	FRI	corrected flag values according to GSM04.08v6.7.0

## Page Mode

Description:

The purpose of the Page Mode information element is to control the action of the mobile station belonging to the paging subgroup corresponding to the paging subchannel.
Reference : [GSM_04.08] section 10.5.2.26

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Page Mode | page_mode | 1 | 1/2 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| spare | .00 |  | 2 |
| Page Mode | pm | 6.142 | 2 |

History:

07-May-97	VK	Initial

## PAN Structure

Description:

This structure contains parameters for handling of counter N3102 during Packet data Transfer. At each cell reselection the mobile station shall set the counter N3102 to the value defined by the optional broadcast parameter PAN_MAX. Whenever the mobile station receives a Packet Ack/Nack that allows the advancement of V(S), the mobile station shall increment N3102 by the broadcast value PAN_INC, however N3102 shall never exceed the value PAN_MAX. Each time T3182 expires the mobile station shall decrement N3102 by the broadcast value PAN_DEC. When N3102 less or equal 0 is reached the mobile station shall perform an abnormal release with cell-reselection.

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24 (GPRS Cell Options).

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| PAN Structure | pan | 9 |

Elements:

| long name | short name | ref | bit len | pres |
| --- | --- | --- | --- | --- |
| PAN_DEC | dec | 6.143 | 3 | M |
| PAN_INC | inc | 6.144 | 3 | M |
| PAN_MAX | pmax | 6.145 | 3 | M |

History:

31-May-99	SL	Initial

## PBCCH Description

Description:

Reference	: [GSM_04.08] section 10.5.2.37b, GSM_04.60 section 11.2.25

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| PBCCH Description | pbcch_des |  | 11-22 |

Elements:

| long name | short name | ref | bit len | pres | ctrl |
| --- | --- | --- | --- | --- | --- |
| Pb | pb | 6.146 | 4 | M |  |
| Training Sequence Code | tsc | 6.222 | 3 | M |  |
| Time Slot Number | tn | 6.214 | 3 | M |  |
| Flag | flag | 6.77 | 1 | M |  |
| Flag2 | flag2 | 6.77 | 1 | C | {flag=0} |
| ARFCN | arfcn | 6.8 | 10 | C | {flag=0 AND flag2=1} |
| Mobile Allocation Index Offset | maio | 6.115 | 6 | C | {flag=1} |

History:

22-Jul-99	SL	Initial

07-Jun-00	CAN	Initial

## PCH and NCH Info

Description:

The PCH and NCH Info is part SI 6 Rest Octets information element.
Reference : [GSM_04.08] section 10.5.2.35a

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| PCH and NCH Info | pch_nch_info | 7 |

Elements:

| long name | short name | ref | bit len | pres |
| --- | --- | --- | --- | --- |
| Paging Channel Restructuring | pch_restruct | 6.141 | 1 | M |
| Notification List Number SACCH | nln_sacch | 6.133 | 2 | M |
| Call Priority | call_prio | 6.38 | 3 | M |
| Notification List Number Status | nln_status | 6.134 | 1 | M |

History:

13-Apr-00	LG	Initial

## Position

Description:

Reference : [1] section 10.5.2.37a (SI 9 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Position | pos | 5-21 |
| Optional Position | poso | 5-21 |

Elements:

| long name | short name | ref | bit len | pres | ctrl |
| --- | --- | --- | --- | --- | --- |
| Modulus | mod | 6.119 | 4 |  |  |
| Relative Position | rel_pos | 6.162 | 16 |  | {mod#0}[.mod+1..16] |
| BCCH Type | bcch_type | 6.29 | 1 |  |  |

History:

27-Apr-00	LG	Initial

## Positions

Description:

Reference : [1] section 10.5.2.37a (SI 9 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Positions | poss | 6-126 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| Position | pos | 5.80 | 5-21 | M |  |  |
| Optional Position | poso | 5.80 | 5-21 | O | CSN1_S1 | [0..MAX_POS] |

History:

27-Apr-00	LG	Initial

## Power Command

Description:

The purpose of the Power Command information element is to provide the power level to be used by the mobile station.
Reference : [GSM_04.08] section 10.5.2.28

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Power Command | pow_cmd | 3 | 2 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| spare | .000 |  | 3 |
| Power Level | pow | 6.152 | 5 |

History:

07-May-97	VK	Initial

## Power Command & Access Type

Description:

The purpose of the Power Command and access type information element is to provide the power level to be used by the mobile station and the indication that the mobile station can avoid the transmission of handover access.
Reference : [GSM_04.08] section 10.5.2.28a

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Power Command & access type | pow_cmd_access | 3 | 2 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Access Type Control | atc | 6.11 | 1 |
| spare | .00 |  | 2 |
| Power Level | pow | 6.152 | 5 |

History:

07-May-97	VK	Initial

## Power Control Parameters

Description:

The Power Control Parameters information element contains parameters the mobile station shall use to determine its TX power level.
Reference : [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment), GSM_04.60 section 12.13

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Power Control Parameters | pwr_ctrl | 12-52 |

Elements:

| long name | short name | ref | bit len | ctrl |
| --- | --- | --- | --- | --- |
| Alpha | alpha | 6.15 | 4 |  |
| Tagged Gamma | tagged_gamma | 5.101 | 1-6 | [8] |

History:

27-Apr-00	LG	Initial

## RACH Control Parameters

Description:

The purpose of the RACH Control Parameters information element is to provide parameters used to control the RACH utilization. This information element is broadcast to mobile stations in SYSTEM INFORMATION TYPE 1, 2, 2bis, 3, and 4 messages.
Reference : [GSM_04.08] section 10.5.2.29

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| RACH Control Parameters | rach_ctrl | 3 | 4 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Maximum Number of Retransmissions | max_retrans | 6.109 | 2 |
| Tx-integer | tx_integer | 6.224 | 4 |
| Cell Barred for Access | cell_bar_access | 6.41 | 1 |
| Call Reestablishment allowed | re | 6.39 | 1 |
| Access Control Class | ac | 6.10 | 16 |

History:

07-May-97	VK	Initial

## Request Reference

Description:

The purpose of the Request Reference information element is to provide the random access information used in the channel request and the frame number, FN modulo 42432 in which the channel request was received.
Reference : [GSM_04.08] section 10.5.2.30

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Request Reference | req_ref | 3 | 4 |
| Request Reference | req_ref_2 | 3 | 4 |
| Request Reference | req_ref_3 | 3 | 4 |
| Request Reference | req_ref_4 | 3 | 4 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Random Access Information | ra | 6.161 | 8 |
| T1' | t1 | 6.199 | 5 |
| T3 | t3 | 6.201 | 6 |
| T2 | t2 | 6.200 | 5 |

History:

07-May-97	VK	Initial

## Routing Area Identification

Description:

The purpose of the Routing Area Identification information element is to provide an unambiguous identification of routing areas within the areas covered by the GSM system.

Reference : [1] section 10.5.5.15

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Routing Area Identification | rout_area_id | 3 | 7 |

Elements:

| long name | short name | ref | bit len | Ctrl |
| --- | --- | --- | --- | --- |
| Mobile Country Code | mcc | 6.117 | 4 | BCDEVEN[3] |
| Mobile Network Code | mnc | 6.118 | 4 | BCDEVEN[2] |
| Location Area Code | lac | 6.103 | 16 |  |
| Routing Area Code | rac | 6.172 | 8 |  |

History:

27-Apr-00	LG	Initial

## Scheduling Info

Description:

Reference : [1] section 10.5.2.37a (SI 9 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Scheduling Info | sched | 12-819 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| Info Type and Positions | itp | 5.40 | 11-133 | M |  |  |
| Info Type and Positions Optional | itpo | 5.40 | 11-133 | O | CSN1_S1 | [0..MAX_ITP] |

History:

27-Apr-00	LG	Initial

## SI 1 Rest Octets

Description:

The SI 1 Rest Octets information element is coded as followed:

{SI1 rest octet} ::={ H{NCH Position} | L}
			{Spare padding}
Reference : [1] section 10.5.2.32

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| SI 1 Rest Octets | si1_rest_oct | 8 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| NCH Position | nch_position | 6.128 | 5 | O | CSN1_SHL |  |
| Spare Padding | .00101011 |  | 0-7 | O | S_PADDING | (22) |

History:

25-Oct-99	VK	Re-introduce rest octets
13-Apr-00	LG	defined as a structure

## SI 3 Rest Octets

Description:

The SI 3 Rest Octets information element is coded as followed:

{SI3 rest octet} ::={Optional selection parameter}
			{Optional Power offset}
			{System Information 2ter Indicator}
			{Early Classmark Sending Control}
			{Scheduling if and where}
			{H {GPRS Indicator} | L}
			{Spare padding}

{Optional Selection Parameter} ::= H {Selection Parameter} | L

{Selection Parameter} ::= {CBQ} {CELL_RESELECT_OFFSET}
			{TEMPORARY_OFFSET} {PENALTY_TIME}

{Optional Power Offset} ::= H {Power Offset} | L

{System Information 2ter Indicator} ::= H | L

{Early Classmark Sending Control} ::= H | L

Scheduling if and where ::= {H {where} | L}

GPRS Indicator ::= {RA color}{SI13 position} 

Reference : [GSM_04.08] section 10.5.2.34

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| SI 3 Rest Octets | si3_rest_oct |  | 8-32 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| Optional Selection Parameters | opt_sel_par | 5.61 | 15 | O | CSN1_SHL |  |
| Power Offset | pow_offs | 6.153 | 2 | O | CSN1_SHL |  |
| System Information 2ter Indicator | si2ter_ind | 5.99 | 0 | O | CSN1_SHL |  |
| Early Classmark Sending Tag | es_ind_tag | 5.19 | 0 | O | CSN1_SHL |  |
| If And Where | if_and_where | 6.92 | 3 | O | CSN1_SHL |  |
| GPRS Indicator | gprs_indic | 5.27 | 4 | O | CSN1_SHL |  |
| Spare Padding | .00101011 |  | 0-7 | O | S_PADDING | (22) |

History:

05-Apr-00	LG	Initial

## SI 4 Rest Octets

Description:

The SI 4 Rest Octets information element includes parameters which are used by the mobile station for cell reselection purposes. It may also include the POWER OFFSET parameter used by DCS1800 Class 3 MS.

The SI 4 Rest Octets information element is coded as followed:

{SI4 rest octet} ::={SI4 Rest Octets_0}
			{H {SI Rest Octets_S} | L}
			{Spare padding}

{ SI4 Rest Octets_0} ::={Optional selection parameters}

{Optional Power offset}{H {GPRS Indicator} | L}

{SI4 Rest Octets_S} ::={H {LSA Parameters} | L}

{H {Cell Identity} | L}{H {LSA ID information} | L}

{Optional Selection Parameters} ::= H {Selection Parameters} | L

{Selection Parameter} ::= {CBQ} {CELL_RESELECT_OFFSET}
			{TEMPORARY_OFFSET} {PENALTY_TIME}

{Optional Power Offset} ::= H {Power Offset} | L

GPRS Indicator ::= {RA color}{SI13 position} 

{LSA Parameters} ::= {Priority Threshold}{LSA Offset}{1 {Optional MCC}{MNC} | 0}

{LSA ID Information} ::= {LSA identity} {0 | 1 {LSA_ID information}}

{LSA identity} ::= {0 {LSA_ID} | 1 {ShortLSA_ID}}

Reference : [GSM_04.08] section 10.5.2.35, 10.5.2.36, 10.5.2.37

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| SI 4 Rest Octets | si4_rest_oct |  | 8-88 |
| SI 7 Rest Octets | si7_rest_oct |  | 8-88 |
| SI 8 Rest Octets | si8_rest_oct |  | 8-88 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| Optional selection Parameters | opt_sel_par | 5.61 | 15 | O | CSN1_SHL |  |
| Power Offset | pow_offs | 6.153 | 2 | O | CSN1_SHL |  |
| GPRS Indicator | gprs_indic | 5.27 | 4 | O | CSN1_SHL |  |
| LSA Parameters | lsa_param | 5.44 | 20-31 | O | CSN1_SHL |  |
| Cell Identity | cell_ident | 6.43 | 16 | O | CSN1_SHL |  |
| LSA ID Information | lsa_id_info | 5.43 | 11-76 | O | CSN1_SHL |  |
| Spare Padding | .00101011 |  | 0-7 | O | S_PADDING | (22) |

History:

05-Apr-00	LG	Initial

## SI 6 Rest Octets

Description:

The SI 6 Rest Octets information element contains information about the paging, notification channels, VBS and VCGS services of the cell.

The SI 6 Rest Octets information element is coded as followed:

{SI6 rest octet} ::={H {PCH and NCH info} | L}
			{H {VBS/VCGS options} | L}
			{Spare padding}

{PCH and NCH info} ::= {Paging channel restructuring} 
			{NLN(SACCH)}
			{Call priority}
			{NLN status}

{VBS/VCGS options} ::= {inband notifications} {inband paging}

Reference : [GSM_04.08] section 10.5.2.35a

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| SI 6 Rest Octets | si6_rest_oct |  | 8-16 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| PCH and NCH Info | pch_nch_info | 5.79 | 7 | O | CSN1_SHL |  |
| VBS/VGCS Options | vbs_vgcs_opt | 5.110 | 2 | O | CSN1_SHL |  |
| Spare Padding | .00101011 |  | 0-7 | O | S_PADDING | (18) |

History:

13-Apr-00	LG	Initial

## SI 9 Rest Octets

Description:

The SI 9 Rest Octets information element contains information about scheduling of some or all of the information on the BCCH.

The SI 9 Rest Octets information element is coded as followed:

{SI9 Rest Octet} ::={H {Scheduling Info} | L}
			{Spare padding}

{Scheduling Info} ::= {Info Type}{Positions} 
			{0 | 1 {Scheduling Info}}

{Info Type} ::= 	0{Info Type 4} 
			| 10{Info Type 5}
			| 11{Info Type 6}

{Positions}  ::= 	{Position}{0 | 1 {Positions}}

{Position}  ::= 	{Modulus}{Relative Position}{BCCH Type}

Reference : [GSM_04.08] section 10.5.2.37a

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| SI 9 Rest Octets | si9_rest_oct |  | 17 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| Scheduling Info | sched | 5.88 | 12-819 | O | CSN1_SHL |  |
| Spare Padding | .00101011 |  | 0-7 | O | S_PADDING | (22) |

History:

27-Apr-00	LG	Initial

## SI 13 Info

Description:

Reference : [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets)

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| SI 13 Info | si13_info |  | 24-331 |

Elements:

| long name | short name | ref | bit len | pres | ctrl |
| --- | --- | --- | --- | --- | --- |
| BCCH Change Mark | bcch_cm | 6.27 | 3 | M |  |
| SI Change Field | si_cf | 6.188 | 4 | M |  |
| Flag | flag | 6.77 | 1 | M |  |
| SI 13 Change Mark | si13_cm | 6.187 | 2 | C | {flag=1} |
| GPRS Mobile Allocation | gprs_ma | 5.29 | 9-97 | C | {flag=1} |
| Flag1 | flag1 | 6.77 | 1 | M |  |
| Routing Area Code | rac | 6.172 | 8 | C | {flag1=0} |
| Split PG Cycle Support | spgc | 6.192 | 1 | C | {flag1=0} |
| Priority Access Threshold | prio_acc_thr | 6.155 | 3 | C | {flag1=0} |
| Network Control Order | nco | 6.131 | 2 | C | {flag1=0} |
| GPRS Cell Options | gprs_opt | 5.26 | 34-98 | C | {flag1=0} |
| Alpha | alpha | 6.15 | 4 | C | {flag1=0} |
| T_AVG_W Value | t_avg_w | 6.207 | 5 | C | {flag1=0} |
| T_AVG_T Value | t_avg_t | 6.206 | 5 | C | {flag1=0} |
| Power Control Measurement Channel | pc_meas_chan | 6.147 | 1 | C | {flag1=0} |
| N_AVG_I Value | n_avg_i | 6.125 | 4 | C | {flag1=0} |
| PSI1 Repeat Period | psi1_period | 6.158 | 4 | C | {flag1=1} |
| PBCCH Description | pbcch_des | 5.78 | 11-22 | C | {flag1=1} |

History:

27-Apr-00	LG	Initial

## SI 13 Rest Octets

Description:

Reference : [GSM_04.08] section 10.5.2.37b

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| SI 13 Rest Octets | si13_rest_oct |  | 21 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| SI 13 Info | si13_info | 5.94 | 24-331 |  | CSN1_SHL |  |
| Spare Padding | .00101011 |  | 0-7 | O | S_PADDING | (22) |

History:

27-Apr-00	LG	Initial

## SI 16 Rest Octets

Description:

The SI 16 Rest Octets information element includes parameters which are used by the mobile station for cell selection and reselection purposes.

Reference : [GSM_04.08] section 10.5.2.37d

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| SI 16 Rest Octets | si16_rest_oct |  | 21 |
| SI 17 Rest Octets | si17_rest_oct |  | 21 |

Elements:

| long name | short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| LSA Parameters and LSA ID | lsa_par_id | 5.45 | 45-108 |  | CSN1_SHL |  |
| Spare Padding | .00101011 |  | 0-7 | O | S_PADDING | (22) |

History:

28-Apr-00	LG	Initial

## Single Block Allocation

Description:

The Single Block Allocation information element contains parameter necessary to define the radio resources of a Single Block Allocation. For example for sending of a Packet Resource Request message in a two phase access or a Measurement Report.
Reference : [GSM_04.08] section 10.5.2.25c

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Single Block Allocation | single_alloc | 9-13 |

Elements:

| long name | short name | ref | bit len | type |
| --- | --- | --- | --- | --- |
| Timeslot Number | tn | 6.214 | 3 |  |
| Alpha Gamma | alpha_gamma | 5.1 | 9 | CSN1_S1 |
| P0 and BTS Power Control Mode | p0_pwr_ctrl_mode | 5.62 | 5 | CSN1_S1 |

History:

26-Apr-00	LG	Initial

23-Oct-00	FRI	Inserted P0andBTS, changed alpha & gamma to alpha_gamma

## Starting Time

Description:

The purpose of the Starting Time information element is to provide the start TDMA frame number, FN modulo 42432.
Reference : [GSM_04.08] section 10.5.2.38

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Starting Time | start_time | 3 | 3 |
| TBF Starting Time | tbf_start_time |  | 2 |
| Measurement Starting Time | meas_start |  | 2 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| T1' | t1 | 6.199 | 5 |
| T3 | t3 | 6.201 | 6 |
| T2 | t2 | 6.200 | 5 |

History:

07-May-97	VK	Initial
25-Apr-00	LG	tbf_start_time and meas_start added

## System Information 2ter Indicator

Description:

The parameter is used to indicate inside the SI 3 rest octets the presence of a SYTEM INFORMATION TYPE 2ter message.
Reference : [GSM_04.08] section 10.5.2.34

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| system information 2ter indicator | si2ter_ind | 0 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |

History:

05-Apr-00	LG	Initial

## Synchronization Indication

Description:

The purpose of Synchronization Indication information element is to indicate which type of handover is to be performed.
Reference : [GSM_04.08] section 10.5.2.39

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Synchronization Indication | synch_ind | 1 | 1 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Normal cell indication | nci | 6.135 | 1 |
| Report Observed Time Difference | rot | 6.163 | 1 |
| Synchronisation indication | si | 6.198 | 2 |

History:

07-May-97	VK	Initial

## Tagged Gamma

Description:

Reference : [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Tagged Gamma | tagged_gamma | 1-6 |

Elements:

| long name | short name | ref | bit len | type |
| --- | --- | --- | --- | --- |
| Gamma | gamma | 6.84 | 5 | CSN1_SHL |

History:

26-Apr-00	LG	Initial

## Tagged Priority

Description:

Reference : [GSM_04.08] section 10.5.2.23, section 10.5.2.24, section 10.5.2.25

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| Tagged Priority | tagged_call_prio |  | 1-4 |

Elements:

| long name | short name | ref | bit len | type |
| --- | --- | --- | --- | --- |
| Call Priority | call_prio | 6.38 | 3 | CSN1_SHL |

History:

26-Apr-00	LG	Initial

## Tagged USF For Timeslot

Description:

Reference : [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| Tagged USF For Timeslot | tagged_usf_tn |  | 1-4 |

Elements:

| long name | short name | ref | bit len | type |
| --- | --- | --- | --- | --- |
| Uplink State Flag | usf | 6.227 | 3 | CSN1_S1 |

History:

26-Apr-00	LG	Initial

23-Oct-00	FRI	corrected CSN1 flag

## Timing Advance

Description:

The purpose of the Timing Advance information element is to provide the timing advance value.
Reference : [GSM_04.08] section 10.5.2.40

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| Timing Advance | time_advance | 3 | 8 |
| Timing Advance | time_advance_2 | 3 | 8 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| spare | .00 |  | 2 |
| Timing advance value | ta | 6.219 | 6 |

History:

07-May-97	VK	Initial

## Timing Advance Index and Number

Description:

The Timing Advance Index and Number field describes the timing advance mode assigned to the mobile station.
Reference : [GSM_04.08] section 10.5.2.26 , GSM_08.60 section 12.12 (Packet Timing Advance)

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| Timing Advance Index and Number | ta_idx_nm |  | 7 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Timing Advance Index | ta_idx | 6.216 | 4 |
| Timing Advance Timeslot Number | ta_nm | 6.217 | 3 |

History:

26-Apr-00	LG	Initial

## TFI Assignment and Allocation

Description:

The TFI Assignment and Allocation field is part of the packet uplink assignment of IA rest octets.

Reference : [GSM_04.08] section 10.5.2.16

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| TFI Assignment and Allocation | tfi_ass_alloc |  | 15-48 |

Elements:

| long name | short name | ref | bit len | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| TFI Assignment | tfi | 6.211 | 5 |  |  |
| Polling | poll | 6.150 | 1 |  |  |
| Allocation flag | allo_flag | 6.77 | 1 |  |  |
| Uplink State Flag | usf | 6.227 | 3 |  | {allo_flag=0} |
| USF Granularity | usf_gran | 6.228 | 1 |  | {allo_flag=0} |
| P0 and PR Mode | p0_prmode | 5.63 | 5 | CSN1_S1 | {allo_flag=0} |
| Allocation Bitmap Length | allo_len5 | 6.17 | 5 |  | {allo_flag#0} |
| Allocation Bitmap | allo_bmp5 | 6.16 | 1 |  | {allo_flag#0}[allo_len5..31] |
| P0 BTS PR Mode | p0_bts_prmode | 5.64 | 6 | CSN1_S1 | {allo_flag#0} |
| Channel Coding Command | ccm | 6.58 | 2 |  |  |
| TLLI Block Channel Coding | tlli_bcc | 6.220 | 1 |  |  |

History:

25-Apr-00	LG	Initial

12-Feb-01	CAN	Allocation Bitmap bit len set to 1

## TFI Assignment and RLC Mode

Description:

The TFI Assignment and RLC Mode field is part of the packet Downlink assignment of IA rest octets.

Reference : [GSM_04.08] section 10.5.2.16

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| TFI Assignment and RLC Mode | tfi_ass_rlc |  | 14-18 |

Elements:

| long name | short name | ref | bit len | type |
| --- | --- | --- | --- | --- |
| TFI Assignment | tfi | 6.211 | 5 |  |
| RLC Mode | rlc_mode | 6.170 | 1 |  |
| Alpha | alpha | 6.15 | 4 | CSN1_S1 |
| Gamma | gamma | 6.84 | 5 |  |
| Polling | poll | 6.150 | 1 |  |
| Timing Advance Valid | ta_valid | 6.218 | 1 |  |

History:

25-Apr-00	LG	Initial

## USF For Timeslot And Gamma (CSN1)

Description:

The USF For Timeslot And Gamma field contains an uplink state flag and a gamma value. 
Reference : [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| USF For Timeslot And Gamma (CSN1) | usf_gamma_csn1 |  | 1-9 |

Elements:

| long name | short name | ref | bit len | type |
| --- | --- | --- | --- | --- |
| USF For Timeslot And Gamma | usf_gamma | 5.109 | 8 | CSN1_S1 |

History:

23-Oct-00	FRI	Initial

## USF For Timeslot And Gamma

Description:

The USF For Timeslot And Gamma field contains an uplink state flag and a gamma value. 
Reference : [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| USF For Timeslot And Gamma | usf_gamma |  | 8 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Uplink State Flag | usf | 6.227 | 3 |
| Gamma | gamma | 6.84 | 5 |

History:

26-Apr-00	LG	Initial

23-Oct-00	FRI	removed CSN1_S1 for usf (s. 5.108)

## VBS/VGCS Options

Description:

VBS/VGCS Options are sent as part of SI6 rest octets.
Reference : [GSM_04.08] section 10.5.2.35a

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| VBS/VGCS Options | vbs_vgcs_opt |  | 2 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Inband Notification | inband_not | 6.93 | 1 |
| Inband pagings | inband_pag | 6.94 | 1 |

History:

13-Apr-00	LG	Initial

## VGCS Target Mode Indication

Description:

Note: The recommendation is ambiguous: In text is explained "... is a type 3 information element with 2 octets length", but in message desciptions and in the desciptions of the IE occurs a length.
Reference : [GSM_04.08] section 10.5.2.42a

Definition:

| long name | short name | type | len |
| --- | --- | --- | --- |
| VGCS Target Mode Indication | vgcs_tmi | 4 | 2 |

Elements:

| long name | short name | ref | bit len |
| --- | --- | --- | --- |
| Target mode | tm | 6.208 | 2 |
| Group Cipher Key Number | group_ckn | 6.86 | 4 |
| Spare | .11 |  | 2 |

History:

28-Apr-00	LG	Initial

## Fixed Allocation Block Structure

Description:

The Fixed Allocation information element contains parameters necessary to define the radio resources of a fixed allocation.
Reference : [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Fixed Allocation Block Structure | fa_blk_ia | 8 |

Elements:

| long name | short name | ref | bit len | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| Blocks Or Block Periods | blp | 6.30 | 1 |  |  |
| Allocation Bitmap Length | allo_len7 | 6.17 | 7 |  |  |
| Allocation Bitmap | allo_bmp7 | 6.16 | 1 |  | [allo_len7..127] |

History:

12-Feb-01	CAN	Initial

# Basic Elements

## A5/1

Description:

Reference :	[GSM_04.08] section 10.5.1.5 (Mobile Station Classmark 1)
		[GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| A5/1 | a5_1 | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | A5_1_YES |  |
| 1 | A5_1_NO |  |

History:

07-May-97	VK	Initial

## A5/2

Description:

Reference :	[GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| A5/2 | a5_2 | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | A5_2_NO | encryption algorithm A5/2 not available |
| 1 | A5_2_YES | encryption algorithm A5/2 available |

History:

07-May-97	VK	Initial

## A5/3

Description:

Reference :	[GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| A5/3 | a5_3 | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | A5_3_NO | encryption algorithm A5/3 not available |
| 1 | A5_3_YES | encryption algorithm A5/3 available |

History:

07-May-97	VK	Initial

## A5/4

Description:

Reference :	[GSM_04.08] section 10.5.1.7 (Mobile Station Classmark 3)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| A5/4 | a5_4 | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | A5_4_NO | encryption algorithm A5/4 not available |
| 1 | A5_4_YES | encryption algorithm A5/4 available |

History:

07-May-97	VK	Initial

## A5/5

Description:

Reference :	[GSM_04.08] section 10.5.1.7 (Mobile Station Classmark 3)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| A5/5 | a5_5 | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | A5_5_NO | encryption algorithm A5/5 not available |
| 1 | A5_5_YES | encryption algorithm A5/5 available |

History:

07-May-97	VK	Initial

## A5/6

Description:

Reference :	[GSM_04.08] section 10.5.1.7 (Mobile Station Classmark 3)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| A5/6 | a5_6 | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | A5_6_NO | encryption algorithm A5/6 not available |
| 1 | A5_6_YES | encryption algorithm A5/6 available |

History:

07-May-97	VK	Initial

## A5/7

Description:

Reference :	[GSM_04.08] section 10.5.1.7 (Mobile Station Classmark 3)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| A5/7 | a5_7 | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | A5_7_NO | encryption algorithm A5/7 not available |
| 1 | A5_7_YES | encryption algorithm A5/7 available |

History:

07-May-97	VK	Initial

## Absolute RF Channel Number

Description:

Reference : [GSM_04.08] section 10.5.2.5 (Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Absolute RF Channel Number | arfcn | 10 |

History:

07-May-97	VK	Initial

## Access Burst Type

Description:

The ACCESS_BURST_TYPE field indicates if 8 or 11 bit access burst shall be used on PRACH and in the PACKET CONTROL ACKNOWLEDGMENT message when the format is four access bursts.
Reference :	[GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24 (GPRS Cell Options)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Access Burst Type | ab_type | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | AB_8_BIT | 8 bit access burst shall be used |
| 1 | AB_11_BIT | 11 bit access burst shall be used |

History:

31-May-99	SL	Initial

## Access Control Class

Description:

Reference :	[GSM_04.08] section 10.5.1.29 (RACH Control Parameters)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Access Control Class | ac | 16 |

History:

07-May-97	VK	Initial

## Access Type Control

Description:

Reference :	[GSM_04.08] section 10.5.2.28a (Power Command and Access Type)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Access Type Control | atc | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | ATC_HANDOV_MAN | Sending of Handover access is mandatory |
| 1 | ATC_HANDOV_OPT | Sending of Handover access is optional |

History:

07-May-97	VK	Initial

## Acknowledge Flag

Description:

Reference :	[GSM_04.08] section 10.5.1.9 (Descriptive Group Or Broadcast Call Reference), 10.5.2.22c (NT/N Rest octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Acknowledge Flag | af | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | AF_NO | acknowledgement is not required |
| 1 | AF_YES | acknowledgement is required |

History:

26-Apr-00	LG	Initial

## ACS

Description:

Additional reselect parameter indication.
Reference : [GSM_04.08] section 10.5.2.4 (Cell Selection Parameters)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Additional reselect parameter indication | acs | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | ACS_USE_SI4 | The SI 4 rest octets, if present, shall be used to derive the value of PI and possibly C2 parameters and/or other parameters |
| 1 | ACS_USE_SI7 | The value of PI and possibly C2 parameters and/or other parameters in a System information type 7 or type 8 message shall be used |

History:

07-May-97	VK	Initial

## Algorithm Identifier

Description:

Reference : [GSM_04.08] section 10.5.2.9 (Cipher Mode Setting)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| algorithm identifier | algo_ident | 3 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | ALGO_A5_1 | cipher with algorithm A5/1 |
| 1 | ALGO_A5_2 | cipher with algorithm A5/2 |
| 2 | ALGO_A5_3 | cipher with algorithm A5/3 |
| 3 | ALGO_A5_4 | cipher with algorithm A5/4 |
| 4 | ALGO_A5_5 | cipher with algorithm A5/5 |
| 5 | ALGO_A5_6 | cipher with algorithm A5/6 |
| 6 | ALGO_A5_7 | cipher with algorithm A5/7 |
| DEF |  | reserved |

History:

07-May-97	VK	Initial

## Alpha

Description:

The Alpha field is used for power output control.
Reference : [GSM_04.08] section 10.5.2.16 (IA Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Alpha | alpha | 4 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0-10 |  | binary representation of tenth of alpha (measured in dB) |
| DEF |  | Reserved |

History:

25-Apr-00	LG	Initial

## Allocation Bitmap

Description:

Reference : [GSM_04.08] section 10.5.2.16 (IA Rest Octets), section 10.5.2.22c (RR Packet Uplink Assignment), section 10.5.2.37b (SI13 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Allocation Bitmap | allo_bmp5 | 1 |
| Allocation Bitmap | allo_bmp6 | 1 |
| Allocation Bitmap | allo_bmp7 | 1 |

History:

25-Apr-00	LG	Initial

12-Feb-01	CAN	bit len set to 1

## Allocation Bitmap Length

Description:

The Allocation Bitmap Length describes the length of Allocation Bitmap.

Reference : [GSM_04.08] section 10.5.2.16 (IA Rest Octets), section 10.5.2.37b (SI13 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Allocation Bitmap Length | allo_len5 | 5 |
| Allocation Bitmap Length | allo_len6 | 6 |
| Allocation Bitmap Length | allo_len7 | 7 |

History:

25-Apr-00	LG	Initial

## Allocation Bitmap Var

Description:

Reference : [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Allocation Bitmap Var | allo_var | 1 |

History:

25-Oct-00	FRI	Initial

12-Feb-01	CAN	bit len set to 1

## ARFCN Index

Description:

Reference : [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets) [GSM_04.60] section 12.10a (GPRS Mobile Allocation)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| ARFCN Index | arfcn_idx | 6 |

History:

28-Apr-00	LG	Initial

## Associated Radio Capability 1

Description:

Reference : [GSM_04.08] section 10.5.1.7 (Mobile Station Classmark 3)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Associated Radio Capability 1 | radio_cap_1 | 4 |

History:

07-May-97	VK	Initial

## Associated Radio Capability 2

Description:

Reference : [GSM_04.08] section 10.5.1.7 (Mobile Station Classmark 3)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Associated Radio Capability 2 | radio_cap_2 | 4 |

History:

07-May-97	VK	Initial

## Attach / detach allowed

Description:

Reference : [GSM_04.08] section 10.5.2.11 (Control Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Attach / detach allowed | att | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | CCD_ATT_NO | MSs are not allowed to apply IMSI attach and detach procedure. |
| 1 | CCD_ATT_YES | MSs in the cell shall apply IMSI attach and detach procedure. |

History:

07-May-97	VK	Initial

## BA used

Description:

Reference : [GSM_04.08] section 10.5.2.20 (Measurement Results)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| BA-USED | ba_used | 1 |

History:

07-May-97	VK	Initial

## Multiband Support

Description:

Reference : [GSM_04.08] section 10.5.1.7 (Mobile Station Classmark 3)

Definition:

| long name | Short name | bit len |
| --- | --- | --- |
| Multiband support | mb_sup | 3 |

History:

29-Jan-01	LE	Initial

## BCCH ARFCN (hi part)

Description:

Reference : [GSM_04.08] section 10.5.2.2 (Cell Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| BCCH ARFCN | bcch_arfcn_hi | 2 |

History:

03-Feb-98	VK	Initial

## BCCH ARFCN (lo part)

Description:

Reference : [GSM_04.08] section 10.5.2.2 (Cell Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| BCCH ARFCN | bcch_arfcn_lo | 8 |

History:

03-Feb-98	VK	Initial

## BCCH Change Mark

Description:

This field indicates the status of the information an the BCCH. The value of this field may be changed when information on BCCH is changed.
Reference : [GSM_04.08] section 10.5.2.37b (SI 13 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| BCCH Change Mark | bcch_cm | 3 |

History:

27-Apr-00	LG	Initial

## BCCH Frequency Ncell

Description:

Reference : [GSM_04.08] section 10.5.2.20 (Measurement Result)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| BCCH Frequency Ncell | bcch_ncell | 5 |

History:

07-May-97	VK	Initial

## BCCH Type

Description:

Reference : [GSM_04.08] section 10.5.2.37a (SI 9 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| BCCH Type | bcch_type | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | BCCH_NORM | on BCCH Norm |
| 1 | BCCH_EXT | on BCCH Ext. |

History:

27-Apr-00	LG	Initial

## Blocks Or Block Periods

Description:

This information field indicates whether the allocation bit map is to be interpreted as blocks or as block periods.
Reference : [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Blocks Or Block Periods | blp | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | BLP_BLK | The allocation bitmap is to be interpreted as blocks. |
| 1 | BLP_PER | The allocation bitmap is to be interpreted as block periods. |

History:

27-Apr-00	LG	Initial

## BS Access Grant Blocks reserved

Description:

Reference : [GSM_04.08] section 10.5.2.11 (Control Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| BS Access Grant Blocks reserved | bs_ag_blks_res | 3 |

History:

07-May-97	VK	Initial

## BS Colour Code

Description:

The BCC field is coded as the binary representation of the BS colour code (see TS. GSM 03.03).
Reference : [GSM_04.08] section 10.5.2.2 (Cell Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| BS Colour Code | bcc | 3 |

History:

07-May-97	VK	Initial

## BS_CV_MAX

Description:

CV – countdown value
Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24 (GPRS Cell Options)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| BS_CV_MAX | bs_cv_max | 4 |

History:

31-May-99	SL	Initial

## BSIC

Description:

Reference :	[GSM_04.08] section 10.5.2.20 (Measurement Results)
		[GSM_04.08] section 9.1.30 (Synchronisation Channel Information)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| BSIC | bsic | 6 |

History:

07-May-97	VK	Initial

## BS Paging Multiframes

Description:

Reference : [GSM_04.08] section 10.5.2.11 (Control Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| BS Paging Multiframes | bs_pa_mfrms | 3 |

History:

07-May-97	VK	Initial

## BTS Power Control Mode

Description:

The  BTS Power Control Mode field indicates the power control mode used by the network as defined in GSM 05.08.

Reference : [GSM_04.08] section 10.5.2.16 (IA Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| BTS Power Control Mode | pwr_ctrl_mode | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | PWR_CTR_A | mode A |
| 1 | PWR_CTR_B | mode B |

History:

03-May-00	LG	Initial

## C Value

Description:

The C Value field is the C parameter as defined in GSM 05.08. 
Reference : [GSM_04.08] section 10.5.2.20a (GPRS Measurement Results)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| C Value | c_val | 6 |

History:

25-Apr-00	LG	Initial

## Call Priority

Description:

Reference : [GSM_04.08] section 10.5.1.9 (Descriptive group or broadcast call reference), 10.5.1.11 (Priority level), 10.5.2.23 (P1 rest octets), 10.5.2.24 (P2 rest octets), 10.5.2.25 (P3 rest octets), 10.5.2.35a (SI 6 rest octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Call Priority | call_prio | 3 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | CALL_PRIO_NO | no priority applied |
| 1 | CALL_PRIO_LEV_4 | call priority level 4 |
| 2 | CALL_PRIO_LEV_3 | call priority level 3 |
| 3 | CALL_PRIO_LEV_2 | call priority level 2 |
| 4 | CALL_PRIO_LEV_1 | call priority level 1 |
| 5 | CALL_PRIO_LEV_0 | call priority level 0 |
| 6 | CALL_PRIO_LEV_A | call priority level A |
| 7 | CALL_PRIO_LEV_B | call priority level B |

History:

13-Apr-00	LG	Initial

## Call Reestablishment allowed

Description:

Reference : [GSM_04.08] section 10.5.2.29 (RACH Control Parameters)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Call Reestablishment allowed | re | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | REESTAB_YES | Call Reestablishment allowed in the cell |
| 1 | REESTAB_NO | Call Reestablishment not allowed in the cell |

History:

07-May-97	VK	Initial

## Cell Bar Qualify

Description:

The parameter is used to control mobile station cell selection and reselection.
Reference : [GSM_04.08] section 10.5.2.34 (SI3 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Cell Bar Qualify | cell_bar_qual | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | CBQ_NO | cell selection and reselection depending on cell bar access |
| 1 | CBQ_YES | cell selection priority always low, reselection always normal |

History:

05-Apr-00	LG	Initial

## Cell Barred for Access

Description:

Reference : [GSM_04.08] section 10.5.2.29 (RACH Control Parameters)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Cell Barred for Access | cell_bar_access | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | BARRED_NO | The cell is not barred |
| 1 | BARRED_YES | The cell is barred |

History:

07-May-97	VK	Initial

## Cell Channel Description

Description:

The purpose of the Cell Channel Description information element is to provide the reference frequency list to be used to decode the mobile allocation information element. There are several formats for the Cell Channel Description information element, distinguished by the "format indicator" subfield. Some formats are frequency bit maps, the others use a special encoding scheme. NOTE: No more than 64 RF channels should be encoded in the Cell Allocation since this is the maximum number of RF channels which can be referenced in the Mobile Allocation IE.
Reference : [GSM_04.08] section 10.5.2.1b (Cell Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Cell Channel Description | cell_chan_desc | 128 |

History:

07-May-97	VK	Initial

## Cell Identity

Description:

The purpose of the Cell Identity information element is to identify a cell within a location area.
Reference : [GSM_04.08] section 10.5.1.1 (Cell Identity)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Cell identity | cell_ident | 16 |

History:

07-May-97	VK	Initial

23-Jul-97	LE	length of cell identity is 16 bit

## Cell Reselect Hysteresis

Description:

The usage of this information is defined in TS GSM 05.08.
Reference : [GSM_04.08] section 10.5.2.4 (Cell Selection Parameters)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| CELL-RESELECT HYSTERESIS | cell_resel_hyst | 3 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | CELL_HYST_0_DB | 0 dB RXLEV hysteresis for LA re-selection |
| 1 | CELL_HYST_2_DB | 2 dB RXLEV hysteresis for LA re-selection |
| 2 | CELL_HYST_4_DB | 4 dB RXLEV hysteresis for LA re-selection |
| 3 | CELL_HYST_6_DB | 6 dB RXLEV hysteresis for LA re-selection |
| 4 | CELL_HYST_8_DB | 8 dB RXLEV hysteresis for LA re-selection |
| 5 | CELL_HYST_10_DB | 10 dB RXLEV hysteresis for LA re-selection |
| 6 | CELL_HYST_12_DB | 12 dB RXLEV hysteresis for LA re-selection |
| 7 | CELL_HYST_14_DB | 14 dB RXLEV hysteresis for LA re-selection |

History:

07-May-97	VK	Initial

## Cell Reselect Offset

Description:

The parameter is used by the mobile station to apply an offset to the value of C2.
Reference : [GSM_04.08] section 10.5.2.34 (SI3 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Cell Reselect Offset | cell_resel_offs | 6 |

History:

05-Apr-00	LG	Initial

## Change Mark 1

Description:

The parameter is the binary representation of the allowed value of the SI change mark associated with the GPRS mobile allocation to which the MA number refers.
Reference : [GSM_04.08] section 10.5.2.25a (Packet Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Change Mark 1 | ch_mark1 | 2 |

History:

05-Apr-00	LG	Initial

## Channel Type and TDMA Offset

Description:

Reference : [GSM_04.08] section 10.5.2.5 (Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Channel type and TDMA offset | chan_type | 5 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 1 | TCH_F | TCH/F + ACCHs |
| 2 | TCH_H_S0 | TCH/H + ACCHs, subchannel 0 |
| 3 | TCH_H_S1 | TCH/H + ACCHs, subchannel 1 |
| 4 | SDCCH_4_S0 | SDCCH/4 + SDCCH/C4 or CBCH (SDCCH/4), subchannel 0 |
| 5 | SDCCH_4_S1 | SDCCH/4 + SDCCH/C4 or CBCH (SDCCH/4), subchannel 1 |
| 6 | SDCCH_4_S2 | SDCCH/4 + SDCCH/C4 or CBCH (SDCCH/4), subchannel 2 |
| 7 | SDCCH_4_S3 | SDCCH/4 + SDCCH/C4 or CBCH (SDCCH/4), subchannel 3 |
| 8 | SDCCH_8_S0 | SDCCH/8 + SDCCH/C8 or CBCH (SDCCH/8), subchannel 0 |
| 9 | SDCCH_8_S1 | SDCCH/8 + SDCCH/C8 or CBCH (SDCCH/8), subchannel 1 |
| 10 | SDCCH_8_S2 | SDCCH/8 + SDCCH/C8 or CBCH (SDCCH/8), subchannel 2 |
| 11 | SDCCH_8_S3 | SDCCH/8 + SDCCH/C8 or CBCH (SDCCH/8), subchannel 3 |
| 12 | SDCCH_8_S4 | SDCCH/8 + SDCCH/C8 or CBCH (SDCCH/8), subchannel 4 |
| 13 | SDCCH_8_S5 | SDCCH/8 + SDCCH/C8 or CBCH (SDCCH/8), subchannel 5 |
| 14 | SDCCH_8_S6 | SDCCH/8 + SDCCH/C8 or CBCH (SDCCH/8), subchannel 6 |
| 15 | SDCCH_8_S7 | SDCCH/8 + SDCCH/C8 or CBCH (SDCCH/8), subchannel 7 |
| 16 | TCH_F_NO_ADD | TCH/F + FACCH/F + SACCH/M, no additional timeslots |
| 17 | TCH_F_ADD_BI1 | TCH/F+ACCHs, additional bidirectional TCH/F/SACCH/M on timeslot n-1 |
| 18 | TCH_F_ADD_BI2 | TCH/F+ACCHs, additional bidirectional TCH/F/SACCH/M on timeslot n+1, n-1 |
| 19 | TCH_F_ADD_BI3 | TCH/F+ACCHs, additional bidirectional TCH/F/SACCH/M on timeslot n+1, n-1, n-2 |
| 20 | TCH_F_ADD_BI4 | TCH/F+ACCHs, additional bidirectional TCH/F/SACCH/M on timeslot n+1, n-1 until n-3 |
| 21 | TCH_F_ADD_BI5 | TCH/F+ACCHs, additional bidirectional TCH/F/SACCH/M on timeslot n+1, n-1 until n-4 |
| 22 | TCH_F_ADD_BI6 | TCH/F+ACCHs, additional bidirectional TCH/F/SACCH/M on timeslot n+1, n-1 until n-5 |
| 23 | TCH_F_ADD_BI7 | TCH/F+ACCHs, additional bidirectional TCH/F/SACCH/M on timeslot n+1, n-1 until n-6 |
| 24 | TCH_F_ADD_UNI1 | TCH/F+ACCHs, additional unidirectional TCH/FD/SACCH/MD on timeslot n-1 |
| 25 | TCH_F_ADD_UNI2 | TCH/F+ACCHs, additional unidirectional TCH/FD/SACCH/MD on timeslot n+1, n-1 |
| 26 | TCH_F_ADD_UNI3 | TCH/F+ACCHs, additional unidirectional TCH/FD/SACCH/MD on timeslot n+1, n-1 , n-2 |
| 30 | TCH_F_ADD_BI_UNI | TCH/F+ACCHs, additional bidirectional TCH/F/SACCH/M and unidirectional TCH/FD/SACCH/MD on timeslot n+1, n-1 |
| DEF |  | channel not defined |

History:

07-May-97	VK	Initial
20-Apr-00	LG	added values for channel description 2

## Channel Needed Info

Description:

Reference : [GSM_04.08] section 10.5.2.8 (Channel Needed) (cn1 and cn2),  section 10.5.2.24 (P2 Rest Octets), section 10.5.2.25 (P3 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Channel Needed for Mobile ID 1 | cn1 | 2 |
| Channel Needed for Mobile ID 2 | cn2 | 2 |
| Channel Needed for Mobile ID 3 | cn3 | 2 |
| Channel Needed for Mobile ID 4 | cn4 | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | CN_ANY_CHAN | Any channel. |
| 1 | CN_SDCCH | SDCCH. |
| 2 | CN_TCH_F | TCH/F (Full rate). |
| 3 | CN_TCH | TCH/H or TCH/F (Dual rate). |

History:

07-May-97	VK	Initial

26-Apr-00	LG	added cn3 and cn4

## Channel Mode

Description:

The Channel Mode information element gives information of the mode on coding/decoding and transcoding. The exact mode is determined by the contents of this IE and the channel type.
Reference : [GSM_04.08] section 10.5.2.6

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Channel Mode | chan_mode | 8 |
| Mode Of 2nd Channel Set | chan_mode2 | 8 |
| Mode Of 3rd Channel Set | chan_mode3 | 8 |
| Mode Of 4th Channel Set | chan_mode4 | 8 |
| Mode Of 5th Channel Set | chan_mode5 | 8 |
| Mode Of 6th Channel Set | chan_mode6 | 8 |
| Mode Of 7th Channel Set | chan_mode7 | 8 |
| Mode Of 8th Channel Set | chan_mode8 | 8 |

Values:

| Value | c-macro | comment |
| --- | --- | --- |
| 0 | CM_SIG_ONLY | signaling only |
| 1 | CM_SPEECH | speech (full rate or half rate), version 1 |
| 0b00000001 | CM_SPEECH_VER1 | speech (full rate or half rate), version 1 |
| 0b00100001 | CM_SPEECH_VER2 | speech (full rate or half rate), version 2 |
| 0b01000001 | CM_SPEECH_VER3 | speech (full rate or half rate), version 3 |
| 0b00001111 | CM_DATA_14_5 | data, 14.5 kbit/s radio interface rate |
| 0b00000011 | CM_DATA_12_0 | data, 12.0 kbit/s radio interface rate |
| 0b00001011 | CM_DATA_6_0 | data, 6.0 kbit/s radio interface rate |
| 0b00010011 | CM_DATA_3_6 | data, 3.6 kbit/s radio interface rate |
| DEF |  | reserved mode |

History:

07-May-97	VK	Initial

03-Jan-00	VK	Add some new 'Values', CM_SPEECH is depreciated -> use CM_SPEECH_VER1 instead

03-May-00	LG	added chan_mode2 till chan_mode8

## Channel Mode 2

Description:

The Channel Mode 2 information element gives information of the mode of coding/decoding and transcoding.
Reference : [GSM_04.08] section 10.5.2.7

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Channel Mode 2 | chan_mode_2 | 8 |

Values:

| short name | value | c-macro | comment |
| --- | --- | --- | --- |
| mode | 0 | CM2_SIG_ONLY | signaling only |
|  | 5 | CM2_SPEECH_HALF | speech half rate |
|  | 0b00000101 | CM2_SPEECH_VER1 | speech (full rate or half rate), version 1 |
|  | 0b00100101 | CM2_SPEECH_VER2 | speech (full rate or half rate), version 2 |
|  | 0b01000101 | CM2_SPEECH_VER3 | speech (full rate or half rate), version 3 |
|  | 0b00001111 | CM2_DATA_6_0 | data, 6.0 kbit/s radio interface rate |
|  | 0b00010111 | CM2_DATA_3_6 | data, 3.6 kbit/s radio interface rate |
|  | DEF |  | reserved mode |

History:

07-May-97	VK	Initial

03-Jan-00	VK	Add some new 'Values', CM2_SPEECH_HALF is depreciated -> use CM2_SPEECH_HALF_VER1 instead

## Channel Request Description Priority

Description:

Reference : [GSM_04.08] section 10.5.2.8a (Channel Request Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Channel Request Description Priority | crd_prio | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | CRD_PRIO_1 | priority level 1 (highest priority) |
| 1 | CRD_PRIO_2 | priority level 2 |
| 2 | CRD_PRIO_3 | priority level 3 |
| 3 | CRD_PRIO_4 | priority level 4 (lowest priority) |

History:

20-Apr-00	LG	Initial

## Channel Set Bitmap

Description:

If bit m of Channel Set Bitmap field n is set to 1 then timeslot m-1 is included in channel set n.
Reference : [GSM_04.08] section 10.5.2.21b (Multislot Allocation)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Channel Set Bitmap | chan_set | 8 |

History:

03-May-00	LG	Initial

## Cipher Response Info

Description:

Reference : [GSM_04.08] section 10.5.2.10 (Cipher Response)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Cipher Response Info | cr | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | INC_IMEISV_NO | IMEISV shall not be included |
| 1 | INC_IMEISV_YES | IMEISV shall be included |

History:

07-May-97	VK	Initial

## Classmark 3

Description:

Reference : [GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Classmark 3 | class_3 | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | MC2_CLASS_3_NO | No additional MS capability information available |
| 1 | MC2_CLASS_3_YES | MS capabilities are described in the Classmark 3 IE |

History:

07-May-97	VK	Initial

## CMSP Supported

Description:

The parameter defines the support of CM service Prompt (network initiated MO CM connection request) by the mobile equipment.
Reference : [GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| CMSP Supported | cmsp_supp | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | CMSP_SUPP_NO | CMSP not supported |
| 1 | CMSP_SUPP_YES | CMSP supported |

History:

19-Apr-00	LG	Initial

## Codec Hysteresis

Description:

The parameter is coded as the binary representation of the hysteresis value associated to the codec  threshold, as defined in GSM 05.09.
Reference : [GSM_04.08] section 10.5.2.21aa (Multirate Configuration)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Codec Hysteresis | codec_hyst | 4 |

History:

25-Apr-00	LG	Initial

## Codec Threshold

Description:

The parameter is coded as the binary representation of value N. N corresponds to the threshold of C/I in dB, as defined in GSM 05.09.
Reference : [GSM_04.08] section 10.5.2.21aa (Multirate Configuration)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Codec Threshold | codec_thr | 6 |

History:

25-Apr-00	LG	Initial

## Coding Scheme

Description:

The parameter defines channel coding scheme to be used on the downlink.
Reference : [GSM_04.08] section 10.5.2.4a (MAC Mode and Channel Coding Requested), section 10.5.2.16 (IA Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Coding Scheme | cod_scheme | 2 |
| Channel Coding Command | ccm | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | COD_S_1 |  |
| 1 | COD_S_2 |  |
| 2 | COD_S_3 |  |
| 3 | COD_S_4 |  |

History:

20-Apr-00	LG	Initial

## Common Control Channel Configuration

Description:

Reference : [GSM_04.08] section 10.5.2.11 (Control Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Common Control Channel Configuration | ccch_conf | 3 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | CCD_CCCH_1_NOT_COMB | 1 basic channel used for CCCH, not combined with SDCCHs |
| 1 | CCD_CCCH_1_COMB | 1 basic channel used for CCCH, combined with SDCCHs |
| 2 | CCD_CCCH_2_NOT_COMB | 2 basic channel used for CCCH, not combined with SDCCHs |
| 4 | CCD_CCCH_3_NOT_COMB | 3 basic channel used for CCCH, not combined with SDCCHs |
| 6 | CCD_CCCH_4_NOT_COMB | 4 basic channel used for CCCH, not combined with SDCCHs |
| DEF |  | reserved |

History:

07-May-97	VK	Initial

## CONTROL_ACK_TYPE

Description:

This field is the binary representation of the default format of the PACKET CONTROL ACKNOWLEDGMENT message.

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24 (GPRS Cell Options)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| CONTROL_ACK_TYPE | ctrl_ack_type | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 |  | default format is four access bursts |
| 1 |  | default format is RLC/MAC control block. |

History:

31-May-99	SL	Initial

## Count Of Codec Properties

Description:

This field is used to implement a variable array of Codec Properties. The spares in Multi Rate Configuration information element depend on the count.

Reference	: [GSM_04.08] section 10.5.2.21aa (Multi Rate Configuration)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Count Of Codec Properties | cnt_cod_prop | 2 |

History:

08-May-00	LG	Initial

## Dedicated Mode or TBF

Description:

Reference	: [GSM_04.08] section 10.5.2.25b (Dedicated Mode Or TBF)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Dedicated Mode or TBF | d_t | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | D_T_DED | the message assigns a resource in dedicated mode |
| 1 | D_T_TBF | the message assigns a TBF |

History:

26-Apr-00	LG	Initial

## Discontinuous Transmission (BCCH)

Description:

Reference : [GSM_04.08] section 10.5.2.3 (Cell Options (BCCH))

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Discontinuous Transmission (BCCH) | dtx_b | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | B_DTX_MAY_USE | The MSs may use uplink DTX |
| 1 | B_DTX_SHALL_USE | The MSs shall use uplink DTX |
| 2 | B_DTX_DONT_USE | The MS shall not use uplink DTX |
| DEF |  | invalid DTX indicator |

History:

07-May-97	VK	Initial

## Discontinuous Transmission (SACCH) (part I)

Description:

Reference : [GSM_04.08] section 10.5.2.3a (Cell Options (SACCH))

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Discontinuous Transmission (SACCH) MSB | dtx2_s | 1 |

History:

07-May-97	VK	Initial

## Discontinuous Transmission (SACCH) (part II)

Description:

Note : the DTX indicator sent on the SACCH is split into two parts. The values defined here are applicable for the union of part I and II of the DTX indicator.

Reference : [GSM_04.08] section 10.5.2.3a (Cell Options (SACCH))

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Discontinuous Transmission (SACCH) MSB | dtx_s | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | S_DTX_F_MAYUSE_H_DONTUSE | May use DTX on TCH-F. Don’t use DTX on TCH-H. |
| 1 | S_DTX_F_USE_H_DONTUSE | Do use DTX on TCH-F. Don’t use DTX on TCH-H. |
| 2 | S_DTX_F_DONTUSE_H_DONTUSE | Don’t use DTX on TCH-F. Don’t use DTX on TCH-H. |
| 3 | S_DTX_F_USE_H_MAYUSE | Do use DTX on TCH-F. May use DTX on TCH-H. |
| 4 | S_DTX_F_MAYUSE_H_MAYUSE | May use DTX on TCH-F. May use DTX on TCH-H. |
| 5 | S_DTX_F_USE_H_USE | Do use DTX on TCH-F. Do use DTX on TCH-H. |
| 6 | S_DTX_F_DONTUSE_H_USE | Don’t use DTX on TCH-F. Do use DTX on TCH-H. |
| 7 | S_DTX_F_MAYUSE_H_USE | May use DTX on TCH-F. Do use DTX on TCH-H. |

History:

07-May-97	VK	Initial

## Discontinuous Transmission used

Description:

Reference : [GSM_04.08] section 10.5.2.20 (Measurement Results)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| DTX USED | dtx_used | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | DTX_USED_NO | DTX was not used |
| 1 | DTX_USED_YES | DTX was used |

History:

07-May-97	VK	Initial

## Downlink Assignment

Description:

Reference	: [GSM_04.08] section 10.5.2.25b (Dedicated Mode Or TBF)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Downlink Assignment | dl | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | DL_0 | no meaning |
| 1 | DL_1 | the message assigns a resource to the MS identified by the IA rest octets |

History:

26-Apr-00	LG	Initial

## Downlink Assignment Bitmap

Description:

Reference	: [GSM_04.08] section 10.5.2.21b (Multislot Configuration)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Downlink Assignment Bitmap | dab | 7 |
| Uplink Assignment Bitmap | uab | 7 |

History:

03-May-00	LG	Initial

## DRX_TIMER_MAX

Description:

This field is the binary representation of the parameter DRX_TIMER_MAX. Range: 0 to 7. The parameter value is given as two taken to the power of the binary value minus one (2 (bv – 1) in units of 1 second. The binary value zero indicates that non-DRX mode is not supported in the cell. (E.g. the parameter takes the values: ‘not supported’, 1 s, 2 s, 4 s, .. 64 s.)

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24 (GPRS Cell Options)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| DRX_TIMER_MAX | drx_t_max | 3 |

History:

31-May-99	SL	Initial

## Early Classmark Sending

Description:

Reference :	[GSM_04.08] section 10.5.1.5 (Mobile Station Classmark 1)
		[GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Early Classmark Sending | es_ind | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | ES_IMPL_NO | 'Early Classmark Sending' option is not implemented |
| 1 | ES_IMPL_YES | 'Early Classmark Sending' option is implemented |

History:

07-May-97	VK	Initial

## Extended Dynamic Allocation

Description:

This bits indicates the medium access mode to be used during TBF.
Reference :	[GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Extended Dynamic Allocation | ext_dyn_all | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | EXT_DYN_NO | Dynamic Allocation |
| 1 | EXT_DYN_YES | Extended Dynamic Allocation |

History:

26-Apr-00	LG	Initial

## Extended Measurement Capability

Description:

This bits indicates whether the mobile station supports extended measurements or not.
Reference :	[GSM_04.08] section 10.5.1.7 (Mobile Station Classmark 3)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Extended Measurement Capability | ext_meas_cap | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | EXT_MEAS_NO | extended measurements not supported |
| 1 | EXT_MEAS_YES | extended measurements supported |

History:

20-Apr-00	LG	Initial

## Extended Measurement Frequency List

Description:

The purpose of the Extended Measurement Frequency List information element is to provide the absolute radio frequency channel numbers of carriers to measure signal strength to.
Reference : [GSM_04.08] section 10.5.2.46

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Extended Measurement Frequency List | ext_meas_freq | 128 |

History:

28-Apr-00	LG	Initial

## Extension Length

Description:

The length of the extension.

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24 (GPRS Cell Options)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Extension Length | ext_len | 6 |

History:

31-May-99	SL	Initial

## Extension Spare Bits

Description:

That field will be defined during future extension.

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24 (GPRS Cell Options)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Extension Spare Bits | sp_ext | 64 |

History:

31-May-99	SL	Initial

## Final Allocation

Description:

This bits indicates whether the allocation is the last allocation of the TBF or not.
Reference :	[GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Final Allocation | final | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | FINAL_NO | allocation is not the last Allocation of TBF |
| 1 | FINAL_YES | allocation is the last Allocation of TBF |

History:

26-Apr-00	LG	Initial

## Flag

Description:

This bits indicates whether an information is present or not.
Reference :	[GSM_04.08]

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Flag | flag | 1 |
| Flag1 | flag1 | 1 |
| Flag2 | flag2 | 1 |
| Allocation Flag | allo_flag | 1 |
| Flag Parameters Present | flag_pp | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | PRES_NO | information is not present |
| 1 | PRES_YES | information is present |

History:

26-Apr-00	LG	Initial

## Frequency Capability

Description:

Reference :	[GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Frequency capability | freq_cap | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | FREQ_CAP_NO | The mobile station does not support the extension band G1. |
| 1 | FREQ_CAP_YES | The mobile station does support the extension band G1. |

History:

07-May-97	VK	Initial

## Frequency List Contents

Description:

Content of a frequency list
Reference : [GSM_04.08] section 10.5.2.13

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Frequency List Contents | flist | 8 |

History:

08-Feb-99	LE	Initial

## Frequency Range Lower

Description:

Reference	: [GSM_04.08] section 10.5.2.1a (BA Range)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Frequency Range Lower | freq_lower | 10 |

History:

07-May-97	VK	Initial

## Frequency Range Higher

Description:

Reference	: [GSM_04.08] section 10.5.2.1a (BA Range)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Frequency Range Higher | freq_higher | 10 |

History:

07-May-97	VK	Initial

## Frequency Short List

Description:

The purpose of the Frequency Short List information element is to provide the list of the absolute radio frequency channel numbers used in a frequency hopping sequence, in a small fixed length information element to obtain when possible the HANDOVER COMMAND message in a single block. The Frequency Short List information element is a type 3 information element of 10 octet length. This element is encoded exactly as the Frequency List information element, except that it has a fixed length instead of a variable length and does not contain a length indicator and that it shall not be encoded in bitmap 0 format.
Reference : [GSM_04.08] section 10.5.2.14

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Frequency Short List | freq_short_list_before | 72 |
| Frequency Short List | freq_short_list_after | 72 |

History:

07-May-97	VK	Initial

## Frequency Short List 2

Description:

The purpose of the Frequency Short List information element is to provide the list of the absolute radio frequency channel numbers used in a frequency hopping sequence, in a small fixed length information element to obtain when possible the NOTIFICATION FACCH message in a single block. The Frequency Short List 2 information element is a type 3 information element of 8 octet length. This element is encoded exactly as the Frequency List information element, except that it has a fixed length instead of a variable length and does not contain a length indicator and that it shall not be encoded in bitmap 0 format.
Reference : [GSM_04.08] section 10.5.2.14a

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Frequency Short List 2 | freq_short_list2 | 56 |

History:

26-Apr-00	LG	Initial

## Gamma

Description:

The Gamma field is used for power output control in units of 2 dB.
Reference : [GSM_04.08] section 10.5.2.16 (IA Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Gamma | gamma | 5 |

History:

25-Apr-00	LG	Initial

## Group Call Reference

Description:

Reference : [GSM_04.08] section 10.5.1.9 (Descriptive Group or Broadcast Call Reference)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Group Call Reference | gcr | 27 |

History:

25-Apr-00	LG	Initial

## Group Cipher Key Number

Description:

The purpose of the Group Cipher Key Number information element is to provide information on the group cipher key to be used for ciphering and deciphering by the mobile station.
Reference : [GSM_04.08] section 10.5.1.10

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Group Cipher Key Number | group_ckn | 4 |

History:

20-Apr-00	LG	Initial

## Half Duplex Mode

Description:

This information field indicates, for multislot classes 19 to 29, whether the mobile station shall operate in half duplex mode.
Reference : [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Half Duplex Mode | hdm | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | HDM_NO | The mobile station shall not operate in half duplex mode. |
| 1 | HDM_YES | The mobile station shall operate in half duplex mode. |

History:

27-Apr-00	LG	Initial

## Handover Reference

Description:

The purpose of the Handover Reference information element is to provide a handover reference value used for access identification.
Reference : [GSM_04.08] section 10.5.2.15

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Handover Reference | handov_ref | 8 |

History:

07-May-97	VK	Initial

## Hopping

Description:

Reference	: [GSM_04.08] section 10.5.2.5 (Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Hopping | hop | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | HOP_NO | frequency hopping is off |
| 1 | HOP_YES | frequency hopping is on |

History:

07-May-97	VK	Initial

07-May-97	VK	Swap HOP_NO and HOP_YES

## Hopping Sequence Number

Description:

Reference	: [GSM_04.08] section 10.5.2.5 (Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Hopping Sequence Number | hsn | 6 |

History:

07-May-97	VK	Initial

## Identity digit

Description:

Reference	: [GSM_04.08] section 10.5.1.4 (Mobile Identity)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Identity digit | ident_dig | 4 |

History:

07-May-97	VK	Initial

## If And Where

Description:

The parameter is used to locate the blocks where SYSTEM INFORMATION TYPE 9 messages are sent
Reference : [GSM_04.08] section 10.5.2.34

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| If And Where | if_and_where | 3 |

History:

05-Apr-00	LG	Initial

## Inband Notifications

Description:

The parameter is used to indicate possible channels for notifications of VBS/VGCS calls.
Reference : [GSM_04.08] section 10.5.2.35a (SI 6 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Inband Notification | inband_not | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | IN_BAND_NOT_NCH | notifications are sent on NCH only |
| 1 | IN_BAND_NOT_FACCH | notifications are sent on FACCH too |

History:

13-Apr-00	LG	Initial

## Inband Pagings

Description:

The parameter is used to indicate possible channels for paging of point-to-point calls.
Reference : [GSM_04.08] section 10.5.2.35a (SI 6 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Inband pagings | inband_pag | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | IN_BAND_PAG_PCH | pagings are sent on PCH only |
| 1 | IN_BAND_PAG_FACCH | pagings are sent on FACCH too |

History:

13-Apr-00	LG	Initial

## Increment Skip of ARFCNn

Description:

The increment skip ARFCN n is coded as the binary representation of the increment of the preceding absolute RF channel number appearing in the sequence of channels used in the frequency hopping: n = 1,...,16.
Range: 0 to 15 
The value 0 indicates that the increment value is 15 but the concerned channel is not used and the next field, i.e. Increment skip ARFCN n+1 (if present) must be added to the increment to determine the next absolute RF channel number in the sequence of channels used in the frequency hopping.

Reference	: [GSM_04.08] section 10.5.2.12 (Frequency Channel Sequence)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| inc skip of ARFCNn | inc_skip | 4 |

History:

07-May-97	VK	Initial

## Indirect

Description:

Reference : [GSM_04.08] section 10.5.2.25a (Packet Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Indirect | indir | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | INDIR_NO | non hopping configuration |
| 1 | INDIR_YES | indirect encoding of hopping RF channel configuration |

History:

26-Apr-00	LG	Initial

## Info Type 4

Description:

All values indicate unknown, unnecessary information and are reserved for future use
Reference : [GSM_04.08] section 10.5.2.37a (SI 9 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Info Type 4 | it4 | 4 |
| Info Type 6 | it6 | 6 |

History:

27-Apr-00	LG	Initial

## Info Type 5

Description:

Reference : [GSM_04.08] section 10.5.2.37a (SI 9 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Info Type 5 | it5 | 5 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | IT5_1 | System Information Type 1 |
| 1 | IT5_2 | System Information Type 2 |
| 2 | IT5_2BIS | System Information Type 2bis |
| 3 | IT5_2TER | System Information Type 2ter |
| 4 | IT5_3 | System Information Type 3 |
| 5 | IT5_4 | System Information Type 4 |
| 6 | IT5_7 | System Information Type 7 |
| 7 | IT5_8 | System Information Type 8 |
| 8 | IT5_9 | System Information Type 9 |
| 9 | IT5_13 | System Information Type 13 |
| 10 | IT5_14 | System Information Type 14 |

History:

27-Apr-00	LG	Initial

## Initial Codec Mode Indicator

Description:

Reference : [GSM_04.08] section 10.5.2.21aa (Multirate Configuration)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Initial Codec Mode Indicator | icmi | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | ICMI_IMPL | initial codec mode defined by implicit rule provided in GSM 05.09 |
| 1 | ICMI_START_MODE | initial codec mode defined by start mode field |

History:

25-Apr-00	LG	Initial

## Key Sequence

Description:

Reference	: [GSM_04.08] section 10.5.1.2 (Ciphering Key Sequence Number)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| key sequence | key_seq | 3 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0-6 |  |  |
| 7 |  | No key is available (MS to network); Reserved (network to MS) |

History:

07-May-97	VK	Initial

## Length Of Frequency Parameters

Description:

Reference	: [GSM_04.08] section 10.5.2.16 (IA Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Length Of Frequency Parameters | fp_len | 6 |

History:

03-May-00	LG	Initial

## LLC Frame Type

Description:

Reference	: [GSM_04.08] section 10.5.2.8a (Channel Request Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| LLC Frame Type | llc_fr_type | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | LLC_IS_ACK | LLC frame is SACK or NACK |
| 1 | LLC_NOT_ACK | LLC frame is not SACK or NACK |

History:

20-Apr-00	LG	Initial

## Location Area Code

Description:

Reference	: [GSM_04.08] section 10.5.1.3 (Location Area Identification)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Location Area Code | lac | 16 |

History:

07-May-97	VK	Initial

## Lowest ARFCN

Description:

The lowest ARFCN field is coded as the binary representation of the lowest absolute RF channel number appearing in the sequence of channels used in the frequency hopping.
Reference	: [GSM_04.08] section 10.5.1.12 (Frequency Channel Sequence)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Lowest ARFCN | low_arfcn | 7 |

Values:

| value | comment |
| --- | --- |
| 1-124 |  |
| DEF | reserved |

History:

07-May-97	VK	Initial

## LSA ID

Description:

The parameter is used to identify the LSA.
Reference : [GSM_04.08] section 10.5.2.35 (SI4 rest octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| LSA ID | lsa_id | 24 |
| Additional LSA IDs | lsa_id_add | 24 |

History:

05-Apr-00	LG	Initial

## LSA Offset

Description:

The parameter defines an offset for LSA reselection between cells with same LSA priorities.
Reference : [GSM_04.08] section 10.5.2.35 (SI4 rest octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| LSA Offset | lsa_offs | 3 |

History:

05-Apr-00	LG	Initial

## MAC Mode

Description:

The parameter defines the medium access method to be used during the TBF.
Reference : [GSM_04.08] section 10.5.2.4a ()MAC Mode and Channel Coding Requested), [GSM_04.60] section 11.2.7 (Packet Downlink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| MAC Mode | mac_mode | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | MAC_DYN_ALLOC | dynamic allocation |
| 1 | MAC_EXT_DYN_ALLOC | extended dynamic allocation |
| 2 | MAC_FIX_ALLOC_NOT_HALF | fixed allocation, not half duplex mode |
| 3 | MAC_FIX_ALLOC | fixed allocation, half duplex mode |

History:

20-Apr-00	LG	Initial

## MA Number Indication

Description:

Reference : [GSM_04.08] section 10.5.2.25a (Packet Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| MA Number Indication | ma_num | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | MA_NUM_14 | MA_NUMBER = 14 |
| 1 | MA_NUM_15 | MA_NUMBER = 15 |

History:

26-Apr-00	LG	Initial

## Maximum Number of Retransmissions

Description:

Reference	: [GSM_04.08] section 10.5.1.29 (RACH Control Parameters)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Maximum Number of Retransmissions | max_retrans | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | MAX_RETRANS_1 | Maximum 1 retransmission |
| 1 | MAX_RETRANS_2 | Maximum 2 retransmissions |
| 2 | MAX_RETRANS_4 | Maximum 4 retransmissions |
| 3 | MAX_RETRANS_7 | Maximum 7 retransmissions |

History:

07-May-97	VK	Initial

## Measurement Bitmap

Description:

The Measurement Bitmap field indicates the timeslots for use during a measurement periods. A bit set in the field means that the MS shall make measurements during the timeslot, a bit reset means that the MS shall receive downlink data during the timeslot.  
Reference	: [GSM_04.08] section 10.5.2.25d (RR Packet Downlink Assignment), [GSM_04.60] section 11.2.7

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Measurement Bitmap | meas_bmp | 8 |

History:

27-Apr-00	LG	Initial

## Measurement Interval

Description:

The Measurement Interval field indicates the number of block periods from the start of one assigned measurement period to the beginning of the next measurement period.   
Reference	: [GSM_04.08] section 10.5.2.25d (RR Packet Downlink Assignment), [GSM_04.60] section 11.2.7

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Measurement Interval | meas_int | 5 |

History:

27-Apr-00	LG	Initial

## Measurement valid

Description:

Reference	: [GSM_04.08] section 10.5.2.20 (Measurement Results)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Measurement valid | meas_valid | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | MEAS_VALID_YES | The measurement results are valid |
| 1 | MEAS_VALID_NO | the measurement results are not valid |

History:

07-May-97	VK	Initial

## Message Type

Description:

Reference	: [GSM_04.08] section 10.4

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Message Type | msg_type | 8 |

History:

07-May-97	VK	Initial

## Mobile Allocation Contents

Description:

Reference	: [GSM_04.08] section 10.5.2.21 (Mobile Allocation)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Mobile Allocation Contents | mac | 8 |

History:

07-May-97	VK	Initial

## Mobile Allocation Index Offset

Description:

Reference	: [GSM_04.08] section 10.5.2.5 (Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Mobile Allocation Index Offset | maio | 6 |

History:

07-May-97	VK	Initial

## Mobile Allocation Length

Description:

Reference : [GSM_04.08] section 9.1.21a (Notification/FACCH), 10.2.22c (NT/N Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Mobile Allocation Length | ma_len | 8 |

History:

08-May-00	LG	Initial

## Mobile Country Code

Description:

Reference	: [GSM_04.08] section 10.5.1.3 (Location Area Identification)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Mobile Country Code | mcc | 4 |

History:

07-May-97	VK	Initial

## Mobile Network Code

Description:

Reference	: [GSM_04.08] section 10.5.1.3 (Location Area Identification)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Mobile Network Code | mnc | 4 |

History:

07-May-97	VK	Initial

## Modulus

Description:

The Modulus field encodes the position modulus. 
Let M be the encoded value.

If  M=0, the position modulus is 0,

if  M>0, the  position modulus is 2M+1. 
Reference	: [GSM_04.08] section 10.5.2.37a (SI 9 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Modulus | mod | 4 |

History:

27-Apr-00	LG	Initial

## MS TXPWR MAX CCH

Description:

The MS-TXPWR-MAX-CCH field is coded as the binary representation of the "power control level" in TS GSM 05.05 corresponding to the maximum TX power level an MS may use when accessing on a Control Channel CCH. This value shall be used by the Mobile Station according to TS GSM 05.08.
Reference	: [GSM_04.08] section 10.5.2.4 (Cell Selection Parameters)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| MS-TXPWR-MAX-CCH | ms_txpwr_max_cch | 5 |

History:

07-May-97	VK	Initial

## MT/MO Origination type

Description:

Reference	: [GSM_04.08] section 10.5.2.8a (Channel Request Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| MT/MO Origination type | or_ty | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | OR_TY_MT | Mobile terminated |
| 1 | OR_TY_MO | Mobile originated |

History:

20-Apr-00	LG	Initial

/*

## Multiband Reporting

Description:

This element is part of the Structured Element 'Add Multiband', which is not used in this implementation.

Reference	: [GSM_04.08] section 10.5.2.44 (Additional Multiband Information)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Multiband Reporting | mr | 2 |

History:

07-May-97	VK	Initial

*/

## Multi Rate Speech Version

Description:

Reference : [GSM_04.08] section 10.5.2.21aa (Multirate Configuration)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Multi Rate Speech Version | mr_vers | 3 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 1 | MR_VERS_1 | Adaptive Multirate speech version 1 |
| DEF |  | reserved |

History:

25-Apr-00	LG	Initial

## Multi Slot Class

Description:

The multi slot class field contains the multislot class like described in GSM 05.02 Annex B if the Mobile station supports the use of multiple timeslots.

Reference : [GSM_04.08] section 10.5.1.7 (Mobile Station Classmark 3)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Multi Slot Class | m_s_class | 5 |

History:

20-Apr-00	LG	Initial

## N_AVG_I Value

Description:

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets).

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| N_AVG_I Value | n_avg_i | 4 |

History:

11-Jun-99	CAN	Initial

## NC Mode

Description:

Reference : [GSM_04.08] section 10.5.2.21c (NC Mode)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| NC Mode | ncm | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | NC_0 |  |
| 1 | NC_1 |  |
| 2 | NC_2 |  |
| 3 | NC_TARGET | information on NC mode to be gathered from the target cell |

History:

25-Apr-00	LG	Initial

## NCC Permitted

Description:

The purpose of the NCC Permitted information element is to provide a definition of the allowed NCCs on the BCCH carriers to be reported in the MEASUREMENT REPORT message by the mobile stations in the cell.
Reference : [GSM_04.08] section 10.5.2.27

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| NCC Permitted | ncc_permit | 8 |

History:

07-May-97	VK	Initial

21-Jul-97	LE	bit len instead of len

## NCH Position

Description:

The values of the NCH position field indicates the block number of the CCCH block which is used for the first NCH block and the number of blocks used for the NCH.

Reference : [GSM_04.08] section 10.5.2.32 (S1 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| NCH Position | nch_position | 5 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | NCH_1BLK_NR0 | 1 block / first is block 0 |
| 1 | NCH_1BLK_NR1 | 1 block / first is block 1 |
| 2 | NCH_1BLK_NR2 | 1 block / first is block 2 |
| 3 | NCH_1BLK_NR3 | 1 block / first is block 3 |
| 4 | NCH_1BLK_NR4 | 1 block / first is block 4 |
| 5 | NCH_1BLK_NR5 | 1 block / first is block 5 |
| 6 | NCH_1BLK_NR6 | 1 block / first is block 6 |
| 7 | NCH_2BLK_NR0 | 2 blocks / first is block 0 |
| 8 | NCH_2BLK_NR1 | 2 blocks / first is block 1 |
| 9 | NCH_2BLK_NR2 | 2 blocks / first is block 2 |
| 10 | NCH_2BLK_NR3 | 2 blocks / first is block 3 |
| 11 | NCH_2BLK_NR4 | 2 blocks / first is block 4 |
| 12 | NCH_2BLK_NR5 | 2 blocks / first is block 5 |
| 13 | NCH_3BLK_NR0 | 3 blocks / first is block 0 |
| 14 | NCH_3BLK_NR1 | 3 blocks / first is block 1 |
| 15 | NCH_3BLK_NR2 | 3 blocks / first is block 2 |
| 16 | NCH_3BLK_NR3 | 3 blocks / first is block 3 |
| 17 | NCH_3BLK_NR4 | 3 blocks / first is block 4 |
| 18 | NCH_4BLK_NR0 | 4 blocks / first is block 0 |
| 19 | NCH_4BLK_NR1 | 4 blocks / first is block 1 |
| 20 | NCH_4BLK_NR2 | 4 blocks / first is block 2 |
| 21 | NCH_4BLK_NR3 | 4 blocks / first is block 3 |
| 22 | NCH_5BLK_NR0 | 5 blocks / first is block 0 |
| 23 | NCH_5BLK_NR1 | 5 blocks / first is block 1 |
| 24 | NCH_5BLK_NR2 | 5 blocks / first is block 2 |
| 25 | NCH_6BLK_NR0 | 6 blocks / first is block 0 |
| 26 | NCH_6BLK_NR1 | 6 blocks / first is block 1 |
| 27 | NCH_7BLK_NR0 | 7 blocks / first is block 0 |
| DEF |  | reserved |

History:

13-Apr-00	LG	Initial

## Neighbour Cells Description

Description:

The purpose of the Neighbour Cells Description information element is to provide the absolute radio frequency channel numbers of the BCCH carriers to be monitored by the mobile stations in the cell.
Reference : [GSM_04.08] section 10.5.2.22

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Neighbour Cells Description | neigh_cell_desc | 128 |

History:

07-May-97	VK	Initial

21-Jul-97	LE	bit len instead of len

## NECI

Description:

Half rate support.
Reference	: [GSM_04.08] section 10.5.2.4 (Cell Selection Parameters)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Half rate support | neci | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | NECI_NO | New establishment causes are not supported |
| 1 | NECI_YES | New establishment causes are supported |

History:

07-May-97	VK	Initial

## Network Control Order

Description:

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Network Control Order | nco | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | NCO_NO_REP | MS controlled cell reselection, no measurement reporting |
| 1 | NCO_MS_RESEL | MS controlled cell reselection, MA sends measurement reports |
| 2 | NCO_NET_RESEL | Network controlled cell reselection, MA sends measurement reports |
| DEF |  | reserved |

History:

27-Apr-00	LG	Initial

## Network Mode of Operation

Description:

This field is the binary representation of the Network Mode of Operation, see GSM 03.60.

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24 (GPRS Cell Options)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| NMO | nmo | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0b00 |  | Network Mode 1 |
| 0b01 |  | Network Mode 2 |
| 0b10 |  | Network Mode 3 |
| 0b11 |  | reserved |

History:

31-May-99	SL	Initial

## Notification List Number

Description:

The notification list number is a counter modulo 4 for a notification of a new VGCS or VBS call.
Reference	: [GSM_04.08] section 3.3.3.3 (Notification Procedure)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Notification List Number PCH | nln_pch | 2 |
| Notification List Number NCH | nln_nch | 2 |
| Notification List Number SACCH | nln_sacch | 2 |

History:

13-Apr-00	LG	Initial

## Notification List Number Status

Description:

A change of the notification list number status indicates a change of information on the NCH which is not related to a new  VGCS or VBS call. 
Reference	: [GSM_04.08] section 3.3.3.3 (Notification Procedure)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Notification List Number Status | nln_status | 1 |

History:

13-Apr-00	LG	Initial

## Normal cell indication

Description:

Reference	: [GSM_04.08] section 10.5.2.39 (Synchronisation Indication)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Normal cell indication | nci | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | NCI_IGNORE | Out of range timing advance is ignored |
| 1 | NCI_TRIGGER | Out of range timing advance shall trigger a handover failure |

History:

07-May-97	VK	Initial

## Number of Ranges

Description:

The number of Ranges parameter indicates in binary the number of ranges to be transmitted in the BA Range IE. It shall have a minimum value of 1.
Reference	: [GSM_04.08] section 10.5.2.1a (BA Range)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Number of Ranges | num_range | 8 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 1-N_MAX_RANGE |  | number of frequency ranges |
| DEF |  | reserved |

History:

07-May-97	VK	Initial

## Number of Cells

Description:

Reference	: [GSM_04.08] section 10.5.2.20 (Measurement Results)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Number of Cells | num_ncell | 3 |

History:

07-May-97	VK	Initial

## Odd/ Even indication

Description:

Reference	: [GSM_04.08] section 10.5.1.4 (Mobile Identity)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Odd/ Even indication | odd_even | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | EVEN | even |
| 1 | ODD | odd |

History:

07-May-97	VK	Initial

## P0

Description:

The P0 field is used to code an optional downlink power control parameter. Its meaning is specific to downlink power control modes A and B. The fields range is 0 to 15, coding 0 dB to 30 dB in steps of 2 dB.

Reference: [GSM_04.08] section 10.5.2.16 (IA Rest Octets), [GSM_04.60] section 11.2.29 (Packet Uplink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| P0 | p0 | 4 |

History:

03-May-00	LG	Initial

## Packet Channel Type

Description:

Reference	: [GSM_04.08] section 10.5.2.25a (Packet Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Packet Channel type | pck_chan_type | 5 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 1 | PCK_CH_1 | for backward compatibility |
| DEF |  | reserved |

History:

26-Apr-00	LG	Initial

## Paging Channel Restructuring

Description:

Reference	: [GSM_04.08] section 10.5.2.35a (SI 6 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Paging Channel Restructuring | pch_restruct | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | PCH_RESTRUCT_NO | paging channel is restructured |
| 1 | PCH_RESTRUCT_YES | paging channel is not restructured |

History:

13-Apr-00	LG	Initial

## Page Mode

Description:

Reference	: [GSM_04.08] section 10.5.2.26 (Page Mode)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Page Mode | pm | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | PAGING_NORM | Normal paging. |
| 1 | PAGING_EXT | Extended paging. |
| 2 | PAGING_REORG | Paging reorganization. |
| 3 | PAGING_SAME | Same as before. |

History:

07-May-97	VK	Initial

## PAN_DEC

Description:

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24 (GPRS Cell Options)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| PAN_DEC | dec | 3 |

History:

31-May-99	SL	Initial

## PAN_INC

Description:

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24 (GPRS Cell Options)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| PAN_INC | inc | 3 |

History:

31-May-99	SL	Initial

## PAN_MAX

Description:

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24 (GPRS Cell Options)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| PAN_MAX | pmax | 3 |

History:

31-May-99	SL	Initial

## Pb

Description:

The Pb field is used to code an power reduce value used by the BTS on PBCCH blocks, relative to the output power used of the BCCH. The fields range is 0 to 15, coding 0 dB to -30 dB in steps of -2 dB.

Reference: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.9 (Global Power Control Parameters)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Pb | pb | 4 |

History:

03-May-00	LG	Initial

## PC_MEAS_CHAN

Description:

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets).

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| PC_MEAS_CHAN | pc_meas_chan | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | PC_MEAS_BCCH | BCCH shall be used for downlink measurements for power control |
| 1 | PC_MEAS_PDCH | PDCH shall be used for downlink measurements for power control |

History:

11-Jun-99	CAN	Initial

## Penalty Time

Description:

The parameter is used by the mobile station to apply an offset to the value of C2 during penalty time.
Reference : [GSM_04.08] section 10.5.2.34 (SI3 Rest Octets), section 10.5.2.35 (SI4 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Penalty Time | penalty_time | 5 |

History:

05-Apr-00	LG	Initial

## PLMN Colour Code

Description:

The NCC field is coded as the binary representation of the PLMN colour code (see TS. GSM 03.03)
Reference	: [GSM_04.08] section 10.5.2.2 (Cell Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| PLMN Colour Code | ncc | 3 |

History:

07-May-97	VK	Initial

## Polling

Description:

The parameter is in packet uplink assignment and packet downlink assignment of IA Rest Octets. It indicates if the mobile station is polled for a PACKET CONTROL ACKNOWLEDGEMENT.
Reference : [GSM_04.08] section 10.5.16. (IA Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Polling | poll | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | POLL_NO_ACTION | no action required from MS |
| 1 | POLL_SEND_ACK | MS shall send a PACKET CONTROL ACKNOWLEDGEMENT |

History:

25-Apr-00	LG	Initial

## Power Control

Description:

Reference	: [GSM_04.08] section 10.5.2.3 (Cell Options (BCCH))
		  [GSM_04.08] section 10.5.2.3a (Cell Options (SACCH))

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Power Control | pow_ctrl | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | POW_CTRL_NO | PWRC is not set |
| 1 | POW_CTRL_YES | PWRC is set |

History:

07-May-97	VK	Initial

## Power Level

Description:

Reference	: [GSM_04.08] section 10.5.2.28 (Power Command)
		: [GSM_04.08] section 10.5.2.28a (Power Command and Access Type)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Power Level | pow | 5 |

History:

07-May-97	VK	Initial

## Power Offset

Description:

The parameter is used by DCS 1800 Class 3 MSs to add a power offset to the value of MS_TXPWR_MAX_CCH.
Reference : [GSM_04.08] section 10.5.2.34 (SI 3 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Power Offset | pow_offs | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | POW_OFFS_0_DB | offset 0 dB |
| 1 | POW_OFFS_2_DB | offset 2 dB |
| 2 | POW_OFFS_4_DB | offset 4 dB |
| 3 | POW_OFFS_6_DB | offset 6 dB |

History:

05-Apr-00	LG	Initial

## PR Mode

Description:

- XXX

Reference : [GSM_04.08] section XXX (XXX)

Definition:

| long name | short name | Bit len |
| --- | --- | --- |
| PR Mode | pr_mode | 1 |

History:

19-Sep-00	MPA	Initial

## Priority Access Threshold

Description:

Reference : [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Priority Access Threshold | prio_acc_thr | 3 |

Values:

| value | c-macro | Comment |
| --- | --- | --- |
| 0 | PAT_NO | Packet access not allowed in the cell |
| 1 | PAT_SPARE1 | Spare, shall be interpreted as 0 |
| 2 | PAT_SPARE2 | Spare, shall be interpreted as 0 |
| 3 | PAT_ACC1 | packet access allowed for priority level 1 |
| 4 | PAT_ACC2 | packet access allowed for priority level 1 to 2 |
| 5 | PAT_ACC3 | packet access allowed for priority level 1 to 3 |
| 6 | PAT_ACC4 | packet access allowed for priority level 1 to 4 |
| 7 | PAT_SPARE6 | Spare, shall be interpreted as 6 |

History:

27-Apr-00	LG	Initial

## Priority Threshold

Description:

The parameter is used to determine whether the mobile shall apply prioritized cell re-selection.
Reference : [GSM_04.08] section 10.5.2.35 (SI4 Rest Octets), section 10.5.2.37e (SI16 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Priority Threshold | prio_thr | 3 |

History:

05-Apr-00	LG	Initial

## Pseudo Synchronisation Capability

Description:

Reference	: [GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| pseudo-synchronisation cap. | ps_cap | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | PS_CAP_NO | PS capability not present |
| 1 | PS_CAP_YES | PS capability present |

History:

07-May-97	VK	Initial

## PSI1 Repeat Period

Description:

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets).

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| PSI1 Repeat Period | psi1_period | 4 |

History:

11-Jun-99	CAN	Initial

## RA Color

Description:

The parameter is used by the mobile station to decide whether two cell belong to different routing areas.
Reference : [GSM_04.08] section 10.5.2.34 (SI3 Rest Octets), section 10.5.2.35 (SI4 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| RA color | ra_color | 3 |

History:

05-Apr-00	LG	Initial

## Radio Link Timeout

Description:

The precise meaning of RADIO-LINK-TIMEOUT parameter can be found in Technical

Specification GSM 05.08.
Reference	: [GSM_04.08] section 10.5.2.3 (Cell Options (BCCH))
		  [GSM_04.08] section 10.5.2.3a (Cell Options (SACCH))

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Radio Link Timeout | rlt | 4 |

History:

07-May-97	VK	Initial

## Random Access Information

Description:

Content of a previous channel request message.
Reference : [GSM_04.08] section 10.5.2.30 (Request Reference)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Random Access Information | ra | 8 |

History:

07-May-97	VK	Initial

## Relative Position

Description:

The length of this field depends on the value M of Modulus field. It contains a non-negative integer number < 2M+1, if M is greater than 0.

Reference	: [GSM_04.08] section 10.5.2.37a (SI 9 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Relative Position | rel_pos | 16 |

History:

03-May-00	LG	Initial

## Report Observed Time Difference

Description:

Reference	: [GSM_04.08] section 10.5.2.39 (Synchronisation Indication)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Report Observed Time Difference | rot | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | TIME_DIFF_NO | Mobile Time Difference IE shall not be included in the HANDOVER COMPLETE message |
| 1 | TIME_DIFF_YES | Mobile Time Difference IE shall be included in the HANDOVER COMPLETE message |

History:

07-May-97	VK	Initial

## Requested Bandwidth

Description:

The requested bandwidth is the binary encoding of the bit rate information expressed in 100 bit/s, starting from 0*100 bit/s until 65535 * 100 bit/s.
Reference	: [GSM_04.08] section 10.5.2.8a (Channel Request Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Requested Bandwidth | rbw | 16 |

History:

20-Apr-00	LG	Initial

## Resumption ACK

Description:

The ACK field is the binary acknowledge of a successful resumption of GPRS services.
Reference	: [GSM_04.08] section 10.5.2.14c (GPRS Resumption)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Resumption ACK | res_ack | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | RES_ACK_NO | resumption of GPRS services not successfully acknowledged |
| 1 | RES_ACK_YES | resumption of GPRS services successfully acknowledged |

History:

20-Apr-00	LG	Initial

## Revision Level

Description:

Reference	: [GSM_04.08] section 10.5.1.5 (Mobile Station Classmark 1)
		  [GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)
		  [GSM_04.08] section 10.5.1.7 (Mobile Station Classmark 3)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Revision Level | rev_lev | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | PHASE_1 | Reserved for phase 1 |
| 1 | PHASE_2 | Used by phase 2 MSs |
| DEF |  | reserved |

History:

07-May-97	VK	Initial

## RFL Number

Description:

The Reference Frequency List Number field contains a reference frequency. 
Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets) [GSM_04.60] section 12.10a (GPRS Mobile Allocation)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| RFL Number | rfln | 4 |

History:

28-Apr-00	LG	Initial

## RF power capability

Description:

Reference	: [GSM_04.08] section 10.5.1.5 (Mobile Station Classmark 1)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| RF power capability | rf_pow_cap | 3 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | RF_CLASS_1 | class 1 for GSM 900 and DCS 1800 |
| 1 | RF_CLASS_2 | class 2 for GSM 900 and DCS 1800 |
| 2 | RF_CLASS_3 | class 3 for GSM 900 and DCS 1800 |
| 3 | RF_CLASS_4 | class 4 for GSM 900, reserved for DCS 1800 |
| 4 | RF_CLASS_5 | class 5 for GSM 900, reserved for DCS 1800 |
| DEF |  | reserved |

History:

07-May-97	VK	Initial

## RLC Data Blocks Granted

Description:

This field indicates the number of RLC data blocks the mobile station shall transfer. The number of blocks to transmit is coded as the binary representation of the count minus 9. 
Reference	: [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| RLC Data Blocks Granted | rlc_blks | 8 |

History:

20-Apr-00	LG	Initial

## RLC Mode

Description:

Reference	: [GSM_04.08] section 10.5.2.8a (Channel Request Description), section 10.5.2.16 (IA rest octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| RLC Mode | rlc_mode | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | RLC_ACK_MODE | RLC acknowledged mode |
| 1 | RLC_UNACK_MODE | RLC unacknowledged mode |

History:

20-Apr-00	LG	Initial

## RLC Octet Count

Description:

This field indicates the number of octets of RLC data the mobile station wishes to transfer. 
Reference	: [GSM_04.08] section 10.5.2.8a (Channel Request Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| RLC Octet Count | rlc_c_oct | 16 |

History:

20-Apr-00	LG	Initial

## Routing Area Code

Description:

This field identifies the routing area. 
Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_03.03]

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Routing Area Code | rac | 8 |

History:

27-Apr-00	LG	Initial

## RR Cause

Description:

The purpose of the RR Cause information element is to provide the reason for release or the reason for completion of an assignment or handover.
Reference : [GSM_04.08] section 10.5.2.31

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| RR Cause | rr_cause | 8 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0b00000000 | RRC_NORMAL_EVENT | Normal event |
| 0b00000001 | RRC_UNSPECIFIED | Abnormal release, unspecified |
| 0b00000010 | RRC_CHAN_UNACCEPT | Abnormal release, channel unacceptable |
| 0b00000011 | RRC_TIMER_EXPIRED | Abnormal release, timer expired |
| 0b00000100 | RRC_NO_ACTIVITY | Abnormal release, no activity on the radio path |
| 0b00000101 | RRC_PREEMEPT | Preemptive release |
| 0b00001000 | RRC_TIME_ADVANCE | Handover impossible, timing advance out of range |
| 0b00001001 | RRC_CHANNEL_MODE | Channel mode unacceptable |
| 0b00001010 | RRC_FREQ_NOT_IMPL | Frequency not implemented |
| 0b01000001 | RRC_CALL_CLEARED | Call already cleared |
| 0b01011111 | RRC_INCORRECT_MSG | Semantically incorrect message |
| 0b01100000 | RRC_INVALID_MAN_INFO | Invalid mandatory information |
| 0b01100001 | RRC_MSG_NOT_IMPL | Message type non-existent or not implemented |
| 0b01100010 | RRC_MSG_NOT_COMPAT | Message type not compatible with protocol state |
| 0b01100100 | RRC_COND_IE_ERROR | Conditional IE error |
| 0b01100101 | RRC_NO_CELL_ALLOC | No cell allocation available |
| 0b01101111 | RRC_PROT_UNSPECIFIED | Protocol error unspecified |

History:

07-May-97	VK	Initial

21-Jul-97	LE	bit len instead of len

## R Support

Description:

The purpose of the R Support value is to provide the power class in case of  R-GSM band is supported.
Reference : [GSM_04.08] section 10.5.1.7 (Mobile Station Classmark 3)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| R Support | r_support | 3 |

History:

20-Apr-00	LG	Initial

## RX Level Access Minimum

Description:

The RXLEV-ACCESS-MIN field is coded as the binary representation of the minimum received signal level at the MS for which it is permitted to access the system.
Reference	: [GSM_04.08] section 10.5.2.4 (Cell Selection Parameters)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| RX Level Access Minimum | rxlev_access_min | 6 |

History:

07-May-97	VK	Initial

## RX Level Full Serving Cell

Description:

Reference	: [GSM_04.08] section 10.5.2.20 (Measurement Result)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| RX Level Full Serving Cell | rxlev_full | 6 |

History:

07-May-97	VK	Initial

## RX Level Sub Serving Cell

Description:

Reference	: [GSM_04.08] section 10.5.2.20 (Measurement Result)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| RX Level Sub Serving Cell | rxlev_sub | 6 |

History:

07-May-97	VK	Initial

## RX Level Ncell

Description:

Reference	: [GSM_04.08] section 10.5.2.20 (Measurement Result)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| RX Level Ncell | rx_lev_ncell | 6 |

History:

07-May-97	VK	Initial

## RX Quality Full Serving Cell

Description:

Reference	: [GSM_04.08] section 10.5.2.20 (Measurement Result), 10.5.2.20a (GPRS Measurement Result)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| RX Quality Full Serving Cell | rxqual_full | 3 |
| RX Quality | rxqual | 3 |

History:

07-May-97	VK	Initial
25-Apr-00	LG	rxqual added

## RX Quality Sub Serving Cell

Description:

Reference	: [GSM_04.08] section 10.5.2.20 (Measurement Result)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| RX Quality Sub Serving Cell | rxqual_sub | 3 |

History:

07-May-97	VK	Initial

## Sequence Code Used

Description:

Sequence Code Used indicates the value of the SEQ-CODE in the extended measurement frequency list information element used for defining the list of frequencies reported on. 
Reference : [GSM_04.08] section 10.5.2.45 (Extended Measurement Results)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Sequence Code Used | sc_used | 1 |

History:

28-Apr-00	LG	Initial

## Service Flag

Description:

Reference : [GSM_04.08] section 10.5.1.9 (Descriptive Group or Broadcast Call Reference)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Service Flag | sf | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | SF_VBS | broadcast call reference |
| 1 | SF_VGCS | group call reference |

History:

25-Apr-00	LG	Initial

## Set of AMR Codec Modes

Description:

The parameter is used to indicate the presence or absence of codecs of certain rates.
Reference : [GSM_04.08] section 10.5.2.21aa (Multirate Configuration)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Set of AMR Codec Modes | set_amr | 8 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0x80 | SET_AMR_12_2 | codec 12.2 kbit/s is part of subset |
| 0x40 | SET_AMR_10_2 | codec 10.2 kbit/s is part of subset |
| 0x20 | SET_AMR_7_95 | codec 7.95 kbit/s is part of subset |
| 0x10 | SET_AMR_7_4 | codec 7.4 kbit/s is part of subset |
| 0x08 | SET_AMR_6_7 | codec 6.7 kbit/s is part of subset |
| 0x04 | SET_AMR_5_9 | codec 5.9 kbit/s is part of subset |
| 0x02 | SET_AMR_5_15 | codec 5.15 kbit/s is part of subset |
| 0x01 | SET_AMR_4_75 | codec 4 .75 kbit/s is part of subset |

History:

25-Apr-00	LG	Initial

## ShortLSA ID

Description:

The parameter is used to identify the short form of an LSA id.
Reference : [GSM_04.08] section 10.5.2.35 (SI4 rest octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| ShortLSA ID | short_lsa_id | 10 |

History:

09-Mai-01	XBA	Initial

## Short Message Capability

Description:

Reference	: [GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Short Message Capability | sm_cap | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | SM_CAP_NO | SM capability not present |
| 1 | SM_CAP_YES | SM capability present |

History:

07-May-97	VK	Initial

## SI 13 Position

Description:

The parameter is used to indicate where SYSTEM INFORMATION TYPE 13 is sent.
Reference : [GSM_04.08] section 10.5.2.34 (SI3 Rest Octets), section 10.5.2.35 (SI4 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| SI13 Position | si13_pos | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | SI13_ON_BCCH_NORM | on BCCH Norm |
| 1 | SI13_ON_BCCH_EXT | on BCCH Ext. |

History:

05-Apr-00	LG	Initial

## SI13 Change Mark

Description:

This field is the binary representation of the SI change mark identifying the GPRS Mobile Allocation provided in SI13 and PSI13 messages.

Reference : [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| SI 13 Change Mark | si13_cm | 2 |

History:

27-Apr-00	LG	Initial

## SI Change Field

Description:

This field is the binary representation of which information was changed at the last indication in BCCH Change Mark field.
Reference : [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| SI Change Field | si_cf | 4 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | SI_CF_UNSPEC | Update of unspecified SI message(s) |
| 1 | SI_CF_1 | Update of SI1 message |
| 2 | SI_CF_2X | Update of SI2, SI2bis or SI2ter message |
| 3 | SI_CF_3UNTIL8 | Update of SI3, SI4, SI7 or SI8 message |
| 4 | SI_CF_9 | Update of SI9 message |
| DEF |  | Update of unknown SI message type |

History:

27-Apr-00	LG	Initial

## Signal Variance

Description:

The Signal Variance field contains the signal variance parameter calculated by the mobile station as defined in GSM 05.08. 
Reference : [GSM_04.08] section 10.5.2.20a (GPRS Measurement Results)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Signal Variance | sign_var | 6 |

History:

25-Apr-00	LG	Initial

## SoLSA Exclusive Access

Description:

The parameter defines an offset for LSA reselection between cells with same LSA priorities.
Reference : [GSM_04.08] section 10.5.2.35 (SI4 Rest Octets), section 10.5.2.37e (SI16 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| SoLSA Exclusive Access | solsa_exc_acc | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | SOLSA_EXCLUSIVE_NO |  |
| 1 | SOLSA_EXCLUSIVE_YES |  |

History:

05-Apr-00	LG	Initial

## SoLSA Supported

Description:

The parameter defines the support of SoLSA by the mobile equipment.
Reference : [GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| SoLSA Supported | solsa_supp | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | SOLSA_SUPP_NO | SoLSA not supported |
| 1 | SOLSA_SUPP_YES | SoLSA supported |

History:

19-Apr-00	LG	Initial

## Split PG Cycle Support

Description:

Reference : [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Split PG Cycle Support | spgc | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | SPGC_NO | SPLIT_PG_CYCLE is not supported on CCCH in this cell |
| 1 | SPGC_YES | SPLIT_PG_CYCLE is supported on CCCH in this cell |

History:

27-Apr-00	LG	Initial

## SS Screening Indicator

Description:

Reference	: [GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| SS Screening Indicator | ss_screen | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | SS_SCREEN_DEFAULT | default value of phase 1 |
| 1 | SS_SCREEN_PHASE_2 | capability of handling of ellipsis notation and phase 2 error handling |
| DEF |  | reserved |

History:

07-May-97	VK	Initial
11-Aug-98	VK	Add Values

## Start Ciphering

Description:

Reference	: [GSM_04.08] section 10.5.2.9 (Cipher Mode Setting)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Start Ciphering | sc | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | START_CIPH_NO | No ciphering |
| 1 | START_CIPH_YES | Start ciphering |

History:

07-May-97	VK	Initial

## Start Mode

Description:

Reference : [GSM_04.08] section 10.5.2.21aa (Multirate Configuration), [GSM_05.09] section 3.4.1

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Start Mode | st_mode | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | CODEC_MODE_1 | lowest bit rate |
| 1 | CODEC_MODE_2 | second lowest rate if codec includes more then 1 mode |
| 2 | CODEC_MODE_3 | third lowest rate if codec includes more then 2 modes |
| 3 | CODEC_MODE_4 | highest mode if codec includes 4 modes |

History:

25-Apr-00	LG	Initial

## Suspension Cause

Description:

The purpose of the Suspension Cause information element is to provide the reason for the establishment of the dedicated circuit which generated the GPRS suspension.

Reference : [GSM_04.08] section 10.5.2.47 (Suspension Cause)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Suspension Cause | susp_cause | 8 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | SUSP_C_CALL | Mobile originated or mobile terminated call |
| 1 | SUSP_C_LU | Location Update |
| 2 | SUSP_C_SMS | Short message service |
| 3 | SUSP_C_SS | Supplementary service activation |
| 4 | SUSP_C_VBS | Voice broadcast or group call |
| DEF |  | shall be treated as 0 |

History:

28-Apr-00	LG	Initial

## Switch Measurement Time

Description:

The fields indicate the time needed for the mobile station to switch from one radio channel to another, perform a neighbour cell power measurement (and to switch to switch from one radio channel to another in case of sws_time) 
The field is coded as the binary value for time needed (in fourth of a timeslot) minus 1.
Reference	: [GSM_04.08] section 10.5.1.7 (Mobile Classmark 3)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Switch Measurement Time | sw_time | 4 |
| Switch Measurement Switch Time | sws_time | 4 |

History:

20-Apr-00	LG	Initial

## Synchronisation indication

Description:

Reference	: [GSM_04.08] section 10.5.2.39 (Synchronisation Indication)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Synchronisation indication | si | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | SYI_NON_SYNCH | Non-synchronized |
| 1 | SYI_NORM_SYNCH | Synchronized |
| 2 | SYI_PRE_SYNCH | Pre-synchronised |
| 3 | SYI_PSEUDO_SYNCH | Pseudo-synchronised |

History:

07-May-97	VK	Initial

## T1'

Description:

Reference	: [GSM_04.08] section 10.5.2.30 (Request Reference)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| T1' | t1 | 5 |

History:

07-May-97	VK	Initial

## T2

Description:

Reference	: [GSM_04.08] section 10.5.2.30 (Request Reference)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| T2 | t2 | 5 |

History:

07-May-97	VK	Initial

## T3

Description:

Reference	: [GSM_04.08] section 10.5.2.30 (Request Reference)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| T3 | t3 | 6 |

History:

07-May-97	VK	Initial

## T3122 time-out value

Description:

The purpose of the Wait Indication information element is to provide the time the mobile station shall wait before attempting another channel request.
Reference : [GSM_04.08] section 10.5.2.43

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| T 3122 Wait Indication | t3122 | 8 |
| T 3122 Wait Indication | t3122_2 | 8 |
| T 3122 Wait Indication | t3122_3 | 8 |
| T 3122 Wait Indication | t3122_4 | 8 |

History:

07-May-97	VK	Initial

## T3168 timeout value

Description:

This field is the binary representation of the timeout value of timer T3168. Range: 0 to 7. The timeout value is given as the binary value plus one in units of 500 millisecond.

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| T3168 | t3168 | 3 |

History:

31-May-99	SL	Initial

## T3192 timeout value

Description:

This field is the binary representation of the timeout value of timer T3192. Range: 0 to 7. The timeout value is given as the binary value plus one in units of 500 millisecond.

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets), [GSM_04.60] section 12.24

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| T3192 | t3192 | 3 |

History:

31-May-99	SL	Initial

## T3212 time-out value

Description:

Reference	: [GSM_04.08] section 10.5.2.11 (Control Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| T 3212 time-out value | t3212 | 8 |

History:

07-May-97	VK	Initial

## T_AVG_T Value

Description:

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets).

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| T_AVG_T Value | t_avg_t | 5 |

History:

11-Jun-99	CAN	Initial

## T_AVG_W Value

Description:

Reference	: [GSM_04.08] section 10.5.2.37b (SI13 Rest Octets).

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| T_AVG_W Value | t_avg_w | 5 |

History:

11-Jun-99	CAN	Initial

## Target Mode

Description:

Reference	: [GSM_04.08] section 10.5.2.42a (VGCS Target Mode Indication)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Target mode | tm | 2 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | TM_DED | dedicated mode |
| 1 | TM_GROUP | group transmit mode |
| DEF |  | reserved |

History:

28-Apr-00	LG	Initial

## Temporary Logical Link Identifier

Description:

The coding of TLLI (temporary logical link identifier) is left to each administration. 
Reference : [GSM_04.08] section 10.5.2.16 (IA rest Octets), [GSM_03.03]

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Temporary Logical Link Identifier | ded_tlli | 32 |

History:

25-Apr-00	LG	Initial

## Temporary Offset

Description:

The parameter is used by the mobile station to apply an offset to the value of C2 during penalty time.
Reference : [GSM_04.08] section 10.5.2.34 (SI3 Rest Octets), section 10.5.2.35 (SI4 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Temporary Offset | temp_offs | 3 |

History:

05-Apr-00	LG	Initial

## TFI Assignment

Description:

The TFI Assignment field is the binary representation of Temporary Flow Identity. 
Reference : [GSM_04.08] section 10.5.2.16 (IA rest Octets), [GSM_04.60]

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| TFI | tfi | 5 |

History:

25-Apr-00	LG	Initial

## Time Difference

Description:

The parameter defines the mobile observed time difference in half bits.
Reference : [GSM_04.08] section 10.5.2.21a (Mobile Time Difference)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Time difference | diff | 21 |

History:

02-Jun-98	LE	Initial

## Time Difference

Description:

The purpose of the Time Difference information element is to provide information about the synchronization difference between the time bases of two Base Stations. This type of information element is used in relation with the pseudo-synchronization scheme, see TS GSM 05.10.
Reference : [GSM_04.08] section 10.5.2.41

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Time Difference | time_diff | 8 |

History:

07-May-97	VK	Initial

## Time Slot

Description:

The TN field is coded as the binary representation of the timeslot number as defined in TS GSM 05.10.
Reference	: [GSM_04.08] section 10.5.2.5 (Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Time Slot | tn | 3 |
| Downlink Control Timeslot | dl_ctrl_tn | 3 |

History:

07-May-97	VK	Initial

27-Apr-00	LG	dl_ctrl_tn added

## Timeslot Allocation

Description:

The Timeslot Allocation field indicates whether a timeslot is used in a TBF or an PCCCH or not. A bit set in the field means that the appropriate timeslot is assigned.  
Reference	: [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Timeslot Allocation | ts_all | 8 |

History:

26-Apr-00	LG	Initial

## Timing Advance Index

Description:

Reference	: [GSM_04.08] section 10.5.2.16 (IA Rest Octets), section 10.5.2.25c (RR Packet Uplink Assignment), [GSM_04.60] section 12.12 (Packet Timing Advance)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Timing Advance Index | ta_idx | 4 |

History:

25-Apr-00	LG	Initial

## Timing Advance Timeslot Number

Description:

Reference	: [GSM_04.08] section 10.5.2.25c (RR Packet Uplink Assignment), [GSM_04.60] section 12.12 (Packet Timing Advance)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Timing Advance Timeslot Number | ta_nm | 3 |

History:

26-Apr-00	LG	Initial

## Timing Advance Valid

Description:

The Timing Advance Valid field indicates the validity of the timing advance value given in the Timing Advance IE.  
Reference	: [GSM_04.08] section 10.5.2.16 (IA Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Timing Advance Valid | ta_valid | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | TA_VALID_NO | the value is not valid |
| 1 | TA_VALID_YES | the value is valid |

History:

25-Apr-00	LG	Initial

## Timing Advance value

Description:

Reference	: [GSM_04.08] section 10.5.2.40 (Timing Advance)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Timing advance value | ta | 6 |

History:

07-May-97	VK	Initial

## TLLI Block Channel Coding

Description:

The parameter indicates the channel coding to be used for RLC data block comprising TLLI for contention resolution.
Reference : [GSM_04.08] section 10.5.2.16 (IA rest Octets), [GSM_03.03]

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| TLLI Block Channel Coding | tlli_bcc | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | TLLI_BCC1 | mobile station shall use CS-1 |
| 1 | TLLI_BCC_CODED | mobile station shall use CS specified by Channel Coding |

History:

25-Apr-00	LG	Initial

## TMSI

Description:

The purpose of the TMSI information element is to provide the Temporary Mobile Subscriber Identity for paging purposes.
Reference : [GSM_04.08] section 10.5.2.42

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| TMSI | tmsi | 32 |
| TMSI | tmsi_2 | 32 |
| TMSI | tmsi_3 | 32 |
| TMSI | tmsi_4 | 32 |

History:

07-May-97	VK	Initial

## Training Sequence Code

Description:

The TSC field is coded as the binary representation of the Training Sequence code as defined in TS GSM 05.03
Reference	: [GSM_04.08] section 10.5.2.5 (Channel Description)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Training Sequence Code | tsc | 3 |

History:

07-May-97	VK	Initial

## Two-message Assignment

Description:

Reference	: [GSM_04.08] section 10.5.2.25b (Dedicated Mode Or TBF)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Two-message Assignment | tma | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | TMA_0 | no meaning |
| 1 | TMA_1 | the message is the first of two in a two message assignment |

History:

26-Apr-00	LG	Initial

## Tx-integer

Description:

Reference	: [GSM_04.08] section 10.5.2.29 (RACH Control Parameters)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Tx-integer | tx_integer | 4 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | SPREAD_TRANS_3 | 3 slots used to spread transmission |
| 1 | SPREAD_TRANS_4 | 4 slots used to spread transmission |
| 2 | SPREAD_TRANS_5 | 5 slots used to spread transmission |
| 3 | SPREAD_TRANS_6 | 6 slots used to spread transmission |
| 4 | SPREAD_TRANS_7 | 7 slots used to spread transmission |
| 5 | SPREAD_TRANS_8 | 8 slots used to spread transmission |
| 6 | SPREAD_TRANS_9 | 9 slots used to spread transmission |
| 7 | SPREAD_TRANS_10 | 10 slots used to spread transmission |
| 8 | SPREAD_TRANS_11 | 11 slots used to spread transmission |
| 9 | SPREAD_TRANS_12 | 12 slots used to spread transmission |
| 10 | SPREAD_TRANS_14 | 14slots used to spread transmission |
| 11 | SPREAD_TRANS_16 | 16 slots used to spread transmission |
| 12 | SPREAD_TRANS_20 | 20 slots used to spread transmission |
| 13 | SPREAD_TRANS_25 | 25 slots used to spread transmission |
| 14 | SPREAD_TRANS_32 | 32 slots used to spread transmission |
| 15 | SPREAD_TRANS_50 | 50 slots used to spread transmission |

History:

07-May-97	VK	Initial

## Type of identity

Description:

Reference	: [GSM_04.08] section 10.5.1.4 (Mobile Identity)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Type of identity | ident_type | 3 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | ID_TYPE_NO_IDENT | No Identity |
| 1 | ID_TYPE_IMSI | IMSI |
| 2 | ID_TYPE_IMEI | IMEI |
| 3 | ID_TYPE_IMEISV | IMEISV |
| 4 | ID_TYPE_TMSI | TMSI |
| DEF |  | reserved |

History:

07-May-97	VK	Initial

## UCS2 Treatment

Description:

The UCS2 treatment field indicates the likely treatment by the mobile station of UCS2 encoded character strings.
Reference : [GSM_04.08] section 10.5.1.7 (Mobile Station Classmark 3)

Definition:

| Long name | short name | bit len |
| --- | --- | --- |
| UCS2 Treatment | ucs2_treat | 1 |

Values:

| Value | c-macro | comment |
| --- | --- | --- |
| 0 | UCS2_DEF_PREF | preference for default alphabet over UCS2 |
| 1 | UCS2_NO_PREF | no preference between default alphabet and UCS2 |

History:

20-Apr-00	LG	Initial

## Uplink State Flag

Description:

The parameter is the binary representation of the uplink state flag.

Reference : [GSM_04.08] section 10.5.2.16 (IA Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Uplink State Flag | usf | 3 |

History:

25-Apr-00	LG	Initial

## USF Granularity

Description:

The parameter indicates the USF granularity to be applied by the mobile station when it is assigned a TBF using Dynamic Allocation.

Reference : [GSM_04.08] section 10.5.2.16 (IA Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| USF Granularity | usf_gran | 1 |

Values:

| value | c-macro | Comment |
| --- | --- | --- |
| 0 | USF_GRAN1 | the mobile shall send one RLC/MAC block |
| 1 | USF_GRAN4 | the mobile shall send four consecutive RLC/MAC blocks |

History:

25-Apr-00	LG	Initial

## VBS Notification Reception

Description:

The parameter defines the VBS capability of the mobile station and whether notifications are wanted.
Reference : [GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)

Definition:

| long name | Short name | bit len |
| --- | --- | --- |
| VBS Notification Reception | vbs_not_rec | 1 |

Values:

| value | c-macro | Comment |
| --- | --- | --- |
| 0 | VBS_NO_CAP_NOT_WONTED | no VBS capability or no notifications wanted |
| 1 | VBS_WANTED | VBS capability and notifications wanted |

History:

19-Apr-00	LG	Initial

## VGCS Notification Reception

Description:

The parameter defines the VGCS capability of the mobile station and whether notifications are wanted.

Reference : [GSM_04.08] section 10.5.1.6 (Mobile Station Classmark 2)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| VGCS Notification Reception | vgcs_not_rec | 1 |

Values:

| value | c-macro | comment |
| --- | --- | --- |
| 0 | VGCS_NO_CAP_NOT_WONTED | no VGCS capability or no notifications wanted |
| 1 | VGCS_WANTED | VGCS capability and notifications wanted |

History:

19-Apr-00	LG	Initial