Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document

GSM Protocol Stack

Message Sequence Charts

GSMS

(Edit via File/Properties/Summary!)

| Document Number: | (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.2 |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1998-Nov-05 |
| Last changed: | 2003-May-15 by XGUTTEFE |
| File Name: | gsms.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1998-Nov-05 | Stefan Lemke |  | 0.1 |  | 1 |
| 2003-May-15 | XGUTTEFE |  | 0.2 | Draft |  |

Notes:

- Initial version

Table of Contents

1.1	References	5

1.2	Abbreviations	7

1.1	Terms	10

2	Overview	10

1.2	GRR (RLC/MAC) – Radio Link Control/Medium Access Control	11

1.3	LLC – Logical Link Control	11

1.4	GMM – GPRS Mobility Management	11

1.5	SM – Session Management	11

1.6	SNDCP - Subnetwork Dependant Convergence Protocol	11

1.7	GACI – GPRS Application Control Interface	11

1.8	USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	11

1.9	TOM – Tunnelling of Messages	11

3	Introduction	12

3.1	Dynamic Configuration	12

3.2	Timer Modi	12

4	Initialisation	12

4.1	Initial Parameters from the SIM card	12

4.2	Reading SMS from SIM card	13

4.3	Reading SMS from Mobile Memory	15

4.4	Set Mobile Originated Short Message Service	15

5	Flow Control	16

5.1	Availability Indication of GPRS	16

6	Mobile Originated Short Message Service	16

6.1	Establishment Phase	16

6.1.1	Initiation by Man Machine Interface	16

6.1.2	Initiation by Man Machine Interface, GPRS preferred but not registered	18

6.1.3	Sending the MO SM in a UNITDATA REQ	19

6.1.4	Timeout TR1M	19

6.1.5	Multiple Request  by Man Machine Interface	19

6.1.6	Confirmation to CP DATA Message	20

6.1.7	Wrong Message received	21

6.1.8	CP Error received	22

6.1.9	Timeout TC1M	23

6.1.10	Timeout TR1M (state GSMS MO WAIT FOR RP ACK)	24

6.2	Active Phase	25

6.2.1	Acknowledge of the Infrastructure	25

6.2.2	Error signalled by the Infrastructure	26

6.2.3	Wrong Message signalled by the Infrastructure (CP layer)	27

6.2.4	Wrong Message signalled by the Infrastructure (RL layer)	28

6.2.5	Timeout TR1M	29

6.3	Mobile Originated Short Message Command	30

7	Mobile Terminated Short Message Service	33

7.1	Reception of a Short Message	35

7.2	Reception of CP ERROR message	36

7.3	Reception of CP ACK or unknown message	36

7.4	Second Mobile Terminated Connection	37

7.5	Additional Message for  Mobile Terminated Connection	38

7.6	Display Message	39

7.7	Store Message in ME memory	40

7.7.1	No Storage available on ME memory, no other opportunities	42

7.8	Store Message in SIM memory	43

7.8.1	Storage on SIM available, writing successfull	43

7.8.2	Storage on SIM available, writing not successfull	44

7.8.3	No Storage on SIM available	45

7.8.4	No Storage available on SIM memory, no other opportunities	46

7.9	Replace Short Message	47

7.9.1	Replace in Mobile Station Memory	47

7.10	Mobile Terminated Short Message Status	48

7.11	SIM Toolkit Data Download	49

7.12	SMS - Depersonalisation	50

8	Short Message Service Procedures	50

9	Data Formats	50

Appendices	52

A.	Acronyms	52

B.	Glossary	52

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 05.02 version 8.0.0 Release 1999
Digital cellular telecommunications system (Phase 2+);
Multiplexing and multiple access on the radio path

[2]	GSM 04.60 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station (MS) - Base Station System (BSS) interface;
Radio Link Control/ Medium Access Control (RLC/MAC) protocol

[3]	GSM 04.08 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface layer 3 specification

[4]	GSM 03.64 version 6.1.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Overall description of the GPRS radio interface; Stage 2

[5]	GSM 03.60 version 6.3.1 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Service description; Stage 2

[6]	GSM 04.07 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface signalling layer 3; General aspects

[7]	GSM 04.64 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station - Serving GPRS Support Node (MS-SGSN)
Logical Link Control (LLC) layer specification

[8]	GSM 05.08 version 6.4.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem link control

[9]	GSM 05.10 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem synchronization

[10]	GSM 03.20 TS 100 929: July 1998 (GSM 03.20 version 6.0.1)
Security related network functions, ETSI

[11]	Draft GSM 03.22: August 1998 (GSM 03.22 version 6.1.0)
Functions related to Mobile Station (MS) in idle mode and group receive mode, ETSI

[12]	GSM 04.65 V6.3.0: Subnetwork Dependant Convergence Protocol

ETSI, March 1999

[13]	ITU-T V42bis ITU-T, Recommendation V.42 bis 1990

[14]	GSM 09.60 GPRS Tunneling Protocol (GTP) across the Gn and Gp Interface

[15]	RFC 1661 IETF STD 51 July 1994
The Point-to-Point Protocol (PPP)

[16]	RFC 1662 IETF STD 51 July 1994
PPP in HDLC-like Framing

[17]	RFC 1570 January 1994
PPP LCP Extensions

[18]	RFC 1989 August 1996
PPP Link Quality Monitoring

[19]	RFC 1332 May 1992
The PPP Internet Protocol Control Protocol (IPCP)

[20]	RFC 1877 December 1995
PPP IPCP Extensions for Name Server Addresses

[21]	RFC 2153 May 1997
PPP Vendor Extensions

[22]	RFC 1334 October 1992
PPP Authentication Protocols (for Password Authentication Protocol only)

[23]	RFC 1994 August 1996
PPP Challenge Handshake Authentication Protocol (CHAP)

[24]	TIA/EIA-136-370
Packet-Data Services – Enhanced General Packet Radio for TIA/EIA-136 (EGPRS-136) - Overview, Telecommunications Industry Association

[25]	TIA/EIA-136-376
Packet-Data Services – EGPRS-136 Mobility Management, Telecommunications Industry Association

[26]	TIA/EIA-136-972
Packet-Data Services – Stage 2 Description, Telecommunications Industry Association

## Abbreviations

ACI	Application Control Interface
AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CCI	Compression and Ciphering Interface
CHAP	Challenge Handshake Authentication Protocol
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DCOMP	Identifier of the user data compression algorithm used for the N-DPU
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

E	Extension bit
EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GACI	GPRS Application Control Interface
GMM	GPRS Mobility Management
GP	Guard Period
GRR	GPRS RR
GSM	Global System for Mobile Communication

HDLC	High-level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
IP	Internet Protocol
IPCP	Internet Protocol Control Protocol 
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Ciphering Key

L	Length Indicator
LAI	Location Area Information
LCP 	Link Control Protocol 
LISR	Low level Interrupt Service Routine
LLC	Logical Link Control
LPD	Link Protocol Discriminator
LQM	Link Quality Monitoring

M	More bit used to indicate the last segment of N-DPU
MAC	Medium Access Control
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MT	Mobile Termination

N(R)	Receive Number
N(S)	Send Number
NC	Network Control
NCC	National Colour Code
NCP 	Network Control Protocol
NECI	New Establishment Causes included
N-PDU	Network Protocol Data Unit
NSAPI	Network Layer Service Access Point Identifier

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PACCH	Packet Associated Control Channel
PAP	Password Authentication Protocol
PBCCH	Packet BCCH
PCCCH	Packet CCCH
PCOMP	Identifier of the protocol control information compression algorithm used for the N-DPU
PDCH	Packet Data Channel
PDP	Packet Data Protocol e.g. IP or X.25
PDTCH	Packet Data Traffic Channel
PRACH	Packet RACH
PSI	Packet System Information
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Data Unit
PL	Physical Layer
PLMN	Public Land Mobile Network
PPC	Packet Physical Convergence
PPP 	Point-to-Point Protocol
PTP	Point to Point

QoS	Quality of Service

RACH	Random Access Channel
REJ	Reject Frame
RLC	Radio Link Control
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Stand alone Dedicated Control Channel
SDU	Service Data Unit
SGSN	Serving GPRS Support Node
SIM	Subscriber Identity Module
SM	Session Management
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SNDCP	Subnetwork Dependant Convergence Protocol
SNSM	SNDCP-SM
SS	Supplementary Services

TAP	Test Application Program
TBF	Temporary Block Flow
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TCP	Transmission Control Protocol
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TFI	Temporary Flow Identifier
TLLI	Temporary Logical Link Identifier
TMSI	Temporary Mobile Subscriber Identity
TOM	Tunnelling of Messages
TQI	Temporary Queuing Identifier

UA	Unnumbered Acknowledgement Frame
UART	Universal Asynchronous Receiver Transmitter
UI	Unnumbered Information Frame
USF	Uplink State Flag

V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable
VPLMN	Visited Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for GPRS consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity.

Figure 2-1: Architecture of the GSM/GPRS protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the GPRS protocol stack are:

## GRR (RLC/MAC) – Radio Link Control/Medium Access Control

This layer contains two functions: The Radio Link Control function provides a radio-solution-dependent reliable link. The Medium Access Control function controls the access signalling (request and grant) procedures for the radio channel, and the mapping of LLC frames onto the GSM physical channel.

## LLC – Logical Link Control

The LLC entity provides multiple highly reliable logical links for asynchronous data transfer between the MS and the network. It supports variable-length information frames, acknowledged and unacknowledged data transfer, flow and sequence control, error detection and recovery, notification of unrecoverable errors, user identity confidentiality, and ciphering of user and signaling data.

## GMM – GPRS Mobility Management

The GMM entity provides procedures for the mobility of the MS, such as informing the network of its present location, and user identity confidentiality. It manages the GMM context (attach, detach, routing area updating), supports security functions such as authentication of user and MS, controls ciphering of data, and initiates the response to paging messages.

## SM – Session Management

The main function of the session management (SM) is to support PDP context handling of the user terminal. Session Management activates, modifies and deletes the contexts for packet data protocols (PDP). Session Management services are provided at the SMREG-SAP and the SNSM-SAP for anonymous and non-anonymous access. The non-anonymous and anonymous access procedures for PDP context activation and PDP context deactivation are available at the SMREG-SAP. In addition there exists a PDP context modification for non-anonymous PDP contexts.

## SNDCP - Subnetwork Dependant Convergence Protocol

- SNDCP carries out all functions related to transfer of Network layer Protocol Data Units (N-PDUs) over GPRS in a transparent way. SNDCP helps to improve channel efficiency by means of compression techniques. The set of protocol entities above SNDCP consists of commonly used network protocols. They all use the same SNDCP entity, which then performs multiplexing of data coming from different sources to be sent using the service provided by the LLC layer.

## GACI – GPRS Application Control Interface

The GACI is the GPRS extension of the ACI. It is specified in GSM 07.07 and 07.60. It is responsible for processing of the GPRS related AT Commands to setup, activate and deactivate the PDP context parameter. It also provides functionality for the interworking between GMM/SM/SNDCP and a packet oriented protocol like PPP.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the GPRS. The driver has to be able to perform flow control.

## TOM – Tunnelling of Messages

The TOM entity is present if and only if HS136 is supported (the feature flag FF_HS136 is enabled).

The main function of TOM is to tunnel non-GSM signalling messages between the MS and the SGSN. The only non-GSM signalling which is currently supported by TOM is for the EGPRS-136 system (according to TIA/EIA-136-376). Data transfer in both uplink and downlink direction is possible. Two different priorities (high, low) of signalling data transfer are supported. TOM uses the unacknowledged mode of LLC and the acknowledged mode of GRR (RLC/MAC).

# Introduction

## Dynamic Configuration

For short message service (SMS) a lot of dynamic configurations are defined. That means it is possible at run-time to change in behaviour of SMS by sending of a string from a test device to SMS.

SMS uses a keyword table to interpretate the strings. The strings have the following general format:

<keyword> = (parameter 1, parameter 2, ...)

Keywords without parameters are defines only by using the keyword. For keywords with one parameter the bracket aren´t necessary.

The following table is an overview of the possible configurations:

| keyword | Parameter |
| --- | --- |
| TIMER_SET TIMER_RESET TIMER_SPEED_UP TIMER_SLOW_DOWN TIMER_SUPPRESS | name value unit name name factor name factor name |

If mobility management is used in a multi instances system the dynamic configurations have the parameter instance.

The keywords TIMER_SET, TIMER_RESET, TIMER_SPEED_UP, TIMER_SLOW_DOWN and TIMER_SUPPRESS are used for manipulting timers. The different timers are identified by their names. The time used by next starting of the timer is manipulated by the dynamic configurations.

## Timer Modi

For each timer of SMS it is possible to define a mode by a dynamic configuration. If a timer is started by SMS the given start value is manipulated according to the timer mode.

The timer mode TIMER_SET defines a new timer value instead of the origin start value.

The default timer mode is TIMER_RESET which makes no manipulation of the start value.

TIMER_SPEED_UP is used to speed up a timer by the given factor. The start value is divided by the factor. The minimum time is one unit.

The opposite mode is TIMER_SLOW_DOWN. The start value is multiplied by the given factor.

TIMER_SUPPRESS is used to suppress the timer start.

# Initialisation

## Initial Parameters from the SIM card

MMI                   SMS                       SIM

|                    |                         |

|                    |   SIM SMS INSERT IND    |

|                    *<========================*

|                 (SMS 1)                      |

|                    |                         |

(SMS 1)

The SIM application sends to SMS the initial parameter from the SIM card. This are the SMS status informations whether short message storage capacity is available on the SIM card, the last used message reference for mobile originated short message service and the SMS download flag. The SMS download flag defines whether data download via SMS for SIM toolkit is possible or not.

The SMS initialisation procedure starts now reading the SMS messages from the SIM card and the mobile memory.

## Reading SMS from SIM card

MMI                   SMS                       SIM

|                    |                         |

|                    |   SIM READ RECORD REQ   |

|                    *<========================*

|                    |                       (SIM 1)

|                    |   SIM READ RECORD CNF   |

|                    *<========================*

|                 (SMS 1)                      |

a) free record

|                    |                         |

|                    *                         |

|                 (SMS 2)                      |

|                    |                         |

b) mobile originated message

|                    |                         |

|  MNSMS ALERT IND   |                         |

*<===================*                         |

(MMI 1)                |                         |

|                    |                         |

c) mobile terminated message

|                    |                         |

|  MNSMS ALERT IND   |                         |

*<===================*                         |

(MMI 1)                |                         |

|                    |                         |

(SIM 1)

Starting with record 1 all SMS records are read. If the answer from the SIM application indicates that all records are read this procedure is finished.

(SMS 1)

The SIM application has read the requested record of the SIM card.

(SMS 2)

If it is a free record the status of the SMS message is stored.

(MMI 1)

If it is a mobile originated message, the status and the protocol identifier are stored from the SMS message. An indication is send to the MMI.

(MMI 1)

If it is a mobile terminated message, the status and the protocol identifier are stored from the SMS message. An indication is send to the MMI.

The status and protocol identifier backup for all SMS messages is used by SMS for faster access to the SIM card.

## Reading SMS from Mobile Memory

MMI                   SMS                       SIM

|                    |                         |

|                    *<pcm_read_record>        |

|                 (SMS 1)                      |

a) free record

|                    |                         |

|                    *                         |

|                 (SMS 2)                      |

|                    |                         |

b) mobile originated message

|                    |                         |

|  MNSMS ALERT IND   |                         |

*<===================*                         |

(MMI 1)                |                         |

|                    |                         |

c) mobile terminated message

|                    |                         |

|  MNSMS ALERT IND   |                         |

*<===================*                         |

(MMI 1)                |                         |

|                    |                         |

(SMS 1)

Starting with record 1 all SMS records are read from the permanent configuration memory. If the answer from the permanent configuration memory indicates that all records are read this procedure is finished.

(SMS 2)

If it is a free record the status of the SMS message is stored.

(MMI 1)

If it is a mobile originated message, the status and the protocol identifier are stored from the SMS message. An indication is send to the MMI.

(MMI 2)

If it is a mobile terminated message, the status and the protocol identifier are stored from the SMS message. An indication is send to the MMI.

The status and protocol identifier backup for all SMS messages is used by SMS for faster access to the permanent configuration memory.

## Set Mobile Originated Short Message Service

MMI                   SMS                       SIM

|                    |                         |

| MNSMS MO SERV REQ  |                         |

*===================>*                         |

| MNSMS MO SERV CNF  |                         |

*<===================*                         |

(MMI 1)                |                         |

|                    |                         |

(MMI 1)

The MMI configures the preferred messages service (GPRS or normal CCT) for mobile originated messages. Possible choices are: use GPRS only, prefer using GPRS, prefer using CCT and use CCT only.

# Flow Control

The flow control between the (G)SMS entity and the LL sublayer is managed in the following way: LL indicates towards SMS its availability to send one data unit by sending a LL UNITREADY IND. SMS indicates that it is ready to receive a unitdata indication by sending a LL GETUNITDATA REQ.

## Availability Indication of GPRS

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |        LL UNITREADY IND      |

|                    |                 *<=============================*

|                    |               (SMS 1)             |            |

|                    |                 |      LL GETUNITDATA REQ      |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |                 |                 |            |

(SMS 1)

The SMS entity assumes that GPRS is not available on startup. It is notified that GPRS is ready to send data units when it receives a LL UNITREADY IND by the GPRS LL.

(LL 1)

The SMS entity informs LL that it is ready to receive data unit indications.

# Mobile Originated Short Message Service

The mobile originated short message service enables the user to send a text message to the infrastructure. The receiving side is the so called service center (SC), which handles sending of the message to the destination address. In the following it is assumed that SMS is configured to prefer using GPRS or to use GPRS only for mobile originated short messages. (see 4.4)

## Establishment Phase

### Initiation by Man Machine Interface

MMI                 SMS(RL)           SMS(CP)            SIM          GMM

|                    |                 |                 |            |

| MNSMS SUBMIT REQ   |                 |                 |            |

*===================>*                 |                 |            |

|                 (SMS 1)              |                 |            |

|                    |          SIM UPDATE REQ           |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |cp_establish_req |                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |      GMMSMS REG STATE REQ    |

|                    |                 *=============================>*

|                    |                 |                 |        (GMM 1)

|                    |                 |      GMMSMS REG STATE RES    |

|                    |                 *<=============================*

|                    |              (SMS 3)              |            |

|                    |                 |                 |            |

(SMS 1)

The user starts sending of a short message.

(SIM 1)

Each short message contains a message reference. This number is used to identifiy this short message for later transactions. The message reference is stored on the SIM card, if it is a phase 2/2+ SIM card.

(SMS 2)

The RP DATA message is created and forwarded to the Control Protocol layer. The timer TR1M is started.

(GMM 1)

The RP DATA message is stored and a registration request is issued in order to check if GPRS is registered and therefore still able to send data units. This is necessary since it might have been a long while since the last UNITREADY IND was received and GPRS might not be registered anymore.

(SMS 3)

Only if a positive registration information is received by the CP layer (meaning that GPRS is still registered and able to send data units) the SMS continues to use GPRS as shown in 7.1.3. If a negative registration information is received and the user has configured to prefer GPRS for MO SM, the SMS entity switches to use CCT (see 7.1.2 for more information). If the user has configured to use only GPRS for MO SM and GPRS is not registered, no information is sent out and another attempt to use GPRS is made when TR1M expires. In this case the user is informed about the negative end of the procedure when the timer TR1M times out and the maximum number of RP layer retransmissions is reached.

### Initiation by Man Machine Interface, GPRS preferred but not registered

MMI                 SMS(RL)           SMS(CP)            SIM        GMM/MM

|                    |                 |                 |            |

| MNSMS SUBMIT REQ   |                 |                 |            |

*===================>*                 |                 |            |

|                 (SMS 1)              |                 |            |

|                    |          SIM UPDATE REQ           |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |cp_establish_req |                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |      GMMSMS REG STATE REQ    |

|                    |                 *=============================>*

|                    |                 |                 |        (GMM 1)

|                    |                 |      GMMSMS REG STATE RES    |

|                    |                 *<=============================*

|                    |              (SMS 3)              |            |

|                    |                 |      MM ESTABLISH REQ        |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |                 |                 |            |

(SMS 1)

The user starts sending of a short message. (Previously the SMS entity was configured to prefer using GPRS for mobile originated messages and to use circuit switched service if GPRS is not available.)

(SIM 1)

Each short message contains a message reference. This number is used to identifiy this short message for later transactions. The message reference is stored on the SIM card, if it is a phase 2/2+ SIM card.

(SMS 2)

The RP DATA message is created and forwarded to the Control Protocol layer. The timer TR1M is started.

(GMM 1)

The RP DATA message is stored and a registration request is issued in order to check if GPRS is registered and therefore still able to send data units. This is necessary since it might have been a long while since the last UNITREADY IND was received and GPRS might not be registered anymore.

(SMS 3)

GPRS is not registered anymore and a negative registration information is received from GMM.

(MM 1)

The SMS entity falls back to using normal circuit switched short message service and sends an establish request to MM. From this point on the procedure continues as described in the message sequence chart document for normal circuit switched SMS.

### Sending the MO SM in a UNITDATA REQ

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |        LL UNITDATA REQ       |

|                    |                 |           (CP DATA)          |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |                 |     LL UNITREADY IND         |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |                 |                 |            |

(LL 1)

The mobile originated SM is sent via LL.

(SMS 1)

The LL layer confirms that it is ready to receive another data unit by the SMS entity.

### Timeout TR1M

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    *<timeout TR1M>   |                 |            |

|                 (SMS 1)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

In the relay layer the timer TR1M times-out. That means that no answer has received from the infrastructure.

(MMI 1)

The user is informed about the abort of the mobile originated short message service.

### Multiple Request  by Man Machine Interface

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|  MNSMS SUBMIT REQ  |                 |                 |            |

*===================>*                 |                 |            |

|                 (SMS 1)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

The user tries to send an additional mobile originated short message althougth the current transaction has not finished. At one time only one transaction is allowed.

(MMI 1)

The user is informed about rejection of the mobile originated short message request.

### Confirmation to CP DATA Message

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |           LL UNITDATA IN     |

|                    |                 |              (CP ACK)        |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |                 |        LL GETUNITDATA REQ    |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |                 |                 |            |

(SMS 1)

Control Protocol is in the state GSMS MO Wait for CP ACK. That means it waits for the acknowledgement of the previous CP-DATA message. This acknowlegment is received. The timer T1CM is stopped and Control Protocol enters the state GSMS MO Wait for RP ACK where it waits for the arrival of the RP layer acknowledgement in a CP-DATA message.

(LL 1)

The SMS entity indicates that it is ready to receive another data unit.

### Wrong Message received

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |           LL UNITDATA IND    |

|                    |                 |       (CP DATA or unknown)   |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |                 |           LL UNITDATA REQ    |

|                    |                 |             (CP ERROR)       |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |                 |                 |            |

|                    |   rl_error_ind  |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |         LL UNITREADY IND     |

|                    |                 *<=============================*

|                    |              (SMS 4)              |            |

|                    |                 |                 |            |

|                    |                 |         LL GETUNITDATA REQ   |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 2)

|                    |                 |                 |            |

(SMS 1)

Control Protocol exspects a CP ACK message as response to its own CP DATA message. Instead of this a CP DATA or unknown message receives. The timer T1CM is stopped.

(LL 1)

Control Protocol answers with a CP ERROR message and the cause #98 for a CP DATA message or #97 for an unknown message.

(SMS 2)

Relay Layer is informed about the error.

(MMI 1)

The user is informed about the error.

(SMS 3)

The control protocol is informed about the release of the connection.

(SMS 4)

LL indicates it is ready to accept another data unit.

(LL 2)

LL is informed that SMS is ready to handle the next data unit.

### CP Error received

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |           LL UNITDATA IND    |

|                    |                 |             (CP ERROR)       |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |   rl_error_ind  |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |        LL GETUNITDATA REQ    |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |                 |                 |            |

(SMS 1)

Control Protocol exspects a CP ACK message as response of its own previous CP DATA message. Instead a CP ERROR message receives. The timer T1CM is stopped.

(SMS 2)

The error cause is forwarded to the relay layer.

(MMI 1)

The user is informed about the error.

(SMS 3)

The release of the CP SMS connection is requested.

(LL 1)

LL is notified that SMS is ready to handle the next primitive.

### Timeout TC1M

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 *<timeout TC1M>   |            |

|                    |              (SMS 1)              |            |

|                    |                 |                 |            |

a) max retransmission not reached

|                    |                 |                 |            |

|                    |                 |          LL UNITDATA REQ     |

|                    |                 |              (CP DATA)       |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |                 |                 |            |

b) max retransmission reached

|                    |                 |                 |            |

|                    |   rl_error_ind  |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |                 |            |

(SMS 1)

The timer T1CM of the control protocol times-out.

(MM 1)

If this happens the first time, the CP DATA message is retransmitted.

(SMS 2)

The timeout occurs a second time. So the maximum of retransmissions is reached and the error is forwarded to the relay layer.

(MMI 1)

The user is informed about the error.

(SMS 3)

The SMS connection is not longer necessary. Relay layer requests the release of CP SMS connection.

### Timeout TR1M (state GSMS MO WAIT FOR RP ACK)

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    *<timeout TR1M>   |                 |            |

|                 (SMS 1)              |                 |            |

|                    |   cp_abort_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |          LL UNITDATA REQ     |

|                    |                 |      (CP ERROR, Cause #111)  |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |                 |            |

(SMS 1)

A SMS connection is established. Then the timer T1RM of the relay layer expires.

(SMS 2)

The control protocol is informed about the abort.

(MM 1)

Control Protocol sends a CP ERROR message with the cause #111 to the infrastructure.

(SMS 3)

The relay layer requests the release of the SMS connection.

## Active Phase

### Acknowledge of the Infrastructure

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |           LL UNITDATA IND    |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |   rl_data_ind   |                 |            |

|                    |     (RP_ACK)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |           LL UNITDATA REQ    |

|                    |                 |               (CP ACK)       |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |         LL UNITREADY IND     |

|                    |                 *<=============================*

|                    |              (SMS 3)              |            |

|                    |                 |                 |            |

|                    |                 |         LL GETUNITDATA REQ   |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 2)

|                    |                 |                 |            |

(SMS 1)

The SMS entity is in state GSMS MO WAIT FOR RP ACK and the response of the network receives. It is a CP DATA message containing a RP ACK message.

(SMS 2)

The RP ACK message is forwarded to the relay layer.

(MMI 1)

The user is informed about the positive end of procedure.

(MM 1)

After reception of the CP DATA message control protocol sends a CP ACK message as response to the infrastructure.

(SMS 2)

Relay Layer releases the SMS CP connection.

(SMS 3)

LL informs the SMS entity that it is ready to handle another unitdata request.

(LL 2)

SMS informs LL that it is able to accept another unitdata indication.

### Error signalled by the Infrastructure

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |           LL UNITDATA IND    |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |                 |           LL UNITDATA REQ    |

|                    |                 |             (CP ACK)         |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |   rl_data_ind   |                 |            |

|                    |    (RP ERROR)   |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |        LL UNITREADY IND      |

|                    |                 *<=============================*

|                    |              (SMS 4)              |            |

|                    |                 |       LL GETUNITDATA REQ     |

|                    |                 *=============================>*

|                    |                 |                           (LL 2)

|                    |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

A SMS connection is established and the response of the network receives. It is a CP DATA message containing a RP ERROR message.

(LL 1)

The CP DATA message containing the RP ERROR message is acknowledged by the CP layer.

(SMS 2)

The RP ERROR message is forwarded to the relay layer.

(MMI 1)

The user is informed about the negative end of procedure.

(SMS 3)

Relay Layer releases the SMS connection.

(SMS 4)

The SMS CP layer receives the notification that LL is ready to receive the next data request.

(LL 2)

Since now the SMS entity is also ready to receive the next data indication by LL, positive flow control is signalled to LL.

### Wrong Message signalled by the Infrastructure (CP layer)

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |         LL UNITDATA IND      |

|                    |                 |        (CP ACK or unknown)   |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |   rl_error_ind  |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |          LL UNITDATA REQ     |

|                    |                 |              (CP ERROR)      |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |          LL UNITREADY IND    |

|                    |                 *<=============================*

|                    |              (SMS 4)                           |

|                    |                 |        LL GETUNITDATA REQ    |

|                    |                 *=============================>*

|                    |                 |                           (LL 2)

|                    |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

A SMS connection is established and the response of the network receives. It is a CP ACK or an unknown message.

(SMS 2)

The error is forwarded to the relay layer.

(MMI 1)

The user is informed about the negative end of procedure.

(LL 1)

Control protocol sends a CP ERROR message as response to the infrastructure. The used cause is #97 for an unknown message and #98 for a CP ACK message.

(SMS 3)

Relay Layer releases the SMS connection.

(SMS 4)

Positive flow control by LL.

(LL 2)

SMS entity is ready to receive next data indication and also issues positive flow control to LL.

### Wrong Message signalled by the Infrastructure (RL layer)

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |          LL UNITDATA IND     |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |   rl_data_ind   |                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |    (RP ERROR)   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |          LL UNITDATA REQ     |

|                    |                 |             (CP DATA)        |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 4)              |            |

|                    |                 |                 |            |

(SMS 1)

A response of the infrastructure is exspected. Control Protocol receives a CP DATA message.

(SMS 2)

This CP DATA message contains a RP DATA message instead of the exspected RP ACK message.

(SMS 3)

Relay Layer builds a RP ERROR message and forwards it to the control procotol.

(LL 1)

The RP ERROR message is included into a CP DATA message and send to the infrastructure.

(MMI 1)

The error is reported to the user.

(SMS 4)

Relay Layer requests the release of the SMS connection.

### Timeout TR1M

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    *<timeout TR1M>   |                 |            |

|                 (SMS 1)              |                 |            |

|                    |  cp_abort_req   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |          LL UNITDATA REQ     |

|                    |                 |          (CP ERROR, #111)    |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

SMS exspects a response of the infrastructure. In the relay layer the timer TR1M timesout.

(SMS 2)

The abort of the SMS connection is requested to the control protocol.

(LL 1)

A CP ERROR message with the cause #111 is send to the infrastructure.

(SMS 3)

The SMS connection is not longer necessary.

(MMI 1)

The user is informed about the error.

## Mobile Originated Short Message Command

MMI                 SMS(RL)           SMS(CP)            SIM        LL/GMM

|                    |                 |                 |            |

|  MNSMS COMMAND REQ |                 |                 |            |

*===================>*                 |                 |            |

|                 (SMS 1)              |                 |            |

|                    |             SIM UPDATE REQ        |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    | cp_establish_req|                 |            |

|                    |     (RP DATA)   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |     GMMSMS REGSTATE REQ      |

|                    |                 *=============================>*

|                    |                 |                          (GMM 1)

|                    |                 |     GMMSMS REGSTATE RES      |

|                    |                 *<============================ *

|                    |              (SMS 3)                           |

|                    |                 |         LL UNITDATA REQ      |

|                    |                 |             (CP DATA)        |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |                 |         LL UNITREADY IND     |

|                    |                 *<=============================*

|                    |              (SMS 4)              |            |

|                    |                 |         LL UNITDATA IND      |

|                    |                 |              (CP ACK)        |

|                    |                 *<=============================*

|                    |              (SMS 5)              |            |

|                    |                 |       LL GETUNITDATA REQ     |

|                    |                 *=============================>*

|                    |                 |                           (LL 2)

|                    |                 |        LL UNITDATA IND       |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |              (SMS 6)              |            |

|                    |                 |        LL UNITDATA REQ       |

|                    |                 |             (CP ACK)         |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 3)

|                    |   rl_data_ind   |                 |            |

|                    |     (RP ACK)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 7)              |                 |            |

| MNSMS REPORT IND   |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 8)              |            |

|                    |                 |          LL UNITREADY IND    |

|                    |                 *<=============================*

|                    |              (SMS 9)                           |

|                    |                 |        LL GETUNITDATA REQ    |

|                    |                 *=============================>*

|                    |                 |                           (LL 4)

|                    |                 |                 |            |

(SMS 1)

The mobile originated short message command procedure is used to send commands to the service center for previous sended short message. The procedure differs from a mobile originated short message service procedure only in the different initial primitive.

The mobile station starts sending of a short message command. The following commands are available:

Status request for a short message

Delete of status report request for a short message and

Delete of a short message.

The named short message is identified by the message reference used for the short message

(SIM 1)

Each short message command has its own message reference. This number is used if other short message commands are related to this short message command. The message reference is incremented by one. If a phase 2 SIM is available the message reference is stored on the SIM card.

(SMS 2)

The relay layer builds a RP DATA message containing the short message command. The message is forwarded to the control protocol. The timer TR1M is started to supervise the response of the infrastructure.

(GMM 1)

Control Protocol requests the GPRS registration status.

(SMS 3)

GMM confirms positive registration status of GPRS.

(LL 1)

The CP DATA message containing the RP DATA message of the relay layer is send to the infrastructure. The timer T1CM is started to supervise response of the infrastructure.

(SMS 4)

Positive flow control by LL.

(SMS 5)

Control Protocol receives the response of the infrastructure. It is a CP ACK message. It now waits for the response of the peer's relay layer.

(LL 2)

SMS entity is ready to process the next unitdata indication.

(SMS 6)

A CP DATA message is received. The content for the relay layer is decoded.

(LL 3)

The SMS CP layer acknowledges the CP DATA message.

(SMS 7)

The content of the message is a RP ACK message which is indicated to SMS RP layer.

(MMI 1)

The user is informed about the positive end of procedure.

(SMS 8)

The connection release is requested.

(SMS 9)

Positive flow control by LL.

(LL 4)

Transaction is completed and SMS entity is ready to process next unitdata indication. Therefore a getunitdata request is sent to LL.

# Mobile Terminated Short Message Service

With the mobile terminated short message service it is possible to send a textual message from the infrastructure to the mobile station. This message contains the data coding scheme describing the message class.

Additionally it is possible to configure the preferred memory (SIM or mobile equipment (ME) memory) and the used mode (MT = 0 .. 3) according the AT command +CNMI. If nothing is configured SMS preferres SIM card memory and MT is set to 4.

The following tables shows the handling in the relay layer of short message service with the following meanings:

- DISPLAY	forward directly to MMI

- MEM_ME	store in ME memory

- MEM_SIM	store in SIM memory

- NOTHING	do nothing

SMS differs between the following data coding schemes:

- NO CLASS

- CLASS 0

- CLASS 1

- CLASS 2

- CLASS 3

- DISCARD		Mail-Waiting Indicator, discard message

- STORE		Mail-Waiting Indicator, store message

The first action if the preferred memory is ME memory is carried out according the following table:

| MT | NO CLASS | CLASS 0 | CLASS 1 | CLASS 2 | CLASS 3 | DISCARD | STORE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MEM_ME | DISPLAY | MEM_ME | MEM_SM | MEM_ME | DISPLAY | MEM_ME |
| 1 | MEM_ME | DISPLAY | MEM_ME | MEM_SM | MEM_ME | DISPLAY | MEM_ME |
| 2 | DISPLAY | DISPLAY | DISPLAY | MEM_SM | DISPLAY | DISPLAY | MEM_ME |
| 3 | MEM_ME | DISPLAY | MEM_ME | MEM_SM | DISPLAY | DISPLAY | MEM_ME |
| 4 (default) | MEM_SM | DISPLAY | MEM_ME | MEM_SM | MEM_SM | DISPLAY | MEM_SM |

The second action if the preferred memory is ME memory is carried out according the following table:

| MT | NO CLASS | CLASS 0 | CLASS 1 | CLASS 2 | CLASS 3 | DISCARD | STORE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | NOTHING | NOTHING | MEM_SM | MEM_ME | MEM_SM | NOTHING | NOTHING |
| 1 | NOTHING | NOTHING | MEM_SM | MEM_ME | MEM_SM | NOTHING | NOTHING |
| 2 | NOTHING | NOTHING | NOTHING | MEM_ME | NOTHING | NOTHING | NOTHING |
| 3 | NOTHING | NOTHING | MEM_SM | MEM_ME | NOTHING | NOTHING | NOTHING |
| 4 (default) | NOTHING | NOTHING | MEM_SM | MEM_ME | MEM_ME | NOTHING | NOTHING |

The first action if the preferred memory is SIM memory is carried out according the following table:

| MT | NO CLASS | CLASS 0 | CLASS 1 | CLASS 2 | CLASS 3 | DISCARD | STORE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MEM_SM | DISPLAY | MEM_SM | MEM_SM | MEM_SM | DISPLAY | MEM_SM |
| 1 | MEM_SM | DISPLAY | MEM_SM | MEM_SM | MEM_SM | DISPLAY | MEM_SM |
| 2 | DISPLAY | DISPLAY | DISPLAY | MEM_SM | DISPLAY | DISPLAY | MEM_SM |
| 3 | MEM_SM | DISPLAY | MEM_SM | MEM_SM | DISPLAY | DISPLAY | MEM_SM |
| 4 (default) | MEM_SM | DISPLAY | MEM_ME | MEM_SM | MEM_SM | DISPLAY | MEM_SM |

The second action if the preferred memory is SIM memory is carried out according the following table:

| MT | NO CLASS | CLASS 0 | CLASS 1 | CLASS 2 | CLASS 3 | DISCARD | STORE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | NOTHING | NOTHING | MEM_ME | MEM_ME | MEM_ME | NOTHING | NOTHING |
| 1 | NOTHING | NOTHING | MEM_ME | MEM_ME | MEM_ME | NOTHING | NOTHING |
| 2 | NOTHING | NOTHING | NOTHING | MEM_ME | NOTHING | NOTHING | NOTHING |
| 3 | NOTHING | NOTHING | MEM_ME | MEM_ME | NOTHING | NOTHING | NOTHING |
| 4 (default) | NOTHING | NOTHING | MEM_SM | MEM_ME | MEM_ME | NOTHING | NOTHING |

## Reception of a Short Message

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|MNSMS CONFIGURE REQ |                 |                 |            |

*===================>*                 |                 |            |

|                 (SMS 1)              |                 |            |

|                    |                 |        LL UNITREADY IND      |

|                    |                 *<=============================*

|                    |               (SMS 2)             |            |

|                    |                 |      LL GETUNITDATA REQ      |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |                 |          LL UNITDATA IND     |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |               (SMS 3)             |            |

|                    | rl_establish_ind|                 |            |

|                    |     (RP_DATA)   |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 4)              |                 |            |

|                    |                 |         LL UNITDATA REQ      |

|                    |                 |             (CP ACK)         |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 2)

|                    |                 |                 |            |

(SMS 1)

MMI configures the reception modes for mobile terminated short messages.

(SMS 2)

Availability of LL is indicated. (see section 6.1)

(LL 1)

SMS indicates that it is ready to receive a data unit.

(SMS 3)

A CP DATA message is received.

(SMS 4)

The content of the CP DATA message is a RP DATA message which is forwarded to the Relay Layer.

(LL 2)

Control Procotol sends a CP ACK message as acknowlegement to the infrastructure.

(SMS 5)

The following handling of the mobile terminated short message depends on the protocol identifier and the data coding scheme of the message and of the configured modi by MMI.

## Reception of CP ERROR message

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |          LL UNITDATA IND     |

|                    |                 |             (CP ERROR)       |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |                 |        LL GETUNITDATA REQ    |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |                 |                 |            |

(SMS 1)

LL indicates a data unit. The incoming message is a CP ERROR message. The message is ignored.

(LL 1)

SMS is ready to process the next data unit by LL.

## Reception of CP ACK or unknown message

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |       LL UNITDATA IND        |

|                    |                 |     (CP ACK or unknown)      |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |                 |       LL UNITDATA REQ        |

|                    |                 |          (CP ERROR)          |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |                 |       LL UNITREADY IND       |

|                    |                 *<=============================*

|                    |              (SMS 2)                           |

|                    |                 |       LL GETUNITDATA REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 2)

|                    |                 |                 |            |

(SMS 1)

LL indicates a data unit. The incoming message is CP ACK or unknown.

(LL 1)

Control Protocol builds a CP ERROR message with cause #97 for an unknown message and #98 for a CP ACK message.

(SMS 2)

LL indicates availability.

(LL 2)

SMS is ready to process the next data unit.

## Second Mobile Terminated Connection

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |          LL UNITDATA IND     |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |    (RP ERROR)   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |          LL UNITDATA REQ     |

|                    |                 |             (CP DATA)        |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 4)              |            |

|                    |                 |                 |            |

(SMS 1)

LL indicates a second mobile terminated short message connection with a CP DATA message.

(SMS 2)

The content of the CP DATA message is forwarded to the Relay Layer.

(SMS 3)

Only one terminated transaction is allowed in parallel. So a RP ERROR message is build by the Relay Layer and forwarded to the Control Protocol.

(LL 1)

The message is included into a CP DATA message and sent to the infrastructure.

(SMS 4)

The SMS connection is released by the Relay Layer.

## Additional Message for  Mobile Terminated Connection

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |          LL UNITDATA IND     |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |   rl_data_ind   |                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |    (RP ERROR)   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |          LL UNITDATA REQ     |

|                    |                 |             (CP DATA)        |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    |                 |                 |            |

(SMS 1)

Control Protocol receives additional CP DATA messages for an existing transaction.

(SMS 2)

The content of this CP DATA message is a RP DATA message. It is forwarded to the Relay Layer.

(SMS 3)

The relay layer build a RP ERROR message and sends it back to the Control Protocol.

(LL 1)

This message is part of a CP DATA message which is forwarded to the infrastructure.

## Display Message

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 *...              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 1)              |                 |            |

|    MNSMS MT IND    |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |     (RP ACK)    |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |        LL UNITDATA REQ       |

|                    |                 |           (CP DATA)          |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message is received by the Relay Layer. Depending on the protocol identifier, the data coding scheme and the configured modi SMS decides that the message shall be displayed immediately and not stored.

(MMI 1)

The message is displayed to the user.

(SMS 2)

Relay Layer builds a RP ACK message and sent it to Control Protocol.

(LL 1)

The message is part of a CP DATA message which is build by Control Protocol. The message is sent to the infrastructure.

(SMS 3)

The SMS connection is released by the Relay Layer.

## Store Message in ME memory

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 *...              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 1)              |                 |            |

|                    |                 |                 |            |

a) storage capacity available in mobile station memory

|                    |                 |                 |            |

|   MNSMS ALERT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |     (RP ACK)    |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |      LL UNITDATA REQ         |

|                    |                 |        (CP DATA)             |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |                 |            |

b) no storage capacity available in mobile station memory

|                    |                 |                 |            |

|                    *...              |                 |            |

|                 (SMS 4)              |                 |            |

|                    |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message is received by the Relay Layer. Depending on the protocol identifier, the data coding scheme and the configured modi SMS decides that the message shall be stored in the ME memory.

(MMI 1)

The message is stored in the memory of the mobile station. An indication is sent to the user. The message can be read by the user with the SMS read procedure.

(SMS 2)

Relay Layer codes a RP ACK message and forwards it to the Control Protocol.

(LL 1)

The RP ACK message is part of the CP DATA message which is sent to the infrastructure.

(SMS 3)

The existing SMS-Connection is not longer necessary. The release is requested by the Relay Layer.

(SMS 4)

If no free entry is available in the mobile station memory, the relay layer starts the second action defined for this type of message or sends an error indication to the network as described in the next chapter.

### No Storage available on ME memory, no other opportunities

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |    (RP ERROR)   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 1)              |            |

|                    |                 |        LL UNITDATA REQ       |

|                    |                 |           (CP DATA)          |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |                 |            |

(SMS 1)

The error, that no memory is available is coded in a RP ERROR message and forwared to the CP layer.

(LL 1)

The RP ERROR message is filled in a CP DATA message and forwarded to the infrastructure.

(SMS 2)

The SMS connection will be released by RL layer.

## Store Message in SIM memory

### Storage on SIM available, writing successfull

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 *...              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 1)              |                 |            |

|                    |       SIM UPDATE RECORD REQ       |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |       SIM UPDATE RECORD CNF       |            |

|                    *<==================================*            |

|                 (SMS 2)              |                 |            |

|   MNSMS ALERT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |     (RP ACK)    |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |       LL UNITDATA REQ        |

|                    |                 |          (CP DATA)           |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 4)              |            |

|                    |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message is received by the Relay Layer. Depending on the protocol identifier, the data coding scheme and the configured modi SMS decides that the message shall be stored in the SIM memory. Using the backup of SMS status bytes from the SIM card a free record is found.

(SIM 1)

The SMS message is written to this record.

(SMS 2)

SMS receives the confirmation of writing the SMS message to the SIM memory.

(MMI 1)

An indication is sent to the user. The message can be read by the user with the SMS read procedure.

(SMS 3)

Relay Layer codes a RP ACK message and forwards it to the Control Protocol.

(LL 1)

The RP ACK message is part of the CP DATA message which is sent to the infrastructure.

(SMS 4)

The existing SMS-Connection is not longer necessary. The release is requested by the Relay Layer.

### Storage on SIM available, writing not successfull

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 *...              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 1)              |                 |            |

|                    |       SIM UPDATE RECORD REQ       |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |       SIM UPDATE RECORD CNF       |            |

|                    *<==================================*            |

|                 (SMS 2)              |                 |            |

|                    |          SIM UPDATE REQ           |            |

|                    *==================================>*            |

|                    |                 |              (SIM 2)         |

|                    |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message is received by the Relay Layer. Depending on the protocol identifier, the data coding scheme and the configured modi SMS decides that the message shall be stored in the SIM memory. Using the backup of SMS status bytes from the SIM card a free record is found.

(SIM 1)

The SMS message is written to this record.

(SMS 2)

SMS indicates a write error during updating the record.

(SIM 2)

A memory capacity exceeded message will be send to the infrastructure to indicate that the SIM memory is not accessible. Therefore the message number in this mobile originated short message attempt is incremented.

### No Storage on SIM available

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 *...              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 1)              |                 |            |

|                    |          SIM UPDATE REQ           |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message is received by the Relay Layer. Depending on the protocol identifier, the data coding scheme and the configured modi SMS decides that the message shall be stored in the SIM memory. Using the backup of SMS status bytes from the SIM card SMS finds no free entry.

(SIM 1)

A memory capacity exceeded message will be sent to the infrastructure to indicate that the SIM memory is not accessible. Therefore the message number in this mobile originated short message attempt is incremented.

### No Storage available on SIM memory, no other opportunities

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |    (RP ERROR)   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 1)              |            |

|                    |                 |         LL UNITDATA REQ      |

|                    |                 |            (CP DATA)         |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |                 |            |

(SMS 1)

The error, that no memory is available is coded in a RP ERROR message and forwarded to the CP layer.

(LL 1)

The RP ERROR message is filled in a CP DATA message and forwarded to the infrastructure.

(SMS 2)

The SMS connection will be released by the Relay Layer.

## Replace Short Message

### Replace in Mobile Station Memory

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |         LL UNITDATA IND      |

|                    |                 |            (CP DATA)         |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|                    *<replace message in memory>        |            |

|                 (SMS 3)              |                 |            |

|                    *<replace message on SIM card>      |            |

|                 (SMS 4)              |                 |            |

|                    |                 |                 |            |

(SMS 1)

LL indicates the reception of a mobile terminated short message.

(SMS 2)

The protocol identifier of the incoming short message indicates that it is a replace short message. That means it shall replace a short message which is already stored in the mobile station.

(SMS 3)

Using the ME backup of status and protocol identifier of all SMS messages in the ME memory a candidate is selected by SMS for replacing. This SMS message must be mobile terminated and must have the same protocol identifier. If such a SMS message is available, SMS reads the whole message from the permanent configuration memory and compares originator address and service center address. If all matches the new message is stored in the mobile memory replacing the old one.

(SMS 4)

If replacing in the mobile memory has failed, the same is carried out for the SIM card memory. A pre-selection of candidates is carried out using the stored status and protocol identifier of the SMS messages stored on the SIM card. Then a possible record is read from the SIM card and the originator address and the service center address are checked. If these parameters are also matching with the received message the SMS record on the SIM card is replaced.

If both checks have failed, normal storing of the incoming message is carried out.

## Mobile Terminated Short Message Status

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 |         LL UNITDATA IND      |

|                    |                 |            (CP DATA)         |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |                 |         LL UNITDATA REQ      |

|                    |                 |            (CP ACK)          |

|                    |                 *=============================>*

|                    |                 |                           (LL 1)

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS STATUS IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |     (RP ACK)    |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 4)              |            |

|                    |                 |         LL UNITREADY IND     |

|                    |                 *<=============================*

|                    |              (SMS 5)                           |

|                    |                 |         LL UNITDATA REQ      |

|                    |                 |            (CP DATA)         |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 2)

|                    |                 |                 |            |

(SMS 1)

LL indicates the reception of a mobile terminated short message.

(LL 1)

The CP DATA message is acknowledged.

(SMS 2)

The received CP DATA message is a SMS STATUS message and indicated to the Relay Layer.

(MMI 1)

The content of the SMS STATUS message is forwarded to MMI.

(SMS 3)

The reception of the RP DATA message is acknowledged by the RP ACK message. Since until now no positive flow control was received, this RP DATA message cannot be forwarded to LL now. It is temporarily stored.

(SMS 4)

The SMS connection is released by RL layer.

(SMS 5)

Availability is indicated by LL.

(LL 2)

A CP DATA message is sent by the Control Layer which contains the temporarily stored RP DATA message.

## SIM Toolkit Data Download

MMI                 SMS(RL)           SMS(CP)            SIM          LL

|                    |                 |                 |            |

|                    |                 *...              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 1)              |                 |            |

|                    |           SIM_TOOLKIT_REQ         |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |           SIM_TOOLKIT_CNF         |            |

|                    *==================================>*            |

|                 (SMS 2)              |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |     (RP ACK)    |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |       LL UNITDATA REQ        |

|                    |                 |          (CP DATA)           |

|                    |                 *=============================>*

|                    |                 |                 |         (LL 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 4)              |            |

|                    |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message is received in the Relay Layer. The protocol identifier indicates SIM Data Download.

(SIM 1)

If the sms download flag indicates support of this feature, the message content is forwarded to the SIM application.

(SMS 2)

The SIM application response is received.

(SMS 3)

Relay Layer builds a RP ACK message which may contain response data from the SIM application and sent it to Control Protocol.

(LL 1)

The message is part of a CP DATA message which is build by Control Protocol. The message is sent to the infrastructure.

(SMS 4)

The SMS connection will be released by the Relay Layer.

## SMS - Depersonalisation

MMI                  SMS(RL)           SMS(CP)            SIM          LL

|                     |                 |                 |            |

|                     |                 *...              |            |

|                     | rl_establish_ind|                 |            |

|                     |    (RP DATA)    |                 |            |

|                     *<----------------*                 |            |

|                  (SMS 1)              |                 |            |

|    MNSMS_MT_IND     |                 |                 |            |

*<====================*                 |                 |            |

(MMI 1)                 |                 |                 |            |

|MNSMS_DELIVER_REP_REQ|                 |                 |            |

*====================>*                 |                 |            |

|                  (SMS 2)              |                 |            |

|                     |   cp_data_req   |                 |            |

|                     |     (RP ACK)    |                 |            |

|                     *---------------->*                 |            |

|                     |              (SMS 3)              |            |

|                     |                 |         LL UNITDATA REQ      |

|                     |                 |            (CP DATA)         |

|                     |                 *=============================>*

|                     |                 |                 |        (LL 1)

|                     | cp_release_req  |                 |            |

|                     *---------------->*                 |            |

|                     |              (SMS 4)              |            |

|                     |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message is received in the Relay Layer. The protocol identifier indicates SMS De-personalisation.

(MMI 1)

The SMS message is forwarded to MMI.

(SMS 2)

MMI sends the response to SMS.

(SMS 3)

Relay Layer builds a RP ACK message containing the the DELIVER REPORT from MMI and sent it to Control Protocol.

(LL 1)

The message is part of a CP DATA message which is build by Control Protocol. The message is sent to the infrastructure.

(SMS 4)

The SMS connection will be released by the Relay Layer.

(LL 2)

The release of SMS connection is forwarded to MM.

# Short Message Service Procedures

Since the handling of short message service procedures does not depend on the kind of underlying message delivery service (GPRS or normal circuit switched) these procedures are the same as those described in the message charts for normal SMS. In order to avoid problems with updating redundant information, the appropriate section will not be repeated here - please refer to the SMS message sequence charts, document no. 6147.202.98.101, section 7, instead.

# Data Formats

Since the short message data format does not depend on the kind of underlying message delivery service (GPRS or normal circuit switched) these procedures are the same as those described in the message charts for normal SMS. In order to avoid problems with updating redundant information, the appropriate section will not be repeated here - please refer to the SMS message sequence charts, document no. 6147.202.98.101, section 8, instead.

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |