Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document - Confidential

GSM General Packet Radio Services

Test Specification

GACI

(Edit via File/Properties/Summary!)

| Document Number: | 8441.407.00.002 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.3 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2000-Jun-20 |
| Last changed: | 2003-May-14 by XGUTTEFE |
| File Name: | gaci.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2000-Jun-20 | BRZ |  | 0.1 |  | 1 |
| 2003-Jan-27 | RM |  | 0.2 |  | 2 |
| 2003-May-14 | XGUTTEFE |  | 0.3 | Draft |  |

Notes:

- Initial version

- add testcases CGPCO an PKTIO

Table of Contents

1.1	References	7

1.2	Abbreviations	9

1.3	Terms	12

2	Overview	12

2.1	GRR (RLC/MAC) – Radio Link Control/Medium Access Control	13

2.2	LLC – Logical Link Control	13

2.3	GMM – GPRS Mobility Management	13

2.4	SM – Session Management	13

2.5	SNDCP - Subnetwork Dependant Convergence Protocol	13

2.6	GACI – GPRS Application Control Interface	13

2.7	USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	13

2.8	TOM – Tunnelling of Messages	13

3	Parameters	14

3.1	Declarations	14

3.1.1	Message Struct Declarations	14

3.2	COMMAND SECTION	15

3.2.1	GSM commands	15

3.2.2	GPRS commands	16

3.2.3	Percent GPRS commands	19

3.2.4	Modem compatibility mode commands	20

3.3	MESSAGE SECTION	21

3.3.1	General messages	21

3.3.2	AT command answer messages	21

3.3.3	Values	26

3.3.4	Fields	28

3.3.5	PLMN list (+COPS=?)	28

3.3.6	Arrays	32

3.4	Primitive Structs	33

4	TEST CASES	36

4.1	Routing (internal)	36

4.1.1	GACI000: Setup the Routing for the GACI test	36

4.1.2	GACI001: Set ME to full functionality state, no PIN required	37

4.1.3	GACI002: Automatic registration (GPRS automatic attach)	39

4.1.4	GACI003: Automatic registration (GPRS manual attach)	41

4.1.5	GACI004: Set ME to minimum functionality state	42

4.1.6	GACI005: change the default mobile class	43

4.1.7	GACI006: Automatic registration (GPRS automatic attach)	44

4.1.8	GACI007: registration failed	45

4.1.9	GACI008: Automatic registration (GPRS automatic attach)	46

4.1.10	GACI009: Automatic registration, but GPRS rejected	48

4.1.11	GACI010: Automatic registration, but GSM rejected	49

4.1.12	GACI011: Automatic registration, but combined rejected	50

4.1.13	GACI012: GPRS only mode: registration failed	52

4.1.14	GACI013: UART Init	53

4.1.15	GACI014: Add second test source	54

4.1.16	GACI015: Set ME to full functionality state, PIN required	55

4.1.17	GACI016: Enter a wrong PIN	56

4.1.18	GACI017: Enter a good PIN and set automatic registration	57

4.2	Root functions	59

4.2.1	GACI020: Attach mobile after it already is attached	59

4.2.2	GACI021: detach mobile	60

4.2.3	GACI022: change the mobile class	60

4.2.4	GACI023: Command change mobile class but no changes possihle	62

4.2.5	GACI024: change the mobile class	63

4.2.6	GACI025: change the default mobile class	64

4.2.7	GACI027: CGAATT	64

4.2.8	GACI030: Attach mobile	65

4.2.9	GACI031: Attach mobile	66

4.2.10	GACI035: Attach status and test commands	67

4.2.11	GACI036: Class status and test commands	68

4.2.12	GACI037: check attach state	69

4.3	Communication with GMM over GMMREG	71

4.3.1	GACI040: Detach indication for GPRS from the network	71

4.3.2	GACI041: Detach indication for GPRS from the network	72

4.4	PDP context definition	72

4.4.1	GACI060: One PDP context defined	72

4.4.2	GACI061: Too many PDP context definitions	73

4.4.3	GACI062: Clear defined PDP context	74

4.4.4	GACI063: One PDP context define	74

4.4.5	GACI075: Context status and test commands	75

4.4.6	GACI076: Context status and test commands	77

4.4.7	GACI077: Context status and test commands	78

4.5	PDP context modification	79

4.5.1	GACI080: Set quality of service profile (requested)	79

4.5.2	GACI081: Set quality of service profile (minimum acceptable)	79

4.5.3	GACI082: Set quality of service profile to undefined	80

4.5.4	GACI083: Set default of quality of service profile	81

4.5.5	GACI095: Quality of service status and test commands	82

GACI096: Quality of service status commands	83

4.5.6	GACI097: Quality of service status and test commands	84

4.6	PDP context activation	85

4.6.1	GACI100: One PDP context activated	85

4.6.2	GACI101: One failed PDP context activated by PPP	91

4.6.3	GACI102: One failed PDP context activated by SM	94

4.6.4	GACI103: One failed PDP context activated by PPP	97

4.6.5	GACI104: One failed PDP context activated by PPP	101

4.6.6	GACI105: One failed PDP context activated by PPP	105

4.6.7	GACI106: One failed PDP context activated by PPP	108

4.6.8	GACI107: One failed PDP context activated by PPP and SM	112

4.6.9	GACI108: One failed PDP context activated by SM	116

4.6.10	GACI109: One failed PDP context activated by SM	120

4.6.11	GACI110: One PDP context deactivation by PPP	124

4.6.12	GACI111: One PDP context deactivation by PPP and SM	125

4.6.13	GACI112: One PDP context deactivation by SM	127

4.6.14	GACI113: Start a PDP context without specified use	128

4.6.15	GACI114: PDP context deactivation by CGACT=0,…	129

4.6.16	GACI115: PDP context deactivation by CGACT=0,…	132

4.6.17	GACI117: One PDP context activated	134

4.6.18	GACI119: Context activation status and test commands	137

4.7	Automatic detach after last context deactivation	138

4.7.1	GACI130: Detach after one PDP context deactivation by SM	138

4.7.2	GACI131: Detach after one PDP context deactivation by PPP	140

4.7.3	GACI132: Automatic detach after failed PDP context activated by PPP	142

4.7.4	GACI133: Automatic detach after failed PDP context activated by SM	145

4.7.5	GACI134: Automatic detach after aborting +CGDATA during GPRS attachment	148

4.7.6	GACI135: Automatic detach after failed PDP context activated by SM	150

4.8	network requested context activation	153

4.8.1	GACI140: Set CGAUTO mode	153

4.8.2	GACI141: modem compatibility mode, GPRS and circuit switched calls (default)	154

4.8.3	GACI142: modem compatibility mode, GPRS and circuit switched calls (default)	155

4.8.4	GACI143: Automatic acceptance of a network request for PDP context activation __FAILED!!!	160

4.8.5	GACI144: set CGAUTO mode = 1 and attach	163

4.8.6	GACI145: Manuel acceptance of a network request for PDP context activation	164

4.8.7	GACI146: context reactivation	165

4.8.8	GACI147: context reactivation	170

4.8.9	GACI148: context reactivation aborted	174

4.8.10	GACI149: context reactivation aborted	176

4.8.11	GACI150: context reactivation	178

4.8.12	GACI155: CGAUTO & CGANS status and test commands	182

4.9	Other GPRS commands	183

4.9.1	GACI175: Select servive for mobile originated SMS massages	183

4.9.2	GACI176: SMS servive status and test commands	184

4.10	Unsolicited reporting	186

4.10.1	GACI180: Control the sending of unsolicited GPRS event reporting	186

4.10.2	GACI181: Control the sending and presentation of unsolicited GPRS network registration status	186

4.11	Root functions 2	187

4.11.1	GACI200: Setting the Error Response	187

4.11.2	GACI220: Setting the GPRS network registration status	188

0.1	More register test cases	188

4.11.3	GACI230: Manuel registration (GPRS automatic attach)	188

4.11.4	GACI231: Manuel registration (GPRS manuel attach)	190

4.11.5	GACI232: Net Search (failed: mistake in testcase)	192

4.11.6	GACI240: One PDP context activated	193

0.2	Network registration state reporting (+CGREG, %CGREG)	197

0.2.1	GACI250: Defining registration state reporting level (AT+CGREG=0,1 or 2; AT%CGREG=0,1 or 2)	197

0.2.2	GACI251: Attaching GPRS including CGREG reporting	199

0.2.3	GACI252: switching mobile off and back on, CGREG should keep ist state!	200

0.2.4	GACI253: Suspend and Resume	204

4.12	Ciphering Indicator Test Cases	205

4.12.1	GACI260: CI is disabled, test set command	205

4.12.2	GACI261: CI is disabled, test query command	206

4.12.3	GACI262: CI is disabled, test test command	206

4.12.4	GACI263: CI is enabled, test set and test command	207

4.12.5	GACI264: CI is enabled, test query command and ciphering indication	208

0.3	additional test cases for context activation and deactivation	209

4.12.6	GACI400: One PDP context activated	209

4.12.7	GACI405: Start a PDP context without specified use	212

4.12.8	GACI406: PDP context deactivation by CGACT=0,…	214

4.12.9	GACI407: A failed PDP context activation	215

4.13	UART_DETECT_IND (GACI420-GACI449)	217

4.13.1	GACI420: One PDP context deactivation by PPP	217

4.13.2	GACI421: PDP context deactivation by PPP (2 UART_DETECTED_IND)	219

4.14	Engineering Mode (GACI450-GACI460)	221

4.14.1	GACI450:  Infrastructure Data - Serving Cell GPRS	221

4.15	Protocol Configuration Options (GACI500-501)	223

4.15.1	GACI500:Set PCO for PDP context activation	223

4.15.2	GACI501:Query PCO for PDP context activation	223

4.16	Send packet data for protocol PKTIO (GACI600-)	225

4.16.1	GACI600: Entity PKTIO registers in ACI, successfullF	225

4.16.2	GACI601: Send AT%DATA to configure the data flow for PKTIO	225

4.16.3	GACI602: Send AT+CGDATA and open dti connection	226

4.16.4	GACI603:  Send AT+CGDATA to open dti connection, disconnection by PKTIO	229

4.16.5	GACI604:  Send AT+CGDATA to open dti connection, disconnection by network	230

4.16.6	GACI605:  Send AT+CGDACT and close dti connection	232

4.16.7	GACI606:  Send AT+CGDACT to close dti connection, disconnection by PKTIO	233

4.16.8	GACI607: Disconnection of dti connection caused by DIO driver	234

4.17	%CGCLASS and Network mode III (GACI700-GACI721)	235

4.17.1	GACI700: change the mobile class over %CGCLASS	235

4.17.2	GACI701: change the mobile class over %CGCLASS (NMO III, attach_cnf)	236

4.17.3	GACI702: change the mobile class over %CGCLASS (NMO III, attach_rej)	238

4.17.4	GACI709: Class status commands	239

4.17.5	GACI710: One PDP context activated	241

4.17.6	GACI715: One PDP context deactivation by PPP	244

4.17.7	GACI720: Start a PDP context without specified use	246

4.17.8	GACI721: Start a PDP context without specified use	248

4.18	Dual port test cases (GACI800-GACI830)	250

4.18.1	GACI800: UART Init (2) and install redirection	250

4.18.2	GACI801: Set ME to full functionality state, no PIN required	252

4.18.3	GACI802: Automatic registration (GPRS automatic attach)	253

4.18.4	GACI810: One PDP context activated	255

Appendices	258

A.	Acronyms	258

B.	Glossary	258

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 05.02 version 8.0.0 Release 1999
Digital cellular telecommunications system (Phase 2+);
Multiplexing and multiple access on the radio path

[2]	GSM 04.60 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station (MS) - Base Station System (BSS) interface;
Radio Link Control/ Medium Access Control (RLC/MAC) protocol

[3]	GSM 04.08 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface layer 3 specification

[4]	GSM 03.64 version 6.1.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Overall description of the GPRS radio interface; Stage 2

[5]	GSM 03.60 version 6.3.1 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Service description; Stage 2

[6]	GSM 04.07 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface signalling layer 3; General aspects

[7]	GSM 04.64 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station - Serving GPRS Support Node (MS-SGSN)
Logical Link Control (LLC) layer specification

[8]	GSM 05.08 version 6.4.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem link control

[9]	GSM 05.10 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem synchronization

[10]	GSM 03.20 TS 100 929: July 1998 (GSM 03.20 version 6.0.1)
Security related network functions, ETSI

[11]	Draft GSM 03.22: August 1998 (GSM 03.22 version 6.1.0)
Functions related to Mobile Station (MS) in idle mode and group receive mode, ETSI

[12]	GSM 04.65 V6.3.0: Subnetwork Dependant Convergence Protocol

ETSI, March 1999

[13]	ITU-T V42bis ITU-T, Recommendation V.42 bis 1990

[14]	GSM 09.60 GPRS Tunneling Protocol (GTP) across the Gn and Gp Interface

[15]	RFC 1661 IETF STD 51 July 1994
The Point-to-Point Protocol (PPP)

[16]	RFC 1662 IETF STD 51 July 1994
PPP in HDLC-like Framing

[17]	RFC 1570 January 1994
PPP LCP Extensions

[18]	RFC 1989 August 1996
PPP Link Quality Monitoring

[19]	RFC 1332 May 1992
The PPP Internet Protocol Control Protocol (IPCP)

[20]	RFC 1877 December 1995
PPP IPCP Extensions for Name Server Addresses

[21]	RFC 2153 May 1997
PPP Vendor Extensions

[22]	RFC 1334 October 1992
PPP Authentication Protocols (for Password Authentication Protocol only)

[23]	RFC 1994 August 1996
PPP Challenge Handshake Authentication Protocol (CHAP)

[24]	TIA/EIA-136-370
Packet-Data Services – Enhanced General Packet Radio for TIA/EIA-136 (EGPRS-136) - Overview, Telecommunications Industry Association

[25]	TIA/EIA-136-376
Packet-Data Services – EGPRS-136 Mobility Management, Telecommunications Industry Association

[26]	TIA/EIA-136-972
Packet-Data Services – Stage 2 Description, Telecommunications Industry Association

## Abbreviations

ACI	Application Control Interface
AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CCI	Compression and Ciphering Interface
CHAP	Challenge Handshake Authentication Protocol
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DCOMP	Identifier of the user data compression algorithm used for the N-DPU
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

E	Extension bit
EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GACI	GPRS Application Control Interface
GMM	GPRS Mobility Management
GP	Guard Period
GRR	GPRS RR
GSM	Global System for Mobile Communication

HDLC	High-level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
IP	Internet Protocol
IPCP	Internet Protocol Control Protocol 
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Ciphering Key

L	Length Indicator
LAI	Location Area Information
LCP 	Link Control Protocol 
LISR	Low level Interrupt Service Routine
LLC	Logical Link Control
LPD	Link Protocol Discriminator
LQM	Link Quality Monitoring

M	More bit used to indicate the last segment of N-DPU
MAC	Medium Access Control
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MT	Mobile Termination

N(R)	Receive Number
N(S)	Send Number
NC	Network Control
NCC	National Colour Code
NCP 	Network Control Protocol
NECI	New Establishment Causes included
N-PDU	Network Protocol Data Unit
NSAPI	Network Layer Service Access Point Identifier

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PACCH	Packet Associated Control Channel
PAP	Password Authentication Protocol
PBCCH	Packet BCCH
PCCCH	Packet CCCH
PCOMP	Identifier of the protocol control information compression algorithm used for the N-DPU
PDCH	Packet Data Channel
PDP	Packet Data Protocol e.g. IP or X.25
PDTCH	Packet Data Traffic Channel
PRACH	Packet RACH
PSI	Packet System Information
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Data Unit
PL	Physical Layer
PLMN	Public Land Mobile Network
PPC	Packet Physical Convergence
PPP 	Point-to-Point Protocol
PTP	Point to Point

QoS	Quality of Service

RACH	Random Access Channel
REJ	Reject Frame
RLC	Radio Link Control
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Stand alone Dedicated Control Channel
SDU	Service Data Unit
SGSN	Serving GPRS Support Node
SIM	Subscriber Identity Module
SM	Session Management
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SNDCP	Subnetwork Dependant Convergence Protocol
SNSM	SNDCP-SM
SS	Supplementary Services

TAP	Test Application Program
TBF	Temporary Block Flow
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TCP	Transmission Control Protocol
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TFI	Temporary Flow Identifier
TLLI	Temporary Logical Link Identifier
TMSI	Temporary Mobile Subscriber Identity
TOM	Tunnelling of Messages
TQI	Temporary Queuing Identifier

UA	Unnumbered Acknowledgement Frame
UART	Universal Asynchronous Receiver Transmitter
UI	Unnumbered Information Frame
USF	Uplink State Flag

V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable
VPLMN	Visited Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for GPRS consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity.

- Figure 2-1: Architecture of the GSM/GPRS protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the GPRS protocol stack are:

## GRR (RLC/MAC) – Radio Link Control/Medium Access Control

This layer contains two functions: The Radio Link Control function provides a radio-solution-dependent reliable link. The Medium Access Control function controls the access signalling (request and grant) procedures for the radio channel, and the mapping of LLC frames onto the GSM physical channel.

## LLC – Logical Link Control

The LLC entity provides multiple highly reliable logical links for asynchronous data transfer between the MS and the network. It supports variable-length information frames, acknowledged and unacknowledged data transfer, flow and sequence control, error detection and recovery, notification of unrecoverable errors, user identity confidentiality, and ciphering of user and signaling data.

## GMM – GPRS Mobility Management

The GMM entity provides procedures for the mobility of the MS, such as informing the network of its present location, and user identity confidentiality. It manages the GMM context (attach, detach, routing area updating), supports security functions such as authentication of user and MS, controls ciphering of data, and initiates the response to paging messages.

## SM – Session Management

The main function of the session management (SM) is to support PDP context handling of the user terminal. Session Management activates, modifies and deletes the contexts for packet data protocols (PDP). Session Management services are provided at the SMREG-SAP and the SNSM-SAP for anonymous and non-anonymous access. The non-anonymous and anonymous access procedures for PDP context activation and PDP context deactivation are available at the SMREG-SAP. In addition there exists a PDP context modification for non-anonymous PDP contexts.

## SNDCP - Subnetwork Dependant Convergence Protocol

- SNDCP carries out all functions related to transfer of Network layer Protocol Data Units (N-PDUs) over GPRS in a transparent way. SNDCP helps to improve channel efficiency by means of compression techniques. The set of protocol entities above SNDCP consists of commonly used network protocols. They all use the same SNDCP entity, which then performs multiplexing of data coming from different sources to be sent using the service provided by the LLC layer.

## GACI – GPRS Application Control Interface

The GACI is the GPRS extension of the ACI. It is specified in GSM 07.07 and 07.60. It is responsible for processing of the GPRS related AT Commands to setup, activate and deactivate the PDP context parameter. It also provides functionality for the interworking between GMM/SM/SNDCP and a packet oriented protocol like PPP.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the GPRS. The driver has to be able to perform flow control.

## TOM – Tunnelling of Messages

The TOM entity is present if and only if HS136 is supported (the feature flag FF_HS136 is enabled).

The main function of TOM is to tunnel non-GSM signalling messages between the MS and the SGSN. The only non-GSM signalling which is currently supported by TOM is for the EGPRS-136 system (according to TIA/EIA-136-376). Data transfer in both uplink and downlink direction is possible. Two different priorities (high, low) of signalling data transfer are supported. TOM uses the unacknowledged mode of LLC and the acknowledged mode of GRR (RLC/MAC).

# Parameters

/*

## Declarations

### Message Struct Declarations

*/

- DECLARATION (A_ECC_FIELD )

DECLARATION (A_AD_FIELD_CI_DISABLED )

DECLARATION (A_AD_FIELD_CI_ENABLED )

- DECLARATION(NO_EC_CODES)

- DECLARATION(NO_PREF_LANG)

- DECLARATION(IMSI)

- DECLARATION(IMSI_ARRAY)

- DECLARATION(SIM_TOOLKIT_PROFILE)

- DECLARATION(PLMN_1)

- DECLARATION(PLMN_262_02)

- DECLARATION(MCC_1)

- DECLARATION(MCC_262)

- DECLARATION(MNC_1)

- DECLARATION(MNC_02)

- DECLARATION(APN_BUF_1)

- DECLARATION(SMREG_APN_0_S)

- DECLARATION(SMREG_APN_1)

- DECLARATION(PDP_ADD_BUF_1)

- DECLARATION(PDP_ADD_BUF_2)

- DECLARATION(PDP_ADD_BUF_3)

- DECLARATION(PDP_ADDRESS_0_S)

- DECLARATION(PDP_ADDRESS_1)

- DECLARATION(PDP_ADDRESS_2)

- DECLARATION(PDP_ADDRESS_3)

- DECLARATION(SMREG_QOS_0)

- DECLARATION(SMREG_QOS_1)

- DECLARATION(SMREG_QOS_2)

- DECLARATION(SMREG_QOS_3)

- DECLARATION(SMREG_QOS_4)

- DECLARATION(SNDCP_NAME_BUF)

- DECLARATION(UART_NAME_BUF)

- DECLARATION(SNDCP_CHANNEL)

- DECLARATION(UART_CHANNEL)

- DECLARATION(SDU)

- DECLARATION(SDU_2)

- DECLARATION(SDU_3)

- DECLARATION(SDU_CTX_REQ_DEF)

- DECLARATION(SDU_NET_DNS_1)

- DECLARATION(SDU_NET_GATEW_2)

- DECLARATION(PREF_PLMN)

- DECLARATION(PREF_PLMN_ARRAY)

- DECLARATION(UNUSED_PDP_ADDRESS)

- DECLARATION(UNUSED_QOS)

- DECLARATION(UNUSED_APN)

- DECLARATION(PLMN_LST)

- DECLARATION(PLMN_LST_1)

- DECLARATION(PLMN_LST_2)

- DECLARATION(PLMN_LST_3)

- DECLARATION(PLMN_LST_4)

- DECLARATION(PLMN_LST_5)

- DECLARATION(S_NET_CTRL)

- DECLARATION(EMPTY_PLMN_LST)

- DECLARATION(MCC_PLMN_LST_1)

- DECLARATION(MCC_PLMN_LST_2)

- DECLARATION(MCC_PLMN_LST_3)

- DECLARATION(MCC_PLMN_LST_4)

- DECLARATION(MCC_PLMN_LST_5)

- DECLARATION(MNC_PLMN_LST_1)

- DECLARATION(MNC_PLMN_LST_2)

- DECLARATION(MNC_PLMN_LST_3)

- DECLARATION(MNC_PLMN_LST_4)

- DECLARATION(MNC_PLMN_LST_5)

- DECLARATION(FRB_PLMN_LST)

- DECLARATION(RXL_PLMN_LST)

- DECLARATION(GPRS_STATUS_LST)

- DECLARATION (DEFAULT_DCB_FROM_DIO_1)
DECLARATION (DEFAULT_DCB_FROM_DIO_2)
DECLARATION (DEFAULT_DCB_FROM_ACI_1)
DECLARATION (DEFAULT_DCB_FROM_ACI_2)
DECLARATION (DEFAULT_DCB_FROM_DIO_3)

/*

## COMMAND SECTION

### GSM commands

*/

/* command: CFUN */

STRING(C_CFUN_0, "AT+CFUN=0\r")

STRING(C_CFUN_1, "AT+CFUN=1\r")

/*STRING(C_CFUN_1, "AT+CFUN=1\r")*/

BYTE LC_CFUN 10

/* command:  CPIN*/

STRING(C_CPIN_WRONG, "AT+CPIN=\"1234\"\r")

STRING(C_CPIN_IO, "AT+CPIN=\"4321\"\r")

- BYTE LC_CPIN 15

/* command:  COPS*/

STRING(C_COPS_0, "AT+COPS=0\r")

STRING(C_COPS_1, "AT+COPS=1\r")

STRING(C_PLUS_COPS_MAN_NUM, "AT+COPS=1,2,\"26202\"" )

STRING(C_COPS_Q, "AT+COPS=?\r")

- BYTE LC_COPS 10

- BYTE LC_PLUS_COPS_MAN_NUM 19

/* command:  CMEE*/

STRING(C_CMEE_1, "AT+CMEE=1\r")

- BYTE LC_CMEE_1 10

STRING(C_AND_D2, "AT&D2\r")

BYTE LC_AND_D2 6

/*Command AT+CGDATA set up packet data connection*/

STRING(C_PLUS_CGDATA_PKT_DEV_1, "AT+CGDATA=\"M-PKT\",1")

/*

### GPRS commands

*/

/* command:  CGATT*/

STRING(C_CGATT_0, "AT+CGATT=0\r")

STRING(C_CGATT_1, "AT+CGATT=1\r")

STRING(C_CGATT_R, "AT+CGATT?\r")

STRING(C_CGATT_T, "AT+CGATT=?\r")

- BYTE LC_CGATT 11

- BYTE LC_CGATT_R 10

BYTE LC_CGATT_T 11

/* command:  +CGCLASS*/

STRING(C_CGCLASS_A, "AT+CGCLASS=\"A\"\r")

STRING(C_CGCLASS_B, "AT+CGCLASS=\"B\"\r")

STRING(C_CGCLASS_C, "AT+CGCLASS=\"C\"\r")

STRING(C_CGCLASS_CG, "AT+CGCLASS=\"CG\"\r")

STRING(C_CGCLASS_CC, "AT+CGCLASS=\"CC\"\r")

STRING(C_CGCLASS_R, "AT+CGCLASS?\r")

STRING(C_CGCLASS_T, "AT+CGCLASS=?\r")

- BYTE LC_CGCLASS_X 15

- BYTE LC_CGCLASS_XX 16

BYTE LC_CGCLASS_R 12

BYTE LC_CGCLASS_T 13

/* command: %CGCLASS */

STRING(C_PERCENT_CGCLASS_A, "AT%CGCLASS=\"A\"\r")

STRING(C_PERCENT_CGCLASS_B, "AT%CGCLASS=\"B\"\r")

STRING(C_PERCENT_CGCLASS_BX, "AT%CGCLASS=\"BX\"\r")

STRING(C_PERCENT_CGCLASS_BG, "AT%CGCLASS=\"BG\"\r")

STRING(C_PERCENT_CGCLASS_BC, "AT%CGCLASS=\"BC\"\r")

STRING(C_PERCENT_CGCLASS_C, "AT%CGCLASS=\"C\"\r")

STRING(C_PERCENT_CGCLASS_CG, "AT%CGCLASS=\"CG\"\r")

STRING(C_PERCENT_CGCLASS_CC, "AT%CGCLASS=\"CC\"\r")

STRING(C_PERCENT_CGCLASS_R, "AT%CGCLASS?\r")

STRING(C_PERCENT_CGCLASS_T, "AT%CGCLASS=?\r")

/* command:  CGDCONT*/

/*STRING(C_CGDCONT_1, "AT+CGDCONT=1,\"IP\",,\"255.255.255.255\",0,0\r")*/

STRING(C_CGDCONT_1, "AT+CGDCONT=1,\"IP\",\"APN\",\"255.255.255.255\",0,0\r")

STRING(C_CGDCONT_1_oIAP, "AT+CGDCONT=1,,,,0,0\r")

STRING(C_CGDCONT_2, "AT+CGDCONT=2,\"IP\",\"APN\",\"255.255.255.255\",1,1\r")

STRING(C_CGDCONT_3, "AT+CGDCONT=3,\"IP\",\"APN\",\"255.255.255.255\",1,1\r")

STRING(C_CGDCONT_4, "AT+CGDCONT=4,\"IP\",\"APN\",\"255.255.255.255\",0,0\r")

STRING(C_CGDCONT_5, "AT+CGDCONT=5,\"IP\",\"APN\",\"255.255.255.255\",1,1\r")

STRING(C_CGDCONT_6, "AT+CGDCONT=6,\"IP\",\"APN\",\"255.255.255.255\",1,1\r")

STRING(C_CGDCONT_7, "AT+CGDCONT=7,\"IP\",\"APN\",\"255.255.255.255\",0,0\r")

STRING(C_CGDCONT_8, "AT+CGDCONT=8,\"IP\",\"APN\",\"255.255.255.255\",1,1\r")

STRING(C_CGDCONT_9, "AT+CGDCONT=9,\"IP\",\"APN\",\"255.255.255.255\",1,1\r")

STRING(C_CGDCONT_10, "AT+CGDCONT=10,\"IP\",\"APN\",\"255.255.255.255\",0,0\r")

STRING(C_CGDCONT_11, "AT+CGDCONT=11,\"IP\",\"APN\",\"255.255.255.255\",0,0\r")

STRING(C_CGDCONT_12, "AT+CGDCONT=12,\"IP\",\"APN\",\"255.255.255.255\",0,0\r")

STRING(C_CGDCONT_13, "AT+CGDCONT=13,\"IP\",\"APN\",\"255.255.255.255\",0,0\r")

STRING(C_CGDCONT_14, "AT+CGDCONT=14,\"IP\",\"APN\",\"255.255.255.255\",0,0\r")

STRING(C_CGDCONT_15, "AT+CGDCONT=15,\"IP\",\"APN\",\"255.255.255.255\",0,0\r")

STRING(C_CGDCONT_16, "AT+CGDCONT=16,\"IP\",\"APN\",\"255.255.255.255\",0,0\r")

STRING(C_CGDCONT_1_CLEAR, "AT+CGDCONT=1\r")

STRING(C_CGDCONT_CLEAR, "AT+CGDCONT=\r")

STRING(C_CGDCONT_R, "AT+CGDCONT?\r")

STRING(C_CGDCONT_T, "AT+CGDCONT=?\r")

/*BYTE LC_CGDCONT_X 37*/

BYTE LC_CGDCONT_X 46

BYTE LC_CGDCONT_X_oIAP 20

BYTE LC_CGDCONT_XX 47

BYTE LC_CGDCONT_CLEAR 13

BYTE LC_CGDCONT_R 12

BYTE LC_CGDCONT_T 13

BYTE LC_CGDCONT_CLEAR 12

/* special command sequence */

STRING(C_CGDCONT_SPECIAL, "AT+CGDCONT=,\"IP\",\"APN\",\"18.52.86.120\",0,0;+CGQREQ=,1,1,3,1,1;+CGQMIN=,2,2,4,1,1\r")

- BYTE LC_CGDCONT_SPECIAL 80

/* command:  CGQREQ*/

STRING(C_CGQREQ_DEF_0, "AT+CGQREQ=,0,0,0,0,0\r")

STRING(C_CGQREQ_DEF_1, "AT+CGQREQ=,1,2,3,4,5\r")

STRING(C_CGQREQ_0, "AT+CGQREQ=1\r")

STRING(C_CGQREQ_1, "AT+CGQREQ=1,1,2,1,4,5\r")

STRING(C_CGQREQ_2, "AT+CGQREQ=1,3,4,3,2,1\r")

STRING(C_CGQREQ_R, "AT+CGQREQ?\r")

STRING(C_CGQREQ_T, "AT+CGQREQ=?\r")

- BYTE LC_CGQREQ_DEF 21

- BYTE LC_CGQREQ_0 12

BYTE LC_CGQREQ_X 22

BYTE LC_CGQREQ_R 11

BYTE LC_CGQREQ_T 12

/* command:  CGQMIN*/

STRING(C_CGQMIN_DEF_0, "AT+CGQMIN=,0,0,0,0,0\r")

STRING(C_CGQMIN_DEF_1, "AT+CGQMIN=,1,2,3,4,5\r")

STRING(C_CGQMIN_0, "AT+CGQMIN=1\r")

STRING(C_CGQMIN_1, "AT+CGQMIN=1,1,2,3,4,5\r")

STRING(C_CGQMIN_2, "AT+CGQMIN=1,3,4,3,2,1\r")

STRING(C_CGQMIN_R, "AT+CGQMIN?\r")

STRING(C_CGQMIN_T, "AT+CGQMIN=?\r")

- BYTE LC_CGQMIN_DEF 21

- BYTE LC_CGQMIN_0 12

BYTE LC_CGQMIN_X 22

BYTE LC_CGQMIN_R 11

BYTE LC_CGQMIN_T 12

/* command:  CGQREG and CGQMIN*/

STRING(C_CGQCOMB_DEF_1, "AT+CGQREQ=,1,2,1,4,5;+CGQMIN=,1,2,1,4,5\r")

- BYTE LC_CGQCOMB_DEF 40

/* command:  CGACT*/

STRING(C_CGACT_10, "AT+CGACT=0,1\r")

STRING(C_CGACT_11, "AT+CGACT=1,1\r")

STRING(C_CGACT_20, "AT+CGACT=0,2\r")

STRING(C_CGACT_21, "AT+CGACT=1,2\r")

STRING(C_CGACT_121, "AT+CGACT=1,1,2\r")

STRING(C_CGACT_R, "AT+CGACT?\r")

STRING(C_CGACT_T, "AT+CGACT=?\r")

- BYTE LC_CGACT_0 11

- BYTE LC_CGACT_X 13

- BYTE LC_CGACT_X2 15

BYTE LC_CGACT_R 10

BYTE LC_CGACT_T 11

STRING(C_CGACT_0, "AT+CGACT=0\r")

STRING(C_CGACT_1, "AT+CGACT=1\r")

/* command:  CGDATA*/

STRING(C_CGDATA_P1, "AT+CGDATA=\"PPP\",1\r")

STRING(C_CGDATA_01, "AT+CGDATA=,1\r")

STRING(C_CGDATA_P2, "AT+CGDATA=\"PPP\",2\r")

STRING(C_CGDATA_02, "AT+CGDATA="",2\r")

STRING(C_CGDATA_BAD, "AT+CGDATA=\"PPP\",1,2\r")

STRING(C_CGDATA_T, "AT+CGDATA=?\r")

- BYTE LC_CGDATA_PX 18

- BYTE LC_CGDATA_0X 13

BYTE LC_CGDATA_BAD 20

BYTE LC_CGDATA_T 12

/* command:  CGAUTO*/

STRING(C_CGAUTO_0, "AT+CGAUTO=0\r")

STRING(C_CGAUTO_1, "AT+CGAUTO=1\r")

STRING(C_CGAUTO_2, "AT+CGAUTO=2\r")

STRING(C_CGAUTO_3, "AT+CGAUTO=3\r")

STRING(C_CGAUTO_R, "AT+CGAUTO?\r")

STRING(C_CGAUTO_T, "AT+CGAUTO=?\r")

- BYTE LC_CGAUTO_X 12

- BYTE LC_CGAUTO_R 11

BYTE LC_CGAUTO_T 12

/* command:  CGANS*/

STRING(C_CGANS_0, "AT+CGANS=0\r")

STRING(C_CGANS_1, "AT+CGANS=1\r")

STRING(C_CGANS_2X, "AT+CGANS=1,\"PPP\",2\r")

STRING(C_CGANS_T, "AT+CGANS=?\r")

- BYTE LC_CGANS_X 11

- BYTE LC_CGANS_X2 19

BYTE LC_CGANS_T 11

/* command:  CGPADDR*/

STRING(C_CGPADDR_, "AT+CGPADDR=\r")

STRING(C_CGPADDR_1, "AT+CGPADDR=1\r")

STRING(C_CGPADDR_2, "AT+CGPADDR=1,2\r")

STRING(C_CGPADDR_100, "AT+CGPADDR=100\r")

STRING(C_CGPADDR_T, "AT+CGPADDR=?\r")

- BYTE LC_CGPADDR_ 11

- BYTE LC_CGPADDR_1 13

BYTE LC_CGPADDR_2 15

BYTE LC_CGPADDR_T 13

/* command:  CGSMS*/

STRING(C_CGSMS_0, "AT+CGSMS=0\r")

STRING(C_CGSMS_1, "AT+CGSMS=1\r")

STRING(C_CGSMS_2, "AT+CGSMS=2\r")

STRING(C_CGSMS_3, "AT+CGSMS=3\r")

STRING(C_CGSMS_R, "AT+CGSMS?\r")

STRING(C_CGSMS_T, "AT+CGSMS=?\r")

- BYTE LC_CGSMS_X 11

- BYTE LC_CGSMS_R 10

BYTE LC_CGSMS_T 11

/* command:  CGEREP */

STRING(C_CGEREP_0, "AT+CGEREP=0\r")

STRING(C_CGEREP_1, "AT+CGEREP=1\r")

STRING(C_CGEREP_2, "AT+CGEREP=2\r")

STRING(C_CGEREP_10, "AT+CGEREP=1,0\r")

STRING(C_CGEREP_11, "AT+CGEREP=1,1\r")

STRING(C_CGEREP_20, "AT+CGEREP=2,0\r")

STRING(C_CGEREP_21, "AT+CGEREP=2,1\r")

STRING(C_CGEREP_R, "AT+CGEREP?\r")

STRING(C_CGEREP_T, "AT+CGEREP=?\r")

- BYTE LC_CGEREP_X 12

- BYTE LC_CGEREP_XX 14

BYTE LC_CGEREP_R 11

BYTE LC_CGEREP_T 12

/* command:  CGREG */

STRING(C_CGREG_0, "AT+CGREG=0\r")

STRING(C_CGREG_1, "AT+CGREG=1\r")

STRING(C_CGREG_2, "AT+CGREG=2\r")

STRING(C_CGREG_R, "AT+CGREG?\r")

STRING(C_CGREG_T, "AT+CGREG=?\r")

- BYTE LC_CGREG_X 11

- BYTE LC_CGREG_R 10

BYTE LC_CGREG_T 11

/*

### Percent GPRS commands

command: CGAATT */

STRING(C_CGAATT_00, "AT%CGAATT=0,0\r")

STRING(C_CGAATT_01, "AT%CGAATT=0,1\r")

STRING(C_CGAATT_10, "AT%CGAATT=1,0\r")

STRING(C_CGAATT_11, "AT%CGAATT=1,1\r")

STRING(C_CGAATT_R, "AT%CGAATT?\r")

STRING(C_CGAATT_T, "AT%CGAATT=?\r")

- BYTE LC_CGAATT_X 14

- BYTE LC_CGAATT_R 11

BYTE LC_CGAATT_T 12

/* command: %CUNS */

STRING(C_CUNS_0, "AT%CUNS=0\r")

STRING(C_CUNS_1, "AT%CUNS=1\r")

STRING(C_CUNS_2, "AT%CUNS=2\r")

BYTE LC_CUNS  10

/* command:  %CGREG */

STRING(C_P_CGREG_0, "AT%CGREG=0\r")

STRING(C_P_CGREG_1, "AT%CGREG=1\r")

STRING(C_P_CGREG_2, "AT%CGREG=2\r")

STRING(C_P_CGREG_R, "AT%CGREG?\r")

STRING(C_P_CGREG_T, "AT%CGREG=?\r")

- BYTE LC_P_CGREG_X 11

- BYTE LC_P_CGREG_R 10

BYTE LC_P_CGREG_T 11

/*  Engineering mode – AT-CMD */

/*

- Command: 	%EM

get infrastructure data serving cell GPRS

*/

- STRING(C_PERCENT_EM_QUERY_2, "AT%EM=2,2 " )

- BYTE LC_PERCENT_EM_QUERY_2 9

/*

- Message:	%EM

get infrastructure data serving cell GPRS

*/

- STRING(M_PERCENT_EM_2, "%EM: 0,1,2,3,4,5,6,7,8,9" )

- BYTE LM_PERCENT_EM_2 24

/*

### Modem compatibility mode commands

command: D*/

STRING(C_GD_0, "ATD*99#\r")

STRING(C_GD_1, "ATDT*99*12345#\r")

STRING(C_GD_2, "ATDP*99**1#\r")

STRING(C_GD_3, "ATD*99*12345*1*2#\r")

- BYTE LC_GD_0 8

- BYTE LC_GD_1 15

BYTE LC_GD_2 12

BYTE LC_GD_3 18

/* command: A */

STRING(C_A, "ATA\r")

- BYTE LC_A 4

/* command: H */

STRING(C_H, "ATH\r")

- BYTE LC_H 4

/* command: Z */

STRING(C_Z, "ATZ\r")

- BYTE LC_Z 4

/* command: S0 */

STRING(C_S0_0, "ATS0=0\r")

STRING(C_S0_1, "ATS0=1\r")

- BYTE LC_S0_0 4

- BYTE LC_S0_1 4

- BYTE MSID_NO 100

/*

## MESSAGE SECTION

### General messages

*/

- STRING(M_OK, "OK")

- BYTE LM_OK 2

- STRING(M_CONNECT, "CONNECT")

- BYTE LM_CONNECT 7

- STRING(M_RING, "RING")

- BYTE LM_RING 4

- STRING(M_NO_CARRIER, "NO CARRIER")

- BYTE LM_NO_CARRIER 10

- STRING(M_ERROR, "ERROR")

- BYTE LM_ERROR 5

/*

message:	%SIMREM

SIM removed

*/

STRING(M_PERCENT_SIMREM_ERR_0, "%SIMREM: 0" )

- BYTE LM_PERCENT_SIMREM_ERR_0 10

/*

### AT command answer messages

*/

/* message: CGATT */

STRING(M_CGATT_R0, "+CGATT: 0")

- STRING(M_CGATT_R1, "+CGATT: 1")

- STRING(M_CGATT_T, "+CGATT: (0,1)")

- BYTE LM_CGATT_R 9

- BYTE LM_CGATT_T 13

/* message: +CGCLASS */

STRING(M_CGCLASS_RB, "+CGCLASS: \"B\"")

- STRING(M_CGCLASS_RC, "+CGCLASS: \"C\"")

- STRING(M_CGCLASS_RCG, "+CGCLASS: \"CG\"")

- STRING(M_CGCLASS_RCC, "+CGCLASS: \"CC\"")

- STRING(M_CGCLASS_T, "+CGCLASS: (\"B\",\"C\",\"CG\",\"CC\")")

- BYTE LM_CGCLASS_RX 13

- BYTE LM_CGCLASS_RXX 14

BYTE LM_CGCLASS_T 29

/* message: %CGCLASS */

STRING(M_PERCENT_CGCLASS_RBXBC, "%CGCLASS: \"BX\",\"BC\"")

STRING(M_PERCENT_CGCLASS_RBXBG, "%CGCLASS: \"BX\",\"BG\"")

STRING(M_PERCENT_CGCLASS_RB, "%CGCLASS: \"B\"")

STRING(M_PERCENT_CGCLASS_RBG, "%CGCLASS: \"BG\"")

STRING(M_PERCENT_CGCLASS_RBC, "%CGCLASS: \"BC\"")

STRING(M_PERCENT_CGCLASS_RC, "%CGCLASS: \"C\"")

STRING(M_PERCENT_CGCLASS_RCG, "%CGCLASS: \"CG\"")

STRING(M_PERCENT_CGCLASS_RCC, "%CGCLASS: \"CC\"")

- BYTE LM_PERCENT_CGCLASS_RBX 19

/* command:  CGDCONT*/

STRING(M_CGDCONT_R0, "+CGDCONT: ")

- STRING(M_CGDCONT_R1, "+CGDCONT: 1,\"IP\",\"APN\",\"255.255.255.255\",0,0")

STRING(M_CGDCONT_R2, "+CGDCONT: 2,\"IP\",\"APN\",\"255.255.255.255\",1,1")

- STRING(M_CGDCONT_T, "+CGDCONT: (1-2),\"IP\",,,(0,1),(0,1)")

- BYTE LM_CGDCONT_R0 10

- BYTE LM_CGDCONT_R1 44

BYTE LM_CGDCONT_R 44

BYTE LM_CGDCONT_T 34

/* special command sequence */

STRING(M_CGDCONT_SP1, "+CGDCONT: 1,\"IP\",\"APN\",\"18.52.86.120\",0,0")

STRING(M_CGDCONT_SP2, "+CGDCONT: 2,\"IP\",\"APN\",\"18.52.86.120\",0,0")

- BYTE LM_CGDCONT_SP1 41

- BYTE LM_CGDCONT_SP 41

/* command:  CGQREQ*/

STRING(M_CGQREQ_R0, "+CGQREQ: 1,0,0,0,0,0")

- STRING(M_CGQREQ_R1, "+CGQREQ: 1,1,2,1,4,5")

- STRING(M_CGQREQ_R2, "+CGQREQ: 1,3,4,3,2,1")

STRING(M_CGQREQ_R3, "+CGQREQ: 2,0,0,0,0,0")

- STRING(M_CGQREQ_T, "+CGQREQ: \"IP\",(1-3),(1-4),(1-5),(1-9),(1-18,31)")

- BYTE LM_CGQREQ_RX 20

BYTE LM_CGQREQ_T 47

/* command:  CGQMIN*/

STRING(M_CGQMIN_R0, "+CGQMIN: 1,0,0,0,0,0")

- STRING(M_CGQMIN_R1, "+CGQMIN: 1,1,2,3,4,5")

- STRING(M_CGQMIN_R2, "+CGQMIN: 1,3,4,3,2,1")

STRING(M_CGQMIN_R3, "+CGQMIN: 2,0,0,0,0,0")

- STRING(M_CGQMIN_T, "+CGQMIN: \"IP\",(1-3),(1-4),(1-5),(1-9),(1-18,31)")

- BYTE LM_CGQMIN_RX 20

BYTE LM_CGQMIN_T 47

/* command:  CGACT*/

STRING(M_CGACT_R00, "+CGACT: ")

- STRING(M_CGACT_R10, "+CGACT: 1,0")

- STRING(M_CGACT_R11, "+CGACT: 1,1")

STRING(M_CGACT_R20, "+CGACT: 1,0\r\n+CGACT: 2,0")

STRING(M_CGACT_R21, "+CGACT: 1,1\r\n+CGACT: 2,0")

- STRING(M_CGACT_T, "+CGACT: (0,1)")

- BYTE LM_CGACT_R0 8

- BYTE LM_CGACT_R1 11

BYTE LM_CGACT_R2 24

BYTE LM_CGACT_T 13

/* command:  CGDATA*/

STRING(M_CGDATA_T, "+CGDATA: \"PPP\"")

- BYTE LM_CGDATA_T 14

/* command:  CGAUTO*/

STRING(M_CGAUTO_R0, "+CGAUTO: 0")

- STRING(M_CGAUTO_R1, "+CGAUTO: 1")

- STRING(M_CGAUTO_R2, "+CGAUTO: 2")

- STRING(M_CGAUTO_R3, "+CGAUTO: 3")

- STRING(M_CGAUTO_T, "+CGAUTO: (0-3)")

- BYTE LM_CGAUTO_R 10

- BYTE LM_CGAUTO_T 14

/* command:  CGANS*/

STRING(M_CGANS_T, "+CGANS: (0,1),(\"PPP\")")

- BYTE LM_CGANS_T 21

/* command:  CGPADDR*/

STRING(M_CGPADDR_, "+CGPADDR: ")

- STRING(M_CGPADDR_1_, "+CGPADDR: 1")

- STRING(M_CGPADDR_1, "+CGPADDR: 1,\"255.255.255.255\"")

- STRING(M_CGPADDR_1X, "+CGPADDR: 1,\"254.254.254.254\"")

- STRING(M_CGPADDR_2, "+CGPADDR: 2,\"255.255.255.255\"")

- STRING(M_CGPADDR_T0, "+CGPADDR: ()")

- STRING(M_CGPADDR_T1, "+CGPADDR: (1)")

- STRING(M_CGPADDR_T2, "+CGPADDR: (2)")

- STRING(M_CGPADDR_T12, "+CGPADDR: (1,2)")

- BYTE LM_CGPADDR_ 10

- BYTE LM_CGPADDR_X_ 11

BYTE LM_CGPADDR_X 29

BYTE LM_CGPADDR_X 29

BYTE LM_CGPADDR_T0 12

BYTE LM_CGPADDR_T1 13

BYTE LM_CGPADDR_T2 13

BYTE LM_CGPADDR_T12 15

/* command:  CGSMS*/

STRING(M_CGSMS_R0, "+CGSMS: 0")

- STRING(M_CGSMS_R1, "+CGSMS: 1")

- STRING(M_CGSMS_R2, "+CGSMS: 2")

- STRING(M_CGSMS_R3, "+CGSMS: 3")

- STRING(M_CGSMS_T, "+CGSMS: (0-3)")

- BYTE LM_CGSMS_RX 9

- BYTE LM_CGSMS_T 13

/* command:  CGEREP */

/*STRING(C_CGEREP_0, "AT+CGEREP=0\r")

STRING(C_CGEREP_1, "AT+CGEREP=1\r")

STRING(C_CGEREP_2, "AT+CGEREP=2\r")

STRING(C_CGEREP_10, "AT+CGEREP=1,0\r")

STRING(C_CGEREP_11, "AT+CGEREP=1,1\r")

STRING(C_CGEREP_20, "AT+CGEREP=2,0\r")

STRING(C_CGEREP_21, "AT+CGEREP=2,1\r")

STRING(C_CGEREP_R, "AT+CGEREP?\r")

STRING(C_CGEREP_T, "AT+CGEREP=?\r")

- BYTE LC_CGEREP_X 12

- BYTE LC_CGEREP_XX 14

BYTE LC_CGEREP_R 11

BYTE LC_CGEREP_T 12*/

/* command:  CGREG */

STRING(M_CGREG_00, "+CGREG: 0,0")

STRING(M_CGREG_01, "+CGREG: 0,1")

STRING(M_CGREG_02, "+CGREG: 0,2")

- BYTE LM_CGREG_R 11

STRING(M_CGREG_0, "+CGREG: 0")

STRING(M_CGREG_1, "+CGREG: 1")

STRING(M_CGREG_2, "+CGREG: 2")

STRING(M_CGREG_3, "+CGREG: 3")

STRING(M_CGREG_4, "+CGREG: 4")

STRING(M_CGREG_5, "+CGREG: 5")

- BYTE LM_CGREG 9

/* command: %CGREG */

STRING(M_P_CGREG_00, "%CGREG: 0,0")

STRING(M_P_CGREG_01, "%CGREG: 0,1")

STRING(M_P_CGREG_02, "%CGREG: 0,2")

- BYTE LM_P_CGREG_R 11

STRING(M_P_CGREG_0, "%CGREG: 0")

STRING(M_P_CGREG_1, "%CGREG: 1")

STRING(M_P_CGREG_2, "%CGREG: 2")

STRING(M_P_CGREG_3, "%CGREG: 3")

STRING(M_P_CGREG_4, "%CGREG: 4")

STRING(M_P_CGREG_5, "%CGREG: 5")

STRING(M_P_CGREG_6, "%CGREG: 6")

STRING(M_P_CGREG_7, "%CGREG: 7")

STRING(M_P_CGREG_8, "%CGREG: 8")

- BYTE LM_P_CGREG 9

/* command:  CGAATT*/

STRING(M_CGAATT_R00, "%CGAATT: 0,0")

- STRING(M_CGAATT_R01, "%CGAATT: 0,1")

- STRING(M_CGAATT_R10, "%CGAATT: 1,0")

- STRING(M_CGAATT_R11, "%CGAATT: 1,1")

- STRING(M_CGAATT_T, "%CGAATT: (0,1),(0,1)")

- BYTE LM_CGAATT_R 12

- BYTE LM_CGAATT_T 20

/* command:  COPS*/

STRING(M_PLUS_COPS_LST, "+COPS: (1,\"T-Mobile D\",\"TMO D\",\"26201\"),(1,\"Vodafone D2\",\"Voda D2\",\"26202\"),(1,\"E-Plus\",\"E-Plus\",\"26203\")" )

BYTE LM_PLUS_COPS_LST 105

/*

Command:	%CPRI

ciphering indication set command

*/

STRING(C_PERCENT_CPRI_S1, "AT%CPRI=1" )

BYTE LC_PERCENT_CPRI_S1 9

/*

Command:	%CPRI

ciphering indication set command

*/

STRING(C_PERCENT_CPRI_S2, "AT%CPRI=0" )

BYTE LC_PERCENT_CPRI_S2 9

/*

Command:	%CPRI

ciphering indication query command

*/

STRING(C_PERCENT_CPRI_Q1, "AT%CPRI?" )

BYTE LC_PERCENT_CPRI_Q1 8

/*

Message:	%CPRI

ciphering indication query message

*/

STRING(M_PERCENT_CPRI_Q1, "%CPRI: 2" )

BYTE LM_PERCENT_CPRI_Q1 8

/*

Command:	%CPRI

ciphering indication test command

*/

STRING(C_PERCENT_CPRI_T1, "AT%CPRI=?" )

BYTE LC_PERCENT_CPRI_T1 9

/*

Message:	%CPRI

ciphering indication test message

*/

STRING(M_PERCENT_CPRI_T1, "%CPRI: (0,1)" )

BYTE LM_PERCENT_CPRI_T1 12

/*

Message:	%CPRI

ciphering indication query message (2)

*/

STRING(M_PERCENT_CPRI_Q2, "%CPRI: 1" )

BYTE LM_PERCENT_CPRI_Q2 8

/*

Message:	%CPRI

ciphering indication unsolicited message

*/

STRING(M_PERCENT_CPRI_U1, "%CPRI: 2,1" )

BYTE LM_PERCENT_CPRI_U1 10

/* command:  %CGPCO */

/*Command %CGPCO = Set protocol configuration options for PDP context, ASCII:

Auth prot
user
password
dns1
dns2
cid */

STRING(C_PERC_CGPCO_1, "AT%CGPCO=0,1,\"PAP,USER,PASSW,145.100.255.001,255.255.255.255\",0")

BYTE LC_PERC_CGPCO_1 63

/*Command %CGPCO = Set protocol configuration options for PDP context, ASCII:

Auth prot
dns1
cid

*/

STRING(C_PERC_CGPCO_2, "AT%CGPCO=0,1,\"PAP,,,145.100.255.001\",0")

BYTE LC_PERC_CGPCO_2 38

STRING(C_PERC_CGPCO_3, "AT%CGPCO=0,1,\"PAP,userd1,passd1,0.0.0.0, 0.0.0.0\"")

/*Command %CGPCO = Query protocol configuration options for PDP context, ASCII:
 	DNS1
	DNS2
	GATEWAY

*/

STRING(QUERY_PERC_CGPCO_1,"AT%CGPCO=1,1")

BYTE LQ_PERC_CGPCO_1 12

/*Message Indication %CGPCO output pco’s */

STRING(M_PERC_CGPCO_1, "%CGPCO:145.100.255.001,255.255.255.255,,0" )

BYTE LM_PERC_CGPCO_1 41

STRING(M_PERC_CGPCO_2, "%CGPCO:145.100.255.001,255.255.255.255,,1" )

BYTE LM_PERC_CGPCO_2 41

STRING(M_QPERC_CGPCO_1,"%CGPCO:141.64.24.129,141.64.254.131,0")

STRING(M_QPERC_CGPCO_2,"%CGPCO:141.64.102.3,0")

STRING(C_P_DATA_DEF, "AT%DATA=1,\"PKTIO\",0,, \"PKT\",,,,1")

STRING(C_P_DATA_ALL, "AT%DATA=1,\"PKTIO\",0,0,\"PKT\",\"UART\",0,0,1")

/* for UART dual port testing */

STRING(C_P_DATA_DP, "AT%DATA=2,\"UART\",1,,\"SER\",\"UART\",0")

/*---------- ALL OTHER STUFF --------------------------------------------------------*/

/*

### Values

*/

BYTE PIN_3_ATTEMPTS	3

BYTE PIN_2_ATTEMPTS	2

BYTE PUK_10_ATTEMPTS	10

BYTE PIN_1	1

BYTE PIN_2	2

/* all profiles supported  0xE0 */

BYTE MMI_AND_FDN_BDN	0xE0

BYTE GACI_TUI	1

BYTE UART_TUI	2

BYTE PPP_TUI		3

BYTE OP_UNACK	0x00

BYTE OP_ACK	0x01

BYTE DTI_PID_UTCP	0x2F

/* digits 0 to 10 */

BYTE NUM_0 0

BYTE NUM_1 1

BYTE NUM_2 2

BYTE NUM_3 3

BYTE NUM_4 4

BYTE NUM_5 5

BYTE NUM_6 6

BYTE NUM_7 7

BYTE NUM_8 8

BYTE NUM_9 9

BYTE NUM_10 10

BYTE NUM_12 12

BYTE NUM_21 21

BYTE NUM_22 22

BYTE NUM_FF 255

BYTE NUM_1000 1000

LONG VAL_T3314 44000
LONG VAL_T3312 3240000

USHORT NSAPI_SET_NSAPI_5 =	0x0020;

USHORT NSAPI_SET_NSAPI_6 =	0x0040;

USHORT NSAPI_SET_NSAPI_56 =	0x0060;

ULONG STRING_POINTER = 0xfe1234ef;

ULONG STRING_NULL_POINTER = 0;

#define DEVICE_1 	NUM_0

#define DEVICE_2 	NUM_4

ULONG UART_DTI_ID =	0x00000100;

ULONG UART_DTI_ID_2 =	0x00000200;

ULONG SNDCP_DTI_ID =	0x00000101;

ULONG SNDCP_DTI_ID_2 =	0x00000201;

ULONG SNDCP_NULL_DTI_ID =	0x00000200;

ULONG SNDCP_NULL_DTI_ID_2 =	0x00000300;

ULONG PKT_LINK_ID =	0x00000200;

#define STANDARD_DLCI 	NUM_FF

ULONG IP_ADDRESS_1 = 0xFFFFFFFF;

BYTE PEER_DIRECTION	1

BYTE PROT_DIRECTION	0

/* values for the elimination of warnings */

#define UNUSED_IN_TESTCASE 0

- /* data PKTIO */

/* PKTIO data */

LONG	LONG_0	0x00000000
BYTE DIO_CONV_PACKET 0x02

BYTE DIO_CONV_FAIL 0x09

BYTE DIO_MODE_DATA 0x02

BYTE DIO_SLEEP_DISABLE 0x02

LONG DIO_BAUD_812500 0x80000

LONG DIO_BAUD_14400 0x0400

BYTE DIO_CHAR_8 0x80

BYTE DIO_STOP_1 0x01

BYTE DIO_PARITY_NO 0x01

SHORT DIO_FLOW_HW_HW 0x100

BYTE DIO_ESC_OFF 0x00

SHORT PKTCS_SUCCESS 0xD200

SHORT PKTCS_DISCONNECT 0xD203

BYTE NORMAL_WAY 0x01

BYTE DTI_CHANNEL_TO_LOWER_LAYER 1 /* defined in dti.h */

/* Maximum transmission unit
 */

SHORT	MTU_1500	1500

/* All baud rates allowed on shared memory except DIO_BAUD_AUTO
 */
LONG	DIO_BAUD_SHAREDMEM	0x000FFFFE

LONG	DIO_BAUD_TEST_1	0x000005FE

/*

### Fields

*/

/* SIM Service Table with Nr. 4 */

FIELD (F_SIM_SRV_4) 0xC0, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDFIELD (F_SIM_SRV_4, 10)

FIELD (PIN_1_VALUE) 0x34,0x33,0x32,0x31,0xFF,0xFF,0xFF,0xFF
ENDFIELD (PIN_1_VALUE, 8)

FIELD (PIN_1_WRONG) 0x31,0x32,0x33,0x34,0xFF,0xFF,0xFF,0xFF
ENDFIELD (PIN_1_WRONG, 8)

- FIELD (AIR_ADD_INFO_1)

32,0,

0,0,

0x1, 0x2, 0x3, 0x4, 0x0, 0x0

- ENDFIELD(AIR_ADD_INFO_1, 10)

/*

### PLMN list (+COPS=?)

*/

BEGINARRAY (MCC_PLMN_LST_1, 3) 0x02, 0x06, 0x02 ENDARRAY

BEGINARRAY (MNC_PLMN_LST_1, 3) 0x00, 0x01,0x0F ENDARRAY

BEGIN_PSTRUCT ("plmn", PLMN_LST_1)
	SET_COMP ("v_plmn", V_PLMN_PRES)
	SET_COMP ("mcc", MCC_PLMN_LST_1)
	SET_COMP ("mnc", MNC_PLMN_LST_1)
ENDSTRUCT

BEGINARRAY (MCC_PLMN_LST_2, 3) 0x02, 0x06, 0x02 ENDARRAY

BEGINARRAY (MNC_PLMN_LST_2, 3) 0x00, 0x02,0x0F ENDARRAY

BEGIN_PSTRUCT ("plmn", PLMN_LST_2)
	SET_COMP ("v_plmn", V_PLMN_PRES)
	SET_COMP ("mcc", MCC_PLMN_LST_2)
	SET_COMP ("mnc", MNC_PLMN_LST_2)
ENDSTRUCT

BEGINARRAY (MCC_PLMN_LST_3, 3) 0x02, 0x06, 0x02 ENDARRAY

BEGINARRAY (MNC_PLMN_LST_3, 3) 0x00, 0x03,0x0F ENDARRAY

BEGIN_PSTRUCT ("plmn", PLMN_LST_3)
	SET_COMP ("v_plmn", V_PLMN_PRES)
	SET_COMP ("mcc", MCC_PLMN_LST_3)
	SET_COMP ("mnc", MNC_PLMN_LST_3)
ENDSTRUCT

BEGINARRAY (MCC_PLMN_LST_4, 3) 0x00, 0x00, 0x00 ENDARRAY
BEGINARRAY (MNC_PLMN_LST_4, 3) 0x00, 0x00,0x0F ENDARRAY

BEGIN_PSTRUCT ("plmn", PLMN_LST_4)
	SET_COMP ("v_plmn", V_PLMN_PRES)
	SET_COMP ("mcc", MCC_PLMN_LST_4)
	SET_COMP ("mnc", MNC_PLMN_LST_4)
ENDSTRUCT

BEGINARRAY (MCC_PLMN_LST_5, 3) 0x00, 0x00, 0x00 ENDARRAY
BEGINARRAY (MNC_PLMN_LST_5, 3) 0x00, 0x00,0x0F ENDARRAY

BEGIN_PSTRUCT ("plmn", PLMN_LST_5)
	SET_COMP ("v_plmn", V_PLMN_PRES)
	SET_COMP ("mcc", MCC_PLMN_LST_5)
	SET_COMP ("mnc", MNC_PLMN_LST_5)
ENDSTRUCT

BEGIN_PSTRUCT ("plmn", EMPTY_PLMN_LST)
	SET_COMP ("v_plmn", GMMREG_PLMN_NOT_PRES)
	SKIP_COMP ("mcc")
	SKIP_COMP ("mnc")
ENDSTRUCT

BEGINARRAY (FRB_PLMN_LST, 7) 
	0x00, 0x00, 0x00, 0x01, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (RXL_PLMN_LST, 7)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

BEGINARRAY (GPRS_STATUS_LST, GMMREG_MAX_PLMN_ID)
	GMMREG_GPRS_GSM,

GMMREG_GPRS_GSM,

GMMREG_GPRS_GSM,

GMMREG_GPRS_GSM,

GMMREG_GPRS_GSM,

GMMREG_GPRS_GSM,

GMMREG_GPRS_GSM,

GMMREG_GPRS_GSM,

GMMREG_GPRS_GSM,

GMMREG_GPRS_GSM,

GMMREG_GPRS_GSM,

GMMREG_GPRS_GSM

ENDARRAY

BEGIN_PSTRUCT_ARRAY (PLMN_LST, GMMREG_MAX_PLMN_ID) 
            PLMN_LST_1, 
            PLMN_LST_2, 
            PLMN_LST_3, 
            EMPTY_PLMN_LST, 
            EMPTY_PLMN_LST,
            EMPTY_PLMN_LST, 
            EMPTY_PLMN_LST, 
            EMPTY_PLMN_LST, 
            EMPTY_PLMN_LST, 
            EMPTY_PLMN_LST, 
            EMPTY_PLMN_LST, 
            EMPTY_PLMN_LST
ENDARRAY

/*DIO Capabilities from DIO */

BEGIN_PSTRUCT ("dio_dcb", DEFAULT_DCB_FROM_DIO_1)
	SET_COMP ("convergence", DIO_CONV_PACKET)
	SET_COMP ("data_mode", DIO_MODE_DATA)
	SET_COMP ("sleep_mode", DIO_SLEEP_DISABLE)
	SET_COMP ("mux_configuration", LONG_0)
	SET_COMP ("n1", NUM_0)
	SET_COMP ("n2", NUM_0)
	SET_COMP ("t1", NUM_0)
	SET_COMP ("t2", NUM_0)
	SET_COMP ("t3", NUM_0)
	SET_COMP ("k", NUM_0)
	SET_COMP ("mtu", MTU_1500)
	SET_COMP ("baud", DIO_BAUD_SHAREDMEM)
	SET_COMP ("data_bits", DIO_CHAR_8) /* Some defaults */
	SET_COMP ("stop_bits", DIO_STOP_1) /* Some defaults */
	SET_COMP ("parity", DIO_PARITY_NO) /* Some defaults */
	SET_COMP ("flow_control", DIO_FLOW_HW_HW) /* Shared memory is HW */
	SET_COMP ("xon", NUM_0)
	SET_COMP ("xoff", NUM_0)
	SET_COMP ("esc_char", NUM_0)
	SET_COMP ("guard_period", DIO_ESC_OFF) /* No guard pattern detection */
ENDSTRUCT

BEGIN_PSTRUCT ("dio_dcb", DEFAULT_DCB_FROM_DIO_2)
	SET_COMP ("convergence", DIO_CONV_PACKET)
	SET_COMP ("data_mode", DIO_MODE_DATA)
	SET_COMP ("sleep_mode", DIO_SLEEP_DISABLE)
	SET_COMP ("mux_configuration", LONG_0)
	SET_COMP ("n1", NUM_0)
	SET_COMP ("n2", NUM_0)
	SET_COMP ("t1", NUM_0)
	SET_COMP ("t2", NUM_0)
	SET_COMP ("t3", NUM_0)
	SET_COMP ("k", NUM_0)
	SET_COMP ("mtu", MTU_1500)
	SET_COMP ("baud", DIO_BAUD_TEST_1)
	SET_COMP ("data_bits", DIO_CHAR_8) /* Some defaults */
	SET_COMP ("stop_bits", DIO_STOP_1) /* Some defaults */
	SET_COMP ("parity", DIO_PARITY_NO) /* Some defaults */
	SET_COMP ("flow_control", DIO_FLOW_HW_HW) /* Shared memory is HW */
	SET_COMP ("xon", NUM_0)
	SET_COMP ("xoff", NUM_0)
	SET_COMP ("esc_char", NUM_0)
	SET_COMP ("guard_period", DIO_ESC_OFF) /* No guard pattern detection */
ENDSTRUCT

BEGIN_PSTRUCT ("dio_dcb", DEFAULT_DCB_FROM_DIO_3)
	SET_COMP ("convergence", DIO_CONV_FAIL)
	SET_COMP ("data_mode", DIO_MODE_DATA)
	SET_COMP ("sleep_mode", DIO_SLEEP_DISABLE)
	SET_COMP ("mux_configuration", LONG_0)
	SET_COMP ("n1", NUM_0)
	SET_COMP ("n2", NUM_0)
	SET_COMP ("t1", NUM_0)
	SET_COMP ("t2", NUM_0)
	SET_COMP ("t3", NUM_0)
	SET_COMP ("k", NUM_0)
	SET_COMP ("mtu", MTU_1500)
	SET_COMP ("baud", DIO_BAUD_SHAREDMEM)
	SET_COMP ("data_bits", DIO_CHAR_8) /* Some defaults */
	SET_COMP ("stop_bits", DIO_STOP_1) /* Some defaults */
	SET_COMP ("parity", DIO_PARITY_NO) /* Some defaults */
	SET_COMP ("flow_control", DIO_FLOW_HW_HW) /* Shared memory is HW */
	SET_COMP ("xon", NUM_0)
	SET_COMP ("xoff", NUM_0)
	SET_COMP ("esc_char", NUM_0)
	SET_COMP ("guard_period", DIO_ESC_OFF) /* No guard pattern detection */
ENDSTRUCT

/*DIO Capabilities from ACI */

BEGIN_PSTRUCT ("dio_dcb", DEFAULT_DCB_FROM_ACI_1)
	SET_COMP ("convergence", DIO_CONV_PACKET)
	SET_COMP ("data_mode", DIO_MODE_DATA)
	SET_COMP ("sleep_mode", DIO_SLEEP_DISABLE)
	SET_COMP ("mux_configuration", LONG_0)
	SET_COMP ("n1", NUM_0)
	SET_COMP ("n2", NUM_0)
	SET_COMP ("t1", NUM_0)
	SET_COMP ("t2", NUM_0)
	SET_COMP ("t3", NUM_0)
	SET_COMP ("k", NUM_0)
	SET_COMP ("mtu", MTU_1500)
	SET_COMP ("baud", DIO_BAUD_812500) /* Negotiated highest common */
	SET_COMP ("data_bits", DIO_CHAR_8)
	SET_COMP ("stop_bits", DIO_STOP_1)
	SET_COMP ("parity", DIO_PARITY_NO)
	SET_COMP ("flow_control", DIO_FLOW_HW_HW)
	SET_COMP ("xon", NUM_0)
	SET_COMP ("xoff", NUM_0)
	SET_COMP ("esc_char", NUM_0)
	SET_COMP ("guard_period", DIO_ESC_OFF)
ENDSTRUCT

BEGIN_PSTRUCT ("dio_dcb", DEFAULT_DCB_FROM_ACI_2)
	SET_COMP ("convergence", DIO_CONV_PACKET)
	SET_COMP ("data_mode", DIO_MODE_DATA)
	SET_COMP ("sleep_mode", DIO_SLEEP_DISABLE)
	SET_COMP ("mux_configuration", LONG_0)
	SET_COMP ("n1", NUM_0)
	SET_COMP ("n2", NUM_0)
	SET_COMP ("t1", NUM_0)
	SET_COMP ("t2", NUM_0)
	SET_COMP ("t3", NUM_0)
	SET_COMP ("k", NUM_0)
	SET_COMP ("mtu", MTU_1500)
	SET_COMP ("baud", DIO_BAUD_14400) /* Negotiated highest common */
	SET_COMP ("data_bits", DIO_CHAR_8)
	SET_COMP ("stop_bits", DIO_STOP_1)
	SET_COMP ("parity", DIO_PARITY_NO)
	SET_COMP ("flow_control", DIO_FLOW_HW_HW)
	SET_COMP ("xon", NUM_0)
	SET_COMP ("xoff", NUM_0)
	SET_COMP ("esc_char", NUM_0)
	SET_COMP ("guard_period", DIO_ESC_OFF)
ENDSTRUCT

/*

### Arrays

#### Access point name – APN

*/

- BEGINARRAY (APN_BUF_1, 4) 0x03, 0x41, 0x50, 0x4E

ENDARRAY

/*

#### Packet Data Protocol address – PDP address

*/

- BEGINARRAY (PDP_ADD_BUF_1, 4)

0xFF, 0xFF, 0xFF, 0xFF

- ENDARRAY

- BEGINARRAY (PDP_ADD_BUF_2, 4)

0xFE, 0xFE, 0xFE, 0xFE

- ENDARRAY

- BEGINARRAY (PDP_ADD_BUF_3, 4)

0x12, 0x34, 0x56, 0x78

- ENDARRAY

/*

#### Mobile Country Code and Mobile Network Code

*/

/* EF ECC field array */

- BEGINARRAY (A_ECC_FIELD,12) 0x11, 0xF2, 0xFF, 0x99, 0xF9, 0xFF, 0x21, 0x43, 0x65, 0xFF, 0xFF, 0xFF

ENDARRAY

/* EF AD field array , disable CI */

BEGINARRAY_PART (A_AD_FIELD_CI_DISABLED,4) 0x00, 0x00, 0x00, 0x02

ENDARRAY

/* EF AD field array , enable CI */

BEGINARRAY_PART (A_AD_FIELD_CI_ENABLED,4) 0x00, 0x00, 0x01, 0x02

ENDARRAY

- BEGINARRAY (PREF_PLMN_ARRAY,3) 0x00,0x04,0x09

ENDARRAY

- BEGINARRAY (MCC_1,3) 0x00,0x04,0x09

ENDARRAY

- BEGINARRAY (MCC_262,3) 0x02,0x06,0x02

ENDARRAY

- BEGINARRAY (MNC_1,3) 0x00,0x04,0x0F

ENDARRAY

- BEGINARRAY (MNC_02,3) 0x00,0x02,0x0F

ENDARRAY

- BEGINARRAY (SIM_TOOLKIT_PROFILE,12) 0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00

ENDARRAY

BEGINARRAY (NO_EC_CODES, 15)
	0xFF,0xFF,0xFF,0xFF,0xFF,
	0xFF,0xFF,0xFF,0xFF,0xFF,
	0xFF,0xFF,0xFF,0xFF,0xFF
ENDARRAY

BEGINARRAY (NO_PREF_LANG,5) 0xFF,0xFF,0xFF,0xFF,0xFF
ENDARRAY

BEGINARRAY (IMSI_ARRAY,9) 0x29, 0x26, 0x10, 0x74, 0x11, 0x94, 0x21, 0xFF, 0xFF
ENDARRAY

BEGINARRAY (SNDCP_NAME_BUF, 6) 0x53, 0x4E, 0x44, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (UART_NAME_BUF, 6) 0x55, 0x41, 0x52, 0x54, 0x00, 0x00
ENDARRAY

/*

## Primitive Structs

*/

SET_SDU (SDU, 32, 0)

0x01, 0x02, 0x03, 0x04

ENDSDU

SET_SDU (SDU_2, 40, 0)

0x05, 0x06, 0x07, 0x08, 0x09

ENDSDU

SET_SDU (SDU_3, 48, 0)

0x04, 0x05, 0x06, 0x07, 0x08, 0x09

ENDSDU

SET_SDU (SDU_CTX_REQ_DEF, 160, 0)

0x80,0x80,0x21,0x10,0x01,0x01,0x00,0x10,0x81,0x06,

0x00,0x00,0x00,0x00,0x83,0x06,0x00,0x00,0x00,0x00

ENDSDU

/* Code CONF_NAK-> no gateway */

SET_SDU (SDU_NET_DNS_1, 160, 0)

0x80,

0x80, 0x21, 0x10,

0x03, 0x01, 0x00, 0x10, 0x81, 0x06, 0x8d, 0x40, 0x18, 0x81,
	0x83, 0x06, 0x8d, 0x40, 0xfe, 0x83

ENDSDU

/* Code CONF_REQ -> no primary and secondary DNS */

SET_SDU (SDU_NET_GATEW_2, 112, 0)

0x80,

0x80, 0x21, 0x0A,

0x01, 0x01, 0x00, 0x0A, 0x03, 0x06, 0x8d, 0x40, 0x66, 0x03

ENDSDU

/*

#### PLMN

*/

BEGIN_PSTRUCT("plmn", PLMN_1)

SET_COMP("v_plmn", v_plmn_PRES)

SET_COMP("mcc", MCC_1)

SET_COMP("mnc", MNC_1)

- ENDSTRUCT

BEGIN_PSTRUCT("plmn", PLMN_262_02)

SET_COMP("v_plmn", v_plmn_PRES)

SET_COMP("mcc", MCC_262)

SET_COMP("mnc", MNC_02)

- ENDSTRUCT

BEGIN_PSTRUCT("pref_plmn", PREF_PLMN)

SET_COMP("c_pref", NUM_3)

SET_COMP("pref", PREF_PLMN_ARRAY)

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_apn", SMREG_APN_0_S)

SHOW_COMP ("buffer") /* this is an empty APN and mean: use the subscribed APN */

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_apn", SMREG_APN_1)

SET_COMP("buffer", APN_BUF_1) /* this is not a valid APN */

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_apn", UNUSED_APN)

SET_COMP("buffer", APN_BUF_1) /* this is not a valid APN */

- ENDSTRUCT

BEGIN_PSTRUCT("pdp_address", PDP_ADDRESS_0_S)

SHOW_COMP ("buff") /* this is an empty PDP address and mean: use the subscribed PDP address */

- ENDSTRUCT

BEGIN_PSTRUCT("pdp_address", PDP_ADDRESS_1)

SET_COMP("buff", PDP_ADD_BUF_1) /* this is not a valid PDP address */

- ENDSTRUCT

BEGIN_PSTRUCT("pdp_address", PDP_ADDRESS_2)

SET_COMP("buff", PDP_ADD_BUF_2) /* this is not a valid PDP address */

- ENDSTRUCT

BEGIN_PSTRUCT("pdp_address", PDP_ADDRESS_3)

SET_COMP("buff", PDP_ADD_BUF_3) /* this is not a valid PDP address */

- ENDSTRUCT

BEGIN_PSTRUCT("pdp_address", UNUSED_PDP_ADDRESS)

SET_COMP("buff", PDP_ADD_BUF_2) /* this is not a valid PDP address */

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_qos", SMREG_QOS_0)

SET_COMP("delay", NUM_0)

SET_COMP("relclass", NUM_0)

SET_COMP("peak", NUM_0)

SET_COMP("preced", NUM_0)

SET_COMP("mean", NUM_0)

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_qos", SMREG_QOS_1)

SET_COMP("delay", NUM_2)

SET_COMP("relclass", NUM_1)

SET_COMP("peak", NUM_4)

SET_COMP("preced", NUM_1)

SET_COMP("mean", NUM_5)

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_qos", SMREG_QOS_2)

SET_COMP("delay", NUM_4)

SET_COMP("relclass", NUM_3)

SET_COMP("peak", NUM_2)

SET_COMP("preced", NUM_3)

SET_COMP("mean", NUM_1)

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_qos", SMREG_QOS_3)

SET_COMP("delay", NUM_1)

SET_COMP("relclass", NUM_3)

SET_COMP("peak", NUM_1)

SET_COMP("preced", NUM_1)

SET_COMP("mean", NUM_1)

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_qos", SMREG_QOS_4)

SET_COMP("delay", NUM_2)

SET_COMP("relclass", NUM_4)

SET_COMP("peak", NUM_1)

SET_COMP("preced", NUM_2)

SET_COMP("mean", NUM_1)

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_qos", UNUSED_QOS)

SET_COMP("delay", NUM_4)

SET_COMP("relclass", NUM_3)

SET_COMP("peak", NUM_2)

SET_COMP("preced", NUM_3)

SET_COMP("mean", NUM_1)

- ENDSTRUCT

BEGIN_PSTRUCT("protocol_channel", SNDCP_CHANNEL)

SET_COMP("protocol_entity", SNDCP_NAME_BUF)

- ENDSTRUCT

BEGIN_PSTRUCT("peer_channel", UART_CHANNEL)

SET_COMP("peer_entity", UART_NAME_BUF)

- ENDSTRUCT

BEGIN_PSTRUCT("imsi_field", IMSI)

SET_COMP("c_field", NUM_9)

SET_COMP("field", IMSI_ARRAY)

- ENDSTRUCT

BEGIN_PSTRUCT ("net_ctrl", S_NET_CTRL)
	SET_COMP ("spgc_ccch_sup",	NUM_2)
	SET_COMP ("priority_access_thr",	NUM_3)
ENDSTRUCT

# TEST CASES

## Routing (internal)

### GACI000: Setup the Routing for the GACI test

Description:

Routings for the GACI tests are set.

Preamble:

None

APL		ACI		PS

|		|		|

COMMAND (TAP RESET)
COMMAND (CC RESET)
COMMAND (MM RESET)
COMMAND (SIM RESET)
COMMAND (SS RESET)
COMMAND (MMI RESET)
COMMAND (SMS RESET)
COMMAND (GMM RESET)
COMMAND (SM RESET)
COMMAND (SND RESET)
COMMAND (PPP RESET)

COMMAND (UART REDIRECT MMI NULL)

COMMAND (UART RESET)
COMMAND (PKT RESET)
COMMAND (PL RESET)
	 |		|		|
COMMAND (TAP REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
COMMAND (MM REDIRECT CLEAR)
COMMAND (SIM REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
COMMAND (MMI REDIRECT CLEAR)
COMMAND (SMS REDIRECT CLEAR)
COMMAND (GMM REDIRECT CLEAR)
COMMAND (SM REDIRECT CLEAR)
COMMAND (SND REDIRECT CLEAR)
COMMAND (PPP REDIRECT CLEAR)
COMMAND (UART REDIRECT CLEAR)
COMMAND (PKT REDIRECT CLEAR)
COMMAND (PL REDIRECT CLEAR)
	 |		|		|
COMMAND (MMI REDIRECT CC TAP)
COMMAND (MMI REDIRECT MM TAP)
COMMAND (MMI REDIRECT SIM TAP)
COMMAND (MMI REDIRECT SS TAP)
COMMAND (MMI REDIRECT MMI TAP)
COMMAND (MMI REDIRECT SMS TAP)
COMMAND (MMI REDIRECT PL TAP)
COMMAND (MMI REDIRECT GMM TAP)
COMMAND (MMI REDIRECT SM TAP)
COMMAND (MMI REDIRECT SND TAP)
COMMAND (MMI REDIRECT PPP TAP)
COMMAND (MMI REDIRECT PKT TAP)
COMMAND (MMI REDIRECT UART TAP)
COMMAND (PL REDIRECT MMI NULL)
	|		|		|
COMMAND (TAP REDIRECT TAP MMI)
COMMAND (MMI REDIRECT MMI TAP)
	|		|		|

Parametrization:

Primitive	Parameter	Value

History:	20-6-00	BRZ	Initial

### GACI001: Set ME to full functionality state, no PIN required

Description:

<A> First command. The default class is “BG".

<B> Reset ME to full functionality state. This is used to check the reset behavior of the GACI entity.

<C> The default class is set to “CG" before.

<D> The default class is set to “CC" before.

<E> The default class is set to “CC" after reset.

ciphering indication disabled (A-E) / enabled (F)

Variants:

- <A>...<F>

Preamble:

- <A>GACI013

- <B>GACI004

- <C>GACI005a

- <D>GACI005b

<E>GACI005c
<F>GACI013

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CFUN=1)	|		|

*	==========================>	*		|

(2)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(4)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(7)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(9)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CFUN
	cmd_seq	C_CFUN_1

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NO_EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF	
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
<A>	trans_data	A_AD_FIELD_CI_DISABLED
<B>	trans_data	A_AD_FIELD_CI_DISABLED
<C>	trans_data	A_AD_FIELD_CI_DISABLED
<D>	trans_data	A_AD_FIELD_CI_DISABLED
<E>	trans_data	A_AD_FIELD_CI_DISABLED
<F>	trans_data	A_AD_FIELD_CI_ENABLED

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

01-07-2001	brz		split +CFUN and +COPS command

28-04-2001	brz		clear unecpected primitives

16-02-2000	FK		Initial

### GACI002: Automatic registration (GPRS automatic attach)

Description:

<A> ME is in full functionality state.

<B> Reset ME to full functionality state. This is used to check the reset behavior of the GACI entity.

<C> The default class is set to “CC" before.

<D> The default class is set to “CC" after reset.

Variants:

- <A>...<D>

Preamble:

- <A>GACI001a

- <B>GACI001b

- <C>GACI001d

- <D>GACI001e

APL		ACI		PS

- COMMAND (MMI CONFIG AUTO_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=0)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(5)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_COPS
	cmd_seq	C_COPS_0

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
<A>	mobile_class	GMMREG_CLASS_BG
<B>	mobile_class	GMMREG_CLASS_BG
<C>	mobile_class	GMMREG_CLASS_CC
<D>	mobile_class	GMMREG_CLASS_CC
<A>	attach_type	GMMREG_AT_COMB
<B>	attach_type	GMMREG_AT_COMB
<C>	attach_type	GMMREG_AT_IMSI
<D>	attach_type	GMMREG_AT_IMSI
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
<A>	attach_type	GMMREG_AT_COMB
<B>	attach_type	GMMREG_AT_COMB
<C>	attach_type	GMMREG_AT_IMSI
<D>	attach_type	GMMREG_AT_IMSI
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

01-07-2001	brz		split +CFUN and +COPS command

28-04-2001	brz		clear unecpected primitives

16-02-2000	brz		Initial

### GACI003: Automatic registration (GPRS manual attach)

Description:

<A> The default class is “BG".

<B> The default class is set to “CC" before.

<C> The default class is set to “CC" after reset.

Variants:

- <A>...<C>

Preamble:

- <A>GACI001a

- <B>GACI001d

- <C>GACI001e

APL		ACI		PS

- COMMAND (MMI CONFIG MAN_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=0)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(5)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_COPS
	cmd_seq	C_COPS_0

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
<A>	mobile_class	GMMREG_CLASS_BG
<B>	mobile_class	GMMREG_CLASS_CC
<C>	mobile_class	GMMREG_CLASS_CC
<A>	attach_type	GMMREG_AT_IMSI
<B>	attach_type	GMMREG_AT_IMSI
<C>	attach_type	GMMREG_AT_IMSI
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
<A>	attach_type	GMMREG_AT_IMSI
<B>	attach_type	GMMREG_AT_IMSI
<C>	attach_type	GMMREG_AT_IMSI
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

01-07-2001	brz		split +CFUN and +COPS command

28-04-2001	brz		clear unecpected primitives

16-02-2000	brz		Initial

### GACI004: Set ME to minimum functionality state

Description:

Preamble:

- GACI002a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CFUN=0)	|		|

*	==========================>	*		|

(2)	|		|	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_DETACH_REQ	|

|		*	==========================>	*

(4)	|		|	SIM_SYNC_CNF	|

|		*	<==========================	*

(5)	|		| 	GMMREG_DETACH_CNF	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CFUN
	cmd_seq	C_CFUN_0

- SIM_SYNC_REQ
	synccs	SYNC_DEACTIVATE

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_POWER_OFF

- SIM_SYNC_CNF
	cause	SYNC_DEACTIVATE

- GMMREG_DETACH_CNF
	detach_type	GMMREG_DT_POWER_OFF

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	28-09-2000	BRZ	Initial

### GACI005: change the default mobile class

Description:

First command before set ME to minimum functionality state.

Variants:

- <A>...<C>

Preamble:

<A>GACI013

<B>GACI013

<C>GACI004

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGCLASS=CG)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGCLASS_XX
<B>	cmd_len	LC_CGCLASS_XX
<C>	cmd_len	LC_CGCLASS_XX
<A>	cmd_seq	C_CGCLASS_CG
<B>	cmd_seq	C_CGCLASS_CC
<C>	cmd_seq	C_CGCLASS_CC

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

01-07-2001	brz		Initial

### GACI006: Automatic registration (GPRS automatic attach)

Description:

<A> The default class is set to “CG" before.

<B> The default class is set to “CG" before.

Variants:

- <A>...<B>

Preamble:

- <A>GACI001c

- <B>GACI001c

APL		ACI		PS

- COMMAND (MMI CONFIG AUTO_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=0)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(4)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_COPS
	cmd_seq	C_COPS_0

- GMMREG_ATTACH_REQ
<A>	mobile_class	GMMREG_CLASS_CG
<B>	mobile_class	GMMREG_CLASS_CG
<A>	attach_type	GMMREG_AT_GPRS
<B>	attach_type	GMMREG_AT_GPRS
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- GMMREG_ATTACH_CNF
<A>	attach_type	GMMREG_AT_GPRS
<B>	attach_type	GMMREG_AT_GPRS
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:

08-07-2001	brz		Indication changed (OK)

01-07-2001	brz		Initial

### GACI007: registration failed

Description:

<A> The default class is “BG".

<B> The default class is “CC".

Variants:

- <A>...<B>

Preamble:

- <A>GACI001a

- <B>GACI001d

APL		ACI		PS

- COMMAND (MMI CONFIG AUTO_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=0)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REJ	|

|		*	<==========================	*

(5)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_COPS
	cmd_seq	C_COPS_0

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
<A>	mobile_class	GMMREG_CLASS_BG
<B>	mobile_class	GMMREG_CLASS_CC
<A>	attach_type	GMMREG_AT_COMB
<B>	attach_type	GMMREG_AT_IMSI
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_REJ
	detach_type	GMMREG_DT_COMB
	cause	GMMCS_IMSI_UNKNOWN
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:

08-07-2001	brz		add variant for class CC

01-07-2001	brz		change predecessor test case

21-03-2001	ANS	Initial

### GACI008: Automatic registration (GPRS automatic attach)

Description:

Preamble:

GACI001a

APL		ACI		PS

- COMMAND (MMI CONFIG AUTO_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=0)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(5)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(7)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_COPS
	cmd_seq	C_COPS_0

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_IMSI
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:

- 07-07-2001	brz		add primitive

- 01-03-2001	brz		Initial

### GACI009: Automatic registration, but GPRS rejected

Description:

Preamble:

GACI001a

APL		ACI		PS

- COMMAND (MMI CONFIG AUTO_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=0)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(5)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(7)	|		| 	GMMREG_ATTACH_REJ	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_COPS
	cmd_seq	C_COPS_0

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_IMSI
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- GMMREG_ATTACH_REJ
	detach_type	GMMREG_DT_GPRS
	cause	GMMCS_IMSI_UNKNOWN
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:

- 08-07-2001	brz		Initial

### GACI010: Automatic registration, but GSM rejected

Description:

Preamble:

GACI001a

APL		ACI		PS

- COMMAND (MMI CONFIG AUTO_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=0)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REJ	|

|		*	<==========================	*

(5)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

(7)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_COPS
	cmd_seq	C_COPS_0

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_REJ	detach_type	GMMREG_DT_IMSI
	cause	GMMCS_IMSI_UNKNOWN
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

- GMMREG_ATTACH_CNF	attach_type	GMMREG_AT_GPRS
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:

- 08-07-2001	brz		Initial

### GACI011: Automatic registration, but combined rejected

Description:

Preamble:

GACI001a

APL		ACI		PS

- COMMAND (MMI CONFIG AUTO_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=0)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REJ	|

|		*	<==========================	*

(5)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

(7)	|		| 	GMMREG_ATTACH_REJ	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_COPS
	cmd_seq	C_COPS_0

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_REJ
	detach_type	GMMREG_DT_IMSI
	cause	GMMCS_IMSI_UNKNOWN
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

- GMMREG_ATTACH_REJ
	detach_type	GMMREG_DT_COMB
	cause	GMMCS_IMSI_UNKNOWN
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:

- 08-07-2001	brz		Initial

### GACI012: GPRS only mode: registration failed

Description:

The default class is “CG".

Preamble:

- GACI001c

APL		ACI		PS

- COMMAND (MMI CONFIG AUTO_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=0)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(4)	|		| 	GMMREG_ATTACH_REJ	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_COPS
	cmd_seq	C_COPS_0

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_CG
	attach_type	GMMREG_AT_GPRS
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- GMMREG_ATTACH_REJ
	detach_type	GMMREG_DT_COMB
	cause	GMMCS_IMSI_UNKNOWN
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:

08-07-2001	BRZ	Initial

### GACI013: UART Init

Description: MMI reset is done after in the initialization... Means, every primitive that has to be sent at the very beginning should be here defined.

.

Preamble:

GACI000

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG EXPAND_ATI_SRC_TST)

(1)	|		|	UART_PARAMETERS_REQ	|

|		*	==========================>	*

(2)	|		|	UART_PARAMETERS_CNF	|

|		*	<==========================	*

- (3)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (4)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (5)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (6)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_PARAMETERS_REQ
	device	DEVICE_1
	comPar	NOT_USED

- UART_PARAMETERS_CNF
	device	DEVICE_1

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

History:

26-08-2002	brz	Adaptation to DTI2

30-07-2001	brz	changes for MULTI INSTANCE ACI

15-02-2001	clb	Initial

### GACI014: Add second test source

Description: MMI reset is done after in the initialization... Means, every primitive that has to be sent at the very beginning should be here defined.

.

Variants:

- <A>...<B>

Preamble:

<A>GACI100a

<B>GACI400a

<C>GACI100K

<D>GACI100L

<E>GACI400c

<F>GACI400d

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG ADD_TST_SRC UART)

(1)	|		|	UART_PARAMETERS_REQ	|

|		*	==========================>	*

(2)	|		|	UART_PARAMETERS_CNF	|

|		*	<==========================	*

- (3)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (4)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (5)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (6)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_PARAMETERS_REQ
	device	DEVICE_2
	comPar	NOT_USED

- UART_PARAMETERS_CNF
	device	DEVICE_2

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID_2
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_2
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID_2
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID_2
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_2
	dlci	STANDARD_DLCI

History:

24-09-2001	clb	Initial

11-12-2002	rm	add variants C – F for query network pco

### GACI015: Set ME to full functionality state, PIN required

Description:

Preamble:

- GACI013

APL		ACI		PS

- COMMAND (MMI CONFIG AUTO_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CFUN=1)	|		|

*	==========================>	*		|

(2)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CFUN
	cmd_seq	C_CFUN_1

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_cnt	PIN_3_ATTEMPTS
	puk_cnt	PUK_10_ATTEMPTS
	pin2_cnt	PIN_3_ATTEMPTS
	puk2_cnt	PUK_10_ATTEMPTS
	ec_code	NO_EC_CODES
	pref_lang	NO_PREF_LANG

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	16-02-2000	brz	Initial

### GACI016: Enter a wrong PIN

Description:

Preamble:

- GACI015

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CPIN="1234")	|		|

*	==========================>	*		|

(2)	|		|	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(3)	|		|	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPIN
	cmd_seq	C_CPIN_WRONG

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_WRONG
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_id	PIN_1
	pin_cnt	PIN_2_ATTEMPTS
	puk_cnt	PUK_10_ATTEMPTS
	pin2_cnt	PIN_3_ATTEMPTS
	puk2_cnt	PUK_10_ATTEMPTS

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	16-02-2000	FK	Initial

### GACI017: Enter a good PIN and set automatic registration

Description:

Preamble:

- GACI016

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CPIN="4321")	|		|

*	==========================>	*		|

(2)	|		|	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(3)	|		|	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

(4)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		|	SIM_READ_CNF	|

|		*	<==========================	*

(7)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(8)	|		|	SIM_READ_CNF	|

|		*	<==========================	*

(9)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(10)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=0)	|		|

*	==========================>	*		|

(11)	|		|	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(12)	|		|	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(13)	|		|	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(14)	|		|	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(15)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CPIN
	cmd_seq	C_CPIN_IO

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	PIN_3_ATTEMPTS
	puk_cnt	PUK_10_ATTEMPTS
	pin2_cnt	PIN_3_ATTEMPTS
	puk2_cnt	PUK_10_ATTEMPTS

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF	
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_COPS
	cmd_seq	C_COPS_0

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	16-02-2000	FK	Initial

## Root functions

### GACI020: Attach mobile after it already is attached

Description:

GPRS will be attaching again. GACI will just return OK since the mobile has already been attached in preamble GACI002A

Preamble:

- GACI002A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGATT=1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGATT
	cmd_seq	C_CGATT_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

26-08-2001	JKH	removed gmm_attach_req & cnf since mobile is attached

01-03-2001	brz		change interface to GMM

06-07-2000	brz		Initial

### GACI021: detach mobile

Description:

GPRS will be detaching.

Preamble:

- GACI020

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGATT=0)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_DETACH_REQ	|

|		*	==========================>	*

- MUTE (2000)

(3)	|		| 	GMMREG_DETACH_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGATT
	cmd_seq	C_CGATT_0

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_GPRS

- GMMREG_DETACH_CNF
	detach_type	GMMREG_DT_GPRS

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	07-07-2000	brz	Initial

### GACI022: change the mobile class

Description:

<A>The protocol stack is attached and we change the mobile class.

<B>The protocol stack is attached and we change back the mobile class.

<C>The protocol stack is detached and we change the mobile class.

<D>The protocol stack is attached and we change the mobile class.

<E>The protocol stack is attached and we change the mobile class.

<F>The protocol stack is attached and we change the mobile class.

Variants:

- <A>...<F>

Preamble:

<A>GACI020

<B>GACI020

<C>GACI022a

<D>GACI021

<E>GACI020

<F>GACI020

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGCLASS=CG)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

- MUTE (2000)

(3)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGCLASS_XX
<B>	cmd_len	LC_CGCLASS_X
<C>	cmd_len	LC_CGCLASS_X
<D>	cmd_len	LC_CGCLASS_X
<E>	cmd_len	LC_CGCLASS_X
<F>	cmd_len	LC_CGCLASS_XX
<A>	cmd_seq	C_CGCLASS_CG
<B>	cmd_seq	C_CGCLASS_B
<C>	cmd_seq	C_CGCLASS_B
<D>	cmd_seq	C_CGCLASS_B
<E>	cmd_seq	C_CGCLASS_C
<F>	cmd_seq	C_CGCLASS_CC

- GMMREG_ATTACH_REQ
<A>	mobile_class	GMMREG_CLASS_CG
<B>	mobile_class	GMMREG_CLASS_BG
<C>	mobile_class	GMMREG_CLASS_BG
<D>	mobile_class	GMMREG_CLASS_BG
<E>	mobile_class	GMMREG_CLASS_CG
<F>	mobile_class	GMMREG_CLASS_CC
<A>	attach_type	GMMREG_AT_GPRS
<B>	attach_type	GMMREG_AT_COMB
<C>	attach_type	GMMREG_AT_COMB
<D>	attach_type	GMMREG_AT_COMB
<E>	attach_type	GMMREG_AT_GPRS
<F>	attach_type	GMMREG_AT_IMSI
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
<A>	attach_type	GMMREG_AT_GPRS
<B>	attach_type	GMMREG_AT_COMB
<C>	attach_type	GMMREG_AT_COMB
<D>	attach_type	GMMREG_AT_COMB
<E>	attach_type	GMMREG_AT_GPRS
<F>	attach_type	GMMREG_AT_IMSI
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

22-02-2001	brz		add Variants

20-09-2000	brz		add Variants

07-07-2000	brz		Initial

### GACI023: Command change mobile class but no changes possihle

Description:

The protocol stack is attached and the mobile class is not supported.

Preamble:

- GACI020

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGCLASS=A)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGCLASS_X
	cmd_seq	C_CGCLASS_A

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:

20-09-2000	brz	Variants

07-07-2000	brz	Initial

### GACI024: change the mobile class

Description:

The protocol stack is attached and we change the mobile class.

Preamble:

GACI002a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGCLASS=CG)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

- MUTE (2000)

(3)	|		| 	GMMREG_ATTACH_REJ	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGCLASS_XX
	cmd_seq	C_CGCLASS_CG

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_CG
	attach_type	GMMREG_AT_GPRS
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_REJ
	detach_type	GMMREG_DT_COMB
	cause	GMMCS_NET_FAIL
	search_running	GMMREG_SEARCH_NOT_RUNNING

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:

29-06-2001	brz		Initial

### GACI025: change the default mobile class

Description:

The protocol stack is off (before +CFUN=1).

Variants:

- <A>...<D>

Preamble:

GACI013

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGCLASS=CG)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGCLASS_X
<B>	cmd_len	LC_CGCLASS_X
<C>	cmd_len	LC_CGCLASS_XX
<D>	cmd_len	LC_CGCLASS_XX
<A>	cmd_seq	C_CGCLASS_B
<B>	cmd_seq	C_CGCLASS_C
<C>	cmd_seq	C_CGCLASS_CG
<D>	cmd_seq	C_CGCLASS_CC

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

01-07-2001	brz		Initial

### GACI027: CGAATT

Description:

Variants:

<A>....<D>

Preamble:

- GACI013

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGAATT=0,0)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGAATT_X
<B>	cmd_len	LC_CGAATT_X
<C>	cmd_len	LC_CGAATT_X
<D>	cmd_len	LC_CGAATT_X
<A>	cmd_seq	C_CGAATT_00
<B>	cmd_seq	C_CGAATT_01
<C>	cmd_seq	C_CGAATT_10
<D>	cmd_seq	C_CGAATT_11

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

07-05-2001	brz	Initial

### GACI030: Attach mobile

Description:

start in the state: combined detached.

Preamble:

GACI001a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGATT=1)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

- MUTE (2000)

(3)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGATT
	cmd_seq	C_CGATT_1

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_GPRS
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_GPRS
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

- 01-03-2001	brz		Initial

### GACI031: Attach mobile

Description:

start in the state: combined detached.

Preamble:

GACI001a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGATT=1)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

- MUTE (2000)

(3)	|		| 	GMMREG_ATTACH_REJ	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGATT
	cmd_seq	C_CGATT_1

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_GPRS
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_REJ
	detach_type	GMMREG_DT_COMB
	cause	GMMCS_IMSI_UNKNOWN
	search_running	GMMREG_SEARCH_NOT_RUNNING

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:

- 08-07-2001	brz		Initial

### GACI035: Attach status and test commands

Description:

<A> Test command

<B> Read command. The protocol stack is not attached.

<C> Read command. The protocol stack is attached.

<D> Read command. The protocol stack is not attached after detach command.

Variants:

- <A>....<D>

Preamble:

<A>GACI002a

<B>GACI013

<C>GACI020

<D>GACI021

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGATT?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGATT_T
<B>	cmd_len	LC_CGATT_R
<C>	cmd_len	LC_CGATT_R
<D>	cmd_len	LC_CGATT_R
<A>	cmd_seq	C_CGATT_T
<B>	cmd_seq	C_CGATT_R
<C>	cmd_seq	C_CGATT_R
<D>	cmd_seq	C_CGATT_R

- ACI_CMD_IND
<A>	cmd_len	LM_CGATT_T
<B>	cmd_len	LM_CGATT_R
<C>	cmd_len	LM_CGATT_R
<D>	cmd_len	LM_CGATT_R
<A>	cmd_seq	M_CGATT_T
<B>	cmd_seq	M_CGATT_R0
<C>	cmd_seq	M_CGATT_R1
<D>	cmd_seq	M_CGATT_R0

History:

20-09-2000	brz	remove check after automatically attach

07-08-2000	brz	Initial

### GACI036: Class status and test commands

Description:

<A> Test command

<B> Read the default parameter before attaching.

<C> Read the class after attaching with the default parameter.

<D> Read the class after changing to CG.

<E> Read the class after changing to BG.

<F> Read the class after impossible changing to A.

<G> Read the class after changing to C.

<H> Read the class after changing to CC.

Variants:

- <A>....<L>

Preamble:

<A>GACI002a

<B>GACI013

<C>GACI020

<D>GACI022a

<E>GACI022b

<F>GACI023

<G>GACI022e

<H>GACI022f

<I>GACI025a

<J>GACI025b

<K>GACI025c

<L>GACI025d

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGCLASS?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGCLASS_T
<B>	cmd_len	LC_CGCLASS_R
<C>	cmd_len	LC_CGCLASS_R
<D>	cmd_len	LC_CGCLASS_R
<E>	cmd_len	LC_CGCLASS_R
<F>	cmd_len	LC_CGCLASS_R
<G>	cmd_len	LC_CGCLASS_R
<H>	cmd_len	LC_CGCLASS_R
<I>	cmd_len	LC_CGCLASS_R
<J>	cmd_len	LC_CGCLASS_R
<K>	cmd_len	LC_CGCLASS_R
<L>	cmd_len	LC_CGCLASS_R
<A>	cmd_seq	C_CGCLASS_T
<B>	cmd_seq	C_CGCLASS_R
<C>	cmd_seq	C_CGCLASS_R
<D>	cmd_seq	C_CGCLASS_R
<E>	cmd_seq	C_CGCLASS_R
<F>	cmd_seq	C_CGCLASS_R
<G>	cmd_seq	C_CGCLASS_R
<H>	cmd_seq	C_CGCLASS_R
<I>	cmd_seq	C_CGCLASS_R
<J>	cmd_seq	C_CGCLASS_R
<K>	cmd_seq	C_CGCLASS_R
<L>	cmd_seq	C_CGCLASS_R

- ACI_CMD_IND
<A>	cmd_len	LM_CGCLASS_T
<B>	cmd_len	LM_CGCLASS_RX
<C>	cmd_len	LM_CGCLASS_RX
<D>	cmd_len	LM_CGCLASS_RXX
<E>	cmd_len	LM_CGCLASS_RX
<F>	cmd_len	LM_CGCLASS_RX
<G>	cmd_len	LM_CGCLASS_RXX
<H>	cmd_len	LM_CGCLASS_RXX
<I>	cmd_len	LM_CGCLASS_RX
<J>	cmd_len	LM_CGCLASS_RXX
<K>	cmd_len	LM_CGCLASS_RXX
<L>	cmd_len	LM_CGCLASS_RXX
<A>	cmd_seq	M_CGCLASS_T
<B>	cmd_seq	M_CGCLASS_RB
<C>	cmd_seq	M_CGCLASS_RB
<D>	cmd_seq	M_CGCLASS_RCG
<E>	cmd_seq	M_CGCLASS_RB
<F>	cmd_seq	M_CGCLASS_RB
<G>	cmd_seq	M_CGCLASS_RCG
<H>	cmd_seq	M_CGCLASS_RCC
<I>	cmd_seq	M_CGCLASS_RB
<J>	cmd_seq	M_CGCLASS_RCG
<K>	cmd_seq	M_CGCLASS_RCG
<L>	cmd_seq	M_CGCLASS_RCC

History:

22-02-2001	brz	add 2 Variants

07-08-2000	brz	Initial

### GACI037: check attach state

Description:

<A> Read command.

<B> Read command. The protocol stack is not attached.

<C> Read command. The protocol stack is attached.

<D> Read command. The protocol stack is not attached after detach command.

Variants:

- <A>....<M>

Preamble:

<A>GACI002a

<B>GACI008

<C>GACI009

<D>GACI010

<E>GACI011

<F>GACI007a

<G>GACI003a

<H>GACI012

<I>GACI030

<J>GACI007b

<K>GACI006a

<L>GACI002c

<M>GACI002c

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGATT?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGATT_R
<B>	cmd_len	LC_CGATT_R
<C>	cmd_len	LC_CGATT_R
<D>	cmd_len	LC_CGATT_R
<E>	cmd_len	LC_CGATT_R
<F>	cmd_len	LC_CGATT_R
<G>	cmd_len	LC_CGATT_R
<H>	cmd_len	LC_CGATT_R
<I>	cmd_len	LC_CGATT_R
<J>	cmd_len	LC_CGATT_R
<K>	cmd_len	LC_CGATT_R
<L>	cmd_len	LC_CGATT_R
<M>	cmd_len	LC_CGATT_R
<A>	cmd_seq	C_CGATT_R
<B>	cmd_seq	C_CGATT_R
<C>	cmd_seq	C_CGATT_R
<D>	cmd_seq	C_CGATT_R
<E>	cmd_seq	C_CGATT_R
<F>	cmd_seq	C_CGATT_R
<G>	cmd_seq	C_CGATT_R
<H>	cmd_seq	C_CGATT_R
<I>	cmd_seq	C_CGATT_R
<J>	cmd_seq	C_CGATT_R
<K>	cmd_seq	C_CGATT_R
<L>	cmd_seq	C_CGATT_R
<M>	cmd_seq	C_CGATT_R

- ACI_CMD_IND
<A>	cmd_len	LM_CGATT_R
<B>	cmd_len	LM_CGATT_R
<C>	cmd_len	LM_CGATT_R
<D>	cmd_len	LM_CGATT_R
<E>	cmd_len	LM_CGATT_R
<F>	cmd_len	LM_CGATT_R
<G>	cmd_len	LM_CGATT_R
<H>	cmd_len	LM_CGATT_R
<I>	cmd_len	LM_CGATT_R
<J>	cmd_len	LM_CGATT_R
<K>	cmd_len	LM_CGATT_R
<L>	cmd_len	LM_CGATT_R
<M>	cmd_len	LM_CGATT_R
<A>	cmd_seq	M_CGATT_R1
<B>	cmd_seq	M_CGATT_R1
<C>	cmd_seq	M_CGATT_R0
<D>	cmd_seq	M_CGATT_R1
<E>	cmd_seq	M_CGATT_R0
<F>	cmd_seq	M_CGATT_R0
<G>	cmd_seq	M_CGATT_R0
<H>	cmd_seq	M_CGATT_R0
<I>	cmd_seq	M_CGATT_R1
<J>	cmd_seq	M_CGATT_R0
<K>	cmd_seq	M_CGATT_R1
<L>	cmd_seq	M_CGATT_R0
<M>	cmd_seq	M_CGATT_R0

History:

- 08-07-2001	brz	add new variants

- 01-03-2001	brz	Initial

## Communication with GMM over GMMREG

### GACI040: Detach indication for GPRS from the network

Description:

Combined detach indication from the network.

Preamble:

- GACI020

APL		ACI		PS

|		|		|

(1)	|		| 	GMMREG_DETACH_IND	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- GMMREG_DETACH_IND
	detach_type	GMMREG_DT_COMB
	cause	GMMCS_NET_FAIL
	search_running	GMMREG_SEARCH_RUNNING

History:	13.07.2000	brz	Initial

### GACI041: Detach indication for GPRS from the network

Description:

Combined detach indication from the network.

Preamble:

- GACI020

APL		ACI		PS

|		|		|

(1)	|		| 	GMMREG_DETACH_IND	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- GMMREG_DETACH_IND
	detach_type	GMMREG_DT_COMB
	cause	GMMCS_NET_FAIL
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:	13.07.2000	brz	Initial

## PDP context definition

### GACI060: One PDP context defined

Description:

<A> One PDP context will be defined.

<B> One PDP context will be defined with ommitted parameter.

<C> One PDP context will be defined with a command list.

<D> One PDP context will be defined with a command list.

Variants:

- <A>....<D>

Preamble:

<A>GACI020

<B>GACI020

<C>GACI020

<D>GACI060c

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDCONT=…)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGDCONT_X
<B>	cmd_len	LC_CGDCONT_X_oIAP
<C>	cmd_len	LC_CGDCONT_SPECIAL
<D>	cmd_len	LC_CGDCONT_SPECIAL
<A>	cmd_seq	C_CGDCONT_1
<B>	cmd_seq	C_CGDCONT_1_oIAP
<C>	cmd_seq	C_CGDCONT_SPECIAL
<D>	cmd_seq	C_CGDCONT_SPECIAL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

01-03-2001	brz	add variants

24-02-2001	brz	add variants

01-08-2000	brz	Initial

### GACI061: Too many PDP context definitions

Description:

The last PDP context is too much.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDCONT=…)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDCONT=…)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDCONT_X
	cmd_seq	C_CGDCONT_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDCONT_X
	cmd_seq	C_CGDCONT_3

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	01.08.2000	brz	Initial

### GACI062: Clear defined PDP context

Description:

The PDP context 1 will be undefined.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDCONT=…)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDCONT_CLEAR
	cmd_seq	C_CGDCONT_1_CLEAR

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	01.08.2000	brz	Initial

### GACI063: One PDP context define

Description:

<A> One PDP context will be defined after all PDP context undefine.

<B> A second PDP context will be defined.

Variants:

- <A>....<B>

Preamble:

<A>GACI062

<B>GACI063a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDCONT=…)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGDCONT_X
<B>	cmd_len	LC_CGDCONT_X
<A>	cmd_seq	C_CGDCONT_1
<B>	cmd_seq	C_CGDCONT_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	01.08.2000	brz	Initial

### GACI075: Context status and test commands

Description:

<A> CGDCONT Test command

<B> CGDCONT Read command. No PDP context is defined.

<C> CGDCONT Read parameter from one PDP context.

<D> CGDCONT Read command after undefined all PDP contexts.

<E> CGDCONT Read parameter from the new defined PDP context after undefined all contexts.

<F> CGPADDR Test command without defined contexts

<G> CGPADDR Test command with one defined context

<H> CGPADDR Test command with two defined contexts

<I> CGPADDR Execution command with no defined context

<J> CGPADDR Execution command on defined context

<K>

<L> CGPADDR Execution command with invalid cid

Variants:

<A>....<L>

Preamble:

<A>GACI002a

<B>GACI013

<C>GACI060a

<D>GACI062

<E>GACI063a

<F>GACI020

<G>GACI060a

<H>GACI063b

<I>GACI020

<J>GACI060a

<K>GACI060c

<L>GACI060c

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDCONT=?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: ...)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGDCONT_T
<B>	cmd_len	LC_CGDCONT_R
<C>	cmd_len	LC_CGDCONT_R
<D>	cmd_len	LC_CGDCONT_R
<E>	cmd_len	LC_CGDCONT_R
<F>	cmd_len	LC_CGPADDR_T
<G>	cmd_len	LC_CGPADDR_T
<H>	cmd_len	LC_CGPADDR_T
<I>	cmd_len	LC_CGPADDR_1
<J>	cmd_len	LC_CGPADDR_1
<K>	cmd_len	LC_CGDCONT_R
<L>	cmd_len	LC_CGPADDR_1
<A>	cmd_seq	C_CGDCONT_T
<B>	cmd_seq	C_CGDCONT_R
<C>	cmd_seq	C_CGDCONT_R
<D>	cmd_seq	C_CGDCONT_R
<E>	cmd_seq	C_CGDCONT_R
<F>	cmd_seq	C_CGPADDR_T
<G>	cmd_seq	C_CGPADDR_T
<H>	cmd_seq	C_CGPADDR_T
<I>	cmd_seq	C_CGPADDR_1
<J>	cmd_seq	C_CGPADDR_1
<K>	cmd_seq	C_CGDCONT_R
<L>	cmd_seq	C_CGPADDR_100

- ACI_CMD_IND
<A>	cmd_len	LM_CGDCONT_T
<B>	cmd_len	LM_OK
<C>	cmd_len	LM_CGDCONT_R1
<D>	cmd_len	LM_OK
<E>	cmd_len	LM_CGDCONT_R1
<F>	cmd_len	LM_CGPADDR_T0
<G>	cmd_len	LM_CGPADDR_T1
<H>	cmd_len	LM_CGPADDR_T12
<I>	cmd_len	LM_CGPADDR_X_
<J>	cmd_len	LM_CGPADDR_X
<K>	cmd_len	LM_CGDCONT_SP1
<L>	cmd_len	LM_ERROR
<A>	cmd_seq	M_CGDCONT_T
<B>	cmd_seq	M_OK
<C>	cmd_seq	M_CGDCONT_R1
<D>	cmd_seq	M_OK
<E>	cmd_seq	M_CGDCONT_R1
<F>	cmd_seq	M_CGPADDR_T0
<G>	cmd_seq	M_CGPADDR_T1
<H>	cmd_seq	M_CGPADDR_T12
<I>	cmd_seq	M_CGPADDR_1_
<J>	cmd_seq	M_CGPADDR_1
<K>	cmd_seq	M_CGDCONT_SP1
<L>	cmd_seq	M_ERROR

History:

05-10-2001	brz	add a execution command with an invalid cid

12-08-2001	brz	move some variants and delete some unnecessary variants

01-03-2001	brz	add variants

24-02-2001	brz	add CGPADDR commands

07-08-2000	brz	Initial

### GACI076: Context status and test commands

Description:

<A> CGDCONT Read parameter of two PDP contexts.

<B> CGPADDR Execution command after context activation of defined context

Variants:

<A>....<B>

Preamble:

<A>GACI063b

<B>GACI060d

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDCONT=?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: ...)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(msg: ...)	|		|

*	<==========================	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGDCONT_R
<B>	cmd_len	LC_CGDCONT_R
<A>	cmd_seq	C_CGDCONT_R
<B>	cmd_seq	C_CGDCONT_R

- ACI_CMD_IND
<A>	cmd_len	LM_CGDCONT_R
<B>	cmd_len	LM_CGDCONT_SP
<A>	cmd_seq	M_CGDCONT_R1
<B>	cmd_seq	M_CGDCONT_SP1

- ACI_CMD_IND
<A>	cmd_len	LM_CGDCONT_R
<B>	cmd_len	LM_CGDCONT_SP
<A>	cmd_seq	M_CGDCONT_R2
<B>	cmd_seq	M_CGDCONT_SP2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

12-08-2001	brz	Initial

### GACI077: Context status and test commands

Description:

<A> CGPADDR Execution command with requested context 1,2

<B> CGPADDR Execution command without requested context

Variants:

- <A>....<B>

Preamble:

<A>GACI063b

<B>GACI063b

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGPADDR=)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: ...)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(msg: ...)	|		|

*	<==========================	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGPADDR_2
<B>	cmd_len	LC_CGPADDR_
<A>	cmd_seq	C_CGPADDR_2
<B>	cmd_seq	C_CGPADDR_

- ACI_CMD_IND
	cmd_len	LM_CGPADDR_X
	cmd_seq	M_CGPADDR_1

- ACI_CMD_IND
	cmd_len	LM_CGPADDR_X
	cmd_seq	M_CGPADDR_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

- 24-02-2001	brz	Initial

## PDP context modification

### GACI080: Set quality of service profile (requested)

Description:

<A> The profile will be set for the first defined PDP context.

<B> The profile will be set for the first defined PDP context after all contexts become undefined.

Variants:

<A>....<B>

Preamble:

<A>GACI060a

<B>GACI063b

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGQREQ=1,1,2,3,4,5)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGQREQ_X
	cmd_seq	C_CGQREQ_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	01.08.2000	brz	Initial

### GACI081: Set quality of service profile (minimum acceptable)

Description:

<A> The profile will be set for the first defined PDP context.

<B> The profile will be set for the first defined PDP context after all contexts become undefined.

Variants:

<A>....<B>

Preamble:

<A>GACI080a

<B>GACI080b

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGQMIN=1,5,4,3,2,1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGQMIN_X
	cmd_seq	C_CGQMIN_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	01.08.2000	brz	Initial

### GACI082: Set quality of service profile to undefined

Description:

<A> – for the requested QOS profile.

<B> – for the minimum acceptable QOS profile.

Variants:

- <A>....<B>

Preamble:

- GACI081a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGQREQ=1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGQREQ_0
<B>	cmd_len	LC_CGQMIN_0
<A>	cmd_seq	C_CGQREQ_0
<B>	cmd_seq	C_CGQMIN_0

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	01.08.2000	brz	Initial

### GACI083: Set default of quality of service profile

Description:

<A> – for the requested QOS profile.

<B> – for the minimum acceptable QOS profile.

<C> – for the requested QOS and the minimum acceptable QOS profile.

Variants:

- <A>....<C>

Preamble:

<A>GACI020

<B>GACI020

<C>GACI020

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGQREQ=1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGQREQ_DEF
<B>	cmd_len	LC_CGQMIN_DEF
<C>	cmd_len	LC_CGQCOMB_DEF
<A>	cmd_seq	C_CGQREQ_DEF_1
<B>	cmd_seq	C_CGQMIN_DEF_1
<C>	cmd_seq	C_CGQCOMB_DEF_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	11.12.2000	brz	Initial

### GACI095: Quality of service status and test commands

Description:

<A> Test command of requested QOS.

<B> Test command of minimum acceptable QOS.

<C> Read the default parameter if one PDP context is defined (requested QOS).

<D> Read the default parameter if one PDP context is defined (minimum acceptable QOS).

<E> Read the parameter if a QOS is set (requested QOS).

<F> Read the parameter if a QOS is set (minimum acceptable QOS).

<G> Read parameter after set QOS to undefined (requested QOS).

<H> Read parameter after set QOS to undefined (minimum acceptable QOS).

Variants:

- <A>....<H>

Preamble:

<A>GACI002A

<B>GACI002A

<C>GACI060a

<D>GACI060a

<E>GACI080a

<F>GACI081a

<G>GACI082a

<H>GACI082b

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGQREQ=?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: ...)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGQREQ_T
<B>	cmd_len	LC_CGQMIN_T
<C>	cmd_len	LC_CGQREQ_R
<D>	cmd_len	LC_CGQMIN_R
<E>	cmd_len	LC_CGQREQ_R
<F>	cmd_len	LC_CGQMIN_R
<G>	cmd_len	LC_CGQREQ_R
<H>	cmd_len	LC_CGQMIN_R
<A>	cmd_seq	C_CGQREQ_T
<B>	cmd_seq	C_CGQMIN_T
<C>	cmd_seq	C_CGQREQ_R
<D>	cmd_seq	C_CGQMIN_R
<E>	cmd_seq	C_CGQREQ_R
<F>	cmd_seq	C_CGQMIN_R
<G>	cmd_seq	C_CGQREQ_R
<H>	cmd_seq	C_CGQMIN_R

- ACI_CMD_IND
<A>	cmd_len	LM_CGQREQ_T
<B>	cmd_len	LM_CGQMIN_T
<C>	cmd_len	LM_CGQREQ_RX
<D>	cmd_len	LM_CGQMIN_RX
<E>	cmd_len	LM_CGQREQ_RX
<F>	cmd_len	LM_CGQMIN_RX
<G>	cmd_len	LM_CGQREQ_RX
<H>	cmd_len	LM_CGQMIN_RX
<A>	cmd_seq	M_CGQREQ_T
<B>	cmd_seq	M_CGQMIN_T
<C>	cmd_seq	M_CGQREQ_R0
<D>	cmd_seq	M_CGQMIN_R0
<E>	cmd_seq	M_CGQREQ_R1
<F>	cmd_seq	M_CGQMIN_R2
<G>	cmd_seq	M_CGQREQ_R0
<H>	cmd_seq	M_CGQMIN_R0

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

12-08-2001	brz	move variants to gaci076 and delete some unnecessary variants

07-08-2000	brz	Initial

### GACI096: Quality of service status commands

Description:

<A> No PDP context is defined (requested QOS).

<B> No PDP context is defined (minimum acceptable QOS).

Variants:

- <A>....<B>

Preamble:

<A>GACI002A

<B>GACI002A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGQREQ=?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: ...)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGQREQ_R
<B>	cmd_len	LC_CGQMIN_R
<A>	cmd_seq	C_CGQREQ_R
<B>	cmd_seq	C_CGQMIN_R

- ACI_CMD_IND
<A>	cmd_len	LM_OK
<B>	cmd_len	LM_OK
<A>	cmd_seq	M_OK
<B>	cmd_seq	M_OK

History:	12.08.2001	brz	Initial

### GACI097: Quality of service status and test commands

Description:

<A> Read parameter if two PDP contexts are defined (requested QOS).

<B> Read parameter if two PDP contexts are defined (minimum acceptable QOS).

Variants:

- <A>....<B>

Preamble:

<A>GACI080b

<B>GACI081b

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGQREQ=?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: ...)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(msg: ...)	|		|

*	<==========================	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGQREQ_R
<B>	cmd_len	LC_CGQMIN_R
<A>	cmd_seq	C_CGQREQ_R
<B>	cmd_seq	C_CGQMIN_R

- ACI_CMD_IND
<A>	cmd_len	LM_CGQREQ_RX
<B>	cmd_len	LM_CGQMIN_RX
<A>	cmd_seq	M_CGQREQ_R1
<B>	cmd_seq	M_CGQMIN_R2

- ACI_CMD_IND
<A>	cmd_len	LM_CGQREQ_RX
<B>	cmd_len	LM_CGQMIN_RX
<A>	cmd_seq	M_CGQREQ_R3
<B>	cmd_seq	M_CGQMIN_R3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	12.08.2001	brz	Initial

## PDP context activation

### GACI100: One PDP context activated

Description:

<A> No PDP context is defined.

<B> One PDP context is defined and will be activated.

<C> One PDP context is defined and will be activated. The requested quality of service profile is set.

<D> One PDP context is defined and will be activated. Both quality of service profiles are set.

<E> One PDP context is defined and will be activated. The minimum acceptable QOS profile is set and the requested quality of service profile is undefined.

<F> With +ATD and no PDP context is defined.

<G> With +ATD and no PDP context is defined.

<H> With +ATD and no PDP context is defined.

<I> With +ATD and no PDP context is defined.

<J> With +ATD and another PDP context is defined.

<K> PCO from network with DNS1, DNS2 and gateway.

<L> PCO from network with DNS1, DNS2 and gateway.

Variants:

- <A>...<L>

Preamble:

<A>GACI020

<B>GACI060a

<C>GACI080a

<D>GACI081a

<E>GACI082a

<F>GACI020

<G>GACI020

<H>GACI020

<I>GACI020

<J>GACI060a

<K>GACI020

<L>GACI020

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT&D2)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(5)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_IND	|

|		|	(UART_DISCONNECT_DTI)	|

|		*	<==========================	*

(7)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(8)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(9)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(10)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(11)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(12)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(13)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(14)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(15)	|		|	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(16)	|		|	PPP_ESTABLISH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_AND_D2
	cmd_seq	C_AND_D2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGDATA_PX
<B>	cmd_len	LC_CGDATA_PX
<C>	cmd_len	LC_CGDATA_PX
<D>	cmd_len	LC_CGDATA_PX
<E>	cmd_len	LC_CGDATA_PX
<F>	cmd_len	LC_GD_0
<G>	cmd_len	LC_GD_1
<H>	cmd_len	LC_GD_2
<I>	cmd_len	LC_GD_3
<J>	cmd_len	LC_GD_2
<K>	cmd_len	LC_CGDATA_PX
<L>	cmd_len	LC_CGDATA_PX
<A>	cmd_seq	C_CGDATA_P1
<B>	cmd_seq	C_CGDATA_P1
<C>	cmd_seq	C_CGDATA_P1
<D>	cmd_seq	C_CGDATA_P1
<E>	cmd_seq	C_CGDATA_P1
<F>	cmd_seq	C_GD_0
<G>	cmd_seq	C_GD_1
<H>	cmd_seq	C_GD_2
<I>	cmd_seq	C_GD_3
<J>	cmd_seq	C_GD_2
<K>	cmd_seq	C_CGDATA_P1
<L>	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND 
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
<A>	smreg_qos	SMREG_QOS_0
<B>	smreg_qos	SMREG_QOS_0
<C>	smreg_qos	SMREG_QOS_1
<D>	smreg_qos	SMREG_QOS_1
<E>	smreg_qos	SMREG_QOS_0
<F>	smreg_qos	SMREG_QOS_0
<G>	smreg_qos	SMREG_QOS_0
<H>	smreg_qos	SMREG_QOS_0
<I>	smreg_qos	SMREG_QOS_0
<J>	smreg_qos	SMREG_QOS_0
<K>	smreg_qos	SMREG_QOS_0
<L>	smreg_qos	SMREG_QOS_0
<A>	smreg_min_qos	SMREG_QOS_0
<B>	smreg_min_qos	SMREG_QOS_0
<C>	smreg_min_qos	SMREG_QOS_0
<D>	smreg_min_qos	SMREG_QOS_2
<E>	smreg_min_qos	SMREG_QOS_2
<F>	smreg_min_qos	SMREG_QOS_0
<G>	smreg_min_qos	SMREG_QOS_0
<H>	smreg_min_qos	SMREG_QOS_0
<I>	smreg_min_qos	SMREG_QOS_0
<J>	smreg_min_qos	SMREG_QOS_0
<K>	smreg_min_qos	SMREG_QOS_0
<L>	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
<A>	pdp_address	PDP_ADDRESS_0_S
<B>	pdp_address	PDP_ADDRESS_1
<C>	pdp_address	PDP_ADDRESS_1
<D>	pdp_address	PDP_ADDRESS_1
<E>	pdp_address	PDP_ADDRESS_1
<F>	pdp_address	PDP_ADDRESS_0_S
<G>	pdp_address	PDP_ADDRESS_0_S
<H>	pdp_address	PDP_ADDRESS_0_S
<I>	pdp_address	PDP_ADDRESS_0_S
<J>	pdp_address	PDP_ADDRESS_1
<K>	pdp_address	PDP_ADDRESS_0_S
<L>	pdp_address	PDP_ADDRESS_0_S
<A>	smreg_apn	SMREG_APN_0_S
<B>	smreg_apn	SMREG_APN_1
<C>	smreg_apn	SMREG_APN_1
<D>	smreg_apn	SMREG_APN_1
<E>	smreg_apn	SMREG_APN_1
<F>	smreg_apn	SMREG_APN_0_S
<G>	smreg_apn	SMREG_APN_0_S
<H>	smreg_apn	SMREG_APN_0_S
<I>	smreg_apn	SMREG_APN_0_S
<J>	smreg_apn	SMREG_APN_1
<K>	smreg_apn	SMREG_APN_0_S
<L>	smreg_apn	SMREG_APN_0_S
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
<A>	smreg_qos	SMREG_QOS_1
<B>	smreg_qos	SMREG_QOS_1
<C>	smreg_qos	SMREG_QOS_2
<D>	smreg_qos	SMREG_QOS_2
<E>	smreg_qos	SMREG_QOS_1
<F>	smreg_qos	SMREG_QOS_1
<G>	smreg_qos	SMREG_QOS_1
<H>	smreg_qos	SMREG_QOS_1
<I>	smreg_qos	SMREG_QOS_1
<J>	smreg_qos	SMREG_QOS_1
<K>	smreg_qos	SMREG_QOS_1
<L>	smreg_qos	SMREG_QOS_1
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
<A>	sdu	SDU_2
<B>	sdu	SDU_2
<C>	sdu	SDU_2
<D>	sdu	SDU_2
<E>	sdu	SDU_2
<F>	sdu	SDU_2
<G>	sdu	SDU_2
<H>	sdu	SDU_2
<I>	sdu	SDU_2
<J>	sdu	SDU_2
<K>	sdu	SDU_NET_DNS_1
<L>	sdu	SDU_NET_GATEW_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
<A>	sdu	SDU_2
<B>	sdu	SDU_2
<C>	sdu	SDU_2
<D>	sdu	SDU_2
<E>	sdu	SDU_2
<F>	sdu	SDU_2
<G>	sdu	SDU_2
<H>	sdu	SDU_2
<I>	sdu	SDU_2
<J>	sdu	SDU_2
<K>	sdu	SDU_NET_DNS_1
<L>	sdu	SDU_NET_GATEW_2

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	UNUSED_IN_TESTCASE
	dns1	UNUSED_IN_TESTCASE
	dns2	UNUSED_IN_TESTCASE

History:

11-12-2002	rm	add variants K/L for query network pco

30-07-2001	brz	changes for MULTI INSTANCE ACI

11-01-2001	brz	add more ATD commands

27-12-2000	brz	add ATD command

05-10-2000	brz	add UART entity

20-08-2000	brz	activation without context definition

06-08-2000	brz	add different quality of service

01-08-2000	brz	Initial

### GACI101: One failed PDP context activated by PPP

Description:

The establishment of PPP fails in step one.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

- MUTE (2000)

(8)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(9)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (10)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (11)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (12)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (13)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(14)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_NO_RESPONSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

06-08-2001	brz	changes for MULTI INSTANCE ACI

05-10-2000	brz	add UART entity

17-08-2000	brz	Initial

### GACI102: One failed PDP context activated by SM

Description:

SM can’t activate a context.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(8)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(9)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(10)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(11)	|		|	SMREG_PDP_ACTIVATE_REJ	|

|		*	<==========================	*

(12)	|		|	PPP_PDP_ACTIVATE_REJ	|

|		*	==========================>	*

- MUTE (2000)

(13)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(14)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (15)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (16)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (17)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (18)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(19)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SMREG_RC_NETWORK_FAILURE
	smreg_nsapi	SMREG_NSAPI_5

- PPP_PDP_ACTIVATE_REJ
	ppp_cause	SMREG_RC_NETWORK_FAILURE

- PPP_TERMINATE_IND
	ppp_cause	SMREG_RC_NETWORK_FAILURE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

06-08-2001	brz	changes for MULTI INSTANCE ACI

05-10-2000	brz	add UART entity

17-08-2000	brz	Initial

### GACI103: One failed PDP context activated by PPP

Description:

The establishment of PPP fails in step two.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(8)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(9)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(10)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(11)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(12)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(13)	|		|	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(14)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(15)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(16)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(17)	|		|	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

- (18)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (19)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (20)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (21)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(22)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU_2

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_NO_RESPONSE

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

06-08-2001	brz	changes for MULTI INSTANCE ACI

05-10-2000	brz	add UART entity

17-08-2000	brz	Initial

### GACI104: One failed PDP context activated by PPP

Description:

The establishment of PPP fails in step two and SM deactivated the context after activation.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(8)	|		| 	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(9)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(10)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(11)	|		| 	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(12)	|		| 	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(13)	|		| 	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(14)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(15)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(16)	|		|	SMREG_PDP_DEACTIVATE_IND	|

|		*	<==========================	*

(17)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (18)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (19)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (20)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (21)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(22)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU_2

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_NO_RESPONSE

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- SMREG_PDP_DEACTIVATE_IND	nsapi_set	NSAPI_SET_NSAPI_5

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

06-08-2001	brz	changes for MULTI INSTANCE ACI

05-10-2000	brz	add UART entity

17-08-2000	brz	Initial

### GACI105: One failed PDP context activated by PPP

Description:

PPP terminated during the context activation of SM.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(8)	|		| 	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(9)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(10)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(11)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(12)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(13)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(14)	|		| 	SMREG_PDP_DEACTIVATE_CNF	|

- |		*	<==========================	*

- (15)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (16)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (17)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (18)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(19)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_PROT_REJ_UNEXPECTED

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

06-08-2001	brz	changes for MULTI INSTANCE ACI

05-10-2000	brz	add UART entity

17-08-2000	brz	Initial

### GACI106: One failed PDP context activated by PPP

Description:

PPP terminated after context activation of SM, but before the confirm primitive is received.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(8)	|		| 	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(9)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(10)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(11)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(12)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(13)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(14)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(15)	|		|	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

- (16)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (17)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (18)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (19)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(20)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_PROT_REJ_UNEXPECTED

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

06-08-2001	brz	changes for MULTI INSTANCE ACI

05-10-2000	brz	add UART entity

17-08-2000	brz	Initial

### GACI107: One failed PDP context activated by PPP and SM

Description:

PPP terminated after a failed context activation of SM, but before the reject primitive is received.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(8)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(9)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(10)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(11)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(12)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(13)	|		|	SMREG_PDP_ACTIVATE_REJ	|

|		*	<==========================	*

- MUTE (2000)

(14)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (15)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (16)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (17)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (18)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(19)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ
	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_PROT_REJ_UNEXPECTED

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SMREG_RC_NETWORK_FAILURE
	smreg_nsapi	SMREG_NSAPI_5

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

06-08-2001	brz	changes for MULTI INSTANCE ACI

05-10-2000	brz	add UART entity

17-08-2000	brz	Initial

### GACI108: One failed PDP context activated by SM

Description:

SM deactivated the context during the step two of establishment of PPP.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(8)	|		| 	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(9)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(10)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(11)	|		| 	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(12)	|		| 	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(13)	|		| 	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(14)	|		|	SMREG_PDP_DEACTIVATE_IND	|

|		*	<==========================	*

(15)	|		|	PPP_TERMINATE_REQ	|

|		*	==========================>	*

- MUTE (2000)

(16)	|		| 	PPP_TERMINATE_IND	|

|		*	<==========================	*

(17)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (18)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (19)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (20)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (21)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(22)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU_2

- SMREG_PDP_DEACTIVATE_IND
	nsapi_set	NSAPI_SET_NSAPI_5

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

06-08-2001	brz	changes for MULTI INSTANCE ACI

05-10-2000	brz	add UART entity

17-08-2000	brz	Initial

### GACI109: One failed PDP context activated by SM

Description:

SM deactivated the context after the step two of establishment of PPP, but before the confirm primitive is received.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(8)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(9)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(10)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(11)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(12)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(13)	|		|	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(14)	|		|	SMREG_PDP_DEACTIVATE_IND	|

|		*	<==========================	*

(15)	|		|	PPP_TERMINATE_REQ	|

|		*	==========================>	*

(16)	|		|	PPP_ESTABLISH_CNF	|

|		*	<==========================	*

- MUTE (2000)

(17)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(18)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (19)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (20)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (21)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (22)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(23)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU_2

- SMREG_PDP_DEACTIVATE_IND
	nsapi_set	NSAPI_SET_NSAPI_5

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	UNUSED_IN_TESTCASE
	dns1	UNUSED_IN_TESTCASE
	dns2	UNUSED_IN_TESTCASE

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

06-08-2001	brz	changes for MULTI INSTANCE ACI

05-10-2000	brz	add UART entity

17-08-2000	brz	Initial

### GACI110: One PDP context deactivation by PPP

Description:

PPP terminated an activated context.

Preamble:

GACI100b

APL		ACI		PS

|		|		|

(1)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(2)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(4)	|		| 	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

- (5)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (6)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (7)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (8)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(9)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_PEER

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

24-09-2001	brz	return NO_CARRIER

06-08-2001	brz	changes for MULTI INSTANCE ACI

05-10-2000	brz	add UART entity

17-08-2000	brz	Initial

### GACI111: One PDP context deactivation by PPP and SM

Description:

PPP and SM terminated an activated context in the same time. In first the PPP terminate primitive is received.

Preamble:

GACI100b

APL		ACI		PS

|		|		|

(1)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(2)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

- MUTE (2000)

(3)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(4)	|		|	SMREG_PDP_DEACTIVATE_IND	|

|		*	<==========================	*

- (5)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (6)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (7)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (8)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(9)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_PEER

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- SMREG_PDP_DEACTIVATE_IND
	nsapi_set	NSAPI_SET_NSAPI_5

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

24-09-2001	brz	return NO_CARRIER

06-08-2001	brz	changes for MULTI INSTANCE ACI

05-10-2000	brz	add UART entity

17-08-2000	brz	Initial

### GACI112: One PDP context deactivation by SM

Description:

SM deactivated an activated context.

Preamble:

GACI100b

APL		ACI		PS

|		|		|

(1)	|		|	SMREG_PDP_DEACTIVATE_IND	|

|		*	<==========================	*

(2)	|		|	PPP_TERMINATE_REQ	|

|		*	==========================>	*

- MUTE (2000)

(3)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (5)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (6)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (7)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (8)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(9)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_DEACTIVATE_IND
	nsapi_set	NSAPI_SET_NSAPI_5

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

24-09-2001	brz	return NO_CARRIER

06-08-2001	brz	changes for MULTI INSTANCE ACI

05-10-2000	brz	add UART entity

17-08-2000	brz	Initial

### GACI113: Start a PDP context without specified use

Description:

It is possible to activate a PDP context without an application (eg WAP)

Variants:

<A>...<D>

Preamble:

<A>GACI020

<B>GACI114b

<C>GACI020

<D>GACI020

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGACT=1)	|		|

*	==========================>	*		|

(2)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGACT_X
	cmd_seq	C_CGACT_11

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_0_S
	smreg_apn	SMREG_APN_0_S
<A>	dti_linkid	SNDCP_NULL_DTI_ID
<B>	dti_linkid	SNDCP_NULL_DTI_ID_2
<C>	dti_linkid	SNDCP_NULL_DTI_ID
<D>	dti_linkid	SNDCP_NULL_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_NEIGHBOR
<A>	sdu	SDU_CTX_REQ_DEF
<B>	sdu	SDU
<C>	sdu	SDU_CTX_REQ_DEF
<D>	sdu	SDU_CTX_REQ_DEF

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_1
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
<A>	sdu	SDU_2
<B>	sdu	SDU_3
<C>	sdu	SDU_NET_DNS_1
<D>	sdu	SDU_NET_GATEW_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

11-12-2002	rm	add variants C/D with additional network pco’s

09-10-2001	brz	add context activation primitives

12-08-2001	brz	Initial

### GACI114: PDP context deactivation by CGACT=0,…

Description:

Variants:

<A>...<C>

Preamble:

<A>GACI014a

<B>GACI014b

<C>GACI014a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGACT=0)	|		|

*	==========================>	*		|

(2)	|		|	PPP_TERMINATE_REQ	|

|		*	==========================>	*

(3)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(4)	|		|	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

(5)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(7)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (8)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (9)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (10)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (11)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(12)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	NUM_2
<A>	cmd_len	LC_CGACT_X
<B>	cmd_len	LC_CGACT_X
<C>	cmd_len	LC_CGACT_0
<A>	cmd_seq	C_CGACT_10
<B>	cmd_seq	C_CGACT_10
<C>	cmd_seq	C_CGACT_0

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

26-09-2001	brz	Initial

### GACI115: PDP context deactivation by CGACT=0,…

Description:

Preamble:

GACI014a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGACT=0)	|		|

*	==========================>	*		|

(2)	|		|	PPP_TERMINATE_REQ	|

|		*	==========================>	*

(3)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(4)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(5)	|		|	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(7)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (8)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (9)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (10)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (11)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(12)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	NUM_2
	cmd_len	LC_CGACT_X
	cmd_seq	C_CGACT_10

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

26-09-2001	brz	Initial

### GACI117: One PDP context activated

Description:

No PDP context is defined.

Preamble:

- GACI021

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(5)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(7)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(8)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(9)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(10)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(11)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(12)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(13)	|		| 	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(14)	|		| 	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(15)	|		| 	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(16)	|		| 	PPP_ESTABLISH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_0_S
	smreg_apn	SMREG_APN_0_S
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_1
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU_2

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	UNUSED_IN_TESTCASE
	dns1	UNUSED_IN_TESTCASE
	dns2	UNUSED_IN_TESTCASE

History:

30-07-2001	brz	changes for MULTI INSTANCE ACI

- 07-05-2001	brz	Initial

### GACI119: Context activation status and test commands

Description:

<A> Test command CGACT.

<B> Read command without defined contexts.

<C> Read command with one defined but not activated context.

<D> Read command with one defined and activated context.

<E> Read command with two defined and one activated context.

Variants:

- <A>....<E>

Preamble:

<A>GACI002A

<B>GACI002A

<C>GACI060a

<D>GACI111

<E>GACI108

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGACT=?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: ...)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGACT_T
<B>	cmd_len	LC_CGACT_R
<C>	cmd_len	LC_CGACT_R
<D>	cmd_len	LC_CGACT_R
<E>	cmd_len	LC_CGACT_R
<A>	cmd_seq	C_CGACT_T
<B>	cmd_seq	C_CGACT_R
<C>	cmd_seq	C_CGACT_R
<D>	cmd_seq	C_CGACT_R
<E>	cmd_seq	C_CGACT_R

- ACI_CMD_IND
<A>	cmd_len	LM_CGACT_T
<B>	cmd_len	LM_OK
<C>	cmd_len	LM_CGACT_R1
<D>	cmd_len	LM_CGACT_R1
<E>	cmd_len	LM_CGACT_R1
<A>	cmd_seq	M_CGACT_T
<B>	cmd_seq	M_OK
<C>	cmd_seq	M_CGACT_R10
<D>	cmd_seq	M_CGACT_R10
<E>	cmd_seq	M_CGACT_R10
History:	07.08.2000	brz	Initial

## Automatic detach after last context deactivation

### GACI130: Detach after one PDP context deactivation by SM

Description:

SM deactivated an activated context.

Preamble:

- GACI100a

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_DETACH)

(1)	|		|	SMREG_PDP_DEACTIVATE_IND	|

|		*	<==========================	*

(2)	|		|	PPP_TERMINATE_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_DETACH_REQ	|

|		*	==========================>	*

(4)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(5)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (6)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (7)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (8)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (9)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(10)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

(11)	|		| 	GMMREG_DETACH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_DEACTIVATE_IND
	nsapi_set	NSAPI_SET_NSAPI_5

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_GPRS

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

- GMMREG_DETACH_CNF
	detach_type	GMMREG_DT_GPRS

History:

24-09-2001 	brz	return NO_CARRIER

12-08-2001	brz	Initial

### GACI131: Detach after one PDP context deactivation by PPP

Description:

PPP deactivated an activated context.

Preamble:

- GACI100a

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_DETACH)

(1)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(2)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(6)	|		|	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

- (4)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (5)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

(7)	|		|	GMMREG_DETACH_REQ	|

|		*	==========================>	*

- (8)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (9)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(10)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

(11)	|		|	GMMREG_DETACH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_PEER

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_GPRS

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

- GMMREG_DETACH_CNF
	detach_type	GMMREG_DT_GPRS

History:

26-12-2001	brz	Initial

### GACI132: Automatic detach after failed PDP context activated by PPP

Description:

The establishment of PPP fails in step one.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_DETACH)

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

- MUTE (2000)

(8)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(9)	|		|	GMMREG_DETACH_REQ	|

|		*	==========================>	*

(10)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (11)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (12)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (13)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (14)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(15)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

(16)	|		| 	GMMREG_DETACH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_NO_RESPONSE

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_GPRS

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

- GMMREG_DETACH_CNF
	detach_type	GMMREG_DT_GPRS

History:

26-12-2001	brz	Initial

### GACI133: Automatic detach after failed PDP context activated by SM

Description:

SM can’t activate a context.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_DETACH)

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(8)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(9)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(10)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(11)	|		|	SMREG_PDP_ACTIVATE_REJ	|

|		*	<==========================	*

(12)	|		|	GMMREG_DETACH_REQ	|

|		*	==========================>	*

(13)	|		|	PPP_PDP_ACTIVATE_REJ	|

|		*	==========================>	*

- MUTE (2000)

(14)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(15)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (16)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (17)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (18)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (19)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(20)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

(21)	|		|	GMMREG_DETACH_CNF	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SMREG_RC_NETWORK_FAILURE
	smreg_nsapi	SMREG_NSAPI_5

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_GPRS

- PPP_PDP_ACTIVATE_REJ
	ppp_cause	SMREG_RC_NETWORK_FAILURE

- PPP_TERMINATE_IND
	ppp_cause	SMREG_RC_NETWORK_FAILURE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

- GMMREG_DETACH_CNF
	detach_type	GMMREG_DT_GPRS

History:

26-12-2001	brz	Initial

### GACI134: Automatic detach after aborting +CGDATA during GPRS attachment

Description:

The attachment will be aborted.

Preamble:

GACI021

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_DETACH)

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|		|	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(3)	|	ACI_ABORT_REQ	|		|

*	==========================>	*		|

(4)	|		|	GMMREG_DETACH_REQ	|

|		*	==========================>	*

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

MUTE (2000)

(6)	|		|	GMMREG_DETACH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- ACI_ABORT_REQ
	cmd_src	CMD_SRC_EXT
	cause	ABT_ABORT_CMD

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_GPRS

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- GMMREG_DETACH_CNF
	detach_type	GMMREG_DT_GPRS

History:

08-08-2002	brz	Initial

### GACI135: Automatic detach after failed PDP context activated by SM

Description:

SM can’t activate a context.

Preamble:

GACI060a

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_DETACH)

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(8)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(9)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(10)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(11)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(12)	|		|	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(13)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(14)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(15)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(16)	|		|	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

- (17)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (18)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

(19)	|		|	GMMREG_DETACH_REQ	|

|		*	==========================>	*

- (20)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

- MUTE (2000)

- (21)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

(22)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

(23)	|		|	GMMREG_DETACH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_3
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU_2

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_PROT_REJ_UNEXPECTED

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_GPRS

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

- GMMREG_DETACH_CNF
	detach_type	GMMREG_DT_GPRS

History:

08-08-2002	brz	Initial

## network requested context activation

### GACI140: Set CGAUTO mode

Description:

Variants:

<A>...<C>

Preamble:

<A>GACI020

<B>GACI020

<C>GACI020

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGAUTO=…)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGAUTO_X
<A>	cmd_seq	C_CGAUTO_0
<B>	cmd_seq	C_CGAUTO_2
<C>	cmd_seq	C_CGAUTO_3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	01.08.2000	brz	Initial

### GACI141: modem compatibility mode, GPRS and circuit switched calls (default)

Description:

Manuel rejection of a network request for PDP context activation

Variants:

<A>...<D>

Preamble:

- <A>GACI020

- <B>GACI020

- <C>GACI141a

- <D>GACI141b

APL		ACI		PS

|		|		|

(1)	|		| 	SMREG_PDP_ACTIVATE_IND	|

|		*	<==========================	*

- TIMEOUT (2000)

(2)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

- TIMEOUT (2000)

(3)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

- TIMEOUT (2000)

(4)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_REQ	|		|

|	(cmd: ATH)	|		|

*	==========================>	*		|

(6)	|		| 	SMREG_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_IND
	smreg_qos	SMREG_QOS_0
	smreg_ti	NUM_8
	smreg_apn	SMREG_APN_1
	pdp_type	IP_V_4
	pdp_address	PDP_ADDRESS_1

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_H
<B>	cmd_len	LC_CGANS_X
<C>	cmd_len	LC_H
<D>	cmd_len	LC_CGANS_X
<A>	cmd_seq	C_H
<B>	cmd_seq	C_CGANS_0
<C>	cmd_seq	C_H
<D>	cmd_seq	C_CGANS_0

- SMREG_PDP_ACTIVATE_RES
	smreg_ti	NUM_8
	smreg_cause	SMREG_RC_ACT_REJ_UNSPEC

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

- 27-03-2002	brz	2 variants added

- 01-09-2000	brz	Initial

### GACI142: modem compatibility mode, GPRS and circuit switched calls (default)

Description:

Manuel acceptance of a network request for PDP context activation

Variants:

<A>...<D>

Preamble:

<A>GACI060c

<B>GACI060c

<C>GACI060c

<D>GACI060c

APL		ACI		PS

|		|		|

(1)	|		| 	SMREG_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(2)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

- TIMEOUT (2000)

(3)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

- TIMEOUT (2000)

(4)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_REQ	|		|

|	(cmd: ATA)	|		|

*	==========================>	*		|

(6)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(7)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(8)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(9)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(10)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(11)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(12)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(13)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(14)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(15)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(16)	|		| 	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(17)	|		| 	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(18)	|		| 	PPP_ESTABLISH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_IND
	smreg_qos	SMREG_QOS_1
	smreg_ti	NUM_8
	smreg_apn	SMREG_APN_1
	pdp_type	IP_V_4
	pdp_address	PDP_ADDRESS_1

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_A
<B>	cmd_len	LC_CGANS_X
<C>	cmd_len	LC_CGANS_X2
<D>	cmd_len	LC_GD_0
<A>	cmd_seq	C_A
<B>	cmd_seq	C_CGANS_1
<C>	cmd_seq	C_CGANS_2X
<D>	cmd_seq	C_GD_0

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_3
	smreg_min_qos	SMREG_QOS_4
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_8
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_3
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU_2

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	UNUSED_IN_TESTCASE
	dns1	UNUSED_IN_TESTCASE
	dns2	UNUSED_IN_TESTCASE

History:

30-07-2001	brz	changes for MULTI INSTANCE ACI

30-04-2001	brz	change some parameter

24-02-2001	brz	add variants with CGANS

18-01-2001	brz	add context activation

01-10-2000	brz	Initial

### GACI143: Automatic acceptance of a network request for PDP context activation __FAILED!!!

Description:

Manuel acceptance of a network request for PDP context activation

Preamble:

- GACI144

APL		ACI		PS

|		|		|

(1)	|		| 	SMREG_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(2)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(4)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(5)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(6)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(7)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(8)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(9)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(10)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(11)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(12)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(13)	|		| 	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(14)	|		| 	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(15)	|		| 	PPP_ESTABLISH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_IND
	smreg_qos	SMREG_QOS_1
	smreg_ti	NUM_8
	smreg_apn	SMREG_APN_1
	pdp_type	IP_V_4
	pdp_address	PDP_ADDRESS_1

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_8
	pdp_address	PDP_ADDRESS_0_S
	smreg_apn	SMREG_APN_0_S
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_3
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU_2

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	UNUSED_IN_TESTCASE
	dns1	UNUSED_IN_TESTCASE
	dns2	UNUSED_IN_TESTCASE

History:

30-07-2001	brz	changes for MULTI INSTANCE ACI

18-01-2001	brz	Initial

### GACI144: set CGAUTO mode = 1 and attach

Description:

Preamble:

GACI021

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGAUTO=1)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGAUTO_X
	cmd_seq	C_CGAUTO_1

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	02.03.2001	brz	Initial

### GACI145: Manuel acceptance of a network request for PDP context activation

Description:

Manuel acceptance of a network request for PDP context activation

Preamble:

GACI100a

APL		ACI		PS

|		|		|

(1)	|		| 	SMREG_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(2)	|		|	SMREG_PDP_ACTIVATE_RES	|

|		*	==========================>	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_IND
	smreg_qos	SMREG_QOS_1
	smreg_ti	NUM_8
	smreg_apn	SMREG_APN_1
	pdp_type	IP_V_4
	pdp_address	PDP_ADDRESS_1

- SMREG_PDP_ACTIVATE_RES
	smreg_ti	NUM_8
	smreg_cause	SMREG_RC_INSUF_RES

History:

- 07-05-2001	brz	Initial

### GACI146: context reactivation

Description:

Manuel acceptance of a network request for PDP context activation

Preamble:

GACI142a

APL		ACI		PS

|		|		|

(1)	|		|	SMREG_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(2)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	PPP_TERMINATE_REQ	|

|		*	==========================>	*

- MUTE (2000)

(4)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(5)	|		|	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

(6)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (7)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (8)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (9)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (10)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(11)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

(12)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

- TIMEOUT (2000)

(13)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

- TIMEOUT (2000)

(14)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

(15)	|	ACI_CMD_REQ	|		|

|	(cmd: ATA)	|		|

*	==========================>	*		|

(16)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(17)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(18)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(19)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(20)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(21)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(22)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(23)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(24)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(25)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(26)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(27)	|		|	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(28)	|		|	PPP_ESTABLISH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_IND
	smreg_qos	SMREG_QOS_1
	smreg_ti	NUM_8
	smreg_apn	SMREG_APN_1
	pdp_type	IP_V_4
	pdp_address	PDP_ADDRESS_1

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_LOCAL

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_A
	cmd_seq	C_A

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_3
	smreg_min_qos	SMREG_QOS_4
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_8
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_3
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU_2

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	UNUSED_IN_TESTCASE
	dns1	UNUSED_IN_TESTCASE
	dns2	UNUSED_IN_TESTCASE

History:

24-09-2001 	brz	return NO_CARRIER

30-07-2001	brz	changes for MULTI INSTANCE ACI

- 07-05-2001	brz	Initial

### GACI147: context reactivation

Description:

Preamble:

GACI142a

APL		ACI		PS

|		|		|

(1)	|		|	SMREG_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(2)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	PPP_TERMINATE_REQ	|

|		*	==========================>	*

- MUTE (2000)

(4)	|		|	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

(5)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(6)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (7)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (8)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (9)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (10)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(11)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

(12)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

- TIMEOUT (2000)

(13)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

TIMEOUT (2000)

(14)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

(15)	|	ACI_CMD_REQ	|		|

|	(cmd: ATA)	|		|

*	==========================>	*		|

(16)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(17)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(18)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(19)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(20)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(21)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(22)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(23)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(24)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(25)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(26)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(27)	|		|	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(28)	|		|	PPP_ESTABLISH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_IND
	smreg_qos	SMREG_QOS_1
	smreg_ti	NUM_8
	smreg_apn	SMREG_APN_1
	pdp_type	IP_V_4
	pdp_address	PDP_ADDRESS_1

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_LOCAL

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_A
	cmd_seq	C_A

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_3
	smreg_min_qos	SMREG_QOS_4
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_8
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_3
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU_2

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	UNUSED_IN_TESTCASE
	dns1	UNUSED_IN_TESTCASE
	dns2	UNUSED_IN_TESTCASE

History:

24-09-2001 	brz	retrun NO_CARRIER

30-07-2001	brz	changes for MULTI INSTANCE ACI

- 14-05-2001	brz	Initial

### GACI148: context reactivation aborted

Description:

Preamble:

GACI142a

APL		ACI		PS

|		|		|

(1)	|		|	SMREG_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(2)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	PPP_TERMINATE_REQ	|

|		*	==========================>	*

- MUTE (2000)

(4)	|		|	SMREG_PDP_DEACTIVATE_IND	|

|		*	<==========================	*

(5)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(6)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (7)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (8)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (9)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (10)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(11)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_IND
	smreg_qos	SMREG_QOS_1
	smreg_ti	NUM_8
	smreg_apn	SMREG_APN_1
	pdp_type	IP_V_4
	pdp_address	PDP_ADDRESS_1

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_LOCAL

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- SMREG_PDP_DEACTIVATE_IND
	nsapi_set	NSAPI_SET_NSAPI_5

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

24-09-2001	brz	retrun NO_CARRIER

13-09-2001	brz	change behavior after deactivate_ind

30-07-2001	brz	changes for MULTI INSTANCE ACI

- 15-05-2001	brz	Initial

### GACI149: context reactivation aborted

Description:

Preamble:

GACI142a

APL		ACI		PS

|		|		|

(1)	|		|	SMREG_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(2)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	PPP_TERMINATE_REQ	|

|		*	==========================>	*

- MUTE (2000)

(4)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(5)	|		|	SMREG_PDP_DEACTIVATE_IND	|

|		*	<==========================	*

(6)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (7)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (8)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (9)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (10)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(11)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_IND
	smreg_qos	SMREG_QOS_1
	smreg_ti	NUM_8
	smreg_apn	SMREG_APN_1
	pdp_type	IP_V_4
	pdp_address	PDP_ADDRESS_1

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_LOCAL

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

- SMREG_PDP_DEACTIVATE_IND
	nsapi_set	NSAPI_SET_NSAPI_5

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

24-09-2001	brz	retrun NO_CARRIER

13-09-2001	brz	change behavior after deactivate_ind

30-07-2001	brz	changes for MULTI INSTANCE ACI

- 15-05-2001	brz	Initial

### GACI150: context reactivation

Description:

Manuel acceptance of a network request for PDP context activation

Preamble:

GACI142a

APL		ACI		PS

|		|		|

(1)	|		|	SMREG_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(2)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	PPP_TERMINATE_REQ	|

|		*	==========================>	*

- MUTE (2000)

(4)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(5)	|		|	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

(6)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- (7)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (8)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (9)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (10)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(11)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

(12)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

(13)	|	ACI_CMD_REQ	|		|

|	(cmd: ATZ)	|		|

*	==========================>	*		|

(14)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

TIMEOUT (2000)

(15)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

TIMEOUT (2000)

(16)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

(17)	|	ACI_CMD_REQ	|		|

|	(cmd: ATA)	|		|

*	==========================>	*		|

(18)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(19)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(20)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(21)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(22)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(23)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(24)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(25)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(26)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(27)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(28)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(29)	|		|	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(30)	|		|	PPP_ESTABLISH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_IND
	smreg_qos	SMREG_QOS_1
	smreg_ti	NUM_8
	smreg_apn	SMREG_APN_1
	pdp_type	IP_V_4
	pdp_address	PDP_ADDRESS_1

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_LOCAL

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_Z
	cmd_seq	C_Z

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_IND
	cmd_len	LM_RING
	cmd_seq	M_RING

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_A
	cmd_seq	C_A

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_3
	smreg_min_qos	SMREG_QOS_4
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_8
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_3
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU_2

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	UNUSED_IN_TESTCASE
	dns1	UNUSED_IN_TESTCASE
	dns2	UNUSED_IN_TESTCASE

History:

- 15-04-2002	brz	Initial

### GACI155: CGAUTO & CGANS status and test commands

Description:

<A> CGAUTO Test command

<B> CGAUTO Read command. (n=0)

<C> CGAUTO Read command. (n=1)

<D> CGAUTO Read command. (n=2)

<E> CGAUTO Read command. (n=3)

<F> CGANS TEST command.

Variants:

- <A>....<F>

Preamble:

<A>GACI020

<B>GACI140a

<C>GACI144

<D>GACI140b

<E>GACI140c

<F>GACI020

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGAUTO?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGAUTO_T
<B>	cmd_len	LC_CGAUTO_R
<C>	cmd_len	LC_CGAUTO_R
<D>	cmd_len	LC_CGAUTO_R
<E>	cmd_len	LC_CGAUTO_R
<F>	cmd_len	LC_CGANS_T
<A>	cmd_seq	C_CGAUTO_T
<B>	cmd_seq	C_CGAUTO_R
<C>	cmd_seq	C_CGAUTO_R
<D>	cmd_seq	C_CGAUTO_R
<E>	cmd_seq	C_CGAUTO_R
<F>	cmd_seq	C_CGANS_T

- ACI_CMD_IND
<A>	cmd_len	LM_CGAUTO_T
<B>	cmd_len	LM_CGAUTO_R
<C>	cmd_len	LM_CGAUTO_R
<D>	cmd_len	LM_CGAUTO_R
<E>	cmd_len	LM_CGAUTO_R
<F>	cmd_len	LM_CGANS_T
<A>	cmd_seq	M_CGAUTO_T
<B>	cmd_seq	M_CGAUTO_R0
<C>	cmd_seq	M_CGAUTO_R1
<D>	cmd_seq	M_CGAUTO_R2
<E>	cmd_seq	M_CGAUTO_R3
<F>	cmd_seq	M_CGANS_T

History:

24-02-2001	brz	Initial

## Other GPRS commands

### GACI175: Select servive for mobile originated SMS massages

Description:

<A> Set servive to GPRS

<B> Set servive to circuit switched

<C> Set servive to GPRS preferred (use circuit switched if GPRS not avalable)

<D> Set servive to circuit switched preferred (use gprs if circuit switched not avalable)

Variants:

<A>...<D>

Preamble:

<A>GACI020

<B>GACI020

<C>GACI020

<D>GACI175a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGSMS=…)	|		|

*	==========================>	*		|

(2)	|		| 	MNSMS_MO_SERV_REQ	|

|		*	==========================>	*

- MUTE (2000)

(3)	|		| 	MNSMS_MO_SERV_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGSMS_X
<A>	cmd_seq	C_CGSMS_0
<B>	cmd_seq	C_CGSMS_1
<C>	cmd_seq	C_CGSMS_2
<D>	cmd_seq	C_CGSMS_3

- MNSMS_MO_SERV_REQ
<A>	mo_sms_serv	GPRS_SMS_GPRS_ONLY
<B>	mo_sms_serv	GPRS_SMS_CCT_ONLY
<C>	mo_sms_serv	GPRS_SMS_GPRS_PREF
<D>	mo_sms_serv	GPRS_SMS_CCT_PREF

- MNSMS_MO_SERV_CNF
<A>	mo_sms_serv	GPRS_SMS_GPRS_ONLY
<B>	mo_sms_serv	GPRS_SMS_CCT_ONLY
<C>	mo_sms_serv	GPRS_SMS_GPRS_PREF
<D>	mo_sms_serv	GPRS_SMS_CCT_PREF

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	21.02.2001	brz	Initial

### GACI176: SMS servive status and test commands

Description:

<A> Test command

<B> Read command. (service=0)

<C> Read command. (service=1)

<D> Read command. (service=2)

<E> Read command. (service=3)

Variants:

- <A>...<E>

Preamble:

<A>GACI020

<B>GACI175a

<C>GACI175b

<D>GACI175c

<E>GACI175d

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGSMS=…)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: +CGSMS=…)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGSMS_T
<B>	cmd_len	LC_CGSMS_R
<C>	cmd_len	LC_CGSMS_R
<D>	cmd_len	LC_CGSMS_R
<E>	cmd_len	LC_CGSMS_R
<A>	cmd_seq	C_CGSMS_T
<B>	cmd_seq	C_CGSMS_R
<C>	cmd_seq	C_CGSMS_R
<D>	cmd_seq	C_CGSMS_R
<E>	cmd_seq	C_CGSMS_R

- ACI_CMD_IND
<A>	cmd_len	LM_CGSMS_T
<B>	cmd_len	LM_CGSMS_RX
<C>	cmd_len	LM_CGSMS_RX
<D>	cmd_len	LM_CGSMS_RX
<E>	cmd_len	LM_CGSMS_RX
<A>	cmd_seq	M_CGSMS_T
<B>	cmd_seq	M_CGSMS_R0
<C>	cmd_seq	M_CGSMS_R1
<D>	cmd_seq	M_CGSMS_R2
<E>	cmd_seq	M_CGSMS_R3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

08-10-2001	brz	add OK indication

21-02-2001	brz	Initial

## Unsolicited reporting

### GACI180: Control the sending of unsolicited GPRS event reporting

Description:

<A>

<B>

<C>

Variants:

- <A>...<D>

Preamble:

- GACI020

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGEREP=…)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGEREP_X
<B>	cmd_len	LC_CGEREP_X
<C>	cmd_len	LC_CGEREP_XX
<D>	cmd_len	LC_CGEREP_XX
<A>	cmd_seq	C_CGEREP_0
<B>	cmd_seq	C_CGEREP_1
<C>	cmd_seq	C_CGEREP_10
<D>	cmd_seq	C_CGEREP_11

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	21.02.2001	brz	Initial

### GACI181: Control the sending and presentation of unsolicited GPRS network registration status

Description:

<A> Disable network registration unsolicited result code

<B> enable network registration unsolicited result code +CGREG: <stat>

<C> enable network registration and location information unsolicited result code +CGREG: <stat>[,<lac>,<ci>]

Variants:

- <A>...<B>

Preamble:

- GACI020

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGREG=…)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGREG_X
<A>	cmd_seq	C_CGREG_0
<B>	cmd_seq	C_CGREG_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	21.02.2001	brz	Initial

## Root functions 2

### GACI200: Setting the Error Response

Description:

The error response is set to numeric.

Preamble:

- GACI013

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CMEE=1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CMEE_1
	cmd_seq	C_CMEE_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

15-02-2000	FK	Initial

### GACI220: Setting the GPRS network registration status

Description:

Variants:

- <A>...<C>

Preamble:

- <A> GACI220b

- <B> GACI020

- <C> GACI020

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGREG=)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGREG_X
<A>	cmd_seq	C_CGREG_0
<B>	cmd_seq	C_CGREG_1
<C>	cmd_seq	C_CGREG_2

- ACI_CMD_IND
<A>	cmd_len	LM_OK
<B>	cmd_len	LM_OK
<C>	cmd_len	LM_OK
<A>	cmd_seq	M_OK
<B>	cmd_seq	M_OK
<C>	cmd_seq	M_OK

History:

12-08-2001	BRZ	Initial

## More register test cases

### GACI230: Manuel registration (GPRS automatic attach)

Description:

<A> ME is in full functionality state.

<B> Reset ME to full functionality state. This is used to check the reset behavior of the GACI entity.

<C> The default class is set to “CC" before.

<D> The default class is set to “CC" after reset.

Variants:

- <A>...<E>

Preamble:

- <A>GACI001a

- <B>GACI001b

- <C>GACI001d

- <D>GACI001e

- <E>GACI003a

APL		ACI		PS

- COMMAND (MMI CONFIG AUTO_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=1,1,...)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_PLMN_RES	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(5)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_COPS_MAN_NUM
	cmd_seq	C_PLUS_COPS_MAN_NUM

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_MAN

- GMMREG_PLMN_RES
	plmn	PLMN_262_02
<A>	mobile_class	GMMREG_CLASS_BG
<B>	mobile_class	GMMREG_CLASS_BG
<C>	mobile_class	GMMREG_CLASS_CC
<D>	mobile_class	GMMREG_CLASS_CC
<E>	mobile_class	GMMREG_CLASS_BG
<A>	attach_type	GMMREG_AT_COMB
<B>	attach_type	GMMREG_AT_COMB
<C>	attach_type	GMMREG_AT_IMSI
<D>	attach_type	GMMREG_AT_IMSI
<E>	attach_type	GMMREG_AT_COMB

- GMMREG_ATTACH_CNF
<A>	attach_type	GMMREG_AT_COMB
<B>	attach_type	GMMREG_AT_COMB
<C>	attach_type	GMMREG_AT_IMSI
<D>	attach_type	GMMREG_AT_IMSI
<E>	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_MAN

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

19-09-2001	brz		Initial

### GACI231: Manuel registration (GPRS manuel attach)

Description:

<A> ME is in full functionality state.

<B> Reset ME to full functionality state. This is used to check the reset behavior of the GACI entity.

<C> The default class is set to “CC" before.

<D> The default class is set to “CC" after reset.

Variants:

- <A>...<E>

Preamble:

- <A>GACI001a

- <B>GACI001b

- <C>GACI001d

- <D>GACI001e

- <E>GACI003a

APL		ACI		PS

- COMMAND (MMI CONFIG MAN_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=1,1,...)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_PLMN_RES	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(5)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_COPS_MAN_NUM
	cmd_seq	C_PLUS_COPS_MAN_NUM

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_MAN

- GMMREG_PLMN_RES
	plmn	PLMN_262_02
<A>	mobile_class	GMMREG_CLASS_BG
<B>	mobile_class	GMMREG_CLASS_BG
<C>	mobile_class	GMMREG_CLASS_CC
<D>	mobile_class	GMMREG_CLASS_CC
<E>	mobile_class	GMMREG_CLASS_BG
<A>	attach_type	GMMREG_AT_IMSI
<B>	attach_type	GMMREG_AT_IMSI
<C>	attach_type	GMMREG_AT_IMSI
<D>	attach_type	GMMREG_AT_IMSI
<E>	attach_type	GMMREG_AT_IMSI

- GMMREG_ATTACH_CNF
<A>	attach_type	GMMREG_AT_IMSI
<B>	attach_type	GMMREG_AT_IMSI
<C>	attach_type	GMMREG_AT_IMSI
<D>	attach_type	GMMREG_AT_IMSI
<E>	attach_type	GMMREG_AT_IMSI
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_MAN

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

19-09-2001	brz		Initial

### GACI232: Net Search (failed: mistake in testcase)

Description:

Scan for available PLMN's

Preamble:

GACI002a

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG MAN_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=?)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_NET_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_PLMN_IND	|

|		*	<==========================	*

(5)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: +COPS: ...)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_COPS
	cmd_seq	C_COPS_Q

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_MAN

- GMMREG_NET_REQ

- GMMREG_PLMN_IND
	cause	MMCS_SUCCESS
	plmn	PLMN_LST
	forb_ind	FRB_PLMN_LST
	rxlevel	RXL_PLMN_LST
	gprs_status	GPRS_STATUS_LST

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- ACI_CMD_IND
	cmd_len	LM_PLUS_COPS_LST
	cmd_seq	M_PLUS_COPS_LST

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	22.12.98	AK	Initial

### GACI240: One PDP context activated

Description:

<A> No PDP context is defined.

<B> One PDP context is defined and will be activated.

<C> One PDP context is defined and will be activated. The requested quality of service profile is set.

<D> One PDP context is defined and will be activated. Both quality of service profiles are set.

<E> One PDP context is defined and will be activated. The minimum acceptable QOS profile is set and the requested quality of service profile is undefined.

<F> With +ATD and no PDP context is defined.

<G> With +ATD and no PDP context is defined.

<H> With +ATD and no PDP context is defined.

<I> With +ATD and no PDP context is defined.

<J> With +ATD and another PDP context is defined.

Variants:

- <A>...<J>

Preamble:

<A>GACI102

<B>GACI107

<C>GACI110

<D>GACI111

<E>GACI112

<F>GACI101

<G>GACI103

<H>GACI104

<I>GACI105

<J>GACI106

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(8)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(9)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(10)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(11)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(12)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(13)	|		|	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(14)	|		|	PPP_ESTABLISH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGDATA_PX
<B>	cmd_len	LC_CGDATA_PX
<C>	cmd_len	LC_CGDATA_PX
<D>	cmd_len	LC_CGDATA_PX
<E>	cmd_len	LC_CGDATA_PX
<F>	cmd_len	LC_GD_0
<G>	cmd_len	LC_GD_1
<H>	cmd_len	LC_GD_2
<I>	cmd_len	LC_GD_3
<J>	cmd_len	LC_GD_2
<A>	cmd_seq	C_CGDATA_P1
<B>	cmd_seq	C_CGDATA_P1
<C>	cmd_seq	C_CGDATA_P1
<D>	cmd_seq	C_CGDATA_P1
<E>	cmd_seq	C_CGDATA_P1
<F>	cmd_seq	C_GD_0
<G>	cmd_seq	C_GD_1
<H>	cmd_seq	C_GD_2
<I>	cmd_seq	C_GD_3
<J>	cmd_seq	C_GD_2

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
<A>	smreg_qos	SMREG_QOS_0
<B>	smreg_qos	SMREG_QOS_0
<C>	smreg_qos	SMREG_QOS_0
<D>	smreg_qos	SMREG_QOS_0
<E>	smreg_qos	SMREG_QOS_0
<F>	smreg_qos	SMREG_QOS_0
<G>	smreg_qos	SMREG_QOS_0
<H>	smreg_qos	SMREG_QOS_0
<I>	smreg_qos	SMREG_QOS_0
<J>	smreg_qos	SMREG_QOS_0
<A>	smreg_min_qos	SMREG_QOS_0
<B>	smreg_min_qos	SMREG_QOS_0
<C>	smreg_min_qos	SMREG_QOS_0
<D>	smreg_min_qos	SMREG_QOS_0
<E>	smreg_min_qos	SMREG_QOS_0
<F>	smreg_min_qos	SMREG_QOS_0
<G>	smreg_min_qos	SMREG_QOS_0
<H>	smreg_min_qos	SMREG_QOS_0
<I>	smreg_min_qos	SMREG_QOS_0
<J>	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
<A>	pdp_address	PDP_ADDRESS_0_S
<B>	pdp_address	PDP_ADDRESS_1
<C>	pdp_address	PDP_ADDRESS_1
<D>	pdp_address	PDP_ADDRESS_1
<E>	pdp_address	PDP_ADDRESS_1
<F>	pdp_address	PDP_ADDRESS_0_S
<G>	pdp_address	PDP_ADDRESS_0_S
<H>	pdp_address	PDP_ADDRESS_0_S
<I>	pdp_address	PDP_ADDRESS_0_S
<J>	pdp_address	PDP_ADDRESS_1
<A>	smreg_apn	SMREG_APN_0_S
<B>	smreg_apn	SMREG_APN_1
<C>	smreg_apn	SMREG_APN_1
<D>	smreg_apn	SMREG_APN_1
<E>	smreg_apn	SMREG_APN_1
<F>	smreg_apn	SMREG_APN_0_S
<G>	smreg_apn	SMREG_APN_0_S
<H>	smreg_apn	SMREG_APN_0_S
<I>	smreg_apn	SMREG_APN_0_S
<J>	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
<A>	smreg_qos	SMREG_QOS_1
<B>	smreg_qos	SMREG_QOS_1
<C>	smreg_qos	SMREG_QOS_2
<D>	smreg_qos	SMREG_QOS_2
<E>	smreg_qos	SMREG_QOS_1
<F>	smreg_qos	SMREG_QOS_1
<G>	smreg_qos	SMREG_QOS_1
<H>	smreg_qos	SMREG_QOS_1
<I>	smreg_qos	SMREG_QOS_1
<J>	smreg_qos	SMREG_QOS_1
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU_2

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	UNUSED_IN_TESTCASE
	dns1	UNUSED_IN_TESTCASE
	dns2	UNUSED_IN_TESTCASE

History:

30-07-2001	brz	changes for MULTI INSTANCE ACI

11-01-2001	brz	add more ATD commands

27-12-2000	brz	add ATD command

05-10-2000	brz	add UART entity

20-08-2000	brz	activation without context definition

06-08-2000	brz	add different quality of service

01-08-2000	brz	Initial

## Network registration state reporting (+CGREG, %CGREG)

### GACI250: Defining registration state reporting level (AT+CGREG=0,1 or 2; AT%CGREG=0,1 or 2)

Description:

Uses the command AT+CGREG and AT%CGREG =0, 1 or 2 to define the level of CGREG messages.

Variants:

<A> ... <F>

Preamble:

<A> GACI001a

<B> GACI001a

<C> GACI001a

<D> GACI001a

<E> GACI001a

<F> GACI001a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: %CUNS = 1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: CGREG=)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CUNS
	cmd_seq	C_CUNS_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGREG_X
<B>	cmd_len	LC_CGREG_X
<C>	cmd_len	LC_CGREG_X
<D>	cmd_len	LC_P_CGREG_X
<E>	cmd_len	LC_P_CGREG_X
<F>	cmd_len	LC_P_CGREG_X
<A>	cmd_seq	C_CGREG_0
<B>	cmd_seq	C_CGREG_1
<C>	cmd_seq	C_CGREG_2
<D>	cmd_seq	C_P_CGREG_0
<E>	cmd_seq	C_P_CGREG_1
<F>	cmd_seq	C_P_CGREG_2

- ACI_CMD_IND
<A>	cmd_len	LM_OK
<B>	cmd_len	LM_OK
<C>	cmd_len	LM_OK
<D>	cmd_len	LM_OK
<E>	cmd_len	LM_OK
<F>	cmd_len	LM_OK
<A>	cmd_seq	M_OK
<B>	cmd_seq	M_OK
<C>	cmd_seq	M_OK
<D>	cmd_seq	M_OK
<E>	cmd_seq	M_OK
<F>	cmd_seq	M_OK

History:

21-08-2001	JKH	Initial

### GACI251: Attaching GPRS including CGREG reporting

Description:

Attaches GPRS and reports GPRS attach state using CGREG messages.

Variants:

<A> ... <B>

Preamble:

<A> GACI250B

<B> GACI250E

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGATT=1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(cmd: CGREG=2)	|		|

*	<==========================*		|

(3)	|		|	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(4)	|		|	GMMREG_ATTACH_REJ	|

|		*	<==========================*

(5)	|		|	GMMREG_ATTACH_CNF	|

|		*	<==========================*

(6)	|	ACI_CMD_IND	|		|

|	(cmd: CGREG=1)	|		|

*	<==========================*		|

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGATT
	cmd_seq	C_CGATT_1

- ACI_CMD_IND
<A>	cmd_len	LM_CGREG
<B>	cmd_len	LM_P_CGREG
<A>	cmd_seq	M_CGREG_2
<B>	cmd_seq	M_P_CGREG_2

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_GPRS
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_REJ
	detach_type	GMMREG_DT_COMB
	cause	MMCS_CONGESTION
	search_running	GMMREG_SEARCH_RUNNING

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_GPRS
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- ACI_CMD_IND
<A>	cmd_len	LM_CGREG
<B>	cmd_len	LM_P_CGREG
<A>	cmd_seq	M_CGREG_5
<B>	cmd_seq	M_P_CGREG_5

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

28-02-2000	BRZ	CGATT=1 requests an only GPRS attach

04-12-2001	JKH	Initial

### GACI252: switching mobile off and back on, CGREG should keep ist state!

Description:

Switches mobile off and on, then checks the state of CGREG.

Variants:

<A> ... <B>

Preamble:

<A> GACI251A

<B> GACI251B

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CFUN=0)	|		|

*	==========================>	*		|

(2)	|		|	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_DETACH_REQ	|

|		*	==========================>	*

(4)	|		|	SIM_SYNC_CNF	|

|		*	<==========================	*

(5)	|		| 	GMMREG_DETACH_CNF	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(cmd: CGREG=0)	|		|

*	<==========================*		|

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(8)	|	ACI_CMD_REQ	|		|

|	(cmd: CFUN=1)	|		|

*	==========================>	*		|

(9)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(10)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(11)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(12)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(13)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(14)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(15)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(16)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(17)	|	ACI_CMD_REQ	|		|

|	(cmd: CGATT=1)	|		|

*	==========================>	*		|

(18)	|		|	GMMREG_ATTACH_REQ	|
	|		*	==========================>	*

(19)	|	ACI_CMD_IND	|		|

|	(cmd: CGREG=2)	|		|

*	<==========================*		|

(20)	|		|	GMMREG_ATTACH_REJ	|
	|		*	<==========================*

(21)	|		|	GMMREG_ATTACH_CNF	|
	|		*	<==========================*

(22)	|	ACI_CMD_IND	|		|

|	(cmd: CGREG=1)	|		|

*	<==========================*		|

(23)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CFUN
	cmd_seq	C_CFUN_0

- SIM_SYNC_REQ
	synccs	SYNC_DEACTIVATE

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_POWER_OFF

- SIM_SYNC_CNF
	cause	SYNC_DEACTIVATE

- GMMREG_DETACH_CNF
	detach_type	GMMREG_DT_POWER_OFF

- ACI_CMD_IND
<A>	cmd_len	LM_CGREG
<B>	cmd_len	LM_P_CGREG
<A>	cmd_seq	M_CGREG_0
<B>	cmd_seq	M_P_CGREG_0

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CFUN
	cmd_seq	C_CFUN_1

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NO_EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF	
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGATT
	cmd_seq	C_CGATT_1

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_GPRS
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- ACI_CMD_IND
<A>	cmd_len	LM_CGREG
<B>	cmd_len	LM_P_CGREG
<A>	cmd_seq	M_CGREG_2
<B>	cmd_seq	M_P_CGREG_2

- GMMREG_ATTACH_REJ
	detach_type	GMMREG_DT_COMB
	cause	MMCS_CONGESTION
	search_running	GMMREG_SEARCH_RUNNING

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- ACI_CMD_IND
<A>	cmd_len	LM_CGREG
<B>	cmd_len	LM_P_CGREG
<A>	cmd_seq	M_CGREG_5
<B>	cmd_seq	M_P_CGREG_5

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

28-02-2000	BRZ	CGATT=1 requests an only GPRS attach

04-12-2001	JKH	Initial

### GACI253: Suspend and Resume

Description:

GMM suspends and resumes GPRS for different reasons

Variants:

<A> ... <C>

Preamble:

<A> GACI251B

<B> GACI251B

<C> GACI251B

APL		ACI		PS

|		|		|

(1)	|		|	GMMREG_SUSPEND_IND	|

|		*	<==========================	*

(2)	|	ACI_CMD_IND	|		|

|	(cmd: %CGREG=6,7,8)	|		|

*	<==========================*		|

(3)	|		|	GMMREG_RESUME_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(cmd: %CGREG=5)	|		|

*	<==========================*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- GMMREG_SUSPEND_IND
<A>	cell_state	GMMREG_LIMITED_SERVICE
<B>	cell_state	GMMREG_CS_CALL
<C>	cell_state	GMMREG_GPRS_NOT_SUPPORTED

- ACI_CMD_IND
<A>	cmd_len	LM_P_CGREG
<B>	cmd_len	LM_P_CGREG
<C>	cmd_len	LM_P_CGREG
<A>	cmd_seq	M_P_CGREG_6
<B>	cmd_seq	M_P_CGREG_7
<C>	cmd_seq	M_P_CGREG_8

- GMMREG_RESUME_IND

- ACI_CMD_IND
<A>	cmd_len	LM_P_CGREG
<B>	cmd_len	LM_P_CGREG
<C>	cmd_len	LM_P_CGREG
<A>	cmd_seq	M_P_CGREG_5
<B>	cmd_seq	M_P_CGREG_5
<C>	cmd_seq	M_P_CGREG_5

History:

19-12-2001	JKH	Initial

## Ciphering Indicator Test Cases

### GACI260: CI is disabled, test set command

Description:

Preamble:

GACI001A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: %CPRI=1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: op not allowed)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PERCENT_CPRI_S1
	cmd_seq	C_PERCENT_CPRI_S1

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	29.10.01	TLU	Initial

### GACI261: CI is disabled, test query command

Description:

Preamble:

GACI001A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: %CPRI?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: %CPRI: 2)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PERCENT_CPRI_Q1
	cmd_seq	C_PERCENT_CPRI_Q1

- ACI_CMD_IND
	cmd_len	LM_PERCENT_CPRI_Q1
	cmd_seq	M_PERCENT_CPRI_Q1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	29.10.01	TLU	Initial

### GACI262: CI is disabled, test test command

Description:

Preamble:

GACI001A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: %CPRI=?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: %CPRI: (0,1)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PERCENT_CPRI_T1
	cmd_seq	C_PERCENT_CPRI_T1

- ACI_CMD_IND
	cmd_len	LM_PERCENT_CPRI_T1
	cmd_seq	M_PERCENT_CPRI_T1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	29.10.01	TLU	Initial

### GACI263: CI is enabled, test set and test command

Description:

Preamble:

GACI001F

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: %CPRI=1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: %CPRI=?)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: %CPRI: (0,1)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PERCENT_CPRI_S1
	cmd_seq	C_PERCENT_CPRI_S1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PERCENT_CPRI_T1
	cmd_seq	C_PERCENT_CPRI_T1

- ACI_CMD_IND
	cmd_len	LM_PERCENT_CPRI_T1
	cmd_seq	M_PERCENT_CPRI_T1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	29.10.01	TLU	Initial

### GACI264: CI is enabled, test query command and ciphering indication

Description:

Preamble:

GACI263

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: %CPRI?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: %CPRI: 1)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(4)	|		| 	GMMREG_CIPHERING_IND	|

|		*	<==========================	*

(5)	|	ACI_CMD_IND	|		|

|	(msg: %CPRI: 1,2)	|		|

*	<==========================	*		|

(6)	|		| 	SIM_REMOVE_IND	|

|		*	<==========================	*

(7)	|	ACI_CMD_IND	|		|

|	(msg: %SIMREM: 0)	|		|

*	<==========================	*		|

(8)	|	ACI_CMD_REQ	|		|

|	(cmd: %CPRI=0)	|		|

*	==========================>	*		|

(9)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PERCENT_CPRI_Q1
	cmd_seq	C_PERCENT_CPRI_Q1

- ACI_CMD_IND
	cmd_len	LM_PERCENT_CPRI_Q2
	cmd_seq	M_PERCENT_CPRI_Q2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- GMMREG_CIPHERING_IND
	gsm_ciph	CIPH_NA
	gprs_ciph	CIPH_ON

- ACI_CMD_IND
	cmd_len	LM_PERCENT_CPRI_U1
	cmd_seq	M_PERCENT_CPRI_U1

- SIM_REMOVE_IND
	cause	SIM_NO_ERROR

- ACI_CMD_IND
	cmd_len	LM_PERCENT_SIMREM_ERR_0
	cmd_seq	M_PERCENT_SIMREM_ERR_0

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PERCENT_CPRI_S2
	cmd_seq	C_PERCENT_CPRI_S2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	29.10.01	TLU	Initial

## additional test cases for context activation and deactivation

### GACI400: One PDP context activated

Description:

No PDP context is defined.

Variants:

<A> ... <D>

Preamble:

- <A>GACI113a

- <B>GACI113b

- <C>GACI113c

<D>GACI113d

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(5)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(7)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(8)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(9)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(10)	|		|	SN_SWITCH_REQ	|

|		*	==========================>	*

(11)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(12)	|		|	SN_SWITCH_CNF	|

|		*	<==========================	*

(13)	|		|	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(14)	|		|	PPP_ESTABLISH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SN_SWITCH_REQ
	nsapi	SMREG_NSAPI_5
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SN_SWITCH_CNF
	nsapi	SMREG_NSAPI_5

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	ip	IP_ADDRESS_1
<A>	sdu	SDU_2
<B>	sdu	SDU_3
<C>	sdu	SDU_NET_DNS_1
<D>	sdu	SDU_NET_GATEW_2

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	ip	UNUSED_IN_TESTCASE
	dns1	UNUSED_IN_TESTCASE
	dns2	UNUSED_IN_TESTCASE

History:

- 11-12-2002	rm	add variants C/D for additional network pco’s

- 09-10-2001	brz	Initial

### GACI405: Start a PDP context without specified use

Description:

It is possible to activate a PDP context without an application (eg WAP)

Variants:

<A>...<B>

Preamble:

<A>GACI081a

<B>GACI406a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGACT=1)	|		|

*	==========================>	*		|

(2)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(4)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(5)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGACT_X2
	cmd_seq	C_CGACT_121

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_1
	smreg_min_qos	SMREG_QOS_2
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_NULL_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_NEIGHBOR
	sdu	SDU_CTX_REQ_DEF

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_1
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_6
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_0_S
	smreg_apn	SMREG_APN_0_S
	dti_linkid	SNDCP_NULL_DTI_ID_2
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_NEIGHBOR
	sdu	SDU_CTX_REQ_DEF

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_1
	smreg_nsapi	SMREG_NSAPI_6
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

09-10-2001	brz	add context activation primitives

12-08-2001	brz	Initial

### GACI406: PDP context deactivation by CGACT=0,…

Description:

Variants:

<A>...<B>

Preamble:

<A>GACI405a

<B>GACI405b

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGACT=0)	|		|

*	==========================>	*		|

(2)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

- MUTE (2000)

(3)	|		|	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

- MUTE (2000)

(4)	|		|	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGACT_0
	cmd_seq	C_CGACT_0

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_56
	smreg_local	SMREG_NONLOCAL

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_6

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

26-09-2001	brz	Initial

### GACI407: A failed PDP context activation

Description:

Variants:

<A>...<B>

Preamble:

<A>GACI020

<B>GACI407a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGACT=1)	|		|

*	==========================>	*		|

(2)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	SMREG_PDP_ACTIVATE_REJ	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGACT_X
	cmd_seq	C_CGACT_11

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_0_S
	smreg_apn	SMREG_APN_0_S
	dti_linkid	SNDCP_NULL_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_NEIGHBOR
	sdu	SDU_CTX_REQ_DEF

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SMREG_RC_NETWORK_FAILURE
	smreg_nsapi	SMREG_NSAPI_5

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:

22-10-2001	brz	Initial

## UART_DETECT_IND (GACI420-GACI449)

### GACI420: One PDP context deactivation by PPP

Description:

PPP terminated an activated context.

Preamble:

GACI100b

APL		ACI		PS

|		|		|

(1)	|		|	UART_DETECTED_IND	|

|		*	<==========================	*

(2)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	PPP_TERMINATE_REQ	|

|		*	==========================>	*

- MUTE (2000)

(4)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(5)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(6)	|		|	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

- (7)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (8)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (9)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (10)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(11)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- UART_DETECTED_IND
	device	DEVICE_1
	dlci	STANDARD_DLCI
	cause	UART_DETECT_ESC

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

13-08-2002	brz	Initial

### GACI421: PDP context deactivation by PPP (2 UART_DETECTED_IND)

Description:

PPP terminated an activated context. 2 UART_DETECTED_IND (DTR-off) were received.

Preamble:

GACI100b

APL		ACI		PS

|		|		|

(1)	|		|	UART_DETECTED_IND	|

|		*	<==========================	*

(2)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	PPP_TERMINATE_REQ	|

|		*	==========================>	*

- MUTE (2000)

(4)	|		|	PPP_TERMINATE_IND	|

|		*	<==========================	*

(5)	|		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

(6)	|		|	UART_DETECTED_IND	|

|		*	<==========================	*

(7)	|		|	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

- (8)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (9)	|		|	UART_DTI_REQ	|
	|		*	==========================>	*

- (10)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- (11)	|		|	UART_DTI_CNF	|
	|		*	<==========================	*

(12)	|	ACI_CMD_IND	|		|

|	(msg: NO CARRIER)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- UART_DETECTED_IND
	device	DEVICE_1
	dlci	STANDARD_DLCI
	cause	UART_DETECT_DTR

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DETECTED_IND
	device	DEVICE_1
	dlci	STANDARD_DLCI
	cause	UART_DETECT_DTR

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

History:

03-01-2003	tlu	Initial

## Engineering Mode (GACI450-GACI460)

### GACI450:  Infrastructure Data - Serving Cell GPRS

Description:

Request serving cell GPRS data from GMM/GRR

Preamble:

- GACI001a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT%EM=2,2)	|		|

*	==========================>	*		|

(2)	|		| 	EM_SC_GPRS_INFO_REQ	|

|		*	==========================>	*

(3)	|		|	EM_SC_GPRS_INFO_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: %EM: ...)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PERCENT_EM_QUERY_2

- cmd_seq	C_PERCENT_EM_QUERY_2

- EM_SC_GPRS_INFO_REQ
	data	num_0

- EM_SC_GPRS_INFO_CNF
	tn	num_0
	nmo	num_1
	net_ctrl	S_NET_CTRL
	cba	num_4
	rac	num_5
	tav	num_6
	dsc	num_7
	c31	num_8
	c32	num_9

- ACI_CMD_IND
	cmd_len	LM_PERCENT_EM_2
	cmd_seq	M_PERCENT_EM_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	15.11.2001	OT	Initial

## Protocol Configuration Options (GACI500-501)

### GACI500:Set PCO for PDP context activation

Description:

Variants:

<A>...<C>

Preamble:

- GACI001A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATPercCGPCO=0,..)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PERC_CGPCO_1
<B>	cmd_len	LC_PERC_CGPCO_2
<C>	cmd_len	NUM_ELEMENTS(C_PERC_CGPCO_3)
<A>	cmd_seq	C_PERC_CGPCO_1
<B>	cmd_seq	C_PERC_CGPCO_2
<C>	cmd_seq	C_PERC_CGPCO_3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	26.11.02	RM	Initial

### GACI501:Query PCO for PDP context activation

Description:

Variants:

<A>...<D>

Preamble:

- <A> GACI500A

- <B> GACI014D

- <C> GACI014E

- <D> GACI014F

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATPercCGPCO=1,1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: %CGPCO)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	NUM_2
	cmd_len	LQ_PERC_CGPCO_1
	cmd_seq	QUERY_PERC_CGPCO_1

- ACI_CMD_IND
<A>	cmd_len	NUM_ELEMENTS(M_QPERC_CGPCO_1)
<B>	cmd_len	NUM_ELEMENTS(M_QPERC_CGPCO_2)
<C>	cmd_len	NUM_ELEMENTS(M_QPERC_CGPCO_1)
<D>	cmd_len	NUM_ELEMENTS(M_QPERC_CGPCO_2)
<A>	cmd_seq	M_QPERC_CGPCO_1
<B>	cmd_seq	M_QPERC_CGPCO_2
<C>	cmd_seq	M_QPERC_CGPCO_1
<D>	cmd_seq	M_QPERC_CGPCO_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	03.12.02	RM	Initial

## Send packet data for protocol PKTIO (GACI600-)

### GACI600: Entity PKTIO registers in ACI, successfullF

Description:

Variants:

<A>...<B>

Preamble:

- GACI002A

APL		ACI		PS

|		|		|

(1)	|		|	PKT_CONNECT_IND	|
	|		*	<==========================	*

(2)	|		|	PKT_CONNECT_RES	|
	|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- PKT_CONNECT_IND
	device_no	NUM_0
<A>	dio_dcb	DEFAULT_DCB_FROM_DIO_1
<B>	dio_dcb	DEFAULT_DCB_FROM_DIO_2

- PKT_CONNECT_RES
	device_no	NUM_0
<A>	dio_dcb	DEFAULT_DCB_FROM_ACI_1
<B>	dio_dcb	DEFAULT_DCB_FROM_ACI_2

History:	10.01.03	RM	Initial

### GACI601: Send AT%DATA to configure the data flow for PKTIO

Description:

Variants:

<A>...<E>

Preamble:

<A> GACI600a

<B> GACI600b

<C> GACI603b

<D> GACI605

<E> GACI606

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
|	(cmd: %DATA)	|		|
*	==========================>	*		|

- |	ACI_CMD_IND	|		|
|	(msg: OK)	|		|
*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	NUM_ELEMENTS(C_P_DATA_DEF)
<B>	cmd_len	NUM_ELEMENTS(C_P_DATA_ALL)
<C>	cmd_len	NUM_ELEMENTS(C_P_DATA_ALL)
<D>	cmd_len	NUM_ELEMENTS(C_P_DATA_ALL)
<E>	cmd_len	NUM_ELEMENTS(C_P_DATA_ALL)
<A>	cmd_seq	C_P_DATA_DEF
<B>	cmd_seq	C_P_DATA_ALL
<C>	cmd_seq	C_P_DATA_ALL
<D>	cmd_seq	C_P_DATA_ALL
<E>	cmd_seq	C_P_DATA_ALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	14.01.03	brz	Initial

### GACI602: Send AT+CGDATA and open dti connection

Description:

Variants:

<A>...<E>

Preamble:

<A>GACI601a

- <B>GACI601b

- <C>GACI601c

- <D>GACI601d

- <E>GACI601e

APL		ACI		PS

|		|		|

- TIMEOUT (2000)

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CGDATA)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	PKT_DTI_OPEN_REQ	|
	|		*	==========================>	*

(4)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(5)	|		|	PKT_DTI_OPEN_CNF	|
	|		*	<==========================	*

(6)	|		| 	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

MUTE(2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	NUM_ELEMENTS(C_PLUS_CGDATA_PKT_DEV_1)
	cmd_seq	C_PLUS_CGDATA_PKT_DEV_1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- PKT_DTI_OPEN_REQ
	device_no	NUM_0
	peer	NOT_USED
	link_id	PKT_LINK_ID
	dti_direction	DTI_CHANNEL_TO_LOWER_LAYER

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_0_S
	smreg_apn	SMREG_APN_0_S
	dti_linkid	PKT_LINK_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU_CTX_REQ_DEF

- PKT_DTI_OPEN_CNF
	device_no	NUM_0
	cause	PKTCS_SUCCESS

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_1
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

History:	10.01.03	RM	Initial

### GACI603:  Send AT+CGDATA to open dti connection, disconnection by PKTIO

Description:

Variants:

<A>...<B>

Preamble:

<A>GACI601a

- <B>GACI601b

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CGDATA)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(3)	|		|	PKT_DTI_OPEN_REQ	|
	|		*	==========================>	*

(4)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(5)	|		|	PKT_DTI_CLOSE_IND	|
	|		*	<==========================	*

(6)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(7)	|		|	PKT_DISCONNECT_IND	|
	|		*	<==========================	*

(8)	|	ACI_CMD_IND	|		|

|	(msg: no carrier)	|		|

*	<==========================	*		|

(9)	|		| 	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

MUTE(2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	NUM_ELEMENTS(C_PLUS_CGDATA_PKT_DEV_1)
	cmd_seq	C_PLUS_CGDATA_PKT_DEV_1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- PKT_DTI_OPEN_REQ
	device_no	NUM_0
	peer	NOT_USED
	link_id	PKT_LINK_ID
	dti_direction	DTI_CHANNEL_TO_LOWER_LAYER

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_0_S
	smreg_apn	SMREG_APN_0_S
	dti_linkid	PKT_LINK_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU_CTX_REQ_DEF

- PKT_DTI_CLOSE_IND
	device_no	NUM_0

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- PKT_DISCONNECT_IND
	device_no	NUM_0
	cause	PKTCS_DISCONNECT

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

History:	27.01.03	RM	Update

### GACI604:  Send AT+CGDATA to open dti connection, disconnection by network

Description:

Preamble:

<A>GACI601a

- <B>GACI601b

APL		ACI		PS

|		|		|
(1)	|	ACI_CMD_REQ	|		|
	|	(cmd: AT+CGDATA)	|		|
	*	==========================>	*		|
(2)	|	ACI_CMD_IND	|		|
	|	(msg: CONNECT)	|		|
	*	<==========================	*		|
(3)	|		|	PKT_DTI_OPEN_REQ	|
	|		*	==========================>	*
(4)	|		|	SMREG_PDP_ACTIVATE_REQ	|
	|		*	==========================>	*
(5)	|		|	PKT_DTI_OPEN_CNF	|
	|		*	<==========================	*
(6)	|		|	SMREG_PDP_ACTIVATE_REJ	|
	|		*	<==========================	*

(7)	|		|	PKT_DTI_CLOSE_REQ	|
	|		*	==========================>	*

(8)	|	ACI_CMD_IND	|		|
	|	(msg: no carrier)	|		|
	*	<==========================	*		|
(9)	|		|	PKT_DTI_CLOSE_CNF	|
	|		*	<==========================	*
MUTE(2000) 
	|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	NUM_ELEMENTS(C_PLUS_CGDATA_PKT_DEV_1)
	cmd_seq	C_PLUS_CGDATA_PKT_DEV_1

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- PKT_DTI_OPEN_REQ
	device_no	NUM_0
	peer	NOT_USED
	link_id	PKT_LINK_ID
	dti_direction	DTI_CHANNEL_TO_LOWER_LAYER

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_0_S
	smreg_apn	SMREG_APN_0_S
	dti_linkid	PKT_LINK_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU_CTX_REQ_DEF

- PKT_DTI_OPEN_CNF
	device_no	NUM_0
	cause	PKTCS_SUCCESS

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SMREG_RC_NETWORK_FAILURE
	smreg_nsapi	SMREG_NSAPI_5

- PKT_DTI_CLOSE_REQ
	device_no	NUM_0

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

- PKT_DTI_CLOSE_CNF
	device_no	NUM_0

History:	28.01.03	RM	Initial

### GACI605:  Send AT+CGDACT and close dti connection

Description:

Preamble:

- GACI602b

APL		ACI		PS

|		|		|

- TIMEOUT (2000)

- (1)	|	ACI_CMD_REQ	|		|

- |	(cmd: AT+CGACT=0)	|		|

- *	==========================>	*		|

- (2)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

- |		*	==========================>	*

- (3)	|		|	PKT_DTI_CLOSE_REQ	|
	|		*	==========================>	*

- (4)	|		|	PKT_DTI_CLOSE_CNF	|
	|		*	<==========================	*

- (5)	|		| 	SMREG_PDP_DEACTIVATE_CNF	|

- |		*	<==========================	*

- (6)	|	ACI_CMD_IND	|		|

- |	(msg: OK)	|		|

- *	<==========================	*		|

- MUTE(2000)

- |		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	NUM_ELEMENTS(C_CGACT_10)
	cmd_seq	C_CGACT_10

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- PKT_DTI_CLOSE_REQ
	device_no	NUM_0

- PKT_DTI_CLOSE_CNF
	device_no	NUM_0

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:		28.01.03	RM	Update

### GACI606:  Send AT+CGDACT to close dti connection, disconnection by PKTIO

Description:

Preamble:

- GACI602b

APL		ACI		PS

|		|		|

- TIMEOUT (2000)

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CGACT)	|		|

*	==========================>	*		|

(2)	|		|	PKT_DTI_CLOSE_REQ	|
	|		*	==========================>	*

(3)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(4)	|		|	PKT_DTI_CLOSE_IND	|
	|		*	<==========================	*

(5)	|		|	PKT_DISCONNECT_IND	|
	|		*	<==========================	*

(6)	|		| 	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	NUM_ELEMENTS(C_CGACT_10)
	cmd_seq	C_CGACT_10

- PKT_DTI_CLOSE_REQ
	device_no	NUM_0

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- PKT_DTI_CLOSE_IND
	device_no	NUM_0

- PKT_DISCONNECT_IND
	device_no	NUM_0
	cause	PKTCS_DISCONNECT

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	28.01.03	RM	Update

### GACI607: Disconnection of dti connection caused by DIO driver

Description:

Preamble:

- GACI602b

APL		ACI		PS

|		|		|

(1)	|		|	PKT_DTI_CLOSE_IND	|
	|		*	<==========================	*

(2)	|		|	SMREG_PDP_DEACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	PKT_DISCONNECT_IND	|
	|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: no carrier)	|		|

*	<==========================	*		|

(5)	|		| 	SMREG_PDP_DEACTIVATE_CNF	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- PKT_DTI_CLOSE_IND
	device_no	NUM_0

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- PKT_DISCONNECT_IND
	device_no	NUM_0
	cause	PKTCS_DISCONNECT

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

History:	15.11.02	RM	Initial

## %CGCLASS and Network mode III (GACI700-GACI721)

### GACI700: change the mobile class over %CGCLASS

Description:

The protocol stack is attached and we change the mobile class.

Variants:

- <A>...<F>

Preamble:

<A>GACI081a

<B>GACI700a

<C>GACI700b

<D>GACI700c

<E>GACI700d

<F>GACI700e

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGCLASS=XX)	|		|

*	==========================>	*		|

(2)	|		|	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

- MUTE (2000)

(3)	|		|	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGCLASS_XX
<B>	cmd_len	LC_CGCLASS_XX
<C>	cmd_len	LC_CGCLASS_XX
<D>	cmd_len	LC_CGCLASS_X
<E>	cmd_len	LC_CGCLASS_XX
<F>	cmd_len	LC_CGCLASS_XX
<A>	cmd_seq	C_PERCENT_CGCLASS_BX
<B>	cmd_seq	C_PERCENT_CGCLASS_BG
<C>	cmd_seq	C_PERCENT_CGCLASS_BC
<D>	cmd_seq	C_PERCENT_CGCLASS_C
<E>	cmd_seq	C_PERCENT_CGCLASS_CC
<F>	cmd_seq	C_PERCENT_CGCLASS_CG

- GMMREG_ATTACH_REQ
<A>	mobile_class	GMMREG_CLASS_BC
<B>	mobile_class	GMMREG_CLASS_BG
<C>	mobile_class	GMMREG_CLASS_BC
<D>	mobile_class	GMMREG_CLASS_CG
<E>	mobile_class	GMMREG_CLASS_CC
<F>	mobile_class	GMMREG_CLASS_CG
<A>	attach_type	GMMREG_AT_COMB
<B>	attach_type	GMMREG_AT_COMB
<C>	attach_type	GMMREG_AT_COMB
<D>	attach_type	GMMREG_AT_GPRS
<E>	attach_type	GMMREG_AT_IMSI
<F>	attach_type	GMMREG_AT_GPRS
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
<A>	attach_type	GMMREG_AT_COMB
<B>	attach_type	GMMREG_AT_COMB
<C>	attach_type	GMMREG_AT_COMB
<D>	attach_type	GMMREG_AT_GPRS
<E>	attach_type	GMMREG_AT_IMSI
<F>	attach_type	GMMREG_AT_GPRS
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

22-10-2002	brz		Initial

### GACI701: change the mobile class over %CGCLASS (NMO III, attach_cnf)

Description:

The protocol stack is attached and we change the mobile class.

Variants:

- <A>...<F>

Preamble:

<A>GACI081a

<B>GACI700a

<C>GACI700b

<D>GACI700c

<E>GACI700d

<F>GACI700e

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGCLASS=XX)	|		|

*	==========================>	*		|

(2)	|		|	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

- MUTE (2000)

(3)	|		|	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGCLASS_XX
<B>	cmd_len	LC_CGCLASS_XX
<C>	cmd_len	LC_CGCLASS_XX
<D>	cmd_len	LC_CGCLASS_X
<E>	cmd_len	LC_CGCLASS_XX
<F>	cmd_len	LC_CGCLASS_XX
<A>	cmd_seq	C_PERCENT_CGCLASS_BX
<B>	cmd_seq	C_PERCENT_CGCLASS_BG
<C>	cmd_seq	C_PERCENT_CGCLASS_BC
<D>	cmd_seq	C_PERCENT_CGCLASS_C
<E>	cmd_seq	C_PERCENT_CGCLASS_CC
<F>	cmd_seq	C_PERCENT_CGCLASS_CG

- GMMREG_ATTACH_REQ
<A>	mobile_class	GMMREG_CLASS_BC
<B>	mobile_class	GMMREG_CLASS_BG
<C>	mobile_class	GMMREG_CLASS_BC
<D>	mobile_class	GMMREG_CLASS_CG
<E>	mobile_class	GMMREG_CLASS_CC
<F>	mobile_class	GMMREG_CLASS_CG
<A>	attach_type	GMMREG_AT_COMB
<B>	attach_type	GMMREG_AT_COMB
<C>	attach_type	GMMREG_AT_COMB
<D>	attach_type	GMMREG_AT_GPRS
<E>	attach_type	GMMREG_AT_IMSI
<F>	attach_type	GMMREG_AT_GPRS
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
<A>	attach_type	GMMREG_AT_IMSI
<B>	attach_type	GMMREG_AT_GPRS
<C>	attach_type	GMMREG_AT_IMSI
<D>	attach_type	GMMREG_AT_GPRS
<E>	attach_type	GMMREG_AT_IMSI
<F>	attach_type	GMMREG_AT_GPRS
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

22-10-2002	brz		Initial

### GACI702: change the mobile class over %CGCLASS (NMO III, attach_rej)

Description:

The protocol stack is attached and we change the mobile class.

Variants:

- <A>...<F>

Preamble:

<A>GACI081a

<B>GACI700a

<C>GACI700b

<D>GACI700c

<E>GACI700d

<F>GACI700e

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGCLASS=XX)	|		|

*	==========================>	*		|

(2)	|		|	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

- MUTE (2000)

(3)	|		|	GMMREG_ATTACH_REJ	|

|		*	<==========================	*

(4)	|		|	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_CGCLASS_XX
<B>	cmd_len	LC_CGCLASS_XX
<C>	cmd_len	LC_CGCLASS_XX
<D>	cmd_len	LC_CGCLASS_X
<E>	cmd_len	LC_CGCLASS_XX
<F>	cmd_len	LC_CGCLASS_XX
<A>	cmd_seq	C_PERCENT_CGCLASS_BX
<B>	cmd_seq	C_PERCENT_CGCLASS_BG
<C>	cmd_seq	C_PERCENT_CGCLASS_BC
<D>	cmd_seq	C_PERCENT_CGCLASS_C
<E>	cmd_seq	C_PERCENT_CGCLASS_CC
<F>	cmd_seq	C_PERCENT_CGCLASS_CG

- GMMREG_ATTACH_REQ
<A>	mobile_class	GMMREG_CLASS_BC
<B>	mobile_class	GMMREG_CLASS_BG
<C>	mobile_class	GMMREG_CLASS_BC
<D>	mobile_class	GMMREG_CLASS_CG
<E>	mobile_class	GMMREG_CLASS_CC
<F>	mobile_class	GMMREG_CLASS_CG
<A>	attach_type	GMMREG_AT_COMB
<B>	attach_type	GMMREG_AT_COMB
<C>	attach_type	GMMREG_AT_COMB
<D>	attach_type	GMMREG_AT_GPRS
<E>	attach_type	GMMREG_AT_IMSI
<F>	attach_type	GMMREG_AT_GPRS
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_REJ
	detach_type	GMMREG_DT_COMB
	cause	GMMCS_IMSI_UNKNOWN
	search_running	GMMREG_SEARCH_RUNNING

- GMMREG_ATTACH_CNF
<A>	attach_type	GMMREG_AT_IMSI
<B>	attach_type	GMMREG_AT_GPRS
<C>	attach_type	GMMREG_AT_IMSI
<D>	attach_type	GMMREG_AT_GPRS
<E>	attach_type	GMMREG_AT_IMSI
<F>	attach_type	GMMREG_AT_GPRS
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

22-10-2002	brz		Initial

### GACI709: Class status commands

Description:

Read mobile class.

Variants:

- <A>....<L>

Preamble:

<A>GACI700a

<B>GACI700b

<C>GACI700c

<D>GACI700d

<E>GACI700e

<F>GACI700f

<G>GACI701a

<H>GACI701b

<I>GACI701c

<J>GACI701d

<K>GACI701e

<L>GACI701f

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGCLASS?)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: %CGCLASS="XX")	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGCLASS_R
	cmd_seq	C_PERCENT_CGCLASS_R

- ACI_CMD_IND
<A>	cmd_len	LM_PERCENT_CGCLASS_RBX
<B>	cmd_len	LM_CGCLASS_RXX
<C>	cmd_len	LM_CGCLASS_RXX
<D>	cmd_len	LM_CGCLASS_RX
<E>	cmd_len	LM_CGCLASS_RXX
<F>	cmd_len	LM_CGCLASS_RXX
<G>	cmd_len	LM_PERCENT_CGCLASS_RBX
<H>	cmd_len	LM_CGCLASS_RXX
<I>	cmd_len	LM_CGCLASS_RXX
<J>	cmd_len	LM_CGCLASS_RX
<K>	cmd_len	LM_CGCLASS_RXX
<L>	cmd_len	LM_CGCLASS_RXX
<A>	cmd_seq	M_PERCENT_CGCLASS_RBXBC
<B>	cmd_seq	M_PERCENT_CGCLASS_RBG
<C>	cmd_seq	M_PERCENT_CGCLASS_RBC
<D>	cmd_seq	M_PERCENT_CGCLASS_RC
<E>	cmd_seq	M_PERCENT_CGCLASS_RCC
<F>	cmd_seq	M_PERCENT_CGCLASS_RCG
<G>	cmd_seq	M_PERCENT_CGCLASS_RBXBC
<H>	cmd_seq	M_PERCENT_CGCLASS_RBG
<I>	cmd_seq	M_PERCENT_CGCLASS_RBC
<J>	cmd_seq	M_PERCENT_CGCLASS_RC
<K>	cmd_seq	M_PERCENT_CGCLASS_RCC
<L>	cmd_seq	M_PERCENT_CGCLASS_RCG

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

22-10-2002	brz	Initial

### GACI710: One PDP context activated

Description:

No PDP context is defined.

Variants:

- <A>...<C>

Preamble:

- <A>GACI700a

- <B>GACI701a

- <C>GACI702a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGDATA="PPP",1)	|		|

*	==========================>	*		|

(2)	|		|	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(3)	|		|	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(5)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_IND	|

|		|	(UART_DISCONNECT_DTI)	|

|		*	<==========================	*

(7)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(8)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(9)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(10)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(11)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(12)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(13)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(14)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(15)	|		|	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(16)	|		|	PPP_ESTABLISH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGDATA_PX
	cmd_seq	C_CGDATA_P1

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_GPRS
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	NUM_0
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID
	prot_link_id	SNDCP_DTI_ID

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND 
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_1
	smreg_min_qos	SMREG_QOS_2
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_2
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	UNUSED_IN_TESTCASE
	dns1	UNUSED_IN_TESTCASE
	dns2	UNUSED_IN_TESTCASE

History:

- 22-10-2002	brz	Initial

### GACI715: One PDP context deactivation by PPP

Description:

PPP terminated an activated context.

Variants:

- <A>...<C>

Preamble:

<A>GACI710a

<B>GACI710b

<C>GACI710c

APL		ACI		PS

|		|		|

- |		|	PPP_TERMINATE_IND	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	SMREG_PDP_DEACTIVATE_REQ	|
	|		*	==========================>	*
MUTE (2000)

- |		|	SMREG_PDP_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	GMMREG_ATTACH_REQ	|
	|		*	==========================>	*

- |	ACI_CMD_IND	|		|
	|	(msg: NO CARRIER)	|		|
	*	<==========================	*		|

- |		|	GMMREG_ATTACH_CNF	|
	|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- PPP_TERMINATE_IND
	ppp_cause	NOT_USED

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_NSAPI_5
	smreg_local	SMREG_NONLOCAL

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_NSAPI_5

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ 
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	DEVICE_1
	dlci	STANDARD_DLCI

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BC
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- ACI_CMD_IND
	cmd_len	LM_NO_CARRIER
	cmd_seq	M_NO_CARRIER

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_IMSI
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:

22-10-2002	brz	Initial

### GACI720: Start a PDP context without specified use

Description:

It is possible to activate a PDP context without an application

Variants:

<A>...<C>

Preamble:

<A>GACI701a

<B>GACI715b

<C>GACI721b

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGACT=1)	|		|

*	==========================>	*		|

(2)	|		|	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(3)	|		|	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(4)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(5)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGACT_X
	cmd_seq	C_CGACT_11

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_GPRS
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_1
	smreg_min_qos	SMREG_QOS_2
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_1
	dti_linkid	SNDCP_NULL_DTI_ID
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_NEIGHBOR
<A>	sdu	SDU_CTX_REQ_DEF
<B>	sdu	SDU
<C>	sdu	SDU_CTX_REQ_DEF

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	NUM_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_2
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

22-10-2002	brz	Initial

### GACI721: Start a PDP context without specified use

Description:

It is possible to activate a PDP context without an application (eg WAP)

Variants:

<A>...<C>

Preamble:

<A>GACI700a

<B>GACI701a

<C>GACI702a

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CGACT=1)	|		|

*	==========================>	*		|

(2)	|		|	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(3)	|		|	GMMREG_ATTACH_REJ	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CGACT_X
	cmd_seq	C_CGACT_11

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_REJ
	detach_type	GMMREG_DT_COMB
	cause	GMMCS_IMSI_UNKNOWN
	search_running	GMMREG_SEARCH_NOT_RUNNING

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:

22-10-2002	brz	Initial

## Dual port test cases (GACI800-GACI830)

### GACI800: UART Init (2) and install redirection

Description:

Initialise two UART-ACI connections.

Preamble:

GACI000

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG ADD_TST_SRC UART_DP)

- |		|	UART_PARAMETERS_REQ	|
	|		*	==========================>	*

- COMMAND (MMI CONFIG ADD_TST_SRC UART_DP)

- |		|	UART_PARAMETERS_REQ	|
	|		*	==========================>	*

- |		|	UART_PARAMETERS_CNF	|
	|		*	<==========================	*

- |		|	UART_PARAMETERS_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |	ACI_CMD_REQ	|		|
	|	(cmd: %DATA)	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_PARAMETERS_REQ
	device	NUM_0
	comPar	NOT_USED

- UART_PARAMETERS_REQ
	device	NUM_1
	comPar	NOT_USED

- UART_PARAMETERS_CNF
	device	NUM_0

- UART_PARAMETERS_CNF
	device	NUM_1

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	UART_DTI_ID
	entity_name	STRING_POINTER

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_1
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	UART_DTI_ID_2
	entity_name	STRING_POINTER

- DTI2_CONNECT_REQ
	link_id	UART_DTI_ID_2
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_1
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_CNF
	link_id	UART_DTI_ID_2
	version	DTI_VERSION_10

- ACI_CMD_REQ
	cmd_src	NUM_2
	cmd_len	NUM_ELEMENTS(C_P_DATA_DP)
	cmd_seq	C_P_DATA_DP

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

14-02-2003	tlu	Initial

### GACI801: Set ME to full functionality state, no PIN required

Description:

First command. The default class is “BG".

Preamble:

- GACI800

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: CFUN=1)	|		|

*	==========================>	*		|

(2)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(3)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(4)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(7)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(9)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	NUM_2
	cmd_len	LC_CFUN
	cmd_seq	C_CFUN_1

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NO_EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF	
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

14-02-2003	tlu		Initial

### GACI802: Automatic registration (GPRS automatic attach)

Description:

ME is in full functionality state.

Preamble:

- <A>GACI801

APL		ACI		PS

- COMMAND (MMI CONFIG AUTO_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COPS=0)	|		|

*	==========================>	*		|

(2)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(5)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	NUM_2
	cmd_len	LC_COPS
	cmd_seq	C_COPS_0

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	NUM_1
	rac	NUM_1
	cid	NUM_1
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:

10.02.2003	TLU	Initial

### GACI810: One PDP context activated

Description:

No PDP context is defined.

Preamble:

GACI802

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT&D2)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD*99#)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: CONNECT)	|		|

*	<==========================	*		|

(5)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(6)	|		|	UART_DTI_IND	|

|		|	(UART_DISCONNECT_DTI)	|

|		*	<==========================	*

(7)	|		|	UART_DTI_REQ	|

|		*	==========================>	*

(8)	|		|	UART_DTI_CNF	|

|		*	<==========================	*

(9)	|		|	PPP_ESTABLISH_REQ	|

|		*	==========================>	*

(10)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(11)	|		|	PPP_PDP_ACTIVATE_IND	|

|		*	<==========================	*

(12)	|		|	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

(13)	|		|	PPP_DTI_CONNECTED_IND	|

|		*	<==========================	*

(14)	|		|	SMREG_PDP_ACTIVATE_CNF	|

|		*	<==========================	*

(15)	|		|	PPP_PDP_ACTIVATE_RES	|

|		*	==========================>	*

(16)	|		|	PPP_ESTABLISH_CNF	|

|		*	<==========================	*

- MUTE (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	NUM_2
	cmd_len	LC_AND_D2
	cmd_seq	C_AND_D2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	NUM_2
	cmd_len	LC_GD_0
	cmd_seq	C_GD_0

- ACI_CMD_IND
	cmd_len	LM_CONNECT
	cmd_seq	M_CONNECT

- DTI2_DISCONNECT_REQ	link_id	UART_DTI_ID_2
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_1
	dlci	UART_DLCI_NOT_MULTIPLEXED

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_1
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	UART_DTI_ID_2
	entity_name	STRING_POINTER

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_1
	dlci	UART_DLCI_NOT_MULTIPLEXED

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_AUTO
	login	NOT_USED
	accm	PPP_ACCM_OFF
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	NOT_USED
	ip	NOT_USED
	dns1	NOT_USED
	dns2	NOT_USED
	peer_channel	UART_CHANNEL
	protocol_channel	SNDCP_CHANNEL
	peer_direction	PEER_DIRECTION
	prot_direction	PROT_DIRECTION
	peer_link_id	UART_DTI_ID_2
	prot_link_id	SNDCP_DTI_ID_2

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- PPP_PDP_ACTIVATE_IND 
	ppp_hc	PPP_HC_VJ
	msid	MSID_NO
	sdu	SDU

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NUM_FF
	pdp_address	PDP_ADDRESS_0_S
	smreg_apn	SMREG_APN_0_S
	dti_linkid	SNDCP_DTI_ID_2
	dti_neighbor	STRING_POINTER
	dti_direction	SMREG_HOME
	sdu	SDU

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_1
	smreg_nsapi	SMREG_NSAPI_5
	pdp_address	PDP_ADDRESS_1
	sdu	SDU_2

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	IP_ADDRESS_1
	sdu	SDU_2

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_NO
	ip	UNUSED_IN_TESTCASE
	dns1	UNUSED_IN_TESTCASE
	dns2	UNUSED_IN_TESTCASE

History:

14-02-2003	tlu	Initial

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |