GSM/GPRS Protocol Stack

MMI Framework – Test Specification

Test Control Language Script for the MMI

| Date: | 09-October-2003 |
| --- | --- |
| ID: | 8415.xxx.00.001 |
|  | MFW.DOC |

| Author: | Condat UK Ltd 38 Melville Street Edinburgh Scotland |
| --- | --- |

| Condat UK Proprietary Information NDA - Confidential Do Not Copy |  |
| --- | --- |

Table of Contents

0	Document Control	8

0.1	Document History	8

0.2	References	8

1	Introduction	10

1.1	Abbreviations	11

1.2	Terms	14

2	Protocol Stack	15

2.1	GRR (RLC/MAC) – Radio Link Control/Medium Access Control	15

2.2	LLC – Logical Link Control	15

2.3	GMM – GPRS Mobility Management	16

2.4	SM – Session Management	16

2.5	SNDCP - Subnetwork Dependant Convergence Protocol	16

2.6	GACI – GPRS Application Control Interface	16

2.7	USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	16

2.8	TOM – Tunnelling of Messages	16

2.9	Protocol Stack Entities	16

2.10	Test Script description	18

I/F with Entities	18

3	Parameters	20

4	TEST CASES	106

4.1	Routing (internal)	106

4.1.1	MFW001:  Setup the Routing and the PCO view for the MMI test. The CC configuration and SIM activation signals are output.	106

4.2	MMI Component Tests - case : Power On Sequences.	107

4.2.1	MFW010: Power on Sequence, ME to Idle State, no PIN required	107

4.2.2	MFW011: Power On Sequence ME to Idle state, no PIN required. Combined IMSI/GPRS Attach.	110

4.2.3	MFW012: Power on Sequence. ME to Idle state, no PIN required, Limited Service.	112

4.2.4	MFW014: Power on Sequence. ME to Idle state, no PIN required. Automatic registration and Attachment follows.	114

4.2.5	MFW015: Power On Sequence. ME to Idle State, No PIN required. Automatic registration and Attachment follows.	117

4.2.6	MFW016: Power on Sequence, ME to Idle State, No PIN required.	119

4.2.7	MFW018: Power On Sequence. Initialise the UART and flush any data.	120

4.2.8	MFW020: Set the Date	120

4.2.9	MFW022: For test of Xpanel.	122

4.2.10	MFW024: Power on Sequence, enter PIN required. ME to Idle State.	124

4.2.11	MFW026: Power on Sequence. No SIM card. Limited mode. Emergency Call.	127

4.2.12	MFW028: Power on Sequence. No SIM card. No Service.	129

4.2.13	MFW030: Power On Sequence, Invalid PIN, entering more than 3 digits , delete and emergency call	131

4.2.14	MFW032: Power On Sequence, No SIM card, error in emergency call, delete and emergency call	133

4.2.15	MFW034: Power On Sequence, Blocked SIM card, limited mode, emergency call	135

4.2.16	MFW036: Power On Sequence, Blocked SIM Card, enter PUK1 code.	138

4.2.17	MFW038: Power On Sequence: PIN disabled, sim unlock(all checks), IdleScreen with clock.	142

4.2.18	MFW040: Power On Sequence with PIN entering and sim unlock	144

4.2.19	MFW042: MO call setup, followed by removel of the SIM card.	147

4.3	MMI Component Tests - case : Pin Entering from Power On.	148

4.3.1	MFW050: Power on the ME. PIN required. This will be the initial preamble for PIN entering tests.	148

4.3.2	MFW052: PIN entering from Power On, enter 4 digits, successfull	149

4.3.3	MFW054: PIN entering from Power On , clear with long press, 4 digits.	152

4.3.4	MFW056: PIN entering from Power On and clearing several times. 8 digits.	155

4.3.5	MFW058: PIN entering from Power On and clear all entered digits. 8 digits.	158

4.3.6	MFW060: PIN entering from Power On, clear if nothing to clear and wrong keys.	161

4.3.7	MFW062: PIN entering from Power On, wrong digit during entering.	164

4.3.8	MFW064: PIN entering from Power On, END and MENU UP keys during entering.	167

4.3.9	MFW066: PIN entering from Power On, enter digits. Successful, enter extraneous digit afterwards. Correctly ignored.	170

4.3.10	MFW068: PIN entering from Power On, enter digits with delay while entering.	172

4.3.11	MFW070: PIN entering from Power On, error in PIN, abort information screen, followed by correct PIN.	173

4.3.12	MFW072: PIN entering from Power On via GSM command string, unsuccessful gsm code change. Test Fails.	176

4.3.13	MFW074: PUK entering from Power On via GSM command string, unsuccessful GSM code change. Test Fails.	179

4.4	MMI Component Tests - case : PUK entering from Power On	182

4.4.1	MFW100: PIN entering from Power On, repeated failures on verification, initial case for PUK	182

4.4.2	MFW102: PUK entering from Power On, enter the PUK.  Successful case.	183

4.4.3	MFW104: PUK entering from Power On, PIN verification, new PIN is different.	186

4.4.4	MFW106: PUK entering from Power On, error in PUK, re-input the correct PUK.	189

4.4.5	MFW108: PUK entering from Power On, wrong digit, clear, wrong digit, long pressed clear. Correct PUK.	193

4.4.6	MFW110: PUK entering from Power On, wrong digit, clear, wrong digit, End key. Correct PUK.	196

4.4.7	MFW112: PUK entering from Power On, Emergency call setup.	199

4.5	MMI Component Tests - case : Main Menu, Security, Pin1 codes.	201

4.5.1	MFW150: Main Menu, Security, Pin code: PIN changed, successful,	201

4.5.2	MFW152: Main Menu, Security,PIN, Change, enter old Pin code, abort, back to Idle	203

4.5.3	MFW154: Main Menu, Security, PIN, Change, enter Pin code, error in new pin, correct new pin.	205

4.5.4	MFW156: Main Menu, Security, PIN, Change, Pin code, wrong pin, wrong lhs. Change PIN.	208

4.5.5	MFW158: Main Menu, Security, PIN, Change, Pin code, new Pin again: abort clear	211

4.5.6	MFW160: Main Menu, Security, PIN2, PIN 2 changed, with long delay while entering.	213

4.5.7	MFW162: Main Menu, Security, PIN enabled by default, Pin, Pin disabled.	215

4.5.8	MFW164: Main Menu, Change PIN by GSM command string **04*.	218

4.5.9	MFW166: Main Menu, Security, PIN, PIN changed via GSM code, 1st attempt fail, 2nd success.	220

4.5.10	MFW168: Main Menu, Security, Pin2, PIN2 changed via GSM code, sucessfull (Pin enabled)	223

4.5.11	MFW170: Main Menu, Security, PIN, Change PIN by GSM command string, twice in a row.	225

4.5.12	MFW172: Main Menu, Change PIN by GSM command string, error in code. Re enter code.	228

4.5.13	MFW174: Main Menu, Change PIN, Error 3 times in entry, so the PUK is entered by gsm code.	231

4.5.14	MFW176: Main Menu, Change PIN, 3 times error in input, error in PUK GSM code, correct PUK.	233

4.5.15	MFW178: PIN Deactivated. Main Menu, Security, PIN, Activate, Deactivate, Activate in sequence.	237

4.5.16	MFW180: PIN enabled, enter emergency call and successfull pin entering. Test fails.	241

4.5.17	MFW182: PIN disabled. Main menu, Security, Activate, PIN. unsuccessful, re-try with correct PIN.	244

4.5.18	MFW184: PIN disabled. Power On Sequence Main menu, Security, PIN, Activate. Not successful, Delete.	246

4.5.19	MFW186: PIN enabled. Activate PIN. Advisary message is displayed.	247

4.5.20	MFW188: Power On Sequence Main Menu, Security, Pin, Activate, unsuccessful PIN enabled 3 times. PUK code.	248

4.5.21	MFW190: Default PIN disabled. Power On Sequence Main Menu, Security, Pin, Activate failed GSM command string, then manually enter.	251

4.5.22	MFW192: . Power On Sequence Main Menu, Security, PIN, Deactivate, enter PIN, wrong PIN then correct PIN.	253

4.5.23	MFW194: PIN enabled by default. Power On Sequence, Main menu, Security, Pin, Deactivate, failure and back to IdleScreen.	254

4.5.24	MFW196: PIN enabled by default. Power On Sequence, Main Menu, Security, Pin, Deactivate, enter by GSM code(error), then correctly.	256

4.5.25	MFW198: PIN disabled by default. Power On Sequence Main Menu, Security, Pin, Deactivate, no pin needed for disable.	258

4.6	MMI Component Tests - case : Main Menu, Security, Pin2 codes.	259

4.6.1	MFW250: Main Menu, Security, Pin2, enter, with time out while entering, successful	259

4.6.2	MFW252: Main Menu, Security, Pin2, confirm PIN 2 entry in error, then successful	261

4.6.3	MFW254: Main Menu, Security, Pin2, enter old PIN 2, then clear and abort back.	264

4.6.4	MFW256: Main Menu, Security, PIN2, PIN2 changed, wrong new pin and wrong key left.	266

4.6.5	MFW258: Main Menu, Security, PIN2, PIN2 changed, correct new pin and abort.	269

4.6.6	MFW260: Main Menu, Security, PIN2, error in old PIN2, retry, new PIN2, confirm PIN2.	270

4.6.7	MFW262: Main Menu, Security, PIN2, twice error in old PIN2, new PIN2, confirm PIN2.	273

4.6.8	MFW264: Main Menu, Security, PIN2, repeated errors in old PIN2, PUK2 code entered.	276

4.6.9	MFW266: Main Menu, Security, PIN2, repeated errors in old PIN2, error in PUK2, re-enter.PUK2.	280

4.6.10	MFW268: Main Menu, Security, PIN2, repeated errors in old PIN2, PUK2 code entered via GSM string.	281

4.7	MMI Component Tests - case :  Main Menu, Phone Settings, Date and Time.	286

4.7.1	MFW300: Main Menu, Phone Settings, Clock, Time/Date, enter date and time successfully and back to idle screen.	286

4.7.2	MFW302: Main Menu, Phone Settings, Clock, Time/Date, enter incorrect date and time, then correct.	288

4.7.3	MFW304: Main Menu, Phone Settings, Clock, Time/Date, enter date and time clearing both times.	290

4.7.4	MFW306: Main Menu, Phone Settings, Clock, Time/Date, enter date, long press clear; without time entering.	292

4.8	MMI Component Tests - case : Power Down Sequence	293

4.8.1	MFW350: Power Down Sequence from the IdleScreen (No PIN required), power down key.	293

4.8.2	MFW352: Power up Sequence from idle when entering PIN, Power down key.	294

4.8.3	MFW354: Power up Sequence from idle when entering an emergency number, Power down key.	295

4.8.4	MFW356: Power Down Sequence from the IdleScreen, Power up, Power down, Power up. Test Fails.	296

4.8.5	MFW358: Power Down Sequence from the WelcomeScreen, power down key.	298

4.8.6	MFW360: Power Down sequence. Remove SIM, Power Down.	299

4.9	MMI Component Tests - case : Main Menu, Security, Fixed Names	300

4.9.1	MFW380: Main Menu, Security, Fixed Names, Activate with time out while entering, successful	300

4.9.2	MFW382: Main Menu, Security, Activate Fixed Names, during entry LHS and CLEAR	302

4.9.3	MFW384: Main Menu, Security, Activate Fixed Names, 2 wrong PIN2 entries, followed by correct PIN2.	304

4.9.4	MFW386: Main Menu, Security, Activate Fixed Names, 3 wrong PIN2 entries, followed by PUK2.	307

4.9.5	MFW388: After correct PUK2 code has been entered, then activate SIM..	310

4.9.6	MFW390: After incorrect PUK2 code has been entered, re-enter PUK2 correctly then activate SIM.	311

4.9.7	MFW392: After incorrect PUK2 code has been entered, re-enter PUK2 correctly but wrong PIN 1, re-enter.	313

4.9.8	MFW394: After incorrect PUK2 code has been entered, abort during re-entry of PUK2..	315

4.9.9	MFW396: Main Menu, Security, Fixed Names, Deactivate.	316

4.9.10	MFW398: Main Menu, Security, Fixed Names, Deactivate, Abort and Deactivate.	317

4.9.11	MFW400: Main Menu, Security, Dectivate Fixed Names, 3 wrong PIN2 entries, followed by PUK2.	319

4.9.12	MFW402: After correct PUK2 code has been entered, then Deactiavate FDN.	323

4.9.13	MFW404: After incorrect PUK2 code has been entered, re-enter PUK2 correctly. Deactivate FDN.	324

4.9.14	MFW406: Main Menu, Security, Fixed Names, Activate then Deactivate.	325

4.10	MMI Component Tests - case: Main Menu, Names, Phonebook.	328

4.10.1	MFW430: Power On Sequence with Phonebook(ADN enabled).	328

4.10.2	MFW432: Power On Sequence with Phonebook(FDN enabled).	334

4.10.3	MFW440: FDN Enabled make Mobile Originated Call when call is in the FDN.	339

4.10.4	MFW442: FDN Enabled make Mobile Originated Call when call is not in the FDN.	341

4.10.5	MFW444: FDN Enabled make Emergency Call when call is not in the FDN.	341

4.10.6	MFW450: From the Idle Screen, Names, Add new Entry into the Phonebook.	342

4.10.7	MFW452: From Idle, Search for a name in the Phonebook, and Call.	344

4.10.8	MFW454: From Idle, Search for a name in the Phonebook, and Delete.	346

4.10.9	MFW456: From Idle, Search for a name in the Phonebook, and Change.	347

4.10.10	MFW458: From Main menu, Update the Call Centre and Validity period, ready for SMS. Make MO Call.	348

4.10.11	MFW460: From Idle, Names, Display Memory Stat.	351

4.10.12	MFW462: From Idle, Names, Set the Phonebook to SIM then Phone	351

4.10.13	MFW464: From Idle, Search for a name in the Phonebook, and Copy entry to SIM.	352

4.10.14	MFW466: From Idle, Search for a name in the Phonebook, Send Message.	353

4.10.15	MFW468: From Idle, Names, Service Number	356

4.10.16	MFW470: From Idle, Names, Change My Number	356

4.10.17	MFW472: From Idle, Search for a name in the Phonebook, and Move entry to SIM.	358

4.11	MMI Component Tests case : Main Menu, Network Services, Activate Call Waiting.	359

4.11.1	MFW490: From Main menu, Network services, Set Call Waiting Enabled, ready for Call Processing tests.	359

4.12	MMI Component Tests - case : MO Call Processing	360

4.12.1	MFW500: Mobile Originated Call.	360

4.12.2	MFW502: End a Mobile Originated Call.	361

4.12.3	MFW504: Hold and Retrieve a Mobile Originated Call.	362

4.12.4	MFW506: Hold failure for a Mobile Originated Call, followed by Hold and retrieve.	363

4.12.5	MFW508: MO call terminated by remote disconnection.	364

4.13	MMI Component Tests - case : MT Call Processing	365

4.13.1	MFW510: Mobile Terminated call setup, Voice	365

4.13.2	MFW511: Mobile Terminated call Setup, Data	366

4.13.3	MFW512: Mobile Terminated call Setup, FAX	369

4.13.4	MFW513: End of Mobile Terminated Call.	371

4.14	MMI Component Tests - case : MO / MT Call Processing	372

4.14.1	MFW514: MO Call followed by a MT Call, first call put on Hold.	372

4.14.2	MFW516: Using the Send Key, Swap the two calls twice so that the MT call is active and the MO call is held.	374

4.14.3	MFW518: In Call Menu, Swap the two calls twice so that the MT call is active and the MO call is held.	375

4.14.4	MFW520: When in-call, initiate a 2nd call using DTMF tones.	377

4.14.5	MFW522: End  the MT call, In Call Menu, Hold, Activate the held call.	380

4.14.6	MFW524: Reject a MT call, followed by a MO call.	381

4.14.7	MFW526: Missed calls in Idle mode.	383

4.14.8	MFW528: An MT call is rejected.	385

4.14.9	MFW530: In Call Menu, End both calls.	387

4.14.10	MFW532: MO Emergency Call.	388

4.14.11	MFW534: Cancel a MO call during Setup.	390

4.14.12	MFW536: Call Independent Supplementary Services, Register for Forward All Calls.	391

4.15	MMI Component Tests case : In Call Menu	393

4.15.1	MFW538: In Call Menu, make a 2nd call from the Phonebook.	393

4.15.2	MFW540: In Call Menu, End all calls.	395

4.15.3	MFW541: In Call Menu, During an active call, send a text message. End the Call.	396

4.15.4	MFW542: In Call Menu, MO call held, MT call active, send a text message.	397

4.16	MMI Component Tests case : Missed Calls	400

4.16.1	MFW544: Make a call from the missed call list – Main Menu, Recent Calls, Missed Calls	400

4.16.2	MFW546: Make a call from the missed call list. Left Key to display number, Send to initiate Call.	401

4.16.3	MFW547: From Main Menu, Recent Calls, Missed Calls, store callers number then send SMS text message.	402

4.17	MMI Component Tests case : Main Menu, Messages, Voicemail, Set Voicemail	406

4.17.1	MFW548: Set the Voicemail options (number), then call Voicemail.	406

4.17.2	MFW550: Call Voicemail using the “1” key.	408

4.18	Mmi Component Tests case : Main Menu, Phone Settings, set Auto Redial On	409

4.18.1	MFW552: Set the Auto Redial On .	409

4.18.2	MFW554: Auto Redial when called party is busy.	410

4.18.3	MFW556: Override Auto Redial by calling immediately, then Cancel re-dial.	413

4.18.4	MFW558: During a call, increase / decrease the Volume.	415

4.19	MMI Component Tests case : Main Menu, Phone Settings, Set Any Key Answer On	416

4.19.1	MFW560: Set the Any Key Answer On. Accept a MT call by any key.	416

4.20	MMI Component Tests case : Conference Call	418

4.20.1	MFW562: Start a Multiparty Call  from the in-calls option Menu, then Split into a private call. Finally Retrieve the call.	418

4.21	MMI Component Tests case : Advice of Charge	421

4.21.1	MFW564: Mobile Originated Call with Forward Advice of Charge Information	421

4.22	MMI Component Tests case : Main Menu, Phone Settings, set Auto Answer On	422

4.22.1	MFW566: Set the Auto Answer On. Incoming MT call is connected.	422

4.22.2	MFW568: Main Menu, Phone Settings, Change the Language.	424

4.23	MMI Component Tests case : Keypad Lock	426

4.23.1	MFW570: Keypad Locked, make an Emergency Call.	426

4.23.2	MFW572: Keypad Locked, make a MO Call.	427

4.24	MMI Component Tests - case : GSM Command Strings. Supplementary services: Call Divert.	428

4.24.1	MFW580: SS GSM Command String, enter string to Call Forward(Diversion) All Calls.	428

4.24.2	MFW582: SS GSM Command String, enter string to Call Forward(Diversion) If Busy.	429

4.24.3	MFW584: SS GSM Command String, enter string to Call Forward(Diversion) If No Reply.	429

4.24.4	MFW586: SS GSM Command String, enter string to Call Forward(Diversion) If Not Reachable.	430

4.24.5	MFW588: SS GSM Command String, enter string to Call Forward(Diversion) All configured call divertions.	430

4.24.6	MFW590: SS GSM Command String, enter string to Call Forward(Diversion) All Diverts to be activated.	431

4.24.7	MFW592: SS GSM Command String, enter string to Call Forward(Diversion) Unconditionally divert all calls.	431

4.24.8	MFW600: SS GSM Command String, Call Forward(Diversion). Enter Diverting number.	432

4.24.9	MFW602: SS GSM Command String, Register various Call Forward(Diversions) for Voice.	433

4.24.10	MFW650: SS GSM Command String, Register various Call Forward(Diversions) for FAX.	434

4.24.11	MFW652: SS GSM Command String, Register various Call Forward(Diversions) for Data.	435

4.24.12	MFW654: SS GSM Command String, enter string to De-Register All Calls.	437

4.24.13	MFW655: SS GSM Command String, enter string to De-Register All Calls Unconditionally.	437

4.24.14	MFW656: SS GSM Command String, enter string to De-Register if Busy.	438

4.24.15	MFW657: SS GSM Command String, enter string to De-Register if No Reply.	438

4.24.16	MFW658: SS GSM Command String, enter string to De-Register all Configured calls.	439

4.24.17	MFW659: SS GSM Command String, enter string to De-Register if Not Reachable.	439

4.24.18	MFW660: SS GSM Command String, enter string for Voice.	440

4.24.19	MFW662: SS GSM Command String, enter string for Fax.	440

4.24.20	MFW664: SS GSM Command String, enter string for Data.	441

4.24.21	MFW666: SS GSM Command String, De-register various Call Forward(Diversions) for Voice, FAX and Data.	442

4.24.22	MFW604: SS GSM Command String, Register and Activate Call Forward(Diversion) for Voice, Fax and Data.	444

4.24.23	MFW606: SS GSM Command String, Deactivate Call Forward(Diversion) for Voice, Fax and Data.	445

4.24.24	MFW608: SS GSM Command String, Activate Call Forward(Diversion) All Calls.	446

4.25	MMI Component Tests - case :Main Menu, Network Services, Call Divert.	447

4.25.1	MFW610: Main Menu; Network services, Divert; All Calls.	447

4.25.2	MFW611: Main Menu; Network services, Divert; If Busy.	448

4.25.3	MFW612: Main Menu; Network services, Divert; If No Answer.	449

4.25.4	MFW613: Main Menu; Network services, Divert; If Not Reachable.	450

4.25.5	MFW614: Main Menu; Network services, Divert; If No Service.	451

4.25.6	MFW615: Main Menu; Network services, Call Divert for Voice	451

4.25.7	MFW616: Main Menu; Network services, Call Divert for Fax	453

4.25.8	MFW617: Main Menu; Network services, Call Divert for Data	455

4.25.9	MFW618: Main Menu; Network services, Call Divert for Voice if Not Reachable	458

4.25.10	MFW619: Main Menu; Network services, Call Divert for Fax if Not Reachable	460

4.25.11	MFW620: Main Menu; Network services, Call Divert for Data if Not Reachable	462

4.25.12	MFW621: Main Menu; Network Services, Call Divert De-activation for Voice.	463

4.25.13	MFW622: Main Menu; Network Services, Call Divert De-activation for Fax.	465

4.25.14	MFW623: Main Menu; Network Services, Call Divert De-activation for Data.	466

4.25.15	MFW624: Main Menu; Network Services, Call Divert If Not Reachable De-activation for Voice.	467

4.25.16	MFW625: Main Menu; Network Services, Call Divert If Not Reachable De-activation for Fax.	468

4.25.17	MFW626: Main Menu; Network Services, Call Divert If Not Reachable De-activation for Data.	469

4.25.18	MFW627: Main Menu; Network Services, Call Divert – Cancel All.	470

4.25.19	MFW628: Main Menu; Network Services, Call Divert Status for Voice.	471

4.25.20	MFW607: Main Menu; Network Services, Call Divert Status for Fax.	472

4.25.21	MFW609: Main Menu; Network Services, Call Divert Status for Data.	473

4.25.22	MFW629: Main Menu; Network services, Status Check, Hide ID Activate, followed by Deactivate. Test fails.	475

4.26	MMI Component Tests - case : Main Menu, Network Services, Call Barring.	478

4.26.1	MFW630: Main Menu; Network Services, Call Barring.	478

4.26.2	MFW632: Check the Status of Barring All Outgoing Calls.	479

4.26.3	MFW633: Check the Status of Barring Outgoing International Calls.	480

4.26.4	MFW634: Check the Status of Barring Outgoing International except directed to Home PLMN.	481

4.26.5	MFW635: Check the Status of Barring Incoming Calls.	482

4.26.6	MFW636: Check the Status of Barring Incoming calls when Roaming outside the Home PLMN.	483

4.26.7	MFW637: Cancel all Call Barring option	484

4.26.8	MFW638: Change Password option	485

4.26.9	MFW640: Send Each Call Barring SS option to the network.	486

4.27	MMI Component Tests - case : Main Menu, Network Services, Call Back.	487

4.27.1	MFW642: Activate Call Back. Main Menu, Network Services, Call Back, Activate. Test fails.	487

4.28	MMI Component Tests - case : Main Menu, Network Services, Call Waiting	488

4.28.1	MFW644: Main Menu, Network Services, Call Waiting, Check Status.	488

4.29	MMI Component Tests - case : Main Menu, Network Services, Manual Selection.	489

4.29.1	MFW646: Network Sevices, Manual Selection and select vodafone D2	489

4.30	MMI Component Tests - case : Main Menu, Network Services, Call Deflect	490

4.30.1	MFW648: Main Menu, Network Services, Call Deflect, Activate. MT Call Deflected. Test Fails.	490

4.31	MMI Component Tests - case : Main Menu, Recent Calls	493

4.31.1	MFW670: Make a call from the Answered Call List – Main Menu, Recent Calls, Answered Calls.	493

4.31.2	MFW672: Store a number and send SMS from the Answered Call List – Main Menu, Recent Calls, Answered Calls	494

4.31.3	MFW674: Delete record from the Answered Call List – Main Menu, Recent Calls, Answered Calls, Delete.	497

4.31.4	MFW676: Make a call from the Outgoing Call List – Main Menu, Recent Calls, Outgoing Calls.	498

4.31.5	MFW678: Store a number and send SMS from the Outgoing Call List – Main Menu, Recent Calls, Outgoing Calls	499

4.32	MMI Component Tests - case : Main Menu, Messages	502

4.32.1	MFW700: Menu, Messages, Activate Concatenation, Setup Call Centre and Validy Period defaults.	502

4.32.2	MFW702: Setup a MO Originated SMS text string  from the Messages window, Send. Check for error condition.	504

4.32.3	MFW704: From the Messages Window, write a MO Short Message and save it to the SIM.	506

4.32.4	MFW706: From the Messages Window, store a Short Message to SIM memory, then Send message.	508

4.32.5	MFW708: Setup a maximum length MO Short Message of 160 characters in length.	510

4.32.6	MFW710: Setup a MO Concatenated Short Message of 160 7 bit characters in length.	517

4.32.7	MFW712: Continue Concatenated Short Message of 196 characters in length, which leads to two SM’s to the network	523

4.32.8	MFW714: Read the list of Short Messages and then delete all.	527

4.32.9	MFW716: Send an empty MO text message.	528

4.32.10	MFW720: A Class 0 MT Short Message is displayed on the MMI. Afterwards send a MO msg.	529

4.32.11	MFW723: Delete a saved msg. Check for error condition.	531

4.32.12	MFW724: A MT Concatenated Short Message is indicated on the MMI. Read this message and Reply to it.	533

4.32.13	MFW726: A MT Concatenated Short Message is indicated on the MMI. Read this message and call the number.	537

4.32.14	MFW728: A MT Concatenated Short Message is indicated on the MMI. Read this message and extract number.	541

4.32.15	MFW730: A MT Concatenated Short Message is indicated on the MMI. Read this message, edit and Discard.	544

4.32.16	MFW732: A MT Concatenated Short Message is indicated on the MMI. Read this message and Delete.	547

4.32.17	MFW736: Set Voicemail then call Voicemail.	551

4.32.18	MFW740: Activate Cell Broadcast and Read. Start new Broadcast.	553

4.33	MMI Component Tests - case : Main Menu, Phone Settings, GPRS.	555

4.33.1	MFW750: GPRS Attach from the Main Menu.	555

4.33.2	MFW752: GPRS Attach from the Main Menu. Attach is rejected by the network.	559

4.33.3	MFW754: GPRS Detach from the Main Menu.	562

4.33.4	MFW756: SMS CSD Preferred Attach from the Main Menu, same as the default settings, so no change.	563

4.33.5	MFW757: SMS GPRS Attach from the Main Menu..	564

4.33.6	MFW758: SMS CSD Attach from the Main Menu..	566

4.33.7	MFW759: SMS GPRS Prefered Attach from the Main Menu..	568

4.33.8	MFW760: Data Count from the Main Menu, Incoming	570

4.33.9	MFW761: Data Count from the Main Menu, Outgoing	571

4.33.10	MFW764: Main Menu, Define a PDP Context.	572

4.33.11	MFW766: Main Menu, Activate a PDP Context.	573

4.34	MMI Component Tests - case : SIM Application Toolkit.	574

4.34.1	MFW800: SIM Toolkit request to display text, less than 128 bytes.	574

4.34.2	MFW802: SIM Toolkit Setup the Main Menu.	574

4.34.3	MFW804: SIM Toolkit Select Item From the Application Toolkit Menu.	575

4.34.4	MFW806: SIM Toolkit, The Menu Item selected requests a key input.	575

4.34.5	MFW808: SIM Toolkit,.The Menu Item requests a minimum of 3 characters.	576

4.34.6	MFW810: SIM Toolkit,.Send a Short Message.	577

4.34.7	MFW812: SIM Toolkit,.Send a Supplementry Service.	578

4.34.8	MFW814: SIM Toolkit,.Send a Setup Call.	579

4.34.9	MFW816: SIM Toolkit,.Send a Setup Call. First attempt rejected due to busy ME. Automatic Redial. Test Fails.	580

4.34.10	MFW830: SIM Toolkit,.SAT Class e. Test Fails.	581

# Document Control

ã Copyright Condat UK Ltd, 2003.

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement.  reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Condat UK Ltd. Condat UK Ltd accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a license agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Condat UK Ltd.

Condat UK Ltd

Research and Development

38 Melville Street

Edinburgh EH3 7HA

Scotland

Telephone:	+44 (0)131 226 5838

Fax:	+44 (0)131 226 7055

Internet:	http://www.condat.co.uk

## Document History

ID	Author	Date	Status	Remarks

8415.xxx.00.001	EF 	Jan. 29, 02	Being Processed	Initial

Oct 09, 02		Major change to make MMI testing stable. MMI is initialised at the end of the routings so that TAP does not time out. Add CC config signals and initial SIM signal.

EF	Dec. 02	Being Processed	SIM, GMMREG, MNSMS, MNCC saps changed to the cause concept.

EF	Mar. 03	Being Processed	Add Sim Toolkit tests

EF	Jul. 03	Being Processed	Add data calls, Multiparty, Call Back, Manual network selection, MT SMS Concatenation.

DR	Jul. 03	Being Processed	Add Menu driven Call Divert combinations, Auto Answer.

EF	Aug. 03	Being Processed	Comment all key presses and Menus. Re-check old tests and update. Add GPRS PDP activation.

EF	Oct. 03	Being Processed	Add some more Emergency Call tests & keypad lock.plus changes to Menu and keypad entry.

## References

[GSM 2.30]	ETS 300 511: July 1995 (GSM 02.30 version 4.13.0)
Man-Machine Interface (MMI) of the Mobile Station (MS), ETSI

[GSM 11.11]	TS 100.977. Release 1999, GSM Specification of the SIM-ME interface. Version 8.2.0.

8415.028.99.300	TCC Test Case Control, User Guide

[C_6147.302]	Test Case Script Language User Guide; Condat

      - VSI/PEI - Frame/Body Interfaces

8434.100.01.001	FUG Frame Users Guide

8350.301_sa	Specifying Service Access Points

  - PCO2 Userguide

  - Dynamic Entity Configuration Commands

G23 Basic MMI (BMI) Test Documantation

# Introduction

- G23 is a software package implementing Layers 2 and 3 of the ETSI-defined GSM air interface signaling protocol, and as such represents the part of a GSM mobile station's protocol software which is both, platform and manufacturer independent. Therefore, G23 can be viewed as a building block providing standardized functionality through generic interfaces for easy integration.

The G23 suite of products consists of the following items:

- Layers 2 and 3 for speech & short message services,

- Layers 2 and 3 for fax & data services,

- Application Control Interface/AT Command Interface,

- MMI and MMI Framework (MFW) and

- Test and integration support tools.

This document establishes a suite of tests for the MMI environment, and includes the simulation of key presses and signalling with the Protocol Stack interface. The testing environment runs under Windows and an executable file is created linking the Protocol Stack and the MMI entities. The MMI can be viewed as a basic MMI element and an MFW framework, together with an ACI SAP for AT commands and key press simulation. A front end environment is provided by “TapCaller”, which records the results of testing, and which loads the exe file mentioned above, together with the test execution program “TAP” and then runs the selected test/tests, reporting a pass or fail. Traces from the MMI and Protocol Stack are stored in debug files (.dbg) and can be viwed from “TapCaller”.

## Abbreviations

ACI	Application Control Interface
AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CCI	Compression and Ciphering Interface
CHAP	Challenge Handshake Authentication Protocol
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DCOMP	Identifier of the user data compression algorithm used for the N-DPU
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

E	Extension bit
EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GACI	GPRS Application Control Interface
GMM	GPRS Mobility Management
GP	Guard Period
GRR	GPRS RR
GSM	Global System for Mobile Communication

HDLC	High-level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
IP	Internet Protocol
IPCP	Internet Protocol Control Protocol 
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Ciphering Key

L	Length Indicator
LAI	Location Area Information
LCP 	Link Control Protocol 
LISR	Low level Interrupt Service Routine
LLC	Logical Link Control
LPD	Link Protocol Discriminator
LQM	Link Quality Monitoring

M	More bit used to indicate the last segment of N-DPU
MAC	Medium Access Control
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MT	Mobile Termination

N(R)	Receive Number
N(S)	Send Number
NC	Network Control
NCC	National Colour Code
NCP 	Network Control Protocol
NECI	New Establishment Causes included
N-PDU	Network Protocol Data Unit
NSAPI	Network Layer Service Access Point Identifier

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PACCH	Packet Associated Control Channel
PAP	Password Authentication Protocol
PBCCH	Packet BCCH
PCCCH	Packet CCCH
PCOMP	Identifier of the protocol control information compression algorithm used for the N-DPU
PDCH	Packet Data Channel
PDP	Packet Data Protocol e.g. IP or X.25
PDTCH	Packet Data Traffic Channel
PRACH	Packet RACH
PSI	Packet System Information
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Data Unit
PL	Physical Layer
PLMN	Public Land Mobile Network
PPC	Packet Physical Convergence
PPP 	Point-to-Point Protocol
PTP	Point to Point

QoS	Quality of Service

RACH	Random Access Channel
REJ	Reject Frame
RLC	Radio Link Control
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Stand alone Dedicated Control Channel
SDU	Service Data Unit
SGSN	Serving GPRS Support Node
SIM	Subscriber Identity Module
SM	Session Management
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SNDCP	Subnetwork Dependant Convergence Protocol
SNSM	SNDCP-SM
SS	Supplementary Services

TAP	Test Application Program
TBF	Temporary Block Flow
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TCP	Transmission Control Protocol
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TFI	Temporary Flow Identifier
TLLI	Temporary Logical Link Identifier
TMSI	Temporary Mobile Subscriber Identity
TOM	Tunnelling of Messages
TQI	Temporary Queuing Identifier

UA	Unnumbered Acknowledgement Frame
UART	Universal Asynchronous Receiver Transmitter
UI	Unnumbered Information Frame
USF	Uplink State Flag

V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable
VPLMN	Visited Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Protocol Stack

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for GPRS consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity.

MMI

- Figure 2-1: Architecture of the GSM/GPRS protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the GPRS protocol stack are:

## GRR (RLC/MAC) – Radio Link Control/Medium Access Control

This layer contains two functions: The Radio Link Control function provides a radio-solution-dependent reliable link. The Medium Access Control function controls the access signalling (request and grant) procedures for the radio channel, and the mapping of LLC frames onto the GSM physical channel.

## LLC – Logical Link Control

The LLC entity provides multiple highly reliable logical links for asynchronous data transfer between the MS and the network. It supports variable-length information frames, acknowledged and unacknowledged data transfer, flow and sequence control, error detection and recovery, notification of unrecoverable errors, user identity confidentiality, and ciphering of user and signaling data.

## GMM – GPRS Mobility Management

The GMM entity provides procedures for the mobility of the MS, such as informing the network of its present location, and user identity confidentiality. It manages the GMM context (attach, detach, routing area updating), supports security functions such as authentication of user and MS, controls ciphering of data, and initiates the response to paging messages.

## SM – Session Management

The main function of the session management (SM) is to support PDP context handling of the user terminal. Session Management activates, modifies and deletes the contexts for packet data protocols (PDP). Session Management services are provided at the SMREG-SAP and the SNSM-SAP for anonymous and non-anonymous access. The non-anonymous and anonymous access procedures for PDP context activation and PDP context deactivation are available at the SMREG-SAP. In addition there exists a PDP context modification for non-anonymous PDP contexts.

## SNDCP - Subnetwork Dependant Convergence Protocol

- SNDCP carries out all functions related to transfer of Network layer Protocol Data Units (N-PDUs) over GPRS in a transparent way. SNDCP helps to improve channel efficiency by means of compression techniques. The set of protocol entities above SNDCP consists of commonly used network protocols. They all use the same SNDCP entity, which then performs multiplexing of data coming from different sources to be sent using the service provided by the LLC layer.

## GACI – GPRS Application Control Interface

The GACI is the GPRS extension of the ACI. It is specified in GSM 07.07 and 07.60. It is responsible for processing of the GPRS related AT Commands to setup, activate and deactivate the PDP context parameter. It also provides functionality for the interworking between GMM/SM/SNDCP and a packet oriented protocol like PPP.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the GPRS. The driver has to be able to perform flow control.

## TOM – Tunnelling of Messages

The TOM entity is present if and only if HS136 is supported (the feature flag FF_HS136 is enabled).

The main function of TOM is to tunnel non-GSM signalling messages between the MS and the SGSN. The only non-GSM signalling which is currently supported by TOM is for the EGPRS-136 system (according to TIA/EIA-136-376). Data transfer in both uplink and downlink direction is possible. Two different priorities (high, low) of signalling data transfer are supported. TOM uses the unacknowledged mode of LLC and the acknowledged mode of GRR (RLC/MAC).

## Protocol Stack Entities

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages. Additionally the reception of cell broadcast short messages is included.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

The GSM specification do not set out any implementation of the Protocol Stack. All entities except the Man Machine Interface and Physical Layer are implemented as part of the Protocol Stack.

## Test Script description

This document defines the tests for the whole protocol stack using the MMI Framework. The following table describes the interface with the PS Entities and the corresponding tests that apply.

| ### I/F with Entities | Test Section |
| --- | --- |
| L2R/RLP/RA/T30 | MT Call Processing, Sim Toolkit |
| SIM/GMM/UART | PIN tests from Power On, PIN and PUK entering from menus, GSM Strings |
| CC/SS/SMS | Call Processing tests for MO and MT, SIM Toolkit |
| SMS | Messages from the Menu, SIM Toolkit |
| GMM | Setting GPRS options in Settings Menu |
| SS | Call Divert options from the Menu or GSM strings, SIM Toolkit |
| SM | PDP Context Activation tests |

# Parameters

DECLARATION (A_ECC_FIELD )

- DECLARATION (APN_BUF_1)

DECLARATION (A_AD_FIELD_CI_DISABLED )

DECLARATION (A_AD_FIELD_CI_ENABLED )

DECLARATION (BC_PARA_ASYNC_1200)

DECLARATION (BC_PARA_SPEECH)

DECLARATION (BC_PARA_FAX)

DECLARATION (BC_PARA_NO_SERVICE)

DECLARATION (CBCH_MSG_ID_ARR)

DECLARATION (CBCH_DCS_ID_ARR)

DECLARATION (CLED_PARTY_112)

DECLARATION (CLED_PARTY_01317180537)

DECLARATION (CLED_PARTY_9876543210_FRED)

DECLARATION (CLED_PARTY_SUB)

DECLARATION (CONNECTED_PARTY_SUB_NONE)

DECLARATION (CONNECTED_NUMBER_112)
DECLARATION (CONNECTED_NUMBER_01317180537)
DECLARATION (CLG_PARTY_01317180537)

DECLARATION (CLG_PARTY_SUB)

DECLARATION (CLED_PARTY_654321)

DECLARATION (CH_SIG_ONLY)

DECLARATION (CH_DATA_9_6)

DECLARATION (CLED_PARTY_SUB_NONE)

DECLARATION (CONNECTED_NUMBER_654321)

DECLARATION (CLG_PARTY_654321)

- DECLARATION (CLED_PARTY_ERIC_NT)

- DECLARATION (CLED_PARTY_ERIC_T)

- DECLARATION (CLED_PARTY_VOICEMBX)

DECLARATION (CHANNEL_MODE_1)

DECLARATION (DA_654321)

DECLARATION (DA_654321_NB)

DECLARATION (DEREGISTER_SS_DIVERT_ALLCALLS)

DECLARATION (DAY_25)

DECLARATION (DIVERT_STATUS_CALLS_CONTENTS_V)

DECLARATION (DIVERT_STATUS_CALLS_CONTENTS_F)

DECLARATION (DIVERT_STATUS_CALLS_CONTENTS_D)

DECLARATION (DIVERT_STATUS_IFBUSY_CONTENTS_V)

DECLARATION (DIVERT_STATUS_IFBUSY_CONTENTS_F)

DECLARATION (DIVERT_STATUS_IFBUSY_CONTENTS_D)

DECLARATION (DIVERT_STATUS_IFNOREPLY_CONTENTS_V)

DECLARATION (DIVERT_STATUS_IFNOREPLY_CONTENTS_F)

DECLARATION (DIVERT_STATUS_IFNOREPLY_CONTENTS_D)

DECLARATION (DIVERT_STATUS_IFNONETWORK_CONTENTS_V)

DECLARATION (DIVERT_STATUS_IFNONETWORK_CONTENTS_F)

DECLARATION (DIVERT_STATUS_IFNONETWORK_CONTENTS_D)

- DECLARATION (EC_CODES )

DECLARATION (F_SIM_SRV_4)

DECLARATION (F_SIM_SRV)

DECLARATION (F_SIM_SRV_4_15_16)

DECLARATION (F_SIM_SRV_PHB)

DECLARATION (F_SIM_SRV_FDN_PHB)

DECLARATION (FACILITY_DIVERT_ARR)

DECLARATION (FACILITY_REGISTER)

DECLARATION (FAC_DIVERT_ALLCALLS)

DECLARATION (FAC_DEACTIVATE_ALLCALLS)

DECLARATION (FAC_DIVERT_ALLCALLS_VOICE)

DECLARATION (FAC_DIVERT_ALLCALLS_V)

DECLARATION (FAC_DIVERT_ALLCALLS_F)

DECLARATION (FAC_DIVERT_ALLCALLS_D)

DECLARATION (FAC_DIVERT_IFBUSY)

DECLARATION (FAC_DIVERT_IFBUSY_V)

DECLARATION (FAC_DIVERT_IFBUSY_F)

DECLARATION (FAC_DIVERT_IFBUSY_D)

DECLARATION (FAC_DIVERT_IFNOREPLY)

DECLARATION (FAC_DIVERT_IFNOREPLY_V)

DECLARATION (FAC_DIVERT_IFNOREPLY_F)

DECLARATION (FAC_DIVERT_IFNOREPLY_D)

DECLARATION (FAC_DIVERT_IFNOSERVICE_V)

DECLARATION (FAC_DIVERT_IFNOSERVICE_F)

DECLARATION (FAC_DIVERT_IFNOSERVICE_D)

DECLARATION (FAC_DIVERT_IFNOTREACHABLE)

DECLARATION (FAC_DIVERT_IFNOTREACHABLE_V)

DECLARATION (FAC_DIVERT_IFNOTREACHABLE_F)

DECLARATION (FAC_DIVERT_IFNOTREACHABLE_D)

DECLARATION (FAC_DIVERT_ALL_CONDITIONAL_V)

DECLARATION (FAC_DIVERT_ALL_CONDITIONAL_F)

DECLARATION (FAC_DIVERT_ALL_CONDITIONAL_D)

DECLARATION (FAC_DIVERT_ALLCONFIGURED_V)

DECLARATION (FAC_DIVERT_ALLCONFIGURED_F)

DECLARATION (FAC_DIVERT_ALLCONFIGURED_D)

DECLARATION (FAC_DIVERT_ALLCONFIGURED)

DECLARATION (FAC_DIVERT_ALLDIVERTS)

DECLARATION (FAC_DIVERT_ALLDIVERTS_V)

DECLARATION (FAC_DIVERT_ALLDIVERTS_F)

DECLARATION (FAC_DIVERT_ALLDIVERTS_D)

DECLARATION (FAC_DIVERT_STATUS_CALLS_V)

DECLARATION (FAC_DIVERT_STATUS_CALLS_F)

DECLARATION (FAC_DIVERT_STATUS_CALLS_D)

DECLARATION (FAC_DIVERT_STATUS_IFBUSY_V)

DECLARATION (FAC_DIVERT_STATUS_IFBUSY_F)

DECLARATION (FAC_DIVERT_STATUS_IFBUSY_D)

DECLARATION (FAC_DIVERT_STATUS_IFNOREPLY_V)

DECLARATION (FAC_DIVERT_STATUS_IFNOREPLY_F)

DECLARATION (FAC_DIVERT_STATUS_IFNOREPLY_D)

DECLARATION (FAC_DIVERT_STATUS_IFNONETWORK_V)

DECLARATION (FAC_DIVERT_STATUS_IFNONETWORK_F)

DECLARATION (FAC_DIVERT_STATUS_IFNONETWORK_D)

DECLARATION (FAC_DIVERT_UNCONDITIONALLY)

DECLARATION (FAC_DIVERT_UNCONDITIONALLY_V)

DECLARATION (FAC_DIVERT_UNCONDITIONALLY_F)

DECLARATION (FAC_DIVERT_UNCONDITIONALLY_D)

DECLARATION (FAC_BAR_OG_CALLS)

DECLARATION (FAC_BAR_INTER_CALLS)

DECLARATION (FAC_BAR_INTER_BUT_HOME_CALLS)

DECLARATION (FAC_BAR_IC_CALLS)

DECLARATION (FAC_BAR_IC_ROAMING_CALLS)

DECLARATION (FAC_CANCEL_ALLBAR)

DECLARATION (FAC_REG_PW)

DECLARATION (FAC_REG_PW_CONTENT)

DECLARATION (FAC_PW_RES)

DECLARATION (FAC_PW_RES_CONTENT)

DECLARATION (FAC_SET_WAITING_ENABLED)

DECLARATION (FAC_ERASE_ALLDIVERTS)

DECLARATION (FAC_ERASE_IFNOSERVICE_V)

DECLARATION (FAC_ERASE_IFNOSERVICE_F)

DECLARATION (FAC_ERASE_IFNOSERVICE_D)

DECLARATION (FAC_ERASE_ALLDIVERTS_V)

DECLARATION (FAC_ERASE_UNCONDITIONALLY_V)

DECLARATION (FAC_ERASE_IFBUSY_V)

DECLARATION (FAC_ERASE_IFNOREPLY_V)

DECLARATION (FAC_ERASE_ALLCONFIGURED_V)

DECLARATION (FAC_ERASE_IFNOTREACHABLE_V)

DECLARATION (FAC_ERASE_ALLDIVERTS_F)

DECLARATION (FAC_ERASE_UNCONDITIONALLY_F)

DECLARATION (FAC_ERASE_IFBUSY_F)

DECLARATION (FAC_ERASE_IFNOREPLY_F)

DECLARATION (FAC_ERASE_ALLCONFIGURED_F)

DECLARATION (FAC_ERASE_IFNOTREACHABLE_F)

DECLARATION (FAC_ERASE_ALLDIVERTS_D)

DECLARATION (FAC_ERASE_UNCONDITIONALLY_D)

DECLARATION (FAC_ERASE_IFBUSY_D)

DECLARATION (FAC_ERASE_IFNOREPLY_D)

DECLARATION (FAC_ERASE_ALLCONFIGURED_D)

DECLARATION (FAC_ERASE_IFNOTREACHABLE_D)

DECLARATION (ERASE_ALLDIVERTS_CONTENTS)

DECLARATION (ERASE_ALLDIVERTS_CONTENTS_V)

DECLARATION (ERASE_IFNOSERVICE_CONTENTS_V)

DECLARATION (ERASE_IFNOSERVICE_CONTENTS_F)

DECLARATION (ERASE_IFNOSERVICE_CONTENTS_D)

DECLARATION (ERASE_UNCONDITIONALLY_CONTENTS_V)

DECLARATION (ERASE_IFBUSY_CONTENTS_V)

DECLARATION (ERASE_IFNOREPLY_CONTENTS_V)

DECLARATION (ERASE_ALLCONFIGURED_CONTENTS_V)

DECLARATION (ERASE_IFNOTREACHABLE_CONTENTS_V)

DECLARATION (ERASE_ALLDIVERTS_CONTENTS_F)

DECLARATION (ERASE_UNCONDITIONALLY_CONTENTS_F)

DECLARATION (ERASE_IFBUSY_CONTENTS_F)

DECLARATION (ERASE_IFNOREPLY_CONTENTS_F)

DECLARATION (ERASE_ALLCONFIGURED_CONTENTS_F)

DECLARATION (ERASE_IFNOTREACHABLE_CONTENTS_F)

DECLARATION (ERASE_ALLDIVERTS_CONTENTS_D)

DECLARATION (ERASE_UNCONDITIONALLY_CONTENTS_D)

DECLARATION (ERASE_IFBUSY_CONTENTS_D)

DECLARATION (ERASE_IFNOREPLY_CONTENTS_D)

DECLARATION (ERASE_ALLCONFIGURED_CONTENTS_D)

DECLARATION (ERASE_IFNOTREACHABLE_CONTENTS_D)

DECLARATION (CALL_WAITING_CONTENTS)

DECLARATION (RESULT_ERASE_ALLDIVERTS_V)

DECLARATION (RESULT_ERASE_ALL_DIVERTS_CONTENTS)

DECLARATION (HOUR_12)

- DECLARATION (IMSI)

- DECLARATION (IMSI_ARRAY)

DECLARATION (MONTH_05)
DECLARATION (MINUTE_34)

- DECLARATION (MCC_1)

- DECLARATION (MCC_262)

- DECLARATION (MNC_1)

- DECLARATION (MNC_02)

- DECLARATION (MNC_14)

- DECLARATION (NO_EC_CODES)

- DECLARATION (NO_PREF_LANG)

DECLARATION (NUM_112)

DECLARATION (NUM_VOICEMBX_121)

DECLARATION (NUM_01317180537)

DECLARATION (NUM_9876543210)

DECLARATION (NUM_654321)

- DECLARATION (NUM_X)

- DECLARATION (PREF_PLMN)

- DECLARATION (PREF_PLMN_ARRAY)

- DECLARATION (PUK_1_VALUE)

- DECLARATION (PUK_2_VALUE_11223344)

DECLARATION (PIN_1_VALUE_1234)

DECLARATION (PIN_1_VALUE_B)

DECLARATION (PIN_1_VALUE_11111111)

DECLARATION (PIN_1_VALUE_1239)

DECLARATION (PIN_1_VALUE_1233)

DECLARATION (PHB_ADN_RECORD_1)

DECLARATION (PHB_ADN_RECORD_2)

DECLARATION (PHB_ADN_RECORD_3)

DECLARATION (PHB_ADN_RECORD_4)

DECLARATION (PHB_ADN_RECORD_5)

DECLARATION (PHB_ADN_RECORD_6)

DECLARATION (PHB_ADN_RECORD_EMPTY)

DECLARATION (PHB_ADN_RECORD_1_DELETE)

DECLARATION (PHB_ADN_RECORD_1_CHANGE)

DECLARATION (PHB_ADN_RECORD_ERIC_MT_SAVE)

DECLARATION (PHB_ADN_RECORD_ERIC_MO_SAVE)

DECLARATION (PHB_ADN_RECORD_SID)

- DECLARATION (PLMN_1)

- DECLARATION (PLMN_262_14)

- DECLARATION (PLMN_262_02)

DECLARATION (RED_PARTY)

DECLARATION (RED_PARTY_SUB)

DECLARATION (RELEASE_COMPLETE_DEACTIVATE)

DECLARATION (REGISTER_SS_BAR_OG_CALLS)

DECLARATION (REGISTER_SS_BAR_IC_CALLS)

DECLARATION (REGISTER_SS_BAR_INTER_CALLS)

DECLARATION (REGISTER_SS_BAR_INTER_BUT_HOME_CALLS)

DECLARATION (REGISTER_SS_BAR_IC_ROAMING_CALLS)

DECLARATION (REGISTER_SS_CANCEL_ALLBAR)

DECLARATION (REGISTER_SS_DIVERT_ALLCALLS)

DECLARATION (REGISTER_SS_DIVERT_ALLCALLS_V)

DECLARATION (REGISTER_SS_DIVERT_ALLCALLS_F)

DECLARATION (REGISTER_SS_DIVERT_ALLCALLS_D)

DECLARATION (REGISTER_SS_DIVERT_IFBUSY)

DECLARATION (REGISTER_SS_DIVERT_IFBUSY_V)

DECLARATION (REGISTER_SS_DIVERT_IFBUSY_F)

DECLARATION (REGISTER_SS_DIVERT_IFBUSY_D)

DECLARATION (REGISTER_SS_DIVERT_IFNOREPLY)

DECLARATION (REGISTER_SS_DIVERT_IFNOREPLY_V)

DECLARATION (REGISTER_SS_DIVERT_IFNOREPLY_F)

DECLARATION (REGISTER_SS_DIVERT_IFNOREPLY_D)

DECLARATION (REGISTER_SS_DIVERT_IFNOTREACHABLE)

DECLARATION (REGISTER_SS_DIVERT_IFNOTREACHABLE_V)

DECLARATION (REGISTER_SS_DIVERT_IFNOTREACHABLE_F)

DECLARATION (REGISTER_SS_DIVERT_IFNOTREACHABLE_D)

DECLARATION (REGISTER_SS_DIVERT_ALLCONFIGURED)

DECLARATION (REGISTER_SS_DIVERT_ALLCONFIGURED_V)

DECLARATION (REGISTER_SS_DIVERT_ALLCONFIGURED_F)

DECLARATION (REGISTER_SS_DIVERT_ALLCONFIGURED_D)

DECLARATION (REGISTER_SS_DIVERT_ALL_CONDITIONAL_V)

DECLARATION (REGISTER_SS_DIVERT_ALL_CONDITIONAL_F)

DECLARATION (REGISTER_SS_DIVERT_ALL_CONDITIONAL_D)

DECLARATION (REGISTER_SS_DIVERT_ALLDIVERTS)

DECLARATION (REGISTER_SS_DIVERT_ALLDIVERTS_V)

DECLARATION (REGISTER_SS_DIVERT_ALLDIVERTS_F)

DECLARATION (REGISTER_SS_DIVERT_ALLDIVERTS_D)

DECLARATION (REGISTER_SS_DIVERT_ALLCALLS_VOICE)

DECLARATION (REGISTER_SS_DIVERT_UNCONDITIONALLY)

DECLARATION (REGISTER_SS_DIVERT_UNCONDITIONALLY_V)

DECLARATION (REGISTER_SS_DIVERT_UNCONDITIONALLY_F)

DECLARATION (REGISTER_SS_DIVERT_UNCONDITIONALLY_D)

DECLARATION (REGISTER_SS_DIVERT_IFNOSERVICE_V)

DECLARATION (REGISTER_SS_DIVERT_IFNOSERVICE_F)

DECLARATION (REGISTER_SS_DIVERT_IFNOSERVICE_D)

DECLARATION (RELEASE_COMPLETE_FACILITY)

DECLARATION (RELEASE_COMPLETE_ARR)

DECLARATION (RELEASE_COMPLETE_DEACTIVARR)

- DECLARATION (REGISTER_SS_1_ARR)

DECLARATION (CALL_WAITING_RETURN_RESULT)

DECLARATION (CALL_WAITING_RET_RES_ARR)

DECLARATION (CALL_FORWARD_RETURN_RESULT)

DECLARATION (CALL_FORWARD_RET_RES_ARR)

DECLARATION (SC_ADDR_1)

DECLARATION (SC_ADDR_1_NB)

DECLARATION (SMS_SDU_1)

DECLARATION (SMS_MSG_1)

DECLARATION (SMS_NO_TEXT)

DECLARATION (SMS_NO_TEXT_CONTENT)

DECLARATION (SMS_SDU_2)

DECLARATION (SMS_MSG_2)

DECLARATION (SMS_PHB)

DECLARATION (SMS_SDU_CONC_1)

DECLARATION (SMS_SDU_MAX_1)

DECLARATION (SMS_MSG_CONC_1)

DECLARATION (SMS_MSG_MAX_1)

DECLARATION (SMS_SDU_CONC_2)

DECLARATION (SMS_MSG_CONC_2)

DECLARATION (SMS_SDU_EMPTY)

DECLARATION (SMS_MSG_EMPTY)

DECLARATION (SMS_MSG_MT_CONCAT)

DECLARATION (SMS_MSG_MT_CONCAT_SDU)

DECLARATION (SECOND_56)

DECLARATION (SMS_SDU_ERIC_NAMES)

DECLARATION (SMS_MSG_ERIC_NAMES)

DECLARATION (SIM_SERV_PHASE_2_ADN_BDN)

DECLARATION (SIM_SERV_PHASE_2)

DECLARATION (SMS_SDU_MISSED_CALLS)

DECLARATION (SMS_MSG_MISSED_CALLS)

DECLARATION (SMS_SDU_OUTGOING_CALLS)

DECLARATION (SMS_MSG_OUTGOING_CALLS)

DECLARATION (STK_SUPPORTED)

- DECLARATION (SIM_TOOLKIT_PROFILE)

DECLARATION (SIM_CPHS_FIELD)

- DECLARATION (SMREG_APN_0_S)

- DECLARATION (SMREG_APN_1)

- DECLARATION (SDU_ARRAY)

- DECLARATION (SDU_SHOW)

- DECLARATION (SDU)

- DECLARATION (SDU_1)

- DECLARATION (SDU_2)

- DECLARATION (SDU_3)

- DECLARATION (SNDCP_CHANNEL)

- DECLARATION (VER_SS_1_ARR)

- DECLARATION (VERSION_SS)

DECLARATION (VP_A9801071234564)
DECLARATION (YEAR_02)

DECLARATION (SMS_SDU_MO_ABS)

DECLARATION (SMS_SDU_MO_ABS_BUF)

DECLARATION (STK_DISPLAY_TEXT_SHORT)

DECLARATION (STK_DISPLAY_TEXT_SHORT_CONTENT)

DECLARATION (STK_TERM_RESP_DISPLAY_TEXT)

DECLARATION (STK_TERM_RESP_DISPLAY_TEXT_CONTENT)

DECLARATION (STK_SETUP_MENU_CONTENT)

DECLARATION (STK_SETUP_MENU)

DECLARATION (STK_TERM_RESP_SETUP_MENU)

DECLARATION (STK_TERM_RESP_SETUP_MENU_CONTENT)

DECLARATION (STK_SELECT_ITEM_CONTENT)

DECLARATION (STK_SELECT_ITEM)

DECLARATION (STK_TERM_RESP_SELECT_ITEM)

DECLARATION (STK_TERM_RESP_SELECT_ITEM_CONTENT)

DECLARATION (STK_GET_INKEY)

DECLARATION (STK_GET_INKEY_CONTENT)

DECLARATION (STK_TERM_RESP_GET_INKEY)

DECLARATION (STK_TERM_RESP_GET_INKEY_CONTENT)

DECLARATION (STK_GET_INPUT_CONTENT)

DECLARATION (STK_GET_INPUT)

DECLARATION (STK_TERM_RESP_GET_INPUT_CONTENT)

DECLARATION (STK_TERM_RESP_GET_INPUT)

DECLARATION (STK_SEND_SMS)

DECLARATION (STK_SEND_SMS_CONTENT)

DECLARATION (STK_TERM_RESP_SEND_SMS)

DECLARATION (STK_TERM_RESP_SEND_SMS_CONTENT)

DECLARATION (SMS_SDU_STK)

DECLARATION (SMS_SDU_STK_CONTENT)

DECLARATION (STK_SEND_SS)

DECLARATION (STK_SEND_SS_CONTENT)

DECLARATION (STK_TERM_RESP_SEND_SS)

DECLARATION (STK_TERM_RESP_SEND_SS_CONTENT)

DECLARATION (STK_DIVERT_ALLCALLS)

DECLARATION (STK_DIVERT_ALLCALLS_CONTENT)

DECLARATION (STK_SETUP_CALL)

DECLARATION (STK_SETUP_CALL_CONTENT)

DECLARATION (STK_TERM_RESP_SETUP_CALL)

DECLARATION (STK_TERM_RESP_SETUP_CALL_CONTENT)

DECLARATION (STK_TERM_RESP_SETUP_CLASSE)

DECLARATION (STK_TERM_RESP_SETUP_CLASSE_CONTENT)

DECLARATION (STK_CLED_PARTY_01317180537)

DECLARATION (STK_SETUP_CALL_BUSY)

DECLARATION (STK_SETUP_CALL_BUSY_CONTENT)

- DECLARATION (STK_OPEN_CHANNEL_IMM_CSD)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_CONTENT)

DECLARATION (STK_OPEN_CHANNEL_IM_UDP)

DECLARATION (STK_OPEN_CHANNEL_IM_UDP_CONTENT)

DECLARATION (STK_OPEN_CHANNEL_IM_L2R)

DECLARATION (STK_OPEN_CHANNEL_IM_L2R_CONTENT)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_DATA_CIRCUIT_ASYNC_UDI)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_0)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_DATA_CIRCUIT_SYNCH_UDI)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_1)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_PAD_ACCESS_ASYNCH_UDI)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_2)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_PACKET_ACCESS_SYNCH_UDI)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_3)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_DATA_CIRCUIT_ASYNCH_RDI)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_4)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_DATA_CIRCUIT_SYNCH_RDI)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_5)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_PAD_ACCESS_ASYNCH_RDI)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_6)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_PACKET_ACCESS_SYNCH_RDI)

DECLARATION (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_7)

DECLARATION (BC_PARA_ASYNC_9600)

DECLARATION (BC_PARA_SYNC_9600)

DECLARATION (BC_PARA_PAD_ACCESS_ASYNC)

DECLARATION (CHAN_MODE_DATA_9_6)

DECLARATION (FAC_MULTIPARTY)

DECLARATION (FAC_MULTIPARTY_SPLIT)

DECLARATION (MULTIPARTY_ARR)

DECLARATION (MULTIPARTY_SPLIT_ARR)

DECLARATION (MULTIPARTY_RETURN_RESULT)

DECLARATION (MULTIPARTY_RET_RES_ARR)

DECLARATION (MULTIPARTY_SPLIT_RETURN_RESULT)

DECLARATION (MULTIPARTY_SPLIT_RET_RES_ARR)

DECLARATION (FAC_MULTIPARTY_RETRIEVE)

DECLARATION (FAC_AOCI)

DECLARATION (AOCI_ARR)

DECLARATION (MULTIPARTY_RETRIEVE_ARR)

DECLARATION (MULTIPARTY_RETRIEVE_RETURN_RESULT)

DECLARATION (MULTIPARTY_RETRIEVE_RET_RES_ARR)

DECLARATION (TEST_FAC_INF)

DECLARATION (INTERROGATE_SS)

DECLARATION (INTERROGATE_SS_ARR)

DECLARATION (INTERROGATE_SS_OG)

DECLARATION (INTERROGATE_SS_OG_CONTENT)

DECLARATION (INTERROGATE_SS_OG_INTER)

DECLARATION (INTERROGATE_SS_OG_INTER_CONTENT)

DECLARATION (INTERROGATE_SS_OG_INTER_BUT_HOME)

DECLARATION (INTERROGATE_SS_OG_INTER_BUT_HOME_CONTENT)

DECLARATION (INTERROGATE_SS_IC)

DECLARATION (INTERROGATE_SS_IC_CONTENT)

DECLARATION (INTERROGATE_SS_IC_ROAMING)

DECLARATION (INTERROGATE_SS_IC_ROAMING_CONTENT)

DECLARATION (INTERROGATE_SS_CLIR)

DECLARATION (INTERROGATE_SS_CLIR_CONTENT)

DECLARATION (INTERROGATE_SS_RESULT)

DECLARATION (INTERROGATE_SS_RESULT_ARR)

DECLARATION (PLMN_LIST)

DECLARATION (PLMN_NULL)

DECLARATION (LAC_LIST)

DECLARATION (FORB_PLMN_LIST)

DECLARATION (RX_LEVEL_LIST)

DECLARATION (GPRS_STATUS_LIST)

DECLARATION (REGISTER_CLIR)

DECLARATION (REGISTER_CLIR_ARR)

DECLARATION (DEREGISTER_CLIR)

DECLARATION (DEREGISTER_CLIR_ARR)

DECLARATION (RELEASE_COMPLETE_CLIR)

DECLARATION (RELEASE_COMPLETE_CLIR_ARR)

DECLARATION (CH_MODE_FAX_TCH_F)

DECLARATION (CH_MODE_SPEECH_TCH_F)

DECLARATION (CH_MODE_DATA_TCH_F)

DECLARATION (CAP_DCS0)

DECLARATION (CAP_DIS0)

DECLARATION (CAPS_DCS0)

DECLARATION (TP_OA)

DECLARATION (TP_SCTS)

DECLARATION (TP_UD1)

DECLARATION (DTI2_DTR_PARAMETERS1)

DECLARATION (SERIAL_LINE_STATES_1)

DECLARATION (DTI2_SDU_1)

DECLARATION (SMS_DELIVER_CONC_1)

DECLARATION (SMS_DELIVER_CONC_2)

DECLARATION (SMS_SDU_MT_CONC_1)

DECLARATION (SMS_SDU_MT_CONC_2)

DECLARATION (SIM_CPHS_VMW_FIELD)

DECLARATION (CBCH_MSG_IDS)

DECLARATION (CBCH_DCS_IDS)

DECLARATION (CBCH_MSG_1)

DECLARATION (SMS_CONFIG)

DECLARATION (CBCH_MSG_IDS_NEW_BC)

DECLARATION (COM_PARS)

DECLARATION (SMREG_QOS_0)

DECLARATION (PDP_ADDRESS_1)

DECLARATION (SMREG_APN_NAME)

DECLARATION (SMREG_CODING_OPTIONS)

DECLARATION (ADDRESS_BUF_1)

DECLARATION (APN)

/* AT command:  COPS*/

STRING(C_COPS_0, "AT+COPS=0\r")

STRING(C_COPS_1, "AT+COPS=1\r")

STRING(C_PLUS_COPS_MAN_NUM, "AT+COPS=1,2,\"26202\"" )

STRING(C_COPS_Q, "AT+COPS=?\r")

- STRING(M_OK, "OK")

STRING(M_PERCENT_SIMREM_ERR_0, "%SIMREM: 0" )

STRING(M_RING, "RING")

- BYTE LM_RING_LEN 4

- BYTE LM_PERCENT_SIMREM_ERR_0 10

- BYTE LM_OK 5

- BYTE LC_COPS 10

- BYTE LC_PLUS_COPS_MAN_NUM 19

BYTE TI_MO_0						0x00

BYTE TI_MO_1						0x01

BYTE TI_MT_9						0x09

BYTE TI_RESP_M0					0x08

BYTE PIN_3_ATTEMPTS	3

BYTE PIN_2_ATTEMPTS	2

BYTE PUK_10_ATTEMPTS	10

BYTE PIN_1	1

BYTE PIN_2	2

BYTE PARAM_OK	0

BYTE ZERO_VALUE	0x00
BYTE RAC_89		0x89

/* all profiles supported  0xE0 */

BYTE MMI_AND_FDN_BDN	0xE0

BYTE NUM_0 0

BYTE NUM_1 1

BYTE NUM_2 2

BYTE NUM_3 3

BYTE NUM_4 4

BYTE NUM_5 5

BYTE NUM_6 6

BYTE NUM_7 7

BYTE NUM_8 8

BYTE NUM_9 9

BYTE NUM_10 10

BYTE NUM_12 12

BYTE NUM_14 14

BYTE NUM_17 17

BYTE NUM_20 20

BYTE NUM_21 21

BYTE NUM_22 22

BYTE NUM_FF 255

BYTE NUM_1000 1000

BYTE UART_TUI 2

BYTE GACI_TUI 1

BYTE PHB_RECORD_1 1

BYTE PHB_RECORD_2 2

BYTE PHB_RECORD_3 3

BYTE PHB_RECORD_4 4

BYTE PHB_RECORD_5 5

BYTE PHB_RECORD_6 6

BYTE PHB_RECORD_7 7

BYTE PHB_RECORD_8 8

BYTE PHB_MAX_RECORD_8 8

BYTE PHB_LENGTH_1 1

- BYTE PHB_LENGTH_ADN 52

- BYTE TOTAL_SIM_RECORDS_10 10

- BYTE USED_SIM_RECORDS_1 1

- BYTE STATUS_SIM_RECORDS SMS_RECORDS_FREE

- BYTE TOTAL_ME_RECORDS_0 0

- BYTE USED_ME_RECORDS_0 0

- BYTE STATUS_ME_RECORDS SMS_RECORD_DEFECT

- BYTE DUMMY_VALUE 0

- BYTE REC_NUM_0 0

- BYTE REC_NUM_1 1

- BYTE REC_NUM_2 2

BYTE L_FAC_64 64

BYTE L_FAC_104 104

BYTE L_FAC_128 128

BYTE L_FAC_176 176

BYTE L_FAC_200 200

BYTE L_FAC_600 600
BYTE L_FAC_7 7

BYTE NUM_DATA_BITS 8

BYTE NUM_STOP_BITS_1 1

BYTE NUM_STOP_BITS_2 2

BYTE L2R_CODE_STR_LEN 6

BYTE L2R_RETRANS_ATTEMPTS 0x06

BYTE L2R_RELAY_DELAY 0

BYTE L2R_RELAY_DELAY_20 20

BYTE L2R_ACK_TIMER 0x30

BYTE HALF_RATE_FLAG_0 0x00

BYTE TCF_THRESHOLD_90 90

BYTE HDLC_REPORT_1 1

BYTE TP_VT_MTI   SMS_VT_DELIVER	/* virtual msg type */

BYTE RP_SRI_UDH_MMS_MTI 0x04	/* reply path, no data hdr,no stat rep,no more msg’s waiting,MT sms */

BYTE TP_PID   SMS_PID_DEFAULT	/* protocol id */

BYTE TP_DCS  SMS_DCS_DATA_CLASS	/* data coding scheme with msg class */

BYTE TP_TIME_ZONE 0x40	/* 1 hour passed GMT */

BYTE MAX_SLOT_ID_0 0 	/* max slot id for SMREG */

BYTE BIP_CH_ID_1 1	/* bearer independent channel */

BYTE SIM_DTI_NORMAL_DIRECTION 0

BYTE SIM_DTI_REQ_NOT_APPLICABLE 0

BYTE SAT_NTW_CS_NO_ERROR 0x00		/* no error cause */

BYTE SIM_DTI_CONNECT_BIP_OPEN 0x05	/* DTI Connect and BIP Open? */

UBYTE DTI2_DTR_PARAMETER_ID = 0x04;

USHORT L2R_WIN_SIZE_IWF_MS = 0x003d;

USHORT L2r_WIN_SIZE_MS_IWF = 0x003d;

USHORT L2R_NUM_CODE_WORDS = 0x0200;

USHORT L2R_BUFFER_SIZE_2048 = 2048;

USHORT L2R_BYTES_PER_PRIM_250 = 250;

USHORT NO_FRAMES_PER_PRIM_3 = 3;

USHORT TRANS_RATE_9600 = 9600;

USHORT NSAPI_SET_NSAPI_5 =	0x0020;

USHORT NSAPI_SET_NSAPI_6 =	0x0040;

USHORT NSAPI_SET_NSAPI_56 =	0x0060;

USHORT MNSS_CAUSE_0600 = 0x0600;

USHORT LAC_1234 =	0x1234;

USHORT CID_0001 =	0x0001;

USHORT RX_LEVEL_100 =	0x0064;

LONG VAL_T3314 44000
LONG VAL_T3312 3240000

ULONG STRING_POINTER = 0xfe1234ef;

ULONG STRING_NULL_POINTER = 0;

ULONG DTI2_LINK_ID_0 = 0;

ULONG DTI_LINK_ID_0 = 0;

LONG DTI_LINKID_NAME 0x00000100

LONG DTI_NEIGHBOR_NAME 0xFE1234EF	/* this is an address so could change */

LONG TIME_10SEC 0x00000064			/* time to release DTI connection if no xfer */

#define PUK_1				    3

#define DEVICE_1 	NUM_0

#define DEVICE_2 	NUM_3

#define UART_DTI_ID 	NUM_4

#define UART_DTI_ID_2 	NUM_3

#define STANDARD_DLCI 	NUM_FF

/* values for the elimination of warnings */

#define UNUSED_IN_TESTCASE 0

#define OFF_HOOK_KEY	0x0d

#define  PROACTIVE_SIM_CMD_TAG	0xD0		/* proactive SIM command tag */

#define  CMD_DETAILS_TAG	0x81		/* command details tag */

#define  DEV_ID_TAG	0x82		/* device identities tag */

#define  ADDR_TAG	0x86		/* address tag */

#define  BEAR_DESC_TAG	0xB5		/* bearer description tag */

#define  BUF_SIZE_TAG	0xB9		/* buffer size tag */

#define  SIM_ME_TRANS_TAG	0xbc		/* SIM/ME interf. transp. level tag */

#define  DESTINATION_PORT_1	0x07		/* 1. byte of port 1800 */

#define  DESTINATION_PORT_2	0x08		/* 2. byte of port 1800 */

#define  DEST_ADDR_TAG	0xbe		/* Data destination address tag */

#define  DESTINATION_IP_1	0x0a		/* 1. byte of IP address 10.11.12.13 */

#define  DESTINATION_IP_2	0x0b		/* 2. byte of IP address 10.11.12.13 */

#define  DESTINATION_IP_3	0x0c		/* 3. byte of IP address 10.11.12.13 */

#define  DESTINATION_IP_4	0x0d		/* 4. byte of IP address 10.11.12.13 */

#define  SAT_CMD_OPEN_CHANNEL		0x40

#define QLF_OPCH_IMMDT_LINK_EST		0x1

#define QLF_OPCH_NO_AUTO_RECONNECT		0x0

#define DEV_SRC_SIM				0x81

#define DEV_DST_ME				0x82

#define BT_CSD					0x1

#define UDP					0x1

#define IPv4					0x21

#define DLCI_0 	NUM_0

- /* Parameters for DTI2_DATA_TEST_REQ */

BEGIN_PSTRUCT ("st_lines", SERIAL_LINE_STATES_1)

SET_COMP ("st_flow", DTI_FLOW_ON)
SET_COMP ("st_line_sa", DTI_SA_ON)
SET_COMP ("st_line_sb", DTI_SB_OFF)
SET_COMP ("st_break_len", DTI_BREAK_OFF)

ENDSTRUCT

BEGIN_PSTRUCT ("parameters", DTI2_DTR_PARAMETERS1)

SET_COMP ("p_id", DTI2_DTR_PARAMETER_ID)
SET_COMP ("st_lines", SERIAL_LINE_STATES_1)

ENDSTRUCT

/* originator address tp_oa */

BEGINARRAY (TP_OA, 8)
	0x0B,	/* number of digits */

0x28,	/* ton+npi */

0x01,0x13, 0x17, 0x08, 0x35, 0xF7

ENDARRAY

- /* Parameters for UART Serial link */

BEGIN_PSTRUCT ("comPar", COM_PARS)

SET_COMP ("speed", UART_IO_SPEED_115200)
SET_COMP ("bpc", UART_IO_BPC_8)
SET_COMP ("nsb", UART_IO_SB_1)
SET_COMP ("parity", UART_IO_PA_NONE)

SET_COMP ("flow_rx", UART_IO_FC_RX_RTS)
SET_COMP ("flow_tx", UART_IO_FC_TX_RTS)
SET_COMP ("xon_valid", UART_IO_XON_VALID)
SET_COMP ("xon", UART_IO_XON_DEFAULT)

SET_COMP ("xoff_valid", UART_IO_XOFF_VALID)
SET_COMP ("xoff", UART_IO_XOFF_DEFAULT)

SET_COMP ("esc_valid", UART_IO_ESC_UNDEF)
SET_COMP ("esc_char", 0)

SET_COMP ("esc_gp", UART_IO_ESC_OFF)

ENDSTRUCT

/* service centre time stamp tp_scts */

BEGINARRAY (TP_SCTS, 7)
	0x03,	/* year */

0x06,	/* month */

0x12,	/* day */

0x11,	/*hour */

0x15,	/* minute */

0x95,	/* second */

TP_TIME_ZONE

ENDARRAY

/* read sms msgs at phone initialisation */

/* MO SMS-SDU (VP-ABS) */

BEGIN_PSTRUCT ("sms_sdu", SMS_SDU_MO_ABS)
	SET_COMP ("l_buf", 1400)
	SET_COMP ("o_buf", 0)
	SET_COMP ("buf", SMS_SDU_MO_ABS_BUF)
ENDSTRUCT

BEGINARRAY (SMS_SDU_MO_ABS_BUF,163)
	0x07, 0x91, 0x44, 0x97, 0x37, 0x01, 0x90, 0x37, 0x44, 0x0C, 0x91, 0x44, 0x87, 0x71, 0x61,
	0x35, 0x87, 0x00, 0x00, 0x20, 0x50, 0x72, 0x31, 0x40, 0x55, 0x40, 0x13, 0x05, 0x00, 0x03,
	0xC8, 0x03, 0x03, 0xF0, 0xCC, 0x69, 0x72, 0xF8, 0x04, 0x99, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF
ENDARRAY

/* SIM Service Table with Nr. 4 */

BEGINARRAY (F_SIM_SRV_4,10)
	0xC0, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

BEGINARRAY (SIM_SERV_PHASE_2_ADN_BDN, 16)
	0xCF, 0x3F, 0xFF, 0x0F,
	0xFF, 0x00, 0x00, 0x30,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (SIM_SERV_PHASE_2, 16)
	0xCF, 0xFF, 0x3F, 0x03,
	0xFF, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (F_SIM_SRV,10)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

BEGINARRAY (PIN_1_VALUE_1234,8)
	0x31,0x32,0x33,0x34,0xFF,0xFF, 0xFF,0xFF

ENDARRAY

BEGINARRAY (PIN_1_VALUE_B,8)
	0x31,0x31,0x32,0x33,0x34, 0xFF,0xFF,0xFF

ENDARRAY

BEGINARRAY (PIN_1_VALUE_11111111,8)
	0x31,0x31,0x31,0x31,0x31,0x31,0x31,0x31

ENDARRAY

BEGINARRAY (PIN_1_VALUE_1239,8)
	0x31,0x32,0x33,0x39,0xFF,0xFF,0xFF,0xFF

ENDARRAY

BEGINARRAY (PIN_1_VALUE_1233,8)
	0x31,0x32,0x33,0x33,0xFF, 0xFF,0xFF,0xFF

ENDARRAY

BEGINARRAY (F_SIM_SRV_4_15_16,10)
	0x00, 0x00, 0x00, 0xf0, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (F_SIM_SRV_PHB,10)
	0x0C, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (F_SIM_SRV_FDN_PHB,10)
	0x30, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

/* phonebook records */

/* DONALD */

BEGINARRAY (PHB_ADN_RECORD_ERIC_MT_SAVE,256)

0x45, 0x72, 0x69, 0x63, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0x07, 0x91,

0x10, 0x13, 0x17, 0x08, 0x35, 0xF7, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

/* Sid */

BEGINARRAY (PHB_ADN_RECORD_SID,256)

0x53, 0x69, 
0x64, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 
0xFF, 0xFF, 0xFF, 0xFF, 0x06, 0x81, 0x89, 0x67, 
0x45, 0x23, 0x01, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 
0xFF, 0xFF, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

BEGINARRAY (PHB_ADN_RECORD_ERIC_MO_SAVE,256)

0x45, 0x72, 0x69, 0x63, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0x07, 0x81,

0x10, 0x13, 0x17, 0x08, 0x35, 0xF7, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

/* JIE*/

/*

BEGINARRAY (PHB_ADN_RECORD_1,52)

0x4A, 0x49, 0x45, 0x45, 0x45, 0x45, 0x45, 0x45,
0x4A, 0x49, 0x45, 0x45, 0x45, 0x45, 0x45, 0x45, 0x4A, 0x49, 0x45, 0x45, 0x45, 0x45, 0x45,
0x45, 0x4A, 0x49, 0x45, 0x45, 0x45, 0x45, 0x45, 0x45, 0x4A, 0x49, 0x45, 0x45, 0x45, 0xFF,
0x06, 0x81, 0x30, 0x30, 0x09, 0x49, 0x01, 0xF3, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF

ENDARRAY

*/

/* ERIC ERIC */

BEGINARRAY (PHB_ADN_RECORD_3,52)

0x45, 0x52, 0x49, 0x43, 0x20, 0x45, 0x52, 0x49,
0x43, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0x07, 0x81, 
0x10, 0x13, 0x17, 0x08, 0x35, 0xF7, 0xFF, 0xFF, 0xFF, 0xFF,0xFF, 0xFF

ENDARRAY

BEGINARRAY (PHB_ADN_RECORD_EMPTY,52)

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF

ENDARRAY

/* ERWIN*/

BEGINARRAY (PHB_ADN_RECORD_4,52)

0x45, 0x52, 0x57, 0x49, 0x4E, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x01, 0xF8, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF

ENDARRAY

/* ANDREAS */

BEGINARRAY (PHB_ADN_RECORD_1,52)

0x41, 0x4E, 0x44, 0x52, 0x45, 0x41, 0x53, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0x05, 0x81, 0x10, 0x17, 0x11, 0x42, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF

ENDARRAY

/* BERND */

BEGINARRAY (PHB_ADN_RECORD_2,52)

0x42, 0x45, 0x52, 0x4E, 0x44, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0x06, 0x81, 0x94, 0x93, 0x90, 0x14, 0x61, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF

ENDARRAY

/* JVJ */

BEGINARRAY (PHB_ADN_RECORD_5,52)

0x4A, 0x56, 0x4A, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0x07, 0x81, 0x30,0x30, 0x89, 0x84, 0x76, 0xF5, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF

ENDARRAY

/* STEFAN */

BEGINARRAY (PHB_ADN_RECORD_6,52)

0x53, 0x74, 0x65, 0x66, 0x61, 0x6E, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0x07, 0x81,
0x30, 0x30, 0x09, 0x49, 0x11, 0xF7, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF

ENDARRAY

/* ANDREAS */

BEGINARRAY (PHB_ADN_RECORD_1_DELETE,256)

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

/* ANDREAS */

BEGINARRAY (PHB_ADN_RECORD_1_CHANGE,256)

0x41, 0x4E, 0x44, 0x52, 0x45, 0x41, 0x53, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0x05, 0x81, 0x10, 0x17, 0x11, 0x42, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

- BEGINARRAY_PART (NUM_654321, 6)

- 0x06, 0x05, 0x04, 0x03, 0x02, 0x01

- ENDARRAY

- BEGINARRAY_PART (NUM_VOICEMBX_121, 3)

- 0x01, 0x02, 0x01

- ENDARRAY

/* BC_PARA_SPEECH

| 0 | User rate |
| --- | --- |
| 5 | Bearer serv speech |
| 0 | Connection element |
| 0 | Stop bits |
| 0 | Data bits |
| 0 | Parity |
| 0 | Flow control |
| 0 | Modem type |

*/

BEGIN_PSTRUCT ("bcpara", BC_PARA_SPEECH)

SET_COMP ("rate", UR_NOT_PRES)
SET_COMP ("bearer_serv", BEARER_SERV_SPEECH)
SET_COMP ("conn_elem", CONN_ELEM_NOT_PRES)
SET_COMP ("stop_bits",	ZERO_VALUE)
SET_COMP ("data_bits",	DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", NO_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)

ENDSTRUCT

/* BC_PARA_ASYNC_1200 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_ASYNC_1200)

SET_COMP ("rate", UR_1_2_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V22)

ENDSTRUCT

/* BC_PARA_FAX */

BEGIN_PSTRUCT ("bcpara", BC_PARA_FAX)

SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_FAX)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V22)

ENDSTRUCT

/* for sim toolkit CSD channel */

BEGIN_PSTRUCT ("bcpara", BC_PARA_ASYNC_9600)

SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", NO_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V34)

ENDSTRUCT

BEGIN_PSTRUCT ("bcpara", BC_PARA_SYNC_9600)

SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_SYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", NO_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V34)

ENDSTRUCT

BEGIN_PSTRUCT ("bcpara", BC_PARA_PAD_ACCESS_ASYNC)

SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_PAD_ACCESS)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", NO_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V34)

ENDSTRUCT

/* BC_PARA_NO_SERVICE

| 0 | User rate |
| --- | --- |
| 255 | Bearer serv none |
| 0 | Connection element |
| 0 | stop bits |
| 0 | data bits |
| 0 | Parity |
| 0 | flow control |
| 0 | Modem type |

*/
BEGIN_PSTRUCT ("bcpara", BC_PARA_NO_SERVICE)

SET_COMP ("rate", UR_NOT_PRES)
SET_COMP ("bearer_serv", BEARER_SERV_NOT_PRES)
SET_COMP ("conn_elem",  CONN_ELEM_NOT_PRES)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", NO_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)

ENDSTRUCT

BEGIN_PSTRUCT ("called_party", CLED_PARTY_112)
	SET_COMP ("ton", TON_UNKNOWN)
	SET_COMP ("npi", NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num", 3)
	SET_COMP ("called_num", NUM_112)
ENDSTRUCT

BEGIN_PSTRUCT ("called_party", CLED_PARTY_01317180537)
	SET_COMP ("ton", TON_UNKNOWN)
	SET_COMP ("npi", NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num", 11)
	SET_COMP ("called_num", NUM_01317180537)
ENDSTRUCT

BEGINARRAY_PART (NUM_01317180537, 11) 0x00, 0x01, 0x03, 0x01, 0x07, 0x01, 0x08, 0x00, 0x05, 0x03, 0x07 ENDARRAY

BEGIN_PSTRUCT ("called_party", CLED_PARTY_9876543210_FRED)
	SET_COMP ("ton", TON_UNKNOWN)
	SET_COMP ("npi", NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num", 10)
	SET_COMP ("called_num", NUM_9876543210)
ENDSTRUCT

BEGINARRAY_PART (NUM_9876543210, 10) 0x09, 0x08, 0x07, 0x06, 0x05, 0x04, 0x03, 0x02, 0x01, 0x00 ENDARRAY

- BEGIN_PSTRUCT ("called_party", CLED_PARTY_654321)
	SET_COMP ("ton", TON_UNKNOWN)
	SET_COMP ("npi", NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num", 6)
	SET_COMP ("called_num", NUM_654321)
ENDSTRUCT

- BEGIN_PSTRUCT ("called_party", CLED_PARTY_ERIC_NT)
	SET_COMP ("ton", TON_UNKNOWN)
	SET_COMP ("npi", NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num", 11)
	SET_COMP ("called_num", NUM_X)
ENDSTRUCT

- BEGINARRAY_PART (NUM_X, 11)  0x00, 0x01, 0x03, 0x01, 0x07, 0x01, 0x08, 0x00, 0x05, 0x03, 0x07 ENDARRAY

- BEGIN_PSTRUCT ("called_party", CLED_PARTY_ERIC_T)
	SET_COMP ("ton", TON_INT_NUMB)
	SET_COMP ("npi", NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num", 11)
	SET_COMP ("called_num", NUM_X)
ENDSTRUCT

BEGIN_PSTRUCT ("called_party", CLED_PARTY_VOICEMBX)
	SET_COMP ("ton", TON_UNKNOWN)
	SET_COMP ("npi", NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num", 3)
	SET_COMP ("called_num", NUM_VOICEMBX_121)
ENDSTRUCT

BEGIN_PSTRUCT ("calling_party", CLG_PARTY_01317180537)
	SET_COMP ("ton", TON_NAT_NUMB)
	SET_COMP ("npi", NPI_NAT_NUMB_PLAN)
	SET_COMP (“present”, PRES_PRES_ALLOW)
	SET_COMP (“screen”, SCREEN_USER_PROV_NOT_SCREEN)

SET_COMP ("c_num", 11)
	SET_COMP ("num", NUM_01317180537)
ENDSTRUCT

BEGIN_PSTRUCT ("calling_party", CLG_PARTY_654321)
	SET_COMP ("ton", TON_NAT_NUMB)
	SET_COMP ("npi", NPI_NAT_NUMB_PLAN)
	SET_COMP (“present”, PRES_PRES_ALLOW)
	SET_COMP (“screen”, SCREEN_USER_PROV_NOT_SCREEN)

SET_COMP ("c_num", 6)
	SET_COMP ("num", NUM_654321)
ENDSTRUCT

BEGIN_PSTRUCT ("connected_number", CONNECTED_NUMBER_112)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("present",	PRES_PRES_ALLOW)
	SET_COMP ("screen",	SCREEN_USER_PROV_NOT_SCREEN)
	SET_COMP ("c_num",	3)
	SET_COMP ("num",	NUM_112)
ENDSTRUCT

BEGINARRAY_PART (NUM_112, 3)

0x01, 0x01, 0x02

ENDARRAY

BEGIN_PSTRUCT ("connected_number", CONNECTED_NUMBER_01317180537)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("present",	PRES_PRES_ALLOW)
	SET_COMP ("screen",	SCREEN_USER_PROV_NOT_SCREEN)
	SET_COMP ("c_num",	11)
	SET_COMP ("num",	NUM_01317180537)
ENDSTRUCT

BEGIN_PSTRUCT ("connected_number", CONNECTED_NUMBER_654321)
	SET_COMP ("ton",	TON_NAT_NUMB)
	SET_COMP ("npi",	NPI_NAT_NUMB_PLAN)
	SET_COMP ("present",	PRES_PRES_ALLOW)
	SET_COMP ("screen",	SCREEN_USER_PROV_NOT_SCREEN)
	SET_COMP ("c_num",	6)
	SET_COMP ("num",	NUM_654321)
ENDSTRUCT

/* calling/called/connected_party_sub (Calling/Called/Connected party subaddress) */
BEGIN_PSTRUCT ("called_party_sub", CLED_PARTY_SUB)
	SET_COMP ("tos", TOS_NOT_PRES)
	SET_COMP ("odd_even", OE_ODD)
	SET_COMP ("c_subaddr", 6)
	SET_COMP ("subaddr", NUM_654321)
ENDSTRUCT

BEGIN_PSTRUCT ("connected_number_sub", CONNECTED_PARTY_SUB_NONE)
	SET_COMP ("tos", TOS_NOT_PRES)
	SET_COMP ("odd_even", OE_ODD)
	SET_COMP ("c_subaddr", ZERO_VALUE)
	SET_COMP ("subaddr", NUM_654321)
ENDSTRUCT

BEGIN_PSTRUCT ("called_party_sub", CLED_PARTY_SUB_NONE)
	SET_COMP ("tos", TOS_NOT_PRES)
	SET_COMP ("odd_even", OE_EVEN)
	SET_COMP ("c_subaddr", ZERO_VALUE)
	SKIP_COMP ("subaddr")
ENDSTRUCT

BEGIN_PSTRUCT ("calling_party_sub", CLG_PARTY_SUB)
	SET_COMP ("tos", TOS_NSAP)
	SET_COMP ("odd_even", OE_ODD)
	SET_COMP ("c_subaddr", 11)
	SET_COMP ("subaddr", NUM_01317180537)
ENDSTRUCT

- BEGIN_PSTRUCT (“redirecting_party”, RED_PARTY)

- SET_COMP (“ton”, TON_NAT_NUMB)
	SET_COMP (“npi”, NPI_NAT_NUMB_PLAN)
	SET_COMP (“present”, PRES_PRES_ALLOW)
	SET_COMP (“screen”, SCREEN_USER_PROV_NOT_SCREEN)
	SET_COMP (“c_redir_num”, NUM_6)
	SET_COMP (“redir_num”, NUM_654321)

- ENDSTRUCT

BEGIN_PSTRUCT ("redirecting_party_sub", RED_PARTY_SUB)
	SET_COMP ("tos", TOS_NSAP)
	SET_COMP ("odd_even", OE_EVEN)
	SET_COMP ("c_subaddr", 6)
	SET_COMP ("subaddr", NUM_654321)
ENDSTRUCT

- BEGIN_PSTRUCT ("chm", CHANNEL_MODE_1)
	SET_COMP ("ch_type", CH_TCH_F)
	SET_COMP ("ch_mode", CHM_SPEECH)
ENDSTRUCT

BEGINARRAY (FACILITY_DIVERT_ARR,12)

0x40, 0x00, 0x00, 0x00, 0xA1, 0x06, 0x02, 0x01, 0x00, 0x02, 0x01, 0x7E ENDARRAY

- BEGINARRAY (RELEASE_COMPLETE_ARR,251)

- 0xA2, 0x14, 0x02, 0x01, 0x0A, 0x30, 0x0C, 0x04, 0x01, 0x21, 0x84, 0x07, 0x81, 0x03, 0x10, 0x13, 0x17, 0x08, 0x35, 0xF7,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

- BEGINARRAY (RELEASE_COMPLETE_DEACTIVARR,251)

- 0xA2, 0x14, 0x02, 0x01, 0x0A, 0x30, 0x0C, 0x04, 0x01, 0x21, 0x84, 0x07, 0x81, 0x03, 0x10, 0x13, 0x17, 0x08, 0x35, 0xF7,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/*

- BEGINARRAY (RELEASE_COMPLETE_ARR,24)

- 0xA1, 0x14, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0C, 0x04, 0x01, 0x21, 0x84, 0x07, 0x81, 0X03,
0x10, 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00

- ENDARRAY

*/

/* FACILITY_REGISTER */

BEGIN_PSTRUCT ("fac_inf", FACILITY_REGISTER)

SET_COMP ("l_fac", L_FAC_176)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_1_ARR)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_1_ARR,251)
0xA1, 0x14, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0C, 0x04, 0x01, 0x21, 0x83, 0x01, 0x10, 0x84, 0x04, 0x81, 0x21,

- 0x43, 0x65, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

/* CLIR */

BEGIN_PSTRUCT ("fac_inf", REGISTER_CLIR)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_CLIR_ARR)

ENDSTRUCT

- BEGINARRAY (REGISTER_CLIR_ARR,251)
0xA1, 0x0B, 		/* component invoke tag, length */

- 0x02, 0x01, 0x01,		/* invoke id tag, length, invoke id (2nd invoke) */

- 0x02, 0x01, 0x0C,		/* operation code tag, length, operation (activate) */

- 0x30, 0x03,		/* parameter id tag, length */

- 0x04, 0x01, 0x12,		/* ss-code tag, length, ss-code (clir) */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("fac_inf", DEREGISTER_CLIR)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", DEREGISTER_CLIR_ARR)

ENDSTRUCT

- BEGINARRAY (DEREGISTER_CLIR_ARR,251)
0xA1, 0x0B, 		/* component invoke tag, length */

- 0x02, 0x01, 0x00,		/* invoke id tag, length, invoke id (1st invoke) */

- 0x02, 0x01, 0x0D,		/* operation code tag, length, operation (deactivate) */

- 0x30, 0x03,		/* parameter id tag, length */

- 0x04, 0x01, 0x12,		/* ss-code tag, length, ss-code (clir) */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

/* INTERROGATE SS RESULT */

BEGIN_PSTRUCT ("fac_inf", RELEASE_COMPLETE_CLIR)

SET_COMP ("l_fac", L_FAC_128)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", RELEASE_COMPLETE_CLIR_ARR)

ENDSTRUCT

- BEGINARRAY (RELEASE_COMPLETE_CLIR_ARR,251)
0xA2, 0x0E, 		/* return result component tag, length */

- 0x02, 0x01, 0x00,		/* invoke id tag, length, invoke id (1st invoke) */

0x30, 0x10, 		/*sequence tag, Length */

- 0x02, 0x01, 0x0C,		/* operation code tag, length, operation (activate) */

- /* start of parameters */

- 0x80, 0x01,0x03,		/* ss status id tag, length, value (registered&active) */

- 0x04, 0x01, 0x12,		/* ss-code tag, length, ss-code (call waiting) */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

/* INTERROGATE SS */

BEGIN_PSTRUCT ("fac_inf", INTERROGATE_SS)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", INTERROGATE_SS_ARR)

ENDSTRUCT

- BEGINARRAY (INTERROGATE_SS_ARR,251)
0xA1, 0x0B, 		/* component invoke tag, length */

- 0x02, 0x01, 0x00,		/* invoke id tag, length, invoke id (1st invoke) */

- 0x02, 0x01, 0x0E,		/* operation code tag, length, operation (interrogate) */

- 0x30, 0x03,		/* parameter id tag, length */

- 0x04, 0x01, 0x41,		/* ss-code tag, length, ss-code (call waiting) */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("fac_inf", INTERROGATE_SS_OG)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", INTERROGATE_SS_OG_CONTENT)

ENDSTRUCT

- BEGINARRAY (INTERROGATE_SS_OG_CONTENT,251)
0xA1, 0x0B, 		/* component invoke tag, length */

- 0x02, 0x01, 0x00,		/* invoke id tag, length, invoke id (1st invoke) */

- 0x02, 0x01, 0x0E,		/* operation code tag, length, operation (interrogate) */

- 0x30, 0x03,		/* parameter id tag, length */

- 0x04, 0x01, 0x92,		/* ss-code tag, length, ss-code (all Outgoing) */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("fac_inf", INTERROGATE_SS_OG_INTER)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", INTERROGATE_SS_OG_INTER_CONTENT)

ENDSTRUCT

- BEGINARRAY (INTERROGATE_SS_OG_INTER_CONTENT,251)
0xA1, 0x0B, 		/* component invoke tag, length */

- 0x02, 0x01, 0x00,		/* invoke id tag, length, invoke id (1st invoke) */

- 0x02, 0x01, 0x0E,		/* operation code tag, length, operation (interrogate) */

- 0x30, 0x03,		/* parameter id tag, length */

- 0x04, 0x01, 0x93,		/* ss-code tag, length, ss-code (all Outgoing International) */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("fac_inf", INTERROGATE_SS_OG_INTER_BUT_HOME)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", INTERROGATE_SS_OG_INTER_BUT_HOME_CONTENT)

ENDSTRUCT

- BEGINARRAY (INTERROGATE_SS_OG_INTER_BUT_HOME_CONTENT,251)
0xA1, 0x0B, 		/* component invoke tag, length */

- 0x02, 0x01, 0x00,		/* invoke id tag, length, invoke id (1st invoke) */

- 0x02, 0x01, 0x0E,		/* operation code tag, length, operation (interrogate) */

- 0x30, 0x03,		/* parameter id tag, length */

- 0x04, 0x01, 0x94,		/* ss-code tag, length, ss-code (all Outgoing International except those directed to home PLMN) */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("fac_inf", INTERROGATE_SS_IC)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", INTERROGATE_SS_IC_CONTENT)

ENDSTRUCT

- BEGINARRAY (INTERROGATE_SS_IC_CONTENT,251)
0xA1, 0x0B, 		/* component invoke tag, length */

- 0x02, 0x01, 0x00,		/* invoke id tag, length, invoke id (1st invoke) */

- 0x02, 0x01, 0x0E,		/* operation code tag, length, operation (interrogate) */

- 0x30, 0x03,		/* parameter id tag, length */

- 0x04, 0x01, 0x9A,		/* ss-code tag, length, ss-code (all Incoming) */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("fac_inf", INTERROGATE_SS_IC_ROAMING)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", INTERROGATE_SS_IC_ROAMING_CONTENT)

ENDSTRUCT

- BEGINARRAY (INTERROGATE_SS_IC_ROAMING_CONTENT,251)
0xA1, 0x0B, 		/* component invoke tag, length */

- 0x02, 0x01, 0x00,		/* invoke id tag, length, invoke id (1st invoke) */

- 0x02, 0x01, 0x0E,		/* operation code tag, length, operation (interrogate) */

- 0x30, 0x03,		/* parameter id tag, length */

- 0x04, 0x01, 0x9B,		/* ss-code tag, length, ss-code (all Incoming when Roaming outside home PLMN) */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("fac_inf", INTERROGATE_SS_CLIR)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", INTERROGATE_SS_CLIR_CONTENT)

ENDSTRUCT

- BEGINARRAY (INTERROGATE_SS_CLIR_CONTENT,251)
0xA1, 0x0B, 		/* component invoke tag, length */

- 0x02, 0x01, 0x00,		/* invoke id tag, length, invoke id (1st invoke) */

- 0x02, 0x01, 0x0E,		/* operation code tag, length, operation (interrogate) */

- 0x30, 0x03,		/* parameter id tag, length */

- 0x04, 0x01, 0x12,		/* ss-code tag, length, ss-code (ss code for CLIR) */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

/* INTERROGATE SS RESULT */

BEGIN_PSTRUCT ("fac_inf", INTERROGATE_SS_RESULT)

SET_COMP ("l_fac", L_FAC_128)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", INTERROGATE_SS_RESULT_ARR)

ENDSTRUCT

- BEGINARRAY (INTERROGATE_SS_RESULT_ARR,251)
0xA2, 0x0E, 		/* return result component tag, length */

- 0x02, 0x01, 0x00,		/* invoke id tag, length, invoke id (1st invoke) */

0x30, 0x10, 		/*sequence tag, Length */

- 0x02, 0x01, 0x0E,		/* operation code tag, length, operation (interrogate SS) */

- /* start of parameters */

- 0x80, 0x01,0x03,		/* ss status id tag, length, value (registered&active) */

- 0x04, 0x01, 0x41,		/* ss-code tag, length, ss-code (call waiting) */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

/* FACILITY_REGISTER DIVERT VOICE*/

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALLCALLS_V)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALLCALLS_V)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALLCALLS_V,251)

- 0xA1, 0x17,		/* invoke component, length */

- 0x02, 0x01, 0x00,		/* invoke id tag , length, id (1st) */

- 0x02, 0x01, 0x0A,		/* operation code tag, length, register */

- 0x30, 0x0F,		/* sequence code tag, length */

- 0x04, 0x01, 0x21,		/* ss code tag, length, ss-code (CF) */

- 0x83, 0x01, 0x10,		/* teleservices tag, length, value (all speech services) */

- 0x84, 0x07,		/* forward to tag, length */

- 0x81, 0x10,		/* noa-npi, telephone digits 01 etc */

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* FACILITY_REGISTER DIVERT FAX*/

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALLCALLS_F)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALLCALLS_F)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALLCALLS_F,251)

- 0xA1, 0x17,		/* invoke component, length */

- 0x02, 0x01, 0x00,		/* invoke id tag , length, id (1st) */

- 0x02, 0x01, 0x0A,		/* operation code tag, length, register */

- 0x30, 0x0F,		/* sequence code tag, length */

- 0x04, 0x01, 0x21,		/* ss code tag, length, ss-code (CF) */

- 0x83, 0x01, 0x60,		/* teleservices tag, length, value (all FAX services) */

- 0x84, 0x07,		/* forward to tag, length */

- 0x81, 0x10,		/* noa-npi, telephone digits 01 etc */

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* FACILITY_REGISTER DIVERT DATA*/

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALLCALLS_D)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALLCALLS_D)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALLCALLS_D,251)

- 0xA1, 0x17,		/* invoke component, length */

- 0x02, 0x01, 0x00,		/* invoke id tag , length, id (1st) */

- 0x02, 0x01, 0x0A,		/* operation code tag, length, register */

- 0x30, 0x0F,		/* sequence code tag, length */

- 0x04, 0x01, 0x21,		/* ss code tag, length, ss-code (CF) */

- 0x82, 0x01, 0x00,		/* bearer services tag, length, value (all data services) */

- 0x84, 0x07,		/* forward to tag, length */

- 0x81, 0x10,		/* noa-npi, telephone digits 01 etc */

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALLCALLS_VOICE)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALLCALLS_VOICE)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALLCALLS_VOICE,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x21, 0x83, 0x01, 0x10, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35,0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFBUSY_V)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFBUSY_V)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFBUSY_V,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x29, 0x83, 0x01, 0x10, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFBUSY_F)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFBUSY_F)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFBUSY_F,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x29, 0x83, 0x01, 0x60, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFBUSY_D)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFBUSY_D)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFBUSY_D,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x29, 0x82, 0x01, 0x00, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFNOREPLY_V)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFNOREPLY_V)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFNOREPLY_V,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x2A, 0x83, 0x01, 0x10, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFNOREPLY_F)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFNOREPLY_F)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFNOREPLY_F,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x2A, 0x83, 0x01, 0x60, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFNOREPLY_D)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFNOREPLY_D)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFNOREPLY_D,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x2A, 0x82, 0x01, 0x00, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFNOTREACHABLE_V)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFNOTREACHABLE_V)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFNOTREACHABLE_V,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x2B, 0x83, 0x01, 0x10, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFNOTREACHABLE_F)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFNOTREACHABLE_F)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFNOTREACHABLE_F,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x2B, 0x83, 0x01, 0x60, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFNOTREACHABLE_D)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFNOTREACHABLE_D)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFNOTREACHABLE_D,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x2B, 0x82, 0x01, 0x00, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALL_CONDITIONAL_V)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALL_CONDITIONAL_V)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALL_CONDITIONAL_V,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x28, 0x83, 0x01, 0x10, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALL_CONDITIONAL_D)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALL_CONDITIONAL_D)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALL_CONDITIONAL_D,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x28, 0x82, 0x01, 0x00, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALL_CONDITIONAL_F)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALL_CONDITIONAL_F)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALL_CONDITIONAL_F,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x28, 0x83, 0x01, 0x60, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFNOSERVICE_V)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFNOSERVICE_V)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFNOSERVICE_V,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x2B, 0x83, 0x01, 0x10, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFNOSERVICE_F)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFNOSERVICE_F)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFNOSERVICE_F,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x2B, 0x83, 0x01, 0x60, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFNOSERVICE_D)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFNOSERVICE_D)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFNOSERVICE_D,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x2B, 0x82, 0x01, 0x00, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALLCONFIGURED_V)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALLCONFIGURED_V)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALLCONFIGURED_V,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x28, 0x83, 0x01, 0x10, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALLCONFIGURED_F)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALLCONFIGURED_F)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALLCONFIGURED_F,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x28, 0x83, 0x01, 0x60, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALLCONFIGURED_D)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALLCONFIGURED_D)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALLCONFIGURED_D,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x28, 0x82, 0x01, 0x00, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALLDIVERTS_V)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALLDIVERTS_V)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALLDIVERTS_V,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x20, 0x83, 0x01, 0x10, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALLDIVERTS_F)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALLDIVERTS_F)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALLDIVERTS_F,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x20, 0x83, 0x01, 0x60, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALLDIVERTS_D)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALLDIVERTS_D)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALLDIVERTS_D,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x20, 0x82, 0x01, 0x00, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_UNCONDITIONALLY_V)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_UNCONDITIONALLY_V)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_UNCONDITIONALLY_V,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x21, 0x83, 0x01, 0x10, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_UNCONDITIONALLY_F)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_UNCONDITIONALLY_F)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_UNCONDITIONALLY_F,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x21, 0x83, 0x01, 0x60, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_UNCONDITIONALLY_D)

SET_COMP ("l_fac", L_FAC_200)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_UNCONDITIONALLY_D)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_UNCONDITIONALLY_D,251)

- 0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x21, 0x82, 0x01, 0x00, 0x84, 0x07, 0x81, 0x10,

- 0x13, 0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_AOCI)

SET_COMP ("l_fac", L_FAC_176)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", AOCI_ARR)

ENDSTRUCT

- BEGINARRAY_PART (AOCI_ARR,251)
0xA1, 0x14,		/* invoke component tag, length */

- 0x02, 0x01, 0x00,		/* invoke ID tag, length, value (1st facility) */

- 0x02, 0x01, 0x7D,		/* operation code tag, length, value */

- 0x30, 0x0C,		/* sequence tag, length */

- 0x80, 0x01,0x71,		/*IEI for SS-code, length, ss-code AOCI */

- 0xA1, 0x07,		/* IEI for charging info, length */

- 0x81, 0x01, 0x3C,		/* IEI for e1, length, value of e1 */

- 0x82, 0x02, 0x00, 0x8C,	/* IEI for e2, length, value of e2(0) and e2(1) */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

/* FACILITY CALL DEPENDENT MULTIPARTY */

BEGIN_PSTRUCT ("fac_inf", FAC_MULTIPARTY)

SET_COMP ("l_fac", L_FAC_64)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", MULTIPARTY_ARR)

ENDSTRUCT

- BEGINARRAY_PART (MULTIPARTY_ARR,251)
0xA1, 0x06, 0x02, 0x01, 0x00, 0x02, 0x01, 0x7C, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_MULTIPARTY_SPLIT)

SET_COMP ("l_fac", L_FAC_64)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", MULTIPARTY_SPLIT_ARR)

ENDSTRUCT

- BEGINARRAY_PART (MULTIPARTY_SPLIT_ARR,251)
0xA1, 0x06, 0x02, 0x01, 0x01, 0x02, 0x01, 0x79, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_MULTIPARTY_RETRIEVE)

SET_COMP ("l_fac", L_FAC_64)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", MULTIPARTY_RETRIEVE_ARR)

ENDSTRUCT

- BEGINARRAY_PART (MULTIPARTY_RETRIEVE_ARR,251)
0xA1, 0x06, 0x02, 0x01, 0x02, 0x02, 0x01, 0x7A, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

- BEGIN_PSTRUCT (“fac_inf”, MULTIPARTY_RETURN_RESULT)

- SET_COMP (“l_fac”, L_FAC_176)
SET_COMP (“o_fac”, ZERO_VALUE)
SET_COMP (“fac”, MULTIPARTY_RET_RES_ARR)

- ENDSTRUCT

- BEGINARRAY_PART (MULTIPARTY_RET_RES_ARR,251)

0xA2, 0x08, 0x02, 0x01, 0x00, 0x30, 0x03, 0x02, 0x01, 0x7C, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

- BEGIN_PSTRUCT (“fac_inf”, MULTIPARTY_SPLIT_RETURN_RESULT)

- SET_COMP (“l_fac”, L_FAC_176)
SET_COMP (“o_fac”, ZERO_VALUE)
SET_COMP (“fac”, MULTIPARTY_SPLIT_RET_RES_ARR)

- ENDSTRUCT

- BEGINARRAY_PART (MULTIPARTY_SPLIT_RET_RES_ARR,251)

0xA2, 0x08, 0x02, 0x01, 0x00, 0x30, 0x03, 0x02, 0x01, 0x79, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

- BEGIN_PSTRUCT (“fac_inf”, MULTIPARTY_RETRIEVE_RETURN_RESULT)

- SET_COMP (“l_fac”, L_FAC_176)
SET_COMP (“o_fac”, ZERO_VALUE)
SET_COMP (“fac”, MULTIPARTY_RETRIEVE_RET_RES_ARR)

- ENDSTRUCT

- BEGINARRAY_PART (MULTIPARTY_RETRIEVE_RET_RES_ARR,251)

0xA2, 0x08, 0x02, 0x01, 0x00, 0x30, 0x03, 0x02, 0x01, 0x7A, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* FACILITY REGISTER DIVERT*/

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALLCALLS)

SET_COMP ("l_fac", L_FAC_176)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALLCALLS)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALLCALLS,251)
0xA1, 0x14, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0C, 0x04, 0x01, 0x21, 0x84, 0x07, 0x81, 0x10, 0x13, 0x17, 0x08,

- 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFBUSY)

SET_COMP ("l_fac", L_FAC_176)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFBUSY)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFBUSY,251)

- 0xA1, 0x14, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0C, 0x04, 0x01, 0x29, 0x84, 0x07, 0x81, 0x10, 0x13, 0x17, 0x08,

- 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFNOREPLY)

SET_COMP ("l_fac", L_FAC_176)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFNOREPLY)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFNOREPLY,251)

- 0xA1, 0x14, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0C, 0x04, 0x01, 0x2A, 0x84, 0x07, 0x81, 0x10, 0x13, 0x17, 0x08,

- 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_IFNOTREACHABLE)

SET_COMP ("l_fac", L_FAC_176)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_IFNOTREACHABLE)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_IFNOTREACHABLE,251)

- 0xA1, 0x14, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0C, 0x04, 0x01, 0x2B, 0x84, 0x07, 0x81, 0x10, 0x13, 0x17, 0x08,

- 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALLCONFIGURED)

SET_COMP ("l_fac", L_FAC_176)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALLCONFIGURED)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALLCONFIGURED,251)

- 0xA1, 0x14, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0C, 0x04, 0x01, 0x28, 0x84, 0x07, 0x81, 0x10, 0x13, 0x17, 0x08,

- 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_ALLDIVERTS)

SET_COMP ("l_fac", L_FAC_176)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_ALLDIVERTS)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_ALLDIVERTS,251)

- 0xA1, 0x14, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0C, 0x04, 0x01, 0x20, 0x84, 0x07, 0x81, 0x10, 0x13, 0x17, 0x08,

- 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_UNCONDITIONALLY)

SET_COMP ("l_fac", L_FAC_176)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_DIVERT_UNCONDITIONALLY)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_DIVERT_UNCONDITIONALLY,251)

- 0xA1, 0x14, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0C, 0x04, 0x01, 0x21, 0x84, 0x07, 0x81, 0x10, 0x13, 0x17, 0x08,

- 0x35, 0xF7, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* FACILITY REGISTER BARRING */

BEGIN_PSTRUCT ("fac_inf", FAC_BAR_OG_CALLS)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_BAR_OG_CALLS)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_BAR_OG_CALLS,251)

- 0xA1, 0x0B, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x92, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_BAR_INTER_CALLS)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_BAR_INTER_CALLS)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_BAR_INTER_CALLS,251)

- 0xA1, 0x0B, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x93, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_BAR_INTER_BUT_HOME_CALLS)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_BAR_INTER_BUT_HOME_CALLS)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_BAR_INTER_BUT_HOME_CALLS,251)

- 0xA1, 0x0B, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x94, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_BAR_IC_CALLS)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_BAR_IC_CALLS)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_BAR_IC_CALLS,251)

- 0xA1, 0x0B, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x9A, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_BAR_IC_ROAMING_CALLS)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_BAR_IC_ROAMING_CALLS)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_BAR_IC_ROAMING_CALLS,251)

- 0xA1, 0x0B, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x9B, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_CANCEL_ALLBAR)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", REGISTER_SS_CANCEL_ALLBAR)

ENDSTRUCT

- BEGINARRAY (REGISTER_SS_CANCEL_ALLBAR,251)

- 0xA1, 0x0B, 		/* component invoke tag, length */

- 0x02, 0x01, 0x00,		/* invoke id tag, length, invoke id */

- 0x02, 0x01, 0x0D,		/* operation code tag, length, code(deactivate) */

- 0x30, 0x03, 		/* sequence tag, length */

- 0x04, 0x01, 0x90, 		/* ss code tag, length, ss code(all call barring) */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_REG_PW)

SET_COMP ("l_fac", 0x0058)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", FAC_REG_PW_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (FAC_REG_PW_CONTENT, 11)

0xA1, 0x09, 0x02, 0x01, 0x00, 0x02, 0x01, 0x11, 0x04, 0x01, 0x90

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_PW_RES)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", FAC_PW_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (FAC_PW_RES_CONTENT, 20)

0xA2, 0x12,		/* return result, length */

0x02, 0x01, 0x00,	/* invoke id tag, length, value */

0x30, 0x0D, 		/* sequence tag, length */

0x02, 0x01, 0x11, 	/* operation code tag, length, value (register password) */

0x12, 0x04, 		/* new password, length */

0x39, 0x38, 0x37, 0x36, 0x00, 0x00, 0x00, 0x00	/* new password, old password */

- ENDARRAY

/* FACILITY DEREGISTER AND DEACTIVATE ALL DIVERTS*/

BEGIN_PSTRUCT ("fac_inf", FAC_DEACTIVATE_ALLCALLS)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", DEREGISTER_SS_DIVERT_ALLCALLS)

ENDSTRUCT

- BEGINARRAY_PART (DEREGISTER_SS_DIVERT_ALLCALLS,251)

- 0xA1, 0x0B, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0B, 0x30, 0x03,0x04, 0x01, 0x21, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_ALLDIVERTS)

SET_COMP ("l_fac", L_FAC_104)

SET_COMP ("o_fac", ZERO_VALUE)

SET_COMP ("fac", ERASE_ALLDIVERTS_CONTENTS)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_ALLDIVERTS_CONTENTS, 251)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x03, 0x04, 0x01, 0x20, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* FACILITY CALL WAITING ENABLED */

BEGIN_PSTRUCT ("fac_inf", FAC_SET_WAITING_ENABLED)

SET_COMP ("l_fac", L_FAC_104)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", CALL_WAITING_CONTENTS)

ENDSTRUCT

BEGINARRAY_PART(CALL_WAITING_CONTENTS, 251)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x41, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

BEGINARRAY (VER_SS_1_ARR,8)

0x00, 0x00,0x00,0x00, 0x00,0x00, 0x00, 0x00

ENDARRAY

- BEGIN_PSTRUCT (“ss_ver”, VERSION_SS)

- SET_COMP (“len”, 1)
SET_COMP (“ver”, VER_SS_1_ARR)

- ENDSTRUCT

- BEGIN_PSTRUCT (“fac_inf”, RELEASE_COMPLETE_FACILITY)

- SET_COMP (“l_fac”, L_FAC_176)
SET_COMP (“o_fac”, ZERO_VALUE)
SET_COMP (“fac”, RELEASE_COMPLETE_ARR)

- ENDSTRUCT

- BEGIN_PSTRUCT (“fac_inf”, CALL_WAITING_RETURN_RESULT)

- SET_COMP (“l_fac”, L_FAC_176)
SET_COMP (“o_fac”, ZERO_VALUE)
SET_COMP (“fac”, CALL_WAITING_RET_RES_ARR)

- ENDSTRUCT

- BEGINARRAY_PART (CALL_WAITING_RET_RES_ARR,251)

0xA2, 0x03, 0x02, 0x01, 0x0A, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

- BEGIN_PSTRUCT (“fac_inf”, CALL_FORWARD_RETURN_RESULT)

- SET_COMP (“l_fac”, L_FAC_176)
SET_COMP (“o_fac”, ZERO_VALUE)
SET_COMP (“fac”, CALL_FORWARD_RET_RES_ARR)

- ENDSTRUCT

- BEGINARRAY_PART (CALL_FORWARD_RET_RES_ARR, 250)

- 0xA2, 0x29,		/* return result component tag, length */

- 0x02, 0x01, 0x00,		/* Invoke id tag, length, value */

- 0x30, 0x24,		/* sequence tag, length */

- 0x02, 0x01, 0x0C,		/* operation code tag (activate), length, value */

- 0xA0, 0x1F,		/* parameters follow. forwarding feature, length */

0x04,0x01,0x21, 		/* ss code CFU */

- 0x30, 0x1A,		/* forwarding feature list sequence tag, length */

- 0x82, 0x01, 0x00,		/* bearer services tag, length, value (all services) */

- 0x83, 0x01, 0x10,		/* teleservices tag, length, value (all speech transmission services) */

- 0x84, 0x01, 0x07,		/* ss status tag, length, value (provisioned,registered,active) */

- 0x85, 0x07,		/* forwarded to number tag, length */

- 0x81, 0x10, 0x13, 0x17, 0x08, 0x35, 0xF7,	/* noa+npi, number */

- 0x86, 0x01, 0x0B,		/* forwarding options tag, length, value(version 2 notify+unconditional) */

- 0x87, 0x01, 0x00,	 	/* no reply condition tag, length, value */

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

- 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

- BEGIN_PSTRUCT (“fac_inf”, RELEASE_COMPLETE_DEACTIVATE)

- SET_COMP (“l_fac”, L_FAC_176)
SET_COMP (“o_fac”, ZERO_VALUE)
SET_COMP (“fac”, RELEASE_COMPLETE_DEACTIVARR)

- ENDSTRUCT

/* erase CF for voice,fax,data */

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_ALLDIVERTS_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_ALLDIVERTS_CONTENTS_V)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_ALLDIVERTS_CONTENTS_V, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x20, 0x83, 0x01, 0x10

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_UNCONDITIONALLY_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_UNCONDITIONALLY_CONTENTS_V)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_UNCONDITIONALLY_CONTENTS_V, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x21, 0x83, 0x01, 0x10

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_IFBUSY_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_IFBUSY_CONTENTS_V)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_IFBUSY_CONTENTS_V, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x29, 0x83, 0x01, 0x10

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_IFNOREPLY_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_IFNOREPLY_CONTENTS_V)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_IFNOREPLY_CONTENTS_V, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x83, 0x01, 0x10

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_IFNOSERVICE_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_IFNOSERVICE_CONTENTS_V)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_IFNOSERVICE_CONTENTS_V, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x2B, 0x83, 0x01, 0x10

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_IFNOSERVICE_F)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_IFNOSERVICE_CONTENTS_F)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_IFNOSERVICE_CONTENTS_F, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x2B, 0x83, 0x01, 0x60

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_ALLCONFIGURED_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_ALLCONFIGURED_CONTENTS_V)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_ALLCONFIGURED_CONTENTS_V, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x28, 0x83, 0x01, 0x10

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_IFNOTREACHABLE_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_IFNOTREACHABLE_CONTENTS_V)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_IFNOTREACHABLE_CONTENTS_V, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x28, 0x83, 0x01, 0x10

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_ALLDIVERTS_F)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_ALLDIVERTS_CONTENTS_F)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_ALLDIVERTS_CONTENTS_F, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x20, 0x83, 0x01, 0x60

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_UNCONDITIONALLY_F)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_UNCONDITIONALLY_CONTENTS_F)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_UNCONDITIONALLY_CONTENTS_F, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x21, 0x83, 0x01, 0x60

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_IFBUSY_F)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_IFBUSY_CONTENTS_F)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_IFBUSY_CONTENTS_F, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x29, 0x83, 0x01, 0x60

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_IFNOREPLY_F)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_IFNOREPLY_CONTENTS_F)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_IFNOREPLY_CONTENTS_F, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x83, 0x01, 0x60

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_ALLCONFIGURED_F)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_ALLCONFIGURED_CONTENTS_F)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_ALLCONFIGURED_CONTENTS_F, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x28, 0x83, 0x01, 0x60

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_IFNOTREACHABLE_F)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_IFNOTREACHABLE_CONTENTS_F)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_IFNOTREACHABLE_CONTENTS_F, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x28, 0x83, 0x01, 0x60

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_ALLDIVERTS_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_ALLDIVERTS_CONTENTS_D)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_ALLDIVERTS_CONTENTS_D, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x20, 0x82, 0x01, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_UNCONDITIONALLY_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_UNCONDITIONALLY_CONTENTS_D)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_UNCONDITIONALLY_CONTENTS_D, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x21, 0x82, 0x01, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_IFBUSY_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_IFBUSY_CONTENTS_D)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_IFBUSY_CONTENTS_D, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x29, 0x82, 0x01, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_IFNOSERVICE_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_IFNOSERVICE_CONTENTS_D)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_IFNOSERVICE_CONTENTS_D, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x2B, 0x82, 0x01, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_IFNOREPLY_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_IFNOREPLY_CONTENTS_D)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_IFNOREPLY_CONTENTS_D, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x82, 0x01, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_ALLCONFIGURED_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_ALLCONFIGURED_CONTENTS_D)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_ALLCONFIGURED_CONTENTS_D, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x28, 0x82, 0x01, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_ERASE_IFNOTREACHABLE_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", ERASE_IFNOTREACHABLE_CONTENTS_D)

- ENDSTRUCT

BEGINARRAY_PART (ERASE_IFNOTREACHABLE_CONTENTS_D, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x28, 0x82, 0x01, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", RESULT_ERASE_ALLDIVERTS_V)

SET_COMP ("l_fac", 0x00F8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", RESULT_ERASE_ALL_DIVERTS_CONTENTS)

- ENDSTRUCT

BEGINARRAY_PART (RESULT_ERASE_ALL_DIVERTS_CONTENTS, 23)

0xA2, 0x15, 0x02, 0x01, 0x00, 0x30, 0x10, 0x02, 0x01, 0x0B, 0xA0, 0x0B, 0x04, 0x01, 0x28, 0x30, 0x06, 0x83, 0x01, 0x10, 0x84, 0x01, 0x04

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_STATUS_CALLS_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", DIVERT_STATUS_CALLS_CONTENTS_V)

- ENDSTRUCT

BEGINARRAY_PART (DIVERT_STATUS_CALLS_CONTENTS_V, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x21, 0x83, 0x01, 0x10

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_STATUS_CALLS_F)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", DIVERT_STATUS_CALLS_CONTENTS_F)

- ENDSTRUCT

BEGINARRAY_PART (DIVERT_STATUS_CALLS_CONTENTS_F, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x21, 0x83, 0x01, 0x60

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_STATUS_CALLS_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", DIVERT_STATUS_CALLS_CONTENTS_D)

- ENDSTRUCT

BEGINARRAY_PART (DIVERT_STATUS_CALLS_CONTENTS_D, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x21, 0x82, 0x01, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_STATUS_IFBUSY_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", DIVERT_STATUS_IFBUSY_CONTENTS_V)

- ENDSTRUCT

BEGINARRAY_PART (DIVERT_STATUS_IFBUSY_CONTENTS_V, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x29, 0x83, 0x01, 0x10

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_STATUS_IFBUSY_F)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", DIVERT_STATUS_IFBUSY_CONTENTS_F)

- ENDSTRUCT

BEGINARRAY_PART (DIVERT_STATUS_IFBUSY_CONTENTS_F, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x29, 0x83, 0x01, 0x60

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_STATUS_IFBUSY_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", DIVERT_STATUS_IFBUSY_CONTENTS_D)

- ENDSTRUCT

BEGINARRAY_PART (DIVERT_STATUS_IFBUSY_CONTENTS_D, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x29, 0x82, 0x01, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_STATUS_IFNOREPLY_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", DIVERT_STATUS_IFNOREPLY_CONTENTS_V)

- ENDSTRUCT

BEGINARRAY_PART (DIVERT_STATUS_IFNOREPLY_CONTENTS_V, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x83, 0x01, 0x10

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_STATUS_IFNOREPLY_F)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", DIVERT_STATUS_IFNOREPLY_CONTENTS_F)

- ENDSTRUCT

BEGINARRAY_PART (DIVERT_STATUS_IFNOREPLY_CONTENTS_F, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x83, 0x01, 0x60

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_STATUS_IFNOREPLY_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", DIVERT_STATUS_IFNOREPLY_CONTENTS_D)

- ENDSTRUCT

BEGINARRAY_PART (DIVERT_STATUS_IFNOREPLY_CONTENTS_D, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x82, 0x01, 0x00

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_STATUS_IFNONETWORK_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", DIVERT_STATUS_IFNONETWORK_CONTENTS_V)

- ENDSTRUCT

BEGINARRAY_PART (DIVERT_STATUS_IFNONETWORK_CONTENTS_V, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x2B, 0x83, 0x01, 0x10

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_STATUS_IFNONETWORK_F)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", DIVERT_STATUS_IFNONETWORK_CONTENTS_F)

- ENDSTRUCT

BEGINARRAY_PART (DIVERT_STATUS_IFNONETWORK_CONTENTS_F, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x2B, 0x83, 0x01, 0x60

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", FAC_DIVERT_STATUS_IFNONETWORK_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", DIVERT_STATUS_IFNONETWORK_CONTENTS_D)

- ENDSTRUCT

BEGINARRAY_PART (DIVERT_STATUS_IFNONETWORK_CONTENTS_D, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x2B, 0x82, 0x01, 0x00

- ENDARRAY

- BEGINARRAY (CBCH_MSG_ID_ARR,40)

- 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF

ENDARRAY

- BEGINARRAY (CBCH_DCS_ID_ARR,20)

- 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF

ENDARRAY

- BEGINARRAY (CBCH_MSG_IDS,40)

- 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF

ENDARRAY

- BEGINARRAY (CBCH_MSG_IDS_NEW_BC,40)

- 0x7B, 0x00, 0x7B, 0x00, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF

ENDARRAY

- BEGINARRAY (CBCH_DCS_IDS,20)

- 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF

ENDARRAY

- BEGINARRAY (CBCH_MSG_1,8)

- 0x12, 0x34, 0x56, 0x78, 0x9A, 0xBC, 0xDE, 0xFF

ENDARRAY

- BEGINARRAY (SMS_CONFIG, 8)

0x20, 0x00,  /* sdu.l_buf ( in bit!!! )*/

0x00, 0x00, /* sdu.o_buf */

0x03, 0x00, 0x00, 0x02  /* sdu.buf */
ENDARRAY

- BEGINARRAY (APN_BUF_1, 4)

- 0x03, 0x41, 0x50, 0x4E

ENDARRAY

/* EF ECC field array */

- BEGINARRAY (A_ECC_FIELD,12)

- 0x11, 0xF2, 0xFF, 0x99, 0xF9, 0xFF, 0x21, 0x43, 0x65, 0xFF, 0xFF, 0xFF

ENDARRAY

/* EF AD field array , disable CI */

BEGINARRAY_PART (A_AD_FIELD_CI_DISABLED,4)

0x00, 0x00, 0x00, 0x02

ENDARRAY

/* EF AD field array , enable CI */

BEGINARRAY_PART (A_AD_FIELD_CI_ENABLED,4)

0x00, 0x00, 0x01, 0x02

ENDARRAY

- BEGINARRAY_PART (SIM_CPHS_FIELD,3)

- 0x02, 0x80, 0x00

- ENDARRAY

/* EF - CPHS VMW  */

BEGINARRAY_PART (SIM_CPHS_VMW_FIELD,2)

0x55, 0x55

ENDARRAY

- BEGINARRAY (PREF_PLMN_ARRAY,3)

- 0x00,0x04,0x09

ENDARRAY

- BEGINARRAY (MCC_1,3)

- 0x00,0x04,0x09

ENDARRAY

- /* germany */

- BEGINARRAY (MCC_262,3)

- 0x02,0x06,0x02

ENDARRAY

- BEGINARRAY (MNC_1,2)

- 0x00,0x04

ENDARRAY

- BEGINARRAY (MNC_02,3)

- 0x00,0x02,0x0F

ENDARRAY

- BEGINARRAY (MNC_14,3)

- 0x00,0x01,0x0F

ENDARRAY

BEGIN_PSTRUCT("plmn", PLMN_1)

SET_COMP("v_plmn", v_plmn_PRES)

SET_COMP("mcc", MCC_1)

SET_COMP("mnc", MNC_1)

- ENDSTRUCT

/* vodafone D2 */

BEGIN_PSTRUCT("plmn", PLMN_262_02)

SET_COMP("v_plmn", v_plmn_PRES)

SET_COMP("mcc", MCC_262)

SET_COMP("mnc", MNC_02)

- ENDSTRUCT

BEGIN_PSTRUCT("plmn", PLMN_262_14)

SET_COMP("v_plmn", v_plmn_PRES)

SET_COMP("mcc", MCC_262)

SET_COMP("mnc", MNC_14)

- ENDSTRUCT

- /* for gmmreg_plmn_ind */

- BEGIN_PSTRUCT_ARRAY (PLMN_LIST,GMMREG_MAX_PLMN_ID)

- PLMN_262_02, PLMN_262_14,

- PLMN_NULL, PLMN_NULL, PLMN_NULL, PLMN_NULL, PLMN_NULL,

- PLMN_NULL, PLMN_NULL, PLMN_NULL, PLMN_NULL, PLMN_NULL

- ENDARRAY

BEGIN_SHORT_ARRAY (LAC_LIST,GMMREG_MAX_PLMN_ID)

1234, 5678,0,0,0,0,0,0,0,0,0,0

ENDARRAY

BEGINARRAY (FORB_PLMN_LIST, GMMREG_MAX_PLMN_ID)

FORB_PLMN_NOT_INCLUDED, FORB_PLMN_NOT_INCLUDED, 0,0,0,0,0,0,0,0,0,0

ENDARRAY

BEGINARRAY (RX_LEVEL_LIST, GMMREG_MAX_PLMN_ID)

0x0c, 0x1F, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

BEGINARRAY (GPRS_STATUS_LIST, GMMREG_MAX_PLMN_ID)

GMMREG_GPRS_GSM, GMMREG_GPRS_ONLY,0,0,0,0,0,0,0,0,0,0

ENDARRAY

BEGIN_PSTRUCT("plmn", PLMN_NULL)

SET_COMP("v_plmn", GMMREG_PLMN_NOT_PRES)

SKIP_COMP("mcc")

SKIP_COMP("mnc")

- ENDSTRUCT

- BEGINARRAY (SIM_TOOLKIT_PROFILE,12)

- 0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00

ENDARRAY

BEGINARRAY (NO_EC_CODES, 15)
	0xFF,0xFF,0xFF,0xFF,0xFF,
	0xFF,0xFF,0xFF,0xFF,0xFF,
	0xFF,0xFF,0xFF,0xFF,0xFF
ENDARRAY

BEGINARRAY (EC_CODES, 15)
	0x11, 0xF2, 0xFF,
	0x99, 0xF9, 0xFF,
	0xFF,0xFF,0xFF,0xFF,0xFF,
	0xFF,0xFF,0xFF,0xFF
ENDARRAY

BEGINARRAY (PUK_1_VALUE,8)
	0x31,0x32,0x33,0x34,0x35, 0x36,0x37,0x38

ENDARRAY

BEGINARRAY (PUK_2_VALUE_11223344,8)
	0x31,0x31,0x32,0x32,0x33, 0x33,0x34,0x34

ENDARRAY

BEGINARRAY (NO_PREF_LANG,5)

0xFF,0xFF,0xFF,0xFF,0xFF

ENDARRAY

BEGINARRAY (STK_SUPPORTED, 20)

0x4D,0x7F,0x97,0x3F,0x7F,0x00,0x00,0xFC,0x03,0x00,0,0x13,0x23,0,0,0,0x02,0,0,0

ENDARRAY

BEGINARRAY (IMSI_ARRAY,9)

0x29, 0x26, 0x10, 0x74, 0x11, 0x94, 0x21, 0xFF, 0xFF

ENDARRAY

- BEGINARRAY (DTI2_SDU_1, 8)

0x20, 0x00,  /* sdu.l_buf ( in bit!!! )*/

0x00, 0x00, /* sdu.o_buf */

0x52, 0x49, 0x4e, 0x47  /* sdu.buf */
ENDARRAY

- BEGINARRAY (SDU_1, 9)

0x28, 0x00,  /* sdu.l_buf ( in bit!!! )*/

0x00, 0x00, /* sdu.o_buf */

0x55, 0x41, 0x52, 0x54, 0x00  /* sdu.buf */
ENDARRAY

- BEGINARRAY (SDU_2, 10)

0x30, 0x00,  /* sdu.l_buf ( in bit!!! )*/

0x00, 0x00, /* sdu.o_buf */

0x55, 0x41, 0x52, 0x54, 0xFF, 0x00
ENDARRAY

- BEGINARRAY (SDU_3, 11)

0x38, 0x00,  /* sdu.l_buf ( in bit!!! )*/

0x00, 0x00, /* sdu.o_buf */

0x55, 0x41, 0x52, 0x54, 0xFF, 0xFE, 0x00
ENDARRAY

- BEGINARRAY (SDU_ARRAY, 1)

0x03

- ENDARRAY

BEGIN_PSTRUCT("sdu", SDU)

SET_COMP("l_buf", NUM_1)

SET_COMP("o_buf", NUM_0)

SET_COMP("buf", SDU_ARRAY)

- ENDSTRUCT

BEGIN_PSTRUCT("sdu", SDU_SHOW)

SHOW_COMP("l_buf")

SHOW_COMP("o_buf")

SHOW_COMP("buf")

- ENDSTRUCT

BEGIN_PSTRUCT("pref_plmn", PREF_PLMN)

SET_COMP("c_pref", NUM_3)

SET_COMP("pref", PREF_PLMN_ARRAY)

- ENDSTRUCT

BEGIN_PSTRUCT("imsi_field", IMSI)

SET_COMP("c_field", NUM_9)

SET_COMP("field", IMSI_ARRAY)

- ENDSTRUCT

/* CH MODE DATA 9.6K */

BEGIN_PSTRUCT ("chm", CH_MODE_DATA_TCH_F)

SET_COMP ("ch_type", CH_TCH_F)

SET_COMP ("ch_mode", CHM_DATA_9_6)

ENDSTRUCT

/* CH MODE SIGNALLING */

BEGIN_PSTRUCT ("chm", CH_SIG_ONLY)

SET_COMP ("ch_type", CH_SDCCH)

SET_COMP ("ch_mode", CHM_SIG_ONLY)

ENDSTRUCT

/* CH MODE FAX */

BEGIN_PSTRUCT ("chm", CH_MODE_FAX_TCH_F)

SET_COMP ("ch_type", CH_TCH_F)

SET_COMP ("ch_mode", CHM_DATA_9_6)

ENDSTRUCT

/* CH MODE SPEECH */

BEGIN_PSTRUCT ("chm", CH_MODE_SPEECH_TCH_F)

SET_COMP ("ch_type", CH_TCH_F)

SET_COMP ("ch_mode", CHM_SPEECH_V3)

ENDSTRUCT

/* All things to do with SMS */

BYTE MSG_TYPE_1D  0x1D

BYTE PROT_ID  0

BEGINARRAY (YEAR_02, 2) 0x00, 0x02
ENDARRAY

BEGINARRAY (MONTH_05, 2) 0x00, 0x05
ENDARRAY

BEGINARRAY (DAY_25, 2) 0x02, 0x05
ENDARRAY

BEGINARRAY (HOUR_12, 2) 0x01, 0x02
ENDARRAY

BEGINARRAY (MINUTE_34, 2) 0x03, 0x04
ENDARRAY

BEGINARRAY (SECOND_56, 2) 0x05, 0x06
ENDARRAY

BEGINARRAY (TP_UD1, 161)

0x07, 0x91, 0x44, 0x97, 0x37, 0x01, 0x90, 0x37, 0x44, 0x0C, 0x91, 0x44, 0x87, 0x71, 0x61,
	0x35, 0x87, 0x00, 0x00, 0x20, 0x50, 0x72, 0x31, 0x40, 0x55, 0x40, 0x13, 0x05, 0x00, 0x03,
	0xC8, 0x03, 0x03, 0xF0, 0xCC, 0x69, 0x72, 0xF8, 0x04, 0x99, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF

ENDARRAY

BEGIN_PSTRUCT ("sms_sdu", SMS_SDU_MISSED_CALLS)
	SET_COMP ("l_buf",	0xA0)
	SET_COMP ("o_buf",	NUM_0)
	SET_COMP ("buf",	SMS_MSG_MISSED_CALLS)
ENDSTRUCT

/* SMS containing “Eric” as input from the keys*/

BEGINARRAY (SMS_MSG_MISSED_CALLS, 175)
	0x01, 0x81, 0x11, 0x00, 0x0B, 0x91, 0x10, 0x13,

0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0xA7, 0x04,

0x45, 0x79, 0x7A, 0x0C, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("sms_sdu", SMS_SDU_OUTGOING_CALLS)
	SET_COMP ("l_buf",	0xA0)
	SET_COMP ("o_buf",	NUM_0)
	SET_COMP ("buf",	SMS_MSG_OUTGOING_CALLS)
ENDSTRUCT

/* SMS containing “Eric” as input from the keys*/

BEGINARRAY (SMS_MSG_OUTGOING_CALLS, 175)
	0x01, 0x81, 0x11, 0x00, 0x0B, 0x81, 0x10, 0x13,

0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0xA7, 0x04,

0x45, 0x79, 0x7A, 0x0C, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("sms_sdu", SMS_PHB)
	SET_COMP ("l_buf",	0xA0)
	SET_COMP ("o_buf",	NUM_0)
	SET_COMP ("buf",	SMS_MSG_1)
ENDSTRUCT

BEGIN_PSTRUCT ("sms_sdu", SMS_SDU_1)
	SET_COMP ("l_buf",	0xD8)
	SET_COMP ("o_buf",	NUM_0)
	SET_COMP ("buf",	SMS_MSG_1)
ENDSTRUCT

/* SMS containing “Eric” as input from the keys*/

BEGINARRAY (SMS_MSG_1, 175)
	0x08, 0x81, 0x00, 0x44, 0x97, 0x37, 0x01, 0x90,

0x17, 0x11, 0x00, 0x0B, 0x81, 0x10, 0x13, 0x17,
	0x08, 0x35, 0xF7, 0x00, 0x00, 0xA7, 0x04, 0x45,
	0x79, 0x7A, 0x0C, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("sms_sdu", SMS_SDU_ERIC_NAMES)
	SET_COMP ("l_buf",	0xA0)
	SET_COMP ("o_buf",	NUM_0)
	SET_COMP ("buf",	SMS_MSG_ERIC_NAMES)
ENDSTRUCT

BEGINARRAY (SMS_MSG_ERIC_NAMES, 175)
	0x01,0x81, 0x11, 0x00, 0x0B, 0x81, 0x10, 0x13,
 	0x17, 0x08, 0x35, 0xF7, 0x00, 0x00, 0xA7, 0x04,
	0x45, 0x79, 0x7A, 0x0C, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("sms_sdu", SMS_SDU_2)
	SET_COMP ("l_buf",	0xA8)
	SET_COMP ("o_buf",	NUM_0)
	SET_COMP ("buf",	SMS_MSG_2)
ENDSTRUCT

/* SMS containing SMS_MSG_1 as retrieved from SIM memory */

BEGINARRAY (SMS_MSG_2, 175)
	0x08, 0x81, 0x00, 0x44, 0x97, 0x37, 0x01, 0x90,

0x17, 0x11, 0x00, 0x00, 0x80, 0x00, 0x00, 0xA7,
	0x04, 0x45, 0x79, 0x7A, 0x0C, 0x00, 0x00, 0x00, 
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("sms_sdu", SMS_NO_TEXT)
	SET_COMP ("l_buf",	0xB8)
	SET_COMP ("o_buf",	NUM_0)
	SET_COMP ("buf",	SMS_NO_TEXT_CONTENT)
ENDSTRUCT

/* Empty SMS text msg*/

BEGINARRAY (SMS_NO_TEXT_CONTENT, 175)
	0x08, 0x81, 0x00, 0x44, 0x97, 0x37, 0x01, 0x90,

0x17, 0x11, 0x00, 0x0B, 0x81, 0x10, 0x13, 0x17,
	0x08, 0x35, 0xF7, 0x00, 0x00, 0xA7, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("sms_sdu", SMS_SDU_CONC_1)
	SET_COMP ("l_buf",	0x518)
	SET_COMP ("o_buf",	NUM_0)
	SET_COMP ("buf",	SMS_MSG_CONC_1)
ENDSTRUCT

/* SMS containing the first letter from eack text input key, holding 160 7 bit chars as input from the keys*/

BEGINARRAY (SMS_MSG_CONC_1,175)

0x08, 0x81, 0x00, 0x44, 0x97, 0x37, 0x01, 0x90,

0x17, 0x51, 0x00, 0x0B, 0x81, 0x10, 0x13, 0x17,

0x08, 0x35, 0xF7, 0x00, 0x00, 0xA7, 0xA0, 0x05,

0x00, 0x03, 0x01, 0x02, 0x01, 0x82, 0xE4, 0xB3,

0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA,

0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D,

0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7,

0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF,

0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3,

0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4,

0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3,

0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA,

0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D,

0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7,

0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF,

0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3,

0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4,

0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3,

0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA,

0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D,

0xA7, 0xDF, 0xC3, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

/*BEGINARRAY (SMS_MSG_CONC_1,175)

0x08, 0x81, 0x00, 0x44, 0x97, 0x37, 0x01, 0x90,
	0x17, 0x11, 0x00, 0x0B, 0x81, 0x10, 0x13, 0x17,
	0x08, 0x35, 0xF7, 0x00, 0x00, 0xA7, 0xA0, 0x41,
	0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2,

0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59,

0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD,

0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD, 0x86,

0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD, 0x86, 0xD3,

0xEF, 0x61, 0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF,

0x61, 0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61,

0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2,

0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59,

0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD,

0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD, 0x86,

0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD, 0x86, 0xD3,

0xEF, 0x61, 0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF,

0x61, 0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61,

0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2,

0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59,

0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD,

0x86, 0xD3, 0xEF, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

*/

BEGIN_PSTRUCT ("sms_sdu", SMS_SDU_MAX_1)
	SET_COMP ("l_buf",	0x518)
	SET_COMP ("o_buf",	NUM_0)
	SET_COMP ("buf",	SMS_MSG_MAX_1)
ENDSTRUCT

/* max SMS msg */

BEGINARRAY (SMS_MSG_MAX_1,175)

0x08, 0x81, 0x00, 0x44, 0x97, 0x37, 0x01, 0x90,
	0x17, 0x11, 0x00, 0x0B, 0x81, 0x10, 0x13, 0x17,
	0x08, 0x35, 0xF7, 0x00, 0x00, 0xA7, 0xA0, 0x41,
	0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2,

0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59,

0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD,

0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD, 0x86,

0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD, 0x86, 0xD3,

0xEF, 0x61, 0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF,

0x61, 0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61,

0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2,

0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59,

0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD,

0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD, 0x86,

0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD, 0x86, 0xD3,

0xEF, 0x61, 0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF,

0x61, 0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61,

0xF2, 0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2,

0x59, 0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59,

0xDD, 0x86, 0xD3, 0xEF, 0x61, 0xF2, 0x59, 0xDD,

0x86, 0xD3, 0xEF, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

BEGIN_PSTRUCT ("sms_sdu", SMS_SDU_MT_CONC_2)
	SET_COMP ("l_buf",	0x518)
	SET_COMP ("o_buf",	NUM_0)
	SET_COMP ("buf",	SMS_DELIVER_CONC_2)
ENDSTRUCT

BEGIN_PSTRUCT ("sms_sdu", SMS_SDU_CONC_2)
	SET_COMP ("l_buf",	0x200)
	SET_COMP ("o_buf",	NUM_0)
	SET_COMP ("buf",	SMS_MSG_CONC_2)
ENDSTRUCT

/* SMS containing the last letters from eack text input key, holding 16 7 bit chars as input from the keys*/

BEGINARRAY (SMS_MSG_CONC_2,175)

0x08, 0x81, 0x00, 0x44, 0x97, 0x37, 0x01, 0x90,
	0x17, 0x51, 0x00, 0x0B, 0x81, 0x10, 0x13, 0x17,
	0x08, 0x35, 0xF7, 0x00, 0x00, 0xA7, 0x2E, 0x05,
	0x00, 0x03, 0x01, 0x02, 0x02, 0xC8, 0x67, 0x75,
	0x1B, 0x4E, 0xBF, 0x87, 0xC9, 0x67, 0x75, 0x1B,
	0x4E, 0xBF, 0x87, 0xC9, 0x67, 0x75, 0x1B, 0x4E,
	0xBF, 0x87, 0xC9, 0x67, 0x75, 0x1B, 0x4E, 0xBF,
	0x87, 0xC9, 0x67, 0x75, 0x1B, 0x4E, 0xBF, 0x03,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

BEGIN_PSTRUCT ("sms_sdu", SMS_SDU_EMPTY)
	SET_COMP ("l_buf",	0x00)
	SET_COMP ("o_buf",	NUM_0)
	SET_COMP ("buf",	SMS_MSG_EMPTY)
ENDSTRUCT

BEGINARRAY (SMS_MSG_EMPTY, 175)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

/* SMS delivery msg containg rp addr and the tpdu */

BEGINARRAY (SMS_DELIVER_CONC_1, 181)

/* rp addr */

0x07,			/* num digits in rp addr */
0x91,			/* ton+npi */

0x10,0x32, 0x54, 0x76, 0x98, 0xF9,		/* rp number */
0x44,		/* no reply path, data hdr present, no stat rep,no more msg’s waiting,MT sms */
0x0B,0x28,0x10,0x13,0x17,0x08,0x35,0xF7,	/* originator address, num digits, ton, npi, number */
0x00,			/* protocol identifier, default */
0xF0,			/* data coding scheme, with msg class */
0x03,0x06,0x20, 0x11,0x15,0x95,0x40,	/* service centre time stamp, year,month,day,hour,min,sec,timezone */
/* user data */

0xA7, 0xA0, 0x00,	/* length 167 , 1st data byte = zero = concat */

0xA4, 0x00, 0x02, 0x01,	/* length udh, udh ref num, udh max num, udh seq num */
0x01, 0x82, 0xE4, 0xB3,

0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA,

0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D,

0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7,

0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF,

0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3,

0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4,

0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3,

0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA,

0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D,

0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7,

0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF,

0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3,

0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4,

0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3,

0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA,

0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D,

0xA7, 0xDF, 0xC3, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

BEGIN_PSTRUCT ("sms_sdu", SMS_SDU_MT_CONC_1)
	SET_COMP ("l_buf",	0x518)
	SET_COMP ("o_buf",	NUM_0)
	SET_COMP ("buf",	SMS_DELIVER_CONC_1)
ENDSTRUCT

/* SMS delivery msg containg rp addr and the tpdu */

BEGINARRAY (SMS_DELIVER_CONC_2, 181)

/* rp addr */

0x07,			/* num digits in rp addr */
0x91,			/* ton+npi */

0x10,0x32, 0x54, 0x76, 0x98, 0xF9,		/* rp number */
0x44,		/* no reply path, data hdr present, no stat rep,no more msg’s waiting,MT sms */
0x0B,0x28,0x10,0x13,0x17,0x08,0x35,0xF7,	/* originator address, num digits, ton, npi, number */
0x00,			/* protocol identifier, default */
0xF0,			/* data coding scheme, with msg class */
0x03,0x06,0x20, 0x11,0x15,0x95,0x40,	/* service centre time stamp, year,month,day,hour,min,sec,timezone */
/* user data */

0xA7, 0xA0, 0x00,	/* length 167 , 1st data byte = zero = concat */

0xA4, 0x00, 0x02, 0x02,	/* length udh, udh ref num, udh max num, udh seq num */
0x01, 0x82, 0xE4, 0xB3,

0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA,

0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D,

0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7,

0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF,

0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3,

0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4,

0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3,

0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA,

0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D,

0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7,

0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF,

0xC3, 0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3,

0xE4, 0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4,

0xB3, 0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3,

0xBA, 0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA,

0x0D, 0xA7, 0xDF, 0xC3, 0xE4, 0xB3, 0xBA, 0x0D,

0xA7, 0xDF, 0xC3, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

ENDARRAY

BEGINARRAY ( SC_ADDR_1_NB, 9)
	0x09, 0x08, 0x07, 0x06, 0x05,
	0x04, 0x03, 0x02, 0x01
ENDARRAY

/* sim toolkit */

BEGIN_PSTRUCT ("stk_cmd", STK_DISPLAY_TEXT_SHORT)

SET_COMP ("l_cmd", 0x0140)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_DISPLAY_TEXT_SHORT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_DISPLAY_TEXT_SHORT_CONTENT, 40)
	0xD0, 	/* proactive SIM command tag	*/
	38,	/* following length	*/
	0x81,	/* command details tag	*/
	3,	/* command details length	*/
	1,	/* command number	*/
	0x21,	/* command DISPLAY TEXT	*/
	1,	/* high priority	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x02,	/* destination Display	*/
	0x8D,	/* text string tag	*/
	27,	/* text string length	*/
	0,	/* data coding scheme 8 bit	*/
	0x41, 0x42, 0x43, 0x44, 0x45,    /* text	*/
	0x46, 0x47, 0x48, 0x49, 0x4A,
	0x4B, 0x4C, 0x4D, 0x4E, 0x4F,
	0x50, 0x51, 0x52, 0x53, 0x54,
	0x55, 0x56, 0x57, 0x58, 0x59,
	0x5A
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_TERM_RESP_DISPLAY_TEXT)

SET_COMP ("l_cmd", 0x0060)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_TERM_RESP_DISPLAY_TEXT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_TERM_RESP_DISPLAY_TEXT_CONTENT, 12)
	0x81, 	/* command details tag	*/
	3, 	/* command details length	*/
	1,	/* command number	*/
	0x21,	/* command DISPLAY TEXT	*/
	1,	/* high priority	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x82,	/* source ME	*/
	0x81,	/* destination SIM	*/
	0x83,	/* result tag	*/
	1,	/* result length	*/
	0	/* result OK	*/
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_SETUP_MENU)

SET_COMP ("l_cmd", 0x0138)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_SETUP_MENU_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_SETUP_MENU_CONTENT, 39)
	0xD0, 	/* proactive SIM command tag	*/
	37,	/* following length	*/
	0x81,	/* command details tag	*/
	3,	/* command details length	*/
	6,	/* command number	*/
	0x25,	/* command SET UP MENU	*/
	0,	/* not used	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x82,	/* destination ME	*/
	0x85,	/* alpha identifier tag	*/
	10,	/* alpha identifier length	*/
	0x4D, 0x45, 0x4E, 0x55, 0x20,   /* menu title	*/
	0x54, 0x49, 0x54, 0x4C, 0x45,
	0x8F,	/* item tag	*/
	6,	/* item length	*/
	0x49, 0x54, 0x45,	/* item 1	*/
	0x4d, 0x20, 0x31,
	0x0F,	/* item tag	*/
	6,	/* item length	*/
	0x49, 0x54, 0x45,	/* item 2	*/
	0x4d, 0x20, 0x32
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_TERM_RESP_SETUP_MENU)

SET_COMP ("l_cmd", 0x0060)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_TERM_RESP_SETUP_MENU_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_TERM_RESP_SETUP_MENU_CONTENT, 12)
	0x81, 	/* command details tag	*/
	3, 	/* command details length	*/
	6,	/* command number	*/
	0x25,	/* command DISPLAY TEXT	*/
	0,	/* low priority	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x82,	/* source ME	*/
	0x81,	/* destination SIM	*/
	0x83,	/* result tag	*/
	1,	/* result length	*/
	0	/* result OK	*/
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_SELECT_ITEM)

SET_COMP ("l_cmd", 0x00D8)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_SELECT_ITEM_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_SELECT_ITEM_CONTENT, 26)
	0xD0, 	/* proactive SIM command tag	*/
	25,	/* following length	*/
	0x81,	/* command details tag	*/
	3,	/* command details length	*/
	7,	/* command number	*/
	0x24,	/* command SELECT ITEM	*/
	0,	/* not used	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x82,	/* destination ME	*/
	0x8F,	/* item tag	*/
	6,	/* item length	*/
	0x69, 0x54, 0x45,	/* item 1	*/
	0x4d, 0x20, 0x31,
	0x0F,	/* item tag	*/
	6,	/* item length	*/
	0x49, 0x54, 0x45,	/* item 2	*/
	0x4d, 0x20, 0x32
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_TERM_RESP_SELECT_ITEM)

SET_COMP ("l_cmd", 0x0078)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_TERM_RESP_SELECT_ITEM_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_TERM_RESP_SELECT_ITEM_CONTENT, 15)
	0x81, 	/* command details tag	*/
	3, 	/* command details length	*/
	7,	/* command number	*/
	0x24,	/* command SELECT_ITEM	*/
	0,	/* low priority	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x82,	/* source ME	*/
	0x81,	/* destination SIM	*/
	0x83,	/* result tag	*/
	1,	/* result length	*/
	0,	/* result OK	*/
	0x90,	/* item id tag*/
	1,	/* item length*/
	0x69	/* ident of item chosen*/
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_GET_INKEY)

SET_COMP ("l_cmd", 0x0140)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_GET_INKEY_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_GET_INKEY_CONTENT, 40)
	0xD0, 	/* proactive SIM command tag	*/
	38,	/* following length	*/
	0x81,	/* command details tag	*/
	3,	/* command details length	*/
	3,	/* command number	*/
	0x22,	/* command GET INKEY	*/
	1,	/* sms default alphabet	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x82,	/* destination ME	*/
	0x8D,	/* text string tag	*/
	27,	/* text string length	*/
	1,	/* data coding scheme 8 bit	*/
	0x41, 0x42, 0x43, 0x44, 0x45,    /* text	*/
	0x46, 0x47, 0x48, 0x49, 0x4A,
	0x4B, 0x4C, 0x4D, 0x4E, 0x4F,
	0x50, 0x51, 0x52, 0x53, 0x54,
	0x55, 0x56, 0x57, 0x58, 0x59,
	0x5A
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_TERM_RESP_GET_INKEY)

SET_COMP ("l_cmd", 0x0080)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_TERM_RESP_GET_INKEY_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_TERM_RESP_GET_INKEY_CONTENT, 15)
	0x81, 	/* command details tag	*/
	3, 	/* command details length */
	3,	/* command number	*/
	0x22,	/* command GET_INKEY	*/
	1,	/* medium priority	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x82,	/* source ME	*/
	0x81,	/* destination SIM	*/
	0x83,	/* result tag	*/
	1,	/* result length	*/
	0,	/* result OK	*/
	0x8D,	/* text string tag*/
	2,	/* length*/
	4,	/* DCS 8 bit data */
	0x57	/* “W” in ASCII, key 9*/
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_GET_INPUT)

SET_COMP ("l_cmd", 0x0160)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_GET_INPUT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_GET_INPUT_CONTENT, 44)
	0xD0, 	/* proactive SIM command tag	*/
	42,	/* following length	*/
	0x81,	/* command details tag	*/
	3,	/* command details length	*/
	4,	/* command number	*/
	0x23,	/* command GET INPUT	*/
	1,	/* sms default alphabet	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x82,	/* destination ME	*/
	0x8D,	/* text string tag	*/
	27,	/* text string length	*/
	1,	/* data coding scheme 8 bit	*/
	0x41, 0x42, 0x43, 0x44, 0x45,    /* text	*/
	0x46, 0x47, 0x48, 0x49, 0x4A,
	0x4B, 0x4C, 0x4D, 0x4E, 0x4F,
	0x50, 0x51, 0x52, 0x53, 0x54,
	0x55, 0x56, 0x57, 0x58, 0x59,
	0x5A,
	0x91,	/* response length tag	*/
	2,	/* response length length	*/
	3,	/* minimum response length	*/
	10	/* maximum response length	*/
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_TERM_RESP_GET_INPUT)

SET_COMP ("l_cmd", 0x0090)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_TERM_RESP_GET_INPUT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_TERM_RESP_GET_INPUT_CONTENT, 15)
	0x81, 	/* command details tag	*/
	3, 	/* command details length */
	4,	/* command number	*/
	0x23,	/* command GET_INPUT	*/
	1,	/* medium priority	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x82,	/* source ME	*/
	0x81,	/* destination SIM	*/
	0x83,	/* result tag	*/
	1,	/* result length	*/
	0,	/* result OK	*/
	0x8D,	/* text string tag*/
	4,	/* length*/
	4,	/* DCS 8 bit data */
	0x41,	/* “A” in ASCII, key 2*/
	0x64,	/* “d” in ASCII, key 3*/
	0x67	/* “g” in ASCII, key 4*/
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_SEND_SMS)

SET_COMP ("l_cmd", 0x0110)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_SEND_SMS_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_SEND_SMS_CONTENT, 34)
	0xD0, 	/* proactive SIM command tag	*/
	32,	/* following length	*/
	0x81,	/* command details tag	*/
	3,	/* command details length	*/
	8,	/* command number	*/
	0x13,	/* type of command SEND SMS	*/
	0,	/* command qualifier, no packing	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x83,	/* destination Network	*/
	0x8B,	/* SMS TPDU tag	*/
	21,	/* SMS TPDU length	*/
	0x19,	/* SUBMIT, VP-ABS */
	0x04,
	0x06, 0x91, 0x56, 0x34, 0x12,	/* TP_ADDR_SBM */
	0x00, 0x00,		/* pid, no class gsm alphabet */
	0x89, 0x10, 0x70, 0x21, 0x43, 0x65, 0x40,	/* GMT time plus 1 hour */
	0x04, 0x45, 0x79, 0x7A, 0x0C		/* sms text Eric */

ENDARRAY

BEGIN_PSTRUCT (“sms_sdu“,SMS_SDU_STK)

SET_COMP (“l_buf”, 0xB0)

SET_COMP (“o_buf”, 0x00)

SET_COMP (“buf”, SMS_SDU_STK_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (SMS_SDU_STK_CONTENT, 22)

0x00, 0x19, 0x04, 0x06, 0x91, 0x56, 0x34, 0x12, 0x00, 0x00, 0x89, 0x10, 0x70, 0x21, 0x43, 0x65, 0x40,

0x04, 0x45, 0x79, 0x7A, 0x0c

ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_TERM_RESP_SEND_SMS)

SET_COMP ("l_cmd", 0x0060)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_TERM_RESP_SEND_SMS_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_TERM_RESP_SEND_SMS_CONTENT, 12)
	0x81, 	/* command details tag	*/
	3, 	/* command details length */
	8,	/* command number	*/
	0x13,	/* command SEND_SMS	*/
	0,	/* low priority	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x82,	/* source ME	*/
	0x81,	/* destination SIM	*/
	0x83,	/* result tag	*/
	1,	/* result length	*/
	0	/* result OK	*/
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_SEND_SS)

SET_COMP ("l_cmd", 0x00B8)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_SEND_SS_CONTENT)

- ENDSTRUCT

- BEGINARRAY_PART (STK_SEND_SS_CONTENT, 23)
	0xD0, 	/* proactive SIM command tag	*/
	21,	/* following length	*/
	0x81,	/* command details tag	*/
	3,	/* command details length	*/
	9,	/* command number	*/
	0x11,	/* command SEND SS	*/
	0,	/* not used	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x83,	/* destination Network	*/
	0x89,	/* SS string tag	*/
	10,	/* SS string length	*/
	145,	/* TON and NPI	*/
	0xAA, 0x12, 0x0A, 0x31, 0x71, 0x81, 0x50, 0x73, 0x0B		/* coded in BCD */

- ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_TERM_RESP_SEND_SS)

SET_COMP ("l_cmd", 0x0060)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_TERM_RESP_SEND_SS_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_TERM_RESP_SEND_SS_CONTENT, 12)
	0x81, 	/* command details tag	*/
	3, 	/* command details length */
	9,	/* command number	*/
	0x11,	/* command SEND_SS	*/
	0,	/* low priority	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x82,	/* source ME	*/
	0x81,	/* destination SIM	*/
	0x83,	/* result tag	*/
	1,	/* result length	*/
	0	/* result OK	*/
ENDARRAY

- BEGINARRAY_PART (STK_DIVERT_ALLCALLS_CONTENT,22)

- 0xA1, 0x14, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0C, 0x04, 0x01, 0x21, 0x84, 0x07, 0x91, 0x10, 0x13, 0x17, 0x08,
0x35, 0xF7

- ENDARRAY

BEGIN_PSTRUCT ("fac_inf", STK_DIVERT_ALLCALLS)

SET_COMP ("l_fac", 0x00B0)
SET_COMP ("o_fac", 0x0000)
SET_COMP ("fac", STK_DIVERT_ALLCALLS_CONTENT)

ENDSTRUCT

BEGIN_PSTRUCT ("stk_cmd", STK_SETUP_CALL)

SET_COMP ("l_cmd", 0x00A0)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_SETUP_CALL_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_SETUP_CALL_CONTENT, 20)
	0xD0, 	/* proactive SIM command tag	*/
	18,	/* following length	*/
	0x81,	/* command details tag	*/
	3,	/* command details length	*/
	10,	/* command number	*/
	0x10,	/* command SET UP CALL	*/
	0,	/* Setup call if not busy	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x83,	/* destination Network	*/
	0x86,	/* address tag	*/
	7,	/* address length	*/
	0x91,	/* TON(1) and NPI(1)	*/
	0x10, 0x13, 0x17, 0x08, 0x35, 0xF7
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_TERM_RESP_SETUP_CALL)

SET_COMP ("l_cmd", 0x0060)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_TERM_RESP_SETUP_CALL_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_TERM_RESP_SETUP_CALL_CONTENT, 12)
	0x81, 	/* command details tag	*/
	3, 	/* command details length */
	10,	/* command number	*/
	0x10,	/* command SETUP CAL	*/
	0,	/* low priority	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x82,	/* source ME	*/
	0x81,	/* destination SIM	*/
	0x83,	/* result tag	*/
	1,	/* result length	*/
	0	/* result OK	*/
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_TERM_RESP_SETUP_CLASSE)

SET_COMP ("l_cmd", 0x00D0)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_TERM_RESP_SETUP_CLASSE_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_TERM_RESP_SETUP_CLASSE_CONTENT, 28)
	0x81, 	/* command details tag	*/
	3, 	/* command details length */
	01,	/* command number	*/
	0x40,	/* command CLASS E	*/
	01,	/* low priority	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x82,	/* source ME	*/
	0x81,	/* destination SIM	*/
	0x83,	/* result tag	*/
	1,	/* result length	*/
	0,	/* result OK	*/
	0xB8,	/* destination SIM	*/
	0x02,	/* result tag	*/
	0x81,	/* result length	*/
	0,	/* result OK	*/
	0xB5,	/* destination SIM	*/
	0x04,	/* result tag	*/
	1,	/* result length	*/
	0x0C,	/* result OK	*/
                          0x00, 0x01, 0xB9, 0x02, 0x05, 0xDC, 0x00, 0x00

ENDARRAY

BEGIN_PSTRUCT ("called_party", STK_CLED_PARTY_01317180537)
	SET_COMP ("ton", TON_INT_NUMB)
	SET_COMP ("npi", NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num", 11)
	SET_COMP ("called_num", NUM_01317180537)
ENDSTRUCT

BEGIN_PSTRUCT ("stk_cmd", STK_SETUP_CALL_BUSY)

SET_COMP ("l_cmd", 0x00A0)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_SETUP_CALL_BUSY_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_SETUP_CALL_BUSY_CONTENT, 20)
	0xD0, 	/* proactive SIM command tag	*/
	18,	/* following length	*/
	0x81,	/* command details tag	*/
	3,	/* command details length	*/
	10,	/* command number	*/
	0x10,	/* command SET UP CALL	*/
	1,	/* Setup call by redial if busy	*/
	0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x83,	/* destination Network	*/
	0x86,	/* address tag	*/
	7,	/* address length	*/
	0x91,	/* TON(1) and NPI(1)	*/
	0x10, 0x13, 0x17, 0x08, 0x35, 0xF7
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_OPEN_CHANNEL_IMM_CSD_DATA_CIRCUIT_ASYNC_UDI)

SET_COMP ("l_cmd", 0x00F0)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_OPEN_CHANNEL_IMM_CSD_CONTENT_0)

- ENDSTRUCT

BEGINARRAY_PART (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_0, 28)
	0xD0, 	/* proactive SIM command tag	*/
	28,	/* following length */
	0x81,	/* command details tag */
	3,	/* command details length */
	1,	/* command number */
	0x40, 	/* command OPEN_CHANNEL	*/

0x01,	/* comm qual, Immediate, no auto reconnect	*/

0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x82,	/* destination ME	*/
	0x86,	/* address tag	*/
	7,	/* address length */
	0x91,	/* TON(1) and NPI(1) */
	0x10, 0x13, 0x17, 0x08, 0x35, 0xF7,
	0xB5,	/* bearer description tag */
	4,	/* bearer length */
	0x01,	/* bearer type CSD */
	12,	/* speed 9600 bps */
	0,	/* name, data circuit asynch UDI*/
	1,	/* connection element non-transparent */
	0xB9,	/* buffer size tag */
	2,	/* buffer size length */
	0x02, 0x32	/* buffer size of 562 bytes */
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_OPEN_CHANNEL_IMM_CSD_DATA_CIRCUIT_SYNCH_UDI)

SET_COMP ("l_cmd", 0x00F0)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_OPEN_CHANNEL_IMM_CSD_CONTENT_1)

- ENDSTRUCT

BEGINARRAY_PART (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_1, 28)
	0xD0, 	/* proactive SIM command tag	*/
	28,	/* following length */
	0x81,	/* command details tag */
	3,	/* command details length */
	1,	/* command number */
	0x40, 	/* command OPEN_CHANNEL	*/

0x01,	/* comm qual, Immediate, no auto reconnect	*/

0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x82,	/* destination ME	*/
	0x86,	/* address tag	*/
	7,	/* address length */
	0x91,	/* TON(1) and NPI(1) */
	0x10, 0x13, 0x17, 0x08, 0x35, 0xF7,
	0xB5,	/* bearer description tag */
	4,	/* bearer length */
	0x01,	/* bearer type CSD */
	12,	/* speed 9600 bps */
	1,	/* name, data circuit synch UDI*/
	1,	/* connection element non-transparent */
	0xB9,	/* buffer size tag */
	2,	/* buffer size length */
	0x02, 0x32	/* buffer size of 562 bytes */
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_OPEN_CHANNEL_IMM_CSD_PAD_ACCESS_ASYNCH_UDI)

SET_COMP ("l_cmd", 0x00F0)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_OPEN_CHANNEL_IMM_CSD_CONTENT_2)

- ENDSTRUCT

BEGINARRAY_PART (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_2, 28)
	0xD0, 	/* proactive SIM command tag	*/
	28,	/* following length */
	0x81,	/* command details tag */
	3,	/* command details length */
	1,	/* command number */
	0x40, 	/* command OPEN_CHANNEL	*/

0x01,	/* comm qual, Immediate, no auto reconnect	*/

0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x82,	/* destination ME	*/
	0x86,	/* address tag	*/
	7,	/* address length */
	0x91,	/* TON(1) and NPI(1) */
	0x10, 0x13, 0x17, 0x08, 0x35, 0xF7,
	0xB5,	/* bearer description tag */
	4,	/* bearer length */
	0x01,	/* bearer type CSD */
	12,	/* speed 9600 bps */
	2,	/* name, PAD access asynch UDI*/
	1,	/* connection element non-transparent */
	0xB9,	/* buffer size tag */
	2,	/* buffer size length */
	0x02, 0x32	/* buffer size of 562 bytes */
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_OPEN_CHANNEL_IMM_CSD_PACKET_ACCESS_SYNCH_UDI)

SET_COMP ("l_cmd", 0x00F0)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_OPEN_CHANNEL_IMM_CSD_CONTENT_3)

- ENDSTRUCT

BEGINARRAY_PART (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_3, 28)
	0xD0, 	/* proactive SIM command tag	*/
	28,	/* following length */
	0x81,	/* command details tag */
	3,	/* command details length */
	1,	/* command number */
	0x40, 	/* command OPEN_CHANNEL	*/

0x01,	/* comm qual, Immediate, no auto reconnect	*/

0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x82,	/* destination ME	*/
	0x86,	/* address tag	*/
	7,	/* address length */
	0x91,	/* TON(1) and NPI(1) */
	0x10, 0x13, 0x17, 0x08, 0x35, 0xF7,
	0xB5,	/* bearer description tag */
	4,	/* bearer length */
	0x01,	/* bearer type CSD */
	12,	/* speed 9600 bps */
	3,	/* name, packet access synch UDI */
	1,	/* connection element non-transparent */
	0xB9,	/* buffer size tag */
	2,	/* buffer size length */
	0x02, 0x32	/* buffer size of 562 bytes */
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_OPEN_CHANNEL_IMM_CSD_DATA_CIRCUIT_ASYNCH_RDI)

SET_COMP ("l_cmd", 0x00F0)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_OPEN_CHANNEL_IMM_CSD_CONTENT_4)

- ENDSTRUCT

BEGINARRAY_PART (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_4, 28)
	0xD0, 	/* proactive SIM command tag	*/
	28,	/* following length */
	0x81,	/* command details tag */
	3,	/* command details length */
	1,	/* command number */
	0x40, 	/* command OPEN_CHANNEL	*/

0x01,	/* comm qual, Immediate, no auto reconnect	*/

0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x82,	/* destination ME	*/
	0x86,	/* address tag	*/
	7,	/* address length */
	0x91,	/* TON(1) and NPI(1) */
	0x10, 0x13, 0x17, 0x08, 0x35, 0xF7,
	0xB5,	/* bearer description tag */
	4,	/* bearer length */
	0x01,	/* bearer type CSD */
	12,	/* speed 9600 bps */
	4,	/* name, data circuit asynch RDI*/
	1,	/* connection element non-transparent */
	0xB9,	/* buffer size tag */
	2,	/* buffer size length */
	0x02, 0x32	/* buffer size of 562 bytes */
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_OPEN_CHANNEL_IMM_CSD_DATA_CIRCUIT_SYNCH_RDI)

SET_COMP ("l_cmd", 0x00F0)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_OPEN_CHANNEL_IMM_CSD_CONTENT_5)

- ENDSTRUCT

BEGINARRAY_PART (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_5, 28)
	0xD0, 	/* proactive SIM command tag	*/
	28,	/* following length */
	0x81,	/* command details tag */
	3,	/* command details length */
	1,	/* command number */
	0x40, 	/* command OPEN_CHANNEL	*/

0x01,	/* comm qual, Immediate, no auto reconnect	*/

0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x82,	/* destination ME	*/
	0x86,	/* address tag	*/
	7,	/* address length */
	0x91,	/* TON(1) and NPI(1) */
	0x10, 0x13, 0x17, 0x08, 0x35, 0xF7,
	0xB5,	/* bearer description tag */
	4,	/* bearer length */
	0x01,	/* bearer type CSD */
	12,	/* speed 9600 bps */
	5,	/* name, data circuit synch RDI*/
	1,	/* connection element non-transparent */
	0xB9,	/* buffer size tag */
	2,	/* buffer size length */
	0x02, 0x32	/* buffer size of 562 bytes */
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_OPEN_CHANNEL_IMM_CSD_PAD_ACCESS_ASYNCH_RDI)

SET_COMP ("l_cmd", 0x00F0)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_OPEN_CHANNEL_IMM_CSD_CONTENT_6)

- ENDSTRUCT

BEGINARRAY_PART (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_6, 28)
	0xD0, 	/* proactive SIM command tag	*/
	28,	/* following length */
	0x81,	/* command details tag */
	3,	/* command details length */
	1,	/* command number */
	0x40, 	/* command OPEN_CHANNEL	*/

0x01,	/* comm qual, Immediate, no auto reconnect	*/

0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x82,	/* destination ME	*/
	0x86,	/* address tag	*/
	7,	/* address length */
	0x91,	/* TON(1) and NPI(1) */
	0x10, 0x13, 0x17, 0x08, 0x35, 0xF7,
	0xB5,	/* bearer description tag */
	4,	/* bearer length */
	0x01,	/* bearer type CSD */
	12,	/* speed 9600 bps */
	6,	/* name, PAD access asynch RDI*/
	1,	/* connection element non-transparent */
	0xB9,	/* buffer size tag */
	2,	/* buffer size length */
	0x02, 0x32	/* buffer size of 562 bytes */
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_OPEN_CHANNEL_IMM_CSD_PACKET_ACCESS_SYNCH_RDI)

SET_COMP ("l_cmd", 0x00F0)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_OPEN_CHANNEL_IMM_CSD_CONTENT_7)

- ENDSTRUCT

BEGINARRAY_PART (STK_OPEN_CHANNEL_IMM_CSD_CONTENT_7, 28)
	0xD0, 	/* proactive SIM command tag	*/
	28,	/* following length */
	0x81,	/* command details tag */
	3,	/* command details length */
	1,	/* command number */
	0x40, 	/* command OPEN_CHANNEL	*/

0x01,	/* comm qual, Immediate, no auto reconnect	*/

0x82,	/* device details tag	*/
	2,	/* device details length	*/
	0x81,	/* source SIM	*/
	0x82,	/* destination ME	*/
	0x86,	/* address tag	*/
	7,	/* address length */
	0x91,	/* TON(1) and NPI(1) */
	0x10, 0x13, 0x17, 0x08, 0x35, 0xF7,
	0xB5,	/* bearer description tag */
	4,	/* bearer length */
	0x01,	/* bearer type CSD */
	12,	/* speed 9600 bps */
	7,	/* name, packet access synch RDI*/
	1,	/* connection element non-transparent */
	0xB9,	/* buffer size tag */
	2,	/* buffer size length */
	0x02, 0x32	/* buffer size of 562 bytes */
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_OPEN_CHANNEL_IM_UDP)

SET_COMP ("l_cmd", 0x0140)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_OPEN_CHANNEL_IM_UDP_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_OPEN_CHANNEL_IM_UDP_CONTENT, 40)
	PROACTIVE_SIM_CMD_TAG,

38,	/* following length */
	CMD_DETAILS_TAG,
	3,	/* command details length */
	1,	/* command number */
	SAT_CMD_OPEN_CHANNEL,

(QLF_OPCH_IMMDT_LINK_EST | QLF_OPCH_NO_AUTO_RECONNECT),
	DEV_ID_TAG,
	2,	/* length */
	DEV_SRC_SIM,
	DEV_DST_ME,
	ADDR_TAG,
	5,	/* address length */
	145,	/* TON and NPI */
	0x31, 0x33,	/* dialling numbers */
	0x34, 0x35,	/* dialling numbers */
	BEAR_DESC_TAG,
	4,	/* length */
	BT_CSD,
	12,	/* speed */
	2,	/* name: PAD Access (async) (UDI) */
	0,	/* connection element: transparent */
	BUF_SIZE_TAG,
	2,	/* length */
	0x02, 0x32,	/* buffer size of 562 bytes */
	SIM_ME_TRANS_TAG,
	3,	/* length */
	UDP,
	DESTINATION_PORT_1,	/* port number 1800 */

DESTINATION_PORT_2,
	DEST_ADDR_TAG,
	5,	/* length */
	IPv4,
	DESTINATION_IP_1,	/* Address: 10.11.12.13 */
	DESTINATION_IP_2,
	DESTINATION_IP_3,
	DESTINATION_IP_4
ENDARRAY

BEGIN_PSTRUCT ("stk_cmd", STK_OPEN_CHANNEL_IM_L2R)

SET_COMP ("l_cmd", 0x00e0)

SET_COMP ("o_cmd", 0x0000)

SET_COMP ("cmd", STK_OPEN_CHANNEL_IM_L2R_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (STK_OPEN_CHANNEL_IM_L2R_CONTENT, 28)
	PROACTIVE_SIM_CMD_TAG,

26,	/* following length */
	CMD_DETAILS_TAG,
	3,	/* command details length */
	1,	/* command number */
	SAT_CMD_OPEN_CHANNEL,

(QLF_OPCH_IMMDT_LINK_EST | QLF_OPCH_NO_AUTO_RECONNECT),
	DEV_ID_TAG,
	2,	/* length */
	DEV_SRC_SIM,
	DEV_DST_ME,
	ADDR_TAG,
	5,	/* address length */
	145,	/* TON and NPI */
	0x31, 0x33,	/* dialling numbers */
	0x34, 0x35,	/* dialling numbers */
	BEAR_DESC_TAG,
	4,	/* length */
	BT_CSD,
	12,	/* speed */
	2,	/* name: PAD Access (async) (UDI) */
	1,	/* connection element: non-transparent */
	BUF_SIZE_TAG,
	2,	/* length */
	0x02, 0x32	/* buffer size of 562 bytes */
ENDARRAY

BEGIN_PSTRUCT (“chm“, CHAN_MODE_DATA_9_6)

SET_COMP (“ch_type”, CH_TCH_F)

SET_COMP (“ch_mode”, CHM_DATA_9_6)

ENDSTRUCT

/* capability: DIS (cap 0 only) */
BEGIN_PSTRUCT ("dis", CAP_DIS0)
	SET_COMP ("v8",	0)
	SET_COMP ("n_byte",	0)
	SET_COMP ("ready_tx_fax",	0)
	SET_COMP ("rec_fax_op",	1)
	SET_COMP ("data_sig_rate",	8)
	SET_COMP ("R8_lines_pels",	0)
	SET_COMP ("two_dim_coding",	0)
	SET_COMP ("rec_width",	0)
	SET_COMP ("max_rec_len",	0)
	SET_COMP ("min_scan_time",	7)
	SKIP_COMP ("uncomp_mode")
	SKIP_COMP ("err_corr_mode")
	SKIP_COMP ("frame_size")
	SKIP_COMP ("t6_coding")
	SKIP_COMP ("R8_lines")
	SKIP_COMP ("r_300_pels")
	SKIP_COMP ("R16_lines_pels")
	SKIP_COMP ("resolution_type")
	SKIP_COMP ("i_res_pref")
	SKIP_COMP ("m_res_pref")
	SKIP_COMP ("min_scan_time_hr")
	SKIP_COMP ("sel_polling")
	SKIP_COMP ("subaddr")
	SKIP_COMP ("password")
	SKIP_COMP ("ready_tx_doc")
	SKIP_COMP ("bft")
	SKIP_COMP ("dtm")
	SKIP_COMP ("edi")
	SKIP_COMP ("btm")
	SKIP_COMP ("ready_tx_mixed")
	SKIP_COMP ("char_mode")
	SKIP_COMP ("mixed_mode")
	SKIP_COMP ("proc_mode_26")
	SKIP_COMP ("dig_network_cap")
	SKIP_COMP ("duplex")
	SKIP_COMP ("jpeg")
	SKIP_COMP ("full_colour")
	SKIP_COMP ("huffman_tables")
	SKIP_COMP ("r_12_bits_pel_comp")
	SKIP_COMP ("no_subsamp")
	SKIP_COMP ("cust_illum")
	SKIP_COMP ("cust_gamut")
	SKIP_COMP ("na_letter")
	SKIP_COMP ("na_legal")
	SKIP_COMP ("sing_prog_seq_coding_basic")
	SKIP_COMP ("sing_prog_seq_coding_L0")
ENDSTRUCT

/* capability: DCS (cap 0 only) */
BEGIN_PSTRUCT ("dcs", CAP_DCS0)
	SKIP_COMP ("v8")
	SKIP_COMP ("n_byte")
	SKIP_COMP ("ready_tx_fax")
	SET_COMP ("rec_fax_op",	0)
	SET_COMP ("data_sig_rate",	1)
	SET_COMP ("R8_lines_pels",	1)
	SET_COMP ("two_dim_coding",	1)
	SET_COMP ("rec_width",	1)
	SET_COMP ("max_rec_len",	2)
	SET_COMP ("min_scan_time",	4)
	SKIP_COMP ("uncomp_mode")
	SKIP_COMP ("err_corr_mode")
	SKIP_COMP ("frame_size")
	SKIP_COMP ("t6_coding")
	SKIP_COMP ("R8_lines")
	SKIP_COMP ("r_300_pels")
	SKIP_COMP ("R16_lines_pels")
	SKIP_COMP ("resolution_type")
	SKIP_COMP ("i_res_pref")
	SKIP_COMP ("m_res_pref")
	SKIP_COMP ("min_scan_time_hr")
	SKIP_COMP ("sel_polling")
	SKIP_COMP ("subaddr")
	SKIP_COMP ("password")
	SKIP_COMP ("ready_tx_doc")
	SKIP_COMP ("bft")
	SKIP_COMP ("dtm")
	SKIP_COMP ("edi")
	SKIP_COMP ("btm")
	SKIP_COMP ("ready_tx_mixed")
	SKIP_COMP ("char_mode")
	SKIP_COMP ("mixed_mode")
	SKIP_COMP ("proc_mode_26")
	SKIP_COMP ("dig_network_cap")
	SKIP_COMP ("duplex")
	SKIP_COMP ("jpeg")
	SKIP_COMP ("full_colour")
	SKIP_COMP ("huffman_tables")
	SKIP_COMP ("r_12_bits_pel_comp")
	SKIP_COMP ("no_subsamp")
	SKIP_COMP ("cust_illum")
	SKIP_COMP ("cust_gamut")
	SKIP_COMP ("na_letter")
	SKIP_COMP ("na_legal")
	SKIP_COMP ("sing_prog_seq_coding_basic")
	SKIP_COMP ("sing_prog_seq_coding_L0")
ENDSTRUCT

/* capability info with DCS (cap 0 only) */
BEGIN_PSTRUCT ("hdlc_info", CAPS_DCS0)
	SET_COMP ("crp",	FALSE)
	SET_COMP ("c_pwd",	0)
	SKIP_COMP ("pwd")
	SET_COMP ("c_sub",	0)
	SKIP_COMP ("sub")
	SET_COMP ("c_sep",	0)
	SKIP_COMP ("sep")
	SET_COMP ("c_nsc",	0)
	SKIP_COMP ("nsc")
	SET_COMP ("c_nsf",	0)
	SKIP_COMP ("nsf")
	SET_COMP ("c_tsi",	0)
	SKIP_COMP ("tsi")
	SET_COMP ("c_cig",	0)
	SKIP_COMP ("cig")
	SET_COMP ("c_csi",	0)
	SKIP_COMP ("csi")
	SET_COMP ("c_nss",	0)
	SKIP_COMP ("nss")
	SET_COMP ("v_dis",	TRUE)
	SET_COMP ("dis", 	CAP_DIS0)
	SET_COMP ("v_dcs",	FALSE)
	SKIP_COMP ("dcs")
	SET_COMP ("v_dtc",	FALSE)
	SKIP_COMP ("dtc")
ENDSTRUCT

/* empty capability */
/*BEGIN_PSTRUCT ("hdlc_info", CAPS_DCS0)
	SET_COMP ("crp",	FALSE)
	SET_COMP ("c_pwd",	0)
	SKIP_COMP ("pwd")
	SET_COMP ("c_sub",	0)
	SKIP_COMP ("sub")
	SET_COMP ("c_sep",	0)
	SKIP_COMP ("sep")
	SET_COMP ("c_nsc",	0)
	SKIP_COMP ("nsc")
	SET_COMP ("c_nsf",	0)
	SKIP_COMP ("nsf")
	SET_COMP ("c_tsi",	0)
	SKIP_COMP ("tsi")
	SET_COMP ("c_cig",	0)
	SKIP_COMP ("cig")
	SET_COMP ("c_csi",	0)
	SKIP_COMP ("csi")
	SET_COMP ("c_nss",	0)
	SKIP_COMP ("nss")
	SET_COMP ("v_dis",	FALSE)
	SKIP_COMP ("dis")
	SET_COMP ("v_dcs",	FALSE)
	SKIP_COMP ("dcs")
	SET_COMP ("v_dtc",	FALSE)
	SKIP_COMP ("dtc")
ENDSTRUCT*/

BEGIN_PSTRUCT("smreg_qos", SMREG_QOS_0)

SET_COMP("delay", SMREG_DELAY_SUB)

SET_COMP("relclass", SMREG_RELCLASS_SUB)

SET_COMP("peak", SMREG_PEAK_SUB)

SET_COMP("preced", SMREG_PRECED_SUB)

SET_COMP("mean", SMREG_MEAN_SUB)

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_apn", SMREG_APN_NAME)

SET_COMP("buffer", APN)

- ENDSTRUCT

BEGINARRAY (APN,8)

0x07,	/* bell , default value vizzavi as held in MMI*/

0x56, 0x69, 0x7A, 0x7A, 0x61, 0x76, 0x69

ENDARRAY

- BEGINARRAY_PART (SMREG_CODING_OPTIONS,25)			/* defailt values as held in MMI */

/* length */

0xA0, 0,

/* offset */

0x0, 0x0,

/* pco data */

0x80, 0x80, 0x21, 0x10, 0x01, 0x01, 0x00, 0x10, 0x81, 0x06, 0x00, 0x00, 0x00,

0x00, 0x83, 0x06, 0x00, 0x00, 0x00, 0x00,0x00

- ENDARRAY

BEGIN_PSTRUCT("pdp_address", PDP_ADDRESS_1)

SET_COMP("buff", ADDRESS_BUF_1)

- ENDSTRUCT

- BEGINARRAY(ADDRESS_BUF_1,4)

0xD4, 0xB7, 0x89, 0x0C	/* default IP 212.183.137.012 values as held in MMI */

- ENDARRAY

# TEST CASES

## Routing (internal)

### MFW001:  Setup the Routing and the PCO view for the MMI test. The CC configuration and SIM activation signals are output.

Description: Routings for the MFW tests are set, and initial signals are output.

Preamble: None

APL		ACI		PS

|		|		|

|		|		|
COMMAND (MMI REDIRECT CC TAP)
COMMAND (MMI REDIRECT MM TAP)
COMMAND (MMI REDIRECT SIM TAP)
COMMAND (MMI REDIRECT SS TAP)
COMMAND (MMI REDIRECT SMS TAP)
COMMAND (MMI REDIRECT PL TAP)
COMMAND (MMI REDIRECT GMM TAP)

COMMAND (MMI REDIRECT GRR TAP)
COMMAND (MMI REDIRECT SM TAP)
COMMAND (MMI REDIRECT SND TAP)
COMMAND (MMI REDIRECT PPP TAP)
COMMAND (MMI REDIRECT UART TAP)
COMMAND (MMI REDIRECT RA TAP)
COMMAND (MMI REDIRECT RLP TAP)
COMMAND (MMI REDIRECT L2R TAP)
COMMAND (MMI REDIRECT T30 TAP)
COMMAND (PL REDIRECT MMI NULL)
	|		|		|
COMMAND (TAP REDIRECT TAP MMI)
COMMAND (MMI REDIRECT MMI TAP)
COMMAND (TAP RESET)

(1)	|		| 	MMI_KEYPAD_IND	|
	|		*	<==========================	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MMI_KEYPAD_IND
	key_code	23
	key_stat	KEY_STAT_PRS

History:

25-02-2002	ef		initial

09-10-2002	ef		Re-write the routings and reset parameters to make MMI more

stable in a runtime environment. Add config signals to CC and

the initial SIM signal.

21-02-2003	ef		add the power on signal and remove the MMI RESET

03-04-2003	ef		add RA and RLP signals for Sim Toolkit

02-06-2003	ef		add L2R and T30 for data/FAX call setup

## MMI Component Tests - case : Power On Sequences.

### MFW010: Power on Sequence, ME to Idle State, no PIN required

Description: After Power On of the mobile and the SIM_ACTIVATE_REQ sent in the start up phase of the mobile from MFW. This primitive initialises the SIM and leads to a SIM_ACTIVATE_CNF primitive to be returned. The SIM_MMI_INSERT_IND is used by the SIM process to inform MFW that a SIM card has been inserted and whether it is necessary to enter a PIN. MFW requests Emergency Call Codes (ECC) and the mandatory Administration data (AD) data from the SIM. MAN_ATTACH assumes an IMSI Attach for a Class BG mobile. The 5 secs delay is to allow the idle window to be output.

Preamble: 	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG MAN_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(8)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(9)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(10)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(11)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (5000)

(12)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(13)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NO_EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF
	datafield	SIM_CPHS_CINF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_IMSI
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_IMSI
	plmn	PLMN_262_14
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

25-02-2002	ef		initial

01-08-2002	ef		updated for CPHS read

09-08-2002	ef		change to SIM profile

19-12-2002	ef		add cause to activate cnf

27-08-2003	ef		add GMMREG_PLMN_MODE_REQ

### MFW011: Power On Sequence ME to Idle state, no PIN required. Combined IMSI/GPRS Attach.

Description: Set the Mobile Class to be Type A: Combined GPRS, and Set Auto Attach ON. Then Power On, should come up to the Idle state.

Preamble: 	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG CLASS_A)

- COMMAND (MMI CONFIG AUTO_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(9)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(10)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(11)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (5000)

(12)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(13)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NO_EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- SIM_READ_CNF	
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- SIM_READ_CNF
	datafield	SIM_CPHS_CINF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_A
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_262_14
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

25-02-2002	ef		initial

01-08-2002	ef		updated for CPHS read

09-08-2002	ef		change to SIM profile

19-12-2002	ef		add cause to activate cnf

### MFW012: Power on Sequence. ME to Idle state, no PIN required, Limited Service.

Description: Set the Mobile Class to be Type CG: GPRS Only, and Set Auto Attach ON. Then Power On, should come up to the Idle state. No Attach so Limited/No Service.

Preamble: 	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG CLASS_CG)

- COMMAND (MMI CONFIG AUTO_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(8)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(9)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(10)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(11)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NO_EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- SIM_READ_CNF
	datafield	SIM_CPHS_CINF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

History:

25-02-2002	ef		initial

01-08-2002	ef		updated for CPHS read

09-08-2002	ef		change to SIM profile

19-12-2002	ef		add cause to activate cnf

### MFW014: Power on Sequence. ME to Idle state, no PIN required. Automatic registration and Attachment follows.

Description: A Class BG mobile is automatically Attached. No PIN is required.

Preamble: 	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

- COMMAND (MMI CONFIG MAN_DETACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		| 	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(9)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(10)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(11)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (5000)

(12)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(13)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- SIM_READ_CNF
	datafield	SIM_CPHS_CINF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

30-01-2002	ef		initial

30-07-2002	ef		updated for CPHS read

19-12-2002	ef		add cause to activate cnf

### MFW015: Power On Sequence. ME to Idle State, No PIN required. Automatic registration and Attachment follows.

Description: Set up an IMSI(non-GPRS) Attach.

Preamble:  MFW001

APL		ACI		PS

|		|		|

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		| 	SIM_READ_REQ	|

|		*	<==========================	*

(7)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(8)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(9)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(10)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(11)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (5000)

(12)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(13)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	SIM_SERV_PHASE_2_ADN_BDN
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_IMSI
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- SIM_READ_CNF
	datafield	SIM_CPHS_CINF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_IMSI
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

30-01-2002	ef		initial

30-07-2000	ef		updated for CPHS read

19-12-2002	ef		add cause to activate cnf

### MFW016: Power on Sequence, ME to Idle State, No PIN required.

Description: No Config defaults

Preamble: 	MFW001

APL		ACI		PS

|		|		|

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	NOT_USED
	access_acmmax	NOT_USED
	access_puct	NOT_USED

History:

30-01-2002	ef		initial

19-12-2002	ef		add cause to activate cnf

### MFW018: Power On Sequence. Initialise the UART and flush any data.

Description: During MMI Power On, initialise the UART and disconnect ready for a new connect..

Preamble: 	MFW001

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG EXPAND_ATI_SRC_TST)

(1)	|		|	UART_PARAMETERS_REQ	|

|		*	==========================>	*

(2)	|		|	UART_PARAMETERS_CNF	|

|		 *<	==========================	*

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_PARAMETERS_REQ
	device	DEVICE_1
	comPar	COM_PARS

- UART_PARAMETERS_CNF
	device	DEVICE_1

- DTI2_DISCONNECT_REQ
	link_id	NUM_0
	cause	DTI_CAUSE_NORMAL_CLOSE

History:

31-01-2002	ef	initial.

19-12-2002	ef	all parameters changed.

27-08-2003	ef	signals changed

### MFW020: Set the Date

Description: After Power On of the mobile, The Main Menu is entered; Phone Settings; Clock, date/time:- date entering ; date/time: enter time successfull and back to idleScreen.

Note:  Sim Toolkit Menu does not appear. It can only be explicitly called by the Sim Toolkit tests defined later.

Preamble: 	MFW014

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Phone Settings */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Clock */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Time-Date */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=4)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=9)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (3000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to Idle Screen */
TIMEOUT_WAIT (1000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

History:

07-02-2002	ef	initial

10-04-2003	ef	update menu

### MFW022: For test of Xpanel.

Description:  For test of xpanel. Set Date and Time.

Preamble: 	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=UP)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=UP)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=UP)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=UP)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=UP)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=9)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (1000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

History:

08-02-2002	ef		initial

### MFW024: Power on Sequence, enter PIN required. ME to Idle State.

Description: After Power On of the mobile, enter the PIN number as requested.

Preamble: 	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(5)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(6)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

- TIMEOUT_WAIT (10000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

- TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

- TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

- TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

- TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(7)	|		| 	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

(9)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(10)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(11)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(12)	|		| 	SIM_READ_REQ	|

|		*	==========================>	*

(13)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(14)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(15)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(16)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(17)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

- TIMEOUT_WAIT (5000)

(18)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

- TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- SIM_READ_CNF
	datafield	SIM_CPHS_CINF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:

11-02-2002	ef		initial

30-07-2002	ef		updated for CPHS read

19-12-2002	ef		add cause to activate cnf

15-01-2003	ef		change the cause of SIM_ACTIVATE_CNF to

sim_cause_pin1_expect – is this correct?

Also add time delay for keyboard handler to be created

05-02-2003	ef		change order of GMM_PLMN_MODE_REQ

10-04-2003	ef		increase timeout waiting for PIN windows retry

### MFW026: Power on Sequence. No SIM card. Limited mode. Emergency Call.

Description: After Power On of the mobile, limited mode, enter the  emergency number after the “Insert SIM” menu is displayed.

Preamble:	MFW001

APL		ACI		PS

|		|		|

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(5)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(6)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*
TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_PRESS=LEFT)

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_PRESS=LEFT)

(7)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(9)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(10)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

(11)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_CAUSE_CARD_REMOVED
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NO_EC_CODES
	pref_lang	NO_PREF_LANG

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_IMSI
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_IMSI
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_EMERG_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_112
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_END_TO_END_PLMN
	connected_number	CONNECTED_NUMBER_112
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

History:

28-02-2002	ef		initial

30-07-2002	ef		updated

13-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal from all tests

19-12-2002	ef		add cause to SIM_ACTIVATE_CNF and SIM_SYNC_CNF

11-08-2003	ef		remove the POWER_ON key press. Add TCH signal from CC

### MFW028: Power on Sequence. No SIM card. No Service.

Description: After Power On of the mobile, invalid SIM, no service from network.

Preamble:	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(5)	|		| 	GMMREG_ATTACH_REJ	|

|		*	<==========================	*

TIMEOUT_WAIT (4000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_CAUSE_CARD_REMOVED
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_REJ
	detach_type	GMMREG_DT_LIMITED_SERVICE
	cause	MMCS_NETWORK_FAILURE
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:

11-02-2002	ef		initial

30-07-2002	ef		updated

19-12-02	ef		change to the cause type

19-12-2002	ef		add cause to activate cnf

04-02-2003	ef		omit 2nd GMMREG_PLMN_MODE_REQ

### MFW030: Power On Sequence, Invalid PIN, entering more than 3 digits , delete and emergency call

Description:.After Power On and enter invalid PIN, an Emergency Call is initiated, error digits are deleted. Call is set up.

Preamble:	MFW001

APL			ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(5)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(6)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Delete */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(7)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(9)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

(10)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_EMERG_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_112
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_END_TO_END_PLMN
	connected_number	CONNECTED_NUMBER_112
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

History:

11-02-2002	ef	initial

01-08-2002	ef	updated the GMM_ATTACH_CNF

13-08-2002	ef	remove the MMI_SPEECH_MODE_REQ signal

19-12-2002	ef	add cause to SIM_ACTIVATE_CNF.

15-01-2003	ef	changed cause of above, is SIM_CAUSE_PIN1_EXPECT correct?

15-01-2003	ef	add cause to SIM_SYNC_CNF

### MFW032: Power On Sequence, No SIM card, error in emergency call, delete and emergency call

Description:.After Power on, with no SIM card, attempt an incorrect emergency call, cancel and then enter correct emergency call.

Preamble:	MFW001

APL			ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(5)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(6)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*
TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Emergency Call? */
(7)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(9)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(10)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

(11)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_CAUSE_CARD_REMOVED
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NOT_USED
	pref_lang	NOT_USED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_EMERG_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_112
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_END_TO_END_PLMN
	connected_number	CONNECTED_NUMBER_112
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

History:

25-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF

13-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

19-12-2002	ef		add cause to SIM_ACTIVATE_CNF

15-01-2003	ef		add cause to SIM_SYNC_CNF

16-01-2003	ef		update keyboard entry to reflect the test case

14-08-2003	ef		added TCH indication

### MFW034: Power On Sequence, Blocked SIM card, limited mode, emergency call

Description:.The SIM reports that it is blocked and only limited mode is allowed. An Emergency Call (112) is initiated.

Preamble:	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(5)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(6)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*
TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(7)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(9)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

(10)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_CAUSE_PUK1_EXPECT
	pin_cnt	NUM_0
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_EMERG_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_112
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_END_TO_END_PLMN
	connected_number	CONNECTED_NUMBER_112
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

History:

25-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF

13-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

19-12-2002	ef		add cause to SIM_ACTIVATE_CNF

15-01-2003	ef		add cause to SIM_SYNC_CNF

15-01-2003	ef		change cause in SIM_ACTIVATE_CNF to

SIM_CAUSE_PUK1_EXPECT

### MFW036: Power On Sequence, Blocked SIM Card, enter PUK1 code.

Description:.The SIM is Blocked on Pwer Up. Enter the PUK1 code when requested. Then enter new PIN1 and verify PIN1.

Preamble:	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(5)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(6)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PUK1 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* PIN1 */

TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* PIN1 */

(7)	|		| 	SIM_UNBLOCK_REQ	|
	|		*	==========================>	*
(8)	|		| 	SIM_UNBLOCK_CNF	|
	|		*	<==========================	*
(9)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(10)	|		| 	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*
(11)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(12)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(13)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(14)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(15)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(16)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(17)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(18)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

|			|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_CAUSE_PUK1_EXPECT
	pin_cnt	NUM_0
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_1_VALUE
	pin	PIN_1_VALUE_1234
	pin_id	PUK_1

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PUK_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

11-02-2002	ef		initial

30-07-2002	ef		updated the GMM_ATTACH_CNF

19-12-2002	ef		add cause to SIM_ACTIVATE_CNF

16-12-2003	ef		change cause on above to SIM_CAUSE_PUK1_EXPECT

03-07-2003	ef		chang to primitive sequences + extra SIM_READ_REQ for CPHS Voice ind

### MFW038: Power On Sequence: PIN disabled, sim unlock(all checks), IdleScreen with clock.

Description:. All SIM Locks are enabled,. Unblock PIN for all SIM Locks requested.

Preamble:	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

COMMAND (MMI CONFIG SIMLOCK=1)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_ACTIVATE_CNF	|
	|		*	<==========================	*

(3)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		| 	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_READ_REQ	|

|		*	==========================>	*

(9)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(10)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(11)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(12)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(13)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(14)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(15)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(16)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(17)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NOT_USED

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV_4_15_16
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	NOT_USED
	access_acmmax	NOT_USED
	access_puct	NOT_USED

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_GID1
	length	NOT_PRESENT_8BIT
	max_length	NUM_5

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_GID1
	length	NOT_PRESENT_8BIT
	max_length	NUM_5

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_GID2
	length	NOT_PRESENT_8BIT
	max_length	NUM_5

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- SIM_READ_CNF
	datafield	SIM_GID1
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_GID2
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- SIM_READ_CNF
	datafield	SIM_CPHS_CINF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:

14-02-2002	ef		initial

30-07-2002	ef		added SIM_GID2 fields andGMM_ATTACH_CNF

19-12-2002	ef		add cause to SIM_ACTIVATE_CNF

### MFW040: Power On Sequence with PIN entering and sim unlock

Description:. Enter the PIN code as requested.

Preamble:	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

COMMAND (MMI CONFIG SIMLOCK=1)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(5)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(6)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(7)	|		| 	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

(9)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(10)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(11)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(12)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(13)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(14)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(15)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NOT_USED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF
	datafield	SIM_CPHS_CINF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

History:

14-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF and added CPHS read

19-12-2002	ef		add cause to SIM_ACTIVATE_CNF

16-01-2003	ef		change cause on above to CAUSE_PIN1_EXPECT

### MFW042: MO call setup, followed by removel of the SIM card.

Description:.After a MO call has connected, the SIM is removed. This causes the call to be disconnected and the mobile returns to the Idle state.

Preamble:	MFW500

APL		ACI		PS

|		|		|

(1)	|		|	SIM_REMOVE_IND	|

|		*	<==========================	*

(2)	|		|	MMI_CBCH_REQ	|
	|			*==========================>*
(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(4)	|		|	MMR_NREG_IND	|
	|			*<==========================*
(5)	|		|	GMMREG_DETACH_IND	|
	|			*<==========================*
	|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_REMOVE_IND
	cause	SIM_NO_ERROR

- MMI_CBCH_REQ
	msg_id	CBCH_MSG_ID_ARR
	dcs_id	CBCH_DCS_ID_ARR
	modus	MMI_CBCH_STOP

- ACI_CMD_IND
	cmd_len	LM_PERCENT_SIMREM_ERR_0
	cmd_seq	M_PERCENT_SIMREM_ERR_0

- MMR_NREG_IND
	service	NREG_LIMITED_SERVICE
	search_running	SEARCH_NOT_RUNNING
	new_forb_plmn	PLMN_262_02
	cause	MMCS_SIM_REMOVED

- GMMREG_DETACH_IND
	detach_type	GMMREG_DT_SIM_REMOVED
	cause	MMCS_SIM_REMOVED
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:

12-06-2002	ef		initial

19-12-2002	ef		new cause in MMR_NREG_IND & GMMREG_DETACH_IND

## MMI Component Tests - case : Pin Entering from Power On.

### MFW050: Power on the ME. PIN required. This will be the initial preamble for PIN entering tests.

Description:. Power ON and Auto Attach, PIN required.

Preamble: 	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(5)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(6)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

- TIMEOUT_WAIT (3500)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

18-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF

14-08-02	ef		POWER key apparently not needed anymore

09-10-02	ef		allow the welcome window time to output

19-12-2002	ef		add cause to SIM_ACTIVATE_CNF

16-01-2003	ef		change cause above to SIM_CAUSE_PIN1_EXPECT

### MFW052: PIN entering from Power On, enter 4 digits, successfull

Description:. Enter the PIN1 code.

Preamble: 	MFW050

APL		ACI		PS

|		|		|
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)
TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)
TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (3000)

(3)	|		| 	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*
(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(9)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(10)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(11)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (6000)

(12)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(13)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	SIM_SERV_PHASE_2_ADN_BDN
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_SPN
	length	NUM_17
	max_length	NUM_17

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

18-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF and added CPHS read.

20-12-2002	ef		add cause to SIM_VERIFY signals, and for subsequents.

16-01-2003	ef		add SIM_READ_REQ for service provider name.

06-02-2003	ef		change order of signals

### MFW054: PIN entering from Power On , clear with long press, 4 digits.

Description:. During PIN1 code entry, Clear one of the digits in error.

Preamble: 	MFW050

APL		ACI		PS

|		|		|
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_PRESS=RSOFT)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(1)	|		| 	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (3000)

(3)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

(9)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(10)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(11)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(12)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_B
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

18-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF

06-02-2003	ef		change order of signals

### MFW056: PIN entering from Power On and clearing several times. 8 digits.

Description:. During PIN1 entry, clear several error digits.

Preamble: 	MFW050

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2200)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2200)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (3000)

(3)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

(9)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(10)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(11)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(12)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_11111111
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

19-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF

06-02-2003	ef		added CPHS read and change order of SIM_READ_CNF’s

13-02-03	ef		remove KEY_PRESS as it deletes all digits

### MFW058: PIN entering from Power On and clear all entered digits. 8 digits.

Description:. During PIN1 entry, clear a complete line.

Preamble: 	MFW050

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2200)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2200)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2200)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2200)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (3000)

(3)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

(9)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(10)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(11)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(12)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_11111111
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

19-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF

06-02-2003	ef		change order of signal and use KEY_SEQUENCE

### MFW060: PIN entering from Power On, clear if nothing to clear and wrong keys.

Description:.Before a digit has been entered, Clear.

Preamble: 	MFW050

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (3000)

(3)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

(9)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(10)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(11)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_11111111
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:

19-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF

06-02-2003	ef		change order of signal

### MFW062: PIN entering from Power On, wrong digit during entering.

Description:.Wrong PIN1 is entered, re-enter correct PIN1.

Preamble: 	MFW050

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_PRESS=LEFT)

(3)	|		| 	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(4)	|		| 	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

(5)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(8)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(9)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(10)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(11)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(12)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(13)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(14)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(15)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_id	PIN_1
	pin_cnt	NUM_1
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF
	datafield	SIM_CPHS_CINF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

21-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF

16-01-2003	ef		change cause of SIM_VERIFY_PIN_CNF

to SIM_CAUSE_PIN1_EXPECT, is this ok?

### MFW064: PIN entering from Power On, END and MENU UP keys during entering.

Description:.During PIN entry, enter END and MENU UP keys. These are ignored.

Preamble: 	MFW050

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT(4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=END)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=UP)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		| 	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(4)	|		| 	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

(5)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(8)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(9)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(10)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(11)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(12)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(13)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(14)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(15)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_id	PIN_1
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF
	datafield	SIM_CPHS_CINF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

21-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF

16-01-2003	ef		change cause in SIM_VERIFY_PIN1_CNF

to SIM_CAUSE_PIN1_EXPECT

13-02-2003	ef		add the END and POWER keys to PIN1 input

10-04-2003	ef		power key causes goodbye. Use Menu up instead

### MFW066: PIN entering from Power On, enter digits. Successful, enter extraneous digit afterwards. Correctly ignored.

Description:.An extra digit is entered after the PIN1 has been accepted. This is ignored.

Preamble: 	MFW050

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*

(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (2500)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

(3)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

History:

22-02-2002	ef		initial

### MFW068: PIN entering from Power On, enter digits with delay while entering.

Description:.The PIN is requested and after 2 digits a 1 minute delay. Continue with last two digits.

Preamble: 	MFW050

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (60000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
(3)	|		| 	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

History:

22-02-2002	ef		initial

### MFW070: PIN entering from Power On, error in PIN, abort information screen, followed by correct PIN.

Description:. The PIN is requested, but is entered in error. The procedure is aborted by hitting the RHS, and the correct PIN is entered.

Preamble: 	MFW050

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* abort this menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		| 	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(4)	|		| 	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

(5)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(8)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(9)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(10)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(11)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(12)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(13)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(14)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(15)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_id	PIN_1
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF
	datafield	SIM_CPHS_CINF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

22-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF

16-01-2003	ef		change cause on SIM_VERIFY_CNF

### MFW072: PIN entering from Power On via GSM command string, unsuccessful gsm code change. Test Fails.

Description:. The PIN code is requested after Power On, and this is entered via a GSM command string code.

Preamble: 	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(5)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(6)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(7)	|		| 	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

(9)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	NOT_USED
	access_acmmax	NOT_USED
	access_puct	NOT_USED

History:

25-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF

16-01-2003	ef		change cause on SIM_ACTIVATE_CNF

### MFW074: PUK entering from Power On via GSM command string, unsuccessful GSM code change. Test Fails.

Description:. The PUK code is requested after Power On, and this is entered via a GSM command string.

Preamble: 	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(4)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(5)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(6)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(7)	|		| 	SIM_UNBLOCK_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_UNBLOCK_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)

(9)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_CAUSE_PUK1_EXPECT
	pin_cnt	NUM_0
	puk_cnt	NUM_9
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_1_VALUE
	pin	PIN_1_VALUE_1234
	pin_id	PUK_1

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PUK_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	NOT_USED
	access_acmmax	NOT_USED
	access_puct	NOT_USED

History:

28-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF

16-01-2003	ef		change cause on

## MMI Component Tests - case : PUK entering from Power On

### MFW100: PIN entering from Power On, repeated failures on verification, initial case for PUK

Description:.The PIN code is entered incorrectly 3 times. The PUK code must now be entered.

Preamble: 	MFW050

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 2nd attempt Enter PIN1 code */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=9)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=#)
(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 3rd attemp Enter PIN1 code */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=9)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=#)
(3)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_id	PIN_1
	pin_cnt	NUM_1
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PUK1_EXPECT
	pin_id	PIN_1
	pin_cnt	NUM_0
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

25-02-2002	ef		initial

16-01-2003	ef		change cause in SIM_VERIFY_PIN_CNF

### MFW102: PUK entering from Power On, enter the PUK.  Successful case.

Description: After Power on, and 3 unsuccessful PIN code entries, the PUK code is requested, together with the PIN code and its verification.

Preamble:	MFW100

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PUK1 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter New PIN1 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Confirm PIN1 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_UNBLOCK_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_UNBLOCK_CNF	|

|		*	<==========================	*
(3)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(4)	|		| 	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*
(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(7)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(9)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(10)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(11)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(12)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_1_VALUE
	pin	PIN_1_VALUE_1234
	pin_id	PUK_1

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PUK_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

25-02-2002	ef		initial

01-08-2002	ef		updated the GMM_ATTACH_CNF and added CPHS read

### MFW104: PUK entering from Power On, PIN verification, new PIN is different.

Description:. Enter the PUK code, followed PIN but error in verification. Repeat. Re-enter PIN twice OK.

Preamble: 	MFW100

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PUK1 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN1 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Confirm PIN1 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN1 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Confirm PIN1 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_UNBLOCK_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_UNBLOCK_CNF	|

|		*	<==========================	*

(3)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(7)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(9)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(10)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(11)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(12)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_1_VALUE
	pin	PIN_1_VALUE_1234
	pin_id	PUK_1

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PUK_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

25-02-2002	ef		initial

01-08-2002	ef		updated for CPHS read and updated the GMM_ATTACH_CNF

### MFW106: PUK entering from Power On, error in PUK, re-input the correct PUK.

Description:. The PUK1 code is requested after Power On, enter incorrect PUK1, followed by correct PUK1.

Preamble: 	MFW100

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 1st attempt err Enter PUK1 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN1 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Confirm PIN1 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_UNBLOCK_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_UNBLOCK_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PUK1 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN1 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Confirm PIN1 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		| 	SIM_UNBLOCK_REQ	|

|		*	==========================>	*

(4)	|		| 	SIM_UNBLOCK_CNF	|

|		*	<==========================	*

(5)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(9)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(10)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(11)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(12)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(13)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(14)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_1_VALUE
	pin	PIN_1_VALUE_1234
	pin_id	PUK_1

- SIM_UNBLOCK_CNF
	cause	SIM_CAUSE_PUK1_REMAIN9
	pin_id	PUK_1
	pin_cnt	NUM_3
	puk_cnt	NUM_9
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_1_VALUE
	pin	PIN_1_VALUE_1234
	pin_id	PUK_1

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PUK_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

25-02-2002	ef		initial

01-08-2002 	ef		updated for CPHS read and updated the GMM_ATTACH_CNF

### MFW108: PUK entering from Power On, wrong digit, clear, wrong digit, long pressed clear. Correct PUK.

Description:. The PUK1 code is requested after power on. During entry two digits are cleared by the RHS.

Preamble: 	MFW100

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PUK1 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Clear */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Clear */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Verify PIN */

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_UNBLOCK_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_UNBLOCK_CNF	|

|		*	<==========================	*

(3)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(7)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(9)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(10)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(11)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(12)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_1_VALUE
	pin	PIN_1_VALUE_1234
	pin_id	PUK_1

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PUK_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

25-02-2002	ef		initial

01-08-2001	ef		updated for CPHS read and updated the GMM_ATTACH_CNF

### MFW110: PUK entering from Power On, wrong digit, clear, wrong digit, End key. Correct PUK.

Description:.After Power On, the PUK1 code is requested. During entry all digits are cleared by long RHS press.

Preamble: 	MFW100

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PUK1 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_PRESS=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Verify PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_UNBLOCK_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_UNBLOCK_CNF	|

|		*	<==========================	*

(3)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(7)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(9)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(10)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(11)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(12)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_UNBLOCK_REQ
		source	SRC_MMI
		unblock_key	PUK_1_VALUE
		pin	PIN_1_VALUE_1234
		pin_id	PUK_1

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PUK_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

25-02-2002	ef		initial

01-08-2002	ef		updated for CPHS read and the GMMREG_ATTACH_CNF

13-02-2003	ef		long rhs deletes all digits on the PIN window.

### MFW112: PUK entering from Power On, Emergency call setup.

Description:.The PUK1 code is requested after Power On. An Emergency Call (112) is Setup.

Preamble: 	MFW100

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(4)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

(5)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_EMERG_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_112
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_END_TO_END_PLMN
	connected_number	CONNECTED_NUMBER_112
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

History:

25-02-2002	ef		initial

13-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

15-01-2003	ef		add cause to SIM_SYNC_CNF

14-08-2003	ef		add TCH indication

## MMI Component Tests - case : Main Menu, Security, Pin1 codes.

### MFW150: Main Menu, Security, Pin code: PIN changed, successful,

Description:.From Idle go to Security, PIN, Change PIN. Old PIN1 requested, followed by new PIN1 and then confirm new PIN1.

Preamble: 	MFW052

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Change */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter new PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(3)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PIN_1

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

04-03-2002	ef		initial

16-01-2003	ef		longer delay to allow PIN menu to be output.

27-08-2003	ef		add RSK key to bring back to idle

### MFW152: Main Menu, Security,PIN, Change, enter old Pin code, abort, back to Idle

Description:.From Idle screen, open Security, PIN, Change, and input old PIN, delete all and return back to Idle Screen by 4  RHS presses.

Preamble: 	MFW052

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Change */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)		/* enter old PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Back to Idle Screen */

TIMEOUT_WAIT (2000)

Parametrization:

Primitive	Parameter	Value

History:

12-03-2002	ef		initial

14-08-2003	ef		one extra RSK for Idle mode

### MFW154: Main Menu, Security, PIN, Change, enter Pin code, error in new pin, correct new pin.

Description:.From Idle open Security, PIN, Change, and input old PIN, confirm with error for new pin once, then correct PIN.

Preamble: 	MFW052

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Change */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter old PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 1st attempt Enter new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Confirm new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(3)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 2nd attempt Enter new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Confirm new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(5)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)
	|		|		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PIN_1

- SIM_CHANGE_PIN_CNF
	cause	SIM_CAUSE_PIN1_REMAIN2
	pin_id	PIN_1
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PIN_1

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- History:

12-03-2002	ef		initial

16-01-2003	ef		longer delay to allow PIN menu to be output

### MFW156: Main Menu, Security, PIN, Change, Pin code, wrong pin, wrong lhs. Change PIN.

Description:.From Idle screen, open Security,PIN, Change and input old PIN, wrong lhs, confirm with error each time. Pin change success.

Preamble: 	MFW052

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Change */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter old PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)		/* Confirm old PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter old PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)		/* Enter old PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter old PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Confirm new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(3)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (5000)
	|		|		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PIN_1

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- History:

13-03-2002	ef		initial

16-01-2003	ef		longer delay to allow PIN menu to init.

### MFW158: Main Menu, Security, PIN, Change, Pin code, new Pin again: abort clear

Description:.From Idle, open Security, PIN, Change, and input correct PIN, Clear (abort), back to main menu.

Preamble: 	MFW052

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Change */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter old PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=END)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=UP)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=UP)
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- History:

13-03-2002	ef		initial

16-01-2003	ef		longer delay for PIN menu

### MFW160: Main Menu, Security, PIN2, PIN 2 changed, with long delay while entering.

Description:.From Idle screen, open Security, PIN2, change PIN2 but introduce long delay while entering the confirm PIN2.

Preamble: 	MFW052

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN2 */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* ignore */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter old PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (10000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Confirm new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
TIMEOUT_WAIT (5000)

(4)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1234
	pin_id	PIN_2

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_2
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- History:

13-03-2002	ef		initial

### MFW162: Main Menu, Security, PIN enabled by default, Pin, Pin disabled.

Description:. PIN enabled. From the Menu, Check we can disable the PIN.

Preamble: 	MFW050

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (2000)

(3)	|		| 	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*
(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(8)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(9)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(10)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (5000)

(11)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(12)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Deactivate */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN 1 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(13)	|		| 	SIM_DISABLE_PIN_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (6000)

(14)	|		|	SIM_DISABLE_PIN_CNF	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	SIM_SERV_PHASE_2_ADN_BDN
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- SIM_DISABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_DISABLE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- History:

14-03-2002	ef		initial

01-08-2002	ef		updated for CPHS read and GMMREG_ATTACH_CNF

### MFW164: Main Menu, Change PIN by GSM command string **04*.

Description:.From Idle screen, open Security, and Change PIN via a GSM command string.

Preamble: 	MFW052

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Change */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

(1)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PIN_1

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

15-03-2002	ef		initial

### MFW166: Main Menu, Security, PIN, PIN changed via GSM code, 1st attempt fail, 2nd success.

Description:.From Idle screen, open Security, and Change PIN via a GSM command string, first attempt unsuccessful, second attempt successful.

Preamble: 	MFW052

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Change */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

(1)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter old PIN */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Confirm PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(5)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
TIMEOUT_WAIT (4000)

(6)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PIN_1

- SIM_CHANGE_PIN_CNF
	cause	SIM_CAUSE_PIN1_REMAIN2
	pin_id	PIN_1
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PIN_1

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

15-03-2002	ef		initial

### MFW168: Main Menu, Security, Pin2, PIN2 changed via GSM code, sucessfull (Pin enabled)

Description:.PIN2 defaults to enabled. Main Menu window and open Security, and Change PIN2 via a GSM command string, first attempt successful.

Preamble: 	MFW052

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN2 */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

(1)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Back to Idle Screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PIN_2

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_2
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

15-03-2002	ef		initial

16-01-2003	ef		longer delay to allow PIN menu display

### MFW170: Main Menu, Security, PIN, Change PIN by GSM command string, twice in a row.

Description:.From Idle screen, open Security, PIN, Change, enter  PIN using GSM command string twice.

Preamble: 	MFW052

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Change */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

(1)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

(3)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PIN_1

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PIN_1

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

15-03-2002	ef		initial

27-08-2003	ef		add RSK to bring back to idle

### MFW172: Main Menu, Change PIN by GSM command string, error in code. Re enter code.

Description:.From the idle screen, open Security, and Change PIN via a GSM command string. Error in code, re-enter old pin as directed.

Preamble: 	MFW052

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Change */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Confirm new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PIN_1

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

18-03-2002	ef		initial

16-01-2003	ef		longer delay for PIN menu

### MFW174: Main Menu, Change PIN, Error 3 times in entry, so the PUK is entered by gsm code.

Description:.From the Idle screen, open Security, PIN, Change, and enter PIN,. Error in verification 3 times, so enter PUK code via GSM command string.

Note: 1st PIN failure is in the preamble.

Preamble: 	MFW052

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Change */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 2nd attempt enter old PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 3rd attempt enter old PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

(5)	|		| 	SIM_UNBLOCK_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_UNBLOCK_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (4000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_id	PIN_1
	pin_cnt	NUM_1
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PUK1_EXPECT
	pin_id	PIN_1
	pin_cnt	NUM_0
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_1_VALUE
	pin	PIN_1_VALUE_1234
	pin_id	PUK_1

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PUK_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

19-03-2002	ef		initial

16-01-2003	ef		longer delay for PIN menu

16-01-2003	ef		change cause in SIM_VERIFY_PIN_CNF

### MFW176: Main Menu, Change PIN, 3 times error in input, error in PUK GSM code, correct PUK.

Description:.From the Idle screen, open Security, PIN, Change, and enter PIN,. Error in verification 3 times, so enter PUK code via a GSM command string. The first attempt is in error, next is correct.

Preamble: 	MFW052

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Change */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 2nd attempt enter old PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 3rd attempt enter old PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

(5)	|		| 	SIM_UNBLOCK_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_UNBLOCK_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

(7)	|		| 	SIM_UNBLOCK_REQ	|
	|		*	==========================>	*
(8)	|		| 	SIM_UNBLOCK_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (4000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_id	PIN_1
	pin_cnt	NUM_1
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PUK1_EXPECT
	pin_id	PIN_1
	pin_cnt	NUM_0
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_1_VALUE
	pin	PIN_1_VALUE_1234
	pin_id	PUK_1

- SIM_UNBLOCK_CNF
	cause	SIM_CAUSE_PUK1_REMAIN9
	pin_id	PUK_1
	pin_cnt	NUM_3
	puk_cnt	NUM_9
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_1_VALUE
	pin	PIN_1_VALUE_1234
	pin_id	PUK_1

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PUK_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

19-03-2002	ef		initial

16-01-2003	ef		longer delay for PIN menu

16-01-2003	ef		change cause in SIM_VERIFY_PIN_CNF

### MFW178: PIN Deactivated. Main Menu, Security, PIN, Activate, Deactivate, Activate in sequence.

Description:.PIN Deactivated in Security menu, back to Main Menu, Security, PIN, Activate, Deactivate, Activate. Then back to the Main Menu.

Preamble: 	MFW015

TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Activate */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		|	SIM_ENABLE_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_ENABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Deactivate */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		|	SIM_DISABLE_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_DISABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=UP)		/* Activate */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(5)	|		|	SIM_ENABLE_PIN_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_ENABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ENABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_ENABLE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_DISABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_DISABLE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ENABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_ENABLE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

19-03-2002	ef		initial

### MFW180: PIN enabled, enter emergency call and successfull pin entering. Test fails.

Description:.After Power On and before the PIN is input, enter Emergency call number, after Setup, finish call and enter the correct PIN.

Preamble: 	MFW050

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

(4)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* end call */

(5)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(6)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(7)	|		| 	SIM_VERIFY_PIN_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_VERIFY_PIN_CNF	|

|		*	<==========================	*

(9)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(10)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(11)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(12)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(13)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (5000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_EMERG_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_112
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_END_TO_END_PLMN
	connected_number	CONNECTED_NUMBER_112
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_1

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_ENABLED

History:

20-03-2002	ef		initial

16-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		add to payload of MNCC_DISCONNECT_REQ

20-12-2002	ef		update the cause value for MNCC_DISCONNECT_REQ

15-01-2003	ef		add cause to SIM_SYNC_CNF

### MFW182: PIN disabled. Main menu, Security, Activate, PIN. unsuccessful, re-try with correct PIN.

Description:.PIN defaults to disabled. From the Idle screen, Security, PIN, Activate, enter incorrect PIN, followed by the correct PIN.

Preamble: 	MFW015

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Activate */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		|	SIM_ENABLE_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_ENABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		|	SIM_ENABLE_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_ENABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (3700)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ENABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_ENABLE_PIN_CNF
	cause	SIM_CAUSE_PIN1_REMAIN2
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ENABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_ENABLE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

20-03-2002	ef		initial

### MFW184: PIN disabled. Power On Sequence Main menu, Security, PIN, Activate. Not successful, Delete.

Description:.PIN defaults to disabled. From the Idle screen, Security, Pin, enter incorrect PIN in the Enable PIN Menu, followed by the Delete key to go back to the PIN Menu.

Preamble: 	MFW015

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Activate */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		|	SIM_ENABLE_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_ENABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ENABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_ENABLE_PIN_CNF
	cause	SIM_CAUSE_PIN1_REMAIN2
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

20-03-2002	ef		initial

### MFW186: PIN enabled. Activate PIN. Advisary message is displayed.

Description:.PIN is enabled from previous test and in the PIN Menu, LHS pressed to activate PIN. Message PIN Activated is displayed. Back to Idle screen

Preamble: 	MFW182

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

21-03-2002	ef		initial

### MFW188: Power On Sequence Main Menu, Security, Pin, Activate, unsuccessful PIN enabled 3 times. PUK code.

Description:.Default PIN disabled. Main menu, Security, PIN, Activate. Enter PIN 3 times incorrectly. Enter PUK code and correct PIN.

Preamble: 	MFW015

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Activate */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 1st attempt Enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		|	SIM_ENABLE_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_ENABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 2nd attempt enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		|	SIM_ENABLE_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_ENABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 3rd attempt enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(5)	|		|	SIM_ENABLE_PIN_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_ENABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (8000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PUK */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter new PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Confirm PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(7)	|		| 	SIM_UNBLOCK_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_UNBLOCK_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)
	|		|		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ENABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_ENABLE_PIN_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ENABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_ENABLE_PIN_CNF
	cause	SIM_CAUSE_PIN1_EXPECT
	pin_cnt	NUM_1
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ENABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_ENABLE_PIN_CNF
	cause	SIM_CAUSE_PUK1_EXPECT
	pin_cnt	NUM_0
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_1_VALUE
	pin	PIN_1_VALUE_1234
	pin_id	PUK_1

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PUK_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

21-03-2002	ef		initial

18-01-2003	ef

### MFW190: Default PIN disabled. Power On Sequence Main Menu, Security, Pin, Activate failed GSM command string, then manually enter.

Description:.PIN disabled by default. Main menu, Security, PIN, Activate, enter PIN change using GSM command string which is not successful., followed by correct PIN entered via the keypad..

Preamble: 	MFW015

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Activate */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		|	SIM_ENABLE_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_ENABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ENABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_ENABLE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

21-03-2002	ef		initial

### MFW192: . Power On Sequence Main Menu, Security, PIN, Deactivate, enter PIN, wrong PIN then correct PIN.

Description:.PIN enabled by default. Main menu, Security, PIN, Deactivate, enter incorrect PIN1, followed by correct PIN1.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Deactivate */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 1st attempt Enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		|	SIM_DISABLE_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_DISABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 2nd attempt enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		|	SIM_DISABLE_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_DISABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (3200)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_DISABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_DISABLE_PIN_CNF
	cause	SIM_CAUSE_PIN1_REMAIN2
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_DISABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_DISABLE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

21-03-2002	ef		initial

### MFW194: PIN enabled by default. Power On Sequence, Main menu, Security, Pin, Deactivate, failure and back to IdleScreen.

Description: From the idle screen, Main menu, Security, failure to Deactivate and back to idle screen.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Deactivate */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		|	SIM_DISABLE_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_DISABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_DISABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_DISABLE_PIN_CNF
	cause	SIM_CAUSE_PIN1_REMAIN2
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

22-03-2002	ef		initial

14-08-2003	ef		Add RSK to go back to idle

### MFW196: PIN enabled by default. Power On Sequence, Main Menu, Security, Pin, Deactivate, enter by GSM code(error), then correctly.

Description: From the Idle screen, Main menu, Security, failure to Deactivate via GSM command string, followed by correct entry and some LHS presses. Back to idle screen.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Deactivate */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(1)	|		|	SIM_DISABLE_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_DISABLE_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=*)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Back to idle screen */
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_DISABLE_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234

- SIM_DISABLE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

22-03-2002	ef		initial

### MFW198: PIN disabled by default. Power On Sequence Main Menu, Security, Pin, Deactivate, no pin needed for disable.

Description: From the Main menu, Security, PIN, Deactivate.However PIN is already Deactivated so a message is displayed.

Preamble: 	MFW050

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Deactivate /
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
	|		|		|

Parametrization:

Primitive	Parameter	Value

History:

22-03-2002	ef		initial

## MMI Component Tests - case : Main Menu, Security, Pin2 codes.

### MFW250: Main Menu, Security, Pin2, enter, with time out while entering, successful

Description:.From Idle screen, open Security, PIN2, enter old PIN2, start to enter  new PIN2 and Timeout. Re-enter new PIN2. and confirm PIN2.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN2 */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter new PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (40000)			/* timeout */

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* re-enter */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(3)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PHASE_2_PIN_2

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

22-03-2002	ef		initial

### MFW252: Main Menu, Security, Pin2, confirm PIN 2 entry in error, then successful

Description:.From the Idle screen, open Security, and Change PIN2, incorrect confirmation of PIN2, followed by successful confirmation of new PIN2.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN2 */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter new PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* error in PIN2 */
(3)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter new PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */
(5)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PHASE_2_PIN_2

- SIM_CHANGE_PIN_CNF
	cause	SIM_CAUSE_PIN2_REMAIN2
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PHASE_2_PIN_2

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

25-03-2002	ef		initial

17-01-2003	ef		increase time delay at start to allow menu display

### MFW254: Main Menu, Security, Pin2, enter old PIN 2, then clear and abort back.

Description:.From the Idle screen, open Security, PIN2, enter correct old PIN2, then clear all four digits and RHS key to abort back.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN2 */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

25-03-2002	ef		initial

16-01-2003	ef		increase time delay at start

16-08-2003	ef		Add RSK to bring back to idle screen

### MFW256: Main Menu, Security, PIN2, PIN2 changed, wrong new pin and wrong key left.

Description:.From the idle screen, open Security, and Change PIN2, enter correct old PIN2 but incorrect new PIN2 confirm, repeat with correct new PIN2 confirm.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN2 */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */
(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter new PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* ignored */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */
TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)		/* error in confirmation */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter new PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */
(3)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (5000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PHASE_2_PIN_2

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

25-03-2002	ef		initial

### MFW258: Main Menu, Security, PIN2, PIN2 changed, correct new pin and abort.

Description:.From the Idle screen, open Security, and Change PIN2, enter correct old PIN2 but abort the confirm new PIN2 back to PIN2 Menu.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN2 */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter new PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* confirm PIN2 */

TIMEOUT_WAIT (6000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

25-03-2002	ef		initial

### MFW260: Main Menu, Security, PIN2, error in old PIN2, retry, new PIN2, confirm PIN2.

Description:.From the idle screen, open Security, and Change PIN2, enter incorrect old PIN2, then correct old PIN2, followed by new PIN2 and confirm PIN2.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN2 */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* error in enter old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(3)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter new PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */
(5)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_2
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PHASE_2_PIN_2

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

25-03-2002	ef		initial

17-01-2003	ef		change cause of SIM_VERIFY_CNF to SIM_PIN2_EXPECT

from SIM_PIN2_REMAIN2. Is this correct?

### MFW262: Main Menu, Security, PIN2, twice error in old PIN2, new PIN2, confirm PIN2.

Description:.From the Idle screen, open Security, and Change PIN2, enter twice incorrect old PIN2, then correct old PIN2, followed by new PIN2 and confirm PIN2.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN2 */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* error in old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* error in old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(3)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(5)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* new PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */
(7)	|		|	SIM_CHANGE_PIN_REQ	|
	|		*	==========================>	*
(8)	|		| 	SIM_CHANGE_PIN_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_2
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_1
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	PIN_1_VALUE_1234
	new_pin	PIN_1_VALUE_1233
	pin_id	PHASE_2_PIN_2

- SIM_CHANGE_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_2
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

25-03-2002	ef		initial

### MFW264: Main Menu, Security, PIN2, repeated errors in old PIN2, PUK2 code entered.

Description:.From the Idle screen, open Security, and Change PIN2, enter incorrect old PIN2 three times, then enter PUK2 code, followed by new PIN2 and confirm PIN2.

A:	 no error returned in unblock

B: 	PUK error returned in unblock

Variants:		<A>...<B>

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN2 */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 1st error in old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (10000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/*  2nd error in old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(3)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (10000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 3rd error in old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(5)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (10000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* PIN blocked, enter PUK2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

TIMEOUT_WAIT (10000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter new PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

TIMEOUT_WAIT (10000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(7)	|		| 	SIM_UNBLOCK_REQ	|
	|		*	==========================>	*
(8)	|		| 	SIM_UNBLOCK_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_2
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_1
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PUK2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_0
	puk2_cnt	NUM_10

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_2_VALUE_11223344
	pin	PIN_1_VALUE_1233
	pin_id	PHASE_2_PUK_2

- SIM_UNBLOCK_CNF
<A>	cause	SIM_NO_ERROR
<B>	cause	SIM_CAUSE_PUK2_REMAIN9
	pin_id	PHASE_2_PUK_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
<A>	puk2_cnt	NUM_10
<B>	puk2_cnt	NUM_9

History:

26-03-2002	ef		initial

17-01-2003	ef		change cause of SIM_VERIFY_PIN_CNF to

SIM_CAUSE_PIN2_EXPECT and SIM_CAUSE_PUK2_EXPECT

### MFW266: Main Menu, Security, PIN2, repeated errors in old PIN2, error in PUK2, re-enter.PUK2.

Description:.Correctly enter PUK2, followed by new PIN2 and confirm PIN2.

Preamble: 	MFW264B

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* re-enter the PUK2 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter new PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	SIM_UNBLOCK_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_UNBLOCK_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to Idle screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_2_VALUE_11223344
	pin	PIN_1_VALUE_1233
	pin_id	PHASE_2_PUK_2

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PUK_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

27-03-2002	ef		initial

### MFW268: Main Menu, Security, PIN2, repeated errors in old PIN2, PUK2 code entered via GSM string.

Description:.From the Idle screen, open Security, and Change PIN2, enter incorrect old PIN2 three times, then enter PUK2 code via a GSM command string containing the new PIN2..

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* PIN2 */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 1st error in old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 2nd error in old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(3)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 3rd error in old PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(5)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

(7)	|		| 	SIM_UNBLOCK_REQ	|
	|		*	==========================>	*
TIMEOUT_WAIT (3000)

(8)	|		| 	SIM_UNBLOCK_CNF	|
	|		*	<==========================	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_2
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_1
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PUK2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_0
	puk2_cnt	NUM_10

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_2_VALUE_11223344
	pin	PIN_1_VALUE_1233
	pin_id	PHASE_2_PUK_2

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PUK_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

History:

18-06-2002	ef		initial

17-01-2003	ef		change cause in SIM_VERIFY_PIN_CNF to

SIM_CAUSE_PIN2_EXPECT and SIM_CAUSE_PUK2_EXPECT

## MMI Component Tests - case :  Main Menu, Phone Settings, Date and Time.

### MFW300: Main Menu, Phone Settings, Clock, Time/Date, enter date and time successfully and back to idle screen.

Description:.From Settings, Clock, Time/Date, enter the date and time successfully.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Main Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Phone Settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Clock */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Time-Date */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

28-03-2002	ef		initial

23-01-2003	ef		menu has changed

### MFW302: Main Menu, Phone Settings, Clock, Time/Date, enter incorrect date and time, then correct.

Description:.From Settings, Clock, Time/Date, enter error date then correct, error time then correct.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Main Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Phone Settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Clock */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* date-time */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

28-03-2002	ef		initial

23-01-2003	ef		menu has changed

### MFW304: Main Menu, Phone Settings, Clock, Time/Date, enter date and time clearing both times.

Description:.From Phone Settings, Clock, Time/Date, enter date and abort, back to enter date, enter date. Enter time and abort, back to enter date and time. Enter correct date and time.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Main Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Phone Settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Clock */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* date-time */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

28-03-2002	ef		initial

23-01-2003	ef		menu has changed

### MFW306: Main Menu, Phone Settings, Clock, Time/Date, enter date, long press clear; without time entering.

Description:.From Phone Settings, Clock, Time/Date, enter date and abort, back to enter date, enter date. Long press Clear without the Time entered.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Main Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Phone Settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Clock */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* date-time */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_PRESS=CLEAR)

TIMEOUT_WAIT (2200)

COMMAND (MMI CONFIG KEY_RELEASE=CLEAR)

TIMEOUT_WAIT (2200)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (3000)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

02-04-2002	ef		initial

23-01-2003	ef		menu has changed

## MMI Component Tests - case : Power Down Sequence

### MFW350: Power Down Sequence from the IdleScreen (No PIN required), power down key.

Description:.At the Idle screen, press the Power down key. Note that the gmmreg cnf is not required.

Preamble: 	MFW010

APL		ACI		PS

|		|		|
	TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_PRESS=POWER)

(1)	|		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*
(2)	|		|	GMMREG_DETACH_REQ	|
	|			*==========================>*
(3)	|		| 	SIM_SYNC_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (2000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_SYNC_REQ
	synccs	SYNC_DEACTIVATE

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_POWER_OFF

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

History:

02-04-2002	ef		initial

06-08-2002	ef		updated to KEY F2

19-08-2002	ef		remove the F2 Key and use POWER.

15-01-2003	ef		add cause to SIM_SYNC_CNF

### MFW352: Power up Sequence from idle when entering PIN, Power down key.

Description:.At the Idle screen, enter part of the PIN, followed by the Power off key.

Preamble: 	MFW050

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_PRESS=POWER)

(1)	|		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_SYNC_CNF	|
	|		*	<==========================	*
(3)	|		|	GMMREG_DETACH_REQ	|
	|			*==========================>*
TIMEOUT_WAIT (10000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_SYNC_REQ
	synccs	SYNC_DEACTIVATE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_POWER_OFF

History:

03-04-2002	ef		initial

06-08-2002	ef		updated to KEY F2

19-08-2002	ef		remove the F2 Key and use POWER

15-01-2003	ef		add cause to SIM_SYNC_CNF.

### MFW354: Power up Sequence from idle when entering an emergency number, Power down key.

Description:.At the Idle screen, enter emergency number, followed by the Power off key.

Preamble: 	MFW050

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2200)

COMMAND (MMI CONFIG KEY_PRESS=POWER)

(1)	|		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_SYNC_CNF	|
	|		*	<==========================	*
(3)	|		|	GMMREG_DETACH_REQ	|
	|			*==========================>*
TIMEOUT_WAIT (10000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_SYNC_REQ
	synccs	SYNC_DEACTIVATE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_POWER_OFF

History:

03-04-2002	ef		initial

06-08-2002	ef		updated to KEY F2

19-08-2002	ef		remove the F2 Key and use POWER.

15-01-2003	ef		add cause to SIM_SYNC_CNF

### MFW356: Power Down Sequence from the IdleScreen, Power up, Power down, Power up. Test Fails.

Description:.At the Idle screen, press the Power down key. Then successive Power UP, Power Down, Power Up, Power Down.

Preamble: 	MFW010

APL		ACI		PS

|		|		|
COMMAND (MMI CONFIG KEY_PRESS=POWER)

(1)	|		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_SYNC_CNF	|
	|		*	<==========================	*
(3)	|		|	GMMREG_DETACH_REQ	|
	|			*==========================>*
	TIMEOUT_WAIT (6000)
(4)	|		| 	MMI_KEYPAD_IND	|
	|		*	<==========================	*
COMMAND (MMI CONFIG KEY_PRESS=POWER)

(5)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(6)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(7)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(8)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(9)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(10)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(11)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(12)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(13)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(14)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(15)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_SYNC_REQ
	synccs	SYNC_DEACTIVATE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_POWER_OFF

- MMI_KEYPAD_IND
	key_code	23
	key_stat	KEY_STAT_PRS

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NO_EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

History:

03-04-2002	ef		initial

06-08-2002	ef		updated to KEY F2

19-08-2002	ef		remove the F2 Key and use POWER.

### MFW358: Power Down Sequence from the WelcomeScreen, power down key.

Description:.At the PIN screen, press the Power down key.

Preamble: 	MFW001

APL		ACI		PS

|		|		|
(1)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(2)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

COMMAND (MMI CONFIG KEY_PRESS=POWER)

TIMEOUT_WAIT (2000)
(3)	|		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_SYNC_CNF	|
	|		*	<==========================	*
(5)	|		|	GMMREG_DETACH_REQ	|
	|			*==========================>*
TIMEOUT_WAIT (2000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_BDN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_SYNC_REQ
	synccs	SYNC_DEACTIVATE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_POWER_OFF

History:

04-04-2002	ef		initial

06-08-2002	ef		updated to KEY F2

19-08-2002	ef		remove the F2 Key and use POWER

15-01-2003	ef		add cause to SIM_SYNC_CNF.

### MFW360: Power Down sequence. Remove SIM, Power Down.

Description:.At the Idle screen, no PIN required. Remove SIM card, not possible to make an emergency call.Power down.

Preamble: 	MFW014

APL/ACI		MMI		PS

|		|		|
(1)	|		| 	SIM_REMOVE_IND	|
	|		*	<==========================	*

(2)	|		|	MMI_CBCH_REQ	|
	|			*==========================>*
(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

COMMAND (MMI CONFIG KEY_PRESS=POWER)

(4)	|		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*
(5)	|		| 	SIM_SYNC_CNF	|
	|		*	<==========================	*
(6)	|		|	GMMREG_DETACH_REQ	|
	|			*==========================>*
TIMEOUT_WAIT (2000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_REMOVE_IND
	cause	SIM_NO_ERROR

- MMI_CBCH_REQ
	msg_id	CBCH_MSG_ID_ARR
	dcs_id	CBCH_DCS_ID_ARR
	modus	MMI_CBCH_STOP

- ACI_CMD_IND
	cmd_len	LM_PERCENT_SIMREM_ERR_0
	cmd_seq	M_PERCENT_SIMREM_ERR_0

- SIM_SYNC_REQ
	synccs	SYNC_DEACTIVATE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_POWER_OFF

History:

04-04-2002	ef		initial

06-08-2002	ef		updated to KEY F2

19-08-2002	ef		remove the F2 Key and use POWER

15-01-2003	ef		add cause to SIM_SYNC_CNF

07-07-2003	ef		add cause to SIM_REMOVE_IND

## MMI Component Tests - case : Main Menu, Security, Fixed Names

### MFW380: Main Menu, Security, Fixed Names, Activate with time out while entering, successful

Description:.From the Idle screen, open Security, Fixed Names, Yes, Timeout and re-enter PIN2.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Main Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Fixed Names */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Yes */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (40000)			/* timeout */

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
(3)	|		| 	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(4)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to Idle screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ACTIVATE_REQ
	proc	SIM_FDN_ENABLE
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

History:

04-04-2002	ef		initial

### MFW382: Main Menu, Security, Activate Fixed Names, during entry LHS and CLEAR

Description:.From the Idle screen, open Security, Fixed Names, Yes, enter PIN2 and LHS twice followed by CLEAR.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Fixed Names */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Yes */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Delete */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Delete */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
(3)	|		| 	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(4)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ACTIVATE_REQ
	proc	SIM_FDN_ENABLE
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

History:

04-04-2002	ef		initial

### MFW384: Main Menu, Security, Activate Fixed Names, 2 wrong PIN2 entries, followed by correct PIN2.

Description:.From the Idle screen, Menu, open Security, Fixed Names, Yes, enter PIN2 incorrectly two times followed by the correct PIN2.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Fixed Names */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Yes */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 1st error in PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 2nd error in PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(5)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
(7)	|		| 	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(8)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_2
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN2_REMAIN1
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_1
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ACTIVATE_REQ
	proc	SIM_FDN_ENABLE
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

History:

05-04-2002	ef		initial

27-01-2003	ef		change cause on PIN_VERIFY_CNF

### MFW386: Main Menu, Security, Activate Fixed Names, 3 wrong PIN2 entries, followed by PUK2.

Description:.From the Idle screen, Menu, open Security, Fixed Names, Yes, enter PIN2 incorrectly three times followed by the required PUK2 code.

A: 	No error in PUK2 code

B: 	Error in PUK2 code requiring re-input.

Variants:		<A>...<B>

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Fixed names */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Yes */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 1st error in PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 2nd error in PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(3)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* clear confirmation msg */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* 3rd error in PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(5)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)		/* wait for confirmation msg to clear */

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PUK2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(7)	|		| 	SIM_UNBLOCK_REQ	|
	|		*	==========================>	*
(8)	|		| 	SIM_UNBLOCK_CNF	|
	|		*	<==========================	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_2
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_1
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PUK2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_0
	puk2_cnt	NUM_10

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_2_VALUE_11223344
	pin	PIN_1_VALUE_1233
	pin_id	PHASE_2_PUK_2

- SIM_UNBLOCK_CNF
<A>	cause	SIM_NO_ERROR
<B>	cause	SIM_CAUSE_PUK2_REMAIN9
	pin_id	PHASE_2_PUK_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
<A>	puk2_cnt	NUM_10
<B>	puk2_cnt	NUM_9

History:

12-04-2002	ef		initial

### MFW388: After correct PUK2 code has been entered, then activate SIM..

Description:.Continuation of test 386A after correct PUK2 code has been entered.

Preamble: 	MFW386A

APL		ACI		PS

|		|		|

(1)	|		| 	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_FDN_ENABLE
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

History:

12-04-2002	ef		initial

### MFW390: After incorrect PUK2 code has been entered, re-enter PUK2 correctly then activate SIM.

Description:.Continuation of test 386B after incorrect PUK2 code has been entered, enter the correct PUK2.

Preamble: 	MFW386B

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (10000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* re-enter PUK2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_UNBLOCK_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_UNBLOCK_CNF	|
	|		*	<==========================	*
(3)	|		| 	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(4)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_2_VALUE_11223344
	pin	PIN_1_VALUE_1233
	pin_id	PHASE_2_PUK_2

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PUK_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ACTIVATE_REQ
	proc	SIM_FDN_ENABLE
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

History:

12-04-2002	ef		initial

### MFW392: After incorrect PUK2 code has been entered, re-enter PUK2 correctly but wrong PIN 1, re-enter.

Description:.Continuation of test 386B after incorrect PUK2 code has been entered, re-enter correct PUK2 but different new PIN2, repeat with correct PUK2 and twice same PIN2.

Preamble: 	MFW386B

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (10000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PUK2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)		/* error in confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_UNBLOCK_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_UNBLOCK_CNF	|
	|		*	<==========================	*
(3)	|		| 	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(4)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_2_VALUE_11223344
	pin	PIN_1_VALUE_1239
	pin_id	PHASE_2_PUK_2

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PUK_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ACTIVATE_REQ
	proc	SIM_FDN_ENABLE
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

History:

12-04-2002	ef		initial

### MFW394: After incorrect PUK2 code has been entered, abort during re-entry of PUK2..

Description:.Continuation of test 386B after incorrect PUK2 code has been entered, during entery of the correct PUK2, abort back to main menu.

Preamble: 	MFW386B

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (10000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

15-04-2002	ef		initial

16-08-2003	ef		add two RSK to bring to idle screen

### MFW396: Main Menu, Security, Fixed Names, Deactivate.

Description:.Go to the correct Menu window and open Security, Fixed Names, No, enter PIN2 successfully.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Fixed Names */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* No */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
(3)	|		| 	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(4)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ACTIVATE_REQ
	proc	SIM_FDN_DISABLE
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

History:

15-04-2002	ef		initial

### MFW398: Main Menu, Security, Fixed Names, Deactivate, Abort and Deactivate.

Description:.From the Idle screen, Menu, open Security, Fixed Names, No, enter PIN2 and Abort, back to previous menu, No, Deactivate, enter correct PIN2.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Fixed Names */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* No */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=END)		/* back to previous menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
(3)	|		| 	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(4)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ACTIVATE_REQ
	proc	SIM_FDN_DISABLE
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

History:

15-04-2002	ef		initial

18-08-2003	ef		correct use of go back to previous menu ie END key

### MFW400: Main Menu, Security, Dectivate Fixed Names, 3 wrong PIN2 entries, followed by PUK2.

Description:.From the Idle screen, open Security, Fixed Names, No, enter PIN2 incorrectly three times followed by the required PUK2 code, Deactivate FDN successfully.

A: 	No error in PUK2 code

B: 	Error in PUK2 code requiring re-input.

Variants:		<A>...<B>

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Fixed Names */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* No */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (3200)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(3)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(4)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (3200)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(5)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PUK2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter new PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* confirm PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(7)	|		| 	SIM_UNBLOCK_REQ	|
	|		*	==========================>	*
(8)	|		| 	SIM_UNBLOCK_CNF	|
	|		*	<==========================	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_2
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PIN2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_1
	puk2_cnt	NUM_10

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1239
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_CAUSE_PUK2_EXPECT
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_0
	puk2_cnt	NUM_10

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_2_VALUE_11223344
	pin	PIN_1_VALUE_1233
	pin_id	PHASE_2_PUK_2

- SIM_UNBLOCK_CNF
<A>	cause	SIM_NO_ERROR
<B>	cause	SIM_CAUSE_PUK2_REMAIN9
	pin_id	PHASE_2_PUK_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
<A>	puk2_cnt	NUM_10
<B>	puk2_cnt	NUM_9

History:

12-04-2002	ef		initial

27-01-2003	ef		change cause of SIM_VERIFY_PIN_CNF, is this correct?

### MFW402: After correct PUK2 code has been entered, then Deactiavate FDN.

Description:.Continuation of test 386A after correct PUK2 code has been entered.

Preamble: 	MFW400A

APL		ACI		PS

|		|		|

(1)	|		| 	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_FDN_DISABLE
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

History:

12-04-2002	ef		initial

### MFW404: After incorrect PUK2 code has been entered, re-enter PUK2 correctly. Deactivate FDN.

Description:.Continuation of test 400B after incorrect PUK2 code has been entered, enter the correct PUK2.

Preamble: 	MFW400B

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (10000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PUK2 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter PIN2 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Confirm PIN2 code */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_UNBLOCK_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_UNBLOCK_CNF	|
	|		*	<==========================	*
(3)	|		| 	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(4)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	PUK_2_VALUE_11223344
	pin	PIN_1_VALUE_1233
	pin_id	PHASE_2_PUK_2

- SIM_UNBLOCK_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PUK_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ACTIVATE_REQ
	proc	SIM_FDN_DISABLE
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

History:

12-04-2002	ef		initial

### MFW406: Main Menu, Security, Fixed Names, Activate then Deactivate.

Description:.From the Idle screen, open Security, Fixed Names, Yes, enter PIN2 to Activate FDN, then No to Deactivate FDN.

Preamble: 	MFW052

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Security */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Fixed Names */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Yes */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (40000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(2)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
(3)	|		| 	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(4)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* return to previous menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Fixed Names */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* No */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter PIN2 */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(5)	|		| 	SIM_VERIFY_PIN_REQ	|
	|		*	==========================>	*
(6)	|		| 	SIM_VERIFY_PIN_CNF	|
	|		*	<==========================	*
(7)	|		| 	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(8)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ACTIVATE_REQ
	proc	SIM_FDN_ENABLE
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_VERIFY_PIN_REQ
	source	SRC_MMI
	pin	PIN_1_VALUE_1234
	pin_id	PHASE_2_PIN_2

- SIM_VERIFY_PIN_CNF
	cause	SIM_NO_ERROR
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- SIM_ACTIVATE_REQ
	proc	SIM_FDN_DISABLE
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NO_PREF_LANG

History:

15-04-2002	ef		initial

18-08-2003	ef		add extra RSK to go back to idle

## MMI Component Tests - case: Main Menu, Names, Phonebook.

### MFW430: Power On Sequence with Phonebook(ADN enabled).

Description:.Power on the mobile, with no PIN required. The ADN in the Phonebook is enabled.

Preamble: 	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(8)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(9)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(10)	|		|	MNSMS_REPORT_IND	|

|			*<==========================*

(11)	|		| 	MNSMS_MESSAGE_IND	|

|			*<==========================*

(12)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(13)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (10000)
(14)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(15)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(16)	|		|	MNSMS_REPORT_IND	|

|			*<==========================*

(17)	|		|	MMI_CBCH_REQ	|
	|			*==========================>*
(18)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(19)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(20)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(21)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(22)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(23)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(24)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(25)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(26)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(27)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(28)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(29)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(30)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(31)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(32)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(33)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(34)	|		|	MNSMS_READ_REQ	|

|		*	==========================>	*

(35)	|		|	MNSMS_READ_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (6000)
	|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NO_EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV_PHB
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF
	datafield	SIM_CPHS_CINF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

- MNSMS_REPORT_IND
	state	SMS_STATE_INITIALISING

- MNSMS_MESSAGE_IND
	mem_type	MEM_SM
	rec_num	NUM_0
	rec_max	NUM_0
	status	SMS_RECORD_REC_READ
	sms_sdu	SMS_SDU_1

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- MNSMS_REPORT_IND
	state	SMS_STATE_READY

- MMI_CBCH_REQ
	msg_id	CBCH_MSG_ID_ARR
	dcs_id	CBCH_DCS_ID_ARR
	modus	MMI_CBCH_STOP

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_1
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_ADN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_1
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_1

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_2
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_ADN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_2
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_2

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_3
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_ADN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_3
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_3

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_4
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_ADN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_4
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_4

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_5
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_ADN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_5
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_5

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_6
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_ADN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_6
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_6

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_7
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_ADN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_7
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_EMPTY

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_8
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_ADN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_8
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_EMPTY

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	REC_NUM_0
	read_mode	READ_PREVIEW
	status	NUM_FF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_0
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	REC_NUM_1
	cause	SIM_NO_ERROR
	rec_status	SMS_RECORD_NOT_EXIST
	status	SMS_RECORD_STO_UNSENT
	sms_sdu	SMS_SDU_MO_ABS

History:

16-04-2002	ef		initial

30-07-2002	ef		updated

09-10-2002	ef		add time for idle window to be output

22-10-2002	ef		add cphs sim info

29-10-2002	ef		add MNSMS_REPORT_IND for initialisation.

16-01-2003	ef		add cause to MNSMS_READ_CNF

16-01-2003	ef		add MMI_CBCH_REQ signal to stop reading CBCH

### MFW432: Power On Sequence with Phonebook(FDN enabled).

Description:.Power on the mobile, with no PIN required. The FDN in the Phonebook is enabled.

Preamble: 	MFW001

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG AUTO_ATTACH)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(8)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(9)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(10)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(11)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (5000)
(12)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

(13)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(14)	|		|	MNSMS_REPORT_IND	|

|			*<==========================*

(15)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(16)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(17)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(18)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(19)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(20)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(21)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(22)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(23)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(24)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(25)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(26)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(27)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(28)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(29)	|		|	SIM_READ_RECORD_REQ	|

|			*==========================>*

(30)	|		|	SIM_READ_RECORD_CNF	|

|			*<==========================*

(31)	|		|	MNSMS_READ_REQ	|

|		*	==========================>	*

(32)	|		|	MNSMS_READ_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (1000)
	|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NO_EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_FDN_ENABLED
	sim_serv	F_SIM_SRV_FDN_PHB
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF
	datafield	SIM_CPHS_CINF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_COMB
	plmn	PLMN_1
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- MNSMS_REPORT_IND
	state	SMS_STATE_READY

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_FDN
	record	PHB_RECORD_1
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_FDN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_1
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_1

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_FDN
	record	PHB_RECORD_2
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_FDN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_2
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_2

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_FDN
	record	PHB_RECORD_3
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_FDN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_3
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_3

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_FDN
	record	PHB_RECORD_4
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_FDN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_4
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_4

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_FDN
	record	PHB_RECORD_5
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_FDN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_5
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_5

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_FDN
	record	PHB_RECORD_6
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_FDN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_6
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_6

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_FDN
	record	PHB_RECORD_7
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_FDN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_7
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_EMPTY

- SIM_READ_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_FDN
	record	PHB_RECORD_8
	length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_FDN
	cause	SIM_NO_ERROR
	record	PHB_RECORD_8
	max_record	PHB_MAX_RECORD_8
	length	PHB_LENGTH_ADN
	linear_data	PHB_ADN_RECORD_EMPTY

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	REC_NUM_0
	read_mode	READ_PREVIEW
	status	NUM_FF

- MNSMS_READ_CNF
	mem_type	MEM_ME
	rec_num	REC_NUM_0
	rec_next	NOT_USED
	rec_max	NOT_USED
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_STAT_STRD
	sms_sdu	SMS_SDU_1

History:

16-04-2002	ef		initial

31-07-2002	ef		updated

09-10-2002	ef		add time for idle window to be output

16-01-2003	ef		add cause to MNSMS_READ_CNF

### MFW440: FDN Enabled make Mobile Originated Call when call is in the FDN.

Description:.FDN is Enabled. Key in phone number of entry in the FDN and press accept. An outgoing call is setup.

Preamble: 	MFW432

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

(4)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(5)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

(6)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
(7)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (3000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_01317180537
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

History:

11-06-2002	ef		initial

20-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

18-08-2003	ef		add TCH indication and correct ti parameter on Call_proceeding

### MFW442: FDN Enabled make Mobile Originated Call when call is not in the FDN.

Description:.FDN is Enabled. Key in phone number of entry in the FDN and press accept. NO call is setup.

Preamble: 	MFW432

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=654321)

TIMEOUT_WAIT (3000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

History:

11-06-2002	ef		initial

### MFW444: FDN Enabled make Emergency Call when call is not in the FDN.

Description:.FDN is Enabled. Key in phone number of entry in the FDN and press accept. Emergency Call is setup.

Preamble: 	MFW432

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* enter number */

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(4)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

(5)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (3000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_EMERG_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_112
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_END_TO_END_PLMN
	connected_number	CONNECTED_NUMBER_112
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

History:

30-09-2003	ef		initial

### MFW450: From the Idle Screen, Names, Add new Entry into the Phonebook.

Description:.From the Idle Screen, Names, Add New Entry. Key in the number (9876543210) and name (Sid), and back to Main Menu.

Preamble: 	MFW430

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Names */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* add new entry */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=9)		/* Enter Number */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)		/* enter name */

TIMEOUT_WAIT (100)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (100)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (100)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (100)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (100)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_UPDATE_RECORD_REQ	|

|			*==========================>*
(2)	|		| 	SIM_UPDATE_RECORD_CNF	|

|		*<	==========================	*

TIMEOUT_WAIT (5000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_UPDATE_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_7
	length	PHB_LENGTH_ADN
 	linear_data	PHB_ADN_RECORD_SID

- SIM_UPDATE_RECORD_CNF
	datafield	SIM_ADN
	record	PHB_RECORD_7
	cause	SIM_NO_ERROR

History:

20-06-2002	ef		initial

18-08-2003	ef		add one more menu down for add names, add primitive

### MFW452: From Idle, Search for a name in the Phonebook, and Call.

Description:.From the Idle Screen, Names, Search for ERIC, Options, Call Number.

Preamble: 	MFW430

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Names */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Search */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Call Number */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

(4)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

(5)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(6)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

(7)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
	|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_01317180537
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

History:

20-06-2002	ef		initial

### MFW454: From Idle, Search for a name in the Phonebook, and Delete.

Description:.From the Idle Screen, Names, Search, First entry (ANDREAS) and  Options, Delete.

Preamble: 	MFW450

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Names */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Search */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* 1st entry */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* press OK to delete */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	SIM_UPDATE_RECORD_REQ	|

|			*==========================>*
(2)	|		| 	SIM_UPDATE_RECORD_CNF	|

|		*<	==========================	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_UPDATE_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_1
	length	PHB_LENGTH_ADN
 	linear_data	PHB_ADN_RECORD_1_DELETE

- SIM_UPDATE_RECORD_CNF
	datafield	SIM_ADN
	record	PHB_RECORD_1
	cause	SIM_NO_ERROR

History:

24-06-2002	ef		initial

### MFW456: From Idle, Search for a name in the Phonebook, and Change.

Description:.From the Idle Screen, Names, Search, First entry (ANDREAS) and  Options, Change. Leave number and name unchanged.

Preamble: 	MFW450

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Names */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Search */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* 1st entry */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Change */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* new number */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* new name */

(1)	|		| 	SIM_UPDATE_RECORD_REQ	|

|			*==========================>*
(2)	|		| 	SIM_UPDATE_RECORD_CNF	|

|		*<	==========================	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_UPDATE_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_1
	length	PHB_LENGTH_ADN
 	linear_data	PHB_ADN_RECORD_1_CHANGE

- SIM_UPDATE_RECORD_CNF
	datafield	SIM_ADN
	record	PHB_RECORD_1
	cause	SIM_NO_ERROR

History:

24-06-2002	ef		initial

### MFW458: From Main menu, Update the Call Centre and Validity period, ready for SMS. Make MO Call.

Description:.From the Main Menu, Messages, Settings, Set Validy Period of 1 Hour, Up the Menu, Add the Service Centre Number. Then back to the Main Menu and set up a MO Call.

Preamble: 	MFW430

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Messages */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Settings */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Validity */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* 1 hour */

TIMEOUT_WAIT (6000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Validity */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=UP)		/* Service Centre */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter number */

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)     /*Back to idle screen*/

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

(3)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

(4)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

(5)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
(6)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (3000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_01317180537
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

History:

28-08-2002	ef		initial

15-01-2003	ef		add cause to SIM_SYNC_CNF

### MFW460: From Idle, Names, Display Memory Stat.

Description:.From the Idle screen, Names, Memory Stat..

Preamble: 	MFW430

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Names */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Memory Stat */

TIMEOUT_WAIT (4000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

History:

05-08-2003	ef		initial

### MFW462: From Idle, Names, Set the Phonebook to SIM then Phone

Description:.From the Idle screen, Names, Select Book, Sim, then the Phone.

Preamble: 	MFW430

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Names */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select Book */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* SIM */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Phone */

TIMEOUT_WAIT (6000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Back to Idle */

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

History:

05-08-2003	ef		initial

### MFW464: From Idle, Search for a name in the Phonebook, and Copy entry to SIM.

Description:.From the Idle Screen, Names, Search, First entry (ANDREAS) and  Options, Copy.

Preamble: 	MFW430

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Names */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Search */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* 1st entry */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Copy */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* This entry */

(1)	|		| 	SIM_UPDATE_RECORD_REQ	|

|			*==========================>*
(2)	|		| 	SIM_UPDATE_RECORD_CNF	|

|		*<	==========================	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_UPDATE_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_1
	length	PHB_LENGTH_ADN
 	linear_data	PHB_ADN_RECORD_1_DELETE

- SIM_UPDATE_RECORD_CNF
	datafield	SIM_ADN
	record	PHB_RECORD_1
	cause	SIM_NO_ERROR

History:

05-08-2003	ef		initial

### MFW466: From Idle, Search for a name in the Phonebook, Send Message.

Description:.From the Idle Screen, Names, Search, First entry (ANDREAS) and  Options, Send Message (“Eric”).

Note: Pass over the special characters àand á which appear on the ABC button. This will be removed at a later stage.

Preamble: 	MFW430

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Main Menu */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Messages */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Settings */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Validity */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* 1 hour */

TIMEOUT_WAIT (6000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=UP)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Service Centre */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Back to Idle */

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Names */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Search */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* 3rd  entry */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Send Message */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Enter Number */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Send */

(1)	|		| 	MNSMS_SUBMIT_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (6000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNSMS_SUBMIT_REQ
	mem_type	MEM_SM
	rec_num	NUM_0
	condx	SMS_CONDX_OVR_NON
	modify	SMS_MODIFY_NON
	sms_sdu	SMS_SDU_ERIC_NAMES

History:

06-08-2003	ef		initial

19-08-2003	ef		skip over the special chars à and á. Remove later.

09-10-2003	ef		remove the chars à and á

### MFW468: From Idle, Names, Service Number

Description:.From the Idle screen, Names, Service Number

Note: this test always pass as only internal memory is read.

Preamble: 	MFW430

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Names */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (5000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Back to Idle */

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

History:

05-08-2003	ef		initial

### MFW470: From Idle, Names, Change My Number

Description:.From the Idle screen, Names, My Number, Consult, Change, Enter new number and name.

Note: this test always pass as only internal memory is read.

Preamble: 	MFW430

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Names */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* My Number */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Consult */

TIMEOUT_WAIT (5000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Change */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Enter Name */
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
	|		|		|

- Parametrization:

Primitive	Parameter	Value

History:

13-08-2003	ef		initial

### MFW472: From Idle, Search for a name in the Phonebook, and Move entry to SIM. Test fails.

Description:.From the Idle Screen, Names, Search, First entry (ANDREAS) and  Options, Move.

Note: Test fails because the phonebook code has not(yet) been included in the MFW project.

Preamble: 	MFW430

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Names */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Search */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* 1st entry */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Move */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* This entry */

(1)	|		| 	SIM_UPDATE_RECORD_REQ	|

|			*==========================>*
(2)	|		| 	SIM_UPDATE_RECORD_CNF	|

|		*<	==========================	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_UPDATE_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_1
	length	PHB_LENGTH_ADN
 	linear_data	PHB_ADN_RECORD_1_DELETE

- SIM_UPDATE_RECORD_CNF
	datafield	SIM_ADN
	record	PHB_RECORD_1
	cause	SIM_NO_ERROR

History:

06-10-2003	ef		initial

## MMI Component Tests case : Main Menu, Network Services, Activate Call Waiting.

### MFW490: From Main menu, Network services, Set Call Waiting Enabled, ready for Call Processing tests.

Description:.From the Main Menu, Network Settings, Call Waiting, Set Enabled and back to Idle Screen.

Note: The extra LEFT key after SS OK is returned by the network. This will taking out at a later date.

Preamble: 	MFW700

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Network Services */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Call Waiting */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Activate */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (2000)

(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* clear msg confirmation */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	FAC_SET_WAITING_ENABLED
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_WAITING_RETURN_RESULT

History:

14-04-2003	ef		initial

12-08-2003	ef		change preamble to init the Message Service centre

## MMI Component Tests - case : MO Call Processing

### MFW500: Mobile Originated Call.

Description:.Set Call Waiting Enabled and then from Idle mode, key in phone number and accept. An outgoing call is setup.

Preamble: 	MFW490

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)
 (1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

(4)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

(5)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(6)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

(7)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (3000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_01317180537
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

History:

17-04-2002	ef		initial
	03-05-2002	ef		Add Alert Indication and Call Proceed Indication

13-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

14-08-2003	ef		add TCH indication

### MFW502: End a Mobile Originated Call.

Description:.Clear down the outgoing call.

Preamble: 	MFW500

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=END)

(1)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(2)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(3)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR

History:

17-04-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		add to payload of MNCC_DISCONNECT_REQ

20-12-2002	ef		update the cause value for MNCC_DISCONNECT_REQ

### MFW504: Hold and Retrieve a Mobile Originated Call.

Description:.The HOLD and RETRIEVE option are initiated for this MO call..

Preamble: 	MFW500

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNCC_HOLD_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (1000)

(2)	|		| 	MNCC_HOLD_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (4000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(3)	|		| 	MNCC_RETRIEVE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (1000)

(4)	|		| 	MNCC_RETRIEVE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (3000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_HOLD_REQ
	ti	TI_MO_0

- MNCC_HOLD_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_HOLD_SUCCESS

- MNCC_RETRIEVE_REQ
	ti	TI_MO_0

- MNCC_RETRIEVE_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_RETRIEVE_SUCCESS

History:

17-04-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

20-12-2002	ef		cause updated MNCC_HOLD_CNF/MNCC_RETRIEVE_CNF

### MFW506: Hold failure for a Mobile Originated Call, followed by Hold and retrieve.

Description:.The HOLD option is initiated for this MO call, but this fails so there is no change in call status.Afterwards a successful Hold and Retrieve is made.

Preamble: 	MFW500

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNCC_HOLD_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (1000)

(2)	|		| 	MNCC_HOLD_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(3)	|		| 	MNCC_HOLD_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (1000)

(4)	|		| 	MNCC_HOLD_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (4000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(5)	|		| 	MNCC_RETRIEVE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (1000)

(6)	|		| 	MNCC_RETRIEVE_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (3000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_HOLD_REQ
	ti	TI_MO_0

- MNCC_HOLD_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_TEMP_FAIL

- MNCC_HOLD_REQ
	ti	TI_MO_0

- MNCC_HOLD_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_HOLD_SUCCESS

- MNCC_RETRIEVE_REQ
	ti	TI_MO_0

- MNCC_RETRIEVE_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_RETRIEVE_SUCCESS

History:

17-04-2002	ef		initial

20-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

20-12-2002	ef		cause updated MNCC_HOLD_CNF/MNCC_RETRIEVE_CNF

### MFW508: MO call terminated by remote disconnection.

Description: A Mobile Originated call is disconnected by a backward clear

Preamble: 	MFW500

APL		ACI		PS

|		|		|

|		|		|
(1)	|		| 	MNCC_DISCONNECT_IND	|

|		*	<==========================	*

(2)	|		| 	MNCC_RELEASE_CNF	|

|		*	<==========================	*

(3)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(4)	|		| 	SIM_SYNC_CNF	|

|		*	<==========================	*

- TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_DISCONNECT_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR
	diagnostic	NOT_PRESENT_8BIT
	progress_desc	PROG_END_TO_END_PLMN

- MNCC_RELEASE_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

History:

17-04-2002	ef		initial

15-01-2003	ef		add cause to SIM_SYNC_CNF

## MMI Component Tests - case : MT Call Processing

### MFW510: Mobile Terminated call setup, Voice

Description: From the idle screen, a MT call is indicated and accepted.

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

- TIMEOUT_WAIT (4000)

(1)	|		| 	MMI_RXLEV_IND	|
	|		*<	==========================	*

(2)	|		| 	GMMREG_CIPHERING_IND	|
	|		*<	==========================	*

(3)	|		| 	MNCC_SETUP_IND	|
	|		*<	==========================	*

(4)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(6)	|		| 	MNCC_ALERT_REQ	|
	|		*	==========================>	*
(7)	|		| 	MMI_RXLEV_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(8)	|		| 	MNCC_SETUP_RES	|
	|		*	==========================>	*
(9)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(10)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(11)	|		| 	MNCC_SETUP_COMPL_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (4000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MMI_RXLEV_IND
	rxlev	NUM_8

- GMMREG_CIPHERING_IND

- gsm_ciph	CIPH_ON
	gprs_ciph	CIPH_ON

- MNCC_SETUP_IND
	ti	TI_MT_9
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_SPEECH
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIG_DIAL_TONE_ON
	calling_party	CLG_PARTY_01317180537
	calling_party_sub	CLG_PARTY_SUB
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB
	redirecting_party	RED_PARTY
	redirecting_party_sub	RED_PARTY_SUB

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_SIG_ONLY

- ACI_CMD_IND
	cmd_len	LM_RING_LEN
	cmd_seq	M_RING

- MNCC_ALERT_REQ
	ti	TI_MT_9

- MMI_RXLEV_IND
	rxlev	NUM_8

- MNCC_SETUP_RES
	ti	TI_MT_9

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_SETUP_COMPL_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_SUCCESS

History:

17-04-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

20-12-2002	ef		updated cause in MNCC_SYNC_IND

02-06-2003	ef		add TCH indication.

12-08-2003	ef		change preamble to set SMS service centre for text reply

### MFW511: Mobile Terminated call Setup, Data

Description: From the idle screen, a MT non-transparent data call is indicated and accepted.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

- TIMEOUT_WAIT (4000)

(1)	|		| 	MMI_RXLEV_IND	|
	|		*<	==========================	*

(2)	|		| 	GMMREG_CIPHERING_IND	|
	|		*<	==========================	*

(3)	|		| 	MNCC_SETUP_IND	|
	|		*<	==========================	*

(4)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(6)	|		| 	MNCC_ALERT_REQ	|
	|		*	==========================>	*
(7)	|		| 	MMI_RXLEV_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(8)	|		| 	MNCC_SETUP_RES	|
	|		*	==========================>	*
(9)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(10)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(11)	|		| 	MNCC_SETUP_COMPL_IND	|

|		*	<==========================	*

(12)	|		| 	RA_ACTIVATE_REQ	|

|		*	==========================>	*

(13)	|		|	RA_ACTIVATE_CNF	|
	|		*	<==========================	*

(14)	|		| 	L2R_ACTIVATE_REQ	|

|		*	==========================>	*

(15)	|		|	L2R_ACTIVATE_CNF	|
	|		*	<==========================	*

(16)	|		| 	L2R_CONNECT_REQ	|

|		*	==========================>	*

(17)	|		|	L2R_CONNECT_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (4000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MMI_RXLEV_IND
	rxlev	NUM_8

- GMMREG_CIPHERING_IND

- gsm_ciph	CIPH_ON
	gprs_ciph	CIPH_ON

- MNCC_SETUP_IND
	ti	TI_MT_9
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_ASYNC_9600
	bcpara2	BC_PARA_ASYNC_9600
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIG_DIAL_TONE_ON
	calling_party	CLG_PARTY_01317180537
	calling_party_sub	CLG_PARTY_SUB
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB
	redirecting_party	RED_PARTY
	redirecting_party_sub	RED_PARTY_SUB

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_SIG_ONLY

- ACI_CMD_IND
	cmd_len	LM_RING_LEN
	cmd_seq	M_RING

- MNCC_ALERT_REQ
	ti	TI_MT_9

- MMI_RXLEV_IND
	rxlev	NUM_8

- MNCC_SETUP_RES
	ti	TI_MT_9

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_DATA_TCH_F

- MNCC_SETUP_COMPL_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_SUCCESS

- RA_ACTIVATE_REQ
	model	RA_MODEL_RLP
	tra_rate	TRA_FULLRATE_9600
	user_rate	URA_9600
	ndb	NUM_DATA_BITS
	nsb	NUM_STOP_BITS_1

- RA_ACTIVATE_CNF
	ack_flg	RA_ACK

- L2R_ACTIVATE_REQ
	k_ms_iwf	L2r_WIN_SIZE_MS_IWF
	k_iwf_ms	L2R_WIN_SIZE_IWF_MS
	t1	L2R_ACK_TIMER
	t2	L2R_RELAY_DELAY_20
	n2	L2R_RETRANS_ATTEMPTS
	pt	L2R_COMPR_TYPE_V42BIS
	p0	L2R_COMP_DIR_NONE
	p1	L2R_NUM_CODE_WORDS
	p2	L2R_CODE_STR_LEN
	uil2p	L2R_ISO6429
	bytes_per_prim	L2R_BYTES_PER_PRIM_250
	buffer_size	L2R_BUFFER_SIZE_2048
	rate	L2R_FULLRATE_9600

- L2R_ACTIVATE_CNF
	ack_flg	L2R_ACK

- L2R_CONNECT_REQ

- L2R_CONNECT_CNF
	ack_flg	L2R_ACK

History:

02-06-2003	ef		initial

### MFW512: Mobile Terminated call Setup, FAX

Description: From the idle screen, a MT FAX call is indicated and accepted.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

- TIMEOUT_WAIT (4000)

(1)	|		| 	MMI_RXLEV_IND	|
	|		*<	==========================	*

(2)	|		| 	GMMREG_CIPHERING_IND	|
	|		*<	==========================	*

(3)	|		| 	MNCC_SETUP_IND	|
	|		*<	==========================	*

(4)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(6)	|		| 	MNCC_ALERT_REQ	|
	|		*	==========================>	*
(7)	|		| 	MMI_RXLEV_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(8)	|		| 	MNCC_SETUP_RES	|
	|		*	==========================>	*
(9)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(10)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(11)	|		| 	MNCC_SETUP_COMPL_IND	|

|		*	<==========================	*

(12)	|		| 	RA_ACTIVATE_REQ	|

|		*	==========================>	*

(13)	|		|	RA_ACTIVATE_CNF	|
	|		*	<==========================	*

(14)	|		| 	T30_ACTIVATE_REQ	|

|		*	==========================>	*

(15)	|		|	T30_ACTIVATE_CNF	|
	|		*	<==========================	*

(16)	|		| 	T30_CONFIG_REQ	|

|		*	==========================>	*

(17)	|		|	T30_CAP_REQ	|
	|		*	==========================>	*

TIMEOUT_WAIT (4000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MMI_RXLEV_IND
	rxlev	NUM_8

- GMMREG_CIPHERING_IND
	gsm_ciph	CIPH_ON
	gprs_ciph	CIPH_ON

- MNCC_SETUP_IND
	ti	TI_MT_9
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_FAX
	bcpara2	BC_PARA_FAX
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIG_DIAL_TONE_ON
	calling_party	CLG_PARTY_01317180537
	calling_party_sub	CLG_PARTY_SUB
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB
	redirecting_party	RED_PARTY
	redirecting_party_sub	RED_PARTY_SUB

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_SIG_ONLY

- ACI_CMD_IND
	cmd_len	LM_RING_LEN
	cmd_seq	M_RING

- MNCC_ALERT_REQ
	ti	TI_MT_9

- MMI_RXLEV_IND
	rxlev	NUM_8

- MNCC_SETUP_RES
	ti	TI_MT_9

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_FAX_TCH_F

- MNCC_SETUP_COMPL_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_SUCCESS

- RA_ACTIVATE_REQ
	model	RA_MODEL_FAX
	tra_rate	TRA_FULLRATE_9600
	user_rate	URA_9600
	ndb	NUM_DATA_BITS
	nsb	NUM_STOP_BITS_1

- RA_ACTIVATE_CNF
	ack_flg	RA_ACK

- T30_ACTIVATE_REQ
	trans_rate	TRANS_RATE_9600
	half_rate	HALF_RATE_FLAG_0
	threshold	TCF_THRESHOLD_90
	frames_per_prim	NO_FRAMES_PER_PRIM_3
	bitorder	FBO_NRM_DATA

- T30_ACTIVATE_CNF
	buf_size_rx	L2R_BUFFER_SIZE_2048
	buf_size_tx	L2R_BUFFER_SIZE_2048

- T30_CONFIG_REQ
	hdlc_report	HDLC_REPORT_1
	test_mode	TST_OFF

- T30_CAP_REQ
	hdlc_info	CAPS_DCS0

History:

02-06-2003	ef		initial

### MFW513: End of Mobile Terminated Call.

Description: Clear the call down by pressing the end key.

Preamble: 	MFW510

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=END)

(1)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(2)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(3)	|		| 	MNCC_RELEASE_IND	|

|		*	<==========================	*

- TIMEOUT_WAIT (5000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	TI_MT_9
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CALL_CLEAR

History:

01-05-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		add to payload of MNCC_DISCONNECT_REQ

20-12-2002	ef		update the cause value for MNCC_DISCONNECT_REQ

## MMI Component Tests - case : MO / MT Call Processing

### MFW514: MO Call followed by a MT Call, first call put on Hold.

Description: During a MO call, a 2nd call arrives. The 1st call is put on Hold and the waiting MT call is accepted.

A: 	Speech

B: 	Data

Variants:		<A>..<B>

Preamble: 	MFW500

APL/ACI		MMI		PS

|		|		|

- TIMEOUT_WAIT (4000)

(1)	|		| 	MMI_RXLEV_IND	|
	|		*<	==========================	*

(2)	|		| 	GMMREG_CIPHERING_IND	|
	|		*<	==========================	*

(3)	|		| 	MNCC_SETUP_IND	|
	|		*<	==========================	*

(4)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

- (5)	|		| 	MNCC_ALERT_REQ	|
	|		*	==========================>	*
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Accept Call */

(6)	|		| 	MNCC_HOLD_REQ	|

|		*	==========================>	*

(7)	|		| 	MNCC_HOLD_CNF	|

|		*	<==========================	*

(8)	|		| 	MNCC_SETUP_RES	|
	|		*	==========================>	*
(9)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(10)	|		| 	MNCC_SETUP_COMPL_IND	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MMI_RXLEV_IND
	rxlev	NUM_8

- GMMREG_CIPHERING_IND

- gsm_ciph	CIPH_ON
	gprs_ciph	CIPH_ON

- MNCC_SETUP_IND
	ti	TI_MT_9
	ri	NOT_PRESENT_8BIT
<A>	bcpara	BC_PARA_SPEECH
<A>	bcpara2	BC_PARA_SPEECH
<B>	bcpara	BC_PARA_ASYNC_1200
<B>	bcpara2	BC_PARA_ASYNC_1200
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIG_DIAL_TONE_ON
	calling_party	CLG_PARTY_01317180537
	calling_party_sub	CLG_PARTY_SUB
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB
	redirecting_party	RED_PARTY
	redirecting_party_sub	RED_PARTY_SUB

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_SIG_ONLY

- MNCC_ALERT_REQ
	ti	TI_MT_9

- MNCC_HOLD_REQ
	ti	TI_MO_0

- MNCC_HOLD_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_HOLD_SUCCESS

- MNCC_SETUP_RES
	ti	TI_MT_9

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_SETUP_COMPL_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_SUCCESS

History:

02-05-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

20-12-2002	ef		cause updated MNCC_SETUP_COMPL_IND/MNCC_SYNC_IND

28-01-2003	ef		new parameter “redirecting party” for MNCC_SETUP_IND

07-07-2003	ef		change SEND key to LEFT key

14-08-2003	ef		add the TCH indication

### MFW516: Using the Send Key, Swap the two calls twice so that the MT call is active and the MO call is held.

Description: The MT active call and the waiting MO call are swapped by twice by the use of the Send key.

Preamble: 	MFW514A

APL		MMI		PS

|		|		|

- TIMEOUT_WAIT (4000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNCC_HOLD_REQ	|

|		*	==========================>	*

(2)	|		| 	MNCC_RETRIEVE_REQ	|

|		*	==========================>	*

(3)	|		| 	MNCC_HOLD_CNF	|

|		*	<==========================	*

(4)	|		| 	MNCC_RETRIEVE_CNF	|

|		*	<==========================	*

- TIMEOUT_WAIT (4000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(5)	|		| 	MNCC_HOLD_REQ	|

|		*	==========================>	*

(6)	|		| 	MNCC_RETRIEVE_REQ	|

|		*	==========================>	*

(7)	|		| 	MNCC_HOLD_CNF	|

|		*	<==========================	*

(8)	|		| 	MNCC_RETRIEVE_CNF	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_HOLD_REQ
	ti	TI_MT_9

- MNCC_RETRIEVE_REQ
	ti	TI_MO_0

- MNCC_HOLD_CNF
	ti	TI_MT_9
	cause	MNCC_CAUSE_HOLD_SUCCESS

- MNCC_RETRIEVE_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_RETRIEVE_SUCCESS

- MNCC_HOLD_REQ
	ti	TI_MO_0

- MNCC_RETRIEVE_REQ
	ti	TI_MT_9

- MNCC_HOLD_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_HOLD_SUCCESS

- MNCC_RETRIEVE_CNF
	ti	TI_MT_9
	cause	MNCC_CAUSE_RETRIEVE_SUCCESS

History:

02-05-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

20-12-2002	ef		cause updated MNCC_HOLD_CNF/MNCC_RETRIEVE_CNF

### MFW518: In Call Menu, Swap the two calls twice so that the MT call is active and the MO call is held.

Description: The MT active call and the waiting MO call are swapped by twice by the use of the In Call Menu and the Swap option.

Preamble: 	MFW514A

APL		MMI		PS

|		|		|

- TIMEOUT_WAIT (4000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* options */

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Swap */

- (1)	|		| 	MNCC_HOLD_REQ	|

|		*	==========================>	*

(2)	|		| 	MNCC_RETRIEVE_REQ	|

|		*	==========================>	*

(3)	|		| 	MNCC_HOLD_CNF	|

|		*	<==========================	*

(4)	|		| 	MNCC_RETRIEVE_CNF	|

|		*	<==========================	*

- TIMEOUT_WAIT (4000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Swap */

(5)	|		| 	MNCC_HOLD_REQ	|

|		*	==========================>	*

(6)	|		| 	MNCC_RETRIEVE_REQ	|

|		*	==========================>	*

(7)	|		| 	MNCC_HOLD_CNF	|

|		*	<==========================	*

(8)	|		| 	MNCC_RETRIEVE_CNF	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_HOLD_REQ
	ti	TI_MT_9

- MNCC_RETRIEVE_REQ
	ti	TI_MO_0

- MNCC_HOLD_CNF
	ti	TI_MT_9
	cause	MNCC_CAUSE_HOLD_SUCCESS

- MNCC_RETRIEVE_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_RETRIEVE_SUCCESS

- MNCC_HOLD_REQ
	ti	TI_MO_0

- MNCC_RETRIEVE_REQ
	ti	TI_MT_9

- MNCC_HOLD_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_HOLD_SUCCESS

- MNCC_RETRIEVE_CNF
	ti	TI_MT_9
	cause	MNCC_CAUSE_RETRIEVE_SUCCESS

History:

02-05-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

20-12-2002	ef		cause updated MNCC_HOLD_CNF/MNCC_RETRIEVE_CNF

07-07-2003	ef		in call menu changes

### MFW520: When in-call, initiate a 2nd call using DTMF tones.

Description: A MO call is active. Another MO call is initiated using DTMF tones The Send key is pressed and the 1st call(active) is placed on Hold, and the 2nd call is started.

Preamble: 	MFW500

APL		MMI		PS

|		|		|

- TIMEOUT_WAIT (4000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=6)

(1)	|		| 	MNCC_START_DTMF_REQ	|

|		*	==========================>	*

(2)	|		| 	MNCC_START_DTMF_CNF	|

|		*<	==========================	*

- TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=5)

(3)	|		| 	MNCC_START_DTMF_REQ	|

|		*	==========================>	*

(4)	|		| 	MNCC_START_DTMF_CNF	|

|		*<	==========================	*

- TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

(5)	|		| 	MNCC_START_DTMF_REQ	|

|		*	==========================>	*

(6)	|		| 	MNCC_START_DTMF_CNF	|

|		*<	==========================	*

- TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)

(7)	|		| 	MNCC_START_DTMF_REQ	|

|		*	==========================>	*

(8)	|		| 	MNCC_START_DTMF_CNF	|

|		*<	==========================	*

- TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=2)

(9)	|		| 	MNCC_START_DTMF_REQ	|

|		*	==========================>	*

(10)	|		| 	MNCC_START_DTMF_CNF	|

|		*<	==========================	*

- TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

(11)	|		| 	MNCC_START_DTMF_REQ	|

|		*	==========================>	*

(12)	|		| 	MNCC_START_DTMF_CNF	|

|		*<	==========================	*

- TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

- (13)	|		| 	MNCC_HOLD_REQ	|

|		*	==========================>	*

(14)	|		| 	MNCC_HOLD_CNF	|

|		*	<==========================	*

(15)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(16)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

(17)	|		| 	MNCC_PROGRESS_IND	|

|		*	<==========================	*

(18)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(19)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	* 
(20)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (3000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_START_DTMF_REQ
	ti	TI_MO_0
	key	DIG_6
	dtmf_mod	DTMF_MOD_AUTO

- MNCC_START_DTMF_CNF
	ti	TI_MO_0
	key	DIG_6
	cause	MNCC_CAUSE_DTMF_START_SUCCESS
	dtmf_mod	DTMF_MOD_AUTO

- MNCC_START_DTMF_REQ
	ti	TI_MO_0
	key	DIG_5
	dtmf_mod	DTMF_MOD_AUTO

- MNCC_START_DTMF_CNF
	ti	TI_MO_0
	key	DIG_5
	cause	MNCC_CAUSE_DTMF_START_SUCCESS
	dtmf_mod	DTMF_MOD_AUTO

- MNCC_START_DTMF_REQ
	ti	TI_MO_0
	key	DIG_4
	dtmf_mod	DTMF_MOD_AUTO

- MNCC_START_DTMF_CNF
	ti	TI_MO_0
	key	DIG_4
	cause	MNCC_CAUSE_DTMF_START_SUCCESS
	dtmf_mod	DTMF_MOD_AUTO

- MNCC_START_DTMF_REQ
	ti	TI_MO_0
	key	DIG_3
	dtmf_mod	DTMF_MOD_AUTO

- MNCC_START_DTMF_CNF
	ti	TI_MO_0
	key	DIG_3
	cause	MNCC_CAUSE_DTMF_START_SUCCESS
	dtmf_mod	DTMF_MOD_AUTO

- MNCC_START_DTMF_REQ
	ti	TI_MO_0
	key	DIG_2
	dtmf_mod	DTMF_MOD_AUTO

- MNCC_START_DTMF_CNF
	ti	TI_MO_0
	key	DIG_2
	cause	MNCC_CAUSE_DTMF_START_SUCCESS
	dtmf_mod	DTMF_MOD_AUTO

- MNCC_START_DTMF_REQ
	ti	TI_MO_0
	key	DIG_1
	dtmf_mod	DTMF_MOD_AUTO

- MNCC_START_DTMF_CNF
	ti	TI_MO_0
	key	DIG_1
	cause	MNCC_CAUSE_DTMF_START_SUCCESS
	dtmf_mod	DTMF_MOD_AUTO

- MNCC_HOLD_REQ
	ti	TI_MO_0

- MNCC_HOLD_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_HOLD_SUCCESS

- MNCC_SETUP_REQ
	ti	TI_MO_1
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_1
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_PROGRESS_IND
	ti	TI_MO_1
	progress_desc	PROG_END_TO_END_PLMN

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_ALERT_IND
	ti	TI_MO_1
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_1
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_654321
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

History:

03-05-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

20-12-2002	ef		cause updated MNCC_HOLD_CNF/MNCC_START_DTMF_CNF

14-08-2003	ef		add TCH indication

### MFW522: End  the MT call, In Call Menu, Hold, Activate the held call.

Description: The Active MT call is terminated by the End key. Using the In call menu, the MO Call Held is then made Active.

Preamble: 	MFW514A

APL		MMI		PS

|		|		|

- TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

(1)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(2)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

- TIMEOUT_WAIT (5000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Unhold */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Active */

(3)	|		| 	MNCC_RETRIEVE_REQ	|

|		*	==========================>	*

(4)	|		| 	MNCC_RETRIEVE_CNF	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_DISCONNECT_REQ
	ti	TI_MT_9
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CALL_CLEAR

- MNCC_RETRIEVE_REQ
	ti	TI_MO_0

- MNCC_RETRIEVE_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_HOLD_SUCCESS

History:

08-05-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		add to payload of MNCC_DISCONNECT_REQ

20-12-2002	ef		update the cause value for MNCC_DISCONNECT_REQ

19-08-2003	ef		replace END key with RSK, because END key ends both calls

09-10-2003	ef		increase wait timer for the in call menu to correctly appear

### MFW524: Reject a MT call, followed by a MO call.

Description: .From the Idle screen, reject an incoming MT call, then wait and follow with a MO call

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

(1)	|		| 	MNCC_SETUP_IND	|
	|		*<	==========================	*

(2)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(4)	|		| 	MNCC_ALERT_REQ	|
	|		*	==========================>	*
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Reject Call */

(5)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(6)	|		| 	MNCC_RELEASE_IND	|

|		*	<==========================	*

- COMMAND (MMI CONFIG KEY_SEQUENCE=<01317180537>)

- COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(7)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(8)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(9)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

(10)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(11)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

(12)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
	|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_IND
	ti	TI_MT_9
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_SPEECH
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIG_DIAL_TONE_ON
	calling_party	CLG_PARTY_01317180537
	calling_party_sub	CLG_PARTY_SUB
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB
	redirecting_party	RED_PARTY
	redirecting_party_sub	RED_PARTY_SUB

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_SIG_ONLY

- ACI_CMD_IND
	cmd_len	LM_RING_LEN
	cmd_seq	M_RING

- MNCC_ALERT_REQ
	ti	TI_MT_9

- MNCC_DISCONNECT_REQ
	ti	TI_MT_9
	cause	MNCC_CAUSE_USER_BUSY
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CALL_CLEAR

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_1
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_01317180537
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

History:

08-05-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		add to payload of MNCC_DISCONNECT_REQ

20-12-2002	ef		cause updated MNCC_SYNC_IND

28-01-2003	ef		add parameter “redirecting party” to MNCC_SETUP_IND

14-08-2003	ef		add TCH indication

### MFW526: Missed calls in Idle mode.

Description: .In the idle mode, a MT call is not answered, twice in succession.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

(1)	|		| 	MNCC_SETUP_IND	|
	|		*<	==========================	*

(2)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(4)	|		| 	MNCC_ALERT_REQ	|
	|		*	==========================>	*
TIMEOUT_WAIT (6000)

(5)	|		| 	MNCC_DISCONNECT_IND	|

|		*	<==========================	*

(6)	|		| 	MNCC_RELEASE_IND	|

|		*	<==========================	*

(7)	|		| 	MNCC_SETUP_IND	|
	|		*<	==========================	*

(8)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(9)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(10)	|		| 	MNCC_ALERT_REQ	|
	|		*	==========================>	*
TIMEOUT_WAIT (6000)

(11)	|		| 	MNCC_DISCONNECT_IND	|

|		*	<==========================	*

(12)	|		| 	MNCC_RELEASE_IND	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_IND
	ti	TI_MT_9
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_SPEECH
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIG_DIAL_TONE_ON
	calling_party	CLG_PARTY_01317180537
	calling_party_sub	CLG_PARTY_SUB
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB
	redirecting_party	RED_PARTY
	redirecting_party_sub	RED_PARTY_SUB

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_SIG_ONLY

- ACI_CMD_IND
	cmd_len	LM_RING_LEN
	cmd_seq	M_RING

- MNCC_ALERT_REQ
	ti	TI_MT_9

- MNCC_DISCONNECT_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_UNSPECIFIED
	diagnostic	NOT_PRESENT_8BIT
	progress_desc	PROG_NOT_PRES

- MNCC_RELEASE_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_USER_BUSY

- MNCC_SETUP_IND
	ti	TI_MT_9
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_SPEECH
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIG_DIAL_TONE_ON
	calling_party	CLG_PARTY_01317180537
	calling_party_sub	CLG_PARTY_SUB
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB
	redirecting_party	RED_PARTY
	redirecting_party_sub	RED_PARTY_SUB

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_SIG_ONLY

- ACI_CMD_IND
	cmd_len	LM_RING_LEN
	cmd_seq	M_RING

- MNCC_ALERT_REQ
	ti	TI_MT_9

- MNCC_DISCONNECT_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_UNSPECIFIED
	diagnostic	NOT_PRESENT_8BIT
	progress_desc	PROG_NOT_PRES

- MNCC_RELEASE_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_USER_BUSY

History:

09-05-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

20-12-2002	ef		updated cause in MNCC_SYNC_IND

28-01-2003	ef		add parameter “redirecting party” to MNCC_SETUP_IND

### MFW528: An MT call is rejected.

Description: .During a MO call, an incoming MT call is rejected by the user.

Preamble: 	MFW500

APL/ACI		MMI		PS

|		|		|

(1)	|		| 	MNCC_SETUP_IND	|
	|		*<	==========================	*

(2)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(3)	|		| 	MNCC_ALERT_REQ	|
	|		*	==========================>	*
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=END)

(4)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(5)	|		| 	MNCC_RELEASE_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=END)

(6)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(7)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(8)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_IND
	ti	TI_MT_9
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_SPEECH
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIG_DIAL_TONE_ON
	calling_party	CLG_PARTY_01317180537
	calling_party_sub	CLG_PARTY_SUB
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB
	redirecting_party	RED_PARTY
	redirecting_party_sub	RED_PARTY_SUB

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_SIG_ONLY

- MNCC_ALERT_REQ
	ti	TI_MT_9

- MNCC_DISCONNECT_REQ
	ti	TI_MT_9
	cause	MNCC_CAUSE_USER_BUSY
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_USER_BUSY

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR

History:

09-05-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		add to payload of MNCC_DISCONNECT_REQ

20-12-2002	ef		updated cause in MNCC_SYNC_IND

28-01-2003	ef		add parameter “redirecting party” to MNCC_SETUP_IND

29-01-2003	ef		change cause on 2nd MNCC_DISCONNECT_REQ

### MFW530: In Call Menu, End both calls.

Description: .From the In call Menu, Options,End All. This ends both calls..

Preamble: 	MFW516

APL/ACI		MMI		PS

|		|		|

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* End All */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(1)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(2)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

(3)	|		| 	SIM_SYNC_REQ	|

- |		*	==========================>	*

(4)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (3000)

(5)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_DISCONNECT_REQ
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	TI_MT_9
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CALL_CLEAR

History:

10-05-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		add to payload of MNCC_DISCONNECT_REQ

29-01-2003	ef		change cause in all above signals.

07-07-2003	ef		in call menu changes

### MFW532: MO Emergency Call.

Description: From Idle mode, key in the Emergency phone number and accept. An outgoing call is setup. Afterwards the call is cleared down.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(4)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

(5)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=END)

(6)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(7)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(8)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

TIMEOUT_WAIT (3000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_EMERG_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_112
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_END_TO_END_PLMN
	connected_number	CONNECTED_NUMBER_112
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR

History:

10-05-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		add to payload of MNCC_DISCONNECT_REQ

15-01-2003	ef		add cause to SIM_SYNC_CNF

29-01-2003	ef		change cause for MNCC_DISCONNECT_REQ

14-08-2003	ef		add TCH indication

### MFW534: Cancel a MO call during Setup.

Description: From Idle mode, initiate a MO call, then Cancel the call before it becomes active.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		| 	MNCC_PROGRESS_IND	|

|		*	<==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=END)

(4)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(5)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(6)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

TIMEOUT_WAIT (3000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_PROGRESS_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR

History:

13-05-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		add to payload of MNCC_DISCONNECT_REQ

29-01-2003	ef		change cause of MNCC_DISCONNECT_REQ

### MFW536: Call Independent Supplementary Services, Register for Forward All Calls.

Description: .From Idle, Main Menu, Netw. Services, Divert, All Calls, Voice, Set Divert, and input the Divert number.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Network services */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Divert */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */ 
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* All Calls */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Voice */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Set Divert */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter Number */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=4)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=6)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(1)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(3)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* clear message */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (1000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CFF
	length	NOT_PRESENT_8BIT
	max_length	NUM_2

- SIM_READ_CNF
	datafield	SIM_CPHS_CFF
	cause	SIM_NO_ERROR
	length	NUM_3
	trans_data	SIM_CPHS_FIELD

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	FACILITY_REGISTER
	ss_ver	VERSION_SS

History:

14-05-2002	ef		initial

20-08-2002	ef		add the SIM call forwarding flags request

29-01-2003	ef		change menu for Divert

## MMI Component Tests case : In Call Menu

### MFW538: In Call Menu, make a 2nd call from the Phonebook.

Description: .While a MO call is active, from the In Call Menu, Contacts, access the Phonebook and initiate another call. Afterwards go back to main menu.

Preamble: 	MFW500

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Contacts */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Search */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select 3rd entry */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Call Number */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Call */
(1)	|		| 	MNCC_HOLD_REQ	|

|		*	==========================>	*

(2)	|		| 	MNCC_HOLD_CNF	|

|		*	<==========================	*

(3)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(4)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

(5)	|		| 	MNCC_PROGRESS_IND	|

|		*	<==========================	*

(6)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(7)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	* 
(8)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_HOLD_REQ
	ti	TI_MO_0

- MNCC_HOLD_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_HOLD_SUCCESS

- MNCC_SETUP_REQ
	ti	TI_MO_1
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_ERIC_NT
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_1
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_PROGRESS_IND
	ti	TI_MO_1
	progress_desc	PROG_END_TO_END_PLMN

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_ALERT_IND
	ti	TI_MO_1
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_1
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_654321
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

History:

15-05-2002	ef		initial

20-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

14-08-2003	ef		add TCH indication

### MFW540: In Call Menu, End all calls.

Description: One active MO call, one Held MO call. From the In Call Menu, choose End All.

Preamble: 	MFW538

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* End All */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(1)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(2)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

(3)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(4)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(5)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_DISCONNECT_REQ
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	TI_MO_1
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MO_1
	cause	MNCC_CAUSE_CALL_CLEAR

History:

16-05-2002	ef		initial

20-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		add to payload of MNCC_DISCONNECT_REQ

07-07-07	ef		changes to in call menu

### MFW541: In Call Menu, During an active call, send a text message. End the Call.

Description: One Active MO call. From In call Menu, Send Message, Select.

Enter msg (“Eric”), Select, enter Number, press Send. After msg is sent, discard the msg and end the call.

Note: Pass over the special characters à and á which appear on the ABC button. This will be removed at a later stage.

Preamble: 	MFW458

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Send Message */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Send */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Send */
(1)	|		| 	MNSMS_SUBMIT_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* discard */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* in call menu, End call */
TIMEOUT_WAIT (2000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_SUBMIT_REQ
	mem_type	MEM_SM
	rec_num	NUM_0
	condx	SMS_CONDX_OVR_NON
	modify	SMS_MODIFY_NON
	sms_sdu	SMS_SDU_1

History:

17-05-2002	ef		initial

19-08-2003	ef		skip over the special chars à and á. Remove later.

09-10-2003	ef		remove tha chars à and á.

### MFW542: In Call Menu, MO call held, MT call active, send a text message.

Description: One Active MT call, one Held MO Call. From In Call Menu, Send Message, Select.

Enter msg (“Eric”), Select, enter Number, press Send. After msg is sent, discard the msg.

Note: Pass over the special characters à and á which appear on the ABC button. This will be removed at a later stage.

Preamble: 	MFW514A

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Send Message */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Send */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Send */
(1)	|		| 	MNSMS_SUBMIT_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* discard */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_SUBMIT_REQ
	mem_type	MEM_SM
	rec_num	NUM_0
	condx	SMS_CONDX_OVR_NON
	modify	SMS_MODIFY_NON
	sms_sdu	SMS_SDU_1

History:

21-08-2003	ef		initial

09-10-2003	ef		remove tha chars à and á.

## MMI Component Tests case : Missed Calls

### MFW544: Make a call from the missed call list – Main Menu, Recent Calls, Missed Calls

Description: An incoming call is not answered. Afterwards the Missed Call is accessed from the Main Menu; Recent Calls and the first entry is chosen, a call is setup.

Preamble: 	MFW526

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Recent Calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Missed Calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)
(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

(3)	|		| 	MNCC_PROGRESS_IND	|

|		*	<==========================	*

(4)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(5)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	* 
(6)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
(7)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_ERIC_T
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_PROGRESS_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_654321
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

History:

21-05-2002	ef		initial

20-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

14-08-2003	ef		add TCH indication

### MFW546: Make a call from the missed call list. Left Key to display number, Send to initiate Call.

Description: An incoming call is not answered. The LSK (Read) is pressed to display the number and the Send Key to setup the call.

Preamble: 	MFW526

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)
(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

(3)	|		| 	MNCC_PROGRESS_IND	|

|		*	<==========================	*

(4)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(5)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	* 
(6)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
(7)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*
TIMEOUT_WAIT (4000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_ERIC_T
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_PROGRESS_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_654321
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

History:

22-05-2002	ef		initial

20-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

14-08-2003	ef		add TCH indiaction

### MFW547: From Main Menu, Recent Calls, Missed Calls, store callers number then send SMS text message.

Description: An incoming call is not answered. Afterwards the Missed Call is accessed from the Main Menu, Recent Calls and the number is stored from the Options menu. The idle screen is then returned to, the Missed Call accessed again through the Main Menu and Recent Calls and an SMS created and sent from the Options menu.

Note1: Pass over the special characters à and á which appear on the ABC button. This will be removed at a later stage.

Note2: When the test finishes a reserved address is accessed which causes a windows error. However just leave it, test passes ok

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

(1)	|		| 	MNCC_SETUP_IND	|
	|		*<	==========================	*

(2)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(4)	|		| 	MNCC_ALERT_REQ	|
	|		*	==========================>	*
TIMEOUT_WAIT (6000)

(5)	|		| 	MNCC_DISCONNECT_IND	|

|		*	<==========================	*

(6)	|		| 	MNCC_RELEASE_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Main Menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* recent calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* missed calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* store number */
TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)		/* enter a name */

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=4)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=2)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(7)	|		| 	SIM_UPDATE_RECORD_REQ	|

|			*==========================>*
TIMEOUT_WAIT (3000)
(8)	|		| 	SIM_UPDATE_RECORD_CNF	|

|		*<	==========================	*

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* main menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* recent calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* send message */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)		/* enter text */

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=4)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=2)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Send */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(9)	|		| 	MNSMS_SUBMIT_REQ	|

|			*==========================>*
(10)	|		| 	MNSMS_SUBMIT_CNF	|

|			*<==========================*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_IND
	ti	TI_MT_9
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_SPEECH
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIG_DIAL_TONE_ON
	calling_party	CLG_PARTY_01317180537
	calling_party_sub	CLG_PARTY_SUB
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB
	redirecting_party	RED_PARTY
	redirecting_party_sub	RED_PARTY_SUB

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_SIG_ONLY

- ACI_CMD_IND
	cmd_len	LM_RING_LEN
	cmd_seq	M_RING

- MNCC_ALERT_REQ
	ti	TI_MT_9

- MNCC_DISCONNECT_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_UNSPECIFIED
	diagnostic	NOT_PRESENT_8BIT
	progress_desc	PROG_NOT_PRES

- MNCC_RELEASE_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_USER_BUSY

- SIM_UPDATE_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_7
	length	PHB_LENGTH_ADN
 	linear_data	PHB_ADN_RECORD_ERIC_MT_SAVE

- SIM_UPDATE_RECORD_CNF
	datafield	SIM_ADN
	record	PHB_RECORD_7
	cause	SIM_NO_ERROR

- MNSMS_SUBMIT_REQ
	mem_type		MEM_SM
	rec_num		NUM_0
	condx		SMS_CONDX_OVR_NON
	modify		SMS_MODIFY_NON
	sms_sdu		SMS_SDU_MISSED_CALLS

- MNSMS_SUBMIT_CNF
	mem_type	MEM_SM
	rec_num	SMS_RECORD_NOT_EXIST
	cause	SMS_NO_ERROR
	tp_mr	NUM_2
	sms_sdu	SMS_SDU_EMPTY

History:

23-06-2003	dr		initial

08-07-2003	ef		initialise the SMS parameters first ie new preamble

19-08-2003	ef		skip over the special chars à and á. Remove later.

09-10-2003	ef		remove tha chars à and á.

## MMI Component Tests case : Main Menu, Messages, Voicemail, Set Voicemail

### MFW548: Set the Voicemail options (number), then call Voicemail.

Description: .Main Menu, Messages, VoiceMail, Set VoiceMail, enter voicemail number (121) and accept, Menu up 1 and call Voicemail to setup the call. Clear down the call afterwards.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Main Menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Messages */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Voice Mail */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Set Voice Mail */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter Number */

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=UP)		/* Voice Mail */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Calling 121 */
(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*
(3)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=END)
(4)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(5)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(6)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_VOICEMBX
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_654321
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR

History:

22-05-2002	ef		initial

20-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		add to payload of MNCC_DISCONNECT_REQ

20-12-2002	ef		update the cause value for MNCC_DISCONNECT_REQ

26-09-2003	ef		SIM_SYNC_REQ after the Setup Req

### MFW550: Call Voicemail using the “1” key.

Description: .Using the hot key “1”, long press, and setup the voice call. Clear the call afterwards..

Preamble: 	MFW548

APL/ACI		MMI		PS

|		|		|

TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_PRESS=1)
TIMEOUT_WAIT (2000)
(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*
(3)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=END)
(4)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(5)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(6)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_VOICEMBX
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_654321
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR

History:

22-05-2002	ef		initial

21-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		add to payload of MNCC_DISCONNECT_REQ

20-12-2002	ef		update the cause value for MNCC_DISCONNECT_REQ

26-09-2003	ef		SIM_SYNC_REQ comes after Setup request

## Mmi Component Tests case : Main Menu, Phone Settings, set Auto Redial On

### MFW552: Set the Auto Redial On .

Description: .From the Main Menu, Settings, Auto Redial, Set ON. Back to Idle Screen.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Phone Settings */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Auto Redial */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* On */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

History:

22-05-2002	ef		initial

29-01-2003	ef		change menu structure

### MFW554: Auto Redial when called party is busy.

Description: .A MO call is setup and is rejected by the Network. Because auto redial is “on”, a 2nd attemp is automatically made.

Preamble: 	MFW552

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		| 	MNCC_DISCONNECT_IND	|

|		*	<==========================	*

(4)	|		| 	MNCC_RELEASE_CNF	|

|		*	<==========================	*

(5)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(6)	|		| 	SIM_SYNC_CNF	|

|		*	<==========================	*

(7)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(8)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(9)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

(10)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(11)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

(12)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
(13)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (10000)
	|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_DISCONNECT_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_USER_BUSY
	diagnostic	NOT_PRESENT_8BIT
	progress_desc	PROG_NOT_PRES

- MNCC_RELEASE_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_USER_BUSY

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_01317180537
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

History:

22-05-2002	ef		initial

21-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

15-01-2003	ef		add cause to SIM_SYNC_CNF

14-08-2003	ef		add TCH indication

### MFW556: Override Auto Redial by calling immediately, then Cancel re-dial.

Description: .A MO call is setup and is rejected by the Network. Because auto redial is “on”, a 2nd attempt is automatically made. However the re-attempt is initiated immediately. This is then cancelled and a return to Idle.

Preamble: 	MFW552

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (3000)
(3)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

(4)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

(5)	|		| 	MNCC_DISCONNECT_IND	|

|		*	<==========================	*

(6)	|		| 	MNCC_RELEASE_CNF	|

|		*	<==========================	*

(7)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(8)	|		| 	SIM_SYNC_CNF	|

|		*	<==========================	*

- COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(9)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(10)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(11)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

(12)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

(13)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(14)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(15)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_DISCONNECT_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_USER_BUSY
	diagnostic	NOT_PRESENT_8BIT
	progress_desc	PROG_NOT_PRES

- MNCC_RELEASE_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_USER_BUSY

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR

History:

23-05-2002	ef		initial

21-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		add to payload of MNCC_DISCONNECT_REQ

20-12-2002	ef		update the cause value for MNCC_DISCONNECT_REQ

15-01-2003	ef		add cause to SIM_SYNC_CNF

### MFW558: During a call, increase / decrease the Volume.

Description: .The Volume up and Vol down keys are pressed during a MO call. However the low level calls to the Drivers are simulated.

Preamble: 	MFW500

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=VOL_PLUS)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE= VOL_PLUS)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE= VOL_PLUS)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=VOL_MINUS)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE= VOL_MINUS)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE= VOL_MINUS)

|		|		|

- Parametrization:

Primitive	Parameter	Value

History:

23-05-2002	ef		initial

## MMI Component Tests case : Main Menu, Phone Settings, Set Any Key Answer On

### MFW560: Set the Any Key Answer On. Accept a MT call by any key.

Description: .From the Main Menu, Phone Settings, Any key answer, Set ON. Back to Idle Screen. A MT call is accepted by the “7” key.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Phone Settings */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
 TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Any Key Answer */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* On */
TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Back to Idle */
TIMEOUT_WAIT (4000)
(1)	|		| 	MMI_RXLEV_IND	|
	|		*<	==========================	*

(2)	|		| 	GMMREG_CIPHERING_IND	|
	|		*<	==========================	*

(3)	|		| 	MNCC_SETUP_IND	|
	|		*<	==========================	*

(4)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(6)	|		| 	MNCC_ALERT_REQ	|
	|		*	==========================>	*
(7)	|		| 	MMI_RXLEV_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

(8)	|		| 	MNCC_SETUP_RES	|
	|		*	==========================>	*
(9)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(10)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(11)	|		| 	MNCC_SETUP_COMPL_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (4000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MMI_RXLEV_IND
	rxlev	NUM_8

- GMMREG_CIPHERING_IND

- gsm_ciph	CIPH_ON
	gprs_ciph	CIPH_ON

- MNCC_SETUP_IND
	ti	TI_MT_9
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_SPEECH
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIG_DIAL_TONE_ON
	calling_party	CLG_PARTY_01317180537
	calling_party_sub	CLG_PARTY_SUB
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB
	redirecting_party	RED_PARTY
	redirecting_party_sub	RED_PARTY_SUB

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_SIG_ONLY

- ACI_CMD_IND
	cmd_len	LM_RING_LEN
	cmd_seq	M_RING

- MNCC_ALERT_REQ
	ti	TI_MT_9

- MMI_RXLEV_IND
	rxlev	NUM_8

- MNCC_SETUP_RES
	ti	TI_MT_9

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_SETUP_COMPL_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_SUCCESS

- History:

24-05-2002	ef		initial

22-08-2002	ef		remove MMI_SPEECH_MODE_REQ signal

14-11-2002	ef		change the MNCC_SETUP_IND payload

20-12-2002	ef		updated cause in MNCC_SYNC_IND

14-08-2003	ef		add TCH indication

20-08-2003	ef		correct the menu for any key answer

## MMI Component Tests case : Conference Call

### MFW562: Start a Multiparty Call  from the in-calls option Menu, then Split into a private call. Finally Retrieve the call.

Description: One active MO call, one Held MO call. From the in-calls option menu, choose Conference. Both calls are now Active.

After a short delay the Conference call is split by Selecting the In Call screen and the first option PRIVATE. The 2nd call is now on Hold.

After a short delay, from the in-calls options menu, the Private call is then Retrieved, by the SWAP option.

Note: This test case will need all the menu Down keys deleted after “Conference” is added to the options menu.

Preamble: 	MFW538

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Conference */

(1)	|		| 	MNCC_FACILITY_REQ	|

|		*	==========================>	*

(2)	|		| 	MNCC_SYNC_REQ	|

|		*	==========================>	*

(3)	|		| 	MNCC_FACILITY_IND	|

- |		*	<==========================	*

(4)	|		| 	MNCC_SYNC_REQ	|

|		*	==========================>	*

- TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Private */

(5)	|		| 	MNCC_FACILITY_REQ	|

|		*	==========================>	*

(6)	|		| 	MNCC_SYNC_REQ	|

|		*	==========================>	*

(7)	|		| 	MNCC_FACILITY_IND	|

- |		*	<==========================	*

(8)	|		| 	MNCC_SYNC_REQ	|

|		*	==========================>	*

- TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Swap */

(9)	|		| 	MNCC_HOLD_REQ	|

|		*	==========================>	*

(10)	|		| 	MNCC_HOLD_CNF	|

- |		*	<==========================	*

(11)	|		| 	MNCC_FACILITY_REQ	|

|		*	==========================>	*

(12)	|		| 	MNCC_SYNC_REQ	|

|		*	==========================>	*

(13)	|		| 	MNCC_FACILITY_IND	|

- |		*	<==========================	*

(14)	|		| 	MNCC_SYNC_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_FACILITY_REQ
	ti	TI_MO_1
	fac_inf	FAC_MULTIPARTY
	ss_version	NOT_USED

- MNCC_SYNC_REQ
	synccs	SYNCCS_MPTY_EVENT
	ti	TI_MO_1
	mpty_event	MPTY_BUILD_SENT

- MNCC_FACILITY_IND
	ti	TI_MO_1
	fac_context	FAC_IN_FACILITY
	fac_inf	MULTIPARTY_RETURN_RESULT

- MNCC_SYNC_REQ
	synccs	SYNCCS_MPTY_EVENT
	ti	TI_MO_1
	mpty_event	MPTY_BUILD_SUCCESS

- MNCC_FACILITY_REQ
	ti	TI_MO_1
	fac_inf	FAC_MULTIPARTY_SPLIT
	ss_version	NOT_USED

- MNCC_SYNC_REQ
	synccs	SYNCCS_MPTY_EVENT
	ti	TI_MO_1
	mpty_event	MPTY_SPLIT_SENT

- MNCC_FACILITY_IND
	ti	TI_MO_1
	fac_context	FAC_IN_FACILITY
	fac_inf	MULTIPARTY_SPLIT_RETURN_RESULT

- MNCC_SYNC_REQ
	synccs	SYNCCS_MPTY_EVENT
	ti	TI_MO_1
	mpty_event	MPTY_SPLIT_SUCCESS

- MNCC_HOLD_REQ
	ti	TI_MO_1

- MNCC_HOLD_CNF
	ti	TI_MO_1
	cause	MNCC_CAUSE_HOLD_SUCCESS

- MNCC_FACILITY_REQ
	ti	TI_MO_0
	fac_inf	FAC_MULTIPARTY_RETRIEVE
	ss_version	NOT_USED

- MNCC_SYNC_REQ
	synccs	SYNCCS_MPTY_EVENT
	ti	TI_MO_0
	mpty_event	MPTY_RETRIEVE_SENT

- MNCC_FACILITY_IND
	ti	TI_MO_0
	fac_context	FAC_IN_FACILITY
	fac_inf	MULTIPARTY_RETRIEVE_RETURN_RESULT

- MNCC_SYNC_REQ
	synccs	SYNCCS_MPTY_EVENT
	ti	TI_MO_0
	mpty_event	MPTY_RETRIEVE_SUCCESS

History:

14-05-2003	ef		initial

08-07-2003	ef		change to in call menu

## MMI Component Tests case : Advice of Charge

### MFW564: Mobile Originated Call with Forward Advice of Charge Information

Description:. An outgoing call is setup and the network sends AOCI information to the mobile.

Preamble: 	MFW490

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

(4)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

(5)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(6)	|		| 	MNCC_FACILITY_IND	|

- |		*	<==========================	*

TIMEOUT_WAIT (2000)

(7)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

(8)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (3000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_FACILITY_IND
	ti	TI_MO_0
	fac_context	FAC_IN_CONNECT
	fac_inf	FAC_AOCI

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_01317180537
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

History:

17-04-2002	ef		initial
	03-05-2002	ef		Add Alert Indication and Call Proceed Indication

13-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

14-08-2003	ef		add TCH indication

## MMI Component Tests case : Main Menu, Phone Settings, set Auto Answer On

### MFW566: Set the Auto Answer On. Incoming MT call is connected.

Description: From the idle screen, Menu, Phone Settings, Set Any Auto Answer On. An incoming MT call is indicated and accepted automatically.

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Phone Settings */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Auto Answer */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* On */
TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Back to Idle */
TIMEOUT_WAIT (4000)

(1)	|		| 	MMI_RXLEV_IND	|
	|		*<	==========================	*

(2)	|		| 	GMMREG_CIPHERING_IND	|
	|		*<	==========================	*

(3)	|		| 	MNCC_SETUP_IND	|
	|		*<	==========================	*

(4)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(6)	|		| 	MNCC_ALERT_REQ	|
	|		*	==========================>	*
(7)	|		| 	MMI_RXLEV_IND	|
	|		*<	==========================	*

(8)	|		| 	MNCC_SETUP_RES	|
	|		*	==========================>	*
(9)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(10)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(11)	|		| 	MNCC_SETUP_COMPL_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (4000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MMI_RXLEV_IND
	rxlev	NUM_8

- GMMREG_CIPHERING_IND

- gsm_ciph	CIPH_ON
	gprs_ciph	CIPH_ON

- MNCC_SETUP_IND
	ti	TI_MT_9
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_SPEECH
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIG_DIAL_TONE_ON
	calling_party	CLG_PARTY_01317180537
	calling_party_sub	CLG_PARTY_SUB
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB
	redirecting_party	RED_PARTY
	redirecting_party_sub	RED_PARTY_SUB

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_SIG_ONLY

- ACI_CMD_IND
	cmd_len	LM_RING_LEN
	cmd_seq	M_RING

- MNCC_ALERT_REQ
	ti	TI_MT_9

- MMI_RXLEV_IND
	rxlev	NUM_8

- MNCC_SETUP_RES
	ti	TI_MT_9

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_SETUP_COMPL_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_SUCCESS

History:

20-08-2003	ef		initial

### MFW568: Main Menu, Phone Settings, Change the Language.

Description: From the idle screen, Menu, Phone Settings, Language, change to German.

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Phone Settings */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Language */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* German */
TIMEOUT_WAIT (2000)
 COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Back to Idle */
TIMEOUT_WAIT (4000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

History:

29-09-2003	ef		initial

## MMI Component Tests case : Keypad Lock

### MFW570: Keypad Locked, make an Emergency Call.

Description: From Idle mode, lock keypad then key in the Emergency phone number and accept. An outgoing call is setup. Afterwards the call is cleared down.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (500)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)		/* keypad lock */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(4)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

(5)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=END)

(6)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(7)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(8)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

TIMEOUT_WAIT (3000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_EMERG_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_112
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_MODE_SPEECH_TCH_F

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_END_TO_END_PLMN
	connected_number	CONNECTED_NUMBER_112
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR

History:

29-09-2003	ef		initial

### MFW572: Keypad Locked, make a MO Call.

Description: From Idle mode, lock keypad then key in the phone number and accept. No call is set up

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (500)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)		/* keypad lock */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=<01317180537>)

COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

TIMEOUT_WAIT (3000)

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

30-09-2003	ef		initial

## MMI Component Tests - case : GSM Command Strings. Supplementary services: Call Divert.

### MFW580: SS GSM Command String, enter string to Call Forward(Diversion) All Calls.

Description: .Setup the Register and Activate string.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

29-05-2002	ef		initial

### MFW582: SS GSM Command String, enter string to Call Forward(Diversion) If Busy.

Description: . Setup the Register and Activate string.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

29-05-2002	ef		initial

### MFW584: SS GSM Command String, enter string to Call Forward(Diversion) If No Reply.

Description: . Setup the Register and Activate string.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

29-05-2002	ef		initial

### MFW586: SS GSM Command String, enter string to Call Forward(Diversion) If Not Reachable.

Description: . Setup the Register and Activate string.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

29-05-2002	ef		initial

### MFW588: SS GSM Command String, enter string to Call Forward(Diversion) All configured call divertions.

Description: . Setup the Register and Activate string.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

29-05-2002	ef		initial

### MFW590: SS GSM Command String, enter string to Call Forward(Diversion) All Diverts to be activated.

Description: . Setup the Register and Activate string.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

29-05-2002	ef		initial

### MFW592: SS GSM Command String, enter string to Call Forward(Diversion) Unconditionally divert all calls.

Description: . Setup the Register and Activate string.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

29-05-2002	ef		initial

### MFW600: SS GSM Command String, Call Forward(Diversion). Enter Diverting number.

Description: .As part of the GSM command string, enter the number to Divert to.

A: 	All Calls

B: 	If Busy

C: 	If no reply

D: 	If not reachable

E: 	All configured Call Divertions

F: 	All Diverts to be activated

G: 	Unconditionally divert all calls

Variants:		<A>...<G>

Preamble: 	<A>MFW580
<B>MFW582
<C>MFW584
<D>MFW586
<E>MFW588
<F>MFW590
<G>MFW592

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

30-05-2002	ef		initial

### MFW602: SS GSM Command String, Register various Call Forward(Diversions) for Voice.

Description: .Send a Call Independent Supervisory Service, for Call Forwarding.forVoice for various conditions ie

A: 	All Calls

B:	If Busy

C: 	If no reply

D: 	If not reachable

E: 	All conditional Call Divertions

F:	All Diverts to be activated

G: 	Unconditionally divert all calls

Variants:		<A>...<G>

Preamble: 	<A>MFW600A
<B>MFW600B
<C>MFW600C
<D>MFW600D
<E>MFW600E
<F>MFW600F
<G>MFW600G

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_DIVERT_ALLCALLS_V
<B>	fac_inf	FAC_DIVERT_IFBUSY_V
<C>	fac_inf	FAC_DIVERT_IFNOREPLY_V
<D>	fac_inf	FAC_DIVERT_IFNOTREACHABLE_V
<E>	fac_inf	FAC_DIVERT_ALLCONFIGURED_V
<F>	fac_inf	FAC_DIVERT_ALLDIVERTS_V
<G>	fac_inf	FAC_DIVERT_UNCONDITIONALLY_V
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

30-05-2002	ef		initial

04-06-2003	ef		change for voice

### MFW650: SS GSM Command String, Register various Call Forward(Diversions) for FAX.

Description: .Send a Call Independent Supervisory Service, for Call Forwarding.forFAX for various conditions ie.

A: 	All Calls

B:	If Busy

C: 	If no reply

D: 	If not reachable

E: 	All conditional Call Divertions

F:	All Diverts to be activated

G: 	Unconditionally divert all calls

Variants:		<A>...<G>

Preamble: 	<A>MFW600A
<B>MFW600B
<C>MFW600C
<D>MFW600D
<E>MFW600E
<F>MFW600F
<G>MFW600G

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_DIVERT_ALLCALLS_F
<B>	fac_inf	FAC_DIVERT_IFBUSY_F
<C>	fac_inf	FAC_DIVERT_IFNOREPLY_F
<D>	fac_inf	FAC_DIVERT_IFNOTREACHABLE_F
<E>	fac_inf	FAC_DIVERT_ALLCONFIGURED_F
<F>	fac_inf	FAC_DIVERT_ALLDIVERTS_F
<G>	fac_inf	FAC_DIVERT_UNCONDITIONALLY_F
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

30-05-2002	ef		initial

04-06-2003	ef		change for fax

### MFW652: SS GSM Command String, Register various Call Forward(Diversions) for Data.

Description: .Send a Call Independent Supervisory Service, for Call Forwarding.forData for various conditions ie.

A: 	All Calls

B:	If Busy

C: 	If no reply

D: 	If not reachable

E: 	All conditional Call Divertions

F:	All Diverts to be activated

G: 	Unconditionally divert all calls

Variants:		<A>...<G>

Preamble: 	<A>MFW600A
<B>MFW600B
<C>MFW600C
<D>MFW600D
<E>MFW600E
<F>MFW600F
<G>MFW600G

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_DIVERT_ALLCALLS_D
<B>	fac_inf	FAC_DIVERT_IFBUSY_D
<C>	fac_inf	FAC_DIVERT_IFNOREPLY_D
<D>	fac_inf	FAC_DIVERT_IFNOTREACHABLE_D
<E>	fac_inf	FAC_DIVERT_ALLCONFIGURED_D
<F>	fac_inf	FAC_DIVERT_ALLDIVERTS_D
<G>	fac_inf	FAC_DIVERT_UNCONDITIONALLY_D
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

30-05-2002	ef		initial

04-06-2003	ef		change for data

### MFW654: SS GSM Command String, enter string to De-Register All Calls.

Description: .setup the deativate string.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

06-06-2003	ef		initial

### MFW655: SS GSM Command String, enter string to De-Register All Calls Unconditionally.

Description: .Setup the deativate string.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

06-06-2003	ef		initial

### MFW656: SS GSM Command String, enter string to De-Register if Busy.

Description: .Setup the deativate string.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

06-06-2003	ef		initial

### MFW657: SS GSM Command String, enter string to De-Register if No Reply.

Description: .Setup the deativate string.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

06-06-2003	ef		initial

### MFW658: SS GSM Command String, enter string to De-Register all Configured calls.

Description: .Setup the deativate string.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

06-06-2003	ef		initial

### MFW659: SS GSM Command String, enter string to De-Register if Not Reachable.

Description: .Setup the deativate string.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

06-06-2003	ef		initial

### MFW660: SS GSM Command String, enter string for Voice.

Description: .Setup string, for Erasing Call Forwarding.for Voice for various conditions ie.

A: 	All Calls

B:	All Unconditional calls

C: 	If busy calls

D: 	If no Reply

E: 	All Configured calls

F:	All not Reachable

Variants:		<A>...<F>

Preamble: 	<A>MFW654
<B>MFW655
<C>MFW656
<D>MFW657
<E>MFW658
<F>MFW659

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

06-06-2003	ef		initial

### MFW662: SS GSM Command String, enter string for Fax.

Description: .Setup string, for Erasing Call Forwarding.for FAX for various conditions ie.

A: 	All Calls

B:	All Unconditional calls

C: 	If busy calls

D: 	If no Reply

E: 	All Configured calls

F:	All not Reachable

Variants:		<A>...<F>

Preamble: 	<A>MFW654
<B>MFW655
<C>MFW656
<D>MFW657
<E>MFW658
<F>MFW659

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

06-06-2003	ef		initial

### MFW664: SS GSM Command String, enter string for Data.

Description: .Setup string, for Erasing Call Forwarding.for Data for various conditions ie.

A: 	All Calls

B:	All Unconditional calls

C: 	If busy calls

D: 	If no Reply

E: 	All Configured calls

F:	All not Reachable

Variants:		<A>...<F>

Preamble: 	<A>MFW654
<B>MFW655
<C>MFW656
<D>MFW657
<E>MFW658
<F>MFW659

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- History:

06-06-2003	ef		initial

### MFW666: SS GSM Command String, De-register various Call Forward(Diversions) for Voice, FAX and Data.

Description: .Send a Call Independent Supervisory Service, for Erasing Call Forwarding.for various conditions ie.

VOICE	A-F

A: 	All Calls

B: 	All unconditional Call Divertions

C:	If Busy

D: 	If no reply

E: 	All configured Call Divertions

F: 	If not reachable

FAX 	G-L

DATA	M-R

Variants:		<A>...<G>

Preamble: 	<A>MFW660A
<B>MFW660B
<C>MFW660C
<D>MFW660D
<E>MFW660E
<F>MFW660F
<G>MFW662A
<H>MFW662B
<I>MFW662C
<J>MFW662D
<K>MFW662E
<L>MFW662F
<M>MFW664A
<N>MFW664B
<O>MFW664C
<P>MFW664D
<Q>MFW664E
<R>MFW664F

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_ERASE_ALLDIVERTS_V
<B>	fac_inf	FAC_ERASE_UNCONDITIONALLY_V
<C>	fac_inf	FAC_ERASE_IFBUSY_V
<D>	fac_inf	FAC_ERASE_IFNOREPLY_V
<E>	fac_inf	FAC_ERASE_ALLCONFIGURED_V
<F>	fac_inf	FAC_ERASE_IFNOTREACHABLE_V
<G>	fac_inf	FAC_ERASE_ALLDIVERTS_F
<H>	fac_inf	FAC_ERASE_UNCONDITIONALLY_F
<I>	fac_inf	FAC_ERASE_IFBUSY_F
<J>	fac_inf	FAC_ERASE_IFNOREPLY_F
<K>	fac_inf	FAC_ERASE_ALLCONFIGURED_F
<L>	fac_inf	FAC_ERASE_IFNOTREACHABLE_F
<M>	fac_inf	FAC_ERASE_ALLDIVERTS_D
<N>	fac_inf	FAC_ERASE_UNCONDITIONALLY_D
<O>	fac_inf	FAC_ERASE_IFBUSY_D
<P>	fac_inf	FAC_ERASE_IFNOREPLY_D
<Q>	fac_inf	FAC_ERASE_ALLCONFIGURED_D
<R>	fac_inf	FAC_ERASE_IFNOTREACHABLE_D
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

30-05-2002	ef		initial

04-06-2003	ef		change for data

### MFW604: SS GSM Command String, Register and Activate Call Forward(Diversion) for Voice, Fax and Data.

Description: .Send a Call Independent Supervisory Service, for Call Forwarding. The Network responds by sending Activated containing a Release Complete message which is passed to the MMI.

A: 	All Calls

B: 	If Busy

C: 	If no reply

D: 	If not reachable

E: 	All configured Call Divertions

F: 	All Diverts to be activated

G: 	Unconditionally divert all calls

Variants:		<A>...<G>

Preamble: 	<A>MFW600A
<B>MFW600B
<C>MFW600C
<D>MFW600D
<E>MFW600E
<F>MFW600F
<G>MFW600G

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_DIVERT_ALLCALLS
<B>	fac_inf	FAC_DIVERT_IFBUSY
<C>	fac_inf	FAC_DIVERT_IFNOREPLY
<D>	fac_inf	FAC_DIVERT_IFNOTREACHABLE
<E>	fac_inf	FAC_DIVERT_ALLCONFIGURED
<F>	fac_inf	FAC_DIVERT_ALLDIVERTS
<G>	fac_inf	FAC_DIVERT_UNCONDITIONALLY
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

11-06-20002	ef		initial

29-01-2003	ef		change cause in MNCC_END_IND

23-04-2003	ef		update the facility msg from the network

### MFW606: SS GSM Command String, Deactivate Call Forward(Diversion) for Voice, Fax and Data.

Description: .Send a Call Independent Supervisory Service, for Deactivating Call Forwarding. A Release Complete from the network is passed to MMI.

Preamble: 	MFW604A

APL/ACI		MMI		PS

|		|		|

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	FAC_DEACTIVATE_ALLCALLS
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	RELEASE_COMPLETE_DEACTIVATE

History:

13-06-20002	ef		initial

29-01-2003	ef		change cause in MNCC_END_IND

23-04-2003	ef		add a delay to ensure network response window is deleted

### MFW608: SS GSM Command String, Activate Call Forward(Diversion) All Calls.

Description: .Send a Call Independent Supervisory Service, to Activate Call Forwarding(Divert) All Calls. Note that Activate and Register and Activate produce the same result.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=*)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=#)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	FAC_DIVERT_ALLCALLS
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
 	cause	MNSS_CAUSE_CALL_CLEAR
 	fac_inf	RELEASE_COMPLETE_FACILITY

History:

19-06-20002	ef		initial

29-01-2003	ef		change cause in MNCC_END_IND

## MMI Component Tests - case :Main Menu, Network Services, Call Divert.

### MFW610: Main Menu; Network services, Divert; All Calls.

Description: Common script for Call Divert – All Calls

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Network Services */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* All Calls */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

Parametrization:

Primitive	Parameter	Value

History:

26-06-2003	dr		initial

### MFW611: Main Menu; Network services, Divert; If Busy.

Description: Common script for Call Divert – If Busy

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Network services */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* If Busy */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

Parametrization:

Primitive	Parameter	Value

History:

26-06-2003	dr		initial

### MFW612: Main Menu; Network services, Divert; If No Answer.

Description: Common script for Call Divert – No Reply

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Network Services */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* If No Answer */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

Parametrization:

Primitive	Parameter	Value

History:

26-06-2003	dr		initial

### MFW613: Main Menu; Network services, Divert; If Not Reachable.

Description: Common script for Call Divert – Not Reachable

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Network Services */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* If Not Reachable */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

Parametrization:

Primitive	Parameter	Value

History:

26-06-2003	dr		initial

### MFW614: Main Menu; Network services, Divert; If No Service.

Description: Common script for Call Divert – No Service

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Network Services */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* If No Service */

TIMEOUT_WAIT (1000)

Parametrization:

Primitive	Parameter	Value

History:

26-06-2003	dr		initial

### MFW615: Main Menu; Network services, Call Divert for Voice

Description: Set type of call divert through preamble, then enter number and register divert. Finally go back to main menu.

A: 	All Calls

B: 	If Busy

C: 	If no reply

D: 	If no service

Variants:		<A>...<D>

Preamble: 	<A>MFW610
<B>MFW611
<C>MFW612
<D>MFW614

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Voice */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Set Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (6000)

(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* close confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_DIVERT_ALLCALLS_V
<B>	fac_inf	FAC_DIVERT_IFBUSY_V
<C>	fac_inf	FAC_DIVERT_IFNOREPLY_V
<D>	fac_inf	FAC_DIVERT_IFNOSERVICE_V
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
 	cause	MNSS_CAUSE_CALL_CLEAR
 	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

### MFW616: Main Menu; Network services, Call Divert for Fax

Description: Set type of call divert through preamble, then enter number and register divert. Finally go back to main menu.

A: 	All Calls

B: 	If Busy

C: 	If no reply

D: 	If no service

Variants:		<A>...<D>

Preamble: 	<A>MFW610
<B>MFW611
<C>MFW612
<D>MFW614

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* FAX */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Set Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (6000)

(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* close confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_DIVERT_ALLCALLS_F
<B>	fac_inf	FAC_DIVERT_IFBUSY_F
<C>	fac_inf	FAC_DIVERT_IFNOREPLY_F
<D>	fac_inf	FAC_DIVERT_IFNOSERVICE_F
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
 	cause	MNSS_CAUSE_CALL_CLEAR
 	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

### MFW617: Main Menu; Network services, Call Divert for Data

Description: Set type of call divert through preamble, then enter number and register divert. Finally go back to main menu.

A: 	All Calls

B: 	If Busy

C: 	If no reply

D: 	If no service

Variants:		<A>...<D>

Preamble: 	<A>MFW610
<B>MFW611
<C>MFW612
<D>MFW614

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Data */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Set Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Close Confirmation OK*/

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_DIVERT_ALLCALLS_D
<B>	fac_inf	FAC_DIVERT_IFBUSY_D
<C>	fac_inf	FAC_DIVERT_IFNOREPLY_D
<D>	fac_inf	FAC_DIVERT_IFNOSERVICE_D
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
 	cause	MNSS_CAUSE_CALL_CLEAR
 	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

### MFW618: Main Menu; Network services, Call Divert for Voice if Not Reachable

Description: Set type of call divert through preamble, then enter number and register divert. Finally go back to main menu

Note: The Menu item Not Reachable actually is the SS code for All Conditional. Depends what your definition of not reachable is.

Preamble: 	MFW613

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Voice */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Set Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (6000)

(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	FAC_DIVERT_ALL_CONDITIONAL_V
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
 	cause	MNSS_CAUSE_CALL_CLEAR
 	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

19-08-2003	ef		from menu, not reachable is actually “all conditional”

### MFW619: Main Menu; Network services, Call Divert for Fax if Not Reachable

Description: Set type of call divert through preamble, then enter number and register divert. Finally go back to main menu.

Note: The Menu item Not Reachable actually is the SS code for All Conditional. Depends what your definition of not reachable is.

Preamble: 	MFW613

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Fax */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Set Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (6000)

(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	FAC_DIVERT_ALL_CONDITIONAL_F
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
 	cause	MNSS_CAUSE_CALL_CLEAR
 	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

19-08-2003	ef		from menu, not reachable is actually “all conditional”

### MFW620: Main Menu; Network services, Call Divert for Data if Not Reachable

Description: Set type of call divert through preamble, then enter number and register divert. Finally go back to main menu.

Note: The Menu item Not Reachable actually is the SS code for All Conditional. Depends what your definition of not reachable is.

Preamble: 	MFW613

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Data */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Set Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* OK */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (6000)

(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	FAC_DIVERT_ALL_CONDITIONAL_D
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
 	cause	MNSS_CAUSE_CALL_CLEAR
 	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

19-08-2003	ef		from menu, not reachable is actually “all conditional”

### MFW621: Main Menu; Network Services, Call Divert De-activation for Voice.

Description: Set type of call divert through preamble, then de-register divert. Finally go back to main menu.

A: 	All Calls

B:	If Busy

C: 	If no reply

D: 	If no service

Variants:		<A>...<D>

Preamble: 	<A>MFW610
<B>MFW611
<C>MFW612
<D>MFW614

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Voice */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Cancel Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_ERASE_UNCONDITIONALLY_V _V
<B>	fac_inf	FAC_ERASE_IFBUSY_V
<C>	fac_inf	FAC_ERASE_IFNOREPLY_V
<D>	fac_inf	FAC_ERASE_IFNOSERVICE_V
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

### MFW622: Main Menu; Network Services, Call Divert De-activation for Fax.

Description: Set type of call divert through preamble, then de-register divert. Finally go back to main menu.

A: 	All Calls

B:	If Busy

C: 	If no reply

D: 	If no service

Variants:		<A>...<D>

Preamble: 	<A>MFW610
<B>MFW611
<C>MFW612
<D>MFW614

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* FAX */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Cancel Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_ERASE_UNCONDITIONALLY_F
<B>	fac_inf	FAC_ERASE_IFBUSY_F
<C>	fac_inf	FAC_ERASE_IFNOREPLY_F
<D>	fac_inf	FAC_ERASE_IFNOSERVICE_F
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

### MFW623: Main Menu; Network Services, Call Divert De-activation for Data.

Description: Set type of call divert through preamble, then de-register divert. Finally go back to main menu.

A: 	All Calls

B:	If Busy

C: 	If no reply

D: 	If no service

Variants:		<A>...<D>

Preamble: 	<A>MFW610
<B>MFW611
<C>MFW612
<D>MFW614

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Data */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Cancel Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_ERASE_UNCONDITIONALLY_D
<B>	fac_inf	FAC_ERASE_IFBUSY_D
<C>	fac_inf	FAC_ERASE_IFNOREPLY_D
<D>	fac_inf	FAC_ERASE_IFNOSERVICE_D
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

### MFW624: Main Menu; Network Services, Call Divert If Not Reachable De-activation for Voice.

Description: Set type of call divert through preamble, then de-register divert. Finally go back to main menu.

Preamble: 	MFW613

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Voice */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Cancel Diverts */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	FAC_ERASE_IFNOTREACHABLE_V
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

### MFW625: Main Menu; Network Services, Call Divert If Not Reachable De-activation for Fax.

Description: Set type of call divert through preamble, then de-register divert. Finally go back to main menu.

Preamble: 	MFW613

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* FAX */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Cancel Diverts */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	FAC_ERASE_IFNOTREACHABLE_F
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

### MFW626: Main Menu; Network Services, Call Divert If Not Reachable De-activation for Data.

Description: Set type of call divert through preamble, then de-register divert. Finally go back to main menu.

Preamble: 	MFW613

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Data */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Cancel Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	FAC_ERASE_IFNOTREACHABLE_D
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

### MFW627: Main Menu; Network Services, Call Divert – Cancel All.

Description: Set type of call divert through preamble, then cancel all diverts. Finally go back to main menu.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Network Services */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Divert */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Cancel All */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	FAC_ERASE_ALLDIVERTS
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	RELEASE_COMPLETE_DEACTIVATE

History:

26-06-2003	dr		initial

### MFW628: Main Menu; Network Services, Call Divert Status for Voice.

Description: Set type of call divert through preamble, then de-register divert. Finally go back to main menu.

A: 	All Calls

B:	If Busy

C: 	If no reply

D: 	If no service

Variants:		<A>...<D>

Preamble: 	<A>MFW610
<B>MFW611
<C>MFW612
<D>MFW614

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Voice */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Check Status */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_DIVERT_STATUS_CALLS_V
<B>	fac_inf	FAC_DIVERT_STATUS_IFBUSY_V
<C>	fac_inf	FAC_DIVERT_STATUS_IFNOREPLY_V
<D>	fac_inf	FAC_DIVERT_STATUS_IFNONETWORK_V
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

### MFW607: Main Menu; Network Services, Call Divert Status for Fax.

Description: Set type of call divert through preamble, then de-register divert. Finally go back to main menu.

A: 	All Calls

B:	If Busy

C: 	If no reply

D: 	If no service

Variants:		<A>...<D>

Preamble: 	<A>MFW610
<B>MFW611
<C>MFW612
<D>MFW614

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* FAX */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Check Status */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_DIVERT_STATUS_CALLS_F
<B>	fac_inf	FAC_DIVERT_STATUS_IFBUSY_F
<C>	fac_inf	FAC_DIVERT_STATUS_IFNOREPLY_F
<D>	fac_inf	FAC_DIVERT_STATUS_IFNONETWORK_F
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

### MFW609: Main Menu; Network Services, Call Divert Status for Data.

Description: Set type of call divert through preamble, then de-register divert. Finally go back to main menu.

A: 	All Calls

B:	If Busy

C: 	If no reply

D: 	If no service

Variants:		<A>...<D>

Preamble: 	<A>MFW610
<B>MFW611
<C>MFW612
<D>MFW614

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Data */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Check Status */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete confirmation OK */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_DIVERT_STATUS_CALLS_D
<B>	fac_inf	FAC_DIVERT_STATUS_IFBUSY_D
<C>	fac_inf	FAC_DIVERT_STATUS_IFNOREPLY_D
<D>	fac_inf	FAC_DIVERT_STATUS_IFNONETWORK_D
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	CALL_FORWARD_RETURN_RESULT

History:

26-06-2003	dr		initial

### MFW629: Main Menu; Network services, Status Check, Hide ID Activate, followed by Deactivate. Test fails.

Description: .Check the status of CLIR then Send a Call Independent Supervisory Service, for Calling Line Identification Restriction. A Release Complete from the network is passed to MMI.

Note: The Activate selection, only does a Status Check. So test fail.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* from the Idle screen, Menu */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* network services */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Hide ID */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Check Status */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (5000)

(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* left key response */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Activate */

(3)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (3000)

(4)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* left key response */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Deactivate */

(5)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (3000)

(6)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* left key response */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	INTERROGATE_SS_CLIR
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	INTERROGATE_SS_RESULT

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	REGISTER_CLIR
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
 	cause	MNSS_CAUSE_CALL_CLEAR
 	fac_inf	RELEASE_COMPLETE_CLIR

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	DEREGISTER_CLIR
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
 	cause	MNSS_CAUSE_CALL_CLEAR
 	fac_inf	RELEASE_COMPLETE_CLIR

History:

30-05-2003	ef		initial

09-07-2003	ef		change in menu and add the deactivation part.

16-07-2003	ef		add Check Status of CLIR

## MMI Component Tests - case : Main Menu, Network Services, Call Barring.

### MFW630: Main Menu; Network Services, Call Barring.

Description: .Send a Call Independent Supervisory Service Call Barring. First Enter the Password.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Network Services */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Call Barring */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)		/* Enter Password */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

TIMEOUT_WAIT (1000)

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

13-06-20002	ef		initial

30-01-2003	ef		change menu

### MFW632: Check the Status of Barring All Outgoing Calls.

Description: .Status Check of Barring of Outgoing calls..

Preamble: 	MFW630

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Outgoing */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Check Status */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* left key response */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT) 		/* Set Bar */

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	INTERROGATE_SS_OG
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	INTERROGATE_SS_RESULT

History:

17-06-20002	ef		initial

14-07-2003	ef		add Check Status

### MFW633: Check the Status of Barring Outgoing International Calls.

Description: .Status Check of Outgoing International Calls Barring

Preamble: 	MFW630

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Outgoing International calls */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Check Status */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (3000)

(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* left key response */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Set Bar */

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	INTERROGATE_SS_OG_INTER
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	INTERROGATE_SS_RESULT

History:

17-06-20002	ef		initial

14-07-2003	ef		add Check Status

### MFW634: Check the Status of Barring Outgoing International except directed to Home PLMN.

Description: .Status Check of Outgoing International calls except directed to Home PLMN Barring.

Preamble: 	MFW630

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* OG Internat but home PLMN */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Check Status */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (3000)

(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* left key response */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT) 		/* Set Bar */

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	INTERROGATE_SS_OG_INTER_BUT_HOME
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	INTERROGATE_SS_RESULT

History:

17-06-20002	ef		initial

14-07-2003	ef		add Check Status

### MFW635: Check the Status of Barring Incoming Calls.

Description: .Status Check of Incomming calls Barring.

Preamble: 	MFW630

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Incoming call */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Check Status */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (3000)

(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* left key response */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Set Bar */

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	INTERROGATE_SS_IC
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	INTERROGATE_SS_RESULT

History:

17-06-20002	ef		initial

14-07-2003	ef		add Check Status

### MFW636: Check the Status of Barring Incoming calls when Roaming outside the Home PLMN.

Description: Status Check of Incomming calls Barring when Roaming outside the home PLMN.

Preamble: 	MFW630

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Incoming call when Roaming */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Check Status */

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (3000)

(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* left key response */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* set Bar */

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	INTERROGATE_SS_IC_ROAMING
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	INTERROGATE_SS_RESULT

History:

11-07-2003	ef		initial

14-07-2003	ef		add Check Status

### MFW637: Cancel all Call Barring option

Description: .Set Cancel all Call Barring

Preamble: 	MFW630

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Cancel All Barring */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

17-06-20002	ef		initial

11-07-2003	ef		change in menu

### MFW638: Change Password option

Description: .Set a new password for the use of Call Barring

Preamble: 	MFW630

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Change Password */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* enter new password */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* confirm new password */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=6)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* confirm new password */

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

11-07-2003	ef		initial

### MFW640: Send Each Call Barring SS option to the network.

Description: Send a Call Independent Supervisory Service Call Barring option to the network. The network replies with a SS Release Complete.

A: 	Bar Outgoing Calls

B: 	Bar Outgoing International Calls

C: 	Bar Outgoing International Calls except Home PLMN

D: 	Bar Incoming Calls

E: 	Bar Incoming when Roaming Calls

F: 	Cancel all Barring

G:	Change Password

Variants:		<A>...<G>

Preamble: 	<A>MFW632
<B>MFW633
<C>MFW634
<D>MFW635
<E>MFW636
<F>MFW637
<G>MFW638

APL/ACI		MMI		PS

|		|		|

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (5000)

(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
<A>	fac_inf	FAC_BAR_OG_CALLS
<B>	fac_inf	FAC_BAR_INTER_CALLS
<C>	fac_inf	FAC_BAR_INTER_BUT_HOME_CALLS
<D>	fac_inf	FAC_BAR_IC_CALLS
<E>	fac_inf	FAC_BAR_IC_ROAMING_CALLS
<F>	fac_inf	FAC_CANCEL_ALLBAR
<G>	fac_inf	FAC_REG_PW
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR

- <A>	fac_inf	RELEASE_COMPLETE_FACILITY
<B>	fac_inf	RELEASE_COMPLETE_FACILITY
<C>	fac_inf	RELEASE_COMPLETE_FACILITY
<D>	fac_inf	RELEASE_COMPLETE_FACILITY
<E>	fac_inf	RELEASE_COMPLETE_FACILITY
<F>	fac_inf	RELEASE_COMPLETE_FACILITY
<G>	fac_inf	FAC_PW_RES

History:

17-06-20002	ef		initial

30-01-2003	ef		change cause

14-07-2003	ef		add change password and check status

## MMI Component Tests - case : Main Menu, Network Services, Call Back.

### MFW642: Activate Call Back. Main Menu, Network Services, Call Back, Activate. Test fails.

Description: Note: This facility is not yet integrated.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Network Services */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Call Back */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Activate */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* left key response */

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

28-05-20003	ef		initial

## MMI Component Tests - case : Main Menu, Network Services, Call Waiting

### MFW644: Main Menu, Network Services, Call Waiting, Check Status.

Description: Note: The result of the Check Status is to send a facility message containing a Notify SS to the network.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Network Services */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Call Waiting */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Check Status */

|		|		|

(1)	|		| 	MNSS_BEGIN_REQ	|

|			*==========================>*
TIMEOUT_WAIT (5000)

(2)	|		| 	MNSS_END_IND	|
	|		*<	==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	INTERROGATE_SS
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	INTERROGATE_SS_RESULT

History:

29-05-20003	ef		initial

## MMI Component Tests - case : Main Menu, Network Services, Manual Selection.

### MFW646: Network Sevices, Manual Selection and select vodafone D2

Description: .From Idle, Main Menu, Network Services, Network Selection, Manual. The first plmn is chosen.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Network Services */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Network Selection */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Manual */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
(1)	|		|	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(2)	|		| 	GMMREG_NET_REQ	|

|		*	==========================>	*

(3)	|		| 	GMMREG_PLMN_IND	|

|		*	<==========================	*

(4)	|		|	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select 1st PLMN */
(5)	|		|	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(6)	|		| 	GMMREG_PLMN_RES	|

|		*	==========================>	*

(7)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (1000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_MAN

- GMMREG_NET_REQ

- GMMREG_PLMN_IND
	cause	MMCS_SUCCESS
	plmn	PLMN_LIST
	forb_ind	FORB_PLMN_LIST
	lac_list	LAC_LIST
	rxlevel	RX_LEVEL_LIST
	gprs_status	GPRS_STATUS_LIST

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_MAN

- GMMREG_PLMN_RES
	plmn	PLMN_262_02
	mobile_class	GMMREG_CLASS_BG
	attach_type	GMMREG_AT_COMB

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_GPRS
	plmn	PLMN_262_02
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

History:

28-05-2003	ef		initial

21-08-2003	ef		add the attach_cnf

## MMI Component Tests - case : Main Menu, Network Services, Call Deflect

### MFW648: Main Menu, Network Services, Call Deflect, Activate. MT Call Deflected. Test Fails.

Description: After setting Call Deflect On, an incomming Call is first rejected by the Mobile, then deflected by invoking the SS Call Deflect Facility.

Note: The test fails converting the CD Facility message to the Air interface bitstream in CCD encoder, and no Facility message is sent to the network.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Network Services */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Call Deflect */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* On */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Back to Idle */

TIMEOUT_WAIT (2000)

(1)	|		| 	MNCC_SETUP_IND	|
	|		*<	==========================	*

(2)	|		| 	MNCC_SYNC_IND	|
	|		*<	==========================	*

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(4)	|		| 	MNCC_ALERT_REQ	|
	|		*	==========================>	*
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Deflect Accept */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

(5)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(6)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* LSK response */

TIMEOUT_WAIT (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_IND
	ti	TI_MT_9
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_SPEECH
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIG_DIAL_TONE_ON
	calling_party	CLG_PARTY_01317180537
	calling_party_sub	CLG_PARTY_SUB
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB
	redirecting_party	RED_PARTY
	redirecting_party_sub	RED_PARTY_SUB

- MNCC_SYNC_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CH_SIG_ONLY

- ACI_CMD_IND
	cmd_len	LM_RING_LEN
	cmd_seq	M_RING

- MNCC_ALERT_REQ
	ti	TI_MT_9

- MNCC_DISCONNECT_REQ
	ti	TI_MT_9
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MT_9
	cause	MNCC_CAUSE_CALL_CLEAR

History:

06-08-20003	ef		initial

## MMI Component Tests - case : Main Menu, Recent Calls

### MFW670: Make a call from the Answered Call List – Main Menu, Recent Calls, Answered Calls.

Description: After a Mobile Terminated call, from the Main Menu, Recent Calls, Answered Calls, the first entry is chosen, a call is setup.

Preamble: 	MFW513

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Main Menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Recent Calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Answered Calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)
(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

(3)	|		| 	MNCC_PROGRESS_IND	|

|		*	<==========================	*

(4)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	* 
(5)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
(6)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_ERIC_T
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_PROGRESS_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_654321
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

History:

21-05-2002	ef		initial

20-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

### MFW672: Store a number and send SMS from the Answered Call List – Main Menu, Recent Calls, Answered Calls

Description: An MT call is received, and the callers number is stored. Main Menu, Recent Calls, Answered calls and the first entry is chosen, store the number. Back to the Recent calls Menu, Aswered Calls and send SMS text message.

Note: first set the Service Cente Settings

Note: Pass over the special characters à and á which appear on the ABC button. This will be removed at a later stage.

Preamble: 	MFW513

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Main Menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* recent calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* answered calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* store number */
TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* enter a name */

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(1)	|		| 	SIM_UPDATE_RECORD_REQ	|

|			*==========================>*
TIMEOUT_WAIT (3000)
(2)	|		| 	SIM_UPDATE_RECORD_CNF	|

|		*<	==========================	*

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* main menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* recent calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* answered calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* send message */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* enter text */

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* send */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(3)	|		| 	MNSMS_SUBMIT_REQ	|

|			*==========================>*
(4)	|		| 	MNSMS_SUBMIT_CNF	|

|			*<==========================*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_UPDATE_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_7
	length	PHB_LENGTH_ADN
 	linear_data	PHB_ADN_RECORD_ERIC_MT_SAVE

- SIM_UPDATE_RECORD_CNF
	datafield	SIM_ADN
	record	PHB_RECORD_7
	cause	SIM_NO_ERROR

- MNSMS_SUBMIT_REQ
	mem_type		MEM_SM
	rec_num		NUM_0
	condx		SMS_CONDX_OVR_NON
	modify		SMS_MODIFY_NON
	sms_sdu		SMS_SDU_MISSED_CALLS

- MNSMS_SUBMIT_CNF
	mem_type	MEM_SM
	rec_num	SMS_RECORD_NOT_EXIST
	cause	SMS_NO_ERROR
	tp_mr	NUM_2
	sms_sdu	SMS_SDU_EMPTY

History:

12-08-2003	ef		initial

20-08-2003	ef		skip over the special chars à and á. Remove later.

08-10-2003	ef		remove the à and á characters

### MFW674: Delete record from the Answered Call List – Main Menu, Recent Calls, Answered Calls, Delete.

Description: After a Mobile Originated call, from the Main Menu, Recent Calls, Answered Calls, Delete.

Note: This test will always pass as only internal phone memory is erased. No signal is output.

Preamble: 	MFW513

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Main Menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Recent Calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Answered Calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Delete */
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (5000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

History:

13-08-2003	ef		initial

### MFW676: Make a call from the Outgoing Call List – Main Menu, Recent Calls, Outgoing Calls.

Description: After a Mobile Originated call, from the Main Menu, Recent Calls, Outgoing Calls, the first entry is chosen, a call is setup.

Preamble: 	MFW513

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Main Menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Recent Calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Answered Calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=SEND)
(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

(3)	|		| 	MNCC_PROGRESS_IND	|

|		*	<==========================	*

(4)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	* 
(5)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
(6)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_ERIC_T
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_PROGRESS_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_654321
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

History:

21-05-2002	ef		initial

20-08-2002	ef		remove the MMI_SPEECH_MODE_REQ signal

### MFW678: Store a number and send SMS from the Outgoing Call List – Main Menu, Recent Calls, Outgoing Calls

Description: An MT call is received, and the callers number is stored. Main Menu, Recent Calls, Outgoing calls and the first entry is chosen, store the number. Back to the Recent calls Menu, Outgoing Calls and send SMS text message.

Note 1: first set the Service Cente Settings

Note 2: Pass over the special characters à and á which appear on the ABC button. This will be removed at a later stage.

Preamble: 	MFW502

APL/ACI		MMI		PS

|		|		|

TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Main Menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* recent calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Outgoing Calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* store number */
TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* enter a name */

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(1)	|		| 	SIM_UPDATE_RECORD_REQ	|

|			*==========================>*
TIMEOUT_WAIT (3000)
(2)	|		| 	SIM_UPDATE_RECORD_CNF	|

|		*<	==========================	*

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* main menu */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* recent calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Outgoing calls */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select 1st entry*/
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* send message */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* enter text */

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (100)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* send */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Enter Number, Send */
(3)	|		| 	MNSMS_SUBMIT_REQ	|

|			*==========================>*
(4)	|		| 	MNSMS_SUBMIT_CNF	|

|			*<==========================*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_UPDATE_RECORD_REQ
	source	SRC_MMI
	datafield	SIM_ADN
	record	PHB_RECORD_7
	length	PHB_LENGTH_ADN
 	linear_data	PHB_ADN_RECORD_ERIC_MO_SAVE

- SIM_UPDATE_RECORD_CNF
	datafield	SIM_ADN
	record	PHB_RECORD_7
	cause	SIM_NO_ERROR

- MNSMS_SUBMIT_REQ
	mem_type		MEM_SM
	rec_num		NUM_0
	condx		SMS_CONDX_OVR_NON
	modify		SMS_MODIFY_NON
	sms_sdu		SMS_SDU_OUTGOING_CALLS

- MNSMS_SUBMIT_CNF
	mem_type	MEM_SM
	rec_num	SMS_RECORD_NOT_EXIST
	cause	SMS_NO_ERROR
	tp_mr	NUM_2
	sms_sdu	SMS_SDU_EMPTY

History:

12-08-2003	ef		initial

20-08-2003	ef		skip over the special chars à and á. Remove later.

08-10-2003	ef		remove the à and á characters

## MMI Component Tests - case : Main Menu, Messages

### MFW700: Menu, Messages, Activate Concatenation, Setup Call Centre and Validy Period defaults.

Description: From the Main Menu, Messages, Settings, Set Activate SMS concatenation, Set Validy Period of 1 Hour, Add the Service Centre Number. Then back to the Main Menu.

Preamble: 	MFW430

APL		ACI		PS

|		|		|

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

- TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Messages */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Settings */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Set Service Centre */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=9)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Validity */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* 1 hour */

TIMEOUT_WAIT (6000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to parent menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* skip eazy text */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Concatenate */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* set Concatenate On */

TIMEOUT_WAIT (6000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/*  */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

TIMEOUT_WAIT (2000)

- Parametrization:

Primitive	Parameter	Value

History:

05-09-2002	ef		initial

30-01-2003	ef		change menu

24-04-2003	ef		use right key after validity and concat to clear the window

### MFW702: Setup a MO Originated SMS text string  from the Messages window, Send. Check for error condition.

Description: Main Menu, Messages, Write, enter text (“Eric”), enter number, Send.

Note: Pass over the special characters àand á which appear on the ABC button. This will be removed at a later stage.

A: 	No error in SMS.

B: 	SMS CP error detected

Variants:		<A>...<B>

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Messages */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Write */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Enter Text */

TIMEOUT_WAIT (4000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Send */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Enter Number */
TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Send */
(1)	|		| 	MNSMS_SUBMIT_REQ	|

|		*	==========================>	*

(2)	|		|	MNSMS_SUBMIT_CNF	|

|			*<==========================*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_SUBMIT_REQ
	mem_type		MEM_SM
	rec_num		NUM_0
	condx		SMS_CONDX_OVR_NON
	modify		SMS_MODIFY_NON
	sms_sdu		SMS_SDU_1

- MNSMS_SUBMIT_CNF
	mem_type	MEM_SM
	rec_num	SMS_RECORD_NOT_EXIST
<A>	cause	SMS_NO_ERROR
<B>	cause	SMS_CP_CS_NETWORK_FAILURE
	tp_mr	NUM_2
	sms_sdu	SMS_SDU_EMPTY

History:

04-09-2002	ef		initial
	06-01-2003	ef		update to cause concept

19-08-2003	ef		skip over the special chars à and á. Remove later

08-10-2003	ef		characters à and á have been removed

### MFW704: From the Messages Window, write a MO Short Message and save it to the SIM.

Description: Main Menu, Messages, Write, enter text (“Eric”), and Save, using Delete key 5 times, back to the Messages Menu, then back to Main Menu.

Note: Pass over the special characters à and á which appear on the ABC button. This will be removed at a later stage.

Preamble: 	MFW700

APL		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Messages */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Write */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* Enter Text */

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Save */
(1)	|		| 	MNSMS_STORE_REQ	|

|		*	==========================>	*

(2)	|		|	MNSMS_STORE_CNF	|

|			*<==========================*

TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_STORE_REQ
	mem_type	MEM_SM
	rec_num	NUM_0
	condx	SMS_CONDX_OVR_NON
	status	SMS_RECORD_STO_UNSENT
	sms_sdu	SMS_SDU_2

- MNSMS_STORE_CNF
	mem_type	MEM_SM
	rec_num	NUM_0
	cause	SMS_NO_ERROR

History:

06-09-2002	ef		initial

20-08-2003	ef		skip over the special chars à and á. Remove later.

08-10-2003	ef		characters à and á have been removed

### MFW706: From the Messages Window, store a Short Message to SIM memory, then Send message.

Description: Main Menu, Messages, Write the text “Eric”, Save, Menu Down 1 to Send, enter the number and then Send.

Note: Pass over the special characters à and á which appear on the ABC button. This will be removed at a later stage.

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Messages */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Write */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* Enter Text */

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Save */
(1)	|		| 	MNSMS_STORE_REQ	|

|		*	==========================>	*

(2)	|		|	MNSMS_STORE_CNF	|

|			*<==========================*

TIMEOUT_WAIT (5000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Send */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Send */
(3)	|		| 	MNSMS_SUBMIT_REQ	|

|		*	==========================>	*

(4)	|		|	MNSMS_REPORT_IND	|

|			*<==========================*

TIMEOUT_WAIT (6000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_STORE_REQ
	mem_type	MEM_SM
	rec_num	NUM_0
	condx	SMS_CONDX_OVR_NON
	status	SMS_RECORD_STO_UNSENT
	sms_sdu	SMS_SDU_2

- MNSMS_STORE_CNF
	mem_type	MEM_SM
	rec_num	NUM_0
	cause	SMS_NO_ERROR

- MNSMS_SUBMIT_REQ
	mem_type	MEM_SM
	rec_num	NUM_0
	condx	SMS_CONDX_OVR_NON
	modify	SMS_MODIFY_NON
	sms_sdu	SMS_SDU_1

- MNSMS_REPORT_IND
	state	SMS_STATE_READY

History:

09-09-2002	ef		initial

20-08-2003	ef		skip over the special chars à and á. Remove later.

08-10-2003	ef		characters à and á have been removed

### MFW708: Setup a maximum length MO Short Message of 160 characters in length.

Description: Main Menu, Messages, Write, enter 176 text characters, Options, Send, enter number, Accept.

Note. Sometimes the contents of the output msg changes. May need to run again, or use SMS_SDU_CONC_1.

A: 	No error in SMS.

B: 	SMS CP error detected

Variants:		<A>...<B>

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Messages */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Write */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (4000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */
TIMEOUT_WAIT (6000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Send */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Send */
(1)	|		| 	MNSMS_SUBMIT_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (6000)
(2)	|		|	MNSMS_SUBMIT_CNF	|

|			*<==========================*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_SUBMIT_REQ
	mem_type	MEM_SM
	rec_num	NUM_0
	condx	SMS_CONDX_OVR_NON
	modify	SMS_MODIFY_NON
	sms_sdu	SMS_SDU_MAX_1

- MNSMS_SUBMIT_CNF
	mem_type	MEM_SM
	rec_num	SMS_RECORD_NOT_EXIST
<A>	cause	SMS_NO_ERROR
<B>	cause	SMS_CP_CS_NETWORK_FAILURE
	tp_mr	NUM_2
	sms_sdu	SMS_SDU_EMPTY

History:

10-09-2002	ef		initial

06-01-2003	ef		update to cause concept

### MFW710: Setup a MO Concatenated Short Message of 160 7 bit characters in length.

Description: Main Menu, Messages, Write, enter 160 text characters.

Preamble: 	MFW700

APL		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Messages */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Write */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (4000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

26-11-2002	ef		initial

### MFW712: Continue Concatenated Short Message of 196 characters in length, which leads to two SM’s to the network

Description: Continue to enter text characters, Options, Send, enter number, Accept.

Note1. Sometimes the contents of the output msg changes. May need to run again, or use SMS_SDU_MAX_1.

Note2: The count of chars in the header seems wrong. However for the test it is set to what the MMI says. So if this is changed at a later date, then the SMS message will have to change as well.

Preamble: 	MFW710

APL		MMI		PS

|		|		|

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY5)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY6)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY8)

- TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=KEY9)

TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

TIMEOUT_WAIT (6000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Send */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Send */
(1)	|		| 	MNSMS_SUBMIT_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (6000)
(2)	|		|	MNSMS_SUBMIT_CNF	|

|			*<==========================*

(3)	|		| 	MNSMS_SUBMIT_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (6000)
(4)	|		|	MNSMS_SUBMIT_CNF	|

|			*<==========================*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_SUBMIT_REQ
	mem_type	MEM_SM
	rec_num	NUM_0
	condx	SMS_CONDX_OVR_NON
	modify	SMS_MODIFY_NON
	sms_sdu	SMS_SDU_CONC_1

- MNSMS_SUBMIT_CNF
	mem_type	MEM_SM
	rec_num	SMS_RECORD_NOT_EXIST
	cause	SMS_NO_ERROR
	tp_mr	NUM_2
	sms_sdu	SMS_SDU_EMPTY

- MNSMS_SUBMIT_REQ
	mem_type	MEM_SM
	rec_num	NUM_0
	condx	SMS_CONDX_OVR_NON
	modify	SMS_MODIFY_NON
	sms_sdu	SMS_SDU_CONC_2

- MNSMS_SUBMIT_CNF
	mem_type	MEM_SM
	rec_num	SMS_RECORD_NOT_EXIST
	cause	SMS_NO_ERROR
	tp_mr	NUM_2
	sms_sdu	SMS_SDU_EMPTY

History:

26-11-2002	ef		initial

06-01-2003	ef		update to cause concept

### MFW714: Read the list of Short Messages and then delete all.

Description: Main Menu, Messages, Read, Back, Delete All.

Preamble: 	MFW704

APL		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Messages */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Read */

(1)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(2)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete all */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* delete? OK */

(3)	|		| 	MNSMS_DELETE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (6000)
(4)	|		|	MNSMS_DELETE_CNF	|

|			*<==========================*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_0
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_0
	rec_next	REC_NUM_1
	rec_max	NOT_USED
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_STO_UNSENT
	sms_sdu	SMS_SDU_1

- MNSMS_DELETE_REQ
	mem_type	MEM_SM
	rec_num	NUM_0

- MNSMS_DELETE_CNF

- mem_type	MEM_SM
	rec_num	NUM_0
	cause	SIM_NO_ERROR

History:

09-09-2002	ef		initial

31-01-2003	ef		change cause in MNSMS_READ_CNF

24-04-2003	ef		update the sequence

### MFW716: Send an empty MO text message.

Description: From the Idle screen, Main Menu, Messages, Write, enter number, Send.

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Messages */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Write */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* no text, Options */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Send */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Send */
(1)	|		| 	MNSMS_SUBMIT_REQ	|

|		*	==========================>	*

(2)	|		|	MNSMS_SUBMIT_CNF	|

|			*<==========================*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_SUBMIT_REQ
	mem_type		MEM_SM
	rec_num		NUM_0
	condx		SMS_CONDX_OVR_NON
	modify		SMS_MODIFY_NON
	sms_sdu		SMS_NO_TEXT

- MNSMS_SUBMIT_CNF
	mem_type	MEM_SM
	rec_num	SMS_RECORD_NOT_EXIST
	cause	SMS_NO_ERROR
	tp_mr	NUM_2
	sms_sdu	SMS_SDU_EMPTY

History:

15-07-2003	ef		initial

### MFW720: A Class 0 MT Short Message is displayed on the MMI. Afterwards send a MO msg.

Description: Main Menu, MT SM is indicated, Read, Select, and the message is displayed. Options, Menu Down to Reply and Select, enter optional text (“Eric”) and Options. Menu Down to Send and enter number to send, Send.

Note: Pass over the special characters à and á which appear on the ABC button. This will be removed at a later stage.

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (4000)

(1)	|		| 	MNSMS_MESSAGE_IND	|

|			*<==========================*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Read Message */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Reply */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (6000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* Enter Text */

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
TIMEOUT_WAIT (6000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Send */

TIMEOUT_WAIT (6000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)		/* Enter Number */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=1)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=8)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=0)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=5)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=7)
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Send */
(2)	|		| 	MNSMS_SUBMIT_REQ	|

|		*	==========================>	*

(3)	|		|	MNSMS_SUBMIT_CNF	|

|			*<==========================*

TIMEOUT_WAIT (6000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_MESSAGE_IND
	mem_type	MEM_SM
	rec_num	NUM_0
	rec_max	NUM_0
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_1

- MNSMS_SUBMIT_REQ
	mem_type	MEM_SM
	rec_num	NUM_0
	condx	SMS_CONDX_OVR_NON
	modify	SMS_MODIFY_NON
	sms_sdu	SMS_SDU_1

- MNSMS_SUBMIT_CNF
	mem_type	MEM_SM
	rec_num	SMS_RECORD_NOT_EXIST
	cause	NOT_USED
	tp_mr	NUM_2
	sms_sdu	SMS_SDU_EMPTY

History:

13-09-2002	ef		initial

20-08-2003	ef		skip over the special chars à and á. Remove later.

08-10-2003	ef		characters à and á have been removed

### MFW723: Delete a saved msg. Check for error condition.

Description: Main Menu, Messages, Write, enter text and Save. Delete the text and back up to the Delete All option and Select. Report an error back from the SIM

Note: Pass over the special characters à and á which appear on the ABC button. This will be removed at a later stage.

Preamble: 	MFW700

APL		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Messages */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Write */

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* Enter Text */

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */
TIMEOUT_WAIT (1000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Save */
(1)	|		| 	MNSMS_STORE_REQ	|

|		*	==========================>	*

(2)	|		|	MNSMS_STORE_CNF	|

|			*<==========================*

TIMEOUT_WAIT (4000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to Messages Menu(Write) */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Delete All */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Delete All? */
(3)	|		| 	MNSMS_DELETE_REQ	|

|		*	==========================>	*

(4)	|		|	MNSMS_DELETE_CNF	|

|			*<==========================*

TIMEOUT_WAIT (2000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_STORE_REQ
	mem_type	MEM_SM
	rec_num	NUM_0
	condx	SMS_CONDX_OVR_NON
	status	SMS_RECORD_STO_UNSENT
	sms_sdu	SMS_SDU_2

- MNSMS_STORE_CNF
	mem_type	MEM_SM
	rec_num	NUM_0
	cause	SIM_NO_ERROR

- MNSMS_DELETE_REQ
	mem_type	MEM_SM
	rec_num	NUM_0

- MNSMS_DELETE_CNF
	mem_type	MEM_SM
	rec_num	NUM_0
	cause	SMS_CAUSE_SIM_BUSY

- History:

19-09-2002	ef		initial

20-08-2003	ef		skip over the special chars à and á. Remove later.

08-10-2003	ef		characters à and á have been removed

### MFW724: A MT Concatenated Short Message is indicated on the MMI. Read this message and Reply to it.

Description: On the Idle screen, MT SM is indicated, Read, Select, and the message is displayed. Options, Menu Down to Reply and Select, enter optional text (“Eric”) and Options. Menu Down to Send and enter number to send, Send.

Note1: for some reason the mnsms_read_req is sent to the sim twice, followed by a status check. Seems a bit odd.

Note2: Pass over the special characters à and á which appear on the ABC button. This will be removed at a later stage.

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

(1)	|		| 	MNSMS_MESSAGE_IND	|

|			*<==========================*

(2)	|		| 	MNSMS_MESSAGE_IND	|

|			*<==========================*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Read Message */

(3)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(4)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(5)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(6)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

TIMEOUT_WAIT (9000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(7)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(8)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(9)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(10)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(11)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(12)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(13)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(14)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

TIMEOUT_WAIT (9000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Reply */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (6000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)		/* Enter Text */

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)		/* “E” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)		/* “r” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=4)		/* “i” */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=2)		/* “c” */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */
TIMEOUT_WAIT (6000)
COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Send */

TIMEOUT_WAIT (6000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Enter Number, Send */
(15)	|		| 	MNSMS_SUBMIT_REQ	|

|		*	==========================>	*

(16)	|		|	MNSMS_SUBMIT_CNF	|

|			*<==========================*

TIMEOUT_WAIT (6000)
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_MESSAGE_IND
	mem_type	MEM_SM
	rec_num	NUM_1
	rec_max	NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_MESSAGE_IND
	mem_type	MEM_SM
	rec_num	NUM_2
	rec_max	NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_STATUS_CHANGE
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_STATUS_CHANGE
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_SUBMIT_REQ
	mem_type	MEM_SM
	rec_num	REC_NUM_0
	condx	SMS_CONDX_OVR_NON
	modify	SMS_MODIFY_NON
	sms_sdu	SMS_SDU_1

- MNSMS_SUBMIT_CNF
	mem_type	MEM_SM
	rec_num	SMS_RECORD_NOT_EXIST
	cause	NOT_USED
	tp_mr	NUM_2
	sms_sdu	SMS_SDU_EMPTY

History:

19-06-2003	ef		initial

20-08-2003	ef		skip over the special chars à and á. Remove later.

08-10-2003	ef		characters à and á have been removed

### MFW726: A MT Concatenated Short Message is indicated on the MMI. Read this message and call the number.

Description: On the Idle screen, MT SM is indicated, Read, Select, and the message is displayed. Options, Menu Down to Call Number and Select.

Note: for some reason the mnsms_read_req is sent to the sim twice, followed by a status check. Seems a bit odd.

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

(1)	|		| 	MNSMS_MESSAGE_IND	|

|			*<==========================*

(2)	|		| 	MNSMS_MESSAGE_IND	|

|			*<==========================*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Read Message */

(3)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(4)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(5)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(6)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

TIMEOUT_WAIT (9000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(7)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(8)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(9)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(10)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(11)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(12)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(13)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(14)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

TIMEOUT_WAIT (9000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Call Number */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Enter Number, Call */

(15)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(16)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(17)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

(18)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

(19)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

(20)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_MESSAGE_IND
	mem_type	MEM_SM
	rec_num	NUM_1
	rec_max	NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_MESSAGE_IND
	mem_type	MEM_SM
	rec_num	NUM_2
	rec_max	NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_STATUS_CHANGE
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_STATUS_CHANGE
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_01317180537
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

History:

26-06-2003	ef		initial

### MFW728: A MT Concatenated Short Message is indicated on the MMI. Read this message and extract number.

Description: On the Idle screen, MT SM is indicated, Read, Select, and the message is displayed. Options, Menu Down to Extract Number and Select.

Note: for some reason the mnsms_read_req is sent to the sim twice, followed by a status check. Seems a bit odd.

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

(1)	|		| 	MNSMS_MESSAGE_IND	|

|			*<==========================*

(2)	|		| 	MNSMS_MESSAGE_IND	|

|			*<==========================*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Read Message */

(3)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(4)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(5)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(6)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

TIMEOUT_WAIT (9000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(7)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(8)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(9)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(10)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(11)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(12)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(13)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(14)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

TIMEOUT_WAIT (9000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)		/* Extract Number */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* number back */

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_MESSAGE_IND
	mem_type	MEM_SM
	rec_num	NUM_1
	rec_max	NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_MESSAGE_IND
	mem_type	MEM_SM
	rec_num	NUM_2
	rec_max	NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_STATUS_CHANGE
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_STATUS_CHANGE
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

History:

26-06-2003	ef		initial

### MFW730: A MT Concatenated Short Message is indicated on the MMI. Read this message, edit and Discard.

Description: On the Idle screen, MT SM is indicated, Read, Select, and the message is displayed. Options, Menu Down to Edit , Options, Discard.

Note: fo some reason the mnsms_read_req is sent to the sim twice, followed by a status check. Seems a bit odd.

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

(1)	|		| 	MNSMS_MESSAGE_IND	|

|			*<==========================*

(2)	|		| 	MNSMS_MESSAGE_IND	|

|			*<==========================*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Read Message*/

(3)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(4)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(5)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(6)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

TIMEOUT_WAIT (9000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(7)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(8)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(9)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(10)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(11)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(12)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(13)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(14)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

TIMEOUT_WAIT (9000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Edit */

TIMEOUT_WAIT (5000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* options */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Discard */

TIMEOUT_WAIT (4000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_MESSAGE_IND
	mem_type	MEM_SM
	rec_num	NUM_1
	rec_max	NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_MESSAGE_IND
	mem_type	MEM_SM
	rec_num	NUM_2
	rec_max	NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_STATUS_CHANGE
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_STATUS_CHANGE
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

History:

26-06-2003	ef		initial

### MFW732: A MT Concatenated Short Message is indicated on the MMI. Read this message and Delete.

Description: On the Idle screen, MT SM is indicated, Read, Select, and the message is displayed. Options, Delete.

Note: fo some reason the mnsms_read_req is sent to the sim twice, followed by a status check. Seems a bit odd.

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

(1)	|		| 	MNSMS_MESSAGE_IND	|

|			*<==========================*

(2)	|		| 	MNSMS_MESSAGE_IND	|

|			*<==========================*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Read Message */

(3)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(4)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(5)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(6)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

TIMEOUT_WAIT (9000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Select */

(7)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(8)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(9)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(10)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(11)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(12)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

(13)	|		| 	MNSMS_READ_REQ	|

|		*	==========================>	*

(14)	|		|	MNSMS_READ_CNF	|

|			*<==========================*

TIMEOUT_WAIT (9000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Options */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Delete */

(15)	|		| 	MNSMS_DELETE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (6000)
(16)	|		|	MNSMS_DELETE_CNF	|

|			*<==========================*

TIMEOUT_WAIT (6000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_MESSAGE_IND
	mem_type	MEM_SM
	rec_num	NUM_1
	rec_max	NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_MESSAGE_IND
	mem_type	MEM_SM
	rec_num	NUM_2
	rec_max	NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_PREVIEW
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_1
	read_mode	READ_STATUS_CHANGE
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_1
	rec_next	REC_NUM_2
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_1
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_1

- MNSMS_READ_REQ
	mem_type	MEM_SM
	rec_num	NUM_2
	read_mode	READ_STATUS_CHANGE
	status	0xFF

- MNSMS_READ_CNF
	mem_type	MEM_SM
	rec_num	REC_NUM_2
	rec_next	SMS_RECORD_NOT_EXIST
	rec_max	NUM_10
	cause	SIM_NO_ERROR
	rec_status	REC_NUM_2
	status	SMS_RECORD_REC_UNREAD
	sms_sdu	SMS_SDU_MT_CONC_2

- MNSMS_DELETE_REQ
	mem_type	MEM_SM
	rec_num	NUM_1

- MNSMS_DELETE_CNF

- mem_type	MEM_SM
	rec_num	NUM_1
	cause	SIM_NO_ERROR

History:

16-07-2003	ef		initial

### MFW736: Set Voicemail then call Voicemail.

Description: From the Idle screen, Menu,Messages, Voicemail, Set Voicemail, Call Voicemail.

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* messages */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* voicemail */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Set Voicemail */

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=8)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=0)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=5)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)

TIMEOUT_WAIT (1000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=7)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Voicemail number */
TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* call voicemail */

(1)	|		| 	MNCC_SETUP_REQ	|

|			*==========================>*
(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

(4)	|		| 	MNCC_CALL_PROCEED_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

(5)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

(6)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- SIM_SYNC_CNF
	cause	SIM_NO_ERROR

- MNCC_CALL_PROCEED_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_01317180537
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

History:

05-08-2003	ef		initial

### MFW740: Activate Cell Broadcast and Read. Start new Broadcast.

Description: From the Idle screen, Menu,Messages, Broadcast, Activate, On, Read, new Broadcast.

Preamble: 	MFW700

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* messages */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* broadcast */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Activate */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* On */

(1)	|		| 	MNSMS_CONFIGURE_REQ	|

|			*========================>*

(2)	|		| 	MMI_CBCH_REQ	|

|			*========================>*

(3)	|		| 	MMI_CBCH_IND	|

|			*<==========================*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* menu */

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* read */

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* new broadcast */

TIMEOUT_WAIT (6000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY3)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY7)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY4)

TIMEOUT_WAIT (2000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=KEY2)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* name of broadcast */
TIMEOUT_WAIT (4000)

- COMMAND (MMI CONFIG KEY_SEQUENCE=1)

- COMMAND (MMI CONFIG KEY_SEQUENCE=2)

- COMMAND (MMI CONFIG KEY_SEQUENCE=3)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* MID */
(4)	|		| 	MNSMS_CONFIGURE_REQ	|

|			*========================>*

(5)	|		| 	MMI_CBCH_REQ	|

|			*========================>*

(6)	|		| 	MMI_CBCH_REQ	|

|			*========================>*

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* List */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* save */

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_CONFIGURE_REQ
	pref_mem_3	MEM_SM
	mt	MT0
	ds	DS0
	mhc	SMS_MHC_PH2

- MMI_CBCH_REQ
	msg_id	CBCH_MSG_IDS
	dcs_id	CBCH_DCS_IDS
	modus	CBCH_ACCEPT

- MMI_CBCH_IND
	cbch_msg	CBCH_MSG_1
	cbch_len	NUM_9

- MNSMS_CONFIGURE_REQ
	pref_mem_3	MEM_SM
	mt	MT0
	ds	DS0
	mhc	SMS_MHC_PH2

- MMI_CBCH_REQ
	msg_id	CBCH_MSG_IDS
	dcs_id	CBCH_DCS_IDS
	modus	CBCH_ACCEPT

- MMI_CBCH_REQ
	msg_id	CBCH_MSG_IDS_NEW_BC
	dcs_id	CBCH_DCS_IDS
	modus	CBCH_ACCEPT

History:

16-07-2003	ef		initial

## MMI Component Tests - case : Main Menu, Phone Settings, GPRS.

### MFW750: GPRS Attach from the Main Menu.

Description: Main Menu, Phone Settings, GPRS, Attach.

Note: The 5 secs delay is essential to allow the Idle window time to appear.

Preamble: 	MFW001

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG CLASS_A)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

(8)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (6000)

(9)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(10)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(11)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* phone settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* GPRS */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* attach */
(12)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(13)	|		| 	GMMREG_ATTACH_CNF	|

|		*	<==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* accept the confirmation msg */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NO_EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_A
	attach_type	GMMREG_AT_IMSI
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_A
	attach_type	GMMREG_AT_GPRS
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_CNF
	attach_type	GMMREG_AT_GPRS
	plmn	PLMN_262_14
	lac	LAC_1234
	rac	RAC_89
	cid	CID_0001
	gprs_indicator	GMM_GPRS_SUPP_YES
	search_running	GMMREG_SEARCH_NOT_RUNNING

- History:

24-09-2002	ef		initial

08-10-2002	ef		changes to allow the PIN window first.

03-02-2003	ef		change to menu

10-06-2003	ef		add SIM_ACTIVATE_REQ signal and back to main menu

### MFW752: GPRS Attach from the Main Menu. Attach is rejected by the network.

Description: Main Menu, Phone Settings, GPRS, Attach.

Preamble: 	MFW001

APL/ACI		MMI		PS

|		|		|

- COMMAND (MMI CONFIG CLASS_A)

(1)	|		|	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		|	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		|	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(6)	|		|	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(8)	|		| 	GMMREG_PLMN_MODE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (6000)

(9)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(10)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(11)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* phone settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* GPRS */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* attach */
(12)	|		| 	GMMREG_ATTACH_REQ	|

|		*	==========================>	*

(13)	|		| 	GMMREG_ATTACH_REJ	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	MMI_AND_FDN_BDN
	stk_pro_file	STK_SUPPORTED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NO_EC_CODES
	pref_lang	NO_PREF_LANG

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	IMSI
	pref_plmn	PREF_PLMN
	phase	PHASE_2_SIM
	access_acm	ACCESS_ALWAYS
	access_acmmax	ACCESS_ALWAYS
	access_puct	ACCESS_ALWAYS

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_A
	attach_type	GMMREG_AT_IMSI
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_PLMN_MODE_REQ
	net_selection_mode	GMMREG_NET_SEL_MODE_AUTO

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_CLASS_A
	attach_type	GMMREG_AT_GPRS
	service_mode	SERVICE_MODE_FULL
	t3314_ready_val	VAL_T3314
	t3312_standby_rau_val	VAL_T3312

- GMMREG_ATTACH_REJ
	detach_type	GMMREG_AT_GPRS
	cause	MMCS_NETWORK_FAILURE
	search_running	GMMREG_SEARCH_RUNNING

- History:

25-09-2002	ef		initial

08-10-2002	ef		changes to allow the PIN window first.

19-12-2002	ef		change the cause type

03-02-2003	ef		change to menu

### MFW754: GPRS Detach from the Main Menu.

Description: Main Menu, Phone Settings, GPRS, Detach.

Preamble: 	MFW750

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* phone settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* GPRS */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(1)	|		| 	GMMREG_DETACH_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (6000)

(2)	|		| 	GMMREG_DETACH_CNF	|

|		*	<==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* accept the confirmation msg */
TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- GMMREG_DETACH_REQ
	detach_type	GMMREG_DT_GPRS

- GMMREG_DETACH_CNF
	detach_type	GMMREG_DT_GPRS

- History:

25-09-2002	ef		initial

### MFW756: SMS CSD Preferred Attach from the Main Menu, same as the default settings, so no change.

Description: Main Menu, Phone Settings, GPRS, SMS, CSD Preferred Attach.

Preamble: 	MFW750

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* phone settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* GPRS */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* SMS */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* CSD preferred */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* accept the confirmation msg */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */

|		|		|

- Parametrization:

Primitive	Parameter	Value

- History:

30-09-2002	ef		initial

08-10-2002	ef		changes to allow the PIN window first.

03-02-2003	ef		change to menu

09-06-2003	ef		return to main menu

### MFW757: SMS GPRS Attach from the Main Menu..

Description: Main Menu, Phone Settings, GPRS, SMS, GPRS Attach..

Preamble: 	MFW750

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Phone Settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* gprs */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* SMS */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* GPRS */
(1)	|		| 	MNSMS_MO_SERV_REQ	|

|		*	==========================>	*

(2)	|		| 	MNSMS_MO_SERV_CNF	|

|		*	<==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* accept confirmation msg */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNSMS_MO_SERV_REQ
	mo_sms_serv	GPRS_SMS_GPRS_ONLY

- MNSMS_MO_SERV_CNF
	mo_sms_serv	GPRS_SMS_GPRS_ONLY

- History:

09-06-2003	ef		initial

08-10-2003	ef		menu change, GPRS is now 1st iem

### MFW758: SMS CSD Attach from the Main Menu..

Description: Main Menu, Phone Settings, GPRS, SMS, CSD Attach.

Preamble: 	MFW750

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Phone Settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* GPRS */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* SMS */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* CSD attach */
(1)	|		| 	MNSMS_MO_SERV_REQ	|

|		*	==========================>	*

(2)	|		| 	MNSMS_MO_SERV_CNF	|

|		*	<==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* accept confirmation msg */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNSMS_MO_SERV_REQ
	mo_sms_serv	GPRS_SMS_CCT_ONLY

- MNSMS_MO_SERV_CNF
	mo_sms_serv	GPRS_SMS_CCT_ONLY

- History:

09-06-2003	ef		initial

08-10-2003	ef		menu change, CSD is now 2nd item

### MFW759: SMS GPRS Prefered Attach from the Main Menu..

Description: Main Menu, Phone Settings, GPRS, SMS, GPRS Preferred Attach.

Preamble: 	MFW750

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Phone Settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* GPRS */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* SMS */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* GPRS prefered */
(1)	|		| 	MNSMS_MO_SERV_REQ	|

|		*	==========================>	*

(2)	|		| 	MNSMS_MO_SERV_CNF	|

|		*	<==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* accept confirmation msg */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNSMS_MO_SERV_REQ
	mo_sms_serv	GPRS_SMS_GPRS_PREF

- MNSMS_MO_SERV_CNF
	mo_sms_serv	GPRS_SMS_GPRS_PREF

- History:

10-06-2003	ef		initial

08-10-2003	ef		menu change, GPRS preferred is now 3rd item

### MFW760: Data Count from the Main Menu, Incoming

Description: Main Menu, Phone Settings, GPRS, Data Count, Incoming.

Preamble: 	MFW750

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Phone Settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* GPRS */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Data Count */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Incoming Calls */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* accept confirmation msg */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- History:

10-06-2003	ef		initial

### MFW761: Data Count from the Main Menu, Outgoing

Description: Main Menu, Phone Settings, GPRS, Data Count, Outgoing.

Preamble: 	MFW750

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Phone Settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* GPRS */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Data Count */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* Outgoing Calls */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* accept confirmation msg */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* back to idle screen */
TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- History:

10-06-2003	ef		initial

### MFW764: Main Menu, Define a PDP Context.

Description: Main Menu, Phone Settings, GPRS, GPRS Context Define.

Note1: These PDP tests are not necessary yet, as the Menu is not included in the build. However in the future….

Note2: The PDP context is hard coded GGSN name and IP address. This will change if ever this Menu is included.

Preamble: 	MFW750

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)		/* Menu */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* phone settings */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* GPRS */
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=DOWN)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* GPRS Context Define */
TIMEOUT_WAIT (5000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- History:

28-08-2003	ef		initial

### MFW766: Main Menu, Activate a PDP Context. This test fails as there is no menu for context define(yet).

Description: Main Menu, Phone Settings, GPRS, GPRS Activate.

Note:If and when a menu is used again for GPRS Context Definition and Activation then we can use this test.

Preamble: 	MFW764

APL/ACI		MMI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)		/* GPRS Activate */
(1)	|		| 	SMREG_PDP_ACTIVATE_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (5000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MAX_SLOT_ID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_NAME
	dti_linkid	DTI_LINKID_NAME
	dti_neighbor	DTI_NEIGHBOR_NAME
	dti_direction	SMREG_NEIGHBOR
	sdu	SMREG_CODING_OPTIONS

- History:

28-08-2003	ef		initial

## MMI Component Tests - case : SIM Application Toolkit.

### MFW800: SIM Toolkit request to display text, less than 128 bytes.

Description: In the Idle screen, the SIM application toolkit requests a display. The display waits for 10 seconds.

Preamble: 	MFW011

APL/ACI		MMI		PS

|		|		|

(1)	|		|	SIM_TOOLKIT_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (8000)

(2)	|		| 	SIM_TOOLKIT_RES	|

|		*	==========================>	*

TIMEOUT_WAIT (8000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_TOOLKIT_IND
	stk_cmd	STK_DISPLAY_TEXT_SHORT

- SIM_TOOLKIT_RES
	stk_cmd	STK_TERM_RESP_DISPLAY_TEXT

- History:

26-02-2003	ef		initial

### MFW802: SIM Toolkit Setup the Main Menu.

Description: In the Idle screen, the SIM application toolkit requests the main menu Application Toolkit.

Preamble: 	MFW011

APL/ACI		MMI		PS

|		|		|

(1)	|		|	SIM_TOOLKIT_IND	|

|		*	<==========================	*

(2)	|		| 	SIM_TOOLKIT_RES	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_TOOLKIT_IND
	stk_cmd	STK_SETUP_MENU

- SIM_TOOLKIT_RES
	stk_cmd	STK_TERM_RESP_SETUP_MENU

- History:

28-02-2003	ef		initial

### MFW804: SIM Toolkit Select Item From the Application Toolkit Menu.

Description: In the Sim Application Main Menu, the SIM requests an Item from the Toolkit Application Menu previously created. The Response is issued after the item is selected by the accept key.

Preamble: 	MFW802

APL/ACI		MMI		PS

|		|		|

(1)	|		|	SIM_TOOLKIT_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(2)	|		| 	SIM_TOOLKIT_RES	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_TOOLKIT_IND
	stk_cmd	STK_SELECT_ITEM

- SIM_TOOLKIT_RES
	stk_cmd	STK_TERM_RESP_SELECT_ITEM

- History:

28-02-2003	ef		initial

### MFW806: SIM Toolkit, The Menu Item selected requests a key input.

Description: From the Item selected , the SIM requests an answer from the keypad. Response issued after item is selected by accept key.

Preamble: 	MFW804

APL/ACI		MMI		PS

|		|		|

(1)	|		|	SIM_TOOLKIT_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (8000)

COMMAND (MMI CONFIG KEY_SEQUENCE=9)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(2)	|		| 	SIM_TOOLKIT_RES	|

|		*	==========================>	*

TIMEOUT_WAIT (15000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_TOOLKIT_IND
	stk_cmd	STK_GET_INKEY

- SIM_TOOLKIT_RES
	stk_cmd	STK_TERM_RESP_GET_INKEY

- History:

05-03-2003	ef		initial

### MFW808: SIM Toolkit,.The Menu Item requests a minimum of 3 characters.

Description: From the Item selected , the SIM requests an answer from the keypad. The minimum 3 characters are input and the Response issued after accept key pressed.

Preamble: 	MFW804

APL/ACI		MMI		PS

|		|		|

(1)	|		|	SIM_TOOLKIT_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (8000)

COMMAND (MMI CONFIG KEY_SEQUENCE=2)
TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE=3)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=4)
TIMEOUT_WAIT (2000)
COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(2)	|		| 	SIM_TOOLKIT_RES	|

|		*	==========================>	*

TIMEOUT_WAIT (8000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_TOOLKIT_IND
	stk_cmd	STK_GET_INPUT

- SIM_TOOLKIT_RES
	stk_cmd	STK_TERM_RESP_GET_INPUT

- History:

05-03-2003	ef		initial

### MFW810: SIM Toolkit,.Send a Short Message.

Description: From the Item selected , the SIM forwards a Short Message to the MMI for transmission to the network. After the SMS delivery report has been received, the Terminal response is forwarded to the SIM. Note that the SMS entity must have indicated enabled in the preamble.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

(1)	|		|	SIM_TOOLKIT_IND	|

|		*	<==========================	*

(2)	|		| 	MNSMS_SUBMIT_REQ	|

|		*	==========================>	*

(3)	|		|	MNSMS_SUBMIT_CNF	|

|			*<==========================*

(4)	|		| 	SIM_TOOLKIT_RES	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_TOOLKIT_IND
	stk_cmd	STK_SEND_SMS

- MNSMS_SUBMIT_REQ
	mem_type		MEM_SM
	rec_num		NUM_0
	condx		SMS_CONDX_OVR_NON
	modify		SMS_MODIFY_NON
	sms_sdu		SMS_SDU_STK

- MNSMS_SUBMIT_CNF
	mem_type	MEM_SM
	rec_num	SMS_RECORD_NOT_EXIST
	cause	SMS_NO_ERROR
	tp_mr	NUM_2
	sms_sdu	SMS_SDU_EMPTY

- SIM_TOOLKIT_RES
	stk_cmd	STK_TERM_RESP_SEND_SMS

- History:

07-03-2003	ef		initial

### MFW812: SIM Toolkit,.Send a Supplementry Service.

Description: From the Item selected , the SIM forwards a SS Message to the MMI for transmission to the network. If successful, the network sends an Activat containing a Release Complete message which is passed to the MMI. The Terminal response is then forwarded to the SIM.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

(1)	|		|	SIM_TOOLKIT_IND	|

|		*	<==========================	*

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		|	MNSS_END_IND	|

|			*<==========================*

(4)	|		| 	SIM_TOOLKIT_RES	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_TOOLKIT_IND
	stk_cmd	STK_SEND_SS

- MNSS_BEGIN_REQ
	ti	TI_MO_0
	fac_inf	STK_DIVERT_ALLCALLS
	ss_ver	VERSION_SS

- MNSS_END_IND
	ti	TI_MO_0
	cause	MNSS_CAUSE_CALL_CLEAR
	fac_inf	RELEASE_COMPLETE_FACILITY

- SIM_TOOLKIT_RES
	stk_cmd	STK_TERM_RESP_SEND_SS

- History:

10-03-2003	ef		initial

### MFW814: SIM Toolkit,.Send a Setup Call.

Description: From the Item selected , the SIM forwards Setup Call to the MMI after being accepted by the user, which then formats a Call Setup to the Network. If accepted by the network the MMI is Alerted and after connection the Terminal response is then forwarded to the SIM.

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

(1)	|		|	SIM_TOOLKIT_IND	|

|		*	<==========================	*

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(2)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

(3)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

(4)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
(5)	|		| 	SIM_TOOLKIT_RES	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_TOOLKIT_IND
	stk_cmd	STK_SETUP_CALL

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	STK_CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_END_TO_END_PLMN

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_END_TO_END_PLMN
	connected_number	CONNECTED_NUMBER_01317180537
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_TOOLKIT_RES
	stk_cmd	STK_TERM_RESP_SETUP_CALL

- History:

11-03-2003	ef		initial

### MFW816: SIM Toolkit,.Send a Setup Call. First attempt rejected due to busy ME. Automatic Redial. Test Fails.

Description: From the Item selected , the SIM forwards Setup Call to the MMI and is accepted by the user. However the mobile is Busy with another call which is eventually cleared down. The Sim automatically retries the call setup. If accepted by the network the MMI is Alerted and after connection the Terminal response is then forwarded to the SIM.

Preamble: 	MFW500

APL/ACI		MMI		PS

|		|		|

(1)	|		|	SIM_TOOLKIT_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (3000)

COMMAND (MMI CONFIG KEY_SEQUENCE=RSOFT)	/* clear call */
(2)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(3)	|		| 	MNCC_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|		| 	MNCC_RELEASE_IND	|

- |		*	<==========================	*

TIMEOUT_WAIT (6000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(5)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

(6)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

(7)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
(8)	|		| 	SIM_TOOLKIT_RES	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_TOOLKIT_IND
	stk_cmd	STK_SETUP_CALL_BUSY

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CALL_CLEAR

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_SPEECH
	bcpara2	BC_PARA_NO_SERVICE
	called_party	STK_CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_01317180537
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- SIM_TOOLKIT_RES
	stk_cmd	STK_TERM_RESP_SETUP_CALL

- History:

12-03-2003	ef		initial

### MFW830: SIM Toolkit,.SAT Class e. Test Fails.

Description: From the Item selected , the SIM forwards Setup Call to the MMI after being accepted by the user, which then formats a Call Setup to the Network. If accepted by the network the MMI is Alerted and after connection the Terminal response is then forwarded to the SIM.

A:	data circuit asynch UDI

B:	data circuit synch UDI

C:	PAD access asynch UDI

D:	packet access synch UDI

E:	data circuit asynch RDI

F:	data circuit synch RDI

G:	PAD access asynch RDI

H:	packet access synch RDI

Variants:		<A>..<B>

Preamble: 	MFW430

APL/ACI		MMI		PS

|		|		|

(1)	|		|	SIM_TOOLKIT_IND	|

|		*	<==========================	*

TIMEOUT_WAIT (10000)

COMMAND (MMI CONFIG KEY_SEQUENCE=LSOFT)
(2)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

(3)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(4)	|		|	MNCC_SYNC_IND	|
	|		*	<==========================	*

(5)	|		| 	MNCC_ALERT_IND	|

|		*	<==========================	*

(6)	|		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*
(7)	|		| 	RA_ACTIVATE_REQ	|

|		*	==========================>	*

(8)	|		|	RA_ACTIVATE_CNF	|
	|		*	<==========================	*

(9)	|		| 	L2R_ACTIVATE_REQ	|

|		*	==========================>	*

(10)	|		|	L2R_ACTIVATE_CNF	|
	|		*	<==========================	*

(11)	|		| 	L2R_CONNECT_REQ	|

|		*	==========================>	*

(12)	|		|	L2R_CONNECT_CNF	|
	|		*	<==========================	*

(13)	|		| 	SIM_DTI_REQ	|

|		*	==========================>	*

(14)	|		| 	L2R_DTI_REQ	|

|		*	==========================>	*

(15)	|		|	L2R_DTI_CNF	|
	|		*	<==========================	*

(16)	|		|	SIM_DTI_CNF	|
	|		*	<==========================	*

(17)	|		| 	SIM_TOOLKIT_RES	|

|		*	==========================>	*

TIMEOUT_WAIT (2000)

|		|		|

- Parametrization:

Primitive	Parameter	Value

- SIM_TOOLKIT_IND
<A>	stk_cmd	STK_OPEN_CHANNEL_IMM_CSD_DATA_CIRCUIT_ASYNC_UDI
<B>	stk_cmd	STK_OPEN_CHANNEL_IMM_CSD_DATA_CIRCUIT_SYNCH_UDI

- MNCC_SETUP_REQ
	ti	TI_MO_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
<A>	bcpara	BC_PARA_ASYNC_9600
<B>	bcpara	BC_PARA_SYNC_9600
	bcpara2	BC_PARA_NO_SERVICE
	called_party	STK_CLED_PARTY_01317180537
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- MNCC_SYNC_IND
	ti	TI_MO_0
	cause	MNCC_CAUSE_CHANNEL_SYNC
	chm	CHAN_MODE_DATA_9_6

- MNCC_ALERT_IND
	ti	TI_MO_0
	progress_desc	PROG_NOT_PRES

- MNCC_SETUP_CNF
	ti	TI_MO_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	CONNECTED_NUMBER_01317180537
	connected_number_sub	CONNECTED_PARTY_SUB_NONE

- RA_ACTIVATE_REQ
	model	RA_MODEL_RLP
	tra_rate	TRA_FULLRATE_9600
	user_rate	URA_9600
	ndb	NUM_DATA_BITS
	nsb	NUM_STOP_BITS_1

- RA_ACTIVATE_CNF
	ack_flg	RA_ACK

- L2R_ACTIVATE_REQ
	k_ms_iwf	L2r_WIN_SIZE_MS_IWF
	k_iwf_ms	L2R_WIN_SIZE_IWF_MS
	t1	L2R_ACK_TIMER
	t2	L2R_RELAY_DELAY_20
	n2	L2R_RETRANS_ATTEMPTS
	pt	L2R_COMPR_TYPE_V42BIS
	p0	L2R_COMP_DIR_NONE
	p1	L2R_NUM_CODE_WORDS
	p2	L2R_CODE_STR_LEN
	uil2p	L2R_ISO6429
	bytes_per_prim	L2R_BYTES_PER_PRIM_250
	buffer_size	L2R_BUFFER_SIZE_2048
	rate	L2R_FULLRATE_9600

- L2R_ACTIVATE_CNF
	ack_flg	L2R_ACK

- L2R_CONNECT_REQ

- L2R_CONNECT_CNF
	ack_flg	L2R_ACK

- SIM_DTI_REQ
	link_id	DTI_LINKID_NAME
	dti_conn	SIM_DTI_CONNECT_BIP_OPEN
	bip_ch_id	BIP_CH_ID_1
	con_type	SIM_CON_TYPE_SERIAL
	dti_direction	SIM_DTI_NORMAL_DIRECTION
	entity_name	NOT_USED
	local_ip	SIM_DTI_REQ_NOT_APPLICABLE
	destination_ip	SIM_DTI_REQ_NOT_APPLICABLE
	destination_port	SIM_DTI_REQ_NOT_APPLICABLE
	general_result	RSLT_PERF_SUCCESS
	add_info_result	SAT_NTW_CS_NO_ERROR
	release_time	SIM_DTI_REQ_NOT_APPLICABLE

- L2R_DTI_REQ
	dti_conn	L2R_CONNECT_DTI
	entity_name	NOT_USED
	link_id	DTI_LINKID_NAME
	dti_direction	SIM_DTI_NORMAL_DIRECTION

- L2R_DTI_CNF
	dti_conn	L2R_CONNECT_DTI
	link_id	DTI_LINKID_NAME

- SIM_DTI_CNF
	link_id	DTI_LINKID_NAME
	dti_conn	SIM_DTI_CONNECT_BIP_OPEN
	bip_ch_id	BIP_CH_ID_1

- SIM_TOOLKIT_RES
	stk_cmd	STK_TERM_RESP_SETUP_CLASSE

- History:

02-04-2003	ef		initial

02-09-2003	ef		continiue this test, add the L2R primitives