Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document

GSM Protocol Stack

G23

GDI – Generic Driver Interface

Functional Specification

(Edit via File/Properties/Summary!)

| Document Number: | 8415.026.99.020 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.18(Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1998-Sep-08 |
| Last changed: | 2003-May-16 by Joerg Deiss |
| File Name: | 8415_026.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1998-Sep-08 | LM et al. |  | 0.1 |  | 1 |
| 1998-Oct-28 | LM et al. |  | 0.2 |  | 2 |
| 1998-Nov-24 | MS et al. |  | 0.3 |  | 3 |
| 1998-Dec-14 | LM et al. |  | 0.4 |  | 4 |
| 1998-Dec-15 | LM et al. |  | 0.5 |  | 5 |
| 1998-Dec-17 | LM et al. |  | 0.6 |  | 6 |
| 1999-Mar-02 | MS et al. |  | 0.7 |  | 7 |
| 1999-Mar-10 | LM et al. |  | 0.8 |  | 8 |
| 1999-Jun-04 | LE et al. |  | 0.9 |  | 9 |
| 1999-Dec-01 | TSE et al. |  | 0.10 |  | 10 |
| 1999-Dec-08 | TSE et al. |  | 0.11 |  | 11 |
| 2000-Jan-11 | MP et al. |  | 0.12 |  | 12 |
| 2000-Jan-19 | MP et al. |  | 0.13 |  | 13 |
| 2000-Jan-19 | MP et al. |  | 0.14 |  | 14 |
| 2000-Jun-07 | MP |  | 0.15 |  | 15 |
| 2001-Oct-14 | SBK |  | 0.16 |  | 16 |
| 2001-Nov-13 | MP |  | 0.17 |  | 17 |

Notes:

- Initial version

- API changed

- Editorial

- Complete API change

- Editorial, auto data types changed

- Draft version

- New document template/English check

- Introduction of the chapter “Signals”/Clarification of the use of ProcHandle

- Consistency check/Submitted

- Introduction, variable type updated

- Drv_SignalID_Type updated

- Redesign of data types and functions

- T_DRV_SIGNAL modified

- T_DRV_SIGNAL modified

- Update the current gdi.h

- Editorial improvements, minor corrections

- Change length values to ULONG

Table of Contents

1.1	References	5

1.2	Abbreviations	5

3.1	Data types	7

3.1.1	T_DRV_DCB – Driver Control Block	7

3.1.2	T_DRV_CB_FUNC – Driver Call-Back Function	8

3.1.3	T_DRV_SIGNAL – Driver Signal	8

3.1.4	T_DRV_FUNC – Driver Functions	8

3.1.5	T_DRV_LIST_ENTRY – Driver List	8

3.1.6	T_DRV_EXPORT – Driver's Properties (exported by the Driver)	9

3.2	Constants	10

3.3	Signals	11

3.3.1	DRV_SIGTYPE_READ	11

3.3.2	DRV_SIGTYPE_WRITE	11

3.3.3	DRV_SIGTYPE_CLEAR	11

3.3.4	DRV_SIGTYPE_FLUSH	12

3.4	Driver Setup and Call-back Mechanism	13

3.5	Functions	14

3.5.1	drv_Init – Driver Initialization	15

3.5.2	drv_Exit – Driver Finalization	16

3.5.3	drv_Read - Read Data from the Driver	17

3.5.4	drv_Write – Write Data to the Driver	18

3.5.5	drv_Look – Look at Data from the Driver	19

3.5.6	drv_Clear – Clear internal Driver Buffers	20

3.5.7	drv_Flush – Flush internal Driver Buffers	21

3.5.8	drv_SetSignal – Setup a Signal	22

3.5.9	drv_ResetSignal – Remove a Signal	23

3.5.10	drv_SetConfig – Set a Driver Configuration	24

3.5.11	drv_GetConfig – Retrieve the Driver Configuration	25

3.5.12	drv_Callback – Callback Function of the Driver	26

A.	Acronyms	27

B.	Glossary	27

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[C_8415.033]	8415.033, VSI/PEI- Frame/Body Interfaces; Condat

## Abbreviations

API	Application Programming Interface

GDI	Generic Driver Interface

NVRAM	Non Volatile Random Access Memory

# Introduction

G23 is a software package implementing Layers 2 and 3 of the ETSI-defined GSM air interface signaling protocol, and as such represents the part of a GSM mobile station's protocol software which is both, platform and manufacturer independent. Therefore, G23 can be viewed as a building block providing standardized functionality through generic interfaces for easy integration.

The G23 suite of products consists of the following items:

- Layers 2 and 3 for speech & short message services,

- Layers 2 and 3 for fax & data services,

- Application Control Interface/AT Command Interface,

- MMI and MMI Framework (MFW) and

- Test and integration support tools.

This document describes the general design of the drivers and driver interfaces (driver APIs) used in the G23 Protocol Stack. Drivers are function libraries that export reusable functionality for different applications (processes). In general, drivers do not attend to scheduling.

There are two kinds of drivers recognized in the context of the G23 Protocol Stack, communication drivers and "plain" function drivers.

Communication drivers include communication functionality. This means that they can send data to and receive data from a hardware device such as a serial interface. The communication drivers signal specific events, such as the reception of data, to the parent application using one of two methods (signals or call-backs). Signals are part of the common driver API specification. Call-back functionality is an optional implementation and is therefore not described in this document.

In contrast to communication drivers, plain drivers act as simple common function libraries that can access hardware devices. An LED driver, or a ring buffer manager are examples for such a plain drivers. They allow processes to write in and read from the NVRAM via one standardized interface, the driver.

The following chapters describe the functions from which the drivers must be derived. There is also a differentiation between plain drivers and communication drivers, in that a different amount of standard functions must be implemented. All drivers must implement the functions drv_Init() and drv_Exit(). All communication drivers must implement the functions drv_Read(), drv_Write(), drv_Look(), drv_Flush(),  drv_SetSignal(), drv_ResetSignal() and drv_Clear(). Drivers may implement the functions drv_SetConfig(), drv_GetConfig() and drv_Callback().

| function | plain driver | communica-tion driver |
| --- | --- | --- |
| drv_Init() | mandatory | mandatory |
| drv_Exit() | mandatory | mandatory |
| drv_Read() | optional | mandatory |
| drv_Write() | optional | mandatory |
| drv_Look() | optional | mandatory |
| drv_Flush() | optional | mandatory |
| drv_SetSignal() | optional | mandatory |
| drv_ResetSignal() | optional | mandatory |
| drv_Clear() | optional | mandatory |
| drv_SetConfig() | optional | optional |
| drv_GetConfig() | optional | optional |
| drv_Callback() | optional | optional |

drv signifies the name of the specific driver e.g. emi_Init(). Instances of drv will have to be substituted by a sequence of letters denoting the actual driver.

The standard driver API as defined by this document may be extended by adding additional functions, constants and data types.

# Generic Driver Interface

## Data types

Name	Description

ULONG					Unsigned 32 bit integer data type

USHORT					Unsigned 16 bit integer data type

SHORT		Signed 16 bit integer data type

T_VOID_STRUCT				Unsigned 32 bit integer data type (equivalent to unsigned long)

T_DRV_DCB		Driver Control Block (may be different for different drivers)
T_DRV_SIGNAL		Signal information data type

T_DRV_CB_FUNC		Signal call-back function type

T_DRV_FUNC				Driver's function type

T_DRV_LIST				Drivers List data type

T_DRV_EXPORT				Driver's property type (exported by the driver)

### T_DRV_DCB – Driver Control Block

Definition:

typedef struct T_DRV_DCB
{
	...
 } T_DRV_DCB;

Description:

The driver control block data type T_DRV_DCB is a prototype for driver-specific implementations of a driver control block. A driver control block contains all parameters used to configure a driver.

### T_DRV_CB_FUNC – Driver Call-Back Function

Definition:

typedef void (*T_DRV_CB_FUNC) (T_DRV_SIGNAL * Signal) ;

Description:

This type defines a call-back function used to signal driver events, e.g. driver is ready to accept data. The driver calls the signal call-back function when a specific event occurs and the driver has been instructed to signal the event to a specific process (see 3.5.8).

A process can set or reset event signaling by calling one of the driver functions drv_SetSignal(), drv_ResetSignal(). Event signaling can only be performed when a signal call-back function has been installed at driver initialization.

The signal call-back has only one single parameter Signal containing all data required to identify the signal. For more information about the T_DRV_SIGNAL data type, refer to 3.1.3.

### T_DRV_SIGNAL – Driver Signal

Definition:

typedef struct
{
	USHORT	SignalType;
	USHORT	DrvHandle;
	ULONG	DataLength;
	T_VOID_STRUCT *	UserData;
} T_DRV_SIGNAL

Description:

This type defines the signal information data used to identify a signal. This data type is used to define and to report a signal. A signal is defined by a process by calling the driver function drv_SetSignal(). An event is signaled by a driver by calling the pre-defined signal call-back function (see 3.5.1).

### T_DRV_FUNC – Driver Functions

Definition:

typedef struct
{

void (*drv_Exit)();

USHORT (*drv_Read)();

USHORT (*drv_Write)();

USHORT (*drv_Look)();

USHORT (*drv_Clear)();

USHORT (*drv_Flush)();

USHORT (*drv_SetSignal)();

USHORT (*drv_ResetSignal)();

USHORT (*drv_SetConfig)();

USHORT (*drv_GetConfig)();

void (*drv_Callback)();
} T_DRV_FUNC

Description:

This type defines the functions exported by the driver. If any of the functions is not implemented for a driver, a NULL-pointer has to be entered in this table.

### T_DRV_LIST_ENTRY – Driver List

Definition:

typedef struct
{
   char *	Name;

USHORT               	 (*drv_Init)( USHORT, T_DRV_CB_FUNC, T_DRV_EXPORT const **);

char * 	Process;

void *                  	DrvConfig;
} T_DRV_LIST_ENTRY

Description:

This data type defines the parameters needed to setup a driver.

Name		name of the driver

drv_Init()	driver initialization function

Process		process to be notified by driver call-back, e.g. "TST"

DrvConfig	pointer to driver control block

### T_DRV_EXPORT – Driver's Properties (exported by the Driver)

Definition:

typedef struct
{
	char *	Name
	USHORT	Flags
	T_DRV_FUNC	DrvFunc

} T_DRV_EXPORT

Description:

This data type defines the properties exported by the driver.

Name		Name of the driver

Flags		Bit (0): Call-back function is called during ISR(1)/not called during ISR(0)

DrvFunc		functions to access the driver

## Constants

Name	Description

DRV_BUFFER_FULL	The internal buffer is exhausted
DRV_DISABLED	Driver is not enabled
DRV_ENABLED 	Driver is enabled
DRV_NOTCONFIGURED	Driver is not configured
DRV_INITFAILURE	Driver initialization failed
DRV_INITIALIZED	Driver is already initialized
DRV_INTERNAL_ERROR	Unspecified internal driver error
DRV_INPROCESS	The requested function is currently being executed
DRV_INVALID_PARAMS	One or more parameters are out of range or invalid
DRV_OK	Return value indicating the function completed successfully
DRV_SIGFCT_NOTAVAILABLE	The requested event signaling functionality is not available
DRV_SIGTYPE_CLEAR	Used to specify clear ready event signaling (refer to 3.5.8)
DRV_SIGTYPE_FLUSH	Used to specify flush ready event signaling (refer to 3.5.8)
DRV_SIGTYPE_READ	Used to specify read event signaling (refer to 3.5.8)
DRV_SIGTYPE_USER	Used to specify additional driver dependent signals (refer to 3.5.8)
DRV_SIGTYPE_WRITE	Used to specify write event signaling (refer to 3.5.8)
DRV_BUFTYPE_WRITE	Used to specify the write buffer type of buffer (refer to 3.5.6)
DRV_BUFTYPE_READ	Used to specify the read buffer type of buffer (refer to 3.5.6)
DRV_MAX_SIGNAL	Maximum number of processes to be notified by a driver call-back

## Signals

Signals are used to inform the using process about selected events asynchronously. Signaling is done by passing a signal call-back function to the driver at the time of initialization (see “3.5.1 drv_Init – Driver Initialization”). When no call-back function is defined event signaling cannot be performed. A signal can be set using the function drv_SetSignal() which can be found in Chapter 3.5.8. Event signaling can be disabled by calling the function drv_ResetSignal(), for more details on this function refer to Chapter 3.5.9.

The following chapters describe the contents of the T_DRV_SIGNAL information structures defined for the common driver signals.

### DRV_SIGTYPE_READ

This signal is indicated when the driver has received data which can now be read using the drv_Read() function. A prerequisite to being informed asynchronously about this event is that the signal has been set using the drv_SetSignal() function. The event will only be signaled each time new data is available. The behavior depends on the driver's functionality i.e. the event may be signaled for example each time a character is received via an RX line of a RS232 HW or each time a complete data block is available. Call the drv_Look() or drv_Read() function as a reaction to the signal.

| Parameter | Value |
| --- | --- |
| SignalType | DRV_SIGTYPE_READ |
| DataLength | not used |
| UserData | not used |

### DRV_SIGTYPE_WRITE

This signal will be indicated when the write buffer is ready to take new data. A prerequisite to being informed asynchronously about this event is that the signal has been set using the drv_SetSignal() function. The event will only be signaled once, however each time the write buffer is ready to take new data using the drv_Write() function.

| Parameter | Value |
| --- | --- |
| SignalType | DRV_SIGTYPE_WRITE |
| DataLength | not used |
| UserData | not used |

### DRV_SIGTYPE_CLEAR

This signal is indicated when the read and/or write buffer of the driver has been cleared asynchronously. Prerequisite to being informed asynchronously about this event is that the signal has been set using the drv_SetSignal() function and a call to the drv_Clear() function has been performed which returned DRV_INPROCESS meaning that the buffers could not be cleared at once (synchronously) or clearing is currently in process. The event will be signaled as soon as the selected buffer(s) is/are cleared.

| Parameter | Value |
| --- | --- |
| SignalType | DRV_SIGTYPE_CLEAR |
| DataLength | sizeof(USHORT) |
| UserData | Buffertype (Bitmask, values: DRV_BUFTYPE_WRITE, DRV_BUFTYPE_READ) |

### DRV_SIGTYPE_FLUSH

This signal is indicated when the driver buffer has been flushed asynchronously. Prerequisite to being informed asynchronously about this event is that the signal has been set using the drv_SetSignal() function and that a call to the drv_Flush() function has been performed which returned DRV_INPROCESS meaning that the buffers could not be flushed at once (synchronously) or flushing is currently being performed.

| Parameter | Value |
| --- | --- |
| SignalType | DRV_SIGTYPE_FLUSH |
| DataLength | not used |
| UserData | not used |

## Driver Setup and Call-back Mechanism

All test interface drivers used in the protocol stack are entered in a driver list. In this list the address of the drv_Init() function, the names of the entitiy, that must be notified in the case of a driver call-back, and a pointer to a default configuration string.

The frame reads the driver list, calls the drv_Init() function and stores the handles of the process to be in a table under the index corresponding to the index in the driver list. This index is also passed to the drv_Init() function and serves as the driver handle. The address of a call-back function that is located in the frame is also passed to the drv_Init() function and a pointer to the properties exported by the driver is stored in the driver table.

In the case of a test interface driver call-back, the frame finally notifies the process which name is entered for the uppermost driver in the driver list and passes a signal to it if the corresponding signal type is enabled. The frame acts as dispatcher.

To enable/disable signal types the functions vsi_d_setsignal() or vsi_d_resetsignal() must be used, refer to [C_8415.033].

## Functions

Name	Description

drv_Init	Initialization of the driver
drv_Exit	Termination of the driver
drv_Clear	Re-initialize all buffers
drv_Write	Write data to the driver 
drv_Look	Read data from a driver but leave data unchanged 
drv_Read	Read data from the driver
drv_Flush	Flush all buffers
drv_SetSignal	Define a signal the driver uses to indicate an event
drv_ResetSignal	Un-define a signal the driver uses to indicate an event
drv_SetConfig	Set driver configuration 
drv_GetConfig	Get driver configuration
drv_Callback	Callback function of the higher layer driver
drv_Tick	Entry for driver external clock generation

### drv_Init – Driver Initialization

Definition:

USHORT drv_Init
(

U16      	Caller

USHORT	DrvHandle

T_DRV_CB_FUNC 	CallbackFunc

T_DRV_EXPORT **	DrvInfo

) ;

Parameters:

Name	Description

Caller	handle of calling process

DrvHandle	unique handle of the driver

CallbackFunc	This parameter points to the function that is called at the time an event that is to be signaled occurs. This value can be set to NULL if event signaling should not be possible. This function must not be confused with the drv_Callback() function in 3.5.12

DrvInfo	Pointer to the driver parameters.

Return values:

Name	Description

DRV_OK	Initialization successful
DRV_INITIALIZED 	Driver already initialized
DRV_INITFAILURE	Initialization failed

Description

This function needs to be implemented in all drivers.

The driver exports its properties like its name, the functions to access driver functionality and a bitfield called flags. As the drivers exports only the address of its properties, these must not be located on the stack but as static structure.

The function initializes the driver's internal data. The function returns DRV_OK in the case of a successful completion.

The function returns DRV_INITIALIZED if the driver has already been initialized and is ready to be used or is already in use. In case of an initialization failure, which means that the driver cannot be used, the function returns DRV_INITFAILURE.

### drv_Exit – Driver Finalization

Definition:

void drv_Exit
(
	U16      	Caller

void

) ;

Parameters:

Name	Description

Caller	handle of calling process

-	-

Return values:

Name	Description

-	-

Description

This function needs to be implemented in all drivers.

The function is called when the driver functionality is no longer needed. The function “de-allocates” all allocated resources and finalizes the driver.

### drv_Read - Read Data from the Driver

Definition:

USHORT drv_Read
(

U16	Caller
	void *	Buffer
	ULONG *	Length
) ;

Parameters:

Name	Description

Caller	handle of calling process

Buffer	This parameter points to the buffer wherein the data is to be copied

Length	On call: number of characters to read. If the function returns DRV_OK, it contains the number of characters read. If the function returns DRV_INPROCESS, it contains 0.

Return values:

Name	Description

DRV_OK	Function successful

DRV_INPROCESS	The driver is currently reading data. The data is incomplete.

Description

This function needs to be implemented in all communication drivers.

This function is used to read data from a driver. The data is copied into the buffer to which Buffer points. The parameter *Length contains the size of the buffer in characters.

In the case of a successful completion, the driver's buffer is cleared. The driver keeps the data available when calling the function drv_Look().

NOTE: When calling the function with a buffer size of 0, the function will return DRV_OK. The size of the buffer needed to store the available data is stored in the parameter *Length. In this case, Buffer can be set to NULL.

### drv_Write – Write Data to the Driver

Definition:

USHORT drv_Write
(
	U16	Caller
	void *	Buffer
	ULONG *	Length
) ;

Parameters:

Name	Description

Caller	handle of calling process

Buffer	This parameter points to the buffer that is passed to the driver for further processing

Length	On call: number of characters to write. If the function returns DRV_BUFFER_FULL, it contains the maximum number of characters that can be written. If the function returns DRV_OK, it contains the number of characters written. If the function returns DRV_INPROCESS, it contains 0.

Return values:

Name	Description

DRV_OK	Function successful
DRV_BUFFER_FULL	Not enough space
DRV_INPROCESS	Driver is busy writing data

Description

This function needs to be implemented in all communication drivers.

This function is used to write data to the driver. The parameter * Length contains the number of characters to write.

In the case of a successful completion, the function returns DRV_OK.

If the data cannot be written because the storage capacity of the driver has been exhausted, the function returns DRV_BUFFER_FULL and the maximum number of characters that can be written in * Length.

If the driver is currently busy writing data and therefore cannot accept further data to be written, it returns DRV_INPROCESS and sets the parameter *Length to 0.

NOTE:	When calling the function with a buffer size of 0, the function will return the number of characters that can be written in the parameter *Length. In this case, Buffer can be set to NULL.

### drv_Look – Look at Data from the Driver

Definition:

USHORT drv_Look
(
	U16	Caller
	void *	Buffer
	ULONG *	Length
) ;

Parameters:

Name	Description

Caller	handle of calling process

Buffer	This parameter points to the buffer wherein the data is to be copied

Length	On call: number of characters to read. If the function returns DRV_OK, it contains the number of characters read. If the function returns DRV_INPROCESS, it contains 0.

Return values:

Name	Description

DRV_OK	Function successful

DRV_INPROCESS	The driver is currently reading data. The data is incomplete.

Description

This function needs to be implemented in all communication drivers.

This function is used to read data from the driver. The data is copied into the buffer to which Buffer points. The parameter *Length contains the size of the buffer in characters. The driver's internal buffer is not cleared.

In the case of a successful completion, the function returns DRV_OK and sets the value of *Length to the number of characters read.

NOTE:	When calling the function with a buffer size of 0, the function will return DRV_OK. The size of the buffer needed to store the available data is stored in the parameter *Length. In this case, Buffer can be set to NULL.

### drv_Clear – Clear internal Driver Buffers

Definition:

USHORT drv_Clear
(
	U16	Caller
	USHORT	BufferType
) ;

Parameters:

Name	Description

Caller	handle of calling process

BufferType	Bit-mask used to specify if the read, write or read and write buffer is cleared

Return values:

Name	Description

DRV_OK	Function successful

DRV_INPROCESS	The driver could not complete the clearance of the buffers at once. The driver is busy clearing the buffers.

Description

This function needs to be implemented in all communication drivers.

This function is used to clear the driver's internal buffers. The parameter BufferType is used to specify which buffer is to be cleared. The value of BufferType can be one of the values or a combination of the values defined in the following table. Combining DRV_BUFTYPE_READ and DRV_BUFTYPE_WRITE using bit wise OR will cause the driver to clear the read and write buffers.

| Buffer type | Value |
| --- | --- |
| DRV_BUFTYPE_WRITE | 1 |
| DRV_BUFTYPE_READ | 2 |

Figure 1

### drv_Flush – Flush internal Driver Buffers

Definition:

USHORT drv_Flush
(
	U16	Caller

void
) ;

Parameters:

Name	Description

Caller	handle of calling process

-	-

Return values:

Name	Description

DRV_OK	Function successful

DRV_INPROCESS	The driver could not complete flushing the buffers at once. The driver is busy flushing the buffers.

Description

This function needs to be implemented in all communication drivers.

This function is used to flush the driver's internal buffers. This means data that is currently stored in the driver's internal write buffer is written to the device at once.

### drv_SetSignal – Setup a Signal

Definition:

USHORT drv_SetSignal
(
	U16	Caller
	USHORT	SignalType
) ;

Parameters:

Name	Description

Caller	handle of calling process

SignalType	Signal type to be set

Return values:

Name	Description

DRV_OK	Function completed successfully
DRV_INVALID_PARAMS	Invalid signal type
DRV_SIGFCT_NOTAVAILABLE	Event signaling functionality is not available

Description

This function needs to be implemented in all communication drivers.

This function is used to define a single signal or multiple signals that is/are indicated to the process when the event identified by SignalType occurs. Standard signals are defined in the following table. The signals can be extended for derived drivers, using DRV_SIGTYPE_USER shifted left to define new signals.

| Signal | Value |
| --- | --- |
| DRV_SIGTYPE_WRITE | 0x0001 |
| DRV_SIGTYPE_READ | 0x0002 |
| DRV_SIGTYPE_FLUSH | 0x0004 |
| DRV_SIGTYPE_CLEAR | 0x0008 |
| DRV_SIGTYPE_USER | 0x0010 |

Figure 2

To remove a signal, call the function drv_ResetSignal().

If the passed signal type is not implemented, the driver returns DRV_INVALID_PARAMS.

If no signal call-back function has been defined at the time of initialization, the driver returns DRV_SIGFCT_NOTAVAILABLE.

This function is normally not called directly by the process, but indirectly via the frame function vsi_d_setsignal(): For dynamic enabling of signals, the process calls vsi_d_setsignal() to enable the frame to store the new signal type mask for the calling process (refer to 3.4).

### drv_ResetSignal – Remove a Signal

Definition:

USHORT drv_ResetSignal
(
	U16	Caller
	USHORT	SignalType
) ;

Parameters:

Name	Description

Caller	handle of calling process

SignalType	Signal type to be reset

Return values:

Name	Description

DRV_OK	Function completed successfully
DRV_INVALID_PARAMS	Invalid signal type
DRV_SIGFCT_NOTAVAILABLE	Event signaling functionality is not available

Description

This function needs to be implemented in all communication drivers.

It is used to remove a single signal or multiple signals that have previously been set. The signal type is a bit mask containing the signal(s) to be reset.

If the passed signal type is not implemented the driver returns DRV_INVALID_PARAMS.

If no signal call-back function has been defined at the time of initialization, the driver returns DRV_SIGFCT_NOTAVAILABLE.

This function is normally not called directly by the process but indirectly via the frame function vsi_d_resetsignal(): For dynamic disabling of signals the process calls vsi_d_resetsignal() to enable the frame to store the new signal type mask for the calling process (refer to 3.4).

### drv_SetConfig – Set a Driver Configuration

Definition:

USHORT drv_SetConfig
(
	U16	Caller
	T_DRV_DCB *	DCBPtr
) ;

Parameters:

Name	Description

Caller	handle of calling process

DCBPtr	Pointer to the driver control block

Return values:

Name	Description

DRV_OK	Function successfully completed
DRV_INVALID_PARAMS	One or more values are out of range or invalid in that combination

Description

Implementation of this function is optional for all types of drivers and governed by the functional specification of the respective driver.

This function is used to configure a driver (transmission rate, flow control, etc). For detailed information about the contents of the driver control block, refer to the functional specification of the specific driver.

If any value of the configuration is out of range or invalid in combination with any other value of the configuration, the function returns DRV_INVALID_PARAMS.

Call the drv_GetConfig() function to retrieve the driver's current configuration.

### drv_GetConfig – Retrieve the Driver Configuration

Definition:

USHORT drv_GetConfig
(
	U16	Caller
	T_DRV_DCB *	DCBPtr
) ;

Parameters:

Name	Description

Caller	handle of calling process

DCBPtr	Pointer to the driver control block

Return values:

Name	Description

DRV_OK	Function successfully completed
DRV_NOTCONFIGURED	The driver is not yet configured

Description

Implementation of this function is optional for all types of drivers and governed by the functional specification of the respective driver.

This function is used to retrieve the configuration of the driver. The configuration is returned in the driver control block to which the pointer provided DCBPtr points. For detailed information about the contents of the driver control block, refer to the functional specification of the specific driver.

Some drivers may have a default configuration. If these drivers have not been configured a call of drv_GetConfig() returns a pointer to a control block containing this default configuration data and DRV_OK as return value. For drivers that have to be configured, in this case DRV_NOTCONFIGURED is returned.

Call the drv_SetConfig() function to configure the driver.

### drv_Callback – Callback Function of the Driver

Definition:

void drv_Callback
(
	U16	Caller
	T_DRV_SIGNAL *	Signal
) ;

Parameters:

Name	Description

Caller	handle of calling process

Signal	Pointer to the driver signal (information data)

Return values:

Name	Description

-	-

Description

This function must not be confused with the parameter CallbackFunc passed to drv_Init().

This function is only needed for cascaded drivers where the lower layer driver calls the call-back function of the upper layer driver via the frame and thus in general optional. It is the callback entry for the lower layer driver.

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |