Report Nr 3559 at GNATS

How to adapt to the changes in CCD and CCDGEN to support

Encoding and decoding of SI 4 Rest Octet

- The chapter 3 of the message document will have a new entry:

# Types

Definition:

| name | add bit | ctrl | Comment |
| --- | --- | --- | --- |
| GSM1_V | 0 |  | mandatory IE V-component in one nibble |
| GSM1_TV | 4 | optional | optional with V-Component |
| GSM2_T | 8 | optional | optional IE, contains IEI only (no V-component) |
| GSM3_V | 0 |  | mandatory / conditional, no IEI, V-component only |
| GSM3_TV | 8 | optional | optional IE with V-component |
| GSM4_LV | 8 |  | mandatory / conditional, length and V-component |
| GSM4_TLV | 16 | optional | optional IE with length indicator and V-component |
| GSM5_V | 0 | optional | optional IE (bitstream to end of message) |
| BCDODD | 4 |  | binary coded decimal number starting with digit1 |
| BCDEVEN | 4 |  | binary coded decimal number starting with digit2 |
| CSN1_S1 | 1 | optional | CSN1 coded value with 0/1 tag |
| CSN1_SHL | 1 | optional | CSN1 coded value with L/H tag |
| S_PADDING | 0 | optional | spare padding |
| LH_FLAG | 0 |  | presence bit for XOR-ed pairs of CSN1_SHL elements |

And the message description in rr.doc should look like this:

Elements:

| long name | Short name | ref | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- | --- |
| SI4 Rest Octets_O | Blabla |  | Xx | O |  |  |
| L H Bit | flag |  | 1 | M | HL_FLAG |  |
| Break Indicator | break_indic |  | xx | O |  | {flag=0} |
| SI Rest Octets_S | si4_rest_octets_s |  | xx | O |  | {flag=1} |
| Spare Padding | .00101011 |  | 0-7 | O | S_PADDING | (22) |

Please keep in mind that the value 0 for flag is associated with the bit evaluation L. The value 1 refers to H.

2) The type definition will look like this:

typedef struct

{

T_si4_rest_octets_o     si4_rest_octets_o;

UBYTE      flag;

UBYTE                        v_break_indic;          /* validity-flag */

T_ break_indic             break_indic;

UBYTE                        v_si4_rest_octets_s; /* validity-flag */

T_si4_rest_octets_s     si4_rest_octets_s;

} T_si4_rest_octets;

3) What does a PS developer need to do when encoding:

In the following example we assume the presence of break_indic in the air message

- set flag to 0;

- set v_break_indic to 1 and v_si4_rest_octets_s to 0; complete the C-Structure for break_indic;

- Call ccd_codeMsg();

4) What does a PS developer need to do when decoding:

In the following example we assume the presence of break_indic in the air message

- Call ccd_decodeMsg();

- You can use both v_break_indic and v_ si4_rest_octets_s to check which of them is present. Alternatively you can use the flag to distinguish between the two cases.

- Process the entries in the C-Structure for break_indic completely or si4_rest_octets_s depending on the information in flag or v_break_indic and v_ si4_rest_octets_s;

# List of modules to be changed

- rr.doc: message document as described above

- ccdgen: no change in the current version; (In future we need a fixed coding type number for HL_FLAG)

- ccd: new encoding and decoding functions for CCDTYPE_HL_FLAG

- test: test cases related to SI4 Rest Octets may need to be updated (??)

GLOBAL SHORT cdc_decode_HL_FLAG (USHORT cRef, USHORT eRef, T_CCD_Globs *globs)

{

*(globs->pstruct + globs->pstructOffs) = (UBYTE) GET_HL_PREV (bf_readBit (globs) );

globs->pstructOffs++;

}

GLOBAL SHORT cdc_encode_HL_FLAG (USHORT cRef, USHORT eRef, T_CCD_Globs *globs)

{

bf_writeBit (GET_HL (globs->pstruct + globs->pstructOffs), globs);

globs->pstructOffs++;

}