Table of Contents

GSM Fax & Data Services

Test Specification

ACI LC

Author:	Texas Instruments Berlin AG
	Alt-Moabit 91d
	D-10559 Berlin
	Germany

Date:	24. September 2002 
Document No.:	8462.400.01.00
File:	input.doc

0	Table of Contents	2

1	Document Control	3

1.1	Document History	3

1.2	References	4

1.3	Abbreviations	7

1.4	Terms	9

2	Overview	10

2.1	RA - Rate Adaptation	10

2.2	RLP - Radio Link Protocol	10

2.3	L2R - Layer 2 Relay Functionality	10

2.4	FAD 03.45 - Fax Adaptation Protocol	11

2.5	T.30 - Fax Protocol Entity	11

2.6	ACI - AT Command Interpreter	11

2.7	USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	11

3	Parameters	12

4	TEST CASES	19

4.1	Routing (internal) (ACILC001 -)	19

4.1.1	ACILC001:  Setup the Routing and the PCO view for the ACI test, and set ACI to transparent mode	19

4.2	Initialisation (ACILC011 - ACILC020)	20

4.2.1	ACILC011:  Power On	20

4.2.2	ACILC020:  Set/Query Location Service Source and Destination Address	22

4.2.3	ACILC021:  Initialize Location Service, receive SMS with MLC source address.(CLSA)	23

4.2.4	ACILC022:  Set Operating Mode of Location Service (CLOM)	26

4.2.5	ACILC023:  Set Operating Mode of Location Service 2 (CLOM)	30

4.2.6	ACILC024:  Set Operating Mode of Location Service 3 (CLOM)	32

4.2.7	ACILC025:  Set Periodic Update  of Location Service (CLPS, failed, is to rebuilt)	34

4.2.8	ACILC026:  Set Privacy Parameter for LC client (CLSR)	38

4.2.9	ACILC027:  Request position data, send data in SMS to MLC destination address.(CLSA, failed currently)	40

# Document Control

|	Copyright Condat AG, 2002.

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Condat GmbH reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Condat GmbH. Condat GmbH accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a licence agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Condat GmbH.

Texas Instruments Berlin AG

Alt Moabit 91d

10559 Berlin

Germany

- Telephone:	+49.30.3983-0

Fax:	+49.30.3983-

Internet:

E-mail:

## Document History

Document Id.	Date	Author	Remarks

8462.400.01.00.	24 September 2002	RM	Initial

## References

[1]	Rec. T.4 Standardisation of group 3 facsimile apparatus for document transmission;
(CCITT-T.4, 1984)

[2]	ITU-T Recommendation T.30; Series T: Terminal equipments and protocols for telematic services;
Procedures for document facsimile transmission in the general switched
telephone network;
(ITU-T.30, 1996)

[3]	ITU-T Recommendation T.31; Terminals for telematic services;
Asynchronous facsimile DCE control - service class 1
(ITU-T.31, 1995)

[4]	ITU-T Recommendation T.32; Terminals for telematic services;
Asynchronous facsimile DCE control - service class 2
(ITU-T.32, 1995)

[5]	Rec. T.35; Terminal equipment and protocols for telematic services;
Procedures for the allocation of CCITT definde codes for non-standard facilities;
(CCITT-T.35, 1991)

[6]	ITU-T Recommendation V.25 ter; Series V: data communication over the telephone network;
Interfaces and voiceband modems; Serial asynchronous automatic dialling and control
(ITU-T V.25 ter, 1997)

[7]	Rec. V.42 bis Data compression procedures for data circuit terminating equipment (DCE) using error correction procedures;
(CCITT-V.42 bis, 1990)

[8]	Rec. V.110 (Blue book, Vol. VIII, Fascicle VIII.1) Support of data terminal equipments (DTEs) with V-series type interfaces by an integrated services digital network (ISDN);
(CCITT-V.110, 1988)

[9]	European digital cellular telecommunications system (Phase 2);
GSM Public Land Mobile Network (PLMN) connection types;
(GSM 3.10, September 1994, version 4.3.1)

[10]	European digital cellular telecommunications system (Phase 2);
Technical realisation of facsimile group 3 transparent;
(GSM 3.45, September 1995, version 4.5.0)

[11]	Digital cellular telecommunications system (Phase 2);
Mobile radio interface layer 3 specification;
(GSM 4.08, November 1996, version 4.17.0)

[12]	European digital cellular telecommunications system (Phase 2);
Rate adaptation on the Mobile Station - Base Station System (MS - BSS) Interface;
(GSM 4.21, May 1995, version 4.6.0)

[13]	European digital cellular telecommunications system (Phase 2);
Radio Link Protocol (RLP) for data and telematic services on the Mobile Station - Base Station System (MS - BSS) interface and the Base Station System - Mobile-service Switching Centre (BSS - MSC) interface
(GSM 4.22, September 1994, version 4.3.0)

[14]	European digital cellular telecommunications system (Phase 2);
Radio Link Protocol (RLP) for data and telematic services on the Mobile Station - Base Station System (MS - BSS) interface and the Base Station System - Mobile-service Switching Centre (BSS - MSC) interface
(Amendment prA1 for GSM 4.22, version 4.3.0)
(GSM 4.22, March 1995, version 4.4.0)

[15]	European digital cellular telecommunications system (Phase 2);
General on Terminal Adaptation Functions (TAF) for Mobile Stations (MS);
(GSM 7.01, December 1995, version 4.10.0)

[16]	European digital cellular telecommunications system (Phase 2);
Terminal Adaptation Functions (TAF) for services using asynchronous bearer capabilities;
(GSM 7.02, September 1994, version 4.5.1)

[17]	European digital cellular telecommunications system (Phase 2);
Terminal Adaptation Functions (TAF) for services using synchronous bearer capabilities;
(GSM 7.03, September 1994, version 4.5.1)

[18]	Digital cellular telecommunications system (Phase 2);
Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Services (CBS);
(GSM 7.05, November 1996, version 4.8.0)

[19]	Digital cellular telecommunications system (Phase 2);
AT command set for GSM Mobile Equipment (ME)
(GSM 7.07, May 1996, version 4.1.0)

[20]	Digital cellular telecommunication system (Phase 2);
Mobile Station (MS) conformance specification;
Part 1: Conformance specification
(GSM 11.10-1, November 1996, version 4.17.0)

[21]	Digital cellular telecommunications system (Phase 2);
Mobile Station (MS) conformance specification;
Part 2: Protocol Implementation Conformance Statement (PICS)
proforma specification
(GSM 11.10-2, May 1996, version 4.15.0)

[22]	Digital cellular telecommunications system (Phase 2);
Mobile Station (MS) conformance specification;
Part 3: Layer 3 (L3) Abstract Test Suite (ATS)
(GSM 11.10-3, November 1996, version 4.17.0)

[23]	Proposal for Rate Adaptation implemented on a DSP;
(C. Bianconi, Texas Instruments, January 1998, version 1.0)

[24]	MCU-DSP Interfaces for Data Applications;
Specification S844
(C. Bianconi, Texas Instruments, March 1998, version 0.1)

[25]	Users Guide
6147.300.96.100; Condat GmbH

[26]	Service Access Point RA
8411.100.98.100; Condat GmbH

[27]	Service Access Point RLP
8411.101.98.100; Condat GmbH

[28]	Service Access Point L2R
8411.102.98.100; Condat GmbH

[29]	Service Access Point FAD
8411.103.98.100; Condat GmbH

[30]	Service Access Point T30
8411.104.98.100; Condat GmbH

[31]	Service Access Point ACI
8411.105.98.100; Condat GmbH

[32]	Message Sequence Charts RLP
8411.201.98.100; Condat GmbH

[33]	Message Sequence Charts L2R
8411.202.98.100; Condat GmbH

[34]	Message Sequence Charts FAD
8411.203.98.100; Condat GmbH

[35]	Message Sequence Charts T30
8411.204.98.100; Condat GmbH

[36]	Message Sequence Charts ACI
8411.205.98.100; Condat GmbH

[37]	Proposal for Fax & Data Integration; March 1998
8411.300.98.100; Condat GmbH

[38]	Test Specification RLP
8411.401.98.100; Condat GmbH

[39]	Test Specification L2R
8411.402.98.100; Condat GmbH

[40]	Test Specification FAD
8411.403.98.100; Condat GmbH

[41]	Test Specification T30
8411.404.98.100; Condat GmbH

[42]	Test Specification ACI
8411.405.98.100; Condat GmbH

[43]	SDL Specification RLP
8411.501.98.100; Condat GmbH

[44]	SDL Specification L2R
8411.502.98.100; Condat GmbH

[45]	SDL Specification FAD
8411.503.98.100; Condat GmbH

[46]	SDL Specification T30
8411.504.98.100; Condat GmbH

[47]	SDL Specification ACI
8411.505.98.100; Condat GmbH

[48]	Technical Documentation RLP
8411.701.98.100; Condat GmbH

[49]	Technical Documentation L2R
8411.702.98.100; Condat GmbH

[50]	Technical Documentation FAD
8411.703.98.100; Condat GmbH

[51]	Technical Documentation T30
8411.704.98.100; Condat GmbH

[52]	Technical Documentation ACI
8411.705.98.100; Condat GmbH

## Abbreviations

ACI	AT Command Interpreter
AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BCS	Binary Coded Signals
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface
EOL	End Of Line

F	Final Bit
F&D	Fax and Data Protocol Stack
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HDLC	High level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LISR	Low level Interrupt Service Routine
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MSG	Message phase in the GSM 3.45 protocol

N®	Receive Number
N(S)	Send Number
NCC	National Colour Code
NECI	New Establishment Causes included

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Description Unit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

- T.4	CCITT Standardisation for Document coding of Group 3 Facsimile Apparatus
TAP	Test Application Program
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

V(A)	Acknowledgement State Variable
V®	Receive State Variable
V(S)	Send State Variable
VPLMN	Visiting Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for fax and data transmission consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity. The entity, which is described in this document, is coloured grey in the following figure :

- Figure 2-1: Architecture of the fax and data protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the fax and data protocol stack are:

## RA - Rate Adaptation

This entity performs an adaptation between an asynchronous or synchronous data stream with several bit rates on to the fixed bit rate used at the TCH. This is performed by the rate adaptation functions RA1’ and RA0 described in GSM 04.21.

## RLP - Radio Link Protocol

This entity provides a Layer 2 protocol for asynchronous reliable data transfer as specified in GSM 04.22. It includes error correction, sequence numbers and a mechanism for repeating corrupted and lost messages.

## L2R - Layer 2 Relay Functionality

The L2R provides relay functions in order to adapt the character-oriented data received from the TE via USART to the bit-oriented RLP protocol.

## FAD 03.45 - Fax Adaptation Protocol

The fax adaptation protocol, as specified in GSM 03.45, provides synchronisation with the BCS and MSG modems of the peer entity. It uses byte repetition in conjunction with a voting algorithm to handle corruption on the TCH data stream. The non-transparent fax protocol in accordance with GSM 03.46 is not part of this implementation.

The fax adapter enables T.30 to send BCS at 300 BPS and T.4 MSG in 2400, 4800, 7200 and 9600 BPS.

## T.30 - Fax Protocol Entity

The protocol uses binary coded signals packed in HDLC frames to set up and release a connection in the message phase of the FAX transmission. This entity is specified in the ITU-T.30. The main tasks of this unit are:

- Building the HDLC frames with CRC.

- Performing bit stuffing/de-stuffing.

- Executing a sequence of 5 phases: 1.) set up, 2.) pre-message procedures, 3.) transmission/reception, 4.) post message procedures, 5.) waiting for call release.

## ACI - AT Command Interpreter

The ACI is specified in GSM 07.07. It is responsible for call establishment via the GSM voice protocol stack and terminal adaptation for asynchronous transparent character-oriented data transmission. The ACI is able to receive AT commands and send the replies over the USART driver to a remote PC. This makes it possible to control the voice and data protocol stack from a remote application running on a PC. The ACI also provides a unique interface for an internal MMI in the MS.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the F&D. The driver has to be able to perform flow control.

# Parameters

/* array declarations */

DECLARATION (F_SIM_SRV_NO_CC)

DECLARATION (A_ECC_FIELD )

DECLARATION (A_AD_FIELD_CI_DISABLED )

DECLARATION (CBM_MID_DEF)

DECLARATION (CBM_DCS_DEF)

DECLARATION (F_SIM_SRV_4)

DECLARATION(DELIVER_01)

DECLARATION(D_DELIVER_01)

DECLARATION(DELIVER_02)

DECLARATION(D_DELIVER_02)

DECLARATION(SM7_ABCDEFGHI_08)

DECLARATION(D_SM7_ABCDEFGHI_08)

DECLARATION (POS_DATA1)

/* structure declarations */

/* Number definitions */

BYTE NUM_0 0
BYTE NUM_1 1
BYTE NUM_2 2
BYTE NUM_3 3
BYTE NUM_4 4
BYTE NUM_5 5
BYTE NUM_6 6
BYTE NUM_7 7
BYTE NUM_8 8
BYTE NUM_9 9
BYTE NUM_10 10
BYTE NUM_12 12
BYTE NUM_FF 0xFF
BYTE REC_NUM_MAX 0xFF

BYTE L_DELIVER_01 31

BYTE L_DELIVER_02 31

BYTE L_SM7_ABCDEFGHI_08 30

/*Message Indication OK*/

- STRING(M_OK, "OK" )

- BYTE LM_OK 2

/* message reference */

SHORT	MSG_REF_1	1
BYTE VALID 0

/*Command: +CFUN set phone functionality*/

- STRING(C_PLUS_CFUN_FULL, "AT+CFUN=1 " )

- BYTE LC_PLUS_CFUN_FULL 9

/*Message Indication +CME	 error result code*/

STRING(M_ERR_PIN_REQ, "+CME ERROR: SIM PIN required" )

- BYTE LM_ERR_PIN_REQ 28

/*Message Indication +CMEE extended error report mode*/

- STRING(C_PLUS_CMEE_VERB, "AT+CMEE=2 " )

- BYTE LC_PLUS_CMEE_VERB 9

/*Command +CLSA = Set source and destination address of mobile location center (E-OTD)*/

- STRING(C_PLUS_CLSA_1234, "AT+CLSA=+4412345678,+449876543210")

- BYTE LC_PLUS_CLSA_1234 33

/*Command +CLSA = Set source and destination address of mobile location center (E-OTD)*/

- STRING(C_PLUS_CLSA_1234_1, "AT+CLSA=4412345678,449876543210")

- BYTE LC_PLUS_CLSA_1234_1 31

/*Command +CLSA? query source and destination address of mobile location center (E-OTD)*/

- STRING(C_PLUS_CLSA_QUERY, "AT+CLSA?")

- BYTE LC_PLUS_CLSA_QUERY 8

/*Message Indication+CLSA source/destination address MLC*/

- STRING(M_PLUS_CLSA,"+CLSA:+4412345678,+449876543210")

- BYTE LM_PLUS_CLSA 31

/*Command +CLOM = Set LCS Operating Mode (E-OTD) ON/ALL clients*/

- STRING(C_PLUS_CLOM_ON_ALL, "AT+CLOM=1,1" )

- BYTE LC_PLUS_CLOM_ON_ALL 11

- STRING(C_PLUS_CLOM_ON_0, "AT+CLOM=1,0" )

- BYTE LC_PLUS_CLOM_ON_0 11

/*Command +CLOM = Set LCS Operating Mode (E-OTD) ON/ client*/

- STRING(C_PLUS_CLOM_ON_1, "AT+CLOM=1,4412345678")

- BYTE LC_PLUS_CLOM_ON_1 20

- STRING(C_PLUS_CLOM_ON_2, "AT+CLOM=1,+4412345678")

- BYTE LC_PLUS_CLOM_ON_2 21

- STRING(C_PLUS_CLOM_ON_3, "AT+CLOM=1,9876543210")

- BYTE LC_PLUS_CLOM_ON_3 20

/*Command +CLOM = Set LCS Operating Mode (E-OTD)OFF/ALL clients*/

- STRING(C_PLUS_CLOM_OFF_ALL, "AT+CLOM=0,0")

- BYTE LC_PLUS_CLOM_OFF_ALL 11

/*Command +CLOM = Set LCS Operating Mode (E-OTD)OFF/ client*/

- STRING(C_PLUS_CLOM_OFF_1, "AT+CLOM=0,4412345678")

- BYTE LC_PLUS_CLOM_OFF_1 20

- STRING(C_PLUS_CLOM_OFF_2, "AT+CLOM=0,+4412345678")

- BYTE LC_PLUS_CLOM_OFF_2 21

- STRING(C_PLUS_CLOM_OFF_3, "AT+CLOM=0,9876543210")

- BYTE LC_PLUS_CLOM_OFF_3 20

/*Command +CLOM = Set LCS Operating Mode (E-OTD)Create/ client*/

- STRING(C_PLUS_CLOM_CREA_1, "AT+CLOM=3,4412345678")

- BYTE LC_PLUS_CLOM_CREA_1 20

- STRING(C_PLUS_CLOM_CREA_2, "AT+CLOM=3,+4412345678")

- BYTE LC_PLUS_CLOM_CREA_2 21

- STRING(C_PLUS_CLOM_CREA_3, "AT+CLOM=3,9876543210")

- BYTE LC_PLUS_CLOM_CREA_3 20

/*Command +CLOM = Set LCS Operating Mode (E-OTD)Delete/ client*/

- STRING(C_PLUS_CLOM_DEL_1, "AT+CLOM=2,+4412345678")

- BYTE LC_PLUS_CLOM_DEL_1 21

- STRING(C_PLUS_CLOM_DEL_2, "AT+CLOM=2,4412345678")

- BYTE LC_PLUS_CLOM_DEL_2 20

- STRING(C_PLUS_CLOM_DEL_3, "AT+CLOM=2,9876543210")

- BYTE LC_PLUS_CLOM_DEL_3 20

/*Command +CLOM? query LC operating mode (E-OTD)*/

- STRING(C_PLUS_CLOM_QUERY, "AT+CLOM?")

- BYTE LC_PLUS_CLOM_QUERY 8

- STRING(C_PLUS_CLOM_DEL_5, "AT+CLOM=2,6666663210")

- BYTE LC_PLUS_CLOM_DEL_5 20

/*Message Indication+CLOM operating mode */

STRING(M_PLUS_CLOM_QU1,"+CLOM:3" )

STRING(M_PLUS_CLOM_QU5,"+CLOM:2" )

STRING(M_PLUS_CLOM_QU11,"0,4412345678")

STRING(M_PLUS_CLOM_QU51,"1,4412345678")

STRING(M_PLUS_CLOM_QU12,"0,+4412345678")

STRING(M_PLUS_CLOM_QU52,"1,+4412345678")

STRING(M_PLUS_CLOM_QU13,"0,9876543210")

BYTE LM_PLUS_CLOM_QU1 7

BYTE LM_PLUS_CLOM_QU5 7

BYTE LM_PLUS_CLOM_QU11 12

BYTE LM_PLUS_CLOM_QU51 12

BYTE LM_PLUS_CLOM_QU12 13

BYTE LM_PLUS_CLOM_QU52 13

BYTE LM_PLUS_CLOM_QU13 12

/*Message Indication+CLOM operating mode */

STRING(M_PLUS_CLOM_QU2,"+CLOM:2" )

STRING(M_PLUS_CLOM_QU21,"0,+4412345678")

STRING(M_PLUS_CLOM_QU22,"0,9876543210")

BYTE LM_PLUS_CLOM_QU2 7

BYTE LM_PLUS_CLOM_QU21 13

BYTE LM_PLUS_CLOM_QU22 12

/*Message Indication+CLOM operating mode */

STRING(M_PLUS_CLOM_QU4,"+CLOM:2" )

STRING(M_PLUS_CLOM_QU41,"0,4412345678")

STRING(M_PLUS_CLOM_QU42,"0,+4412345678")

BYTE LM_PLUS_CLOM_QU4 7

BYTE LM_PLUS_CLOM_QU41 12

BYTE LM_PLUS_CLOM_QU42 13

/*Message Indication+CLOM operating mode */

STRING(M_PLUS_CLOM_QU3,"+CLOM:2" )

STRING(M_PLUS_CLOM_QU31,"0,4412345678")

STRING(M_PLUS_CLOM_QU32,"0,9876543210")

BYTE LM_PLUS_CLOM_QU3 7

BYTE LM_PLUS_CLOM_QU31 12

BYTE LM_PLUS_CLOM_QU32 12

/*Command +CLPS = Set Periodic Update (E-OTD) ON/ ALL clients*/

- STRING(C_PLUS_CLPS_ON_ALL, "AT+CLPS=1,1")

- BYTE LC_PLUS_CLPS_ON_ALL 11

- STRING(C_PLUS_CLPS_ON_1, "AT+CLPS=1,+4412345678,30")

- BYTE LC_PLUS_CLPS_ON_1 24

- STRING(C_PLUS_CLPS_ON_2, "AT+CLPS=1,4412345678,120")

- BYTE LC_PLUS_CLPS_ON_2 24

- STRING(C_PLUS_CLPS_ON_3, "AT+CLPS=1,9876543210,15")

- BYTE LC_PLUS_CLPS_ON_3 23

/*Command +CLPS = Set Periodic Update (E-OTD)OFF/ALL clients*/

- STRING(C_PLUS_CLPS_OFF_ALL, "AT+CLPS=0,0")

- BYTE LC_PLUS_CLPS_OFF_ALL 11

/*Command +CLPS = Remove Periodic Update (E-OTD)OFF/ client*/

- STRING(C_PLUS_CLPS_OFF_2, "AT+CLPS=0,4412345678")

- BYTE LC_PLUS_CLPS_OFF_2 20

/*Command +CLPS? query LC operating mode (E-OTD)*/

- STRING(C_PLUS_CLPS_QUERY, "AT+CLPS?")

- BYTE LC_PLUS_CLPS_QUERY 8

/*Message Indication+CLPS operating mode */

STRING(M_PLUS_CLPS_QU1,"+CLPS:2" )

STRING(M_PLUS_CLPS_QU11,"+4412345678,30")

STRING(M_PLUS_CLPS_QU12,"9876543210,15")

BYTE LM_PLUS_CLPS_QU1 7

BYTE LM_PLUS_CLPS_QU11 14

BYTE LM_PLUS_CLPS_QU12 13

/*Message Indication+CLPS delivered position data*/

STRING(M_CLPS_POS1,"%CLPS:12345678901234567890
12345678901234567890123456789012345678901234567890123456789012345678901234567890
12345678901234567890123456789012345678901234567890123456789012345678901234567890
12345678901234567890123456789012345678901234567890123456789012345678901234567890

12345678901288997766")

BYTE LM_CLPS_POS1 146

/*Command +CLSR = Set Privacy Parameter*/

- STRING(C_PLUS_CLSR_2_NNNC, "AT+CLSR=0,0,+4412345678")

- BYTE LC_PLUS_CLSR_2_NNNC 23

- STRING(C_PLUS_CLSR_1_NNC, "AT+CLSR=1,0,4412345678")

- BYTE LC_PLUS_CLSR_1_NNC 22

- STRING(C_PLUS_CLSR_3_NC, "AT+CLSR=1,1,9876543210")

- BYTE LC_PLUS_CLSR_3_NC 22

/* command: CNMI */

STRING(C_CNMI_ON, "AT+CNMI=1,2,2,1,0")

BYTE LC_CNMI_ON 17

/* message: CMT */

STRING(M_CMT_ABCDEFGHI_HEADER_1,

"+CMT: \"4412345678\",,\"98/01/07,12:34:56+04\",145,0,64,0,\"12345\",129,9")

BYTE LM_CMT_ABCDEFGHI_HEADER_1 67

STRING(M_CMT_ABCDEFGHI_HEADER_2,

"+CMT: \"4412345678\",,\"98/01/07,12:34:56+04\",129,0,64,0,\"12345\",129,9")

BYTE LM_CMT_ABCDEFGHI_HEADER_2 67

STRING(M_CMT_ABCDEFGHI, "ABCDEFGHI")

BYTE LM_CMT_ABCDEFGHI 9

/* command: CMGF */

STRING(C_CMGF_SET_TXT, "AT+CMGF=1")

BYTE LC_CMGF_SET_TXT 9

STRING(C_CMGF_SET_PDU, "AT+CMGF=0")

BYTE LC_CMGF_SET_PDU 9

/* SIM EF(SMSP) Requests */

/* empty list of CB message identifier ranges */

BEGIN_SHORT_ARRAY (CBM_MID_DEF, 20) 
0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF
ENDARRAY

/* empty list of CB data coding scheme ranges */

BEGINARRAY (CBM_DCS_DEF, 20)
0xFF, 0xFF,
0xFF, 0xFF,
0xFF, 0xFF,
0xFF, 0xFF,
0xFF, 0xFF,
0xFF, 0xFF,
0xFF, 0xFF,
0xFF, 0xFF,
0xFF, 0xFF,
0xFF, 0xFF
ENDARRAY

/* SMS-DELIVER */

/* SA_12345, OA_4412345678, PID_SM_TYPE_0, DCS_DEF_ALPH, VP_A9801071234564 (TP-SCTS), 
    SM7_ABCDEFGHI */

BEGIN_PSTRUCT ("sms_sdu", DELIVER_01)
	SET_COMP ("l_buf",	L_DELIVER_01*8)
	SET_COMP ("o_buf",	0x00)
	SET_COMP ("buf",	D_DELIVER_01)
ENDSTRUCT

BEGINARRAY_PART(D_DELIVER_01, L_DELIVER_01)
	0x04, 0x81, 0x21, 0x43, 0xF5,
	0x00, 0x0A, 0x91, 0x44, 0x21, 0x43, 0x65, 0x87,
	0x40, 0x00, 0x89, 0x10, 0x70, 0x21, 0x43, 0x65, 0x40,
	0x09, 0x41, 0xE1, 0x90, 0x58, 0x34, 0x1E, 0x91, 0x49
ENDARRAY

/* SA_12345, OA_+4412345678, PID_SM_TYPE_0, DCS_DEF_ALPH, VP_A9801071234564 (TP-SCTS), 
    SM7_ABCDEFGHI */

BEGIN_PSTRUCT ("sms_sdu", DELIVER_02)
	SET_COMP ("l_buf",	L_DELIVER_02*8)
	SET_COMP ("o_buf",	0x00)
	SET_COMP ("buf",	D_DELIVER_02)
ENDSTRUCT

BEGINARRAY_PART(D_DELIVER_02, L_DELIVER_02)
	0x04, 0x81, 0x21, 0x43, 0xF5,
	0x00, 0x0A, 0x81, 0x44, 0x21, 0x43, 0x65, 0x87,
	0x40, 0x00, 0x89, 0x10, 0x70, 0x21, 0x43, 0x65, 0x40,
	0x09, 0x41, 0xE1, 0x90, 0x58, 0x34, 0x1E, 0x91, 0x49
ENDARRAY

/* message: CMGS

STRING(M_CMGS_MSG_REF_1, "+CMGS: 1")

BYTE LM_CMGS_MSG_REF_1 8*/

/* SIM service table */

BEGINARRAY (F_SIM_SRV_NO_CC, 10)

0x03, 0x00, 0x00, 0x00, 0x00, 0x00, 0x3C, 0x03, 0x00, 0x30
ENDARRAY

/* SIM Service Table with Nr. 4 */

BEGINARRAY (F_SIM_SRV_4, 10) 0xC0, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

/* EF ECC field array */

BEGINARRAY (A_ECC_FIELD,12) 0x11, 0xF2, 0xFF, 0x99, 0xF9, 0xFF, 0x21, 0x43, 0x65, 0xFF, 0xFF, 0xFF ENDARRAY

/* EF AD field array , disable CI */

BEGINARRAY_PART (A_AD_FIELD_CI_DISABLED,4) 0x00, 0x00, 0x00, 0x02 ENDARRAY

/* DA_654321, SA_12345, PID_SM_TYPE_0, DCS_DEF_ALPH, VP_A9801071234564, SM7_ABCDEFGHI */

BEGIN_PSTRUCT ("sms_sdu", SM7_ABCDEFGHI_08)

SET_COMP ("l_buf",	L_SM7_ABCDEFGHI_08*8)

SET_COMP ("o_buf",	0x00)

SET_COMP ("buf",	D_SM7_ABCDEFGHI_08)

ENDSTRUCT

BEGINARRAY_PART(D_SM7_ABCDEFGHI_08, L_SM7_ABCDEFGHI_08)

0x04, 0x81, 0x21, 0x43, 0xF5,

0x19, 0x00, 0x06, 0x81, 0x56, 0x34, 0x12,

0x40, 0x00, 0x89, 0x10, 0x70, 0x21, 0x43, 0x65, 0x40,

0x09, 0x41, 0xE1, 0x90, 0x58, 0x34, 0x1E, 0x91, 0x49

ENDARRAY

/* SMS field array for position data */

BEGINARRAY (POS_DATA1, 140) 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x34, 0x56, 0x78, 0x90, 0x12, 0x88, 0x99, 0x77, 0x66 ENDARRAY

# TEST CASES

## Routing (internal) (ACILC001 -)

### ACILC001:  Setup the Routing and the PCO view for the ACI test, and set ACI to transparent mode

Description:

Routings for the ACI tests are set.

Preamble:

None

APL		ACI		PS

|		|		|

COMMAND (TAP RESET)
COMMAND (CC RESET)
COMMAND (MM RESET)
COMMAND (SIM RESET)
COMMAND (SS RESET)
COMMAND (MMI RESET)
COMMAND (SMS RESET)
COMMAND (RR RESET)
COMMAND (hCommGRR RESET)
COMMAND (hCommGMM RESET)

COMMAND (PL RESET)
COMMAND (hCommLC RESET)
	 |		|		|
COMMAND (TAP REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
COMMAND (MM REDIRECT CLEAR)
COMMAND (SIM REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
COMMAND (MMI REDIRECT CLEAR)
COMMAND (SMS REDIRECT CLEAR)

COMMAND (RR REDIRECT CLEAR)
COMMAND (hCommGRR REDIRECT CLEAR)
COMMAND (hCommGMM REDIRECT CLEAR)
COMMAND (PL REDIRECT CLEAR)

COMMAND (hCommLC REDIRECT CLEAR)

|		|		|
COMMAND (MMI REDIRECT CC TAP)
COMMAND (MMI REDIRECT MM TAP)
COMMAND (MMI REDIRECT SIM TAP)
COMMAND (MMI REDIRECT SS TAP)
COMMAND (MMI REDIRECT MMI TAP)
COMMAND (MMI REDIRECT SMS TAP)
COMMAND (MMI REDIRECT T30 TAP)
COMMAND (MMI REDIRECT L2R TAP)
COMMAND (MMI REDIRECT RA TAP)

COMMAND (MMI REDIRECT RR TAP)
COMMAND (MMI REDIRECT hCommGRR TAP)
COMMAND (MMI REDIRECT hCommGMM TAP)

COMMAND (MMI REDIRECT hCommLC TAP)
COMMAND (PL REDIRECT MMI NULL)
	|		|		|
COMMAND (TAP REDIRECT TAP MMI)
COMMAND (MMI REDIRECT MMI TAP)
	|		|		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

History:	14.12.98	AK	Initial

## Initialisation (ACILC011 - ACILC020)

### ACILC011:  Power On

Description:

activate SIM card at power on, SIM indicates SAT features

Preamble:

ACILC001

APL		ACI		PS
	|		|		|
(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CMEE=2)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CFUN=1)	|		|
	*	==========================>	*		|

(4)	|		| 	SIM_ACTIVATE_REQ	|
	|		*	==========================>	*

(5)	|		| 	SIM_ACTIVATE_CNF	|
	|		*	<==========================	*

(6)	|		| 	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*

(7)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(8)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(9)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(10)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(11)	|	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CMEE_VERB
	cmd_seq	C_PLUS_CMEE_VERB

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CFUN_FULL
	cmd_seq	C_PLUS_CFUN_FULL

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	NOT_USED
	stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	NOT_USED
	pref_lang	NOT_USED

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	F_SIM_SRV_4
	imsi_field	NOT_USED
	pref_plmn	NOT_USED
	phase	PHASE_2PLUS_SIM
	access_acm	NOT_USED
	access_acmmax	NOT_USED
	access_puct	NOT_USED

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	25.09.2002	RM	Initial

### ACILC020:  Set/Query Location Service Source and Destination Address

Description:

Preamble:

- ACILC011

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM ON)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLSA =)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLSA?)	|		|

*	==========================>	*		|

(6)	|	ACI_CMD_IND	|		|

|	(msg: +CLSA: "+44...)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_ON_ALL
	cmd_seq	C_PLUS_CLOM_ON_ALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLSA_1234
	cmd_seq	C_PLUS_CLSA_1234

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLSA_QUERY
	cmd_seq	C_PLUS_CLSA_QUERY

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLSA
	cmd_seq	M_PLUS_CLSA

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.10.02	RM	Initial

### ACILC021:  Initialize Location Service, receive SMS with MLC source address.(CLSA)

Description:

Variants:	<A>...<B>

Preamble:

- ACILC011

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM ON)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLSA =)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_REQ	|		|
	|	(cmd: CMGF=0)	|		|
	*	==========================>	*		|

(6)	|	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|

- TIMEOUT(1000)

(7)	|		| 	MNSMS_REPORT_IND	|
	|		*	<==========================	*

- (8)	|		|	MNSMS_MESSAGE_IND	|
	|		*	<==========================	*

- (9)	|		|	MNSMS_MESSAGE_IND	|
	|		*	<==========================	*

- (10)	|		| 	MNSMS_REPORT_IND	|
	|		*	<==========================	*

(11)	|	ACI_CMD_REQ	|		|
	|	(cmd: CNMI)	|		|
	*	==========================>	*		|

(12)	|		|	MNSMS_CONFIGURE_REQ	|
	|		*	==========================>	*

(13)	|	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|

(14)	|		|	MNSMS_MESSAGE_IND	|
	|		*	<==========================	*

(15)	|	ACI_CMD_IND	|		|
	|	(msg: CMT)	|		|
	*	<==========================	*		|

(16)	|	ACI_CMD_IND	|		|
	|	(msg: CMT edit)	|		|
	*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_ON_ALL
	cmd_seq	C_PLUS_CLOM_ON_ALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CLSA_1234
<B>	cmd_len	LC_PLUS_CLSA_1234_1
<A>	cmd_seq	C_PLUS_CLSA_1234
<B>	cmd_seq	C_PLUS_CLSA_1234_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CMGF_SET_TXT
	cmd_seq	C_CMGF_SET_TXT

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNSMS_REPORT_IND
	state	SMS_STATE_INITIALISING

- MNSMS_MESSAGE_IND 	mem_type	MEM_ME
	rec_num	SMS_RECORD_NOT_EXIST
	rec_max	REC_NUM_MAX
	status	SMS_RECORD_FREE
	sms_sdu	NOT_USED

- MNSMS_MESSAGE_IND 	mem_type	MEM_SM
	rec_num	SMS_RECORD_NOT_EXIST
	rec_max	REC_NUM_MAX
	status	SMS_RECORD_FREE
	sms_sdu	NOT_USED

- MNSMS_REPORT_IND
	state	SMS_STATE_READY

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_CNMI_ON
	cmd_seq	C_CNMI_ON

- MNSMS_CONFIGURE_REQ
	pref_mem_3	NOT_USED
	mt	NOT_USED
	ds	NOT_USED
	mhc	NOT_USED

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNSMS_MESSAGE_IND
	mem_type	MEM_ME
	rec_num	SMS_RECORD_NOT_EXIST
	rec_max	REC_NUM_MAX
	status	SMS_RECORD_REC_UNREAD
<A>	sms_sdu	DELIVER_01
<B>	sms_sdu	DELIVER_02

- ACI_CMD_IND
<A>	cmd_len	LM_CMT_ABCDEFGHI_HEADER_1
<B>	cmd_len	LM_CMT_ABCDEFGHI_HEADER_2
<A>	cmd_seq	M_CMT_ABCDEFGHI_HEADER_1
<B>	cmd_seq	M_CMT_ABCDEFGHI_HEADER_2

- ACI_CMD_IND
	cmd_len	LM_CMT_ABCDEFGHI
	cmd_seq	M_CMT_ABCDEFGHI

History:	10.10.02	RM	Initial

### ACILC022:  Set Operating Mode of Location Service (CLOM)

Description: switch on/off EOTD Flag, create, activate, deactivate, remove LC clients

Variants:	<A>...<C>

Preamble:

- ACILC011

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM ON)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM CR)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM CR)	|		|

*	==========================>	*		|

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM CR)	|		|

*	==========================>	*		|

(8)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(9)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM OFF)	|		|

*	==========================>	*		|

(10)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(11)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM ON)	|		|

*	==========================>	*		|

(12)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(13)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM DIS)	|		|

*	==========================>	*		|

(14)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(15)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM?)	|		|

*	==========================>	*		|

(16)	|	ACI_CMD_IND	|		|

|	(msg: +CLOM:...)	|		|

*	<==========================	*		|

(17)	|	ACI_CMD_IND	|		|

|	(msg: +CLOM:...)	|		|

*	<==========================	*		|

(18)	|	ACI_CMD_IND	|		|

|	(msg: +CLOM:...)	|		|

*	<==========================	*		|

(19)	|	ACI_CMD_IND	|		|

|	(msg: +CLOM:...)	|		|

*	<==========================	*		|

(20)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(21)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM DEL)	|		|

*	==========================>	*		|

(22)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(23)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM?)	|		|

*	==========================>	*		|

(24)	|	ACI_CMD_IND	|		|

|	(msg: +CLOM:...)	|		|

*	<==========================	*		|

(25)	|	ACI_CMD_IND	|		|

|	(msg: +CLOM:...)	|		|

*	<==========================	*		|

(26)	|	ACI_CMD_IND	|		|

|	(msg: +CLOM:...)	|		|

*	<==========================	*		|

(27)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_ON_ALL
	cmd_seq	C_PLUS_CLOM_ON_ALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_CREA_1
	cmd_seq	C_PLUS_CLOM_CREA_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_CREA_2
	cmd_seq	C_PLUS_CLOM_CREA_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_CREA_3
	cmd_seq	C_PLUS_CLOM_CREA_3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_OFF_ALL
	cmd_seq	C_PLUS_CLOM_OFF_ALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CLOM_ON_1
<B>	cmd_len	LC_PLUS_CLOM_ON_2
<C>	cmd_len	LC_PLUS_CLOM_ON_3
<A>	cmd_seq	C_PLUS_CLOM_ON_1
<B>	cmd_seq	C_PLUS_CLOM_ON_2
<C>	cmd_seq	C_PLUS_CLOM_ON_3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CLOM_OFF_1
<B>	cmd_len	LC_PLUS_CLOM_OFF_2
<C>	cmd_len	LC_PLUS_CLOM_OFF_3
<A>	cmd_seq	C_PLUS_CLOM_OFF_1
<B>	cmd_seq	C_PLUS_CLOM_OFF_2
<C>	cmd_seq	C_PLUS_CLOM_OFF_3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_QUERY
	cmd_seq	C_PLUS_CLOM_QUERY

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLOM_QU1
	cmd_seq	M_PLUS_CLOM_QU1

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLOM_QU11
	cmd_seq	M_PLUS_CLOM_QU11

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLOM_QU12
	cmd_seq	M_PLUS_CLOM_QU12

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLOM_QU13
	cmd_seq	M_PLUS_CLOM_QU13

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CLOM_DEL_2
<B>	cmd_len	LC_PLUS_CLOM_DEL_1
<C>	cmd_len	LC_PLUS_CLOM_DEL_3
<A>	cmd_seq	C_PLUS_CLOM_DEL_2
<B>	cmd_seq	C_PLUS_CLOM_DEL_1
<C>	cmd_seq	C_PLUS_CLOM_DEL_3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_QUERY
	cmd_seq	C_PLUS_CLOM_QUERY

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLOM_QU2
	cmd_seq	M_PLUS_CLOM_QU2

- ACI_CMD_IND
<A>	cmd_len	LM_PLUS_CLOM_QU21
<B>	cmd_len	LM_PLUS_CLOM_QU31
<C>	cmd_len	LM_PLUS_CLOM_QU41
<A>	cmd_seq	M_PLUS_CLOM_QU21
<B>	cmd_seq	M_PLUS_CLOM_QU31
<C>	cmd_seq	M_PLUS_CLOM_QU41

- ACI_CMD_IND
<A>	cmd_len	LM_PLUS_CLOM_QU22
<B>	cmd_len	LM_PLUS_CLOM_QU32
<C>	cmd_len	LM_PLUS_CLOM_QU42
<A>	cmd_seq	M_PLUS_CLOM_QU22
<B>	cmd_seq	M_PLUS_CLOM_QU32
<C>	cmd_seq	M_PLUS_CLOM_QU42

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	09.10.02	RM	Initial

### ACILC023:  Set Operating Mode of Location Service 2 (CLOM)

Description: switch on/off EOTD Flag, create, activate, deactivate, remove LC clients

Preamble:

- ACILC011

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM ON)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM CR)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM CR)	|		|

*	==========================>	*		|

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM?)	|		|

*	==========================>	*		|

(8)	|	ACI_CMD_IND	|		|

|	(msg: +CLOM:...)	|		|

*	<==========================	*		|

(9)	|	ACI_CMD_IND	|		|

|	(msg: +CLOM:...)	|		|

*	<==========================	*		|

(10)	|	ACI_CMD_IND	|		|

|	(msg: +CLOM:...)	|		|

*	<==========================	*		|

(11)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(12)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM DEL)	|		|

*	==========================>	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_ON_ALL
	cmd_seq	C_PLUS_CLOM_ON_ALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_CREA_1
	cmd_seq	C_PLUS_CLOM_CREA_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_CREA_2
	cmd_seq	C_PLUS_CLOM_CREA_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_QUERY
	cmd_seq	C_PLUS_CLOM_QUERY

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLOM_QU5
	cmd_seq	M_PLUS_CLOM_QU5

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLOM_QU51
	cmd_seq	M_PLUS_CLOM_QU51

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLOM_QU52
	cmd_seq	M_PLUS_CLOM_QU52

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_DEL_5
	cmd_seq	C_PLUS_CLOM_DEL_5

History:	01.11.02	RM	Initial

### ACILC024:  Set Operating Mode of Location Service 3 (CLOM)

Description: switch on/off EOTD Flag, create, activate, deactivate, remove LC clients

Preamble:

- ACILC011

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM ON)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM CR)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM CR)	|		|

*	==========================>	*		|

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM?)	|		|

*	==========================>	*		|

(8)	|	ACI_CMD_IND	|		|

|	(msg: +CLOM:...)	|		|

*	<==========================	*		|

(9)	|	ACI_CMD_IND	|		|

|	(msg: +CLOM:...)	|		|

*	<==========================	*		|

(10)	|	ACI_CMD_IND	|		|

|	(msg: +CLOM:...)	|		|

*	<==========================	*		|

(11)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(12)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM ON,0)	|		|

*	==========================>	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_ON_ALL
	cmd_seq	C_PLUS_CLOM_ON_ALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_CREA_1
	cmd_seq	C_PLUS_CLOM_CREA_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_CREA_2
	cmd_seq	C_PLUS_CLOM_CREA_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_QUERY
	cmd_seq	C_PLUS_CLOM_QUERY

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLOM_QU5
	cmd_seq	M_PLUS_CLOM_QU5

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLOM_QU51
	cmd_seq	M_PLUS_CLOM_QU51

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLOM_QU52
	cmd_seq	M_PLUS_CLOM_QU52

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_ON_0
	cmd_seq	C_PLUS_CLOM_ON_0

History:	01.11.02	RM	Initial

### ACILC025:  Set Periodic Update  of Location Service (CLPS, failed, is to rebuilt)

Description: switch on/off Periodic Update  Flag, set/reset Periodic Update for clients, set periodic update value

Preamble:

- ACILC011

APL		ACI		PS

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM ON)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM CR)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM CR)	|		|

*	==========================>	*		|

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM CR)	|		|

*	==========================>	*		|

(8)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(9)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLPS ON)	|		|

*	==========================>	*		|

(10)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(11)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLPS OFF)	|		|

*	==========================>	*		|

(12)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(13)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLPS ON1)	|		|

*	==========================>	*		|

(14)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(15)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLPS ON2)	|		|

*	==========================>	*		|

(16)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(17)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLPS ON3)	|		|

*	==========================>	*		|

(18)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(19)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLPS OF2)	|		|

*	==========================>	*		|

(20)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(21)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLPS?)	|		|

*	==========================>	*		|

(22)	|	ACI_CMD_IND	|		|

|	(msg: +CLPS:...)	|		|

*	<==========================	*		|

(23)	|	ACI_CMD_IND	|		|

|	(msg: +CLPS:...)	|		|

*	<==========================	*		|

(24)	|	ACI_CMD_IND	|		|

|	(msg: +CLPS:...)	|		|

*	<==========================	*		|

(25)	|	ACI_CMD_IND	|		|

|	(msg: +CLPS:...)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_ON_ALL
	cmd_seq	C_PLUS_CLOM_ON_ALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_CREA_1
	cmd_seq	C_PLUS_CLOM_CREA_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_CREA_2
	cmd_seq	C_PLUS_CLOM_CREA_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_CREA_3
	cmd_seq	C_PLUS_CLOM_CREA_3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLPS_ON_ALL
	cmd_seq	C_PLUS_CLPS_ON_ALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLPS_OFF_ALL
	cmd_seq	C_PLUS_CLPS_OFF_ALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLPS_ON_1
	cmd_seq	C_PLUS_CLPS_ON_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLPS_ON_2
	cmd_seq	C_PLUS_CLPS_ON_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLPS_ON_3
	cmd_seq	C_PLUS_CLPS_ON_3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLPS_OFF_2
	cmd_seq	C_PLUS_CLPS_OFF_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLPS_QUERY
	cmd_seq	C_PLUS_CLPS_QUERY

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLPS_QU1
	cmd_seq	M_PLUS_CLPS_QU1

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLPS_QU11
	cmd_seq	M_PLUS_CLPS_QU11

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLPS_QU12
	cmd_seq	M_PLUS_CLPS_QU12

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	15.10.02	RM	Initial

### ACILC026:  Set Privacy Parameter for LC client (CLSR)

Description: set/reset notifiy and confirmation state for LC clients

Preamble:

- ACILC011

APL		ACI		PS

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM ON)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM CR)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM CR)	|		|

*	==========================>	*		|

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLOM CR)	|		|

*	==========================>	*		|

(8)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(9)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLRS NNNC)	|		|

*	==========================>	*		|

(10)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(11)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLRS NNC)	|		|

*	==========================>	*		|

(12)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(13)	|	ACI_CMD_REQ	|		|

|	(cmd: AT+CLRS NC)	|		|

*	==========================>	*		|

(14)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_ON_ALL
	cmd_seq	C_PLUS_CLOM_ON_ALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_CREA_1
	cmd_seq	C_PLUS_CLOM_CREA_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_CREA_2
	cmd_seq	C_PLUS_CLOM_CREA_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLOM_CREA_3
	cmd_seq	C_PLUS_CLOM_CREA_3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLSR_2_NNNC
	cmd_seq	C_PLUS_CLSR_2_NNNC

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLSR_1_NNC
	cmd_seq	C_PLUS_CLSR_1_NNC

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLSR_3_NC
	cmd_seq	C_PLUS_CLSR_3_NC

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	25.10.02	RM	Initial

### ACILC027:  Request position data, send data in SMS to MLC destination address.(CLSA, failed currently)

Description:

Preamble:

ACILC021B

APL		ACI		PS

|		|		|

TIMEOUT(1000)

(1)	|		|	MNLC_SMS_MEAS_CNF	|

|		*	<==========================	*

(2)	|	ACI_CMD_IND	|		|

|	(msg: CLPS)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(4)	|		|	MNSMS_SUBMIT_REQ	|

|		*	==========================>	*

(5)	|		|	MNSMS_REPORT_IND	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: CMGS)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNLC_SMS_MEAS_CNF
	reference	NUM_1
	data_valid	VALID
	sms_text	POS_DATA1

- ACI_CMD_IND
	cmd_src	CMD_SRC_EXT
	cmd_len	LM_CLPS_POS1
	cmd_seq	M_CLPS_POS1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	25.10.02	RM	Initial