GSM PROTOCOL STACK

SOFTWARE LOW LEVEL DESIGN DOCUMENT

SIM PERSONALIZATION FEATURE

| Document Version | Author(s) | Approval(s) |
| --- | --- | --- |
| 0.1 (Initial Draft) merging VLE5-SMLK and new LoCosto features | Sasken |  |
| 1.0 (re-formatting to suit TI-LoCosto design template) | Sasken | Manish |
| 2.0 (Restoring 0.1 content with cover page, header, footer as per TI-LoCosto design template) | Sasken |  |

IMPORTANT NOTICE

Texas Instruments Incorporated and its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products and services at any time and to discontinue any product or service without notice. Customers should obtain the latest relevant information before placing orders and should verify that such information is current and complete. All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment.

TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI components. To minimize the risks associated with customer products and applications, customers should provide adequate design and operating safeguards.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products or services are used. Information published by TI regarding third–party products or services does not constitute a license from TI to use such products or services or a warranty or endorsement thereof. Use of such information may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

Reproduction of information in TI data books or data sheets is permissible only if reproduction is without alteration and is accompanied by all associated warranties, conditions, limitations, and notices. Reproduction of this information with alteration is an unfair and deceptive business practice. TI is not responsible or liable for such altered documentation.

Resale of TI products or services with statements different from or beyond the parameters stated by TI for that product or service voids all express and any implied warranties for the associated TI product or service and is an unfair and deceptive business practice. TI is not responsible or liable for any such statements.

Mailing Address:

Texas Instruments

Post Office Box 655303

Dallas, Texas 75265

Copyright ©2003, Texas Instruments Incorporated

TABLE OF CONTENTS

1	Introduction	5

1.1	Scope	5

1.2	Design Representation	5

1.3	Terms/Abbreviations	5

1.4	References:	5

2	Global variables	6

3	Functionality	7

3.1	Personalization Check – Power On	7

3.2	Auto-personalization	16

3.3	Personalization Process	20

3.4	Locking & Unlocking	21

3.5	Disable Personalization (Permanent Unlocking)	24

3.6	Password Change	24

3.7	Status Check	26

3.8	Failure Counter Reset	28

3.9	Supplementary Info	28

3.10	Over The Air De-personalisation	28

4	New Interfaces added to ACI	34

# Introduction

This document presents the Low-level Design for ME Personalization (SIM Lock) functionality.

## Scope

The scope of this document is limited to the ME Personalization functionality implemented at the ACI layer. The document also presents the data structure format for the personalization data to be stored in the Security Driver.

The document does not discuss details about the MMI design for the Personalzition functionality. MMI design shall be presented in a separate document.

## Design Representation

Flow-charts and pseudo-code have been used to explain the detailed design of each functionality. Data structures have been presented using standard `C` convention.

## Terms/Abbreviations

| Sr. No. | Term | Expansion |
| --- | --- | --- |
|  | ACI | Application Control Interface |
|  | API | Application Programming Interface |
|  | ATCI | AT Command Interpreter |
|  | CK | Control Key |
|  | IMSI | International Mobile Subscriber Identity |
|  | ME | Mobile Equipment |
|  | SIM | Subscriber Identity Module |

## References:

| Sr. No | Document/Filename | Version |
| --- | --- | --- |
|  | 3GPP TS 22.022 Personalization of ME | V3.2.1 (2002-06) |
|  | 3GPP TS 27.007 AT Command set for User Equipment | V 3.13.0 (2003-03) |
|  | 3GPP TS 11.11 Mobile Equipment (SIM-ME) Interface | V 8.9.1 (2003-06) |
|  | ME Personalization Requirements - SIM-ME-LOCK VLE5 SIM-ME-LOCK on VLE5 Handsets | 0.2 |
|  | ME Personalization Requirements - SIM-ME-LOCK SIM-ME-LOCK on Alcatel Handsets | 0.1 |
|  | TA_SIM_Lock_TR_Frozen.xls |  |
|  | ACI-ME Personalization – Interface Description | Draft |
|  | Security Driver – High level design | 0.2 |
|  | ME Personalization – High level design | 0.1 |

# Global variables

- T_SEC_LCK_CFG aci_slock_config

- T_ACI_SIM_CONFIG aci_slock_sim_config;     /* SIM configuration*/

typedef struct

{

UBYTE deper_key [16];

UBYTE phase;

UBYTE oper_mode;       /* SIM card functionality   */

UBYTE pref_lang[5];

UBYTE access_acm;

UBYTE access_acmmax;

UBYTE access_puct;

UBYTE sim_gidl1[MAX_GID];

UBYTE sim_gidl2[MAX_GID];

T_SIM_TYPE sim_type;		-- newly introduced to identify the SIM Type

UBYTE sim_read_gid1;  /* added for SP, CP */

UBYTE sim_read_gid2;  /* added for SP, CP */

UBYTE gid1_len;

UBYTE gid2_len;

UBYTE sim_read_ad_first_byte;

} T_ACI_SIM_CONFIG;

where T_SIM_TYPE is defined as

typedef enum

{

SIM_NORMAL,

SIM_TYPEAPPROVAL,

SIM_TEST_CPHS

}T_SIM_TYPE ;

- T_SIM_SHRD_PRM simShrdPrm; - Currently exists in the code

- T_ACI_SLOCK_SHARED AciSLockShrd

typedef struct

{

T_SIMLOCK_STATUS status[SIMLOCK_FIRST_SIM+1];  /* one status for every

personalisation lock type */

UINT16 dependency[SIMLOCK_FIRST_SIM+1];

UBYTE blocked;                 /* blocked flag */

T_SIMLOCK_TYPE current_lock;            /* currently checked lock */

UBYTE auto_personalisation_done ;

UBYTE pb_load ;

}T_ACI_SLOCK_SHARED;

# Functionality

## Personalization Check – Power On

The personalization check is done when a SIM_MMI_INSERT_IND is received. The psa_sim_mmi_insert_ind receives this indication.

Firstly ACI sends a request to read the1st byte of EF(AD)=0x80 to identify if it is a type Approval SIM. Then it is checked if the SIM is a test sim. ACI checks if the PLMN value is 00101. If yes, then the personalization check proceeds based on theTestSIM Mode configured in MEPD. If not, then the personalizations check proceeds based on normal SIM. If it is a normal SIM, the aci_slock_check_personalization function is called. The personalization code in the SIM is compared with the normal codes stored in the MEPD. If none of the normal codes match, the personalization code in the SIM is verified using the Interval data stored in MEPD. If personalization check succeeds, ME start with normal operation. If it fails, the user is asked to enter the control keys for the failed categories.

The following flowcharts explain the personalization check process. If SP or CP category is set in MEPD, while doing check personalization first check will be done for whether GID1/GID2 is read from the sim. If it is not read from the sim, sim read request is called before check personalization of SP/CP category. If check done for all the category is successful the aci_slcok_check_done (SIMLOCK_ENABLED) function is called. If it is unsuccessful the aci_slcok_check_done (SIMLOCK_BLOCKED) function is called.

The command handler function cmh_SIM_SIMInserted is called from aci_slock_check_done function after the personalization check is done.  This handler function returns a Pin Request message to the MMI if the personalization check fails. (RAT_CME in the flow-chart is an error response to the AT Command in case of a failure. Post this response, the user is requested for the Control key in case of failure.)

## Auto-personalization

The ME personalizes to the first Normal SIM inserted if Autolock is enabled for any of the categories.

When a SIM is inserted for the first time into the ME:

ACI checks if it is a Test SIM.

If it is a Test SIM, ME is not auto-personalized.

If it is a Normal SIM, ACI gets the auto-lock record from Security Driver.

If the status of auto-lock record is DISABLED, then ME is not personalized.

If the status of the auto-lock record is ENABLED, ACI checks if the personalization code for the auto-lock categories already exists in MEPD(by default).

If code does not exist, it is added to the respective code group in the MEPD using the sec_get_rec & sec_set_rec APIs.

If the code already exists in the MEPD by default, no new code is added.

The security driver API sec_rec_autolock(record num) is then called to lock the autolock category(s). sec_rec_autolock is a new API that needs to be added to the Security Driver. It should internally set the status of the category to LOCKED without any checks.

After the first auto-personalization is done, the status of the AUTOLOCK record status is set to DISABLED to avoid any subsequent auto-personalization.

When a SIM is already auto-personalized and a new SIM is inserted:

No autopersonalization is done since the AUTOLOCK record status would be disabled.

When the same SIM is inserted again:

Here again, no autopersonalization is done since the AUTOLOCK record status would be disabled.

Flow-charts for auto-personalization are as follows. A separate flow-chart is provided for auto-personalization for SP and CP categories. This is because auto-personalization for these categories is initiated only when a read confirmation for GID1 and GID2 files is received.

## Personalization Process

This functionality is implemented for the SIM Personalization option provided from the MMI.

## Locking & Unlocking

This functionality is provided to lock or unlock a personalization category from the UI or via AT commands issued to the ME. The following steps are performed for Locking/Unlocking of  the category from the MMI.

- Select the category to be locked/unlocked from the MMI.

- Enter the password for the category

- sAT_PlusCLCK function is called with the category type and password.

- sAT_PlusCLCK calls the aci_slock_lock/aci_slock_unlock function for locking & unlocking respectively.

For Lock operation:

ACI checks if the category code already exists in the MEPD. If yes, then ACI calls aci_slock_lock with the category and password.

If the category code does not exist in the MEPD, ACI checks the ’Add New Imsi’ flag to check if a new code can be added to MEPD.

If the Add New Imsi flag is on, ACI adds the code to MEPD using sec_get_rec & sec_set_rec APIs.

If the  Add New Imsi flag is off, ACI just sets the category status to locked.

If the password entered by the user is Incorrect, the user is informed of failure.

If the category is already locked, the user is notified.

For Unlock operation:

ACI calls the sec_rec_unlock function with the category and password. If the password is

incorrect, Security Driver increments the failure counter. User is notified of the incorrect

password.

The user is given FC_MAX number of chances to enter the right password.(as configured

in the security driver.). If the user doesn’t enter the right password in within the give

chances, user is notified that the ME is blocked.

## Disable Personalization (Permanent Unlocking)

The sAT_PercentCLCK API shall be provided for permanent unlocking of a category. (Currently, this option shall not be available from the MMI as it is not a part of the current requirements.) ACI shall provide an additional API aci_slock_permanent_unlock.

## Password Change

MMI provides options to change passwords for a particular category. The following steps are performed for password change from the MMI:

  - Select the ’password change’ option from the MMI.

  - Enter the current password and new password.

  - If the current password is incorrect, the user is prompted to enter the current password again.

  - If the current password is correct, the new password is updated in the Security Driver. The user is informed of a successful password change.

For AT commands issued over Modem SW/AT interface, the sAT_PlusCPWD function is invoked with the current and new password. The rest of the operations are similar to that in password change from MMI.

(Flowchart on next page)

## Status Check

This functionality is provided to check the lock/unlock status of a particular category from the MMI or via AT Commands. The qAT_PlusCLCK function receives the AT command. This function calls the ACI API aci_personalization_get_status which retrieves the status from the Security Driver. The status is returned to the MMI or to the AT Command interface.

## Failure Counter Reset

The MMI provides an option to reset the failure counter.. The AT+CLCK command shall be used for this functionality. A new <fac> value FC shall be introduced.

  - Select the ’Reset Failure Counter’ option from the MMI.

  - Enter the failure counter reset password.

  - sAT_PlusCLCK(FC,0,PWD) is invoked. This function calls the aci_slock_reset_fc API provided by ACI-SLOCK.

  - aci_slock_reset_fc invokes the aci_ext_slock_reset_fc function.

  - aci_ext_reset_fc calls the Security Driver API sec_FC_Reset with the user-entered password and the password length.

  - If Security Driver returns failure, the user is informed about the reset failure due to incorrect password.

## Supplementary Info

This facility is provided to inform the maximum number of attempts allowed for entering the Control key (FCMAX), current attempts left to enter control key (FCATTEMPTSLEFT), maximum number of reset failure counter allowed (FCRESETFAILMAX), current reset failure counter attempts left (FCRESETFAILATTEMPTSLEFT), maximum number of successful attempts allowed to reset failure counter(FCRESETSUCCESSMAX), current successful attempts left for reset failure counter(FCRESETSUCCESSATTEMPTSLEFT), Timer flag , ETSI Flag and Airtel indication flag . The qAT_PercentMEPD function is implemented for the same. This function reads the global config data to get the Max failure counter value.

This value is returned via the pointer variable passed to the qAT_PercentMEPD function.

## Over The Air De-personalisation

As an optional ME feature, the ME may be de-personalised over-the-air (OTA) by the network. The network (mcc+mnc in imsi), network subset (hlr code in imsi), Service Provider (gid1) and corporate (gid1 + gid2) categories may be de-personalised in this way. More than one category may be de-personalised at the same time. The process results in the relevant personalisation indicator(s) being set to "off". The ME must be registered on a network.

Two OTA methods are defined both of which use MT SMS-PP messages. With the first method, the IMEI of the ME to be de-personalised and the Control Key(s) of the personalisation categories to be de-personalised are sent directly to the ME. The ME performs checks on both the IMEI and the key values and the outcome of the attempted de-personalisation(s) is acknowledged to the network.

With the second method, the keys of the personalisation categories to be de-personalised are sent to the ME via the SIM/USIM. The IMEI is not included and the de-personalisation process only checks the keys. The outcome of the attempted de-personalisation(s) is acknowledged to the network.

The network de-personalises the ME by one of the following methods:

### SMS-PP, ME-specific:

- A point-to-point SMS message is sent by the network to the MS , the message being marked as being destined for the ME only and for the purposes of ME de-personalisation. The message will have (in SMS PDU)

  - Data Coding Scheme – Uncompressed, Default Alphabet, Message Class1 (ME specific) i.e. DCS = 0x11

  - Protocol Identifier – PID = 0x7E

  - User Data – De-personalisation keys and IMEI as below (default alphabet coding). ‘FFFFFFF’ to denote ‘de-personalisation not required

| Character | Description |
| --- | --- |
| 1 – 40 | Operator specific text padded with spaces to character 40. |
| 41 – 48 | Network control key |
| 49 – 56 | Network subset control key |
| 57 – 64 | SP control key |
| 65 – 72 | Corporate control key |
| 73 – 88 | IMEI |

- ME compares the IMEI value and then category key values

- ME sends acknowledgement to network; User Data in SMS-DELIVER-REPORT for RP-ACK is as follows:

| Character | Description |
| --- | --- |
| 1-16 | IMEI of ME |
| 17 | Network personalisation status |
| 18 | Network subset personalisation status |
| 19 | SP personalisation status |
| 20 | Corporate personalisation status |

| Status code | Description |
| --- | --- |
| 0 | Currently not personalised |
| 1 | Permanently not personalised |
| 2 | Personalised |
| 3 | IMEI mismatch |
| Other | RFU |

  - If IMEI values differ, no effect on personalisation status and return status code = ‘3’ for all the categories

  - If any key values differ, no effect on corresponding personalisation status and return result code = ‘0/1/2’

  - If key value match, de-personalisation for corresponding category and return result code = ‘0’

MMI

SMS

MM

MMSMS_DATA_IND

(CP-DATA)

[preferably function tl_handle_message]

Decoding the message

.

.

If (PID = 0x7E) and

(DCS = 0x11)

{

1) MMI callback

function (User Data)

2) Based on results

passed by MMI,

encode User Data to

be sent in RP ACK

(coding as per req.

analysis)

}

.

.

Send RP ACK

MMI callback function

{

[as req. analysis]

1) Decoding of User Data

2) IMEI check

3) De-personalisation

(function

aci_slock_unlock)

4) Passback the result of

de-personalisation

}

MMSMS_DATA_REQ

(CP-DATA)

MMSMS_DATA_REQ

(CP-ACK)

MMSMS_DATA_IND

(CP-ACK)

### SMS-PP Data Download:

- The network sends a PP SMS message to the SIM for updating EFDCK  (via Data download procedure of SIM Tool Kit). As a result, SIM issues proactive command REFRESH.

| Identifier: '6F2C' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: 16 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ CHV1 UPDATE CHV1 INVALIDATE ADM REHABILITATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to 4 | 8 digits of network de-personalization control key |  |  | M |  | 4 bytes |
| 5 to 8 | 8 digits of network subset de-personalization control key |  |  | M |  | 4 bytes |
| 9 to 12 | 8 digits of service provider de-personalization control key |  |  | M |  | 4 bytes |
| 13 to 16 | 8 digits of corporate de-personalization control key |  |  | M |  | 4 bytes |

- During initialization procedure (triggered due to REFRESH command), ME reads EFDCK if DCK service is “allocated and activated”.

- For empty control key (coded as all ‘FF’), personalisation status shall not be changed

- If control key matches, de-personalisation for that category

- If control key differ, no changes in personalisation status

- EFDCK is reset to default values.

MMI

SIM

SIM

REFRESH command

File Update Function for EFDCK

[Note: this would be bound during personalisation initialization via psaSAT_FURegister]

{

Read EFDCK  with

callback

}

TERMINAL RESPONSE

SIM_FILE_UPDATE_RES

SIM_FILE_UPDATE_IND

(EFDCK)

SIM_READ_REQ

(EFDCK)

SIM_READ_CNF

(EFDCK)

Callback for read of EFDCK

{

(as per req. analysis)

Interpret EF contents

De-personalisation

(function

aci_slock_unlock)

Write EFDCK with default

contents (stored during

personalisation

initialization)

}

SIM_WRITE_REQ

(EFDCK)

SIM_WRITE_CNF

(EFDCK)

MMI

SIM

SIM_MMI_INSERT_IND

SIM_READ_REQ

(EFDCK)

SIM_READ_CNF

(EFDCK)

Personalisation initialization

{

1) Bind File Update function

via psaSAT_FURegister

2) Read EFDCK with

callback

}

Callback function for EFDCK

{

1) Store EF contents

[Note: as per my opinion,

we need not do

de-personalisation here as

this is hit during SIM

activation]

}

# New Interfaces added to ACI

- aci_slock_set_CFG

To initialize the global configuration data by reading part1 of the MEPD from Security Driver.

- aci_slock_autopersonalize

To auto-personalize to the first normal SIM inserted into the ME based on the autolock settings in

the Security Driver.

- aci_slock_autopersonalize_SP_CP

To auto-personalize ME on the SP & CP categories for the first normal SIM inserted into the ME.

- aci_slock_permanent_unlock

To permanently unlock a category.

- aci_slock_reset_fc

To reset the Failure counter.

- aci_ext_add_code

To add code in ME Data.

Appendix A

- Supported regular expressions for storing interval information

“xx123x“ where x is a single digit from 0-9