High Level Design Specification

PSI Enhancement

| Department: | WTBU - Cellular Systems |  |  |
| --- | --- | --- | --- |
| Creation Date: | 2005-01-17 |  |  |
| Last Modified: | 2005-04-28 by RM |  |  |
| ID: | 16_04_03_00961 | Version: | 101 |
| Status: | Updated | ECCN: | Not Applicable |

- Document Control

 2005 Texas Instruments Incorporated. All rights reserved.

Texas Instruments Incorporated and / or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and / or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and / or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and / or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and / or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and / or the licensing of software do not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of TI.

  - Export Control Statement

Recipient agrees that it will not knowingly export or re-export, directly or indirectly, any product or technical data (as defined by the U.S, EU and other Export Administration Regulations) including software, or any controlled product restricted by other applicable national regulations, received from Disclosing party under this Agreement, or any direct product of such technology, to any destination to which such export or re-export is restricted or prohibited by U.S or other applicable laws, without obtaining prior authorisation from U.S. Department of Commerce and other competent Government authorities to the extent required by those laws. This provision shall survive termination or expiration of this Agreement.

According to our best knowledge of the state and end-use of this product or technology, and in compliance with the export control regulations of dual-use goods in force in the origin and exporting countries, this technology is classified as given on the front page.

This product or technology may require export or re-export license for shipping it in compliance with certain countries regulations.

  - Document History

| Date | Version | Status | Author |
| --- | --- | --- | --- |
| 2005-04-28 | 101 | Updated | RM |
| Updated version. |  |  |  |
| 2005-02-14 | 100 | Approved | RM |
| Reviewed version. |  |  |  |
| 2005-01-17 | 001 | Draft | RM |
| Initial version. |  |  |  |

  - References

| [1] | ISO 9000:2000: International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |
| [2] | 8462.720.01.001High Level Description Protocol Stack Interface (PSI) |
| [3] | 88_08_03_01166 Data I/O Interface (DIO) |
| [4] | 8462.725.04.001Low Level Description Protocol Stack Interface (PSI) |
| [5] | 19_04_03_01398: The High-Level Design Specification of the BAT library |

  - Abbreviations

BAT		Binary AT Interface

DIO IL		Data I/O Interface Layer

FFS		Flash File System

PKTIO		Packet Data I/O Entity

PSI		Protocol Stack Interface

SAP		Service Access Point

UART		Universal Asynchronous Receiver-Transmitter

Table of Contents

1	Introduction	6

2	General Enhancement of PSI	7

3	UART entity	9

3.1	Current Integration of UART driver	9

3.2	New Integration of UART driver	10

3.2.1	Enhancement in PSI	10

3.2.2	Adaptation in ACI	10

3.2.2.1	Replace UART SAP by PSI SAP	11

3.2.2.2	Enhancement PSI part in ACI by handling device type UART	11

3.2.2.2.1	No MUX mode	11

3.2.2.2.2	MUX mode	16

3.2.2.3	Handling specific UART AT commands and additional enhancements	18

3.2.2.4	UART/MUX simulation tests	19

4	PKTIO entity	20

4.1	Current Integration of PKT driver	20

4.2	New Integration of PKTIO driver	20

4.2.1	Enhancement in PSI	21

4.2.2	Adaptation in ACI	21

4.2.2.1	Replace PKTIO SAP by PSI SAP	21

4.2.2.2	Enhancement PSI part in ACI by handling device type PKT	21

4.2.2.3	PKTIO simulation tests	24

5	BAT enhancement	25

5.1	New Integration of BAT driver	25

5.1.1	Enhancement in PSI	25

5.1.2	MCS Communication BAT driver - PSI	26

5.1.2.1	BAT simulation tests	28

List of Figures and Tables

Figure 1 Fragment of current PSI TX state machine during the flushing procedure	7

Figure 2 Fragment of enhanced PSI TX state machine during the flushing procedure	8

Figure 3 UART configuration before PSI implementation project phase 1A	9

Figure 4 Current UART driver / UART entity configuration	9

Figure 5 New configuration regarding UART driver and PSI entity	10

Figure 6 MSC à UART device registration in PSI and ACI	11

Figure 7 MSC à DTI connection close/open between PSI and ACI	12

Figure 8 MSC à DTI close indication from PSI to ACI	12

Figure 9 MSC à UART device disconnection from PSI and ACI	13

Figure 10 MSC à Updating of UART configuration	13

Figure 11 MSC à Changing of UART line states	14

Figure 12 MSC à UART line states indication	14

Figure 13 MSC à UART device close request by ACI	15

Figure 14 MSC à MUX configuration by ACI and PSI	16

Figure 15 MSC à MUX release by UART/MUX driver	17

Figure 16 Current PKT driver / PKTIO entity configuration	20

Figure 17 New configuration regarding PKT driver and PSI entity	20

Figure 18 MSC à PKT device registration in PSI and ACI	21

Figure 19 MSC à DTI connection open between PSI and ACI	22

Figure 20 MSC à DTI close indication from PSI to ACI	22

Figure 21 MSC à PKT device disconnection from PSI and ACI	23

Figure 22 MSC à Updating of PKT configuration parameter	23

Figure 23 MSC à PKT device close request by ACI	24

Figure 24 New configuration regarding BAT driver and PSI entity	25

Figure 25  MSC à BAT device registration in PSI and ACI	26

Figure 26 MSC à DTI close indication from PSI to ACI	27

Figure 27 MSC à BAT device disconnection from PSI and ACI	27

Figure 28 MSC à BAT device close request by ACI	28

# Introduction

As described in [2] a unified interface for the Protocol Stack (PSI - Protocol Stack Interface) is introduced which increases the flexibility, reduces the effort of maintenance, and simplifies the internal handling and the tests. In addition it makes simpler extensions to new drivers and internal applications because only a small layer needs to be adapted. The DIO interface layer (DIO IL), version 4,[3 ]serves controlling of driver access to the TI Protocol Stack. It provides a functional interface to PSI. Up to now PSI is only used for access to an USB driver (phase 1B of PSI implementation project [2]).This document describes the needed effort of PSI/ACI adaptation for replacement of the entities UART and PKTIO (phase 2). From ACI and PSI point of view each driver or application using DIO interface layer is generally called “device”.(Provide a short description of this document.)

# General Enhancement of PSI

- The possibility of each device to transport AT command strings and/or user data is called “device data mode”. The values are part of the PSI SAP and have to be stored in the FFS in the manufacturing process.  PSI uses these FFS entries for sending the according value to ACI in the device registration session.

- The configuration parameters of each device have to be stored in the FFS in the manufacturing process too. PSI uses these FFS entries for the device default configuration.

In order to make PSI more stable it is necessary to improve the design in some points:

- If ACI sends changed line states for serial devices like DCD state in the primitive PSI_LINE_STATE_REQ or in the sent DTI2 data parameters, PSI has to start the flush procedure as the same way as changing of device configuration parameters (Figure_10 ).

Figure 1 Fragment of current PSI TX state machine during the flushing procedure

Figure 2 Fragment of enhanced PSI TX state machine during the flushing procedure

Minor changes:

- PSI sets segment size of read buffer with respect to a more effectively usage of GPF memory partition pool.

- If ACI wants to do a deregistration of the device and it sends PSI_CLOSE_REQ, PSI has to check the status of DTI connection

# UART entity

## Current Integration of UART driver

Retrospect: In the past multiplexer support (MUX) had been part of the UART entity.

Figure 3 UART configuration before PSI implementation project phase 1A

The multiplexer functionality was removed from the UART entity in the phase 1A of the PSI implementation project [2]. Both the UART driver and a new separate multiplexer layer MUX process the multiplexed serial data transmission and support the DIO interface, v4. The small Serial Adaptation Module (SAM) adapts the UF interface of the UART driver to the DIO interface, v4. The UART entity is able to communicate via DIO interface, v4 with the DIO interface layer, as well.

Figure 4 Current UART driver / UART entity configuration

## New Integration of UART driver

Since the current PSI implementation supports serial data transmission the entity UART is not more necessary. The following scheme shows the new configuration of the according entities.

Figure 5 New configuration regarding UART driver and PSI entity

### Enhancement in PSI

- Handling of MUX device capability and MUX configuration parameter in the entity.

- In general: PSI contains DIO dummy functions and test primitives for the simulation test of the functional DIO interface. The dummy functions send primitives called PSI_DIOSIM_XXX_REQ to TAP and wait for the confirmation primitives called PSI_DIOSIM_XXX_CNF sent by TAP. The provided different result values or parameters in these primitives allow testing several driver scenarios.
Some further test primitives testing the PSI SAP are necessary because of the fact that TDC doesn’t support casting. They are named PSI_XXX_TEST. 
For the MUX driver simulation enhancements of the simulation functions described above are necessary.

- Sending of default configuration of UART driver read from FFS.

- Testing:  several PSI instances; handling more than one provided buffer segment, enhanced flush procedure

### Adaptation in ACI

In general: ACI uses the parameter device_type of structure T_DIO_CAP for distinguish the device data type support (instead of evaluation of parameter devId). The number of max DTI links has to be calculated in a new way (used as parameter for DTI lib configuration).

#### Replace UART SAP by PSI SAP

The MUX related primitives of the UART SAP are not used anymore. The remained UART primitives are be covered by the PSI SAP.

#### Enhancement PSI part in ACI by handling device type UART

The UART lists uart_src_params and uart_com_par are covered by the existing lists psi_dev_list and psi_src_params. The list psi_dcb_par ACI used for PSI handling can be removed. The list content is added to psi_dev_list.

##### No MUX mode

- The UART device registration ensues as same way as for the USB device in the PSI part of ACI. PSI sends the first UART device configuration to the device (instead of ACI). The DTI control manager starts immediately the new DTI connection (Figure 6).

Figure 6 MSC à UART device registration in PSI and ACI

- The DTI manager closes the existing DTI connection before it switches to a new DTI connection, i.e. UART device – PSI – L2R entity (Figure_7)

Figure 7 MSC à DTI connection close/open between PSI and ACI

- PSI informs ACI about disconnected DTI connection i.e. if the data transmission breaks down (Figure 8).

Figure 8 MSC à DTI close indication from PSI to ACI

- If the UART device disconnects the connection PSI starts the closing of the according DTI connection and of the according device controlling in PSI and informs ACI (Figure 9.)

Figure 9 MSC à UART device disconnection from PSI and ACI

- The UART configuration parameter are changeable by AT commands like AT+IPR, AT+ICF and setting of Escape Sequence Detection On/Off. It is handled in the primitive PSI_SETCONF_REQ. PSI agrees the changing by PSI_SETCONF_CNF (Figure_10) .

Figure 10 MSC à Updating of UART configuration

- ACI sends the serial line states like DCD state and RING which are to be set by requirements of several PS entities and AT commands in primitive PSI_LINE_STAT_REQ. PSI agrees with the primitive PSI_LINE_STATE_CNF (Figure_11).

Figure 11 MSC à Changing of UART line states

- The UART device indicates Escape Sequence Detection and DTR line drop. PSI sends these indications in primitive PSI_LINE_STAT_IND to ACI. The according DTI connection is closed (Figure_12)..

Figure 12 MSC à UART line states indication

- If the user wants to close the UART device connection ACI sends this requirement in primitive PSI_CLOSE_REQ to PSI. After closing the DIO connection PSI confirms with PSI_CLOSE_CNF  (Figure_13).

Figure 13 MSC à UART device close request by ACI

##### MUX mode

- Precondition is a registered device which is able to work in MUX mode. The device is handled as described from Figure 6 to Figure_13 and works first in serial mode (the parameter device_ mode of structure T_DIO_DCB_SER_MUX is set on DIO_MODE_SER). The driver name is UART. The device capabilities contain serial and MUX parameters.

- The MUX feature is activated by AT+CMUX. Before the switching from serial in MUX mode ACI checks if data are still buffered in ACI. If all of buffered data are sent ACI disconnects the DTI connection and sends the MUX configuration parameter in primitive PSI_SETCONF_REQ to PSI (the parameter device_ mode of structure T_DIO_DCB_SER_MUX has to be set on DIO_MODE_MUX)   (Figure_14) .

Figure 14 MSC à MUX configuration by ACI and PSI

The multiplex data channels are registered in the same way as described from Figure 6 to Figure_13. They are handled as usual serial data channels. The driver name is MUX.

- The multiplex data channel release procedure is the same like in (Figure_9)

- If the multiplex control channel does not work any longer in MUX mode, PSI informs ACI in primitive PSI_LINE_STAT_IND. ACI starts a new DTI connection  (Figure_15).

Figure 15 MSC à MUX release by UART/MUX driver

#### Handling specific UART AT commands and additional enhancements

AT+CMUX:

The functions sAT_PlusCMUX (cmh_uarts.c) and cmh_sig_dti_tx_buffer_ready_ind (sap_dti.c) are to be changed with respect to the MUX handling of PSI. Query of MUX settings are not supported currently.

AT+IPR:

It is not necessary to change the process of this command. The structure uart_ipr_param [] should be renamed by a more common name, the same for the function cmhUART_init_ipr_params ().

AT+ICF:

Currently ACI doesn’t send OK after changing of character framing. For this reason it is necessary to extend the function psa_psi_setconf_cnf ().

AT+IFC:

Currently ACI doesn’t send OK after changing of flow control (V25 ter). For this reason it is necessary to extend the function psa_psi_setconf_cnf ().

AT+FLO:

Currently ACI doesn’t send OK after changing of flow control (T.32). For this reason it is necessary to extend the function psa_psi_setconf_cnf ().

Further open issues:

UART constants for VAL parity (defined in UART SAP): has to be defined in ACI and renamed. The DIO SAP does not support proper parity values but the supported parity is part of character frame constants.

UART constants for VAL_nsb (defined in UART SAP): has to be defined in ACI and renamed. The DIO SAP does not support proper stop bit values but the supported number of stop bits is part of character frame constants.

UART constants for VAL_bpc (defined in UART SAP): has to be defined in ACI and renamed. The DIO SAP does not support proper bits per character values but the supported number of bits per character is part of character frame constants.

The parameter dlci is not longer used because each multiplexed serial channel has its own device identifier.

The UART SAP constant UART_DLCI_NOT_MULTIPLEXED has to be replaced by new constant and to be defined in ACI for backward compatibility.

Since the UART constants UART_DTI_FLOW_OFF, UART_DTI_SB_BIT, UART_DTI_SA_BIT and UART_RING_RUNNING (defined in psa_uart.h) are not more used all functions processing these constants have to be adapted to the PSI constants PSI_DTI_FLOW_OFF, PSI_DTI_SB_BIT, PSI_DTI_SA_BIT and PSI_RING_RUNNING (defined in psa_psi.h).

The function cmhUARTtest_srcId () has to be replaced by cmhPSItest_srcId () completely (T30).

Minor issues:

GPRS connection: parameter of T_GPRS_CONT_CLASS link_uart_id is to be renamed with link_psi_id.

The parameter uart_conn of global structure T_L2R_SHRD_PRM is to be renamed with psi_conn.

The global used structure T_ACI_AT_CMD uartEntcurCmd [CMD_SRC_MAX] is to be renamed with psiEntcurCmd [CMD_SRC_MAX].

The function PPP_UART_connect_dti_cb () is to be renamed with PPP_PSI_connect_dti_cb ().

The function uart_is_mt_te_link () is to be renamed with psi_is_mt_te_link ().

#### UART/MUX simulation tests

The primitives and structures of the PSI SAP have to replace the current used of UART SAP in the UART/MUX simulation tests of ACI entity. It concerns ACI_DTI, ACI_FD, ACI_SS, GACI and ASC.

# PKTIO entity

## Current Integration of PKT driver

The following interface for packet data transmission via the entity PKTIO is currently implemented.

The PKTIO entity is able to communicate via DIO Interface, v4.

Figure 16 Current PKT driver / PKTIO entity configuration

## New Integration of PKTIO driver

The PSI entity is able to support packet data transmission. The following scheme shows the new entity configuration.

Figure 17 New configuration regarding PKT driver and PSI entity

### Enhancement in PSI

- Handling of packet device capability and packet configuration parameter

- Enhancement of buffer handling regarding IP protocol field

- Testing like described in Chap_4223
For the packet driver simulation enhancements of the simulation functions described above are necessary

- Sending of default configuration of PKT driver read from FFS.

- Testing:  several PSI instances; handling more than one provided buffer segment

### Adaptation in ACI

#### Replace PKTIO SAP by PSI SAP

The PKT primitives are be covered by the PSI SAP.

#### Enhancement PSI part in ACI by handling device type PKT

The PKTIO list pktio_dev_list is covered by the existing list psi_dev_list.

- The PKT device registration uses the same primitive as for the USB device in the PSI part of ACI but this device is not treated as ACI source. PSI sends the first PKT device configuration to the device (instead of ACI)(Figure_18).

Figure 18 MSC à PKT device registration in PSI and ACI

- The user must redirect the data flow via the AT command %DATA because no AT commands (ATD, AT+CGDATA) can received on the PKTIO channel. In the GPRS Call set up phase the DTI manager initiates the needed DTI connection (Figure_19) .

Figure 19 MSC à DTI connection open between PSI and ACI

- PSI informs ACI about disconnected DTI connection i.e. if the data transmission breaks down  (Figure_20).

Figure 20 MSC à DTI close indication from PSI to ACI

- If the PKT device disconnects the connection PSI starts the closing of the according DTI connection and of the according device controlling in PSI and informs ACI (Figure_21)

Figure 21 MSC à PKT device disconnection from PSI and ACI

- The changeable PKT device parameters are handled in the primitive PSI_SETCONF_REQ. PSI agrees the changing by PSI_SETCONF_CNF. (Figure_22)

Figure 22 MSC à Updating of PKT configuration parameter

- If the user wants to close the PKT device connection ACI sends this requirement in primitive PSI_CLOSE_REQ to PSI. PSI confirms with PSI_CLOSE_CNF (Figure_23) .

Figure 23 MSC à PKT device close request by ACI

#### PKTIO simulation tests

The primitives and structures of the PSI SAP have to replace the current used of PKTIO SAP in the PKTIO simulation tests of ACI entity. It concerns ACI_PKT.

# BAT enhancement

The BAT library offers a functional interface for execution of AT commands. Several mobile phone configurations (such as one-processor, two-processor, and hybrid execution environments) must be taken into consideration. From a general and high-level point of view, the application is running in the so-called application domain, and sends binary AT commands via the BAT library to the protocol stack (PS) running in the so-called modem domain. The BAT library uses a communication channel between the two domains to transfer information about the transmitted AT commands and their responses. 
The BAT Adapter is a software component situated in the application-domain. It connects the BAT library to PSI. In this scenario, the application and the protocol stack communicate via the protocol stack interface (PSI) entity, the entry point to the protocol stack executing AT commands. The main task of the BAT adapter is to decouple the BAT lib from DIOv4 being communication medium to transfer data to PSI. It also provides a context switch between PSI and the application.

## New Integration of BAT driver

The data transmitted between application and ACI are formated as packets. The following scheme shows the new entity configuration.

Figure 24 New configuration regarding BAT driver and PSI entity

### Enhancement in PSI

- Handling of packet device capability and packet configuration parameter

- Enhancement of buffer handling regarding IP protocol field

- Testing like described in Chap_5121 .
For the packet driver simulation additional enhancements of the simulation functions (described above) are necessary

- Sending of default configuration of BAT driver read from FFS.

- Testing:  several PSI instances; handling more than one provided buffer segment

### MCS Communication BAT driver - PSI

- The BAT device registration uses the same primitive as for the USB device in the PSI part of ACI. PSI sends the first BAT device configuration to the device (instead of ACI) (Figure_25).

Figure 25  MSC à BAT device registration in PSI and ACI

- PSI informs ACI about disconnected DTI connection i.e. if the data transmission breaks down (Figure_26 ).

Figure 26 MSC à DTI close indication from PSI to ACI

- If the BAT device disconnects the connection PSI starts the closing of the according DTI connection and of the according device controlling in PSI and informs ACI (Figure_27 )

Figure 27 MSC à BAT device disconnection from PSI and ACI

- If the user wants to close the BAT device connection ACI sends this requirement in primitive PSI_CLOSE_REQ to PSI. PSI confirms with PSI_CLOSE_CNF(Figure_28 ).

Figure 28 MSC à BAT device close request by ACI

#### BAT simulation tests

The existing PSI simulation tests of the entity PSI are to be enhanced for packet data.

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |