Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document

GSM Protocol Stack

G23

IP and ICMP

Message Sequence Charts

(Edit via File/Properties/Summary!)

| Document Number: | 8444.201.00.001 (Edit via File/Properties/Custom!) |
| --- | --- |
| Version: | 0.4 (Edit via File/Properties/Custom!) |
| Status: | Draft (Edit via File/Properties/Custom!) |
| Approval Authority: | Udo Sprute (Edit via File/Properties/Custom!) |
| Creation Date: | 1998-Mar-25 (Edit via File/Properties/Custom!) |
| Last changed: | 2003-Aug-30 by Jacek Kwasnik |
| File Name: | input.doc (Edit via File/Properties/Custom!) |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1998-Mar-25 | NI et al |  | 0.1 | Being Processed | 1 |
| 2000-May-29 | SG et al |  | 0.2 | Being Processed |  |
| 2003-May-15 | XGUTTEFE |  | 0.3 | Draft |  |
| 2003-Aug-30 | Jacek Kwasnik | Udo Sprute | 0.4 | Draft | 2 |

Notes:

- Initial version

- Brief review minor corrections

Table of Contents

1.1	Standards and Supplementary Documents	6

1.2	IP Basics	7

1.2.1	Features	7

1.2.2	IP Netmasks, Address Classes, Subnetting, Supernetting (CIDR)	9

1.2.3	IP Packet header	10

1.3	ICMP messages	15

1.3.1	Destination unreachable (Error message)	16

Value	17

1.3.2	Time Exceeded (Error message)	18

Value	18

1.3.3	Echo/Echo Reply (Query message)	19

1.4	IP Algorithm	19

1.4.1	IP output	19

1.4.2	IP input	20

1.5	Requirement Levels	20

Req. Level	21

2.1	Deactivated State	25

2.1.1	Activation of IP	25

2.1.2	Reception of DTI2_DATA_REQ in Deactivated State	25

2.1.3	Reception of DTI2_READY_IND in Deactivated State	26

2.1.4	Reception of DTI2_DATA_IND in Deactivated State	26

2.1.5	Reception of IP_ADDR_REQ in Deactivated State	26

2.2	Activated and not-configured State	27

2.2.1	Configuration of an Interface (up)	27

2.2.2	Reception of DTI2_DATA_REQ in not-configured State	27

2.2.3	Reception of DTI2_READY_IND in not-configured State	27

2.2.4	Reception of DTI2_DATA_IND in not-configured State	28

2.2.5	Reception of IP_ADDR_REQ in not-configured State	28

2.2.6	Deactivation of IP	28

2.3	Configured State	29

2.3.1	Configuration of an Interface (down)	29

2.3.2	Configuration of an Interface (up)	29

2.3.3	Reception of IP_ADDR_REQ in Configured State	30

2.3.4	Reception of DTI2_DATA_REQ in Configured State	30

2.3.5	Reception of DTI2_DATA_REQ in Configured State	31

2.3.6	Reception of DTI2_DATA_REQ in Configured State (ERROR/ICMP)	31

A.	Acronyms	32

B.	Glossary	32

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

# Introduction

The Internet Protocol (IP) has been designed to enable connectionless best-effort delivery of packets (called datagrams) host-to-host over a network. It is in use on the world-wide Internet since the early 1980s and has increasingly been adopted for other wide-area and local networks since. The current version of the Internet Protocol is 4 (IPv4). Deployment of the succeeding version 6 (IPv6) has not yet left the R&D area. Due to the large world-wide investments in IPv4-based technology it will take several years until IPv6 becomes dominant on the Internet and other networks.

The transport layer protocols built on top of IP are mainly UDP, which hands through IP’s capabilities to applications, and TCP, which offers a reliable byte-stream data transmission. Since TCP is the dominant Internet transport protocol, IP and the protocols built on top of it, are often collectively called TCP/IP or the Internet protocols suite.

The Internet Control Message Protocol (ICMP) is based on IP. It is used to transmit control information (mostly about error conditions) regarding the communication in the IP layer and the transport layer (usually UDP and TCP). Implementation of ICMP is mandatory for all IP implementations.

## Standards and Supplementary Documents

Internet standards like IP are described in the “Request for Comments” (RFC) series of documents. The RFCs are available online from http://www.rfc-editor.org/ (inside TI from http://chuck/rfcs/). The following RFCs are relevant for IP and ICMP:

[RFC 791]

Jon Postel (ed.): Internet Protocol. RFC 791, September 1981.
http://chuck/rfcs/rfc0791.txt or ftp://ftp.isi.edu/in-notes/rfc791.txt

The initial IP specification.

[RFC 792]

Jon Postel: Internet Control Message Protocol. RFC 792, September 1981.
http://chuck/rfcs/rfc0792.txt or ftp://ftp.isi.edu/in-notes/rfc792.txt

The initial ICMP specification.

[RFC 1071]

R.T. Braden, D.A. Borman, C. Partridge: Computing the Internet checksum. RFC 1071, September 1988.

http://chuck/rfcs/rfc1071.txt or ftp://ftp.isi.edu/in-notes/rfc1071.txt

This RFC gives several examples for implementation of the Internet checksum algorithm. RFC 1141 adds a method for incremental update of the checksum, e. g. on TTL decrement.

[RFC 1122]

Robert. T. Braden (ed.): Requirements for Internet Hosts – Communication Layers. RFC 1122, October 1989.
http://chuck/rfcs/rfc1122.txt or ftp://ftp.isi.edu/in-notes/rfc1122.txt

Among other topics, RFC 1122 contains clarifications and additions to the initial IP and ICMP specifications.

[RFC 1700]

Joyce K. Reynolds, Jon Postel: Assigned Numbers. RFC 1700, October 1994.
http://chuck/rfcs/rfc1700.txt or ftp://ftp.isi.edu/in-notes/rfc1700.txt

RFC 1700 contains numeric values and other definitions in use with the TCP/IP protocol suite.

[RFC 1812]

Fred Baker (ed.): Requirements for IP Version 4 Routers. RFC 1812, June 1995.

http://chuck/rfcs/rfc1812.txt or ftp://ftp.isi.edu/in-notes/rfc1812.txt

This RFC contains an updated list of the ICMP types and codes.

[RFC 2474]

Kathleen Nichols, Steven Blake, Fred Baker, David L. Black: Definition of the Differentiated Services Field (DS Field)in the IPv4 and IPv6 Headers. RFC 2474, December 1998.

http://chuck/rfcs/rfc2474.txt or ftp://ftp.isi.edu/in-notes/rfc2474.txt

This document redefines the meaning of the Type of Service field in the IP header.

In addition to the RFCs, the following books are valuable sources of information:

[MacKusick et al. 1996]

Marshal Kirk McKusick, Keith Bostic, Michael J. Karels, John S. Quarterman: The Design and Implementation of the 4.4 BSD Operating System. Addison-Wesley 1996.

Chapter 13 contains a very concise overview of IP and ICMP.

[Stevens 1994a]

W. Richard Stevens: TCP/IP Illustrated Volume 1 – The Protocols. Addison-Wesley, 1994.

This book contains an excellent introduction to the TCP/IP protocol suite. Relevant to IP and ICMP are the chapters 3, 6, and 9.

[Stevens 1994b]

W. Richard Stevens: TCP/IP Illustrated Volume 2 – The Implementation. Addison-Wesley, 1994.

In this book Stevens annotates the complete 4.4 BSD TCP/IP source code. Relevant to IP and ICMP are chapters 8 to 11.

These RFCs and books were the main sources used in preparation of this document.

The 4.4 BSD TCP/IP software documented in [Stevens 1994a] is an excellent example of a full-featured and very mature implementation of the Internet protocol suite. While it is for several reasons not suited for direct integration into a mobile phone, it is a good reference for any TCP/IP implementer. The TCP/IP code of FreeBSD, a direct successor of 4.4 BSD, can been found at ftp://ftp.freebsd.org/pub/FreeBSD/FreeBSD-stable/src/sys/netinet/ (current version of the “stable” development branch) or, inside TI, at http://chuck/freebsd/src/sys/netinet/ (from FreeBSD release 3.4 of  December 1999).

## IP Basics

### Features

IP is the network layer (ISO/OSI layer 3) protocol of the TCP/IP protocol suite. ICMP messages are encapsulated in IP datagrams, but ICMP belongs conceptually to the same layer as it transmits control information related to the network layer.

Figure 1-1: ISO/OSI model layer of IP and ICMP

From [Stevens 1994a]:

IP is the workhorse of the TCP/IP protocol suite. All TCP, UDP, ICMP, and IGMP data gets transmitted as IP datagrams [...]. A fact that amazes many newcomers to TCP/IP, especially those from an X.25 or SNA background, is that IP provides an unreliable, connectionless datagram delivery service.

By unreliable we mean there are no guarantees that an IP datagram successfully gets to its destination. IP provides a best effort service. When something goes wrong, such as a router temporarily running out of buffers, IP has a simple error handling algorithm: throw away the datagram and try to send an ICMP message back to the source. Any required reliability must be provided by the upper layers (e. g., TCP).

The term connectionless means that IP does not maintain any state information about successive datagrams. Each datagram is handled independently from all other datagrams. This also means that IP datagrams can get delivered out of order. If a source sends two consecutive datagrams (first A, then B) to the same destination, each is routed independently and can take different routes, with B arriving before A.

[Emphasis in the first paragraph is mine.]

The main features of IP are:

Addressing and Routing

IP addresses are 32-bit values, usually written as four decimal numbers, each representing an 8-bit-byte portion of the address, most significant byte first. This notation is called “decimal dotted” notation of IP addresses. Example: 172.20.48.48 for the binary value 10101100000101000011000000110000 (most significant bit first).

An IP packet contains the address of the destination host as well as the address of the source host. If an IP packet cannot be sent on a direct point-to-point link or shared network from source to destination, it must travel via several connections (hops), on the way being forwarded by intermediate hosts acting as routers. A router is a dual- or multi-homed host (i. e. a host with two or more network interfaces) that can forward packets destined to another host from one link or network to another.

Figure 1-2: IP Routing

In each router, the packet is handed to the IP layer, which decides whether and which way to forward the packet according to the destination address of the packet.

To bound maximum packet lifetime, e. g. when encountering routing loops, a Time-To-Live (TTL) field in the packet header is decremented by each router. When the TTL value reaches zero, the packet is discarded.

Fragmentation and Reassembly

If an IP packet (either a complete datagram or a packet already containing a datagram fragment) is too large to be sent over a link, the IP layer in the sending host (either the source of the datagram or a router on the way) can fragment the packet. It is split into smaller packets called fragments, each containing an IP header and a portion of the payload. The IP layer in the destination host then reassembles the original datagram from the fragments received. If not all fragments reach the destination host, the datagram is discarded. (To be precise, the datagram is discarded when the reassembly timer expires before all fragments are received.)

“[...] note the terminology: an IP datagram is the is the unit of end-to-end transmission at the IP layer (before fragmentation and after reassembly), and a packet is the unit of data passed between the IP layer and the link layer. A packet can be a complete IP datagram or a fragment of an IP datagram.” [Stevens 1994a]

There are several IP options that can be set in a datagram. They are more or less obsolete in the Internet context and can safely be ignored in the mobile phone context. They must be safely ignored, though, in the sense that incoming IP datagrams with options set must not cause the IP layer to crash or deliver invalid data to the upper layers.

IP supports multicast (all hosts of a specified group) and broadcast (all hosts on a specified network) destinations. In the context of a mobile phone both do not make much sense and can be ignored.

### IP Netmasks, Address Classes, Subnetting, Supernetting (CIDR)

Applicability

At least currently netmasks will not be relevant in the mobile phone context; this description is provided only for completeness. Even if another IP-capable device uses the mobile to access the Internet, the connection between the mobile and the other device will be a point-to-point link, such that handling two routes will suffice – a host route to the other device and a default route over the air interface. The concept of netmasks does not apply here.

Netmasks

An IP address consists of a network part and a host part. The network part is determined by the netmask. For example, in the IP address 172.20.48.48 with a netmask of 255.255.0.0, the first 16 bits determine the network address (172.20.0.0), and the last 16 bits determine the host inside this network. In words of the C programming language: network = address & netmask.

address bits:		10101100.00010100.00110000.00110000 (172.20.48.48)
netmask bits:		11111111.11111111.00000000.00000000 (255.255.0.0)
network address:		10101100.00010100.00000000.00000000 (172.20.0.0)

Figure 1-3: IP Netmask Example

The first possible address in a network (in this case 172.20.0.0) is consequently used as a symbol for the network itself, and the last possible address (in this case 172.20.255.255) is used as the broadcast address for that network. The host part of the address (here 0.0.48.48) is rarely if ever used separately. The network part of the address is used by routers – this way a router needs to know only where to send packets destined for a network, but not for every single host address in that network.

Address Classes

Originally IP addresses have been divided into five address classes:

| Class | Address bits | Decimal Addresses | Netmask | Max. hosts | Purpose |
| --- | --- | --- | --- | --- | --- |
| A | 0xxxxxxx.* | 0.0.0.0 – 126.255.255.255 | 255.0.0.0 | 16777214 | Very large networks |
| B | 10xxxxxx.* | 128.0.0.0 – 191.255.255.255 | 255.255.0.0 | 65534 | Mid-size networks |
| C | 110xxxxx.* | 192.0.0.0 – 223.255.255.255 | 255.255.0.0 | 254 | Small networks |
| D | 1110xxxx.* | 224.0.0.0 – 239.255.255.255 | n. a. | n. a. | Multicast addresses |
| E | 1111xxxx.* | 240.0.0.0 – 255.255.255.254 | n. a. | n. a. | Experimental |

Table 1-1: IP Address Classes

The addresses 127.0.0.0 – 127.255.255.255 are reserved for loopback interfaces; the address 255.255.255.255 is an additional address for a broadcast on the local network.

Subnetting

To enable separation of a network into several segments, e. g. for different buildings, subnetting has been introduced. With subnetting, each network interface is assigned an explicit netmask along with its IP address. This way a class C network can be separated into several subnetworks, e. g. four separate networks with a netmask of 255.255.255.192, each with up to 62 hosts, or even into three networks with one using a netmask of 255.255.255.128 and two using 255.255.255.192.

With subnetting, the netmask technically needs not be a consecutive bitmask, but it usually is, except for pathological cases. For convenience, the netmask of a network or subnet is thus usually described only by its length and written with a slash behind the (often abbreviated) network address: 192.168.220/24 is the network 192.168.220.0 with a 24-bit long netmask (255.255.255.0), i. e. a regular class C network.

An example may show how a subnetted network may look like:

Figure 1-4: IP Subnetting Example

Supernetting (CIDR)

With the growth of the Internet, two problems became evident:

- The division of the IP address space into classes proved to be not flexible enough.

- The routing tables in Internet routers grew beyond the abilities of hardware, software, and people.

Supernetting, also called Classless Inter-Domain Routing (CIDR), was introduced as a solution. CIDR abandons the concept of network classes at all. Every network address is now associated with a netmask regardless of its class. This way class A networks can be divided into networks with e. g. 24-bit netmasks or even smaller ones; routes to several networks can, if possible, be aggregated to a route to a supernet, e. g. all networks containing addresses from 133.0.0.0 – 133.255.255.255, class B networks in the old notation, can be seen as a supernet 133.0.0.0/8 or shorter 133/8, needing only one entry in the routing table of an Internet router instead of 256. (RFCs 1517 – 1519)

### IP Packet header

All IP header fields are in network order, i. e. big endian (most significant bit/byte first). C library implementations usually supply the functions or macros ntohl(), htonl(), ntohs(), htons() to convert long and short values between network order and host order.

Figure 1-5: IP Packet Header

The lengths in the diagram above are given in bits.

Version

Protocol version of IP. Always 4 with IP version 4.

Internet Header Length (IHL)

Length of the IP packet header in 32-bit words. The minimum value (i. e. if no IP options are present) is 5.

Type Of Service (TOS)

The TOS field allows the sender to select a tradeoff between high throughput, high reliability, and low delay. As the  meaning of the TOS field is currently undergoing changes (a standard is proposed as [RFC 2474]), it is best to send IP packets with a TOS of zero if there is no compelling reason to do otherwise (i. e. answering to an IMCP query with TOS set).

Total Length

Total length of the packet in “octets” (8-bit bytes). The total length is adjusted to the length of the fragment when a packet is fragmented. The maximum value is 65535 (216–1), but since a datagram of this size would for most link layers have to be split into lots of fragments, this value is not used in common applications. Common lengths are up to 8192 bytes in LANs and 1500 bytes (or less) on WANs. Each IP implementation must be capable of receiving datagrams with a total length of up to 576 bytes. (The total length of a fragmented datagram is not known on the receiving side before the last fragment arrives.)

Identification

Identification of the datagram, used when the datagram is reassembled from fragments. The combination of source address, destination address, protocol number, and identification must be unique (at least until all reassembl timers have expired). The identification is usually incremented with each datagram sent by a host.

Flags

Fragmentation flags. Bit 0: reserved, must be zero; Bit 1: (DF) 0 = May Fragment,  1 = Don't Fragment;      Bit 2: (MF) 0 = Last Fragment, 1 = More Fragments. MF is set to 1 in all fragments of a datagram except the last.

Figure 1-6: IP Datagram Fragmentation Flags

Fragment Offset

Contains the offset of the current fragment from the beginning of the datagram in units of 8 8-bit bytes. The first fragment (or the only packet of a non-fragmented datagram) has a fragment offset of 0.

The meaning of the various combinations of the flags and offset values can best be put in a table:

| DF | MF | Offset |  |
| --- | --- | --- | --- |
| 0 | 0 | 0 | unfragmented datagram |
| 0 | 0 | != 0 | last fragment |
| 0 | 1 | 0 | first fragment |
| 0 | 1 | != 0 | fragment in the middle |
| 1 | 0 | 0 | unfragmented datagram |
| 1 | 0 | != 0 | invalid combination |
| 1 | 1 | 0 | invalid combination |
| 1 | 1 | != 0 | invalid combination |

Table 1-2: IP Flags and Fragment Offset Combinations

When sending an IP packet, we need not set DF.

Time to Live

The Time to Live (TTL) is set to an initial value by the IP layer of the source host. It is decremented by each router that forwards the packet. When the TTL has reached zero, the packet is discarded and an ICMP error of the type “TTL expired” is sent to the source host. The currently recommended initial value for the TTL is 64 [RFC 1700]. No packet must be sent with a TTL of 0.

Protocol

Protocol number of the next-higher-level protocol. The protocol numbers are defined in [RFC 1700], or, more current, in http://www.isi.edu/in-notes/iana/assignments/protocol-numbers. Relevant in this context are the numbers for ICMP (1) and UDP (17).

Header Checksum

Checksum over all header fields. This checksum must be recalculated on each change of a header field. This includes fragmentation and TTL decrement.

“The checksum field is the 16 bit one's complement of the one's complement sum of all 16 bit words in the header. For purposes of computing the checksum, the value of the checksum field is zero.” [RFC 791] “A host MUST verify the IP header checksum on every received datagram and silently discard every datagram that has a bad checksum.” [RFC 1122]

A C implementation of the checksum algorithm can be found in [RFC 1071]. Unfortunately this implementation is incorrect. A corrected version looks like this:

{

/* Compute Internet Checksum for "count" bytes

*         beginning at location "addr", which is a

*         pointer to unsigned short (16 bits).

*/

register long sum = 0;

while( count > 1 )  {

/*  This is the inner loop */

sum += *addr++;

count -= 2;

}

/*  Add left-over byte, if any */

if( count > 0 )

sum += * (unsigned char *) addr;

/*  Fold 32-bit sum to 16 bits */

while (sum>>16)

sum = (sum & 0xffff) + (sum >> 16);

checksum = ~sum;

}

Source Address,
Destination Address

IP address of the interface of the source host that sends this packet; IP address of the destination host.

Options

Setting of IP options is optional, but not their implementation. In our context, though, it is legitimate to ignore the next-higher-level protocol’s request to set IP options, as well as to ignore IP options in incoming datagrams. This is a violation of the standard, but the use of IP options can be considered non-necessary today. IP options can, though, without any effort be passed to the next higher-layer protocol, if simply the whole IP datagram is passed.

It is necessary, of course, to handle incoming IP packets with IP options correctly, even when ignoring the option values. The full length of the option fields is determined through the Header Length field.

## ICMP messages

An ICMP message is embedded in an IP packet. The IP header field “protocol” is 1 for ICMP, Type of Service depends on the type of the message; all other fields are set as determined by the usual considerations. ICMP messages are of different types; some types carry additionally different codes.

ICMP messages can be error messages or query messages. ICMP error messages must never be sent in response to

- an ICMP error message (as opposed to an ICMP query message)

- an IP datagram sent as IP broadcast or link-layer broadcast

- a fragment other than the first

- a datagram with a source address that does not define a single host, i. e. a multicast or broadcast address, a network address, a loopback address, or a class E address.

According to http://www.isi.edu/in-notes/iana/assignments/icmp-parameters, the ICMP types shown in the table below are currently defined. The code in the “Relevant” column shows whether the specified type is to be implemented in the mobile phone context () or not (–). The relevant types are described in the following sections.

| Type | Name | Relevant |
| --- | --- | --- |
| 0 | Echo Reply |  |
| 1 | Unassigned | – |
| 2 | Unassigned | – |
| 3 | Destination Unreachable |  |
| 4 | Source Quench | – |
| 5 | Redirect | – |
| 6 | Alternate Host Address | – |
| 7 | Unassigned | – |
| 8 | Echo |  |
| 9 | Router Advertisement | – |
| 10 | Router Selection | – |
| 11 | Time Exceeded |  |
| 12 | Parameter Problem | – |
| 13 | Timestamp | – |
| 14 | Timestamp Reply | – |
| 15 | Information Request | – |
| 16 | Information Reply | – |
| 17 | Address Mask Request | – |
| 18 | Address Mask Reply | – |
| 19 | Reserved (for Security) | – |
| 20-29 | Reserved (for Robustness Experiment) | – |
| 30 | Traceroute | – |
| 31 | Datagram Conversion Error | – |
| 32 | Mobile Host Redirect | – |
| 33 | IPv6 Where-Are-You | – |
| 34 | IPv6 I-Am-Here | – |
| 35 | Mobile Registration Request | – |
| 36 | Mobile Registration Reply | – |
| 37 | Domain Name Request | – |
| 38 | Domain Name Reply | – |
| 39 | SKIP | – |
| 40 | Photuris | – |
| 41-255 | Reserved | – |

Figure 1-7: ICMP Message Types

An ICMP error message is always sent with a Type Of Service (TOS field of the IP header) of 0. An ICMP request message may be sent with any value in the TOS field. An ICMP reply message is sent with the same value in the TOS field as was used in the corresponding ICMP request message.

We need not send any ICMP code not marked as relevant in the table above. Incoming ICMP messages with these codes can be ignored.

### Destination unreachable (Error message)

Figure 1-8: ICMP Header Destination unreachable

The ICMP type “destination unreachable” has the value 3. The different codes are described in the table below. When receiving these messages, not all of these codes need not be maintained as separate error conditions in the mobile phone context; the codes can be grouped into three different error conditions, which can then be passed to the application:

CONNECTION_REFUSED

The IP packet reached the destination host, but on the destination host there is no entity listening for the specified transport protocol and the specified port. (“Protocol Unreachable” and “Port Unreachable”)

MESSAGE_TOO_LONG

The IP packet could not be routed to the destination host because for the next hop link the packet would have to be fragmented, but the DF (don’t fragment) flag is set in the packet. (“Fragmentation Needed and DF Set”)

NO_ROUTE_TO_HOST

The IP packet could not be routed to the destination host for some other reason. (all other codes)

| Code | ### Value | Description | Condition |
| --- | --- | --- | --- |
| Network Unreachable | 0 | generated by a router if a forwarding path (route) to the destination network is not available | NO_ROUTE_TO_HOST |
| Host unreachable | 1 | generated by a router if a forwarding path (route) to the destination host on a directly connected network is not available | NO_ROUTE_TO_HOST |
| Protocol Unreachable | 2 | generated if the transport protocol designated in a datagram is not supported in the transport layer of the final destination | CONNECTION_REFUSED |
| Port Unreachable | 3 | generated if the designated transport protocol (e.g., UDP) is unable to demultiplex the datagram in the transport layer of the final destination but has no protocol mechanism to inform the sender | CONNECTION_REFUSED |
| Fragmentation Needed and DF Set | 4 | generated if a router needs to fragment a datagram but cannot since the DF flag is set | MESSAGE_TOO_LONG |
| Source Route Failed | 5 | generated if a router cannot forward a packet to the next hop in a source route option | NO_ROUTE_TO_HOST |
| Destination Network Unknown | 6 | This code SHOULD NOT be generated since it would imply on the part of the router that the destination network does not exist (net unreachable code 0 SHOULD be used in place of code 6) | NO_ROUTE_TO_HOST |
| Destination Host Unknown | 7 | generated only when a router can determine (from link layer advice) that the destination host does not exist | NO_ROUTE_TO_HOST |
| Network Unreachable For Type Of Service | 11 | generated by a router if a forwarding path (route) to the destination network with the requested or default TOS is not available | NO_ROUTE_TO_HOST |
| Host Unreachable For Type Of Service | 12 | generated if a router cannot forward a packet because its route(s) to the destination do not match either the TOS requested in the datagram or the default TOS (0) | NO_ROUTE_TO_HOST |
| Communication Administratively Prohibited | 13 | generated if a router cannot forward a packet due to administrative filtering | NO_ROUTE_TO_HOST |
| Host Precedence Violation | 14 | Sent by the first hop router to a host to indicate that a requested precedence is not permitted for the particular combination of source/destination host or network, upper layer protocol, and source/destination port | NO_ROUTE_TO_HOST |
| Precedence cutoff in effect | 15 | The network operators have imposed a minimum level of precedence required for operation, the datagram was sent with a precedence below this level | NO_ROUTE_TO_HOST |

Table 1-3: ICMP Codes for Destination Unreachable [RFC 1812]

### Time Exceeded (Error message)

Figure 1-9: ICMP Header Time Exceeded

The “Time Exceeded” type has the value 11. When receiving these messages, both codes must be reported to the transport layer as separate error conditions.

| Code | ### Value | Description | Comment |
| --- | --- | --- | --- |
| TTL exceeded in transit | 0 | generated by a router when it has dropped an IP packet because the TTL field of the packet is zero | The mobile should not throw away IP packets with a TTL of zero that are addressed to the mobile; however, it must drop them when it routes IP packets between a locally attached device and the Internet, and send a TTL exceeded message. |
| Fragment reassembly time exceeded | 1 | generated by the destination host when it has not received all fragments of a datagram during the configured reassembly time |  |

Table 1-4: ICMP Codes for Time Exceeded

### Echo/Echo Reply (Query message)

Figure 1-10: ICMP Header Echo/Echo Reply

The “Echo” type (value 8) requests the destination host to send back an ICMP message of type “Echo reply” (0). The Code field is zero for both types. “The identifier and sequence number may be used by the echo sender to aid in matching the replies with the echo requests. For example, the identifier might be used like a port in TCP or UDP to identify a session, and the sequence number might be incremented on each echo request sent. The echoer returns these same values in the echo reply.” [RFC 792]

The Data is arbitrary data of arbitrary length as chosen by the sender of the Echo request; the data received must be returned in the Echo Reply message.

The ICMP Echo and Echo Reply messages are used by the “ping” program.

## IP Algorithm

This section is not intended as a comprehensive description of the IP layer algorithms, but rather as a general outline.

The numbered headlines in ths section are taken from [McKusick et al. 1996] (with omissions).

### IP output

The IP layer is called to send a message for ICMP or some upper layer. This message may already have a pseudo IP header attached with some values filled in (e. g. destination address).

1. Insert any IP options.

(We omit this step.)

2. Fill in the remaining header fields (IP version, zero offset, header length, and a new packet identification) if the packet contains an IP pseudoheader.

3. Determine the route (i. e., outgoing interface and next-hop destination).

If we have only a WAP browser running on the mobile, we need not worry about routes at all – there is only one route via the air interface. If an IP-capable device (i. e. a host) is attached to one of the local ports (USART, IRDA, Bluetooth, etc.), we need to remember this port as a route to this host (host route); all packets destined to other hosts are routed via the air interface (default route).

4. Check whether the destination is a multicast address.

For now, we need not treat multicast addresses any different from unicast addresses.

5. Check whether the destination is a broadcast address.

The mobile won’t have any broadcast-capable IP interfaces, but it can send broadcast packets over the default route. It must not send packets with the broadcast address 255.255.255.255.

6. If the packet size is no larger than the maximum packet size for the outgoing interface, compute the checksum and call the interface output routine.

7. If the packet size is larger than the maximum packet size for the outgoing interface, break the packet into fragments and send each in turn.

On fragmentation, the Offset, Flags, and Total Length fields must be adjusted.

### IP input

The IP layer is called to handle a packet that has been received by the link layer.

1. Verify that the packet is at least as long as an IP header, and ensure that the header is contiguous.

If it is not, drop the packet. (I am not sure what McKusick et al. mean with “contiguous”.)

2. Checksum the header of the packet, and discard the packet if there is an error.

3. Verify that the packet is at least as long as the header indicates, and drop the packet if it is not.

4. Process any IP options in the header.

Here: Ignore any IP options in the header.

5. Check whether the packet is for this host. If it is, continue processing the packet. If it is not, and if doing packet forwarding, try to forward the packet. Otherwise, drop the packet.

If the packet is for a host attached to the mobile:

Decrement the packet’s TTL. If it is zero:

Drop the packet and send an ICMP time exceeded message with the code 0 (TTL exceeded in transit).

Otherwise forward the packet over the link to this host.

If the packet is for the mobile:

Continue with step 6.

If the packet is neither for an attached host nor for the mobile itself, we return an ICMP destination unreachable message with code 7 (Destination host unknown).

6. If the packet has been fragmented, keep it until all fragments are received and reassembled, or until it is too old to keep.

If this is the first packet, start the reassembly timer with the reassemby timeout value. If it expires before all fragments have arrived, drop the packet and return an ICMP time exceeded message with code 1 (Fragmentation reassembly time exceeded).

7. Pass the packet to the input routine of the next-higher-level protocol.

## Requirement Levels

[RFC 1122] lists a couple of features that an IP/ICMP implementation must, should, may, should not, and must not implement. The meaning of these terms is defined in RFC 2119 (“Key words for use in RFCs to Indicate Requirement Levels”, Scott Bradner, March 1997):

- MUST   This word, or the terms "REQUIRED" or "SHALL", mean that the definition is an absolute requirement of the specification.

- MUST NOT   This phrase, or the phrase "SHALL NOT", mean that the definition is an absolute prohibition of the specification.

- SHOULD   This word, or the adjective "RECOMMENDED", mean that there may exist valid reasons in particular circumstances to ignore a particular item, but the full implications must be understood and carefully weighed before choosing a different course.

- SHOULD NOT   This phrase, or the phrase "NOT RECOMMENDED" mean that there may exist valid reasons in particular circumstances when the particular behavior is acceptable or even useful, but the full implications should be understood and the case carefully weighed before implementing any behavior described with this label.

- MAY   This word, or the adjective "OPTIONAL", mean that an item is truly optional.  One vendor may choose to include the item because a particular marketplace requires it or because the vendor feels that it enhances the product while another vendor may omit the same item. An implementation which does not include a particular option MUST be prepared to interoperate with another implementation which does include the option, though perhaps with reduced functionality. In the same vein an implementation which does include a particular option MUST be prepared to interoperate with another implementation which does not include the option (except, of course, for the feature the option provides.)

The standard RFC 1122 is targeted at general-purpose Internet host. The situation with the mobile phone is vastly different, so the standard is not applicable (“n. a.”) to all issues mentioned in RFC 1122. The restrictions are:

- Sending IP options is not supported; incoming options are passed to the application layer, but not evaluated in the IP layer.

- The only ICMP types supported are 0, 3, 8, 11 (Echo Request and Reply, Destination Unreachable, Time Exceeded).

- Multihoming is not supported, but will probably be in a later version, when a locally attached IP-capable device is supported.

- Netmasks are not supported. (There is currently only one point-to-point link to the ISP, so netmasks are not needed.)

The following table contains the list of requirement levels for IP from [RFC 1122].

| ### Req. Level | Feature | We do | Comment |
| --- | --- | --- | --- |
| MUST | Implement IP and ICMP |  |  |
| MUST | Handle remote multihoming in application layer |  | i. e. the IP layer does not care |
| MAY | Support local multihoming | – | not at the moment |
| MUST | Meet gateway specs if forward datagrams | – | n. a. |
| MUST | Configuration switch for embedded gateway | – | n. a. |
| MUST | Config switch default to non-gateway | – | n. a. |
| MUST NOT | Auto-config based on number of interfaces | – | n. a. |
| SHOULD | Able to log discarded datagrams | – | not necessary or desirable in a mobile |
| SHOULD | Record in counter | – |  |
| MUST | Silently discard Version != 4 |  |  |
| MUST | Verify IP checksum, silently discard bad dgram |  |  |
|  | Addressing: |  |  |
| MUST | Subnet addressing (RFC-950) | – | n. a. |
| MUST | Src address must be host's own IP address |  |  |
| MUST | Silently discard datagram with bad dest addr |  |  |
| MUST | Silently discard datagram with bad src addr |  |  |
| MUST | Support reassembly |  |  |
| MAY | Retain same Id field in identical datagram |  |  |
|  | TOS: |  |  |
| MUST | Allow transport layer to set TOS | – | obsolete and unnecessary |
| SHOULD | Pass received TOS up to transport layer |  | this is easier than not doing it |
| SHOULD NOT | Use RFC-795 link-layer mappings for TOS | – |  |
|  | TTL: |  |  |
| MUST NOT | Send packet with TTL of 0 | – |  |
| MUST NOT | Discard received packets with TTL < 2 | – |  |
| MUST | Allow transport layer to set TTL |  |  |
| MUST | Fixed TTL is configurable |  | at ROM programming time |
|  | IP Options: |  |  |
| MUST | Allow transport layer to send IP options | – | no need to set IP options for our applications |
| MUST | Pass all IP options rcvd to higher layer |  | whole IP packet is passed to transport layer |
| MUST | IP layer silently ignore unknown options |  |  |
| MAY | Security option | – |  |
| SHOULD NOT | Send Stream Identifier option | – |  |
| MUST | Silently ignore Stream Identifier option |  |  |
| MAY | Record Route option | – |  |
| MAY | Timestamp option | – |  |
|  | Source Route Option: |  |  |
| MUST | Originate & terminate Source Route options | – |  |
| MUST | Datagram with completed SR passed up to TL |  | whole IP packet is passed to transport layer |
| MUST | Build correct (non-redundant) return route | – |  |
| MUST NOT | Send multiple SR options in one header | – |  |
|  | ICMP: |  |  |
| MUST | Silently discard ICMP msg with unknown type | – |  |
| MAY | Include more than 8 octets of orig datagram | – |  |
| MUST | Included octets same as received |  |  |
| MUST | Demux ICMP Error to transport protocol |  |  |
| SHOULD | Send ICMP error message with TOS=0 |  |  |
|  | Send ICMP error message for: |  |  |
| MUST NOT | - ICMP error msg | – |  |
| MUST NOT | - IP b'cast or IP m'cast | – |  |
| MUST NOT | - Link-layer b'cast | – |  |
| MUST NOT | - Non-initial fragment | – |  |
| MUST NOT | - Datagram with non-unique src address | – |  |
| MUST | Return ICMP error msgs (when not prohibited) |  |  |
|  | Dest Unreachable: |  |  |
| SHOULD | Generate Dest Unreachable (code 2/3) |  |  |
| MUST | Pass ICMP Dest Unreachable to higher layer |  |  |
| SHOULD | Higher layer act on Dest Unreach |  | this is up to the higher layer |
| MUST | Interpret Dest Unreach as only hint |  | this is up to the higher layer |
|  | Redirect: |  |  |
| SHOULD NOT | Host send Redirect | – |  |
| MUST | Update route cache when recv Redirect | – | n. a. |
| MUST | Handle both Host and Net Redirects | – | n. a. |
| SHOULD | Discard illegal Redirect |  |  |
|  | Source Quench: |  |  |
| MAY | Send Source Quench if buffering exceeded | – |  |
| MUST | Pass Source Quench to higher layer | – | obsolete by modern flow-control algorithms |
| SHOULD | Higher layer act on Source Quench | – | obsolete by modern flow-control algorithms |
| MUST | Time Exceeded: pass to higher layer |  |  |
|  | Parameter Problem: |  |  |
| SHOULD | Send Parameter Problem messages | – | due to performance and memory reasons |
| MUST | Pass Parameter Problem to higher layer | – |  |
| MAY | Report Parameter Problem to user | – | user is not interested |
|  | ICMP Echo Request or Reply: |  |  |
| MUST | Echo server and Echo client |  |  |
| SHOULD | Echo client |  |  |
| MAY | Discard Echo Request to broadcast address |  |  |
| MAY | Discard Echo Request to multicast address |  |  |
| MUST | Use specific-dest addr as Echo Reply src |  |  |
| MUST | Send same data in Echo Reply |  |  |
| MUST | Pass Echo Reply to higher layer |  |  |
| SHOULD | Reflect Record Route, Time Stamp options | – | IP options not supported |
| MUST | Reverse and reflect Source Route option | – | IP options not supported |
| SHOULD NOT | ICMP Information Request or Reply: | – |  |
| MAY | ICMP Timestamp and Timestamp Reply: | – |  |
| SHOULD | Minimize delay variability | – |  |
| MAY | Silently discard b'cast Timestamp |  |  |
| MAY | Silently discard m'cast Timestamp |  |  |
| MUST | Use specific-dest addr as TS Reply src | – | n. a. |
| SHOULD | Reflect Record Route, Time Stamp options | – | n. a. |
| MUST | Reverse and reflect Source Route option | – | n. a. |
| MUST | Pass Timestamp Reply to higher layer | – |  |
| MUST | Obey rules for "standard value" | – | n. a. |
|  | ICMP Address Mask Request and Reply: |  |  |
| MUST | Addr Mask source configurable | – |  |
| MUST | Support static configuration of addr mask | – |  |
| MAY | Get addr mask dynamically during booting | – |  |
| MAY | Get addr via ICMP Addr Mask Request/Reply | – |  |
| MUST | Retransmit Addr Mask Req if no Reply | – |  |
| SHOULD | Assume default mask if no Reply | – |  |
| MUST | Update address mask from first Reply only | – |  |
| SHOULD | Reasonableness check on Addr Mask | – |  |
| MUST NOT | Send unauthorized Addr Mask Reply msgs | – |  |
| MUST | Explicitly configured to be agent | – |  |
| SHOULD | Static config=> Addr-Mask-Authoritative flag | – |  |
| MUST | Broadcast Addr Mask Reply when init. | – |  |
|  | ROUTING OUTBOUND DATAGRAMS: |  |  |
| MUST | Use address mask in local/remote decision | – | n. a. |
| MUST | Operate with no gateways on conn network | – | n. a. |
| MUST | Maintain "route cache" of next-hop gateways |  | configured by PPP |
| SHOULD | Treat Host and Net Redirect the same |  | i. e. not |
| MUST | If no cache entry, use default gateway |  |  |
| MUST | Support multiple default gateways | – |  |
| MAY | Provide table of static routes |  | i. e. one static default route |
| MAY | Flag: route overridable by Redirects | – |  |
| MAY | Key route cache on host, not net address | – |  |
| SHOULD | Include TOS in route cache | – |  |
| MUST | Able to detect failure of next-hop gateway |  |  |
| SHOULD NOT | Assume route is good forever | – | until PPP gives an error |
| MUST NOT | Ping gateways continuously | – |  |
| MUST | Ping only when traffic being sent |  | will not ping gateway |
| MUST | Ping only when no positive indication |  | will not ping gateway |
| SHOULD | Higher and lower layers give advice |  | PPP |
| MUST | Switch from failed default g'way to another | – |  |
| MUST | Manual method of entering config info | – | configured by PPP |
|  | REASSEMBLY and FRAGMENTATION: |  |  |
| MUST | Able to reassemble incoming datagrams |  |  |
| MUST | At least 576 byte datagrams |  |  |
| SHOULD | EMTU_R configurable or indefinite |  |  |
| MUST | Transport layer able to learn MMS_R |  | configured by PPP |
| MUST | Send ICMP Time Exceeded on reassembly timeout |  |  |
| SHOULD | Fixed reassembly timeout value |  |  |
| MUST | Pass MMS_S to higher layers |  |  |
| MAY | Local fragmentation of outgoing packets |  |  |
| MUST | Else don't send bigger than MMS_S | – | n. a. |
| SHOULD | Send max 576 to off-net destination | – | common value is 1500 now |
| MAY | All-Subnets-MTU configuration flag | – | n. a. |
|  | MULTIHOMING: |  |  |
| SHOULD | Reply with same addr as spec-dest addr |  |  |
| MUST | Allow application to choose local IP addr |  | if applicable |
| MAY | Silently discard d'gram in "wrong" interface |  |  |
| MAY | Only send d'gram through "right" interface |  |  |
|  | SOURCE-ROUTE FORWARDING: |  |  |
| MAY | Forward datagram with Source Route option | – |  |
| MUST | Obey corresponding gateway rules | – |  |
| MUST | Update TTL by gateway rules | – |  |
| MUST | Able to generate ICMP err code 4, 5 | – | 4 only |
| MAY | IP src addr not local host | – |  |
| MUST | Update Timestamp, Record Route options | – |  |
| MUST | Configurable switch for non-local SRing | – |  |
| MUST | Defaults to OFF |  |  |
| MUST | Satisfy gwy access rules for non-local SRing | – |  |
| SHOULD | If not forward, send Dest Unreach (cd 5) | – |  |
|  | BROADCAST: |  |  |
| MUST NOT | Broadcast addr as IP source addr | – |  |
| SHOULD | Receive 0 or -1 broadcast formats OK | – |  |
| MAY | Config'ble option to send 0 or -1 b'cast | – |  |
| SHOULD | Default to -1 broadcast | – | n. a. |
| MUST | Recognize all broadcast address formats | – | n. a. |
| MUST | Use IP b'cast/m'cast addr in link-layer b'cast | – | n. a. |
| SHOULD | Silently discard link-layer-only b'cast dg's |  |  |
| SHOULD | Use Limited Broadcast addr for connected net | – | n. a. |
|  | MULTICAST: |  |  |
| SHOULD | Support local IP multicasting (RFC-1112) | – |  |
| MAY | Support IGMP (RFC-1112) | – |  |
| SHOULD | Join all-hosts group at startup | – |  |
| SHOULD | Higher layers learn i'face m'cast capability | – |  |
|  | INTERFACE: |  |  |
| MUST | Allow transport layer to use all IP mechanisms | – | all except options |
| MUST | Pass interface ident up to transport layer |  |  |
| MUST | Pass all IP options up to transport layer |  |  |
| MUST | Transport layer can send certain ICMP messages |  |  |
| MUST | Pass spec'd ICMP messages up to transp. layer |  |  |
| MUST | Include IP hdr+8 octets or more from orig. |  |  |
| SHOULD | Able to leap tall buildings at a single bound | – | mobile can be thrown by the user |

Table 1-5: Summary of the Requirements for Internet Hosts (IP and ICMP)

# Protocol

## Deactivated State

### Activation of IP

ACI                         UDP                        IP

|                           |                          |

|                   IPA_ACTIVATE_REQ                   |

*=====================================================>*

|                           |                        (IP 1)

|                           |                          |

|                           |       DTI2_READY_IND     |

|                           *<=========================*

|                        (UDP 1)                       |

|                           |                          |

|                           |                          |

|                   IPA_ACTIVATE_CNF                   |

*<=====================================================*

(ACI 1)                       |                          |

|                           |                          |

(IP 1)

The IP entity receives the IPA_ACTIVATE_REQ and is switched to the activated state. At this point every related transport layer must be known.

(UDP 1)

IP informs UDP that it is ready to receive data. (Occurs with each transport layer)

(ACI 1)

IP informs ACI that the entity is activated.

### Reception of DTI2_DATA_REQ in Deactivated State

UDP                        IP

|                          |

|      DTI2_DATA_REQ       |

*=========================>*

|                        (IP 1)

|                          |

(IP 1)

The IP entity receives the DTI2_DATA_REQ from a transport layer and ignores it.

### Reception of DTI2_READY_IND in Deactivated State

IP                         PPP

|                          |

|      DTI2_READY_IND      |

*<=========================*

(IP 1)                       |

|                          |

(IP 1)

The IP entity receives the DTI2_READY_IND from an interface layer and stores it.

### Reception of DTI2_DATA_IND in Deactivated State

IP                         PPP

|                          |

|      DTI2_DATA_IND       |

*<=========================*

(IP 1)                       |

|                          |

(IP 1)

The IP entity receives the DTI2_DATA_IND from an interface layer and ignores it.

### Reception of IP_ADDR_REQ in Deactivated State

UDP                        IP

|                          |

|      IP_ADDR_REQ         |

*=========================>*

|                        (IP 1)

|                          |

|      IP_ADDR_CNF         |

*<=========================*

(UDP 1)                      |

|                          |

(IP 1)

The IP entity receives the IP_ADDR_REQ.

(UDP 1)

IP sends an IP_ADDR_CNF with the err field set to IP_NOT_READY.

## Activated and not-configured State

### Configuration of an Interface (up)

ACI                         IP                         PPP

|                           |                          |

|      IPA_CONFIG_REQ       |                          |

*==========================>*                          |

|                         (IP 1)                       |

|                           |                          |

|                           |      DTI2_GETDATA_REQ    |

|                           *=========================>*

|                           |                       (PPP 1)

|                           |                          |

|                           |                          |

|      IPA_CONFIG_CNF       |                          |

*<==========================*                          |

(ACI 1)                       |                          |

|                           |                          |

(IP 1)

ACI configures a transport layer interface  (ip, mtu, ttl, netmask, UP)

(PPP 1)

IP indicates that the entity is ready to get the next DTI2_DATA_IND primitive from the transport layer.

(ACI 2)

IP confirms the configuration and is now in configured and activated state.

### Reception of DTI2_DATA_REQ in not-configured State

UDP                        IP

|                          |

|      DTI2_DATA_REQ       |

*=========================>*

|                        (IP 1)

|                          |

(IP 1)

The IP entity receives the DTI2_DATA_REQ from a transport layer and ignores it.

### Reception of DTI2_READY_IND in not-configured State

IP                         PPP

|                          |

|      DTI2_READY_IND      |

*<=========================*

(IP 1)                       |

|                          |

(IP 1)

The IP entity receives the DTI2_READY_IND from an interface layer and stores it.

### Reception of DTI2_DATA_IND in not-configured State

IP                        PPP

|                          |

|      DTI2_DATA_IND       |

*<=========================*

(IP 1)                       |

|                          |

(IP 1)

The IP entity receives the DTI2_DATA_IND from an interface layer and ignores it.

### Reception of IP_ADDR_REQ in not-configured State

UDP                        IP

|                          |

|       IP_ADDR_REQ        |

*=========================>*

|                        (IP 1)

|                          |

|       IP_ADDR_CNF        |

*<=========================*

(UDP 1)                      |

|                          |

(IP 1)

The IP entity receives the IP_ADDR_REQ.

(UDP 1)

IP sends an IP_ADDR_CNF with the err field set to IP_NOT_READY.

### Deactivation of IP

ACI                         UDP                        IP

|                           |                          |

|                  IPA_DEACTIVATE_REQ                  |

*=====================================================>*

|                           |                        (IP 1)

|                           |                          |

|                           |                          |

|                  IPA_DEACTIVATE_CNF                  |

*<=====================================================*

(ACI 1)                       |                          |

|                           |                          |

(IP 1)

The IP entity receives the IPA_DEACTIVATE_REQ and is switched to the deactivated state.

(ACI 1)

IP informs ACI that the entity is deactivated.

## Configured State

### Configuration of an Interface (down)

ACI                         IP

|                           |

|      IPA_CONFIG_REQ       |

*==========================>*

|                         (IP 1)

|                           |

|      IPA_CONFIG_CNF       |

*<==========================*

(ACI 1)                       |

|                           |

(IP 1)

ACI configures a transport layer interface  (…, DOWN)

(ACI 2)

IP confirms the configuration and ignores all occurring DTI2_DATA_IND from the closed interface layer.
If no other interface is configured, the unit goes in the not-configured state.

### Configuration of an Interface (up)

ACI                         IP                         PPP

|                           |                          |

|      IPA_CONFIG_REQ       |	                          |

*==========================>*                          |

|                         (IP 1)                       |

|                           |                          |

|                           |      DTI2_GETDATA_REQ    |

|                           *=========================>*

|                           |                       (PPP 1)

|                           |                          |

|                           |                          |

|      IPA_CONFIG_CNF       |                          |

*<==========================*                          |

(ACI 1)                       |                          |

|                           |                          |

(IP 1)

ACI configures a transport layer interface  (ip, peer_ip, mtu, ui, netmask, UP)

(PPP 1)

IP indicates that the entity is ready to get the next DTI2_DATA_IND primitive from the transport layer.

(ACI 2)

IP confirms the configuration.

### Reception of IP_ADDR_REQ in Configured State

UDP                        IP

|                          |

|      IP_ADDR_REQ         |

*=========================>*

|                        (IP 1)

|                          |

|      IP_ADDR_CNF         |

*<=========================*

(UDP 1)                      |

|                          |

(IP 1)

The IP entity receives the IP_ADDR_REQ.

(UDP 1)

IP sends an IP_ADDR_CNF with the src_addr field set to the correct source address or the err field set to IP_ADDR_NOROUTE.

### Reception of DTI2_DATA_REQ in Configured State

UDP                         IP                         PPP

|                           |                          |

|     DTI2_DATA_REQ         |                          |

*==========================>*                          |

|                         (IP 1)                       |

|                           |                          |

|                           |       DTI2_DATA_REQ      |

|                           *=========================>*

|                           |                       (PPP 1)

|                           |                          |

|     DTI2_DATA_REQ         |	                          |

*<==========================*                          |

(UDP 1)                       |                          |

|                           |                          |

(IP 1)

IP receives a datagram or an ICMP message from the transport layer.

(PPP 1)

IP determines the relevant interface and sends an IP datagram via DTI2_DATA_REQ.

(UDP 1)

IP  indicates that the entity is ready to get the next DTI2_DATA_REQ primitive from this transport layer.

### Reception of DTI2_DATA_REQ in Configured State

UDP                         IP                         PPP

|                           |                          |

|                           |       DTI2_DATA_IND      |

|                           *<=========================*

|                         (IP 1)                       |

|                           |                          |

|      DTI2_DATA_IND        |	                          |

*<==========================*                          |

(UDP 1)                       |                          |

|                           |                          |

|                           |     DTI2_GETDATA_REQ     |

|                           *=========================>*

|                           |                       (PPP 1)

|                           |                          |

(IP 1)

IP receives a datagram from the interface layer.

(UDP 1)

IP determines the relevant transport layer  and sends an IP datagram via DTI2_DATA_REQ.

(PPP 2)

IP indicates that the entity is ready to get the next DTI2_DATA_IND primitive.

### Reception of DTI2_DATA_REQ in Configured State (ERROR/ICMP)

UDP                         IP                         PPP

|                           |                          |

|                           |       DTI2_DATA_IND      |

|                           *<=========================*

|                         (IP 1)                       |

|                           |                          |

|                           |       DTI2_DATA_REQ      |

|                           *=========================>*

|                           |                       (PPP 1)

|                           |                          |

|                           |    DTI2_GETDATA_REQ      |

|                           *=========================>*

|                           |                       (PPP 2)

|                           |                          |

(IP 1)

IP receives a datagram from the interface layer.

(PPP 1)

IP sends an ICMP Message via DTI2_DATA_REQ.

(PPP 2)

IP indicates that the entity is ready to get the next DTI2_DATA_IND primitive.

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. URL: http://www.itu.int/home |
| --- | --- |

Sometimes, however, it is still convenient to talk about networks with netmasks of 8, 16, and 24 bits as class A, B, and C networks, regardless of their place in the address space, e. g. “the class B network 62.207.0.0”.