Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document – Confidential

GSM Protocol Stack

Test Specification

PKTIO

(Edit via File/Properties/Summary!)

| Document Number: | 8443.407.02.002 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.3 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2002-Oct-15 |
| Last changed: | 2003-May-22 by XGUTTEFE |
| File Name: | pktio.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2002-Oct-15 | HM |  | 0.1 |  | 1 |
| 2003-Apr-03 | HM |  | 0.2 |  | 2 |
| 2003-May-22 | XGUTTEFE |  | 0.3 | Draft |  |

Notes:

- Initial version

- Fixed typo

Table of Contents

1.1	Terms	6

2	Overview	6

2.1	GRR (RLC/MAC) – Radio Link Control/Medium Access Control	7

2.2	LLC – Logical Link Control	7

2.3	GMM – GPRS Mobility Management	7

2.4	SM – Session Management	7

2.5	SNDCP - Subnetwork Dependant Convergence Protocol	7

2.6	GACI – GPRS Application Control Interface	7

2.7	USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	7

2.8	TOM – Tunnelling of Messages	8

3	Parameters	8

4	TEST CASES	16

4.1	Internal Routing	16

4.1.1	PKTIO000: Setup the routing and PCO view for the PKTIO tests	16

4.2	Connection phase	16

4.2.1	PKTIO010: Connection of first device requested	16

4.2.2	PKTIO011: Response to connection of first device - positive	17

4.2.3	PKTIO012: Response to connection of first device negative	18

4.2.4	PKTIO013: Received DRV_SIGTYPE_DISCONNECT in conn. phase	19

4.2.5	PKTIO014: Clash of DISCONNECT signal and PKT_CONNECT_RES	20

4.2.6	PKTIO015: Clash of DISCONNECT signal and PKT_CONNECT_REJ	20

4.2.7	PKTIO016: DRV_SIGTYPE_DISCONNECT in state KER_NO_DTI	21

4.2.8	PKTIO020: Opening of DTI connection requested	21

4.2.9	PKTIO021: Opening of DTI connection confirmed	22

4.2.10	PKTIO022: Initial flow control primive from peer received	23

4.2.11	PKTIO023: Opening of DTI connection rejected	23

4.2.12	PKTIO024: DTI disconnection in state KER_READY	24

4.2.13	PKTIO025:KER_DTI_READY, DRV_SIGTYPE_DISCONNECT received	25

4.2.14	PKTIO026: KER_DTI_OPENING, DRV_SIGTYPE_DISCONNECT received	26

4.2.15	PKTIO027: More than one read buffer supported by DIO driver	27

4.2.16	PKTIO028: PKT_DTI_CLOSE_REQ received	29

4.3	Modifications	29

4.3.1	PKTIO050: KER_READY, modification of driver settings immediately possible	29

4.3.2	PKTIO051: KER_READY, Modification of driver settings requested	30

4.3.3	PKTIO052: KER_MODIFY, end of flushing in signalled	31

4.3.4	PKTIO053: KER_MODIFY, DTI2_DISCONNECT_REQ	33

4.3.5	PKTIO054: KER_MODIFY_NO_DTI, DRV_SIGTYPE_FLUSH signalled	33

4.3.6	PKTIO055: KER_MODIFY, DRV_SIGTYPE_DISCONNECT signalled	34

4.3.7	PKTIO056: KER_MODIFY_NO_DTI and DRV_SIGTYPE_DISCONNECT	36

4.3.8	PKTIO057: KER_MODIFY / TX_FLUSH_BUFFER, end of flushing	36

4.3.9	PKTIO058: KER_MODIFY / TX_FLUSHING, end of flushing	38

4.3.10	PKTIO059: TX_FLUSHING or TX_FLUSH_BUFFER, packet written	39

4.3.11	PKTIO060: KER_DTI_OPENING, PKT_MODIFY_REQ immediate	40

4.3.12	PKTIO061: KER_DTI_OPENING, PKT_MODIFY_REQ after flush	41

4.3.13	PKTIO062: KER_MODIFY_DTI_OPENING, DRV_SIGTYPE_FLUSH	42

4.3.14	PKTIO063: KER_MODIFY_DTI_OPENING, DTI2_DISCONNECT_REQ	43

4.3.15	PKTIO064: KER_MODIFY_DTI_OPENING, PKT_DTI_OPEN_CNF	43

4.3.16	PKTIO065: KER_MODIFY_DTI_OPENING, DRV_SIGTYPE_DISCONNECT	44

4.3.17	PKTIO070: KER_NO_DTI, PKT_MODIFY_REQ immediate	45

4.3.18	PKTIO071: KER_DTI_OPENING, PKT_MODIFY_REQ after flush	46

4.4	Connected phase - focus on RX / DTX	47

4.4.1	PKTIO100: KER_READY / DTX_READY, UL packet received	47

4.4.2	PKTIO101: Flow control primitive from DTI after DL data tranfer	48

4.4.3	PKTIO102: No flow control primitive from DTI after DL data tranfer	49

4.4.4	PKTIO103: No flow control primitive from DTI after DL data tranfer	50

4.4.5	PKTIO104: DTI becomes ready while one descriptor present in PKTIO	51

4.4.6	PKTIO106: Reception of flow control primitive, no filled read buffer present	52

4.4.7	PKTIO107: Reception of flow control primitive, filled read buffer present	52

4.4.8	PKTIO108: RX_BUFFER, DRV_SIGTYPE_DISCONNECT from DIO	53

4.4.9	PKTIO109: KER_READY, RX_READY, DTX_READY, packet received	54

4.4.10	PKTIO110: KER_READY, RX_READY, DTX_NOT_READY, flow control	55

4.4.11	PKTIO111: KER_READY, RX_READY, DTX_NOT_READY, packets received	56

4.4.12	PKTIO112: KER_READY, RX_BUFFER, DTX_NOT_READY, flow control	57

4.4.13	PKTIO113: KER_READY, RX_BUFFER, DTX_NOT_READY, flow control	58

4.5	Connected phase - focus on RX / DTX, multi instance tests	60

4.6	Connected phase - focus on TX / DRX	60

4.6.1	PKTIO200: DRX_READY, DL packet received	60

4.6.2	PKTIO201: DRX_READY, DL packet received, DIO driver has one packet	61

4.6.3	PKTIO202: DRX_READY, TX_READY, packet written by driver	62

4.6.4	PKTIO203: DRX_NOT_READY, TX_BUFFER, packet written by driver	63

4.6.5	PKTIO204: DRX_NOT_READY, TX_BUFFER, disconnect by driver	64

4.6.6	PKTIO205: DRX_READY, TX_FLUSHING, DL packet received	65

4.6.7	PKTIO206: DRX_NOT_READY, TX_BUFFER, disconnect by peer	65

4.6.8	PKTIO207: DRX_NOT_READY, TX_BUFFER, disconnect req. by ACI	66

4.6.9	PKTIO208: DRX_DEAD_NOT_READY, TX_BUFFER	67

4.6.10	PKTIO209: DRX_DEAD_NOT_READY, TX_BUFFER	68

4.6.11	PKTIO210: DRX_DEAD_READY, TX_READY, driver disconnects	69

4.6.12	PKTIO211: DRX_READY, DL packet received, DIO driver has one packet	70

4.7	Config primitives	71

4.7.1	PKTIO301: Config Primitive SEND	71

4.7.2	PKTIO302: Driver returns correct looped back data after config primitive SEND	71

4.7.3	PKTIO303: Driver returns looped back data after config primitive SEND, wrong length	72

4.7.4	PKTIO304: Driver returns looped back data after config primitive SEND, length is ok but content is wrong	74

4.7.5	PKTIO305: Second Config Primitive SEND	75

4.7.6	PKTIO306: Flow control primive from peer received	75

4.7.7	PKTIO307: Driver returns second correct looped back data after config primitive SEND	76

5	Suites	77

Appendices	82

A.	Acronyms	82

B.	Glossary	82

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for GPRS consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity.

- Figure 2-1: Architecture of the GSM/GPRS protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the GPRS protocol stack are:

## GRR (RLC/MAC) – Radio Link Control/Medium Access Control

This layer contains two functions: The Radio Link Control function provides a radio-solution-dependent reliable link. The Medium Access Control function controls the access signalling (request and grant) procedures for the radio channel, and the mapping of LLC frames onto the GSM physical channel.

## LLC – Logical Link Control

The LLC entity provides multiple highly reliable logical links for asynchronous data transfer between the MS and the network. It supports variable-length information frames, acknowledged and unacknowledged data transfer, flow and sequence control, error detection and recovery, notification of unrecoverable errors, user identity confidentiality, and ciphering of user and signaling data.

## GMM – GPRS Mobility Management

The GMM entity provides procedures for the mobility of the MS, such as informing the network of its present location, and user identity confidentiality. It manages the GMM context (attach, detach, routing area updating), supports security functions such as authentication of user and MS, controls ciphering of data, and initiates the response to paging messages.

## SM – Session Management

The main function of the session management (SM) is to support PDP context handling of the user terminal. Session Management activates, modifies and deletes the contexts for packet data protocols (PDP). Session Management services are provided at the SMREG-SAP and the SNSM-SAP for anonymous and non-anonymous access. The non-anonymous and anonymous access procedures for PDP context activation and PDP context deactivation are available at the SMREG-SAP. In addition there exists a PDP context modification for non-anonymous PDP contexts.

## SNDCP - Subnetwork Dependant Convergence Protocol

- SNDCP carries out all functions related to transfer of Network layer Protocol Data Units (N-PDUs) over GPRS in a transparent way. SNDCP helps to improve channel efficiency by means of compression techniques. The set of protocol entities above SNDCP consists of commonly used network protocols. They all use the same SNDCP entity, which then performs multiplexing of data coming from different sources to be sent using the service provided by the LLC layer.

## GACI – GPRS Application Control Interface

The GACI is the GPRS extension of the ACI. It is specified in GSM 07.07 and 07.60. It is responsible for processing of the GPRS related AT Commands to setup, activate and deactivate the PDP context parameter. It also provides functionality for the interworking between GMM/SM/SNDCP and a packet oriented protocol like PPP.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the GPRS. The driver has to be able to perform flow control.

## TOM – Tunnelling of Messages

The TOM entity is present if and only if HS136 is supported (the feature flag FF_HS136 is enabled).

The main function of TOM is to tunnel non-GSM signalling messages between the MS and the SGSN. The only non-GSM signalling which is currently supported by TOM is for the EGPRS-136 system (according to TIA/EIA-136-376). Data transfer in both uplink and downlink direction is possible. Two different priorities (high, low) of signalling data transfer are supported. TOM uses the unacknowledged mode of LLC and the acknowledged mode of GRR (RLC/MAC).

# Parameters

FIELD (AN_EXAMPLE_FIELD)
	0x00, 0x01
ENDFIELD (AN_EXAMPLE_FIELD, 2)

/*
 * Some SDU test data
 */

FIELD (A_UL_SDU)
	0x00, 0x03,		/* Length in bits */
	0x00, 0x00,		/* Offset in bits */
	0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07,	/* 96 Data bytes*/
	0x08, 0x09, 0x0A, 0x0B, 0x0C, 0x0D, 0x0E, 0x0F,
	0x10, 0x11, 0x12, 0x13, 0x14, 0x15, 0x16, 0x17,
	0x18, 0x19, 0x1A, 0x1B, 0x1C, 0x1D, 0x1E, 0x1F,
	0x20, 0x21, 0x22, 0x23, 0x24, 0x25, 0x26, 0x27,
	0x28, 0x29, 0x2A, 0x2B, 0x2C, 0x2D, 0x2E, 0x2F,
	0x30, 0x31, 0x32, 0x33, 0x34, 0x35, 0x36, 0x37,
	0x38, 0x39, 0x3A, 0x3B, 0x3C, 0x3D, 0x3E, 0x3F,
	0x40, 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47,
	0x48, 0x49, 0x4A, 0x4B, 0x4C, 0x4D, 0x4E, 0x4F,
	0x50, 0x51, 0x52, 0x53, 0x54, 0x55, 0x56, 0x57,
	0x58, 0x59, 0x5A, 0x5B, 0x5C, 0x5D, 0x5E, 0x5F
ENDFIELD (A_UL_SDU, 100)

FIELD (A_FIRST_UL_SDU)
	0x40, 0x03,		/* Length in bits */
	0x00, 0x00,		/* Offset in bits */
	0x01, 0x01, 0x01, 0x01, 0x01, 0x01, 0x01, 0x01,	/* 104 Data bytes*/
	0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07,
	0x08, 0x09, 0x0A, 0x0B, 0x0C, 0x0D, 0x0E, 0x0F,
	0x10, 0x11, 0x12, 0x13, 0x14, 0x15, 0x16, 0x17,
	0x18, 0x19, 0x1A, 0x1B, 0x1C, 0x1D, 0x1E, 0x1F,
	0x20, 0x21, 0x22, 0x23, 0x24, 0x25, 0x26, 0x27,
	0x28, 0x29, 0x2A, 0x2B, 0x2C, 0x2D, 0x2E, 0x2F,
	0x30, 0x31, 0x32, 0x33, 0x34, 0x35, 0x36, 0x37,
	0x38, 0x39, 0x3A, 0x3B, 0x3C, 0x3D, 0x3E, 0x3F,
	0x40, 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47,
	0x48, 0x49, 0x4A, 0x4B, 0x4C, 0x4D, 0x4E, 0x4F,
	0x50, 0x51, 0x52, 0x53, 0x54, 0x55, 0x56, 0x57,
	0x58, 0x59, 0x5A, 0x5B, 0x5C, 0x5D, 0x5E, 0x5F
ENDFIELD (A_FIRST_UL_SDU, 108)

FIELD (A_SECOND_UL_SDU)
	0x40, 0x03,		/* Length in bits */
	0x00, 0x00,		/* Offset in bits */
	0x02, 0x02, 0x02, 0x02, 0x02, 0x02, 0x02, 0x02,	/* 104 Data bytes*/
	0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07,
	0x08, 0x09, 0x0A, 0x0B, 0x0C, 0x0D, 0x0E, 0x0F,
	0x10, 0x11, 0x12, 0x13, 0x14, 0x15, 0x16, 0x17,
	0x18, 0x19, 0x1A, 0x1B, 0x1C, 0x1D, 0x1E, 0x1F,
	0x20, 0x21, 0x22, 0x23, 0x24, 0x25, 0x26, 0x27,
	0x28, 0x29, 0x2A, 0x2B, 0x2C, 0x2D, 0x2E, 0x2F,
	0x30, 0x31, 0x32, 0x33, 0x34, 0x35, 0x36, 0x37,
	0x38, 0x39, 0x3A, 0x3B, 0x3C, 0x3D, 0x3E, 0x3F,
	0x40, 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47,
	0x48, 0x49, 0x4A, 0x4B, 0x4C, 0x4D, 0x4E, 0x4F,
	0x50, 0x51, 0x52, 0x53, 0x54, 0x55, 0x56, 0x57,
	0x58, 0x59, 0x5A, 0x5B, 0x5C, 0x5D, 0x5E, 0x5F
ENDFIELD (A_SECOND_UL_SDU, 108)

FIELD (A_THIRD_UL_SDU)
	0x40, 0x03,		/* Length in bits */
	0x00, 0x00,		/* Offset in bits */
	0x03, 0x03, 0x03, 0x03, 0x03, 0x03, 0x03, 0x03,	/* 104 Data bytes*/
	0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07,
	0x08, 0x09, 0x0A, 0x0B, 0x0C, 0x0D, 0x0E, 0x0F,
	0x10, 0x11, 0x12, 0x13, 0x14, 0x15, 0x16, 0x17,
	0x18, 0x19, 0x1A, 0x1B, 0x1C, 0x1D, 0x1E, 0x1F,
	0x20, 0x21, 0x22, 0x23, 0x24, 0x25, 0x26, 0x27,
	0x28, 0x29, 0x2A, 0x2B, 0x2C, 0x2D, 0x2E, 0x2F,
	0x30, 0x31, 0x32, 0x33, 0x34, 0x35, 0x36, 0x37,
	0x38, 0x39, 0x3A, 0x3B, 0x3C, 0x3D, 0x3E, 0x3F,
	0x40, 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47,
	0x48, 0x49, 0x4A, 0x4B, 0x4C, 0x4D, 0x4E, 0x4F,
	0x50, 0x51, 0x52, 0x53, 0x54, 0x55, 0x56, 0x57,
	0x58, 0x59, 0x5A, 0x5B, 0x5C, 0x5D, 0x5E, 0x5F
ENDFIELD (A_THIRD_UL_SDU, 108)

FIELD (A_DL_SDU)
	0x00, 0x03,		/* Length in bits */
	0x00, 0x00,		/* Offset in bits */
	0x5F, 0x5E, 0x5D, 0x5C, 0x5B, 0x5A, 0x59, 0x58,	/* 96 Data bytes*/
	0x57, 0x56, 0x55, 0x54, 0x53, 0x52, 0x51, 0x50,
	0x4F, 0x4E, 0x4D, 0x4C, 0x4B, 0x4A, 0x49, 0x48,
	0x47, 0x46, 0x45, 0x44, 0x43, 0x42, 0x41, 0x40,
	0x3F, 0x3E, 0x3D, 0x3C, 0x3B, 0x3A, 0x39, 0x38,
	0x37, 0x36, 0x35, 0x34, 0x33, 0x32, 0x31, 0x30,
	0x2F, 0x2E, 0x2D, 0x2C, 0x2B, 0x2A, 0x29, 0x28,
	0x27, 0x26, 0x25, 0x24, 0x23, 0x22, 0x21, 0x20,
	0x1F, 0x1E, 0x1D, 0x1C, 0x1B, 0x1A, 0x19, 0x18,
	0x17, 0x16, 0x15, 0x14, 0x13, 0x12, 0x11, 0x10,
	0x0F, 0x0E, 0x0D, 0x0C, 0x0B, 0x0A, 0x09, 0x08,
	0x07, 0x06, 0x05, 0x04, 0x03, 0x02, 0x01, 0x00
ENDFIELD (A_DL_SDU, 100)

FIELD (A_FIRST_DL_SDU)
	0x40, 0x03,		/* Length in bits */
	0x00, 0x00,		/* Offset in bits */
	0x01, 0x01, 0x01, 0x01, 0x01, 0x01, 0x01, 0x01,	/* 104 Data bytes*/
	0x5F, 0x5E, 0x5D, 0x5C, 0x5B, 0x5A, 0x59, 0x58,
	0x57, 0x56, 0x55, 0x54, 0x53, 0x52, 0x51, 0x50,
	0x4F, 0x4E, 0x4D, 0x4C, 0x4B, 0x4A, 0x49, 0x48,
	0x47, 0x46, 0x45, 0x44, 0x43, 0x42, 0x41, 0x40,
	0x3F, 0x3E, 0x3D, 0x3C, 0x3B, 0x3A, 0x39, 0x38,
	0x37, 0x36, 0x35, 0x34, 0x33, 0x32, 0x31, 0x30,
	0x2F, 0x2E, 0x2D, 0x2C, 0x2B, 0x2A, 0x29, 0x28,
	0x27, 0x26, 0x25, 0x24, 0x23, 0x22, 0x21, 0x20,
	0x1F, 0x1E, 0x1D, 0x1C, 0x1B, 0x1A, 0x19, 0x18,
	0x17, 0x16, 0x15, 0x14, 0x13, 0x12, 0x11, 0x10,
	0x0F, 0x0E, 0x0D, 0x0C, 0x0B, 0x0A, 0x09, 0x08,
	0x07, 0x06, 0x05, 0x04, 0x03, 0x02, 0x01, 0x00
ENDFIELD (A_FIRST_DL_SDU, 108)

FIELD (A_SECOND_DL_SDU)
	0x40, 0x03,		/* Length in bits */
	0x00, 0x00,		/* Offset in bits */
	0x02, 0x02, 0x02, 0x02, 0x02, 0x02, 0x02, 0x02,	/* 104 Data bytes*/
	0x5F, 0x5E, 0x5D, 0x5C, 0x5B, 0x5A, 0x59, 0x58,
	0x57, 0x56, 0x55, 0x54, 0x53, 0x52, 0x51, 0x50,
	0x4F, 0x4E, 0x4D, 0x4C, 0x4B, 0x4A, 0x49, 0x48,
	0x47, 0x46, 0x45, 0x44, 0x43, 0x42, 0x41, 0x40,
	0x3F, 0x3E, 0x3D, 0x3C, 0x3B, 0x3A, 0x39, 0x38,
	0x37, 0x36, 0x35, 0x34, 0x33, 0x32, 0x31, 0x30,
	0x2F, 0x2E, 0x2D, 0x2C, 0x2B, 0x2A, 0x29, 0x28,
	0x27, 0x26, 0x25, 0x24, 0x23, 0x22, 0x21, 0x20,
	0x1F, 0x1E, 0x1D, 0x1C, 0x1B, 0x1A, 0x19, 0x18,
	0x17, 0x16, 0x15, 0x14, 0x13, 0x12, 0x11, 0x10,
	0x0F, 0x0E, 0x0D, 0x0C, 0x0B, 0x0A, 0x09, 0x08,
	0x07, 0x06, 0x05, 0x04, 0x03, 0x02, 0x01, 0x00
ENDFIELD (A_SECOND_DL_SDU, 108)

FIELD (A_THIRD_DL_SDU)
	0x40, 0x03,		/* Length in bits */
	0x00, 0x00,		/* Offset in bits */
	0x03, 0x03, 0x03, 0x03, 0x03, 0x03, 0x03, 0x03,	/* 104 Data bytes*/
	0x5F, 0x5E, 0x5D, 0x5C, 0x5B, 0x5A, 0x59, 0x58,
	0x57, 0x56, 0x55, 0x54, 0x53, 0x52, 0x51, 0x50,
	0x4F, 0x4E, 0x4D, 0x4C, 0x4B, 0x4A, 0x49, 0x48,
	0x47, 0x46, 0x45, 0x44, 0x43, 0x42, 0x41, 0x40,
	0x3F, 0x3E, 0x3D, 0x3C, 0x3B, 0x3A, 0x39, 0x38,
	0x37, 0x36, 0x35, 0x34, 0x33, 0x32, 0x31, 0x30,
	0x2F, 0x2E, 0x2D, 0x2C, 0x2B, 0x2A, 0x29, 0x28,
	0x27, 0x26, 0x25, 0x24, 0x23, 0x22, 0x21, 0x20,
	0x1F, 0x1E, 0x1D, 0x1C, 0x1B, 0x1A, 0x19, 0x18,
	0x17, 0x16, 0x15, 0x14, 0x13, 0x12, 0x11, 0x10,
	0x0F, 0x0E, 0x0D, 0x0C, 0x0B, 0x0A, 0x09, 0x08,
	0x07, 0x06, 0x05, 0x04, 0x03, 0x02, 0x01, 0x00
ENDFIELD (A_THIRD_DL_SDU, 108)

FIELD (CONFIG_64_SDU)
	0x00, 0x02,		/* Length in bits */
	0x00, 0x00,		/* Offset in bits */
  0x00, 0x00, 0x00, 0x01, 0x00, 0x02, 0x00, 0x03,
  0x00, 0x04, 0x00, 0x05, 0x00, 0x06, 0x00, 0x07,
  0x00, 0x08, 0x00, 0x09, 0x00, 0x0a, 0x00, 0x0b,
  0x00, 0x0c, 0x00, 0x0d, 0x00, 0x0e, 0x00, 0x0f,
  0x00, 0x10, 0x00, 0x11, 0x00, 0x12, 0x00, 0x13,
  0x00, 0x14, 0x00, 0x15, 0x00, 0x16, 0x00, 0x17,
  0x00, 0x18, 0x00, 0x19, 0x00, 0x1a, 0x00, 0x1b,
  0x00, 0x1c, 0x00, 0x1d, 0x00, 0x1e, 0x00, 0x1f

ENDFIELD (CONFIG_64_SDU, 68)

FIELD (CONFIG_32_SDU)
	0x00, 0x01,		/* Length in bits */
	0x00, 0x00,		/* Offset in bits */
  0x00, 0x00, 0x00, 0x01, 0x00, 0x02, 0x00, 0x03,
  0x00, 0x04, 0x00, 0x05, 0x00, 0x06, 0x00, 0x07,
  0x00, 0x08, 0x00, 0x09, 0x00, 0x0a, 0x00, 0x0b,
  0x00, 0x0c, 0x00, 0x0d, 0x00, 0x0e, 0x00, 0x0f
ENDFIELD (CONFIG_32_SDU, 36)

FIELD (CONFIG_64_WRONG_CONTENT)
	0x00, 0x02,		/* Length in bits */
	0x00, 0x00,		/* Offset in bits */
  0x00, 0x00, 0x00, 0x01, 0x00, 0x02, 0x00, 0x03,
  0x00, 0x04, 0x00, 0x05, 0x00, 0x06, 0x00, 0x07,
  0x00, 0x08, 0x00, 0x09, 0x00, 0x0a, 0x00, 0x0b,
  0x00, 0x0c, 0x00, 0x0d, 0x00, 0x0e, 0x00, 0x0f,
  0x00, 0x10, 0x00, 0x11, 0x00, 0x12, 0x00, 0x13,
  0x00, 0x14, 0x00, 0x15, 0x00, 0x16, 0x00, 0x17,
  0x00, 0x18, 0x00, 0x19, 0x00, 0x1a, 0x00, 0x1b,
  0x00, 0x1c, 0x00, 0x1d, 0x00, 0x1e, 0x00, 0xff

ENDFIELD (CONFIG_64_WRONG_CONTENT, 68)

FIELD (CONFIG_WRONG_LEN)
	0x40, 0x00,		/* Length in bits */
	0x00, 0x00,		/* Offset in bits */
  0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDFIELD (CONFIG_WRONG_LEN, 12)

FIELD (CONFIG_WRONG_CONTENT)
	0x40, 0x00,		/* Length in bits */
	0x00, 0x00,		/* Offset in bits */
  0xff, 0xff, 0xff, 0xff, 0xff, 0xff, 0xff, 0xff
ENDFIELD (CONFIG_WRONG_CONTENT, 12)

DECLARATION (AN_EXAMPLE_ARRAY)
DECLARATION (DEFAULT_DCB_FROM_DIO)
DECLARATION (DEFAULT_DCB_FROM_ACI)
DECLARATION (MODIFIED_DCB_FROM_ACI)
DECLARATION (TRASH_DCB_FROM_ACI)
/* DECLARATION (AN_EXAMPLE_MSTRUCT) */
DECLARATION (AN_EXAMPLE_PSTRUCT_ARRAY)

/*
 * Simple ordinal numbers from zero to nine, type BYTE
 */
BYTE	BYTE_0	0
BYTE	BYTE_1	1
BYTE	BYTE_2	2
BYTE	BYTE_3	3
BYTE	BYTE_4	4
BYTE	BYTE_5	5
BYTE	BYTE_6	6
BYTE	BYTE_7	7
BYTE	BYTE_8	8
BYTE	BYTE_9	9

/*
 * Device numbers
 */
/*
BYTE	DEVICE_0	0
BYTE	DEVICE_1	1
BYTE	DEVICE_2	2
BYTE	DEVICE_3	3
BYTE	DEVICE_4	4
BYTE	DEVICE_5	5
BYTE	DEVICE_6	6
BYTE	DEVICE_7	7
BYTE	DEVICE_8	8
BYTE	DEVICE_9	9
*/
BYTE	DEVICE_47	0x47	/* No low device numbers guaranteed */
BYTE	DEVICE_66	0x66	/* Some other device */

BYTE	DTI_DIRECTION_NORMAL	0x00
BYTE	DTI_DIRECTION_INVERTED	0x01

/*
 * Simple ordinal numbers from zero to nine, type SHORT
 */
SHORT	SHORT_0	0x0000
SHORT	SHORT_1	0x0001
SHORT	SHORT_2	0x0002
SHORT	SHORT_3	0x0003
SHORT	SHORT_4	0x0004
SHORT	SHORT_5	0x0005
SHORT	SHORT_6	0x0006
SHORT	SHORT_7	0x0007
SHORT	SHORT_8	0x0008
SHORT	SHORT_9	0x0009

/*
 * Maximum transmission unit
 */

SHORT	MTU_1500	1500
SHORT	MTU_512	512
SHORT	MTU_TRASH	10000

/*
 * Return values as defined in gdi.h
 */
SHORT	DRV_OK	0x0000
SHORT	DRV_BUFFER_FULL	0x0001
SHORT	DRV_DISABLED	0x0002
SHORT	DRV_ENABLED	0x0003
SHORT	DRV_INITFAILURE	0x0004
SHORT	DRV_INITIALIZED	0x0005
SHORT	DRV_INTERNAL_ERROR	0x0006
SHORT	DRV_INPROCESS	0x0007
SHORT	DRV_INVALID_PARAMS	0x0008
SHORT	DRV_NOTCONFIGURED	0x0009

/*
 * Return values as defined in gdi.h
 */
SHORT	DRV_BUFTYPE_READ	0x0001
SHORT	DRV_BUFTYPE_WRITE	0x0002

/*
 * Signal events as defined in gdi.h
 */
SHORT	DRV_SIGTYPE_CLEAR	0x0002
SHORT	DRV_SIGTYPE_FLUSH	0x0004
SHORT	DRV_SIGTYPE_READ	0x0008
SHORT	DRV_SIGTYPE_WRITE	0x0010
SHORT	DRV_SIGTYPE_CONNECT	0x0020
SHORT	DRV_SIGTYPE_DISCONNECT	0x0040

/*
 * Simple ordinal numbers from zero to nine, type LONG
 */
LONG	LONG_0	0x00000000
LONG	LONG_1	0x00000001
LONG	LONG_2	0x00000002
LONG	LONG_3	0x00000003
LONG	LONG_4	0x00000004
LONG	LONG_5	0x00000005
LONG	LONG_6	0x00000006
LONG	LONG_7	0x00000007
LONG	LONG_8	0x00000008
LONG	LONG_9	0x00000009

/*
 * All baud rates allowed on shared memory except DIO_BAUD_AUTO
 */
LONG	DIO_BAUD_SHAREDMEM	0x000FFFFE

/*
 * The DTI peer. This is casted to a pointer to a C-String in real life, so it is impossible to set this
 * to a meaningful value here in the simulation environment. This restriction has to be handled in
 * the C-Code of the entity using a conditional compile switch.
 */
LONG	PEER_SNDCP	0x47110000

/*

* The DTI link identifier. Here an arbitrary value
 */

LONG	LINK_ID_47	0x12345647
LONG	LINK_ID_66	0x12345666

/*
 * MASK and STATE for DIO_Write(). These are not used in PKTIO and therefore shall be 0.
 */

LONG	DIO_STATE_NONE	0x00000000
LONG	DIO_STATE_IP	0x00000021
LONG	DIO_MASK_NONE	0x00000000

/*
 * ARRAYs
 */

BEGINARRAY (AN_EXAMPLE_ARRAY, 3)
	0x00, 0x01, 0x02
ENDARRAY

/*
 * PSTRUCTs
 */

BEGIN_PSTRUCT ("dio_dcb", DEFAULT_DCB_FROM_DIO)
	SET_COMP ("convergence", DIO_CONV_PACKET)
	SET_COMP ("data_mode", DIO_MODE_DATA)
	SET_COMP ("sleep_mode", DIO_SLEEP_DISABLE)
	SET_COMP ("mux_configuration", LONG_0)
	SET_COMP ("n1", BYTE_0)
	SET_COMP ("n2", BYTE_0)
	SET_COMP ("t1", BYTE_0)
	SET_COMP ("t2", BYTE_0)
	SET_COMP ("t3", BYTE_0)
	SET_COMP ("k", BYTE_0)
	SET_COMP ("mtu", MTU_1500)
	SET_COMP ("baud", DIO_BAUD_SHAREDMEM)
	SET_COMP ("data_bits", DIO_CHAR_8) /* Some defaults */
	SET_COMP ("stop_bits", DIO_STOP_1) /* Some defaults */
	SET_COMP ("parity", DIO_PARITY_NO) /* Some defaults */
	SET_COMP ("flow_control", DIO_FLOW_HW_HW) /* Shared memory is HW */
	SET_COMP ("xon", BYTE_0)
	SET_COMP ("xoff", BYTE_0)
	SET_COMP ("esc_char", BYTE_0)
	SET_COMP ("guard_period", DIO_ESC_OFF) /* No guard pattern detection */
ENDSTRUCT

BEGIN_PSTRUCT ("dio_dcb", DEFAULT_DCB_FROM_ACI)
	SET_COMP ("convergence", DIO_CONV_PACKET)
	SET_COMP ("data_mode", DIO_MODE_DATA)
	SET_COMP ("sleep_mode", DIO_SLEEP_DISABLE)
	SET_COMP ("mux_configuration", LONG_0)
	SET_COMP ("n1", BYTE_0)
	SET_COMP ("n2", BYTE_0)
	SET_COMP ("t1", BYTE_0)
	SET_COMP ("t2", BYTE_0)
	SET_COMP ("t3", BYTE_0)
	SET_COMP ("k", BYTE_0)
	SET_COMP ("mtu", MTU_1500)
	SET_COMP ("baud", DIO_BAUD_812500) /* Negotiated highest common */
	SET_COMP ("data_bits", DIO_CHAR_8)
	SET_COMP ("stop_bits", DIO_STOP_1)
	SET_COMP ("parity", DIO_PARITY_NO)
	SET_COMP ("flow_control", DIO_FLOW_HW_HW)
	SET_COMP ("xon", BYTE_0)
	SET_COMP ("xoff", BYTE_0)
	SET_COMP ("esc_char", BYTE_0)
	SET_COMP ("guard_period", DIO_ESC_OFF)
ENDSTRUCT

BEGIN_PSTRUCT ("dio_dcb", MODIFIED_DCB_FROM_ACI)
	SET_COMP ("convergence", DIO_CONV_PACKET)
	SET_COMP ("data_mode", DIO_MODE_DATA)
	SET_COMP ("sleep_mode", DIO_SLEEP_DISABLE)
	SET_COMP ("mux_configuration", LONG_0)
	SET_COMP ("n1", BYTE_0)
	SET_COMP ("n2", BYTE_0)
	SET_COMP ("t1", BYTE_0)
	SET_COMP ("t2", BYTE_0)
	SET_COMP ("t3", BYTE_0)
	SET_COMP ("k", BYTE_0)
	SET_COMP ("mtu", MTU_512)
	SET_COMP ("baud", DIO_BAUD_812500) /* Negotiated highest common */
	SET_COMP ("data_bits", DIO_CHAR_8)
	SET_COMP ("stop_bits", DIO_STOP_1)
	SET_COMP ("parity", DIO_PARITY_NO)
	SET_COMP ("flow_control", DIO_FLOW_HW_HW)
	SET_COMP ("xon", BYTE_0)
	SET_COMP ("xoff", BYTE_0)
	SET_COMP ("esc_char", BYTE_0)
	SET_COMP ("guard_period", DIO_ESC_OFF)
ENDSTRUCT

BEGIN_PSTRUCT ("dio_dcb", TRASH_DCB_FROM_ACI)
	SET_COMP ("convergence", DIO_CONV_PACKET)
	SET_COMP ("data_mode", DIO_MODE_DATA)
	SET_COMP ("sleep_mode", DIO_SLEEP_DISABLE)
	SET_COMP ("mux_configuration", LONG_0)
	SET_COMP ("n1", BYTE_0)
	SET_COMP ("n2", BYTE_0)
	SET_COMP ("t1", BYTE_0)
	SET_COMP ("t2", BYTE_0)
	SET_COMP ("t3", BYTE_0)
	SET_COMP ("k", BYTE_0)
	SET_COMP ("mtu", MTU_TRASH)
	SET_COMP ("baud", DIO_BAUD_812500) /* Negotiated highest common */
	SET_COMP ("data_bits", DIO_CHAR_8)
	SET_COMP ("stop_bits", DIO_STOP_1)
	SET_COMP ("parity", DIO_PARITY_NO)
	SET_COMP ("flow_control", DIO_FLOW_HW_HW)
	SET_COMP ("xon", BYTE_0)
	SET_COMP ("xoff", BYTE_0)
	SET_COMP ("esc_char", BYTE_0)
	SET_COMP ("guard_period", DIO_ESC_OFF)
ENDSTRUCT

/*
 * MSTRUCTs
 */

/*

BEGIN_MSTRUCT("an_example_mstruct", AN_EXAMPLE_MSTRUCT)
ENDSTRUCT
*/

/*
 * PSTRUCT_ARRAYs
*/

/*

BEGIN_PSTRUCT_ARRAY (AN_EXAMPLE_PSTRUCT_ARRAY, 1)
	AN_EXAMPLE_PSTRUCT
ENDARRAY

*/

# TEST CASES

## Internal Routing

### PKTIO000: Setup the routing and PCO view for the PKTIO tests

Description:	Routings for the PKTIO tests are set.

Preamble:	None

DIO		 PKTIO		ACI
	 |		  |		  |
COMMAND (TAP RESET)
COMMAND (MMI RESET)
COMMAND (PKT RESET)
COMMAND (SND RESET)
	 |		  |		  |
COMMAND (TAP REDIRECT CLEAR)
COMMAND (PKT REDIRECT CLEAR)
	 |		  |		  |
COMMAND (MMI REDIRECT PKT NULL)
	 |		  |		  |
COMMAND (PKT REDIRECT MMI TAP)
COMMAND (PKT REDIRECT SND TAP)
	 |		  |		  |
	 |		  |		  |
COMMAND (TAP REDIRECT TAP PKT)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

History:	15-Oct-02	HM	Initial

## Connection phase

### PKTIO010: Connection of first device requested

Description:	The DIO driver requests a connection for the first device by issuing the signalling callback with the parameter DRV_SIGTYPE_CONNECT. The PKTIO entity gets the information driver's capabilities using the DIO_GetCapabilities() function and informs ACI about the driver's wish to connect.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	PKTIO000

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_CONNECT)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETCAP_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETCAP_CNF	  |
	 |		  *<==============================*
(4)	 |	PKT_CONNECT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
>		signal_type	DRV_SIGTYPE_CONNECT

- PKT_DIO_GETCAP_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_GETCAP_CNF
		retval	DRV_OK
		dio_dcb	DEFAULT_DCB_FROM_DIO

- PKT_CONNECT_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		dio_dcb	DEFAULT_DCB_FROM_DIO

History:	15-Oct-02	HM	Initial

### PKTIO011: Response to connection of first device - positive

Description:	The PKTIO entity gets a PKT_CONNECT_RES response from ACI. The driver is configured successfully. Only one read buffer supported by the driver.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO010A
		<B>	PKTIO010B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_CONNECT_RES	  |		  |
	 *==============================>*		  |
(2)	 |		  |	PKT_DIO_SETCONFIG_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_SETCONFIG_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *===============================>*
(5)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *===============================>*
(7)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_CONNECT_RES
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		dio_dcb	DEFAULT_DCB_FROM_ACI

- PKT_DIO_SETCONFIG_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		dio_dcb	DEFAULT_DCB_FROM_ACI

- PKT_DIO_SETCONFIG_CNF
		retval	DRV_OK

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_OK

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_BUFFER_FULL

History:	15-Oct-02	HM	Initial

### PKTIO012: Response to connection of first device negative

Description:	The PKTIO entity gets a PKT_CONNECT_REJ response from ACI. The device is closed.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO010A
		<B>	PKTIO010B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_CONNECT_REJ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	PKT_DIO_CLOSEDEVICE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_CLOSEDEVICE_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_CONNECT_REJ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_CLOSEDEVICE_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_CLOSEDEVICE_CNF
		retval	DRV_OK

History:	15-Oct-02	HM	Initial

### PKTIO013: Received DRV_SIGTYPE_DISCONNECT in conn. phase

Description:	The PKTIO entity, while awaiting the PKT_CONNECT_RES primitive from ACI, gets a DRV_SIGTYPE_DISCONNECT from the driver. The disconnection is announced to ACI.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO010A
		<B>	PKTIO010B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_CLOSEDEVICE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_CLOSEDEVICE_CNF	  |
	 |		  *<==============================*
(4)	 |	PKT_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_DISCONNECT

- PKT_DIO_CLOSEDEVICE_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_CLOSEDEVICE_CNF
		retval	DRV_OK

- PKT_DISCONNECT_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		cause	PKTCS_DISCONNECT

History:	15-Oct-02	HM	Initial

### PKTIO014: Clash of DISCONNECT signal and PKT_CONNECT_RES

Description:	After the driver indicated the disconnection, PKTIO receives the PKT_CONNECT_RES primitive from ACI. It is expected that nothing happens in this collision case, the primitive from ACI is simply ignored.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO013A
		<B>	PKTIO013B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_CONNECT_RES	  |		  |
	 *==============================>*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_CONNECT_RES
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		dio_dcb	DEFAULT_DCB_FROM_ACI

History:	15-Oct-02	HM	Initial

### PKTIO015: Clash of DISCONNECT signal and PKT_CONNECT_REJ

Description:	After the driver indicated the disconnection, PKTIO receives the PKT_CONNECT_REJ primitive from ACI. No action is expected by the PKTIO entity.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO013A
		<B>	PKTIO013B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_CONNECT_REJ	  |		  |
	 *==============================>*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_CONNECT_REJ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

History:	15-Oct-02	HM	Initial

### PKTIO016: DRV_SIGTYPE_DISCONNECT in state KER_NO_DTI

Description:	In state KER_NO_DTI the driver sends the DRV_SIGTYPE_DISCONNECT signal. It has to be ensured that the buffer descriptors are got back from the DIO driver.
Note: This is the same as PKTIO013, but with different preamble.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO011A
		<B>	PKTIO011B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_CLOSEDEVICE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_CLOSEDEVICE_CNF	  |
	 |		  *<==============================*
(4)	 |	PKT_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_DISCONNECT

- PKT_DIO_CLOSEDEVICE_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_CLOSEDEVICE_CNF
		retval	DRV_OK

- PKT_DISCONNECT_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		cause	PKTCS_DISCONNECT

History:	15-Oct-02	HM	Initial

### PKTIO020: Opening of DTI connection requested

Description:	In state KER_NO_DTI the ACI requests the opening of a DTI connection.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO011A
		<B>	PKTIO011B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_DTI_OPEN_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	DTI2_CONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DTI_OPEN_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		peer	PEER_SNDCP
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		dti_direction	DTI_DIRECTION_NORMAL

- DTI2_CONNECT_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		version	DTI_VERSION_10

History:	15-Oct-02	HM	Initial

### PKTIO021: Opening of DTI connection confirmed

Description:	In state KER_DTI_OPENING the ACI answered positive. The PKTIO entity confirms the opening of the DTI connection. The initial flow control primitive ist sent by PKTIO.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO020A
		<B>	PKTIO020B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_CONNECT_RES	  |		  |
	 *==============================>*		  |
(2)	 |	DTI2_READY_IND	  |		  |
	 *<==============================*		  |
(3)	 |	PKT_DTI_OPEN_CNF	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_CONNECT_RES
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		version	DTI_VERSION_10

- DTI2_READY_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

- PKT_DTI_OPEN_CNF
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		cause	PKTCS_SUCCESS

History:	15-Oct-02	HM	Initial

### PKTIO022: Initial flow control primive from peer received

Description:	In state DTX_NOT_READY immediately after opening PKTIO receives the first flow control primitive. DTX state changes to DTX_READY.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO021A
		<B>	PKTIO021B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

History:	15-Oct-02	HM	Initial

### PKTIO023: Opening of DTI connection rejected

Description:	In state KER_DTI_OPENING the DTI2 answers negative, the connection is not to be opened. ACI is informed and the next state is KER_NO_DTI. The RX and TX states are not affected by this.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO020A
		<B>	PKTIO020B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_DISCONNECT_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	PKT_DTI_CLOSE_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_DISCONNECT_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		cause	DTI_CAUSE_NORMAL_CLOSE

- PKT_DTI_CLOSE_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

History:	15-Oct-02	HM	Initial

### PKTIO024: DTI disconnection in state KER_READY

Description:	In state KER_READY the disconnection of the DTI connection is requested.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO022A
		<B>	PKTIO022B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_DISCONNECT_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	PKT_DTI_CLOSE_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_DISCONNECT_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		cause	DTI_CAUSE_NORMAL_CLOSE

- PKT_DTI_CLOSE_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

History:	15-Oct-02	HM	Initial

### PKTIO025:KER_DTI_READY, DRV_SIGTYPE_DISCONNECT received

Description:	In state KER_DTI_READY the driver sends a DRV_SIGTYPE_DISCONNECT. All PKTIO state machines are expected to be in the NULL state after the state transitions are finished.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO022A
		<B>	PKTIO022B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	DTI2_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |	PKT_DTI_CLOSE_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	PKT_DIO_CLOSEDEVICE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	PKT_DIO_CLOSEDEVICE_CNF	  |
	 |		  *<==============================*
(6)	 |	PKT_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_DISCONNECT

- DTI2_DISCONNECT_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		cause	DTI_CAUSE_NORMAL_CLOSE

- PKT_DTI_CLOSE_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_CLOSEDEVICE_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_CLOSEDEVICE_CNF
		retval	DRV_OK

- PKT_DISCONNECT_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		cause	PKTCS_DISCONNECT

History:	15-Oct-02	HM	Initial
06-Nov-02	HK	DTI close before driver close.

### PKTIO026: KER_DTI_OPENING, DRV_SIGTYPE_DISCONNECT received

Description:	In state KER_DTI_OPENING the driver sends a DRV_SIGTYPE_DISCONNECT. All PKTIO state machines are expected to be in the NULL state after the state transitions are finished.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO020A
		<B>	PKTIO020B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	DTI2_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |	PKT_DTI_CLOSE_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	PKT_DIO_CLOSEDEVICE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	PKT_DIO_CLOSEDEVICE_CNF	  |
	 |		  *<==============================*
(6)	 |	PKT_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_DISCONNECT

- DTI2_DISCONNECT_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		cause	DTI_CAUSE_NORMAL_CLOSE

- PKT_DTI_CLOSE_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_CLOSEDEVICE_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_CLOSEDEVICE_CNF
		retval	DRV_OK

- PKT_DISCONNECT_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		cause	PKTCS_DISCONNECT

History:	15-Oct-02	HM	Initial
06-Nov-02	HK	DTI close before driver close.

### PKTIO027: More than one read buffer supported by DIO driver

Description:	Instead of the case which is tested in PKTIO011 - PKTIO022, here more than one read buffer is supported.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO010A
		<B>	PKTIO010B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_CONNECT_RES	  |		  |
	 *==============================>*		  |
(2)	 |		  |	PKT_DIO_SETCONFIG_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_SETCONFIG_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
(8)	 |	PKT_DTI_OPEN_REQ	  |		  |
	 *==============================>*		  |
(9)	 |	DTI2_CONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
(10)	 |	DTI2_CONNECT_RES	  |		  |
	 *==============================>*		  |
(11)	 |	DTI2_READY_IND	  |		  |
	 *<==============================*		  |
(12)	 |	PKT_DTI_OPEN_CNF	  |		  |
	 *<==============================*		  |
MUTE (1000)
(13)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_CONNECT_RES
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		dio_dcb	DEFAULT_DCB_FROM_ACI

- PKT_DIO_SETCONFIG_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		dio_dcb	DEFAULT_DCB_FROM_ACI

- PKT_DIO_SETCONFIG_CNF
		retval	DRV_OK

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_OK

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_OK

- PKT_DTI_OPEN_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		peer	PEER_SNDCP
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		dti_direction	DTI_DIRECTION_NORMAL

- DTI2_CONNECT_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		version	DTI_VERSION_10

- DTI2_CONNECT_RES
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		version	DTI_VERSION_10

- DTI2_READY_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

- PKT_DTI_OPEN_CNF
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		cause	PKTCS_SUCCESS

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

History:	15-Oct-02	HM	Initial
	29-Oct-02	HM	Revised

### PKTIO028: PKT_DTI_CLOSE_REQ received

Description:	In state KER_DTI_OPENING the PKTIO entity receives a PKT_DTI_CLOSE_REQ primitive from ACI.
<A> DTI2 disconnection requested in state KER_DTI_OPENING
<B> DTI2 disconnection requested in state KER_DTI_READY
<C> DTI2 disconnection requested in state KER_DTI_READY, flow control received

Variants:	<A>....<C>

Preamble:	<A>	PKTIO020A
		<B>	PKTIO021A
		<C>	PKTIO022A

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_DTI_CLOSE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	DTI2_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |	PKT_DTI_CLOSE_CNF	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DTI_CLOSE_REQ
		device_no	DEVICE_47

- DTI2_DISCONNECT_IND
		link_id	LINK_ID_47
		cause	DTI_CAUSE_NORMAL_CLOSE

- PKT_DTI_CLOSE_CNF
		device_no	DEVICE_47

History:	15-Oct-02	HM	Initial

## Modifications

### PKTIO050: KER_READY, modification of driver settings immediately possible

Description:	In state KER_READY / TX_READY the driver settings are to be modified. The driver is flushed by calling DIO_Flush(), the function returns with DRV_OK as there is nothing to flush so that the modification immeditately can take place. The state KER_MODIFY needs not to be entered, the next state is KER_READY again.
<A> Successful case
<B> Parameters from ACI were trash, driver refused.

Variants:	<A>....<B>

Preamble:	PKTIO022A

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_MODIFY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	PKT_DIO_FLUSH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_FLUSH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	PKT_DIO_SETCONFIG_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	PKT_DIO_SETCONFIG_CNF	  |
	 |		  *<==============================*
(8)	 |	PKT_MODIFY_CNF	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_MODIFY_REQ
		device_no	DEVICE_47
<A>		dio_dcb	MODIFIED_DCB_FROM_ACI
<B>		dio_dcb	TRASH_DCB_FROM_ACI

- PKT_DIO_FLUSH_REQ
		device_no	DEVICE_47

- PKT_DIO_FLUSH_CNF
		retval	DRV_OK

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_SETCONFIG_REQ
		device_no	DEVICE_47
<A>		dio_dcb	MODIFIED_DCB_FROM_ACI
<B>		dio_dcb	TRASH_DCB_FROM_ACI

- PKT_DIO_SETCONFIG_CNF
<A>		retval	DRV_OK
<B>		retval	DRV_INVALID_PARAMS

- PKT_MODIFY_CNF
		device_no	DEVICE_47
<A>		cause	PKTCS_SUCCESS
<B>		cause	PKTCS_INVALID_PARAMS

History:	15-Oct-02	HM	Initial

### PKTIO051: KER_READY, Modification of driver settings requested

Description:	The driver settings are to be modified. The driver is flushed by calling DIO_Flush(), the function returns with DRV_INPROCESS as flushing needs time. The state KER_MODIFY / TX_FLUSHING is entered for cases <A> and <B>, for cases <C> and <D> KER_MODIFY / TX_FLUSH_BUFFER is entered.
<A> Parameters from ACI are good, initial TX_READY
<B> Parameters from ACI are trash, driver will refuse, initial TX_READY
<C> Parameters from ACI are good, initial TX_BUFFER
<D> Parameters from ACI are trash, driver will refuse, initial TX_BUFFER

Variants:	<A>....<D>

Preamble:	<A>	PKTIO200A
		<B>	PKTIO200A
		<C>	PKTIO201A
		<D>	PKTIO201A

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_MODIFY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	PKT_DIO_FLUSH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_FLUSH_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_MODIFY_REQ
		device_no	DEVICE_47
<A>		dio_dcb	MODIFIED_DCB_FROM_ACI
<B>		dio_dcb	TRASH_DCB_FROM_ACI
<C>		dio_dcb	MODIFIED_DCB_FROM_ACI
<D>		dio_dcb	TRASH_DCB_FROM_ACI

- PKT_DIO_FLUSH_REQ
		device_no	DEVICE_47

- PKT_DIO_FLUSH_CNF
		retval	DRV_INPROCESS

History:	15-Oct-02	HM	Initial

### PKTIO052: KER_MODIFY, end of flushing in signalled

Description:	In state KER_MODIFY / TX_FLUSHING the driver signals end of flushing activity by sending DRV_SIGTYPE_FLUSH. The requested modification is performed. The next state is regardless of the result of the modification KER_READY / TX_READY.
<A> Parameters from ACI were good, TX_FLUSHING
<B> Parameters from ACI were trash, driver will refuse, TX_FLUSHING

Variants:	<A>....<B>

Preamble:	<A>	PKTIO051A
		<B>	PKTIO051B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_WRITE)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *===============================>*
(3)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
MUTE (500)
(4)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_FLUSH)	  |
	 |		  *<==============================*
(5)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	PKT_DIO_SETCONFIG_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	PKT_DIO_SETCONFIG_CNF	  |
	 |		  *<==============================*
(9)	 |	PKT_MODIFY_CNF	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_WRITE

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_FLUSH

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_SETCONFIG_REQ
		device_no	DEVICE_47
<A>		dio_dcb	MODIFIED_DCB_FROM_ACI
<B>		dio_dcb	TRASH_DCB_FROM_ACI

- PKT_DIO_SETCONFIG_CNF
<A>		retval	DRV_OK
<B>		retval	DRV_INVALID_PARAMS

- PKT_MODIFY_CNF
		device_no	DEVICE_47
<A>		cause	PKTCS_SUCCESS
<B>		cause	PKTCS_INVALID_PARAMS

History:	15-Oct-02	HM	Initial

### PKTIO053: KER_MODIFY, DTI2_DISCONNECT_REQ

Description:	In state KER_MODIFY / TX_FLUSHING the DTI connection is closed. This is indicated to ACI and the main state is changed to KER_MODIFY_NO_DTI, the DRX state changes from DRX_READY to DRX_DEAD_READY and the DTX state changes from DTX_READY to DTX_DEAD.
Note: The state transitions are not tested here. Only the emission of primives is tested in this testcase.
<A> Parameters from ACI are good
<B> Parameters from ACI are trash, driver will refuse

Variants:	<A>....<B>

Preamble:	<A>	PKTIO051A
		<B>	PKTIO051B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_DISCONNECT_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	PKT_DTI_CLOSE_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_DISCONNECT_REQ
		link_id	LINK_ID_47
		cause	DTI_CAUSE_NORMAL_CLOSE

- PKT_DTI_CLOSE_IND
		device_no	DEVICE_47

History:	15-Oct-02	HM	Initial

### PKTIO054: KER_MODIFY_NO_DTI, DRV_SIGTYPE_FLUSH signalled

Description:	In state KER_MODIFY_NO_DTI / TX_FLUSHING the driver signals the end of flushing. The driver's configuration is changed, ACI is informed about the result of the modification and the new main state KER_NO_DTI is entered.
Note: The state transitions are not tested here. Only the emission of primives is tested in this testcase.
<A> Parameters from ACI are good
<B> Parameters from ACI are trash, driver will refuse

Variants:	<A>....<B>

Preamble:	<A>	PKTIO053A
		<B>	PKTIO053B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_WRITE)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
MUTE (500)
(4)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_FLUSH)	  |
	 |		  *<==============================*
(5)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	PKT_DIO_SETCONFIG_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	PKT_DIO_SETCONFIG_CNF	  |
	 |		  *<==============================*
(9)	 |	PKT_MODIFY_CNF	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_WRITE

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_FLUSH

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_SETCONFIG_REQ
		device_no	DEVICE_47
<A>		dio_dcb	MODIFIED_DCB_FROM_ACI
<B>		dio_dcb	TRASH_DCB_FROM_ACI

- PKT_DIO_SETCONFIG_CNF
<A>		retval	DRV_OK
<B>		retval	DRV_INVALID_PARAMS

- PKT_MODIFY_CNF
		device_no	DEVICE_47
<A>		cause	PKTCS_SUCCESS
<B>		cause	PKTCS_INVALID_PARAMS

History:	15-Oct-02	HM	Initial

### PKTIO055: KER_MODIFY, DRV_SIGTYPE_DISCONNECT signalled

Description:	In state KER_MODIFY the driver sends a DRV_SIGTYPE_DISCONNECT. All PKTIO state machines are expected to be in the NULL state after the state transitions are finished.
<A> The driver disconnect is received in state TX_FLUSHING
<B> The driver disconnect is received in state TX_FLUSH_BUFFER

Variants:	<A>....<B>

Preamble:	<A>	PKTIO051A
		<B>	PKTIO051C

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	DTI2_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |	PKT_DTI_CLOSE_IND	  |		  |
	 *<==============================*		  |
(4)	 |	PKT_MODIFY_CNF	  |		  |
	 *<==============================*		  |
(5)	 |		  |	PKT_DIO_CLOSEDEVICE_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	PKT_DIO_CLOSEDEVICE_CNF	  |
	 |		  *<==============================*
(7)	 |	PKT_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_DISCONNECT

- DTI2_DISCONNECT_IND
		link_id	LINK_ID_47
		cause	DTI_CAUSE_NORMAL_CLOSE

- PKT_DTI_CLOSE_IND
		device_no	DEVICE_47

- PKT_MODIFY_CNF
		device_no	DEVICE_47
		cause	PKTCS_DISCONNECT

- PKT_DIO_CLOSEDEVICE_REQ
		device_no	DEVICE_47

- PKT_DIO_CLOSEDEVICE_CNF
		retval	DRV_OK

- PKT_DISCONNECT_IND
		device_no	DEVICE_47
		cause	PKTCS_DISCONNECT

History:	15-Oct-02	HM	Initial
	06-Nov-02	HK	DTI close before driver close.

### PKTIO056: KER_MODIFY_NO_DTI and DRV_SIGTYPE_DISCONNECT

Description:	In state KER_MODIFY_NO_DTI the driver sends a DRV_SIGTYPE_DISCONNECT. All PKTIO state machines are expected to be in the NULL state after the state transitions are finished.

Preamble:	PKTIO053A

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	PKT_MODIFY_CNF	  |		  |
	 *<==============================*		  |
(3)	 |		  |	PKT_DIO_CLOSEDEVICE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	PKT_DIO_CLOSEDEVICE_CNF	  |
	 |		  *<==============================*
(5)	 |	PKT_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_DISCONNECT

- PKT_MODIFY_CNF
		device_no	DEVICE_47
		cause	PKTCS_DISCONNECT

- PKT_DIO_CLOSEDEVICE_REQ
		device_no	DEVICE_47

- PKT_DIO_CLOSEDEVICE_CNF
		retval	DRV_OK

- PKT_DISCONNECT_IND
		device_no	DEVICE_47
		cause	PKTCS_DISCONNECT

History:	15-Oct-02	HM	Initial

### PKTIO057: KER_MODIFY / TX_FLUSH_BUFFER, end of flushing

Description:	In state KER_MODIFY / TX_FLUSH_BUFFER the driver signals end of flushing activity by sending DRV_SIGTYPE_FLUSH. As there is still one descriptor stored in the PKTIO entity which has not been send, this is now submmitted to the DIO driver and flushing is performed again for this descriptor. Before end of flush in is assumed the driver indicates the successfully written packet by sending DRV_SIGTYPE_WRITE. This shall have no influence on the behaviour of PKTIO.
<A> Parameters from ACI were good, initial TX_FLUSH_BUFFER
<B> Parameters from ACI are trash, driver will refuse, initial TX_FLUSH_BUFFER

Variants:	<A>....<B>

Preamble:	<A>	PKTIO051C
		<B>	PKTIO051D

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_WRITE)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
MUTE (500)
(4)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_FLUSH)	  |
	 |		  *<==============================*
(5)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	PKT_DIO_WRITE_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	PKT_DIO_WRITE_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	PKT_DIO_FLUSH_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	PKT_DIO_FLUSH_CNF	  |
	 |		  *<==============================*
MUTE (1000)
(11)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_WRITE)	  |
	 |		  *<==============================*
(12)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_WRITE

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_FLUSH

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_WRITE_REQ
		device_no	DEVICE_47
		state	DIO_STATE_IP
		mask	DIO_MASK_NONE
		sdu	A_DL_SDU

- PKT_DIO_WRITE_CNF
		retval	DRV_OK

- PKT_DIO_FLUSH_REQ
		device_no	DEVICE_47

- PKT_DIO_FLUSH_CNF
		retval	DRV_INPROCESS

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_WRITE

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

History:	15-Oct-02	HM	Initial

### PKTIO058: KER_MODIFY / TX_FLUSHING, end of flushing

Description:	In state KER_MODIFY / TX_FLUSHING the driver signals end of flushing activity by sending DRV_SIGTYPE_FLUSH.
<A> Parameters from ACI were good, initial TX_FLUSHING
<B> Parameters from ACI are trash, driver will refuse, initial TX_FLUSHING

Variants:	<A>....<B>

Preamble:	<A>	PKTIO057A
		<B>	PKTIO057B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_WRITE)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*

(4)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_FLUSH)	  |
	 |		  *<==============================*
(5)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	PKT_DIO_SETCONFIG_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	PKT_DIO_SETCONFIG_CNF	  |
	 |		  *<==============================*
(9)	 |	PKT_MODIFY_CNF	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_WRITE

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_FLUSH

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_SETCONFIG_REQ
		device_no	DEVICE_47
<A>		dio_dcb	MODIFIED_DCB_FROM_ACI
<B>		dio_dcb	TRASH_DCB_FROM_ACI

- PKT_DIO_SETCONFIG_CNF
<A>		retval	DRV_OK
<B>		retval	DRV_INVALID_PARAMS

- PKT_MODIFY_CNF
		device_no	DEVICE_47
<A>		cause	PKTCS_SUCCESS
<B>		cause	PKTCS_INVALID_PARAMS

History:	15-Oct-02	HM	Initial

### PKTIO059: TX_FLUSHING or TX_FLUSH_BUFFER, packet written

Description:	In state KER_MODIFY / TX_FLUSHING or TX_FLUSH_BUFFER the driver signals that a packet has been written. This shall not cause a state change of any states of the PKTIO entity, but the entity gets back the written descriptors at the soonest opportunity which is possible.
<A> TX_FLUSHING => TX_FLUSHING
<B> TX_FLUSH_BUFFER => TX_FLUSH_BUFFER

Variants:	<A>....<B>

Preamble:	<A>	PKTIO051A
		<B>	PKTIO051C

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_WRITE)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_WRITE

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

History:	15-Oct-02	HM	Initial

### PKTIO060: KER_DTI_OPENING, PKT_MODIFY_REQ immediate

Description:	In state KER_DTI_OPENING the PKTIO entity receives a PKT_MODIFY_REQ. The modification can be immediately done as DIO_Flush returns with DRV_OK.
Note: This is almost the same as PKTIO050, but the state is different.
<A> Parameters from ACI are good
<B> Parameters from ACI are trash, driver will refuse

Variants:	<A>....<B>

Preamble:	PKTIO020A

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_MODIFY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	PKT_DIO_FLUSH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_FLUSH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	PKT_DIO_SETCONFIG_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	PKT_DIO_SETCONFIG_CNF	  |
	 |		  *<==============================*
(8)	 |	PKT_MODIFY_CNF	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_MODIFY_REQ
		device_no	DEVICE_47
<A>		dio_dcb	MODIFIED_DCB_FROM_ACI
<B>		dio_dcb	TRASH_DCB_FROM_ACI

- PKT_DIO_FLUSH_REQ
		device_no	DEVICE_47

- PKT_DIO_FLUSH_CNF
		retval	DRV_OK

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_SETCONFIG_REQ
		device_no	DEVICE_47
<A>		dio_dcb	MODIFIED_DCB_FROM_ACI
<B>		dio_dcb	TRASH_DCB_FROM_ACI

- PKT_DIO_SETCONFIG_CNF
<A>		retval	DRV_OK
<B>		retval	DRV_INVALID_PARAMS

- PKT_MODIFY_CNF
		device_no	DEVICE_47
<A>		cause	PKTCS_SUCCESS
<B>		cause	PKTCS_INVALID_PARAMS

History:	15-Oct-02	HM	Initial

### PKTIO061: KER_DTI_OPENING, PKT_MODIFY_REQ after flush

Description:	In state KER_DTI_OPENING the driver settings are to be modified. The driver is flushed by calling DIO_Flush(), the function returns with DRV_INPROCESS as flushing needs time. The state KER_MODIFY_DTI_OPENING is entered.
Note: As no data packets were sent by DTI, in real life the driver never will return with DRV_INPROCESS, but always with DRV_OK. On the other hand, this should not do any harm for the test purpose.
Note: This is almost the same as PKTIO051, but the state is different.
<A> Parameters from ACI are good
<B> Parameters from ACI are trash, driver will refuse

Variants:	<A>....<B>

Preamble:	PKTIO020A

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_MODIFY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	PKT_DIO_FLUSH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_FLUSH_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_MODIFY_REQ
		device_no	DEVICE_47
<A>		dio_dcb	MODIFIED_DCB_FROM_ACI
<B>		dio_dcb	TRASH_DCB_FROM_ACI

- PKT_DIO_FLUSH_REQ
		device_no	DEVICE_47

- PKT_DIO_FLUSH_CNF
		retval	DRV_INPROCESS

History:	15-Oct-02	HM	Initial

### PKTIO062: KER_MODIFY_DTI_OPENING, DRV_SIGTYPE_FLUSH

Description:	In state KER_MODIFY_DTI_OPENING / TX_FLUSHING the driver signals end of flushing activity by sending DRV_SIGTYPE_FLUSH. The requested modification is performed. The next state is regardless of the result of the modification KER_DTI_OPENING / TX_READY.
<A> Parameters from ACI are good
<B> Parameters from ACI are trash, driver will refuse

Variants:	<A>....<B>

Preamble:	<A>	PKTIO061A
		<B>	PKTIO061B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_FLUSH)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	PKT_DIO_SETCONFIG_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	PKT_DIO_SETCONFIG_CNF	  |
	 |		  *<==============================*
(6)	 |	PKT_MODIFY_CNF	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_FLUSH

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_SETCONFIG_REQ
		device_no	DEVICE_47
<A>		dio_dcb	MODIFIED_DCB_FROM_ACI
<B>		dio_dcb	TRASH_DCB_FROM_ACI

- PKT_DIO_SETCONFIG_CNF
<A>		retval	DRV_OK
<B>		retval	DRV_INVALID_PARAMS

- PKT_MODIFY_CNF
		device_no	DEVICE_47
<A>		cause	PKTCS_SUCCESS
<B>		cause	PKTCS_INVALID_PARAMS

History:	15-Oct-02	HM	Initial

### PKTIO063: KER_MODIFY_DTI_OPENING, DTI2_DISCONNECT_REQ

Description:	In state KER_MODIFY_DTI_OPENING / TX_FLUSHING the opening of the DTI connection is closed. This is indicated to ACI and the main state is changed to KER_MODIFY_NO_DTI.
Note: This is almost the same as PKTIO053, but the start state is different.
<A> Parameters from ACI are good
<B> Parameters from ACI are trash, driver will refuse

Variants:	<A>....<B>

Preamble:	<A>	PKTIO061A
		<B>	PKTIO061B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_DISCONNECT_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	PKT_DTI_CLOSE_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_DISCONNECT_REQ
		link_id	LINK_ID_47
		cause	DTI_CAUSE_NORMAL_CLOSE

- PKT_DTI_CLOSE_IND
		device_no	DEVICE_47

History:	15-Oct-02	HM	Initial

### PKTIO064: KER_MODIFY_DTI_OPENING, PKT_DTI_OPEN_CNF

Description:	In state KER_MODIFY_DTI_OPENING / TX_FLUSHING the opening of the DTI connection is confirmed. Also the first flow control primitive from the DTI partner is received. The next main state is KER_MODIFY.
<A> Parameters from ACI are good
<B> Parameters from ACI are trash, driver will refuse

Variants:	<A>....<B>

Preamble:	<A>	PKTIO061A
		<B>	PKTIO061B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_CONNECT_RES	  |		  |
	 *==============================>*		  |
(2)	 |	DTI2_READY_IND	  |		  |
	 *<==============================*		  |
(3)	 |	PKT_DTI_OPEN_CNF	  |		  |
	 *<==============================*		  |
MUTE (1000)
(4)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_CONNECT_RES
		link_id	LINK_ID_47
		version	DTI_VERSION_10

- DTI2_READY_IND
		link_id	LINK_ID_47

- PKT_DTI_OPEN_CNF
		device_no	DEVICE_47
		cause	PKTCS_SUCCESS

- DTI2_GETDATA_REQ
		link_id	LINK_ID_47

History:	15-Oct-02	HM	Initial

### PKTIO065: KER_MODIFY_DTI_OPENING, DRV_SIGTYPE_DISCONNECT

Description:	In state KER_MODIFY_DTI_OPENING the driver signals a DRV_SIGTYPE_DISCONNECT. All PKTIO state machines are expected to be in the NULL state after the state transitions are finished.

Preamble:	PKTIO061A

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	DTI2_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |	PKT_DTI_CLOSE_IND	  |		  |
	 *<==============================*		  |
(4)	 |	PKT_MODIFY_CNF	  |		  |
	 *<==============================*		  |
(5)	 |		  |	PKT_DIO_CLOSEDEVICE_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	PKT_DIO_CLOSEDEVICE_CNF	  |
	 |		  *<==============================*
(7)	 |	PKT_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_DISCONNECT

- DTI2_DISCONNECT_IND
		link_id	LINK_ID_47
		cause	DTI_CAUSE_NORMAL_CLOSE

- PKT_DTI_CLOSE_IND
		device_no	DEVICE_47

- PKT_MODIFY_CNF
		device_no	DEVICE_47
		cause	PKTCS_DISCONNECT

- PKT_DIO_CLOSEDEVICE_REQ
		device_no	DEVICE_47

- PKT_DIO_CLOSEDEVICE_CNF
		retval	DRV_OK

- PKT_DISCONNECT_IND
		device_no	DEVICE_47
		cause	PKTCS_DISCONNECT

History:	15-Oct-02	HM	Initial

### PKTIO070: KER_NO_DTI, PKT_MODIFY_REQ immediate

Description:	In state KER_NO_DTI the PKTIO entity receives a PKT_MODIFY_REQ. The modification can be immediately done as DIO_Flush returns with DRV_OK.
Note: This is almost the same as PKTIO050 / PKTIO060, but the state is different.
<A> Parameters from ACI are good
<B> Parameters from ACI are trash, driver will refuse

Variants:	<A>....<B>

Preamble:	PKTIO011A

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_MODIFY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	PKT_DIO_FLUSH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_FLUSH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	PKT_DIO_SETCONFIG_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	PKT_DIO_SETCONFIG_CNF	  |
	 |		  *<==============================*
(8)	 |	PKT_MODIFY_CNF	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_MODIFY_REQ
		device_no	DEVICE_47
<A>		dio_dcb	MODIFIED_DCB_FROM_ACI
<B>		dio_dcb	TRASH_DCB_FROM_ACI

- PKT_DIO_FLUSH_REQ
		device_no	DEVICE_47

- PKT_DIO_FLUSH_CNF
		retval	DRV_OK

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_SETCONFIG_REQ
		device_no	DEVICE_47
<A>		dio_dcb	MODIFIED_DCB_FROM_ACI
<B>		dio_dcb	TRASH_DCB_FROM_ACI

- PKT_DIO_SETCONFIG_CNF
<A>		retval	DRV_OK
<B>		retval	DRV_INVALID_PARAMS

- PKT_MODIFY_CNF
		device_no	DEVICE_47
<A>		cause	PKTCS_SUCCESS
<B>		cause	PKTCS_INVALID_PARAMS

History:	15-Oct-02	HM	Initial

### PKTIO071: KER_DTI_OPENING, PKT_MODIFY_REQ after flush

Description:	In state KER_DTI_OPENING the driver settings are to be modified. The driver is flushed by calling DIO_Flush(), the function returns with DRV_INPROCESS as flushing needs time. The state KER_MODIFY_NO_DTI is entered.
Note: As no data packets were sent by DTI, in real life the driver never will return with DRV_INPROCESS, but always with DRV_OK. On the other hand, this should not do any harm for the test purpose.
Note: This is almost the same as PKTIO051 / PKTIO061, but the state is different.
<A> Parameters from ACI are good
<B> Parameters from ACI are trash, driver will refuse

Variants:	<A>....<B>

Preamble:	PKTIO011A

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_MODIFY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	PKT_DIO_FLUSH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_FLUSH_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_MODIFY_REQ
		device_no	DEVICE_47
<A>		dio_dcb	MODIFIED_DCB_FROM_ACI
<B>		dio_dcb	TRASH_DCB_FROM_ACI

- PKT_DIO_FLUSH_REQ
		device_no	DEVICE_47

- PKT_DIO_FLUSH_CNF
		retval	DRV_INPROCESS

History:	15-Oct-02	HM	Initial

## Connected phase - focus on RX / DTX

### PKTIO100: KER_READY / DTX_READY, UL packet received

Description:	In state KER_READY / DTX_READY the driver indicates the reception of a packet. As DTI is ready, the packet is fetched from the driver and transferred to the DTI peer. A new read buffer is provided to the driver.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO022A
		<B>	PKTIO022B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_READ)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETDATA_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETDATA_CNF	  |
	 |		  *<==============================*
(4)	 |	DTI2_DATA_TEST_IND	  |		  |
	 *<==============================*		  |
(5)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_READ

- PKT_DIO_GETDATA_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_GETDATA_CNF
		retval	DRV_OK
		state	DIO_SA
		sdu	A_UL_SDU

- DTI2_DATA_TEST_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_UL_SDU

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_OK

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_BUFFER_FULL

History:	15-Oct-02	HM	Initial

### PKTIO101: Flow control primitive from DTI after DL data tranfer

Description:	After having transferred a DTI packet, a flow control primitive is received. State becomes READY again.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO100A
		<B>	PKTIO100B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

History:	15-Oct-02	HM	Initial

### PKTIO102: No flow control primitive from DTI after DL data tranfer

Description:	After having transferred a DTI packet, no flow control primitive is received by the PKTIO entity. The entity receives a second DL data packet. Afterwards there is one filled descriptor stored by the PKTIO entity and one empty descriptor owned by the DIO driver.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO100A
		<B>	PKTIO100B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_READ)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETDATA_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETDATA_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_READ

- PKT_DIO_GETDATA_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_GETDATA_CNF
		retval	DRV_OK
		state	DIO_SA
		sdu	A_FIRST_UL_SDU

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_OK

History:	15-Oct-02	HM	Initial
	28-Oct-02	HM	Revised

### PKTIO103: No flow control primitive from DTI after DL data tranfer

Description:	The initial state here is that DTX is not READY, the DIO driver has one read buffer and the PKTIO entity has one read buffer filled with data. While DTI has not become READY in the meantime a packet is received. It is checked that the PKTIO entity doesn't offer a read buffer to the DIO driver to ensure the number of parallel occupied descriptors in the system PKTIO / DIO does not exceed two. The final state is that the DTX process is not READY, there is no read buffer present in the driver and there are two read buffers filled with data present in the PKTIO entity.
<A> Device_47
<B> Device_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO102A
		<B>	PKTIO102B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_READ)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETDATA_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETDATA_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_READ

- PKT_DIO_GETDATA_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_GETDATA_CNF
		retval	DRV_OK
		state	DIO_SA
		sdu	A_SECOND_UL_SDU

History:	15-Oct-02	HM	Initial
	28-Oct-02	HM	Revised

### PKTIO104: DTI becomes ready while one descriptor present in PKTIO

Description:	The initial state here is that DTX is not READY. The PKTIO entity has no free read buffer and the DIO driver has no filled read buffer. In case <A>, the PKTIO entity has one filled read buffer and the DIO driver has one free read buffer, in case <B> the PKTIO entity has two filled read buffers and the DIO driver has no read buffer at all. The expected behaviour is that the PKTIO entity sends the filled read data to the DTI peer and offeres exactly a new empy read buffer to the DIO driver.
<A> Initial: PKTIO 1 filled read buffer, DIO 1 free read buffer
<A> Final: PKTIO 1 free read buffer, DIO 1 free read buffer
<B> Initial: PKTIO 2 filled read buffers
<B> Final: PKTIO 1 filled read buffer, DIO 1 free read buffer
<C>,,,<D> same as before, but with DEVICE_66

Variants:	<A>....<D>

Preamble:	<A>	PKTIO102A
		<B>	PKTIO103A
		<C>	PKTIO102B
		<D>	PKTIO103B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	DTI2_DATA_TEST_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_47
<C>		link_id	LINK_ID_66
<D>		link_id	LINK_ID_66

- DTI2_DATA_TEST_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_47
<C>		link_id	LINK_ID_66
<D>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_FIRST_UL_SDU

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_47
<C>		device_no	DEVICE_66
<D>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_OK

History:	15-Oct-02	HM	Initial
	28-Oct-02	HM	Revised

### PKTIO106: Reception of flow control primitive, no filled read buffer present

Description:	The PKTIO entity receives a DTI flow control primitive. Neither DIO nor PKTIO have a filled read buffer to provide immediately, so nothing happens.
Note: This is almost the same as PKTIO101, so these testcases could be merged to a variant testcase using different preambles.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO104A
		<B>	PKTIO104C

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

History:	15-Oct-02	HM	Initial

### PKTIO107: Reception of flow control primitive, filled read buffer present

Description:	The PKTIO entity receives a DTI flow control primitive. The PKTIO entity has a filled read buffer to provide immediately. This is provided to the DTI peer. The PKTIO entity tries to provide the newly free read buffer to the DIO entity which, as it already has stored one free read buffer, does not accept it.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO104B
		<B>	PKTIO104D

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	DTI2_DATA_TEST_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *===============================>*
(4)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<===============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

- DTI2_DATA_TEST_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_SECOND_UL_SDU

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_BUFFER_FULL

History:	15-Oct-02	HM	Initial
	28-Oct-02	HM	Revised

### PKTIO108: RX_BUFFER, DRV_SIGTYPE_DISCONNECT from DIO

Description:	The PKTIO entity has one or more filled read buffers from the application and therefore its RX process is in state RX_BUFFER. The driver signals that the device is to be closed.
<A> The entitiy has one filled read buffer while the driver has one free read buffer
<B> The entity has two filled read buffers while the driver has no free read buffer
<C> The entity has no filled read buffer, DTX_NOT_READY after data transfer

Variants:	<A>....<C>

Preamble:	PKTIO102A
		PKTIO103A
		PKTIO100A

(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	DTI2_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |	PKT_DTI_CLOSE_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	PKT_DIO_CLOSEDEVICE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	PKT_DIO_CLOSEDEVICE_CNF	  |
	 |		  *<==============================*
(4)	 |	PKT_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_DISCONNECT

- DTI2_DISCONNECT_IND
		link_id	LINK_ID_47
		cause	DTI_CAUSE_NORMAL_CLOSE

- PKT_DTI_CLOSE_IND
		device_no	DEVICE_47

- PKT_DIO_CLOSEDEVICE_REQ
		device_no	DEVICE_47

- PKT_DIO_CLOSEDEVICE_CNF
		retval	DRV_OK

- PKT_DISCONNECT_IND
		device_no	DEVICE_47
		cause	PKTCS_DISCONNECT

History:	15-Oct-02	HM	Initial
	29-Oct-02	HM	Revised

### PKTIO109: KER_READY, RX_READY, DTX_READY, packet received

Description:	We have a DIO driver which supports 2 (more than 2?) read buffers. The driver signals that a packet has been received. It is tested that exactly one new empty read buffer is handed over to the driver. The expected final state is KER_READY, RX_READY, DTX_NOT_READY.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO027A
		<B>	PKTIO027B

(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_READ)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETDATA_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETDATA_CNF	  |
	 |		  *<==============================*
(4)	 |	DTI2_DATA_TEST_IND	  |		  |
	 *<==============================*		  |
(5)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_READ

- PKT_DIO_GETDATA_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_GETDATA_CNF
		retval	DRV_OK
		state	DIO_SA
		sdu	A_UL_SDU

- DTI2_DATA_TEST_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_UL_SDU

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_OK

History:	15-Oct-02	HM	Initial

### PKTIO110: KER_READY, RX_READY, DTX_NOT_READY, flow control

Description:	We have a DIO driver which supports 2 read buffers. The PKTIO entity receives a flow control primitive from the peer. It is expected that no visible reaction to the outside is seen. The expected end state is KER_READY, RX_READY, DTX_READY.
Note: See comments for PKTIO101, PKTIO106. Could be unified.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO109A
		<B>	PKTIO109B

(1)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

History:	15-Oct-02	HM	Initial

### PKTIO111: KER_READY, RX_READY, DTX_NOT_READY, packets received

Description:	We have a DIO driver which supports 2 read buffers. The driver signals that a packet has been received. The DTI peer is not ready. The PKTIO entity fetches the read buffer and stores it, awaiting a DTI primitive from the peer. The foreseen final state is KER_READY, RX_BUFFER, DTX_NOT_READY, the PKTIO entity owning two filled read buffers and the DIO driver owning no empty read buffer.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO109A
		<B>	PKTIO109B

(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_READ)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETDATA_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETDATA_CNF	  |
	 |		  *<==============================*
MUTE (1000)
(4)	 |		  |	PKT_DIO_SIGNAL_IND	  |	 |		  |	(DRV_SIGTYPE_READ)	  |
	 |		  *<==============================*
(5)	 |		  |	PKT_DIO_GETDATA_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	PKT_DIO_GETDATA_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_READ

- PKT_DIO_GETDATA_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_GETDATA_CNF
		retval	DRV_OK
		state	DIO_SA
		sdu	A_FIRST_UL_SDU

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_READ

- PKT_DIO_GETDATA_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_GETDATA_CNF
		retval	DRV_OK
		state	DIO_SA
		sdu	A_SECOND_UL_SDU

History:	15-Oct-02	HM	Initial

### PKTIO112: KER_READY, RX_BUFFER, DTX_NOT_READY, flow control

Description:	We have a DIO driver which supports 2 read buffers. The PKTIO entity possesses two filled read buffers, the DIO driver possesses no read buffer. The DTI peer signals flow control, a stored packed is sent and the DIO driver provided with an empty read buffer. The intermediate state is KER_READY,. RX_BUFFER, DTX_NOT_READY, still one filled read buffer stored in the PKTIO entity and one empty read buffer stored in the DIO driver. Subsequently we get the second flow control primitive, PKTIO sends the second stored read data and provides a second empty read buffer to the DIO driver. The foreseen second intermediate state state is KER_READY, RX_READY, DTX_NOT_READY, the PKTIO entity possessing no read buffer and the DIO driver possessing two empty read buffers. After receiving a flow control primitive from the DTI peer, the final DTX state becomes DTX_READY without any observable reaction from the test machine.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO111A
		<B>	PKTIO111B

(1)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	DTI2_DATA_TEST_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
(5)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
(6)	 |	DTI2_DATA_TEST_IND	  |		  |
	 *<==============================*		  |
(7)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
(9)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

- DTI2_DATA_TEST_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_FIRST_UL_SDU

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_OK

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

- DTI2_DATA_TEST_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_SECOND_UL_SDU

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_OK

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

History:	15-Oct-02	HM	Initial

### PKTIO113: KER_READY, RX_BUFFER, DTX_NOT_READY, flow control

Description:	This is almost the same as PKTIO112, with the difference that here after the DIO_Read() calls packets are received by the driver. We want to check if order of received packets is maintained.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO111A
		<B>	PKTIO111B

(1)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	DTI2_DATA_TEST_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
(5)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_READ)	  |
	 |		  *<==============================*
(6)	 |		  |	PKT_DIO_GETDATA_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	PKT_DIO_GETDATA_CNF	  |
	 |		  *<==============================*
MUTE (1000)
(8)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
(9)	 |	DTI2_DATA_TEST_IND	  |		  |
	 *<==============================*		  |
(10)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
(12)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
(13)	 |	DTI2_DATA_TEST_IND	  |		  |
	 *<==============================*		  |
(14)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
(16)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

- DTI2_DATA_TEST_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_FIRST_UL_SDU

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_OK

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_READ

- PKT_DIO_GETDATA_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_GETDATA_CNF
		retval	DRV_OK
		state	DIO_SA
		sdu	A_THIRD_UL_SDU

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

- DTI2_DATA_TEST_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_SECOND_UL_SDU

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_OK

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

- DTI2_DATA_TEST_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_THIRD_UL_SDU

- PKT_DIO_READ_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_READ_CNF
		retval	DRV_OK

- DTI2_GETDATA_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

History:	15-Oct-02	HM	Initial

## Connected phase - focus on RX / DTX, multi instance tests

## Connected phase - focus on TX / DRX

### PKTIO200: DRX_READY, DL packet received

Description:	In state DRX_READY a packet from the DTI2 peer is received. It can be successfully written to the driver. As there is exactly one place in the PKTIO entity where a further DTI2_DATA_REQ could be buffered, the flow control primitive is sent immediately.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO022A
		<B>	PKTIO022B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_DATA_TEST_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	PKT_DIO_WRITE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_WRITE_CNF	  |
	 |		  *<==============================*
(4)	 |	DTI2_READY_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_DL_SDU

- PKT_DIO_WRITE_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		state	DIO_STATE_IP
		mask	DIO_MASK_NONE
		sdu	A_DL_SDU

- PKT_DIO_WRITE_CNF
		retval	DRV_OK

- DTI2_READY_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

History:	15-Oct-02	HM	Initial

### PKTIO201: DRX_READY, DL packet received, DIO driver has one packet

Description:	In state DRX_READY a packet from the DTI2 peer is received. It can not be successfully written to the driver and therefore has to be buffered in the PKTIO entity.
The TX state is expected to become TX_BUFFER, the the DRX state is expected to become DRX_NOT_READY.
<A> The driver supports exactly one TX buffer, DEVICE_47
<B> The driver supports exactly one TX buffer, DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO200A
		<B>	PKTIO200B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_DATA_TEST_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	PKT_DIO_WRITE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_WRITE_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_DL_SDU

- PKT_DIO_WRITE_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		state	DIO_STATE_IP
		mask	DIO_MASK_NONE
		sdu	A_DL_SDU

- PKT_DIO_WRITE_CNF
		retval	DRV_BUFFER_FULL

History:	15-Oct-02	HM	Initial

### PKTIO202: DRX_READY, TX_READY, packet written by driver

Description:	In state DRX_READY, TX_READY the PKTIO entity receives a signal from the DIO driver indicating that a packet could successfully be written by the driver. The PKTIO entity fetches the written descriptor out of control of the DIO driver. Now flow control for the DTI peer is necessary here as this already has been done, no DL packet is waiting in the PKTIO entity.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO200A
		<B>	PKTIO200B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_WRITE)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_WRITE

- PKT_DIO_GETBUFFER_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

History:	15-Oct-02	HM	Initial

### PKTIO203: DRX_NOT_READY, TX_BUFFER, packet written by driver

Description:	In state DRX_NOT_READY, TX_BUFFER the PKTIO entity receives a signal from the DIO driver indicating that a packet could successfully be written by the driver. As the PKTIO entity has stored a descriptor which is waiting to be sent, this is transferred to the driver. The PKTIO entity indicates by using a flow control primitive that it is ready to receive further data.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO201A
		<B>	PKTIO201B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_WRITE)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	PKT_DIO_WRITE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	PKT_DIO_WRITE_CNF	  |
	 |		  *<==============================*
(6)	 |	DTI2_READY_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_WRITE

- PKT_DIO_GETBUFFER_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_WRITE_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		state	DIO_STATE_IP
		mask	DIO_MASK_NONE
		sdu	A_DL_SDU

- PKT_DIO_WRITE_CNF
		retval	DRV_OK

- DTI2_READY_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

History:	15-Oct-02	HM	Initial

### PKTIO204: DRX_NOT_READY, TX_BUFFER, disconnect by driver

Description:	In state DRX_NOT_READY, TX_BUFFER the PKTIO entity receives a signal from the DIO driver indicating that the device is to be closed. The device is closed, the closing is indicated to the DTI peer, to the DTI manager within ACI and to ACI for the device itself.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO201A
		<B>	PKTIO201B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	DTI2_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |	PKT_DTI_CLOSE_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	PKT_DIO_CLOSEDEVICE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	PKT_DIO_CLOSEDEVICE_CNF	  |
	 |		  *<==============================*
(6)	 |	PKT_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		signal_type	DRV_SIGTYPE_DISCONNECT

- DTI2_DISCONNECT_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		cause	DTI_CAUSE_NORMAL_CLOSE

- PKT_DTI_CLOSE_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_CLOSEDEVICE_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- PKT_DIO_CLOSEDEVICE_CNF
		retval	DRV_OK

- PKT_DISCONNECT_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		cause	PKTCS_DISCONNECT

History:	15-Oct-02	HM	Initial

### PKTIO205: DRX_READY, TX_FLUSHING, DL packet received

Description:	In state DRX_READY, TX_FLUSHING the PKTIO entity receives a DL packet from the DTI peer. As the driver is in the process of flushing the PKTIO entity won't send the packet to the DIO driver but store it internally. The next states are DRX_NOT_READY, TX_FLUSH_BUFFER.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO201A
		<B>	PKTIO201B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_DATA_TEST_REQ	  |		  |
	 *==============================>*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_DL_SDU

History:	15-Oct-02	HM	Initial

### PKTIO206: DRX_NOT_READY, TX_BUFFER, disconnect by peer

Description:	In state DRX_NOT_READY, TX_BUFFER the PKTIO entity receives a DTI2_DISCONNECT_REQ primitive from the DTI peer. The next state is DRX_DEAD_NOT_READY. The driver remains fully functional.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO201A
		<B>	PKTIO201B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_DISCONNECT_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	PKT_DTI_CLOSE_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_DISCONNECT_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		cause	DTI_CAUSE_NORMAL_CLOSE

- PKT_DTI_CLOSE_IND
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

History:	15-Oct-02	HM	Initial

### PKTIO207: DRX_NOT_READY, TX_BUFFER, disconnect req. by ACI

Description:	In state DRX_NOT_READY, TX_BUFFER the PKTIO entity receives a PKT_DTI_DISCONNECT_REQ primitive from ACI.  The next state is DRX_DEAD_NOT_READY. The driver remains fully functional.
<A> DEVICE_47
<B> DEVICE_66

Variants:	<A>....<B>

Preamble:	<A>	PKTIO201A
		<B>	PKTIO201B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_DTI_CLOSE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	DTI2_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(2)	 |	PKT_DTI_CLOSE_CNF	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DTI_CLOSE_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

- DTI2_DISCONNECT_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		cause	DTI_CAUSE_NORMAL_CLOSE

- PKT_DTI_CLOSE_CNF
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66

History:	15-Oct-02	HM	Initial

### PKTIO208: DRX_DEAD_NOT_READY, TX_BUFFER

Description:	In state DRX_DEAD_NOT_READY, TX_BUFFER the PKTIO entity receives a signal that the data could be written. As only the DTI connection is dead, it is expected that the buffered data is handed over to the DIO driver and no flow control primitive to the (non-existant) DTI2 peer is sent. The next state is TX_READY, DRX_DEAD_READY (expected, but not tested).
<A> DTI2 DISCONNECT was by peer
<B> DTI2 DISCONNECT was by ACI

Variants:	<A>....<B>

Preamble:	<A>	PKTIO206A
		<B>	PKTIO207A

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_WRITE)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	PKT_DIO_WRITE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	PKT_DIO_WRITE_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_WRITE

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_WRITE_REQ
		device_no	DEVICE_47
		state	DIO_STATE_IP
		mask	DIO_MASK_NONE
		sdu	A_DL_SDU

- PKT_DIO_WRITE_CNF
		retval	DRV_OK

History:	15-Oct-02	HM	Initial

### PKTIO209: DRX_DEAD_NOT_READY, TX_BUFFER

Description:	In state DRX_DEAD_NOT_READY, TX_BUFFER the DTI2 connection is reopened. As there is still buffered data in the PKTIO entity, it is expected that no flow control primitive is sent to the DTI2 peer. The state is DRX_NOT_READY until the driver indicates that the data could be written, then in the state transition to DRX_READY the flow control primitive is sent.

Preamble:	PKTIO206A

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	PKT_DTI_OPEN_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	DTI2_CONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
(3)	 |	DTI2_CONNECT_RES	  |		  |
	 *==============================>*		  |
(4)	 |	PKT_DTI_OPEN_CNF	  |		  |
	 *<==============================*		  |
MUTE (1000)
(5)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_WRITE)	  |
	 |		  *<==============================*
(6)	 |		  |	PKT_DIO_GETBUFFER_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	PKT_DIO_GETBUFFER_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	PKT_DIO_WRITE_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	PKT_DIO_WRITE_CNF	  |
	 |		  *<==============================*
(10)	 |	DTI2_READY_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DTI_OPEN_REQ
		device_no	DEVICE_47
		peer	PEER_SNDCP
		link_id	LINK_ID_47
		dti_direction	DTI_DIRECTION_NORMAL

- DTI2_CONNECT_IND
		link_id	LINK_ID_47
		version	DTI_VERSION_10

- DTI2_CONNECT_RES
		link_id	LINK_ID_47
		version	DTI_VERSION_10

- PKT_DTI_OPEN_CNF
		device_no	DEVICE_47
		cause	PKTCS_SUCCESS

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_WRITE

- PKT_DIO_GETBUFFER_REQ
		device_no	DEVICE_47

- PKT_DIO_GETBUFFER_CNF
		retval	DRV_OK

- PKT_DIO_WRITE_REQ
		device_no	DEVICE_47
		state	DIO_STATE_IP
		mask	DIO_MASK_NONE
		sdu	A_DL_SDU

- PKT_DIO_WRITE_CNF
		retval	DRV_OK

- DTI2_READY_IND
		link_id	LINK_ID_47

History:	15-Oct-02	HM	Initial

### PKTIO210: DRX_DEAD_READY, TX_READY, driver disconnects

Description:	In state DRX_DEAD_READY, TX_READY the drivers signals disconnection.
Note: Looks more or less like PKTIO108.

Preamble:	PKTIO208A

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_CLOSEDEVICE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_CLOSEDEVICE_CNF	  |
	 |		  *<==============================*
(4)	 |	PKT_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_DISCONNECT

- PKT_DIO_CLOSEDEVICE_REQ
		device_no	DEVICE_47

- PKT_DIO_CLOSEDEVICE_CNF
		retval	DRV_OK

- PKT_DISCONNECT_IND
		device_no	DEVICE_47
		cause	PKTCS_DISCONNECT

History:	15-Oct-02	HM	Initial

### PKTIO211: DRX_READY, DL packet received, DIO driver has one packet

Description:	In state DRX_READY a packet from the DTI2 peer is received. It can be successfully written to the DIO driver as this is supporting more than only one packet. Subsequently a third DTI primitve comes in, as the PKTIO entity limits the number of TX packets in the system to avoid a memory outrun, this is not written to the driver anymore. The TX state is expected to become TX_BUFFER, the the DRX state is expected to become DRX_NOT_READY.
<A> DEVICE_47
<B> DEVICE_66

Preamble:	<A>	PKTIO200A
		<B>	PKTIO200B

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_DATA_TEST_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	PKT_DIO_WRITE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_WRITE_CNF	  |
	 |		  *<==============================*
(4)	 |	DTI2_READY_IND	  |		  |
	 *<==============================*		  |
MUTE (1000)
(4)	 |	DTI2_DATA_TEST_REQ	  |		  |
	 *==============================>*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_SECOND_DL_SDU

- PKT_DIO_WRITE_REQ
<A>		device_no	DEVICE_47
<B>		device_no	DEVICE_66
		state	DIO_STATE_IP
		mask	DIO_MASK_NONE
		sdu	A_SECOND_DL_SDU

- PKT_DIO_WRITE_CNF
		retval	DRV_OK

- DTI2_READY_IND
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66

- DTI2_DATA_TEST_REQ
<A>		link_id	LINK_ID_47
<B>		link_id	LINK_ID_66
		parameters	NOT_USED
		sdu	A_THIRD_DL_SDU

History:	15-Oct-02	HM	Initial

## Config primitives

### PKTIO301: Config Primitive SEND

Description:	PKTIO has opened driver and DTI connection. For test purposes the config prim SEND is sent to PKTIO. Ths parameter indicates the number of octets to be transmitted.

Preamble:		PKTIO022a

ACI		 PKTIO		DIO
	 |		  |		  |
COMMAND (PKT CONFIG SEND 71 64)

(1)	 |		  |	PKT_DIO_WRITE_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	PKT_DIO_WRITE_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_WRITE_REQ
		device_no	DEVICE_47
		state	DIO_STATE_IP
		mask	DIO_MASK_NONE
		sdu	CONFIG_64_SDU

- PKT_DIO_WRITE_CNF
		retval	DRV_OK

- History:	05-Nov-02	HK	Initial

### PKTIO302: Driver returns correct looped back data after config primitive SEND

Description:	After receiving a config primitive SEND the PKTIO entity has written data to the driver.  Now the driver returns the same data. Length and content of the returned packet match the sent packet. The returned packet is passed to the DTI interface.

Preamble:	PKTIO301

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_READ)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETDATA_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETDATA_CNF	  |
	 |		  *<==============================*
(4)	 |	DTI2_DATA_TEST_IND	  |		  |
	 *<==============================*		  |
(5)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_READ

- PKT_DIO_GETDATA_REQ
		device_no	DEVICE_47

- PKT_DIO_GETDATA_CNF
		retval	DRV_OK
		state	DIO_SA
		sdu	CONFIG_64_SDU

- DTI2_DATA_TEST_IND
		link_id	LINK_ID_47
		parameters	NOT_USED
		sdu	CONFIG_64_SDU

- PKT_DIO_READ_REQ
		device_no	DEVICE_47

- PKT_DIO_READ_CNF
		retval	DRV_OK

- PKT_DIO_READ_REQ
		device_no	DEVICE_47

- PKT_DIO_READ_CNF
		retval	DRV_BUFFER_FULL

History:	05-Nov-02	HK	Initial

### PKTIO303: Driver returns looped back data after config primitive SEND, wrong length

Description:	After receiving a config primitive SEND the PKTIO entity has written data to the driver. The length of the returned packet does not match the length of the sent packet. A packet of length 8 and with all octets set to 0 is sent to the DTI interface to signal that the length was wrong.

Preamble:	PKTIO301

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_READ)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETDATA_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETDATA_CNF	  |
	 |		  *<==============================*
(4)	 |	DTI2_DATA_TEST_IND	  |		  |
	 *<==============================*		  |
(5)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_READ

- PKT_DIO_GETDATA_REQ
		device_no	DEVICE_47

- PKT_DIO_GETDATA_CNF
		retval	DRV_OK
		state	DIO_SA
		sdu	A_THIRD_DL_SDU

- DTI2_DATA_TEST_IND
		link_id	LINK_ID_47
		parameters	NOT_USED
		sdu	CONFIG_WRONG_LEN

- PKT_DIO_READ_REQ
		device_no	DEVICE_47

- PKT_DIO_READ_CNF
		retval	DRV_OK

- PKT_DIO_READ_REQ
		device_no	DEVICE_47

- PKT_DIO_READ_CNF
		retval	DRV_BUFFER_FULL

History:	06-Nov-02	HK	Initial

### PKTIO304: Driver returns looped back data after config primitive SEND, length is ok but content is wrong

Description:	After receiving a config primitive SEND the PKTIO entity has written data to the driver. The length of the returned packet does match the length of the sent packet. But the content is not the same. A packet of length 8 and with all octets set to 0xFF is sent to the DTI interface to signal that the length was wrong.

Preamble:	PKTIO301

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_READ)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETDATA_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETDATA_CNF	  |
	 |		  *<==============================*
(4)	 |	DTI2_DATA_TEST_IND	  |		  |
	 *<==============================*		  |
(5)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_READ

- PKT_DIO_GETDATA_REQ
		device_no	DEVICE_47

- PKT_DIO_GETDATA_CNF
		retval	DRV_OK
		state	DIO_SA
		sdu	CONFIG_64_WRONG_CONTENT

- DTI2_DATA_TEST_IND
		link_id	LINK_ID_47
		parameters	NOT_USED
		sdu	CONFIG_WRONG_CONTENT

- PKT_DIO_READ_REQ
		device_no	DEVICE_47

- PKT_DIO_READ_CNF
		retval	DRV_OK

- PKT_DIO_READ_REQ
		device_no	DEVICE_47

- PKT_DIO_READ_CNF
		retval	DRV_BUFFER_FULL

History:	06-Nov-02	HK	Initial

### PKTIO305: Second Config Primitive SEND

Description:	PKTIO has opened driver and DTI connection. For test purposes the config prim SEND has been to sent to PKTIO and the correct packet has been returned from th driver. Now in this case a second config SEND is received.

Preamble:		PKTIO302

ACI		 PKTIO		DIO
	 |		  |		  |
COMMAND (PKT CONFIG SEND 71 32)

(1)	 |		  |	PKT_DIO_WRITE_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	PKT_DIO_WRITE_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_WRITE_REQ
		device_no	DEVICE_47
		state	DIO_STATE_IP
		mask	DIO_MASK_NONE
		sdu	CONFIG_32_SDU

- PKT_DIO_WRITE_CNF
		retval	DRV_OK

- History:	05-Nov-02	HK	Initial

### PKTIO306: Flow control primive from peer received

Description:	In state DTX_NOT_READY immediately after opening PKTIO receives the first flow control primitive. DTX state changes to DTX_READY.

Preamble:	PKTIO305

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |	DTI2_GETDATA_REQ	  |		  |
	 *==============================>*		  |
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DTI2_GETDATA_REQ
		link_id	LINK_ID_47

History:	15-Oct-02	HM	Initial

### PKTIO307: Driver returns second correct looped back data after config primitive SEND

Description:	After receiving a second config primitive SEND the PKTIO entity has written data to the driver.  Now the driver returns the same data. Length and content of the returned packet match the sent packet. The returned packet is passed to the DTI interface.

Preamble:	PKTIO306

ACI		 PKTIO		DIO
	 |		  |		  |
(1)	 |		  |	PKT_DIO_SIGNAL_IND	  |
	 |		  |	(DRV_SIGTYPE_READ)	  |
	 |		  *<==============================*
(2)	 |		  |	PKT_DIO_GETDATA_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	PKT_DIO_GETDATA_CNF	  |
	 |		  *<==============================*
(4)	 |	DTI2_DATA_TEST_IND	  |		  |
	 *<==============================*		  |
(5)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	PKT_DIO_READ_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	PKT_DIO_READ_CNF	  |
	 |		  *<==============================*
MUTE (1000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- PKT_DIO_SIGNAL_IND
		device_no	DEVICE_47
		signal_type	DRV_SIGTYPE_READ

- PKT_DIO_GETDATA_REQ
		device_no	DEVICE_47

- PKT_DIO_GETDATA_CNF
		retval	DRV_OK
		state	DIO_SA
		sdu	CONFIG_32_SDU

- DTI2_DATA_TEST_IND
		link_id	LINK_ID_47
		parameters	NOT_USED
		sdu	CONFIG_32_SDU

- PKT_DIO_READ_REQ
		device_no	DEVICE_47

- PKT_DIO_READ_CNF
		retval	DRV_OK

- PKT_DIO_READ_REQ
		device_no	DEVICE_47

- PKT_DIO_READ_CNF
		retval	DRV_BUFFER_FULL

History:	05-Nov-02	HK	Initial

# Suites

/* Set internal routings */

INT_ROUTING:	PKTIO000;

/* Bring device 47 to initial ready state, flow control */
SUI_PKTIO22A:	INT_ROUTING,
	PKTIO010A,
	PKTIO011A,
	PKTIO020A,
	PKTIO021A,
	PKTIO022A;

/* Bring device 66 to initial ready state, flow control */
SUI_PKTIO22B:	INT_ROUTING,
	PKTIO010B,
	PKTIO011B,
	PKTIO020B,
	PKTIO021B,
	PKTIO022B;
/* Mix it wildly */
SUI_PKTIO22A_22B:	INT_ROUTING,
	PKTIO010A,
	PKTIO011A,
	PKTIO010B,
	PKTIO020A,
	PKTIO011B,
	PKTIO021A,
	PKTIO020B,
	PKTIO021B,
	PKTIO022B,
	PKTIO022A;

/* Reject opening of DTI connection, device 47 */
SUI_PKTIO23A:	INT_ROUTING,
	PKTIO010A,
	PKTIO011A,
	PKTIO020A,
	PKTIO023A;

/* Reject opening of DTI connection, device 47 */
SUI_PKTIO23B:	INT_ROUTING,
	PKTIO010B,
	PKTIO011B,
	PKTIO020B,
	PKTIO023B;

/* Reject opening of DTI connection, mixed */
SUI_PKTIO23A_23B:	INT_ROUTING,
	PKTIO010B,
	PKTIO010A,
	PKTIO011B,
	PKTIO011A,
	PKTIO020B,
	PKTIO020A,
	PKTIO023B,
	PKTIO023A;

/* Mix 22A and 23B */

SUI_PKTIO22A_23B:	INT_ROUTING,
	PKTIO010A,
	PKTIO010B,
	PKTIO011A,
	PKTIO011B,
	PKTIO020A,
	PKTIO020B,
	PKTIO021A,
	PKTIO022A,
	PKTIO023B;

/* Some UL data transfer, device 47 */

SUI_PKTIO_112A:	INT_ROUTING,
	PKTIO010A,
	PKTIO027A,
	PKTIO109A,
	PKTIO111A,
	PKTIO112A;

/* Some UL data transfer, device 66 */

SUI_PKTIO_112B:	INT_ROUTING,
	PKTIO010B,
	PKTIO027B,
	PKTIO109B,
	PKTIO111B,
	PKTIO112B;

/* Some other UL data transfer, device 47 */

SUI_PKTIO_113A:	INT_ROUTING,
	PKTIO010A,
	PKTIO027A,
	PKTIO109A,
	PKTIO111A,
	PKTIO113A;

/* Some other UL data transfer, device 66 */

SUI_PKTIO_113B:	INT_ROUTING,
	PKTIO010B,
	PKTIO027B,
	PKTIO109B,
	PKTIO111B,
	PKTIO113B;

/* Mix device 47 and device 66 */

SUI_PKTIO_112A_112B:	INT_ROUTING,
	PKTIO010A,
	PKTIO010B,
	PKTIO027A,
	PKTIO027B,
	PKTIO109A,
	PKTIO109B,
	PKTIO111A,
	PKTIO111B,
	PKTIO112A,
	PKTIO112B;

/* And again */

SUI_PKTIO_113A_113B:	INT_ROUTING,
	PKTIO010B,
	PKTIO010A,
	PKTIO027B,
	PKTIO027A,
	PKTIO109B,
	PKTIO109A,
	PKTIO111B,
	PKTIO111A,
	PKTIO113B,
	PKTIO113A;

/* Device 47, downlink data */
SUI_PKTIO_207A:	INT_ROUTING,
	PKTIO010A,
	PKTIO011A,
	PKTIO020A,
	PKTIO021A,
	PKTIO022A,
	PKTIO200A,
	PKTIO201A,
	PKTIO207A;

/* Device 66, downlink data */
SUI_PKTIO_207B:	INT_ROUTING,
	PKTIO010B,
	PKTIO011B,
	PKTIO020B,
	PKTIO021B,
	PKTIO022B,
	PKTIO200B,
	PKTIO201B,
	PKTIO207B;

/* Mix 47 and 66 */

SUI_PKTIO_207A_207B:	INT_ROUTING,
	PKTIO010A,
	PKTIO010B,
	PKTIO011A,
	PKTIO011B,
	PKTIO020A,
	PKTIO020B,
	PKTIO021A,
	PKTIO021B,
	PKTIO022A,
	PKTIO022B,
	PKTIO200A,
	PKTIO200B,
	PKTIO201A,
	PKTIO201B,
	PKTIO207A,
	PKTIO207B;

/* Mix downlink and uplink */
SUI_PKTIO_113B_207A:	INT_ROUTING,
	PKTIO010A,
	PKTIO011A,
	PKTIO010B,
	PKTIO020A,
	PKTIO021A,
	PKTIO027B,
	PKTIO022A,
	PKTIO200A,
	PKTIO109B,
	PKTIO201A,
	PKTIO111B,
	PKTIO113B,
	PKTIO207A;

/* Some very long modification for device 47 to play around with */
SUI_PKTIO_058A:	INT_ROUTING,
	PKTIO010A,
	PKTIO011A,
	PKTIO020A,
	PKTIO021A,
	PKTIO022A,
	PKTIO200A,
	PKTIO201A,
	PKTIO051C,
	PKTIO057A,
	PKTIO058A;

/* Mix 058A (device 47) and 207B (device 66) */

SUI_PKTIO_058A_207B:	INT_ROUTING,
	PKTIO010A,
	PKTIO011A,
	PKTIO010B,
	PKTIO011B,
	PKTIO020A,
	PKTIO021A,
	PKTIO020B,
	PKTIO021B,
	PKTIO022A,
	PKTIO022B,
	PKTIO200A,
	PKTIO200B,
	PKTIO201A,
	PKTIO201B,
	PKTIO051C,
	PKTIO057A,
	PKTIO058A,
	PKTIO207B;

SUI_PKTIO_058A_113B:	INT_ROUTING,
	PKTIO010A,
	PKTIO011A,
	PKTIO010B,
	PKTIO020A,
	PKTIO021A,
	PKTIO022A,
	PKTIO027B,
	PKTIO200A,
	PKTIO109B,
	PKTIO201A,
	PKTIO051C,
	PKTIO111B,
	PKTIO057A,
	PKTIO058A,
	PKTIO113B;

/*
History:	30-Oct-02	HM	Initial
*/

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |