LLD CPHS Interface

| Project | G23-GSM Protocol Stack |
| --- | --- |
| Document Type | Technical Documentation |
| Title | LLD CPHS Interface |
| Author | Marc Droste-Franke |
| Creation Date | 24 October, 2002 |
| Last Modified | 25 July, 2005 |
| ID and Version | 8462.730.02.006 |
| Status | Being Processed |

Copyright  2002-2003 Texas Instruments, Inc. All rights reserved.

Texas Instruments Proprietary Information – Strictly Private

- Document Control

ã Copyright Texas Instruments, Inc. 2002-2003

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Texas Instruments reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Texas Instruments. Texas Instruments accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a license agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Texas Instruments.

  - Document History

ID	Author	Date	Status

8462.730.02.002	CLB	29 October, 2002	Creation

Update document

8462.730.02.002	CLB	11 November, 2002	Being Processed

Update document

8462.730.02.002	KGT	11 December, 2002	Being Processed

8462.730.02.002	KGT	21 January, 2003	Being Processed

Update document: CPHS Information, defined service values changed

8462.730.02.003	TLU	15 April, 2003	Being Processed

document updaded

8462.730.02.004	TLU	30 April, 2003	Being Processed

8462.730.02.005	MDF	13 September, 2004	Being Processed

8462.730.02.006	AJS	25 July, 2005	Being Processed

document updated: behaviour of %CPCFU (Call Forwarding Unconditional) changed

  - References, Abbreviations, Terms

[TI 7010.801]	7010.801, References and Vocabulary, Texas Instruments.

Table of Contents

1	Introduction	4

2	Overview	5

2.1	General	5

2.2	Functionality	5

2.3	Architecture	6

3	Interface Description	7

3.1	Basic Principle	7

3.2	CHANGES	7

3.3	General Data Types	7

3.3.1	T_CPHS_RET	7

3.3.2	T_CPHS_LINES	8

3.3.3	T_CPHS_CB	8

3.3.4	T_CPHS_PARAMS	9

3.3.5	T_CPHS_USER_CB	9

3.4	General Functions	9

3.4.1	cphs_check_status	9

3.4.2	cphs_line_makes_sense	9

3.5	START/END/REFRESH CPHS MODULE	10

3.5.1	ACI/CPHS Module functional interface	10

3.5.1.1	cphs_start	10

3.5.1.2	cphs_refresh_data	10

3.5.1.3	cphs_stop	10

3.5.2	AT command	11

3.5.2.1	AT%CPHS	11

3.5.3	ACI/MFW functional interface	11

3.5.3.1	Data types	11

3.5.4	T_ACI_CPHS_INIT	11

3.5.4.1	sAT_PercentCPHS	12

3.5.4.2	qAT_PercentCPHS	12

3.5.5	MSC	13

3.6	NETWORK OPERATOR NAME	15

3.6.1	ACI/CPHS Module functional interface	15

3.6.1.1	cphs_get_opn	15

3.6.2	AT command	15

3.6.2.1	AT%CPOPN	15

3.6.3	ACI/MFW functional interface	16

3.6.3.1	qAT_PercentCPOPN	16

3.6.4	MSC	17

3.7	HOME COUNTRY ROAMING INDICATOR	17

3.7.1	ACI/CPHS Module functional interface	17

3.7.1.1	cphs_roaming_ind	17

3.7.2	AT command	17

3.7.2.1	%CPROAM	17

3.7.3	ACI/MFW functional interface	18

3.7.3.1	rAT_PercentCPROAM	18

3.7.4	MSC	18

3.8	VOICE MESSAGE WAITING	19

3.8.1	ACI/CPHS Module functional interface	19

3.8.1.1.1	cphs_voice_mail_ind	19

3.8.1.2	Voice Message Waiting Indicator Setting/Clearing/Reading	19

3.8.1.2.1	cphs_set_waiting_flag	19

3.8.1.2.2	cphs_get_waiting_flag	19

3.8.2	AT command	20

3.8.2.1	%CPVWI	20

3.8.3	ACI/MFW functional interface	21

3.8.3.1	sAT_PercentCPVWI	21

3.8.3.2	qAT_PercentCPVWI	21

3.8.3.3	rAT_PercentCPVWI	21

3.8.4	MSC	23

3.9	DIVERTED CALL INDICATOR and CALL FORWARDING FLAG	24

3.9.1	ACI/CPHS Module functional interface	24

3.9.1.1.1	cphs_set_cfu_flag	24

3.9.1.1.2	cphs_get_fwd_flag	25

3.9.2	AT command	25

3.9.2.1	%CPCFU	25

3.9.3	ACI/MFW functional interface	26

3.9.3.1	sAT_PercentCPCFU	26

3.9.3.2	qAT_PercentCPCFU	26

3.9.4	MSC	28

3.10	CURRENT LINE INDICATOR and ALS	29

3.10.1	ACI/CPHS Module functional interface	29

3.10.1.1.1	cphs_get_line	29

3.10.2	AT command	29

3.10.2.1	%CPALS	29

3.10.3	ACI/MFW functional interface	30

3.10.3.1	qAT_PercentCPALS	30

3.10.4	MSC	31

3.11	EMERGENCY CALL 999	31

3.12	CPHS INFORMATION and CUSTOMER SPECIFIC PROFILE	31

3.12.1	Data Types	31

3.12.2	ACI/CPHS Module functional interface	32

3.12.2.1.1	cphs_get_cphs_info	32

3.12.2.1.2	cphs_get_csprof	32

3.12.3	AT command	33

3.12.3.1	%CPINF	33

3.12.4	ACI/MFW functional interface	33

3.12.4.1	qAT_PercentCPINF	33

3.12.5	MSC	34

3.13	READ MAILBOX NUMBERS	34

3.13.1	Data Types	34

3.13.2	ACI/CPHS Module functional interface	34

3.13.2.1.1	cphs_read_mb_number	34

3.13.2.1.2	cphs_first_free	35

3.13.3	AT command	35

3.13.3.1	AT%CPMB	35

3.13.4	ACI/MFW functional interface	36

3.13.4.1	qAT_PercentCPMB	36

3.13.5	MSC	37

3.14	WRITE MAILBOX NUMBERS	37

3.14.1	Data Types	37

3.14.2	ACI/CPHS Module functional interface	37

3.14.2.1.1	cphs_write_mb_number	37

3.14.2.1.2	cphs_get_mb_parameter	38

3.14.3	AT command	38

3.14.3.1	AT%CPMBW	38

3.14.4	ACI/MFW functional interface	39

3.14.4.1	sAT_PercentCPMBW	39

3.14.4.2	tAT_PercentCPMBW	39

3.14.5	MSC	40

3.15	INFORMATION NUMBERS	41

3.15.1	Data Types:	41

3.15.2	ACI/CPHS Module functional interface	41

3.15.2.1.1	cphs_read_info_nb	41

3.15.2.1.2	cphs_explore_info_nbs	41

3.15.2.1.3	cphs_info_num_get_max	42

3.15.3	AT command	42

3.15.3.1	%CPNUMS	42

3.15.4	ACI/MFW functional interface	43

3.15.4.1	qAT_PercentCPNUMS	43

3.15.4.2	sAT_PercentCPNUMS	43

3.15.4.3	rAT_PercentCPNUMS	44

3.15.5	MSC	45

# Introduction

- G23 is a software package implementing Layers 2 and 3 of the ETSI-defined GSM air interface signalling protocol, and as such represents that part of a GSM mobile station’s protocol software which is both, platform and manufacturer independent. Therefore, G23 can be viewed as a building block providing standardised functionality through generic interfaces for easy integration.

The G23 suite of products consists of the following items:

- Layers 2 and 3 for speech & short message services,

- Layers 2 and 3 for fax & data services,

- Application Control Interface,

- Slim MMI [02.30] and

- Test and integration support tools.

# Overview

## General

The CPHS functionality is to be implemented as a separate module of ACI. AT-Interpreter and MFW will access its functionalities by means of an AT-command based interface using some newly defined AT-commands. Another interface also should allow internal communication with ACI. The aim of this document is the description of these two interfaces.

The CPHS module provides the whole functionality described in the document CPHS Phase 2 version 4.2 from 27 February 1997 with the exception of the "Network and Service Provider Lock" section. The latter has indeed already been implemented as an extra module in ACI (see doc 8463.600).

The module caches most of the CPHS SIM values. This allows functions to return immediately and avoids the use of a callback mechanism.

## Functionality

The CPHS library provides the following features:

- Network Operator Name

- Home Country Roaming Indicator

- Voice Message Waiting Indicator

- Diverted Call Indicator

- Current Line Indicator

- Alternate Line Service (ALS)

- Emergency Calling (dialing 999)

- Call Forwarding flag

- Voice message waiting flag

- Customer Service Profile (CSP)

- CPHS Information

- Mailbox Numbers

- Information Numbers

## Architecture

# Interface Description

## Basic Principle

The CPHS functionality is implemented in a separated module of ACI. To ensure the independence of this module, three interfaces are here defined.

The first one is a so-called user interface. This interface will be used by any application willing to use CPHS functionality. In the current application, this interface is used by the Command Handler part of ACI. Some operations may be asynchronous (message sent to lower entity such as SIM: stack returns before having the answer). In such cases, the CPHS module will use a callback previously passed by ACI during initialisation phase.

The CPHS module needs also a so-called access interface so that the protocol stack (through ACI) may exchange information with it. This interface may be used by the Command Handler or the Protocol Stack Adapter.

In some cases, the CPHS module will also have to use some ACI features, such as accessing data on the SIM. The CPHS module should as much as possible use the ACI/MFW functional interface in theses cases. However, given the current implementation of this interface, it is only possible with function immediately returning successfully (synchronous case). This is due to the current impossibility to redirect callbacks. Asynchronous ACI features should only be necessary for SIM accessing: for this case corresponding cmhSIMfunctions may be used. However in order to limit ACI dependency, calls of such functions shall be encapsulated in an ACI adapter layer in the CPHS module.

This document also contains the description of an AT-command-based interface for CPHS features. AT commands and corresponding Command Handler functions (for MFW/BMI use) here described give full control over CPHS functionalities for any type of user (ATI or MFW).

## CHANGES

## General Data Types

### T_CPHS_RET

Definition:

typedef enum

{

CPHS_FAIL 			= -1,

CPHS_OK,

CPHS_EXEC,

CPHS_BUSY,

CPHS_NOT_INIT,

} T_CPHS_RET;

Elements:

CPHS_FAIL				unspecific failure

CPHS_OK				successful execution

CPHS_EXEC				execution in progress

CPHS_BUSY				module is already in use… Retry later

CPHS_NO_INIT			module has not been initialised

Description:

These are the different return codes possible when calling a CPHS interface function.

### T_CPHS_LINES

Elements:

CPHS_LINE_NULL		line parameter is irrelevant

CPHS_LINE1			standard voice line

CPHS_LINE_DATA		bearer data line

CPHS_LINE_FAX		bearer fax line

CPHS_LINE2			auxiliary speech line (in case of ALS)

Description:

These are the different lines CPHS functionalities may apply to.

Definition:

#define  CPHS_LINE_NULL 	(0x0000)

#define  CPHS_LINE1	 	(0x0001)

#define  CPHS_LINE_DATA 	(0x0002)

#define  CPHS_LINE_FAX 	(0x0004)

#define  CPHS_LINE2 		(0x0100)

These values have been chosen for consistency with the concept of classes in AT commands definitions such as CCFC or CLCK.

Type definition:

#define 	T_CPHS_LINES 	USHORT

The reason why no enum is being used here is that CPHS lines are to be used in bit fields. So it is more handy to have these #defines in order to handle CPHS lines.

### T_CPHS_CB

Definition:

typedef enum

{

CPHS_INIT_RES = 	1,

CPHS_ROAM_IND,

CPHS_VOICE_MAIL_IND,

CPHS_VOICE_MAIL_RES,

CPHS_CFU_RES

} T_CPHS_CB;

Elements:

CPHS_INIT_RES		Result of initialisation

CPHS_ROAM_IND		Roaming indicator

CPHS_VOICE_MAIL_IND	Incoming voice mail indication

CPHS_VOICE_MAIL_RES	Read/Write voice mail flag result

CPHS_CFU_RES		Read/Write call diverted flag result

Description:

These are the different contexts where the CPHS module may call the ACI callback.

### T_CPHS_PARAMS

typedef struct

{

T_CPHS_CB cb_type;

T_CPHS_RET operation_result;

UBYTE set_flag;

T_CPHS_LINES line;

} T_CPHS_PARAMS;

Elements:

cb_type			Callback type of ACI callback

operation_result		Return value of the CPHS interface functions

set_flag			Auxiliary setflag

line			Different lines CPHS functionalities may apply to

Description:

This is the structure passed by CPHS module when it has to inform the user. Cb_type informs about what 	CPHS functionality is involved and thus how to interpret the set_flag data.

### T_CPHS_USER_CB

typedef void T_CPHS_USER_CB( T_CPHS_PARAMS *params );

Description:

Prototype type of callback to be used by the CPHS module to inform user (ATI/MFW).

## General Functions

### cphs_check_status

Prototype:

T_CPHS_RET cphs_check_status(void);

Parameters:

void

Return:

CPHS_BUSY			CPHS module is busy

CPHS_OK			CPHS module is initialised and ready to process

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

This function checks whether CPHS module has been initialised or if it is currently

being used.

### cphs_line_makes_sense

Prototype:

BOOL cphs_line_makes_sense(T_CPHS_LINES line);

Parameters:

line				Different lines CPHS functionalities may apply to

Return:

TRUE			line is a CPHS line

FALSE			line is not a CPHS line

Description:

This function returns TRUE if the given line is a valid CPHS line..

## START/END/REFRESH CPHS MODULE

### ACI/CPHS Module functional interface

#### cphs_start

Prototype:

T_CPHS_RET cphs_start (T_CPHS_USER_CB *cphs_user_cb);

Parameters:

cphs_user_cb( )		contains callback to be used by the CPHS module to inform user (ATI/MFW).

Return:

CPHS_EXEC			CPHS module is being initialised

CPHS_FAIL			an error occurred (e.g. parameter passed is a NULL pointer)

CPHS_BUSY			CPHS has already been started to initialise

CPHS_OK			CPHS module already initialised

Description:

Initialises the CPHS module (read some parameters from the SIM).

#### cphs_refresh_data

Prototype:

T_CPHS_RET cphs_refresh_data (void);

Parameters:

void

Return:

CPHS_EXEC			CPHS module is being initialised

CPHS_FAIL			an error occurred

CPHS_BUSY			CPHS module is busy

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Refresh the cached CPHS parameter from SIM.

#### cphs_stop

Prototype:

T_CPHS_RET cphs_stop (void);

Parameters:

void

Return:

CPHS_OK			execution OK

CPHS_BUSY			CPHS module is busy

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Ends CPHS module functionalities. cphs_start( ) will be needed before using the CPHS library again.

### AT command

#### AT%CPHS

Table 1: %CPHS parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| %CPHS=<init_mode> | +CME ERROR: <err> |
| %CPHS? | %CPHS: <init_mode> +CME ERROR: <err> |
| %CPHS=? | %CPHS: (list of supported <init_mode>s) +CME ERROR: <err> |

Description

This set command allows initialising and closing CPHS functionalities. Initialising is mandatory if the user aims at using CPHS functionality (e.g. retrieving voice waiting message). When initialising, the CPHS module proceeds to retrieve data from SIM to cache them internally, this allowing direct availability of these data. Initialising means also enabling all CPHS related indications (e.g. Call Diverted Indicator, Call Waiting Message Indicator, etc…).

Closing CPHS functionality will stop CPHS indications to be displayed. It will also free the related memory needed to cache CPHS data.

If setting fails in an ME error, +CME ERROR: <err> is returned. Refer subclause 9.2 for <err> values.

Read command returns the current state of CPHS functionalities: initialised, refreshing (only while refreshing is in progress. When progress is done, initialised will be shown) or closed.

Test command returns supported <init_mode>-values.

Defined values

<init_mode>: integer type value indicating:

0 – Closes CPHS functionalities (default).

1 – Initializes CPHS functionalities.

2 – Refreshes CPHS data cached from SIM.

### ACI/MFW functional interface

#### Data types

### T_ACI_CPHS_INIT

Definition:

typedef enum

{

ACI_CPHS_CLOSE 	= 0,

ACI_CPHS_INIT,

ACI_CPHS_REFRESH,

ACI_CPHS_BUSY

} T_ACI_CPHS_INIT;

Elements:

ACI_CPHS_CLOSE		close CPHS functionalities

ACI_CPHS_INIT		initialised CPHS functionalities

ACI_CPHS_REFRESH		refreshes CPHS data

ACI_CPHS_BUSY		CPHS module is busy

Description:

These are the different values of parameter init_cphs for sAT_PercentCPHS and qAT_PercentCPHS.

#### sAT_PercentCPHS

Prototype:

T_ACI_RETURN sAT_PercentCPHS(T_ACI_CMD_SRC srcId, T_ACI_CPHS_INIT init_cphs);

Parameters:

init_cphs	Initialising/closing/refreshing CPHS module.

Return:

AT_EXCT		CPHS module is initialising

AT_CMPL		operation successful (can occur: e.g. end CPHS)

AT_FAIL		an error occurred

Description:

Starts/ends/refreshes CPHS functionalities.

When initialising, cphs_start( ) will be called with parameter cphs_user_cb = &cmhCPHS_user_cb( ), then void cmhCPHS_user_cb(T_CPHS_PARAMS *params) is to be implemented in the command handler part of ACI (file cmh_cphs.c). The function will then take care of providing the right user with the right data (using the right R_AT callback).

#### qAT_PercentCPHS

Prototype:

T_ACI_RETURN qAT_PercentCPHS(T_ACI_CMD_SRC srcId, T_ACI_CPHS_INIT * init_cphs);

Parameters:

init_cphs		Contains status of CPHS module.

Return:

AT_CMPL		status of CPHS module has been successfully retrieved

AT_FAIL		an error occurred

Description:

Queries the status of CPHS functionalities.

### MSC

[Starts CPHS functionality or refreshes CPHS data from SIM]

AT%CPHS=1 or AT%CPHS=2

OK

ATI/MFW           CMH              CPHS                 PSA/CMH           SIM

|               |                 |                    |                |

|               |                 |                    |                |

(1): CPHS not initialised yet

|  sAT_%CPHS(1) |                 |                    |                |

*-------------->*                 |                    |                |

|               |   cphs_start()  |                    |                |

|               *---------------->*                    |                |

|               |                 |                    |                |

(end of 1)

OR

(2): CPHS already initialised

|  sAT_%CPHS(2) |                 |                    |                |

*-------------->*                 |                    |                |

|               |cphs_refresh_data|                    |                |

|               *---------------->*                    |                |

|               |                 |                    |                |

(end of 2)

(3)

|               |                 | cmhSIM_ReadTranspEF|                |

|               |                 *------------------->*                |

|               |                 |                    |                |

|               |                 |                    |  SIM_READ_REQ  |

|               |                 |                    *===============>*

|               |                 |                    |                |

|               |    CPHS_EXEC    |                    |                |

|               *<--%ret result%--*                    |                |

|               |                 |                    |                |

|    AT_EXCT    |                 |                    |                |

*<-%ret result%-*                 |                    |                |

|               |                 |                    |                |

|               |                 |                    |  SIM_READ_CNF  |

|               |                 |                    *<===============*

|               |                 |                    |                |

|               |                 | cphs_sim_data_cb() |                |

|               |                 *<-------------------*                |

(end of 3) : (3) can be repeated if needed

|               |                 |                    |                |

|               |cmhCPHS_user_cb()|                    |                |

|               | CPHS_INIT_RES   |                    |                |

|               *<----------------*                    |                |

|               |                 |                    |                |

|   rAT_OK( )   |                 |                    |                |

*<--------------*                 |                    |                |

|               |                 |                    |                |

[Ends CPHS functionality]

AT%CPHS=0

OK

ATI/MFW           CMH              CPHS                 PSA               SIM

|               |                 |                    |                |

| sAT_%CPHS(0)  |                 |                    |                |

*-------------->*                 |                    |                |

|               |                 |                    |                |

|               |   cphs_stop()   |                    |                |

|               *---------------->*                    |                |

(1): CPHS not initialised yet

|               |                 |                    |                |

|               |  CPHS_NOT_INIT  |                    |                |

|               *<--%ret result%--*                    |                |

|               |                 |                    |                |

|   rAT_CME()   |                 |                    |                |

*<--------------*                 |                    |                |

|               |                 |                    |                |

(end of 1)

OR

(2): CPHS had been initialised

|               |                 |                    |                |

|               |    CPHS_OK      |                    |                |

|               *<--%ret result%--*                    |                |

|               |                 |                    |                |

|   rAT_OK( )   |                 |                    |                |

*<--------------*                 |                    |                |

|               |                 |                    |                |

(end of 2)

|               |                 |                    |                |

## NETWORK OPERATOR NAME

### ACI/CPHS Module functional interface

#### cphs_get_opn

Prototype:

T_CPHS_RET cphs_get_opn (CHAR *longname, UBYTE *max_longname, CHAR *shortname,  UBYTE 					*max_shortname);

Parameters:

longname			operator long name

max_longname		maximal length of long name:

IN: should contain the size of memory allocated for longname.

OUT: If memory is not enough, then function returns with CPHS_FAIL and max_longname will contain the amount of memory needed. Otherwise contains amount of written characters.

shortname			operator short name

max_shortname		maximal length of short name:

IN: should contain the size of memory allocated for shortname.

OUT: If memory is not enough, then function returns with CPHS_FAIL and max_shortname will contain the amount of memory needed. Otherwise contains amount of written characters.

Return:

CPHS_OK			execution OK
	CPHS_BUSY			CPHS module is busy

CPHS_FAIL		error occurred: if *max_buffer is not 0, then reason is memory for *buffer is not enough: check max_buffer for amount of memory needed.

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Returns the operator long name and short name if available.

### AT command

#### AT%CPOPN

Table 2: %CPOPN parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| %CPOPN? | %CPOPN: <LONG_NAME>[,<SHORT_NAME>] +CME ERROR: <err> |
| %CPOPN=? |  |

Description

%CPOPN: "LONG NAME"[, "SHORT NAME"] will be displayed as intermediate result if operation is successful (short name is optional: it will be shown only if SIM supports this feature).

This query command returns the long and short names of the operator as stored in the corresponding SIM fields.

Defined values

<LONG_NAME>: string type indicating: Operator Long Name

<SHORT_NAME>: string type indicating: Operator Short Name (optional: may not be present in SIM)

### ACI/MFW functional interface

#### qAT_PercentCPOPN

Prototype:

T_ACI_RETURN qAT_PercentCPOPN(T_ACI_CMD_SRC srcId, CHAR *longname, 
				UBYTE	*max_longname, CHAR *shortname,  UBYTE *max_shortname);

Parameters:

longname			operator long name

max_longname		maximal length of long name:

IN: should contain the size of memory allocated for longname.

OUT: If memory is not enough, then function returns with AT_FAIL and max_longname will contain the amount of memory needed. Otherwise contains amount of written characters.

shortname			operator short name

max_shortname		maximal length of short name:

IN: should contain the size of memory allocated for shortname.

OUT: If memory is not enough, then function returns with AT_FAIL and max_shortname will contain the amount of memory needed. Otherwise contains amount of written characters.

Return:

AT_CMPL			successfully completed

AT_FAIL		an error occurred: if *max_buffer is not 0, then reason is memory for *buffer is not enough: check max_buffer for amount of memory needed.

Description:

Queries long and short names of the operator.

### MSC

[Gets operator long and/or short name]

AT%COPN?

%COPN: “operator long name”,”op”

OK

ATI/MFW             CMH              CPHS                 PSA               SIM

|                 |                 |                    |                |

|  qAT_%CPOPN()   |                 |                    |                |

*---------------->*                 |                    |                |

|                 |                 |                    |                |

|                 | cphs_get_opn()  |                    |                |

|                 *---------------->*                    |                |

|                 |                 |                    |                |

|                 |    CPHS_OK      |                    |                |

|                 *<--%ret result%--*                    |                |

|                 |                 |                    |                |

|     AT_CMPL     |                 |                    |                |

*<--%ret result%--*                 |                    |                |

|                 |                 |                    |                |

## HOME COUNTRY ROAMING INDICATOR

### ACI/CPHS Module functional interface

#### cphs_roaming_ind

Prototype:

void cphs_roaming_ind(UBYTE roaming_status);

Parameters:

roaming_status			roaming status

Values:

#define CPHS_ROAMING_ON  (1)

#define CPHS_ROAMING_OFF (0)

Description:

Informs the CPHS module that the Roaming state has changed.

### AT command

#### %CPROAM

Table 3: %CPROAM parameter unsolicited result syntax

Unsolicited Result

%CPROAM: <roam_status>

Description

This unsolicited result will be displayed whenever the mobile’s Roaming registration state has changed.

Defined values

< roam_status>: integer type value indicating:

0		–  Mobile is not in Roaming state.

1 	– Mobile is in Roaming state (mobile is registered to an other than the home network).

### ACI/MFW functional interface

#### rAT_PercentCPROAM

Prototype:

void rAT_PercentCPROAM(UBYTE roam_status);

Parameters:

roaming_status			roaming status

Values:

#define CPHS_ROAMING_ON  (1)

#define CPHS_ROAMING_OFF (0)

Return:

void

Description:

Informs ATI/MFW that the mobile’s Roaming state has changed.

### MSC

[CPHS Roaming Indicator]

Unsolicited Message:

%CPROAM: 1

ATI/MFW           CMH              CPHS                  PSA             MM/GMM

|               |                 |                    |                 |

(GPRS case)

|               |                 |                    |GMMREG_ATTACH_CNF|

|               |                 |                    *<================*

|               |                 |                    |    “Roaming”    |

|               |                 |                    |                 |

OR

(GSM case)

|               |                 |                    |  MMREG_REG_CNF  |

|               |                 |                    *<================*

|               |                 |                    |    “Roaming”    |

|               |                 |                    |                 |

|               |                 | cphs_roaming_ind() |                 |

|               |                 |  CPHS_ROAMING_ON   |                 |

|               |                 *<-------------------*                 |

|               |                 |                    |                 |

|               |cmhCPHS_user_cb()|                    |                 |

|               | CPHS_ROAM_IND   |                    |                 |

|               *<----------------*                    |                 |

|               |                 |                    |                 |

|rAT_%CPROAM(1) |                 |                    |                 |

*<--------------*                 |                    |                 |

|               |                 |                    |                 |

|               |                 |                    |                 |

|               |                 |                    |                 |

## VOICE MESSAGE WAITING

### ACI/CPHS Module functional interface

##### cphs_voice_mail_ind

Prototype:

void cphs_voice_mail_ind(UBYTE set_flag, T_CPHS_LINES line);

Parameters:

set_flag		waiting flag is to be set/erased on the SIM

line		line to which apply received Voice Mail Indication.

Return:

void

Description:

Indication to the CPHS module that a Voice Mail related SMS has been received from the network.

#### Voice Message Waiting Indicator Setting/Clearing/Reading

##### cphs_set_waiting_flag

Prototype:

T_CPHS_RET cphs_set_waiting_flag(UBYTE set_flag, T_CPHS_LINES lines);

Parameters:

set_flag		waiting flag is to be set/erased on the SIM

Values:

#define CPHS_SET_WAITING_FLAG  (1)

#define CPHS_ERASE_WAITING_FLAG (0)

lines	lines associated to waiting flag indicator: it is a bit mask, so it can be used to change the flag status for several lines at once.

Return:

CPHS_EXEC		operation in progress

CPHS_BUSY		CPHS module is busy

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Controls state of Waiting Indicator Flag on the SIM.

##### cphs_get_waiting_flag

Prototype:

T_CPHS_RET cphs_get_waiting_flag(UBYTE *flag_set, T_CPHS_LINES line);

Parameters:

flag_set		contains flag status.

Values:

#define CPHS_FLAG_DEACTIVATED  (0)

#define CPHS_FLAG_ACTIVATED  (1)

#define CPHS_FLAG_NOT_PRESENT  (2)

#define CPHS_FLAG_ERROR  (3)

line	line associated to waiting flag indicator: it is NOT a bit mask !!

Return:

CPHS_OK		operation successful

CPHS_FAIL		an error occurred (ex: line contains more than one line)

CPHS_BUSY			CPHS module is busy

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Retrieves state of waiting indicator flag for a given bearer capability from the CPHS cached data.

### AT command

#### %CPVWI

Table 4: %CPVWI parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| %CPVWI=<mode>,<line> | %CPVWI: <status>[,<line>] +CME ERROR: <err> |
| %CPVWI? | +CME ERROR: <err> |
| %CPVWI=? | %CPVWI: (list of supported <mode>s,<line>s) +CME ERROR: <err> |

Table 5: %CPVWI parameter unsolicited result syntax

Unsolicited Result

%CPVWI: <status>,<line>

Description

This set command allows setting, clearing, querying of the status of the Voice Waiting Message Flag for one or several lines.

When querying, intermediate result is %CPVWI: status(,lines) (lines is optional: either Voice Waiting Message flag is set for no line and intermediate result is %CPVWI: 0, or Voice Waiting Message flag is set for at least one line and intermediate result is %CPVWI= 1,set_lines).

An unsolicited result %CPVWI: status,line will be displayed upon receiving a Voice Message Waiting SMS from the network.

Defined values

<mode>: integer type value indicating:

0		– Clear Voice Waiting Message Flag.

1 	– Set Voice Waiting Message Flag.

2 	– Query Voice Waiting Message Flag status.

<line>: bit field type value indicating possible sum of following values:

1 	– Line 1

2 	– Data

4 	– Fax

256 	– Line 2

<status>: integer type value indicating:

0		– Clear

1 	– Set

### ACI/MFW functional interface

#### sAT_PercentCPVWI

Prototype:

T_ACI_RETURN sAT_PercentCPVWI(T_ACI_CMD_SRC srcId, UBYTE flag_set, T_CPHS_LINES lines);

Parameters:

flag_set		flag status.

Values:

#define CPHS_SET_WAITING_FLAG  (1)

#define CPHS_ERASE_WAITING_FLAG (0)

lines	line to which change applies: it is a bit mask, so it can be used to change the flag status for several lines at once.

Return:

AT_EXCT	operation in progress

AT_FAIL	an error occurred

Description:

Allows setting or clearing the voice waiting message flag for any line.

#### qAT_PercentCPVWI

Prototype:

T_ACI_RETURN qAT_PercentCPVWI(T_ACI_CMD_SRC srcId, UBYTE * flag_set, T_CPHS_LINES line);

Parameters:

flag_set		flag status.

Values:

#define CPHS_FLAG_DEACTIVATED  (0)

#define CPHS_FLAG_ACTIVATED  (1)

#define CPHS_FLAG_NOT_PRESENT  (2)

#define CPHS_FLAG_ERROR  (3)

line		line to which query applies: it is NOT a bit mask !!

Return:

AT_CMPL	successfully completed

AT_FAIL	an error occurred: (ex: line contains more than one line)

Description:

Queries the current setting for voice waiting message flag for each line.

#### rAT_PercentCPVWI

Prototype:

void rAT_PercentCPVWI(UBYTE flag_set, T_CPHS_LINES line);

Parameters:

flag_set		flag status.

Values:

#define CPHS_SET_WAITING_FLAG  (1)

#define CPHS_ERASE_WAITING_FLAG (0)

line		line to which query applies

Return:

void

Description:

Informs ATI/MFW that a new Voice Message is awaiting retrieval.

### MSC

[CPHS Voice Message Waiting Indicator]

Unsolicited Message:

%CPVWI: 1,1		New Voice Waiting Message for line 1.

ATI/MFW           CMH              CPHS                  PSA              SMS

|               |                 |                    |                |

|               |                 |                    |  MNSMS_MSG_IND |

|               |                 |                    *<===============*

|               |                 |                    |“Voice Msg Wait”|

|               |                 |                    |                |

|               |                 | cphs_voice_mail_ind|                |

|               |                 *<-------------------*                |

|               |                 |                    |                |

|               |                 |psaSIM_AccessSIMData|                |

|               |                 *------------------->*                |

|               |                 |                    |                |

|               |                 |                    | SIM_UPDATE_REQ |

|               |                 |                    *===============>*

|               |                 |                    |                |

|               |                 |                    |                |

|               |                 |                    | SIM_UPDATE_CNF |

|               |                 |                    *<===============*

|               |                 |                    |                |

|               |                 | cphs_sim_data_cb() |                |

|               |                 *<-------------------*                |

|               |                 |                    |                |

|               |cmhCPHS_user_cb()|                    |                |

|               | VOICE_MAIL_IND  |                    |                |

|               *<----------------*                    |                |

|               |                 |                    |                |

|  rAT_%CPVWI() |                 |                    |                |

*<--------------*                 |                    |                |

|               |                 |                    |                |

[CPHS Set/Clear Waiting Flag Indicator]

AT%CPVWI=0	 	Clears flag indicators for all lines.

OK

ATI/MFW           CMH                  CPHS                  PSA             SIM

|               |                     |                    |               |

| sAT_%CPVWI()  |                     |                    |               |

*-------------->*                     |                    |               |

|               |                     |                    |               |

|               |cphs_set_waiting_flag|                    |               |

|               *-------------------->*                    |               |

|               |                     |                    |               |

|               |                     |psaSIM_AccessSIMData|               |

|               |                     *------------------->*               |

|               |                     |                    |               |

|               |                     |                    |SIM_ UPDATE_REQ|

|               |                     |                    *==============>*

|               |                     |                    |               |

|               |      CPHS_EXEC      |                    |               |

|               *<----%ret result%----*                    |               |

|               |                     |                    |               |

|     AT_EXCT   |                     |                    |               |

*<-%ret result%-*                     |                    |               |

|               |                     |                    |               |

|               |                     |                    |SIM_UPDATE_CNF |

|               |                     |                    *<==============*

|               |                     |                    |               |

|               |                     | cphs_sim_data_cb() |               |

|               |                     *<-------------------*               |

|               |                     |                    |               |

|               |  cmhCPHS_user_cb()  |                    |               |

|               | CPHS_VOICE_MAIL_RES |                    |               |

|               *<--------------------*                    |               |

|               |                     |                    |               |

|   rAT_OK( )   |                     |                    |               |

*<--------------*                     |                    |               |

|               |                     |                    |               |

[CPHS Get Waiting Flag Indicator]

AT%CPVWI=2 		Queries for all lines: 1 waiting msg for line 1 and line 2,

none for fax and data

%CPVWI: 1,257

OK

ATI/MFW             CMH                       CPHS             PSA

|                 |                          |               |

| qAT_%CPVWI()    |                          |               |

*---------------->*                          |               |

|                 |                          |               |

|                 | cphs_get_waiting_flag( ) |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

|     AT_CMPL     |                          |               |

*<--%ret result%--*                          |               |

|                 |                          |               |

## DIVERTED CALL INDICATOR and CALL FORWARDING FLAG

### ACI/CPHS Module functional interface

##### cphs_set_cfu_flag

Prototype:

T_CPHS_RET cphs_set_cfu_flag(UBYTE cfu_set, T_CPHS_LINES lines);

Parameters:

cfu_set		Call Forwarding Unconditional flag is to be set/erased on the SIM

Values:

#define CPHS_SET_CFU_FLAG (1)

#define CPHS_ERASE_CFU_FLAG (0)

lines		line(s) to which CFU status change applies (in case of ALS): this is a bit mask.

Return:

CPHS_EXEC		operation in progress

CPHS_BUSY		CPHS module is busy

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Controls state of Unconditional Call Forwarding Flag on the SIM.

##### cphs_get_fwd_flag

Prototype:

T_CPHS_RET cphs_get_fwd_flag(UBYTE *cfu_set, T_CPHS_LINES line);

Parameters:

cfu_set		flags whether Call Forwarding Unconditional is active or not

Values:

#define CPHS_FLAG_DEACTIVATED  (0)

#define CPHS_FLAG_ACTIVATED  (1)

#define CPHS_FLAG_NOT_PRESENT  (2)

#define CPHS_FLAG_ERROR  (3)

line		line to which CFU status applies. (This is NOT a bitmask)

Return:

CPHS_OK		operation successful

CPHS_FAIL		an error occurred (ex: line contains more than one line)

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Retrieves the Call Forwarding flag status stored on the SIM.

### AT command

#### %CPCFU

Table 6: %CPCFU parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| %CPCFU=<mode>,<line> | %CPCFU: <status>[,<line>] +CME ERROR: <err> |
| %CPCFU? | +CME ERROR: <err> |
| %CPCFU=? | %CPCFU: (list of supported <mode>s,<line>s) +CME ERROR: <err> |

Description

This set command allows setting, clearing, querying of the status of the Call Diverted Flag for one or several lines.

This query command allows querying the current status of the Call Diverted Flag on the SIM for one or several lines.

When querying, intermediate result is %CPCFU: status(,lines) (lines is optional: either Unconditional Call Forwarding has not been activated for any line and intermediate result is %CPCFU: 0, or Unconditional Call Forwarding has been activated for at least one line and intermediate result is %CPCFU= 1,set_lines).

Defined values

<mode>: integer type value indicating:

0		– Clear Call Diverted Flag

1 	– Set Call Diverted Flag.

2 	– Query Call Diverted Flag status.

<line>: bit field type value indicating possible sum of following values:

1 	– Line 1

2 	– Data

4 	– Fax

256 	– Line 2

<status>: integer type value indicating:

0		– Activated

1 	– Deactivated

### ACI/MFW functional interface

#### sAT_PercentCPCFU

Prototype:

T_ACI_RETURN sAT_PercentCPCFU(T_ACI_CMD_SRC srcId, UBYTE cfu_set, T_CPHS_LINES lines);

Parameters:

cfu_set		flag status.

Values:

#define CPHS_SET_CFU_FLAG  (1)

#define CPHS_ERASE_CFU_FLAG (0)

lines	line to which change applies: it is a bit mask, so it can be used to change the flag status for several lines at once.

Return:

AT_EXCT	operation in progress

AT_FAIL	an error occurred

Description:

Allows setting or clearing the Call Forwarding Unconditional flag for any line.

#### qAT_PercentCPCFU

Prototype:

T_ACI_RETURN qAT_PercentCPCFU(T_ACI_CMD_SRC srcId, UBYTE *cfu_set, T_CPHS_LINES line);

Parameters:

cfu_set		flags whether Call Forwarding Unconditional is active or not

Values:

#define CPHS_FLAG_DEACTIVATED  (0)

#define CPHS_FLAG_ACTIVATED  (1)

#define CPHS_FLAG_NOT_PRESENT  (2)

#define CPHS_FLAG_ERROR  (3)

line		line to which CFU query applies (this is not a bit mask)

Return:

AT_CMPL	successfully completed

AT_FAIL	an error occurred (ex: line contains more than one line)

Description:

Queries the current call forwarding flag state.

### MSC

[CPHS Set/Clear CFU Flag]

AT%CPCFU=0	 	Clears flag for all lines.

OK

ATI/MFW           CMH                  CPHS                  PSA             SIM

|               |                     |                    |               |

| sAT_%CPCFU()  |                     |                    |               |

*-------------->*                     |                    |               |

|               |                     |                    |               |

|               |  cphs_set_cfu_flag  |                    |               |

|               *-------------------->*                    |               |

|               |                     |                    |               |

|               |                     |psaSIM_AccessSIMData|               |

|               |                     *------------------->*               |

|               |                     |                    |               |

|               |                     |                    |SIM_ UPDATE_REQ|

|               |                     |                    *==============>*

|               |                     |                    |               |

|               |      CPHS_EXEC      |                    |               |

|               *<----%ret result%----*                    |               |

|               |                     |                    |               |

|     AT_EXCT   |                     |                    |               |

*<-%ret result%-*                     |                    |               |

|               |                     |                    |               |

|               |                     |                    |SIM_UPDATE_CNF |

|               |                     |                    *<==============*

|               |                     |                    |               |

|               |                     | cphs_sim_data_cb() |               |

|               |                     *<-------------------*               |

|               |                     |                    |               |

|               |  cmhCPHS_user_cb()  |                    |               |

|               |    CPHS_CFU_RES     |                    |               |

|               *<--------------------*                    |               |

|               |                     |                    |               |

|   rAT_OK( )   |                     |                    |               |

*<--------------*                     |                    |               |

|               |                     |                    |               |

[CPHS Get CFU Flag]

AT%CPCFU=2,1        Queries for line 1.

%CPCFU: 0	 	CFU is inactive for line 1.

OK

ATI/MFW             CMH                       CPHS             PSA

|                 |                          |               |

|  qAT_%CPCFU()   |                          |               |

*---------------->*                          |               |

|                 |                          |               |

|                 |   cphs_get_fwd_flag( )   |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

|     AT_CMPL     |                          |               |

*<--%ret result%--*                          |               |

|                 |                          |               |

## CURRENT LINE INDICATOR and ALS

### ACI/CPHS Module functional interface

##### cphs_get_line

Prototype:

T_CPHS_RET cphs_get_line(UBYTE srcId, UBYTE call_id, T_CPHS_LINES *line, CHAR *line_desc, UBYTE *max_line_desc);

Parameters:

call_id	call id of line being queried. This value is the same as the call id described in GSM 02.30 subclause 4.5.5.1 (also see AT+CLCC). If not present (value 255), then current active line is to be returned.

line		contains line of queried call, or current active line.

line_desc	line identification in SIM

max_line_desc	maximal length of line_desc:

IN: should contain the size of memory allocated for line_desc.

OUT: If memory is not enough, then function returns with CPHS_FAIL and max_line_desc will contain the amount of memory needed. Otherwise contains amount of written characters.

Return:

CPHS_OK			execution OK

CPHS_FAIL	error occurred: if *max_buffer is not 0, then reason is memory for *buffer is not enough: check max_buffer for amount of memory needed.

CPHS_BUSY			CPHS module is busy

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Informs the CPHS module what current line is active for a MT call.

### AT command

#### %CPALS

Table 7: %CPALS parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| %CPALS=<call_id> | %CPALS: <line>,<MSISDN Id> +CME ERROR: <err> |
| %CPALS? | %CPALS: <line>,<MSISDN Id> +CME ERROR: <err> |
| %CPALS=? |  |

Description

This set command allows querying the bearer of a given call. The user has thus the possibility to query the line of a current call.

The query command returns the current active line.

Defined values

<call_id>: integer type value indicating:

call id of line being queried. This value is the same as the call id described in GSM 02.30 subclause 4.5.5.1 (also see AT+CLCC).

<line>: integer type value indicating possible sum of following values:

1 	– Line 1

2 	– Data

4 	– Fax

256 	– Line 2

<MSISDN id>: string type value indicating:

This is the MSISDN identification as found on the SIM. If no MSISDN identification can be found on the SIM then the default strings “Line 1", “Data”, “Fax” and “Line 2” will be applied.

### ACI/MFW functional interface

#### qAT_PercentCPALS

Prototype:

T_ACI_RETURN qAT_PercentCPALS(T_ACI_CMD_SRC srcId, UBYTE call_id, T_CPHS_LINES *line, CHAR *line_desc, UBYTE *max_line _desc);

Parameters:

call_id	call id of line being queried. This value is the same as the call id described in GSM 02.30 subclause 4.5.5.1 (also see AT+CLCC). If not present (value 255), then current active line is to be returned.

line		contains line of queried call, or current active line.

line_desc	line identification in SIM

max_line _desc	maximal length of line_desc:

IN: should contain the size of memory allocated for line_desc.

OUT: If memory is not enough, then function returns with AT_FAIL and max_line_desc will contain the amount of memory needed. Otherwise contains amount of written characters.

Return:

AT_CMPL	successfully completed

AT_FAIL	error occurred: if *max_buffer is not 0, then reason is memory for *buffer is not enough: check max_buffer for amount of memory needed.

Description:

Gives the user the possibility to query at any moment the current active line and for any call its related line (e.g. 	when receiving a MT call)

### MSC

[CPHS Alternate Line Service]

RING

AT+CLCC

+CLCC: 1,1,4,0,0		// incoming voice MT call with cId 1

OK

AT%CPALS=1		// queries line of MT call

%CPALS: 2		// call aimed at line 2 MSISDN

OK

AT%CPALS?		// queries current active line

%CPALS: 1		// active line is line 1 MSISDN

OK

ATI/MFW           CMH              CPHS                    PSA               CC

|               |                  |                     |                |

|               |                  |                     | MNCC_SETUP_IND |

|               |                  |                     *<===============*

|               |                  |                     |                |

|               |         cmhCC_IncomingCall( )          |                |

|               *<---------------------------------------*                |

|               |                  |                     |                |

|  rAT_RING( )  |                  |                     |                |

*<--------------*                  |                     |                |

|               |                  |                     |                |

|               |                  |                     |                |

|               |                  |                     |                |

| qAT_%CPALS    |                  |                     |                |

*-------------->*                  |                     |                |

|               |                  |                     |                |

|               | cphs_get_line()  |                     |                |

|               *----------------->*                     |                |

|               |                  |                     |                |

|               |     CPHS_OK      |                     |                |

|               *<--%ret result%---*                     |                |

|               |                  |                     |                |

|    AT_CMPL    |                  |                     |                |

*<-%ret result%-*                  |                     |                |

|               |                  |                     |                |

|               |                  |                     |                |

## EMERGENCY CALL 999

This feature is already covered by later GSM recommendation. This means CPHS requirements are already fulfilled in the software current implementation.

## CPHS INFORMATION and CUSTOMER SPECIFIC PROFILE

### Data Types

Some macros will be made available to handle CPHS info and CSP:

#define CPHS_CHECK_SST(sst, service, attribute)

Has value TRUE if service has "attribute" status.

Attribute:

ALLOCATED	1

ACTIVATED	2

Services:

CSP			1

SST			2

Mailbox Numbers	3

OpName Shortform	4

Information numbers	5

### ACI/CPHS Module functional interface

##### cphs_get_cphs_info

Prototype:

T_CPHS_RET cphs_get_cphs_info(UBYTE  *phase, USHORT *sst);

Parameters:

phase				either CPHS Phase 1: 1 or CPHS Phase 2: 2

sst				service table: 2 bytes in Hexadecimal format as stored on SIM

Return:

CPHS_OK			execution OK

CPHS_BUSY			CPHS module is busy

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Retrieves the information concerning CPHS.

##### cphs_get_csprof

Prototype:

T_CPHS_RET cphs_get_csprof(CHAR *csp, UBYTE *max_csp_length);

Parameters:

csp			customer service profile: format is the one described in CPHS4_2.ww6

max_csp_length		maximal length of csp:

IN: should contain the size of memory allocated for csp.

OUT: If memory is not enough, then function returns with CPHS_FAIL and max_csp_length will contain the amount of memory needed. Otherwise contains amount of written characters.

Return:

CPHS_OK			execution OK

CPHS_FAIL	error occurred: if max_csp_length is not 0, then reason is memory for csp is not enough: check max_csp_length for amount of memory needed.

CPHS_BUSY			CPHS module is busy

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Retrieves the Customer Service Profile.

### AT command

#### %CPINF

Table 8: %CPINF parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| %CPINF? | %CPHS: <phase>,<sst>[,<csp>] +CME ERROR: <err> |
| %CPINF=? |  |

Description

This query command allows query the CPHS phase, the CPHS service table and the customer service profile.

Defined values

<phase>: integer type value indicating:

1 – Phase 1.

2 – Phase 2.

<sst>: 2 bytes Hexadecimal value indicating:

CPHS Service Table: 2 bytes (format HEX) as defined in CPHS4_2.ww6.

<csp>:  Hexadecimal string indicating:

Customer Service Profile. Format is a hexadecimal string as following: “A1B1A2B2A3B3…” where byte An is the service group code (see CPHS B.4.7.1) and byte Bn the services Byte (see CPHS B.4.7.1)

### ACI/MFW functional interface

#### qAT_PercentCPINF

Prototype:

T_ACI_RETURN qAT_PercentCPINF(T_ACI_CMD_SRC srcId, UBYTE  *phase, USHORT *sst, CHAR *csp, UBYTE *max_csp_size);

Parameters:

phase		either CPHS Phase 1: 1 or CPHS Phase 2: 2

sst		service table: 2 bytes in Hexadecimal format as stored on SIM

csp		customer service profiles

max_cps_size	maximal length of csp:

IN: should contain the size of memory allocated for csp.

OUT: If memory is not enough, then function returns with CPHS_FAIL and max_csp_length will contain the amount of memory needed. Otherwise contains amount of written characters.

Return:

AT_CMPL	successfully completed

AT_FAIL	error occurred: if *max_buffer is not 0, then reason is memory for *buffer is not enough: check max_buffer for amount of memory needed.

Description:

Queries the CPHS phase, service table and customer service profiles.

### MSC

[CPHS Get Information and Customer Service Profile]

Ex: Phase 2, Service Table where “CSP” and “OpName Shortform” are allocated and activated and the rest is not allocated. CSP contains Service Groups “Call Offering”, “Call Completion”, “ValueAdded Services” and “Phase Service 2+”.

AT%CPINF?

%CPINF: 2,C300,01F804F0C00109FA

OK

ATI/MFW             CMH                       CPHS             PSA

|                 |                          |               |

|                 |                          |               |

|  qAT_%CPINF()   |                          |               |

*---------------->*                          |               |

|                 |                          |               |

|                 |  cphs_get_cphs_info( )   |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

|                 |                          |               |

|                 |     phs_get_csprof( )    |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

|     AT_CMPL     |                          |               |

*<--%ret result%--*                          |               |

|                 |                          |               |

## READ MAILBOX NUMBERS

### Data Types

typedef struct

{

T_CPHS_LINES	line;

CHAR			number[40];	// 10 BCD in 6F17 and 10 BCD in EXT 1 at most

UBYTE		toa;		// type of address

CHAR			alpha_id[22];

} T_CPHS_MB

### ACI/CPHS Module functional interface

##### cphs_read_mb_number

Prototype:

T_CPHS_RET cphs_read_mb_number(UBYTE rec_id, T_CPHS_MB *mailbox_entry);

Parameters:

rec_id			Id of queried mailbox number

mailbox_entry		contains all information concerning queried mailbox number

Return:

CPHS_OK			execution OK

CPHS_BUSY			CPHS module is busy

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Returns mailbox number information stored on SIM at a given record.

##### cphs_first_free

Prototype:

T_CPHS_RET cphs_first_free(UBYTE *first_free);

Parameters:

first_free	Index of first unused entry in mailbox list

Return:

CPHS_OK			execution OK

CPHS_FAIL			an error occurred: e.g. no information numbers cached

Description:

Returns the index of the first unused entry in the mailbox list.

### AT command

#### AT%CPMB

Table 9: %CPMB parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| %CPMB=<record_id> | %CPMB: <record_id>,<line>,<”number”>,<toa>, <“alpha id“> +CME ERROR: <err> |
| %CPMB? | %CPMB: <first> +CME ERROR: <err> |
| %CPMB=? | %CPMB: (max value of <record_id>) on the SIM +CME ERROR: <err> |

Description

This set command allows retrieving of the mailbox number by means of its record id on the SIM.

The query command returns the index of the first free entry in the list.

Test command returns the number of the existing record entries of mailbox numbers on the SIM.

Defined values

<record_id>: integer type value indicating:

SIM record id of mailbox

<line>: integer type value indicating:

1 	– Line 1

2 	– Data

4 	– Fax

256 	– Line 2

<number>: string type value indicating:

mailbox number.

<toa>: Integer type value indicating:

type of address (coded as in GSM 04.08)

<alpha id>: string type value indicating:

alpha identifier related to mailbox.

<first> : Integer type value indicating:

index of the first free entry in the list.

### ACI/MFW functional interface

#### qAT_PercentCPMB

Prototype:

T_ACI_RETURN qAT_PercentCPMB(T_ACI_CMD_SRC srcId, UBYTE rec_id, T_CPHS_LINES	line, CHAR *number, T_ACI_TOA_TON ton, T_ACI_TOA_NPI npi, CHAR *alpha_id, UBYTE *first);

Parameters:

rec_id			Id of queried mailbox number

line			line to which apply received Mailbox Number

number			mailbox telephone number

ton			type-of-number of mailbox number

npi			numbering-plan-identifier of mailbox number

alpha_id		alpha identifier related to mailbox

first			first unused entry

Return:

AT_CMPL		successfully completed

AT_FAIL		an error occurred

Description:

Deals with both the set and query aspects of the %CPMB command. If rec_id is present, indicating a set operation, it will return information relating to the specified mailbox number (via parameters line, number, ton, npi and alpha_id). If rec_id is not present, indicating a query operation, it will return the index of the first unused entry in the list (via parameter first).

### MSC

[CPHS Get Mailbox Number]

AT%CPMB=?

%CPMB: 4

OK

AT%CPMB=2

%CPMB: 2,256,"012341234",129,"MB LINE 2"

OK

ATI/MFW             CMH                       CPHS             PSA

|                 |                          |               |

|                 |                          |               |

|   qAT_%CPMB()   |                          |               |

*---------------->*                          |               |

|                 |                          |               |

|                 |  cphs_read_mb_number( )  |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

|     AT_CMPL     |                          |               |

*<--%ret result%--*                          |               |

|                 |                          |               |

## WRITE MAILBOX NUMBERS

### Data Types

typedef struct

{

UBYTE    data[CPHS_MAX_MB_ALPHA_LEN];

UBYTE    len;

} T_CPHS_PB_TEXT;

### ACI/CPHS Module functional interface

##### cphs_write_mb_number

Prototype:

T_CPHS_RET cphs_write_mb_number (UBYTE srcId, UBYTE rec_id, UBYTE *tag, 
	UBYTE tag_len, UBYTE bcd_len, UBYTE *number, UBYTE ton_npi);

Parameters:

source_id		Id of the calling source

rec_id			Id of queried mailbox number

tag			pointer to the alpha indentifier tag

tag_len			length of the alpha identifier tag

bcd_len			length of the bcd encoded  number

number			pointer to the bcd encoded number

ton_npi			ton/npi value of number

Return:

CPHS_EXEC			executing

CPHS_BUSY			CPHS module is busy

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Writes a mailbox entry with given parameters to SIM.

##### cphs_get_mb_parameter

Prototype:

T_CPHS_RET cphs_get_mb_parameter (  SHORT *firstIdx, SHORT *lastIdx, UBYTE *nlength, 	UBYTE *tlength );

Parameters:

firstIdx			pointer to store first index of mailbox phonebook

lastIdx			pointer to store first index of mailbox phonebook

nlength			pointer to the alpha indentifier tag

tlength 			length of the alpha identifier tag

Return:

CPHS_OK			execution OK

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Returns the mailbox phonebook related parameters.

### AT command

#### AT%CPMBW

Table 10: %CPMBW parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| %CPMBW=<record_id>[,[<number>] ,[<toa>],[<text>]] | +CME ERROR: <err> |
| %CPMBW=? | %CPMBW: (list of supported <record_id>s),<nlength>, (list of supported <type>s),<tlength> +CME ERROR: <err> |

Description

Execution command writes phonebook entry in location number <record_id> of the CPHS mailbox phonebook of the SIM. Entry fields written are phone number <number> (in the format <toa>) and text <text> associated with the number. If <number> is omitted, phonebook entry is deleted. If writing fails in an ME error, +CME ERROR: <err> is returned. Refer subclause 9.2 of 3GPP 07.07 for <err> values.

Test command returns location range supported by the CPHS mailbox phonebook as a compound value, the maximum length of <number> field, supported number formats of the storage, and the maximum length of <text> field. If ME is not currently reachable, +CME ERROR: <err> is returned. Refer subclause 9.2 of 3GPP 07.07 for <err> values.

Defined values

<record_id>: integer type values in the range of location number of mailbox phonebook memory.

Attention:

Since there is no support for the parameter the following convention is used 
	(refer to the CPHS specification)

<record_id>	line_id		line description

1		1 		Line 1

2		2 		Data

3		4 		Fax

4		256		Line 2

<number>: string type phone number

<toa>:  type of address octet in integer format (refer GSM 04.08).

<text>: string type field of maximum length <tlength>

<nlength>: integer type value indicating the maximum length of field <number>

<tlength>: integer type value indicating the maximum length of field <text>

### ACI/MFW functional interface

#### sAT_PercentCPMBW

Prototype:

T_ACI_RETURN sAT_PercentCPMBW( T_ACI_CMD_SRC  srcId, SHORT index, 
	CHAR *number, T_ACI_TOA *type, T_CPHS_PB_TEXT *text);

Parameters:

srcId			ID of using source

index			index of record

number			pointer to mailbox telephone number

type			pointer to ton/npi of mailbox number in ACI struct

texi			pointer to numbering-plan-identifier of mailbox number

Return:

AT_EXCT		successfully executing

AT_FAIL		an error occurred

Description:

Writes an entry given by index to the CPHS mailbox phonebook SIM storage and updates the respective ME 	Cache for this entry.

#### tAT_PercentCPMBW

Prototype:

T_ACI_RETURN tAT_PercentCPMBW ( T_ACI_CMD_SRC srcId,  SHORT *firstIdx, 
	SHORT *lastIdx, UBYTE *nlength, UBYTE *tlength );

Parameters:

srcId			ID of using source

firstIdx			pointer to store first index of mailbox phonebook

lastIdx			pointer to store first index of mailbox phonebook

nlength			pointer to the alpha indentifier tag

tlength 			length of the alpha identifier tag

Return:

AT_OK			successfully executed

AT_FAIL		an error occurred

Description:

Returns the CPHS mailbox phonebook related parameters.

### MSC

[CPHS Set Mailbox Number]

AT%CPMBW=2,"012341234",129,"MB LINE 2"

OK

AT%CPMBW=?

%CPMBW:

OK

ATI/MFW             CMH                       CPHS             PSA

|                 |                          |               |

|                 |                          |               |

|   sAT_%CPMBW()  |                          |               |

*---------------->*                          |               |

|                 |                          |               |

|                 |  cphs_write_mb_number( ) |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |                        SIM_UPDATE_RECORD_REQ

|                 |                          *-------------->*

|                 |                          |               |

|                 |                        SIM_UPDATE_RECORD_CNF

|                 |                          *<--------------*

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

|     AT_CMPL     |                          |               |

*<--%ret result%--*                          |               |

|                 |                          |               |

|                 |                          |               |

|   tAT_%CPMBW()  |                          |               |

*---------------->*                          |               |

|                 |                          |               |

|                 | cphs_get_mb_parameter( ) |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

|     AT_CMPL     |                          |               |

*<--%ret result%--*                          |               |

|                 |                          |               |

## INFORMATION NUMBERS

### Data Types:

typedef struct

{

UBYTE	element_index;		// index of element described in this structure

UBYTE	index_level;

CHAR 		alpha_tag[16];

CHAR		number[12];

UBYTE	type_of_address;

BOOL 		premium_flag;		// should it be charged with premium price?

BOOL 		network_flag;		// is it network specific ?

} T_CPHS_INF_NUM

typedef enum

{

CPNUMS_MODE_EXPLORE	= 1,	// should apply to a folder to be explored

CPNUMS_MODE_QUERY	= 2  	// queries information related to an entry

} T_CPHS_CPNUMS_MODE

### ACI/CPHS Module functional interface

##### cphs_read_info_nb

Prototype:

T_CPHS_RET cphs_read_info_nb (UBYTE element_idx, T_CPHS_INF_NUM *inf_num);

Parameters:

element_idx	Index of element chosen

inf_num		structure containing information related to an element

Return:

CPHS_OK			execution OK

CPHS_BUSY			CPHS module is busy

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Returns whole information of a given element.

##### cphs_explore_info_nbs

Prototype:

T_CPHS_RET cphs_explore_info_nbs(UBYTE element_idx, UBYTE *inf_num_indexes, UBYTE *max_elmts);

Parameters:

element_idx	Index of folder element to be explored

inf_num_list	returns list of indexes of elements contained in folder

max_elmts	limits amount of bytes to be written in inf_num_indexes. If numbers of elements to be written is bigger, then function returns with CPHS_EXEC and amount of elements will be written in max_elmts (so that function caller knows how many memory it has to allocate to retrieve the whole list).

Return:

CPHS_OK		operation successful.

CPHS_EXEC		memory for list was too small. Necessary amount is written in max_elmts.

CPHS_FAIL		an error occurred: e.g. element is no folder.

CPHS_BUSY		CPHS module is busy

CPHS_NOT_INIT	CPHS module has not been initialised

Description:

If element_idx has to be a folder. A list of the indexes of the elements contained in the folder will be built and written in inf_num_indexes. Function caller has previously informed of the size of inf_num_indexes by means of max_elmts. If this size is too small then function returns with AT_EXEC and will write the necessary size in max_elmts.

##### cphs_info_num_get_max

Prototype:

T_CPHS_RET cphs_info_num_get_max(UBYTE *max_index);

Parameters:

max_index	maximum count of information numbers

Return:

CPHS_OK			execution OK

CPHS_FAIL			an error occurred: e.g. no information numbers cached

CPHS_BUSY			CPHS module is busy

CPHS_NOT_INIT		CPHS module has not been initialised

Description:

Returns the maximum count of available information numbers.

### AT command

#### %CPNUMS

Table 11: %CPNUMS parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| %CPNUMS=<element_id>,<mode> | %CPNUMS: <element_id>,<alpha tag>,<number>, <index_level>,<premium_flag>, <network_flag> +CME ERROR: <err> |
| %CPNUMS=? | list the whole table: %CPNUMS: <element_id>,<alpha tag>,<number>, <index_level>,<premium_flag>,<network_flag> +CME ERROR: <err> |

Description

This set command has 2 modes: exploring and querying. Exploring is only allowed for elements of type folder. Exploring returns the elements belonging to folder having id "element_id" (To be able to start, an extra element ROOT is here defined with element_id 0. It is of type folder). Querying element returns information related to element having id "element_id".

Test command queries a list of all entries to be found on the SIM.

Defined values

<element_id>: integer type value indicating:

Each information number entry on the SIM gets a unique Id from the CPHS module.

0 – default Id for element ROOT (to be used for getting the first elements: exploring ROOT)

<mode>: integer type value indicating:

1 – Exploring element (only if element is a folder): returns elements belonging to folder.

2 – Querying element: returns information related to element.

<alpha tag>: string type value indicating:

alpha tag of element

<number>: string type value indicating:

telephone number of element (empty string if element is a folder)

<index_level>: integer type value indicating:

index level of element. The elements belonging to ROOT have index level 1.

<premium_flag>: integer type value indicating:

0 – Premium flag is not set for element.

1 – Premium flag is set for element.

<network_flag>: integer type value indicating:

0 – Network specific flag is not set for element.

1 – Network specific flag is set for element.

### ACI/MFW functional interface

#### qAT_PercentCPNUMS

Prototype:

T_ACI_RETURN tAT_PercentCPNUMS(T_ACI_CMD_SRC srcId);

Parameters:

void

Return:

AT_CMPL	successfully completed

AT_FAIL	an error occurred

Description:

Queries whole list.

#### sAT_PercentCPNUMS

Prototype:

T_ACI_RETURN sAT_PercentCPNUMS(T_ACI_CMD_SRC srcId, UBYTE element_index, T_CPHS_CPNUMS_MODE mode);

Parameters:

element_index	index of element to be explored

mode		exploring (only for folders) or querying

Return:

AT_CMPL	successfully completed

AT_FAIL	an error occurred

Description:

Explore folder if mode is exploring, otherwise queries element information.

#### rAT_PercentCPNUMS

Prototype:

void rAT_PercentCPNUMS (UBYTE element_index, UBYTE index_level, CHAR  *alpha_tag,
 CHAR *number, BOOL	premium_flag, BOOL network_flag);

Parameters:

element_index		index of element

index_level		index level of element (starts with 1)

alpha_tag		alpha tag of element

number			telephone number of element

premium_flag		premium flag for element (should it be charged with premium price?)

network_flag		network flag for element (is it network specific?)

Return:

void

Description:

For n elements contained in an explored folder, this function will be called n times so that ATI/MFW knows what elements are included in a given folder.

If element was queried then function will be called once: it will contain the information related to this phone number.

### MSC

[CPHS Get Information Numbers]

Ex: Example here is the one presented in CPHS recommendation B 4.6.1 – Figure 2.

AT%CPNUMS=0,1 		lists elements from root.

%CPNUMS: 1,”TRAVEL”,””,1,0,0

%CPNUMS: 8,”WEATHER”,””,1,0,0

%CPNUMS: 11,”ENTERTAINMENT”,””,1,0,0

OK

AT%CPNUMS=9,1		(Note: 9 is no Folder: Error)

ERROR

AT%CPNUMS=9,2

%CPNUMS: 9,”North”,”323”,2,0,0

OK

AT%CPNUMS=?

%CPNUMS: 1,”TRAVEL”,””,1,0,0

%CPNUMS: 2,”TAXIS”,””,2,0,0

%CPNUMS: 3,”Computercabs”,”111”,3,0,0

%CPNUMS: 4,”Dial-a-cab”,”132”,3,0,0

%CPNUMS: 5,”AIRPORTS”,””,2,0,0

%CPNUMS: 6,”Heathrow”,”345”,3,0,0

%CPNUMS: 7,”Gatwick”,”651”,3,0,0

%CPNUMS: 8,”WEATHER”,””,1,0,0

%CPNUMS: 9,”North”,”323”,2,0,0

%CPNUMS: 10,”South”,”597”,2,0,0

%CPNUMS: 11,”ENTERTAINMENT”,””,1,0,0

%CPNUMS: 12,”Ticketmaster”,”562”,2,0,0

OK

ATI/MFW             CMH                       CPHS             PSA

|                 |                          |               |

|                 |                          |               |

|  tAT_%CPNUMS()  |                          |               |

*---------------->*                          |               |

|                 |                          |               |

|                 |                          |               |

|                 | cphs_info_num_get_max( ) |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

|                 |                          |               |

|                 |  cphs_read_info_nb(1)    |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

| rAT_%CPNUMS(1)  |                          |               |

*<----------------*                          |               |

...              ...                        ...             ...

|                 |                          |               |

|                 |  cphs_read_info_nb(11)   |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

| rAT_%CPNUMS(11) |                          |               |

*<----------------*                          |               |

|                 |                          |               |

|                 |  cphs_read_info_nb(12)   |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

| rAT_%CPNUMS(12) |                          |               |

*<----------------*                          |               |

|                 |                          |               |

|     AT_CMPL     |                          |               |

*<--%ret result%--*                          |               |

|                 |                          |               |

AT%CPNUMS=8,1 		lists elements from “WEATHER”

%CPNUMS: 9,”North”,”323”,2,0,0

%CPNUMS: 10,”South”,”597”,2,0,0

OK

ATI/MFW             CMH                       CPHS             PSA

|                 |                          |               |

|                 |                          |               |

|sAT_%CPNUMS(8,1) |                          |               |

*---------------->*                          |               |

|                 |                          |               |

|                 |                          |               |

|                 | cphs_explore_info_nbs( ) |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

|                 |                          |               |

|                 |  cphs_read_info_nb(9)    |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

|  rAT_%CPNUMS(9) |                          |               |

*<----------------*                          |               |

|                 |                          |               |

|                 |  cphs_read_info_nb(10)   |               |

|                 *------------------------->*               |

|                 |                          |               |

|                 |         CPHS_OK          |               |

|                 *<------%ret result%-------*               |

|                 |                          |               |

|  rAT_%CPNUMS(10)|                          |               |

*<----------------*                          |               |

|                 |                          |               |

|     AT_CMPL     |                          |               |

*<--%ret result%--*                          |               |

|                 |                          |               |