Technical Memo

L1-DSP Recovery

Table of Contents

Table of Contents	ii

Plan Approvals	iii

1.	Introduction	3

2.	Principles	3

2.1.	Generic overview	3

2.2.	TCS5.x applicability	3

3.	Architecture	3

3.1.	Global architecture	3

3.2.	DSP recovery info	3

3.2.1 Non Critical error	3

3.2.2	Critical error	3

3.2.3	Fatal error	3

Plan Approvals

| REV | DATE |
| --- | --- |
| Draft | 1st July 2005 |
| 1.0 | 17th July 2005 |

- Introduction

This document describes L1-DSP recovery design.

The aim of the L1-DSP recovery is to restart properly the system when the DSP seems to be crashed (due to internal DSP error, or a bad L1 management)

- Principles

  - Generic overview

The purpose of DSP Recovery is to provide some debug information regarding DSP failures. If a DSP error occurs, it is outputted by the MCU in the L1 trace (see L1D_SE_TM080.doc).

Both “Diagnose module” and “Recovery module” actions are based on DSP inputs: Several events (DSP errors) are logged in the DSP/MCU communication interface. Several levels of error criticality are defined and permit to know when DSP recovery is needed.

  - TCS5.x applicability

DSP recovery is applicable to all DSP modem modules.

- Architecture

  - Global architecture

The architecture is based on an interaction between DSP, MCU L1 and Protocol Stack (PS):

- MCU L1 analyzes DSP recovery info.

- DSP may recover itself but in some case, MCU-L1 is asked to reset DSP, reload DSP code, and reboot the modem L1-MCU.

  - DSP recovery info

The DSP inputs (events) are available in a circular diagnose buffer, in a shared memory (MCU-DSP).

Each event is constituted of an identifier and associated data.

The event ID gives the error group, sub-group, and criticality of the event as shown hereafter.

The Criticality is declined as:

- Non-critical error detected, but does not compromise the SW integrity. DSP is taking specific actions if required

- Critical error detected that requires a DSP recovery: DSP self-reset, or goes in infinite loop. This is determined by “dsp_recovery_mode” variable defined in MCU/DSP interface at initialization, cf L1D_SE_TM080.doc. Only auto-reset mode will be exposed in the present document. In case the DSP goes in infinite loop, the user has to manually restart the code (for issue investigation).

- Fatal error detected that requires an MCU recovery of DSP (DSP re-install)

Please see the L1D_SE_TM080.doc for a complete description of these errors.

### 3.2.1 Non Critical error

This error has no impact on a Protocol Stack point of view. DSP is the only component to take any action if required

### Critical error

Step 1: The L2 link is broken (blocks are reported with CRC errors). The DSP is reported a critical error, then DSP is resetting itself (RESET OP CODE).

Note: at this stage, in case the number of critical error reported exceed a threshold, please go to 3.2.3 Fatal error, step 2

Step 2: Protocol stack ask for a cell selection (MPHC_RXLEV_REQ), then normal mode operation is resumed.

### Fatal error

Step 1 : The DSP is reported a fatal error, the L2 link is broken.

Step 2 : When the L1 receives a MPHC_RXLEV_REQ :

- the modem is reinit (ABB, TPU drivers and all MCU variables GSM+GPRS)

- the DSP is reloaded and restarted.

Then, normal operation is resumed.