Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document – Confidential

Test Specification

PPP

(Edit via File/Properties/Summary!)

| Document Number: | 8441.408.99.010 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.12 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2000-Jun-14 |
| Last changed: | 2003-May-26 by XGUTTEFE |
| File Name: | ppp.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2000-Jun-14 | Stefan Grundmann |  | 0.1 |  | 1 |
| 2000-Aug-09 | STW |  | 0.2 |  | 2 |
| 2000-Oct-18 | STW |  | 0.3 |  | 3 |
| 2001-Jan-22 | STW |  | 0.4 |  | 4 |
| 2001-Jun-22 | XOF |  | 0.5 |  | 5 |
| 2001-Jul-03 | XOF |  | 0.6 |  | 6 |
| 2001-Jul-12 | STW |  | 0.7 |  | 7 |
| 2001-Sep-20 | XOF |  | 0.8 |  | 8 |
| 2002-Jan-25 | STW |  | 0.9 |  | 9 |
| 2002-Apr-25 | TVO |  | 0.10 |  | 10 |
| 2002-Oct-25 | STW |  | 0.11 |  | 11 |
| 2003-May-26 | XGUTTEFE |  | 0.12 | Draft |  |

Notes:

- Initial version

- Document number changed

- Server mode test cases added

- Special cases of IP address negotiation added

- Channel ID and connect primitive added

- Test case PPP044 and PPP011 corrected

- Test of PCO processing added

- Build DTILIB functionality

- different Flow Control behavior because PPP stores one IP packet

- New tests for closed prot channel (015, 020B, 030B, 040B, 055, 051B, 060B) and changed order of primitives in transparent acknowledgement (014)

- Cause concept

Table of Contents

1.1	References	7

1.2	Abbreviations	9

1.3	Terms	12

2	Overview	12

2.1	GRR (RLC/MAC) – Radio Link Control/Medium Access Control	13

2.2	LLC – Logical Link Control	13

2.3	GMM – GPRS Mobility Management	13

2.4	SM – Session Management	13

2.5	SNDCP - Subnetwork Dependant Convergence Protocol	13

2.6	GACI – GPRS Application Control Interface	13

2.7	USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	13

2.8	TOM – Tunnelling of Messages	13

3	Parameters	14

3.1	Primitive elements	14

3.2	Struct elements	14

3.3	Fields	14

3.3.1	LCP Configure Request	14

3.3.2	LCP Configure Ack	29

3.3.3	LCP Configure Reject	38

3.3.4	LCP Configure NAK	42

3.3.5	LCP Terminate Request	45

3.3.6	LCP Terminate Ack	46

3.3.7	LCP Code Reject	47

3.3.8	LCP Protocol Reject	49

3.3.9	LCP Identification	57

3.3.10	CHAP Challenge	60

3.3.11	CHAP Response	61

3.3.12	CHAP Success	63

3.3.13	PAP Authentication Request	64

3.3.14	PAP Authentication NAK	66

3.3.15	PAP Authentication Ack	67

3.3.16	CCP Configure Request	69

3.3.17	IPCP Configure Request	71

3.3.18	IPCP Configure Reject	85

3.3.19	IPCP Configure NAK	89

3.3.20	IPCP Configure Ack	93

3.3.21	IP packets	100

3.3.22	IP frames	103

3.3.23	HDLC fragments	111

3.3.24	Invalid frames	114

3.3.25	Empty sdu	116

3.3.26	PCO lists	116

3.4	Declarations	123

3.5	Arrays	123

3.6	Structs	124

4	TEST CASES	126

4.1	Setup (PPP001 - PPP010)	126

4.1.1	PPP001:  Setup	126

4.2	Link Establishment Requests (PPP011 – PPP019)	126

4.2.1	PPP011:  Link establishment in client mode (default parameters)	126

4.2.2	PPP012:  Link establishment in client mode (PAP)	128

4.2.3	PPP013:  Link establishment in server mode	130

4.2.4	PPP014:  Link establishment in transparent mode	132

4.2.5	PPP015:  Link establishment in client mode (PAP), only peer dti channel opened	134

4.3	LCP Establishment (PPP020-PPP029)	136

4.3.1	PPP020: LCP Establishment (Client, PAP)	136

4.3.2	PPP021: LCP Establishment (CLient, PAP, ACFC, PFC)	137

4.3.3	PPP022: LCP Establishment (CLient, PAP, ACFC, PFC) with magic number request	138

4.3.4	PPP023: LCP Establishment (Server, CHAP, ACFC, PFC) with magic number request	140

4.3.5	PPP024: LCP Establishment (Server) re-request of rejected configuration options	142

4.4	Authentication Procedure (PPP030-PPP039)	145

4.4.1	PPP030: PAP Authentication in Client Mode	145

4.4.2	PPP031: PAP Authentication in Client Mode (ACFC, PFC)	146

4.4.3	PPP032: PAP authtentication failed (client mode)	147

4.4.4	PPP033: CHAP Authentication in Server Mode (ACFC, PFC) with LCP Identification	149

4.4.5	PPP034: PAP Authentication in Server Mode (ACFC, PFC) with LCP Identification	151

4.5	IPCP Establishment (PPP40-PPP049)	153

4.5.1	PPP040: Normal IPCP Address negotiation	153

4.5.2	PPP041: Normal IPCP Address negotiation (ACFC, PFC)	155

4.5.3	PPP042: CCP request from server during ICPC procedure	157

4.5.4	PPP043: IPCP configuration in server mode with CCP	160

4.5.5	PPP044: PDP activation will be rejected by the network	165

4.6	Link Establishment Confirmation (PPP050-PPP059)	167

4.6.1	PPP050: Confiramtion of  link establishment in client mode	167

4.6.2	PPP051: Open Flow Control for Downlink	168

4.6.3	PPP052: Confiramtion of  link establishment in client mode (ACFC, PFC)	169

4.6.4	PPP053: Open Flow Control for Downlink (ACFC, PFC)	169

4.6.5	PPP055: Confirmation of link establishment in client mode and acknowledgement of prot dti connection	170

4.7	Uplink Transfer (PPP060-PPP69)	171

4.7.1	PPP060: IP Packet in HDLC Frame	171

4.7.2	PPP061: IP Packet in two  HDLC Frames	173

4.7.3	PPP062: Two Packets in three Frames	174

4.7.4	PPP063: IP Packet in HDLC Frame (ACFC, PFC)	176

4.7.5	PPP064: IP Packet in two  HDLC Frames (ACFC, PFC)	177

4.7.6	PPP065: Two Packets in three Frames (ACFC, PFC)	178

4.7.7	PPP066: Three Packets in one Frames	180

4.7.8	PPP067: Three Packets in one Frame (ACFC,PFC)	181

4.7.9	PPP068: TCP Packets without Header Compression Negotiation (ACFC,PFC)	183

4.7.10	PPP069: TCP Packets with Header Compression Negotiation (ACFC,PFC)	184

4.8	Downlink Transfer (PPP070-PPP79)	186

4.8.1	PPP070: Simple Downlink transfer	186

4.8.2	PPP071: Simple Downlink transfer (ACFC, PFC)	187

4.9	PPP Termination (PPP080-PPP083)	188

4.9.1	PPP080: Termination initiated by MMI, lower layer available	188

4.9.2	PPP081: Termination initiated by MMI, lower layer not available	189

4.9.3	PPP082: Termination initiated by PPP Peer	190

4.10	Uplink Transfer with invalid packets (PPP090-PPP099)	191

4.10.1	PPP090:  one frame with invalid control field received	191

4.10.2	PPP091:  one frame with invalid protocol field received	193

4.10.3	PPP092:  one frame with invalid address field received	194

4.10.4	PPP093:  one frame with invalid FSC received	195

4.11	Modification of PPP link (PPP100-PPP109)	197

4.11.1	PPP100:  turn header compression off	197

4.12	Transparent packet transfer (PPP110-PPP119)	200

4.12.1	PPP110:  Transparent packet transfer uplink and downlink	200

4.13	Link termination in transparent mode	203

4.13.1	PPP120: Termination of transparent link	203

4.14	Link reestablishment	204

4.14.1	PPP130: Reestablishment in client mode	204

4.15	IP address negotiation (special cases)	207

4.15.1	PPP140: Server IP address in client mode allowed	207

4.15.2	PPP141: Rejection of dynamic server IP address in client mode	209

4.15.3	PPP142: Allow always static DNS addresses in server mode	212

4.15.4	PPP143: Reject dynamic DNS addresses in server mode if network does not answer	215

Appendices	219

A.	Acronyms	219

B.	Glossary	219

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 05.02 version 8.0.0 Release 1999
Digital cellular telecommunications system (Phase 2+);
Multiplexing and multiple access on the radio path

[2]	GSM 04.60 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station (MS) - Base Station System (BSS) interface;
Radio Link Control/ Medium Access Control (RLC/MAC) protocol

[3]	GSM 04.08 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface layer 3 specification

[4]	GSM 03.64 version 6.1.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Overall description of the GPRS radio interface; Stage 2

[5]	GSM 03.60 version 6.3.1 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Service description; Stage 2

[6]	GSM 04.07 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface signalling layer 3; General aspects

[7]	GSM 04.64 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station - Serving GPRS Support Node (MS-SGSN)
Logical Link Control (LLC) layer specification

[8]	GSM 05.08 version 6.4.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem link control

[9]	GSM 05.10 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem synchronization

[10]	GSM 03.20 TS 100 929: July 1998 (GSM 03.20 version 6.0.1)
Security related network functions, ETSI

[11]	Draft GSM 03.22: August 1998 (GSM 03.22 version 6.1.0)
Functions related to Mobile Station (MS) in idle mode and group receive mode, ETSI

[12]	GSM 04.65 V6.3.0: Subnetwork Dependant Convergence Protocol

ETSI, March 1999

[13]	ITU-T V42bis ITU-T, Recommendation V.42 bis 1990

[14]	GSM 09.60 GPRS Tunneling Protocol (GTP) across the Gn and Gp Interface

[15]	RFC 1661 IETF STD 51 July 1994
The Point-to-Point Protocol (PPP)

[16]	RFC 1662 IETF STD 51 July 1994
PPP in HDLC-like Framing

[17]	RFC 1570 January 1994
PPP LCP Extensions

[18]	RFC 1989 August 1996
PPP Link Quality Monitoring

[19]	RFC 1332 May 1992
The PPP Internet Protocol Control Protocol (IPCP)

[20]	RFC 1877 December 1995
PPP IPCP Extensions for Name Server Addresses

[21]	RFC 2153 May 1997
PPP Vendor Extensions

[22]	RFC 1334 October 1992
PPP Authentication Protocols (for Password Authentication Protocol only)

[23]	RFC 1994 August 1996
PPP Challenge Handshake Authentication Protocol (CHAP)

[24]	TIA/EIA-136-370
Packet-Data Services – Enhanced General Packet Radio for TIA/EIA-136 (EGPRS-136) - Overview, Telecommunications Industry Association

[25]	TIA/EIA-136-376
Packet-Data Services – EGPRS-136 Mobility Management, Telecommunications Industry Association

[26]	TIA/EIA-136-972
Packet-Data Services – Stage 2 Description, Telecommunications Industry Association

## Abbreviations

ACI	Application Control Interface
AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CCI	Compression and Ciphering Interface
CHAP	Challenge Handshake Authentication Protocol
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DCOMP	Identifier of the user data compression algorithm used for the N-DPU
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

E	Extension bit
EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GACI	GPRS Application Control Interface
GMM	GPRS Mobility Management
GP	Guard Period
GRR	GPRS RR
GSM	Global System for Mobile Communication

HDLC	High-level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
IP	Internet Protocol
IPCP	Internet Protocol Control Protocol 
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Ciphering Key

L	Length Indicator
LAI	Location Area Information
LCP 	Link Control Protocol 
LISR	Low level Interrupt Service Routine
LLC	Logical Link Control
LPD	Link Protocol Discriminator
LQM	Link Quality Monitoring

M	More bit used to indicate the last segment of N-DPU
MAC	Medium Access Control
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MT	Mobile Termination

N(R)	Receive Number
N(S)	Send Number
NC	Network Control
NCC	National Colour Code
NCP 	Network Control Protocol
NECI	New Establishment Causes included
N-PDU	Network Protocol Data Unit
NSAPI	Network Layer Service Access Point Identifier

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PACCH	Packet Associated Control Channel
PAP	Password Authentication Protocol
PBCCH	Packet BCCH
PCCCH	Packet CCCH
PCOMP	Identifier of the protocol control information compression algorithm used for the N-DPU
PDCH	Packet Data Channel
PDP	Packet Data Protocol e.g. IP or X.25
PDTCH	Packet Data Traffic Channel
PRACH	Packet RACH
PSI	Packet System Information
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Data Unit
PL	Physical Layer
PLMN	Public Land Mobile Network
PPC	Packet Physical Convergence
PPP 	Point-to-Point Protocol
PTP	Point to Point

QoS	Quality of Service

RACH	Random Access Channel
REJ	Reject Frame
RLC	Radio Link Control
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Stand alone Dedicated Control Channel
SDU	Service Data Unit
SGSN	Serving GPRS Support Node
SIM	Subscriber Identity Module
SM	Session Management
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SNDCP	Subnetwork Dependant Convergence Protocol
SNSM	SNDCP-SM
SS	Supplementary Services

TAP	Test Application Program
TBF	Temporary Block Flow
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TCP	Transmission Control Protocol
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TFI	Temporary Flow Identifier
TLLI	Temporary Logical Link Identifier
TMSI	Temporary Mobile Subscriber Identity
TOM	Tunnelling of Messages
TQI	Temporary Queuing Identifier

UA	Unnumbered Acknowledgement Frame
UART	Universal Asynchronous Receiver Transmitter
UI	Unnumbered Information Frame
USF	Uplink State Flag

V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable
VPLMN	Visited Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for GPRS consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity.

Figure 2-1: Architecture of the GSM/GPRS protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the GPRS protocol stack are:

## GRR (RLC/MAC) – Radio Link Control/Medium Access Control

This layer contains two functions: The Radio Link Control function provides a radio-solution-dependent reliable link. The Medium Access Control function controls the access signalling (request and grant) procedures for the radio channel, and the mapping of LLC frames onto the GSM physical channel.

## LLC – Logical Link Control

The LLC entity provides multiple highly reliable logical links for asynchronous data transfer between the MS and the network. It supports variable-length information frames, acknowledged and unacknowledged data transfer, flow and sequence control, error detection and recovery, notification of unrecoverable errors, user identity confidentiality, and ciphering of user and signaling data.

## GMM – GPRS Mobility Management

The GMM entity provides procedures for the mobility of the MS, such as informing the network of its present location, and user identity confidentiality. It manages the GMM context (attach, detach, routing area updating), supports security functions such as authentication of user and MS, controls ciphering of data, and initiates the response to paging messages.

## SM – Session Management

The main function of the session management (SM) is to support PDP context handling of the user terminal. Session Management activates, modifies and deletes the contexts for packet data protocols (PDP). Session Management services are provided at the SMREG-SAP and the SNSM-SAP for anonymous and non-anonymous access. The non-anonymous and anonymous access procedures for PDP context activation and PDP context deactivation are available at the SMREG-SAP. In addition there exists a PDP context modification for non-anonymous PDP contexts.

## SNDCP - Subnetwork Dependant Convergence Protocol

- SNDCP carries out all functions related to transfer of Network layer Protocol Data Units (N-PDUs) over GPRS in a transparent way. SNDCP helps to improve channel efficiency by means of compression techniques. The set of protocol entities above SNDCP consists of commonly used network protocols. They all use the same SNDCP entity, which then performs multiplexing of data coming from different sources to be sent using the service provided by the LLC layer.

## GACI – GPRS Application Control Interface

The GACI is the GPRS extension of the ACI. It is specified in GSM 07.07 and 07.60. It is responsible for processing of the GPRS related AT Commands to setup, activate and deactivate the PDP context parameter. It also provides functionality for the interworking between GMM/SM/SNDCP and a packet oriented protocol like PPP.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the GPRS. The driver has to be able to perform flow control.

## TOM – Tunnelling of Messages

The TOM entity is present if and only if HS136 is supported (the feature flag FF_HS136 is enabled).

The main function of TOM is to tunnel non-GSM signalling messages between the MS and the SGSN. The only non-GSM signalling which is currently supported by TOM is for the EGPRS-136 system (according to TIA/EIA-136-376). Data transfer in both uplink and downlink direction is possible. Two different priorities (high, low) of signalling data transfer are supported. TOM uses the unacknowledged mode of LLC and the acknowledged mode of GRR (RLC/MAC).

# Parameters

/*

## Primitive elements

*/

BYTE DTI_CHANNEL_TO_LOWER_LAYER	1

BYTE DTI_CHANNEL_TO_HIGHER_LAYER	0

LONG    LINK_ID_PEER		25		/* DTI Link Id in peer direction */

LONG    LINK_ID_PROT		300		/* DTI Link Id in protocol direction */

SHORT	MSID_DUMMY		0		/* max slot identifier dummy*/

SHORT	PPP_MRU_2000		2000		/* maximum receive unit 2000 */

LONG	IP_ADDR		0xc6a80105	/* IP Address 198.168.1.2*/

LONG	PPP_ACCM_0		0		/* Async-Control-Character-Map without mapping */

BYTE  MSID_EXPECTED		15		/* expected msid in PPP_PDP_ACTIVATE_IND primitive */

LONG  PPP_IP_141_64_21_2		0x8d401502	/* IP-Address 141.64.21.2 */

LONG  PPP_PDNS_141_64_24_129		0x8d401881	/* primary DNS-Address 141.64.24.129 */

LONG  PPP_SDNS_141_64_254_131	0x8d40fe83	/* secondary DNS-Address 141.64.254.131 */

LONG  PPP_PDNS_DYNAMIC	0x0		/* dynamic primary DNS-Address */

LONG  PPP_SDNS_DYNAMIC	0x0		/* dynamic secondary DNS-Address */

SHORT PPP_TERM_INSUF_RES	257		/* error cause from SNDCP */

/*

## Struct elements

*/

BYTE LOGIN_NAME_LEN		8					/* length of login name */

BYTE LOGIN_PASSWORD_LEN	8					/* length of login password */

/*

## Fields

*/

/*

### LCP Configure Request

*/

/* HDLC Frame: LCP Configure Request (no options, Identifier = 1)

| 0xc8, 0x00 |  | SDU length in bit (25 Byte = 200 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset in bit |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x08 | 0x7d, 0x20, 0x7d, 0x28 | Length (8 Byte) |
| 0x07, 0x02 | 0x7d, 0x27, 0x7d, 0x22 | PFC |
| 0x08, 0x02 | 0x7d, 0x28, 0x7d, 0x22 | ACFC |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest1)	0xc8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x28,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest1, 29)

/* HDLC Frame: LCP Configure Request (no options, Identifier = 3)

| 0xc8, 0x00 |  | SDU length in bit (25 Byte = 200 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset in bit |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x03 | 0x7d, 0x23 | Identifier |
| 0x00, 0x08 | 0x7d, 0x20, 0x7d, 0x28 | Length (8 Byte) |
| 0x07, 0x02 | 0x7d, 0x27, 0x7d, 0x22 | PFC |
| 0x08, 0x02 | 0x7d, 0x28, 0x7d, 0x22 | ACFC |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest3)	0xc8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x23,

0x7d, 0x20, 0x7d, 0x28,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest3, 29)

/* HDLC Frame: LCP Configure Request (AuthProtocol = PAP, Identifier=4)

| 0xb8, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x04 | 0x7d, 0x24 | Identifier |
| 0x00, 0x08 | 0x7d, 0x20, 0x7d, 0x28 | Length |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0xc0, 0x23 | 0xc0, 0x23 | Option Value (PAP) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest4)	0xb8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x24,

0x7d, 0x20, 0x7d, 0x28,

0x7d, 0x23,

0x7d, 0x24,

0xc0, 0x23,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest4, 27)

/* HDLC Frame: LCP Configure Request (AuthProtocol = PAP, Address-and-Control-Field-Compression, Protocol-Field-Compression, Identifier=6)

| 0xf8, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x06 | 0x7d, 0x26 | Identifier |
| 0x00, 0x0c | 0x7d, 0x20, 0x7d, 0x2c | Length |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0xc0, 0x23 | 0xc0, 0x23 | Option Value (PAP) |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest6)	0xf8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x2c,

0x7d, 0x23,

0x7d, 0x24,

0xc0, 0x23,

0x7d, 0x28,

0x7d, 0x22,

0x7d, 0x27,

0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest6, 35)

/* HDLC Frame: LCP Configure Request (AuthProtocol = PAP, Address-and-Control-Field-Compression, Protocol-Field-Compression, Identifier=1, accm = 0, magic number = 0xa11cfc96)

| 0xa0, 0x01 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x18 | 0x7d, 0x20, 0x7d, 0x38 | Length |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d,0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0xc0, 0x23 | 0xc0, 0x23 | Option Value (PAP) |
| 0x05 | 0x7d, 0x25 | Option Type (magic number) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0xa1, 0x1c, 0xfc, 0x96 | 0xa1, 0x7d, 0x3c, 0xfc, 0x96 | Option Value (magic number) |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequestMag)	0xa0, 0x01,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x38,

0x7d, 0x22,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x23,

0x7d, 0x24,

0xc0, 0x23,

0x7d, 0x25,

0x7d, 0x26,

0xa1, 0x7d, 0x3c, 0xfc, 0x96,

0x7d, 0x27,

0x7d, 0x22,

0x7d, 0x28,

0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequestMag, 56)

/* HDLC Frame: LCP Configure Request (AuthProtocol = PAP, Address-and-Control-Field-Compression, Protocol-Field-Compression, Identifier=2, accm = 0)

| 0x58, 0x01 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x01 | 0x7d, 0x22 | Identifier |
| 0x00, 0x12 | 0x7d, 0x20, 0x7d, 0x38 | Length |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d,0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0xc0, 0x23 | 0xc0, 0x23 | Option Value (PAP) |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequestAccm)	0x58, 0x01,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x32,

0x7d, 0x22,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x23,

0x7d, 0x24,

0xc0, 0x23,

0x7d, 0x27,

0x7d, 0x22,

0x7d, 0x28,

0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequestAccm, 47)

/* HDLC Frame: LCP Configure Request (AuthProtocol = CHAP, Address-and-Control-Field-Compression, Protocol-Field-Compression, Identifier=1)

| 0x08, 0x01 |  | SDU length (33 Byte = 264 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0d | 0x7d, 0x20, 0x7d, 0x2d | Length: 13 |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x05 | 0x7d, 0x25 | Option Length |
| 0xc2, 0x23 | 0xc2, 0x23 | Option Value (CHAP) |
| 0x05 | 0x7d, 0x25 | Algorithm MD5 |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest50)

0x08, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2d,

0x7d, 0x23, 0x7d, 0x25, 0xc2, 0x23, 0x7d, 0x25,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest50, 37)

/* HDLC Frame: LCP Configure Request (ACCM, Magic, ACFC, PFC, Callback, MMRRU, MED, Identifier=0)

| 0xf0, 0x02 |  | SDU length (94 Byte = 752 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x00 | 0x7d, 0x20 | Identifier |
| 0x00, 0x32 | 0x7d, 0x20, 0x32 | Length: 50 |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0x05 | 0x7d, 0x25 | Option Type (Magic Number) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x54, 0x13 | 0x7d, 0x20, 0x7d, 0x20, 0x54, 0x7d, 0x33 | Option Value |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x0d | 0x7d, 0x2d | Option Type (Callback) |
| 0x03 | 0x7d, 0x23 | Option Length |
| 0x06 | 0x7d, 0x26 | Option Value |
| 0x11 | 0x7d, 0x31 | Option Type (Multilink-MRRU) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0x06, 0x4e | 0x7d, 0x26, 4e | Option Value |
| 0x13 | 0x7d, 0x33 | Option Type (Multilink-Endpoint -Discriminator) |
| 0x17 | 0x7d, 0x37 | Option Length (23) |
| 0x01, 0xad, 0xac, 0x80, 0xc0, 0x9b, 0x5e, 0x11, 0xd4, 0x94, 0x16, 0x00, 0x20, 0x18, 0x38, 0xd7, 0x67, 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x21, 0xad, 0xac, 0x80, 0xc0, 0x9b, 0x5e, 0x7d, 0x31, 0xd4, 0x94, 0x7d, 0x36, 0x7d, 0x20, 0x20, 0x7d, 0x38, 0x38, 0xd7, 0x67, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest51)

0xf0, 0x02,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x20,

0x7d, 0x20, 0x32,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x25, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x54, 0x7d, 0x33,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0x7d, 0x2d, 0x7d, 0x23, 0x7d, 0x26,

0x7d, 0x31, 0x7d, 0x24, 0x7d, 0x26, 0x4e,

0x7d, 0x33, 0x7d, 0x37, 0x7d, 0x21, 0xad, 0xac, 0x80, 0xc0, 0x9b, 0x5e,

0x7d, 0x31, 0xd4, 0x94, 0x7d, 0x36, 0x7d, 0x20, 0x20, 0x7d, 0x38, 0x38,

0xd7, 0x67, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest51, 98)

/* HDLC Frame: LCP Configure Request (ACCM, ACFC, PFC, Identifier=1)

| 0x28, 0x01 |  | SDU length (37 Byte = 296 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0e | 0x7d, 0x20, 0x7d, 0x2e | Length: 14 |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest53)

0x28, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2e,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest53, 41)

/* HDLC Frame: Configure Request (LCP, PAP, ACFC, PFC, Identifier=1)

| 0xf8, 0x00 |  | SDU length (31 Byte = 248 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0c | 0x7d, 0x20, 0x7d, 0x2c | Length: 12 |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0xc0, 0x23 | 0xc0, 0x23 | Option Value (PAP) |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest66)

0xf8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2c,

0x7d, 0x23, 0x7d, 0x24, 0xc0, 0x23,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest66, 35)

/* HDLC Frame: Configure Request (LCP, ACFC, PFC, Identifier=1)

| 0xc8, 0x00 |  | SDU length (25 Byte = 200 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x08 | 0x7d, 0x20, 0x7d, 0x28 | Length: 8 |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest68)

0xc8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x28,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest68, 29)

/* HDLC Frame: Configure Request (LCP, CHAP, ACFC, PFC, ACCM, Identifier=1)

| 0x68, 0x01 |  | SDU length (45 Byte = 360 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x13 | 0x7d, 0x20, 0x7d, 0x33 | Length: 19 |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x05 | 0x7d, 0x25 | Option Length |
| 0xc2, 0x23 | 0xc2, 0x23 | Option Value (CHAP) |
| 0x05 | 0x7d, 0x25 | Algorithm MD5 |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest74)

0x68, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x33,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x23, 0x7d, 0x25, 0xc2, 0x23, 0x7d, 0x25,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest74, 49)

/* HDLC Frame: Configure Request (LCP, PAP, ACFC, Identifier=2)

| 0xd8, 0x00 |  | SDU length (27 Byte = 216 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length: 10 |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0xc0, 0x23 | 0xc0, 0x23 | Option Value (PAP) |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest76)

0xd8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23, 0x7d, 0x24, 0xc0, 0x23,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest76, 31)

/* HDLC Frame: Configure Request (LCP, PAP, PFC, Identifier=2)

| 0xd8, 0x00 |  | SDU length (27 Byte = 216 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length: 10 |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0xc0, 0x23 | 0xc0, 0x23 | Option Value (PAP) |
| 0x07 | 0x7d, 0x27 | Option Type (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest78)

0xd8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23, 0x7d, 0x24, 0xc0, 0x23,

0x7d, 0x27, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest78, 31)

/* HDLC Frame: Configure Request (LCP, PAP, ACFC, PFC, Identifier=3)

| 0xf8, 0x00 |  | SDU length (31 Byte = 248 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x03 | 0x7d, 0x23 | Identifier |
| 0x00, 0x0c | 0x7d, 0x20, 0x7d, 0x2c | Length: 12 |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0xc0, 0x23 | 0xc0, 0x23 | Option Value (PAP) |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest80)

0xf8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x23,

0x7d, 0x20, 0x7d, 0x2c,

0x7d, 0x23, 0x7d, 0x24, 0xc0, 0x23,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest80, 35)

/* HDLC Frame: Configure Request (LCP, CHAP, ACFC, PFC, ACCM, MRU, Identifier=1)

| 0xa0, 0x01 |  | SDU length (52 Byte = 416 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x17 | 0x7d, 0x20, 0x7d, 0x37 | Length: 23 |
| 0x01 | 0x7d, 0x21 | Option Type (MRU) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0x07, 0xd0 | 0x7d, 0x27, 0xd0 | Option Value (2000) |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x05 | 0x7d, 0x25 | Option Length |
| 0xc2, 0x23 | 0xc2, 0x23 | Option Value (CHAP) |
| 0x05 | 0x7d, 0x25 | Algorithm MD5 |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest82)

0xa0, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x37,

0x7d, 0x21, 0x7d, 0x24, 0x7d, 0x27, 0xd0,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x23, 0x7d, 0x25, 0xc2, 0x23, 0x7d, 0x25,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest82, 56)

/* HDLC Frame: Configure Request (LCP, CHAP, ACFC, PFC, ACCM, Identifier=2)

| 0x68, 0x01 |  | SDU length (45 Byte = 360 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x13 | 0x7d, 0x20, 0x7d, 0x33 | Length: 19 |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x05 | 0x7d, 0x25 | Option Length |
| 0xc2, 0x23 | 0xc2, 0x23 | Option Value (CHAP) |
| 0x05 | 0x7d, 0x25 | Algorithm MD5 |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest84)

0x68, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x33,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x23, 0x7d, 0x25, 0xc2, 0x23, 0x7d, 0x25,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest84, 49)

/* HDLC Frame: Configure Request (LCP, CHAP, ACFC, PFC, MRU, Identifier=2)

| 0x40, 0x01 |  | SDU length (40 Byte = 320 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x11 | 0x7d, 0x20, 0x7d, 0x31 | Length: 17 |
| 0x01 | 0x7d, 0x21 | Option Type (MRU) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0x07, 0xd0 | 0x7d, 0x27, 0xd0 | Option Value (2000) |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x05 | 0x7d, 0x25 | Option Length |
| 0xc2, 0x23 | 0xc2, 0x23 | Option Value (CHAP) |
| 0x05 | 0x7d, 0x25 | Algorithm MD5 |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest86)

0x40, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x31,

0x7d, 0x21, 0x7d, 0x24, 0x7d, 0x27, 0xd0,

0x7d, 0x23, 0x7d, 0x25, 0xc2, 0x23, 0x7d, 0x25,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest86, 44)

/* HDLC Frame: Configure Request (LCP, CHAP, ACFC, PFC, ACCM, MRU, Identifier=3)

| 0xa0, 0x01 |  | SDU length (52 Byte = 416 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x03 | 0x7d, 0x23 | Identifier |
| 0x00, 0x17 | 0x7d, 0x20, 0x7d, 0x37 | Length: 23 |
| 0x01 | 0x7d, 0x21 | Option Type (MRU) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0x07, 0xd0 | 0x7d, 0x27, 0xd0 | Option Value (2000) |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x05 | 0x7d, 0x25 | Option Length |
| 0xc2, 0x23 | 0xc2, 0x23 | Option Value (CHAP) |
| 0x05 | 0x7d, 0x25 | Algorithm MD5 |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest88)

0xa0, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x23,

0x7d, 0x20, 0x7d, 0x37,

0x7d, 0x21, 0x7d, 0x24, 0x7d, 0x27, 0xd0,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x23, 0x7d, 0x25, 0xc2, 0x23, 0x7d, 0x25,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest88, 56)

/* HDLC Frame: Configure Request (LCP, ACFC, PFC, ACCM, MRU, Identifier=2)

| 0x60, 0x01 |  | SDU length (44 Byte = 352 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x01 | 0x7d, 0x21 | LCP Code (Configure Request) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x12 | 0x7d, 0x20, 0x7d, 0x32 | Length: 18 |
| 0x01 | 0x7d, 0x21 | Option Type (MRU) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0x07, 0xd0 | 0x7d, 0x27, 0xd0 | Option Value (2000) |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfRequest90)

0x60, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x21,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x32,

0x7d, 0x21, 0x7d, 0x24, 0x7d, 0x27, 0xd0,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfRequest90, 48)

/*

### LCP Configure Ack

*/

/* HDLC Frame: LCP Conf Ack (Identifier = 1)

| 0xc8, 0x00 |  | SDU length in bit (25 Byte = 200 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset in bit |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x02 | 0x7d, 0x22 | LCP Code (Configure Ack) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x08 | 0x7d, 0x20, 0x7d, 0x28 | Length (8 Byte) |
| 0x07, 0x02 | 0x7d, 0x27, 0x7d, 0x22 | PFC |
| 0x08, 0x02 | 0x7d, 0x28, 0x7d, 0x22 | ACFC |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfAck1)	0xc8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x22,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x28,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfAck1, 29)

/* HDLC Frame: LCP Conf Ack (Identifier = 3)

| 0xc8, 0x00 |  | SDU length in bit (25 Byte = 200 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset in bit |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x02 | 0x7d, 0x22 | LCP Code (Configure Ack) |
| 0x03 | 0x7d, 0x23 | Identifier |
| 0x00, 0x08 | 0x7d, 0x20, 0x7d, 0x28 | Length (8 Byte) |
| 0x07, 0x02 | 0x7d, 0x27, 0x7d, 0x22 | PFC |
| 0x08, 0x02 | 0x7d, 0x28, 0x7d, 0x22 | ACFC |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfAck3)	0xc8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x22,

0x7d, 0x23,

0x7d, 0x20, 0x7d, 0x28,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfAck3, 29)

/* HDLC Frame: LCP Configure Ack (Identifier = 4)

| 0xb8, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x02 | 0x7d, 0x22 | LCP Code (Configure Ack) |
| 0x04 | 0x7d, 0x24 | Identifier |
| 0x00, 0x08 | 0x7d, 0x20, 0x7d, 0x28 | Length |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0xc0, 0x23 | 0xc0, 0x23 | Option Value (PAP) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfAck4)	0xb8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x22,

0x7d, 0x24,

0x7d, 0x20, 0x7d, 0x28,

0x7d, 0x23,

0x7d, 0x24,

0xc0, 0x23,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfAck4, 27)

/* HDLC Frame: LCP Configure Ack (AuthProtocol = PAP, Address-and-Control-Field-Compression, Protocol-Field-Compression ,Identifier=6)

| 0xf8, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x02 | 0x7d, 0x22 | LCP Code (Configure Ack) |
| 0x06 | 0x7d, 0x26 | Identifier |
| 0x00, 0x0c | 0x7d, 0x20, 0x7d, 0x2c | Length |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0xc0, 0x23 | 0xc0, 0x23 | Option Value (PAP) |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfAck6)	0xf8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x22,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x2c,

0x7d, 0x23,

0x7d, 0x24,

0xc0, 0x23,

0x7d, 0x28,

0x7d, 0x22,

0x7d, 0x27,

0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfAck6, 35)

/* HDLC Frame: LCP Configure Ack (AuthProtocol = PAP, Address-and-Control-Field-Compression, Protocol-Field-Compression, Identifier=2, accm = 0)

| 0x58, 0x01 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x02 | 0x7d, 0x22 | LCP Code (Configure Acknowledge) |
| 0x01 | 0x7d, 0x22 | Identifier |
| 0x00, 0x12 | 0x7d, 0x20, 0x7d, 0x38 | Length |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d,0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0xc0, 0x23 | 0xc0, 0x23 | Option Value (PAP) |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfAccAccm)	0x58, 0x01,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x22,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x32,

0x7d, 0x22,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x23,

0x7d, 0x24,

0xc0, 0x23,

0x7d, 0x27,

0x7d, 0x22,

0x7d, 0x28,

0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfAccAccm, 47)

/* HDLC Frame: LCP Configure Request (AuthProtocol = CHAP, Address-and-Control-Field-Compression, Protocol-Field-Compression, Identifier=1)

| 0x08, 0x01 |  | SDU length (33 Byte = 264 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x02 | 0x7d, 0x22 | LCP Code (Configure Ack) |
| 0x01 | 0x7d, 0x22 | Identifier |
| 0x00, 0x0d | 0x7d, 0x20, 0x7d, 0x2d | Length: 13 |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x05 | 0x7d, 0x25 | Option Length |
| 0xc2, 0x23 | 0xc2, 0x23 | Option Value (CHAP) |
| 0x05 | 0x7d, 0x25 | Algorithm MD5 |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfAck50)

0x08, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x22,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2d,

0x7d, 0x23, 0x7d, 0x25, 0xc2, 0x23, 0x7d, 0x25,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfAck50, 37)

/* HDLC Frame: LCP Configure Ack (ACCM, ACFC, PFC, Identifier=1)

| 0x28, 0x01 |  | SDU length (37 Byte = 296 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x02 | 0x7d, 0x22 | LCP Code (Configure Ack) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0e | 0x7d, 0x20, 0x7d, 0x2e | Length: 14 |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfAck53)

0x28, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x22,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2e,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfAck53, 41)

/* HDLC Frame: Configure Ack (LCP, PAP, ACFC, PFC, Identifier=1)

| 0xf8, 0x00 |  | SDU length (31 Byte = 248 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x02 | 0x7d, 0x22 | LCP Code (Configure Ack) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0c | 0x7d, 0x20, 0x7d, 0x2c | Length: 12 |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0xc0, 0x23 | 0xc0, 0x23 | Option Value (PAP) |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfAck66)

0xf8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x22,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2c,

0x7d, 0x23, 0x7d, 0x24, 0xc0, 0x23,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfAck66, 35)

/* HDLC Frame: Configure Ack (LCP, ACFC, PFC, Identifier=1)

| 0xc8, 0x00 |  | SDU length (25 Byte = 200 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x02 | 0x7d, 0x22 | LCP Code (Configure Ack) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x08 | 0x7d, 0x20, 0x7d, 0x28 | Length: 8 |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfAck68)

0xc8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x22,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x28,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfAck68, 29)

/* HDLC Frame: Configure Ack (LCP, CHAP, ACFC, PFC, ACCM, Identifier=1)

| 0x68, 0x01 |  | SDU length (45 Byte = 360 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x02 | 0x7d, 0x22 | LCP Code (Configure Ack) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x13 | 0x7d, 0x20, 0x7d, 0x33 | Length: 19 |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x05 | 0x7d, 0x25 | Option Length |
| 0xc2, 0x23 | 0xc2, 0x23 | Option Value (CHAP) |
| 0x05 | 0x7d, 0x25 | Algorithm MD5 |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfAck74)

0x68, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x22,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x33,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x23, 0x7d, 0x25, 0xc2, 0x23, 0x7d, 0x25,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfAck74, 49)

/* HDLC Frame: Configure Ack (LCP, PAP, ACFC, PFC, Identifier=3)

| 0xf8, 0x00 |  | SDU length (31 Byte = 248 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x02 | 0x7d, 0x22 | LCP Code (Configure Ack) |
| 0x03 | 0x7d, 0x23 | Identifier |
| 0x00, 0x0c | 0x7d, 0x20, 0x7d, 0x2c | Length: 12 |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0xc0, 0x23 | 0xc0, 0x23 | Option Value (PAP) |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfAck80)

0xf8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x22,

0x7d, 0x23,

0x7d, 0x20, 0x7d, 0x2c,

0x7d, 0x23, 0x7d, 0x24, 0xc0, 0x23,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfAck80, 35)

/* HDLC Frame: Configure Ack (LCP, CHAP, ACFC, PFC, ACCM, MRU, Identifier=3)

| 0xa0, 0x01 |  | SDU length (52 Byte = 416 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x02 | 0x7d, 0x22 | LCP Code (Configure Ack) |
| 0x03 | 0x7d, 0x23 | Identifier |
| 0x00, 0x17 | 0x7d, 0x20, 0x7d, 0x37 | Length: 23 |
| 0x01 | 0x7d, 0x21 | Option Type (MRU) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0x07, 0xd0 | 0x7d, 0x27, 0xd0 | Option Value (2000) |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0x03 | 0x7d, 0x23 | Option Type (Auth Proto) |
| 0x05 | 0x7d, 0x25 | Option Length |
| 0xc2, 0x23 | 0xc2, 0x23 | Option Value (CHAP) |
| 0x05 | 0x7d, 0x25 | Algorithm MD5 |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0x08 | 0x7d, 0x28 | Option Type (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfAck88)

0xa0, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x22,

0x7d, 0x23,

0x7d, 0x20, 0x7d, 0x37,

0x7d, 0x21, 0x7d, 0x24, 0x7d, 0x27, 0xd0,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x23, 0x7d, 0x25, 0xc2, 0x23, 0x7d, 0x25,

0x7d, 0x27, 0x7d, 0x22,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfAck88, 56)

/*

### LCP Configure Reject

*/

/* HDLC Frame: LCP Configure Reject (rejected option: magic number (packet: LCP_ConfRequestMag))

| 0xd0, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x04 | 0x7d, 0x24 | LCP Code (Configure Reject) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length |
| 0x05 | 0x7d, 0x25 | Option Type (magic number) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0xa1, 0x1c, 0xfc, 0x96 | 0xa1, 0x7d, 0x3c, 0xfc, 0x96 | Option Value (magic number) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCPConfigureRejMag)

0xd0, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x24,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x25,

0x7d, 0x26,

0xa1, 0x7d, 0x3c, 0xfc, 0x96,

0xf0, 0xb8,

0x7e

ENDFIELD(LCPConfigureRejMag, 30)

/* HDLC Frame: LCP Configure Reject (Magic, Callback, MMRRU, MED, Identifier=0)

| 0x50, 0x02 |  | SDU length (74 Byte = 592 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x04 | 0x7d, 0x24 | LCP Code (Configure Reject) |
| 0x00 | 0x7d, 0x20 | Identifier |
| 0x00, 0x28 | 0x7d, 0x20, 0x28 | Length: 40 |
| 0x05 | 0x7d, 0x25 | Option Type (Magic Number) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x54, 0x13 | 0x7d, 0x20, 0x7d, 0x20, 0x54, 0x7d, 0x33 | Option Value |
| 0x0d | 0x7d, 0x2d | Option Type (Callback) |
| 0x03 | 0x7d, 0x23 | Option Length |
| 0x06 | 0x7d, 0x26 | Option Value |
| 0x11 | 0x7d, 0x31 | Option Type (Multilink-MRRU) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0x06, 0x4e | 0x7d, 0x26, 4e | Option Value |
| 0x13 | 0x7d, 0x33 | Option Type (Multilink-Endpoint -Discriminator) |
| 0x17 | 0x7d, 0x37 | Option Length (23) |
| 0x01, 0xad, 0xac, 0x80, 0xc0, 0x9b, 0x5e, 0x11, 0xd4, 0x94, 0x16, 0x00, 0x20, 0x18, 0x38, 0xd7, 0x67, 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x21, 0xad, 0xac, 0x80, 0xc0, 0x9b, 0x5e, 0x7d, 0x31, 0xd4, 0x94, 0x7d, 0x36, 0x7d, 0x20, 0x20, 0x7d, 0x38, 0x38, 0xd7, 0x67, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfReject51)

0x50, 0x02,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x24,

0x7d, 0x20,

0x7d, 0x20, 0x28,

0x7d, 0x25, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x54, 0x7d, 0x33,

0x7d, 0x2d, 0x7d, 0x23, 0x7d, 0x26,

0x7d, 0x31, 0x7d, 0x24, 0x7d, 0x26, 0x4e,

0x7d, 0x33, 0x7d, 0x37, 0x7d, 0x21, 0xad, 0xac, 0x80, 0xc0, 0x9b, 0x5e,

0x7d, 0x31, 0xd4, 0x94, 0x7d, 0x36, 0x7d, 0x20, 0x20, 0x7d, 0x38, 0x38,

0xd7, 0x67, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfReject51, 78)

/* HDLC Frame: Configure Reject (LCP, PFC, Identifier=1)

| 0xa8, 0x00 |  | SDU length (21 Byte = 168 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x04 | 0x7d, 0x24 | LCP Code (Configure Reject) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x06 | 0x7d, 0x20, 0x7d, 0x26 | Length: 6 |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfReject66A)

0xa8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x24,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x26,

0x7d, 0x27, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfReject66A, 25)

/* HDLC Frame: Configure Reject (LCP, ACFC, Identifier=1)

| 0xa8, 0x00 |  | SDU length (21 Byte = 168 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x04 | 0x7d, 0x24 | LCP Code (Configure Reject) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x06 | 0x7d, 0x20, 0x7d, 0x26 | Length: 6 |
| 0x08 | 0x7d, 0x28 | Option (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfReject66B)

0xa8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x24,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x26,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfReject66B, 25)

/* HDLC Frame: Configure Reject (LCP, MRU, Identifier=1)

| 0xc0, 0x00 |  | SDU length (24 Byte = 192 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x04 | 0x7d, 0x24 | LCP Code (Configure Reject) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x08 | 0x7d, 0x20, 0x7d, 0x28 | Length: 8 |
| 0x01 | 0x7d, 0x21 | Option Type (MRU) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0x07, 0xd0 | 0x7d, 0x27, 0xd0 | Option Value (2000) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfReject82A)

0xc0, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x24,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x28,

0x7d, 0x21, 0x7d, 0x24, 0x7d, 0x27, 0xd0,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfReject82A, 28)

/* HDLC Frame: Configure Reject (LCP, ACCM, Identifier=1)

| 0xe8, 0x00 |  | SDU length (29 Byte = 232 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x04 | 0x7d, 0x24 | LCP Code (Configure Reject) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length: 10 |
| 0x02 | 0x7d, 0x22 | Option Type (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x0, 0x0, 0x0, 0x0 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (0) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfReject82B)

0xe8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x24,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfReject82B, 33)

/* HDLC Frame: Configure Reject (LCP, AP, Identifier=1)

| 0xc8, 0x00 |  | SDU length (25 Byte = 200 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x04 | 0x7d, 0x24 | LCP Code (Configure Reject) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x09 | 0x7d, 0x20, 0x7d, 0x29 | Length: 9 |
| 0x03 | 0x7d, 0x23 | Option Type (AP) |
| 0x05 | 0x7d, 0x25 | Option Length |
| 0xc2, 0x23, 0x05 | 0xc2, 0x23, 0x7d, 0x25 | Option Value (CHAP) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfReject82C)

0xc8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x24,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x29,

0x7d, 0x23, 0x7d, 0x25, 0xc2, 0x23, 0x7d, 0x25,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfReject82C, 29)

/*

### LCP Configure NAK

*/

/* HDLC Frame: Configure NAK (LCP, PFC, Identifier=2)

| 0xa8, 0x00 |  | SDU length (21 Byte = 168 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x03 | 0x7d, 0x23 | LCP Code (Configure NAK) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x06 | 0x7d, 0x20, 0x7d, 0x26 | Length: 6 |
| 0x07 | 0x7d, 0x27 | Option (PFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfNAK76)

0xa8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x23,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x26,

0x7d, 0x27, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfNAK76, 25)

/* HDLC Frame: Configure NAK (LCP, ACFC, Identifier=2)

| 0xa8, 0x00 |  | SDU length (21 Byte = 168 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x03 | 0x7d, 0x23 | LCP Code (Configure NAK) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x06 | 0x7d, 0x20, 0x7d, 0x26 | Length: 6 |
| 0x08 | 0x7d, 0x28 | Option (ACFC) |
| 0x02 | 0x7d, 0x22 | Option Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfNAK78)

0xa8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x23,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x26,

0x7d, 0x28, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfNAK78, 25)

/* HDLC Frame: Configure NAK (LCP, MRU, Identifier=2)

| 0xc0, 0x00 |  | SDU length (24 Byte = 192 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x03 | 0x7d, 0x23 | LCP Code (Configure NAK) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x08 | 0x7d, 0x20, 0x7d, 0x28 | Length: 8 |
| 0x01 | 0x7d, 0x28 | Option (MRU) |
| 0x04 | 0x7d, 0x24 | Option Length |
| 0x07, 0xd0 | 0x7d, 0x27, 0xd0 | Value (2000) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfNAK84)

0xc0, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x23,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x28,

0x7d, 0x21, 0x7d, 0x24, 0x7d, 0x27, 0xd0,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfNAK84, 28)

/* HDLC Frame: Configure NAK (LCP, ACCM, Identifier=2)

| 0xe8, 0x00 |  | SDU length (29 Byte = 232 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x03 | 0x7d, 0x23 | LCP Code (Configure NAK) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length: 10 |
| 0x02 | 0x7d, 0x28 | Option (ACCM) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Value (0) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfNAK86)

0xe8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x23,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfNAK86, 33)

/* HDLC Frame: Configure NAK (LCP, AP, Identifier=2)

| 0xc8, 0x00 |  | SDU length (25 Byte = 200 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = (LCP) |
| 0x03 | 0x7d, 0x23 | LCP Code (Configure NAK) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x09 | 0x7d, 0x20, 0x7d, 0x29 | Length: 9 |
| 0x03 | 0x7d, 0x23 | Option Type (AP) |
| 0x05 | 0x7d, 0x25 | Option Length |
| 0xc2, 0x23, 0x05 | 0xc2, 0x23, 0x7d, 0x25 | Option Value (CHAP) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ConfNAK90)

0xc8, 0x00,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x23,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x29,

0x7d, 0x23, 0x7d, 0x25, 0xc2, 0x23, 0x7d, 0x25,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ConfNAK90, 29)

/*

### LCP Terminate Request

*/

/* HDLC Frame: Terminate Request (LCP, Identifier = 1, Error-Code = PPP_TERM_OK_MMI)

| 0xa0, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x05 | 0x7d, 0x25 | LCP Code (Terminate Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x06 | 0x7d, 0x20, 0x7d, 0x26 | Length |
| 0xcf, 0x01 | 0xcf, 0x7d, 0x21 | Internal Error Code (PPP_TERM_OK_MMI) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_TermReq0)

0xa0, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x25,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x26,

0xcf, 0x7d, 0x21,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_TermReq0, 24)

/* HDLC Frame: Terminate Request (LCP, Identifier = 1, Error-Code = PPP_TERM_INSUF_RES)

| 0xa8, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x05 | 0x7d, 0x25 | LCP Code (Terminate Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x06 | 0x7d, 0x20, 0x7d, 0x26 | Length |
| 0x01, 0x01 | 0x7d, 0x21, 0x7d, 0x21 | Internal Error Code (PPP_TERM_INSUF_RES) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_TermRequest90)

0xa8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x25,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x26,

0x7d, 0x21, 0x7d, 0x21,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_TermRequest90, 25)

/*

### LCP Terminate Ack

*/

/* HDLC Frame: Terminate Ack (LCP, Identifier = 1, Error-Code = PPP_TERM_OK_MMI)

| 0xa0, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x06 | 0x7d, 0x26 | LCP Code (Terminate Ack) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x06 | 0x7d, 0x20, 0x7d, 0x26 | Length |
| 0xcf, 0x01 | 0xcf, 0x7d, 0x21 | Internal Error Code (== t_cause) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_TermAck0)

0xa0, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x26,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x26,

0xcf, 0x7d, 0x21,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_TermAck0, 24)

/* HDLC Frame: Terminate Ack (LCP, Identifier = 1, Error-Code = PPP_TERM_INSUF_RES)

| 0xa8, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x06 | 0x7d, 0x26 | LCP Code (Terminate Ack) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x06 | 0x7d, 0x20, 0x7d, 0x26 | Length |
| 0x01, 0x01 | 0x7d, 0x21, 0x7d, 0x21 | Internal Error Code (PPP_TERM_INSUF_RES) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_TermAck90)

0xa8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x26,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x26,

0x7d, 0x21, 0x7d, 0x21,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_TermAck90, 25)

/*

### LCP Code Reject

*/

/* HDLC Frame: Code Reject (LCP,  rejection of Identification55, identifier=1)

| 0x58, 0x01 |  | SDU length (43 Bytes = 344 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x07 | 0x7d, 0x27 | LCP Code (Code Reject) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x16 | 0x7d, 0x20, 0x7d, 0x36 | Length: 22 |
| 0x0c | 0x7d, 0x2c | rejected LCP Code (Identification) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x12 | 0x7d, 0x20, 0x7d, 0x32 | Length: 18 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Magic Number |
| 0x4d, 0x53, 0x52, 0x41, 0x53, 0x56, 0x34, 0x2e, 0x30, 0x30 | 0x4d, 0x53, 0x52, 0x41, 0x53, 0x56, 0x34, 0x2e, 0x30, 0x30 | Information "MSRASV4.00" |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_CodeReject55)

0x58, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x27,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x36,

0x7d, 0x2c,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x32,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x4d, 0x53, 0x52, 0x41, 0x53, 0x56, 0x34, 0x2e, 0x30, 0x30,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_CodeReject55, 47)

/* HDLC Frame: Code Reject (LCP, rejection of Identification57, identifier=2)

| 0x60, 0x01 |  | SDU length (44 Byte = 352 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x07 | 0x7d, 0x27 | LCP Code (Identification) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x17 | 0x7d, 0x20, 0x7d, 0x37 | Length: 23 |
| 0x0c | 0x7d, 0x2c | rejected LCP Code (Identification) |
| 0x03 | 0x7d, 0x23 | Identifier |
| 0x00, 0x13 | 0x7d, 0x20, 0x7d, 0x33 | Length: 19 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Magic Number |
| 0x4d, 0x53, 0x52, 0x41, 0x53, 0x2d, 0x31, 0x2d, 0x53, 0x54, 0x57 | 0x4d, 0x53, 0x52, 0x41, 0x53, 0x2d, 0x31, 0x2d, 0x53, 0x54, 0x57 | Information "MSRAS-1-STW" |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_CodeReject57)

0x60, 0x01,

0x00, 0x00,

0x7e,

0xff, 0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x27,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x37,

0x7d, 0x2c,

0x7d, 0x23,

0x7d, 0x20, 0x7d, 0x33,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x4d, 0x53, 0x52, 0x41, 0x53, 0x2d, 0x31, 0x2d, 0x53, 0x54, 0x57,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_CodeReject57, 48)

/*

### LCP Protocol Reject

*/

/* HDLC Frame: LCP Protocol Reject (Identifier = 1, rejected packet is CCPConfReq0)

| 0x38, 0x01 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x08 | 0x7d, 0x28 | LCP Code (Protocol Reject) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x10 | 0x7d, 0x20, 0x7d, 0x30 | Length |
| 0x80, 0xfd | 0x80, 0xfd | Protocol = CCP |
| 0x01 | 0x7d, 0x21 | CCP Code (Conf Req) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length |
| 0x03 | 0x7d, 0x23 | Option Type (OUI) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (dummy) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCPProtoRej0) 	0x38, 0x01,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x28,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x30,

0x80, 0xfd,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(LCPProtoRej0, 43)

/* HDLC Frame: LCP Protocol Reject packet (rejected packet is INVALID_FRAME2)

| 0x30, 0x04 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x08 | 0x7d, 0x28 | LCP Code (Protocol Reject) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x77 | 0x7d, 0x20, 0x77 | Length |
| 0x00 | 0x7d, 0x20 | Rejected packet: |
| 0xf7 | 0xf7 | Address Field (invalid!!!) |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0xa1, 0xa2, 0xa3, …., 0xaf | 0xa1, 0xa2, 0xa3, …., 0xaf | 110 byte data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCPProtoRej1)	0x30, 0x04,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x28,

0x7d, 0x21,

0x7d, 0x20, 0x77,

0x7d, 0x20,

0xf7,

0x7d, 0x23,

0x7d, 0x020, 0x21,

0xa1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xb8,

0x7e

ENDFIELD(LCPProtoRej1, 138)

/* HDLC Frame: Protocol Reject (LCP, Identifier = 1)

| 0x88, 0x0c |  | SDU length (401 Byte = 3208 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x08 | 0x7d, 0x28 | LCP Code (Protocol Reject) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0xdb | 0x7d, 0x20, 0xdb | Length: 219 |
| 0x80, 0xfd | 0x80, 0xfd | Rejected Protocol (CCP) |
| ... | ... | Rejected Packet Content |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ProtoReject59)

0x88, 0x0c,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x28,

0x7d, 0x21,

0x7d, 0x20, 0xdb,

0x80, 0xfd,

0x7d, 0x21,

0x7d, 0x24,

0x7d, 0x20, 0xd5,

0xfe, 0xcb, 0x7d, 0x21, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x74, 0xff,

0xc9, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x68,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0xa0, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x30, 0x30, 0x20, 0x62, 0x7d, 0x25, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0xc0, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0xa4, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0xbc, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x38,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x70, 0x7d, 0x20, 0xea, 0x7d, 0x20,

0xb0, 0xc7, 0x7d, 0x34, 0x7d, 0x20, 0x69, 0xcc, 0xf1, 0x77, 0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x21, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0xe8, 0xfe, 0xc9, 0x7d, 0x20, 0xc8, 0xcd, 0xf1, 0x77, 0x7d,

0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x74, 0x7d, 0x20, 0x6e, 0x7d,

0x20, 0x7d, 0x3d, 0x7d, 0x20, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x21,

0x7d, 0x2b, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0xdb, 0x80, 0xfd, 0x7d, 0x21, 0x7d, 0x24, 0x7d, 0x20, 0xd5,

0xfe, 0xcb, 0x7d, 0x21, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x74, 0xff,

0xc9, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x68,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x32, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x26,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ProtoReject59, 405)

/* HDLC Frame: Protocol Reject (LCP, rejected packet is INVALID_FRAME2, Identifier = 2)

| 0x30, 0x04 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x08 | 0x7d, 0x28 | LCP Code (Protocol Reject) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x77 | 0x7d, 0x20, 0x77 | Length |
| 0x00 | 0x7d, 0x20 | Rejected packet: |
| 0xf7 | 0xf7 | Address Field (invalid!!!) |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0xa1, 0xa2, 0xa3, …., 0xaf | 0xa1, 0xa2, 0xa3, …., 0xaf | 110 byte data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ProtoReject88)

0x30, 0x04,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x28,

0x7d, 0x22,

0x7d, 0x20, 0x77,

0x7d, 0x20,

0xf7,

0x7d, 0x23,

0x7d, 0x020, 0x21,

0xa1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ProtoReject88, 138)

/* HDLC Frame: Protocol Reject (LCP, rejected protocol = uncompressed TCP, identifier = 1)

| 0x10, 0x06 |  | SDU length (194 Byte = 1552 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x08 | 0x7d, 0x28 | LCP Code (Protocol Reject) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x9a | 0x7d, 0x20, 0x9a | Length: 154 |
| 0x00, 0x2f | 0x7d, 0x20, 0x2f | Rejected Protocol (UTCP) |
| ... | ... | Rejected Packet Content |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FCS (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ProtoReject_D0)

0x10, 0x06,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x28,

0x7d, 0x21,

0x7d, 0x20, 0x9a,

0x7d, 0x20, 0x2f,

0x45, 0x7d, 0x20, 0x7d, 0x20, 0x94, 0x7d, 0x24, 0x7d, 0x3e, 0x40, 0x7d, 0x20, 0x75, 0x7d, 0x20,

0xd9, 0x26, 0x3e, 0x60, 0xcf, 0x7d, 0x29, 0x97, 0xbd, 0x82, 0xf8, 0x7d, 0x20, 0x50, 0x7d, 0x24,

0x7d, 0x30, 0xe2, 0x75, 0xdd, 0x43, 0x7d, 0x20, 0x7d, 0x3a, 0xbe, 0x78, 0x50, 0x7d, 0x38, 0x21,

0x7d, 0x29, 0x7d, 0x38, 0xaf, 0x7d, 0x20, 0x7d, 0x20, 0x48, 0x54, 0x54, 0x50, 0x2f, 0x31, 0x2e,

0x31, 0x20, 0x32, 0x30, 0x30, 0x20, 0x4f, 0x4b, 0x7d, 0x2d, 0x7d, 0x2a, 0x53, 0x65, 0x72, 0x76,

0x65, 0x72, 0x3a, 0x20, 0x4d, 0x69, 0x63, 0x72, 0x6f, 0x73, 0x6f, 0x66, 0x74, 0x2d, 0x49, 0x49,

0x53, 0x2f, 0x34, 0x2e, 0x30, 0x7d, 0x2d, 0x7d, 0x2a, 0x44, 0x61, 0x74, 0x65, 0x3a, 0x20, 0x46,

0x72, 0x69, 0x2c, 0x20, 0x30, 0x36, 0x20, 0x4f, 0x63, 0x74, 0x20, 0x32, 0x30, 0x30, 0x30, 0x20,

0x31, 0x32, 0x3a, 0x34, 0x35, 0x3a, 0x32, 0x38, 0x20, 0x47, 0x4d, 0x54, 0x7d, 0x2d, 0x7d, 0x2a,

0x43, 0x6f, 0x6e, 0x74, 0x65, 0x6e, 0x74, 0x2d, 0x54, 0x79, 0x70, 0x65, 0x3a, 0x20, 0x74, 0x65,

0x78, 0x74, 0x2f, 0x68, 0x74, 0x6d, 0x6c, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d, 0x2a,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ProtoReject_D0, 198)

/* HDLC Frame: Protocol Reject (LCP, rejected protocol = uncompressed TCP, identifier = 2)

| 0x10, 0x06 |  | SDU length (194 Byte = 1552 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x08 | 0x7d, 0x28 | LCP Code (Protocol Reject) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x9a | 0x7d, 0x20, 0x9a | Length: 154 |
| 0x00, 0x2f | 0x7d, 0x20, 0x2f | Rejected Protocol (UTCP) |
| ... | ... | Rejected Packet Content |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FCS (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ProtoReject_D0B)

0x10, 0x06,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x28,

0x7d, 0x22,

0x7d, 0x20, 0x9a,

0x7d, 0x20, 0x2f,

0x45, 0x7d, 0x20, 0x7d, 0x20, 0x94, 0x7d, 0x24, 0x7d, 0x3e, 0x40, 0x7d, 0x20, 0x75, 0x7d, 0x20,

0xd9, 0x26, 0x3e, 0x60, 0xcf, 0x7d, 0x29, 0x97, 0xbd, 0x82, 0xf8, 0x7d, 0x20, 0x50, 0x7d, 0x24,

0x7d, 0x30, 0xe2, 0x75, 0xdd, 0x43, 0x7d, 0x20, 0x7d, 0x3a, 0xbe, 0x78, 0x50, 0x7d, 0x38, 0x21,

0x7d, 0x29, 0x7d, 0x38, 0xaf, 0x7d, 0x20, 0x7d, 0x20, 0x48, 0x54, 0x54, 0x50, 0x2f, 0x31, 0x2e,

0x31, 0x20, 0x32, 0x30, 0x30, 0x20, 0x4f, 0x4b, 0x7d, 0x2d, 0x7d, 0x2a, 0x53, 0x65, 0x72, 0x76,

0x65, 0x72, 0x3a, 0x20, 0x4d, 0x69, 0x63, 0x72, 0x6f, 0x73, 0x6f, 0x66, 0x74, 0x2d, 0x49, 0x49,

0x53, 0x2f, 0x34, 0x2e, 0x30, 0x7d, 0x2d, 0x7d, 0x2a, 0x44, 0x61, 0x74, 0x65, 0x3a, 0x20, 0x46,

0x72, 0x69, 0x2c, 0x20, 0x30, 0x36, 0x20, 0x4f, 0x63, 0x74, 0x20, 0x32, 0x30, 0x30, 0x30, 0x20,

0x31, 0x32, 0x3a, 0x34, 0x35, 0x3a, 0x32, 0x38, 0x20, 0x47, 0x4d, 0x54, 0x7d, 0x2d, 0x7d, 0x2a,

0x43, 0x6f, 0x6e, 0x74, 0x65, 0x6e, 0x74, 0x2d, 0x54, 0x79, 0x70, 0x65, 0x3a, 0x20, 0x74, 0x65,

0x78, 0x74, 0x2f, 0x68, 0x74, 0x6d, 0x6c, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d, 0x2a,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ProtoReject_D0B, 198)

/* HDLC Frame: Protocol Reject (LCP, rejected protocol = compressed TCP, identifier = 2)

| 0x98, 0x0e |  | SDU length (467 Byte = 3736 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x08 | 0x7d, 0x28 | LCP Code (Protocol Reject) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x01, 0xa5 | 0x7d, 0x21, 0xa5 | Length: 421 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Rejected Protocol (CTCP) |
| ... | ... | Rejected Packet Content |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FCS (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ProtoReject_E0)

0x98, 0x0e,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x28,

0x7d, 0x22,

0x7d, 0x21, 0xa5,

0x7d, 0x20, 0x2d,

0x3f, 0xbf, 0x67, 0x7d, 0x20, 0x7d, 0x21, 0x7d, 0x20, 0x3c, 0x68, 0x74, 0x6d, 0x6c, 0x3e, 0x7d,

0x2d, 0x7d, 0x2a, 0x3c, 0x68, 0x65, 0x61, 0x64, 0x3e, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d,

0x2a, 0x3c, 0x74, 0x69, 0x74, 0x6c, 0x65, 0x3e, 0x43, 0x6f, 0x6e, 0x64, 0x61, 0x74, 0x3c, 0x2f,

0x74, 0x69, 0x74, 0x6c, 0x65, 0x3e, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d,

0x7d, 0x2a, 0x3c, 0x73, 0x63, 0x72, 0x69, 0x70, 0x74, 0x20, 0x6c, 0x61, 0x6e, 0x67, 0x75, 0x61,

0x67, 0x65, 0x3d, 0x22, 0x4a, 0x61, 0x76, 0x61, 0x53, 0x63, 0x72, 0x69, 0x70, 0x74, 0x22, 0x3e,

0x20, 0x3c, 0x21, 0x2d, 0x2d, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d,

0x2a, 0x66, 0x75, 0x6e, 0x63, 0x74, 0x69, 0x6f, 0x6e, 0x20, 0x4d, 0x4d, 0x5f, 0x73, 0x77, 0x61,

0x70, 0x49, 0x6d, 0x61, 0x67, 0x65, 0x28, 0x29, 0x20, 0x7b, 0x20, 0x2f, 0x2f, 0x76, 0x31, 0x2e,

0x30, 0x7d, 0x2d, 0x7d, 0x2a, 0x20, 0x20, 0x76, 0x61, 0x72, 0x20, 0x69, 0x2c, 0x74, 0x68, 0x65,

0x4f, 0x62, 0x6a, 0x2c, 0x6a, 0x3d, 0x30, 0x2c, 0x73, 0x77, 0x61, 0x70, 0x41, 0x72, 0x72, 0x61,

0x79, 0x3d, 0x6e, 0x65, 0x77, 0x20, 0x41, 0x72, 0x72, 0x61, 0x79, 0x2c, 0x6f, 0x6c, 0x64, 0x41,

0x72, 0x72, 0x61, 0x79, 0x3d, 0x64, 0x6f, 0x63, 0x75, 0x6d, 0x65, 0x6e, 0x74, 0x2e, 0x4d, 0x4d,

0x5f, 0x73, 0x77, 0x61, 0x70, 0x49, 0x6d, 0x67, 0x44, 0x61, 0x74, 0x61, 0x3b, 0x7d, 0x2d, 0x7d,

0x2a, 0x20, 0x20, 0x66, 0x6f, 0x72, 0x20, 0x28, 0x69, 0x3d, 0x30, 0x3b, 0x20, 0x69, 0x20, 0x3c,

0x20, 0x28, 0x4d, 0x4d, 0x5f, 0x73, 0x77, 0x61, 0x70, 0x49, 0x6d, 0x61, 0x67, 0x65, 0x2e, 0x61,

0x72, 0x67, 0x75, 0x6d, 0x65, 0x6e, 0x74, 0x73, 0x2e, 0x6c, 0x65, 0x6e, 0x67, 0x74, 0x68, 0x2d,

0x32, 0x29, 0x3b, 0x20, 0x69, 0x2b, 0x3d, 0x33, 0x29, 0x20, 0x7b, 0x7d, 0x2d, 0x7d, 0x2a, 0x20,

0x20, 0x20, 0x20, 0x74, 0x68, 0x65, 0x4f, 0x62, 0x6a, 0x20, 0x3d, 0x20, 0x65, 0x76, 0x61, 0x6c,

0x28, 0x4d, 0x4d, 0x5f, 0x73, 0x77, 0x61, 0x70, 0x49, 0x6d, 0x61, 0x67, 0x65, 0x2e, 0x61, 0x72,

0x67, 0x75, 0x6d, 0x65, 0x6e, 0x74, 0x73, 0x5b, 0x28, 0x6e, 0x61, 0x76, 0x69, 0x67, 0x61, 0x74,

0x6f, 0x72, 0x2e, 0x61, 0x70, 0x70, 0x4e, 0x61, 0x6d, 0x65, 0x20, 0x3d, 0x3d, 0x20, 0x27, 0x4e,

0x65, 0x74, 0x73, 0x63, 0x61, 0x70, 0x65, 0x27, 0x29, 0x3f, 0x69, 0x3a, 0x69, 0x2b, 0x31, 0x5d,

0x29, 0x7d, 0x2d, 0x7d, 0x2a, 0x20, 0x20, 0x20, 0x20, 0x69, 0x66, 0x20, 0x28, 0x74, 0x68, 0x65,

0x4f, 0x62, 0x6a, 0x20, 0x21, 0x3d, 0x20, 0x6e, 0x75, 0x6c, 0x6c, 0x29, 0x20, 0x7b, 0x7d, 0x2d,

0x7d, 0x2a, 0x20, 0x20, 0x20, 0x20, 0x20, 0x20, 0x73, 0x77, 0x61, 0x70, 0x41, 0x72, 0x72, 0x61,

0x79, 0x5b, 0x6a, 0x2b, 0x2b, 0x5d, 0x20, 0x3d, 0x20, 0x74, 0x68, 0x65, 0x4f, 0x62, 0x6a, 0x3b,

0x7d, 0x2d, 0x7d, 0x2a, 0x20, 0x20, 0x20, 0x20, 0x20, 0x20, 0x73, 0x77, 0x61, 0x70, 0x41, 0x72,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ProtoReject_E0, 471)

/* HDLC Frame: Protocol Reject (LCP, rejected protocol = compressed TCP, identifier = 3)

| 0x98, 0x0e |  | SDU length (467 Byte = 3736 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x08 | 0x7d, 0x28 | LCP Code (Protocol Reject) |
| 0x03 | 0x7d, 0x23 | Identifier |
| 0x01, 0xa5 | 0x7d, 0x21, 0xa5 | Length: 421 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Rejected Protocol (CTCP) |
| ... | ... | Rejected Packet Content |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FCS (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_ProtoReject_E0B)

0x98, 0x0e,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x21,

0x7d, 0x28,

0x7d, 0x23,

0x7d, 0x21, 0xa5,

0x7d, 0x20, 0x2d,

0x3f, 0xbf, 0x67, 0x7d, 0x20, 0x7d, 0x21, 0x7d, 0x20, 0x3c, 0x68, 0x74, 0x6d, 0x6c, 0x3e, 0x7d,

0x2d, 0x7d, 0x2a, 0x3c, 0x68, 0x65, 0x61, 0x64, 0x3e, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d,

0x2a, 0x3c, 0x74, 0x69, 0x74, 0x6c, 0x65, 0x3e, 0x43, 0x6f, 0x6e, 0x64, 0x61, 0x74, 0x3c, 0x2f,

0x74, 0x69, 0x74, 0x6c, 0x65, 0x3e, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d,

0x7d, 0x2a, 0x3c, 0x73, 0x63, 0x72, 0x69, 0x70, 0x74, 0x20, 0x6c, 0x61, 0x6e, 0x67, 0x75, 0x61,

0x67, 0x65, 0x3d, 0x22, 0x4a, 0x61, 0x76, 0x61, 0x53, 0x63, 0x72, 0x69, 0x70, 0x74, 0x22, 0x3e,

0x20, 0x3c, 0x21, 0x2d, 0x2d, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d,

0x2a, 0x66, 0x75, 0x6e, 0x63, 0x74, 0x69, 0x6f, 0x6e, 0x20, 0x4d, 0x4d, 0x5f, 0x73, 0x77, 0x61,

0x70, 0x49, 0x6d, 0x61, 0x67, 0x65, 0x28, 0x29, 0x20, 0x7b, 0x20, 0x2f, 0x2f, 0x76, 0x31, 0x2e,

0x30, 0x7d, 0x2d, 0x7d, 0x2a, 0x20, 0x20, 0x76, 0x61, 0x72, 0x20, 0x69, 0x2c, 0x74, 0x68, 0x65,

0x4f, 0x62, 0x6a, 0x2c, 0x6a, 0x3d, 0x30, 0x2c, 0x73, 0x77, 0x61, 0x70, 0x41, 0x72, 0x72, 0x61,

0x79, 0x3d, 0x6e, 0x65, 0x77, 0x20, 0x41, 0x72, 0x72, 0x61, 0x79, 0x2c, 0x6f, 0x6c, 0x64, 0x41,

0x72, 0x72, 0x61, 0x79, 0x3d, 0x64, 0x6f, 0x63, 0x75, 0x6d, 0x65, 0x6e, 0x74, 0x2e, 0x4d, 0x4d,

0x5f, 0x73, 0x77, 0x61, 0x70, 0x49, 0x6d, 0x67, 0x44, 0x61, 0x74, 0x61, 0x3b, 0x7d, 0x2d, 0x7d,

0x2a, 0x20, 0x20, 0x66, 0x6f, 0x72, 0x20, 0x28, 0x69, 0x3d, 0x30, 0x3b, 0x20, 0x69, 0x20, 0x3c,

0x20, 0x28, 0x4d, 0x4d, 0x5f, 0x73, 0x77, 0x61, 0x70, 0x49, 0x6d, 0x61, 0x67, 0x65, 0x2e, 0x61,

0x72, 0x67, 0x75, 0x6d, 0x65, 0x6e, 0x74, 0x73, 0x2e, 0x6c, 0x65, 0x6e, 0x67, 0x74, 0x68, 0x2d,

0x32, 0x29, 0x3b, 0x20, 0x69, 0x2b, 0x3d, 0x33, 0x29, 0x20, 0x7b, 0x7d, 0x2d, 0x7d, 0x2a, 0x20,

0x20, 0x20, 0x20, 0x74, 0x68, 0x65, 0x4f, 0x62, 0x6a, 0x20, 0x3d, 0x20, 0x65, 0x76, 0x61, 0x6c,

0x28, 0x4d, 0x4d, 0x5f, 0x73, 0x77, 0x61, 0x70, 0x49, 0x6d, 0x61, 0x67, 0x65, 0x2e, 0x61, 0x72,

0x67, 0x75, 0x6d, 0x65, 0x6e, 0x74, 0x73, 0x5b, 0x28, 0x6e, 0x61, 0x76, 0x69, 0x67, 0x61, 0x74,

0x6f, 0x72, 0x2e, 0x61, 0x70, 0x70, 0x4e, 0x61, 0x6d, 0x65, 0x20, 0x3d, 0x3d, 0x20, 0x27, 0x4e,

0x65, 0x74, 0x73, 0x63, 0x61, 0x70, 0x65, 0x27, 0x29, 0x3f, 0x69, 0x3a, 0x69, 0x2b, 0x31, 0x5d,

0x29, 0x7d, 0x2d, 0x7d, 0x2a, 0x20, 0x20, 0x20, 0x20, 0x69, 0x66, 0x20, 0x28, 0x74, 0x68, 0x65,

0x4f, 0x62, 0x6a, 0x20, 0x21, 0x3d, 0x20, 0x6e, 0x75, 0x6c, 0x6c, 0x29, 0x20, 0x7b, 0x7d, 0x2d,

0x7d, 0x2a, 0x20, 0x20, 0x20, 0x20, 0x20, 0x20, 0x73, 0x77, 0x61, 0x70, 0x41, 0x72, 0x72, 0x61,

0x79, 0x5b, 0x6a, 0x2b, 0x2b, 0x5d, 0x20, 0x3d, 0x20, 0x74, 0x68, 0x65, 0x4f, 0x62, 0x6a, 0x3b,

0x7d, 0x2d, 0x7d, 0x2a, 0x20, 0x20, 0x20, 0x20, 0x20, 0x20, 0x73, 0x77, 0x61, 0x70, 0x41, 0x72,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_ProtoReject_E0B, 471)

/*

### LCP Identification

*/

/* HDLC Frame: Identification (LCP, ACFC, PFC, Identifier=2)

| 0x00, 0x01 |  | SDU length (32 Bytes = 256 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x0c | 0x7d, 0x2c | LCP Code (Identification) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x12 | 0x7d, 0x20, 0x7d, 0x32 | Length: 18 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Magic Number |
| 0x4d, 0x53, 0x52, 0x41, 0x53, 0x56, 0x34, 0x2e, 0x30, 0x30 | 0x4d, 0x53, 0x52, 0x41, 0x53, 0x56, 0x34, 0x2e, 0x30, 0x30 | Information "MSRASV4.00" |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_Identification55)

0x00, 0x01,

0x00, 0x00,

0x7e,

0xc0, 0x21,

0x7d, 0x2c,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x32,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x4d, 0x53, 0x52, 0x41, 0x53, 0x56, 0x34, 0x2e, 0x30, 0x30,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_Identification55, 36)

/* HDLC Frame: Identification (LCP, ACFC, PFC, Identifier=3)

| 0x08, 0x01 |  | SDU length (33 Byte = 264 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xc0, 0x21 | 0xc0, 0x21 | Protocol = LCP |
| 0x0c | 0x7d, 0x2c | LCP Code (Identification) |
| 0x03 | 0x7d, 0x23 | Identifier |
| 0x00, 0x13 | 0x7d, 0x20, 0x7d, 0x33 | Length: 19 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Magic Number |
| 0x4d, 0x53, 0x52, 0x41, 0x53, 0x2d, 0x31, 0x2d, 0x53, 0x54, 0x57 | 0x4d, 0x53, 0x52, 0x41, 0x53, 0x2d, 0x31, 0x2d, 0x53, 0x54, 0x57 | Information "MSRAS-1-STW" |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(LCP_Identification57)

0x08, 0x01,

0x00, 0x00,

0x7e,

0xc0, 0x21,

0x7d, 0x2c,

0x7d, 0x23,

0x7d, 0x20, 0x7d, 0x33,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x4d, 0x53, 0x52, 0x41, 0x53, 0x2d, 0x31, 0x2d, 0x53, 0x54, 0x57,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_Identification57, 37)

/* HDLC Frame: Identification (LCP, ACFC, PFC, ACCM=0, Identifier=2)

| 0xc0, 0x00 | SDU length (24 Bytes = 192 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0xc0, 0x21 | Protocol = LCP |
| 0x0c | LCP Code (Identification) |
| 0x02 | Identifier |
| 0x00, 0x12 | Length: 18 |
| 0x00, 0x00, 0x00, 0x00 | Magic Number |
| 0x4d, 0x53, 0x52, 0x41, 0x53, 0x56, 0x34, 0x2e, 0x30, 0x30 | Information "MSRASV4.00" |
| 0xf0, 0xb8 | FSC (dummy) |
| 0x7e | Flag |

*/

FIELD(LCP_Identification77)

0xc0, 0x00,

0x00, 0x00,

0x7e,

0xc0, 0x21,

0x0c,

0x02,

0x00, 0x12,

0x00, 0x00, 0x00, 0x00,

0x4d, 0x53, 0x52, 0x41, 0x53, 0x56, 0x34, 0x2e, 0x30, 0x30,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_Identification77, 28)

/* HDLC Frame: Identification (LCP, ACFC, PFC, ACCM=0, Identifier=3)

| 0xc8, 0x00 | SDU length (25 Byte = 200 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0xc0, 0x21 | Protocol = LCP |
| 0x0c | LCP Code (Identification) |
| 0x03 | Identifier |
| 0x00, 0x13 | Length: 19 |
| 0x00, 0x00, 0x00, 0x00 | Magic Number |
| 0x4d, 0x53, 0x52, 0x41, 0x53, 0x2d, 0x31, 0x2d, 0x53, 0x54, 0x57 | Information "MSRAS-1-STW" |
| 0xf0, 0xb8 | FSC (dummy) |
| 0x7e | Flag |

*/

FIELD(LCP_Identification79)

0xc8, 0x00,

0x00, 0x00,

0x7e,

0xc0, 0x21,

0x0c,

0x03,

0x00, 0x13,

0x00, 0x00, 0x00, 0x00,

0x4d, 0x53, 0x52, 0x41, 0x53, 0x2d, 0x31, 0x2d, 0x53, 0x54, 0x57,

0xf0, 0xb8,

0x7e

ENDFIELD(LCP_Identification79, 29)

/*

### CHAP Challenge

*/

/* HDLC Frame: Challenge  (CHAP, ACFC, PFC, Identifier = 1)

| 0x80, 0x01 |  | SDU length (48 Byte = 384 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xc2, 0x23 | 0xc2, 0x23 | Protocol = CHAP |
| 0x01 | 0x7d, 0x21 | CHAP Code (Challenge) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x15 | 0x7d, 0x20, 0x7d, 0x35 | Length: 21 |
| 0x10 | 0x7d, 0x30 | Value-Size: 16 |
| 0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09, 0x0a, 0x0b, 0x0c, 0x0d, 0x0e, 0x0f | 0x7d, 0x20, 0x7d, 0x21, 0x7d, 0x22, 0x7d, 0x23, 0x7d, 0x24, 0x7d, 0x25, 0x7d, 0x26, 0x7d, 0x27, 0x7d, 0x28, 0x7d, 0x29, 0x7d, 0x2a, 0x7d, 0x2b, 0x7d, 0x2c, 0x7d, 0x2d, 0x7d, 0x2e, 0x7d, 0x2f | Value |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FCS (dummy) |
| 0x7e |  | Flag |

*/

FIELD(CHAP_Challenge54)

0x80, 0x01,

0x00, 0x00,

0x7e,

0xc2, 0x23,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x35,

0x7d, 0x30,

0x7d, 0x20, 0x7d, 0x21, 0x7d, 0x22, 0x7d, 0x23, 0x7d, 0x24, 0x7d, 0x25,

0x7d, 0x26, 0x7d, 0x27, 0x7d, 0x28, 0x7d, 0x29, 0x7d, 0x2a, 0x7d, 0x2b,

0x7d, 0x2c, 0x7d, 0x2d, 0x7d, 0x2e, 0x7d, 0x2f,

0xf0, 0xb8,

0x7e

ENDFIELD(CHAP_Challenge54, 52)

/* HDLC Frame: Challenge  (CHAP, ACFC, PFC, ACCM=0, Identifier = 1)

| 0xd8, 0x00 | SDU length (27 Byte = 216 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0xc2, 0x23 | Protocol = CHAP |
| 0x01 | CHAP Code (Challenge) |
| 0x01 | Identifier |
| 0x00, 0x15 | Length: 21 |
| 0x10 | Value-Size: 16 |
| 0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09, 0x0a, 0x0b, 0x0c, 0x0d, 0x0e, 0x0f | Value |
| 0xf0, 0xb8 | FCS (dummy) |
| 0x7e | Flag |

*/

FIELD(CHAP_Challenge76)

0xd8, 0x00,

0x00, 0x00,

0x7e,

0xc2, 0x23,

0x01,

0x01,

0x00, 0x15,

0x10,

0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09, 0x0a, 0x0b, 0x0c, 0x0d, 0x0e, 0x0f,

0xf0, 0xb8,

0x7e

ENDFIELD(CHAP_Challenge76, 31)

/*

### CHAP Response

*/

/* HDLC Frame: Response  (CHAP, ACFC, PFC, Identifier = 1)

| 0x68, 0x01 |  | SDU length (45 Byte = 360 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xc2, 0x23 | 0xc2, 0x23 | Protocol = CHAP |
| 0x02 | 0x7d, 0x22 | CHAP Code (Response) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x1d | 0x7d, 0x20, 0x7d, 0x3d | Length: 29 |
| 0x10 | 0x7d, 0x30 | Value-Size: 16 |
| 0xd1, 0xe3, 0x7f, 0x02, 0x2a, 0x0c, 0xcd, 0x13, 0x85, 0xd4, 0xe3, 0x13, 0x08, 0xc2, 0x2f, 0xd7 | 0xd1, 0xe3, 0x7f, 0x7d, 0x22, 0x2a, 0x7d, 0x2c, 0xcd, 0x7d, 0x33, 0x85, 0xd4, 0xe3, 0x7d, 0x33, 0x7d, 0x28, 0xc2, 0x2f, 0xd7 | Value |
| 0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65 | 0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65 | Name "testname" |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FCS (dummy) |
| 0x7e |  | Flag |

*/

FIELD(CHAP_Response54)

0x68, 0x01,

0x00, 0x00,

0x7e,

0xc2, 0x23,

0x7d, 0x22,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x3d,

0x7d, 0x30,

0xd1, 0xe3, 0x7f, 0x7d, 0x22, 0x2a, 0x7d, 0x2c, 0xcd, 0x7d, 0x33, 0x85,

0xd4, 0xe3, 0x7d, 0x33, 0x7d, 0x28, 0xc2, 0x2f, 0xd7,

0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65,

0xf0, 0xb8,

0x7e

ENDFIELD(CHAP_Response54, 49)

/* HDLC Frame: Response  (CHAP, ACFC, PFC, ACCM=0, Identifier = 1)

| 0x18, 0x01 | SDU length (35 Byte = 280 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0xc2, 0x23 | Protocol = CHAP |
| 0x02 | CHAP Code (Response) |
| 0x01 | Identifier |
| 0x00, 0x1d | Length: 29 |
| 0x10 | Value-Size: 16 |
| 0xd1, 0xe3, 0x7f, 0x02, 0x2a, 0x0c, 0xcd, 0x13, 0x85, 0xd4, 0xe3, 0x13, 0x08, 0xc2, 0x2f, 0xd7 | Value |
| 0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65 | Name "testname" |
| 0xf0, 0xb8 | FCS (dummy) |
| 0x7e | Flag |

*/

FIELD(CHAP_Response76)

0x18, 0x01,

0x00, 0x00,

0x7e,

0xc2, 0x23,

0x02,

0x01,

0x00, 0x1d,

0x10,

0xd1, 0xe3, 0x7f, 0x02, 0x2a, 0x0c, 0xcd, 0x13, 0x85, 0xd4, 0xe3, 0x13, 0x08, 0xc2, 0x2f, 0xd7,

0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65,

0xf0, 0xb8,

0x7e

ENDFIELD(CHAP_Response76, 39)

/*

### CHAP Success

*/

/* HDLC Frame: Success  (CHAP, ACFC, PFC, Identifier = 1)

| 0x70, 0x00 |  | SDU length (14 Byte = 112 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xc2, 0x23 | 0xc2, 0x23 | Protocol = CHAP |
| 0x03 | 0x7d, 0x23 | CHAP Code (Success) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x04 | 0x7d, 0x20, 0x7d, 0x24 | Length: 4 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FCS (dummy) |
| 0x7e |  | Flag |

*/

FIELD(CHAP_Success54)

0x70, 0x00,

0x00, 0x00,

0x7e,

0xc2, 0x23,

0x7d, 0x23,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x24,

0xf0, 0xb8,

0x7e

ENDFIELD(CHAP_Success54, 18)

/* HDLC Frame: Success  (CHAP, ACFC, PFC, ACCM=0, Identifier = 1)

| 0x50, 0x00 | SDU length (10 Byte = 80 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0xc2, 0x23 | Protocol = CHAP |
| 0x03 | CHAP Code (Success) |
| 0x01 | Identifier |
| 0x00, 0x04 | Length: 4 |
| 0xf0, 0xb8 | FCS (dummy) |
| 0x7e | Flag |

*/

FIELD(CHAP_Success76)

0x50, 0x00,

0x00, 0x00,

0x7e,

0xc2, 0x23,

0x03,

0x01,

0x00, 0x04,

0xf0, 0xb8,

0x7e

ENDFIELD(CHAP_Success76, 14)

/*

### PAP Authentication Request

*/

/* HDLC Frame: Authenticate Request  (PAP, Client Mode)

| 0x20, 0x01 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x23 | 0xc0, 0x23 | Protocol = PAP |
| 0x01 | 0x7d, 0x21 | PAP Code (Auth Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x016 | 0x7d, 0x20, 0x7d, 0x36 | Length |
| 0x08 | 0x7d, 0x28 | Peer-ID Length |
| 0x74, 0x65, 0x73, 0x74, 0x6E, 0x61, 0x6D, 0x65 | 0x74, 0x65, 0x73, 0x74, 0x6E, 0x61, 0x6D, 0x65 | Peer-ID “testname” |
| 0x08 | 0x7d, 0x28 | Password Length |
| 0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73 | 0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73 | Password “testpass” |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FCS (dummy) |
| 0x7e |  | Flag |

*/

FIELD(AuthRequest0)	0x28, 0x01,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x23,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x36,

0x7d, 0x28,

0x74, 0x65, 0x73, 0x74, 0x6E, 0x61, 0x6D, 0x65,

0x7d, 0x28,

0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73,

0xf0, 0xb8,

0x7e

ENDFIELD(AuthRequest0, 41)

/* HDLC Frame: Authenticate Request  (PAP, Client Mode, ACFC, PFC)

| 0x10, 0x01 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xc0, 0x23 | 0xc0, 0x23 | Protocol = PAP |
| 0x01 | 0x7d, 0x21 | PAP Code (Auth Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x016 | 0x7d, 0x20, 0x7d, 0x36 | Length |
| 0x08 | 0x7d, 0x28 | Peer-ID Length |
| 0x74, 0x65, 0x73, 0x74, 0x6E, 0x61, 0x6D, 0x65 | 0x74, 0x65, 0x73, 0x74, 0x6E, 0x61, 0x6D, 0x65 | Peer-ID “testname” |
| 0x08 | 0x7d, 0x28 | Password Length |
| 0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73 | 0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73 | Password “testpass” |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(AuthRequest0_C)	0x10, 0x01,

0x00, 0x00,

0x7e,

0xc0, 0x23,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x36,

0x7d, 0x28,

0x74, 0x65, 0x73, 0x74, 0x6E, 0x61, 0x6D, 0x65,

0x7d, 0x28,

0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73,

0xf0, 0xb8,

0x7e

ENDFIELD(AuthRequest0_C, 38)

/* HDLC Frame: Authenticate Request  (PAP, Client Mode, Identifier 2)

| 0x20, 0x01 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x23 | 0xc0, 0x23 | Protocol = PAP |
| 0x01 | 0x7d, 0x21 | PAP Code (Auth Request) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x016 | 0x7d, 0x20, 0x7d, 0x36 | Length |
| 0x08 | 0x7d, 0x28 | Peer-ID Length |
| 0x74, 0x65, 0x73, 0x74, 0x6E, 0x61, 0x6D, 0x65 | 0x74, 0x65, 0x73, 0x74, 0x6E, 0x61, 0x6D, 0x65 | Peer-ID “testname” |
| 0x08 | 0x7d, 0x28 | Password Length |
| 0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73 | 0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73 | Password “testpass” |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(AuthRequest1)	0x28, 0x01,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x23,

0x7d, 0x21,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x36,

0x7d, 0x28,

0x74, 0x65, 0x73, 0x74, 0x6E, 0x61, 0x6D, 0x65,

0x7d, 0x28,

0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73,

0xf0, 0xb8,

0x7e

ENDFIELD(AuthRequest1, 41)

/* HDLC Frame: Authenticate Request  (PAP, ACFC, PFC, Identifier = 0)

| 0x10, 0x01 |  | SDU length (34 Byte = 272 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xc0, 0x23 | 0xc0, 0x23 | Protocol = PAP |
| 0x01 | 0x7d, 0x21 | PAP Code (Authenticate Request) |
| 0x00 | 0x7d, 0x20 | Identifier |
| 0x00, 0x016 | 0x7d, 0x20, 0x7d, 0x36 | Length: 22 |
| 0x08 | 0x7d, 0x28 | Peer-ID Length: 8 |
| 0x74, 0x65, 0x73, 0x74, 0x6E, 0x61, 0x6D, 0x65 | 0x74, 0x65, 0x73, 0x74, 0x6E, 0x61, 0x6D, 0x65 | Peer-ID “testname” |
| 0x08 | 0x7d, 0x28 | Password Length: 8 |
| 0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73 | 0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73 | Password “testpass” |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(PAP_AuthRequest67)

0x10, 0x01,

0x00, 0x00,

0x7e,

0xc0, 0x23,

0x7d, 0x21,

0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x36,

0x7d, 0x28,

0x74, 0x65, 0x73, 0x74, 0x6E, 0x61, 0x6D, 0x65,

0x7d, 0x28,

0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73,

0xf0, 0xb8,

0x7e

ENDFIELD(PAP_AuthRequest67, 38)

/*

### PAP Authentication NAK

*/

/* HDLC Frame: Authenticate Nack (PAP, Client Mode, Identifier 1)

| 0x98, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x23 | 0xc0, 0x23 | Protocol = PAP |
| 0x03 | 0x7d, 0x23 | PAP Code (Auth Nack) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x05 | 0x7d, 0x20, 0x7d, 0x25 | Length |
| 0x00 | 0x7d, 0x20 | Message Length |
|  |  | Message (None) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(AuthNack0)	0x98, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x23,

0x7d, 0x23,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x25,

0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(AuthNack0, 23)

/*

### PAP Authentication Ack

*/

/* HDLC Frame: Authenticate Ack (PAP, Client Mode)

| 0x98, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0xc0, 0x23 | 0xc0, 0x23 | Protocol = PAP |
| 0x02 | 0x7d, 0x22 | PAP Code (Auth Ack) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x05 | 0x7d, 0x20, 0x7d, 0x25 | Length |
| 0x00 | 0x7d, 0x20 | Message Length |
|  |  | Message (None) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(AuthAck0)		0x98, 0x00,

0, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0xc0, 0x23,

0x7d, 0x22,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x25,

0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(AuthAck0, 23)

/* HDLC Frame: Authenticate Ack (PAP, Client Mode, ACFC, PFC)

| 0x80, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xc0, 0x23 | 0xc0, 0x23 | Protocol = PAP |
| 0x02 | 0x7d, 0x22 | PAP Code (Auth Ack) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x05 | 0x7d, 0x20, 0x7d, 0x25 | Length |
| 0x00 | 0x7d, 0x20 | Message Length |
|  |  | Message (None) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(AuthAck0_C)		0x80, 0x00,

0x00, 0x00,

0x7e,

0xc0, 0x23,

0x7d, 0x22,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x25,

0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(AuthAck0_C, 20)

/* HDLC Frame: Authenticate Ack (PAP, ACFC, PFC, Identifier = 0)

| 0x80, 0x00 |  | SDU length (16 Byte = 128 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xc0, 0x23 | 0xc0, 0x23 | Protocol = PAP |
| 0x02 | 0x7d, 0x22 | PAP Code (Authenticate Ack) |
| 0x00 | 0x7d, 0x20 | Identifier |
| 0x00, 0x05 | 0x7d, 0x20, 0x7d, 0x25 | Length: 5 |
| 0x00 | 0x7d, 0x20 | Message Length |
|  |  | Message (None) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(PAP_AuthAck67)

0x80, 0x00,

0x00, 0x00,

0x7e,

0xc0, 0x23,

0x7d, 0x22,

0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x25,

0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(PAP_AuthAck67, 20)

/*

### CCP Configure Request

*/

/* HDLC Frame: CCP Configure Request (Identifier = 1)

| 0xe8, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x80, 0xfd | 0x80, 0xfd | Protocol = CCP |
| 0x01 | 0x7d, 0x21 | CCP Code (Conf Req) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length |
| 0x03 | 0x7d, 0x23 | Option Type (OUI) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (dummy) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(CCPConfReq0) 	0xe8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0x80, 0xfd,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(CCPConfReq0, 33)

/* HDLC Frame: Configure Request (CCP, ACFC, PFC, Identifier = 4)

| 0x28, 0x0c |  | SDU length (389 Byte = 3112 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0xfd | 0x80, 0xfd | Protocol = CCP |
| 0x01 | 0x7d, 0x21 | CCP Code (Configure Request) |
| 0x04 | 0x7d, 0x24 | Identifier |
| 0x00, 0xd5 | 0x7d, 0x20, 0xd5 | Length: 213 |
| ... | ... | Content |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(CCP_ConfRequest59)

0x28, 0x0c,

0x00, 0x00,

0x7e,

0x80, 0xfd,

0x7d, 0x21,

0x7d, 0x24,

0x7d, 0x20, 0xd5,

0xfe, 0xcb, 0x7d, 0x21, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x74, 0xff,

0xc9, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x68,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0xa0, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x30, 0x30, 0x20, 0x62, 0x7d, 0x25, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0xc0, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0xa4, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0xbc, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x38,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x70, 0x7d, 0x20, 0xea, 0x7d, 0x20,

0xb0, 0xc7, 0x7d, 0x34, 0x7d, 0x20, 0x69, 0xcc, 0xf1, 0x77, 0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x21, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0xe8, 0xfe, 0xc9, 0x7d, 0x20, 0xc8, 0xcd, 0xf1, 0x77, 0x7d,

0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d,

0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x74, 0x7d, 0x20, 0x6e, 0x7d,

0x20, 0x7d, 0x3d, 0x7d, 0x20, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x21,

0x7d, 0x2b, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0xdb, 0x80, 0xfd, 0x7d, 0x21, 0x7d, 0x24, 0x7d, 0x20, 0xd5,

0xfe, 0xcb, 0x7d, 0x21, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x74, 0xff,

0xc9, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x68,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x32, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20,

0x7d, 0x20, 0x7d, 0x26,

0xf0, 0xb8,

0x7e

ENDFIELD(CCP_ConfRequest59, 393)

/* HDLC Frame: Configure Request (CCP, ACFC, PFC, ACCM=0, Identifier = 4)

| 0xd8, 0x06 | SDU length (219 Byte = 1752 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0x80, 0xfd | Protocol = CCP |
| 0x01 | CCP Code (Configure Request) |
| 0x04 | Identifier |
| 0x00, 0xd5 | Length: 213 |
| ... | Content |
| 0xf0, 0xb8 | FSC (dummy) |
| 0x7e | Flag |

*/

FIELD(CCP_ConfRequest81)

0xd8, 0x06,

0x00, 0x00,

0x7e,

0x80, 0xfd,

0x01,

0x04,

0x00, 0xd5,

0xfe, 0xcb, 0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x74, 0xff, 0xc9, 0x00, 0x00, 0x00, 0x00, 0x00, 0x68, 0x00, 0x00, 0x00, 0xa0, 0x00,

0x00, 0x00, 0x30, 0x30, 0x20, 0x62, 0x05, 0x00, 0x00, 0x00, 0xc0, 0x00, 0x00, 0x00, 0xa4, 0x00,

0x00, 0x00, 0xbc, 0x00, 0x00, 0x00, 0x18, 0x00, 0x00, 0x00, 0x70, 0x00, 0xea, 0x00, 0xb0, 0xc7,

0x14, 0x00, 0x69, 0xcc, 0xf1, 0x77, 0x00, 0x00, 0x00, 0x00, 0x01, 0x00, 0x00, 0x00, 0xe8, 0xfe,

0xc9, 0x00, 0xc8, 0xcd, 0xf1, 0x77, 0x06, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x74, 0x00, 0x6e, 0x00, 0x1d, 0x00, 0x20, 0x00, 0x00, 0x01,

0x0b, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0xdb, 0x80, 0xfd, 0x01, 0x04, 0x00, 0xd5, 0xfe, 0xcb,

0x01, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,

0x74, 0xff, 0xc9, 0x00, 0x00, 0x00, 0x00, 0x00, 0x68, 0x00, 0x00, 0x12, 0x06, 0x00, 0x00, 0x00,

0x06,

0xf0, 0xb8,

0x7e

ENDFIELD(CCP_ConfRequest81, 223)

/*

### IPCP Configure Request

*/

/* HDLC Frame: IPCP Configure Request (Identifier = 1)

| 0xe8, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Conf Req) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length |
| 0x03 | 0x7d, 0x23 | Option Type (IP-Adress) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (IP-Adress 0.0.0.0) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCPReq0)		0xe8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCPReq0, 33)

/* HDLC Frame: IPCP Configure Request (ACFC, PFC, Identifier = 1)

| 0xd0, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Conf Req) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length |
| 0x03 | 0x7d, 0x23 | Option Type (IP-Adress) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Option Value (IP-Adress 0.0.0.0) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCPReq0_C)

0xd0, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCPReq0_C, 30)

/* HDLC Frame: IPCP Configure Req (Identifier = 2)

| 0xd8, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Conf Req) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length |
| 0x03 | 0x7d, 0x23 | Option Type (IP-Adress) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0xc6, 0xa8, 0x01, 0x05 | 0xc6, 0xa8, 0x7d, 0x21, 0x7d, 0x25 | Option Value (IP-Adress 192.168.1.5) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCPReq1)		0xd8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23,

0x7d, 0x26,

0xc6, 0xa8, 0x7d, 0x21, 0x7d, 0x25,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCPReq1, 31)

/* HDLC Frame: IPCP Configure Req (ACFC, PFC, Identifier = 2)

| 0xc0, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Conf Req) |
| 0x02 | 0x7d, 0x22 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length |
| 0x03 | 0x7d, 0x23 | Option Type (IP-Adress) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0xc6, 0xa8, 0x01, 0x05 | 0xc6, 0xa8, 0x7d, 0x21, 0x7d, 0x25 | Option Value (IP-Adress 192.168.1.5) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCPReq1_C)		0xc0, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23,

0x7d, 0x26,

0xc6, 0xa8, 0x7d, 0x21, 0x7d, 0x25,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCPReq1_C, 28)

/* HDLC Frame: IPCP Configure Req (Identifier = 5)

| 0x88, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Conf Req) |
| 0x05 | 0x7d, 0x25 | Identifier |
| 0x00, 0x04 | 0x7d, 0x20, 0x7d, 0x24 | Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCPReq2)

0x88, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x25,

0x7d, 0x20, 0x7d, 0x24,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCPReq2, 21)

/* HDLC Frame: IPCP Configure Req (ACFC, PFC, Identifier = 5)

| 0x70, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Conf Req) |
| 0x05 | 0x7d, 0x25 | Identifier |
| 0x00, 0x04 | 0x7d, 0x20, 0x7d, 0x24 | Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCPReq2_C)

0x70, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x25,

0x7d, 0x20, 0x7d, 0x24,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCPReq2_C, 18)

/* HDLC Frame: IPCP Configure Req (ACFC, PFC, IP address, Identifier = 5)

| 0xD0, 0x00 |  | SDU length (26 Byte = 208 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Conf Req) |
| 0x05 | 0x7d, 0x25 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length: 10 |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x0a, 0x00, 0x00, 0x01 | 0x7d, 0x2a, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x21 | Static Address (10.0.0.1) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfRequest3_C)

0xD0, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x25,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23, 0x7d, 0x26, 0x7d, 0x2a, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x21,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest3_C, 30)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, Identifier = 5)

| 0x80, 0x02 |  | SDU length (80 Byte = 640 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Configure Request) |
| 0x05 | 0x7d, 0x25 | Identifier |
| 0x00, 0x28 | 0x7d, 0x20, 0x28 | Length: 40 |
| 0x02 | 0x7d, 0x22 | IP-Header Compression |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Van Jacobson Header Compression |
| 0x0f | 0x7d, 0x2f | Max-Slot-ID: 15 |
| 0x01 | 0x7d, 0x21 | Comp-Slot-ID: compressed slot ID |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x81 | 0x81 | Primary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x82 | 0x82 | Primary NBNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x83 | 0x83 | Secondary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x84 | 0x84 | Secondary NBNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfRequest61)

0x80, 0x02,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x25,

0x7d, 0x20, 0x28,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x2d, 0x7d, 0x2f, 0x7d, 0x21,

0x7d, 0x23, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x81, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x82, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x83, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x84, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest61, 84)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, Identifier = 1)

| 0xc8, 0x00 |  | SDU length (25 Byte = 200 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Configure Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length: 10 |
| 0x02 | 0x7d, 0x22 | IP-Header Compression |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Van Jacobson Header Compression |
| 0x0f | 0x7d, 0x2f | Max-Slot-ID: 15 |
| 0x01 | 0x7d, 0x01 | Comp-Slot-ID: compressed slot ID |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfRequest62)

0xc8, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x2d, 0x7d, 0x2f, 0x7d, 0x21,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest62, 29)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, Identifier = 6)

| 0xd8, 0x01 |  | SDU length (59 Byte = 472 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Configure Request) |
| 0x06 | 0x7d, 0x26 | Identifier |
| 0x00, 0x1c | 0x7d, 0x20, 0x7d, 0x3c | Length: 28 |
| 0x02 | 0x7d, 0x22 | IP-Header Compression |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Van Jacobson Header Compression |
| 0x0f | 0x7d, 0x2f | Max-Slot-ID: 15 |
| 0x01 | 0x7d, 0x01 | Comp-Slot-ID: compressed slot ID |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x81 | 0x81 | Primary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x83 | 0x83 | Secondary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfRequest63)

0xd8, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x3c,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x2d, 0x7d, 0x2f, 0x7d, 0x21,

0x7d, 0x23, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x81, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x83, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest63, 63)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, Identifier = 7)

| 0x90, 0x01 |  | SDU length (50 Byte = 400 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Configure Request) |
| 0x07 | 0x7d, 0x27 | Identifier |
| 0x00, 0x1c | 0x7d, 0x20, 0x7d, 0x3c | Length: 28 |
| 0x02 | 0x7d, 0x22 | IP-Header Compression |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Van Jacobson Header Compression |
| 0x0f | 0x7d, 0x2f | Max-Slot-ID: 15 |
| 0x01 | 0x7d, 0x01 | Comp-Slot-ID: compressed slot ID |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x15, 0x02 | 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22 | Address 141.64.21.2 |
| 0x81 | 0x81 | Primary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x18, 0x81 | 0x8d, 0x40, 0x7d, 0x38, 0x81 | Address 141.64.24.129 |
| 0x83 | 0x83 | Secondary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0xfe, 0x83 | 0x8d, 0x40, 0xfe, 0x83 | Address 141.64.254.131 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfRequest65)

0x90, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x27,

0x7d, 0x20, 0x7d, 0x3c,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x2d, 0x7d, 0x2f, 0x7d, 0x21,

0x7d, 0x23, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22,

0x81, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x38, 0x81,

0x83, 0x7d, 0x26, 0x8d, 0x40, 0xfe, 0x83,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest65, 54)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, VJ with uncompressed CSID, Identifier = 5)

| 0x80, 0x02 |  | SDU length (80 Byte = 640 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Configure Request) |
| 0x05 | 0x7d, 0x25 | Identifier |
| 0x00, 0x28 | 0x7d, 0x20, 0x28 | Length: 40 |
| 0x02 | 0x7d, 0x22 | IP-Header Compression |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Van Jacobson Header Compression |
| 0x0f | 0x7d, 0x2f | Max-Slot-ID: 15 |
| 0x00 | 0x7d, 0x20 | Comp-Slot-ID: uncompressed slot ID |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x81 | 0x81 | Primary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x82 | 0x82 | Primary NBNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x83 | 0x83 | Secondary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x84 | 0x84 | Secondary NBNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfRequest69)

0x80, 0x02,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x25,

0x7d, 0x20, 0x28,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x2d, 0x7d, 0x2f, 0x7d, 0x20,

0x7d, 0x23, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x81, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x82, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x83, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x84, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest69, 84)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, without Header Compression, Identifier = 1)

| 0x70, 0x00 |  | SDU length (14 Byte = 112 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Configure Request) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x04 | 0x7d, 0x20, 0x7d, 0x24 | Length: 4 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfRequest70)

0x70, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x24,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest70, 18)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, without Header Compression, Identifier = 6)

| 0x80, 0x01 |  | SDU length (48 Byte = 384 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Configure Request) |
| 0x06 | 0x7d, 0x26 | Identifier |
| 0x00, 0x16 | 0x7d, 0x20, 0x7d, 0x36 | Length: 22 |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x81 | 0x81 | Primary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x83 | 0x83 | Secondary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfRequest71)

0x80, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x36,

0x7d, 0x23, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x81, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x83, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest71, 52)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, without Header Compression, Identifier = 7)

| 0x38, 0x01 |  | SDU length (39 Byte = 312 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Configure Request) |
| 0x07 | 0x7d, 0x27 | Identifier |
| 0x00, 0x16 | 0x7d, 0x20, 0x7d, 0x36 | Length: 22 |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x15, 0x02 | 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22 | Address 141.64.21.2 |
| 0x81 | 0x81 | Primary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x18, 0x81 | 0x8d, 0x40, 0x7d, 0x38, 0x81 | Address 141.64.24.129 |
| 0x83 | 0x83 | Secondary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0xfe, 0x83 | 0x8d, 0x40, 0xfe, 0x83 | Address 141.64.254.131 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfRequest73)

0x38, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x27,

0x7d, 0x20, 0x7d, 0x36,

0x7d, 0x23, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22,

0x81, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x38, 0x81,

0x83, 0x7d, 0x26, 0x8d, 0x40, 0xfe, 0x83,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest73, 43)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, ACCM=0, Identifier = 5)

| 0x70, 0x01 | SDU length (46 Byte = 368 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0x80, 0x21 | Protocol = IPCP |
| 0x01 | IPCP Code (Configure Request) |
| 0x05 | Identifier |
| 0x00, 0x28 | Length: 40 |
| 0x02 | IP-Header Compression |
| 0x06 | Option Length: 6 |
| 0x00, 0x2d | Van Jacobson Header Compression |
| 0x0f | Max-Slot-ID: 15 |
| 0x01 | Comp-Slot-ID: compressed slot ID |
| 0x03 | IP-Address |
| 0x06 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | Dynamic Address |
| 0x81 | Primary DNS Server Address |
| 0x06 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | Dynamic Address |
| 0x82 | Primary NBNS Server Address |
| 0x06 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | Dynamic Address |
| 0x83 | Secondary DNS Server Address |
| 0x06 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | Dynamic Address |
| 0x84 | Secondary NBNS Server Address |
| 0x06 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | Dynamic Address |
| 0xf0, 0xb8 | FSC (dummy) |
| 0x7e | Flag |

*/

FIELD(IPCP_ConfRequest83)

0x70, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x01,

0x05,

0x00, 0x28,

0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01,

0x03, 0x06, 0x00, 0x00, 0x00, 0x00,

0x81, 0x06, 0x00, 0x00, 0x00, 0x00,

0x82, 0x06, 0x00, 0x00, 0x00, 0x00,

0x83, 0x06, 0x00, 0x00, 0x00, 0x00,

0x84, 0x06, 0x00, 0x00, 0x00, 0x00,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest83, 50)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, ACCM=0, Identifier = 1)

| 0x80, 0x00 | SDU length (16 Byte = 128 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0x80, 0x21 | Protocol = IPCP |
| 0x01 | IPCP Code (Configure Request) |
| 0x01 | Identifier |
| 0x00, 0x0a | Length: 10 |
| 0x02 | IP-Header Compression |
| 0x06 | Option Length: 6 |
| 0x00, 0x2d | Van Jacobson Header Compression |
| 0x0f | Max-Slot-ID: 15 |
| 0x01 | Comp-Slot-ID: compressed slot ID |
| 0xf0, 0xb8 | FSC (dummy) |
| 0x7e | Flag |

*/

FIELD(IPCP_ConfRequest84)

0x80, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x01,

0x01,

0x00, 0x0a,

0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest84, 20)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, ACCM=0, Identifier = 6)

| 0x10, 0x01 | SDU length (34 Byte = 272 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0x80, 0x21 | Protocol = IPCP |
| 0x01 | IPCP Code (Configure Request) |
| 0x06 | Identifier |
| 0x00, 0x1c | Length: 28 |
| 0x02 | IP-Header Compression |
| 0x06 | Option Length: 6 |
| 0x00, 0x2d | Van Jacobson Header Compression |
| 0x0f | Max-Slot-ID: 15 |
| 0x01 | Comp-Slot-ID: compressed slot ID |
| 0x03 | IP-Address |
| 0x06 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | Dynamic Address |
| 0x81 | Primary DNS Server Address |
| 0x06 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | Dynamic Address |
| 0x83 | Secondary DNS Server Address |
| 0x06 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | Dynamic Address |
| 0xf0, 0xb8 | FSC (dummy) |
| 0x7e | Flag |

*/

FIELD(IPCP_ConfRequest85)

0x10, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x01,

0x06,

0x00, 0x1c,

0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01,

0x03, 0x06, 0x00, 0x00, 0x00, 0x00,

0x81, 0x06, 0x00, 0x00, 0x00, 0x00,

0x83, 0x06, 0x00, 0x00, 0x00, 0x00,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest85, 38)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, ACCM=0, Identifier = 7)

| 0x10, 0x01 | SDU length (34 Byte = 272 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0x80, 0x21 | Protocol = IPCP |
| 0x01 | IPCP Code (Configure Request) |
| 0x07 | Identifier |
| 0x00, 0x1c | Length: 28 |
| 0x02 | IP-Header Compression |
| 0x06 | Option Length: 6 |
| 0x00, 0x2d | Van Jacobson Header Compression |
| 0x0f | Max-Slot-ID: 15 |
| 0x01 | Comp-Slot-ID: compressed slot ID |
| 0x03 | IP-Address |
| 0x06 | Option Length: 6 |
| 0x8d, 0x40, 0x15, 0x02 | Address 141.64.21.2 |
| 0x81 | Primary DNS Server Address |
| 0x06 | Option Length: 6 |
| 0x8d, 0x40, 0x18, 0x81 | Address 141.64.24.129 |
| 0x83 | Secondary DNS Server Address |
| 0x06 | Option Length: 6 |
| 0x8d, 0x40, 0xfe, 0x83 | Address 141.64.254.131 |
| 0xf0, 0xb8 | FSC (dummy) |
| 0x7e | Flag |

*/

FIELD(IPCP_ConfRequest87)

0x10, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x01,

0x07,

0x00, 0x1c,

0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01,

0x03, 0x06, 0x8d, 0x40, 0x15, 0x02,

0x81, 0x06, 0x8d, 0x40, 0x18, 0x81,

0x83, 0x06, 0x8d, 0x40, 0xfe, 0x83,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest87, 38)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, without Header Compression, Identifier = 3)

| 0x70, 0x00 |  | SDU length (14 Byte = 112 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Configure Request) |
| 0x03 | 0x7d, 0x23 | Identifier |
| 0x00, 0x04 | 0x7d, 0x20, 0x7d, 0x24 | Length: 4 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfRequest88)

0x70, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x23,

0x7d, 0x20, 0x7d, 0x24,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest88, 18)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, without DNS, Identifier = 6)

| 0x28, 0x01 |  | SDU length (37 Byte = 296 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Configure Request) |
| 0x06 | 0x7d, 0x26 | Identifier |
| 0x00, 0x10 | 0x7d, 0x20, 0x7d, 0x30 | Length: 16 |
| 0x02 | 0x7d, 0x22 | IP-Header Compression |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Van Jacobson Header Compression |
| 0x0f | 0x7d, 0x2f | Max-Slot-ID: 15 |
| 0x01 | 0x7d, 0x01 | Comp-Slot-ID: compressed slot ID |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfRequest89)

0x28, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x30,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x2d, 0x7d, 0x2f, 0x7d, 0x21,

0x7d, 0x23, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest89, 41)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, without DNS, Identifier = 7)

| 0x18, 0x01 |  | SDU length (35 Byte = 280 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x01 | 0x7d, 0x21 | IPCP Code (Configure Request) |
| 0x07 | 0x7d, 0x27 | Identifier |
| 0x00, 0x10 | 0x7d, 0x20, 0x7d, 0x30 | Length: 16 |
| 0x02 | 0x7d, 0x22 | IP-Header Compression |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Van Jacobson Header Compression |
| 0x0f | 0x7d, 0x2f | Max-Slot-ID: 15 |
| 0x01 | 0x7d, 0x01 | Comp-Slot-ID: compressed slot ID |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x15, 0x02 | 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22 | Address 141.64.21.2 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfRequest91)

0x18, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x21,

0x7d, 0x27,

0x7d, 0x20, 0x7d, 0x30,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x2d, 0x7d, 0x2f, 0x7d, 0x21,

0x7d, 0x23, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfRequest91, 39)

/*

### IPCP Configure Reject

*/

/* HDLC Frame: Configure Reject (IPCP, ACFC, PFC, Identifier = 5)

| 0x20, 0x01 |  | SDU length (36 Byte = 288 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x04 | 0x7d, 0x24 | IPCP Code (Configure Reject) |
| 0x05 | 0x7d, 0x25 | Identifier |
| 0x00, 0x10 | 0x7d, 0x20, 0x7d, 0x30 | Length: 16 |
| 0x82 | 0x82 | Primary NBNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x84 | 0x84 | Secondary NBNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfReject61)

0x20, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x24,

0x7d, 0x25,

0x7d, 0x20, 0x7d, 0x30,

0x82, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x84, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfReject61, 40)

/* HDLC Frame: Configure Reject (IPCP, ACFC, PFC, include Header Compression, Identifier = 5)

| 0x78, 0x01 |  | SDU length (47 Byte = 376 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x04 | 0x7d, 0x24 | IPCP Code (Configure Reject) |
| 0x05 | 0x7d, 0x25 | Identifier |
| 0x00, 0x16 | 0x7d, 0x20, 0x7d, 0x36 | Length: 22 |
| 0x02 | 0x7d, 0x22 | Header Compression |
| 0x06 | 0c7d, 0x26 | Option Length: 6 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Van Jacobson Header Compression |
| 0x0f | 0x7d, 0x2f | Max-Slot-ID: 15 |
| 0x01 | 0x7d, 0x21 | Comp-Slot-ID: compressed slot ID |
| 0x82 | 0x82 | Primary NBNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x84 | 0x84 | Secondary NBNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfReject61F)

0x78, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x24,

0x7d, 0x25,

0x7d, 0x20, 0x7d, 0x36,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x2d, 0x7d, 0x2f, 0x7d, 0x21,

0x82, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x84, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfReject61F, 51)

/* HDLC Frame: Configure Reject (IPCP, ACFC, PFC, reject DNS, Identifier = 5)

| 0xd0, 0x01 |  | SDU length (58 Byte = 464 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x04 | 0x7d, 0x24 | IPCP Code (Configure Reject) |
| 0x05 | 0x7d, 0x25 | Identifier |
| 0x00, 0x1c | 0x7d, 0x20, 0x7d, 0x3c | Length: 28 |
| 0x81 | 0x81 | Primary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x82 | 0x82 | Primary NBNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x83 | 0x83 | Secondary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x84 | 0x84 | Secondary NBNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfReject61DNS)

0xd0, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x24,

0x7d, 0x25,

0x7d, 0x20, 0x7d, 0x3c,

0x81, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x82, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x83, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x84, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfReject61DNS, 62)

/* HDLC Frame: Configure Reject (IPCP, ACFC, PFC, include Header Compression, Identifier = 5)

| 0x78, 0x01 |  | SDU length (47 Byte = 376 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x04 | 0x7d, 0x24 | IPCP Code (Configure Reject) |
| 0x05 | 0x7d, 0x25 | Identifier |
| 0x00, 0x16 | 0x7d, 0x20, 0x7d, 0x36 | Length: 22 |
| 0x02 | 0x7d, 0x22 | Header Compression |
| 0x06 | 0c7d, 0x26 | Option Length: 6 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Van Jacobson Header Compression |
| 0x0f | 0x7d, 0x2f | Max-Slot-ID: 15 |
| 0x00 | 0x7d, 0x20 | Comp-Slot-ID: uncompressed slot ID |
| 0x82 | 0x82 | Primary NBNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x84 | 0x84 | Secondary NBNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfReject69)

0x78, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x24,

0x7d, 0x25,

0x7d, 0x20, 0x7d, 0x36,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x2d, 0x7d, 0x2f, 0x7d, 0x20,

0x82, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x84, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfReject69, 51)

/* HDLC Frame: Configure Reject (IPCP, ACFC, PFC, without Header Compression, Identifier = 6)

| 0x80, 0x01 |  | SDU length (48 Byte = 384 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x04 | 0x7d, 0x24 | IPCP Code (Configure Reject) |
| 0x06 | 0x7d, 0x26 | Identifier |
| 0x00, 0x16 | 0x7d, 0x20, 0x7d, 0x36 | Length: 22 |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x81 | 0x81 | Primary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0x83 | 0x83 | Secondary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20 | Dynamic Address |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfReject71)

0x80, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x24,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x36,

0x7d, 0x23, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x81, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0x83, 0x7d, 0x26, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x20,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfReject71, 52)

/* HDLC Frame: Configure Reject (IPCP, ACFC, PFC, ACCM=0, Identifier = 5)

| 0xb0, 0x00 | SDU length (22 Byte = 176 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0x80, 0x21 | Protocol = IPCP |
| 0x04 | IPCP Code (Configure Reject) |
| 0x05 | Identifier |
| 0x00, 0x10 | Length: 16 |
| 0x82 | Primary NBNS Server Address |
| 0x06 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | Dynamic Address |
| 0x84 | Secondary NBNS Server Address |
| 0x06 | Option Length: 6 |
| 0x00, 0x00, 0x00, 0x00 | Dynamic Address |
| 0xf0, 0xb8 | FSC (dummy) |
| 0x7e | Flag |

*/

FIELD(IPCP_ConfReject83)

0xb0, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x04,

0x05,

0x00, 0x10,

0x82, 0x06, 0x00, 0x00, 0x00, 0x00,

0x84, 0x06, 0x00, 0x00, 0x00, 0x00,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfReject83, 26)

/*

### IPCP Configure NAK

*/

/* HDLC Frame: IPCP Configure Nack (Identifier = 1)

| 0xd8, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x03 | 0x7d, 0x23 | IPCP Code (Conf Nack) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length |
| 0x03 | 0x7d, 0x23 | Option Type (IP-Adress) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0xc6, 0xa8, 0x01, 0x05 | 0xc6, 0xa8, 0x7d, 0x21, 0x7d, 0x25 | Option Value (IP-Adress 192.168.1.5) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCPNack0)	0xd8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0x80, 0x21,

0x7d, 0x23,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23,

0x7d, 0x26,

0xc6, 0xa8, 0x7d, 0x21, 0x7d, 0x25,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCPNack0, 31)

/* HDLC Frame: IPCP Configure Nack (ACFC, PFC, Identifier = 1)

| 0xc0, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x03 | 0x7d, 0x23 | IPCP Code (Conf Nack) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length |
| 0x03 | 0x7d, 0x23 | Option Type (IP-Adress) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0xc6, 0xa8, 0x01, 0x05 | 0xc6, 0xa8, 0x7d, 0x21, 0x7d, 0x25 | Option Value (IP-Adress 192.168.1.5) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCPNack0_C)	0xc0, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x23,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23,

0x7d, 0x26,

0xc6, 0xa8, 0x7d, 0x21, 0x7d, 0x25,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCPNack0_C, 28)

/* HDLC Frame: Configure NAK (IPCP, ACFC, PFC, Identifier = 6)

| 0x38, 0x01 |  | SDU length (39 Byte = 312 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x03 | 0x7d, 0x23 | IPCP Code (Configure NAK) |
| 0x06 | 0x7d, 0x26 | Identifier |
| 0x00, 0x16 | 0x7d, 0x20, 0x7d, 0x36 | Length: 22 |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x15, 0x02 | 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22 | Address 141.64.21.2 |
| 0x81 | 0x81 | Primary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x18, 0x81 | 0x8d, 0x40, 0x7d, 0x38, 0x81 | Address 141.64.24.129 |
| 0x83 | 0x83 | Secondary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0xfe, 0x83 | 0x8d, 0x40, 0xfe, 0x83 | Address 141.64.254.131 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfNAK63)

0x38, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x23,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x36,

0x7d, 0x23, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22,

0x81, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x38, 0x81,

0x83, 0x7d, 0x26, 0x8d, 0x40, 0xfe, 0x83,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfNAK63, 43)

/* HDLC Frame: Configure NAK (IPCP, ACFC, PFC, Identifier = 6)

| 0x38, 0x01 |  | SDU length (39 Byte = 312 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x03 | 0x7d, 0x23 | IPCP Code (Configure NAK) |
| 0x06 | 0x7d, 0x26 | Identifier |
| 0x00, 0x16 | 0x7d, 0x20, 0x7d, 0x36 | Length: 22 |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x15, 0x02 | 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22 | Address 141.64.21.2 |
| 0x81 | 0x81 | Primary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x18, 0x81 | 0x8d, 0x40, 0x7d, 0x38, 0x81 | Address 141.64.24.129 |
| 0x83 | 0x83 | Secondary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0xfe, 0x83 | 0x8d, 0x40, 0xfe, 0x83 | Address 141.64.254.131 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfNAK71)

0x38, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x23,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x36,

0x7d, 0x23, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22,

0x81, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x38, 0x81,

0x83, 0x7d, 0x26, 0x8d, 0x40, 0xfe, 0x83,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfNAK71, 43)

/* HDLC Frame: Configure NAK (IPCP, ACFC, PFC, ACCM=0, Identifier = 6)

| 0xe0, 0x00 | SDU length (28 Byte = 224 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0x80, 0x21 | Protocol = IPCP |
| 0x03 | IPCP Code (Configure NAK) |
| 0x06 | Identifier |
| 0x00, 0x16 | Length: 22 |
| 0x03 | IP-Address |
| 0x06 | Option Length: 6 |
| 0x8d, 0x40, 0x15, 0x02 | Address 141.64.21.2 |
| 0x81 | Primary DNS Server Address |
| 0x06 | Option Length: 6 |
| 0x8d, 0x40, 0x18, 0x81 | Address 141.64.24.129 |
| 0x83 | Secondary DNS Server Address |
| 0x06 | Option Length: 6 |
| 0x8d, 0x40, 0xfe, 0x83 | Address 141.64.254.131 |
| 0xf0, 0xb8 | FSC (dummy) |
| 0x7e | Flag |

*/

FIELD(IPCP_ConfNAK85)

0xe0, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x03,

0x06,

0x00, 0x16,

0x03, 0x06, 0x8d, 0x40, 0x15, 0x02,

0x81, 0x06, 0x8d, 0x40, 0x18, 0x81,

0x83, 0x06, 0x8d, 0x40, 0xfe, 0x83,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfNAK85, 32)

/* HDLC Frame: Configure NAK (IPCP, ACFC, PFC, without DNS, Identifier = 6)

| 0xc0, 0x00 |  | SDU length (24 Byte = 192 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x03 | 0x7d, 0x23 | IPCP Code (Configure NAK) |
| 0x06 | 0x7d, 0x26 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length: 10 |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x15, 0x02 | 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22 | Address 141.64.21.2 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfNAK89)

0xc0, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x23,

0x7d, 0x26,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfNAK89, 28)

/*

### IPCP Configure Ack

*/

/* HDLC Frame: IPCP Configure Ack (Identifier = 2)

| 0xd8, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x02 | 0x7d, 0x22 | IPCP Code (Conf Ack) |
| 0x01 | 0x7d, 0x22 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length |
| 0x03 | 0x7d, 0x23 | Option Type (IP-Adress) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0xc6, 0xa8, 0x01, 0x05 | 0xc6, 0xa8, 0x7d, 0x21, 0x7d, 0x25 | Option Value (IP-Adress 192.168.1.5) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCPAck1)		0xd8, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0x80, 0x21,

0x7d, 0x22,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23,

0x7d, 0x26,

0xc6, 0xa8, 0x7d, 0x21, 0x7d, 0x25,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCPAck1, 31)

/* HDLC Frame: IPCP Configure Ack (ACFC, PFC, Identifier = 2)

| 0xc0, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x02 | 0x7d, 0x22 | IPCP Code (Conf Ack) |
| 0x01 | 0x7d, 0x22 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length |
| 0x03 | 0x7d, 0x23 | Option Type (IP-Adress) |
| 0x06 | 0x7d, 0x26 | Option Length |
| 0xc6, 0xa8, 0x01, 0x05 | 0xc6, 0xa8, 0x7d, 0x21, 0x7d, 0x25 | Option Value (IP-Adress 192.168.1.5) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCPAck1_C)		0xc0, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x22,

0x7d, 0x22,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23,

0x7d, 0x26,

0xc6, 0xa8, 0x7d, 0x21, 0x7d, 0x25,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCPAck1_C, 28)

/* HDLC Frame: IPCP Configure Ack (Identifier = 5)

| 0x88, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x02 | 0x7d, 0x22 | IPCP Code (Conf Req) |
| 0x05 | 0x7d, 0x25 | Identifier |
| 0x00, 0x04 | 0x7d, 0x20, 0x7d, 0x24 | Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCPAck2)		0x88, 0x00,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0x80, 0x21,

0x7d, 0x22,

0x7d, 0x25,

0x7d, 0x20, 0x7d, 0x24,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCPAck2, 21)

/* HDLC Frame: IPCP Configure Ack (ACFC, PFC, Identifier = 5)

| 0x70, 0x00 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x02 | 0x7d, 0x22 | IPCP Code (Conf Ack) |
| 0x05 | 0x7d, 0x25 | Identifier |
| 0x00, 0x04 | 0x7d, 0x20, 0x7d, 0x24 | Length |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCPAck2_C)

0x70, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x22,

0x7d, 0x25,

0x7d, 0x20, 0x7d, 0x24,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCPAck2_C, 18)

/* HDLC Frame: IPCP Configure Ack (ACFC, PFC, IP address, Identifier = 5)

| 0xD0, 0x00 |  | SDU length (26 Byte = 208 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x02 | 0x7d, 0x22 | IPCP Code (Conf Ack) |
| 0x05 | 0x7d, 0x25 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x7d, 0x2a | Length: 10 |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x0a, 0x00, 0x00, 0x01 | 0x7d, 0x2a, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x21 | Static Address (10.0.0.1) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfAck3_C)

0xD0, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x22,

0x7d, 0x25,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x23, 0x7d, 0x26, 0x7d, 0x2a, 0x7d, 0x20, 0x7d, 0x20, 0x7d, 0x21,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfAck3_C, 30)

/* HDLC Frame: Configure Ack (IPCP, ACFC, PFC, Identifier = 1)

| 0xc8, 0x00 |  | SDU length (25 Byte = 200 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x02 | 0x7d, 0x22 | IPCP Code (Configure Ack) |
| 0x05 | 0x7d, 0x25 | Identifier |
| 0x00, 0x0a | 0x7d, 0x20, 0x2a | Length: 10 |
| 0x02 | 0x7d, 0x22 | IP-Header Compression |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Van Jacobson Header Compression |
| 0x0f | 0x7d, 0x2f | Max-Slot-ID: 15 |
| 0x01 | 0x7d, 0x01 | Comp-Slot-ID: compressed slot ID |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfAck62)

0xc8, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x22,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x2a,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x2d, 0x7d, 0x2f, 0x7d, 0x21,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfAck62, 29)

/* HDLC Frame: Configure Ack (IPCP, ACFC, PFC, Identifier = 7)

| 0x90, 0x01 |  | SDU length (50 Byte = 400 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x02 | 0x7d, 0x22 | IPCP Code (Configure Ack) |
| 0x07 | 0x7d, 0x27 | Identifier |
| 0x00, 0x1c | 0x7d, 0x20, 0x7d, 0x3c | Length: 28 |
| 0x02 | 0x7d, 0x22 | IP-Header Compression |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Van Jacobson Header Compression |
| 0x0f | 0x7d, 0x2f | Max-Slot-ID: 15 |
| 0x01 | 0x7d, 0x01 | Comp-Slot-ID: compressed slot ID |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x15, 0x02 | 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22 | Address 141.64.21.2 |
| 0x81 | 0x81 | Primary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x18, 0x81 | 0x8d, 0x40, 0x7d, 0x38, 0x81 | Address 141.64.24.129 |
| 0x83 | 0x83 | Secondary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0xfe, 0x83 | 0x8d, 0x40, 0xfe, 0x83 | Address 141.64.254.131 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfAck65)

0x90, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x22,

0x7d, 0x27,

0x7d, 0x20, 0x7d, 0x3c,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x2d, 0x7d, 0x2f, 0x7d, 0x21,

0x7d, 0x23, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22,

0x81, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x38, 0x81,

0x83, 0x7d, 0x26, 0x8d, 0x40, 0xfe, 0x83,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfAck65, 54)

/* HDLC Frame: Configure Ack (IPCP, ACFC, PFC, without Header Compression, Identifier = 1)

| 0x70, 0x00 |  | SDU length (14 Byte = 112 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x02 | 0x7d, 0x22 | IPCP Code (Configure Ack) |
| 0x01 | 0x7d, 0x21 | Identifier |
| 0x00, 0x04 | 0x7d, 0x20, 0x7d, 0x24 | Length: 4 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfAck70)

0xc8, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x22,

0x7d, 0x21,

0x7d, 0x20, 0x7d, 0x24,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfAck70, 18)

/* HDLC Frame: Configure Ack (IPCP, ACFC, PFC, without Header Compression, Identifier = 7)

| 0x38, 0x01 |  | SDU length (39 Byte = 312 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x02 | 0x7d, 0x22 | IPCP Code (Configure Ack) |
| 0x07 | 0x7d, 0x27 | Identifier |
| 0x00, 0x16 | 0x7d, 0x20, 0x7d, 0x36 | Length: 22 |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x15, 0x02 | 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22 | Address 141.64.21.2 |
| 0x81 | 0x81 | Primary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x18, 0x81 | 0x8d, 0x40, 0x7d, 0x38, 0x81 | Address 141.64.24.129 |
| 0x83 | 0x83 | Secondary DNS Server Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0xfe, 0x83 | 0x8d, 0x40, 0xfe, 0x83 | Address 141.64.254.131 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfAck73)

0x38, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x22,

0x7d, 0x27,

0x7d, 0x20, 0x7d, 0x36,

0x7d, 0x23, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22,

0x81, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x38, 0x81,

0x83, 0x7d, 0x26, 0x8d, 0x40, 0xfe, 0x83,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfAck73, 43)

/* HDLC Frame: Configure Ack (IPCP, ACFC, PFC, ACCM=0, Identifier = 1)

| 0x80, 0x00 | SDU length (16 Byte = 128 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0x80, 0x21 | Protocol = IPCP |
| 0x02 | IPCP Code (Configure Ack) |
| 0x01 | Identifier |
| 0x00, 0x0a | Length: 10 |
| 0x02 | IP-Header Compression |
| 0x06 | Option Length: 6 |
| 0x00, 0x2d | Van Jacobson Header Compression |
| 0x0f | Max-Slot-ID: 15 |
| 0x01 | Comp-Slot-ID: compressed slot ID |
| 0xf0, 0xb8 | FSC (dummy) |
| 0x7e | Flag |

*/

FIELD(IPCP_ConfAck84)

0x80, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x02,

0x01,

0x00, 0x0a,

0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfAck84, 20)

/* HDLC Frame: Configure Ack (IPCP, ACFC, PFC, ACCM=0, Identifier = 7)

| 0x10, 0x01 | SDU length (34 Byte = 272 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0x7e | Flag |
| 0x80, 0x21 | Protocol = IPCP |
| 0x02 | IPCP Code (Configure Ack) |
| 0x07 | Identifier |
| 0x00, 0x1c | Length: 28 |
| 0x02 | IP-Header Compression |
| 0x06 | Option Length: 6 |
| 0x00, 0x2d | Van Jacobson Header Compression |
| 0x0f | Max-Slot-ID: 15 |
| 0x01 | Comp-Slot-ID: compressed slot ID |
| 0x03 | IP-Address |
| 0x06 | Option Length: 6 |
| 0x8d, 0x40, 0x15, 0x02 | Address 141.64.21.2 |
| 0x81 | Primary DNS Server Address |
| 0x06 | Option Length: 6 |
| 0x8d, 0x40, 0x18, 0x81 | Address 141.64.24.129 |
| 0x83 | Secondary DNS Server Address |
| 0x06 | Option Length: 6 |
| 0x8d, 0x40, 0xfe, 0x83 | Address 141.64.254.131 |
| 0xf0, 0xb8 | FSC (dummy) |
| 0x7e | Flag |

*/

FIELD(IPCP_ConfAck87)

0x10, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x02,

0x07,

0x00, 0x1c,

0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01,

0x03, 0x06, 0x8d, 0x40, 0x15, 0x02,

0x81, 0x06, 0x8d, 0x40, 0x18, 0x81,

0x83, 0x06, 0x8d, 0x40, 0xfe, 0x83,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfAck87, 38)

/* HDLC Frame: Configure Ack (IPCP, ACFC, PFC, without Header Compression, Identifier = 3)

| 0x70, 0x00 |  | SDU length (14 Byte = 112 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x02 | 0x7d, 0x22 | IPCP Code (Configure Ack) |
| 0x03 | 0x7d, 0x23 | Identifier |
| 0x00, 0x04 | 0x7d, 0x20, 0x7d, 0x24 | Length: 4 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfAck88)

0x70, 0x00,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x22,

0x7d, 0x23,

0x7d, 0x20, 0x7d, 0x24,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfAck88, 18)

/* HDLC Frame: Configure Request (IPCP, ACFC, PFC, without DNS, Identifier = 7)

| 0x18, 0x01 |  | SDU length (35 Byte = 280 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x80, 0x21 | 0x80, 0x21 | Protocol = IPCP |
| 0x02 | 0x7d, 0x22 | IPCP Code (Configure Ack) |
| 0x07 | 0x7d, 0x27 | Identifier |
| 0x00, 0x10 | 0x7d, 0x20, 0x7d, 0x30 | Length: 16 |
| 0x02 | 0x7d, 0x22 | IP-Header Compression |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x00, 0x2d | 0x7d, 0x20, 0x2d | Van Jacobson Header Compression |
| 0x0f | 0x7d, 0x2f | Max-Slot-ID: 15 |
| 0x01 | 0x7d, 0x01 | Comp-Slot-ID: compressed slot ID |
| 0x03 | 0x7d, 0x23 | IP-Address |
| 0x06 | 0x7d, 0x26 | Option Length: 6 |
| 0x8d, 0x40, 0x15, 0x02 | 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22 | Address 141.64.21.2 |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPCP_ConfAck91)

0x18, 0x01,

0x00, 0x00,

0x7e,

0x80, 0x21,

0x7d, 0x22,

0x7d, 0x27,

0x7d, 0x20, 0x7d, 0x30,

0x7d, 0x22, 0x7d, 0x26, 0x7d, 0x20, 0x2d, 0x7d, 0x2f, 0x7d, 0x21,

0x7d, 0x23, 0x7d, 0x26, 0x8d, 0x40, 0x7d, 0x35, 0x7d, 0x22,

0xf0, 0xb8,

0x7e

ENDFIELD(IPCP_ConfAck91, 39)

/*

### IP packets

*/

/* IP Paket: dummy packet A0

| 0x70, 0x03 | SDU length |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0xa1, 0xa2, 0xa3, …., 0xaf | data ( 110 byte ) |

*/

FIELD(IPPACKET_A0)	0x70, 0x03,

0x00, 0x00,

0xa1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

ENDFIELD(IPPACKET_A0,114)

/* IP Paket: dummy packet B0

| 0x70, 0x03 | SDU length |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0xb1, 0xb2, 0xb3, …., 0xbf | data ( 110 byte ) |

*/

FIELD(IPPACKET_B0)	0x70, 0x03,

0x00, 0x00,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x2b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x3b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x4b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x5b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x6b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x7b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x8b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x9b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfc, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xbf,

ENDFIELD(IPPACKET_B0,114)

/* IP Paket: dummy packet C0

| 0x70, 0x03 | SDU length |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| 0xc1, 0xc2, 0xc3, …., 0xcf | Data ( 110 byte ) |

*/

FIELD(IPPACKET_C0)	0x70, 0x03,

0x00, 0x00,

0xc1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xd1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xe1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xf1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xa1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xb1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xc1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xd1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xe1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xf1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xa1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xcf,

ENDFIELD(IPPACKET_C0,114)

/* uncompressed TCP Paket: dummy packet D0

| 0xa0, 0x04 | SDU length (148 Byte = 1184 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| …. | Data ( 148 byte ) |

*/

FIELD(UTCPPACKET_D0)

0xa0, 0x04,

0x00, 0x00,

0x45, 0x00, 0x00, 0x94, 0x04, 0x1e, 0x40, 0x00, 0x75, 0x00, 0xd9, 0x26, 0x3e, 0x60, 0xcf, 0x09,

0x97, 0xbd, 0x82, 0xf8, 0x00, 0x50, 0x04, 0x10, 0xe2, 0x75, 0xdd, 0x43, 0x00, 0x1a, 0xbe, 0x78,

0x50, 0x18, 0x21, 0x09, 0x18, 0xaf, 0x00, 0x00, 0x48, 0x54, 0x54, 0x50, 0x2f, 0x31, 0x2e, 0x31,

0x20, 0x32, 0x30, 0x30, 0x20, 0x4f, 0x4b, 0x0d, 0x0a, 0x53, 0x65, 0x72, 0x76, 0x65, 0x72, 0x3a,

0x20, 0x4d, 0x69, 0x63, 0x72, 0x6f, 0x73, 0x6f, 0x66, 0x74, 0x2d, 0x49, 0x49, 0x53, 0x2f, 0x34,

0x2e, 0x30, 0x0d, 0x0a, 0x44, 0x61, 0x74, 0x65, 0x3a, 0x20, 0x46, 0x72, 0x69, 0x2c, 0x20, 0x30,

0x36, 0x20, 0x4f, 0x63, 0x74, 0x20, 0x32, 0x30, 0x30, 0x30, 0x20, 0x31, 0x32, 0x3a, 0x34, 0x35,

0x3a, 0x32, 0x38, 0x20, 0x47, 0x4d, 0x54, 0x0d, 0x0a, 0x43, 0x6f, 0x6e, 0x74, 0x65, 0x6e, 0x74,

0x2d, 0x54, 0x79, 0x70, 0x65, 0x3a, 0x20, 0x74, 0x65, 0x78, 0x74, 0x2f, 0x68, 0x74, 0x6d, 0x6c,

0x0d, 0x0a, 0x0d, 0x0a

ENDFIELD(UTCPPACKET_D0,152)

/* compressed TCP Paket: dummy packet E0

| 0xf8, 0x0c | SDU length (415 Byte = 3320 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset |
| …. | Data ( 415 byte ) |

*/

FIELD(CTCPPACKET_E0)

0xf8, 0x0c,

0x00, 0x00,

0x3f, 0xbf, 0x67, 0x00, 0x01, 0x00, 0x3c, 0x68, 0x74, 0x6d, 0x6c, 0x3e, 0x0d, 0x0a, 0x3c, 0x68,

0x65, 0x61, 0x64, 0x3e, 0x0d, 0x0a, 0x0d, 0x0a, 0x3c, 0x74, 0x69, 0x74, 0x6c, 0x65, 0x3e, 0x43,

0x6f, 0x6e, 0x64, 0x61, 0x74, 0x3c, 0x2f, 0x74, 0x69, 0x74, 0x6c, 0x65, 0x3e, 0x0d, 0x0a, 0x0d,

0x0a, 0x0d, 0x0a, 0x3c, 0x73, 0x63, 0x72, 0x69, 0x70, 0x74, 0x20, 0x6c, 0x61, 0x6e, 0x67, 0x75,

0x61, 0x67, 0x65, 0x3d, 0x22, 0x4a, 0x61, 0x76, 0x61, 0x53, 0x63, 0x72, 0x69, 0x70, 0x74, 0x22,

0x3e, 0x20, 0x3c, 0x21, 0x2d, 0x2d, 0x0d, 0x0a, 0x0d, 0x0a, 0x0d, 0x0a, 0x66, 0x75, 0x6e, 0x63,

0x74, 0x69, 0x6f, 0x6e, 0x20, 0x4d, 0x4d, 0x5f, 0x73, 0x77, 0x61, 0x70, 0x49, 0x6d, 0x61, 0x67,

0x65, 0x28, 0x29, 0x20, 0x7b, 0x20, 0x2f, 0x2f, 0x76, 0x31, 0x2e, 0x30, 0x0d, 0x0a, 0x20, 0x20,

0x76, 0x61, 0x72, 0x20, 0x69, 0x2c, 0x74, 0x68, 0x65, 0x4f, 0x62, 0x6a, 0x2c, 0x6a, 0x3d, 0x30,

0x2c, 0x73, 0x77, 0x61, 0x70, 0x41, 0x72, 0x72, 0x61, 0x79, 0x3d, 0x6e, 0x65, 0x77, 0x20, 0x41,

0x72, 0x72, 0x61, 0x79, 0x2c, 0x6f, 0x6c, 0x64, 0x41, 0x72, 0x72, 0x61, 0x79, 0x3d, 0x64, 0x6f,

0x63, 0x75, 0x6d, 0x65, 0x6e, 0x74, 0x2e, 0x4d, 0x4d, 0x5f, 0x73, 0x77, 0x61, 0x70, 0x49, 0x6d,

0x67, 0x44, 0x61, 0x74, 0x61, 0x3b, 0x0d, 0x0a, 0x20, 0x20, 0x66, 0x6f, 0x72, 0x20, 0x28, 0x69,

0x3d, 0x30, 0x3b, 0x20, 0x69, 0x20, 0x3c, 0x20, 0x28, 0x4d, 0x4d, 0x5f, 0x73, 0x77, 0x61, 0x70,

0x49, 0x6d, 0x61, 0x67, 0x65, 0x2e, 0x61, 0x72, 0x67, 0x75, 0x6d, 0x65, 0x6e, 0x74, 0x73, 0x2e,

0x6c, 0x65, 0x6e, 0x67, 0x74, 0x68, 0x2d, 0x32, 0x29, 0x3b, 0x20, 0x69, 0x2b, 0x3d, 0x33, 0x29,

0x20, 0x7b, 0x0d, 0x0a, 0x20, 0x20, 0x20, 0x20, 0x74, 0x68, 0x65, 0x4f, 0x62, 0x6a, 0x20, 0x3d,

0x20, 0x65, 0x76, 0x61, 0x6c, 0x28, 0x4d, 0x4d, 0x5f, 0x73, 0x77, 0x61, 0x70, 0x49, 0x6d, 0x61,

0x67, 0x65, 0x2e, 0x61, 0x72, 0x67, 0x75, 0x6d, 0x65, 0x6e, 0x74, 0x73, 0x5b, 0x28, 0x6e, 0x61,

0x76, 0x69, 0x67, 0x61, 0x74, 0x6f, 0x72, 0x2e, 0x61, 0x70, 0x70, 0x4e, 0x61, 0x6d, 0x65, 0x20,

0x3d, 0x3d, 0x20, 0x27, 0x4e, 0x65, 0x74, 0x73, 0x63, 0x61, 0x70, 0x65, 0x27, 0x29, 0x3f, 0x69,

0x3a, 0x69, 0x2b, 0x31, 0x5d, 0x29, 0x0d, 0x0a, 0x20, 0x20, 0x20, 0x20, 0x69, 0x66, 0x20, 0x28,

0x74, 0x68, 0x65, 0x4f, 0x62, 0x6a, 0x20, 0x21, 0x3d, 0x20, 0x6e, 0x75, 0x6c, 0x6c, 0x29, 0x20,

0x7b, 0x0d, 0x0a, 0x20, 0x20, 0x20, 0x20, 0x20, 0x20, 0x73, 0x77, 0x61, 0x70, 0x41, 0x72, 0x72,

0x61, 0x79, 0x5b, 0x6a, 0x2b, 0x2b, 0x5d, 0x20, 0x3d, 0x20, 0x74, 0x68, 0x65, 0x4f, 0x62, 0x6a,

0x3b, 0x0d, 0x0a, 0x20, 0x20, 0x20, 0x20, 0x20, 0x20, 0x73, 0x77, 0x61, 0x70, 0x41, 0x72

ENDFIELD(CTCPPACKET_E0, 419)

/*

### IP frames

*/

/* HDLC Frame: IP Packet A0

| 0xc0, 0x03 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0xa1, 0xa2, 0xa3, …., 0xaf | 0xa1, 0xa2, 0xa3, …., 0xaf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPFRAME_NF_A0)	0xc0, 0x03,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0x7d, 0x020, 0x21,

0xa1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xb8,

0x7e

ENDFIELD(IPFRAME_NF_A0, 124)

/* HDLC Frame: IP Packet A0 (ACFC, PFC)

| 0x98, 0x03 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x21 | 0x21 | Protocol = IP |
| 0xa1, 0xa2, 0xa3, …., 0xaf | 0xa1, 0xa2, 0xa3, …., 0xaf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPFRAME_NF_A0_C)	0x98, 0x03,

0x00, 0x00,

0x7e,

0x21,

0xa1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xb8,

0x7e

ENDFIELD(IPFRAME_NF_A0_C, 119)

/* HDLC Frame: IP Packet B0

| 0xc0, 0x03 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0xa1, 0xa2, 0xa3, …., 0xaf | 0xa1, 0xa2, 0xa3, …., 0xaf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPFRAME_NF_B0)	0xc0, 0x03,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0x7d, 0x020, 0x21,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x2b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x3b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x4b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x5b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x6b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x7b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x8b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x9b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfc, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xbf,

0xf0, 0xb8,

0x7e

ENDFIELD(IPFRAME_NF_B0, 124)

/* HDLC Frame: IP Packet B0 (ACFC, PFC)

| 0x98, 0x03 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x21 | 0x21 | Protocol = IP |
| 0xa1, 0xa2, 0xa3, …., 0xaf | 0xa1, 0xa2, 0xa3, …., 0xaf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPFRAME_NF_B0_C)	0x98, 0x03,

0x00, 0x00,

0x7e,

0x21,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x2b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x3b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x4b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x5b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x6b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x7b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x8b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x9b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfc, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xbf,

0xf0, 0xb8,

0x7e

ENDFIELD(IPFRAME_NF_B0_C, 119)

/* HDLC Frame: IP Packet C0

| 0xc0, 0x03 |  | SDU length (120 Byte = 960 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0xc1, 0xc2, 0xc3, …., 0xcf | 0xc1, 0xc2, 0xc3, …., 0xcf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPFRAME_NF_C0)

0xc0, 0x03,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0x7d, 0x020, 0x21,

0xc1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xd1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xe1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xf1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xa1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xb1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xc1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xd1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xe1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xf1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xa1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xcf,

0xf0, 0xb8,

0x7e

ENDFIELD(IPFRAME_NF_C0, 124)

/* HDLC Frame: Three IP Packets in one Frame

| 0x38, 0x0b |  | SDU length (359 Byte = 2872 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0xa1, 0xa2,…, 0xaf | 0xa1, 0xa2,…, 0xaf | data (110 byte) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0x1b, 0xb2, …., 0xbf | 0x1b, 0xb2, 0xb3, …., 0xbf | data (110 byte) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0x1c, 0xc2, …., 0xcf | 0x1b, 0xb2, …., 0xcf | data (110 byte) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPFRAME_TRIPLE)

0x38, 0x0b,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0x7d, 0x020, 0x21,

0xa1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xb8,

0x7e,

0xff,

0x7d, 0x23,

0x7d, 0x020, 0x21,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x2b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x3b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x4b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x5b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x6b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x7b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x8b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x9b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfc, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xbf,

0xf0, 0xb8,

0x7e,

0x7e,

0xff,

0x7d, 0x23,

0x7d, 0x020, 0x21,

0xc1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xd1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xe1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xf1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xa1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xb1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xc1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xd1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xe1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xf1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xa1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xcf,

0xf0, 0xb8,

0x7e,

ENDFIELD(IPFRAME_TRIPLE, 363)

/* HDLC Frame: Three IP Packets in one Frame (ACFC, PFC)

| 0xc8, 0x0a |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x21 | 0x21 | Protocol = IP |
| 0xa1, 0xa2,…, 0xaf | 0xa1, 0xa2,…, 0xaf | data (110 byte) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |
| 0x7e |  | Flag |
| 0x21 | 0x21 | Protocol = IP |
| 0x1b, 0xb2, …., 0xbf | 0x1b, 0xb2, 0xb3, …., 0xbf | data (110 byte) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |
| 0x7e |  | Flag |
| 0x21 | 0x21 | Protocol = IP |
| 0x1c, 0xc2, …., 0xcf | 0x1b, 0xb2, …., 0xcf | data (110 byte) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FCS (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPFRAME_TRIPLE_C)	0xc8, 0x0a,

0x00, 0x00,

0x7e,

0x21,

0xa1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xb8,

0x7e,

0x7e,

0x21,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x2b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x3b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x4b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x5b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x6b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x7b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x8b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x9b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfc, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xbf,

0xf0, 0xb8,

0x7e,

0x7e,

0x21,

0xc1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xd1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xe1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xf1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xa1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xb1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xc1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xd1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xe1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xf1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xc0,

0xa1, 0xc2, 0xc3, 0xc4, 0xc5, 0xc6, 0xc7, 0xc8, 0xc9, 0xcf,

0xf0, 0xb8,

0x7e,

ENDFIELD(IPFRAME_TRIPLE_C, 349)

/* HDLC Frame: uncompressed TCP (ACFC, PFC)

| 0xa0, 0x05 |  | SDU length (180 Byte = 1440 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x00, 0x2f | 0x2f | Protocol Field (Uncompressed TCP) |
| …. |  | Data ( 175 Byte ) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FCS (dummy) |
| 0x7e |  | Flag |

*/

FIELD(UTCPFRAME_NF_D0_C)

0xa0, 0x05,

0x00, 0x00,

0x7e,

0x2f,

0x45, 0x7d, 0x20, 0x7d, 0x20, 0x94, 0x7d, 0x24, 0x7d, 0x3e, 0x40, 0x7d, 0x20, 0x75, 0x7d, 0x20,

0xd9, 0x26, 0x3e, 0x60, 0xcf, 0x7d, 0x29, 0x97, 0xbd, 0x82, 0xf8, 0x7d, 0x20, 0x50, 0x7d, 0x24,

0x7d, 0x30, 0xe2, 0x75, 0xdd, 0x43, 0x7d, 0x20, 0x7d, 0x3a, 0xbe, 0x78, 0x50, 0x7d, 0x38, 0x21,

0x7d, 0x29, 0x7d, 0x38, 0xaf, 0x7d, 0x20, 0x7d, 0x20, 0x48, 0x54, 0x54, 0x50, 0x2f, 0x31, 0x2e,

0x31, 0x20, 0x32, 0x30, 0x30, 0x20, 0x4f, 0x4b, 0x7d, 0x2d, 0x7d, 0x2a, 0x53, 0x65, 0x72, 0x76,

0x65, 0x72, 0x3a, 0x20, 0x4d, 0x69, 0x63, 0x72, 0x6f, 0x73, 0x6f, 0x66, 0x74, 0x2d, 0x49, 0x49,

0x53, 0x2f, 0x34, 0x2e, 0x30, 0x7d, 0x2d, 0x7d, 0x2a, 0x44, 0x61, 0x74, 0x65, 0x3a, 0x20, 0x46,

0x72, 0x69, 0x2c, 0x20, 0x30, 0x36, 0x20, 0x4f, 0x63, 0x74, 0x20, 0x32, 0x30, 0x30, 0x30, 0x20,

0x31, 0x32, 0x3a, 0x34, 0x35, 0x3a, 0x32, 0x38, 0x20, 0x47, 0x4d, 0x54, 0x7d, 0x2d, 0x7d, 0x2a,

0x43, 0x6f, 0x6e, 0x74, 0x65, 0x6e, 0x74, 0x2d, 0x54, 0x79, 0x70, 0x65, 0x3a, 0x20, 0x74, 0x65,

0x78, 0x74, 0x2f, 0x68, 0x74, 0x6d, 0x6c, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d, 0x2a,

0xf0, 0xb8,

0x7e

ENDFIELD(UTCPFRAME_NF_D0_C, 184)

/* HDLC Frame: compressed TCP (ACFC, PFC)

| 0x28, 0x0e |  | SDU length (453 Byte = 3624 Bit) |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x00, 0x2d | 0x2d | Protocol Field (Compressed TCP) |
| …. |  | Data ( 448 Byte ) |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FCS (dummy) |
| 0x7e |  | Flag |

*/

FIELD(CTCPFRAME_NF_E0_C)

0x28, 0x0e,

0x00, 0x00,

0x7e,

0x2d,

0x3f, 0xbf, 0x67, 0x7d, 0x20, 0x7d, 0x21, 0x7d, 0x20, 0x3c, 0x68, 0x74, 0x6d, 0x6c, 0x3e, 0x7d,

0x2d, 0x7d, 0x2a, 0x3c, 0x68, 0x65, 0x61, 0x64, 0x3e, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d,

0x2a, 0x3c, 0x74, 0x69, 0x74, 0x6c, 0x65, 0x3e, 0x43, 0x6f, 0x6e, 0x64, 0x61, 0x74, 0x3c, 0x2f,

0x74, 0x69, 0x74, 0x6c, 0x65, 0x3e, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d,

0x7d, 0x2a, 0x3c, 0x73, 0x63, 0x72, 0x69, 0x70, 0x74, 0x20, 0x6c, 0x61, 0x6e, 0x67, 0x75, 0x61,

0x67, 0x65, 0x3d, 0x22, 0x4a, 0x61, 0x76, 0x61, 0x53, 0x63, 0x72, 0x69, 0x70, 0x74, 0x22, 0x3e,

0x20, 0x3c, 0x21, 0x2d, 0x2d, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d, 0x2a, 0x7d, 0x2d, 0x7d,

0x2a, 0x66, 0x75, 0x6e, 0x63, 0x74, 0x69, 0x6f, 0x6e, 0x20, 0x4d, 0x4d, 0x5f, 0x73, 0x77, 0x61,

0x70, 0x49, 0x6d, 0x61, 0x67, 0x65, 0x28, 0x29, 0x20, 0x7b, 0x20, 0x2f, 0x2f, 0x76, 0x31, 0x2e,

0x30, 0x7d, 0x2d, 0x7d, 0x2a, 0x20, 0x20, 0x76, 0x61, 0x72, 0x20, 0x69, 0x2c, 0x74, 0x68, 0x65,

0x4f, 0x62, 0x6a, 0x2c, 0x6a, 0x3d, 0x30, 0x2c, 0x73, 0x77, 0x61, 0x70, 0x41, 0x72, 0x72, 0x61,

0x79, 0x3d, 0x6e, 0x65, 0x77, 0x20, 0x41, 0x72, 0x72, 0x61, 0x79, 0x2c, 0x6f, 0x6c, 0x64, 0x41,

0x72, 0x72, 0x61, 0x79, 0x3d, 0x64, 0x6f, 0x63, 0x75, 0x6d, 0x65, 0x6e, 0x74, 0x2e, 0x4d, 0x4d,

0x5f, 0x73, 0x77, 0x61, 0x70, 0x49, 0x6d, 0x67, 0x44, 0x61, 0x74, 0x61, 0x3b, 0x7d, 0x2d, 0x7d,

0x2a, 0x20, 0x20, 0x66, 0x6f, 0x72, 0x20, 0x28, 0x69, 0x3d, 0x30, 0x3b, 0x20, 0x69, 0x20, 0x3c,

0x20, 0x28, 0x4d, 0x4d, 0x5f, 0x73, 0x77, 0x61, 0x70, 0x49, 0x6d, 0x61, 0x67, 0x65, 0x2e, 0x61,

0x72, 0x67, 0x75, 0x6d, 0x65, 0x6e, 0x74, 0x73, 0x2e, 0x6c, 0x65, 0x6e, 0x67, 0x74, 0x68, 0x2d,

0x32, 0x29, 0x3b, 0x20, 0x69, 0x2b, 0x3d, 0x33, 0x29, 0x20, 0x7b, 0x7d, 0x2d, 0x7d, 0x2a, 0x20,

0x20, 0x20, 0x20, 0x74, 0x68, 0x65, 0x4f, 0x62, 0x6a, 0x20, 0x3d, 0x20, 0x65, 0x76, 0x61, 0x6c,

0x28, 0x4d, 0x4d, 0x5f, 0x73, 0x77, 0x61, 0x70, 0x49, 0x6d, 0x61, 0x67, 0x65, 0x2e, 0x61, 0x72,

0x67, 0x75, 0x6d, 0x65, 0x6e, 0x74, 0x73, 0x5b, 0x28, 0x6e, 0x61, 0x76, 0x69, 0x67, 0x61, 0x74,

0x6f, 0x72, 0x2e, 0x61, 0x70, 0x70, 0x4e, 0x61, 0x6d, 0x65, 0x20, 0x3d, 0x3d, 0x20, 0x27, 0x4e,

0x65, 0x74, 0x73, 0x63, 0x61, 0x70, 0x65, 0x27, 0x29, 0x3f, 0x69, 0x3a, 0x69, 0x2b, 0x31, 0x5d,

0x29, 0x7d, 0x2d, 0x7d, 0x2a, 0x20, 0x20, 0x20, 0x20, 0x69, 0x66, 0x20, 0x28, 0x74, 0x68, 0x65,

0x4f, 0x62, 0x6a, 0x20, 0x21, 0x3d, 0x20, 0x6e, 0x75, 0x6c, 0x6c, 0x29, 0x20, 0x7b, 0x7d, 0x2d,

0x7d, 0x2a, 0x20, 0x20, 0x20, 0x20, 0x20, 0x20, 0x73, 0x77, 0x61, 0x70, 0x41, 0x72, 0x72, 0x61,

0x79, 0x5b, 0x6a, 0x2b, 0x2b, 0x5d, 0x20, 0x3d, 0x20, 0x74, 0x68, 0x65, 0x4f, 0x62, 0x6a, 0x3b,

0x7d, 0x2d, 0x7d, 0x2a, 0x20, 0x20, 0x20, 0x20, 0x20, 0x20, 0x73, 0x77, 0x61, 0x70, 0x41, 0x72,

0xf0, 0xb8,

0x7e

ENDFIELD(CTCPFRAME_NF_E0_C, 457)

/*

### HDLC fragments

*/

/* HDLC Frame: IP Packet  A0 Fragment 1 (1/2)

| 0x18, 0x02 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0xa1, 0xa2, 0xa3, …. | 0xa1, 0xa2, 0xa3, …., 0xaf | data |

*/

FIELD(IPFRAME_F_A0F1)	0x18, 0x02,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0x7d, 0x020, 0x21,

0xa1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

ENDFIELD(IPFRAME_F_A0F1, 71)

/* HDLC Frame: IP Packet  A0 Fragment 1 (1/2)  (ACFC, PFC)

| 0xf0, 0x01 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x21 | 0x21 | Protocol = IP |
| 0xa1, 0xa2, 0xa3, …. | 0xa1, 0xa2, 0xa3, …., 0xaf | data |

*/

FIELD(IPFRAME_F_A0F1_C)

0xf0, 0x01,

0x00, 0x00,

0x7e,

0x21,

0xa1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

ENDFIELD(IPFRAME_F_A0F1_C, 66)

/* HDLC Frame: IP Packet  A0 Fragment 2  (2/2)

| 0xa8, 0x01 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| …., 0xaf | …., 0xaf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPFRAME_F_A0F2)	0xa8, 0x01,

0x00, 0x00,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xb8,

0x7e

ENDFIELD(IPFRAME_F_A0F2, 57)

/* HDLC Frame: IP Packet  A0 Fragment 2  (2/2) )  (ACFC, PFC)

| 0xa8, 0x01 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| …., 0xaf | …., 0xaf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPFRAME_F_A0F2_C)

0xa8, 0x01,

0x00, 0x00,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xb8,

0x7e

ENDFIELD(IPFRAME_F_A0F2_C, 57)

/* HDLC Frame: IP Packet  A0 Fragment 2  (2/2) and IP Packet B0 Framgment 1 (1/2)

| 0xb8, 0x03 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| …., 0xaf | …., 0xaf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0x1b, 0xb2, 0xb3, …. | 0x1b, 0xb2, 0xb3, …. | data |

*/

FIELD(IPFRAME_F_A0F2B0F1)	0xb8, 0x03,

0x00, 0x00,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xb8,

0x7e,

0xff,

0x7d, 0x23,

0x7d, 0x020, 0x21,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x2b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x3b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x4b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x5b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x6b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

ENDFIELD(IPFRAME_F_A0F2B0F1, 123)

/* HDLC Frame: IP Packet  A0 Fragment 2  (2/2) and IP Packet B0 Framgment 1 (1/2) (ACFC, PFC)

| 0x90, 0x03 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| …., 0xaf | …., 0xaf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |
| 0x21 | 0x21 | Protocol = IP |
| 0x1b, 0xb2, 0xb3, …. | 0x1b, 0xb2, 0xb3, …. | data |

*/

FIELD(IPFRAME_F_A0F2B0F1_C)

0x90, 0x03,

0x00, 0x00,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xb8,

0x7e,

0x21,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x2b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x3b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x4b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x5b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x6b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

ENDFIELD(IPFRAME_F_A0F2B0F1_C, 118)

/* HDLC Frame: IP Packet  A0 Fragment 2  (2/2)

| 0xa8, 0x01 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| …., 0xaf | …., 0xaf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPFRAME_F_B0F2)	0xa8, 0x01,

0x00, 0x00,

0x7b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x8b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x9b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfc, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xbf,

0xf0, 0xb8,

0x7e

ENDFIELD(IPFRAME_F_B0F2, 57)

/* HDLC Frame: IP Packet  A0 Fragment 2  (2/2) (ACFC, PFC)

| 0xa8, 0x01 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| …., 0xaf | …., 0xaf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(IPFRAME_F_B0F2_C)	0xa8, 0x01,

0x00, 0x00,

0x7b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x8b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0x9b, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfb, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xb0,

0xfc, 0xb2, 0xb3, 0xb4, 0xb5, 0xb6, 0xb7, 0xb8, 0xb9, 0xbf,

0xf0, 0xb8,

0x7e

ENDFIELD(IPFRAME_F_B0F2_C, 57)

/*

### Invalid frames

*/

/* IP Packet in HDLC Frame (invalid Control Field)

| 0xc0, 0x03 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x04 | 0x7d, 0x24 | Control Field (invalid!!!) |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0xa1, 0xa2, 0xa3, …., 0xaf | 0xa1, 0xa2, 0xa3, …., 0xaf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(INVALID_FRAME0) 0xc0, 0x03,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x24,

0x7d, 0x020, 0x21,

0xa1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xb8,

0x7e

ENDFIELD(INVALID_FRAME0, 124)

/* IP Packet in HDLC Frame (invalid protocol field)

| 0xb8, 0x03 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0x08 | 0x08 | Address Field (invalid protocol field!!!) |
| 0x20 | 0x20 | Control Field (invalid protocol field!!!) |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0xa1, 0xa2, 0xa3, …., 0xaf | 0xa1, 0xa2, 0xa3, …., 0xaf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(INVALID_FRAME1) 0xb8, 0x03,

0x00, 0x00,

0x7e,

0x08,

0x20,

0x7d, 0x020, 0x21,

0xa1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xb8,

0x7e

ENDFIELD(INVALID_FRAME1, 123)

/* IP Packet in HDLC Frame (invalid address Field)

| 0xc0, 0x03 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xf7 | 0xf7 | Address Field (invalid!!!) |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0xa1, 0xa2, 0xa3, …., 0xaf | 0xa1, 0xa2, 0xa3, …., 0xaf | data |
| 0xf0, 0xb8 | 0xf0, 0xb8 | FSC (dummy) |
| 0x7e |  | Flag |

*/

FIELD(INVALID_FRAME2) 0xc0, 0x03,

0x00, 0x00,

0x7e,

0xf7,

0x7d, 0x23,

0x7d, 0x020, 0x21,

0xa1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xb8,

0x7e

ENDFIELD(INVALID_FRAME2, 124)

/* IP Packet in HDLC Frame (invalid FSC)

| 0xc0, 0x03 |  | SDU length |
| --- | --- | --- |
| 0x00, 0x00 |  | SDU offset |
| 0x7e |  | Flag |
| 0xff | 0xff | Address Field |
| 0x03 | 0x7d, 0x23 | Control Field |
| 0x00, 0x21 | 0x7d, 0x020, 0x21 | Protocol = IP |
| 0xa1, 0xa2, 0xa3, …., 0xaf | 0xa1, 0xa2, 0xa3, …., 0xaf | data |
| 0xf0, 0xc8 | 0xf0, 0xc8 | FSC (dummy) (invalid!!!) |
| 0x7e |  | Flag |

*/

FIELD(INVALID_FSC) 0xc0, 0x03,

0x00, 0x00,

0x7e,

0xff,

0x7d, 0x23,

0x7d, 0x020, 0x21,

0xa1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf1, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xa2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xb2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xc2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xd2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xe2, 0xa2, 0xa3, 0xa4, 0xa5, 0xa6, 0xa7, 0xa8, 0xa9, 0xa0,

0xf0, 0xc8,

0x7e

ENDFIELD(INVALID_FSC, 124)

/*

### Empty sdu

*/

/* empty sdu

| 0x00, 0x00 | SDU length |
| --- | --- |
| 0x00, 0x00 | SDU offset |

*/

FIELD(EMPTY_SDU)

0x00, 0x00,

0x00, 0x00

ENDFIELD(EMPTY_SDU, 4)

/*

### PCO lists

*/

/* Protocol Configuration Options for PPP_PDP_ACTIVATE_IND with CHAP

| 0x98, 0x03 | SDU length in bit (115 Byte = 920 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x80 | Protocol PPP see GSM04.08 page 569 |
| 0xc0, 0x21 | Protocol ID1: LCP |
| 0x0d | Length of ID1: 13 |
| 0x01, 0x01, 0x00, 0x0d | Code, Identifier and Length Field: Configure-Request |
| 0x01, 0x04, 0x05, 0xdc | MRU 1500 octets |
| 0x03, 0x05, 0xc2, 0x23, 0x05 | Authentication Protocol CHAP with MD5 |
| 0xc0, 0x21 | Protocol ID2: LCP |
| 0x08 | Length of ID2: 8 |
| 0x01, 0x01, 0x00, 0x08 | Code, Identifier and Length Field: Configure-Request |
| 0x01, 0x04, 0x05, 0xdc | MRU 1500 octets |
| 0xc2, 0x23 | Protocol ID3: CHAP |
| 0x15 | Length of ID3: 21 |
| 0x01, 0x01, 0x00, 0x15 | Code, Identifier and Length Field: Challenge |
| 0x10, 0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09, 0x0a, 0x0b, 0x0c, 0x0d, 0x0e, 0x0f | Content of Challenge |
| 0xc2, 0x23 | Protocol ID4: CHAP |
| 0x1d | Length of ID4: 29 |
| 0x02, 0x01, 0x00, 0x1d | Code, Identifier and Length Field: Response |
| 0x10, 0xd1, 0xe3, 0x7f, 0x02, 0x2a, 0x0c, 0xcd, 0x13, 0x85, 0xd4, 0xe3, 0x13, 0x08, 0xc2, 0x2f, 0xd7, 0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65 | Content of Response |
| 0x80, 0x21 | Protocol ID5: IPCP |
| 0x1c | Length of ID5: 28 |
| 0x01, 0x01, 0x00, 0x1c | Code, Identifier and Length Field: Configure-Request |
| 0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01 | Van Jacobson Header Compression |
| 0x03, 0x06, 0x00, 0x00, 0x00, 0x00 | Dynamic IP-Address |
| 0x81, 0x06, 0x00, 0x00, 0x00, 0x00 | Dynamic Primary DNS Address |
| 0x83, 0x06, 0x00, 0x00, 0x00, 0x00 | Dynamic Secondary DNS Address |

*/

FIELD(PCO_LIST_IND_CHAP)

0x98, 0x03,

0x00, 0x00,

0x80,

0xc0, 0x21, 0x0d,

0x01, 0x01, 0x00, 0x0d, 0x01, 0x04, 0x05, 0xdc, 0x03, 0x05, 0xc2, 0x23, 0x05,

0xc0, 0x21, 0x08,

0x01, 0x01, 0x00, 0x08, 0x01, 0x04, 0x05, 0xdc,

0xc2, 0x23, 0x15,

0x01, 0x01, 0x00, 0x15, 0x10, 0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09, 0x0a,

0x0b, 0x0c, 0x0d, 0x0e, 0x0f,

0xc2, 0x23, 0x1d,

0x02, 0x01, 0x00, 0x1d, 0x10, 0xd1, 0xe3, 0x7f, 0x02, 0x2a, 0x0c, 0xcd, 0x13, 0x85, 0xd4, 0xe3,

0x13, 0x08, 0xc2, 0x2f, 0xd7, 0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65,

0x80, 0x21, 0x1c,

0x01, 0x01, 0x00, 0x1c, 0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01, 0x03, 0x06, 0x00, 0x00, 0x00, 0x00,

0x81, 0x06, 0x00, 0x00, 0x00, 0x00, 0x83, 0x06, 0x00, 0x00, 0x00, 0x00

ENDFIELD(PCO_LIST_IND_CHAP, 119)

/* Protocol Configuration Options for PPP_PDP_ACTIVATE_IND with PAP

| 0x98, 0x02 | SDU length in bit (83 Byte = 664 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x80 | Protocol PPP see GSM04.08 page 569 |
| 0xc0, 0x21 | Protocol ID1: LCP |
| 0x0c | Length of ID1: 12 |
| 0x01, 0x01, 0x00, 0x0c | Code, Identifier and Length Field: Configure-Request |
| 0x01, 0x04, 0x05, 0xdc | MRU 1500 octets |
| 0x03, 0x04, 0xc0, 0x23 | Authentication Protocol PAP |
| 0xc0, 0x21 | Protocol ID2: LCP |
| 0x08 | Length of ID2: 8 |
| 0x01, 0x01, 0x00, 0x08 | Code, Identifier and Length Field: Configure-Request |
| 0x01, 0x04, 0x05, 0xdc | MRU 1500 octets |
| 0xc0, 0x23 | Protocol ID3: PAP |
| 0x16 | Length of ID3: 22 |
| 0x01, 0x00, 0x00, 0x16 | Code, Identifier and Length Field: Authenticate-Request |
| 0x08, 0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65 | Length and Peer-ID |
| 0x08, 0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73 | Length and Password |
| 0x80, 0x21 | Protocol ID4: IPCP |
| 0x1c | Length of ID4: 28 |
| 0x01, 0x01, 0x00, 0x1c | Code, Identifier and Length Field: Configure-Request |
| 0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01 | Van Jacobson Header Compression |
| 0x03, 0x06, 0x00, 0x00, 0x00, 0x00 | Dynamic IP-Address |
| 0x81, 0x06, 0x00, 0x00, 0x00, 0x00 | Dynamic Primary DNS Address |
| 0x83, 0x06, 0x00, 0x00, 0x00, 0x00 | Dynamic Secondary DNS Address |

*/

FIELD(PCO_LIST_IND_PAP)

0x98, 0x02,

0x00, 0x00,

0x80,

0xc0, 0x21, 0x0c,

0x01, 0x01, 0x00, 0x0c, 0x01, 0x04, 0x05, 0xdc, 0x03, 0x04, 0xc0, 0x23,

0xc0, 0x21, 0x08,

0x01, 0x01, 0x00, 0x08, 0x01, 0x04, 0x05, 0xdc,

0xc0, 0x23, 0x16,

0x01, 0x00, 0x00, 0x16, 0x08, 0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65, 0x08, 0x74,

0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73,

0x80, 0x21, 0x1c,

0x01, 0x01, 0x00, 0x1c, 0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01, 0x03, 0x06, 0x00, 0x00, 0x00, 0x00,

0x81, 0x06, 0x00, 0x00, 0x00, 0x00, 0x83, 0x06, 0x00, 0x00, 0x00, 0x00

ENDFIELD(PCO_LIST_IND_PAP, 87)

/* Protocol Configuration Options for PPP_PDP_ACTIVATE_IND without authentication

| 0xb0, 0x01 | SDU length in bit (54 Byte = 432 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x80 | Protocol PPP see GSM04.08 page 569 |
| 0xc0, 0x21 | Protocol ID1: LCP |
| 0x08 | Length of ID1: 12 |
| 0x01, 0x01, 0x00, 0x08 | Code, Identifier and Length Field: Configure-Request |
| 0x01, 0x04, 0x05, 0xdc | MRU 1500 octets |
| 0xc0, 0x21 | Protocol ID2: LCP |
| 0x08 | Length of ID2: 8 |
| 0x01, 0x01, 0x00, 0x08 | Code, Identifier and Length Field: Configure-Request |
| 0x01, 0x04, 0x05, 0xdc | MRU 1500 octets |
| 0x80, 0x21 | Protocol ID4: IPCP |
| 0x1c | Length of ID4: 28 |
| 0x01, 0x01, 0x00, 0x1c | Code, Identifier and Length Field: Configure-Request |
| 0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01 | Van Jacobson Header Compression |
| 0x03, 0x06, 0x00, 0x00, 0x00, 0x00 | Dynamic IP-Address |
| 0x81, 0x06, 0x00, 0x00, 0x00, 0x00 | Dynamic Primary DNS Address |
| 0x83, 0x06, 0x00, 0x00, 0x00, 0x00 | Dynamic Secondary DNS Address |

*/

FIELD(PCO_LIST_IND_NO)

0xb0, 0x01,

0x00, 0x00,

0x80,

0xc0, 0x21, 0x08,

0x01, 0x01, 0x00, 0x08, 0x01, 0x04, 0x05, 0xdc,

0xc0, 0x21, 0x08,

0x01, 0x01, 0x00, 0x08, 0x01, 0x04, 0x05, 0xdc,

0x80, 0x21, 0x1c,

0x01, 0x01, 0x00, 0x1c, 0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01, 0x03, 0x06, 0x00, 0x00, 0x00, 0x00,

0x81, 0x06, 0x00, 0x00, 0x00, 0x00, 0x83, 0x06, 0x00, 0x00, 0x00, 0x00

ENDFIELD(PCO_LIST_IND_NO, 58)

/* Protocol Configuration Options for PPP_PDP_ACTIVATE_IND with CHAP

| 0x68, 0x03 | SDU length in bit (109 Byte = 872 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x80 | Protocol PPP see GSM04.08 page 569 |
| 0xc0, 0x21 | Protocol ID1: LCP |
| 0x0d | Length of ID1: 13 |
| 0x01, 0x01, 0x00, 0x0d | Code, Identifier and Length Field: Configure-Request |
| 0x01, 0x04, 0x05, 0xdc | MRU 1500 octets |
| 0x03, 0x05, 0xc2, 0x23, 0x05 | Authentication Protocol CHAP with MD5 |
| 0xc0, 0x21 | Protocol ID2: LCP |
| 0x08 | Length of ID2: 8 |
| 0x01, 0x01, 0x00, 0x08 | Code, Identifier and Length Field: Configure-Request |
| 0x01, 0x04, 0x05, 0xdc | MRU 1500 octets |
| 0xc2, 0x23 | Protocol ID3: CHAP |
| 0x15 | Length of ID3: 21 |
| 0x01, 0x01, 0x00, 0x15 | Code, Identifier and Length Field: Challenge |
| 0x10, 0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09, 0x0a, 0x0b, 0x0c, 0x0d, 0x0e, 0x0f | Content of Challenge |
| 0xc2, 0x23 | Protocol ID4: CHAP |
| 0x1d | Length of ID4: 29 |
| 0x02, 0x01, 0x00, 0x1d | Code, Identifier and Length Field: Response |
| 0x10, 0xd1, 0xe3, 0x7f, 0x02, 0x2a, 0x0c, 0xcd, 0x13, 0x85, 0xd4, 0xe3, 0x13, 0x08, 0xc2, 0x2f, 0xd7, 0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65 | Content of Response |
| 0x80, 0x21 | Protocol ID5: IPCP |
| 0x16 | Length of ID5: 22 |
| 0x01, 0x01, 0x00, 0x16 | Code, Identifier and Length Field: Configure-Request |
| 0x03, 0x06, 0x00, 0x00, 0x00, 0x00 | Dynamic IP-Address |
| 0x81, 0x06, 0x00, 0x00, 0x00, 0x00 | Dynamic Primary DNS Address |
| 0x83, 0x06, 0x00, 0x00, 0x00, 0x00 | Dynamic Secondary DNS Address |

*/

FIELD(PCO_LIST_IND_CHAP_NOVJ)

0x68, 0x03,

0x00, 0x00,

0x80,

0xc0, 0x21, 0x0d,

0x01, 0x01, 0x00, 0x0d, 0x01, 0x04, 0x05, 0xdc, 0x03, 0x05, 0xc2, 0x23, 0x05,

0xc0, 0x21, 0x08,

0x01, 0x01, 0x00, 0x08, 0x01, 0x04, 0x05, 0xdc,

0xc2, 0x23, 0x15,

0x01, 0x01, 0x00, 0x15, 0x10, 0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09, 0x0a,

0x0b, 0x0c, 0x0d, 0x0e, 0x0f,

0xc2, 0x23, 0x1d,

0x02, 0x01, 0x00, 0x1d, 0x10, 0xd1, 0xe3, 0x7f, 0x02, 0x2a, 0x0c, 0xcd, 0x13, 0x85, 0xd4, 0xe3,

0x13, 0x08, 0xc2, 0x2f, 0xd7, 0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65,

0x80, 0x21, 0x16,

0x01, 0x01, 0x00, 0x16, 0x03, 0x06, 0x00, 0x00, 0x00, 0x00, 0x81, 0x06, 0x00, 0x00, 0x00, 0x00,

0x83, 0x06, 0x00, 0x00, 0x00, 0x00

ENDFIELD(PCO_LIST_IND_CHAP_NOVJ, 113)

/* Protocol Configuration Options for PPP_PDP_ACTIVATE_IND with CHAP and static IP and DNS addresses

| 0x98, 0x03 | SDU length in bit (115 Byte = 920 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x80 | Protocol PPP see GSM04.08 page 569 |
| 0xc0, 0x21 | Protocol ID1: LCP |
| 0x0d | Length of ID1: 13 |
| 0x01, 0x01, 0x00, 0x0d | Code, Identifier and Length Field: Configure-Request |
| 0x01, 0x04, 0x05, 0xdc | MRU 1500 octets |
| 0x03, 0x05, 0xc2, 0x23, 0x05 | Authentication Protocol CHAP with MD5 |
| 0xc0, 0x21 | Protocol ID2: LCP |
| 0x08 | Length of ID2: 8 |
| 0x01, 0x01, 0x00, 0x08 | Code, Identifier and Length Field: Configure-Request |
| 0x01, 0x04, 0x05, 0xdc | MRU 1500 octets |
| 0xc2, 0x23 | Protocol ID3: CHAP |
| 0x15 | Length of ID3: 21 |
| 0x01, 0x01, 0x00, 0x15 | Code, Identifier and Length Field: Challenge |
| 0x10, 0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09, 0x0a, 0x0b, 0x0c, 0x0d, 0x0e, 0x0f | Content of Challenge |
| 0xc2, 0x23 | Protocol ID4: CHAP |
| 0x1d | Length of ID4: 29 |
| 0x02, 0x01, 0x00, 0x1d | Code, Identifier and Length Field: Response |
| 0x10, 0xd1, 0xe3, 0x7f, 0x02, 0x2a, 0x0c, 0xcd, 0x13, 0x85, 0xd4, 0xe3, 0x13, 0x08, 0xc2, 0x2f, 0xd7, 0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65 | Content of Response |
| 0x80, 0x21 | Protocol ID5: IPCP |
| 0x1c | Length of ID5: 28 |
| 0x01, 0x01, 0x00, 0x1c | Code, Identifier and Length Field: Configure-Request |
| 0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01 | Van Jacobson Header Compression |
| 0x03, 0x06, 0x8d, 0x40, 0x15, 0x02 | IP-Address 141.64.21.2 |
| 0x81, 0x06, 0x8d, 0x40, 0x18, 0x81 | Primary DNS Address 141.64.24.129 |
| 0x83, 0x06, 0x8d, 0x40, 0xfe, 0x83 | Secondary DNS Address 141.64.254.131 |

*/

FIELD(PCO_LIST_IND_CHAP_STATIC)

0x98, 0x03,

0x00, 0x00,

0x80,

0xc0, 0x21, 0x0d,

0x01, 0x01, 0x00, 0x0d, 0x01, 0x04, 0x05, 0xdc, 0x03, 0x05, 0xc2, 0x23, 0x05,

0xc0, 0x21, 0x08,

0x01, 0x01, 0x00, 0x08, 0x01, 0x04, 0x05, 0xdc,

0xc2, 0x23, 0x15,

0x01, 0x01, 0x00, 0x15, 0x10, 0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09, 0x0a,

0x0b, 0x0c, 0x0d, 0x0e, 0x0f,

0xc2, 0x23, 0x1d,

0x02, 0x01, 0x00, 0x1d, 0x10, 0xd1, 0xe3, 0x7f, 0x02, 0x2a, 0x0c, 0xcd, 0x13, 0x85, 0xd4, 0xe3,

0x13, 0x08, 0xc2, 0x2f, 0xd7, 0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65,

0x80, 0x21, 0x1c,

0x01, 0x01, 0x00, 0x1c, 0x02, 0x06, 0x00, 0x2d, 0x0f, 0x01, 0x03, 0x06, 0x8d, 0x40, 0x15, 0x02,

0x81, 0x06, 0x8d, 0x40, 0x18, 0x81, 0x83, 0x06, 0x8d, 0x40, 0xfe, 0x83

ENDFIELD(PCO_LIST_IND_CHAP_STATIC, 119)

/* Protocol Configuration Options for PPP_PDP_ACTIVATE_RES

| 0xd0, 0x00 | SDU length in bit (26 Byte = 208 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x80 | Protocol PPP see GSM04.08 page 569 |
| 0x80, 0x21 | Protocol ID1: IPCP |
| 0x16 | Length of ID1: 22 |
| 0x03, 0x01, 0x00, 0x16 | Code, Identifier and Length Field: Configure-NAK |
| 0x03, 0x06, 0x8d, 0x40, 0x66, 0x03 | IP-Address 141.64.102.3 |
| 0x81, 0x06, 0x8d, 0x40, 0x18, 0x81 | Primary DNS Address 141.64.24.129 |
| 0x83, 0x06, 0x8d, 0x40, 0xfe, 0x83 | Secondary DNS Address 141.64.254.131 |

*/

FIELD(PCO_LIST_RES)

0xd0, 0x00,

0x00, 0x00,

0x80,

0x80, 0x21, 0x16,

0x03, 0x01, 0x00, 0x16, 0x03, 0x06, 0x8d, 0x40, 0x66, 0x03, 0x81, 0x06, 0x8d, 0x40, 0x18, 0x81,
	0x83, 0x06, 0x8d, 0x40, 0xfe, 0x83

ENDFIELD(PCO_LIST_RES, 30)

/* Protocol Configuration Options for PPP_PDP_ACTIVATE_RES with sdu offset

| 0xd0, 0x00 | SDU length in bit (26 Byte = 208 Bit) |
| --- | --- |
| 0x08, 0x00 | SDU offset in bit (1 Byte = 8 Bit) |
| 0x00 | Offset octet |
| 0x80 | Protocol PPP see GSM04.08 page 569 |
| 0x80, 0x21 | Protocol ID1: IPCP |
| 0x16 | Length of ID1: 22 |
| 0x03, 0x01, 0x00, 0x16 | Code, Identifier and Length Field: Configure-NAK |
| 0x03, 0x06, 0x8d, 0x40, 0x66, 0x03 | IP-Address 141.64.102.3 |
| 0x81, 0x06, 0x8d, 0x40, 0x18, 0x81 | Primary DNS Address 141.64.24.129 |
| 0x83, 0x06, 0x8d, 0x40, 0xfe, 0x83 | Secondary DNS Address 141.64.254.131 |

*/

FIELD(PCO_LIST_RES_OFFSET)

0xd0, 0x00,

0x08, 0x00,

0x00,

0x80,

0x80, 0x21, 0x16,

0x03, 0x01, 0x00, 0x16, 0x03, 0x06, 0x8d, 0x40, 0x66, 0x03, 0x81, 0x06, 0x8d, 0x40, 0x18, 0x81,
	0x83, 0x06, 0x8d, 0x40, 0xfe, 0x83

ENDFIELD(PCO_LIST_RES_OFFSET, 31)

/* Protocol Configuration Options for PPP_PDP_ACTIVATE_RES empty list

| 0x00, 0x00 | SDU length in bit (0 Byte = 0 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |

*/

FIELD(PCO_LIST_RES_EMPTY)

0x00, 0x00,

0x00, 0x00,

ENDFIELD(PCO_LIST_RES_EMPTY, 4)

/*

## Declarations

*/

DECLARATION(AUTH_STRUCT0)

DECLARATION (LOGIN_NAME)

DECLARATION (LOGIN_PASSWORD)

DECLARATION (PEER_CHANNEL0)

DECLARATION (PEER_CHANNEL1)

DECLARATION (PROTOCOL_CHANNEL0)

DECLARATION (PROTOCOL_CHANNEL1)

DECLARATION (DTI_CHANNELNAME_UART)

DECLARATION (DTI_CHANNELNAME_IP)

DECLARATION (DTI_CHANNELNAME_L2R)

DECLARATION (DTI_CHANNELNAME_MMI)

DECLARATION (SMREG_QOS_DUMMY)

DECLARATION (SMREG_MIN_QOS_DUMMY)

DECLARATION (PDP_ADDRESS_DUMMY)

DECLARATION (PDP_ADDRESS_BUF_DUMMY)

DECLARATION (SMREG_APN_DUMMY)

DECLARATION (SMREG_APN_BUF_DUMMY)

DECLARATION ( DTI_PARAMETER_PEER )

DECLARATION ( DTI_PARAMETER_PROT)

DECLARATION ( DTI_PARAMETER_PROT2)

DECLARATION ( DTI_PARAMETER_PROT3)

DECLARATION ( DTI_PARA_ST_LINES_1)

/*

## Arrays

*/

/* Login Name

| 0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65 | name value ("testname") |
| --- | --- |

*/

BEGINARRAY(LOGIN_NAME, LOGIN_NAME_LEN)

0x74, 0x65, 0x73, 0x74, 0x6e, 0x61, 0x6d, 0x65

ENDARRAY

/* Login Password

| 0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73 | name value ("testpass") |
| --- | --- |

*/

BEGINARRAY(LOGIN_PASSWORD, LOGIN_PASSWORD_LEN)

0x74, 0x65, 0x73, 0x74, 0x70, 0x61, 0x73, 0x73

ENDARRAY

/* DTI Communication Channel Name MMI

| 0x4d, 0x4d, 0x49, 0x00, 0x00, 0x0 | name value ("MMI\0") |
| --- | --- |

*/

BEGINARRAY(DTI_CHANNELNAME_MMI, 6)

0x4d, 0x4d, 0x49, 0x00, 0x00, 0x00

ENDARRAY

/* DTI Communication Channel Name UART

| 0x55, 0x41, 0x52, 0x54, 0x00, 0x0 | name value ("UART\0") |
| --- | --- |

*/

BEGINARRAY(DTI_CHANNELNAME_UART, 6)

0x55, 0x41, 0x52, 0x54, 0x00, 0x00

ENDARRAY

/* DTI Communication Channel Name IP

| 0x55, 0x41, 0x52, 0x54, 0x00, 0x0 | name value ("IP\0") |
| --- | --- |

*/

BEGINARRAY(DTI_CHANNELNAME_IP, 6)

0x49, 0x50, 0x00, 0x00, 0x00, 0x00

ENDARRAY

/* DTI Communication Channel Name L2R

| 0x55, 0x41, 0x52, 0x54, 0x00, 0x0 | name value ("L2R\0") |
| --- | --- |

*/

BEGINARRAY(DTI_CHANNELNAME_L2R, 6)

0x4C, 0x32, 0x52, 0x00, 0x00, 0x00

ENDARRAY

/* PDP address dummy

| 0x8d, 0x40, 0x15, 0x02 | PDP address 141.64.21.2 |
| --- | --- |

*/

BEGINARRAY(PDP_ADDRESS_BUF_DUMMY, 4)

0x8d, 0x40, 0x15, 0x02

ENDARRAY

/* smreg access point name dummy

| 0x41, 0x50, 0x4e | name value ("APN\0") |
| --- | --- |

*/

BEGINARRAY(SMREG_APN_BUF_DUMMY, 3)

0x41, 0x50, 0x4e

ENDARRAY

/*

## Structs

*/

BEGIN_PSTRUCT ("st_lines", DTI_PARA_ST_LINES_1)
            SET_COMP ("st_flow",	  DTI_FLOW_ON)
	SET_COMP ("st_line_sa",	  DTI_SA_ON)
	SET_COMP ("st_line_sb",	  DTI_SB_ON)
	SET_COMP ("st_break_len", DTI_BREAK_OFF)
ENDSTRUCT

BEGIN_PSTRUCT ("parameters",  DTI_PARAMETER_PEER)
	SET_COMP ("p_id",	DTI_PID_UOS)
	SET_COMP ("st_lines",	DTI_PARA_ST_LINES_1)
ENDSTRUCT

BEGIN_PSTRUCT ("parameters",  DTI_PARAMETER_PROT)
	SET_COMP ("p_id",	DTI_PID_IP)
	SET_COMP ("st_lines",	DTI_PARA_ST_LINES_1)
ENDSTRUCT

BEGIN_PSTRUCT ("parameters",  DTI_PARAMETER_PROT2)
	SET_COMP ("p_id",	DTI_PID_UTCP)
	SET_COMP ("st_lines",	DTI_PARA_ST_LINES_1)
ENDSTRUCT

BEGIN_PSTRUCT ("parameters",  DTI_PARAMETER_PROT3)
	SET_COMP ("p_id",	DTI_PID_CTCP)
	SET_COMP ("st_lines",	DTI_PARA_ST_LINES_1)
ENDSTRUCT

BEGIN_PSTRUCT("login", AUTH_STRUCT0)

SET_COMP("name_len", LOGIN_NAME_LEN)

SET_COMP("name", LOGIN_NAME)

SET_COMP("password_len", LOGIN_PASSWORD_LEN)

SET_COMP("password", LOGIN_PASSWORD)

ENDSTRUCT

BEGIN_PSTRUCT("peer_channel", PEER_CHANNEL0)

SET_COMP("peer_entity", DTI_CHANNELNAME_UART)

ENDSTRUCT

BEGIN_PSTRUCT("peer_channel", PEER_CHANNEL1)

SET_COMP("peer_entity", DTI_CHANNELNAME_L2R)

ENDSTRUCT

BEGIN_PSTRUCT("protocol_channel", PROTOCOL_CHANNEL0)

SET_COMP("protocol_entity", DTI_CHANNELNAME_MMI)

ENDSTRUCT

BEGIN_PSTRUCT("protocol_channel", PROTOCOL_CHANNEL1)

SET_COMP("protocol_entity", DTI_CHANNELNAME_IP)

ENDSTRUCT

# TEST CASES

## Setup (PPP001 - PPP010)

### PPP001:  Setup

Description:

Initialization of TAP

Preamble:

None

MMI / HIGHER LAYER		PPP		LOWER LAYER

|		|		|

COMMAND (TAP RESET)

COMMAND (MMI RESET)

COMMAND (UART RESET)

COMMAND (PPP RESET)

|		|		 |

COMMAND (TAP REDIRECT CLEAR)

COMMAND (MMI REDIRECT CLEAR)

COMMAND (UART REDIRECT CLEAR)

COMMAND (PPP REDIRECT CLEAR)

|		|		  |

COMMAND (PPP REDIRECT UART TAP)

COMMAND (PPP REDIRECT MMI TAP)

|		|		  |

COMMAND (TAP REDIRECT TAP PPP)

|		|		  |

- Parametrization:

Primitive	Parameter	Value

History:

17-Jun-2000	SG	Initial

## Link Establishment Requests (PPP011 – PPP019)

### PPP011:  Link establishment in client mode (default parameters)

Description:

Establishment in client mode whithout any authentification.

Preamble:

PPP001

MMI / HIGHER LAYER		PPP		LOWER LAYER

|		|		|

(1)	|	PPP_ESTABLISH_REQ	|		|

|	(client mode)	|		|

*	==========================>	*		|

- (2)	|	DTI2_CONNECT_IND	|		|
	*	<==========================	*		|

- (3)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (4)	|	DTI2_CONNECT_RES	|		|
	*	==========================>	*		|

(5)	|	PPP_DTI_CONNECTED_IND	|		|

|	(Prot)	|		|

*	<==========================	*		|

- (6)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

(7)	|	PPP_DTI_CONNECTED_IND	|		|

|	(Peer)	|		|

*	<==========================	*		|

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(9)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- PPP_ESTABLISH_REQ
	mode	PPP_CLIENT
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_NO
	login	AUTH_STRUCT0
	accm	PPP_ACCM_DEFAULT
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	PPP_HC_OFF
	ip	PPP_IP_DYNAMIC
	dns1	PPP_DNS1_DYNAMIC
	dns2	PPP_DNS2_DYNAMIC
	peer_channel	PEER_CHANNEL0
	protocol_channel	PROTOCOL_CHANNEL0
	peer_direction	DTI_CHANNEL_TO_LOWER_LAYER
	prot_direction	DTI_CHANNEL_TO_HIGHER_LAYER
	peer_link_id	LINK_ID_PEER
	prot_link_id	LINK_ID_PROT

- DTI2_CONNECT_IND
	link_id	LINK_ID_PROT
	version	DTI_VERSION_10

- DTI2_CONNECT_REQ
		link_id	LINK_ID_PEER
		version	DTI_VERSION_10

- DTI2_CONNECT_RES
		link_id	LINK_ID_PROT
		version	DTI_VERSION_10

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- DTI2_CONNECT_CNF
		link_id	LINK_ID_PEER
		version	DTI_VERSION_10

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

History:

17-Jun-2000	SG	Initial

22-Juni-2001	XOF	Channel ID and connect primitive added.

03-Juli-2001	XOF	Position of Synchronisation Primitive
changed

12-Jul-2001	STW	remove NOT_USED values

13-Sept-2001	XOF	Build DTILIB functionality

### PPP012:  Link establishment in client mode (PAP)

Description:

Establishment in client mode whithout any authentification.

Preamble:

PPP001

MMI / HIGHER LAYER		PPP		LOWER LAYER

|		|		|

(1)	|	PPP_ESTABLISH_REQ	|		|

|	(client mode)	|		|

*	==========================>	*		|

- (2)	|	DTI2_CONNECT_IND	|		|
	*	<==========================	*		|

- (3)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (4)	|	DTI2_CONNECT_RES	|		|
	*	==========================>	*		|

(5)	|	PPP_DTI_CONNECTED_IND	|		|

|	(Prot)	|		|

*	<==========================	*		|

- (6)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

(7)	|	PPP_DTI_CONNECTED_IND	|		|

|	(Peer)	|		|

*	<==========================	*		|

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(9)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- PPP_ESTABLISH_REQ
	mode	PPP_CLIENT
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_PAP
	login	AUTH_STRUCT0
	accm	PPP_ACCM_DEFAULT
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	PPP_HC_OFF
	ip	PPP_IP_DYNAMIC
	dns1	PPP_DNS1_DYNAMIC
	dns2	PPP_DNS2_DYNAMIC
	peer_channel	PEER_CHANNEL0
	protocol_channel	PROTOCOL_CHANNEL0
	peer_direction	DTI_CHANNEL_TO_LOWER_LAYER
	prot_direction	DTI_CHANNEL_TO_HIGHER_LAYER
	peer_link_id	LINK_ID_PEER
	prot_link_id	LINK_ID_PROT

- DTI2_CONNECT_IND
	link_id	LINK_ID_PROT
	version	DTI_VERSION_10

- DTI2_CONNECT_REQ
		link_id	LINK_ID_PEER
		version	DTI_VERSION_10

- DTI2_CONNECT_RES
		link_id	LINK_ID_PROT
		version	DTI_VERSION_10

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- DTI2_CONNECT_CNF
		link_id	LINK_ID_PEER
		version	DTI_VERSION_10

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

History:

17-Jun-2000	SG	Initial

22-Jun-2001	XOF	Channel ID and connect primitive added.

12-Jul-2001	STW	Remove NOT_USED values.

13-Sept-2001	XOF	Build DTILIB functionality

### PPP013:  Link establishment in server mode

Description:

<A>Establishment in server mode whith automatic authentication protocol negotiation.

<B>Establishment in server mode whith CHAP authentication.

<C>Establishment in server mode whith PAP authentication.

<D>Establishment in server mode whithout any authentication.

<E>Establishment in server mode whith automatic authentication and Async-Control-Character-Map = 0.

<F>Establishment in server mode whith MRU = 2000 and ACCM = 0.

Variants:

<A>....<F>

Preamble:

PPP001

MMI / HIGHER LAYER		PPP		LOWER LAYER

|		|		|

- (1)	|		|	DTI2_CONNECT_IND	|

- |		*	<==========================	*

(2)	|	PPP_ESTABLISH_REQ	|		|

|	(server mode)	|		|

*	==========================>	*		|

- (3)	|	DTI2_CONNECT_IND	|		|

- *	<==========================	*		|

- (4)	|		|	DTI2_CONNECT_RES	|

- |		*	==========================>	*

(5)	|	PPP_DTI_CONNECTED_IND	|		|

|	(Peer)	|		|

*	<==========================	*		|

- (6)	|	DTI2_CONNECT_RES	|		|

- *	==========================>	*		|

(7)	|	PPP_DTI_CONNECTED_IND	|		|

|	(Prot)	|		|

*	<==========================	*		|

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(9)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_CONNECT_IND
	link_id	LINK_ID_PEER
	version	DTI_VERSION_10

- PPP_ESTABLISH_REQ
	mode	PPP_SERVER
<A>	mru	PPP_MRU_DEFAULT
<B>	mru	PPP_MRU_DEFAULT
<C>	mru	PPP_MRU_DEFAULT
<D>	mru	PPP_MRU_DEFAULT
<E>	mru	PPP_MRU_DEFAULT
<F>	mru	PPP_MRU_2000
<A>	ap	PPP_AP_AUTO
<B>	ap	PPP_AP_CHAP
<C>	ap	PPP_AP_PAP
<D>	ap	PPP_AP_NO
<E>	ap	PPP_AP_AUTO
<F>	ap	PPP_AP_AUTO
	login	AUTH_STRUCT0
<A>	accm	PPP_ACCM_DEFAULT
<B>	accm	PPP_ACCM_DEFAULT
<C>	accm	PPP_ACCM_DEFAULT
<D>	accm	PPP_ACCM_DEFAULT
<E>	accm	PPP_ACCM_0
<F>	accm	PPP_ACCM_0
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	PPP_HC_OFF
	ip	PPP_IP_DYNAMIC
	dns1	PPP_DNS1_DYNAMIC
	dns2	PPP_DNS2_DYNAMIC
	peer_channel	PEER_CHANNEL0
	protocol_channel	PROTOCOL_CHANNEL0
	peer_direction	DTI_CHANNEL_TO_LOWER_LAYER
	prot_direction	DTI_CHANNEL_TO_HIGHER_LAYER
	peer_link_id	LINK_ID_PEER
	prot_link_id	LINK_ID_PROT

- DTI2_CONNECT_IND
	link_id	LINK_ID_PROT
	version	DTI_VERSION_10

- DTI2_CONNECT_RES
	link_id	LINK_ID_PEER
	version	DTI_VERSION_10

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- DTI2_CONNECT_RES
	link_id	LINK_ID_PROT
	version	DTI_VERSION_10

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

History:

10-Oct-2000	STW	Initial

22-Jun-2001	XOF	Channel ID and connect primitive added.

12-Jul-2001	STW	Remove NOT_USED values.

13-Sep-2001	XOF	Build DTILIB functionality

24-Oct-2002	STW	Add variant (MRU = 2000, ACCM = 0).

### PPP014:  Link establishment in transparent mode

Description:

Establishment in transparent mode.

Preamble:

PPP001

MMI / HIGHER LAYER		PPP		LOWER LAYER

|		|		|

(1)	|	PPP_ESTABLISH_REQ	|		|

|	(transparent mode)	|		|

*	==========================>	*		|

- (2)	|	DTI2_CONNECT_IND	|		|
	*	<==========================	*		|

- (3)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (4)	|	DTI2_CONNECT_RES	|		|
	*	==========================>	*		|

(5)	|	PPP_DTI_CONNECTED_IND	|		|

|	(Prot)	|		|

*	<==========================	*		|

- (6)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

(7)	|	PPP_DTI_CONNECTED_IND	|		|

|	(Peer)	|		|

*	<==========================	*		|

(8)	|	PPP_ESTABLISH_CNF	|		|

*	<==========================	*		|

(9)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(10)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(11)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(12)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- PPP_ESTABLISH_REQ
	mode	PPP_TRANSPARENT
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_NO
	login	AUTH_STRUCT0
	accm	PPP_ACCM_DEFAULT
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	PPP_HC_OFF
	ip	PPP_IP_DYNAMIC
	dns1	PPP_DNS1_DYNAMIC
	dns2	PPP_DNS2_DYNAMIC
	peer_channel	PEER_CHANNEL0
	protocol_channel	PROTOCOL_CHANNEL0
	peer_direction	DTI_CHANNEL_TO_LOWER_LAYER
	prot_direction	DTI_CHANNEL_TO_HIGHER_LAYER
	peer_link_id	LINK_ID_PEER
	prot_link_id	LINK_ID_PROT

- DTI2_CONNECT_IND
		link_id	LINK_ID_PROT
		version	DTI_VERSION_10

- DTI2_CONNECT_REQ
		link_id	LINK_ID_PEER
		version	DTI_VERSION_10

- DTI2_CONNECT_RES
	link_id	LINK_ID_PROT
	version	DTI_VERSION_10

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- DTI2_CONNECT_CNF
	link_id	LINK_ID_PEER
	version	DTI_VERSION_10

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_DUMMY
	ip	PPP_IP_DYNAMIC
	dns1	PPP_DNS1_DYNAMIC
	dns2	PPP_DNS2_DYNAMIC

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PROT

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_READY_IND
	link_id	LINK_ID_PEER

History:

20-Oct-2000	STW	Initial

22-Jun-2001	XOF	Channel ID and connect primitive added.

12-Jul-2001	STW	Remove NOT_USED values.

13-Sept-2001	XOF	Build DTILIB functionality

### PPP015:  Link establishment in client mode (PAP), only peer dti channel opened

Description:

Establishment in client mode whithout any authentification.

Preamble:

PPP001

MMI / HIGHER LAYER		PPP		LOWER LAYER

|		|		|

(1)	|	PPP_ESTABLISH_REQ	|		|

|	(client mode)	|		|

*	==========================>	*		|

- (2)	|	DTI2_CONNECT_IND	|		|
	*	<==========================	*		|

- (3)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (4)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

(5)	|	PPP_DTI_CONNECTED_IND	|		|

|	(Peer)	|		|

*	<==========================	*		|

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(7)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- PPP_ESTABLISH_REQ
	mode	PPP_CLIENT
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_PAP
	login	AUTH_STRUCT0
	accm	PPP_ACCM_DEFAULT
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	PPP_HC_OFF
	ip	PPP_IP_DYNAMIC
	dns1	PPP_DNS1_DYNAMIC
	dns2	PPP_DNS2_DYNAMIC
	peer_channel	PEER_CHANNEL0
	protocol_channel	PROTOCOL_CHANNEL0
	peer_direction	DTI_CHANNEL_TO_LOWER_LAYER
	prot_direction	DTI_CHANNEL_TO_HIGHER_LAYER
	peer_link_id	LINK_ID_PEER
	prot_link_id	LINK_ID_PROT

- DTI2_CONNECT_IND
	link_id	LINK_ID_PROT
	version	DTI_VERSION_10

- DTI2_CONNECT_REQ
	link_id	LINK_ID_PEER
	version	DTI_VERSION_10

- DTI2_CONNECT_CNF
	link_id	LINK_ID_PEER
	version	DTI_VERSION_10

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

History:

25-Apr-2002	TVO	Initial

## LCP Establishment (PPP020-PPP029)

### PPP020: LCP Establishment (Client, PAP)

Description:

Peer configures PAP.

Variants:

<A>....<B>

Preamble:

<A>PPP012

<B>PPP015

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfReqest1)	|

|		*	==========================>	*

(2)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(3)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfRequest4)	|

|		*	<==========================	*

(4)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfAck4)	|

|		*	==========================>	*

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(7)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfAck1)	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest1

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest4

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfAck4

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfAck1

History:

17-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added

15-Sept-2001	XOF	Build DTILIB functionality

### PPP021: LCP Establishment (CLient, PAP, ACFC, PFC)

Description:

Peer configures PAP,ACFC (Address-and-Control-Field-Compression) am PFC (Protocol-Field-Compression).

Preamble:

PPP012

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfReqest1)	|

|		*	==========================>	*

(2)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(3)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfRequest6)	|

|		*	<==========================	*

(4)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfAck6)	|

|		*	==========================>	*

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(7)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfAck1)	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest1

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest6

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfAck6

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfAck1

History:

17-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP022: LCP Establishment (CLient, PAP, ACFC, PFC) with magic number request

Description:

Peer configures PAP,ACFC (Address-and-Control-Field-Compression) am PFC (Protocol-Field-Compression). Magic number is rejected by PPP client.

Preamble:

PPP012

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfRequestMag)	|

|		*	<==========================	*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfRequest1)	|

|		*	==========================>	*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(4)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCPConfigureRejMag)	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(7)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfRequestAccm)	|

|		*	<==========================	*

(8)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfAccAccm)	|

|		*	==========================>	*

(9)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(10)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(11)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfAck1)	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequestMag

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest1

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCPConfigureRejMag

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequestAccm

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfAccAccm

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfAck1

History:

26-Sep-2000	MT	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP023: LCP Establishment (Server, CHAP, ACFC, PFC) with magic number request

Description:

Server configures

<A>CHAP

<B>CHAP

<C>PAP

<D>no authentication
<E>CHAP and ACCM=0

and ACFC (Address-and-Control-Field-Compression) am PFC (Protocol-Field-Compression). Magic number is rejected by PPP server.

Variants:

<A>....<E>

Preamble:

<A>PPP013A

<B>PPP013B

<C>PPP013C

<D>PPP013D

<E>PPP013E

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfRequest50)	|

|		*	==========================>	*

(2)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfRequest51)	|

|		*	<==========================	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfReject51)	|

|		*	==========================>	*

(6)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfAck50)	|

|		*	<==========================	*

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(9)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfRequest53)	|

|		*	<==========================	*

(10)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfAck53)	|

|		*	==========================>	*

(11)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(12)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	LCP_ConfRequest50
<B>	sdu	LCP_ConfRequest50
<C>	sdu	LCP_ConfRequest66
<D>	sdu	LCP_ConfRequest68
<E>	sdu	LCP_ConfRequest74

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest51

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfReject51

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	LCP_ConfAck50
<B>	sdu	LCP_ConfAck50
<C>	sdu	LCP_ConfAck66
<D>	sdu	LCP_ConfAck68
<E>	sdu	LCP_ConfAck74

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest53

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfAck53

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

History:

10-Oct-2000	STW	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP024: LCP Establishment (Server) re-request of rejected configuration options

Description:

Re-request of already rejected configuration options

<A>PFC

<B>ACFC

<C>MRU

<D>ACCM

<E>Authentication Protocol

Variants:

<A>....<E>

Preamble:

<A>PPP013C

<B>PPP013C

<C>PPP013F

<D>PPP013F

<E>PPP013F

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfRequest66)	|

|		*	==========================>	*

(2)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfRequest51)	|

|		*	<==========================	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfReject51)	|

|		*	==========================>	*

(6)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfReject66)	|

|		*	<==========================	*

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(9)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfRequest76)	|

|		*	==========================>	*

(10)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfRequest53)	|

|		*	<==========================	*

(11)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(12)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(13)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfAck53)	|

|		*	==========================>	*

(14)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfNAK76)	|

|		*	<==========================	*

(15)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(16)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(17)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfRequest80)	|

|		*	==========================>	*

(18)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(19)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfAck80)	|

|		*	<==========================	*

(20)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	LCP_ConfRequest66
<B>	sdu	LCP_ConfRequest66
<C>	sdu	LCP_ConfRequest82
<D>	sdu	LCP_ConfRequest82
<E>	sdu	LCP_ConfRequest82

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest51

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfReject51

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	LCP_ConfReject66A
<B>	sdu	LCP_ConfReject66B
<C>	sdu	LCP_ConfReject82A
<D>	sdu	LCP_ConfReject82B
<E>	sdu	LCP_ConfReject82C

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	LCP_ConfRequest76
<B>	sdu	LCP_ConfRequest78
<C>	sdu	LCP_ConfRequest84
<D>	sdu	LCP_ConfRequest86
<E>	sdu	LCP_ConfRequest90

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest53

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfAck53

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	LCP_ConfNAK76
<B>	sdu	LCP_ConfNAK78
<C>	sdu	LCP_ConfNAK84
<D>	sdu	LCP_ConfNAK86
<E>	sdu	LCP_ConfNAK90

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	LCP_ConfRequest80
<B>	sdu	LCP_ConfRequest80
<C>	sdu	LCP_ConfRequest88
<D>	sdu	LCP_ConfRequest88
<E>	sdu	LCP_ConfRequest88

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	LCP_ConfAck80
<B>	sdu	LCP_ConfAck80
<C>	sdu	LCP_ConfAck88
<D>	sdu	LCP_ConfAck88
<E>	sdu	LCP_ConfAck88

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

History:

15-Aug-2001	STW	Initial

24-Oct-2002	STW	Adaptation to DTI2

## Authentication Procedure (PPP030-PPP039)

### PPP030: PAP Authentication in Client Mode

Description:

PAP authentication in client mode without Address-and-Control-Field-Compression and
Protocol-Field-Compression

Variants:

<A>....<B>

Preamble:

<A>PPP020A

<B>PPP020B

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(AuthRequest0)	|

|		*	==========================>	*

(2)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(AuthAck0)	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	AuthRequest0

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	AuthAck0

History:

26-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP031: PAP Authentication in Client Mode (ACFC, PFC)

Description:

PAP authentication in client mode with Address-and-Control-Field-Compression and
Protocol-Field-Compression

Preamble:

PPP021

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(AuthRequest0_C)	|

|		*	==========================>	*

(2)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(AuthAck0_C)	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	AuthRequest0_C

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	AuthAck0_C

History:

26-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP032: PAP authtentication failed (client mode)

Description:

PAP authentication in client mode, but the server NAK's authentication and terminates the link.

Preamble:

PPP020A

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(AuthRequest0)	|

|		*	==========================>	*

(2)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(AuthNack0)	|

|		*	<==========================	*

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(AuthRequest1)	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_TermReq0)	|

|		*	<==========================	*

(9)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_TermAck0)	|

|		*	==========================>	*

(10)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(11)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

TIMEOUT(3000)

(11)	|	DTI2_DISCONNECT_IND	|		|

*	<==========================	*		|

(12)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(13)	|	PPP_TERMINATE_IND	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	AuthRequest0

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	AuthNack0

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	AuthRequest1

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_TermReq0

- DTI2_DATA_TEST_REQ	
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_TermAck0

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DISCONNECT_IND
	link_id	LINK_ID_PROT
	cause	DTI_CAUSE_NORMAL_CLOSE

- DTI2_DISCONNECT_REQ
	link_id	LINK_ID_PEER
	cause	DTI_CAUSE_NORMAL_CLOSE

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_USE_AUTHED_FAILED

History:

22-Aug-2000	SG	Initial
22-Jun-2001	XOF	Channel ID added
15-Sep-2001	XOF	Build DTILIB functionality

24-Oct-2002	STW	Cause concept

### PPP033: CHAP Authentication in Server Mode (ACFC, PFC) with LCP Identification

Description:

CHAP authentication in server mode. The client sends LCP Identification packets which will be rejected.
<A>with ACFC, PFC
<B>with ACFC, PFC
<C>with ACFC, PFC and ACCM=0

Variants:

<A>....<C>

Preamble:

<A>PPP023A

<B>PPP023B

<C>PPP023E

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CHAP_Challenge54)	|

|		*	==========================>	*

(2)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_Identification55)	|

|		*	<==========================	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_Identification57)	|

|		*	<==========================	*

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(7)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_CodeReject55)	|

|		*	==========================>	*

(8)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CHAP_Response54)	|

|		*	<==========================	*

(9)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(10)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(11)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_CodeReject57)	|

|		*	==========================>	*

(12)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(13)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CHAP_Success54)	|

|		*	==========================>	*

(14)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	CHAP_Challenge54
<B>	sdu	CHAP_Challenge54
<C>	sdu	CHAP_Challenge76

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	LCP_Identification55
<B>	sdu	LCP_Identification55
<C>	sdu	LCP_Identification77

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	LCP_Identification57
<B>	sdu	LCP_Identification57
<C>	sdu	LCP_Identification79

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_CodeReject55

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	CHAP_Response54
<B>	sdu	CHAP_Response54
<C>	sdu	CHAP_Response76

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_CodeReject57

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	CHAP_Success54
<B>	sdu	CHAP_Success54
<C>	sdu	CHAP_Success76

- DTI2_READY_IND
	link_id	LINK_ID_PEER

History:

10-Oct-2000	STW	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP034: PAP Authentication in Server Mode (ACFC, PFC) with LCP Identification

Description:

PAP authentication in server mode. The client sends LCP Identification packets which will be rejected.

Preamble:

PPP023C

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_Identification55)	|

|		*	<==========================	*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_CodeReject55)	|

|		*	==========================>	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_Identification57)	|

|		*	<==========================	*

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|		|	DTI2_DATA_TEST_IND	|

|		|	(PAP_AuthRequest67)	|

|		*	<==========================	*

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(9)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_CodeReject57)	|

|		*	==========================>	*

(10)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(11)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(PAP_AuthAck67)	|

|		*	==========================>	*

(12)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_Identification55

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_CodeReject55

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_Identification57

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	PAP_AuthRequest67

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_CodeReject57

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	PAP_AuthAck67

- DTI2_READY_IND
	link_id	LINK_ID_PEER

History:

13-Oct-2000	STW	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

## IPCP Establishment (PPP40-PPP049)

### PPP040: Normal IPCP Address negotiation

Description:

Successful IPCP negotiation procedure.

Variants:

<A>....<B>

Preamble:

<A>PPP030A

<B>PPP030B

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPReq0)	|

|		*	==========================>	*

(2)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPReq2)	|

|		*	<==========================	*

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPAck2)	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPNack0)	|

|		*	<==========================	*

(9)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPReq1)	|

|		*	==========================>	*

(10)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(11)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(12)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPAck1)	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq0

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq2

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPAck2

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPNack0

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq1

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPAck1

History:

26-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP041: Normal IPCP Address negotiation (ACFC, PFC)

Description:

Successful IPCP negotiation procedure, ACFC and PFC enabled.

Preamble:

PPP031

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPReq0_C)	|

|		*	==========================>	*

(2)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPReq2_C)	|

|		*	<==========================	*

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPAck2_C)	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPNack0_C)	|

|		*	<==========================	*

(9)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPReq1_C)	|

|		*	==========================>	*

(10)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(11)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(12)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPAck1_C)	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq0_C

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq2_C

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPAck2_C

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPNack0_C

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq1_C

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPAck1_C

History:

26-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP042: CCP request from server during ICPC procedure

Description:

PPP receives a CCP (Compression Control Protocol) configure request and sends a LCP protocol reject. The IPCP negotiation procedure continues.

Preamble:

PPP030A

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPReq0)	|

|		*	==========================>	*

(2)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPReq2)	|

|		*	<==========================	*

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPAck2)	|

|		*	==========================>	*

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(7)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(8)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CCPConfReq0)	|

|		*	<==========================	*

(9)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCPProtoRej0)	|

|		*	==========================>	*

(10)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(11)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(12)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPNack0)	|

|		*	<==========================	*

(13)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPReq1)	|

|		*	==========================>	*

(14)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(15)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(16)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPAck1)	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq0

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq2

- DTI2_DATA_TEST_REQ	
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPAck2

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	CCPConfReq0

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCPProtoRej0

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPNack0

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq1

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPAck1

History:

22-Aug-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added

15-Sept-2001	XOF	Build DTILIB functionality

### PPP043: IPCP configuration in server mode with CCP

Description:

IPCP configuration in server mode. PPP receives a CCP (Compression Control Protocol) configure request and sends a LCP protocol reject. The IPCP negotiation procedure continues.
<A>automatic CHAP
<B>CHAP
<C>PAP with PCO sesponse with offset
<D>whithout authentication
<E>without Header Compression – Peer initiated
<F>without Header Compression – MMI initiated
<G>CHAP with ACCM=0

Variants:

<A>....<G>

Preamble:

<A>PPP033A

<B>PPP033B

<C>PPP034

<D>PPP023D

<E>PPP033A

<F>PPP033B

<G>PPP033C

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CCP_ConfRequest59)	|

|		*	<==========================	*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ProtoReject59)	|

|		*	==========================>	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfRequest61)	|

|		*	<==========================	*

(6)	|	PPP_PDP_ACTIVATE_IND	|		|

*	<==========================	*		|

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|	PPP_PDP_ACTIVATE_RES	|		|

*	==========================>	*		|

(9)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfRequest62)	|

|		*	==========================>	*

(10)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(11)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfReject61)	|

|		*	==========================>	*

(12)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(13)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfAck62)	|

|		*	<==========================	*

(14)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(15)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfRequest63)	|

|		*	<==========================	*

(16)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfNAK63)	|

|		*	==========================>	*

(17)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(18)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(19)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfRequest65)	|

|		*	<==========================	*

(20)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfAck65)	|

|		*	==========================>	*

(21)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(22)	|	PPP_ESTABLISH_CNF	|		|

*	<==========================	*		|

(23)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(24)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(25)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	CCP_ConfRequest59
<B>	sdu	CCP_ConfRequest59
<C>	sdu	CCP_ConfRequest59
<D>	sdu	CCP_ConfRequest59
<E>	sdu	CCP_ConfRequest59
<F>	sdu	CCP_ConfRequest59
<G>	sdu	CCP_ConfRequest81

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ProtoReject59

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	IPCP_ConfRequest61
<B>	sdu	IPCP_ConfRequest61
<C>	sdu	IPCP_ConfRequest61
<D>	sdu	IPCP_ConfRequest61
<E>	sdu	IPCP_ConfRequest69
<F>	sdu	IPCP_ConfRequest61
<G>	sdu	IPCP_ConfRequest83

- PPP_PDP_ACTIVATE_IND
<A>	ppp_hc	PPP_HC_VJ
<B>	ppp_hc	PPP_HC_VJ
<C>	ppp_hc	PPP_HC_VJ
<D>	ppp_hc	PPP_HC_VJ
<E>	ppp_hc	PPP_HC_OFF
<F>	ppp_hc	PPP_HC_VJ
<G>	ppp_hc	PPP_HC_VJ
<A>	msid	MSID_EXPECTED
<B>	msid	MSID_EXPECTED
<C>	msid	MSID_EXPECTED
<D>	msid	MSID_EXPECTED
<E>	msid	MSID_DUMMY
<F>	msid	MSID_EXPECTED
<G>	msid	MSID_EXPECTED
<A>	sdu	PCO_LIST_IND_CHAP
<B>	sdu	PCO_LIST_IND_CHAP
<C>	sdu	PCO_LIST_IND_PAP
<D>	sdu	PCO_LIST_IND_NO
<E>	sdu	PCO_LIST_IND_CHAP_NOVJ
<F>	sdu	PCO_LIST_IND_CHAP
<G>	sdu	PCO_LIST_IND_CHAP

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- PPP_PDP_ACTIVATE_RES
<A>	ppp_hc	PPP_HC_VJ
<B>	ppp_hc	PPP_HC_VJ
<C>	ppp_hc	PPP_HC_VJ
<D>	ppp_hc	PPP_HC_VJ
<E>	ppp_hc	PPP_HC_OFF
<F>	ppp_hc	PPP_HC_OFF
<G>	ppp_hc	PPP_HC_VJ
<A>	msid	MSID_EXPECTED
<B>	msid	MSID_EXPECTED
<C>	msid	MSID_EXPECTED
<D>	msid	MSID_EXPECTED
<E>	msid	MSID_DUMMY
<F>	msid	MSID_DUMMY
<G>	msid	MSID_EXPECTED
	ip	PPP_IP_141_64_21_2
<A>	sdu	PCO_LIST_RES
<B>	sdu	PCO_LIST_RES
<C>	sdu	PCO_LIST_RES_OFFSET
<D>	sdu	PCO_LIST_RES
<E>	sdu	PCO_LIST_RES
<F>	sdu	PCO_LIST_RES
<G>	sdu	PCO_LIST_RES

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	IPCP_ConfRequest62
<B>	sdu	IPCP_ConfRequest62
<C>	sdu	IPCP_ConfRequest62
<D>	sdu	IPCP_ConfRequest62
<E>	sdu	IPCP_ConfRequest70
<F>	sdu	IPCP_ConfRequest70
<G>	sdu	IPCP_ConfRequest84

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	IPCP_ConfReject61
<B>	sdu	IPCP_ConfReject61
<C>	sdu	IPCP_ConfReject61
<D>	sdu	IPCP_ConfReject61
<E>	sdu	IPCP_ConfReject69
<F>	sdu	IPCP_ConfReject61F
<G>	sdu	IPCP_ConfReject83

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	IPCP_ConfAck62
<B>	sdu	IPCP_ConfAck62
<C>	sdu	IPCP_ConfAck62
<D>	sdu	IPCP_ConfAck62
<E>	sdu	IPCP_ConfAck70
<F>	sdu	IPCP_ConfAck70
<G>	sdu	IPCP_ConfAck84

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	IPCP_ConfRequest63
<B>	sdu	IPCP_ConfRequest63
<C>	sdu	IPCP_ConfRequest63
<D>	sdu	IPCP_ConfRequest63
<E>	sdu	IPCP_ConfRequest71
<F>	sdu	IPCP_ConfRequest71
<G>	sdu	IPCP_ConfRequest85

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	IPCP_ConfNAK63
<B>	sdu	IPCP_ConfNAK63
<C>	sdu	IPCP_ConfNAK63
<D>	sdu	IPCP_ConfNAK63
<E>	sdu	IPCP_ConfNAK71
<F>	sdu	IPCP_ConfNAK71
<G>	sdu	IPCP_ConfNAK85

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	IPCP_ConfRequest65
<B>	sdu	IPCP_ConfRequest65
<C>	sdu	IPCP_ConfRequest65
<D>	sdu	IPCP_ConfRequest65
<E>	sdu	IPCP_ConfRequest73
<F>	sdu	IPCP_ConfRequest73
<G>	sdu	IPCP_ConfRequest87

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	IPCP_ConfAck65
<B>	sdu	IPCP_ConfAck65
<C>	sdu	IPCP_ConfAck65
<D>	sdu	IPCP_ConfAck65
<E>	sdu	IPCP_ConfAck73
<F>	sdu	IPCP_ConfAck73
<G>	sdu	IPCP_ConfAck87

- DTI2_READY_IND
	link_id	LINK_ID_PROT

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
<A>	ppp_hc	PPP_HC_VJ
<B>	ppp_hc	PPP_HC_VJ
<C>	ppp_hc	PPP_HC_VJ
<D>	ppp_hc	PPP_HC_VJ
<E>	ppp_hc	PPP_HC_OFF
<F>	ppp_hc	PPP_HC_OFF
<G>	ppp_hc	PPP_HC_VJ
<A>	msid	MSID_EXPECTED
<B>	msid	MSID_EXPECTED
<C>	msid	MSID_EXPECTED
<D>	msid	MSID_EXPECTED
<E>	msid	MSID_DUMMY
<F>	msid	MSID_DUMMY
<G>	msid	MSID_EXPECTED
	ip	PPP_IP_141_64_21_2
	dns1	PPP_PDNS_141_64_24_129
	dns2	PPP_SDNS_141_64_254_131

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

12-Oct-2000	STW	Initial

22-Jun-2001	XOF	Channel ID added.

12-Jul-2001	STW	Add PCO with offset.
15-Sep-2001	XOF	Build DTILIB functionality

25-Oct-2002	STW	Cause concept

### PPP044: PDP activation will be rejected by the network

Description:

IPCP configuration in server mode. PPP receives a CCP (Compression Control Protocol) configure request and sends a LCP protocol reject. The IPCP negotiation procedure continues, but PDP activation will be rejected by the network.

Preamble:

PPP033A

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CCP_ConfRequest59)	|

|		*	<==========================	*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ProtoReject59)	|

|		*	==========================>	*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(4)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfRequest61)	|

|		*	<==========================	*

(6)	|	PPP_PDP_ACTIVATE_IND	|		|

*	<==========================	*		|

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(9)	|	PPP_PDP_ACTIVATE_REJ	|		|

*	==========================>	*		|

(10)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_TermReq90)	|

|		*	==========================>	*

(11)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(12)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_TermAck90)	|

|		*	<==========================	*

(13)	|	DTI2_DISCONNECT_IND	|		|

*	<==========================	*		|

(14)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(15)	|	PPP_TERMINATE_IND	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	CCP_ConfRequest59

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ProtoReject59

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfRequest61

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_EXPECTED
	sdu	PCO_LIST_IND_CHAP

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- PPP_PDP_ACTIVATE_REJ
	ppp_cause	PPP_TERM_INSUF_RES

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_TermRequest90

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_TermAck90

- DTI2_DISCONNECT_IND
	link_id	LINK_ID_PROT
	cause	DTI_CAUSE_NORMAL_CLOSE

- DTI2_DISCONNECT_REQ
	link_id	LINK_ID_PEER
	cause	DTI_CAUSE_NORMAL_CLOSE

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_INSUF_RES

History:

18-Oct-2000	STW	Initial

22-Juni-2001	XOF	Channel ID added

03-Juli-2001	XOF	LCP TermAck corrected
15-Sept-2001	XOF	Build DTILIB functionality

25-Oct-2002	STW	Cause concept

## Link Establishment Confirmation (PPP050-PPP059)

### PPP050: Confiramtion of  link establishment in client mode

Description:

PPP sends PPP_ESTABLISH_CNF after link establishment without Address-and-Control-Field-Compression and Protocol-Field-Compression in client mode.

Preamble:

PPP040A

MMI/UPLINK		PPP		DOWNLINK

|		|		|

(1)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(2)	|	PPP_ESTABLISH_CNF	|		|

*	<==========================	*		|

(3)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_READY_IND
	link_id	LINK_ID_PROT

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_DUMMY
	ip	IP_ADDR
	dns1	PPP_DNS1_DYNAMIC
	dns2	PPP_DNS2_DYNAMIC

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

26-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP051: Open Flow Control for Downlink

Description:

After receptin of an DTI2_GETDATA_REQ from UPLINK (PPP050) PPP must send an DTI2_GETDATA_REQ to the DOWNLINK entity.

Variants:

<A>....<B>

Preamble:

<A>PPP050

<B>PPP055

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

History:

26-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP052: Confiramtion of  link establishment in client mode (ACFC, PFC)

Description:

PPP sends PPP_ESTABLISH_CNF after link establishment with Address-and-Control-Field-Compression and Protocol-Field-Compression in client mode.

Preamble:

PPP041

MMI/UPLINK		PPP		DOWNLINK

|		|		|

(1)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(2)	|	PPP_ESTABLISH_CNF	|		|

*	<==========================	*		|

(3)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_READY_IND
	link_id	LINK_ID_PROT

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_DUMMY
	ip	IP_ADDR
	dns1	PPP_DNS1_DYNAMIC
	dns2	PPP_DNS2_DYNAMIC

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

26-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP053: Open Flow Control for Downlink (ACFC, PFC)

Description:

After receptin of an DTI2_GETDATA_REQ from UPLINK (PPP050) PPP must send an DTI2_GETDATA_REQ to the DOWNLINK entity.

Preamble:

PPP052

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

History:

26-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP055: Confirmation of link establishment in client mode and acknowledgement of prot dti connection

Description:

PPP sends PPP_ESTABLISH_CNF after link establishment without Address-and-Control-Field-Compression and Protocol-Field-Compression in client mode.

Preamble:

PPP040B

MMI/UPLINK		PPP		DOWNLINK

|		|		|

(1)	|	PPP_ESTABLISH_CNF	|		|

*	<==========================	*		|

- (2)	|	DTI2_CONNECT_RES	|		|
	*	==========================>	*		|

(3)	|	PPP_DTI_CONNECTED_IND	|		|

|	(Prot)	|		|

*	<==========================	*		|

(4)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(5)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_DUMMY
	ip	IP_ADDR
	dns1	PPP_DNS1_DYNAMIC
	dns2	PPP_DNS2_DYNAMIC

- DTI2_CONNECT_RES
		link_id	LINK_ID_PROT
		version	DTI_VERSION_10

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- DTI2_READY_IND
	link_id	LINK_ID_PROT

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

26-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

## Uplink Transfer (PPP060-PPP69)

### PPP060: IP Packet in HDLC Frame

Description:

Convert an HDLC frame to an IP packet.

Variants:

<A>....<B>

Preamble:

<A>PPP051A

<B>PPP051B

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_NF_A0)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_NF_A0)	|

|		*	<==========================	*

(6)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(7)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(9)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_NF_A0)	|

|		*	<==========================	*

(10)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(11)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(12)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_NF_A0

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_NF_A0

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_NF_A0

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

History:

17-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality
28-Jan-2002	STW	Change Flow COntrol order

### PPP061: IP Packet in two  HDLC Frames

Description:

Convert two HDLC frames to one IP packet.

Preamble:

PPP051A

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_F_A0F1)	|

|		*	<==========================	*

(2)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(3)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_F_A0F2)	|

|		*	<==========================	*

(4)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_F_A0F1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_F_A0F2

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

17-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality
28-Jan-2002	STW	Change Flow Control order

### PPP062: Two Packets in three Frames

Description:

Convert three HDCL frames to two IP packets

Preamble:

PPP051A

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_F_A0F1)	|

|		*	<==========================	*

(2)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(3)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_F_A0F2B0F1)	|

|		*	<==========================	*

(4)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_F_B0F2)	|

|		*	<==========================	*

(7)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(8)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKTE_B0)	|		|

*	<==========================	*		|

(9)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(10)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_F_A0F1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_F_A0F2B0F1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_F_B0F2

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_B0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

17-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality
28-Jan-2002	STW	Change Flow Control order

### PPP063: IP Packet in HDLC Frame (ACFC, PFC)

Description:

Convert HDLC frame with Address-and-Control-Field-Compression and Protocol-Field-Compression to an
IP packet

Variants:

<A>....<B>

Preamble:

<A>PPP043A

<B>PPP053

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_NF_A0_C)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(4)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(3)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_NF_A0_C

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

17-Jun-2000	SG	Initial

17-Oct-2000	STW	add server mode preambles
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality
28-Jan-2002	STW	Change Flow Control order

### PPP064: IP Packet in two  HDLC Frames (ACFC, PFC)

Description:

Convert two HDLC packts with Address-and-Control-Field-Compression and Protocol-Field-Compression to an IP packet.

Variants:

<A>....<D>

Preamble:

<A>PPP063A

<B>PPP063B

<C>PPP043A

<D>PPP053

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_F_A0F1_C)	|

|		*	<==========================	*

(2)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(3)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_F_A0F2_C)	|

|		*	<==========================	*

(4)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_F_A0F1_C

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_F_A0F2_C

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

17-Jun-2000	SG	Initial

17-Oct-2000	STW	add server mode preables
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality
28-Jan-2002	STW	Change Flow Control order

### PPP065: Two Packets in three Frames (ACFC, PFC)

Description:

Convert three HDLC frames with Address-and-Control-Field-Compression and Protocol-Field-Compression to two IP packts.

Variants:

<A>....<F>

Preamble:

<A>PPP064A

<B>PPP064B

<C>PPP064C

<D>PPP064D

<E>PPP043A

<F>PPP053

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_F_A0F1_C)	|

|		*	<==========================	*

(2)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(3)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_F_A0F2B0F1_C)	|

|		*	<==========================	*

(4)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(7)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_F_B0F2_C)	|

|		*	<==========================	*

(8)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKTE_B0)	|		|

*	<==========================	*		|

(9)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(10)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_F_A0F1_C

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_F_A0F2B0F1_C

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
		sdu	IPPACKET_A0

- DTI2_GETDATA_REQ	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_F_B0F2_C

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
		sdu	IPPACKET_B0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

17-Jun-2000	SG	Initial

17-Oct-2000	STW	add server mode preambles
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality
28-Jan-2002	STW	Change Flow Control order

### PPP066: Three Packets in one Frames

Description:

Convert one HDLC frame to three IP packts.

Preamble:

PPP051A

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_TRIPLE)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(3)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(4)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_B0)	|		|

*	<==========================	*		|

(5)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(6)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_C0)	|		|

*	<==========================	*		|

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_TRIPLE

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_B0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_C0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

20-Sep-2000	MT	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality
28-Jan-2002	STW	Change Flow Control order

### PPP067: Three Packets in one Frame (ACFC,PFC)

Description:

Convert one HDLC frame with Address-and-Control-Field-Compression and Protocol-Field-Compression to three IP packts.

Variants:

<A>....<H>

Preamble:

<A>PPP065A

<B>PPP065B

<C>PPP065C

<D>PPP065D

<E>PPP065E

<F>PPP065F

<G>PPP043A

<H>PPP053

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_TRIPLE_C)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(3)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(4)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_B0)	|		|

*	<==========================	*		|

(5)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(6)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_C0)	|		|

*	<==========================	*		|

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_TRIPLE_C

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_B0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_C0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- History:

20-Sep-2000	MT	Initial

17-Oct-2000	STW	add server mode preambles
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality
28-Jan-2002	STW	Change Flow Control order

### PPP068: TCP Packets without Header Compression Negotiation (ACFC,PFC)

Description:

PPP peer sends compressed and uncompressed TCP packets, but Header Compression was not negotiated in IPCP. So these packets will be rejected by LCP-Protocol-Reject.

Variants:

<A>....<B>

Preamble:

<A>PPP067B

<B>PPP043E

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(UTCPFRAME_NF_D0_C)	|

|		*	<==========================	*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ProtoReject_D0)	|

|		*	==========================>	*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(4)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CTCPFRAME_NF_E0_C)	|

|		*	<==========================	*

(6)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ProtoReject_E0)	|

|		*	==========================>	*

(7)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	UTCPFRAME_NF_D0_C

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	LCP_ProtoReject_D0
<B>	sdu	LCP_ProtoReject_D0B

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	CTCPFRAME_NF_E0_C

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	LCP_ProtoReject_E0
<B>	sdu	LCP_ProtoReject_E0B

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

History:

19-Oct-2000	STW	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP069: TCP Packets with Header Compression Negotiation (ACFC,PFC)

Description:

PPP peer sends compressed and uncompressed TCP packets. Header Compression was negotiated in IPCP  so these packets will be pass to the upper layer.

Variants:

<A>....<B>

Preamble:

<A>PPP067A

<B>PPP043A

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(UTCPFRAME_NF_D0_C)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(UTCPPACKET_D0)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CTCPFRAME_NF_E0_C)	|

|		*	<==========================	*

(6)	|	DTI2_DATA_TEST_IND	|		|

|	(CTCPPACKET_E0)	|		|

*	<==========================	*		|

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	UTCPFRAME_NF_D0_C

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT2
	sdu	UTCPPACKET_D0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	CTCPFRAME_NF_E0_C

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT3
	sdu	CTCPPACKET_E0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

20-Oct-2000	STW	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality
28-Jan-2002	STW	Change Flow Control order

## Downlink Transfer (PPP070-PPP79)

### PPP070: Simple Downlink transfer

Description:

Convert an IP packt to a HDLC frame.

Preamble:

PPP051A

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|	DTI2_DATA_TEST_REQ	|		|

|	(IPPACKET_B0)	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPFRAME_NF_B0)	|

|		*	==========================>	*

(3)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_B0

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_NF_B0

- DTI2_READY_IND
	link_id	LINK_ID_PROT

- DTI2_READY_IND
	link_id	LINK_ID_PEER

History:

17-Jun-2000	SG	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality
28-Jan-2002	STW	Change Flow Control order

### PPP071: Simple Downlink transfer (ACFC, PFC)

Description:

Convert an IP packet to a HDLC frame with Address-and-Control-Field-Compression and
Protocol-Field-Compression

Variants:

<A>....<J>

Preamble:

<A>PPP067A

<B>PPP067B

<C>PPP067C

<D>PPP067D

<E>PPP067E

<F>PPP067F

<G>PPP067G

<H>PPP067H

<I>PPP043A

<J>PPP053

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|	DTI2_DATA_TEST_REQ	|		|

|	(IPPACKET_B0)	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPFRAME_NF_B0_C)	|

|		*	==========================>	*

(3)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_B0

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_NF_B0_C

- DTI2_READY_IND
	link_id	LINK_ID_PROT

- DTI2_READY_IND
	link_id	LINK_ID_PEER

History:

17-Jun-2000	SG	Initial

17-Oct-2000	STW	add server mode preables
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality
28-Jan-2002	STW	Change Flow Control order

## PPP Termination (PPP080-PPP083)

### PPP080: Termination initiated by MMI, lower layer available

Description:

Successful termination of PPP while lower layer is available. PPP inform peer about termination.

Variants:

<A>....<C>

Preamble:

<A>PPP071A

<B>PPP071B

<C>PPP062

MMI		PPP		DOWNLINK

|		|		|

(1)	|	PPP_TERMINATE_REQ	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_TermReq0)	|

|		*	==========================>	*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_TermAck0)	|

|		*	<==========================	*

(5)	|	DTI2_DISCONNECT_IND	|		|

*	<==========================	*		|

(6)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(7)	|	PPP_TERMINATE_IND	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_UP

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_TermReq0

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_TermAck0

- DTI2_DISCONNECT_IND
	link_id	LINK_ID_PROT
	cause	DTI_CAUSE_NORMAL_CLOSE

- DTI2_DISCONNECT_REQ
	link_id	LINK_ID_PEER
	cause	DTI_CAUSE_NORMAL_CLOSE

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

History:

17-Jun-2000	SG	Initial

18-Oct-2000	STW	add server mode preambles
22-Jun-2001	XOF	Channel ID added
15-Sep-2001	XOF	Build DTILIB functionality

25-Oct-2002	STW	Cause concept

### PPP081: Termination initiated by MMI, lower layer not available

Description:

Successful termination of PPP while lower layer is not available.

Variants:

<A>....<C>

Preamble:

<A>PPP071A

<B>PPP071B

<C>PPP062

MMI		PPP		DOWNLINK

|		|		|

(1)	|	PPP_TERMINATE_REQ	|		|

*	==========================>	*		|

(2)	|	DTI2_DISCONNECT_IND	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|	PPP_TERMINATE_IND	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_DOWN

- DTI2_DISCONNECT_IND
	link_id	LINK_ID_PROT
	cause	DTI_CAUSE_NORMAL_CLOSE

- DTI2_DISCONNECT_REQ
	link_id	LINK_ID_PEER
	cause	DTI_CAUSE_NORMAL_CLOSE

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

History:

17-Jun-2000	SG	Initial

18-Oct-2000	STW	add server mode preambles
15-Sep-2001	XOF	Build DTILIB functionality

25-Oct-2002	STW	Cause concept

### PPP082: Termination initiated by PPP Peer

Description:

PPP peer desire to terminate the PPP link, PPP informs MMI that the PPP link is terminated.

Variants:

<A>....<C>

Preamble:

<A>PPP071A

<B>PPP071B

<C>PPP062

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_TermReq0)	|
	|		*	<=========================	*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_TermAck0)	|

|		*	==========================>	*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(4)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

TIMEOUT(3000)

(5)	|	DTI2_DISCONNECT_IND	|		|

*	<==========================	*		|

(6)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(7)	|	PPP_TERMINATE_IND	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_TermReq0

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_TermAck0

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DISCONNECT_IND
	link_id	LINK_ID_PROT
	cause	DTI_CAUSE_NORMAL_CLOSE

- DTI2_DISCONNECT_REQ
	link_id	LINK_ID_PEER
	cause	DTI_CAUSE_NORMAL_CLOSE

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_PEER

History:

17-Jun-2000	SG	Initial

18-Oct-2000	STW	add server mode preambles
22-Jun-2001	XOF	Channel ID added
15-Sep-2001	XOF	Build DTILIB functionality

25-Oct-2002	STW	Cause concept

## Uplink Transfer with invalid packets (PPP090-PPP099)

### PPP090:  one frame with invalid control field received

Description:

PPP discards invalid packet and sends “get data request” immediately.

Variants:

<A>....<D>

Preamble:

<A>PPP071A

<B>PPP071B

<C>PPP043A

<D>PPP060A

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(INVALID_FRAME0)	|

|		*	<==========================	*

(2)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(3)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_NF_A0)	|

|		*	<==========================	*

(4)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(5)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	INVALID_FRAME0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	IPFRAME_NF_A0_C
<B>	sdu	IPFRAME_NF_A0
<C>	sdu	IPFRAME_NF_A0_C
<D>	sdu	IPFRAME_NF_A0

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

History:

19-Sep-2000	MT	Initial

18-Oct-2000	STW	add server mode primitives
22-Jun-2001	XOF	Channel ID added
15-Sep-2001	XOF	Build DTILIB functionality

### PPP091:  one frame with invalid protocol field received

Description:

PPP discards invalid packet and sends “get data request” immediately

Variants:

<A>....<D>

Preamble:

<A>PPP090A

<B>PPP090B

<C>PPP043A

<D>PPP060A

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(INVALID_FRAME1)	|

|		*	<==========================	*

(2)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(3)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_NF_A0)	|

|		*	<==========================	*

(4)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(5)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	INVALID_FRAME1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	IPFRAME_NF_A0_C
<B>	sdu	IPFRAME_NF_A0
<C>	sdu	IPFRAME_NF_A0_C
<D>	sdu	IPFRAME_NF_A0

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

History:

19-Sep-2000	MT	Initial

18-Oct-2000	STW	add server mode primitives
22-Jun-2001	XOF	Channel ID added
15-Sep-2001	XOF	Build DTILIB functionality

### PPP092:  one frame with invalid address field received

Description:

PPP sends LCP protocol reject

Variants:

<A>....<D>

Preamble:

<A>PPP091A

<B>PPP091B

<C>PPP043A

<D>PPP060A

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(INVALID_FRAME2)	|

|		*	<==========================	*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCPProtoRej1)	|

|		*	==========================>	*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(4)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_NF_A0)	|

|		*	<==========================	*

(6)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(7)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	INVALID_FRAME2

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	LCP_ProtoReject88
<B>	sdu	LCPProtoRej1
<C>	sdu	LCP_ProtoReject88
<D>	sdu	LCPProtoRej1

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	IPFRAME_NF_A0_C
<B>	sdu	IPFRAME_NF_A0
<C>	sdu	IPFRAME_NF_A0_C
<D>	sdu	IPFRAME_NF_A0

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

History:

20-Sep-2000	MT	Initial

18-Oct-2000	STW	add server mode primitves
22-Jun-2001	XOF	Channel ID added
15-Sep-2001	XOF	Build DTILIB functionality

### PPP093:  one frame with invalid FSC received

Description:

PPP silently discards invalid packet and sends “get data request” immediately

Variants:

<A>....<D>

Preamble:

<A>PPP092A

<B>PPP092B

<C>PPP043A

<D>PPP060A

UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(INVALID_FSC)	|

|		*	<==========================	*

(2)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(3)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_NF_A0)	|

|		*	<==========================	*

(4)	|	DTI2_DATA_TEST_IND	|		|

|	(IPPACKET_A0)	|		|

*	<==========================	*		|

(5)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	INVALID_FSC

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
<A>	sdu	IPFRAME_NF_A0_C
<B>	sdu	IPFRAME_NF_A0
<C>	sdu	IPFRAME_NF_A0_C
<D>	sdu	IPFRAME_NF_A0

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PROT
	sdu	IPPACKET_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

History:

20-Sep-2000	MT	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

## Modification of PPP link (PPP100-PPP109)

### PPP100:  turn header compression off

Description:

PPP initiates a new IPCP negotiation which turns off header compression.

Variants:

<A>....<B>

Preamble:

<A>PPP093A

<B>PPP043A

MMI		PPP		DOWNLINK

|		|		|

(1)	|	PPP_MODIFICATION_REQ	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfRequest70)	|

|		*	==========================>	*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfAck70)	|

|		*	<==========================	*

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfRequest61)	|

|		*	<==========================	*

(7)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfReject61F)	|

|		*	==========================>	*

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(9)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(10)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfRequest71)	|

|		*	<==========================	*

(11)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfNAK71)	|

|		*	==========================>	*

(12)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(13)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(14)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfRequest73)	|

|		*	<==========================	*

(15)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfAck73)	|

|		*	==========================>	*

(16)	|	PPP_MODIFICATION_CNF	|		|

*	<==========================	*		|

(17)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(18)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- PPP_MODIFICATION_REQ
	ppp_hc	PPP_HC_OFF
	msid	MSID_DUMMY

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfRequest88

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfAck88

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfRequest61

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfReject61F

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfRequest71

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfNAK71

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfRequest73

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfAck73

- PPP_MODIFICATION_CNF
	ppp_hc	PPP_HC_OFF
	msid	MSID_DUMMY

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

History:

18-Oct-2000	STW	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

## Transparent packet transfer (PPP110-PPP119)

### PPP110:  Transparent packet transfer uplink and downlink

Description:

PPP packet transfer uplink and downlink.

Preamble:

PPP014

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfRequest1)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(LCP_ConfRequest1)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	DTI2_DATA_TEST_REQ	|		|

|	(LCP_ConfRequest50)	|		|

*	==========================>	*		|

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfRequest50)	|

|		*	==========================>	*

(6)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(7)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(8)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(9)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPFRAME_TRIPLE)	|

|		*	<==========================	*

(10)	|	DTI2_DATA_TEST_IND	|		|

|	(IPFRAME_NF_A0)	|		|

*	<==========================	*		|

(11)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(12)	|	DTI2_DATA_TEST_IND	|		|

|	(IPFRAME_NF_B0)	|		|

*	<==========================	*		|

(13)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(14)	|	DTI2_DATA_TEST_IND	|		|

|	(IPFRAME_NF_C0)	|		|

*	<==========================	*		|

(15)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(16)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest50

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest50

- DTI2_READY_IND
	link_id	LINK_ID_PROT

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_TRIPLE

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_NF_A0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_NF_B0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PROT
	parameters	DTI_PARAMETER_PEER
	sdu	IPFRAME_NF_C0

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

18-Oct-2000	STW	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality
28-Jan-2002	STW	Change Flow Control order

## Link termination in transparent mode

### PPP120: Termination of transparent link

Description:

Termination of PPP link in transparent mode.

Variants:

<A>....<B>

Preamble:

<A>PPP014

<B>PPP110

MMI		PPP		DOWNLINK

|		|		|

(1)	|	PPP_TERMINATE_REQ	|		|

*	==========================>	*		|

(2)	|	DTI2_DISCONNECT_IND	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DISCONNECT_REQ	|

|		*	==========================>	*

(4)	|	PPP_TERMINATE_IND	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- PPP_TERMINATE_REQ
	lower_layer	PPP_LOWER_LAYER_DOWN

- DTI2_DISCONNECT_IND
	link_id	LINK_ID_PROT
	cause	DTI_CAUSE_NORMAL_CLOSE

- DTI2_DISCONNECT_REQ
	link_id	LINK_ID_PEER
	cause	DTI_CAUSE_NORMAL_CLOSE

- PPP_TERMINATE_IND
	ppp_cause	PPP_TERM_OK_MMI

History:

23-Oct-2000	STW	Initial
22-Jun-2001	XOF	Channel ID added
15-Sep-2001	XOF	Build DTILIB functionality

25-Oct-2002	STW	Cause concept

## Link reestablishment

### PPP130: Reestablishment in client mode

Description:

Shows Reestablishment in client mode with PAP authentification, ACFC (Address-and-Control-Field-Compression) am PFC (Protocol-Field-Compression). Magic number is rejected by PPP client.

Variants:

<A>....<C>

Preamble:

<A>PPP082A

<B>PPP082B

<C>PPP082C

MMI/UPLINK		PPP		DOWNLINK

|		|		|

(1)	|	PPP_ESTABLISH_REQ	|		|

|	(client mode)	|		|

*	==========================>	*		|

- (2)	|	DTI2_CONNECT_IND	|		|
	*	<==========================	*		|

- (3)	|		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- (4)	|	DTI2_CONNECT_RES	|		|
	*	==========================>	*		|

(5)	|	PPP_DTI_CONNECTED_IND	|		|

|	(Prot)	|		|

*	<==========================	*		|

- (6)	|		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

(7)	|	PPP_DTI_CONNECTED_IND	|		|

|	(Peer)	|		|

*	<==========================	*		|

(8)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(9)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(10)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfRequestMag)	|

|		*	<==========================	*

(11)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfRequest3)	|

|		*	==========================>	*

(12)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(13)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(14)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCPConfigureRejMag)	|

|		*	==========================>	*

(15)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(16)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfRequestAccm)	|

|		*	<==========================	*

(17)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ConfAccAccm)	|

|		*	==========================>	*

(18)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(19)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(20)	|		|	DTI2_DATA_TEST_IND	|

|		|	(LCP_ConfAck3)	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- PPP_ESTABLISH_REQ
	mode	PPP_CLIENT
	mru	PPP_MRU_DEFAULT
	ap	PPP_AP_PAP
	login	AUTH_STRUCT0
	accm	PPP_ACCM_DEFAULT
	rt	PPP_RT_DEFAULT
	mc	PPP_MC_DEFAULT
	mt	PPP_MT_DEFAULT
	mf	PPP_MF_DEFAULT
	ppp_hc	PPP_HC_OFF
	ip	PPP_IP_DYNAMIC
	dns1	PPP_DNS1_DYNAMIC
	dns2	PPP_DNS2_DYNAMIC
	peer_channel	PEER_CHANNEL0
	protocol_channel	PROTOCOL_CHANNEL0
	peer_direction	DTI_CHANNEL_TO_LOWER_LAYER
	prot_direction	DTI_CHANNEL_TO_HIGHER_LAYER
	peer_link_id	LINK_ID_PEER
	prot_link_id	LINK_ID_PROT

- DTI2_CONNECT_IND
	link_id	LINK_ID_PROT
	version	DTI_VERSION_10

- DTI2_CONNECT_REQ
	link_id	LINK_ID_PEER
	version	DTI_VERSION_10

- DTI2_CONNECT_RES
	link_id	LINK_ID_PROT
	version	DTI_VERSION_10

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PROT

- DTI2_CONNECT_CNF
	link_id	LINK_ID_PEER
	version	DTI_VERSION_10

- PPP_DTI_CONNECTED_IND
	connected_direction	PPP_DTI_CONN_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequestMag

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequest3

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCPConfigureRejMag

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfRequestAccm

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfAccAccm

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ConfAck3

History:

19-Jan-2001	STW	Initial

22-Jun-2001	XOF	Channel ID added

12-Jul-2001	STW	Change primitve order

17-Sept-2001	XOF	Change to DTILIB

## IP address negotiation (special cases)

### PPP140: Server IP address in client mode allowed

Description:

If the server sends its IP address in client mode PPP acknowledged it.

Preamble:

PPP031

MMI/UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPReq0_C)	|

|		*	==========================>	*

(2)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfRequest3_C)	|

|		*	<==========================	*

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfAck3_C)	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPNack0_C)	|

|		*	<==========================	*

(9)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPReq1_C)	|

|		*	==========================>	*

(10)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(11)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(12)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPAck1_C)	|

|		*	<==========================	*

(13)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(14)	|	PPP_ESTABLISH_CNF	|		|

*	<==========================	*		|

(15)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(16)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq0_C

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfRequest3_C

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfAck3_C

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPNack0_C

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq1_C

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPAck1_C

- DTI2_READY_IND
	link_id	LINK_ID_PROT

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_DUMMY
	ip	IP_ADDR
	dns1	PPP_DNS1_DYNAMIC
	dns2	PPP_DNS2_DYNAMIC

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

History:

19-Jan-2001	STW	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP141: Rejection of dynamic server IP address in client mode

Description:

If the server sends a dynamic server IP address in client mode PPP must reject it.

Preamble:

PPP031

MMI/UPLINK		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPReq0_C)	|

|		*	==========================>	*

(2)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfRequest71)	|

|		*	<==========================	*

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfReject71)	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPNack0_C)	|

|		*	<==========================	*

(9)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPReq1_C)	|

|		*	==========================>	*

(10)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(11)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(12)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPReq2_C)	|

|		*	<==========================	*

(13)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCPAck2_C)	|

|		*	==========================>	*

(14)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(15)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(16)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCPAck1_C)	|

|		*	<==========================	*

(17)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(18)	|	PPP_ESTABLISH_CNF	|		|

*	<==========================	*		|

(19)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(20)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq0_C

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfRequest71

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfReject71

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPNack0_C

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq1_C

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPReq2_C

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPAck2_C

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCPAck1_C

- DTI2_READY_IND
	link_id	LINK_ID_PROT

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_OFF
	msid	MSID_DUMMY
	ip	IP_ADDR
	dns1	PPP_DNS1_DYNAMIC
	dns2	PPP_DNS2_DYNAMIC

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

History:

19-Jan-2001	STW	Initial
22-Juni-2001	XOF	Channel ID added
15-Sept-2001	XOF	Build DTILIB functionality

### PPP142: Allow always static DNS addresses in server mode

Description:

IPCP configuration in server mode. If the Network does not respond with Protocol Configuration Options (PCO) PPP ackowledges static DNS addresses. PPP receives a CCP (Compression Control Protocol) configure request and sends a LCP protocol reject. The IPCP negotiation procedure continues.

Preamble:

PPP033A

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CCP_ConfRequest59)	|

|		*	<==========================	*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ProtoReject59)	|

|		*	==========================>	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfRequest65)	|

|		*	<==========================	*

(5)	|	PPP_PDP_ACTIVATE_IND	|		|

*	<==========================	*		|

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(7)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(8)	|	PPP_PDP_ACTIVATE_RES	|		|

*	==========================>	*		|

(9)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfRequest62)	|

|		*	==========================>	*

(10)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(11)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfAck65)	|

|		*	==========================>	*

(12)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfAck62)	|

|		*	<==========================	*

(13)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(14)	|	PPP_ESTABLISH_CNF	|		|

*	<==========================	*		|

(15)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(16)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(17)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	CCP_ConfRequest59

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ProtoReject59

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfRequest65

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_EXPECTED
	sdu	PCO_LIST_IND_CHAP_STATIC

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_VJ
	msid	MSID_EXPECTED
	ip	PPP_IP_141_64_21_2
	sdu	PCO_LIST_RES_EMPTY

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfRequest62

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfAck65

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfAck62

- DTI2_READY_IND
	link_id	LINK_ID_PROT

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_VJ
	msid	MSID_EXPECTED
	ip	PPP_IP_141_64_21_2
	dns1	PPP_PDNS_141_64_24_129
	dns2	PPP_SDNS_141_64_254_131

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

19-Jan-2001	STW	Initial
22-Juni-2001	XOF	Channel ID added
15-Sep-2001	XOF	Build DTILIB functionality

25-Oct-2002	STW	Cause concept

### PPP143: Reject dynamic DNS addresses in server mode if network does not answer

Description:

IPCP configuration in server mode. If the Network does not respond with Protocol Configuration Options (PCO) PPP rejects dynamic DNS addresses. PPP receives a CCP (Compression Control Protocol) configure request and sends a LCP protocol reject. The IPCP negotiation procedure continues.

Preamble:

PPP033A

MMI		PPP		DOWNLINK

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CCP_ConfRequest59)	|

|		*	<==========================	*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(LCP_ProtoReject59)	|

|		*	==========================>	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfRequest61)	|

|		*	<==========================	*

(5)	|	PPP_PDP_ACTIVATE_IND	|		|

*	<==========================	*		|

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(7)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(8)	|	PPP_PDP_ACTIVATE_RES	|		|

*	==========================>	*		|

(9)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfRequest62)	|

|		*	==========================>	*

(10)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(11)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfReject61)	|

|		*	==========================>	*

(12)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfAck62)	|

|		*	<==========================	*

(13)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(14)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(15)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfRequest89)	|

|		*	<==========================	*

(16)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfNAK89)	|

|		*	==========================>	*

(17)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(18)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(19)	|		|	DTI2_DATA_TEST_IND	|

|		|	(IPCP_ConfRequest91)	|

|		*	<==========================	*

(20)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(IPCP_ConfAck91)	|

|		*	==========================>	*

(21)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(22)	|	PPP_ESTABLISH_CNF	|		|

*	<==========================	*		|

(23)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(24)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(25)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	CCP_ConfRequest59

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	LCP_ProtoReject59

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfRequest61

- PPP_PDP_ACTIVATE_IND
	ppp_hc	PPP_HC_VJ
	msid	MSID_EXPECTED
	sdu	PCO_LIST_IND_CHAP

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- PPP_PDP_ACTIVATE_RES
	ppp_hc	PPP_HC_VJ
	msid	MSID_EXPECTED
	ip	PPP_IP_141_64_21_2
	sdu	PCO_LIST_RES_EMPTY

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfRequest62

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfReject61DNS

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfAck62

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfRequest89

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfNAK89

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfRequest91

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_PEER
	parameters	DTI_PARAMETER_PEER
	sdu	IPCP_ConfAck91

- DTI2_READY_IND
	link_id	LINK_ID_PROT

- PPP_ESTABLISH_CNF
	mru	PPP_MRU_DEFAULT
	ppp_hc	PPP_HC_VJ
	msid	MSID_EXPECTED
	ip	PPP_IP_141_64_21_2
	dns1	PPP_PDNS_DYNAMIC
	dns2	PPP_SDNS_DYNAMIC

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PEER

- DTI2_READY_IND
	link_id	LINK_ID_PEER

- DTI2_GETDATA_REQ
	link_id	LINK_ID_PROT

History:

22-Jan-2001	STW	Initial
22-Jun-2001	XOF	Channel ID added
15-Sep-2001	XOF	Build DTILIB functionality
28-Jan-2002	STW	Change Flow Control order

25-Oct-2002	STW	Cause concept

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |