# Using DSP/BIOS with OMAP1510 DSP clock

Arnie Reynoso	Software Applications, TI Santa Barbara

- Overview

The intent of this document is to allow user’s of the OMAP1510 to configure the DSP clock for use with DSP/BIOS within Code Composer Studio (CCS) 2.0 Beta.  DSP/BIOS clock timers are by default configured for a TMS320C5510 core. Therefore the following steps are necessary to use the clock on the OMAP1510 DSP core.  This is an interim solution.  For the final release of CCS 2.0, the clocks will be configurable from the Configuration (CDB) files.

- Description of Interim BIOS Clock Module (CLK) for OMAP510

Two of OMAP1510 "DSP OS Timers" are being used to implement DSP/BIOS clock module functionality:

- Timer 1 is programmed for a 1 msec interrupt rate, which drives the DSP/BIOS low-resolution clock tick.   Calls to CLK_getltime() will return this timer interrupt count.   DSP/BIOS API calls that have timeouts, user-defined CLK functions, and PRDs all run based upon this time base.

- Timer 2 does not generate an interrupt, but free runs, and rolls over on max count. This timer supports the high resolution time functionality.

The following CLK functionality has been verified:

- Timeouts for blocking calls (SEM_pend, SIO_reclaim, TSK_sleep, etc.)

- User CLK functions and PRDs

- CLK_gethtime() – high-resolution time

- CLK_getltime() – low-resolution time

- CLK_countspms() – low-resolution timer counts per msec

- CLK_getprd() – low-resolution timer re-load value

- CLK_itime() – an internal API used for instrumentation.

The following API were changed to support the OMAP1510 DSP timers:

- CLK_start() - sets up the two OMAP1510 timers and starts them.

- CLK_F_getfhtime() - reads the 32-bit time value from the timer peripheral.

- CLK_F_getshtime() - always returns 0 for safety (if configuration is proper, this function should not be called).

- CLK_F_run() - clone of function for C5510 timers, with appropriate IER bit masked for Helen.

- CLK_getprd() – clone of CLK_getprd(), but residing in clk_omap.a55l.

- CLK_countspms() – clone of CLK_countspms(), but residing in clk_omap.a55l.

- CLK_helenImport() - a do-nothing function that should be called by the application to ensure that proper CLK routines for Helen are used.

- CLK_startup - new assembly macro to call Helen timer startup code.

- CLK_getprd - new assembly macro to return the period register re-load value.

- CLK_countspms - new assembly macro to indicate low-resolution clock counts per msec.

- Project/Application Configuration

Before building your project, there are a few basic settings to configure so that Code Composer Studio and DSP/BIOS utilizes the OMAP1510 clock on the DSP core.

Project File:

In the Project, you will need to remove the DSP/BIOS generated Linker command file (e.g., testcfg.cmd) from the project.  Right-click the file and select Remove from project if it’s already been added to the project.

Create a new linker command file (e.g., test.cmd) with the following lines:

-lclk_omap.a55l

-ltestcfg.cmd

Add this new linker command file to the project.

Linker Options:

In the Code Composer Studio menu bar, select Project->Options and click on the Compiler tab.  Under the preprocessor category in the Include Search Path type: C:\ti\c5500\bios\omap1510

- If Code Composer Studio was installed in another directory other than the default, correct the path accordingly (e.g. C:\DEFAULTDIR\ c5500\bios\omap1510).

Click on the Linker tab.  In the Include Search Path type: C:\ti\c5500\bios\omap1510

Make sure that Large Memory Model (-ml) is selected in the Compiler tab (Small Model is not currently support).

BIOS Options:

Open your project’s DSP/BIOS configuration (.cdb) file, and go to the property page for “Global Settings”.  Select ‘other’ as the Chip Support Library (CSL).  Select ‘LARGE’ as the Memory Model.

Go to “Memory Section Manager”. Modify the memory address according to the OMAP1510 DSP core memory configuration.

Go to the properties page for “Clock Manager”.  Check to box Directly Configure on-chip timer registers.  Set PRD Register to 65535.

Go to “Hardware Interrupt Service Routine Manager (HWI)”.  Select the properties page for “HWI_INT23”.  Type: CLK_F_isr in the function box.

Application Option:

The following call must be added to the user’s application in the main() function:

- CLK_helenImport();

This is to ensure that proper CLK routines for OMAP1510 are used.

- Verification of Timer

To properly verify the setup for the timers on the OMAP1510 are working, perform the following test.

Copy the clktest example that is located in the c:\ti\examples\evm5510\bios\clktest directory into c:\ti\myprojects.

Start Code Composer Studio 2.0 Beta

In the Code Composer Studio menu bar, select Project->Open.  Browse to the c:\ti\myproject\clktest\clktest.pjt file and select Open.

In the Project, you will need to remove the DSP/BIOS generated Linker command file (e.g. clktestfcg.cmd) from the project.  Right-click clktestcfg.cmd file and select Remove from project.

Create a new linker command. From the menu bar, select File->New->Source File.  In the new Untitled window add the following lines:

-lclk_omap.a55l

-lclktestcfg.cmd

Save the file.  In the menu bar, select File->Save.  In the File name box type: clktest.  Set the Save as type to: TI Command Language File (*.cmd) and select Save.

Add this file (clktest.cmd) to the project.  From the menu bar, select Project->AddFiletoProject.  Browse to clktest.cmd and click Open.

In the menu bar, select Project->Options and click on the Compiler tab.  Under the Preprocessor category in the Include Search Path type: C:\ti\c5500\bios\omap1510

- If Code Composer Studio was installed in another directory other than the default, correct the path accordingly (e.g., C:\DEFAULTDIR\c5500\bios\omap1510).

Set Large Memory Model (-ml) in the Compiler tab under Advanced category.

Click on the Linker tab.  In the Include Search Path type: C:\ti\c5500\bios\omap1510.  Correct path accordingly if installed in a different directory.

Open the project’s DSP/BIOS configuration (clktest.cdb) file, and go to the property page for “Global Settings”.  Select ‘other’ as the Chip Support Library (CSL).  Select ‘LARGE’ as the Memory Model.

In “Memory Section Manager”, go to the property page for “SARAM”.  Set the base to 0x10100 and the len to 0xff00.

Go to the properties page for “Clock Manager”.  Check to box Directly Configure on-chip timer registers.  Set PRD Register to 65535.

Go to “Hardware Interrupt Service Routine Manager (HWI)”.  Select the properties page for “HWI_INT23”.  Type: CLK_F_isr in the function box.

Open the application source code (clktest.c).  Add the following in the main() function:

CLK_helenImport();

Rebuild the application.  In the menu bar, select Project->RebuildAll.

Load the built file. In the menu bar, select File->Load Program.  Browse to c:\myproject\clktest\Debug\clktest.out, click on Open.

In the menu bar, select DSP/BIOS->Message Log.  A Log window will open up.

Run the program.  In the menu bar, select Debug->Run

In the Log window the following messages will appear:

clktest examples started

task going to sleep for 1 second

… awake! Time is : 1000 ticks

Step similar to the above can be used for any DSP/BIOS application that requires timer support on the OMAP1510.